3GPP TSG RAN WG2 #121bis-e		R2-23xxxxx

Electronic meeting, 17th – 26th April 2023

Title:	Draft LS on report of switching periods in Rel-18 UL Tx switching

Response to:

Release:	Release 18

Work Item:	NR_MC_enh-Core

Source:	NTT DOCOMO, INC. [to be RAN WG2]

To:	RAN WG1, RAN WG4

Cc:	none

Contact person:	Riki Okawa

riki.ookawa.rp@nttdocomo.com

Attachments:	none

1	Overall description

RAN2 has discussed introduction of UE capability for length of switching periods. RAN2 has taken following RAN4 agreement in RAN4#104-e into account.

Agreement: On the length of switching period: - For UL switching period with Tx switching across 3 or 4 bands, RAN4 agreed to reuse the same set of values as in Rel-16/17, i.e., {35 us, 140 us, 210 us} for UL CA and SUL. - The length of switching period is applied per band pair for each band combination. - For each band pair, the switching period can be the same or different for 1Tx-2Tx switching and 2Tx-2Tx switching based on UE reporting, which is similar as in Rel-17. - Note: For UE reporting different periods for 1Tx-2Tx switching and 2Tx-2Tx switching for a band pair, similar to Rel-17, it is RAN4 understanding that the 2Tx-2Tx switching period is applied when 2Tx-2Tx switching mode is configured. - For the same band pair, RAN4 has not concluded on whether the same or a different value can be reported for the specific band pair supporting Tx switching across 3 or 4 bands in Rel-18 compared to Tx switching across 2 bands specified in Rel-16/17.

RAN2 could not achieve conclusion, but has agreed an intention below in RAN2#121bis-e:

In support of RAN4 agreement, RAN2 intend to introduce support for two per-band-pair UE capabilities, a length of a switching period, for 1Tx-2Tx switching (like Rel-16) and that for 2Tx-2Tx switching (like Rel-17).

Question 1. (To RAN1 and RAN4)

RAN2 respectfully asks RAN1 and RAN4 to take above agreement on RAN2 intention into account and asks for feedback if there is any issue.

RAN2 could not conclude whether the UE needs to explicitly report if it supports 2Tx-2Tx switching for every band pair used for Rel-18 UL Tx switching.

RAN2 is not sure which is the correct understanding:

- The UE always supports 2Tx-2Tx switching on a pair of bands if the UE supports 2 layers/ports UL MIMO on the two bands

- The UE may not support 2Tx-2Tx switching on a pair of bands even if the UE supports 2 layers/ports UL MIMO on the two bands (i.e., per-band-pair UE capability to report whether to support 2Tx-2Tx switching is needed, e.g. based on the presence/absence of 2Tx-2Tx switching period).

Question 2. (To RAN4)

RAN2 respectfully asks RAN4 to take below RAN2 assumption into account and and asks for feedback if there is any issue:

RAN2 assume for the band pair supporting 2Tx-2Tx switching, the UE always support 1Tx-2Tx switching.

Question 3. (To RAN4)

RAN2 respectfully asks RAN4 to feedback if there is any issue:

The UE reports whether it supports 2Tx-2Tx switching via per-band-pair UE capability.

RAN2 has discussed how the gNB knows which of the reported switching periods (for 1Tx-2Tx switching or for 2Tx-2Tx) should be applied for every switching but could not conclude.

Question 4. (To RAN4)

RAN2 respectfully asks RAN4 which of the options below matches RAN4 understanding on the selection of applied switching periods when both switching periods of 2Tx-2Tx switching and 1Tx-2Tx switching can be reported for the same band pair.

Option 1: Based on implicit rules, e.g., 2Tx-2Tx switching period is only applicable when performing UL switching between two bands (e.g. 2P+0P<=>0P+2P) and 1Tx-2Tx period is applied for the other switching cases (e.g. UL Tx switching that involves 3 or 4 bands, such as band A + band B<=>band C, band A+ band B <=>band C + band D)-.

Option 2: Based on explicit RRC configuration, i.e., gNB configures which period is applied per band pair.

Question 5. (To RAN1)

RAN2 respectfully asks RAN1 to take above discussion on RAN2 and question to RAN4 into account and asks for feedback if there is any issue.

2	Actions

To RAN WG1

ACTION: 	RAN2 respectfully requests RAN1 to take above discussion and questions into account and asks feedback if there is any issue.

To RAN WG4

ACTION: 	RAN2 respectfully requests RAN4 to provide feedback to the above questions.

3	Dates of next TSG RAN WG2 meetings

TSG RAN WG2 Meeting #122	22nd - 26th May 2023	Incheon, KR

TSG RAN WG2 Meeting #123	21st – 25th August 2023	Toulouse, FR