3GPP RAN WG2 Meeting #122	R2-230xxxx

Incheon, Korea, May 22nd – 26th, 2023

Agenda Item:	7.6.4

Source:	Huawei, HiSilicon (Rapporteur)

Title:	Report of [Post122][113][IoT NTN Enh] Discontinuous coverage (Huawei)

Document for:	Discussion, Decision

# Introduction

This document is intended to discuss possible enhancements for discontinuous coverage (e.g. paging enhancements, RRC connection release enhancements, UE behaviour when in discontinuous coverage) based on contributions submitted to RAN2#122, as per the following:

- [Post122][113][IoT NTN Enh] Discontinuous coverage (Huawei)

Scope: Discuss possible enhancements for discontinuous coverage (e.g. paging enhancements, RRC connection release enhancements, UE behaviour when in discontinuous coverage)

Intended outcome: Summary of the email discussion

Deadline for companies' feedback:   Monday 2023-08-08  10:00 UTC

Deadline for rapporteur's summary: Thursday 2023-08-10 10:00 UTC

# Contact information

| Company | Email address |
| --- | --- |
| NEC | xie_zonghui@nec.cn |
| Qualcomm Inc. | bshrestha@qti.qualcomm.com |
| Transsion Holdings | Wen.wu5@transsion.com |
| CATT | zhangxiangdong@catt.cn |
| Apple | yuqin_chen@apple.com |
| Google | mhtao@google.com |
| MediaTek | Abhishek.Roy@mediatek.com |
| ZTE | lu.ting@zte.com.cn |
| Lenovo | xumin13@lenovo.com |
| Ericsson | emre.yavuz@ericsson.com |
| OPPO | lihaitao@oppo.com |
| Xiaomi | Jiangxiaowei@xiaomi.com |
| Nokia | Srinivasan.selvaganapathy@nokia.com |
| Samsung | c.khirallah@samsung.com |

# Background

In RAN2#121 the following agreements were made:

Agreements: - RAN2 can continue to check whether dedicated RRC signalling can be used for providing satellite information corresponding to discontinuous coverage. - RAN2 will support enhancements in paging and eDRX, in alignment with the work in SA2 and CT1. FFS on the details - RAN2 may consider enhancements for connected UE upon detecting discontinuous coverage (e.g., suspend RLM, RLF detection, and RRC re-establishment process) - Companies supporting the store and forward approach can bring a proposal to the plenary for TEI18 or for updating the WID

In RAN2#121bis the following agreements were made:

Agreements: - RAN2 will not introduce any enhancement to allow a UE in RRC Connected to stay in RRC_CONNECTED during/after a coverage gap (e.g. suspend RLM/RLF, activation time in RRC Reconfiguration, CHO enhancement) - RAN2 to introduce enhancement to RRC Release using one of the following options (FFS which one): - Explicit RRC Release using a new RRC Release cause - UE Autonomous release (e.g. timer based or upon detection of coverage gap)

# Discussion

## Discussion of paging enhancements

The following summarizes contributions from RAN2#122 discussion paging ehancements:

| References | Proposals |
| --- | --- |
| [1] | Proposal 1: Introduce paging offset to solve the misfit issue of the PH/PTW which is currently determined by UE ID. Proposal 2: Consider the PTW extension solution as a complementary solution. Proposal 3: UE assistance information via NAS signaling (unreachability period in TAU) can be used for PTW adjustment when UE finds out the mismatch issue. Proposal 4: Send an LS to SA2/CT1 on unevenly distributed coverage period caused by unevenly distributed satellite from multiple satellites. |
| [2] | Proposal 7：UE/NW autonomous adjustment can be considered for paging and eDRX enhancement |
| [3] | Proposal 2: There is no RAN impacts to adjust the PTW configuration. It is up to UE and eNB implementation to skip the PTW within the discontinuous coverage. |
| [5] | Proposal 2：The network can predict multiple coverage windows by implementation and takes them into account when paging a UE. |
| [6] | Proposal 5: Update the PTW if it falls in a coverage gap by applying an offset. FFS how to determine/configure the offset. Proposal 6: Send an LS to SA2 indicating that RAN2 <have agreed/are considering> shifting the PTW if it falls in a coverage gap, asking what the content/granularity of “UE unreachability period is, whether it can be provided to the gNB, and whether adjusting the PTW is feasible from SA2 point of view. |
| [8] | Proposal 4: RAN2 confirm the PTW position should be configurable (FFS to configure one or multiple PTW positions, FFS the configuration is an offset or replacement of the calculated PTW position), send LS to SA2/CT1 asking for collaboration. |
| [9] | Proposal 5: The PSM and eDRX configurations can be configured to align with the estimated UE unreachability period. The configuration is out of RAN2’s scope. Proposal 6: Network can extend the paging before/after the coverage window if the UE does not respond to paging within the estimated coverage window. Proposal 7: UE can extend the paging monitoring outside the estimated coverage window if radio coverage is available. UE may report to the network to realign the paging monitoring and coverage windows. Proposal 8: UE may report to the network to realign the paging monitoring and coverage windows if UE notices there is radio coverage before or after the expected reachability period. Proposal 9: RAN2 to discuss how to handle UE movement within the same TA during discontinuous coverage. |
| [12] | Proposal 1: In order to ensure that the UE can be reachable when paging arrives, the PTW should aligns with the time duration of coverage and the following two options can be taken into further consideration: Option 1: both UE and network adjust PTW based on a predefined rule Option 2: network provides multiple PTW info Proposal 2: From RAN2 point of view, the existing PSM mechanism needs to be enhanced in order to get better power effect. Proposal 3：The PSM starting mechanism needs to be modified to adapt to the scenario of discontinuous coverage. Proposal 4: A time threshold needs to be introduced for UE to determine whether to start T3324 timer. |
| [13] | Proposal 3: The CN should be aware of the UE’s discontinuous coverage period to CN to avoid paging issues. Proposal 4: The satellite assistance information or the discontinuous coverage information predicted by UE can be provided to CN for paging. |
| [14] | Proposal 1: The issue that UE ends up with no PTW in the in-coverage period should be addressed. Proposal 2: AMF configures the start position (PH) of PTW window. |
| [15] | Proposal 1a: If legacy eDRX is used for keeping alignment between UE and NW during discontinuous coverage, in order to align the starting time of PTW with the out-of-coverage period or unreachability period, it’s suggested to introduce a configurable offset to shift the starting time of PTW. Proposal 1b: The out-of-coverage period or unreachability period should be informed to RAN, e.g., from core network node, to assist RAN to provide a more appropriate paging schedule for UE in idle mode. |

Rapporteur understands the issue mainly comes from that it may be difficult for the CN to align the the configuration of eDRX/PTW with the distribution of coverage windows, leading to no PTW in a coverage window. The potential causes for the issue are as follows:

- The coverage window may be unevenly distributed while the eDRX/PTW is evenly configured

- The position of PTW within the eDRX cycle is determined by the formular rather than configured by CN

- The UE may move within one TA, so that the previous discontinuous coverage information referred to by CN for configuring the PTW is outdated

Solutions are provided by some of the proposals to solve this issue while there is also opinion that this can be left to NW implementation.

Based on the above, the following question is asked:

Q1a)  Which of the following options do you prefer for paging enahncements?

- Option 1: The start position of PTW can be adjusted by gNB and UE based on the discontious coverage information, FFS by an configured offset or by pre-defined rule (The gNB needs to be aware of the discontinuous coverage information)

- Option 2: Leave it to NW implementation

- Option 3: Others (Please describe)

| Company | Prefered Option | Additional comments |
| --- | --- | --- |
| NEC | Option1 with comments | We prefer option1, because UE and network has to be aligned with regarding to where the adjusted PTW is. Suggest to change gNB=> NW in the sentence “(The gNB needs to be aware of the discontinuous coverage information)” or delete this sentence. For paging enhancement purpose, it can be either gNB or CN needs to be aware of the disconitnous coverage information depending on who adjusts the PTW. |
| Qualcomm | Option 2 | Core network knows UE’s ID and when exactly the eDRX/PTW starts and when UE will become unreachable. PTW start error is rather small by 2.56s. SA2/CT1 is working on UE reporting unavailability period to resolve issue. Network does not need to accept the same value reported by UE. It can also set proper values for unreachability period to align with eDRX. Network could configure eDRX properly or with shorter eDRX cycles. Network can also ask UE to send “in-coverage” indication at the end of unavailability period or can configure PSM or can configure proper value of periodic TAU timer. During unreachability period, UE can go to sleep regardless of coverage or PTW. This is the one enhancement we should work in our scope. PTW/eDRX is configured by core network so UE or RAN should not be adjusting it autonomously. Therefore, this whole enhancement should be mainly in the scope of SA2. |
| Transsion Holdings | Option1 with comments Or Option3 | We think it is hard for the network to configure an offset to shift the start position of PTW considering an appropriate granularity and the various unreachability period. As the eDRX cycle is in hyperframes(10.24s), and the unreachability period maybe larger then the hyperframe, we think it is better to configure an PH offset to shift the PH by an offset if the calculated PH has unreachability period. Or Option3: we just exclude the PH in the eDRX cycle if the PH has unreachability period when calculating the PH based on the legacy equation. |
| CATT | Option 1 with comments | We should avoid modifying the existing calculations of PH,PO,or PTW. And we also think it is not necessary to make such modification, because the UE and the eNB (the eNB is the acutual entity to transmit the paging message in the radio interface) know the discoverage information, so they can both delay the determined PO and the PTW to or after the next coverage starting time, based on a pre-defined rule. It is difficult to configure a offset applicable to all the UEs, so we don’t think the red part in “FFS by an configured offset or by pre-defined rule” is needed. |
| Apple | Option 1, Option 3 can be considered | According to the formula how UE determines the Paging Hyper-frame, the HF(s) for UE(s) are scattered in time domain based on UE ID. The duration of one HF is 10.24s. For NB-IoT, the eDRX cycle can be up to 1024 HF(s) which is around 2.9 hours. It could result in a situation where the paging HF corresponding to a UE is not aligned with actual satellite coverage. Thus, we think the MUSIM-like solution can be considered, introducing an offset to current paging HF formula. H-SFN mod TeDRX,H= (UE_ID_H mod TeDRX,H) By the way, we also think the “gNB” in Option 1 can be changed to “network”. |
| Google | Option 1 with comment | Option 1 is feasible as long as the UE, the core network, and the gNB share the same understanding regarding UE’s unreachability periods. Probably we don’t need to introduce a specific offset to the paing HF formula or a specific offset to the UE ID, both UE and NW should be able to adjust/shift the PTW start position automously in the same way (e.g., backwarad for forward) if they share the same understanding regarding UE’s unreachability periods. |
| MediaTek | Option 2 | Support Qualcomm’s view that UE and eNB PTW/eDRX is configured by core network so UE or RAN should not be adjusting it autonomously. It is NOT in scope of RAN2. |
| ZTE | Option 1 | SA2 has agreed the following aspects to support discontinuous coverage in NAS layer, especially on how to (re)negotiate the unavailability period between UE and core network: - The UE may be able to determine, including considering current and expected future locations of the UE, a Start of Unavailability Period and/or Unavailability Period Duration for when it expects to be out of coverage. - The UE triggers a TAU procedure and includes an indication of upcoming loss of coverage in the TAU Request message to the MME. If the UE is able to determine a Start of Unavailability Period and/or Unavailability Period Duration it also includes them in the TAU Request message - If the UE determines an upcoming loss of coverage to the network no longer applies or determines a new Start of Unavailability Period or Unavailability Period Duration related to the upcoming loss of coverage, the UE sends a new TAU Request. - The MME may determine, if not provided by the UE, or update the Unavailability Period Duration. If the MME knows an Unavailability Period Duration (e.g. based on information available to the MME) for the UE, and the UE not include an Unavailability Period Duration or included an Unavailability Period Duration different to the Unavailability Period Duration known to the MME, the MME may use either the Unavailability Period Duration known to the MME or the Unavailability Period Duration from the UE as the Unavailability Period Duration. The MME should include the Unavailability Period Duration known to the MME in the TAU Accept. Furthermore, SA2 specification also indicates that, when MME provides the UE with timers (e.g. periodic TAU timer, extended idle mode DRX, and PSM mode configuration), it may also consider the Unavailability Period Duration (if available) and Start of Unavailability Period (if available). According to above information, we can assume the UE and core network can negotiate the suitable timer length for the related PSM/eDRX timers in the scenario of discontinuous network coverage. However, for example, even CN can configure a suitable eDRX cycle for the UE, the position of Paging Hyperframe (PH), PTW_start and PTW_end are fixed according to the formula given in TS 36.304 (mainly determined by eDRX cycle, PTW length and UE_ID), which may be not aligned with the end of the Unavailability Period (Start of Unavailability Period + Unavailability Period Duration). For example, they can be earlier than the end of the Unavailability Period. Therefore, if just following the existing calculation for PH and PTW_start and no introducing any enhancement, the UE would start monitoring PTW during Unavailability Period, this is undesired. This issue can be seen from the following example figure: The straightforward way to addressing this issue is to try to adjust the PTW_start to align with or after the end of the Unavailability Period. This may have impacts at least on the TS 36.304 specification. Here are some further considerations: - If UE and eNB can determine the offset between PTW_start and the end of the Unavailability Period, they can delay the PTW_start by this offset respectively and still keep consistence on PTW_start. By this way, there is no need for eNB to explicitly configure offset to the UE. But at least the eNB also needs to acquire the information of the Unavailability Period in advance, e.g., from CN. - However, even the Unavailability Period is a UE-specific parameters, it’s still possible for several or many UEs to be very close on the end of the Unavailability Period. Therefore, in order to distribute the PTW_start for different UEs, it may still need eNB to configure UE-speicifc offset (which may be similar or a bit larger than the calculated offset between the PTW_start and the end of the Unavailability Period). - We can understand some above comments for PH adjustment. Considering the granularity of Unavailability Period may be large, e.g., minutes or hours, the offset between PTW_start and the end of the Unavailability Period may be also large, e.g., larger than a Paging Hyperframe. Since the adjustment for PTW_start can only be within one PH, only to adjust PTW_start may be not enough. We may first need to align PH with the end of the Unavailability Period coarsely, and then to align PTW_start with the end of the Unavailability Period precisely. With such understanding, we are fine to also discuss the adjustment of PH. |
| Lenovo | Option 2 | UE can report to network its estimation of coverage interruption, and network can adjust configuration by implementation. |
| Ericsson | Option 1, please see the comments | We agree that UE and the network need to be aligned regarding when UE monitors for paging. Considering the complexity it may introduce we prefer solutions that require minimum, if oppossible no, impact on existing formulas that derive PTW start etc. We do not think leaving it to NW implementation would solve the problem onsidering scenarios where it would be hard to match (e)DRX cycles with coverage gaps. |
| OPPO | Option 2 | We share the same view as Qualcomm. NW implementation can solve this issue, and any enhancement of PTW/eDRX is in the scope of SA2/CT1. |
| Xiaomi | Option 3 | According to 36.304, the PTW starting PH and starting position in a PH are determined based on UE ID. It means that the start PH of PTW can be in any PH of the eDRX cylce. An example of PTW start position is shown in figure 1 below: Figure 1: An example of PTW start position It means that if network configures an improper eDRX cycle, UE may end up with no PTW in the in-coverage period as shown in figure 2 below: Figure 2: An example of PTW for out of coverage Observation: If network improperly configures the eDRX cycle and PTW length, UE may end up with no PTW during the in-coverage period, resulting in not being able to receive paging. Proposal: The issue that UE ends up with no PTW in the in-coverage period should be addressed. Based on the current progress on SA2, UE will report the upcoming discontinuous coverage information (e.g. start time + length, or length). So one option to solve this issue is that AMF sets the eDRX cycle length smaller than the remaining in-coverage period. In this way, network can ensure that there is at least one PTW during in coverage time. An example is shown in the figure 3 below: Figure 3: An example of PTW for in coverage However, the drawback of this option is that network has to configure a small eDRX cycle, which would lead to increased UE power consumption. For example, let us assume the intended eDRX cycle based UE traffic is 1 hour, and the in- coverage period occurs every 20 minutes with length 10 minutes. Based on the remaining in-coverage period, network configures the eDRX cycle close to 10 minutes. So UE will monitor 6 PTWs in one hour instead of the required 1 PTW per hour. To solve this issue, it would be better not to modify the eDRX cycle length, but to adjust the start position of PTW to fall into the in-coverage period, as illustrated in the following figure 4: Figure 4: An example of adjusting PTW for in coverage Regarding how to move the PTW into the in-coverage period, there are following three options: Option 1: AMF is responsible to move the PTW into the in-coverage period; Option 2: eNB is responsible to move the PTW into the in-coverage period. Option 3: UE and eNB move the PTW into the in-coverage period based on the specified rule. For option 1, AMF will tell UE the start position of PTW in the eDRX parameter, and tells eNB the start position of PTW through paging message. Then UE and eNB can align with each other on the PTW start position. And AMF can try to distribute PTW of different UEs evenly in time domain. To balance between indicated start position and UE-id based calculation, AMF can simply indicate the PH, and UE calculates the PF in the PH based on UE id. For option 2, eNB will tell UE the start position of PTW using RRC message. The issue with this option is that when UE reselects from one serving cell to another serving cell, the new serving cell doesn’t know the configured start position of the old serving cell. Besides, eNB needs AMF to tell the unreachability period in advance, and requires UE to connect to eNB to acquire the parameter. For option 3, the issue is related to how to define the rule. Completely rewrite the whole PO calculation fomula is not suggested. And even distribution of PTWs of different UEs in the whole time domain should be achieved. Otherwise, it will impact the paging capacity. Currently, we are not sure how we can predefine a simple rule to achieve this. Based on the above analysis, we prefer to go with option 1: Proposal: AMF configures the start position (PH) of PTW window. |
| Nokia | Option 2 | The current mechanism of PO and PTW calculation based on UE-ID to be maintained. If some changes needed to this based on awareness of discontinuous coverage it should be negotiated between UE and CN and modified values is transperant to RAN. We don’t think RAN changes needed for this. Moreover whether UE will be in-coverage during PTW or not also depends on UE mobility not only the discontinuous coverage pattern. The PTW changes needs to consider this aspect also. |
| Samsung | Option 1 (with comment) | We also understand companies views (above) on supporting Option 1. - The UE and network should be aligned on the Start of Unavailability Period and/or Unavailability Period Duration. - The UE and network can decide the offset required to adjust the PTW start position based on available information on the Start of Unavailability Period and/or Unavailability Period Duration. Moreover, - Option 1: The start position of PTW can be adjusted by gNB the network and UE based on the discontious coverage information, FFS by an configured offset or by pre-defined rule (The gNB needs to be aware of the discontinuous coverage information). |

Besides, it is also mentioned by some proposals that an LS is needed to SA2.

Q1b)  Do you think an LS to SA2 is needed?

| Company | Yes/No | Additional comments |
| --- | --- | --- |
| NEC | Yes |  |
| Qualcomm | Yes | As described in Q1a, coordination is needed with SA2. |
| Transsion Holdings | Yes |  |
| CATT | No with comments | If the pre-defined rule option is seleted by RAN2, there is no impacts on the SA2 specication, so no LS is needed. |
| Apple | Yes | RAN2 can reach out to SA2 on multiple issues and proposals (subject to RAN2 conclusions): 1) PH determination to align the paging HSF with the satellite coverage time duration 2) Multiple eDRX configurations to provide uneven eDRX(s) to support unevenly distributed coverage period from multiple satellites |
| Google | Yes |  |
| MediaTek | No | This is entirely in SA2 and we should let them do it in SA2. An LS is not needed. |
| ZTE | Yes or depends | It depends on what kind of agreements RAN2 can achieved. If these agreements have impacts on SA2, of course, a LS is needed. |
| Lenovo | Depends | Depends on the outcome of RAN2 discussion and whether it (if any) impacts SA2. |
| Ericsson | Yes |  |
| OPPO | Yes |  |
| Xiaomi | Yes |  |
| Nokia | Yes |  |
| Samsung | Yes | Agree that coordination with SA2 is needed. |

## Enhancements of RRC connection release

The following summarizes contributions from RAN2#122 discussing enhancements of RRC connection release:

| References | Proposals |
| --- | --- |
| [1] | Proposal 5: Do not introduce new release cause for coverage gap in RRCRelease message. |
| [2] | Proposal 5：No new RRC Release cause is needed for discontinuous coverage scenario. Proposal 6：UE Autonomous release enhancement for discontinuous coverage is not needed, if AMF/MME shares the UE unreachability period to RAN node. |
| [3] | Proposal 3: A new cause value ‘Release due to discontinuous coverage’ is introduced in RRCRelease message. Proposal 4: The UE can provide the out-of-coverage information to RAN node for e.g, optimization on RRC release or paging. Proposal 5: The UE out-of-coverage information can be kept by RAN node when the UE enters into idle mode. |
| [5] | Proposal 1: UE can indicate “RRC_IDLE” to the Network as the preferred RRC state when it is going to enter a coverage gap and start a timer, similar as the mechanism designed by R17 MUSIM. |
| [6] | Proposal 2: RAN2 to discuss what new behaviour is expected from the UE when NW releases the UE using a new cause e.g. “discontinuous coverage” in the following cases: Case 1: Release to RRC_IDLE Case 2: Release to RRC_INACTIVE |
| [8] | Proposal 1: Consider to support UE providing assistance information on being out-of-coverage to gNB for RRC connection management. Proposal 2: Consider to include remaining coverage time in out-of-coverage information. |
| [9] | Proposal 10: RAN2 to discuss support for UE request for RRC connection release based on DC estimation. FFS support for implicit RRC connection release. |
| [10] | Proposal 1 If the UE is able to predict when the discontinuous coverage starts, reuse NR MUSIM procedure to leave RRC_CONNECTED state where the UE indicates the out-of-coverage to network and starts an out-of-coverage timer. Proposal 2 Reuse NR MUSIM timer T346g behavior, i.e., upon expiry of the out-of-coverage timer, the UE performs the actions upon leaving RRC_CONNECTED, with release cause 'other'. Proposal 3 Further discuss the details on the values of the out-of-coverage timer and message to carry out-of-coverage information. |
| [11] | Proposal 3: UE autonomous release for discontinuous coverage is not pursued. Proposal 4: Explicit RRC Release using a new RRC Release cause for discontinuous coverage is introduced. Proposal 5: Certain idle mode procedures are paused if a UE receives an RRC release with the new RRC release cause. Proposal 6: The new RRC release cause is used to re-direct a UE from a non-discontinuous coverage network to a discontinuous coverage network. Proposal 7: Discontinuous coverage-related information is provided in a re-direct message when being re-directed to a discontinuous coverage network. |
| [13] | Proposal 5: The explict RRC release using a new RRC Release cause(e.g. coverage discontinuity) can be used before the covearage gap. |
| [15] | Proposal 2a: A new release reason, e.g., ‘Release due to discontinuous coverage’ as that introduced in RAN3, can be introduced in RRC release message for indicating UE to stop the subsequent AS layer processes after it is released to idle mode. Proposal 2b: An AS-NAS interaction (e.g., an indication from AS to NAS) also needs to be introduced for indicating UE to stop the subsequent NAS layer processes after it is released to idle mode due to discontinuous coverage. Proposal 2c: The legacy IE extendedWaitTime can be reused to stop the subsequent NAS layer processes after UE is released to idle mode due to discontinuous coverage. The extension to the value range of extendedWaitTime needs to be discussed. Proposal 2d: The UE in connected mode could provide out-of-coverage period or unreachability period information as an assistance to the network (eNB). |

During RAN2#121bis, the following agreement was made:

Agreement: RAN2 to introduce enhancement to RRC Release using one of the following options (FFS which one): - Explicit RRC Release using a new RRC Release cause - UE Autonomous release (e.g. timer based or upon detection of coverage gap)

There are proposals for either of the two solutions and it is also noted that there is also opinon that neither of the two solutions is needed.

Based on the above, the following question is asked:

Q2)  Which of the following options do you prefer for enahncements of RRC connection release?

- Option 1: Explicit RRC Release using a new RRC Release cause (The gNB needs to be aware of the discontinuous coverage information)

- Option 2: UE Autonomous release (e.g. timer based or upon detection of coverage gap)

- Option 3: No enhancement is needed

| Company | Prefered Option | Additional comments |
| --- | --- | --- |
| NEC | Option 1 | There are two cases for UE leaving RRC_CONNECTED after provding out-of-coverage information: - Awaring of UE is going to lose the coverage, eNB can release UE explicitly. - After providing the out-of-coverage information, upon declaring RLF, UE skip cell search and leave RRC_CONNECTED directly. |
| Qualcomm | Option 2 See comment, probably (option1+option2) | Option 1 is not clear on what is new change other than a new RRC release cause. If network wants to release a UE, it can do that any time, it does not have to wait until coverage is gone. It is also not clear what is the purpose of the new release cause. Regardless of release cause, UE and eNB both would know DC is going to happen and follow that procedure. AS NAS interaction on DC is already left to UE implementation. However, option 2 handles the situation where UE and network might have DC state mismatch. It is not a new concept as it is already supported in MUSIM. After providing out-of-coverage information, UE either can receive RRC release (or any pending DL data) from network or let the timer expire to go to IDLE mode autonomously. |
| Transsion Holdings | Option 1 | When discontinuous coverage happens, the RRC connection should be released both in UE and NW to avoid resource consumption and unnecessary AS action.To make the UE and NW consistant understanding of discontinuous coverage, the explict RRC release using a new RRC Release cause(e.g. coverage discontinuity) can be used before the covearege gap. |
| CATT | None | For Option 1, we don’t think new RRC Release cause is needed. The general UE behaviour has been defined in Rel-17, and can be enhanced in Rel-18 (as discussed in 4.3 following). The network can only release the RRC connection, no need to indicate the discoverage casue specially. if entering the discoverage scenario, the UE performs the behaviour accordingly. For Option 2, the existing procedure can be reused, no enhancement is needed. For Option 3, we think more detail or UE-specific discoverage information can be sent to UE via RRC Release message, based on the UE location gotten during the RRC connected mode. |
| Apple | Option 2 | For Option 1, our understanding is since UE has the knowledge about satellite coverage, it seems no extra info is offered by the explicit cause value in RRCConnectionRelease message. |
| Google | Option 2 | UE autonomously releases the RRC connection upon detecting the UE has entered an unreachability period, without starting T310 or T311. If T310 or T311 has been started already upon the detection, the UE stops T310 or T311 and transitions to RRC_IDLE directly. |
| MediaTek | Option 2 | Agree with Qualcomm and Apple. |
| ZTE | Option 1 | As mentioned in comments for Q1a), SA2 has achieved some agreements on how to (re)negotiate the unavailability period between UE and CN. It can be seen that, in order try to maintain consistent understanding on the Unavailability Period between UE and network, frequent or additional TAU procedures would be triggered. From RAN2 perspective, we understand this process would have obvious bad impacts on UE power saving and user experiences, especially for IoT UEs. So in UE’s implementation, we assume such TAU would be minimized and hardly performed by UE in connected mode. As a result, UE and eNB cannot always keep consistent understanding on the Unavailability Period. With this assumption, for UE in connected mode, it’s possible for any side (UE or eNB) to initiate the RRC release earlier than the start of Unavailability Period assumed by other side. If such release is triggered by eNB due to close to the start of Unavailability Period, UE cannot differentiate whether this release is a normal release or a special one due to discontinuous coverage. UE may normally handle it as UE may still think that the coverage will not end until some time later. On one hand, when UE back to idle, AS layer in UE may still perform the normal processes, e.g, paging monitoring or measurement. On the other hand, NAS may trigger another connection setup procedure if it keeps receiving the data from application layer (this may be highly possible as the connection is just interrupted by NW). It’s obvious these AS or NAS layer processes are useless as the current coverage is coming to an end soon. Therefore, in some scenario of inconsistent understanding on Unavailability Period between UE and NW, in order to avoid UE’s attempt for any legacy AS or NAS layer processes after it was deliberately released by the network due to discontinuous coverage, it’s needed to let eNB indicate a special release reason to UE in RRC release message, e.g., a same reason ‘Release due to discontinuous coverage’ as that introduced in RAN3. Moreover, as this new release reason can only be received by AS layer, in order to also stop the procedures which possibly occur in NAS layer, an AS-NAS interaction (e.g., an indication from AS to NAS) also needs to be introduced. |
| Lenovo | Option 1 or 2 | For Option 1, eNB is better to be aware of the coverage interruption at UE. For Option 2, UE may also report its estimation to network for aligned understanding e.g., for RRC release. |
| Ericsson | Option 1 or Option 3 | Option 3 may be sufficient considering that the network can release the UE at any time if it is aware that the coverage will be lost soon and this is UE is somehow expecting that since it is also aware that the coverage will be lost soon. However, as ZTE has explained above, it is also possible that UE may assume there is still room for some further transmission before the coverage is lost, and thus try to establish a connection if it is not aware why the network released the UE. |
| OPPO | Option 3 | For Option 1, we think a special cause value is not needed. NW does not have to indicate the cause for RRC release to UE, since UE knows the satellite coverage information. In Rel-17, UE stops any AS functions upon entering DC, so RRC connection is also released. Therefore, Option 2 is the existing UE behaviour and we don’t need any enhancement. |
| Xiaomi | Option 2 |  |
| Nokia | Option 2 with variation | As UE is aware of discontinuous coverage it can send Leave-indication to inform the network to trigger explicit RRC connection release. The feature is already available in MUSIM scenario where UE can indicate the leave-indication if it switches to other network. Here the UE behaviour is similar with difference that UE is deciding to leave the connection due to coverage issues. |
| Samsung | Option 1 or Option 2 | Considering the understanding from Q1a that both the network and the UE are aware of the discontinuous coverage information, hence, the network and/or the UE may trigger the RRC connection release. |

## UE behaviour in discontinuous coverage

The following summarizes contributions from RAN2#122 discussing UE behaviour in discontinuous coverage:

| References | Proposals |
| --- | --- |
| [2] | Proposal 9: If the remaining time of current cell’s coverage is not sufficient to accommodate a new connection establishment, the UE behavior can be left to UE implementation, no specification impact is expected. |
| [3] | Proposal 6: It is up to UE implementation whether to initiate the connection (re)establishment if the remaining time in the current cell is not sufficient for a new connection establishment, additional assistance information is not needed. |
| [5] | Proposal 3：After RLF is triggered, UE enters RRC_IDLE if the remaining serving time is less than a threshold in discontinuous coverage scenario. |
| [6] | Proposal 3: It is beneficial in terms of UE power saving if the UE can enter RRC_IDLE upon detecting a coverage gap without executing the RLF procedure. |
| [8] | Proposal 3: After providing the out-of-coverage information, UE follows legacy operation in declaring RLF but skip cell search, and leave RRC_CONNECTED directly. |
| [9] | Proposal:11 leave the decision on whether the UE initiates the connection (re-) establishment to the UE implementation when the remaining time in the current cell during discontinuous coverage is insufficient to complete a new connection establishment procedure Proposal 12: RAN2 to discuss UE behavior when the UE has limited remaining GNSS validity duration and the remaining discontinuous coverage time is also short. |
| [11] | Proposal 2(a): The UE verifies whether it has sufficient coverage time to complete a given RRC procedure (or a new connection establishment). Proposal 2(b): The UE initiates a given RRC procedure if it has sufficient coverage time to complete this procedure. Otherwise, the UE will not initiate the procedure or will wait for the next available satellite coverage period. |
| [14] | Error: Reference source not found |

Based on the above proposals, there are two kinds of opinions related to the UE behaviour when RLF is triggered, upon detecting a discontinuous coverage. One is that it is beneficial for UE to directly go to RRC_IDLE if anyway there is not enough time for the UE to finish the procedure of RRC re-establishement due to the discontinuous coverage. The other one is that this can be left to UE implementation.  Based on this, the following question is asked:

Q3)  Do you think it is beneficial for the UE to directly go to RRC_IDLE after RLF is triggered, if there is not enough time for the UE to finish the procedure of RRC re-establishement due to the discontinuous coverage?

| Company | Yes/No | Additional comments |
| --- | --- | --- |
| NEC | Yes | If UE is going to lose the coverage, it’s not worth it for UE to go back to RRC_CONNECTED. But UE and eNB need to be consistence in this situation. Therefore, we think the RLF behaviour should be: After providing the out-of-coverage information, UE follows legacy operation in declaring RLF but skip cell search, and leave RRC_CONNECTED directly. |
| Qualcomm | See comments | This can be handled by UE imeplementation as UE would know when the DC starts and there is no point to start a RRC connection if there is not sufficient time left, which can already happen today. |
| Transsion Holdings | Yes |  |
| CATT | See comments | We think it is beneficial for the UE to directly go to RRC_IDLE after RLF is triggered, if there is not enough time for the UE to finish the procedure of RRC re-establishement due to the discontinuous coverage, but we think this can be left to UE implementation, no further discussion or spec impact is needed. A chairman note can be used, if necessary. |
| Apple | Yes | But there is no need to specify it as it’s not realistic to define the principle. It could be done by UE implementation. |
| Google | See comments | Agree with Apple that nothing needs to be specified (i.e., can leave it to UE implementation). |
| MediaTek | See comments | This can be beneficial, but agree with Apple, Qualcomm and Google that it can left for UE implementation and no specification changes are needed. |
| ZTE | Yes | We think it's needed to specify some UE behavious to facilitate such special RLF processes in the scenario of discontinuous coverage. |
| Lenovo | See comments | UE implementation would be OK. |
| Ericsson | See the comments | This depends on whether there is no coverage, not only with respect to the NTN, but also TN, so it may not be straightforward to come up with such rule. |
| OPPO | See comments | Agree with Qualcomm. It is benefical but can leave to UE implementation without spec changes. |
| Xiaomi | See comments | We can leave this to UE implementation on whether to go to idle state directly or continue with connection reestablishment when RLF occurs. |
| Nokia | See comments | As the UE is aware of ‘coverage window’ whether the RLF is due to other radio conditions or due to DC. If the RLF is declared due to DC UE need not attempt for re-establishment. This can be upto UE implementation. |
| Samsung | Yes | We also think it is needed to specify that the UE goes to RRC_IDLE state, if there is insufficient time to complete the desired RRC procedure due to the discontinuous coverage event. |

## Necessity of providing additional information

The following summarizes contributions from RAN2#122 discussing whether to provide additional information:

| References | Proposals |
| --- | --- |
| [2] | Proposal 3：RAN2 evaluates the effort of introducing new SIB and SIB segmentation for additional satellite information (up to 4) transmission. Proposal 4：RRC Release message can be used to transmit additional satellite information to UE. |
| [3] | Proposal 1: RAN2 discuss to transfer more satellite assistance information via SIB segmentation or multiple SIBs. |
| [4] | Proposal 1 Provide measurement assistance information, e.g., PCI or carrier frequency, in SIB32 to facilitate cell selection and reduce service interruption after an NTN coverage gap. |
| [6] | Proposal 4: RAN2 to discuss how to support signalling of additional satellite information using one of the following options: Option1: Dedicated signalling Option 2: Additional information in SIBs e.g. using an additional new SIB or SIB segmentation |
| [7] | Proposal 2: At least for the same satellite before and after coverage interruption, RAN2 to consider enhancements to RRC connection recovery configuration before coverage interruption, so that UE can recovery RRC connection when coverage restores. |
| [9] | Proposal 2: RAN2 to discuss the provisioning of additional satellites’ ephemeris via dedicated RRC signaling. Proposal 3: RAN2 to discuss how a UE can read only a subset of the SIB with additional satellite assistance information, when the SIB has different information content at different SIB transmission occasions. Proposal 4: RAN2 to include footprint information for the earth-moving cell in discontinuous coverage as an optional field in SIB31. |
| [11] | Proposal 1: Measurement assistance information can be provided to control how the UE shall perform idle mode tasks (i.e. whether a UE can power down specific frequencies during discontinuous coverage) in a discontinuous coverage NTN. |
| [14] | Error: Reference source not found |

The above proposal are related to provding additional information as follows:

- Additional satellite information, e.g., ephemeris or footprint information

- Meassurement information, e.g., PCI, SSB configuration or carrier frequency

Also where to provide the related information is also mentioned, e.g, in SIB or RRC dedicated signaling. Based on the above, the following question is asked:

Q4)  Which of the following information do you think is needed to be provided additionally and where to provide them specifically:

- Ephemeris

- Footprint information

- PCI

- SSB configuration

- Carrier frequency

- Other information (Please describe)

| Company | Additional information | Additional comments(why and where to provide) |
| --- | --- | --- |
| Qualcomm | 1, 2, 5 | Dedicated signaling can be used to provide additional satellite information with ephemeris, foot print information and frequency. |
| CATT | 1,2,5,6 | We support to provide more information to UE for coverage prediction, because the network may have the UE location, then the network can provide more dedtail or UE-specific information to UE, via dedicated signaling. According to the current SIB32, the SatelliteInfo sent to UE can include tle-EphemerisParameters for earth-moving cell case or t-ServiceStart for earth-fixed cell case, so for 1)Ephemeris, we think it should be: - Ephemeris/Service starting time |
| Apple | At least 1, 2, 5 |  |
| Google | 1, 2, and 5 |  |
| MediaTek | 6 | New NTN scenario(GSO/NGSO) indication per satellite in SIB32 will be beneficial to the UE. UE can only support GSO or NGSO cells, but UE cannot derive whether the quasi-earth fixed cell is GSO or NGSO from the current SIB32, as tle-EphemerisParameters is only broadcasted for earth moving cell. So UE would attempt to search the cell that UE does not even support. A new NTN scenario(GSO/NGSO) indication per satellite in SIB32 can help UE to skip the unsupported cells. |
| ZTE | 6 | We see no clear justification for UE to acquire additional satellite assistant information. In our assumption, the (several) neighbour cell satellites acquired from the (several) SIB32(s) would be enough for UE to predict discontinuous coverage after backing to idle mode. However, there is another issue for the information provided via RRC dedicted signaling. In existing spec, the eNB may provide some redirection information (e.g., redirectedCarrierInfo, redirectedCarrierOffsetDedicated, idleModeMobilityControlInfo, etc.) via RRC release messages to the UE to help UE re-select to other cells/carriers after back to RRC idle state. Moreover, some validitiy timer(s) (e.g., T320, T322 etc.) are also provided correspondingly. If such validity timer(s) expire, the related redirection information would be discarded. Different from the above existing scheme, in the scenario of discontinuous coverage, we assume it’s also possible for eNB to provide redirection information when eNB release the UE due to end of current coverage, with the intention to help UE to find/re-select another suitable cell/carrier next time. But it’s obvious such information should be the one which can only be available or used by the UE after the coverage is resumed. Therefore, on the one hand, it’s needed to ensure that after the UE enters the unavailability period, this redirection information will not be discarded due to expiration of the corresponding validity timer(s). And on the other hand, the redirection information may not take effect immediately after the coverage is resumed and the effective time may be different for different rediection cells/carriers, so it may be also needed to indicate the effective time of the redirection information. So the following enhancements can be considered: - The existing validity timers corresponiding to the redirection information in RRC release messages need to be extended; - It’s suggested to introduce epoch time(s) for the redirection information so that they can be accurately used by the UE after the coverage is resumed. |
| Lenovo | 6) | We think it would be helpful to provide UE’s estimation of coverage interruption to eNB for aligned understanding on unreachability. |
| Ericsson | 5 | We think it may be beneficial to provide the information on carrier frequency to make it easier for the UE when searching for the cell after a coverage gap from both latency and energy consumption standpoint. We do not see the need to use dedicated signalling to provide additional ephemeris/footprint information considering what is already possible with the current signalling framework, e.g., broadcasting different versions of SIB32 in the serving cell. |
| OPPO | TBD | We see no clear motivation to introduce these additional satellite assistance information currently, since the current SIB32 has already sufficient for UE for the prediction of discontinuous coverage. |
| Xiaomi | TBD | We don’t see strong motivation to introduce additional assistant information for now. Current SIB32 is enough. |
| Nokia | 1,2,5 |  |
| Samsung | 6) | The design in Release 17 was changed during the correction phase such that the discontinuous coverage is only applied to NTN. This design means that a UE can never not stop measuring on a terrestrial network. In release 18, we should be able to configure or signal to a UE that it does not need to search or monitor for terrestrial frequencies. We think that this can be indicated by configuring whether a frequency is a discontinuous coverage or non-discontinuous frequency. We also think that we can introduce the ability to re-direct from a terrestrial network to a discontinuous coverage network to enable full integration between NTN and TN. |

## Others

Note that the offline only targets on solving the common issues identified by companies.

For a small amount of issues not covered by this offline which is highlighted in Section 6 (mostly never discussed before and mentioned by only one company), they can be submitted to the next meeing via company’s contribution.  But in case you think it is urgent, please indicate here and other companies can provide their opinions correspondingly.

For the details depending on the ouput of the above questions, we can also discuss them later (e.g., whether and how to provide discontinuous coverage information to gNB).

| Company | Other urgent issues |
| --- | --- |
| Qualcomm | UE behavior for TN and NTN IDLE mode tasks during UE’s reported unavailability period. |
| Transsion Holdings | the RACH congestion issue as a large number of UEs may try to access the NTN cell when the discontinuous coverage ends, especially for the Quasi-earth fixed cell. |
| CATT | Agree with Transsion Holdings that, AS layer overload control during the intial RACH when the coverage coming back should be discussed in RAN2. |
| Lenovo | For NB-IoT, disabling neighbour cell measurement triggering in RRC_CONNECTED before coverage interruption due to discontinuous coverage. For eMTC, disabling measurement report triggering before coverage interruption due to discontinuous coverage. |

# Conclusions

To be added

# References (In alphabetical order by company)

| Reference | TDoc | Title | Company | Proposals |
| --- | --- | --- | --- | --- |
| [1] | R2-2306167 | Support on discontinuous coverage in IoT NTN | Apple | Proposal 1: Introduce paging offset to solve the misfit issue of the PH/PTW which is currently determined by UE ID. |
|  |  |  |  | Proposal 2: Consider the PTW extension solution as a complementary solution. |
|  |  |  |  | Proposal 3: UE assistance information via NAS signaling (unreachability period in TAU) can be used for PTW adjustment when UE finds out the mismatch issue. |
|  |  |  |  | Proposal 4: Send an LS to SA2/CT1 on unevenly distributed coverage period caused by unevenly distributed satellite from multiple satellites. |
|  |  |  |  | Proposal 5: Do not introduce new release cause for coverage gap in RRCRelease message. |
|  |  |  |  | Proposal 6: RAN2 to discuss whether to support discontinuous coverage scenario in inactive state. |
| [2] | R2-2304896 | Discussion on enhancements to discontinuous coverage | CATT | Proposal 1：Some enhancement is needed for HARQ process with HARQ enabling when there is no enough time for ACK/NACK feedback because of the incoming coverage hole. |
|  |  |  |  | Proposal 2：RAN2 to discuss the AS overload control enhancement to avoid excessive signalling load on the network upon the network coverage coming back. |
|  |  |  |  | - LS can be sent to SA2 to check detail understanding of the NAS waiting time solution, if necessary |
|  |  |  |  | Proposal 3：RAN2 evaluates the effort of introducing new SIB and SIB segmentation for additional satellite information (up to 4) transmission. |
|  |  |  |  | Proposal 4：RRC Release message can be used to transmit additional satellite information to UE. |
|  |  |  |  | Proposal 5：No new RRC Release cause is needed for discontinuous coverage scenario. |
|  |  |  |  | Proposal 6：UE Autonomous release enhancement for discontinuous coverage is not needed, if AMF/MME shares the UE unreachability period to RAN node. |
|  |  |  |  | Proposal 7：UE/NW autonomous adjustment can be considered for paging and eDRX enhancement. |
|  |  |  |  | Proposal 8：No additional information is needed to assist the UE to verify if the remaining time of current cell’s coverage is sufficient to accommodate a new connection establishment. |
|  |  |  |  | Proposal 9: If the remaining time of current cell’s coverage is not sufficient to accommodate a new connection establishment, the UE behavior can be left to UE implementation, no specification impact is expected. |
| [3] | R2-2305612 | Discussion on the discontinuous coverage for IoT-NTN | CMCC | Proposal 1: RAN2 discuss to transfer more satellite assistance information via SIB segmentation or multiple SIBs. |
|  |  |  |  | Proposal 2: There is no RAN impacts to adjust the PTW configuration. It is up to UE and eNB implementation to skip the PTW within the discontinuous coverage. |
|  |  |  |  | Proposal 3: A new cause value ‘Release due to discontinuous coverage’ is introduced in RRCRelease message. |
|  |  |  |  | Proposal 4: The UE can provide the out-of-coverage information to RAN node for e.g, optimization on RRC release or paging. |
|  |  |  |  | Proposal 5: The UE out-of-coverage information can be kept by RAN node when the UE enters into idle mode. |
|  |  |  |  | Proposal 6: It is up to UE implementation whether to initiate the connection (re)establishment if the remaining time in the current cell is not sufficient for a new connection establishment, additional assistance information is not needed. |
| [4] | R2-2306466 | Enhancements to discontinuous coverage | Ericsson | Proposal 1 Provide measurement assistance information, e.g., PCI or carrier frequency, in SIB32 to facilitate cell selection and reduce service interruption after an NTN coverage gap. |
| [5] | R2-2304812 | Discussion on discontinuous coverage | Huawei, HiSilicon | Proposal 1: UE can indicate “RRC_IDLE” to the Network as the preferred RRC state when it is going to enter a coverage gap and start a timer, similar as the mechanism designed by R17 MUSIM. |
|  |  |  |  | Proposal 2：The network can predict multiple coverage windows by implementation and takes them into account when paging a UE. Proposal 3：After RLF is triggered, UE enters RRC_IDLE if the remaining serving time is less than a threshold in discontinuous coverage scenario. Proposal 4：UE stops the AS idle mode tasks related to TN only when there is no TN cells in the discontinuous coverage. |
| [6] | R2-2305171 | IoT-NTN discontinuous coverage enhancements | Interdigital, Inc. | Proposal 1: For eMTC and NB-IoT: A UE in R17 is allowed not to perform RRC_IDLE mode tasks during a UE unreachability period. Consider whether to explicitly clarify that this means that if a UE in RRC_IDLE or RRC_INACTIVE determines it is in a UE unreachability period, the UE may choose not to perform measurements of the serving cell or neighbour cells, and may postpone moving to “any cell selection” state, and is allowed not to attempt to monitor paging occasions which occur during a UE unreachability period. |
|  |  |  |  | Proposal 2: RAN2 to discuss what new behaviour is expected from the UE when NW releases the UE using a new cause e.g. “discontinuous coverage” in the following cases: Case 1: Release to RRC_IDLE Case 2: Release to RRC_INACTIVE Proposal 3: It is beneficial in terms of UE power saving if the UE can enter RRC_IDLE upon detecting a coverage gap without executing the RLF procedure. |
|  |  |  |  | Proposal 4: RAN2 to discuss how to support signalling of additional satellite information using one of the following options: Option1: Dedicated signalling Option 2: Additional information in SIBs e.g. using an additional new SIB or SIB segmentation |
|  |  |  |  | Proposal 5: Update the PTW if it falls in a coverage gap by applying an offset. FFS how to determine/configure the offset. Proposal 6: Send an LS to SA2 indicating that RAN2 <have agreed/are considering> shifting the PTW if it falls in a coverage gap, asking what the content/granularity of “UE unreachability period is, whether it can be provided to the gNB, and whether adjusting the PTW is feasible from SA2 point of view. |
| [7] | R2-2305714 | Further considerations on discontinuous coverage | Lenovo | Proposal 1: RAN2 to consider enhancements to UE reporting its estimation of coverage interruption to eNB. |
|  |  |  |  | Proposal 2: At least for the same satellite before and after coverage interruption, RAN2 to consider enhancements to RRC connection recovery configuration before coverage interruption, so that UE can recovery RRC connection when coverage restores. |
|  |  |  |  | Proposal 3: RAN2 to consider enhancements to disable neighbour cell measurement triggering in RRC_CONNECTED before coverage interruption due to discontinuous coverage. |
| [8] | R2-2305307 | Considerations on Supporting Discontinuous Coverage | NEC Europe Ltd | Proposal 1: Consider to support UE providing assistance information on being out-of-coverage to gNB for RRC connection management. |
|  |  |  |  | Proposal 2: Consider to include remaining coverage time in out-of-coverage information. |
|  |  |  |  | Proposal 3: After providing the out-of-coverage information, UE follows legacy operation in declaring RLF but skip cell search, and leave RRC_CONNECTED directly. |
|  |  |  |  | Proposal 4: RAN2 confirm the PTW position should be configurable (FFS to configure one or multiple PTW positions, FFS the configuration is an offset or replacement of the calculated PTW position), send LS to SA2/CT1 asking for collaboration. |
| [9] | R2-2305863 | On RAN impacts for Discontineous coverage enhancements | Nokia, Nokia Shanghai Bell | Proposal 1: CN provides the unreachability period(s) for a UE to the RAN. |
|  |  |  |  | Proposal 2: RAN2 to discuss the provisioning of additional satellites’ ephemeris via dedicated RRC signaling. |
|  |  |  |  | Proposal 3: RAN2 to discuss how a UE can read only a subset of the SIB with additional satellite assistance information, when the SIB has different information content at different SIB transmission occasions. |
|  |  |  |  | Proposal 4: RAN2 to include footprint information for the earth-moving cell in discontinuous coverage as an optional field in SIB31. |
|  |  |  |  | Proposal 5: The PSM and eDRX configurations can be configured to align with the estimated UE unreachability period. The configuration is out of RAN2’s scope. Proposal 6: Network can extend the paging before/after the coverage window if the UE does not respond to paging within the estimated coverage window. Proposal 7: UE can extend the paging monitoring outside the estimated coverage window if radio coverage is available. UE may report to the network to realign the paging monitoring and coverage windows. Proposal 8: UE may report to the network to realign the paging monitoring and coverage windows if UE notices there is radio coverage before or after the expected reachability period. |
|  |  |  |  | Proposal 9: RAN2 to discuss how to handle UE movement within the same TA during discontinuous coverage. Proposal 10: RAN2 to discuss support for UE request for RRC connection release based on DC estimation. FFS support for implicit RRC connection release. Proposal:11 leave the decision on whether the UE initiates the connection (re-) establishment to the UE implementation when the remaining time in the current cell during discontinuous coverage is insufficient to complete a new connection establishment procedure Proposal 12: RAN2 to discuss UE behavior when the UE has limited remaining GNSS validity duration and the remaining discontinuous coverage time is also short. |
| [10] | R2-2305201 | RRC release procedure in discontinuous coverage | Qualcomm Incorporated | Proposal 1 If the UE is able to predict when the discontinuous coverage starts, reuse NR MUSIM procedure to leave RRC_CONNECTED state where the UE indicates the out-of-coverage to network and starts an out-of-coverage timer. |
|  |  |  |  | Proposal 2 Reuse NR MUSIM timer T346g behavior, i.e., upon expiry of the out-of-coverage timer, the UE performs the actions upon leaving RRC_CONNECTED, with release cause 'other'. |
|  |  |  |  | Proposal 3 Further discuss the details on the values of the out-of-coverage timer and message to carry out-of-coverage information. |
| [11] | R2-2305785 | Enhancements to discontinuous coverage | Samsung Shenzhen | Proposal 1: Measurement assistance information can be provided to control how the UE shall perform idle mode tasks (i.e. whether a UE can power down specific frequencies during discontinuous coverage) in a discontinuous coverage NTN. |
|  |  |  |  | Proposal 2(a): The UE verifies whether it has sufficient coverage time to complete a given RRC procedure (or a new connection establishment). |
|  |  |  |  | Proposal 2(b): The UE initiates a given RRC procedure if it has sufficient coverage time to complete this procedure. Otherwise, the UE will not initiate the procedure or will wait for the next available satellite coverage period. |
|  |  |  |  | Proposal 3: UE autonomous release for discontinuous coverage is not pursued. |
|  |  |  |  | Proposal 4: Explicit RRC Release using a new RRC Release cause for discontinuous coverage is introduced. |
|  |  |  |  | Proposal 5: Certain idle mode procedures are paused if a UE receives an RRC release with the new RRC release cause. |
|  |  |  |  | Proposal 6: The new RRC release cause is used to re-direct a UE from a non-discontinuous coverage network to a discontinuous coverage network. |
|  |  |  |  | Proposal 7: Discontinuous coverage-related information is provided in a re-direct message when being re-directed to a discontinuous coverage network. |
| [12] | R2-2305560 | Discussion on power saving enhancements for supporting discontinuous coverage | Spreadtrum Communications | Proposal 1: In order to ensure that the UE can be reachable when paging arrives, the PTW should aligns with the time duration of coverage and the following two options can be taken into further consideration: |
|  |  |  |  | Option 1: both UE and network adjust PTW based on a predefined rule |
|  |  |  |  | Option 2: network provides multiple PTW info |
|  |  |  |  | Proposal 2: From RAN2 point of view, the existing PSM mechanism needs to be enhanced in order to get better power effect. |
|  |  |  |  | Proposal 3：The PSM starting mechanism needs to be modified to adapt to the scenario of discontinuous coverage. |
|  |  |  |  | Proposal 4: A time threshold needs to be introduced for UE to determine whether to start T3324 timer. |
| [13] | R2-2305372 | Discussion on enhancement to discontinuous coverage for IoT NTN | Transsion Holdings | Proposal 1: RAN2 needs to consider the RACH congestion issue as a large number of UEs may try to access the NTN cell at the next satellite’s service start time. |
|  |  |  |  | Proposal 2: The discontinuous coverage wait timer can be used in AS to avoid RACH congestion. |
|  |  |  |  | Proposal 3: The CN should be aware of the UE’s discontinuous coverage period to CN to avoid paging issues. |
|  |  |  |  | Proposal 4: The satellite assistance information or the discontinuous coverage information predicted by UE can be provided to CN for paging. |
|  |  |  |  | Proposal 5: The explict RRC release using a new RRC Release cause(e.g. coverage discontinuity) can be used before the covearage gap. |
| [14] | R2-2305672 | Enhancements to discontinuous coverage | Xiaomi | Proposal 2: The issue that UE ends up with no PTW in the in-coverage period should be addressed. |
|  |  |  |  | Proposal 2: AMF configures the start position (PH) of PTW window. |
|  |  |  |  | Error: Reference source not found |
|  |  |  |  | Error: Reference source not found |
| [15] | R2-2305959 | RAN2 enhancements for discontinuous coverage | ZTE Corporation, Sanechips | Proposal 1a: If legacy eDRX is used for keeping alignment between UE and NW during discontinuous coverage, in order to align the starting time of PTW with the out-of-coverage period or unreachability period, it’s suggested to introduce a configurable offset to shift the starting time of PTW. |
|  |  |  |  | Proposal 1b: The out-of-coverage period or unreachability period should be informed to RAN, e.g., from core network node, to assist RAN to provide a more appropriate paging schedule for UE in idle mode. |
|  |  |  |  | Proposal 2a: A new release reason, e.g., ‘Release due to discontinuous coverage’ as that introduced in RAN3, can be introduced in RRC release message for indicating UE to stop the subsequent AS layer processes after it is released to idle mode. |
|  |  |  |  | Proposal 2b: An AS-NAS interaction (e.g., an indication from AS to NAS) also needs to be introduced for indicating UE to stop the subsequent NAS layer processes after it is released to idle mode due to discontinuous coverage. |
|  |  |  |  | Proposal 2c: The legacy IE extendedWaitTime can be reused to stop the subsequent NAS layer processes after UE is released to idle mode due to discontinuous coverage. The extension to the value range of extendedWaitTime needs to be discussed. |
|  |  |  |  | Proposal 2d: The UE in connected mode could provide out-of-coverage period or unreachability period information as an assistance to the network (eNB). |