3GPP TSG-RAN WG2 #123	R2-230xxxx

Toulouse, France, August 21-25, 2023

Agenda Item:	7.9.4

Source: 	LG Electronics Inc.

Title: 	[Draft] Report of [AT121bis-e][419][Relay] Remaining high-priority proposals on multi-path (LG)

Document for:	Discussion and decision

# Introduction

This is the trigger of the following email discussion:

- [Post122][403][Relay] Procedures for multi-path relay (LG)

Scope: Discuss and attempt to develop the multi-path relay procedures:

- Path addition

- Path release

- Path change

- Path failure and recovery

Target developing procedures at a stage 2 level where possible, based on our agreements, and identifying remaining open issues with the procedural design, covering scenarios 1 and 2.

Intended outcome: Report to next meeting

Deadline: Thursday 2023-08-10 1000 UTC

Companies are requested to review the multi-path relay procedures, make comments on the ‘procedure for review’, if any, and provide any remaining open issue necessary to complete the procedures at a stage 2 level.

Note that companies can only provide updates of the ‘procedure for review’ in the corresponding columns for comments, instead of directly modifying the figures and texts of the ‘procedure for review’, and rapporteur may update the procedures and collect open issues before the deadline based on early responses from companies.

# 1. Scenario 1

RAN2 agreed to support the following cases for scenario 1

- Indirect path addition (Case A): The remote UE operating only on the direct path adds the indirect path under the same gNB;

- Direct path addition (Case B): The remote UE operating only on the indirect path adds the direct path under the same gNB;

- Indirect path release (Case C): The remote UE operating in multi-path releases the indirect path;

- Direct path release (Case D): The remote UE operating in multi-path releases the direct path;

- Direct path change (Case E): The remote UE operating in multi-path changes the direct path to a different cell of the same gNB while using the serving relay UE for the indirect path under the same gNB.

- Indirect path change (Case G): The remote UE operating in multi-path changes to a new relay UE for the indirect path while keeping the direct path under the same gNB.

### 1.1 Direct path addition in Scenario 1

##### Related agreements

RAN2 agreements related with this procedure are listed below:

- Support PCell on the direct path only when the UE is in multi-path operation, for both scenario 1 and scenario 2.

- As a baseline, direct path addition for multi-path is a path switch procedure in which the target configuration contains both paths.

- Upon direct path addition for multi-path, one of the serving cells of the added direct path is configured as PCell for the remote UE.

- For Scenario-1, use T304-like timer for direct path addition and change. FFS on expiry behavior.

RAN3 agreed to add Inter-DU Direct Path Addition and Inter-DU Indirect Path Addition procedures in TS 38.401 (in R3-230947). In addition, RAN3 agreed to add the intra-DU path addition procedure in the BL CR (R3-232088) to TS 38.401.

#### 8.xx.3 Intra-DU direct path addition on top of indirect path The signaling flow for intra-DU direct path addition is shown in Fig. 8.xx.3-1. Figure 8.xx.3-1: Signalling procedure of intra-DU direct path addition on top of indirect path - The Uu measurement configuration and measurement report signalling is performed between remote UE and gNB-CU to evaluate both relay link measurement and Uu link measurement. The remote UE may report Uu measurement results of neighboring cells and one or multiple candidate relay UEs. 2. The gNB-CU decides to add the direct path to remote UE under the same gNB-DU. 3. The gNB-CU sends the UE CONTEXT MODIFICATION REQUEST message for the remote UE to the gNB-DU, which contains at least the direct path configuration. 4. The gNB-DU responds to the gNB-CU with a UE CONTEXT MODIFICATION RESPONSE message. 5. The gNB-CU sends an RRCReconfiguration message to the relay UE to update the indirect path configuration if necessary. 6. gNB-CU sends the DL RRC MESSAGE TRANSFER message for remote UE by including the RRCReconfiguration message to the gNB-DU. The contents in the RRCReconfiguration message may include at least direct path addition configuration, RLC channel configuration, bearer mapping and the associated radio bearer(s). 7. The gNB-DU sends the RRCReconfiguration message to the remote UE. 8. The remote UE performs random access procedure at the gNB-DU. 9. The remote UE may complete the direct path addition procedure by sending the RRCReconfigurationComplete message to the gNB-DU. Editor’s Note: FFS whether the remote UE sends the RRCReconfigurationComplete message to the gNB-CU via old path (indirect path) or new path (direct path). 10. The gNB-DU sends the UL RRC MESSAGE TRANSFER message to gNB-CU by including the RRCReconfigurationComplete message.

##### Procedure for review

The procedure based on the related agreements is shown below.

#### 1.1 Direct path addition on top of indirect path Figure 1.1: Signalling procedure of direct path addition on top of indirect path 1. The remote UE and the relay UE perform measurements based on measurement configuration. The remote UE may report measurement results e.g. for Event A1 (Serving becomes better than threshold). 2. The gNB decides to add the direct path for the remote UE. The cell serving the direct path and the cell serving the relay UE on the indirect path belong to the same gNB but can be same or different. 3. The gNB sends an RRCReconfiguration message to the relay UE to update the indirect path configuration, if necessary. 4. The gNB sends the RRCReconfiguration message to the remote UE via the relay UE. The contents in the RRCReconfiguration message may include at least a target cell, direct path addition configuration, RLC channel configuration, bearer mapping and the associated radio bearer(s). 5. The remote UE synchronizes to DL of the target cell serving the direct path and performs random access procedure towards the cell serving the direct path. The remote UE configures the target cell as PCell. 6a. The remote UE sends the RRCReconfigurationComplete message to the gNB at least via the direct path in order to complete the direct path addition procedure. 6b. If a split SRB1 with duplication is configured, the remote UE also sends the RRCReconfigurationComplete message to the gNB via the indirect path served by the relay UE.

Companies are requested to check the procedure and provide comments and remaining open issues on this procedure, if any.

| Company | Comments | Remaining open issues |
| --- | --- | --- |
| ZTE | For step 4, the “RLC channel configuration” here should refer to PC5 relay RLC channel? It is suggested to clarify it since the TS 38.300 generally use Uu Relay RLC channel or PC5 Relay RLC channel. “4. The gNB sends the RRCReconfiguration message to the remote UE via the relay UE. The contents in the RRCReconfiguration message may include at least a target cell, direct path addition configuration, RLC channel configuration, bearer mapping and the associated radio bearer(s).” For 6a, it has not yet determined whether the non-split SRB can be supported on indirect path. If yes, the remote UE may send the RRCReconfigurationComplete message to the gNB only via the indirect path. Taking this into account, it is suggested to leave it as FFS on whether the indirect path or direct path can be used for the sending of RRCReconfigurationComplete message. |  |
| vivo | There are two comments: - General to all steps for the whole Section 1: Comments to all steps for the whole Section 1: since it’s specified for Scenario 1, we should make it clear that the remote UE is the L2 U2N Remote UE and the relay UE is the L2 U2N Relay UE. i.e. replace “remote UE” by “L2 U2N Remote UE” and replace “relay UE” by “L2 U2N Relay UE” in the procedural text. - Step 1: suggest not to mention a specific measurement Event, which is up to gNB configuration. Instead a general sentence could be used here as, e.g., “The measurement results from L2 U2N Remote UE are reported when configured measurement reporting criteria are met” as specified in current subclause 16.12.6.1 of TS 38.300. - Step 4: we think “direct path addition configuration” is a high-level description which can cover all of the remaining configurations. Therefore, suggest to rewording as “The contents in the RRCReconfiguration message may include direct path addition configuration, which includes at least a target PCell, direct path addition configuration, RLC channel configuration, bearer mapping and the associated radio bearer(s).” - Step 5 includes multiple associated UL/DL msgs. The arrow should be modified to be double arrow. |  |
| CATT | 1. For DL/UL data, refer to 38.300, dash line should be used. - For 6a, we share the same view as ZTE, which path(s) is(are) used to carry RRCReconfigurationComplete message depending on SRB1 configuration. RAN2 has no agreement on whether non-split SRB can be configured on indirect path. |  |
| Apple | 1. step 3, we understand this is based on RAN3 diagram, but for relay UE, there is no reason to adjust any configuration related to the remote UE at this time. This step shall only occur after step 6. RAN2 can discuss this and send a LS to RAN3 if agreed to move this optional step to a place after step 6. 2. Step 4, for the information included in RRCReconfiguration, we think this shall be a “multi-path configuration” which includes some detailed configuration about how to split/duplicate traffic for each DRB. It is not just a direct path configuration. So, the current description is insufficient. Also, since there is a PCell change, “ReconfigurationWithSync” needs to be included too. 2. Step 6a. Agree with ZTE and CATT to put FFS on this depending on whether indirect path SRB1 can be supported. |  |
| Nokia | We wonder whether 6a/6b means that the remote UE shouldn’t send the RRCReconfigurationComplete message via the indirect path even if the primary path of the split SRB1 is set to the indirect path without duplication. | It is still FFS where to send the RRCReconfigurationComplete message? - The gNB sets the primary path to the direct path if relay UE is in RRC_CONNECTED and the UE sends the RRCReconfigurationComplete message via (at least) the primary path. - Regardless of the primary path, the remote UE sends the RRCReconfigurationComplete message via the direct path as long as the relay UE is in RRC_CONNECTED. |
| NEC | Step4, for the information included in RRCReconfiguration, we think this configuration should be reorganized as: The contents in the RRCReconfiguration message may include at least a target cell, a direct path addition configuration including at least a target cell, and an indirect path configuration such as RLC Relay channel configuration, bearer mapping and the associated radio bearer(s). |  |
| InterDigital | Step 4, We think the information included in the RRCReconfiguration should be clarified. “In may contain direct path RLC channel configuration, updated radio bearer configuration, including multipath configuration.” Step 6: Agree with ZTE and CATT that non-split via the indirect path is possible and should be left FFS. May consider including a note indicating that the order of 3 and 4 are upto NW implementation. |  |
| Samsung | Step 1 Shall we include the measurement event in the stage-2 procedure? If we want to add this, which event is used may need further discussion. In addition, this step only needs mention the measurement behaviour of remote UE since this is a procedure for remote UE. In Rel-17, for direct to indirect path switch, we don’t include relay UE measurement procedure either. Step 5 “configures the target cell as PCell”: it seems that the PCell is not configured by the remote UE, it is configured by the gNB. Suggest to change it as “consider the target cell to be the PCell” |  |
| Huawei, HiSilicon | Step 4 The RRCReconfiguration message basically is a PCell change command, i.e. with reconfigurationWithSync, which also includes MP configuration. We suggest: The gNB sends the RRCReconfiguration message to the remote UE via the relay UE. The contents in the RRCReconfiguration message, containing the information required to access the target cell and multi-path configuration may include at least a target cell, direct path addition configuration, RLC channel configuration, bearer mapping and the associated radio bearer(s). Step 5 It is strange to say “The remote UE configures the target cell as PCell”, suggest to remove this sentence. Step 6a/6b As some companies commented, how the SRB1 is configured does not need to be mentioned here. Suggest to remove the controversial part for now: 6a. The remote UE sends the RRCReconfigurationComplete message to the gNB in target sideat least via the direct path in order to complete the direct path addition procedure. 6b. If a split SRB1 with duplication is configured, the remote UE also sends the RRCReconfigurationComplete message to the gNB via the indirect path served by the relay UE. |  |
| Ericsson | Step-2: cell serving the direct path and the cell serving the relay UE on the indirect path should belong to the same gNB. But the cells themselves can be same or different. Step-5: - Remote UE performs DL synchronization to the target cell. - Don’t think the remote UE configures the target cell as PCell. This will be done by the network. |  |
| Fraunhofer | Step-5: The UE could be configured to use CFRA instead of regular RAProcedure In any case, the arrow should be in both directions, as the RAProcedure is a multi-message procedure |  |
| LG Electronics | Step 1 The specific event can be removed as commented above. Step 2 DL synchronization may be needed even at a cell under the same gNB. So, ‘if necessary’ can be added. Step 3 FFS can be added to check if RAN2 can confirm what RAN3 agreed or the order can be up to network implementation. Step 4 Some comments on the contents in the message seem Stage 3 details. Considering that we are focusing on Stage 2 level procedure in this email discussion, details about the contents can be removed here and go to Stage 3. Step 4 and 5 The above comments on target PCell can be reflected. FFS on CFRA can be added Step 6 FFS on the complete message only via the indirect path can be added for future discussion. |  |

##### Summary and Updated Procedure

The updated procedure based on the above comments is shown below. Note that any change to the figure is shown in red.

#### 1.1 Direct path addition on top of indirect path Figure 1.1: Signalling procedure of direct path addition on top of indirect path 1. The remote UE and the relay UE perform measurements based on measurement configuration. The remote UE may report measurement results. 2. The gNB decides to add the direct path for the remote UE. The cell serving the direct path and the cell serving the relay UE on the indirect path belong to the same gNB but can be same or different. 3. The gNB sends an RRCReconfiguration message to the relay UE to update the indirect path configuration, if necessary. FFS: Whether RAN2 can confirm the step 3 as agreed in RAN3, noting that this step is based on RAN3 agreement. Whether the order of the step 3 and 4 is up to network implementation. 4. The gNB sends the RRCReconfiguration message to the remote UE via the relay UE for direct path addition. FFS: Stage 3 details about the contents in the RRCReconfiguration message. 5. The remote UE synchronizes to DL of the target PCell serving the direct path, if necessary, and performs random access procedure towards the PCell serving the direct path. FFS: whether CFRA is supported towards PCell serving the direct path. 6a. The remote UE sends the RRCReconfigurationComplete message to the gNB at least via the direct path in order to complete the direct path addition procedure. 6b. If a split SRB1 with duplication is configured, the remote UE also sends the RRCReconfigurationComplete message to the gNB via the indirect path served by the relay UE. FFS: whether the primary path of the split SRB1 can be set to the indirect path without duplication, especially for the RRCReconfigurationComplete message, whether the non-split SRB can be supported on indirect path, and whether the remote UE sends the RRCReconfigurationComplete message to the gNB only via the indirect path.

### 1.2 Indirect path addition in Scenario 1

##### Related agreements

RAN2 agreements related with this procedure are listed below:

- Multi-path Relay is NOT applicable to RRC Setup procedure, for scenario-1 and scenario-2.

- Multi-path Relay is NOT applicable to RRC Resume procedure, for scenario-1 and scenario-2.

- Remote UE storing indirect path configuration (e.g., SRAP and PC5-RLC channel configurations) and resuming directly into multi-path configuration is not supported for scenario 1.

- In scenario 1, when a remote UE configured with multi-path initiates the RRC re-establishment procedure, the remote UE does not perform re-establishment directly into a multi-path configuration.

- For bringing the idle/inactive relay UE to RRC_CONNECTED, the legacy Rel-17 behaviour (Alt 1 in the proposal) is not disabled for indirect path addition when split SRB1 is configured.  A PC5-RRC trigger is specified at least for other cases.

  - FFS if a Rel-17 relay UE is supported for use with multi-path and how the above agreement is reflected in such a case.

- When split SRB1 with duplication is configured, the remote UE sends the RRCReconfigurationComplete message to gNB via both paths for Scenario 1.

- When one of the following conditions is met, the remote UE sends the RRCReconfigurationComplete message to gNB via the direct path for Scenario 1. FFS on need for additional condition.

  - when primary RLC entity of split SRB1 is on direct path

  - when non-split SRB1 is configured on direct path

- For Scenario-1, use T420-like timer for indirect path addition and change. FFS on stop condition and expiry behavior.

RAN3 agreed to add Inter-DU Direct Path Addition and Inter-DU Indirect Path Addition procedures in TS 38.401 (in R3-230947). In addition, RAN3 agreed to add the intra-DU path addition procedure in the BL CR (R3-232088) to TS 38.401.

#### 8.xx.4 Intra-DU indirect path addition on top of direct path The signaling flow for intra-DU indirect path addition is shown in Fig. 8.xx.4-1. Figure 8.xx.4-1: Signalling procedure of intra-DU indirect path addition on top of direct path 1. The Uu measurement configuration and measurement report signalling is performed between remote UE and gNB-CU to evaluate relay link measurement and Uu link measurement. The remote UE may report Uu measurement results of neighboring cells and one or multiple candidate relay UEs. 2. The gNB-CU decides to add the indirect path via relay UE to remote UE under the same gNB-DU. 3. The reconfiguration to relay UE is performed among relay UE, gNB-DU and gNB-CU if relay UE is in RRC_CONNECTED state. The gNB-CU sends an RRCReconfiguration message to the relay UE. If the relay UE is in RRC_IDLE/INACTIVE state, this step is skipped. 4. The gNB-CU sends the UE CONTEXT MODIFICATION REQUEST message for the remote UE to the gNB-DU, which contains the indirect path configuration at least. 5. The gNB-DU responds to the gNB-CU with a UE CONTEXT MODIFICATION RESPONSE message. 6. gNB-CU sends the DL RRC MESSAGE TRANSFER message for remote UE by including the RRCReconfiguration message to gNB-DU. The contents in the RRCReconfiguration message may include at least indirect path addition configuration, PC5 RLC channel configuration for relay traffic, bearer mapping and the associated radio bearer(s). 7. gNB-DU sends the RRCReconfiguration message to the remote UE. 8. The remote UE establishes PC5 connection with relay UE. 9. The remote UE may complete the indirect path addition procedure by sending the RRCReconfigurationComplete message to the gNB-DU. Editor’s Note: FFS whether the remote UE sends the RRCReconfigurationComplete message to the gNB-CU via old path (direct path) or new path (indirect path). Editor’s Note: FFS: In case the relay UE is in RRC_IDLE/ INACTIVE state, how to trigger the relay UE enter RRC_CONNECTED state, RAN3 wait for RAN2’s further progress. 10. The gNB-DU sends the UL RRC MESSAGE TRANSFER message to gNB-CU by including the RRCReconfigurationComplete message.

##### Procedure for review

The procedure based on the related agreements is shown below.

#### 1.2 Indirect path addition on top of direct path Figure 1.2: Signalling procedure of direct path addition on top of indirect path 1. The remote UE and the relay UE perform measurements based on measurement configuration. The remote UE may report measurement results e.g. for Event Y2 (Candidate L2 U2N Relay UE becomes better than threshold). 2. The gNB decides to add the indirect path for remote UE. The cell serving the direct path and the cell serving the relay UE on the indirect path belong to the same gNB but can be same or different. 3. The gNB sends an RRCReconfiguration message to the relay UE to configure the indirect path of the remote UE. 4. The gNB sends the RRCReconfiguration message to the remote UE. The contents in the RRCReconfiguration message may include at least the target relay UE, indirect path addition configuration, RLC channel configuration, bearer mapping and the associated radio bearer(s). 5. If the remote UE has no PC5 unicast link with the target relay UE, the remote UE establishes a PC5 unicast link with the target relay UE. If the remote UE sends the RRCReconfigurationComplete message only on the direct path in Step 6, the remote UE sends a PC5-RRC message to the target relay UE. If the relay UE is not in RRC_CONNECTED, the relay UE initiates a RRC connection establishment upon receiving the PC5-RRC message. FFS: Need for additional condition to trigger the PC5-RRC message (e.g. based on RRC state of the relay UE), which PC5-RRC message is used for this purpose (e.g. new or existing one), and the contents in the PC5-RRC message (e.g. trigger RRC establishment). 6a. The remote UE sends the RRCReconfigurationComplete message to the gNB at least via the direct path in order to complete the indirect path addition procedure. 6b. If a split SRB1 with duplication is configured, the remote UE also sends the RRCReconfigurationComplete message to the gNB via the indirect path served by the relay UE. If relay UE is not in RRC_CONNECTED, the relay UE initiates a RRC connection establishment to send the RRCReconfigurationComplete message to the gNB. FFS: Whether/How to handle the case when the relay UE establishes a RRC connection with a different gNB than the gNB serving the target cell.

Companies are requested to check the procedure and provide comments and remaining open issues on this procedure, if any.

| Company | Comments | Remaining open issues |
| --- | --- | --- |
| OPPO | Step-5, can rename it to PC5 connection establishment, since PC5-S signaling is needed besides PC5-RRC. And considering that relay/non-relay links will be served by different L2 ID, is there still a need to use dash line for step-5? And we wonder if there is a need to mandate step-6a to be after step-5, considering in legacy, RRCReconfigurationComplete for SCell addition or SCG addition does not have to be after the completion of SCell addition or SCG addition. |  |
| Xiaomi | Step 3, if relay UE is in RRC_IDLE/INACTIVE, relay UE shall first enter RRC_CONNECTED triggered by 6a. Therefore, step 3 can be between step 5 and 6 in such case. Regarding OPPO’s comment on step 5, we understand the intention of dash line of step-5 is to cover the PC5-RRC bring IDLE/INACTIVE relay UE to CONNECTED. So, we are fine with the dash line of step 5. But we have similar view with OPPO that step should be added to reflect remote UE and relay UE establish PC5 connection, which can be before step-5. Regarding the last FFS, we understand it refers to inter-gNB multipath, which is not supported in R18. If the intention is to avoid such case, FFS may be modified to ‘how to avoid…’ |  |
| ZTE | For step 5, we share the same view as OPPO. The PC5 links used for U2N relay and for normal sidelink traffic should be different. It is not necessary to check whether the remote UE has PC5 unicast link with the target relay UE. Suggest to use “The U2N Remote UE establishes PC5 connection with target U2N Relay UE” for this step. For step 5 and Step 6b, both mentions the scenario that if the relay UE is not in RRC_Connected. It is suggested to merge the corresponding description. |  |
| vivo | We have the following comments: - Step 1: For this step, suggest not to mention a specific measurement Event, which is up to gNB configuration. Instead a general sentence could be used here as, e.g., “The remote UE may report Uu measurement results of neighboring cells and one or multiple candidate relay UEs.” as in the BL CR (R3-232088) to TS 38.401. - Step 2, we understand that “the cell serving the relay UE on the indirect path” means the cell serving the indirect path. To avoid ambiguity, it is preferred to use the term “the cell serving the indirect path”. Suggest: 2. The gNB decides to add the indirect path for remote UE. The cell serving the direct path and the cell serving the relay UE on the indirect path belong to the same gNB but can be same or different. - Step 4: we think “indirect path addition configuration” is a high-level description which can cover all of the remaining configurations. Therefore, suggest to rewording as “The contents in the RRCReconfiguration message may include indirect path addition configuration, which includes at least a target relay UE, indirect path addition configuration, RLC channel configuration, bearer mapping and the associated radio bearer(s).” - Step 5: - We should also consider the relay UE in in RRC inactive, then this step should reflect this case. Suggest: “If the relay UE is not in RRC_CONNECTED, the relay UE initiates a RRC connection establishment or resume procedure upon receiving the PC5-RRC message.” - It is better to separate Step 5 into two sub-steps, - Step 5a PC5 Connection establishment., Step 5a should be solid line as relay/non-relay links will be served by different L2 ID so there should always be a PC5 RRC connection establishment. - Step 5b notification PC5-RRC message - Step 6b: For Step 6b: the arrow from the relay UE to the gNB should be placed after RRC Connection Establishment box. So, we propose to revise the arrow of Step 6b as below: |  |
| CATT | 1. For step 3, gNB should configure the relay UE after it enters RRC_CONNECTED state if the relay UE is in RRC_IDLE/INACTIVE. Therefore, the RRCReconfiguration message to the relay UE may be happened after RRCSetup/Resume procedure. It is proposed to include both cases in the figure. 2. For step 5, it is suggested to add a “PC5 connection establishment” procedure before PC5-RRC message. And change the step 5 to unidirectional from the remote UE to the relay UE. 3. For the last FFS, it should be deleted since inter-gNB MP is not supported. 4. For DL/UL data, refer to 38.300, dash line should be used. |  |
| Apple | 1. We need replace “serving relay UE” with “target relay UE” in the diagram. 2. Step 3: This step cannot occur in IDLE/INACTIVE target relay case. So, we think it is better to put two different signaling diagrams to depict CONNECTED and IDLE/INACTIVEI case respectively. 3. Step 4. We think there should be a “multi-path configuration” concept in this message, not just indirect path configuration. 4. PC5 link establishment part is missing . Suggest to add this before step 5 PC5-RRC. 5. The 2nd arrow in Step 6b is confusing, we think it is better to depict the IDLE/INATIVE case separately in a new diagrem. 6. We support to keep the second FFS, because this is related to how to avoid or recover from handover failure for IDLE/INACTIVE case, so we need to further discuss, as MP case is different from rel-17 SP case. | 1. Even if the UE is configured to measure and report candidate relays. A MP-capable UE may not always need to be configure with two paths, and gNB may have no clear clue whether it is a good to configure Multi-path or not. So, We think RAN2 should discuss whether remote UE can explicitly request to add indirect path in this case, besides the measurement reporting. 2. The content of PC5-RRC message in Step 5 has not been discussed by RAN2 yet. Based on the outcome of the message design, the Stage-2 spec may need update the description of step 5. |
| Fujitsu | Agree with OPPO and Xiaomi for step 5. A dash line can be added into the figure to cover the PC5-RRC trigger to bring IDLE/INACTIVE relay UE to CONNECTED. In addition, discussions may be needed on when to send RRCReconfigurationComplete message if it is sent via the direct path. |  |
| Nokia | 1. FFS under step 5 should not be only about which message to send to initiate RRC setup/resume for the relay UE not in RRC_CONNECTED, but it is also related to where to send the RRCReconfigurationComplete message: 1) If the relay UE is already in RRC_CONNECTED, the RRCReconfigurationComplete message doesn’t need to be sent over the relay UE. 2) Even if the relay UE is not in RRC_CONNECTED, the gNB may prefer to receive RRCReconfigurationComplete message via the direct path for fast completion of reconfiguration procedure. 2. Regarding step 4, As pointed out by OPPO/Xiaomi, we don’t think the timing of transmission of RRCReconfigurationComplete is linked with step5. Meanwhile, we are fine with dash line of step 5 because the intention is to use PC5-RRC to bring the relay UE to RRC_CONNECTED when RRCReconfigurationComplete is sent over the direct path. 3. FFS under step 6b is not needed because it is not supported in Rel-18. |  |
| NEC | According to current agreements, Remote UE always send RRCReconfigurationComplete message via direct path and it is optional to send this message via indirect path or send PC5-RRC signalling via indirect path, So we have the following suggestions: Step 5. The remote UE sends the RRCReconfigurationComplete message to the gNB at least via the direct path in order to complete the indirect path addition procedure. Step6a. If the remote UE sends the RRCReconfigurationComplete message only on the direct path in Step 5, the remote UE sends a PC5-RRC message to the target relay UE. If the relay UE is not in RRC_CONNECTED, the relay UE initiates a RRC connection establishment upon receiving the PC5-RRC message. Current Step6b is kept. |  |
| InterDigital | Similar comment in previous question on step 4 (contents of the reconfiguration message). Similar common in previous question on step 6: should be clarified to consider the case where non-split SRB is configured on the indirect path only and add this as an FFS. Step 3b may be after step 6 for an IDLE/INACTIVE relay. Step 5 should be 2 steps: 5a) PC5 unicast link establishment and 5b) PC5-RRC message. 5b may not be needed in some cases, and can be shown optional. Our understanding on the FFS after step 5 is that it is not needed: the PC5-RRC message is used/not used depending on the presence of SRB1 on the indirect path. For the FFS after step 6, we should discuss how to avoid this case for Rel18, so it is relevant to keep the FFS. |  |
| Samsung | Caption of Fig It should be “Indirect path addition on top of direct path” Step 1: For stage-2, it may not need to involve the measurement event. In addition, this step only needs mention the measurement behaviour of remote UE since this is a procedure for remote UE. In Rel-17, for direct to indirect path switch, we don’t include relay UE measurement procedure either. Step 3: Shall we also address the case when relay UE is in IDLE/INACTIVE state as RAN3? The step 3 can be carried out if the relay UE is in RRC_CONNECTED status. For the case of relay UE in RRC_IDLE/INACTIVE status, the RRC reconfiguration procedure can be performed to the relay UE together with the step “RRC connection establishment” after step 6a. Step 4: “RLC channel configuration”  “PC5 RLC channel configuration” Step 5: “step 6”  “step 6a”; “RRC connection establishment”  “RRC connection establishement/resume” Step “RRC connection establishment” It should be “RRC connection establishment/resume” |  |
| Huawei, HiSilicon | Step 1 Similar comments as others, here relay UE does not required to perform measurement. And we can generalize the description by skipping measurement details. Step 4 The current description is quite similar to path switch to a relay which is not the case, thus we suggest: 4. The gNB sends the RRCReconfiguration message to the remote UE, containing MP configuration which includes the information required to access the target relay on indirect path. The contents in the RRCReconfiguration message may include at least the target relay UE, indirect path addition configuration, RLC channel configuration, bearer mapping and the associated radio bearer(s). Step 5 We share the same view as Interdigital that the FFS after step 5 is not needed: the PC5-RRC message is used/not used depending on the presence of SRB1 on the indirect path. Step 6a/6b Similar comments to the question in 1.1. |  |
| Ericsson | Step 2: The cell serving the direct path and the cell serving the relay UE on the indirect path should belong to the same gNB. But the cells themselves can be same or different Step 4: “at least the target relay UE”? Is there something missing? Does it mean relay UE ID? |  |
| Fraunhofer | Agree with Appel regarding the MultiPath configuration and request for adding indirect path. If the application requests a more reliable connection or more throughput, this can be solved by requesting an additional indirect path. |  |
| LG Electronics | Step 1 The specific event can be removed as commented above. Step 3 FFS can be added to check if RAN2 can confirm what RAN3 agreed or the order can be up to network implementation for relay UE in RRC_CONNECTED. Step 7 can be added for the relay UE not in RRC_CONNECTED. Step 4 Some comments on the contents in the message seem Stage 3 details. Considering that we are focusing on Stage 2 level procedure in this email discussion, details about the contents can be removed here and go to Stage 3. Step 5a As commented above, PC5-RRC connection establishment can be mandated. Step 5b As commented above, RRC connection resume can be added Step 6b FFS on the order of step 6a and 5 can be added Non-support for inter-gNB multicast case can be clarified in FFS. Step 7 This step can be added for the relay not in RRC_CONNECTED. |  |

##### Summary and Updated Procedure

The updated procedure based on the above comments is shown below. Note that any change to the figure is shown in red.

#### 1.2 Indirect path addition on top of direct path Figure 1.2: Signalling procedure of direct path addition on top of indirect path 1. The remote UE and the relay UE perform measurements based on measurement configuration. The remote UE may report measurement results. 2. The gNB decides to add the indirect path for remote UE. The cell serving the direct path and the cell serving the relay UE on the indirect path belong to the same gNB but can be same or different. 3. If the relay UE is in RRC_CONNECTED (Case 1), the gNB sends an RRCReconfiguration message to the relay UE to configure the indirect path of the remote UE. (Note that if the relay UE is not in RRC_CONNECTED, the gNB sends an RRCReconfiguration message to the relay UE after the relay UE enters RRC_CONNECTED in the step 7.) FFS: Whether RAN2 can confirm the step 3 as agreed in RAN3 for the relay UE in RRC_CONNECTED, noting that this step is based on RAN3 agreement. Whether the order of the step 3 and 4 is up to network implementation for the relay UE in RRC_CONNECTED. 4. The gNB sends the RRCReconfiguration message to the remote UE for indirect path addition. FFS: Stage 3 details about the contents in the RRCReconfiguration message. 5a. The remote UE establishes a PC5-RRC connection with the target relay UE. 5b. If the remote UE sends the RRCReconfigurationComplete message only on the direct path in Step 6a, the remote UE sends a PC5-RRC message to the target relay UE. If the relay UE is not in RRC_CONNECTED, the relay UE initiates a RRC connection establishment or a RRC connection resume upon receiving the PC5-RRC message. FFS: Need for additional condition to trigger the PC5-RRC message (e.g. based on RRC state of the relay UE), which PC5-RRC message is used for this purpose (e.g. new or existing one), and the contents in the PC5-RRC message (e.g. trigger RRC establishment). 6a. The remote UE sends the RRCReconfigurationComplete message to the gNB at least via the direct path in order to complete the indirect path addition procedure. 6b. If a split SRB1 with duplication is configured, the remote UE also sends the RRCReconfigurationComplete message to the gNB via the indirect path served by the relay UE. If relay UE is not in RRC_CONNECTED, the relay UE initiates a RRC connection establishment to send the RRCReconfigurationComplete message to the gNB. FFS: The order of step 5 and 6a e.g. whether step 6a can start before step 5 starts. FFS: Whether/How to avoid/handle the case when the relay UE establishes a RRC connection with a different gNB than the gNB serving the target cell, noting that the inter-gNB multipath case is not supported in Rel-18. 7. If the relay UE is not in RRC_CONNECTED (Case 2), the gNB sends an RRCReconfiguration message to the relay UE after the relay UE enters RRC_CONNECTED.

### 1.3 Direct path change in Scenario 1

##### Related agreements

RAN2 agreements related with this procedure are listed below:

- The following case can be supported via separate release-and-add for scenario 1 (B+D in separate reconfigurations):

E.	The remote UE operating in multi-path changes the direct path to a different cell of the same gNB while using the serving relay UE for the indirect path under the same gNB.

FFS if a single procedure for this case would be supported.

- Support PCell on the direct path only when the UE is in multi-path operation, for both scenario 1 and scenario 2.

- Change of direct path while keeping the indirect path can be done with a release-and-add in a single RRC message.  This does not exclude a gNB implementation from using separate release and add procedures instead.

- For Scenario-1, use T304-like timer for direct path addition and change. FFS on expiry behavior.

##### Procedure for review

The procedure based on the related agreements is shown below.

#### 1.3 Direct path change Figure 1.3: Signalling procedure of direct path change under a single procedure 0. The remote UE performs data transmission and reception by using both the direct path on PCell and the indirect path served by a relay UE. 1. The remote UE and the relay UE perform measurements based on measurement configuration. The remote UE may report measurement results e.g. for Event A3 (Neighbour becomes offset better than SpCell). 2. The gNB decides to change the direct path of the remote UE from the PCell (i.e. source PCell) to a new cell (i.e. target PCell). The source/target PCell serving the old/new direct path and the cell serving the relay UE on the indirect path belong to the same gNB but can be same or different. 3. The gNB sends an RRCReconfiguration message to the relay UE to update the indirect path configuration, if necessary. 4. The gNB sends the RRCReconfiguration message to the remote UE via the relay UE. The contents in the RRCReconfiguration message may include at least a target PCell, direct path release and addition configuration, RLC channel configuration, bearer mapping and the associated radio bearer(s). 5. The remote UE synchronizes to DL of the target PCell serving the new direct path and performs random access procedure towards the target PCell serving the new direct path. 6a. The remote UE sends the RRCReconfigurationComplete message to the gNB at least via the direct path in order to complete the direct path change procedure. 6b. If a split SRB1 with duplication is configured, the remote UE also sends the RRCReconfigurationComplete message to the gNB via the indirect path served by the target relay UE.

Companies are requested to check the procedure and provide comments and remaining open issues on this procedure, if any.

| Company | Comments | Remaining open issues |
| --- | --- | --- |
| OPPO | In step-4, for the following part, the detailed configuration would be up to stage-3, e.g., there may be no explicit “release” or “addition” configuration, but just in a form of reconfiguration-with-sync from pcell1 to pcell2. 4. The gNB sends the RRCReconfiguration message to the remote UE via the relay UE. The contents in the RRCReconfiguration message may include at least a target PCell, direct path release and addition configuration, RLC channel configuration, bearer mapping and the associated radio bearer(s). |  |
| Xiaomi | Step 4, we understand the RRCReconfiguration message can be sent on direct and/or indirect path. But seems current step 4 mandates the indirect path. |  |
| ZTE | For step 4, it is not clear why only indirect path can be used for the transmission of RRCReconfiguration message. In our opinion, which path can be used depends on the gNB configuration and implementation. It is suggested to remove “via the relay UE”. In addition, it is suggested to clarify that the RLC channel configuration denotes the PC5 Relay RLC channel configuration. 4. The gNB sends the RRCReconfiguration message to the remote UE via the relay UE. The contents in the RRCReconfiguration message may include at least a target PCell, direct path release and addition configuration, RLC channel configuration, bearer mapping and the associated radio bearer(s). |  |
| vivo | - Step 2: Suggest not to mention a specific measurement Event, which is up to gNB configuration. Instead a general sentence could be used here as, e.g., “The remote UE may report Uu measurement results and one or multiple candidate relay UEs.” as in the BL CR (R3-232088) to TS 38.401. - Step 3: Depending the situation this step may not be executed at this stage or the RRC reconfiguration to serving relay UE can be executed after Step 6, if necessary. So, we suggest the box in the figure to be dash line - Step 5: Random access includes multiple UL/DL msgs, double arrow should be used. | Maybe we should also consider the order in which the reconfiguration between the relay UE and remote UE is performed. |
| CATT | 1. For step 3, suggest using dashed box since the reconfiguration for the relay UE is not mandatory. 2. For step 4, the RRCReconfiguration message can be sent on direct and/or indirect path. depending on SRB1 configuration. 3. For step 6a, which path(s) is(are) used to carry RRCReconfigurationComplete message depending on SRB1 configuration. RAN2 has no agreement on whether non-split SRB can be configured on indirect path. 4. For DL/UL data, refer to 38.300, dash line should be used. |  |
| Apple | 1. Step 3 need to be moved after Step 6. 2. Step 4, which path to send this RRCReconfiguration message needs further discussed. Then, for the information included in RRCReconfiguration, we think this shall be a “multi-path configuration” which includes some detailed configuration about how to split/duplicate traffic for each DRB. It is not just a direct path configuration. So, the current description is insufficient. Also, since there is a PCell change, “ReconfigurationWithSync” needs to be included too. 3. Step 6a. Same view as CATT. |  |
| Fujitsu | Agree with OPPO and Xiaomi. |  |
| Nokia | 1. For step 4, it would be up to NW implementation where to send the reconfiguration message if split SRB1 is configured, i.e, via direct path or indirect path, although it would make sense to send over the indirect path in a case where the direct path needs to be changed due to poor link quality. 2. We wonder whether 6a/6b means that the remote UE shouldn’t send the RRCReconfigurationComplete message via the indirect path even if the primary path of the split SRB1 is set to the indirect path without duplication. | It is still FFS where to send the RRCReconfigurationComplete message? 1) The gNB sets the primary path to the direct path if relay UE is in RRC_CONNECTED and the UE sends the RRCReconfigurationComplete message via (at least) the primary path. 2) Regardless of the primary path, the remote UE sends the RRCReconfigurationComplete message via the direct path as long as the relay UE is in RRC_CONNECTED. |
| NEC | We suggest to add a separate Step4a, where - Step4 is for release and adding, or for direct path change only, and - Step4a is for adding the indirect path after direct path change. Accordingly, another step for RRCReconfigurationComplete message of Step4a is needed. |  |
| InterDigital | Similar comments to steps 4 and 6 from previous question. |  |
| Samsung | Step 1: The measurement event may not need since it is stage-2 procedure. In addition, this step only needs mention the measurement behaviour of remote UE since this is a procedure for remote UE. In Rel-17, for direct to indirect path switch, we don’t include relay UE measurement procedure either. Step 4: - “at least” is not needed since the configurations for RLC channel and bearer mapping may not be changed during direct path change - The RRCReconfiguration message can be sent via direct path in this step. If indirect path is used, the split SRB1 should be configured first. - “RLC channel configuration”  “PC5 RLC channel configuration” Step 6b: “target relay UE”  “relay UE” since relay UE is not changed in this procedure |  |
| Huawei, HiSilicon | Step 4 This is legacy PCell change procedure, thus existing reconfigurationWithSync is sufficient, thus “, direct path release and addition configuration” should be deleted. And the description of the content should be the same as direct path addition. Step 6a/6b Similar comments to the question 1.1. |  |
| Ericsson | Similar comments on Step-4 and Step-6 Also comments from Q1.1 for Step-2 |  |
| Fraunhofer | Similar comments as Vivo, RAProcedure should be two-way arrow. The RA procedure can be configured to be CFRA. |  |
| LG Electronics | Step 1 The specific event can be removed as commented above. Step 3 FFS can be added to check if RAN2 can confirm step 3 or the order can be up to network implementation. Step 4 The RRCReconfiguration message can be sent to the remote UE on one of both or both paths depending on network configuration/implementation. Some comments on the contents in the message seem Stage 3 details. Considering that we are focusing on Stage 2 level procedure in this email discussion, details about the contents can be removed here and go to Stage 3. Step 6 FFS on the complete message only via the indirect path can be added for future discussion. |  |

##### Summary and Updated Procedure

The updated procedure based on the above comments is shown below. Note that any change to the figure is shown in red.

#### 1.3 Direct path change Figure 1.3: Signalling procedure of direct path change under a single procedure 0. The remote UE performs data transmission and reception by using both the direct path on PCell and the indirect path served by a relay UE. 1. The remote UE and the relay UE perform measurements based on measurement configuration. The remote UE may report measurement results. 2. The gNB decides to change the direct path of the remote UE from the PCell (i.e. source PCell) to a new cell (i.e. target PCell). The source/target PCell serving the old/new direct path and the cell serving the relay UE on the indirect path belong to the same gNB but can be same or different. 3. The gNB sends an RRCReconfiguration message to the relay UE to update the indirect path configuration, if necessary. FFS: Whether RAN2 can confirm the step 3. Whether the order of the step 3 and 4 is up to network implementation. 4. The gNB sends the RRCReconfiguration message to the remote UE on the direct path and/or the indirect path for direct path change. FFS: Stage 3 details about the contents in the RRCReconfiguration message. 5. The remote UE synchronizes to DL of the target PCell serving the new direct path and performs random access procedure towards the target PCell serving the new direct path. 6a. The remote UE sends the RRCReconfigurationComplete message to the gNB at least via the direct path in order to complete the direct path change procedure. 6b. If a split SRB1 with duplication is configured, the remote UE also sends the RRCReconfigurationComplete message to the gNB via the indirect path served by the relay UE. FFS: whether the primary path of the split SRB1 can be set to the indirect path without duplication, especially for the RRCReconfigurationComplete message, whether the non-split SRB can be supported on indirect path, and whether the remote UE sends the RRCReconfigurationComplete message to the gNB only via the indirect path.

### 1.4 Indirect path change in Scenario 1

##### Related agreements

RAN2 agreements related with this procedure are listed below:

- For bringing the idle/inactive relay UE to RRC_CONNECTED, the legacy Rel-17 behaviour (Alt 1 in the proposal) is not disabled for indirect path addition when split SRB1 is configured.  A PC5-RRC trigger is specified at least for other cases.

  - FFS if a Rel-17 relay UE is supported for use with multi-path and how the above agreement is reflected in such a case.

- Change of indirect path while keeping the direct path can be done with a release-and-add in a single RRC message.  This does not exclude a gNB implementation from using separate release and add procedures instead.

- For Scenario-1, use T420-like timer for indirect path addition and change. FFS on stop condition and expiry behavior.

##### Procedure for review

The procedure based on the related agreements is shown below.

#### 1.4 Indirect path change Figure 1.4: Signalling procedure of indirect path change under a single procedure 0. The remote UE performs data transmission and reception by using both the direct path on PCell and the indirect path served by a relay UE. 1. The remote UE and the relay UE perform measurements based on measurement configuration. The relay UE may report measurement results e.g. for inter-gNB handover, or the remote UE may report measurement results. FFS: Any need for measurement event from the remote UE for indirect path change. 2. The gNB decides to change the indirect path of remote UE from the relay UE (i.e. source relay UE) to a new relay UE (i.e. target relay UE). The cell serving the direct path and the cell serving the source/target relay UE on the indirect path belong to the same gNB but can be same or different. 3. The gNB sends an RRCReconfiguration message to the source relay UE to release the indirect path of the remote UE. The gNB sends an RRCReconfiguration message to the target relay UE to add the indirect path of the remote UE. FFS: When the RRCReconfiguration messages are sent to the source/target relay UEs. 4. The gNB sends the RRCReconfiguration message to the remote UE. The contents in the RRCReconfiguration message may include at least the target relay UE, indirect path release and addition configuration, RLC channel configuration, bearer mapping and the associated radio bearer(s). 5. If the remote UE has no PC5 unicast link with the target relay UE, the remote UE establishes a PC5 unicast link with the target relay UE. If the remote UE sends the RRCReconfigurationComplete message only on the direct path in Step 6, the remote UE sends a PC5-RRC message to the target relay UE. If the relay UE is not in RRC_CONNECTED, the relay UE initiates a RRC connection establishment upon receiving the PC5-RRC message. FFS: Need for additional condition to trigger the PC5-RRC message (e.g. based on RRC state of the relay UE), which PC5-RRC message is used for this purpose (e.g. new or existing one), and the contents in the PC5-RRC message (e.g. trigger RRC establishment). 6a. The remote UE sends the RRCReconfigurationComplete message to the gNB at least via the direct path in order to complete the indirect path change procedure. 6b. If a split SRB1 with duplication is configured, the remote UE also sends the RRCReconfigurationComplete message to the gNB via the indirect path served by the target relay UE. If relay UE is not in RRC_CONNECTED, the relay UE initiates a RRC connection establishment to send the RRCReconfigurationComplete message to the gNB. FFS: Whether/How to handle the case when the target relay UE establishes a RRC connection with a different gNB than the gNB serving the target cell.

Companies are requested to check the procedure and provide comments and remaining open issues on this procedure, if any.

| Company | Comments | Remaining open issues |
| --- | --- | --- |
| OPPO | Step-5, can rename it to PC5 connection establishment, since PC5-S signaling is needed besides PC5-RRC. And considering that relay/non-relay links will be served by different L2 ID, is there still a need to use dash line for step-5? And we wonder if there is a need to mandate step-6a to be after step-5, considering in legacy, RRCReconfigurationComplete for SCell addition or SCG addition does not have to be after the completion of SCell addition or SCG addition. |  |
| Xiaomi | First FFS, in service continuity agenda, RAN2 agreed Z1 to support indirect to indirect mobility, which can be reused in indirect path change. Step 5, the RRCReconfiguration can be sent on direct and/or indirect path. Regarding OPPO’s comment on step 5, we understand the intention of dash line of step-5 is to cover the PC5-RRC bring IDLE/INACTIVE relay UE to CONNECTED. So, we are fine with the dash line of step 5. But we have similar view with OPPO that step should be added to reflect remote UE and relay UE establish PC5 connection, which can be before step-5. Regarding the last FFS, we understand it refers to inter-gNB multipath, which is not supported in R18. If the intention is to avoid such case, FFS may be modified to ‘how to avoid…’ |  |
| ZTE | For step 4: as we suggested before, it is suggested to clarify that the RLC channel configuration denotes the PC5 Relay RLC channel configuration. For step 5: The PC5 links used for U2N relay and for normal sidelink traffic should be different. It is not necessary to check whether the remote UE has PC5 unicast link with the target relay UE. Suggest to use “The U2N Remote UE establishes PC5 connection with target U2N Relay UE” for this step. For step 5 and Step 6b, both mentions the scenario that if the relay UE is not in RRC_Connected. It is suggested to merge the corresponding description for this scenario. |  |
| vivo | - Step 1: Suggest to modify as in the BL CR (R3-232088) to TS 38.401. “the remote UE may report Uu measurement results and one or multiple candidate relay UEs.” - Step 3: It is depending on the target relay UE’s RRC_CONNECTED state. For a common procedure with the target relay UE in RRC_IDLE and RRC_INACTE as legacy, the sentence “The gNB sends an RRCReconfiguration message to the target relay UE to add the indirect path of the remote UE.” can be removed in step 3 and executed after “RRC connection establishment”. - Step 4: Suggest the following revision “The contents in the RRCReconfiguration message may include at least the target relay UE, indirect path release and addition configuration, which includes the target relay UE, RLC channel configuration, bearer mapping and the associated radio bearer(s).” - Step 5: - We should also consider the relay UE in in RRC inactive, then this step should reflect this case. Suggest: “If the relay UE is not in RRC_CONNECTED, the relay UE initiates a RRC connection establishment or resume procedure upon receiving the PC5-RRC message.” - this step may comprise two sub-steps, - step 5a PC5 RRC Connection Setup. - Step 5b notification PC5-RRC message - Step 6b: - Similar to Step 5 revise as “… RRC connection establishment or resume procedure upon …” - Similar to 1.2, in Step 6b, the arrow from target relay UE to gNB should be changed to double arrow and placed after “RRC Connection Establishment”. |  |
| CATT | 1. For step 3, gNB should configure the relay UE after it enters RRC_CONNECTED state if the relay UE is in RRC_IDLE/INACTIVE. Therefore, the RRCReconfiguration message to the relay UE may be happened after RRCSetup/Resume procedure. It is proposed to include both cases in the figure. 2. For step 5, it is suggested to add a “PC5 connection establishment” procedure before PC5-RRC message. And change the step 5 to unidirectional from the remote UE to the relay UE. 3. For the last FFS, it should be deleted since inter-gNB MP is not supported. 4. For DL/UL data, refer to 38.300, dash line should be used. |  |
| Apple | Similar comments as in Section 1.2 |  |
| Fujitsu | Agree with OPPO and Xiaomi for step 5. A dash line can be added into the figure to cover the PC5-RRC trigger to bring IDLE/INACTIVE relay UE to CONNECTED. In addition, discussions may be needed on when to send RRCReconfigurationComplete message if it is sent via the direct path. |  |
| Nokia | 1. For step 4, it would be up to NW implementation where to send the reconfiguration message if split SRB1 is configured, i.e., via direct path or indirect path, although it would make sense to send over the direct path in a case where the indirect path needs to be changed due to some reasons. 2. FFS under step 5 should not be only about which message to send to initiate RRC setup/resume for the relay UE not in RRC_CONNECTED, but it is also related to where to send the RRCReconfigurationComplete message: 1) If the relay UE is already in RRC_CONNECTED, the RRCReconfigurationComplete message doesn’t need to be sent over the relay UE. 2) Even if the relay UE is not in RRC_CONNECTED, the gNB may prefer to receive RRCReconfigurationComplete message via the direct path for fast completion of reconfiguration procedure. 2. Regarding step 5, as pointed out by OPPO/Xiaomi, we don’t think the timing of transmission of RRCReconfigurationComplete is linked with step5. Meanwhile, we are fine with dash line of step 5 because the intention is to use PC5-RRC to bring the relay UE to RRC_CONNECTED when RRCReconfigurationComplete is sent over the direct path. |  |
| NEC | Similar suggestions as in section1.2 for configuring the target indirect path. |  |
| InterDigital | Similar comments to steps 4, 5, and 6 from previous questions. |  |
| Samsung | Step 1: This step only needs mention the measurement behaviour of remote UE since this is a procedure for remote UE. In Rel-17, for direct to indirect path switch, we don’t include relay UE measurement procedure either. Step 3: Should we consider the relay UE in RRC_CONNECTED and RRC_IDLE/INACTIVE separately? Similar comments as procedure 1.2 Step 4: “RLC channel configuration” “PC5 RLC channel configuration” Step 5: “Step 6”  “step 6a” |  |
| Huawei,HiSilicon | Similar comments to the question in 1.2. |  |
| Ericsson | Same comments as in Q1.2, 1.3 |  |
| Fraunhofer | We think that Step 4 should be kept on the direct path and not sent on the indirect path. |  |
| LG Electronics | Step 3 RRC reconfiguration for the source/target relay UE can be separated in Step 3a/b. FFS can be added to check if RAN2 can confirm Step 3a/b or the order can be up to network implementation for relay UE in RRC_CONNECTED. Step 7 can be added for the target relay UE not in RRC_CONNECTED. Step 4 The RRCReconfiguration message can be sent to the remote UE on one of both or both paths depending on network configuration/implementation. Some comments on the contents in the message seem Stage 3 details. Considering that we are focusing on Stage 2 level procedure in this email discussion, details about the contents can be removed here and go to Stage 3. Step 5a As commented above, PC5-RRC connection establishment can be mandated. Step 5b As commented above, RRC connection resume can be added Step 6b FFS on the order of step 6a and 5 can be added Non-support for inter-gNB multicast case can be clarified in FFS. Step 7 This step can be added for the target relay not in RRC_CONNECTED. |  |

##### Summary and Updated Procedure

The updated procedure based on the above comments is shown below. Note that any change to the figure is shown in red.

#### 1.4 Indirect path change Figure 1.4: Signalling procedure of indirect path change under a single procedure 0. The remote UE performs data transmission and reception by using both the direct path on PCell and the indirect path served by a relay UE. 1. The remote UE and the relay UE perform measurements based on measurement configuration. The relay UE may report measurement results e.g. for inter-gNB handover, or the remote UE may report measurement results. FFS: Any need for measurement event from the remote UE for indirect path change. 2. The gNB decides to change the indirect path of remote UE from the relay UE (i.e. source relay UE) to a new relay UE (i.e. target relay UE). The cell serving the direct path and the cell serving the source/target relay UE on the indirect path belong to the same gNB but can be same or different. 3a. The gNB sends an RRCReconfiguration message to the source relay UE to release the indirect path of the remote UE. FFS: When the RRCReconfiguration messages are sent to the source relay UEs. Whether the order of the step 3 and 4 is up to network implementation for the relay UE in RRC_CONNECTED. 3b. If the target relay UE is in RRC_CONNECTED (Case 1), the gNB sends an RRCReconfiguration message to the target relay UE to add the indirect path of the remote UE. (Note that if the target relay UE is not in RRC_CONNECTED, the gNB sends an RRCReconfiguration message to the target relay UE after the target relay UE enters RRC_CONNECTED in the step 7.) FFS: When the RRCReconfiguration messages are sent to the target relay UEs. Whether the order of the step 3 and 4 is up to network implementation for the relay UE in RRC_CONNECTED. 4. The gNB sends the RRCReconfiguration message to the remote UE on the direct path and/or the indirect path for indirect path change.. FFS: Stage 3 details about the contents in the RRCReconfiguration message. 5a. The remote UE establishes a PC5-RRC connection with the target relay UE. 5b. If the remote UE sends the RRCReconfigurationComplete message only on the direct path in Step 6a, the remote UE sends a PC5-RRC message to the target relay UE. If the relay UE is not in RRC_CONNECTED, the relay UE initiates a RRC connection establishment or a RRC connection resume upon receiving the PC5-RRC message. FFS: Need for additional condition to trigger the PC5-RRC message (e.g. based on RRC state of the relay UE), which PC5-RRC message is used for this purpose (e.g. new or existing one), and the contents in the PC5-RRC message (e.g. trigger RRC establishment). 6a. The remote UE sends the RRCReconfigurationComplete message to the gNB at least via the direct path in order to complete the indirect path change procedure. 6b. If a split SRB1 with duplication is configured, the remote UE also sends the RRCReconfigurationComplete message to the gNB via the indirect path served by the target relay UE. If relay UE is not in RRC_CONNECTED, the relay UE initiates a RRC connection establishment to send the RRCReconfigurationComplete message to the gNB. FFS: The order of step 5 and 6a e.g. whether step 6a can start before step 5 starts. FFS: Whether/How to avoid/handle the case when the target relay UE establishes a RRC connection with a different gNB than the gNB serving the target cell, noting that the inter-gNB multipath case is not supported in Rel-18. 7. If the target relay UE is not in RRC_CONNECTED (Case 2), the gNB sends an RRCReconfiguration message to the target relay UE after target the relay UE enters RRC_CONNECTED.

### 1.5 Direct path failure in Scenario 1

##### Related agreements

RAN2 agreements related with this procedure are listed below:

- When UE operating in multi-path Relay, it performs RLM for Uu interface, for Scenario-1 and Scenario-2.

- RAN2 further study how for UE operating in multi-path Relay operate for RRC Re-establishment procedure.

- Upon detection of 3GPP-defined RLF failure in one path, remote UE (configured with MP) can report path failure via the alternative available path if SRB1 is configured on the alternative path or split SRB1 is configured.

- In case of Uu-RLF, at least for split SRB1, if SRB1 is available on indirect path not suspended, trigger report to network via indirect path to report the failure via a RRC message. Otherwise, RRC Re-establishment is initiated. RAN2 is requested to discuss whether the RRC message is the existing message e.g. MCGFailureInformation or a new message.

- The remote UE initiates RRC re-establishment procedure (to a potentially new PCell as in Rel-17, unless further changes are agreed) when failure occurs on both paths (including either PC5 failure or notification of Uu failure on the indirect path).

- For Scenario-1/2, when reporting direct-path failure via indirect-path, use MCGFailureInformation message. FFS on whether additional IE needs to be introduced.

- For Scenario-1/2, if MCGFailureInformation is agreed for direct path failure recovery in P5, reuse T316 timer for the direct path failure recovery.

- WA: For scenario 1, primary path of the split SRB1 and SRB2 is always configured on direct path.  This does not preclude having the case where the UE switches the primary path to the indirect path for reporting after direct path failure.

##### Procedure for review

The procedure based on the related agreements is shown below.

#### 1.5 Direct path failure detection and recovery without RRC re-establishment Figure 1.5: Signalling procedure of direct path failure detection and recovery without RRC re-establishment 0. The remote UE performs data transmission and reception by using both the direct path on PCell and the indirect path served by a relay UE. 1. The remote UE performs Uu RLM on the direct path and SL RLM on the indirect path and may detect RLF on the direct path as specified in 38.331. 2. Upon RLF detection on the direct path, if SRB1 has been configured on the indirect path or split SRB1 has been configured, and if the indirect path has been not suspended, the remote UE suspends the direct path, starts timer T316 and sends the MCGFailureInformation message to the gNB via the indirect path as specified in 38.331. Otherwise, the remote UE initiates the RRC connection re-establishment procedure as specified in 38.331. 3. Upon receiving the MCGFailureInformation message, the gNB may send a RRCReconfiguration message to the remote UE in order to change the direct path (as described in 1.3) or release the direct path (as described in 1.7) for the remote UE e.g. based on measurement results on the serving cell(s) and neighboring cells in the MCGFailureInformation message.

Companies are requested to check the procedure and provide comments and remaining open issues on this procedure, if any.

| Company | Comments | Remaining open issues |
| --- | --- | --- |
| OPPO | Considering legacy spec does not capture this failure handling procedure using signaling flow + step-based description in stage-2, we slightly prefer avoid such description in 300. If it is to be captured, we understand the following part is still pending 2. Upon RLF detection on the direct path, if SRB1 has been configured on the indirect path or split SRB1 has been configured, and if the indirect path has been not suspended, the remote UE suspends the direct path, starts timer T316 and sends the MCGFailureInformation message to the gNB via the indirect path as specified in 38.331. Otherwise, the remote UE initiates the RRC connection re-establishment procedure as specified in 38.331. |  |
| Xiaomi | It’s incomplete to only specify the T316 start and not say how T316 is stopped and the expiry result. We suggest to not specify anything about T316. Such details can be left to stage3. |  |
| ZTE | For step 2, agree with oppo that the following part is still FFS “if SRB1 has been configured on the indirect path” |  |
| vivo | - Step 1: There is no “SL RLM” concept for PC5. Suggest to rewording as “SL RLM RLF detection as specified in TS 38.331” according to the following agreement: Agreement: For PC5 interface in Scenario-1, it performs sidelink RLF detection based on Rel-16 V2X specification. - Step 2: - This “SRB1 has been configured on the indirect path” case has not been agreed. Suggest to remove it or add an Editor Note. For example: Editor Note FFS if supporting SRB1 configured on the indirect path. - There is too much stage 3 description such as “…starts timer T316…” prefer to remove them and follow similar stage 2 description as what we have in TS 37.340 Clause 7.7. |  |
| CATT | Share the same view as OPPO. |  |
| Apple | For step 2, similar view as Vivo |  |
| Fujitsu | We think that step 3 is up to gNB implementation and can be removed. In addition, the FFS in the following RAN2 agreement is proposed to be captured. “For Scenario-1/2, when reporting direct-path failure via indirect-path, use MCGFailureInformation message. FFS on whether additional IE needs to be introduced.” |  |
| Nokia | 1. For step 1, we can simplify the description by saying that ‘the remote UE detects an RLF on the direct path as specified in 38.331’ because SL RLF detection has no impact on this procedure. 2. For step 2, it is still WA whether the remote UE can switch the primary path to the indirect path for transmission of the MCGFailureInformation via the indirect path. If it is not allowed, our understanding is that the remote UE initiates the RRC connection re-establishment procedure because the remote UE cannot inform the gNB of the failure. Perhaps, it would be good to 1) change [has been not suspended] to [is available] and 2) add FFS under step 2 saying that ‘Whether the remote UE switches the primary path to the indirect path upon Uu-RLF detection so that the indirect path of split SRB1 becomes available for transmission of MCGFailureInformation’. |  |
| NEC | This section is not needed for stage 2 level. |  |
| InterDigital | Same comments as Vivo. |  |
| Samsung | Step 2: FFS should be added w.r.t. configuring SRB1 at indirect path |  |
| Huawei, HiSilicon | We think a more generic description is sufficient like how MCG failure recovery is captured in TS 37.340. For the technical part, we also agree with vivo. |  |
| Ericsson | Agree with Huawei | SRB1 issue is still pending |
| Fraunhofer | On expiry of T316 – according to 38.331 – the UE will perform RRCReestablishment, which the title says, the UE shall not do. There needs to be spec defining the behaviour if T316 expires. | T316 expiration impact |
| LG Electronics | Step 1 SL RLM can be removed Step 2 FFS can be added to need to confirm the WA on the primary path to the indirect path FFS on SRB1 on the indirect path for reporting the failure can be added FFS on additional IE in the MCGFailureInformation message can be added. |  |

##### Summary and Updated Procedure

The updated procedure based on the above comments is shown below. Note that any change to the figure is shown in red.

#### 1.5 Direct path failure detection and recovery Figure 1.5: Signalling procedure of direct path failure detection and recovery without RRC re-establishment 0. The remote UE performs data transmission and reception by using both the direct path on PCell and the indirect path served by a relay UE. 1. The remote UE performs Uu RLM on the direct path and may detect RLF on the direct path as specified in 38.331. 2. Upon RLF detection on the direct path, if split SRB1 has been configured, and if the indirect path has been not suspended, the remote UE suspends the direct path and sends the MCGFailureInformation message to the gNB via the indirect path as specified in 38.331. Otherwise, the remote UE initiates the RRC connection re-establishment procedure as specified in 38.331. FFS: Need to confirm that the WA whether the remote UE can switch the primary path to the indirect path for transmission of the MCGFailureInformation via the indirect path. If it is not allowed, whether the remote UE initiates the RRC connection re-establishment procedure. FFS: whether SRB1 can be configured on the indirect path for reporting the failure. FFS: whether additional IE needs to be introduced in the MCGFailureInformation message for this case. 3. Upon receiving the MCGFailureInformation message, the gNB may send a RRCReconfiguration message to the remote UE in order to change the direct path (as described in 1.3) or release the direct path (as described in 1.7) for the remote UE e.g. based on measurement results on the serving cell(s) and neighboring cells in the MCGFailureInformation message.

### 1.6 Indirect path failure in Scenario 1

##### Related agreements

RAN2 agreements related with this procedure are listed below:

- For PC5 interface in Scenario-1, it performs sidelink RLF detection based on Rel-16 V2X specification.

- RAN2 further study how for UE operating in multi-path Relay operate for RRC Re-establishment procedure.

- Upon detection of 3GPP-defined RLF failure in one path, remote UE (configured with MP) can report path failure via the alternative available path if SRB1 is configured on the alternative path or split SRB1 is configured.

- In case of PC5-RLF, if SRB1 is available on direct path not suspended, trigger report to network via direct path to report the failure via a RRC message.  FFS if an alternative case exists and what would be done in that case.  FFS which message is used.

- The remote UE initiates RRC re-establishment procedure (to a potentially new PCell as in Rel-17, unless further changes are agreed) when failure occurs on both paths (including either PC5 failure or notification of Uu failure on the indirect path).

- The existing PC5-RRC Notification Message procedure is reused for the relay UE to inform the remote UE about Uu failure of the relay UE as currently specified in 38.331.

- A remote UE in RRC_CONNECTED, upon reception of Uu RLF indication from the relay UE, suspends transmissions on the indirect path and informs the network if SRB1 is available on the direct path and not suspended, otherwise triggers re-establishment.

- For Scenario-1/2, no specification effort to handle the case when the relay UE moves to RRC_IDLE following expiry of dataInactivityTimer, i.e., not pursue relay UE notifying remote UE, and remote UE notifying network.

##### Procedure for review

The procedure based on the related agreements is shown below.

#### 1.6 Direct path failure detection and recovery without RRC re-establishment Figure 1.6: Signalling procedure of direct path failure detection and recovery without RRC re-establishment - The remote UE performs data transmission and reception by using both the direct path on PCell and the indirect path served by a relay UE. 1a. The remote UE performs Uu RLM on the direct path and SL RLM on the indirect path and may detect SL RLF on the indirect path as specified in 38.331. 1b. The relay UE may initiate the Notification Message procedure and sends the NotificationMessageSidelink message to the remote UE as specified in 38.331. 2. Upon SL RLF detection on the indirect path or reception of the NotificationMessageSidelink message from the relay UE, if SRB1 has been configured on the direct path and if the direct path has been not suspended, the remote UE suspends the indirect path and reports indirect path failure to the gNB via the direct path. Otherwise, the remote UE initiates the RRC connection re-establishment procedure as specified in 38.331. FFS: Which message is used to report indirect path failure. - Upon receiving the report of the indirect path failure, the gNB may send a RRCReconfiguration message to the remote UE in order to change the indirect path (as described in 1.4) or release the indirect path (as described in 1.8) for the remote UE.

Companies are requested to check the procedure and provide comments and remaining open issues on this procedure, if any.

| Company | Comments | Remaining open issues |
| --- | --- | --- |
| OPPO | Considering legacy spec does not capture this failure handling procedure using signaling flow + step-based description in stage-2, we slightly prefer avoid such description in 300. If it is to be captured, we understand the following part is still pending (we assume there is no case where SRB1 is not configured on the direct path) 2. Upon SL RLF detection on the indirect path or reception of the NotificationMessageSidelink message from the relay UE, if SRB1 has been configured on the direct path and if the direct path has been not suspended, the remote UE suspends the indirect path and reports indirect path failure to the gNB via the direct path. Otherwise, the remote UE initiates the RRC connection re-establishment procedure as specified in 38.331. |  |
| ZTE | For step 2, it mentions that the remote UE reports the indirect path failure to the gNB via diret path. However, the figures shows that indirect path is used since the message pass through the relay UE. 2. Upon SL RLF detection on the indirect path or reception of the NotificationMessageSidelink message from the relay UE, if SRB1 has been configured on the direct path and if the direct path has been not suspended, the remote UE suspends the indirect path and reports indirect path failure to the gNB via the direct path. Otherwise, the remote UE initiates the RRC connection re-establishment procedure as specified in 38.331. |  |
| vivo | - Step 1a: There is no “SL RLM” concept for PC5. Suggest to rewording as “SL RLM RLF detection on the indirect path” also see comments to Figure 1.5 step 1. - Step 2: - In Step 2, the indirect path failure report should be transmitted via direct path instead of indirect path. - This “…. UE initiates the RRC connection re-establishment…” seems inconsistent with the title and figure “Direct path failure detection and recovery without RRC re-establishment”. |  |
| CATT | 1. For step 2, the circle on the line should be deleted since the message is sent on the direct path. 2. For DL/UL data, refer to 38.300, dash line should be used. |  |
| Apple | Step 1b: The description is quite confusing. We shall clarify that the message indicates Uu hop failure for the indirect path. Step 2: The depiction and the description do not match. The case of “initiating RRC reestablishment procedure” is not depicted. |  |
| Fujitsu | We think that step 3 is up to gNB implementation and can be removed. |  |
| Nokia | For step 1, we can simplify the description by saying that ‘the remote UE detects an RLF on the indirect path as specified in 38.331’ because Uu RLF detection has no impact on this procedure. |  |
| NEC | This section is not needed for stage 2 level. |  |
| InterDigital | Same comments as vivo and Apple. |  |
| Huawei, HiSilicon | Similar comments to the question in 1.5. |  |
| Ericsson | Same concerns as in Q1.5 |  |
| Fraunhofer | Already commented by others: should be “indirect path failure” in title and figure caption. |  |
| LG Electronics | Step 1a SL RLM can be removed Step 2 SRB1 on the direct path can be removed considering that SRB1 is always configured on the direct path. |  |

##### Summary and Updated Procedure

The updated procedure based on the above comments is shown below. Note that any change to the figure is shown in red.

#### 1.6 Indirect path failure detection and recovery Figure 1.6: Signalling procedure of direct path failure detection and recovery without RRC re-establishment - The remote UE performs data transmission and reception by using both the direct path on PCell and the indirect path served by a relay UE. 1a. The remote UE may detect SL RLF on the indirect path as specified in 38.331. 1b. The relay UE may initiate the Notification Message procedure and sends the NotificationMessageSidelink message to the remote UE as specified in 38.331. 2. Upon SL RLF detection on the indirect path or reception of the NotificationMessageSidelink message from the relay UE, if the direct path has been not suspended, the remote UE suspends the indirect path and reports indirect path failure to the gNB via the direct path. Otherwise, the remote UE initiates the RRC connection re-establishment procedure as specified in 38.331. FFS: Which message is used to report indirect path failure. - Upon receiving the report of the indirect path failure, the gNB may send a RRCReconfiguration message to the remote UE in order to change the indirect path (as described in 1.4) or release the indirect path (as described in 1.8) for the remote UE.

### 1.7 Direct path release in Scenario 1

##### Related agreements

RAN2 agreements related with this procedure are listed below:

##### Procedure for review

There is no agreement directly related to this procedure. However, rapporteur thinks that the procedure can be easily drawn based on the RRC reconfiguration procedure agreed for path addition, as shown below.

#### 1.7 Direct path release Figure 1.7: Signalling procedure of direct path release 0. The remote UE performs data transmission and reception by using both the direct path on PCell and the indirect path served by a relay UE. 1. The remote UE and the relay UE perform measurements based on measurement configuration. The remote UE may report measurement results e.g. for Event A2 (Serving becomes worse than threshold). 2. The gNB decides to release the direct path to remote UE. 3. The gNB sends an RRCReconfiguration message to the relay UE to update the indirect path configuration, if necessary. 4. The gNB sends the RRCReconfiguration message to the remote UE. The contents in the RRCReconfiguration message may include at least direct path release configuration. It is up to gNB implementation which path is used to send the RRCReconfiguration message to the remote UE. 5. The remote UE sends the RRCReconfigurationComplete message to the gNB via the indirect path in order to complete the direct path release procedure.

Companies are requested to check the procedure and provide comments and remaining open issues on this procedure, if any.

| Company | Comments | Remaining open issues |
| --- | --- | --- |
| OPPO | In step-4, for the following part, the detailed configuration would be up to stage-3, e.g., there may be no explicit “release” configuration, but just in a form of reconfiguration-with-sync from pcell at direct-path to pcell at indirect-path. 4. The gNB sends the RRCReconfiguration message to the remote UE. The contents in the RRCReconfiguration message may include at least direct path release configuration. It is up to gNB implementation which path is used to send the RRCReconfiguration message to the remote UE. |  |
| Xiaomi | Step 4, we agree it’s up to gNB to decide which path is used. However, in the fig, only direct path is showed. Maybe additional dash line can be added to reflect indirect path, similar as Figure 1.3. |  |
| vivo | - Step 1: suggest not to mention a specific measurement Event, which is up to gNB configuration. Instead a general sentence could be used here, e.g., “The measurement results from L2 U2N Remote UE are reported when configured measurement reporting criteria are met” as specified in current subclause 16.12.6.1 of TS 38.300. - Step 4: On “…It is up to gNB implementation. …” We do not think it’s gNB implementation. It depends on RAN2 conclusion on whether SRB1 configured on the indirect path is supported. If RAN2 does not support SRB1 configured on the indirect path, then the RRCReconfiguration message would always be sent on the direct path. |  |
| CATT | For DL/UL data, refer to 38.300, dash line should be used. |  |
| Apple | 2. Step 4. Same view as Vivo about which path is to be used.. Also, the content of this message can be just a single path configuration so ther eis n |  |
| Fujitsu | Agree with OPPO. In step 4, reconfiguration-with-sync is needed, since the PCell of the remote UE should be changed to the PCell of the relay UE after direct path release. | Mode 2 configuration may need to be included in RRC Reconfiguration message in step 4 if the remote UE is configured with Mode 1 before the direct path release. |
| Nokia | For step 4, it would be up to NW where to send the reconfiguration message if split SRB1 is configured. |  |
| NEC | Agree with OPPO for Step4. |  |
| InterDigital | On step4, we agree it is NW implementation are fine to leave it this way. Regardless of the outcome of SRB1 on indirect, SRB1 can be configured as split and the message can be sent via indirect. So in any case, this is NW implementation. |  |
| Samsung | Step 1: We don’t need indicate the measurement event in stage-2 procedure. In addition, this step only needs mention the measurement behaviour of remote UE since this is a procedure for remote UE. In Rel-17, for direct to indirect path switch, we don’t include relay UE measurement procedure either. Step 4: “it is up to gNB …”, this depends on whether split SRB1 is configured or not, which is not completely gNB implementation issue. In case SRB1 is configured to direct path, only direct path can be used; in case split SRB1 is configured, it is gNB implementation issue |  |
| Huawei, HiSilicon | In general, we understand this is a MP to indirect path HO, which can reuse D2I path switch procedure and signalling. Step 4 4. The gNB sends the RRCReconfiguration message to the remote UE containing the information to access the target relay in target side. The contents in the RRCReconfiguration message may include at least direct path release configuration. It is up to gNB implementation which path is used to send the RRCReconfiguration message to the remote UE. |  |
| Ericsson | Similar concerns for Step-4 as listed above |  |
| LG Electronics | Step 1 The specific event can be removed as commented above. Step 4 The RRCReconfiguration message can be sent to the remote UE on one of both or both paths, at least when split SRB1 is configured with/without duplication. Some comments on the contents in the message seem Stage 3 details. Considering that we are focusing on Stage 2 level procedure in this email discussion, details about the contents can be removed here and go to Stage 3. |  |

##### Summary and Updated Procedure

The updated procedure based on the above comments is shown below. Note that any change to the figure is shown in red.

#### 1.7 Direct path release Figure 1.7: Signalling procedure of direct path release 0. The remote UE performs data transmission and reception by using both the direct path on PCell and the indirect path served by a relay UE. 1. The remote UE and the relay UE perform measurements based on measurement configuration. The remote UE may report measurement results. 2. The gNB decides to release the direct path to remote UE. 3. The gNB sends an RRCReconfiguration message to the relay UE to update the indirect path configuration, if necessary. 4. The gNB sends the RRCReconfiguration message to the remote UE on the direct path and/or the indirect path in order to release the direct path. FFS: Stage 3 details about the contents in the RRCReconfiguration message. 5. The remote UE sends the RRCReconfigurationComplete message to the gNB via the indirect path in order to complete the direct path release procedure.

### 1.8 Indirect path release in Scenario 1

##### Related agreements

RAN2 agreements related with this procedure are listed below:

- For a remote UE and relay UE in RRC_CONNECTED, the network is expected to release the multipath configuration related to this relay at the remote UE before it releases the relay UE to RRC_IDLE/RRC_INACTIVE.

- For Scenario-1/2, not pursue remote UE notifying network upon reception of notification message indicating relay UE handover. FFS whether rely on network to release configuration of relay UE at remote UE before relay UE handover, or rely on remote UE to suspend the indirect path upon reception of notification message indicating relay UE handover.

- For Scenario-1/2, no specification effort to handle the case when the relay UE moves to RRC_IDLE following expiry of dataInactivityTimer, i.e., not pursue relay UE notifying remote UE, and remote UE notifying network.

##### Procedure for review

The procedure based on the related agreements is shown below.

#### 1.8 Indirect path release Figure 1.8: Signalling procedure of indirect path release 0. The remote UE performs data transmission and reception by using both the direct path on PCell and the indirect path served by a relay UE. 1. The remote UE and the relay UE perform measurements based on measurement configuration. The remote UE may report measurement results e.g. for Event X2 (Serving L2 U2N Relay UE becomes worse than threshold) or the relay UE may report measurement results e.g. for inter-gNB handover. 2. The gNB decides to release the indirect path of remote UE e.g. when the relay UE performs inter-gNB handover or leaves RRC_CONNECTED. 3. The gNB sends an RRCReconfiguration message to the relay UE to release the indirect path configuration, if necessary. 4. The gNB sends the RRCReconfiguration message to the remote UE. The contents in the RRCReconfiguration message may include at least indirect path release configuration. It is up to gNB implementation which path is used to send the RRCReconfiguration message to the remote UE. 6. The remote UE sends the RRCReconfigurationComplete message to the gNB via the direct path in order to complete the indirect path release procedure. It is up to UE implementation whether/when PC5 unicast link between the remote UE and the relay UE is released after the indirect path release.

Companies are requested to check the procedure and provide comments and remaining open issues on this procedure, if any.

| Company | Comments | Remaining open issues |
| --- | --- | --- |
| OPPO | The order of step-3 can be FFS. For step-6, considering relay/non-relay link would be served by separate L2 ID, there seems no such ambiguity of “whether”. For “when”, there seems no ambiguity either since AS-layer should indicate to upper layer for link release upon reception of reconfiguration message? 6. The remote UE sends the RRCReconfigurationComplete message to the gNB via the direct path in order to complete the indirect path release procedure. It is up to UE implementation whether/when PC5 unicast link between the remote UE and the relay UE is released after the indirect path release. |  |
| Xiaomi | Step 4, we agree it’s up to gNB to decide which path is used. However, in the fig, only direct path is showed. Maybe additional dash line can be added to reflect indirect path, similar as Figure 1.3. On step 6, we see some benefit to keep connection with relay to continue receiving the buffered data at relay UE, similar as R17. One typo, step 6 should be 5. |  |
| ZTE | 1.For step 6, we think that the remote UE should release the PC5 unicast link with relay UE since the indirect path has been released. 2.After the step 4 and 5, the DL/UL data should use direct path for data delivery between remote UE and gNB. However, the figure shows indirect path is used. |  |
| vivo | - Step 1: - Similar comments as above. “The remote UE may report measurement results for serving relay UE and one or multiple candidate relay UEs.” - Similar comments as above, there should be no restriction about what measurement results are to be used by the gNB, suggest to remove “e.g. for Event X2 (Serving L2 U2N Relay UE becomes worse than threshold) …” - Step 4: Similar to Step 4 of Figure 1.7 |  |
| CATT | 1. For DL/UL data, refer to 38.300, dash line should be used. 2. Step 3 should be after step 5 since step 4/5 may be sent on the indirect path. 3. After step 5, the DL/UL data should be carried via direct path. |  |
| Apple | Step 4. Similar comments as 1.7 | .Even if the UE is configured to have a Multi-path, the UE may feel that using a single direct path is sufficient and more efficient. So,. A MP-capable UE can explicitly request to release indirect path in this case. RAN2 need discuss this as an open issue. |
| Fujitsu | Agree with rapporteur on step 6 that it is up to UE implementation whether/when PC5 unicast link between the remote UE and the relay UE is released after the indirect path release, which is similar as Rel-17. |  |
| Nokia | For step 4, whether to send RRCReconfiguration message only via the direct path depends on the order of step3 and step 4. If step 3 is prior to step 4 as shown in the Figure 1.8, the gNB will send the reconfiguration message only via the direct path because the indirect path is released already in step 3 from the relay perspective. However, if the order of step3 and step4 are in the other way round, the gNB may send the reconfiguration message via the indirect path. After step 5, the DL/UL data should be via the direct path instead of indirect path. | RAN2 need to discuss whether the reconfiguration of the relay UE is always prior to reconfiguration of the remote UE in case of indirect path release. |
| NEC | After Step5 ,for data forwarding: - Add a line for DL/UL via direct path. - How to handle date via indirect path need FFS. |  |
| InterDigital | Fine with the procedure, taking into account our comments from previous question. |  |
| Samsung | Step 1: There is no need to give the measurement event for stage-2 procedure. In addition, this step only needs mention the measurement behaviour of remote UE since this is a procedure for remote UE. In Rel-17, for direct to indirect path switch, we don’t include relay UE measurement procedure either. Step 2: There is no need for “e.g., …” part since it is just one example. Step 3: “if necessary” is not needed since indirect path is released. Step 4: “it is up to gNB …”, this depends on whether split SRB1 is configured or not, which is not completely gNB implementation issue. In case SRB1 is configured to direct path, only direct path can be used; in case split SRB1 is configured, it is gNB implementation issue |  |
| LG Electronics | Step 1 The specific event can be removed as commented above. Step 2 The example can be removed. Step 3 FFS on the order of step 3 and 4 can be added Step 4 The RRCReconfiguration message can be sent to the remote UE on one of both or both paths, at least when split SRB1 is configured with/without duplication. Some comments on the contents in the message seem Stage 3 details. Considering that we are focusing on Stage 2 level procedure in this email discussion, details about the contents can be removed here and go to Stage 3. Step 6 The step need to change to step 5. FFS can be added to capture the issues on buffered data delivery and when PC5 unicast link is released. |  |

##### Summary and Updated Procedure

The updated procedure based on the above comments is shown below. Note that any change to the figure is shown in red.

#### 1.8 Indirect path release Figure 1.8: Signalling procedure of indirect path release 0. The remote UE performs data transmission and reception by using both the direct path on PCell and the indirect path served by a relay UE. 1. The remote UE and the relay UE perform measurements based on measurement configuration. The remote UE may report measurement results. FFS: Whether the remote UE having multi-path can explicitly request release of the indirect path to gNB. 2. The gNB decides to release the indirect path of remote UE. 3. The gNB sends an RRCReconfiguration message to the relay UE to release the indirect path configuration. FFS: When the RRCReconfiguration message is sent to the relay UE. Whether the order of the step 3 and 4 is up to network implementation 4. The gNB sends the RRCReconfiguration message to the remote UE on the direct path and/or the indirect path in order to release the indirect path. FFS: Stage 3 details about the contents in the RRCReconfiguration message. 5. The remote UE sends the RRCReconfigurationComplete message to the gNB via the direct path in order to complete the indirect path release procedure. 6. PC5 unicast link between the remote UE and the relay UE is released after the indirect path release. FFS: When PC5-RRC connection between the remote UE and the relay UE is released e.g. it is immediately released after the indirect path release or it can be maintained before buffered data on the indirect path is delivered.

# Scenario 2

RAN2 agreed to support the following cases for scenario 2

- Indirect path addition (Case A): The remote UE operating only on the direct path adds the indirect path under the same gNB;

- Indirect path release (Case C): The remote UE operating in multi-path releases the indirect path;

- Indirect path change (Case G): The remote UE operating in multi-path changes to a new relay UE for the indirect path while keeping the direct path under the same gNB.

RAN2 additionally agreed the followings for Scenario 2:

- Case B and case D are not supported for Scenario 2.

- For Scenario 2, Case E is not supported.

- For Scenario 2, whether to support Case G is discussed in normative phase, but RAN2 will not do additional work to enable it for Scenario 2 over Scenario 1.

- Non-split SRB1 and 2 over indirect path is not supported in Scenario 2.

- Split SRB1 and 2 are supported in Scenario 2 and primary path of the split SRB 1 and 2 is always on direct path.

### 2.1 Indirect path addition in Scenario 2

##### Related agreements

RAN2 agreements related with this procedure are listed below:

- Working assumption: For multi-path Relay Scenario-2, leave it to relay and remote UE implementation on how to trigger the RRC_IDLE/RRC_INACTIVE target relay UE to initiate RRC connection establishment procedure. RAN2 further discuss the solution for Scenario-1.

- Multi-path Relay is NOT applicable to RRC Setup procedure, for scenario-1 and scenario-2.

- Multi-path Relay is NOT applicable to RRC Resume procedure, for scenario-1 and scenario-2.

- Remote UE storing indirect path configuration or not and use it to resume to MP configuration in scenario 2 is not supported.

- As agreed before, RAN2 deprioritizes association mechanism between remote UE and relay UE from CN to RAN.

- When split SRB1 with duplication is configured, the remote UE sends the RRCReconfigurationComplete message to gNB via both paths for Scenario 2.

- When one of the following conditions is met, the remote UE sends the RRCReconfigurationComplete message to gNB via the direct path for Scenario 2.

  - when primary RLC entity of split SRB1 is on direct path

  - when non-split SRB1 is configured on direct path

- In scenario 2, if both remote and relay UE are in RRC_CONNECTED, the remote UE reports relay UE’s ID to gNB for indirect path addition.  Need for reporting in the idle/inactive case can be further discussed.  FFS what ID is used.

- WA: For scenario 2, remote-UE reports the RRC_CONNECTED relay-UE C-RNTI and serving cell ID (e.g., NCGI) for indirect path addition.

RAN3 agreed to add Inter-DU Direct Path Addition and Inter-DU Indirect Path Addition procedures in TS 38.401 (in R3-230947). In addition, RAN3 agreed to add the intra-DU path addition procedure in the BL CR (R3-232088) to TS 38.401.

#### 8.xx.4 Intra-DU indirect path addition on top of direct path The signaling flow for intra-DU indirect path addition is shown in Fig. 8.xx.4-1. Figure 8.xx.4-1: Signalling procedure of intra-DU indirect path addition on top of direct path 1. The Uu measurement configuration and measurement report signalling is performed between remote UE and gNB-CU to evaluate relay link measurement and Uu link measurement. The remote UE may report Uu measurement results of neighboring cells and one or multiple candidate relay UEs. 2. The gNB-CU decides to add the indirect path via relay UE to remote UE under the same gNB-DU. 3. The reconfiguration to relay UE is performed among relay UE, gNB-DU and gNB-CU if relay UE is in RRC_CONNECTED state. The gNB-CU sends an RRCReconfiguration message to the relay UE. If the relay UE is in RRC_IDLE/INACTIVE state, this step is skipped. 4. The gNB-CU sends the UE CONTEXT MODIFICATION REQUEST message for the remote UE to the gNB-DU, which contains the indirect path configuration at least. 5. The gNB-DU responds to the gNB-CU with a UE CONTEXT MODIFICATION RESPONSE message. 6. gNB-CU sends the DL RRC MESSAGE TRANSFER message for remote UE by including the RRCReconfiguration message to gNB-DU. The contents in the RRCReconfiguration message may include at least indirect path addition configuration, PC5 RLC channel configuration for relay traffic, bearer mapping and the associated radio bearer(s). 7. gNB-DU sends the RRCReconfiguration message to the remote UE. 8. The remote UE establishes PC5 connection with relay UE. 9. The remote UE may complete the indirect path addition procedure by sending the RRCReconfigurationComplete message to the gNB-DU. Editor’s Note: FFS whether the remote UE sends the RRCReconfigurationComplete message to the gNB-CU via old path (direct path) or new path (indirect path). Editor’s Note: FFS: In case the relay UE is in RRC_IDLE/ INACTIVE state, how to trigger the relay UE enter RRC_CONNECTED state, RAN3 wait for RAN2’s further progress. 10. The gNB-DU sends the UL RRC MESSAGE TRANSFER message to gNB-CU by including the RRCReconfigurationComplete message.

##### Procedure for review

The procedure based on the related agreements is shown below.

#### 2.1 Indirect path addition on top of direct path Figure 2.1: Signalling procedure of indirect path addition on top of direct path 0. The remote UE establishes a RRC connection with the gNB and transmits and receives data on the direct path. 1. The remote UE is associated with a relay UE. The remote UE may report the relay UE information on the relay UE to the gNB via the direct path. [The relay UE information includes at least C-RNTI of the relay UE and cell ID (e.g., NCGI) serving the relay UE]. The association and UE-to-UE link between the remote UE and the relay UE is not in the scope of 3GPP specifications. Editor’s note: The step in the brackets need to be confirmed by SA3. FFS: Which message is used to report the relay UE information for scenario 2. 2. Upon receiving the relay UE information, if the cell serving the relay UE and the cell serving the direct path of the remote UE belong to the same gNB, the gNB may decide to add the indirect path to remote UE. The cell serving the direct path and the cell serving the relay UE on the indirect path can be same or different. 3. The gNB sends an RRCReconfiguration message to the relay UE to configure the indirect path of the remote UE. 4. The gNB sends the RRCReconfiguration message to the remote UE. The contents in the RRCReconfiguration message may include at least the target relay UE, indirect path addition configuration, bearer mapping and the associated radio bearer(s). 5a. The remote UE sends the RRCReconfigurationComplete message to the gNB at least via the direct path in order to complete the indirect path addition procedure. 5b. If a split SRB1 with duplication is configured, the remote UE also sends the RRCReconfigurationComplete message to the gNB via the indirect path served by the relay UE.

Companies are requested to check the procedure and provide comments and remaining open issues on this procedure, if any.

| Company | Comments | Remaining open issues |
| --- | --- | --- |
| CATT | 1. For step 3, gNB should configure the relay UE after it enters RRC_CONNECTED state if the relay UE is in RRC_IDLE/INACTIVE. Therefore, the RRCReconfiguration message to the relay UE may be happened after step 4/5a. 2. After step 5, suggest adding one dashed box for RRC connection establishment/resume since the non-connected relay UE is also supported for MP2. 3. For DL/UL data, refer to 38.300, dash line should be used. |  |
| Apple | 1. Since there is no measurement reported, the gNB decision in Step 2 to add indirect path seems just based on the report from remote UE in Step 1. Then, we think Step 1 is more of a “MP-request” nature and the remote UE shall clearly indicate whether it wants to have MP configuration or not in this step, instead of just merely report C-RNTI of relay UE, which seems quite passive. 2. On the other hand, the triggering condition of Step 1 needs to be more clearly manifested in the remote UE side. Why and when remote UE sends this message? 2. We do not agree with CATT comment that relay UE enters CONNECTED after Step 5, This has to happen before step 2. | Before Step 1, the relay UE shall be already in CONNECTED state. So, we suggest to discuss whether there should be one more arrow from remote UE to relay UE to trigger it enter CONNECTED state. |
| CMCC | We have some concern about the indirect path addition trigger event for scenario 2. remote UE report its associated relay UE identification as in step1 as preparation for MP, and then the network or remote UE can trigger the indirect path addition. The details about the trigger event are still open. | The details about the trigger event are still open. |
| Samsung | Step 1: We didn’t reach conclusion on the whether the remote UE can have multiple candidate relay UEs. Suggest to add EN: FFS on whether the remote UE can have multiple candidate relay UEs |  |
| Huawei, HiSlicon | Step 1 As commented by others, although the content of relay UE info has been agreed, how to trigger remote UE to report such information needs further discussion. |  |
| Ericsson | Step 1: Agree with the comments above. Step 4: Not sure what “at least the target relay UE” means. Not sure if this is required and can be removed. General question, why would it be a serving relay UE? |  |
| Fraunhofer | Agree with other companies regarding Step-1 The trigger to add the indirect path should be more clear. |  |
| LG Electronics | Step 1 - Clarify that the relay UE is in RRC_CONNECTED for reporting of relay UE ID. - FFS on why/when the remote UE sends this message can be added - FFS on whether/how the network or remote UE can trigger the indirect path addition can be added. |  |

##### Summary and Updated Procedure

The updated procedure based on the above comments is shown below. Note that any change to the figure is shown in red.

#### 2.1 Indirect path addition on top of direct path Figure 2.1: Signalling procedure of indirect path addition on top of direct path 0. The remote UE establishes a RRC connection with the gNB and transmits and receives data on the direct path. 1. The remote UE is associated with a relay UE. While the relay UE is in RRC_CONNECTED, the remote UE may report the relay UE information on the relay UE to the gNB via the direct path. [The relay UE information includes at least C-RNTI of the relay UE and cell ID (e.g., NCGI) serving the relay UE]. The association and UE-to-UE link between the remote UE and the relay UE is not in the scope of 3GPP specifications. Editor’s note: The step in the brackets need to be confirmed by SA3. FFS: Which message is used to report the relay UE information for scenario 2. FFS: Why/when the remote UE sends this message e.g. only for reporting the relay UE’s ID or for the remote UE to request/trigger the indirect path addition. FFS: Whether/how the network or remote UE can trigger the indirect path addition e.g. by reporting of the relay UE’s ID or a separate signaling. 2. Upon receiving the relay UE information, if the cell serving the relay UE and the cell serving the direct path of the remote UE belong to the same gNB, the gNB may decide to add the indirect path to remote UE. The cell serving the direct path and the cell serving the relay UE on the indirect path can be same or different. 3. The gNB sends an RRCReconfiguration message to the relay UE to configure the indirect path of the remote UE. 4. The gNB sends the RRCReconfiguration message to the remote UE. The contents in the RRCReconfiguration message may include at least the target relay UE, indirect path addition configuration, bearer mapping and the associated radio bearer(s). 5a. The remote UE sends the RRCReconfigurationComplete message to the gNB at least via the direct path in order to complete the indirect path addition procedure. 5b. If a split SRB1 with duplication is configured, the remote UE also sends the RRCReconfigurationComplete message to the gNB via the indirect path served by the relay UE.

### 2.2 Direct path change in Scenario 2

##### Related agreements

RAN2 agreements related with this procedure are listed below:

- Support PCell on the direct path only when the UE is in multi-path operation, for both scenario 1 and scenario 2.

- Change of direct path while keeping the indirect path can be done with a release-and-add in a single RRC message.  This does not exclude a gNB implementation from using separate release and add procedures instead.

##### Procedure for review

The procedure based on the related agreements is shown below.

#### 2.2 Direct path change Figure 2.2: Signalling procedure of direct path change under a single procedure - The remote UE performs data transmission and reception by using both the direct path on PCell and the indirect path served by a relay UE. 1. The remote UE and the relay UE perform measurements based on measurement configuration. The remote UE may report measurement results e.g. for Event A3 (Neighbour becomes offset better than SpCell). 2. The gNB decides to change the direct path of the remote UE from the PCell (i.e. source PCell) to a new cell (i.e. target PCell). The source/target PCell serving the old/new direct path and the cell serving the relay UE on the indirect path belong to the same gNB but can be same or different. 3. The gNB sends an RRCReconfiguration message to the relay UE to update the indirect path configuration, if necessary. 4. The gNB sends the RRCReconfiguration message to the remote UE via the relay UE. The contents in the RRCReconfiguration message may include at least a target PCell, direct path release and addition configuration, bearer mapping and the associated radio bearer(s). 5. The remote UE synchronizes to DL of the target PCell serving the new direct path and performs random access procedure towards the target PCell serving the new direct path. 6a. The remote UE sends the RRCReconfigurationComplete message to the gNB at least via the direct path in order to complete the direct path change procedure. 6b. If a split SRB1 with duplication is configured, the remote UE also sends the RRCReconfigurationComplete message to the gNB via the indirect path served by the target relay UE.

Companies are requested to check the procedure and provide comments and remaining open issues on this procedure, if any.

| Company | Comments | Remaining open issues |
| --- | --- | --- |
| OPPO | Given R2 conclusion that - For Scenario 2, Case E is not supported. There is no need to discuss this case? Sorry if any misunderstanding. |  |
| Xiaomi | Similar view as OPPO. |  |
| ZTE | We share the same understanding as oppo, i.e. case E is not supported for scenario 2. |  |
| vivo | The same comments as for 1.3 | The same open issue as for 1.3 |
| CATT | Case E is not supported for scenario 2. |  |
| Apple | This is not supported in R18 |  |
| Fujitsu | Agree with OPPO. |  |
| Nokia | We have the same understanding as commented by other companies that Case E is not supported for scenario 2. |  |
| CMCC | Agree with OPPO. |  |
| NEC | Agree with OPPO. | NEC |
| InterDigital | Agree with OPPO |  |
| Samsung | Agree with OPPO |  |
| Ericsson | Similar understanding as others, Case E is not supported |  |
| LG Electronics | OK with a majority’s view |  |

##### Summary and Updated Procedure

Based on the above comments, we do not need to further discuss the direct path change for Scenario 2. No proposal is suggested for the direct path change.

### 2.3 Indirect path change in Scenario 2

##### Related agreements

RAN2 agreements related with this procedure are listed below:

- Working assumption: For multi-path Relay Scenario-2, leave it to relay and remote UE implementation on how to trigger the RRC_IDLE/RRC_INACTIVE target relay UE to initiate RRC connection establishment procedure. RAN2 further discuss the solution for Scenario-1.

- Change of indirect path while keeping the direct path can be done with a release-and-add in a single RRC message.  This does not exclude a gNB implementation from using separate release and add procedures instead.

##### Procedure for review

There is no agreement on whether to support indirect path change for Scenario 2. If supported, the procedure can be shown below based on the agreements indirectly related to this procedure.

#### Indirect path change Figure 2.3: Signalling procedure of indirect path change under a single procedure FFS: whether to support indirect path change for Scenario 2. 0. The remote UE transmits and receives data on the direct path and the indirect path served by a relay UE (i.e. source relay UE) associated with the remote UE. 1. The remote UE is associated with a new relay UE (i.e. target relay UE). The remote UE may report the relay UE information on the new relay UE to the gNB via the direct path. [The relay UE information includes at least C-RNTI of the new relay UE and cell ID (e.g., NCGI) serving the new relay UE]. The association and UE-to-UE link between the remote UE and the relay UE is not in the scope of 3GPP specifications. Editor’s note: The step in the brackets need to be confirmed by SA3. FFS: Which message is used to report the relay UE information for scenario 2. 2. The gNB decides to change the indirect path of remote UE from the source relay UE to the target relay UE. The cell serving the direct path and the cell serving the source/target relay UE on the indirect path belong to the same gNB but can be same or different. 3. The gNB sends an RRCReconfiguration message to the source relay UE to release the indirect path of the remote UE. The gNB sends an RRCReconfiguration message to the target relay UE to add the indirect path of the remote UE. FFS: When the RRCReconfiguration messages are sent to the source/target relay UEs. - The gNB sends the RRCReconfiguration message to the remote UE. The contents in the RRCReconfiguration message may include at least the target relay UE, indirect path release and addition configuration, bearer mapping and the associated radio bearer(s). 5a. The remote UE sends the RRCReconfigurationComplete message to the gNB at least via the direct path in order to complete the indirect path change procedure. 5b. If a split SRB1 with duplication is configured, the remote UE also sends the RRCReconfigurationComplete message to the gNB via the indirect path served by the target relay UE.

Companies are requested to check the procedure and provide comments and remaining open issues on this procedure, if any. Note that RAN2 has no agreement on whether to support indirect path change for Scenario 2.

| Company | Comments | Remaining open issues |
| --- | --- | --- |
| OPPO | There seems no need to discuss how to implement Sce-G since it is still pending whether to support this scenario in this release. |  |
| Xiaomi | Similar view as OPPO. |  |
| Vivo | This issue should be down-prioritized as it is not sure whether to support this or not. |  |
| CATT | We share the same view as OPPO. |  |
| Apple | We support to remove Case G in R18. |  |
| Fujitsu | Agree with OPPO. |  |
| Nokia | We also have the same understanding that support of case G for scenario 2 is still FFS (as indicated by the rapporteur). Meanwhile, we don’t think RAN2 has enough time to discuss and decide on FFS parts to support case G for scenario 2. Thus, we would like to conclude in the next meeting that case G is not supported for scenario 2 in this release. |  |
| CMCC | We support to include the case G in R18. |  |
| NEC | Agree with OPPO. |  |
| InterDigital | Agree with OPPO |  |
| Samsung | Agree with OPPO |  |
| Huawei, HiSilicon | We understand the stage 2 procedure of case G is quite similar in scenario1 and scenario 2. We can extend the procedure agreed for scenario 1 to scenario 2 if case G is agreed for scenario 2. |  |
| Ericsson | Agree with others, need not be discussed now |  |
| Fraunhofer | Agree with others |  |
| LG Electronics | OK with a majority’s view |  |

##### Summary and Updated Procedure

Based on the above comments, no proposal is suggested for the indirect path change.

### 2.4 Direct path failure in Scenario 2

##### Related agreements

RAN2 agreements related with this procedure are listed below:

- RAN2 further study how for UE operating in multi-path Relay operate for RRC Re-establishment procedure.

- Upon detection of 3GPP-defined RLF failure in one path, remote UE (configured with MP) can report path failure via the alternative available path if SRB1 is configured on the alternative path or split SRB1 is configured.

- In case of Uu-RLF, at least for split SRB1, if SRB1 is available on indirect path not suspended, trigger report to network via indirect path to report the failure via a RRC message. Otherwise, RRC Re-establishment is initiated. RAN2 is requested to discuss whether the RRC message is the existing message e.g. MCGFailureInformation or a new message.

- The remote UE initiates RRC re-establishment procedure (to a potentially new PCell as in Rel-17, unless further changes are agreed) when failure occurs on both paths (including either PC5 failure or notification of Uu failure on the indirect path).

- For Scenario-1/2, when reporting direct-path failure via indirect-path, use MCGFailureInformation message. FFS on whether additional IE needs to be introduced.

- For Scenario-1/2, if MCGFailureInformation is agreed for direct path failure recovery in P5, reuse T316 timer for the direct path failure recovery.

- WA: For scenario 1, primary path of the split SRB1 and SRB2 is always configured on direct path.  This does not preclude having the case where the UE switches the primary path to the indirect path for reporting after direct path failure.

##### Procedure for review

The procedure based on the related agreements is shown below.

#### 2.4 Direct path failure detection and recovery without RRC re-establishment Figure 2.4: Signalling procedure of direct path failure detection and recovery without RRC re-establishment - The remote UE performs data transmission and reception by using both the direct path on PCell and the indirect path served by a relay UE. - The remote UE performs Uu RLM on the direct path and UE-to-UE RLM on the indirect path and may detect RLF on the direct path as specified in 38.331. How the UE performs UE-to-UE RLM on the indirect path is up to UE implementation. 2. Upon RLF detection on the direct path, if SRB1 has been configured on the indirect path or split SRB1 has been configured, and if the indirect path has been not suspended, the remote UE suspends the direct path, starts timer T316 and sends the MCGFailureInformation message to the gNB via the indirect path as specified in 38.331. Otherwise, the remote UE initiates the RRC connection re-establishment procedure as specified in 38.331. 3. Upon receiving the MCGFailureInformation message, the gNB may send a RRCReconfiguration message to the remote UE in order to change the direct path (as described in 2.2) for the remote UE e.g. based on measurement results on the serving cell(s) and neighboring cells in the MCGFailureInformation message.

Companies are requested to check the procedure and provide comments and remaining open issues on this procedure, if any.

| Company | Comments | Remaining open issues |
| --- | --- | --- |
| OPPO | Considering legacy spec does not capture this failure handling procedure using signaling flow + step-based description in stage-2, we slightly prefer avoid such description in 300. If it is to be captured, we understand the following part is still pending 2. Upon RLF detection on the direct path, if SRB1 has been configured on the indirect path or split SRB1 has been configured, and if the indirect path has been not suspended, the remote UE suspends the direct path, starts timer T316 and sends the MCGFailureInformation message to the gNB via the indirect path as specified in 38.331. Otherwise, the remote UE initiates the RRC connection re-establishment procedure as specified in 38.331. And there are two step-2 in the description. |  |
| Xiaomi | It’s incomplete to only specify the T316 start and not say how T316 is stopped and the expiry result. We suggest to not specify anything about T316. Such details can be left to stage3. |  |
| ZTE | For Step 3, since the Case E is not supported, it is not feasible to change the direct path for the remote UE for this case. 3.Upon receiving the MCGFailureInformation message, the gNB may send a RRCReconfiguration message to the remote UE in order to change the direct path (as described in 2.2) for the remote UE e.g. based on measurement results on the serving cell(s) and neighboring cells in the MCGFailureInformation message. |  |
| vivo | Step 2: “RLM” is 3GPP terminology, we suggest to use more general wording as “UE-UE link failure detection”. |  |
| CATT | We share the same view as OPPO that failure handling procedure is not yet captured in stege-2 spec. Take one step back, the step order should be align with the figure. Furthermore, for step 2, non-split SRB1 can’t be configured on the indirect path for scenario 2. |  |
| Apple | Same view as CATT |  |
| Nokia | 1. For step 2, the highlighted part below needs to be discussed further. 2. Upon RLF detection on the direct path, if SRB1 has been configured on the indirect path or split SRB1 has been configured, and if the indirect path has been not suspended, the remote UE suspends the direct path, starts timer T316 and sends the MCGFailureInformation message to the gNB via the indirect path as specified in 38.331. Otherwise, the remote UE initiates the RRC connection re-establishment procedure as specified in 38.331. RAN2 agreed in #121bis meeting that the primary path of the split SRB 1 and 2 is always on direct path. Meanwhile, the WA made in #122 is only for scenario 1. If the UE is not allowed to switch the primary path to the indirect path for scenario 2, the highlighted part needs to be updated as : split SRB1 has been configured with duplication, and if the indirect path has not been suspended’, meaning that if split SRB is configured without duplication, the remote UE will initiate re-establishment even if the indirect path is not suspended. | In scenario 2, it needs to be discussed whether the remote UE switches the primary path of the split SRB1/2 from the direct path to the indirect path for reporting the direct path failure. |
| NEC | This section is not needed for stage 2 level. |  |
| InterDigital | Similar comments to the question in Scenario 1 |  |
| Samsung | Step 2: - This step indicates that the recovery of direct path is possible, which means that the gNB can add a new direct path while keeping the indirect path for scenario 2. Is it conflict with our agreement of not supporting Case E (direct path change)? -FFS should be added w.r.t. configuring SRB1 at indirect path Step 2&3: FFS should be added w.r.t. the message to report direct path failure |  |
| Huawei, HiSilicon | Similar comments to the question in 1.7. |  |
| Ericsson | Need not be captured. Moreover, it was already agreed that non-split SRB over indirect path is not supported in Scenario-2. This essentially boils down to that. |  |
| Fraunhofer | Share the view from CATT |  |
| LG Electronics | Step 1 UE to UE RLM can be removed Step 2 FFS on whether the WA on the primary path to the indirect path can be also applied to Scenario 2. FFS on SRB1 on the indirect path for reporting the failure can be added FFS on additional IE in the MCGFailureInformation message can be added. Step 3 Direct path change can be removed. Since direct path change is not supported in Scenario 2, we think that gNB should release indirect path (i.e. not keeping the indirect path) and perform PCell handover to a single direct path (i.e. no MP) by sending the RRC reconfiguration message. |  |

##### Summary and Updated Procedure

The updated procedure based on the above comments is shown below. Note that any change to the figure is shown in red.

#### 2.4 Direct path failure detection and recovery Figure 2.4: Signalling procedure of direct path failure detection and recovery without RRC re-establishment - The remote UE performs data transmission and reception by using both the direct path on PCell and the indirect path served by a relay UE. - The remote UE performs Uu RLM on the direct path and may detect RLF on the direct path as specified in 38.331. 2. Upon RLF detection on the direct path, if split SRB1 has been configured with duplication, and if the indirect path has been not suspended, the remote UE suspends the direct path and sends the MCGFailureInformation message to the gNB via the indirect path as specified in 38.331. Otherwise, the remote UE initiates the RRC connection re-establishment procedure as specified in 38.331. FFS: Whether the WA on switching the primary path to the indirect path is also applied to Scenario 2, especially if split SRB1 has been configured without duplication. FFS: whether SRB1 can be configured on the indirect path for reporting the failure. FFS: whether additional IE needs to be introduced in the MCGFailureInformation message for this case. 3. Upon receiving the MCGFailureInformation message, the gNB may send a RRCReconfiguration message to the remote UE.

### 2.5 Indirect path failure in Scenario 2

##### Related agreements

RAN2 agreements related with this procedure are listed below:

- For UE-UE link in Scenario-2, whether/how to have failure detection is out of 3GPP scope.

  - FFS whether there is impact to layers under our control from a failure of the UE-UE link in scenario 2.

- RAN2 further study how for UE operating in multi-path Relay operate for RRC Re-establishment procedure.

- Upon detection of 3GPP-defined RLF failure in one path, remote UE (configured with MP) can report path failure via the alternative available path if SRB1 is configured on the alternative path or split SRB1 is configured.

- The remote UE initiates RRC re-establishment procedure (to a potentially new PCell as in Rel-17, unless further changes are agreed) when failure occurs on both paths (including either PC5 failure or notification of Uu failure on the indirect path).

- In scenario 1, when a remote UE configured with multi-path initiates the RRC re-establishment procedure, the remote UE does not perform re-establishment directly into a multi-path configuration.

- If UE-UE link failure is detected on indirect path in Scenario 2, the remote UE can report UE-UE link failure to gNB over direct path.  Details of the reporting mechanism can be further discussed.

- A remote UE in RRC_CONNECTED, upon reception of Uu RLF indication from the relay UE, suspends transmissions on the indirect path and informs the network if SRB1 is available on the direct path and not suspended, otherwise triggers re-establishment.

- For Scenario-1/2, no specification effort to handle the case when the relay UE moves to RRC_IDLE following expiry of dataInactivityTimer, i.e., not pursue relay UE notifying remote UE, and remote UE notifying network.

##### Procedure for review

The procedure based on the related agreements is shown below.

#### Direct path failure detection and recovery without RRC re-establishment Figure 2.5: Signalling procedure of indirect path failure detection and recovery without RRC re-establishment - The remote UE performs data transmission and reception by using both the direct path on Pcell and the indirect path served by a relay UE. - The remote UE performs Uu RLM on the direct path and UE-to-UE RLM on the indirect path and may detect RLF on the direct path as specified in 38.331. The relay UE also performs Uu RLM and UE-to-UE RLM on the indirect path. How the UE performs UE-to-UE RLM on the indirect path is up to UE implementation. - Upon reception of Uu RLF indication from the relay UE or UE-to-UE link failure detection on the indirect path, if SRB1 has been configured on the direct path and if the direct path has been not suspended, the remote UE suspends the indirect path and reports indirect path failure to the gNB via the direct path. Otherwise, the remote UE initiates the RRC connection re-establishment procedure as specified in 38.331. Indication of Uu RLF indication from the relay UE to the remote UE is not in the scope of 3GPP specifications. FFS: Which message is used to report indirect path failure. - Upon receiving the report of the indirect path failure, the gNB may send a RRCReconfiguration message to the remote UE in order to change the indirect path (as described in 2.1) or release the indirect path (as described in 2.6) for the remote UE.

Companies are requested to check the procedure and provide comments and remaining open issues on this procedure, if any.

| Company | Comments | Remaining open issues |
| --- | --- | --- |
| OPPO | Considering legacy spec does not capture this failure handling procedure using 78ignalling flow + step-based description in stage-2, we slightly prefer avoid such description in 300. If it is to be captured, we understand the following part is still pending (we assume there is no case where SRB1 is not configured on the direct path) - Upon reception of Uu RLF indication from the relay UE or UE-to-UE link failure detection on the indirect path, if SRB1 has been configured on the direct path and if the direct path has been not suspended, the remote UE suspends the indirect path and reports indirect path failure to the gNB via the direct path. Otherwise, the remote UE initiates the RRC connection re-establishment procedure as specified in 38.331. Indication of Uu RLF indication from the relay UE to the remote UE is not in the scope of 3GPP specifications. |  |
| ZTE | For scenario 2, the following agreement has been reached. So the remote UE can anyway report the indirect path failure to gNB via direct path in step 3. “Non-split SRB1 and 2 over indirect path is not supported in Scenario 2. Split SRB1 and 2 are supported in Scenario 2 and primary path of the split SRB 1 and 2 is always on direct path.” |  |
| Vivo | - The similar comments as for 1.6 - Step 2: “RLM” is 3GPP terminology, we suggest to use more general wording as “UE-UE link failure detection”. |  |
| CATT | We share the same view as OPPO that failure handling procedure is not yet captured in stege-2 spec. Take one step back, the step order should be align with the figure. Furthermore, for step 3, SRB1 will always be configured on the direct path. “if SRB1 has been configured on the direct path and” should be deleted. |  |
| Apple | Step 2. The drawing is wrong. This shall occur in direct path. |  |
| NEC | This section is not needed for stage 2 level. |  |
| InterDigital | Similar comments to the question in Scenairo 1 |  |
| Samsung | Step 2: “and may detect RLF on the direct path as specified in 38.331” is not needed since this procedure is about indirect path failure. |  |
| Huawei, HiSilicon | Similar comments to the question in 1.6. |  |
| Ericsson | Need not capture this in Stage-2 |  |
| Fraunhofer | Share the view from CATT |  |
| LG Electronics | Step 2 RLM can be removed. We can focus on failure detection. Step 3 SRB1 on direct path can be deleted as others commented. Step 4 The indirect path change can be removed based on comments in section 2.3. |  |

##### Summary and Updated Procedure

The updated procedure based on the above comments is shown below. Note that any change to the figure is shown in red.

#### Indirect path failure detection and recovery Figure 2.5: Signalling procedure of indirect path failure detection and recovery without RRC re-establishment 1. The remote UE performs data transmission and reception by using both the direct path on Pcell and the indirect path served by a relay UE. 2. The remote UE detect UE-to-UE link failure on the indirect path, or the relay UE detects Uu RLF. 3. Upon reception of Uu RLF indication from the relay UE or UE-to-UE link failure detection on the indirect path, if the direct path has been not suspended, the remote UE suspends the indirect path and reports indirect path failure to the gNB via the direct path. Otherwise, the remote UE initiates the RRC connection re-establishment procedure as specified in 38.331. Indication of Uu RLF indication from the relay UE to the remote UE is not in the scope of 3GPP specifications. FFS: Which message is used to report indirect path failure. 4. Upon receiving the report of the indirect path failure, the gNB may send a RRCReconfiguration message to the remote UE in order to release the indirect path (as described in 2.6) for the remote UE.

### 2.6 Indirect path release in Scenario 2

##### Related agreements

RAN2 agreements related with this procedure are listed below:

- For a remote UE and relay UE in RRC_CONNECTED, the network is expected to release the multipath configuration related to this relay at the remote UE before it releases the relay UE to RRC_IDLE/RRC_INACTIVE.

- For Scenario-1/2, not pursue remote UE notifying network upon reception of notification message indicating relay UE handover. FFS whether rely on network to release configuration of relay UE at remote UE before relay UE handover, or rely on remote UE to suspend the indirect path upon reception of notification message indicating relay UE handover.

- For Scenario-1/2, no specification effort to handle the case when the relay UE moves to RRC_IDLE following expiry of dataInactivityTimer, i.e., not pursue relay UE notifying remote UE, and remote UE notifying network.

##### Procedure for review

The procedure based on the related agreements is shown below.

#### 2.6 Indirect path release Figure 2.6: Signalling procedure of indirect path release - The remote UE performs data transmission and reception by using both the direct path on PCell and the indirect path served by a relay UE. 1. The remote UE and the relay UE perform measurements based on measurement configuration. The relay UE may report measurement results e.g. for inter-gNB handover. 2. The gNB decides to release the indirect path of remote UE e.g. when the relay UE performs inter-gNB handover or leaves RRC_CONNECTED. 3. The gNB sends an RRCReconfiguration message to the relay UE to release the indirect path configuration, if necessary. 4. The gNB sends the RRCReconfiguration message to the remote UE. The contents in the RRCReconfiguration message may include at least indirect path release configuration. It is up to gNB implementation which path is used to send the RRCReconfiguration message to the remote UE. 5. The remote UE sends the RRCReconfigurationComplete message to the gNB via the direct path in order to complete the indirect path release procedure.

Companies are requested to check the procedure and provide comments and remaining open issues on this procedure, if any.

| Company | Comments | Remaining open issues |
| --- | --- | --- |
| OPPO | The order of step-3 can be FFS. |  |
| Xiaomi | Step 4, we agree it’s up to gNB to decide which path is used. However, in the fig, only direct path is showed. Maybe additional dash line can be added to reflect indirect path, similar as Figure 1.3. |  |
| ZTE | The last step of UL/DL data delivery in the figure should be via direct path instead of indirect path. |  |
| Vivo | - Step 1: there should be no restriction about what measurement results is to be used by the gNB in step 2. Suggest “The relay UE may report measurement results e.g. for inter-gNB handover in accordance with measurement report configuration.” |  |
| CATT | 1. Setp3 should be after step 5. 2. After step5, the DL/UL data should be carried via direct path. 3. For DL/UL data, refer to 38.300, dash line should be used. |  |
| Apple | In general, we think the case depicted in the diagram is a corner case. The remote UE and relay UE are very close to each other, and it does not make sense that relay UE has Uu failure but remote UE still maintain good-quality direct path. Anyway, if this case is to be considered, there are following comments: - Step 1. Why the measurement configuration also covers remote UE in this case. I think it is better to just draw a single arrow as relay UE reporting measurement to gNB to indicate the bad Uu RSRP. - Last arrow: shall be “direct path” | In general, the indirect path can be triggered from NW (due to relay UE mobility) or triggered by remote UE (due to no longer in need of using multiple paths). The latter case, which is more plausible, shall be considered. For example, remote UE can report that it does not need the indirect path, so that the gNB can confidently release the indirect path with the procedure 2.6. |
| Nokia | For step 4, it seems to make sense to send the reconfiguration message via the direct path because the relay UE may already release the indirect path related configuration in step 3. |  |
| NEC | Agree with OPPO for Step3 and Xiaomi for Step4. |  |
| InterDigital | Similar comments to the question in scenario 1 |  |
| Samsung | Step 2: “e.g. ” part can be removed since there are other potential examples. Step 3: “if necessary” can be removed since this procedure is to release the indirect path. Step 4: “it is up to gNB …”, this depends on whether split SRB1 is configured or not, which is not completely gNB implementation issue. In case SRB1 is configured to direct path, only direct path can be used; in case split SRB1 is configured, it is gNB implementation issue |  |
| Ericsson | Step-1: These details are yet to be discussed, suggest an FFS for step-1 i.e., triggering conditions. Step-4: Would it not depend on the split SRB configuration? |  |
| Fraunhofer | Like ZTE indicated already, last UL/DL shall be direct path. | Is there an option that the remote UE triggers the release explicitly and not triggered by measurement reporting? |
| LG Electronics | Step 1 and 2 Examples can be removed. Step 3 FFS on the order of the step 3 and 4 can be added Step 4 Details of the contents can be left to Stage 3. |  |

##### Summary and Updated Procedure

The updated procedure based on the above comments is shown below. Note that any change to the figure is shown in red.

#### 2.6 Indirect path release Figure 2.6: Signalling procedure of indirect path release - The remote UE performs data transmission and reception by using both the direct path on PCell and the indirect path served by a relay UE. 1. The remote UE and the relay UE perform measurements based on measurement configuration. The relay UE may report measurement results. 2. The gNB decides to release the indirect path of remote UE. 3. The gNB sends an RRCReconfiguration message to the relay UE to release the indirect path configuration. FFS: When the RRCReconfiguration message is sent to the relay UE. Whether the order of the step 3 and 4 is up to network implementation 4. The gNB sends the RRCReconfiguration message to the remote UE in order to release the indirect path. FFS: Stage 3 details about the contents in the RRCReconfiguration message. 5. The remote UE sends the RRCReconfigurationComplete message to the gNB via the direct path in order to complete the indirect path release procedure.

# Conclusion and recommendation

Note that even though the outcome of this email discussion could be used for future discussion on CR to 38.300, we mainly intend to discuss and attempt to develop supported cases and identify remaining open issues for each case, not directly for draft CR to 38.300.

In conclusion, Rapporteur recommends agreeing the following proposals:

Proposal 1: Consider the above updated procedures (except direct/indirect path change in Scenario 2) and FFS issues as a baseline and future work.

Proposal 2: RAN2 is requested to include at least the following updated procedures in the running CR to 38.300, possibly with corrections and additional inputs.

- For Scenario 1

  - Direct path addition in section 1.1

  - Indirect path addition in section 1.2

- For Scenario 2

  - Indirect path addition in section 2.1

Proposal 3: RAN2 is requested to discuss whether to include the following updated procedures in the running CR to 38.300, possibly with corrections and additional inputs.

- For Scenario 1

  - Direct path change in section 1.3

  - Indirect path change in section 1.4

  - Direct path release in section 1.7

  - Indirect path release in section 1.8

- For Scenario 2

  - Indirect path release in section 2.6

# Annex A: RAN2 Agreements on Multi-path

- RAN2#119-e

- RAN2 anticipate benefits from multi-path in the following areas:

  - Relay and direct multi-path operation (including both scenarios 1 and 2) can provide efficient path switching between direct path and indirect path

  - The remote UE in multi-path operation can provide enhanced user data throughput and reliability compared to a single link

  - gNB can offload the direct connection of the remote UE in congestion to indirect connection via the relay UE (e.g. at different intra/inter-frequency cells)

- RAN2 can confirm the justifiable benefits that multi-path with relay and UE aggregation can improve the throughput and reliability/robustness, e.g., for UE at the edge of a cell, and UE with limited UL transmission power.

- The terms “relay UE” and “remote UE” are used for scenarios 1 and 2.  FFS if we would use additional terms specific to scenario 2.

- Confirm the remote UE in Scenario 1 and the remote UE in Scenario 2 as follows:

  - Scenario 1: the remote UE is connected to the same gNB using one direct path and one indirect path via 1) Layer-2 UE-to-Network relay,

  - Scenario 2: the remote UE is connected to the same gNB using one direct path and one indirect path via 2) via another UE (where the UE-UE inter-connection is assumed to be ideal).

- RAN2 assumes that the relation between remote UE and relay UE in scenario 2 is pre-configured or static and how the relation is pre-configured or static is out of the 3GPP scope.

- RAN2 deprioritizes discussion on authorization and association mechanism between remote UE and relay UE in scenario 2.

- Support the following cell deployment scenarios for multi-path relaying in Rel-18:

  - Scenario C1: The relay UE and remote UE are served by a same cell.

  - Scenario C2: The relay UE and remote UE are served by different intra-frequency cells of a same gNB

  - Scenario C3: The relay UE and remote UE are served by different inter-frequency cells of a same gNB

- Support the following sidelink scenarios for multi-path:

  - Scenario S1: SL TX/RX and Uu share the same carrier at the remote UE.

  - Scenario S2: SL TX/RX and Uu use different carriers at the remote UE.

  - Scenario S3: SL TX/RX and Uu share the same carrier at the relay UE.

  - Scenario S4: SL TX/RX and Uu use different carriers at the relay UE.

- Support direct bearer (bearer mapped to direct path on Uu), indirect bearer (bearer mapped to indirect path via relay UE), and MP split bearer (bearer mapped to both paths, based on the existing split bearer framework).

- For a MP split bearer in scenario 1, one PDCP entity at the remote UE is configured with one direct Uu RLC channel and one indirect PC5 RLC channel.

  - For upstream, a PDCP entity delivers to a Uu RLC entity and a PC5 RLC entity with SRAP entity in the remote UE side.

  - For downstream, a PDCP entity receives from a Uu RLC entity and a PC5 RLC entity with SRAP entity in the remote UE side.

- FFS if we need to take decisions on the mapping of protocol entities in scenario 2.

- RAN2#119bis-e

- The following cases are to be supported for Scenario 1.

A.	The remote UE operating only on the direct path adds the indirect path under the same gNB;

B.	The remote UE operating only on the indirect path adds the direct path under the same gNB;

C.	The remote UE operating in multi-path releases the indirect path;

D.	The remote UE operating in multi-path releases the direct path;

G.	The remote UE operating in multi-path changes to a new relay UE for the indirect path while keeping the direct path under the same gNB.  FFS if this case would be supported via separate release-and-add (A+C in separate reconfigurations) or a single switch procedure (e.g. similar to i2i service continuity).

- The following case is to be not supported for Scenario 1 as a group mobility scenario.

F.	The remote UE configured with multi-path keeps the serving relay UE for the indirect path and the serving cell of the remote UE for the direct path while the serving relay UE changes the serving cell of the relay UE under the same gNB;

- The following case can be supported via separate release-and-add for scenario 1 (B+D in separate reconfigurations):

E.	The remote UE operating in multi-path changes the direct path to a different cell of the same gNB while using the serving relay UE for the indirect path under the same gNB.

FFS if a single procedure for this case would be supported.

- The following cases are proposed to be supported for Scenario 2.

A.	The remote UE configured only on the direct path adds the indirect path under the same gNB;

C.	The remote UE configured with multi-path releases the indirect path;

- The following case is proposed to be not supported for Scenario 2.

F.	The remote UE configured with multi-path keeps the serving relay UE for the indirect path and the serving cell of the remote UE for the direct path while the serving relay UE changes the serving cell of the relay UE under the same gNB;

- Whether to support the following case can be further discussed for Scenario 2.

B.	The remote UE configured only on the indirect path adds the direct path under the same gNB;

D.	The remote UE configured with multi-path releases the direct path;

E.	The remote UE configured with multi-path changes the serving cell of the remote UE for the direct path while keeping the serving relay UE for the indirect path under the same gNB;

G.	The remote UE configured with multi-path changes to a new relay UE for the indirect path while keeping the direct path under the same gNB.

- For scenario 1, SRB1 and SRB2 can be configured on either the direct or the indirect path, or on both at least with duplication.  FFS if they can be configured on different paths from one another.

- For scenario 2, SRB1 and SRB2 can be configured at least on the direct path.  FFS if there are restrictions on the configuration and if they can be configured on both paths.

- FFS CPDU submission; if legacy CPDU submission behaviour is supported, the primary RLC entity of the MP split bearer for DRB can be configured on any of the paths for Scenario 1.

- PDCP DRB duplication is supported for the MP split bearer in Scenario 1 based on the existing framework.

- PDCP DRB duplication is supported for the MP split bearer in Scenario 2 based on the existing framework.

- The relay UE is restricted to serve only one remote UE in Scenario 2.

- For Scenario 2, different Uu logical channels are configured for identification of data directed to/originating from the relay UE and data relayed from/to the remote UE over the Uu link of the indirect path, as in Rel-17.

- RAN2 assumes that in Scenario 2, without the adaptation layer over non-3GPP link, a PDCP PDU can be delivered to an intended PDCP entity or RLC entity for support of more than one RB over UE-to-UE link based on UE implementation.

- RAN2 does not impose a requirement for interoperability between two UEs from different vendors for scenario 2 in this release.

- RAN2 understand that UE identification in L2 PDU over non-3GPP link is not in 3GPP scope in Scenario 2.

- Do not specify adaptation layer over UE-to-UE link for scenario 2 in RAN2.

- UE identification is not needed over Uu link in Scenario 2, if relay UE serves only one remote UE  and different Uu RLC channels can be assumed for the remote UE and the relay UE.

- Working assumptions:

  - Bearer identification except LCID is not needed in L2 PDU over Uu link in Scenario 2. Only 1:1 bearer mapping is supported over Uu link for the indirect path.  FFS how to configure the mapping.

  - Without the adaptation layer over Uu link in scenario 2, a PDCP PDU can be delivered to an intended PDCP entity or RLC entity for support of more than one RB over Uu link e.g. by configuring 1:1 bearer mapping and different Uu RLC channels for relay UE local traffic and relay traffic for PDU delivery.

  - Do not specify adaptation layer over Uu link for scenario 2 in RAN2.

- Multi-path Relay is applicable to RRC_CONNECTED remote-UE, for scenario-1 and scenario-2.

- Multi-path Relay is NOT applicable to RRC_IDLE remote-UE, for scenario-1 and scenario-2.

- For multi-path Relay, support RRC_IDLE/RRC_INACTIVE target relay UE, for the path switching scenario where there is an addition of indirect path or a change of indirect path.

- When UE operating in multi-path Relay, it performs RLM for Uu interface, for Scenario-1 and Scenario-2. For PC5 interface in Scenario-1, it performs sidelink RLF detection based on Rel-16 V2X specification. For UE-UE link in Scenario-2, whether/how to have failure detection is out of 3GPP scope.

  - FFS whether there is impact to layers under our control from a failure of the UE-UE link in scenario 2.

- RAN2 aims at reusing R17 mechanism of paging delivery for R18 U2N Relay on the indirect path and legacy mechanism on the direct path, in the multi-path setting when paging is applicable for RRC_CONNECTED.

- Multi-path Relay is NOT applicable to RRC Setup procedure, for scenario-1 and scenario-2.

- Working assumption: For multi-path Relay Scenario-2, leave it to relay and remote UE implementation on how to trigger the RRC_IDLE/RRC_INACTIVE target relay UE to initiate RRC connection establishment procedure. RAN2 further discuss the solution for Scenario-1.

- Multi-path Relay is NOT applicable to RRC_INACTIVE remote-UE, for scenario-1 and scenario-2. Support storing direct path configuration for potential resume as legacy operation (to single-path configuration), FFS if the UE can also store indirect path configuration and resume directly into multi-path.

- Multi-path Relay is NOT applicable to RRC Resume procedure, for scenario-1 and scenario-2. RAN2 further study how for UE operating in multi-path Relay operate for RRC Re-establishment procedure.

- RAN2#120

- Support PCell on the direct path only when the UE is in multi-path operation, for both scenario 1 and scenario 2.

- RAN2 confirms the following WA for Scenario 2.

  - Bearer identification except LCID is not needed in L2 PDU over Uu link in Scenario 2. Only 1:1 bearer mapping is supported over Uu link for the indirect path. FFS how to configure the mapping.

  - Without the adaptation layer over Uu link in scenario 2, a PDCP PDU can be delivered to an intended PDCP entity or RLC entity for support of more than one RB over Uu link e.g. by configuring 1:1 bearer mapping and different Uu RLC channels for relay UE local traffic and relay traffic for PDU delivery.

  - Do not specify adaptation layer over Uu link for scenario 2 in RAN2.

- How to configure 1:1 bearer mapping and potential spec impact can be discussed in normative phase.

- In principle, Mode 1 RA can be supported for the remote UE configured with multi-path in Scenario 1.

- RAN2 confirms that split SRB can be configured with or without duplication as a baseline, for both scenarios (assuming it is supported in scenario 2 as proposed elsewhere). Further restrictions can be discussed in normative phase.

- For scenario 2, non-split SRB1/2 is allowed to be configured on direct path.

- Remote UE storing indirect path configuration (e.g., SRAP and PC5-RLC channel configurations) and resuming directly into multi-path configuration is not supported for scenario 1.

- If CSS for SI is configured within the active BWP on PCell, the remote UE can perform direct system information acquisition on PCell as currently specified in 38.331; besides, dedicated signaling can be used to deliver SIB via SRB1 configured on direct and/or indirect path as currently specified in 38.331.

- Upon detection of 3GPP-defined RLF failure in one path, remote UE (configured with MP) can report path failure via the alternative available path if SRB1 is configured on the alternative path or split SRB1 is configured.

- PDCP Control PDU is not duplicated.

- RAN2 do not define a control plane primary path concept in the study phase; FFS if something needs to be defined in normative work, but it should be driven by functionality and technical benefits.

- Case B and case D are not supported for Scenario 2.

- For Scenario 2, Case E is not supported.

- For Scenario 2, whether to support Case G is discussed in normative phase, but RAN2 will not do additional work to enable it for Scenario 2 over Scenario 1.

- Whether SRB1/2 can be configured in different path for Scenario 1 can be discussed in normative phase.

- Whether non-split SRB1/2 is allowed to be configured on indirect path for scenario 2 and whether split SRB1/2 is supported for scenario 2 can be discussed in normative work.

- Remote UE storing indirect path configuration or not and use it to resume to MP configuration in scenario 2 is not supported.

- RAN2 will downselect the solution for triggering IDLE/INACTIVE relay UE to enter CONNECTED state from:

  - Option 1 (SL-RLC or UP-based approach (excluding SL-RLC1)),

  - Option 3 (PC5-RRC approach)

  - Option 4( RRCReconfigurationComplete-based approach),

Discovery/PC5-S-based solution can be further discussed if initiated from SA2.

- Multi-path relay study phase is complete and can proceed to normative work from RAN2 perspective, for both scenarios 1 and 2.

- RAN2#121

- UP-based approach (excluding SL-RLC1) in Option 1 is excluded for relay UE to enter RRC_CONNECTED.

- As a baseline, direct path addition for multi-path is a path switch procedure in which the target configuration contains both paths.

- Upon direct path addition for multi-path, one of the serving cells of the added direct path is configured as PCell for the remote UE.

- In case of Uu-RLF, at least for split SRB1, if SRB1 is available on indirect path not suspended, trigger report to network via indirect path to report the failure via a RRC message. Otherwise, RRC Re-establishment is initiated. RAN2 is requested to discuss whether the RRC message is the existing message e.g. MCGFailureInformation or a new message.

- In case of PC5-RLF, if SRB1 is available on direct path not suspended, trigger report to network via direct path to report the failure via a RRC message.  FFS if an alternative case exists and what would be done in that case.  FFS which message is used.

- The remote UE initiates RRC re-establishment procedure (to a potentially new PCell as in Rel-17, unless further changes are agreed) when failure occurs on both paths (including either PC5 failure or notification of Uu failure on the indirect path).

- The existing PC5-RRC Notification Message procedure is reused for the relay UE to inform the remote UE about Uu failure of the relay UE as currently specified in 38.331.

- In scenario 1, when a remote UE configured with multi-path initiates the RRC re-establishment procedure, the remote UE does not perform re-establishment directly into a multi-path configuration.

- The remote UE in MP operation receives system information at least PBCH/MIB on the direct path and directly acquires SFN from MIB on the direct path, if necessary.

- If CSS for Paging is configured within the active BWP on the direct path on PCell, the remote UE in multi-path operation in RRC_CONNECTED monitors paging on PCell for updated system information or ETWS/CMAS indication, as currently specified in 38.331. The gNB can also provide updated system information or warning message(s) to the remote UE on SRB1, as currently specified.

- As agreed before, RAN2 deprioritizes association mechanism between remote UE and relay UE from CN to RAN.

- gNB provides bearer mapping information to relay UE through dedicated signalling.

- For bringing the idle/inactive relay UE to RRC_CONNECTED, the legacy Rel-17 behaviour (Alt 1 in the proposal) is not disabled for indirect path addition when split SRB1 is configured.  A PC5-RRC trigger is specified at least for other cases.

- FFS if a Rel-17 relay UE is supported for use with multi-path and how the above agreement is reflected in such a case.

- Change of direct path while keeping the indirect path can be done with a release-and-add in a single RRC message.  This does not exclude a gNB implementation from using separate release and add procedures instead.

- Change of indirect path while keeping the direct path can be done with a release-and-add in a single RRC message.  This does not exclude a gNB implementation from using separate release and add procedures instead.

- RAN2#121bis

- The concept of the ‘primary path and primary RLC entity’ is adopted for each MP split bearer configuration according to the existing definition.

- In case of duplication, PDCP control PDU only transmits on the primary RLC entity same as legacy.

- Non-split SRB1 and 2 over indirect path is not supported in Scenario 2.

- Split SRB1 and 2 are supported in Scenario 2 and primary path of the split SRB 1 and 2 is always on direct path.

- If UE-UE link failure is detected on indirect path in Scenario 2, the remote UE can report UE-UE link failure to gNB over direct path.  Details of the reporting mechanism can be further discussed.

- When split SRB1 with duplication is configured, the remote UE sends the RRCReconfigurationComplete message to gNB via both paths for Scenario 1.

- When one of the following conditions is met, the remote UE sends the RRCReconfigurationComplete message to gNB via the direct path for Scenario 1. FFS on need for additional condition.

  - when primary RLC entity of split SRB1 is on direct path

  - when non-split SRB1 is configured on direct path

- When split SRB1 with duplication is configured, the remote UE sends the RRCReconfigurationComplete message to gNB via both paths for Scenario 2.

- When one of the following conditions is met, the remote UE sends the RRCReconfigurationComplete message to gNB via the direct path for Scenario 2.

  - when primary RLC entity of split SRB1 is on direct path

  - when non-split SRB1 is configured on direct path

- The bearer type configuration is provided per SRB.  It is up to network implementation whether to configure SRB1 and SRB2 with same or different bearer types (within the bearer types that are supported).

  - FFS if there are cases where the configuration of non-split SRBs over indirect path is useful.

- In scenario 2, if both remote and relay UE are in RRC_CONNECTED, the remote UE reports relay UE’s ID to gNB for indirect path addition.  Need for reporting in the idle/inactive case can be further discussed.  FFS what ID is used.

- WA: For a remote UE and relay UE in RRC_CONNECTED, the network is expected to release the multipath configuration related to this relay at the remote UE before it releases the relay UE to RRC_IDLE/RRC_INACTIVE.

- A remote UE in RRC_CONNECTED, upon reception of Uu RLF indication from the relay UE, suspends transmissions on the indirect path and informs the network if SRB1 is available on the direct path and not suspended, otherwise triggers re-establishment.  FFS whether to apply the same behaviour 1) when the relay UE informs the remote UE of HO; 2) When the relay UE moves to IDLE following expiry of dataInactivityTimer, if the timer is supported for the relay UE.  This agreement does not imply any conclusion on non-split SRB1 on indirect path.

- A remote UE in multipath that is released to RRC_IDLE/RRC_INACTIVE can apply legacy cell/relay selection behaviour, thus moving to single-path operation on either path according to implementation.

- LS to SA2 on authorization for multi-path Scenario 2 is approved in [105].

- RAN2#122

- For Scenario-1/2, PDCP duplication of DRB is controlled by legacy Duplication Activation/Deactivation MAC CE and Duplication RLC Activation/Deactivation MAC CE delivered via direct path.

- For Scenario-1/2, RRC sets the initial state of PDCP duplication for split SRB/DRB as in legacy.

- For Scenario-1/2, when reporting direct-path failure via indirect-path, use MCGFailureInformation message. FFS on whether additional IE needs to be introduced.

- For Scenario-1/2, if MCGFailureInformation is agreed for direct path failure recovery in P5, reuse T316 timer for the direct path failure recovery.

- For Scenario-1/2, confirm the WA that: for a remote UE and relay UE in RRC_CONNECTED, the network is expected to release the multipath configuration related to this relay at the remote UE before it releases the relay UE to RRC_IDLE/INACTIVE. No spec impact is foreseen.

- For Scenario-1/2, not pursue remote UE notifying network upon reception of notification message indicating relay UE handover. FFS whether rely on network to release configuration of relay UE at remote UE before relay UE handover, or rely on remote UE to suspend the indirect path upon reception of notification message indicating relay UE handover.

- For Scenario-1/2, optionally configure UL data split threshold for split DRB. Usage of the threshold follows legacy behavior.

- For Scenario-1/2, no specification effort to handle the case when the relay UE moves to RRC_IDLE following expiry of dataInactivityTimer, i.e., not pursue relay UE notifying remote UE, and remote UE notifying network.

- For Scenario-1, use T304-like timer for direct path addition and change. FFS on expiry behavior.

- For Scenario-1, use T420-like timer for indirect path addition and change. FFS on stop condition and expiry behavior.

- FFS if these two timers are new or reuse the existing timers.

- WA: For scenario 2, remote-UE reports the RRC_CONNECTED relay-UE C-RNTI and serving cell ID (e.g., NCGI) for indirect path addition.

  - LS to SA3 to check if there is any security concern with the WA is approved in [209].

- WA: For Scenario-1/2, MP remote UE is configured with a single cell group, i.e., MCG, for the direct path, and SL configuration, for the indirect path.

- WA: For scenario 1, primary path of the split SRB1 and SRB2 is always configured on direct path.  This does not preclude having the case where the UE switches the primary path to the indirect path for reporting after direct path failure.

- For scenario 1, SRB1 and SRB2 are not decoupled in terms of support of non-split SRB on indirect path; i.e., if SRB1 can be supported on indirect path, so can SRB2.

- For Scenario-1, mode-1 scheduling for remote UE is supported at least for intra-DU case, with the SR/BSR and grant sent on the direct path; whether it is supported for inter-DU case is up to R3, but R2 do not intend to make specification changes to support this case, and for specification purposes RAN2 intend to model it as a single MAC entity at the UE.

  - LS to R3 to notify this conclusion, with “take into account” action is approved in [212].

# Annex B: RAN3 Agreements on Multi-path

- RAN3#117

- From RAN3 perspective, multi-path scenario should be supported in Rel-18.

- Both intra-DU and inter-DU cases will be supported under the same gNB.

- RAN3 waits for the RAN2 progress on how to define control plane and user plane scenarios for multi-path support.

- RAN3 waits for the RAN2 progress on whether and how to define the Primary path in multi-path support.

- Addition of direct/indirect path are supported as follows:

  - Add direct path, after the establishment of the indirect path.

  - Add indirect path, after the establishment of the direct path.

  - This does not imply the exclusion of any other path addition possibility.

- RAN3 will study the signaling impact on the direct or indirect path change under the same gNB for a UE connected via multi-path. The other mobility scenarios can be further considered based on RAN2 decision.

- The following use cases are not supported in Rel-18.

  - Configure two indirect paths

  - More than two paths

  - Inter-gNB multi-path support

- RAN3#117bis-e

- For Scenario 1, the responsibility of gNB-CU and gNB-DU in Rel-17 SL relay can be reused as a baseline. Whether to enhance the responsibility of gNB-CU and gNB-DU in Rel-18 is FFS.

- For the responsibility of gNB-CU and gNB-DU in Scenario 2, the RAN3 waits for RAN2’s progress on protocol stack for Scenario 2.

- For the multi-path support, the gNB-CU takes the responsibility to decide the addition/modification/release of the path.

- For intra-DU and inter-DU cases, the UE Context Setup / Modification procedure can be reused to configure the 2nd path with possible enhancements. The details will be discussed based on RAN2 progress.

- The RAN3 waits for the RAN2 progress on whether the gNB-DU knows the path information of each configured path.

- WA: The direct path and indirect path cannot be configured for a remote UE simultaneously in this release, depending on RAN2 decision.

- The gNB-CU is responsible to determine the data split among two paths for a DRB for both intra-DU and inter-DU cases.

- For intra-DU case, two F1-U tunnels are setup between CU and DU for a split DRB. FFS on how to support the multi-path delivery of split SRB.

- WA: For inter-DU case, legacy DC based data split/duplication mechanism can be reused as baseline for split DRB/SRB.

- WA: The RAN3 will specify the details of the path change procedure after introducing the procedure of the direct/indirect path addition.

- Previous RAN3 agreement is updated as follows:

  - For Scenario 1, addition of direct/indirect path are supported as follows:

    - Add direct path, after the establishment of the indirect path.

    - Add indirect path, after the establishment of the direct path.

  - For Scenario 2, addition of direct/indirect path are supported as follows:

    - Add indirect path, after the establishment of the direct path.

    - Whether to add direct path, after the establishment of the indirect path is pending to RAN2 decision.

- For Scenario 2, interface between UEs are non-3GPP defined. Therefore, in the UE context setup/modification procedure, the PC5 Relay RLC channel configurations are not needed for remote UE and relay UE.

- RAN3#118

- From RAN3 point of view, the Multi-path study phase is completed, and the Multi-path can move to normative work phase.

- RAN3#119

- Add Multi-path authorization from AMF in TS 38.413, 38.423, 38.473 (in R3-230896, R3-230897, R3-230898)

- Turn the following WAs to agreement:

  - NG-RAN receives the multi-path authorization from the AMF.

  - The multi-path authorization can be added in the 5G ProSe Authorized IE if it is needed.

  - The direct path and indirect path cannot be configured for a remote UE simultaneously in this release, depending on RAN2 decision.

  - For inter-DU case, legacy DC based data split/duplication mechanism can be reused as baseline for split DRB/SRB.

  - The RAN3 will specify the details of the path change procedure after introducing the procedure of the direct/indirect path addition.

- Add Inter-DU Direct Path Addition and Inter-DU Indirect Path Addition procedures in TS 38.401 (in R3-230947).

- RAN3#119bis-e

- RAN3 further wait for RAN2 progress before removing two Editor’s notes in BL CR to TS 38.401.

- For intra-DU case, the gNB-CU should inform the gNB-DU about the path to be added, released or modified.

- The Relay UE L2 ID for indirect path addition and the PCell ID for direct path addition should be provided to the gNB-DU. Whether to inform the gNB-DU of other information needs to be further discussed.

- The radio bearer type and channels mapping to be added can be provided to the gNB-DU by Rel-17 U2N relay signaling design.

- Add the intra-DU path addition procedure in the BL CR to TS 38.401. The BL CR in R3-232088 is agreed

- For Scenario 2, the responsibility for gNB-CU and gNB-DU are defined as follows:

  - gNB-CU’s responsibility:

    - Remote UE and relay UE context maintenance

    - Remote UE bearer mapping

    - Relaying Uu RLC channel management

  - gNB-DU’s responsibility:

    - Determine the RLC/MAC/PHY Configuration for the Uu Relay RLC channels of relay UE

  - The responsibility for gNB-CU and gNB-DU can be further enhanced based on RAN2 progress.

- WA: For the intra-DU case, the gNB-DU should take the responsibility of mode 1 resource scheduling for both U2N relay UE and U2N remote UE.

- RAN3#120

- WA: For indirect path addition, the Remote UE Local ID IE and the T420-like timer are sent to the gNB-DU.

- RAN3 reuses the existing F1AP signaling to achieve the operations for ‘Path to be modified’ by means of modifying the configurations associated to the path(s).