3GPP TSG-RAN WG2 Meeting #123bis	R2-23xxxxx

Chicago, US, 13th – 17th Nov 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.331 | CR |  | rev | - | Current version: | 17.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Running RRC CR for NR MUSIM enhancements |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | vivo |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_DualTxRx_MUSIM-Core |  |  |  |  |  | Date: |  |  | 2023-10-16 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Dual Transmission/Reception (Tx/Rx) Multi-SIM for NR. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - The changes can be summarized as follows: - Indication in the OtherConfig to indicate whether UE is allowed to report MUSIM gap priority preference via UAI. - Support the preference reporting and configuration of the periodic MUSIM gap priorities. - Capability restrictions indication of maximum MIMO layers. - Temporary capability restrictions (e.g. via UAI) only after the NW signals via RRC that this is allowed. - Use Msg5 for early indication of MUSIM capability restriction for UEs in IDLE. - MUSIM temporary capability restriction only if the network indicates that it is allowed in SIB1.UE can indicate the temporary maximum MIMO layers for specific serving cells for both UL and DL. - UE can explicitly request specific serving cells or serving cell group to be released for Rel-18 MUSIM purpose. UE can explicitly request specific serving cells or serving cell group to be released for Rel-18 MUSIM purpose. - Use inter-node messages to convey Rel-17 MUSIM gap configuration from MN to SN in NW A when UE is in NR-DC - The UE can indicate that some constrained band combinations . - The UL/DL MIMO layer and/or the UL/DL supported bandwidth restriction (if supported) shall only work for the restricted frequencies for the proactive case. - Introduce ‘wait timer’ for the reactive approach - Introduce ‘prohibit timer’ for the proactive approach - Early capability restriction indication is provided in Msg5 - UE can indicate the temporary capability restriction of measurement gap - Baseline for the proactive approach: Maximum MIMO layers restriction - No need to request gap priority or configure gap priority for aperiodic gap - Introduce single bit indication in MUSIM assistance information to indicate the UE preference of “keep” option - The prohibit timer configuration for R17 MUSIM gap preference (i.e. musim-GapProhibitTimer) is also apply to R18 MUSIM gap priority preference |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Temporary capability restriction and gap priority are not supported for MUSIM operation. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.3.3.4, 5.3.5.9, 5.3.5.9a, 5.3.5.x, 5.3.7.2, 5.3.13.2, 5.3.13.4, 5.7.4.1, 5.7.4.2, 5.7.4.3, 6.2.2, 6.3.2, 6.3.3, 6.3,4, 7.1.1,11.2.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 38.300: "NR; Overall description; Stage 2".

[3]	3GPP TS 38.321: "NR; Medium Access Control (MAC); Protocol specification".

[4]	3GPP TS 38.322: "NR; Radio Link Control (RLC) protocol specification".

[5]	3GPP TS 38.323: "NR; Packet Data Convergence Protocol (PDCP) protocol specification".

[6]	ITU-T Recommendation X.680 (08/2015) "Information Technology – Abstract Syntax Notation One (ASN.1): Specification of basic notation" (Same as the ISO/IEC International Standard 8824-1).

[7]	ITU-T Recommendation X.681 (08/2015) "Information Technology – Abstract Syntax Notation One (ASN.1): Information object specification" (Same as the ISO/IEC International Standard 8824-2).

[8]	ITU-T Recommendation X.691 (08/2015) "Information technology – ASN.1 encoding rules: Specification of Packed Encoding Rules (PER)" (Same as the ISO/IEC International Standard 8825-2).

[9]	3GPP TS 38.215: "NR; Physical layer measurements".

[10]	3GPP TS 36.331: "Evolved Universal Terrestrial Radio Access (E-UTRA) Radio Resource Control (RRC); Protocol Specification".

[11]	3GPP TS 33.501: "Security Architecture and Procedures for 5G System".

[12]	3GPP TS 38.104: "NR; Base Station (BS) radio transmission and reception".

[13]	3GPP TS 38.213: "NR; Physical layer procedures for control".

[14]	3GPP TS 38.133: "NR; Requirements for support of radio resource management".

[15]	3GPP TS 38.101-1: "NR; User Equipment (UE) radio transmission and reception; Part 1: Range 1 Standalone".

[16]	3GPP TS 38.211: "NR; Physical channels and modulation".

[17]	3GPP TS 38.212: "NR; Multiplexing and channel coding".

[18]	ITU-T Recommendation X.683 (08/2015) "Information Technology – Abstract Syntax Notation One (ASN.1): Parameterization of ASN.1 specifications" (Same as the ISO/IEC International Standard 8824-4).

[19]	3GPP TS 38.214: "NR; Physical layer procedures for data".

[20]	3GPP TS 38.304: "NR; User Equipment (UE) procedures in Idle mode and RRC Inactive state".

[21]	3GPP TS 23.003: "Numbering, addressing and identification".

[22]	3GPP TS 36.101: "E-UTRA; User Equipment (UE) radio transmission and reception".

[23]	3GPP TS 24.501: "Non-Access-Stratum (NAS) protocol for 5G System (5GS); Stage 3".

[24]	3GPP TS 37.324: "Service Data Adaptation Protocol (SDAP) specification".

[25]	3GPP TS 22.261: "Service requirements for the 5G System".

[26]	3GPP TS 38.306: "User Equipment (UE) radio access capabilities".

[27]	3GPP TS 36.304: "E-UTRA; User Equipment (UE) procedures in idle mode".

[28]	ATIS 0700041: "WEA 3.0: Device-Based Geo-Fencing".

[29]	3GPP TS 23.041: "Technical realization of Cell Broadcast Service (CBS)".

[30]	3GPP TS 33.401: "3GPP System Architecture Evolution (SAE); Security architecture".

[31]	3GPP TS 36.211: "E-UTRA; Physical channels and modulation".

[32]	3GPP TS 23.501: "System Architecture for the 5G System; Stage 2".

[33]	3GPP TS 36.104:"E-UTRA; Base Station (BS) radio transmission and reception".

[34]	3GPP TS 38.101-3 "NR; User Equipment (UE) radio transmission and reception; Part 3: Range 1 and Range 2 Interworking operation with other radios".

[35]	3GPP TS 38.423: "NG-RAN, Xn application protocol (XnAP)".

[36]	3GPP TS 38.473: "NG-RAN; F1 application protocol (F1AP)".

[37]	3GPP TS 36.423: "E-UTRA; X2 application protocol (X2AP)".

[38]	3GPP TS 24.008: "Mobile radio interface layer 3 specification; Core network protocols; Stage 3".

[39]	3GPP TS 38.101-2 "NR; User Equipment (UE) radio transmission and reception; Part 2: Range 2 Standalone".

[40]	3GPP TS 36.133:"E-UTRA; Requirements for support of radio resource management".

[41]	3GPP TS 37.340: "E-UTRA and NR; Multi-connectivity; Stage 2".

[42]	3GPP TS 38.413: "NG-RAN, NG Application Protocol (NGAP)".

[43]	3GPP TS 23.502: "Procedures for the 5G System; Stage 2".

[44]	3GPP TR 36.816: "Evolved Universal Terrestrial Radio Access (E-UTRA); Study on signalling and procedure for interference avoidance for in-device coexistence ".

[45]	3GPP TS 25.331: "Universal Terrestrial Radio Access (UTRA); Radio Resource Control (RRC); Protocol specification".

[46]	3GPP TS 25.133: "Requirements for Support of Radio Resource Management (FDD)".

[47]	3GPP TS 38.340: "Backhaul Adaptation Protocol (BAP) specification"

[48]	3GPP TS 37.213: "Physical layer procedures for shared spectrum channel access".

[49]	3GPP TS 37.355: "LTE Positioning Protocol (LPP)".

[50]	IEEE 802.11-2012, Part 11: Wireless LAN Medium Access Control (MAC) and Physical Layer (PHY) specifications, IEEE Std.

[51]	Bluetooth Special Interest Group: "Bluetooth Core Specification v5.0", December 2016.

[52]	3GPP TS 32.422: "Telecommunication management; Subscriber and equipment trace; Trace control and configuration management".

[53]	3GPP TS 38.314: "NR; layer 2 measurements".

[54]	Void.

[55]	3GPP TS 23.287: "Architecture enhancements for 5G System (5GS) to support Vehicle-to-Everything (V2X) services".

[56]	3GPP TS 23.285: "Technical Specification Group Services and System Aspects; Architecture enhancements for V2X services".

[57]	3GPP TS 24.587: " Technical Specification Group Core Network and Terminals; Vehicle-to-Everything (V2X) services in 5G System (5GS)".

[58]	Military Standard WGS84 Metric MIL-STD-2401 (11 January 1994): "Military Standard Department of Defence World Geodetic System (WGS)".

[59]	3GPP TS 38.101-4 "NR; User Equipment (UE) radio transmission and reception; Part 4: Performance Requirements".

[60]	3GPP TS 33.536: "Technical Specification Group Services and System Aspects; Security aspects of 3GPP support for advanced Vehicle-to-Everything (V2X) services".

[61]	3GPP TS 37.320: "Radio measurement collection for Minimization of Drive Tests (MDT); Overall description; Stage 2".

[62]	3GPP TS 36.306: "User Equipment (UE) radio access capabilities".

[63]	3GPP TS 38.174: "NR; Integrated Access and Backhaul (IAB) radio transmission and reception".

[64]	3GPP TS 38.472: "NG-RAN; F1 signalling transport".

[65]	3GPP TS 23.304: "Proximity based Services (ProSe) in the 5G System (5GS)".

[66]	3GPP TS 38.351: "NR; Sidelink Relay Adaptation Protocol (SRAP) Specification".

[67]	3GPP TS 23.247: "Architectural enhancements for 5G multicast-broadcast services; Stage 2"

[68]	3GPP TS 26.247: "Transparent end-to-end Packet-switched Streaming Service (PSS); Progressive Download and Dynamic Adaptive Streaming over HTTP (3GP-DASH)".

[69]	3GPP TS 26.114: "IP Multimedia Subsystem (IMS); Multimedia Telephony; Media handling and interaction".

[70]	3GPP TS 26.118: "Virtual Reality (VR) profiles for streaming applications".

[71]	NIMA TR 8350.2, Third Edition, Amendment 1, 3 January 2000: "DEPARTMENT OF DEFENSE WORLD GEODETIC SYSTEM 1984".

[72]	3GPP TS 24.554: "Technical Specification Group Core Network and Terminals; Proximity-services (ProSe) in 5G System (5GS) protocol".

[73]	3GPP TS 38.305: "NG Radio Access Network (NG-RAN); Stage 2 functional specification of User Equipment (UE) positioning in NG-RAN".

[74]	3GPP TS 23.122: "Non-Access-Stratum (NAS) functions related to Mobile Station (MS) in idle mode".

[75]	3GPP TS 38.101-5: "User Equipment (UE) radio transmission and reception; Part 5: Satellite access Radio Frequency (RF) and performance requirements".

# 3	Definitions, symbols and abbreviations

## 3.1	Definitions

For the purposes of the present document, the terms and definitions given in TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in TR 21.905 [1].

AM MRB: An MRB associated with at least an AM RLC bearer for PTP transmission.

BH RLC channel: An RLC channel between two nodes, which is used to transport backhaul packets.

Broadcast MRB: A radio bearer configured for MBS broadcast delivery.

CEIL: Mathematical function used to 'round up' i.e. to the nearest integer having a higher or equal value.

DAPS bearer: a bearer whose radio protocols are located in both the source gNB and the target gNB during DAPS handover to use both source gNB and target gNB resources.

Dedicated signalling: Signalling sent on DCCH logical channel between the network and a single UE.

Dormant BWP: The dormant BWP is one of downlink BWPs configured by the network via dedicated RRC signalling. In the dormant BWP, the UE stops monitoring PDCCH on/for the SCell, but continues performing CSI measurements, Automatic Gain Control (AGC) and beam management, if configured. For each serving cell other than the SpCell or PUCCH SCell, the network may configure one BWP as a dormant BWP.

Field: The individual contents of an information element are referred to as fields.

FLOOR: Mathematical function used to 'round down' i.e. to the nearest integer having a lower or equal value.

Frequency Selection Area ID: An identity used for broadcast MBS session to guide the frequency selection of the UE as defined in TS 23.247 [67].

Global cell identity: An identity to uniquely identifying an NR cell. It is consisted of cellIdentity and plmn-Identity of the first PLMN-Identity in plmn-IdentityList in SIB1.

Information element: A structural element containing single or multiple fields is referred as information element.

MBS Radio Bearer: A radio bearer that is configured for MBS delivery.

Multicast/Broadcast Service: A point-to-multipoint service as defined in TS 23.247 [67].

Multicast MRB: A radio bearer configured for MBS multicast delivery.

MUSIM gap: Period that the UE may use to perform MUSIM operations.

NCSG: Network controlled small gap as defined in TS 38.133 [14].

NPN-only Cell: A cell that is only available for normal service for NPNs' subscriber. An NPN-capable UE determines that a cell is NPN-only Cell by detecting that the cellReservedForOtherUse IE is set to true while the npn-IdentityInfoList IE is present in CellAccessRelatedInfo.

NR sidelink communication: AS functionality enabling at least V2X Communication as defined in TS 23.287 [55], and ProSe Communication (including ProSe UE-to-Network Relay and non-Relay communication) as defined in TS 23.304 [65] between two or more nearby UEs, using NR technology but not traversing any network node.

NR sidelink discovery: AS functionality enabling ProSe non-Relay Discovery and ProSe UE-to-Network Relay discovery for Proximity based Services as defined in TS 23.304 [65] between two or more nearby UEs, using NR technology but not traversing any network node.

PNI-NPN identity: an identifier of a PNI-NPN comprising of a PLMN ID and a CAG -ID combination.

Primary Cell: The MCG cell, operating on the primary frequency, in which the UE either performs the initial connection establishment procedure or initiates the connection re-establishment procedure.

PC5 Relay RLC channel: An RLC channel between L2 U2N Remote UE and L2 U2N Relay UE, which is used to transport packets over PC5 for L2 UE-to-Network relay.

Primary SCG Cell: For dual connectivity operation, the SCG cell in which the UE performs random access when performing the Reconfiguration with Sync procedure.

Primary Timing Advance Group: Timing Advance Group containing the SpCell.

PUCCH SCell: An SCell configured with PUCCH by PUCCH-Config.

PUSCH-Less SCell: An SCell configured without PUSCH.

RedCap UE: A UE with reduced capabilities as specified in clause 4.2.21.1 in TS 38.306 [26].

RLC bearer configuration: The lower layer part of the radio bearer configuration comprising the RLC and logical channel configurations.

Secondary Cell: For a UE configured with CA, a cell providing additional radio resources on top of Special Cell.

Secondary Cell Group: For a UE configured with dual connectivity, the subset of serving cells comprising of the PSCell and zero or more secondary cells.

Serving Cell: For a UE in RRC_CONNECTED not configured with CA/DC there is only one serving cell comprising of the primary cell. For a UE in RRC_CONNECTED configured with CA/ DC the term 'serving cells' is used to denote the set of cells comprising of the Special Cell(s) and all secondary cells.

Small Data Transmission: A procedure used for transmission of data and/or signalling over allowed radio bearers in RRC_INACTIVE state (i.e. without the UE transitioning to RRC_CONNECTED state).

SNPN identity: an identifier of an SNPN comprising of a PLMN ID and an NID combination.

Special Cell: For Dual Connectivity operation the term Special Cell refers to the PCell of the MCG or the PSCell of the SCG, otherwise the term Special Cell refers to the PCell.

Split SRB: In MR-DC, an SRB that supports transmission via MCG and SCG as well as duplication of RRC PDUs as defined in TS 37.340 [41].

SSB Frequency: Frequency referring to the position of resource element RE=#0 (subcarrier #0) of resource block RB#10 of the SS block.

U2N Relay UE: A UE that provides functionality to support connectivity to the network for U2N Remote UE(s).

U2N Remote UE: A UE that communicates with the network via a U2N Relay UE.

Uu Relay RLC channel: An RLC channel between L2 U2N Relay UE and gNB, which is used to transport packets over Uu for L2 UE-to-Network relay.

UE Inactive AS Context: UE Inactive AS Context is stored when the connection is suspended and restored when the connection is resumed. It includes information as defined in clause 5.3.8.3.

V2X sidelink communication: AS functionality enabling V2X Communication as defined in TS 23.285 [56], between nearby UEs, using E-UTRA technology but not traversing any network node.

## 3.2	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1].

5GC	5G Core Network

ACK	Acknowledgement

AM	Acknowledged Mode

ARQ	Automatic Repeat Request

AS	Access Stratum

ASN.1	Abstract Syntax Notation One

BAP	Backhaul Adaptation Protocol

BCD	Binary Coded Decimal

BFD	Beam Failure Detection

BH	Backhaul

BLER	Block Error Rate

BWP	Bandwidth Part

CA	Carrier Aggregation

CAG	Closed Access Group

CAG-ID	Closed Access Group Identifier

CAPC	Channel Access Priority Class

CBR	Channel Busy Ratio

CCCH	Common Control Channel

CFR	Common Frequency Resources

CG	Cell Group

CHO	Conditional Handover

CLI	Cross Link Interference

CMAS	Commercial Mobile Alert Service

CP	Control Plane

CPA	Conditional PSCell Addition

CPC	Conditional PSCell Change

C-RNTI	Cell RNTI

CSI	Channel State Information

DAPS	Dual Active Protocol Stack

DC	Dual Connectivity

DCCH	Dedicated Control Channel

DCI	Downlink Control Information

DCP	DCI with CRC scrambled by PS-RNTI

DFN	Direct Frame Number

DL	Downlink

DL-PRS	Downlink Positioning Reference Signal

DL-SCH	Downlink Shared Channel

DM-RS	Demodulation Reference Signal

DRB	(user) Data Radio Bearer

DRX	Discontinuous Reception

DTCH	Dedicated Traffic Channel

ECEF	Earth-Centered, Earth-Fixed

ECI	Earth-Centered Inertial

EN-DC	E-UTRA NR Dual Connectivity with E-UTRA connected to EPC

EPC	Evolved Packet Core

EPS	Evolved Packet System

ETWS	Earthquake and Tsunami Warning System

E-UTRA	Evolved Universal Terrestrial Radio Access

E-UTRA/5GC	E-UTRA connected to 5GC

E-UTRA/EPC	E-UTRA connected to EPC

E-UTRAN	Evolved Universal Terrestrial Radio Access Network

FDD	Frequency Division Duplex

FFS	For Further Study

G-CS-RNTI	Group Configured Scheduling RNTI

GERAN	GSM/EDGE Radio Access Network

GIN	Group ID for Network selection

GNSS	Global Navigation Satellite System

G-RNTI	Group RNTI

GSM	Global System for Mobile Communications

HARQ	Hybrid Automatic Repeat Request

HRNN	Human Readable Network Name

HSDN	High Speed Dedicated Network

H-SFN	Hyper SFN

IAB	Integrated Access and Backhaul

IAB-DU	IAB-node DU

IAB-MT	IAB Mobile Termination

IDC	In-Device Coexistence

IE	Information element

IMSI	International Mobile Subscriber Identity

kB	Kilobyte (1000 bytes)

L1	Layer 1

L2	Layer 2

L3	Layer 3

LBT	Listen Before Talk

LEO	Low Earth Orbit

MAC	Medium Access Control

MBS	Multicast/Broadcast Service

MBS FSAI	MBS Frequency Selection Area Identity

MCCH	MBS Control Channel

MCG	Master Cell Group

MDT	Minimization of Drive Tests

MIB	Master Information Block

MPE	Maximum Permissible Exposure

MRB	MBS Radio Bearer

MR-DC	Multi-Radio Dual Connectivity

MTCH	MBS Traffic Channel

MTSI	Multimedia Telephony Service for IMS

MUSIM	Multi-Universal Subscriber Identity Module

N/A	Not Applicable

NE-DC	NR E-UTRA Dual Connectivity

(NG)EN-DC	E-UTRA NR Dual Connectivity (covering E-UTRA connected to EPC or 5GC)

NGEN-DC	E-UTRA NR Dual Connectivity with E-UTRA connected to 5GC

NID	Network Identifier

NPN	Non-Public Network

NR-DC	NR-NR Dual Connectivity

NR/5GC	NR connected to 5GC

NSAG	Network Slice AS Group

NTN	Non-Terrestrial Network

PCell	Primary Cell

PDCP	Packet Data Convergence Protocol

PDU	Protocol Data Unit

PEI	Paging Early Indication

PEI-O	Paging Early Indication-Occasion

PLMN	Public Land Mobile Network

PNI-NPN	Public Network Integrated Non-Public Network

posSIB	Positioning SIB

PPW	PRS Processing Window

PRS	Positioning Reference Signal

PSCell	Primary SCG Cell

PTM	Point to Multipoint

PTP	Point to Point

PWS	Public Warning System

QoE	Quality of Experience

QoS	Quality of Service

RAN	Radio Access Network

RAT	Radio Access Technology

RLC	Radio Link Control

RLM	Radio Link Monitoring

RMTC	RSSI Measurement Timing Configuration

RNA	RAN-based Notification Area

RNTI	Radio Network Temporary Identifier

ROHC	Robust Header Compression

RPLMN	Registered Public Land Mobile Network

RRC	Radio Resource Control

RS	Reference Signal

SBAS	Satellite Based Augmentation System

SCell	Secondary Cell

SCG	Secondary Cell Group

SCS	Subcarrier Spacing

SD-RSRP	Sidelink Discovery RSRP

SDT	Small Data Transmission

SFN	System Frame Number

SFTD	SFN and Frame Timing Difference

SI	System Information

SIB	System Information Block

SL	Sidelink

SLSS	Sidelink Synchronisation Signal

SNPN	Stand-alone Non-Public Network

SpCell	Special Cell

SRAP	Sidelink Relay Adaptation Protocol

SRB	Signalling Radio Bearer

SRS	Sounding Reference Signal

SSB	Synchronization Signal Block

TAG	Timing Advance Group

TDD	Time Division Duplex

TEG	Timing Error Group

TM	Transparent Mode

TMGI	Temporary Mobile Group Identity

U2N	UE-to-Network

UDC	Uplink Data Compression

UE	User Equipment

UL	Uplink

UM	Unacknowledged Mode

UP	User Plane

VR	Virtual Reality

In the ASN.1, lower case may be used for some (parts) of the above abbreviations e.g. c-RNTI.

# 4	General

---------------------------------------------------------Skip Unchanged----------------------------------------------------------

# 5	Procedures

---------------------------------------------------------Skip Unchanged----------------------------------------------------------

## 5.3	Connection control

### 5.3.1	Introduction

#### 5.3.1.1	RRC connection control

RRC connection establishment involves the establishment of SRB1. The network completes RRC connection establishment prior to completing the establishment of the NG connection, i.e. prior to receiving the UE context information from the 5GC. Consequently, AS security is not activated during the initial phase of the RRC connection. During this initial phase of the RRC connection, the network may configure the UE to perform measurement reporting, but the UE only sends the corresponding measurement reports after successful AS security activation. However, the UE only accepts a re-configuration with sync message when AS security has been activated.

Upon receiving the UE context from the 5GC, the RAN activates AS security (both ciphering and integrity protection) using the initial AS security activation procedure. The RRC messages to activate AS security (command and successful response) are integrity protected, while ciphering is started only after completion of the procedure. That is, the response to the message used to activate AS security is not ciphered, while the subsequent messages (e.g. used to establish SRB2, DRBs and multicast MRBs) are both integrity protected and ciphered. After having initiated the initial AS security activation procedure, the network may initiate the establishment of SRB2 and DRBs and/or multicast MRBs, i.e. the network may do this prior to receiving the confirmation of the initial AS security activation from the UE. In any case, the network will apply both ciphering and integrity protection for the RRC reconfiguration messages used to establish SRB2, DRBs and/or multicast MRBs. The network should release the RRC connection if the initial AS security activation and/ or the radio bearer establishment fails. A configuration with SRB2 without DRB or multicast MRB, or with DRB or multicast MRB without SRB2 is not supported (i.e., SRB2 and at least one DRB or multicast MRB must be configured in the same RRC Reconfiguration message, and it is not allowed to release all the DRBs and multicast MRBs without releasing the RRC Connection). For IAB-MT, a configuration with SRB2 without any DRB/MRB is supported.

The release of the RRC connection normally is initiated by the network. The procedure may be used to re-direct the UE to an NR frequency or an E-UTRA carrier frequency.

The suspension of the RRC connection is initiated by the network. When the RRC connection is suspended, the UE stores the UE Inactive AS context and any configuration received from the network, and transits to RRC_INACTIVE state. The RRC message to suspend the RRC connection is integrity protected and ciphered.

The resumption of a suspended RRC connection is initiated by upper layers when the UE needs to transit from RRC_INACTIVE state to RRC_CONNECTED state or by RRC layer to perform a RNA update or by RAN paging from NG-RAN or for SDT. When the RRC connection is resumed, network configures the UE according to the RRC connection resume procedure based on the stored UE Inactive AS context and any RRC configuration received from the network. The RRC connection resume procedure re-activates AS security and re-establishes SRB(s) and DRB(s) and/or multicast MRB(s), if configured.

Upon initiating the resume procedure for SDT, AS security (both ciphering and integrity protection) is re-activated for SRB2 (if configured for SDT) and for SRB1. In addition, AS security is also re-activated (if security is configured) for all the DRBs configured for SDT. Further, the PDCP entities of SRB1 and PDCP entities of the radio bearers configured for SDT are re-established and resumed whilst the UE remains in RRC_INACTIVE state. Transmission and reception of data and/or signalling messages over radio bearers configured for SDT can happen whilst the UE is in RRC_INACTIVE state and SDT procedure is ongoing.

In response to a request to resume the RRC connection or in response to a resume procedure initiated for SDT, the network may resume the suspended RRC connection and send UE to RRC_CONNECTED, or reject the request to resume and send UE to RRC_INACTIVE (with a wait timer), or directly re-suspend the RRC connection and send UE to RRC_INACTIVE, or directly release the RRC connection and send UE to RRC_IDLE, or instruct the UE to initiate NAS level recovery (in this case the network sends an RRC setup message).

NOTE:	In case the UE receives the configurations for NR sidelink communication via the E-UTRA, the configurations for NR sidelink communication in SIB12 and sl-ConfigDedicatedNR within RRCReconfiguration used in clause 5.3 are provided by the configurations in SystemInformationBlockType28 and sl-ConfigDedicatedForNR within RRCConnectionReconfiguration as specified in TS 36.331[10], respectively.

#### 5.3.1.2	AS Security

AS security comprises of the integrity protection and ciphering of RRC signalling (SRBs) and user data (DRBs).

RRC handles the configuration of the AS security parameters which are part of the AS configuration: the integrity protection algorithm, the ciphering algorithm, if integrity protection and/or ciphering is enabled for a DRB and two parameters, namely the keySetChangeIndicator and the nextHopChainingCount, which are used by the UE to determine the AS security keys upon reconfiguration with sync (with key change), connection re-establishment and/or connection resume.

The integrity protection algorithm is common for SRB1, SRB2, SRB3 (if configured), SRB4 (if configured) and DRBs configured with integrity protection, with the same keyToUse value. The ciphering algorithm is common for SRB1, SRB2, SRB3 (if configured), SRB4 (if configured) and DRBs configured with the same keyToUse value. Neither integrity protection nor ciphering applies for SRB0.

NOTE 0:	All DRBs related to the same PDU session have the same enable/disable setting for ciphering and the same enable/disable setting for integrity protection, as specified in TS 33.501 [11].

RRC integrity protection and ciphering are always activated together, i.e. in one message/procedure. RRC integrity protection and ciphering for SRBs are never de-activated. However, it is possible to switch to a 'NULL' ciphering algorithm (nea0).

The 'NULL' integrity protection algorithm (nia0) is used only for SRBs and for the UE in limited service mode, see TS 33.501 [11] and when used for SRBs, integrity protection is disabled for DRBs. In case the ′NULL' integrity protection algorithm is used, 'NULL' ciphering algorithm is also used.

NOTE 1:	Lower layers discard RRC messages for which the integrity protection check has failed and indicate the integrity protection verification check failure to RRC.

The AS applies four different security keys: one for the integrity protection of RRC signalling (KRRCint), one for the ciphering of RRC signalling (KRRCenc), one for integrity protection of user data (KUPint) and one for the ciphering of user data (KUPenc). All four AS keys are derived from the KgNB key. The KgNB key is based on the KAMF key (as specified in TS 33.501 [11]), which is handled by upper layers.

The integrity protection and ciphering algorithms can only be changed with reconfiguration with sync. The AS keys (KgNB, KRRCint, KRRCenc, KUPint and KUPenc) change upon reconfiguration with sync (if masterKeyUpdate is included), and upon connection re-establishment and connection resume.

For each radio bearer an independent counter (COUNT, as specified in TS 38.323 [5]) is maintained for each direction. For each radio bearer, the COUNT is used as input for ciphering and integrity protection.

It is not allowed to use the same COUNT value more than once for a given security key. As specified in TS 33.501 clause 6.9.4.1 [11], the network is responsible for avoiding reuse of the COUNT with the same RB identity and with the same key, e.g. due to the transfer of large volumes of data, release and establishment of new RBs, and multiple termination point changes for RLC-UM bearers and multiple termination point changes for RLC-AM bearer with SN terminated PDCP re-establishment (COUNT reset) due to SN only full configuration whilst the key stream inputs (i.e. bearer ID, security key) at MN have not been updated. In order to avoid such re-use, the network may e.g. use different RB identities for RB establishments, change the AS security key, or an RRC_CONNECTED to RRC_IDLE/RRC_INACTIVE and then to RRC_CONNECTED transition.

In order to limit the signalling overhead, individual messages/ packets include a short sequence number (PDCP SN, as specified in TS 38.323 [5]). In addition, an overflow counter mechanism is used: the hyper frame number (HFN, as specified in TS 38.323 [5]). The HFN needs to be synchronized between the UE and the network.

For each SRB, the value provided by RRC to lower layers to derive the 5-bit BEARER parameter used as input for ciphering and for integrity protection is the value of the corresponding srb-Identity with the MSBs padded with zeroes.

For a UE provided with an sk-counter, keyToUse indicates whether the UE uses the master key (KgNB) or the secondary key (S-KeNB or S-KgNB) for a particular DRB. The secondary key is derived from the master key and sk-Counter, as defined in TS 33.501[11]. Whenever there is a need to refresh the secondary key, e.g. upon change of MN with KgNB change or to avoid COUNT reuse, the security key update is used (see 5.3.5.7). When the UE is in NR-DC, the network may provide a UE configured with an SCG with an sk-Counter even when no DRB is setup using the secondary key (S-KgNB) in order to allow the configuration of SRB3. The network can also provide the UE with an sk-Counter, even if no SCG is configured, when using SN terminated MCG bearers.

### 5.3.2	Paging

#### 5.3.2.1	General

Figure 5.3.2.1-1: Paging

The purpose of this procedure is:

-	to transmit paging information to a UE in RRC_IDLE or RRC_INACTIVE.

-	to transmit paging information for a L2 U2N Remote UE in RRC_IDLE or RRC_INACTIVE to its serving L2 U2N Relay UE in any RRC state.

#### 5.3.2.2	Initiation

The network initiates the paging procedure by transmitting the Paging message at the UE's paging occasion as specified in TS 38.304 [20]. The network may address multiple UEs within a Paging message by including one PagingRecord for each UE. The network may also include one or multiple TMGI(s) in the Paging message to page UEs for specific MBS multicast session(s).

#### 5.3.2.3	Reception of the Paging message by the UE or PagingRecord by the L2 U2N Remote UE

Upon receiving the Paging message by the UE or receiving PagingRecord from its connected L2 U2N Relay UE by a L2 U2N Remote UE, the UE shall:

1>	if in RRC_IDLE, for each of the PagingRecord, if any, included in the Paging message, or

1>	if in RRC_IDLE, for each of the PagingRecord, if any, included in the UuMessageTransferSidelink message received from the connected L2 U2N Relay UE:

2>	if the ue-Identity included in the PagingRecord matches the UE identity allocated by upper layers:

3>	if upper layers indicate the support of paging cause:

4>	forward the ue-Identity, accessType (if present) and paging cause (if determined) to the upper layers;

3>	else:

4>	forward the ue-Identity and accessType (if present) to the upper layers;

1>	if in RRC_INACTIVE, for each of the PagingRecord, if any, included in the Paging message, or

1>	if in RRC_INACTIVE, for each of the PagingRecord, if any, included in the UuMessageTransferSidelink message received from the connected L2 U2N Relay UE:

2>	if the ue-Identity included in the PagingRecord matches the UE's stored fullI-RNTI:

3>	if the UE is configured by upper layers with Access Identity 1:

4>	initiate the RRC connection resumption procedure according to 5.3.13 with resumeCause set to mps-PriorityAccess;

3>	else if the UE is configured by upper layers with Access Identity 2:

4>	initiate the RRC connection resumption procedure according to 5.3.13 with resumeCause set to mcs-PriorityAccess;

3>	else if the UE is configured by upper layers with one or more Access Identities equal to 11-15:

4>	initiate the RRC connection resumption procedure according to 5.3.13 with resumeCause set to highPriorityAccess;

3>	else:

4>	initiate the RRC connection resumption procedure according to 5.3.13 with resumeCause set to mt-Access;

NOTE:	A MUSIM UE may not initiate the RRC connection resumption procedure, e.g. when it decides not to respond to the Paging message due to UE implementation constraints as specified in TS 24.501 [23].

2>	else if the ue-Identity included in the PagingRecord matches the UE identity allocated by upper layers:

3>	if upper layers indicate the support of paging cause:

4>	forward the ue-Identity, accessType (if present) and paging cause (if determined) to the upper layers;

3>	else:

4>	forward the ue-Identity and accessType (if present) to the upper layers;

3>	perform the actions upon going to RRC_IDLE as specified in 5.3.11 with release cause 'other';

1>	if in RRC_IDLE, for each TMGI included in pagingGroupList, if any, included in the Paging message:

2>	if the UE has joined an MBS session indicated by the TMGI included in the pagingGroupList:

3>	forward the TMGI to the upper layers;

1>	if in RRC_INACTIVE and the UE has joined one or more MBS session(s) indicated by the TMGI(s) included in the pagingGroupList:

2>	if none of the ue-Identity included in any of the PagingRecord, if included in the Paging message, matches the UE identity allocated by upper layers:

3>	initiate the RRC connection resumption procedure according to 5.3.13 with resumeCause set as below:

4>	if the UE is configured by upper layers with Access Identity 1:

5>	set resumeCause to mps-PriorityAccess;

4>	else if the UE is configured by upper layers with Access Identity 2:

5>	set resumeCause to mcs-PriorityAccess;

4>	else if the UE is configured by upper layers with one or more Access Identities equal to 11-15:

5>	set resumeCause to highPriorityAccess;

4>	else:

5>	set resumeCause to mt-Access;

2>	else:

3>	forward the TMGI(s) to the upper layers;

1>	if the UE is acting as a L2 U2N Relay UE, for each of the PagingRecord, if any, included in the Paging message:

2>	if the ue-Identity included in the PagingRecord in the Paging message matches the UE identity in sl-PagingIdentityRemoteUE included in sl-PagingInfo-RemoteUE received in RemoteUEInformationSidelink message from a L2 U2N Remote UE:

3>	inititate the Uu Message transfer in sidelink to that UE as specified in 5.8.9.9;

### 5.3.3	RRC connection establishment

#### 5.3.3.1	General

Figure 5.3.3.1-1: RRC connection establishment, successful

Figure 5.3.3.1-2: RRC connection establishment, network reject

The purpose of this procedure is to establish an RRC connection. RRC connection establishment involves SRB1 establishment. The procedure is also used to transfer the initial NAS dedicated information/ message from the UE to the network.

The network applies the procedure e.g.as follows:

-	When establishing an RRC connection;

-	When UE is resuming or re-establishing an RRC connection, and the network is not able to retrieve or verify the UE context. In this case, UE receives RRCSetup and responds with RRCSetupComplete.

#### 5.3.3.1a	Conditions for establishing RRC Connection for NR sidelink communication/discovery/V2X sidelink communication

For NR sidelink communication/discovery, an RRC connection establishment is initiated only in the following cases:

1>	if configured by upper layers to transmit NR sidelink communication/discovery and related data is available for transmission:

2>	if the frequency on which the UE is configured to transmit NR sidelink communication is included in sl-FreqInfoList within SIB12 provided by the cell on which the UE camps; and if the valid version of SIB12 does not include sl-TxPoolSelectedNormal for the concerned frequency; or

2>	if the frequency on which the UE is configured to transmit NR sidelink discovery is included in sl-FreqInfoList within SIB12 provided by the cell on which the UE camps; and if the valid version of SIB12 includes neither sl-DiscTxPoolSelected nor sl-TxPoolSelectedNormal for the concerned frequency;

For L2 U2N Relay UE in RRC_IDLE, an RRC connection establishment is initiated in the following cases:

1>	if any message is received from a L2 U2N Remote UE via SL-RLC0 as specified in 9.1.1.4 or SL-RLC1 as specified in 9.2.4;

For V2X sidelink communication, an RRC connection is initiated only when the conditions specified for V2X sidelink communication in clause 5.3.3.1a of TS 36.331 [10] are met.

NOTE:	Upper layers initiate an RRC connection (except if the RRC connection is initiated at the L2 U2N Relay UE upon reception of a message from a L2 U2N Remote UE via SL-RLC0 or SL-RLC1). The interaction with NAS is left to UE implementation.

#### 5.3.3.2	Initiation

The UE initiates the procedure when upper layers request establishment of an RRC connection while the UE is in RRC_IDLE and it has acquired essential system information, or for sidelink communication as specified in clause 5.3.3.1a.

The UE shall ensure having valid and up to date essential system information as specified in clause 5.2.2.2 before initiating this procedure.

Upon initiation of the procedure, the UE shall:

1>	if the upper layers provide an Access Category and one or more Access Identities upon requesting establishment of an RRC connection:

2>	perform the unified access control procedure as specified in 5.3.14 using the Access Category and Access Identities provided by upper layers;

3>	if the access attempt is barred, the procedure ends;

1>	if the upper layers provide NSAG information and one or more S-NSSAI(s) triggering the access attempt (TS 23.501 [32] and TS 24.501 [23]):

2>	apply the NSAG with highest NSAG priority among the NSAGs that are included in SIB1 (i.e., in FeatureCombination and in RA-PrioritizationSliceInfo), and that are associated with the S-NSSAI(s) triggering the access attempt, in the Random Access procedure (TS 38.321 [3], clause 5.1);

1>	if the UE is acting as L2 U2N Remote UE:

2>	establish a SRAP entity as specified in TS 38.351 [66], if no SRAP entity has been established;

2>	apply the specified configuration of SL-RLC0 as specified in 9.1.1.4;

2>	apply the SDAP configuration and PDCP configuration as specified in 9.1.1.2 for SRB0;

1>	else:

2>	apply the default L1 parameter values as specified in corresponding physical layer specifications except for the parameters for which values are provided in SIB1;

2>	apply the default MAC Cell Group configuration as specified in 9.2.2;

2>	apply the CCCH configuration as specified in 9.1.1.2;

2>	apply the timeAlignmentTimerCommon included in SIB1;

1>	start timer T300;

1>	initiate transmission of the RRCSetupRequest message in accordance with 5.3.3.3;

#### 5.3.3.3	Actions related to transmission of RRCSetupRequest message

The UE shall set the contents of RRCSetupRequest message as follows:

1>	set the ue-Identity as follows:

2>	if upper layers provide a 5G-S-TMSI:

3>	set the ue-Identity to ng-5G-S-TMSI-Part1;

2>	else:

3>	draw a 39-bit random value in the range 0..239-1 and set the ue-Identity to this value;

NOTE 1:	Upper layers provide the 5G-S-TMSI if the UE is registered in the TA of the current cell.

1>	if the establishment of the RRC connection is the result of release with redirect with mpsPriorityIndication (either in NR or E-UTRAN):

2>	set the establishmentCause to mps-PriorityAccess;

1>	else:

2>	set the establishmentCause in accordance with the information received from upper layers;

NOTE 2:	In case the L2 U2N Relay UE initiates RRC connection establishment triggered by reception of message from a L2 U2N Remote UE via SL-RLC0 or SL-RLC1 as specified in 5.3.3.1a, the L2 U2N Relay UE sets the establishmentCause by implementation, but it can only set the emergency, mps-PriorityAccess, or mcs-PriorityAccess as establishmentCause if the same cause value is in the message received from the L2 U2N Remote UE via SL-RLC0.

1>	if ta-Report is configured with value enabled and the UE supports TA reporting:

2>	indicate TA report initiation to lower layers;

The UE shall submit the RRCSetupRequest message to lower layers for transmission.

If the UE is a RedCap UE and the RedCap-specific initial downlink BWP is not associated with CD-SSB, the UE may continue cell re-selection related measurements as well as cell re-selection evaluation, otherwise the UE shall continue cell re-selection related measurements as well as cell re-selection evaluation. If the conditions for cell re-selection are fulfilled, the UE shall perform cell re-selection as specified in 5.3.3.6.

NOTE 3:	For L2 U2N Remote UE in RRC_IDLE, the cell (re)selection procedure as specified in TS 38.304 [20] and relay (re)selection procedure as specified in 5.8.15.3 are performed independently and up to UE implementation to select either a cell or a L2 U2N Relay UE.

#### 5.3.3.4	Reception of the RRCSetup by the UE

The UE shall perform the following actions upon reception of the RRCSetup:

1>	if the RRCSetup is received in response to an RRCReestablishmentRequest; or

1>	if the RRCSetup is received in response to an RRCResumeRequest or RRCResumeRequest1:

2>	if sdt-MAC-PHY-CG-Config is configured:

3>	instruct the MAC entity to stop the cg-SDT-TimeAlignmentTimer, if it is running;

3>	instruct the MAC entity to start the timeAlignmentTimer associated with the PTAG, if it is not running;

2>	if srs-PosRRC-InactiveConfig is configured:

3>	instruct the MAC entity to stop the inactivePosSRS-TimeAlignmentTimer, if it is running;

2>	discard any stored UE Inactive AS context and suspendConfig;

2>	discard any current AS security context including the KRRCenc key, the KRRCint key, the KUPint key and the KUPenc key;

2>	release radio resources for all established RBs except SRB0 and broadcast MRBs, including release of the RLC entities, of the associated PDCP entities and of SDAP;

2>	release the RRC configuration except for the default L1 parameter values, default MAC Cell Group configuration, CCCH configuration and broadcast MRBs;

2>	indicate to upper layers fallback of the RRC connection;

2>	discard any application layer measurement reports which were not transmitted yet;

2>	inform upper layers about the release of all application layer measurement configurations;

2>	stop timer T380, if running;

1>	perform the cell group configuration procedure in accordance with the received masterCellGroup and as specified in 5.3.5.5;

1>	perform the radio bearer configuration procedure in accordance with the received radioBearerConfig and as specified in 5.3.5.6;

1>	if stored, discard the cell reselection priority information provided by the cellReselectionPriorities or inherited from another RAT;

1>	stop timer T300, T301, T319;

1>	if T319a is running:

2>	stop T319a;

2>	consider SDT procedure is not ongoing;

1>	if T390 is running:

2>	stop timer T390 for all access categories;

2>	perform the actions as specified in 5.3.14.4;

1>	if T302 is running:

2>	stop timer T302;

2>	perform the actions as specified in 5.3.14.4;

1>	stop timer T320, if running;

1>	if the RRCSetup is received in response to an RRCResumeRequest, RRCResumeRequest1 or RRCSetupRequest:

2>	if T331 is running:

3>	stop timer T331;

3>	perform the actions as specified in 5.7.8.3;

2>	enter RRC_CONNECTED;

2>	stop the cell re-selection procedure;

2>	stop relay (re)selection procedure if any for L2 U2N Remote UE;

1>	consider the current cell to be the PCell;

1>	perform the L2 U2N Remote UE configuration procedure in accordance with the received sl-L2RemoteUE-Config as specified in 5.3.5.16;

1>	perform the sidelink dedicated configuration procedure in accordance with the received sl-ConfigDedicatedNR as specified in 5.3.5.14;

1>	if the UE has radio link failure or handover failure information available in VarRLF-Report and if the RPLMN is included in plmn-IdentityList stored in VarRLF-Report:

2>	if reconnectCellId in VarRLF-Report is not set after failing to perform reestablishment:

3>	if the UE supports RLF-Report for conditional handover and if choCellId in VarRLF-Report is set:

4>	set timeUntilReconnection in VarRLF-Report to the time that elapsed since the radio link failure or handover failure experienced in the failedPCellId stored in VarRLF-Report;

3>	else:

4>	set timeUntilReconnection in VarRLF-Report to the time that elapsed since the last radio link failure or handover failure;

3>	set nrReconnectCellId in reconnectCellId in VarRLF-Report to the global cell identity and the tracking area code of the PCell;

1>	if the UE supports RLF report for inter-RAT MRO NR as defined in TS 36.306 [62], and if the UE has radio link failure or handover failure information available in VarRLF-Report of TS 36.331 [10] and if the RPLMN is included in plmn-IdentityList stored in VarRLF-Report of TS 36.331 [10]:

2>	if reconnectCellId in VarRLF-Report of TS 36.331[10] is not set after failing to perform reestablishment:

3>	set timeUntilReconnection in VarRLF-Report of TS 36.331[10] to the time that elapsed since the last radio link failure or handover failure in LTE;

3>	set nrReconnectCellId in reconnectCellId in VarRLF-Report of TS 36.331[10] to the global cell identity and the tracking area code of the PCell;

1>	set the content of RRCSetupComplete message as follows:

2>	if upper layers provide a 5G-S-TMSI:

3>	if the RRCSetup is received in response to an RRCSetupRequest:

4>	set the ng-5G-S-TMSI-Value to ng-5G-S-TMSI-Part2;

3>	else:

4>	set the ng-5G-S-TMSI-Value to ng-5G-S-TMSI;

2>	if upper layers selected an SNPN or a PLMN and in case of PLMN UE is either allowed or instructed to access the PLMN via a cell for which at least one CAG ID is broadcast:

3>	set the selectedPLMN-Identity from the npn-IdentityInfoList;

2>	else:

3>	set the selectedPLMN-Identity to the PLMN selected by upper layers from the plmn-IdentityInfoList;

2>	if upper layers provide the 'Registered AMF':

3>	include and set the registeredAMF as follows:

4>	if the PLMN identity of the 'Registered AMF' is different from the PLMN selected by the upper layers:

5>	include the plmnIdentity in the registeredAMF and set it to the value of the PLMN identity in the 'Registered AMF' received from upper layers;

4>	set the amf-Identifier to the value received from upper layers;

3>	include and set the guami-Type to the value provided by the upper layers;

2>	if upper layers provide one or more S-NSSAI (see TS 23.003 [21]):

3>	include the s-NSSAI-List and set the content to the values provided by the upper layers;

2>	if upper layers provide onboarding request indication:

3>	include the onboardingRequest;

2>	set the dedicatedNAS-Message to include the information received from upper layers;

2>	if connecting as an IAB-node:

3>	include the iab-NodeIndication;

2>	if the SIB1 contains idleModeMeasurementsNR and the UE has NR idle/inactive measurement information concerning cells other than the PCell available in VarMeasIdleReport; or

2>	if the SIB1 contains idleModeMeasurementsEUTRA and the UE has E-UTRA idle/inactive measurement information available in VarMeasIdleReport:

3>	include the idleMeasAvailable;

2>	if the UE has logged measurements available for NR and if the RPLMN is included in plmn-IdentityList stored in VarLogMeasReport:

3>	include the logMeasAvailable in the RRCSetupComplete message;

3>	if Bluetooth measurement results are included in the logged measurements the UE has available for NR:

4>	include the logMeasAvailableBT in the RRCSetupComplete message;

3>	if WLAN measurement results are included in the logged measurements the UE has available for NR:

4>	include the logMeasAvailableWLAN in the RRCSetupComplete message;

2>	if the sigLoggedMeasType in VarLogMeasReport is included:

3>	if T330 timer is running and the logged measurements configuration is for NR:

4>	set sigLogMeasConfigAvailable to true in the RRCSetupComplete message;

3>	else:

4>	if the UE has logged measurements available for NR:

5>	set sigLogMeasConfigAvailable to false in the RRCSetupComplete message;

2>	if the UE has connection establishment failure or connection resume failure information available in VarConnEstFailReport or VarConnEstFailReportList and if the RPLMN is equal to plmn-Identity stored in VarConnEstFailReport or in at least one of the entries of VarConnEstFailReportList:

3>	include connEstFailInfoAvailable in the RRCSetupComplete message;

2>	if the UE has radio link failure or handover failure information available in VarRLF-Report and if the RPLMN is included in plmn-IdentityList stored in VarRLF-Report, or

2>	if the UE has radio link failure or handover failure information available in VarRLF-Report of TS 36.331 [10], and if the UE is capable of cross-RAT RLF reporting and if the RPLMN is included in plmn-IdentityList stored in VarRLF-Report of TS 36.331 [10]:

3>	include rlf-InfoAvailable in the RRCSetupComplete message;

2>	if the UE has successful handover information available in VarSuccessHO-Report and if the RPLMN is included in plmn-IdentityList stored in VarSuccessHO-Report:

3>	include successHO-InfoAvailable in the RRCSetupComplete message;

2>	if the UE supports storage of mobility history information and the UE has mobility history information available in VarMobilityHistoryReport:

3>	include the mobilityHistoryAvail in the RRCSetupComplete message;

2>	if the UE supports uplink RRC message segmentation of UECapabilityInformation:

3>	may include the ul-RRC-Segmentation in the RRCSetupComplete message;

2>	if the RRCSetup is received in response to an RRCResumeRequest, RRCResumeRequest1 or RRCSetupRequest:

3>	if speedStateReselectionPars is configured in the SIB2:

4>	include the mobilityState in the RRCSetupComplete message and set it to the mobility state (as specified in TS 38.304 [20]) of the UE just prior to entering RRC_CONNECTED state;

2>	if the SIB1 contains musim-CapabilityRestriction and the UE capability is restricted for MUSIM purpose:

3>	may include the musim-CapabilityRestrictionIndication in the RRCSetupComplete message;

1>	submit the RRCSetupComplete message to lower layers for transmission, upon which the procedure ends.

#### 5.3.3.5	Reception of the RRCReject by the UE

The UE shall:

1>	perform the actions as specified in 5.3.15;

#### 5.3.3.6	Cell re-selection or cell selection or relay (re)selection while T390, T300 or T302 is running (UE in RRC_IDLE)

The UE shall:

1>	if cell reselection occurs while T300 or T302 is running; or

1>	if relay reselection occurs while T300 is running; or

1>	if cell changes due to relay reselection while T302 is running:

2>	perform the actions upon going to RRC_IDLE as specified in 5.3.11 with release cause 'RRC connection failure';

1>	else:

2>	if cell selection or reselection occurs while T390 is running; or

2>	cell change due to relay selection or reselection occurs while T390 is running:

3>	stop T390 for all access categories;

3>	perform the actions as specified in 5.3.14.4.

#### 5.3.3.7	T300 expiry

The UE shall:

1>	if timer T300 expires:

2>	reset MAC, release the MAC configuration and re-establish RLC for all RBs that are established (except broadcast MRBs);

2>	if the UE supports RRC Connection Establishment failure with temporary offset and the T300 has expired a consecutive connEstFailCount times on the same cell for which connEstFailureControl is included in SIB1:

3>	for a period as indicated by connEstFailOffsetValidity:

4>	use connEstFailOffset for the parameter Qoffsettemp for the concerned cell when performing cell selection and reselection according to TS 38.304 [20] and TS 36.304 [27];

NOTE 1:	When performing cell selection, if no suitable or acceptable cell can be found, it is up to UE implementation whether to stop using connEstFailOffset for the parameter Qoffsettemp during connEstFailOffsetValidity for the concerned cell.

2>	if the UE supports multiple CEF report:

3>	if the UE has connection establishment failure information or connection resume failure information available in VarConnEstFailReport and if the RPLMN is equal to plmn-identity stored in VarConnEstFailReport; and

3>	if the cell identity of current cell is not equal to the cell identity stored in measResultFailedCell in VarConnEstFailReport and if the maxCEFReport-r17 has not been reached:

4>	append the VarConnEstFailReport as a new entry in the VarConnEstFailReportList;

2>	if the UE has connection establishment failure information or connection resume failure information available in VarConnEstFailReport and if the RPLMN is not equal to plmn-identity stored in VarConnEstFailReport; or

2>	if the cell identity of current cell is not equal to the cell identity stored in measResultFailedCell in VarConnEstFailReport:

3>	reset the numberOfConnFail to 0;

2>	if the UE supports multiple CEF report and if the UE has connection establishment failure informatoin or connection resume failure information available in VarConnEstFailReportList and if the RPLMN is not equal to plmn-identity stored in any entry of VarConnEstFailReportList:

3>	clear the content included in VarConnEstFailReportList;

2>	clear the content included in VarConnEstFailReport except for the numberOfConnFail, if any;

2>	store the following connection establishment failure information in the VarConnEstFailReport by setting its fields as follows:

3>	set the plmn-Identity to the PLMN selected by upper layers (see TS 24.501 [23]) from the PLMN(s) included in the plmn-IdentityInfoList in SIB1;

3>	set the measResultFailedCell to include the global cell identity, tracking area code, the cell level and SS/PBCH block level RSRP, and RSRQ, and SS/PBCH block indexes, of the failed cell based on the available SSB measurements collected up to the moment the UE detected connection establishment failure;

3>	if available, set the measResultNeighCells, in order of decreasing ranking-criterion as used for cell re-selection, to include neighbouring cell measurements for at most the following number of neighbouring cells: 6 intra-frequency and 3 inter-frequency neighbours per frequency as well as 3 inter-RAT neighbours, per frequency/ set of frequencies per RAT and according to the following:

4>	for each neighbour cell included, include the optional fields that are available;

NOTE 2:	The UE includes the latest results of the available measurements as used for cell reselection evaluation, which are performed in accordance with the performance requirements as specified in TS 38.133 [14].

3>	if available, set the locationInfo as follows:

4>	if available, set the commonLocationInfo to include the detailed location information;

4>	if available, set the bt-LocationInfo to include the Bluetooth measurement results, in order of decreasing RSSI for Bluetooth beacons;

4>	if available, set the wlan-LocationInfo to include the WLAN measurement results, in order of decreasing RSSI for WLAN APs;

4>	if available, set the sensor-LocationInfo to include the sensor measurement results as follows;

5>	if available, include the sensor-MeasurementInformation;

5>	if available, include the sensor-MotionInformation;

NOTE 3:	Which location information related configuration is used by the UE to make the locationInfo available for inclusion in the VarConnEstFailReport is left to UE implementation.

3>	set perRAInfoList to indicate the performed random access procedure related information as specified in 5.7.10.5;

3>	if the numberOfConnFail is smaller than 8:

4>	increment the numberOfConnFail by 1;

2>	inform upper layers about the failure to establish the RRC connection, upon which the procedure ends;

The UE may discard the connection establishment failure or connection resume failure information, i.e. release the UE variable VarConnEstFailReport and the UE variable VarConnEstFailReportList, 48 hours after the last connection establishment failure is detected.

The L2 U2N Relay UE either indicates to upper layers (to trigger PC5 unicast link release) or sends Notification message to the connected L2 U2N Remote UE(s) in accordance with 5.8.9.10.

#### 5.3.3.8	Abortion of RRC connection establishment

If upper layers abort the RRC connection establishment procedure, due to a NAS procedure being aborted as specified in TS 24.501 [23], while the UE has not yet entered RRC_CONNECTED, the UE shall:

1>	stop timer T300, if running;

1>	reset MAC, release the MAC configuration and re-establish RLC for all RBs that are established (except broadcast MRBs).

The L2 U2N Relay UE either indicates to upper layers (to trigger PC5 unicast link release) or sends Notification message to the connected L2 U2N Remote UE(s) in accordance with 5.8.9.10.

The L2 U2N Remote UE indicates to upper layers to trigger PC5 unicast link release with its connected L2 U2N Relay UE.

### 5.3.4	Initial AS security activation

#### 5.3.4.1	General

Figure 5.3.4.1-1: Security mode command, successful

Figure 5.3.4.1-2: Security mode command, failure

The purpose of this procedure is to activate AS security upon RRC connection establishment.

#### 5.3.4.2	Initiation

The network initiates the security mode command procedure to a UE in RRC_CONNECTED. Moreover, the network applies the procedure as follows:

-	when only SRB1 is established, i.e. prior to establishment of SRB2, multicast MRBs and/ or DRBs.

#### 5.3.4.3	Reception of the SecurityModeCommand by the UE

The UE shall:

1>	derive the KgNB key, as specified in TS 33.501 [11];

1>	derive the KRRCint key associated with the integrityProtAlgorithm indicated in the SecurityModeCommand message, as specified in TS 33.501 [11];

1>	request lower layers to verify the integrity protection of the SecurityModeCommand message, using the algorithm indicated by the integrityProtAlgorithm as included in the SecurityModeCommand message and the KRRCint key;

1>	if the SecurityModeCommand message passes the integrity protection check:

2>	derive the KRRCenc key and the KUPenc key associated with the cipheringAlgorithm indicated in the SecurityModeCommand message, as specified in TS 33.501 [11];

2>	derive the KUPint key associated with the integrityProtAlgorithm indicated in the SecurityModeCommand message, as specified in TS 33.501 [11];

2>	configure lower layers to apply SRB integrity protection using the indicated algorithm and the KRRCint key immediately, i.e. integrity protection shall be applied to all subsequent messages received and sent by the UE, including the SecurityModeComplete message;

2>	configure lower layers to apply SRB ciphering using the indicated algorithm, the KRRCenc keyafter completing the procedure, i.e. ciphering shall be applied to all subsequent messages received and sent by the UE, except for the SecurityModeComplete message which is sent unciphered;

2>	consider AS security to be activated;

2>	submit the SecurityModeComplete message to lower layers for transmission, upon which the procedure ends;

1>	else:

2>	continue using the configuration used prior to the reception of the SecurityModeCommand message, i.e. neither apply integrity protection nor ciphering.

2>	submit the SecurityModeFailure message to lower layers for transmission, upon which the procedure ends.

### 5.3.5	RRC reconfiguration

#### 5.3.5.1	General

Figure 5.3.5.1-1: RRC reconfiguration, successful

Figure 5.3.5.1-2: RRC reconfiguration, failure

The purpose of this procedure is to modify an RRC connection, e.g. to establish/modify/release RBs/BH RLC channels/Uu Relay RLC channels/PC5 Relay RLC channels, to perform reconfiguration with sync, to setup/modify/release measurements, to add/modify/release SCells and cell groups, to add/modify/release conditional handover configuration, to add/modify/release conditional PSCell change or conditional PSCell addition configuration. As part of the procedure, NAS dedicated information may be transferred from the Network to the UE.

RRC reconfiguration to perform reconfiguration with sync includes, but is not limited to, the following cases:

-	reconfiguration with sync and security key refresh, involving RA to the PCell/PSCell, MAC reset, refresh of security and re-establishment of RLC and PDCP triggered by explicit indicators;

-	reconfiguration with sync but without security key refresh, involving RA to the PCell/PSCell, MAC reset and RLC re-establishment and PDCP data recovery (for AM DRB or AM MRB) triggered by explicit indicators.

-	reconfiguration with sync for DAPS and security key refresh, involving RA to the target PCell, establishment of target MAC, and

-	for non-DAPS bearer: refresh of security and re-establishment of RLC and PDCP triggered by explicit indicators;

-	for DAPS bearer: establishment of RLC for the target PCell, refresh of security and reconfiguration of PDCP to add the ciphering function, the integrity protection function and ROHC function of the target PCell;

-	for SRB: refresh of security and establishment of RLC and PDCP for the target PCell;

-	reconfiguration with sync for DAPS but without security key refresh, involving RA to the target PCell, establishment of target MAC, and

-	for non-DAPS bearer: RLC re-establishment and PDCP data recovery (for AM DRB or AM MRB) triggered by explicit indicators.

-	for DAPS bearer: establishment of RLC for target PCell, reconfiguration of PDCP to add the ciphering function, the integrity protection function and ROHC function of the target PCell;

-	for SRB: establishment of RLC and PDCP for the target PCell.

-	reconfiguration with sync for direct-to-indirect path switch, not involving RA at target side, involving re-establishment of PDCP /PDCP data recovery (for AM DRB) triggered by explicit indicators.

In (NG)EN-DC and NR-DC, SRB3 can be used for measurement configuration and reporting, for UE assistance (re-)configuration and reporting for power savings, for IP address (re-)configuration and reporting for IAB-nodes, to (re-)configure MAC, RLC, BAP, physical layer and RLF timers and constants of the SCG configuration, and to reconfigure PDCP for DRBs associated with the S-KgNB or SRB3, and to reconfigure SDAP for DRBs associated with S-KgNB in NGEN-DC and NR-DC, and to add/modify/release conditional PSCell change configuration, provided that the (re-)configuration does not require any MN involvement, and to transmit RRC messages between the MN and the UE during fast MCG link recovery. In (NG)EN-DC and NR-DC, only measConfig, radioBearerConfig, conditionalReconfiguration, bap-Config, iab-IP-AddressConfigurationList, otherConfig and/or secondaryCellGroup are included in RRCReconfiguration received via SRB3, except when RRCReconfiguration is received within DLInformationTransferMRDC.

#### 5.3.5.2	Initiation

The Network may initiate the RRC reconfiguration procedure to a UE in RRC_CONNECTED. The Network applies the procedure as follows:

-	the establishment of RBs (other than SRB1, that is established during RRC connection establishment) is performed only when AS security has been activated;

-	the establishment of BH RLC Channels for IAB is performed only when AS security has been activated;

-	the establishment of Uu Relay RLC channels and PC5 Relay RLC channels (other than SL-RLC0 and SL-RLC1) for L2 U2N Relay UE is performed only when AS security has been activated, and the establishment of PC5 Relay RLC channels for L2 U2N Remote UE (other than SL-RLC0 and SL-RLC1) is performed only when AS security has been activated;

-	the addition of Secondary Cell Group and SCells is performed only when AS security has been activated;

-	the reconfigurationWithSync is included in secondaryCellGroup only when at least one RLC bearer or BH RLC channel is setup in SCG;

-	the reconfigurationWithSync is included in masterCellGroup only when AS security has been activated, and SRB2 with at least one DRB or multicast MRB or, for IAB, SRB2, are setup and not suspended;

-	the conditionalReconfiguration for CPC is included only when at least one RLC bearer is setup in SCG;

-	the conditionalReconfiguration for CHO or CPA is included only when AS security has been activated, and SRB2 with at least one DRB or multicast MRB or, for IAB, SRB2, are setup and not suspended.

#### 5.3.5.3	Reception of an RRCReconfiguration by the UE

The UE shall perform the following actions upon reception of the RRCReconfiguration, or upon execution of the conditional reconfiguration (CHO, CPA or CPC):

1>	if the RRCReconfiguration is applied due to a conditional reconfiguration execution upon cell selection performed while timer T311 was running, as defined in 5.3.7.3:

2>	remove all the entries within the MCG and the SCG VarConditionalReconfig, if any;

1>	if the RRCReconfiguration includes the daps-SourceRelease:

2>	reset the source MAC and release the source MAC configuration;

2>	for each DAPS bearer:

3>	release the RLC entity or entities as specified in TS 38.322 [4], clause 5.1.3, and the associated logical channel for the source SpCell;

3>	reconfigure the PDCP entity to release DAPS as specified in TS 38.323 [5];

2>	for each SRB:

3>	release the PDCP entity for the source SpCell;

3>	release the RLC entity as specified in TS 38.322 [4], clause 5.1.3, and the associated logical channel for the source SpCell;

2>	release the physical channel configuration for the source SpCell;

2>	discard the keys used in the source SpCell (the KgNB key, the KRRCenc key, the KRRCint key, the KUPint key and the KUPenc key), if any;

1>	if the RRCReconfiguration is received via other RAT (i.e., inter-RAT handover to NR):

2>	if the RRCReconfiguration does not include the fullConfig and the UE is connected to 5GC (i.e., delta signalling during intra 5GC handover):

3>	re-use the source RAT SDAP and PDCP configurations if available (i.e., current SDAP/PDCP configurations for all RBs from source E-UTRA RAT prior to the reception of the inter-RAT HO RRCReconfiguration message);

1>	else:

2>	if the RRCReconfiguration includes the fullConfig:

3>	perform the full configuration procedure as specified in 5.3.5.11;

1>	if the RRCReconfiguration includes the masterCellGroup:

2>	perform the cell group configuration for the received masterCellGroup according to 5.3.5.5;

1>	if the RRCReconfiguration includes the masterKeyUpdate:

2>	perform AS security key update procedure as specified in 5.3.5.7;

1>	if the RRCReconfiguration includes the sk-Counter:

2>	perform security key update procedure as specified in 5.3.5.7;

1>	if the RRCReconfiguration includes the secondaryCellGroup:

2>	perform the cell group configuration for the SCG according to 5.3.5.5;

1>	if the RRCReconfiguration includes the mrdc-SecondaryCellGroupConfig:

2>	if the mrdc-SecondaryCellGroupConfig is set to setup:

3>	if the mrdc-SecondaryCellGroupConfig includes mrdc-ReleaseAndAdd:

4>	perform MR-DC release as specified in clause 5.3.5.10;

3>	if the received mrdc-SecondaryCellGroup is set to nr-SCG:

4>	perform the RRC reconfiguration according to 5.3.5.3 for the RRCReconfiguration message included in nr-SCG;

3>	if the received mrdc-SecondaryCellGroup is set to eutra-SCG:

4>	perform the RRC connection reconfiguration as specified in TS 36.331 [10], clause 5.3.5.3 for the RRCConnectionReconfiguration message included in eutra-SCG;

2>	else (mrdc-SecondaryCellGroupConfig is set to release):

3>	perform MR-DC release as specified in clause 5.3.5.10;

1>	if the RRCReconfiguration message includes the radioBearerConfig:

2>	perform the radio bearer configuration according to 5.3.5.6;

1>	if the RRCReconfiguration message includes the radioBearerConfig2:

2>	perform the radio bearer configuration according to 5.3.5.6;

1>	if the RRCReconfiguration message includes the measConfig:

2>	perform the measurement configuration procedure as specified in 5.5.2;

1>	if the RRCReconfiguration message includes the dedicatedNAS-MessageList:

2>	forward each element of the dedicatedNAS-MessageList to upper layers in the same order as listed;

1>	if the RRCReconfiguration message includes the dedicatedSIB1-Delivery:

2>	perform the action upon reception of SIB1 as specified in 5.2.2.4.2;

NOTE 0:	If this RRCReconfiguration is associated to the MCG and includes reconfigurationWithSync in spCellConfig and dedicatedSIB1-Delivery, the UE initiates (if needed) the request to acquire required SIBs, according to clause 5.2.2.3.5, only after the random access procedure towards the target SpCell is completed.

1>	if the RRCReconfiguration message includes the dedicatedSystemInformationDelivery:

2>	perform the action upon reception of System Information as specified in 5.2.2.4;

2>	if all the SIB(s) and/or posSIB(s) requested in DedicatedSIBRequest message have been acquired:

3>	stop timer T350, if running;

1>	if the RRCReconfiguration message includes the dedicatedPosSysInfoDelivery:

2>	perform the action upon reception of the contained posSIB(s), as specified in clause 5.2.2.4.16;

2>	if all the SIB(s) and/or posSIB(s) requested in DedicatedSIBRequest message have been acquired:

3>	stop timer T350, if running;

1>	if the RRCReconfiguration message includes the otherConfig:

2>	perform the other configuration procedure as specified in 5.3.5.9;

1>	if the RRCReconfiguration message includes the bap-Config:

2>	perform the BAP configuration procedure as specified in 5.3.5.12;

1>	if the RRCReconfiguration message includes the iab-IP-AddressConfigurationList:

2>	if iab-IP-AddressToReleaseList is included:

3>	perform release of IP address as specified in 5.3.5.12a.1.1;

2>	if iab-IP-AddressToAddModList is included:

3>	perform IAB IP address addition/update as specified in 5.3.5.12a.1.2;

1>	if the RRCReconfiguration message includes the conditionalReconfiguration:

2>	perform conditional reconfiguration as specified in 5.3.5.13;

1>	if the RRCReconfiguration message includes the needForGapsConfigNR:

2>	if needForGapsConfigNR is set to setup:

3>	consider itself to be configured to provide the measurement gap requirement information of NR target bands;

2>	else:

3>	consider itself not to be configured to provide the measurement gap requirement information of NR target bands;

1>	if the RRCReconfiguration message includes the needForGapNCSG-ConfigNR:

2>	if needForGapNCSG-ConfigNR is set to setup:

3>	consider itself to be configured to provide the measurement gap and NCSG requirement information of NR target bands;

2>	else:

3>	consider itself not to be configured to provide the measurement gap and NCSG requirement information of NR target bands;

1>	if the RRCReconfiguration message includes the needForGapNCSG-ConfigEUTRA:

2>	if needForGapNCSG-ConfigEUTRA is set to setup:

3>	consider itself to be configured to provide the measurement gap and NCSG requirement information of E-UTRA target bands;

2>	else:

3>	consider itself not to be configured to provide the measurement gap and NCSG requirement information of E-UTRA target bands;

1>	if the RRCReconfiguration message includes the onDemandSIB-Request:

2>	if onDemandSIB-Request is set to setup:

3>	consider itself to be configured to request SIB(s) or posSIB(s) in RRC_CONNECTED in accordance with clause 5.2.2.3.5;

2>	else:

3>	consider itself not to be configured to request SIB(s) or posSIB(s) in RRC_CONNECTED in accordance with clause 5.2.2.3.5;

3>	stop timer T350, if running;

1>	if the RRCReconfiguration message includes the sl-ConfigDedicatedNR:

2>	perform the sidelink dedicated configuration procedure as specified in 5.3.5.14;

NOTE 0a:	If the sl-ConfigDedicatedNR was received embedded within an E-UTRA RRCConnectionReconfiguration message, the UE does not build an NR RRCReconfigurationComplete message for the received sl-ConfigDedicatedNR.

1>	if the RRCReconfiguration message includes the sl-L2RelayUE-Config:

2>	perform the L2 U2N Relay UE configuration procedure as specified in 5.3.5.15;

1>	if the RRCReconfiguration message includes the sl-L2RemoteUE-Config:

2>	perform the L2 U2N Remote UE configuration procedure as specified in 5.3.5.16;

1>	if the RRCReconfiguration message includes the dedicatedPagingDelivery:

2>	perform the Paging message reception procedure as specified in 5.3.2.3;

1>	if the RRCReconfiguration message includes the sl-ConfigDedicatedEUTRA-Info:

2>	perform related procedures for V2X sidelink communication in accordance with TS 36.331 [10], clause 5.3.10 and clause 5.5.2;

1>	if the RRCReconfiguration message includes the ul-GapFR2-Config:

2>	perform the FR2 UL gap configuration procedure as specified in 5.3.5.13c;

1>	if the RRCReconfiguration message includes the musim-GapConfig:

2>	perform the MUSIM gap configuration procedure as specified in 5.3.5.9a;

1>	if the RRCReconfiguration message includes the appLayerMeasConfig:

2>	perform the application layer measurement configuration procedure as specified in 5.3.5.13d;

1>	if the RRCReconfiguration message includes the ue-TxTEG-RequestUL-TDOA-Config:

2>	if ue-TxTEG-RequestUL-TDOA-Config is set to setup:

3>	perform the UE positioning assistance information procedure as specified in 5.7.14;

2>	else:

3>	release the configuration of UE positioning assistance information;

1>	set the content of the RRCReconfigurationComplete message as follows:

2>	if the RRCReconfiguration includes the masterCellGroup containing the reportUplinkTxDirectCurrent:

3>	include the uplinkTxDirectCurrentList for each MCG serving cell with UL;

3>	include uplinkDirectCurrentBWP-SUL for each MCG serving cell configured with SUL carrier, if any, within the uplinkTxDirectCurrentList;

2>	if the RRCReconfiguration includes the masterCellGroup containing the reportUplinkTxDirectCurrentTwoCarrier:

3>	include in the uplinkTxDirectCurrentTwoCarrierList the list of uplink Tx DC locations for the configured intra-band uplink carrier aggregation in the MCG;

2>	if the RRCReconfiguration includes the masterCellGroup containing the reportUplinkTxDirectCurrentMoreCarrier:

3>	include in the uplinkTxDirectCurrentMoreCarrierList the list of uplink Tx DC locations for the configured intra-band uplink carrier aggregation in the MCG;

2>	if the RRCReconfiguration includes the secondaryCellGroup containing the reportUplinkTxDirectCurrent:

3>	include the uplinkTxDirectCurrentList for each SCG serving cell with UL;

3>	include uplinkDirectCurrentBWP-SUL for each SCG serving cell configured with SUL carrier, if any, within the uplinkTxDirectCurrentList;

2>	if the RRCReconfiguration includes the secondaryCellGroup containing the reportUplinkTxDirectCurrentTwoCarrier:

3>	include in the uplinkTxDirectCurrentTwoCarrierList the list of uplink Tx DC locations for the configured intra-band uplink carrier aggregation in the SCG;

2>	if the RRCReconfiguration includes the secondaryCellGroup containing the reportUplinkTxDirectCurrentMoreCarrier:

3>	include in the uplinkTxDirectCurrentMoreCarrierList the list of uplink Tx DC locations for the configured intra-band uplink carrier aggregation in the SCG;

NOTE 0b:	The UE does not expect that the reportUplinkTxDirectCurrentTwoCarrier or reportUplinkTxDirectCurrentMoreCarrier is received in both masterCellGroup and in secondaryCellGroup. Network only configures at most one of reportUplinkTxDirectCurrent, reportUplinkTxDirectCurrentTwoCarrier or reportUplinkTxDirectCurrentMoreCarrier in one RRC message.

2>	if the RRCReconfiguration message includes the mrdc-SecondaryCellGroupConfig with mrdc-SecondaryCellGroup set to eutra-SCG:

3>	include in the eutra-SCG-Response the E-UTRA RRCConnectionReconfigurationComplete message in accordance with TS 36.331 [10] clause 5.3.5.3;

2> if the RRCReconfiguration message includes the mrdc-SecondaryCellGroupConfig with mrdc-SecondaryCellGroup set to nr-SCG:

3>	include in the nr-SCG-Response the SCG RRCReconfigurationComplete message;

3>	if the RRCReconfiguration message is applied due to conditional reconfiguration execution and the RRCReconfiguration message does not include the reconfigurationWithSync in the masterCellGroup:

4>	include in the selectedCondRRCReconfig the condReconfigId for the selected cell of conditional reconfiguration execution;

2>	if the RRCReconfiguration includes the reconfigurationWithSync in spCellConfig of an MCG:

3>	if the UE has logged measurements available for NR and if the RPLMN is included in plmn-IdentityList stored in VarLogMeasReport:

4>	include the logMeasAvailable in the RRCReconfigurationComplete message;

4>	if Bluetooth measurement results are included in the logged measurements the UE has available for NR:

5>	include the logMeasAvailableBT in the RRCReconfigurationComplete message;

4>	if WLAN measurement results are included in the logged measurements the UE has available for NR:

5>	include the logMeasAvailableWLAN in the RRCReconfigurationComplete message;

3>	if the sigLoggedMeasType in VarLogMeasReport is included:

4>	if T330 timer is running and the logged measurements configuration is for NR:

5>	set sigLogMeasConfigAvailable to true in the RRCReconfigurationComplete message;

4>	else:

5>	if the UE has logged measurements available for NR:

6>	set sigLogMeasConfigAvailable to false in the RRCReconfigurationComplete message;

3>	if the UE has connection establishment failure or connection resume failure information available in VarConnEstFailReport or VarConnEstFailReportList and if the RPLMN is equal to plmn-Identity stored in VarConnEstFailReport or in at least one of the entries of VarConnEstFailReportList:

4>	include connEstFailInfoAvailable in the RRCReconfigurationComplete message;

3>	if the UE has radio link failure or handover failure information available in VarRLF-Report and if the RPLMN is included in plmn-IdentityList stored in VarRLF-Report; or

3>	if the UE has radio link failure or handover failure information available in VarRLF-Report of TS 36.331 [10] and if the UE is capable of cross-RAT RLF reporting and if the RPLMN is included in plmn-IdentityList stored in VarRLF-Report of TS 36.331 [10]:

4>	include rlf-InfoAvailable in the RRCReconfigurationComplete message;

3>	if the UE was configured with successHO-Config when connected to the source PCell; and

3>	if the applied RRCReconfiguration is not due to a conditional reconfiguration execution upon cell selection performed while timer T311 was running, as defined in 5.3.7.3:

4>	perform the actions for the successful handover report determination as specified in clause 5.7.10.6, upon successfully completing the Random Access procedure triggered for the reconfigurationWithSync in spCellConfig of the MCG;

3>	if the UE has successful handover information available in VarSuccessHO-Report and if the RPLMN is included in plmn-IdentityList stored in VarSuccessHO-Report:

4>	include successHO-InfoAvailable in the RRCReconfigurationComplete message;

2>	if the RRCReconfiguration message was received via SRB1, but not within mrdc-SecondaryCellGroup or E-UTRA RRCConnectionReconfiguration or E-UTRA RRCConnectionResume:

3>	if the UE is configured to provide the measurement gap requirement information of NR target bands:

4>	if the RRCReconfiguration message includes the needForGapsConfigNR; or

4>	if the NeedForGapsInfoNR information is changed compared to last time the UE reported this information:

5>	include the NeedForGapsInfoNR and set the contents as follows:

6>	include intraFreq-needForGap and set the gap requirement information of intra-frequency measurement for each NR serving cell;

6>	if requestedTargetBandFilterNR is configured:

7>	for each supported NR band that is also included in requestedTargetBandFilterNR, include an entry in interFreq-needForGap and set the gap requirement information for that band;

6>	else:

7>	include an entry in interFreq-needForGap and set the corresponding gap requirement information for each supported NR band;

3>	if the UE is configured to provide the measurement gap and NCSG requirement information of NR target bands:

4>	if the RRCReconfiguration message includes the needForGapNCSG-ConfigNR; or

4>	if the needForGapNCSG-InfoNR information is changed compared to last time the UE reported this information:

5>	include the NeedForGapNCSG-InfoNR and set the contents as follows:

6>	include intraFreq-needForNCSG and set the gap and NCSG requirement information of intra-frequency measurement for each NR serving cell;

6>	if requestedTargetBandFilterNCSG-NR is configured:

7>	for each supported NR band included in requestedTargetBandFilterNCSG-NR, include an entry in interFreq-needForNCSG and set the NCSG requirement information for that band;

6>	else:

7>	include an entry for each supported NR band in interFreq-needForNCSG and set the corresponding NCSG requirement information;

3>	if the UE is configured to provide the measurement gap and NCSG requirement information of E-UTRA target bands:

4>	if the RRCReconfiguration message includes the needForGapNCSG-ConfigEUTRA; or

4>	if the needForGapNCSG-InfoEUTRA information is changed compared to last time the UE reported this information:

5>	include the NeedForGapNCSG-InfoEUTRA and set the contents as follows:

6>	if requestedTargetBandFilterNCSG-EUTRA is configured, for each supported E-UTRA band included in requestedTargetBandFilterNCSG-EUTRA, include an entry in needForNCSG-EUTRA and set the NCSG requirement information for that band; otherwise, include an entry for each supported E-UTRA band in needForNCSG-EUTRA and set the corresponding NCSG requirement information;

1>	if the UE is configured with E-UTRA nr-SecondaryCellGroupConfig (UE in (NG)EN-DC):

2>	if the RRCReconfiguration message was received via E-UTRA SRB1 as specified in TS 36.331 [10]; or

2>	if the RRCReconfiguration message was received via E-UTRA RRC message RRCConnectionReconfiguration within MobilityFromNRCommand (handover from NR standalone to (NG)EN-DC);

3>	if the RRCReconfiguration is applied due to a conditional reconfiguration execution for CPC which is configured via conditionalReconfiguration contained in nr-SecondaryCellGroupConfig specified in TS 36.331 [10]:

4>	submit the RRCReconfigurationComplete message via the E-UTRA MCG embedded in E-UTRA RRC message ULInformationTransferMRDC as specified in TS 36.331 [10], clause 5.6.2a.

3>	else if the RRCReconfiguration message was included in E-UTRA RRCConnectionResume message:

4>	submit the RRCReconfigurationComplete message via E-UTRA embedded in E-UTRA RRC message RRCConnectionResumeComplete as specified in TS 36.331 [10], clause 5.3.3.4a;

3>	else:

4>	submit the RRCReconfigurationComplete via E-UTRA embedded in E-UTRA RRC message RRCConnectionReconfigurationComplete as specified in TS 36.331 [10], clause 5.3.5.3/5.3.5.4/5.4.2.3;

3>	if the scg-State is not included in the E-UTRA message (RRCConnectionReconfiguration or RRCConnectionResume) containing the RRCReconfiguration message:

4>	perform SCG activation as specified in 5.3.5.13a;

4>	if reconfigurationWithSync was included in spCellConfig of an SCG:

5>	initiate the Random Access procedure on the PSCell, as specified in TS 38.321 [3];

4>	else if the SCG was deactivated before the reception of the E-UTRA RRC message containing the RRCReconfiguration message:

5>	if bfd-and-RLM was not configured to true before the reception of the E-UTRA RRCConnectionReconfiguration or RRCConnectionResume message containing the RRCReconfiguration message or if lower layers indicate that a Random Access procedure is needed for SCG activation:

6>	initiate the Random Access procedure on the SpCell, as specified in TS 38.321 [3];

5>	else the procedure ends;

4>	else the procedure ends;

3>	else:

4>	perform SCG deactivation as specified in 5.3.5.13b;

4>	the procedure ends;

2>	if the RRCReconfiguration message was received within nr-SecondaryCellGroupConfig in RRCConnectionReconfiguration message received via SRB3 within DLInformationTransferMRDC:

3>	submit the RRCReconfigurationComplete via E-UTRA embedded in E-UTRA RRC message RRCConnectionReconfigurationComplete as specified in TS 36.331 [10], clause 5.3.5.3/5.3.5.4;

3>	if the scg-State is not included in the RRCConnectionReconfiguration:

4>	if reconfigurationWithSync was included in spCellConfig of an SCG:

5>	initiate the Random Access procedure on the SpCell, as specified in TS 38.321 [3];

4>	else the procedure ends;

3>	else:

4>	perform SCG deactivation as specified in 5.3.5.13b;

4>	the procedure ends;

NOTE 1:	The order the UE sends the RRCConnectionReconfigurationComplete message and performs the Random Access procedure towards the SCG is left to UE implementation.

2>	else (RRCReconfiguration was received via SRB3) but not within DLInformationTransferMRDC:

3>	submit the RRCReconfigurationComplete message via SRB3 to lower layers for transmission using the new configuration;

NOTE 2:	In (NG)EN-DC and NR-DC, in the case RRCReconfiguration is received via SRB1 or within DLInformationTransferMRDC via SRB3, the random access is triggered by RRC layer itself as there is not necessarily other UL transmission. In the case RRCReconfiguration is received via SRB3 but not within DLInformationTransferMRDC, the random access is triggered by the MAC layer due to arrival of RRCReconfigurationComplete.

1>	else if the RRCReconfiguration message was received via SRB1 within the nr-SCG within mrdc-SecondaryCellGroup (UE in NR-DC, mrdc-SecondaryCellGroup was received in RRCReconfiguration or RRCResume via SRB1):

2>	if the RRCReconfiguration is applied due to a conditional reconfiguration execution for CPC which is configured via conditionalReconfiguration contained in nr-SCG within mrdc-SecondaryCellGroup:

3>	submit the RRCReconfigurationComplete message via the NR MCG embedded in NR RRC message ULInformationTransferMRDC as specified in clause 5.7.2a.3.

2>	if the scg-State is not included in the RRCReconfiguration or RRCResume message containing the RRCReconfiguration message:

3>	perform SCG activation as specified in 5.3.5.13a;

3>	if reconfigurationWithSync was included in spCellConfig in nr-SCG:

4>	initiate the Random Access procedure on the PSCell, as specified in TS 38.321 [3];

3>	else if the SCG was deactivated before the reception of the NR RRC message containing the RRCReconfiguration message:

4>	if bfd-and-RLM was not configured to true before the reception of the RRCReconfiguration or RRCResume message containing the RRCReconfiguration message; or

4>	if lower layers indicate that a Random Access procedure is needed for SCG activation:

5>	initiate the Random Access procedure on the PSCell, as specified in TS 38.321 [3];

4>	else the procedure ends;

3>	else the procedure ends;

2>	else

3>	perform SCG deactivation as specified in 5.3.5.13b;

3>	the procedure ends;

NOTE 2a:	The order in which the UE sends the RRCReconfigurationComplete message and performs the Random Access procedure towards the SCG is left to UE implementation.

1>	else if the RRCReconfiguration message was received via SRB3 (UE in NR-DC):

2>	if the RRCReconfiguration message was received within DLInformationTransferMRDC:

3>	if the RRCReconfiguration message was received within the nr-SCG within mrdc-SecondaryCellGroup (NR SCG RRC Reconfiguration):

4>	if the scg-State is not included in the RRCReconfiguration message containing the RRCReconfiguration message:

5>	if reconfigurationWithSync was included in spCellConfig in nr-SCG:

6>	initiate the Random Access procedure on the PSCell, as specified in TS 38.321 [3];

5>	else:

6>	the procedure ends;

4>	else:

5>	perform SCG deactivation as specified in 5.3.5.13b;

5>	the procedure ends;

3>	else:

4>	if the RRCReconfiguration does not include the mrdc-SecondaryCellGroupConfig:

5>	if the RRCReconfiguration includes the scg-State:

6>	perform SCG deactivation as specified in 5.3.5.13b;

4>	submit the RRCReconfigurationComplete message via SRB1 to lower layers for transmission using the new configuration;

2>	else:

3>	submit the RRCReconfigurationComplete message via SRB3 to lower layers for transmission using the new configuration;

1>	else (RRCReconfiguration was received via SRB1):

2>	if the UE is in NR-DC and;

2>	if the RRCReconfiguration does not include the mrdc-SecondaryCellGroupConfig:

3>	if the RRCReconfiguration includes the scg-State:

4>	perform SCG deactivation as specified in 5.3.5.13b;

3>	else:

4>	perform SCG activation without SN message as specified in 5.3.5.13b1;

2>	if the reconfigurationWithSync was included in spCellConfig of an MCG:

3>	if ta-Report is configured with value enabled and the UE supports TA reporting:

4>	indicate TA report initiation to lower layers;

2>	submit the RRCReconfigurationComplete message via SRB1 to lower layers for transmission using the new configuration;

2>	if this is the first RRCReconfiguration message after successful completion of the RRC re-establishment procedure:

3>	resume SRB2, SRB4, DRBs, multicast MRB, and BH RLC channels for IAB-MT, and Uu Relay RLC channels for L2 U2N Relay UE, that are suspended;

1>	if reconfigurationWithSync was included in spCellConfig of an MCG or SCG and when MAC of an NR cell group successfully completes a Random Access procedure triggered above; or,

1>	if sl-PathSwitchConfig was included in reconfigurationWithSync included in spCellConfig of an MCG, and when successfully sending RRCReconfigurationComplete message (i.e., PC5 RLC acknowledgement is received from target L2 U2N Relay UE):

2>	stop timer T304 for that cell group if running;

2>	if sl-PathSwitchConfig was included in reconfigurationWithSync:

3>	stop timer T420;

3>	release all radio resources, including release of the RLC entities and the MAC configuration at the source side;

3>	reset MAC used in the source cell;

NOTE 2b:	PDCP and SDAP configured by the source prior to the path switch that are reconfigured and re-used by target when delta signalling is used, are not released as part of this procedure.

2>	stop timer T310 for source SpCell if running;

2>	apply the parts of the CSI reporting configuration, the scheduling request configuration and the sounding RS configuration that do not require the UE to know the SFN of the respective target SpCell, if any;

2>	apply the parts of the measurement and the radio resource configuration that require the UE to know the SFN of the respective target SpCell (e.g. measurement gaps, periodic CQI reporting, scheduling request configuration, sounding RS configuration), if any, upon acquiring the SFN of that target SpCell;

2>	for each DRB configured as DAPS bearer, request uplink data switching to the PDCP entity, as specified in TS 38.323 [5];

2>	if the reconfigurationWithSync was included in spCellConfig of an MCG:

3>	if T390 is running:

4>	stop timer T390 for all access categories;

4>	perform the actions as specified in 5.3.14.4.

3>	if T350 is running:

4>	stop timer T350;

3>	if RRCReconfiguration does not include dedicatedSIB1-Delivery and

3>	if the active downlink BWP, which is indicated by the firstActiveDownlinkBWP-Id for the target SpCell of the MCG, has a common search space configured by searchSpaceSIB1:

4>	acquire the SIB1, which is scheduled as specified in TS 38.213 [13], of the target SpCell of the MCG;

4>	upon acquiring SIB1, perform the actions specified in clause 5.2.2.4.2;

2>	if the reconfigurationWithSync was included in spCellConfig of an MCG; or

2>	if the reconfigurationWithSync was included in spCellConfig of an SCG and the CPA or CPC was configured:

3>	remove all the entries within the MCG and the SCG VarConditionalReconfig, if any;

3>	remove all the entries within VarConditionalReconfiguration as specified in TS 36.331 [10], clause 5.3.5.9.6, if any;

3>	for each measId of the MCG measConfig, if configured, and for each measId of the SCG measConfig, if configured, if the associated reportConfig has a reportType set to condTriggerConfig:

4>	for the associated reportConfigId:

5>	remove the entry with the matching reportConfigId from the reportConfigList within the VarMeasConfig;

4>	if the associated measObjectId is only associated to a reportConfig with reportType set to condTriggerConfig:

5>	remove the entry with the matching measObjectId from the measObjectList within the VarMeasConfig;

4>	remove the entry with the matching measId from the measIdList within the VarMeasConfig;

2>	if reconfigurationWithSync was included in masterCellGroup or secondaryCellGroup:

3>	if the UE initiated transmission of a UEAssistanceInformation message for the corresponding cell group during the last 1 second, and the UE is still configured to provide the concerned UE assistance information for the corresponding cell group; or

3>	if the RRCReconfiguration message is applied due to a conditional reconfiguration execution, and the UE is configured to provide UE assistance information for the corresponding cell group, and the UE has initiated transmission of a UEAssistanceInformation message for the corresponding cell group since it was configured to do so in accordance with 5.7.4.2:

4>	initiate transmission of a UEAssistanceInformation message for the corresponding cell group in accordance with clause 5.7.4.3 to provide the concerned UE assistance information;

4>	start or restart the prohibit timer (if exists) or the leave without response timer for the MUSIM associated with the concerned UE assistance information with the timer value set to the value in corresponding configuration;

3>	if SIB12 is provided by the target PCell, and the UE initiated transmission of a SidelinkUEInformationNR message indicating a change of NR sidelink communication/discovery related parameters relevant in target PCell (i.e. change of sl-RxInterestedFreqList or sl-TxResourceReqList) during the last 1 second preceding reception of the RRCReconfiguration message including reconfigurationWithSync in spCellConfig of an MCG; or

3>	if the RRCReconfiguration message is applied due to a conditional reconfiguration execution and the UE is capable of NR sidelink communication/discovery and SIB12 is provided by the target PCell, and the UE has initiated transmission of a SidelinkUEInformationNR message since it was configured to do so in accordance with 5.8.3.2:

4>	initiate transmission of the SidelinkUEInformationNR message in accordance with 5.8.3.3;

2>	if reconfigurationWithSync was included in masterCellGroup:

3>	if configured with application layer measurements and if application layer measurement report container has been received from upper layers for which the successful transmission of the message or at least one segment of the message has not been confirmed by lower layers:

4>	re-submit the MeasurementReportAppLayer message or all segments of the MeasurementReportAppLayer message to lower layers for transmission via SRB4;

2>	if reconfigurationWithSync was included in masterCellGroup and the target cell provides SIB21:

3>	if the UE initiated transmission of an MBSInterestIndication message during the last 1 second preceding reception of this RRCReconfiguration message; or

3>	if the RRCReconfiguration message is applied due to a conditional reconfiguration execution, and the UE has initiated transmission of an MBSInterestIndication message after having received this RRCReconfiguration message:

4>	initiate transmission of an MBSInterestIndication message in accordance with clause 5.9.4;

2>	the procedure ends.

NOTE 3:	The UE is only required to acquire broadcasted SIB1 if the UE can acquire it without disrupting unicast or MBS multicast data reception, i.e. the broadcast and unicast/MBS multicast beams are quasi co-located.

NOTE 4: The UE sets the content of UEAssistanceInformation according to latest configuration (i.e. the configuration after applying the RRCReconfiguration message) and latest UE preference. The UE may include more than the concerned UE assistance information within the UEAssistanceInformation according to 5.7.4.2. Therefore, the content of UEAssistanceInformation message might not be the same as the content of the previous UEAssistanceInformation message.

#### 5.3.5.4	Secondary cell group release

The UE shall:

1>	as a result of SCG release triggered by E-UTRA (i.e. (NG)EN-DC case) or NR (i.e. NR-DC case):

2>	reset SCG MAC, if configured;

2>	for each RLC bearer that is part of the SCG configuration:

3>	perform RLC bearer release procedure as specified in 5.3.5.5.3;

2>	for each BH RLC channel that is part of the SCG configuration:

3>	perform BH RLC channel release procedure as specified in 5.3.5.5.10;

2>	release the SCG configuration;

2>	remove all the entries within the SCG VarConditionalReconfig, if any;

2>	if SCG release was triggered by NR (i.e. NR-DC case):

3>	remove all the entries within the MCG VarConditionalReconfig for which the RRCReconfiguration within condRRCReconfig does not include the masterCellGroup with reconfigurationWithSync, if any;

2>	else (i.e. EN-DC case):

3>	perform VarConditionalReconfiguration CPC removal as specified in TS 36.331 [10] clause 5.3.5.9.7;

2>	stop timer T310 for the corresponding SpCell, if running;

2>	stop timer T312 for the corresponding SpCell, if running;

2>	stop timer T304 for the corresponding SpCell, if running.

NOTE:	Release of cell group means only release of the lower layer configuration of the cell group but the RadioBearerConfig may not be released.

#### 5.3.5.5	Cell Group configuration

##### 5.3.5.5.1	General

The network configures the UE with Master Cell Group (MCG), and zero or one Secondary Cell Group (SCG). In (NG)EN-DC, the MCG is configured as specified in TS 36.331 [10], and for NE-DC, the SCG is configured as specified in TS 36.331 [10]. The network provides the configuration parameters for a cell group in the CellGroupConfig IE.

The UE performs the following actions based on a received CellGroupConfig IE:

1>	if the CellGroupConfig contains the spCellConfig with reconfigurationWithSync:

2>	perform Reconfiguration with sync according to 5.3.5.5.2;

2>	resume all suspended radio bearers except the SRBs for the source cell group, and resume SCG transmission for all radio bearers, and resume BH RLC channels and resume SCG transmission for BH RLC channels for IAB-MT, if suspended;

NOTE:	If the SCG is deactivated, resuming SCG transmission for all radio bearers does not imply that PDCP PDUs can be transmitted or received on SCG RLC bearers.

1>	if the CellGroupConfig contains the rlc-BearerToReleaseList or rlc-BearerToReleaseListExt:

2>	perform RLC bearer release as specified in 5.3.5.5.3;

1>	if the CellGroupConfig contains the rlc-BearerToAddModList:

2>	perform the RLC bearer addition/modification as specified in 5.3.5.5.4;

1>	if the CellGroupConfig contains the mac-CellGroupConfig:

2>	configure the MAC entity of this cell group as specified in 5.3.5.5.5;

1>	if the CellGroupConfig contains the sCellToReleaseList:

2>	perform SCell release as specified in 5.3.5.5.8;

1>	if the CellGroupConfig contains the spCellConfig:

2>	configure the SpCell as specified in 5.3.5.5.7;

1>	if the CellGroupConfig contains the sCellToAddModList:

2>	perform SCell addition/modification as specified in 5.3.5.5.9;

1>	if the CellGroupConfig contains the bh-RLC-ChannelToReleaseList:

2>	perform BH RLC channel release as specified in 5.3.5.5.10;

1>	if the CellGroupConfig contains the bh-RLC-ChannelToAddModList:

2>	perform the BH RLC channel addition/modification as specified in 5.3.5.5.11;

1>	if the CellGroupConfig contains the uu-RelayRLC-ChannelToReleaseList:

2>	perform Uu Relay RLC channel release as specified in 5.3.5.5.12;

1>	if the CellGroupConfig contains the uu-RelayRLC-ChannelToAddModList:

2>	perform the Uu Relay RLC channel addition/modification as specified in 5.3.5.5.13;

##### 5.3.5.5.2	Reconfiguration with sync

The UE shall perform the following actions to execute a reconfiguration with sync.

1>	if the AS security is not activated, perform the actions upon going to RRC_IDLE as specified in 5.3.11 with the release cause 'other' upon which the procedure ends;

1>	stop timer T430 if running;

1>	start timer T430 with the timer value set to ntn-UlSyncValidityDuration from the subframe indicated by epochTime, according to the target cell NTN-config;

1>	if no DAPS bearer is configured:

2>	stop timer T310 for the corresponding SpCell, if running;

1>	if this procedure is executed for the MCG:

2>	if timer T316 is running;

3>	stop timer T316;

3>	clear the information included in VarRLF-Report, if any;

2>	resume MCG transmission, if suspended.

1>	stop timer T312 for the corresponding SpCell, if running;

1>	if sl-PathSwitchConfig is included:

2>	consider the target L2 U2N Relay UE to be the one indicated by the targetRelayUE-Identity in the sl-PathSwitchConfig;

2>	start timer T420 for the corresponding target L2 U2N Relay UE with the timer value set to T420, as included in the sl-PathSwitchConfig;

2>	apply the value of the newUE-Identity as the C-RNTI;

2>	indicate to upper layer (to trigger the PC5 unicast link establishment) with the target L2 U2N Relay UE indicated by the targetRelayUE-Identity;

2>	apply the default configuration of SL-RLC1 as defined in 9.2.4 for SRB1;

1>	else (sl-PathSwitchConfig is not included):

2>	if this procedure is executed for the MCG or if this procedure is executed for an SCG not indicated as deactivated in the E-UTRA or NR RRC message in which the RRCReconfiguration message is embedded:

3>	start timer T304 for the corresponding SpCell with the timer value set to t304, as included in the reconfigurationWithSync;

2>	if the frequencyInfoDL is included:

3>	consider the target SpCell to be one on the SSB frequency indicated by the frequencyInfoDL with a physical cell identity indicated by the physCellId;

2>	else:

3>	consider the target SpCell to be one on the SSB frequency of the source SpCell with a physical cell identity indicated by the physCellId;

2>	start synchronising to the DL of the target SpCell;

2>	apply the specified BCCH configuration defined in 9.1.1.1 for the target SpCell;

2>	acquire the MIB of the target SpCell, which is scheduled as specified in TS 38.213 [13];

NOTE 1:	The UE should perform the reconfiguration with sync as soon as possible following the reception of the RRC message triggering the reconfiguration with sync, which could be before confirming successful reception (HARQ and ARQ) of this message.

NOTE 2:	The UE may omit reading the MIB if the UE already has the required timing information, or the timing information is not needed for random access.

NOTE 2a:	A UE with DAPS bearer does not monitor for system information updates in the source PCell.

2>	If any DAPS bearer is configured:

3>	create a MAC entity for the target cell group with the same configuration as the MAC entity for the source cell group;

3>	for each DAPS bearer:

4>	establish an RLC entity or entities for the target cell group, with the same configurations as for the source cell group;

4>	establish the logical channel for the target cell group, with the same configurations as for the source cell group;

NOTE 2b:	In order to understand if a DAPS bearer is configured, the UE needs to check the presence of the field daps-Config within the RadioBearerConfig IE received in radioBearerConfig or radioBearerConfig2.

3>	for each SRB:

4>	establish an RLC entity for the target cell group, with the same configurations as for the source cell group;

4>	establish the logical channel for the target cell group, with the same configurations as for the source cell group;

3>	suspend SRBs for the source cell group;

NOTE 3:	Void

3>	apply the value of the newUE-Identity as the C-RNTI in the target cell group;

3>	configure lower layers for the target SpCell in accordance with the received spCellConfigCommon;

3>	configure lower layers for the target SpCell in accordance with any additional fields, not covered in the previous, if included in the received reconfigurationWithSync.

2>	else:

3>	reset the MAC entity of this cell group;

3>	consider the SCell(s) of this cell group, if configured, that are not included in the SCellToAddModList in the RRCReconfiguration message, to be in deactivated state;

3>	apply the value of the newUE-Identity as the C-RNTI for this cell group;

3>	configure lower layers in accordance with the received spCellConfigCommon;

3>	configure lower layers in accordance with any additional fields, not covered in the previous, if included in the received reconfigurationWithSync.

2>	if the UE is acting as L2 U2N Remote UE at the source side:

3>	indicate upper layer to trigger PC5 unicast link release.

Upon L2 U2N Relay UE receiving reconfigurationWithSync, it either indicates to upper layers (to trigger PC5 unicast link release) or sends Notification message to the connected L2 U2N Remote UE(s) in accordance with 5.8.9.10.

##### 5.3.5.5.3	RLC bearer release

The UE shall:

1>	for each logicalChannelIdentity/LogicalChannelIdentityExt value included in the rlc-BearerToReleaseList/rlc-BearerToReleaseListExt that is part of the current UE configuration within the same cell group (LCH release); or

1>	for each logicalChannelIdentity value that is to be released as the result of an SCG release according to 5.3.5.4:

2>	release the RLC entity or entities as specified in TS 38.322 [4], clause 5.1.3;

2>	release the corresponding logical channel.

##### 5.3.5.5.4	RLC bearer addition/modification

For each RLC-BearerConfig received in the rlc-BearerToAddModList IE the UE shall:

1>	if the UE's current configuration contains an RLC bearer with the received logicalChannelIdentity/LogicalChannelIdentityExt within the same cell group:

2>	if the RLC bearer is associated with an DAPS bearer, or

2>	if any DAPS bearer is configured and the RLC bearer is associated with an SRB:

3>	reconfigure the RLC entity or entities for the target cell group in accordance with the received rlc-Config;

3>	reconfigure the logical channel for the target cell group in accordance with the received mac-LogicalChannelConfig;

2>	else:

3>	if reestablishRLC is received:

4>	re-establish the RLC entity as specified in TS 38.322 [4];

3>	reconfigure the RLC entity or entities in accordance with the received rlc-Config;

3>	reconfigure the logical channel in accordance with the received mac-LogicalChannelConfig;

3>	if servedMBS-RadioBearer is received:

4>	associate this logical channel with the PDCP entity identified by servedMBS-RadioBearer;

NOTE 1:	For DRB and SRB, the network does not re-associate an already configured logical channel with another radio bearer. Hence, servedRadioBearer is not present in this case. For MRB, the network does not re-associate an already configured logical channel with DRB or SRB or another MRB (i.e. MRB with another PDCP entity). Hence multicastRLC-BearerConfig is not present in this case. If a radio bearer is released and another is added with the same radio bearer identity, it is considered as a new (different) radio bearer. Hence, the network also releases the RLC bearer(s) associated with the released radio bearer.

NOTE 2:	In DAPS handover, the UE may perform RLC entity re-establishment (if reestablishRLC is set) for an RLC bearer associated with a non-DAPS bearer when indication of successful completion of random access towards target cell is received from lower layers as specified in TS 38.321 [3].

1>	else (a logical channel with the given logicalChannelIdentity/LogicalChannelIdentityExt is not configured within the same cell group, including the case when full configuration option is used):

2>	if the servedRadioBearer associates the logical channel with an SRB and rlc-Config is not included:

3>	establish an RLC entity in accordance with the default configuration defined in 9.2 for the corresponding SRB;

2>	else:

3>	establish an RLC entity in accordance with the received rlc-Config;

2>	if the servedRadioBearer associates the logical channel with an SRB and if mac-LogicalChannelConfig is not included:

3>	configure this MAC entity with a logical channel in accordance to the default configuration defined in 9.2 for the corresponding SRB;

2>	else:

3>	configure this MAC entity with a logical channel in accordance to the received mac-LogicalChannelConfig;

2>	associate this logical channel with the PDCP entity identified by servedRadioBearer or servedMBS-RadioBearer.

##### 5.3.5.5.5	MAC entity configuration

The UE shall:

1>	if SCG MAC is not part of the current UE configuration (i.e. SCG establishment):

2>	create an SCG MAC entity;

1>	if any DAPS bearer is configured:

2>	reconfigure the MAC main configuration for the target cell group in accordance with the received mac-CellGroupConfig excluding tag-ToReleaseList and tag-ToAddModList;

1>	else:

2>	reconfigure the MAC main configuration of the cell group in accordance with the received mac-CellGroupConfig excluding tag-ToReleaseList and tag-ToAddModList;

1>	if the received mac-CellGroupConfig includes the tag-ToReleaseList:

2>	for each TAG-Id value included in the tag-ToReleaseList that is part of the current UE configuration:

3>	release the TAG indicated by TAG-Id;

1>	if the received mac-CellGroupConfig includes the tag-ToAddModList:

2>	for each tag-Id value included in tag-ToAddModList that is not part of the current UE configuration (TAG addition):

3>	add the TAG, corresponding to the tag-Id, in accordance with the received timeAlignmentTimer;

2>	for each tag-Id value included in tag-ToAddModList that is part of the current UE configuration (TAG modification):

3>	reconfigure the TAG, corresponding to the tag-Id, in accordance with the received timeAlignmentTimer.

##### 5.3.5.5.6	RLF Timers & Constants configuration

The UE shall:

1>	if the received rlf-TimersAndConstants is set to release:

2>	if any DAPS bearer is configured:

3>	use values for timers T301, T310, T311 and constants N310, N311 for the target cell group, as included in ue-TimersAndConstants received in SIB1;

2>	else:

3>	use values for timers T301, T310, T311 and constants N310, N311, as included in ue-TimersAndConstants received in SIB1;

1>	else:

2>	if any DAPS bearer is configured:

3>	configure the value of timers and constants for the target cell group in accordance with received rlf-TimersAndConstants;

2>	else:

3>	(re-)configure the value of timers and constants in accordance with received rlf-TimersAndConstants;

3>	stop timer T310 for this cell group, if running;

3>	stop timer T312 for this cell group, if running;

3>	reset the counters N310 and N311.

##### 5.3.5.5.7	SpCell Configuration

The UE shall:

1>	if the UE is acting as L2 U2N Remote UE:

2>	if the SpCellConfig contains the rlf-TimersAndConstants which is set to setup:

3>	use value for timers T311 as received in rlf-TimersAndConstants;

2>	else if rlf-TimersAndConstants is not configured for this cell group or SpCellConfig contains the rlf-TimersAndConstants which is set to release:

3>	use value for timers T311, as included in ue-TimersAndConstants received in SIB1;

1>	else

2>	if the SpCellConfig contains the rlf-TimersAndConstants:

3>	configure the RLF timers and constants for this cell group as specified in 5.3.5.5.6;

2>	else if rlf-TimersAndConstants is not configured for this cell group:

3>	if any DAPS bearer is configured:

4>	use values for timers T301, T310, T311 and constants N310, N311 for the target cell group, as included in ue-TimersAndConstants received in SIB1;

3>	else

4>	use values for timers T301, T310, T311 and constants N310, N311, as included in ue-TimersAndConstants received in SIB1;

2>	if the SpCellConfig contains spCellConfigDedicated:

3>	configure the SpCell in accordance with the spCellConfigDedicated;

3>	consider the bandwidth part indicated in firstActiveUplinkBWP-Id, if included in the spCellConfigDedicated, to be the active uplink bandwidth part;

3>	if the firstActiveDownlinkBWP-Id is included in the spCellConfigDedicated:

4>	if the SpCellConfig is included in an RRCReconfiguration message contained in an NR or E-UTRA RRC message indicating that the SCG is deactivated:

5>	consider the bandwidth part indicated in firstActiveDownlinkBWP-Id to be the bandwidth part for Radio Link Monitoring, Beam Failure Detection and measurements;

4>	else:

5>	consider the bandwith part indicated in firstActiveDownlinkBWP-Id to be the active downlink bandwidth part;

3>	if any of the reference signal(s) that are used for radio link monitoring are reconfigured by the received spCellConfigDedicated:

4>	stop timer T310 for the corresponding SpCell, if running;

4>	stop timer T312 for the corresponding SpCell, if running;

4>	reset the counters N310 and N311.

1>	if the SpCellConfig contains the lowMobilityEvaluationConnected:

2>	the UE may perform the evaluation of the low mobility criterion for this cell group as specified in 5.7.13.1;

1>	if the SpCellConfig contains the goodServingCellEvaluationRLM:

2>	the UE may perform the evaluation of the good serving cell quality criterion for this SpCell as specified in 5.7.13.2;

1>	if the SpCellConfig contains the goodServingCellEvaluationBFD:

2>	the UE may perform the evaluation of the good serving cell quality criterion for this serving cell as specified in 5.7.13.2;

##### 5.3.5.5.8	SCell Release

The UE shall:

1>	if the release is triggered by reception of the sCellToReleaseList:

2>	for each sCellIndex value included in the sCellToReleaseList:

3>	if the current UE configuration includes an SCell with value sCellIndex:

4>	release the SCell.

##### 5.3.5.5.9	SCell Addition/Modification

The UE shall:

1>	for each sCellIndex value included in the sCellToAddModList that is not part of the current UE configuration (SCell addition):

2>	add the SCell, corresponding to the sCellIndex, in accordance with the sCellConfigCommon and sCellConfigDedicated;

2>	if the sCellState is included:

3>	configure lower layers to consider the SCell to be in activated state;

2>	else:

3>	configure lower layers to consider the SCell to be in deactivated state;

2>	for each measId included in the measIdList within VarMeasConfig:

3>	if SCells are not applicable for the associated measurement; and

3>	if the concerned SCell is included in cellsTriggeredList defined within the VarMeasReportList for this measId:

4>	remove the concerned SCell from cellsTriggeredList defined within the VarMeasReportList for this measId;

2>	if the SCellConfig contains the goodServingCellEvaluationBFD:

3>	the UE may perform the evaluation of the good serving cell quality criterion for this serving cell as specified in 5.7.13.2.

1>	for each sCellIndex value included in the sCellToAddModList that is part of the current UE configuration (SCell modification):

2>	modify the SCell configuration in accordance with the sCellConfigDedicated;

2>	if the sCellState is included:

3>	configure lower layers to consider the SCell to be in activated state;

2>	else:

3>	configure lower layers to consider the SCell to be in deactivated state.

2>	if the SCellConfig contains the goodServingCellEvaluationBFD:

3>	the UE may perform the evaluation of the good serving cell quality criterion for this serving cell as specified in 5.7.13.2.

##### 5.3.5.5.10	BH RLC channel release

The IAB-node shall:

1>	for each BH-RLC-ChannelID value included in the bh-RLC-ChannelToReleaseList that is part of the current IAB-node configuration within the same cell group (LCH release); or

1>	for each BH-RLC-ChannelID value that is to be released as the result of an SCG release according to 5.3.5.4:

2>	release the RLC entity or entities as specified in TS 38.322 [4], clause 5.1.3;

2>	release the corresponding logical channel.

##### 5.3.5.5.11	BH RLC channel addition/modification

For each BH-RLC-ChannelConfig received in the bh-RLC-ChannelToAddModList IE the IAB-node shall:

1>	if the current configuration contains a BH RLC Channel with the received bh-RLC-ChannelID within the same cell group:

2>	if reestablishRLC is received:

3>	re-establish the RLC entity as specified in TS 38.322 [4];

2>	reconfigure the RLC entity or entities in accordance with the received rlc-Config;

2>	reconfigure the logical channel in accordance with the received mac-LogicalChannelConfig;

1>	else (a backhaul logical channel with the given BH-RLC-ChannelID was not configured before within the same cell group):

2>	establish an RLC entity in accordance with the received rlc-Config;

2>	configure this MAC entity with a logical channel in accordance to the received mac-LogicalChannelConfig.

##### 5.3.5.5.12	Uu Relay RLC channel release

The L2 U2N Relay UE shall:

1>	for each Uu-RelayRLC-ChannelID value included in the uu-RelayRLC-ChannelToReleaseList that is part of the current configuration within the same cell group (LCH release):

2>	release the RLC entity as specified in TS 38.322 [4], clause 5.1.3;

2>	release the corresponding logical channel.

##### 5.3.5.5.13	Uu Relay RLC channel addition/modification

For each Uu-RelayRLC-ChannelConfig received in the uu-RelayRLC-ChannelToAddModList the L2 U2N Relay UE shall:

1>	if the current configuration contains a Uu Relay RLC channel with the same uu-RelayRLC-ChannelID within the same cell group:

2>	if reestablishRLC is received:

3>	re-establish the RLC entity as specified in TS 38.322 [4];

2>	reconfigure the RLC entity in accordance with the received rlc-Config;

2>	reconfigure the logical channel in accordance with the received mac-LogicalChannelConfig;

1>	else (a logical channel with the given uu-RelayRLC-ChannelID was not configured before within the same cell group):

2>	establish an RLC entity in accordance with the received rlc-Config;

2>	configure this MAC entity with a logical channel in accordance to the received mac-LogicalChannelConfig.

#### 5.3.5.6	Radio Bearer configuration

##### 5.3.5.6.1	General

The UE shall perform the following actions based on a received RadioBearerConfig IE:

1>	if the RadioBearerConfig includes the srb3-ToRelease or srb4-ToRelease:

2>	perform the SRB release as specified in 5.3.5.6.2;

1>	if the RadioBearerConfig includes the srb-ToAddModList or if any DAPS bearer is configured:

2>	perform the SRB addition or reconfiguration as specified in 5.3.5.6.3;

1>	if the RadioBearerConfig includes the drb-ToReleaseList:

2>	perform DRB release as specified in 5.3.5.6.4;

1>	if the RadioBearerConfig includes the drb-ToAddModList:

2>	perform DRB addition or reconfiguration as specified in 5.3.5.6.5;

1>	if the RadioBearerConfig includes the mrb-ToReleaseList:

2>	perform multicast MRB release as specified in 5.3.5.6.6;

1>	if the RadioBearerConfig includes the mrb-ToAddModList:

2>	perform multicast MRB addition or reconfiguration as specified in 5.3.5.6.7;

1>	release all SDAP entities, if any, that have no associated DRB as specified in TS 37.324 [24] clause 5.1.2, and indicate the release of the user plane resources for PDU Sessions associated with the released SDAP entities to upper layers;

1>	release all SDAP entities that have no associated multicast MRB as specified in TS 37.324 [24] clause 5.1.2, and indicate the release of user plane resources for these MBS multicast sessions to upper layers.

##### 5.3.5.6.2	SRB release

The UE shall:

1>	if srb3-ToRelease is included:

2>	release the PDCP entity and the srb-Identity of the SRB3;

1>	if srb4-ToRelease is included

2>	release the PDCP entity and the srb-Identity of the SRB4.

##### 5.3.5.6.3	SRB addition/modification

The UE shall:

1>	If any DAPS bearer is configured, for each SRB:

2>	establish a PDCP entity for the target cell group as specified in TS 38.323 [5], with the same configuration as the PDCP entity for the source cell group;

2>	if the masterKeyUpdate is received:

3>	configure the PDCP entity with the security algorithms according to securityConfig and apply the keys (KRRCenc and KRRCint) associated with the master key (KgNB);

2>	else:

3>	configure the PDCP entity for the target cell group with state variables continuation as specified in TS 38.323 [5], and with the same security configuration as the PDCP entity for the source cell group;

1>	for each srb-Identity value included in the srb-ToAddModList that is not part of the current UE configuration (SRB establishment or reconfiguration from E-UTRA PDCP to NR PDCP):

2>	establish a PDCP entity;

2>	if AS security has been activated:

3>	if target RAT of handover is E-UTRA/5GC; or

3>	if the UE is connected to E-UTRA/5GC:

4>	if the UE is capable of E-UTRA/5GC, but not capable of NGEN-DC:

5>	configure the PDCP entity with the security algorithms and keys (KRRCenc and KRRCint) configured/derived as specified in TS 36.331 [10];

4>	else (i.e., UE capable of NGEN-DC):

5>	configure the PDCP entity with the security algorithms according to securityConfig and apply the keys (KRRCenc and KRRCint) associated with the master key (KeNB) or secondary key (S-KgNB) as indicated in keyToUse, if applicable;

3>	else (i.e., UE connected to NR or UE connected to E-UTRA/EPC):

4>	configure the PDCP entity with the security algorithms according to securityConfig and apply the keys (KRRCenc and KRRCint) associated with the master key (KeNB/ KgNB) or secondary key (S-KgNB) as indicated in keyToUse, if applicable;

2>	if the current UE configuration as configured by E-UTRA in TS 36.331 [10] includes an SRB identified with the same srb-Identity value:

3>	associate the E-UTRA RLC entity and DCCH of this SRB with the NR PDCP entity;

3>	release the E-UTRA PDCP entity of this SRB;

2>	if the pdcp-Config is included:

3>	configure the PDCP entity in accordance with the received pdcp-Config;

2>	else:

3>	configure the PDCP entity in accordance with the default configuration defined in 9.2.1 for the corresponding SRB;

1>	if any DAPS bearer is configured, for each srb-Identity value included in the srb-ToAddModList that is part of the current UE configuration:

2>	if the pdcp-Config is included:

3>	reconfigure the PDCP entity for the target cell group in accordance with the received pdcp-Config;

1>	else, for each srb-Identity value included in the srb-ToAddModList that is part of the current UE configuration:

2>	if the reestablishPDCP is set:

3>	if target RAT of handover is E-UTRA/5GC; or

3>	if the UE is connected to E-UTRA/5GC:

4>	if the UE is capable of E-UTRA/5GC, but not capable of NGEN-DC:

5>	configure the PDCP entity to apply the integrity protection algorithm and KRRCint key configured/derived as specified in TS 36.331 [10], i.e. the integrity protection configuration shall be applied to all subsequent messages received and sent by the UE, including the message used to indicate the successful completion of the procedure;

5>	configure the PDCP entity to apply the ciphering algorithm and KRRCenc key configured/derived as specified in TS 36.331 [10], i.e. the ciphering configuration shall be applied to all subsequent messages received and sent by the UE, including the message used to indicate the successful completion of the procedure;

4>	else (i.e., a UE capable of NGEN-DC):

5>	configure the PDCP entity to apply the integrity protection algorithm and KRRCint key associated with the master key (KeNB) or secondary key (S-KgNB), as indicated in keyToUse, i.e. the integrity protection configuration shall be applied to all subsequent messages received and sent by the UE, including the message used to indicate the successful completion of the procedure;

5>	configure the PDCP entity to apply the ciphering algorithm and KRRCenc key associated with the master key (KeNB) or secondary key (S-KgNB) as indicated in keyToUse, i.e. the ciphering configuration shall be applied to all subsequent messages received and sent by the UE, including the message used to indicate the successful completion of the procedure;

3>	else (i.e., UE connected to NR or UE in EN-DC):

4>	configure the PDCP entity to apply the integrity protection algorithm and KRRCint key associated with the master key (KeNB/KgNB) or secondary key (S-KgNB), as indicated in keyToUse , i.e. the integrity protection configuration shall be applied to all subsequent messages received and sent by the UE, including the message used to indicate the successful completion of the procedure;

4>	configure the PDCP entity to apply the ciphering algorithm and KRRCenc key associated with the master key (KeNB/KgNB) or secondary key (S-KgNB) as indicated in keyToUse, i.e. the ciphering configuration shall be applied to all subsequent messages received and sent by the UE, including the message used to indicate the successful completion of the procedure;

3>	re-establish the PDCP entity of this SRB as specified in TS 38.323 [5];

2>	else, if the discardOnPDCP is set:

3>	trigger the PDCP entity to perform SDU discard as specified in TS 38.323 [5];

2>	if the pdcp-Config is included:

3>	reconfigure the PDCP entity in accordance with the received pdcp-Config.

##### 5.3.5.6.4	DRB release

The UE shall:

1>	for each drb-Identity value included in the drb-ToReleaseList that is part of the current UE configuration; or

1>	for each drb-Identity value that is to be released as the result of full configuration according to 5.3.5.11:

2>	release the PDCP entity and the drb-Identity;

2>	if SDAP entity associated with this DRB is configured:

3>	indicate the release of the DRB to SDAP entity associated with this DRB (TS 37.324 [24], clause 5.3.3);

2>	if the DRB is associated with an eps-BearerIdentity:

3>	if a new bearer is not added either with NR or E-UTRA with same eps-BearerIdentity:

4>	indicate the release of the DRB and the eps-BearerIdentity of the released DRB to upper layers.

NOTE 1:	The UE does not consider the message as erroneous if the drb-ToReleaseList includes any drb-Identity value that is not part of the current UE configuration.

NOTE 2:	Whether or not the RLC and MAC entities associated with this PDCP entity are reset or released is determined by the CellGroupConfig.

##### 5.3.5.6.5	DRB addition/modification

The UE shall:

1>	for each drb-Identity value included in the drb-ToAddModList that is not part of the current UE configuration (DRB establishment including the case when full configuration option is used):

2>	establish a PDCP entity and configure it in accordance with the received pdcp-Config;

2>	if the PDCP entity of this DRB is not configured with cipheringDisabled:

3>	if target RAT of handover is E-UTRA/5GC; or

3>	if the UE is connected to E-UTRA/5GC:

4>	if the UE is capable of E-UTRA/5GC but not capable of NGEN-DC:

5>	configure the PDCP entity with the ciphering algorithm and KUPenc key configured/derived as specified in TS 36.331 [10];

4>	else (i.e., a UE capable of NGEN-DC):

5>	configure the PDCP entity with the ciphering algorithms according to securityConfig and apply the key (KUPenc) associated with the master key (KeNB) or secondary key (S-KgNB) as indicated in keyToUse, if applicable;

3>	else (i.e., UE connected to NR or UE connected to E-UTRA/EPC):

4>	configure the PDCP entity with the ciphering algorithms according to securityConfig and apply the KUPenc key associated with the master key (KeNB/KgNB) or the secondary key (S-KgNB/S-KeNB) as indicated in keyToUse;

2>	if the PDCP entity of this DRB is configured with integrityProtection:

3>	configure the PDCP entity with the integrity protection algorithms according to securityConfig and apply the KUPint key associated with the master (KeNB/KgNB) or the secondary key (S-KgNB) as indicated in keyToUse;

2>	if an sdap-Config is included:

3>	if an SDAP entity with the received pdu-Session does not exist:

4>	establish an SDAP entity as specified in TS 37.324 [24] clause 5.1.1;

4>	if an SDAP entity with the received pdu-Session did not exist prior to receiving this reconfiguration:

5>	indicate the establishment of the user plane resources for the pdu-Session to upper layers;

3>	configure the SDAP entity in accordance with the received sdap-Config as specified in TS 37.324 [24] and associate the DRB with the SDAP entity;

3>	for each QFI value added in mappedQoS-FlowsToAdd, if the QFI value is previously configured, the QFI value is released from the old DRB;

2>	if the DRB is associated with an eps-BearerIdentity:

3>	if the DRB was configured with the same eps-BearerIdentity either by NR or E-UTRA prior to receiving this reconfiguration:

4>	associate the established DRB with the corresponding eps-BearerIdentity;

3>	else:

4>	indicate the establishment of the DRB(s) and the eps-BearerIdentity of the established DRB(s) to upper layers;

1>	for each drb-Identity value included in the drb-ToAddModList that is part of the current UE configuration and configured as DAPS bearer:

2>	reconfigure the PDCP entity to configure DAPS with the ciphering function, integrity protection function and ROHC function of the target cell group as specified in TS 38.323 [5] and configure it in accordance with the received pdcp-Config;

2>	if the masterKeyUpdate is received:

3>	if the ciphering function of the target cell group PDCP entity is not configured with cipheringDisabled:

4>	configure the ciphering function of the target cell group PDCP entity with the ciphering algorithm according to securityConfig and apply the KUPenc key associated with the master key (KgNB), as indicated in keyToUse, i.e. the ciphering configuration shall be applied to all subsequent PDCP PDUs received from the target cell group and sent to the target cell group by the UE;

3>	if the integrity protection function of the target cell group PDCP entity is configured with integrityProtection:

4>	configure the integrity protection function of the target cell group PDCP entity with the integrity protection algorithms according to securityConfig and apply the KUPint key associated with the master key (KgNB) as indicated in keyToUse;

2>	else:

3>	configure the ciphering function and the integrity protection function of the target cell group PDCP entity with the same security configuration as the PDCP entity for the source cell group;

2>	if the sdap-Config is included and when indication of successful completion of random access towards target cell is received from lower layers as specified in [3]:

3>	reconfigure the SDAP entity in accordance with the received sdap-Config as specified in TS 37.324 [24];

3>	for each QFI value added in mappedQoS-FlowsToAdd, if the QFI value is previously configured, the QFI value is released from the old DRB;

1>	for each drb-Identity value included in the drb-ToAddModList that is part of the current UE configuration and not configured as DAPS bearer:

2>	if the reestablishPDCP is set:

3>	if target RAT of handover is E-UTRA/5GC; or

3>	if the UE is connected to E-UTRA/5GC:

4>	if the UE is capable of E-UTRA/5GC but not capable of NGEN-DC:

5>	if the PDCP entity of this DRB is not configured with cipheringDisabled:

6>	configure the PDCP entity with the ciphering algorithm and KUPenc key configured/derived as specified in TS 36.331 [10], clause 5.4.2.3, i.e. the ciphering configuration shall be applied to all subsequent PDCP PDUs received and sent by the UE;

4>	else (i.e., a UE capable of NGEN-DC):

5>	if the PDCP entity of this DRB is not configured with cipheringDisabled:

6>	configure the PDCP entity with the ciphering algorithm and KUPenc key associated with the master key (KeNB) or the secondary key (S-KgNB), as indicated in keyToUse, i.e. the ciphering configuration shall be applied to all subsequent PDCP PDUs received and sent by the UE;

3>	else (i.e., UE connected to NR or UE connected to E-UTRA/EPC (in EN-DC or capable of EN-DC)):

4>	if the PDCP entity of this DRB is not configured with cipheringDisabled:

5>	configure the PDCP entity with the ciphering algorithm and KUPenc key associated with the master key (KeNB/ KgNB) or the secondary key (S-KgNB/S-KeNB), as indicated in keyToUse, i.e. the ciphering configuration shall be applied to all subsequent PDCP PDUs received and sent by the UE;

4>	if the PDCP entity of this DRB is configured with integrityProtection:

5>	configure the PDCP entity with the integrity protection algorithms according to securityConfig and apply the KUPint key associated with the master key (KeNB/KgNB) or the secondary key (S-KgNB) as indicated in keyToUse;

3>	if drb-ContinueROHC is included in pdcp-Config:

4>	indicate to lower layer that drb-ContinueROHC is configured;

3>	if drb-ContinueEHC-DL is included in pdcp-Config:

4>	indicate to lower layer that drb-ContinueEHC-DL is configured;

3>	if drb-ContinueEHC-UL is included in pdcp-Config:

4>	indicate to lower layer that drb-ContinueEHC-UL is configured;

3>	if drb-ContinueUDC is included in pdcp-Config:

4>	indicate to lower layer that drb-ContinueUDC is configured;

3>	re-establish the PDCP entity of this DRB as specified in TS 38.323 [5], clause 5.1.2;

2>	else, if the recoverPDCP is set:

3>	trigger the PDCP entity of this DRB to perform data recovery as specified in TS 38.323 [5];

2>	if the pdcp-Config is included:

3>	reconfigure the PDCP entity in accordance with the received pdcp-Config.

2>	if the sdap-Config is included:

3>	reconfigure the SDAP entity in accordance with the received sdap-Config as specified in TS37.324 [24];

3>	for each QFI value added in mappedQoS-FlowsToAdd, if the QFI value is previously configured, the QFI value is released from the old DRB;

NOTE 1:	Void.

NOTE 2:	When determining whether a drb-Identity value is part of the current UE configuration, the UE does not distinguish which RadioBearerConfig and DRB-ToAddModList that DRB was originally configured in. To re-associate a DRB with a different key (KeNB to S-KgNB, KgNB to S-KeNB, KgNB to S-KgNB, or vice versa), the network provides the drb-Identity value in the (target) drb-ToAddModList and sets the reestablishPDCP flag. The network does not list the drb-Identity in the (source) drb-ToReleaseList.

NOTE 3:	When setting the reestablishPDCP flag for a radio bearer, the network ensures that the RLC receiver entities do not deliver old PDCP PDUs to the re-established PDCP entity. It does that e.g. by triggering a reconfiguration with sync of the cell group hosting the old RLC entity or by releasing the old RLC entity.

NOTE 4:	In this specification, UE configuration refers to the parameters configured by NR RRC unless otherwise stated.

NOTE 5: Ciphering and integrity protection can be enabled or disabled for a DRB. The enabling/disabling of ciphering or integrity protection can be changed only by releasing and adding the DRB.

NOTE 6:	In DAPS handover, the UE may perform PDCP entity re-establishment (if reestablishPDCP is set) or the PDCP data recovery (if recoverPDCP is set) for a non-DAPS bearer when indication of successful completion of random access towards target cell is received from lower layers as specified in TS 38.321 [3]. In this case, the UE suspends data transmission and reception for all non-DAPS bearers in the source MCG for duration of the DAPS handover.

##### 5.3.5.6.6	Multicast MRB release

The UE shall:

1>	for each mrb-Identity value included in the mrb-ToReleaseList that is part of the current UE configuration; or

1>	for each mrb-Identity value that is to be released as the result of full configuration according to 5.3.5.11:

2>	release the PDCP entity and the mrb-Identity;

2>	if there is no other multicast MRB configured with the same mbs-SessionId as configured for the released multicast MRB:

3>	indicate the release of the user plane resources for the mbs-SessionId to upper layers.

NOTE 1:	The UE does not consider the message as erroneous if the mrb-ToReleaseList includes any mrb-Identity value that is not part of the current UE configuration.

NOTE 2:	Whether or not the RLC and MAC entities associated with this PDCP entity are reset or released is determined by the CellGroupConfig.

##### 5.3.5.6.7	Multicast MRB addition/modification

The UE shall for each element in the order of entry in the list mrb-ToAddModList:

1>	if mrb-Identity value included in the mrb-ToAddModList is part of the UE configuration:

2>	if mrb-Identity value included in the mrb-ToAddModList for which mrb-IdentityNew is included (i.e., multicast MRB ID change):

3>	update the mrb-Identity to the value mrb-IdentityNew;

2>	if the reestablishPDCP is set:

3>	if drb-ContinueROHC is included in pdcp-Config:

4>	indicate to lower layer that drb-ContinueROHC is configured;

3>	if drb-ContinueEHC-DL is included in pdcp-Config:

4>	indicate to lower layer that drb-ContinueEHC-DL is configured;

3>	re-establish the PDCP entity of this multicast MRB as specified in TS 38.323 [5], clause 5.1.2;

2>	else, if the recoverPDCP is set:

3>	trigger the PDCP entity of this MRB to perform data recovery as specified in TS 38.323 [5];

2>	if the pdcp-Config is included:

3>	reconfigure the PDCP entity in accordance with the received pdcp-Config;

1>	else if mrb-Identity value included in the mrb-ToAddModList is not part of the UE configuration (i.e., multicast MRB establishment including the case when full configuration option is used):

2>	establish a PDCP entity and configure it in accordance with the received pdcp-Config;

2>	associate the established multicast MRB with the corresponding mbs-SessionId;

2>	if an SDAP entity with the received mbs-SessionId does not exist:

3>	establish an SDAP entity as specified in TS 37.324 [24] clause 5.1.1;

3>	if an SDAP entity with the received mbs-SessionId did not exist prior to receiving this reconfiguration:

4>	indicate the establishment of the user plane resources for the mbs-SessionId to upper layers.

NOTE 1:	When setting the reestablishPDCP flag for a radio bearer, the network ensures that the RLC receiver entities do not deliver old PDCP PDUs to the re-established PDCP entity. The network does that e.g. by triggering a reconfiguration with sync of the cell group hosting the old RLC entity or by releasing the old RLC entity.

NOTE 2:	In this specification, UE configuration refers to the parameters configured by NR RRC unless otherwise stated.

NOTE 3:	When updating the mrb-Identity, the network ensures new MRBs are listed at the end of the mrb-ToAddModList if they have the same MRB ID as in the existing UE configuration.

#### 5.3.5.7	AS Security key update

The UE shall:

1>	if UE is connected to E-UTRA/EPC or E-UTRA/5GC:

2>	upon reception of sk-Counter as specified in TS 36.331 [10]:

3>	update the S-KgNB key based on the KeNB key and using the received sk-Counter value, as specified in TS 33.401 [30] for EN-DC, or TS 33.501 [11] for NGEN-DC;

3>	derive the KRRCenc and KUPenc keys as specified in TS 33.401 [30] for EN-DC, or TS 33.501 [11] for NGEN-DC;

3>	derive the KRRCint and KUPint keys as specified in TS 33.401 [30] for EN-DC or TS 33.501 [11] for NGEN-DC.

1>	else if this procedure was initiated due to reception of the masterKeyUpdate:

2>	if the nas-Container is included in the received masterKeyUpdate:

3>	forward the nas-Container to the upper layers;

2>	if the keySetChangeIndicator is set to true:

3>	derive or update the KgNB key based on the KAMF key, as specified in TS 33.501 [11];

2>	else:

3>	derive or update the KgNB key based on the current KgNB key or the NH, using the nextHopChainingCount value indicated in the received masterKeyUpdate, as specified in TS 33.501 [11];

2>	store the nextHopChainingCount value;

2>	derive the keys associated with the KgNB key as follows:

3>	if the securityAlgorithmConfig is included in SecurityConfig:

4>	derive the KRRCenc and KUPenc keys associated with the cipheringAlgorithm indicated in the securityAlgorithmConfig, as specified in TS 33.501 [11];

4>	derive the KRRCint and KUPint keys associated with the integrityProtAlgorithm indicated in the securityAlgorithmConfig, as specified in TS 33.501 [11];

3>	else:

4>	derive the KRRCenc and KUPenc keys associated with the current cipheringAlgorithm, as specified in TS 33.501 [11];

4>	derive the KRRCint and KUPint keys associated with the current integrityProtAlgorithm, as specified in TS 33.501 [11].

NOTE 1:	Ciphering and integrity protection are optional to configure for the DRBs.

1>	else if this procedure was initiated due to reception of the sk-Counter (UE is in NE-DC, or NR-DC, or is configured with SN terminated bearer(s)):

2>	derive or update the secondary key (S-KgNB or S-KeNB) based on the KgNB key and using the received sk-Counter value, as specified in TS 33.501 [11];

2>	derive the KRRCenc key and the KUPenc key as specified in TS 33.501 [11] using the ciphering algorithms indicated in the RadioBearerConfig associated with the secondary key (S-KgNB or S-KeNB) as indicated by keyToUse;

2>	derive the KRRCint key and the KUPint key as specified in TS 33.501 [11] using the integrity protection algorithms indicated in the RadioBearerConfig associated with the secondary key (S-KgNB or S-KeNB) as indicated by keyToUse;

NOTE 2:	If the UE has no radio bearer configured with keyToUse set to secondary and receives the sk-Counter without any RadioBearerConfig with keyToUse set to secondary, the UE does not consider it as an invalid reconfiguration.

#### 5.3.5.8	Reconfiguration failure

##### 5.3.5.8.1	Void

##### 5.3.5.8.2	Inability to comply with RRCReconfiguration

NOTE 00:	The UE behaviour specified in this clause does not apply to the following, and the UE ignores, i.e. does not take an action on and does not store, the fields that it does not support or does not comprehend:

-	The fields in ServingCellConfigCommon that are defined in Rel-16 and later.

-	The fields of searchSpaceMCCH and searchSpaceMTCH in PDCCH-ConfigCommon that are defined in Rel-17 and later.

The UE shall:

1>	if the UE is in (NG)EN-DC:

2>	if the UE is unable to comply with (part of) the configuration included in the RRCReconfiguration message received over SRB3;

3>	if the RRCReconfiguration message was received as part of ConditionalReconfiguration:

4>	continue using the configuration used prior to when the inability to comply with the RRCReconfiguration message was detected;

3>	else:

4>	continue using the configuration used prior to the reception of RRCReconfiguration message;

3>	if MCG transmission is not suspended:

4>	initiate the SCG failure information procedure as specified in clause 5.7.3 to report SCG reconfiguration error, upon which the connection reconfiguration procedure ends;

3>	else:

4>	initiate the connection re-establishment procedure as specified in TS 36.331 [10], clause 5.3.7, upon which the connection reconfiguration procedure ends;

2>	else, if the UE is unable to comply with (part of) the configuration included in the RRCReconfiguration message received over SRB1;

3>	if the RRCReconfiguration message was received as part of ConditionalReconfiguration:

4>	continue using the configuration used prior to when the inability to comply with the RRCReconfiguration message was detected;

3>	else:

4>	continue using the configuration used prior to the reception of RRCReconfiguration message;

3>	initiate the connection re-establishment procedure as specified in TS 36.331 [10], clause 5.3.7, upon which the connection reconfiguration procedure ends.

1>	else if RRCReconfiguration is received via NR (i.e., NR standalone, NE-DC, or NR-DC):

2>	if the UE is unable to comply with (part of) the configuration included in the RRCReconfiguration message received over SRB3;

NOTE 0:	This case does not apply in NE-DC.

3>	if the RRCReconfiguration message was received as part of ConditionalReconfiguration:

4>	continue using the configuration used prior to when the inability to comply with the RRCReconfiguration message was detected;

3>	else:

4>	continue using the configuration used prior to the reception of RRCReconfiguration message;

3>	if MCG transmission is not suspended:

4>	initiate the SCG failure information procedure as specified in clause 5.7.3 to report SCG reconfiguration error, upon which the connection reconfiguration procedure ends;

3>	else:

4>	initiate the connection re-establishment procedure as specified in clause 5.3.7, upon which the connection reconfiguration procedure ends;

2>	else if the UE is unable to comply with (part of) the configuration included in the RRCReconfiguration message received over the SRB1 or if the upper layers indicate that the nas-Container is invalid:

NOTE 0a:	The compliance also covers the SCG configuration carried within octet strings e.g. field mrdc-SecondaryCellGroupConfig. I.e. the failure behaviour defined also applies in case the UE cannot comply with the embedded SCG configuration or with the combination of (parts of) the MCG and SCG configurations.

NOTE 0b:	The compliance also covers the V2X sidelink configuration carried within an octet string, e.g. field sl-ConfigDedicatedEUTRA. I.e. the failure behaviour defined also applies in case the UE cannot comply with the embedded V2X sidelink configuration.

3>	if the RRCReconfiguration message was received as part of ConditionalReconfiguration:

4>	continue using the configuration used prior to when the inability to comply with the RRCReconfiguration message was detected;

3>	else:

4>	continue using the configuration used prior to the reception of RRCReconfiguration message;

3>	if AS security has not been activated:

4>	perform the actions upon going to RRC_IDLE as specified in 5.3.11, with release cause 'other'

3>	else if AS security has been activated but SRB2 and at least one DRB or multicast MRB or, for IAB, SRB2, have not been setup:

4>	perform the actions upon going to RRC_IDLE as specified in 5.3.11, with release cause 'RRC connection failure';

3>	else:

4>	initiate the connection re-establishment procedure as specified in 5.3.7, upon which the reconfiguration procedure ends;

1>	else if RRCReconfiguration is received via other RAT (Handover to NR failure):

2>	if the UE is unable to comply with any part of the configuration included in the RRCReconfiguration message or if the upper layers indicate that the nas-Container is invalid:

3>	perform the actions defined for this failure case as defined in the specifications applicable for the other RAT.

NOTE 1:	The UE may apply above failure handling also in case the RRCReconfiguration message causes a protocol error for which the generic error handling as defined in clause 10 specifies that the UE shall ignore the message.

NOTE 2:	If the UE is unable to comply with part of the configuration, it does not apply any part of the configuration, i.e. there is no partial success/failure.

NOTE 3:	It is up to UE implementation whether the compliance check for an RRCReconfiguration received as part of ConditionalReconfiguration is performed upon the reception of the message or upon CHO, CPA and CPC execution (when the message is required to be applied).

##### 5.3.5.8.3	T304 expiry (Reconfiguration with sync Failure) or T420 expiry (Path switch failure)

The UE shall:

1>	if T304 of the MCG expires; or

1> if T420 expires; or,

1> if the target L2 U2N Relay UE (i.e., the UE indicated by targetRelayUE-Identity in the received RRCReconfiguration message containing reconfigurationWithSync indicating path switch as specified in 5.3.5.5.2) changes its serving PCell before path switch:

2>	release dedicated preambles provided in rach-ConfigDedicated if configured;

2>	release dedicated msgA PUSCH resources provided in rach-ConfigDedicated if configured;

2>	if any DAPS bearer is configured, and radio link failure is not detected in the source PCell, according to clause 5.3.10.3:

3>	reset MAC for the target PCell and release the MAC configuration for the target PCell;

3>	for each DAPS bearer:

4>	release the RLC entity or entities as specified in TS 38.322 [4], clause 5.1.3, and the associated logical channel for the target PCell;

4>	reconfigure the PDCP entity to release DAPS as specified in TS 38.323 [5];

3>	for each SRB:

4>	if the masterKeyUpdate was not received:

5>	configure the PDCP entity for the source PCell with state variables continuation as specified in TS 38.323 [5];

4>	release the PDCP entity for the target PCell;

4>	release the RLC entity as specified in TS 38.322 [4], clause 5.1.3, and the associated logical channel for the target PCell;

4>	trigger the PDCP entity for the source PCell to perform SDU discard as specified in TS 38.323 [5];

4>	re-establish the RLC entity for the source PCell;

3>	release the physical channel configuration for the target PCell;

3>	discard the keys used in target PCell (the KgNB key, the KRRCenc key, the KRRCint key, the KUPint key and the KUPenc key), if any;

3>	resume suspended SRBs in the source PCell;

3>	for each non-DAPS bearer:

4>	revert back to the UE configuration used for the DRB or multicast MRB in the source PCell, includes PDCP, RLC states variables, the security configuration and the data stored in transmission and reception buffers in PDCP and RLC entities ;

3>	revert back to the UE measurement configuration used in the source PCell;

3>	store the handover failure information in VarRLF-Report as described in the clause 5.3.10.5;

3>	initiate the failure information procedure as specified in clause 5.7.5 to report DAPS handover failure.

2>	else:

3>	revert back to the UE configuration used in the source PCell;

3>	if the associated T304 was not initiated upon cell selection performed while timer T311 was running, as defined in clause 5.3.7.3:

4>	store the handover failure information in VarRLF-Report as described in the clause 5.3.10.5;

3>	initiate the connection re-establishment procedure as specified in clause 5.3.7.

NOTE 1:	In the context above, "the UE configuration" includes state variables and parameters of each radio bearer.

1>	else if T304 of a secondary cell group expires:

2>	if MCG transmission is not suspended:

3>	release dedicated preambles provided in rach-ConfigDedicated, if configured;

3>	release dedicated msgA PUSCH resources provided in rach-ConfigDedicated, if configured;

3>	initiate the SCG failure information procedure as specified in clause 5.7.3 to report SCG reconfiguration with sync failure, upon which the RRC reconfiguration procedure ends;

2>	else:

3>	if the UE is in NR-DC:

4>	initiate the connection re-establishment procedure as specified in clause 5.3.7;

3>	else (the UE is in (NG) EN-DC):

4>	initiate the connection re-establishment procedure as specified in TS 36.331 [10], clause 5.3.7;

1>	else if T304 expires when RRCReconfiguration is received via other RAT (HO to NR failure):

2>	reset MAC;

2>	perform the actions defined for this failure case as defined in the specifications applicable for the other RAT.

NOTE 2:	In this clause, the term 'handover failure' has been used to refer to 'reconfiguration with sync failure'.

#### 5.3.5.9	Other configuration

The UE shall:

1>	if the received otherConfig includes the delayBudgetReportingConfig:

2>	if delayBudgetReportingConfig is set to setup:

3>	consider itself to be configured to send delay budget reports in accordance with 5.7.4;

2>	else:

3>	consider itself not to be configured to send delay budget reports and stop timer T342, if running.

1>	if the received otherConfig includes the overheatingAssistanceConfig:

2>	if overheatingAssistanceConfig is set to setup:

3>	consider itself to be configured to provide overheating assistance information in accordance with 5.7.4;

2>	else:

3>	consider itself not to be configured to provide overheating assistance information and stop timer T345, if running;

1>	if the received otherConfig includes the idc-AssistanceConfig:

2>	if idc-AssistanceConfig is set to setup:

3>	consider itself to be configured to provide IDC assistance information in accordance with 5.7.4;

2>	else:

3>	consider itself not to be configured to provide IDC assistance information;

1>	if the received otherConfig includes the drx-PreferenceConfig:

2>	if drx-PreferenceConfig is set to setup:

3>	consider itself to be configured to provide its preference on DRX parameters for power saving for the cell group in accordance with 5.7.4;

2>	else:

3>	consider itself not to be configured to provide its preference on DRX parameters for power saving for the cell group and stop timer T346a associated with the cell group, if running;

1>	if the received otherConfig includes the maxBW-PreferenceConfig:

2>	if maxBW-PreferenceConfig is set to setup:

3>	consider itself to be configured to provide its preference on the maximum aggregated bandwidth for power saving for the cell group in accordance with 5.7.4;

3>	if otherConfig includes maxBW-PreferenceConfigFR2-2:

4>	consider itself to be configured to provide its preference on the maximum aggregated bandwidth for FR2-2 for power saving for the cell group in accordance with 5.7.4;

2>	else:

3>	consider itself not to be configured to provide its preference on the maximum aggregated bandwidth for power saving for the cell group and stop timer T346b associated with the cell group, if running;

1>	if the received otherConfig includes the maxCC-PreferenceConfig:

2>	if maxCC-PreferenceConfig is set to setup:

3>	consider itself to be configured to provide its preference on the maximum number of secondary component carriers for power saving for the cell group in accordance with 5.7.4;

2>	else:

3>	consider itself not to be configured to provide its preference on the maximum number of secondary component carriers for power saving for the cell group and stop timer T346c associated with the cell group, if running;

1>	if the received otherConfig includes the maxMIMO-LayerPreferenceConfig:

2>	if maxMIMO-LayerPreferenceConfig is set to setup:

3>	consider itself to be configured to provide its preference on the maximum number of MIMO layers for power saving for the cell group in accordance with 5.7.4;

3>	if otherConfig includes maxMIMO-LayerPreferenceConfigFR2-2:

4>	consider itself to be configured to provide its preference on the maximum number of MIMO layers for FR2-2 for power saving for the cell group in accordance with 5.7.4;

2>	else:

3>	consider itself not to be configured to provide its preference on the maximum number of MIMO layers for power saving for the cell group and stop timer T346d associated with the cell group, if running;

1>	if the received otherConfig includes the minSchedulingOffsetPreferenceConfig:

2>	if minSchedulingOffsetPreferenceConfig is set to setup:

3>	consider itself to be configured to provide its preference on the minimum scheduling offset for cross-slot scheduling for power saving for the cell group in accordance with 5.7.4;

3>	if otherConfig includes minSchedulingOffsetPreferenceConfigExt:

4>	consider itself to be configured to provide its preference on the minimum scheduling offset for 480 kHz SCS and/or 960 kHz SCS for cross-slot scheduling for power saving for the cell group in accordance with 5.7.4;

2>	else:

3>	consider itself not to be configured to provide its preference on the minimum scheduling offset for cross-slot scheduling for power saving for the cell group and stop timer T346e associated with the cell group, if running;

1>	if the received otherConfig includes the releasePreferenceConfig:

2>	if releasePreferenceConfig is set to setup:

3>	consider itself to be configured to provide assistance information to transition out of RRC_CONNECTED in accordance with 5.7.4;

2>	else:

3>	consider itself not to be configured to provide assistance information to transition out of RRC_CONNECTED and stop timer T346f, if running.

1>	if the received otherConfig includes the obtainCommonLocation:

2>	include available detailed location information for any subsequent measurement report or any subsequent RLF report and SCGFailureInformation;

NOTE 1:	The UE is requested to attempt to have valid detailed location information available whenever sending a measurement report for which it is configured to include available detailed location information. The UE may not succeed e.g. because the user manually disabled the GPS hardware, due to no/poor satellite coverage. Further details, e.g. regarding when to activate GNSS, are up to UE implementation.

1>	if the received otherConfig includes the btNameList:

2>	if btNameList is set to setup, include available Bluetooth measurement results for any subsequent measurement report or any subsequent RLF report and SCGFailureInformation;

1>	if the received otherConfig includes the wlanNameList:

2>	if wlanNameList is set to setup, include available WLAN measurement results for any subsequent measurement report or any subsequent RLF report and SCGFailureInformation;

1>	if the received otherConfig includes the sensorNameList:

2>	if sensorNameList is set to setup, include available Sensor measurement results for any subsequent measurement report or any subsequent RLF report and SCGFailureInformation;

NOTE 2:	The UE is requested to attempt to have valid Bluetooth measurements, WLAN measurements and Sensor measurements whenever sending a measurement report for which it is configured to include these measurements. The UE may not succeed e.g. because the user manually disabled the WLAN or Bluetooth or Sensor hardware. Further details, e.g. regarding when to activate WLAN or Bluetooth or Sensor, are up to UE implementation.

1>	if the received otherConfig includes the sl-AssistanceConfigNR:

2>	consider itself to be configured to provide configured grant assistance information for NR sidelink communication in accordance with 5.7.4;

1>	if the received otherConfig includes the referenceTimePreferenceReporting:

2>	consider itself to be configured to provide UE reference time assistance information in accordance with 5.7.4;

1>	else:

2>	consider itself not to be configured to provide UE reference time assistance information;

1>	if the received otherConfig includes the successHO-Config:

2>	consider itself to be configured to provide the successful handover information in accordance with 5.7.10.6;

1>	else:

2>	consider itself not to be configured to provide the successful handover information.

1>	if the received otherConfig includes the ul-GapFR2-PreferenceConfig:

2>	consider itself to be configured to provide its preference on FR2 UL gap in accordance with 5.7.4;

1>	else:

2>	consider itself not to be configured to provide its preference on FR2 UL gap;

1>	if the received otherConfig includes the musim-GapAssistanceConfig:

2>	if musim-GapAssistanceConfig is set to setup:

3>	consider itself to be configured to provide MUSIM assistance information for gap preference in accordance with 5.7.4;

2>	else:

3>	consider itself not to be configured to provide MUSIM assistance information for gap preference and stop timer T346h, if running;

1>	if the received otherConfig includes the musim-LeaveAssistanceConfig:

2>	if musim-LeaveAssistanceConfig is set to setup:

3>	consider itself to be configured to provide MUSIM assistance information for leaving RRC_CONNECTED in accordance with 5.7.4;

2>	else:

3>	consider itself not to be configured to provide MUSIM assistance information for leaving RRC_CONNECTED and stop timer T346g, if running.

1>	if the received otherConfig includes the musim-GapPriorityAssistanceConfig:

2>	consider itself to be configured to provide MUSIM assistance information for gap(s) priority in accordance with 5.7.4;

1>	else:

2>	consider itself not to be configured to provide MUSIM assistance information for gap(s) priority and stop timer T346h, if running;

1>	if the received otherConfig includes the musim-CapabilityRestrictionConfig:

2>	if musim-CapabilityRestrictionConfig is set to setup:

3>	consider itself to be configured to provide MUSIM assistance information for capability restriction in accordance with 5.7.4;

2>	else:

3>	consider itself not to be configured to provide MUSIM assistance information for capability restriction and stop timer T3xx and/or T3xy, if running;

1>	if the received otherConfig includes the rlm-RelaxationReportingConfig:

2>	if rlm-RelaxationReportingConfig is set to setup:

3>	consider itself to be configured to report the relaxation state of RLM measurements in accordance with 5.7.4;

2>	else:

3>	consider itself not to be configured to report the relaxation state of RLM measurements and stop timer T346j associated with the cell group, if running;

1>	if the received otherConfig includes the bfd-RelaxationReportingConfig:

2>	if bfd-RelaxationReportingConfig is set to setup:

3>	consider itself to be configured to report the relaxation state of BFD measurements in accordance with 5.7.4;

2>	else:

3>	consider itself not to be configured to report the relaxation state of BFD measurements and stop timer T346k associated with the cell group, if running;

1>	if the received otherConfig includes the scg-DeactivationPreferenceConfig:

2>	if the scg-DeactivationPreferenceConfig is set to setup:

3>	consider itself to be configured to provide its SCG deactivation preference in accordance with 5.7.4;

2>	else:

3>	consider itself not to be configured to provide its SCG deactivation preference and stop timer T346i, if running.

1>	if the received otherConfig includes the propDelayDiffReportConfig:

2>	if the propDelayDiffReportConfig is set to setup:

3>	consider itself to be configured to provide service link propagation delay difference between serving cell and neighbour cell(s) in accordance with 5.7.4;

2>	else:

3>	consider itself not to be configured to provide service link propagation delay difference between serving cell and neighbour cell(s).

1>	if the received otherConfig includes the rrm-MeasRelaxationReportingConfig:

2>	if the rrm-MeasRelaxationReportingConfig is set to setup:

3>	consider itself to be configured to report the fulfilment of the criterion for relaxing RRM measurements in accordance with 5.7.4;

2>	else:

3>	consider itself not to be configured to report the fulfilment of the criterion for relaxing RRM measurements.

#### 5.3.5.9a	MUSIM gap configuration

The UE shall:

1>	if musim-GapConfig is set to setup:

2>	for each musim-GapId included in the received musim-GapToReleaseList:

3>	release the periodic MUSIM gap configuration associated with the musim-GapId;

2>	for each MUSIM-Gap included in the received musim-GapToAddModList:

3>	setup periodic MUSIM gap configuration indicated by the MUSIM-Gap in accordance with the received musim-GapRepetitionAndOffset (providing musim-GapRepetition and Offset value for the following condition) i.e. the first subframe of each periodic MUSIM gap occurs at an SFN and subframe of the NR PCell meeting the following condition:

SFN mod T = FLOOR(Offset/10);

subframe = Offset mod 10;

with T = musim-GapRepetition/10;

3>	set the MUSIM gap priority configuration indicated by musim-GapPriorityToAddModList, if configured, for each periodic MUSIM gap;

NOTE:	If network does not retain the relative priorities among MUSIM gaps, UE behaviour is not specified.

2>	if musim-AperiodicGap is included:

3>	setup aperiodic MUSIM gap configuration indicated by the musim-AperiodicGap in accordance with the received musim-Starting-SFN-AndSubframe, i.e. the first subframe of aperiodic MUSIM gap occurs at an SFN and subframe of the NR PCell meeting the following condition:

SFN = starting-SFN;

subframe = startingSubframe;

1>	else if musim-GapConfig is set to release:

2>	release the MUSIM gap configuration.

#### 5.3.5.10	MR-DC release

The UE shall:

1>	as a result of MR-DC release triggered by E-UTRA or NR:

2>	release SRB3, if established, as specified in 5.3.5.6.2;

2>	release measConfig associated with SCG;

2>	if the UE is configured with NR SCG:

3>	release the SCG configuration as specified in clause 5.3.5.4;

3>	release otherConfig associated with the SCG, if configured;

3>	stop timers T346a, T346b, T346c, T346d, T346e, T346j and T346k associated with the SCG, if running;

3>	release bap-Config associated with the SCG, if configured;

3>	release the BAP entity as specified in TS 38.340 [47], if there is no configured bap-Config;

3>	release iab-IP-AddressConfigurationList associated with the SCG, if configured;

2>	else if the UE is configured with E-UTRA SCG:

3>	release the SCG configuration as specified in TS 36.331 [10], clause 5.3.10.19 to release the E-UTRA SCG;

#### 5.3.5.11	Full configuration

The UE shall:

1>	release/ clear all current dedicated radio configurations except for the following:

-	the MCG C-RNTI;

-	the AS security configurations associated with the master key;

-	the SRB1/SRB2 configurations and DRB/multicast MRB configurations as configured by radioBearerConfig or radioBearerConfig2.

NOTE 1:	Radio configuration is not just the resource configuration but includes other configurations like MeasConfig. Radio configuration also includes the RLC bearer configurations as configured by RLC-BearerConfig, PC5 Relay RLC channel as configured by SL-RLC-ChannelConfig, and Uu Relay RLC channel as configured by Uu-RelayRLC-ChannelConfig. In case NR-DC or NE-DC is configured, this also includes the entire NR or E-UTRA SCG configuration which are released according to the MR-DC release procedure as specified in 5.3.5.10.

NOTE 1a:	For NR sidelink communication/discovery, the radio configuration includes the sidelink RRC configuration received from the network, but does not include the sidelink RRC reconfiguration and sidelink UE capability received from other UEs via PC5-RRC. In addition, the UE considers the new NR sidelink configurations as full configuration, in case of state transition and change of system information used for NR sidelink communication/discovery.

NOTE 1b:	To establish the RLC bearer of SRB(s) after release due to fullConfig, the network can include the srb-Identity within srb-ToAddModList (i.e. the UE applies RLC default configuration) and/or provide rlc-BearerToAddModList of concerned SRB(s) explicitly.

-	the logged measurement configuration;

1>	if the spCellConfig in the masterCellGroup includes the reconfigurationWithSync:

2>	release/ clear all current common radio configurations;

2>	if sl-PathSwitchConfig was included in reconfigurationWithSync:

3>	use the default values specified in 9.2.3 for timer T311;

2>	else:

3>	use the default values specified in 9.2.3 for timers T310, T311 and constants N310, N311;

1>	else (full configuration after re-establishment or during RRC resume):

2>	if the UE is acting as L2 U2N Remote UE:

3>	use value for timer T311, as included in ue-TimersAndConstants received in SIB1

2>	else:

3>	use values for timers T301, T310, T311 and constants N310, N311, as included in ue-TimersAndConstants received in SIB1;

1>	if no measConfigAppLayerId is included:

2>	inform upper layers about the release of all application layer measurement configurations;

2>	discard any received application layer measurement report from upper layers;

2>	consider itself not to be configured to send application layer measurement report.

1>	if the UE is acting as L2 U2N Remote UE at the target side during reconfiguration with sync, or after re-establishment, or during RRC resume:

2>	apply the default configuration of SL-RLC1 as specified in clause 9.2.4 and associate it with the SRB1;

1>	else:

2>	apply the default L1 parameter values as specified in corresponding physical layer specifications except for the following:

-	parameters for which values are provided in SIB1;

2>	apply the default MAC Cell Group configuration as specified in 9.2.2;

2>	for each srb-Identity value included in the srb-ToAddModList (SRB reconfiguration):

3>	establish an RLC entity for the corresponding SRB;

3>	apply the default SRB configuration defined in 9.2.1 for the corresponding SRB;

NOTE 2:	This is to get the SRBs (SRB1 and SRB2 for reconfiguration with sync and SRB2 for resume and reconfiguration after re-establishment) to a known state from which the reconfiguration message can do further configuration.

1>	for each pdu-Session that is part of the current UE configuration:

2>	release the SDAP entity (clause 5.1.2 in TS 37.324 [24]);

2>	release each DRB associated to the pdu-Session as specified in 5.3.5.6.4;

NOTE 3:	This will retain the pdu-Session but remove the DRBs including drb-identity of these bearers from the current UE configuration. Setup of the DRBs within the AS is described in clause 5.3.5.6.5 using the new configuration. The pdu-Session acts as the anchor for associating the released and re-setup DRB. In the AS the DRB re-setup is equivalent with a new DRB setup (including new PDCP and logical channel configurations).

1>	for each mbs-SessionId that is part of the current UE configuration and associated to a multicast MRB:

2>	release the SDAP entity (clause 5.1.2 in TS 37.324 [24]);

2>	release each multicast MRB associated to the mbs-SessionId as specified in 5.3.5.6.6;

NOTE 4:	This will retain the mbs-SessionId but remove the multicast MRBs including mrb-identity of these bearers from the current UE configuration. Setup of the multicast MRBs within the AS is described in clause 5.3.5.6.7 using the new configuration. The mbs-SessionId acts as the anchor for associating the released and re-setup multicast MRB. In the AS the multicast MRB re-setup is equivalent with a new multicast MRB setup (including new PDCP and logical channel configurations).

1>	for each pdu-Session that is part of the current UE configuration but not added with same pdu-Session in the drb-ToAddModList:

2>	if the procedure was triggered due to reconfiguration with sync:

3>	indicate the release of the user plane resources for the pdu-Session to upper layers after successful reconfiguration with sync;

2>	else:

3>	indicate the release of the user plane resources for the pdu-Session to upper layers immediately;

1>	for each mbs-SessionId that is part of the current UE configuration but not added with the same mbs-SessionId in the mrb-ToAddModList:

2>	if the procedure was triggered due to reconfiguration with sync:

3>	indicate the release of the user plane resources for the mbs-SessionId to upper layers after successful reconfiguration with sync;

2>	else:

3>	indicate the release of the user plane resources for the mbs-SessionId to upper layers immediately.

-------------------------------------------Skip Unchanged-----------------------------------------------------

#### 5.3.5.x	T3xx expiry

The UE can apply the temporary UE capability restriction that the UE indicated this time:

---------------------------------------Skip Unchanged---------------------------------------------------

### 5.3.7	RRC connection re-establishment

#### 5.3.7.1	General

Figure 5.3.7.1-1: RRC connection re-establishment, successful

Figure 5.3.7.1-2: RRC re-establishment, fallback to RRC establishment, successful

The purpose of this procedure is to re-establish the RRC connection. A UE in RRC_CONNECTED, for which AS security has been activated with SRB2 and at least one DRB/multicast MRB setup or, for IAB, SRB2, may initiate the procedure in order to continue the RRC connection. The connection re-establishment succeeds if the network is able to find and verify a valid UE context or, if the UE context cannot be retrieved, and the network responds with an RRCSetup according to clause 5.3.3.4.

The network applies the procedure e.g as follows:

-	When AS security has been activated and the network retrieves or verifies the UE context:

-	to re-activate AS security without changing algorithms;

-	to re-establish and resume the SRB1;

-	When UE is re-establishing an RRC connection, and the network is not able to retrieve or verify the UE context:

-	to discard the stored AS Context and release all RBs and BH RLC channels and Uu Relay RLC channels;

-	to fallback to establish a new RRC connection.

If AS security has not been activated, the UE shall not initiate the procedure but instead moves to RRC_IDLE directly, with release cause 'other'. If AS security has been activated, but SRB2 and at least one DRB or multicast MRB or, for IAB, SRB2, are not setup, the UE does not initiate the procedure but instead moves to RRC_IDLE directly, with release cause 'RRC connection failure'.

#### 5.3.7.2	Initiation

The UE initiates the procedure when one of the following conditions is met:

1>	upon detecting radio link failure of the MCG and t316 is not configured, in accordance with 5.3.10; or

1>	upon detecting radio link failure of the MCG while SCG transmission is suspended, in accordance with 5.3.10; or

1>	upon detecting radio link failure of the MCG while PSCell change or PSCell addition is ongoing, in accordance with 5.3.10; or

1>	upon detecting radio link failure of the MCG while the SCG is deactivated, in accordance with 5.3.10; or

1>	upon re-configuration with sync failure of the MCG, in accordance with clause 5.3.5.8.3; or

1>	upon mobility from NR failure, in accordance with clause 5.4.3.5; or

1>	upon integrity check failure indication from lower layers concerning SRB1 or SRB2, except if the integrity check failure is detected on the RRCReestablishment message; or

1>	upon an RRC connection reconfiguration failure, in accordance with clause 5.3.5.8.2; or

1>	upon detecting radio link failure for the SCG while MCG transmission is suspended, in accordance with clause 5.3.10.3 in NR-DC or in accordance with TS 36.331 [10] clause 5.3.11.3 in NE-DC; or

1>	upon reconfiguration with sync failure of the SCG while MCG transmission is suspended in accordance with clause 5.3.5.8.3; or

1>	upon SCG change failure while MCG transmission is suspended in accordance with TS 36.331 [10] clause 5.3.5.7a; or

1>	upon SCG configuration failure while MCG transmission is suspended in accordance with clause 5.3.5.8.2 in NR-DC or in accordance with TS 36.331 [10] clause 5.3.5.5 in NE-DC; or

1>	upon integrity check failure indication from SCG lower layers concerning SRB3 while MCG is suspended; or

1>	upon T316 expiry, in accordance with clause 5.7.3b.5; or

1>	upon detecting sidelink radio link failure by L2 U2N Remote UE in RRC_CONNECTED, in accordance with clause 5.8.9.3; or

1>	upon reception of NotificationMessageSidelink including indicationType by L2 U2N Remote UE in RRC_CONNECTED, in accordance with clause 5.8.9.10; or

1>	upon PC5 unicast link release indicated by upper layer at L2 U2N Remote UE in RRC_CONNECTED.

NOTE 0:	It is up to UE implementation whether to initiate the procedure while T346g is running.

Upon initiation of the procedure, the UE shall:

1>	stop timer T310, if running;

1>	stop timer T312, if running;

1>	stop timer T304, if running;

1>	start timer T311;

1>	stop timer T316, if running;

1>	if UE is not configured with attemptCondReconfig:

2>	reset MAC;

2>	release spCellConfig, if configured;

2>	suspend all RBs, and BH RLC channels for IAB-MT, and Uu Relay RLC channels for L2 U2N Relay UE, except SRB0 and broadcast MRBs;

2>	release the MCG SCell(s), if configured;

2>	if MR-DC is configured:

3>	perform MR-DC release, as specified in clause 5.3.5.10;

2>	release delayBudgetReportingConfig, if configured and stop timer T342, if running;

2>	release overheatingAssistanceConfig, if configured and stop timer T345, if running;

2>	release idc-AssistanceConfig, if configured;

2>	release btNameList, if configured;

2>	release wlanNameList, if configured;

2>	release sensorNameList, if configured;

2>	release drx-PreferenceConfig for the MCG, if configured and stop timer T346a associated with the MCG, if running;

2>	release maxBW-PreferenceConfig for the MCG, if configured and stop timer T346b associated with the MCG, if running;

2>	release maxCC-PreferenceConfig for the MCG, if configured and stop timer T346c associated with the MCG, if running;

2>	release maxMIMO-LayerPreferenceConfig for the MCG, if configured and stop timer T346d associated with the MCG, if running;

2>	release minSchedulingOffsetPreferenceConfig for the MCG, if configured stop timer T346e associated with the MCG, if running;

2>	release rlm-RelaxationReportingConfig for the MCG, if configured and stop timer T346j associated with the MCG, if running;

2>	release bfd-RelaxationReportingConfig for the MCG, if configured and stop timer T346k associated with the MCG, if running;

2>	release releasePreferenceConfig, if configured stop timer T346f, if running;

2>	release onDemandSIB-Request if configured, and stop timer T350, if running;

2>	release referenceTimePreferenceReporting, if configured;

2>	release sl-AssistanceConfigNR, if configured;

2>	release obtainCommonLocation, if configured;

2>	release musim-GapAssistanceConfig, if configured and stop timer T346h, if running;

2>	release musim-LeaveAssistanceConfig, if configured;

2>	release musim-GapPriorityAssistanceConfig, if configured;

2>	release musim-CapabilityRestrictionConfig, if configured and stop timer T3xx, if running;

2>	release ul-GapFR2-PreferenceConfig, if configured;

2>	release scg-DeactivationPreferenceConfig, if configured, and stop timer T346i, if running;

2>	release propDelayDiffReportConfig, if configured;

2>	release rrm-MeasRelaxationReportingConfig, if configured;

2>	release maxBW-PreferenceConfigFR2-2, if configured;

2>	release maxMIMO-LayerPreferenceConfigFR2-2, if configured;

2>	release minSchedulingOffsetPreferenceConfigExt, if configured;

1>	release successHO-Config, if configured;

1>	if any DAPS bearer is configured:

2>	reset the source MAC and release the source MAC configuration;

2>	for each DAPS bearer:

3>	release the RLC entity or entities as specified in TS 38.322 [4], clause 5.1.3, and the associated logical channel for the source SpCell;

3>	reconfigure the PDCP entity to release DAPS as specified in TS 38.323 [5];

2>	for each SRB:

3>	release the PDCP entity for the source SpCell;

3>	release the RLC entity as specified in TS 38.322 [4], clause 5.1.3, and the associated logical channel for the source SpCell;

2>	release the physical channel configuration for the source SpCell;

2>	discard the keys used in the source SpCell (the KgNB key, the KRRCenc key, the KRRCint key, the KUPint key and the KUPenc key), if any;

1>	release sl-L2RelayUE-Config, if configured;

1>	release sl-L2RemoteUE-Config, if configured;

1>	release the SRAP entity, if configured;

1>	if the UE is acting as L2 U2N Remote UE:

2>	if the PC5-RRC connection with the U2N Relay UE is determined to be released:

3>	indicate upper layers to trigger PC5 unicast link release;

3>	perform either cell selection in accordance with the cell selection process as specified in TS 38.304 [20], or relay selection as specified in clause 5.8.15.3, or both;

2>	else (i.e., maintain the PC5 RRC connection):

3>	consider the connected L2 U2N Relay UE as suitable and perform actions as specified in clause 5.3.7.3a;

NOTE 1:	It is up to Remote UE implementation whether to release or keep the current PC5 unicast link.

1> else:

2>	if the UE is capable of L2 U2N Remote UE:

3>	perform either cell selection as specified in TS 38.304 [20], or relay selection as specified in clause 5.8.15.3, or both;

2>	else:

3>	perform cell selection in accordance with the cell selection process as specified in TS 38.304 [20].

NOTE 2:	For L2 U2N Remote UE, if both a suitable cell and a suitable relay are available, the UE can select either one based on its implementation.

#### 5.3.7.3	Actions following cell selection while T311 is running

Upon selecting a suitable NR cell, the UE shall:

1>	ensure having valid and up to date essential system information as specified in clause 5.2.2.2;

1>	stop timer T311;

1>	if T390 is running:

2>	stop timer T390 for all access categories;

2>	perform the actions as specified in 5.3.14.4;

1>	stop the relay (re)selection procedure, if ongoing;

1>	if the cell selection is triggered by detecting radio link failure of the MCG or re-configuration with sync failure of the MCG or mobility from NR failure, and

1>	if attemptCondReconfig is configured; and

1>	if the selected cell is not configured with CondEventT1, or the selected cell is configured with CondEventT1 and leaving condition has not been fulfilled; and

1>	if the selected cell is one of the candidate cells for which the reconfigurationWithSync is included in the masterCellGroup in the MCG VarConditionalReconfig:

2>	if the UE supports RLF-Report for conditional handover, set the choCellId in the VarRLF-Report to the global cell identity, if available, otherwise to the physical cell identity and carrier frequency of the selected cell;

2>	apply the stored condRRCReconfig associated to the selected cell and perform actions as specified in 5.3.5.3;

NOTE 1:	It is left to network implementation to how to avoid keystream reuse in case of CHO based recovery after a failed handover without key change.

1>	else:

2>	if UE is configured with attemptCondReconfig:

3>	reset MAC;

3>	release spCellConfig, if configured;

3>	release the MCG SCell(s), if configured;

3>	release delayBudgetReportingConfig, if configured and stop timer T342, if running;

3>	release overheatingAssistanceConfig , if configured and stop timer T345, if running;

3>	if MR-DC is configured:

4>	perform MR-DC release, as specified in clause 5.3.5.10;

3>	release idc-AssistanceConfig, if configured;

3>	release btNameList, if configured;

3>	release wlanNameList, if configured;

3>	release sensorNameList, if configured;

3>	release drx-PreferenceConfig for the MCG, if configured and stop timer T346a associated with the MCG, if running;

3>	release maxBW-PreferenceConfig for the MCG, if configured and stop timer T346b associated with the MCG, if running;

3>	release maxCC-PreferenceConfig for the MCG, if configured and stop timer T346c associated with the MCG, if running;

3>	release maxMIMO-LayerPreferenceConfig for the MCG, if configured and stop timer T346d associated with the MCG, if running;

3>	release minSchedulingOffsetPreferenceConfig for the MCG, if configured and stop timer T346e associated with the MCG, if running;

3>	release rlm-RelaxationReportingConfig for the MCG, if configured and stop timer T346j associated with the MCG, if running;

3>	release bfd-RelaxationReportingConfig for the MCG, if configured and stop timer T346k associated with the MCG, if running;

3>	release releasePreferenceConfig, if configured and stop timer T346f, if running;

3>	release onDemandSIB-Request if configured, and stop timer T350, if running;

3>	release referenceTimePreferenceReporting, if configured;

3>	release sl-AssistanceConfigNR, if configured;

3>	release obtainCommonLocation, if configured;

3>	release scg-DeactivationPreferenceConfig, if configured, and stop timer T346i, if running;

3>	release musim-GapAssistanceConfig, if configured and stop timer T346h, if running;

3>	release musim-LeaveAssistanceConfig, if configured;

3>	release propDelayDiffReportConfig, if configured;

3>	release ul-GapFR2-PreferenceConfig, if configured;

3>	release rrm-MeasRelaxationReportingConfig, if configured;

3>	release maxBW-PreferenceConfigFR2-2, if configured;

3>	release maxMIMO-LayerPreferenceConfigFR2-2, if configured;

3>	release minSchedulingOffsetPreferenceConfigExt, if configured;

3>	suspend all RBs, and BH RLC channels for the IAB-MT, except SRB0 and broadcast MRBs;

2>	remove all the entries within the MCG VarConditionalReconfig, if any;

2>	for each measId, if the associated reportConfig has a reportType set to condTriggerConfig:

3>	for the associated reportConfigId:

4>	remove the entry with the matching reportConfigId from the reportConfigList within the VarMeasConfig;

3>	if the associated measObjectId is only associated to a reportConfig with reportType set to condTriggerConfig:

4>	remove the entry with the matching measObjectId from the measObjectList within the VarMeasConfig;

3>	remove the entry with the matching measId from the measIdList within the VarMeasConfig;

2>	release the PC5 RLC entity for SL-RLC0, if any;

2>	start timer T301;

2>	apply the default L1 parameter values as specified in corresponding physical layer specifications except for the parameters for which values are provided in SIB1;

2>	apply the default MAC Cell Group configuration as specified in 9.2.2;

2>	apply the CCCH configuration as specified in 9.1.1.2;

2>	apply the timeAlignmentTimerCommon included in SIB1;

2>	initiate transmission of the RRCReestablishmentRequest message in accordance with 5.3.7.4;

NOTE 2:	This procedure applies also if the UE returns to the source PCell.

Upon selecting an inter-RAT cell, the UE shall:

1>	perform the actions upon going to RRC_IDLE as specified in 5.3.11, with release cause 'RRC connection failure'.

#### 5.3.7.3a	Actions following relay selection while T311 is running

Upon selecting a suitable L2 U2N Relay UE, the L2 U2N Remote UE shall:

1>	indicate to upper layer to trigger the PC5 unicast link establishment with the selected L2 U2N Relay UE, if a new L2 U2N Relay UE is selected;

1>	ensure having valid and up to date essential system information as specified in clause 5.2.2.2;

1>	stop timer T311;

1>	if T390 is running:

2>	stop timer T390 for all access categories;

2>	perform the actions as specified in 5.3.14.4;

1>	stop the cell selection procedure, if ongoing;

1>	start timer T301;

1>	release the RLC entity for SRB0, if any;

1>	establish a SRAP entity as specified in TS 38.351 [66], if no SRAP entity has been established;

1>	apply the specified configuration of SL-RLC0 as specified in 9.1.1.4;

1> apply the SDAP configuration and PDCP configuration as specified in 9.1.1.2 for SRB0;

1>	initiate transmission of the RRCReestablishmentRequest message in accordance with 5.3.7.4.

#### 5.3.7.4	Actions related to transmission of RRCReestablishmentRequest message

The UE shall set the contents of RRCReestablishmentRequest message as follows:

1>	if the procedure was initiated due to radio link failure as specified in 5.3.10.3 or reconfiguration with sync failure as specified in 5.3.5.8.3:

2>	set the reestablishmentCellId in the VarRLF-Report to the global cell identity of the selected cell;

1>	set the ue-Identity as follows:

2>	set the c-RNTI to the C-RNTI used in the source PCell (reconfiguration with sync or mobility from NR failure) or used in the PCell in which the trigger for the re-establishment occurred (other cases);

2>	set the physCellId to the physical cell identity of the source PCell (reconfiguration with sync or mobility from NR failure) or of the PCell in which the trigger for the re-establishment occurred (other cases);

2>	set the shortMAC-I to the 16 least significant bits of the MAC-I calculated:

3>	over the ASN.1 encoded as per clause 8 (i.e., a multiple of 8 bits) VarShortMAC-Input;

3>	with the KRRCint key and integrity protection algorithm that was used in the source PCell (reconfiguration with sync or mobility from NR failure) or of the PCell in which the trigger for the re-establishment occurred (other cases); and

3>	with all input bits for COUNT, BEARER and DIRECTION set to binary ones;

1>	set the reestablishmentCause as follows:

2>	if the re-establishment procedure was initiated due to reconfiguration failure as specified in 5.3.5.8.2:

3>	set the reestablishmentCause to the value reconfigurationFailure;

2>	else if the re-establishment procedure was initiated due to reconfiguration with sync failure as specified in 5.3.5.8.3 (intra-NR handover failure) or 5.4.3.5 (inter-RAT mobility from NR failure):

3>	set the reestablishmentCause to the value handoverFailure;

2>	else:

3>	set the reestablishmentCause to the value otherFailure;

1>	re-establish PDCP for SRB1;

1>	if the UE is acting as L2 U2N Remote UE:

2>	establish or re-established (e.g. via release and add) SL RLC entity for SRB1;

2>	apply the default configuration of SL-RLC1 as defined in 9.2.4 for SRB1;

2>	apply the default configuration of PDCP as defined in 9.2.1 for SRB1;

2>	apply the default configuration of SRAP as defined in 9.2.5 for SRB1;

1> else:

2>	re-establish RLC for SRB1;

2>	apply the default configuration defined in 9.2.1 for SRB1;

1>	configure lower layers to suspend integrity protection and ciphering for SRB1;

NOTE:	Ciphering is not applied for the subsequent RRCReestablishment message used to resume the connection. An integrity check is performed by lower layers, but merely upon request from RRC.

1>	resume SRB1;

1>	if ta-Report is configured with value enabled and the UE supports TA reporting:

2>	indicate TA report initiation to lower layers;

1>	submit the RRCReestablishmentRequest message to lower layers for transmission.

#### 5.3.7.5	Reception of the RRCReestablishment by the UE

The UE shall:

1>	stop timer T301;

1>	consider the current cell to be the PCell;

1>	update the KgNB key based on the current KgNB key or the NH, using the received nextHopChainingCount value, as specified in TS 33.501 [11];

1>	store the nextHopChainingCount value indicated in the RRCReestablishment message;

1>	derive the KRRCenc and KUPenc keys associated with the previously configured cipheringAlgorithm, as specified in TS 33.501 [11];

1>	derive the KRRCint and KUPint keys associated with the previously configured integrityProtAlgorithm, as specified in TS 33.501 [11].

1>	request lower layers to verify the integrity protection of the RRCReestablishment message, using the previously configured algorithm and the KRRCint key;

1>	if the integrity protection check of the RRCReestablishment message fails:

2>	perform the actions upon going to RRC_IDLE as specified in 5.3.11, with release cause 'RRC connection failure', upon which the procedure ends;

1>	configure lower layers to resume integrity protection for SRB1 using the previously configured algorithm and the KRRCint key immediately, i.e., integrity protection shall be applied to all subsequent messages received and sent by the UE, including the message used to indicate the successful completion of the procedure;

1>	configure lower layers to resume ciphering for SRB1 using the previously configured algorithm and, the KRRCenc key immediately, i.e., ciphering shall be applied to all subsequent messages received and sent by the UE, including the message used to indicate the successful completion of the procedure;

1>	release the measurement gap configuration indicated by the measGapConfig, if configured;

1>	release the MUSIM gap configuration indicated by the musim-GapConfig, if configured;

1>	release the FR2 UL gap configuration indicated by the ul-GapFR2-Config, if configured;

1>	perform the L2 U2N Remote UE configuration procedure in accordance with the received sl-L2RemoteUE-Config as specified in 5.3.5.16;

1>	set the content of RRCReestablishmentComplete message as follows:

2>	if the UE has logged measurements available for NR and if the RPLMN is included in plmn-IdentityList stored in VarLogMeasReport:

3>	include the logMeasAvailable in the RRCReestablishmentComplete message;

3>	if Bluetooth measurement results are included in the logged measurements the UE has available for NR:

4>	include the logMeasAvailableBT in the RRCReestablishmentComplete message;

3>	if WLAN measurement results are included in the logged measurements the UE has available for NR:

4>	include the logMeasAvailableWLAN in the RRCReestablishmentComplete message;

2>	if the sigLoggedMeasType in VarLogMeasReport is included:

3>	if T330 timer is running and the logged measurements configuration is for NR:

4>	set sigLogMeasConfigAvailable to true in the RRCReestablishmentComplete message;

3>	else:

4>	if the UE has logged measurements available for NR:

5>	set sigLogMeasConfigAvailable to false in the RRCReestablishmentComplete message;

2>	if the UE has connection establishment failure or connection resume failure information available in VarConnEstFailReport or VarConnEstFailReportList and if the RPLMN is equal to plmn-Identity stored in VarConnEstFailReport or in at least one of the entries of VarConnEstFailReportList:

3>	include connEstFailInfoAvailable in the RRCReestablishmentComplete message;

2>	if the UE has radio link failure or handover failure information available in VarRLF-Report and if the RPLMN is included in plmn-IdentityList stored in VarRLF-Report; or

2>	if the UE has radio link failure or handover failure information available in VarRLF-Report of TS 36.331 [10] and if the UE is capable of cross-RAT RLF reporting and if the RPLMN is included in plmn-IdentityList stored in VarRLF-Report of TS 36.331 [10]:

3>	include rlf-InfoAvailable in the RRCReestablishmentComplete message;

2>	if the UE has successful handover information available in VarSuccessHO-Report and if the RPLMN is included in plmn-IdentityList stored in VarSuccessHO-Report:

3>	include successHO-InfoAvailable in the RRCReestablishmentComplete message;

1>	submit the RRCReestablishmentComplete message to lower layers for transmission;

1>	if SIB21 is provided by the PCell:

2>	if the UE initiated transmission of an MBSInterestIndication message during the last 1 second preceding detection of radio link failure:

3>	initiate transmission of an MBSInterestIndication message in accordance with 5.9.4;

1>	the procedure ends.

#### 5.3.7.6	T311 expiry

Upon T311 expiry, the UE shall:

1>	if the procedure was initiated due to radio link failure or handover failure:

2>	set the noSuitableCellFound in the VarRLF-Report to true;

1>	perform the actions upon going to RRC_IDLE as specified in 5.3.11, with release cause 'RRC connection failure'.

#### 5.3.7.7	T301 expiry or selected cell/L2 U2N Relay UE no longer suitable

The UE shall:

1>	if timer T301 expires; or

1>	if the selected cell becomes no longer suitable according to the cell selection criteria as specified in TS 38.304 [20]; or

1>	if the (re)selected L2 U2N Relay UE becomes unsuitable; or

1>	upon receiption of NotificationMessageSidelink indicating relayUE-HO or relayUE-CellReselection:

2>	perform the actions upon going to RRC_IDLE as specified in 5.3.11, with release cause 'RRC connection failure'.

#### 5.3.7.8	Reception of the RRCSetup by the UE

The UE shall:

1>	perform the RRC connection establishment procedure as specified in 5.3.3.4.

### 5.3.8	RRC connection release

#### 5.3.8.1	General

Figure 5.3.8.1-1: RRC connection release, successful

The purpose of this procedure is:

-	to release the RRC connection, which includes the release of the established radio bearers (except for broadcast MRBs), BH RLC channels, Uu Relay RLC channels, PC5 Relay RLC channels as well as all radio resources; or

-	to suspend the RRC connection only if SRB2 and at least one DRB or multicast MRB or, for IAB, SRB2, are setup, which includes the suspension of the established radio bearers (except for broadcast MRBs).

#### 5.3.8.2	Initiation

The network initiates the RRC connection release procedure to transit a UE in RRC_CONNECTED to RRC_IDLE; or to transit a UE in RRC_CONNECTED to RRC_INACTIVE only if SRB2 and at least one DRB or multicast MRB or, for IAB, SRB2, is setup in RRC_CONNECTED; or to transit a UE in RRC_INACTIVE back to RRC_INACTIVE when the UE tries to resume (for resuming a suspended RRC connection or for initiating SDT); or to transit a UE in RRC_INACTIVE to RRC_IDLE when the UE tries to resume (for resuming of a suspended RRC connection or for initiating SDT). The procedure can also be used to release and redirect a UE to another frequency.

#### 5.3.8.3	Reception of the RRCRelease by the UE

The UE shall:

1>	delay the following actions defined in this clause 60 ms from the moment the RRCRelease message was received or optionally when lower layers indicate that the receipt of the RRCRelease message has been successfully acknowledged, whichever is earlier;

1>	stop timer T380, if running;

1>	stop timer T320, if running;

1>	if timer T316 is running;

2>	stop timer T316;

2>	clear the information included in VarRLF-Report, if any;

1>	stop timer T350, if running;

1>	stop timer T346g, if running;

1>	if the AS security is not activated:

2>	ignore any field included in RRCRelease message except waitTime;

2>	perform the actions upon going to RRC_IDLE as specified in 5.3.11 with the release cause 'other' upon which the procedure ends;

1>	if the RRCRelease message includes redirectedCarrierInfo indicating redirection to eutra:

2>	if cnType is included:

3>	after the cell selection, indicate the available CN Type(s) and the received cnType to upper layers;

NOTE 1:	Handling the case if the E-UTRA cell selected after the redirection does not support the core network type specified by the cnType, is up to UE implementation.

2>	if voiceFallbackIndication is included:

3>	consider the RRC connection release was for EPS fallback for IMS voice (see TS 23.502 [43]);

1>	if the RRCRelease message includes the cellReselectionPriorities:

2>	store the cell reselection priority information provided by the cellReselectionPriorities;

2>	if the t320 is included:

3>	start timer T320, with the timer value set according to the value of t320;

1>	else:

2>	apply the cell reselection priority information broadcast in the system information;

1>	if deprioritisationReq is included and the UE supports RRC connection release with deprioritisation:

2>	start or restart timer T325 with the timer value set to the deprioritisationTimer signalled;

2>	store the deprioritisationReq until T325 expiry;

NOTE 1a:	The UE stores the deprioritisation request irrespective of any cell reselection absolute priority assignments (by dedicated or common signalling) and regardless of RRC connections in NR or other RATs unless specified otherwise.

1>	if the RRCRelease includes the measIdleConfig:

2>	if T331 is running:

3> stop timer T331;

3>	perform the actions as specified in 5.7.8.3;

2>	if the measIdleConfig is set to setup:

3>	store the received measIdleDuration in VarMeasIdleConfig;

3>	start timer T331 with the value set to measIdleDuration;

3>	if the measIdleConfig contains measIdleCarrierListNR:

4>	store the received measIdleCarrierListNR in VarMeasIdleConfig;

3>	if the measIdleConfig contains measIdleCarrierListEUTRA:

4>	store the received measIdleCarrierListEUTRA in VarMeasIdleConfig;

3>	if the measIdleConfig contains validityAreaList:

4>	store the received validityAreaList in VarMeasIdleConfig;

1>	if the RRCRelease includes suspendConfig:

2>	reset MAC and release the default MAC Cell Group configuration, if any;

2>	apply the received suspendConfig except the received nextHopChainingCount;

2>	if the sdt-Config is configured:

3>	for each of the DRB in the sdt-DRB-List:

4>	consider the DRB to be configured for SDT;

3>	if sdt-SRB2-Indication is configured:

4>	consider the SRB2 to be configured for SDT;

3>	for each RLC bearer (except those associated with broadcast MRBs) that is not suspended:

4>	re-establish the RLC entity as specified in TS 38.322 [4];

3>	for SRB2 (if it is resumed) and for SRB1:

4>	trigger the PDCP entity to perform SDU discard as specified in TS 38.323 [5];

3>	if sdt-MAC-PHY-CG-Config is configured:

4>	configure the PCell with the configured grant resources for SDT and instruct the MAC entity to start the cg-SDT-TimeAlignmentTimer;

2>	if srs-PosRRC-Inactive is configured:

3>	apply the configuration and instruct MAC to start the inactivePosSRS-TimeAlignmentTimer;

NOTE 1b:	The Network should provide full configuration to UE for SRS for Positioning in RRC_INACTIVE.

2>	remove all the entries within the MCG and the SCG VarConditionalReconfig, if any;

2>	for each measId of the MCG measConfig and for each measId of the SCG measConfig, if configured, if the associated reportConfig has a reportType set to condTriggerConfig:

3>	for the associated reportConfigId:

4>	remove the entry with the matching reportConfigId from the reportConfigList within the VarMeasConfig;

3>	if the associated measObjectId is only associated to a reportConfig with reportType set to condTriggerConfig:

4>	remove the entry with the matching measObjectId from the measObjectList within the VarMeasConfig;

3>	remove the entry with the matching measId from the measIdList within the VarMeasConfig;

2>	if the UE is acting as L2 U2N Remote UE:

3>	if the PC5-RRC connection with the U2N Relay UE is determined to be released:

4>	indicate upper layers to trigger PC5 unicast link release;

3>	else (i.e., maintain the PC5 RRC connection):

4>	establish or re-establish (e.g. via release and add) SL RLC entity for SRB1;

2>	else:

3>	re-establish RLC entities for SRB1;

2>	if the RRCRelease message with suspendConfig was received in response to an RRCResumeRequest or an RRCResumeRequest1:

3>	stop the timer T319 if running;

3>	in the stored UE Inactive AS context:

4>	replace the KgNB and KRRCint keys with the current KgNB and KRRCint keys;

4>	replace the nextHopChainingCount with the value of nextHopChainingCount received in the RRCRelease message;

4>	replace the cellIdentity with the cellIdentity of the cell the UE has received the RRCRelease message;

4>	if the suspendConfig contains the sl-UEIdentityRemote (i.e. the UE is a L2 U2N Remote UE):

5>	replace the C-RNTI with the value of the sl-UEIdentityRemote;

5>	replace the physical cell identity with the value of the sl-PhysCellId in sl-ServingCellInfo contained in the discovery message received from the connected L2 U2N Relay UE;

4> else:

5>	replace the C-RNTI with the C-RNTI used in the cell (see TS 38.321 [3]) the UE has received the RRCRelease message;

5>	replace the physical cell identity with the physical cell identity of the cell the UE has received the RRCRelease message;

3>	replace the nextHopChainingCount with the value associated with the current KgNB;

3>	stop the timer T319a if running and consider SDT procedure is not ongoing;

2>	else:

3>	store in the UE Inactive AS Context the nextHopChainingCount received in the RRCRelease message, the current KgNB and KRRCint keys, the ROHC state, the EHC context(s), the UDC state, the stored QoS flow to DRB mapping rules, the application layer measurement configuration, the C-RNTI used in the source PCell, the cellIdentity and the physical cell identity of the source PCell, the spCellConfigCommon within ReconfigurationWithSync of the NR PSCell (if configured) and all other parameters configured except for:

-	parameters within ReconfigurationWithSync of the PCell;

-	parameters within ReconfigurationWithSync of the NR PSCell, if configured;

-	parameters within MobilityControlInfoSCG of the E-UTRA PSCell, if configured;

-	servingCellConfigCommonSIB;

-	sl-L2RelayUE-Config, if configured;

-	sl-L2RemoteUE-Config, if configured;

NOTE 1c:	suspendConfig is not stored as part of UE Inactive AS Context, except for the fields explicitly specified.

3>	store any previously or subsequently received application layer measurement reports for which no segment, or full message, has been submitted to lower layers for transmission;

NOTE 2:	NR sidelink communication/discovery related configurations and logged measurement configuration are not stored as UE Inactive AS Context, when UE enters RRC_INACTIVE.

2>	suspend all SRB(s) and DRB(s) and multicast MRB(s), except SRB0 and broadcast MRBs;

2>	indicate PDCP suspend to lower layers of all DRBs and multicast MRBs;

2>	release the SRAP entity, if configured;

2>	if the t380 is included:

3>	start timer T380, with the timer value set to t380;

2>	if the RRCRelease message is including the waitTime:

3>	start timer T302 with the value set to the waitTime;

3>	inform upper layers that access barring is applicable for all access categories except categories '0' and '2';

2>	if T390 is running:

3>	stop timer T390 for all access categories;

3>	perform the actions as specified in 5.3.14.4;

2>	indicate the suspension of the RRC connection to upper layers;

2>	if the UE is capable of L2 U2N Remote UE:

3>	enter RRC_INACTIVE, and perform either cell selection as specified in TS 38.304 [20], or relay selection as specified in clause 5.8.15.3, or both;

2>	else:

3>	enter RRC_INACTIVE and perform cell selection as specified in TS 38.304 [20];

1>	else:

2>	perform the actions upon going to RRC_IDLE as specified in 5.3.11, with the release cause 'other'.

NOTE 3:	Whether to release the PC5 unicast link is left to L2 U2N Remote UE's implementation.

NOTE 4:	It is left to UE implementation whether to stop T430, if running, when going to RRC_INACTIVE.

#### 5.3.8.4	T320 expiry

The UE shall:

1>	if T320 expires:

2>	if stored, discard the cell reselection priority information provided by the cellReselectionPriorities or inherited from another RAT;

2>	apply the cell reselection priority information broadcast in the system information.

#### 5.3.8.5	UE actions upon the expiry of DataInactivityTimer

Upon receiving the expiry of DataInactivityTimer from lower layers while in RRC_CONNECTED, the UE shall:

1>	perform the actions upon going to RRC_IDLE as specified in 5.3.11, with release cause 'RRC connection failure'.

#### 5.3.8.6	T346g expiry

The UE shall:

1>	if T346g expires:

2>	perform the actions upon going to RRC_IDLE as specified in 5.3.11, with release cause 'other'.

### 5.3.9	RRC connection release requested by upper layers

#### 5.3.9.1	General

The purpose of this procedure is to release the RRC connection. Access to the current PCell may be barred as a result of this procedure.

#### 5.3.9.2	Initiation

The UE initiates the procedure when upper layers request the release of the RRC connection as specified in TS 24.501 [23]. The UE shall not initiate the procedure for power saving purposes.

The UE shall:

1>	if the upper layers indicate barring of the PCell:

2>	treat the PCell used prior to entering RRC_IDLE as barred according to TS 38.304 [20];

1>	perform the actions upon going to RRC_IDLE as specified in 5.3.11, with release cause 'other'.

### 5.3.10	Radio link failure related actions

#### 5.3.10.1	Detection of physical layer problems in RRC_CONNECTED

The UE shall:

1>	if any DAPS bearer is configured, upon receiving N310 consecutive "out-of-sync" indications for the source SpCell from lower layers and T304 is running:

2>	start timer T310 for the source SpCell.

1>	upon receiving N310 consecutive "out-of-sync" indications for the SpCell from lower layers while neither T300, T301, T304, T311, T316 nor T319 are running:

2>	start timer T310 for the corresponding SpCell.

#### 5.3.10.2	Recovery of physical layer problems

Upon receiving N311 consecutive "in-sync" indications for the SpCell from lower layers while T310 is running, the UE shall:

1>	stop timer T310 for the corresponding SpCell.

1>	stop timer T312 for the corresponding SpCell, if running.

NOTE 1:	In this case, the UE maintains the RRC connection without explicit signalling, i.e. the UE maintains the entire radio resource configuration.

NOTE 2:	Periods in time where neither "in-sync" nor "out-of-sync" is reported by L1 do not affect the evaluation of the number of consecutive "in-sync" or "out-of-sync" indications.

#### 5.3.10.3	Detection of radio link failure

The UE shall:

1>	if any DAPS bearer is configured and T304 is running:

2>	upon T310 expiry in source SpCell; or

2>	upon random access problem indication from source MCG MAC; or

2>	upon indication from source MCG RLC that the maximum number of retransmissions has been reached; or

2>	upon consistent uplink LBT failure indication from source MCG MAC:

3>	consider radio link failure to be detected for the source MCG i.e. source RLF;

3>	suspend the transmission and reception of all DRBs and multicast MRBs in the source MCG;

3>	reset MAC for the source MCG;

3>	release the source connection.

1>	else:

2>	during a DAPS handover: the following only applies for the target PCell;

2>	upon T310 expiry in PCell; or

2>	upon T312 expiry in PCell; or

2>	upon random access problem indication from MCG MAC while neither T300, T301, T304, T311 nor T319 are running and SDT procedure is not ongoing; or

2>	upon indication from MCG RLC that the maximum number of retransmissions has been reached while SDT procedure is not ongoing; or

2>	if connected as an IAB-node, upon BH RLF indication received on BAP entity from the MCG; or

2>	upon consistent uplink LBT failure indication from MCG MAC while T304 is not running:

3>	if the indication is from MCG RLC and CA duplication is configured and activated for MCG, and for the corresponding logical channel allowedServingCells only includes SCell(s):

4>	initiate the failure information procedure as specified in 5.7.5 to report RLC failure.

3>	else:

4>	consider radio link failure to be detected for the MCG, i.e. MCG RLF;

4>	discard any segments of segmented RRC messages stored according to 5.7.6.3;

NOTE:	Void.

4>	if AS security has not been activated:

5>	perform the actions upon going to RRC_IDLE as specified in 5.3.11, with release cause 'other';-

4>	else if AS security has been activated but SRB2 and at least one DRB or multicast MRB or, for IAB, SRB2, have not been setup:

5>	store the radio link failure information in the VarRLF-Report as described in clause 5.3.10.5;

5>	perform the actions upon going to RRC_IDLE as specified in 5.3.11, with release cause 'RRC connection failure';

4>	else:

5>	store the radio link failure information in the VarRLF-Report as described in clause 5.3.10.5;

5>	if T316 is configured; and

5>	if SCG transmission is not suspended; and

5>	if the SCG is not deactivated; and

5>	if neither PSCell change nor PSCell addition is ongoing (i.e. timer T304 for the NR PSCell is not running in case of NR-DC or timer T307 of the E-UTRA PSCell is not running as specified in TS 36.331 [10], clause 5.3.10.10, in NE-DC):

6>	initiate the MCG failure information procedure as specified in 5.7.3b to report MCG radio link failure.

5>	else:

6>	initiate the connection re-establishment procedure as specified in 5.3.7.

A L2/L3 U2N Relay UE shall:

1>	upon detecting radio link failure:

2>	it either indicates to upper layers (to trigger PC5 unicast link release) or sends Notification message to the connected L2/L3 U2N Remote UE(s) in accordance with 5.8.9.10.

The UE shall:

1>	upon T310 expiry in PSCell; or

1>	upon T312 expiry in PSCell; or

1>	upon random access problem indication from SCG MAC; or

1>	upon indication from SCG RLC that the maximum number of retransmissions has been reached; or

1>	if connected as an IAB-node, upon BH RLF indication received on BAP entity from the SCG; or

1>	upon consistent uplink LBT failure indication from SCG MAC:

2>	if the indication is from SCG RLC and CA duplication is configured and activated for SCG, and for the corresponding logical channel allowedServingCells only includes SCell(s):

3>	initiate the failure information procedure as specified in 5.7.5 to report RLC failure.

2>	else:

3>	consider radio link failure to be detected for the SCG, i.e. SCG RLF;

3>	if the SCG is deactivated:

4>	stop radio link monitoring on the SCG;

4>	indicate to lower layers to stop beam failure detection on the PSCell;

3>	if MCG transmission is not suspended:

4>	initiate the SCG failure information procedure as specified in 5.7.3 to report SCG radio link failure.

3>	else:

4>	if the UE is in NR-DC:

5>	initiate the connection re-establishment procedure as specified in 5.3.7;

4>	else (the UE is in (NG)EN-DC):

5>	initiate the connection re-establishment procedure as specified in TS 36.331 [10], clause 5.3.7;

#### 5.3.10.4	RLF cause determination

The UE shall set the rlf-Cause in the VarRLF-Report as follows:

1>	if the UE declares radio link failure due to T310 expiry:

2>	set the rlf-Cause as t310-Expiry;

1>	else if the UE declares radio link failure due to the random access problem indication from MCG MAC:

2>	if the random access procedure was initiated for beam failure recovery:

3>	set the rlf-Cause as beamFailureRecoveryFailure;

2>	else:

3>	set the rlf-Cause as randomAccessProblem;

1>	else if the UE declares radio link failure due to the reaching of maximum number of retransmissions from the MCG RLC:

2>	set the rlf-Cause as rlc-MaxNumRetx;

1>	else if the UE declares radio link failure due to consistent uplink LBT failures:

2>	set the rlf-Cause as lbtFailure;

1>	else if the IAB-MT declares radio link failure due to the reception of a BH RLF indication on BAP entity:

2>	set the rlf-Cause as bh-rlfRecoveryFailure.

1>	else if the UE declares radio link failure due to T312 expiry:

2>	set the rlf-Cause as t312-Expiry;

#### 5.3.10.5	RLF report content determination

The UE shall determine the content in the VarRLF-Report as follows:

1>	clear the information included in VarRLF-Report, if any;

1>	set the plmn-IdentityList to include the list of EPLMNs stored by the UE (i.e. includes the RPLMN);

1>	set the measResultLastServCell to include the cell level RSRP, RSRQ and the available SINR, of the source PCell (in case HO failure) or PCell (in case RLF) based on the available SSB and CSI-RS measurements collected up to the moment the UE detected failure;

1>	if the SS/PBCH block-based measurement quantities are available:

2>	set the rsIndexResults in measResultLastServCell to include all the available measurement quantities of the source PCell (in case HO failure) or PCell (in case RLF), ordered such that the highest SS/PBCH block RSRP is listed first if SS/PBCH block RSRP measurement results are available, otherwise the highest SS/PBCH block RSRQ is listed first if SS/PBCH block RSRQ measurement results are available, otherwise the highest SS/PBCH block SINR is listed first, based on the available SS/PBCH block based measurements collected up to the moment the UE detected failure;

1>	if the CSI-RS based measurement quantities are available:

2>	set the rsIndexResults in measResultLastServCell to include all the available measurement quantities of the source PCell (in case HO failure) or PCell (in case RLF), ordered such that the highest CSI-RS RSRP is listed first if CSI-RS RSRP measurement results are available, otherwise the highest CSI-RS RSRQ is listed first if CSI-RS RSRQ measurement results are available, otherwise the highest CSI-RS SINR is listed first, based on the available CSI-RS based measurements collected up to the moment the UE detected failure;

1>	set the ssbRLMConfigBitmap and/or csi-rsRLMConfigBitmap in measResultLastServCell to include the radio link monitoring configuration of the source PCell (in case HO failure) or PCell (in case RLF), if available;

1>	for each of the configured measObjectNR in which measurements are available:

2>	if the SS/PBCH block-based measurement quantities are available:

3>	set the measResultListNR in measResultNeighCells to include all the available measurement quantities of the best measured cells, other than the source PCell (in case HO failure) or PCell (in case RLF), ordered such that the cell with highest SS/PBCH block RSRP is listed first if SS/PBCH block RSRP measurement results are available, otherwise the cell with highest SS/PBCH block RSRQ is listed first if SS/PBCH block RSRQ measurement results are available, otherwise the cell with highest SS/PBCH block SINR is listed first, based on the available SS/PBCH block based measurements collected up to the moment the UE detected failure;

4>	for each neighbour cell included, include the optional fields that are available;

NOTE 0a:	For the neighboring cells included in measResultListNR in measResultNeighCells ordered based on the SS/PBCH block measurement quantities, UE also includes the CSI-RS based measurement quantities, if available.

2>	if the CSI-RS based measurement quantities are available:

3>	set the measResultListNR in measResultNeighCells to include all the available measurement quantities of the best measured cells, other than the source PCell (in case HO failure) or PCell (in case RLF), ordered such that the cell with highest CSI-RS RSRP is listed first if CSI-RS RSRP measurement results are available, otherwise the cell with highest CSI-RS RSRQ is listed first if CSI-RS RSRQ measurement results are available, otherwise the cell with highest CSI-RS SINR is listed first, based on the available CSI-RS based measurements collected up to the moment the UE detected radio link failure;

4>	for each neighbour cell included, include the optional fields that are available;

NOTE 0b:	For ordering the neighboring cells based on the CSI-RS measurement quantities, UE includes measurements only for the cells not yet included in measResultListNR in measResultNeighCells to avoid overriding SS/PBCH block-based ordered measurements.

2>	for each neighbour cell, if any, included in measResultListNR in measResultNeighCells:

3>	if the UE supports RLF-Report for conditional handover and if the neighbour cell is one of the candidate cells for which the reconfigurationWithSync is included in the masterCellGroup in the MCG VarConditionalReconfig at the moment of the detected failure:

4>	set choConfig in MeasResult2NR to the execution condition for each measId within condTriggerConfig associated to the neighbour cell within the MCG VarConditionalReconfig;

4>	if the first entry of choConfig corresponds to a fulfilled execution condition at the moment of handover failure, or radio link failure; or

4>	if the second entry of choConfig, if available, corresponds to a fulfilled execution condition at the moment of handover failure, or radio link failure:

5>	set firstTriggeredEvent to the execution condition condFirstEvent corresponding to the first entry of choConfig or to the execution condition condSecondEvent corresponding to the second entry of choConfig, whichever execution condition was fulfilled first in time;

5>	set timeBetweenEvents to the elapsed time between the point in time of fullfilling the condition in choConfig that was fulfilled first in time, and the point in time of fullfilling the condition in choConfig that was fulfilled second in time, if both the first execution condition corresponding to the first entry and the second execution condition corresponding to the second entry in the choConfig were fullfilled;

1>	for each of the configured EUTRA frequencies in which measurements are available;

2>	set the measResultListEUTRA in measResultNeighCells to include the best measured cells ordered such that the cell with highest RSRP is listed first if RSRP measurement results are available, otherwise the cell with highest RSRQ is listed first, and based on measurements collected up to the moment the UE detected failure;

3>	for each neighbour cell included, include the optional fields that are available;

NOTE 1:	The measured quantities are filtered by the L3 filter as configured in the mobility measurement configuration. The measurements are based on the time domain measurement resource restriction, if configured. Exclude-listed cells are not required to be reported.

1>	set the c-RNTI to the C-RNTI used in the source PCell (in case HO failure) or PCell (in case RLF);

1>	if the failure is detected due to reconfiguration with sync failure as described in 5.3.5.8.3, set the fields in VarRLF-report as follows:

2>	set the connectionFailureType to hof;

2>	if the UE supports RLF-Report for DAPS handover and if any DAPS bearer was configured while T304 was running:

3>	set lastHO-Type to daps;

3>	if radio link failure was detected in the source PCell, according to clause 5.3.10.3:

4>	set timeConnSourceDAPS-Failure to the time between the initiation of the DAPS handover execution and the radio link failure detected in the source PCell while T304 was running;

4>	set the rlf-Cause to the trigger for detecting the source radio link failure in accordance with clause 5.3.10.4;

2>	if the UE supports RLF-Report for conditional handover and if configuration of the conditional handover is available in the MCG VarConditionalReconfig at the moment of the handover failure:

3>	if the UE executed a conditional handover toward target PCell according to the condRRCReconfig of the target PCell:

4>	set timeSinceCHO-Reconfig to the time elapsed between the execution of the last RRCReconfiguration message including reconfigurationWithSync for the target PCell of the failed conditional handover, and the reception in the source PCell of the last conditionalReconfiguration including the condRRCReconfig of the target PCell of the failed conditional handover;

3>	else:

4>	set timeSinceCHO-Reconfig to the time elapsed between the execution of the last RRCReconfiguration message including reconfigurationWithSync for the target PCell of the failed handover, and the reception in the source PCell of the last conditionalReconfiguration including the condRRCReconfig;

3>	set choCandidateCellList to include the global cell identity, if available, and otherwise to the physical cell identity and carrier frequency of each of the candidate target cells for conditional handover included in condRRCReconfig within the MCG VarConditionalReconfig at the time of the failed handover, excluding the candidate target cells included in measResulNeighCells;

2>	if the UE supports RLF-Report for conditional handover and if the last executed RRCReconfiguration message including reconfigurationWithSync was concerning a conditional handover:

3>	set lastHO-Type to cho;

2>	set the nrFailedPCellId in failedPCellId to the global cell identity and tracking area code, if available, and otherwise to the physical cell identity and carrier frequency of the target PCell of the failed handover;

2>	include nrPreviousCell in previousPCellId and set it to the global cell identity and tracking area code of the PCell where the last RRCReconfiguration message including reconfigurationWithSync was received;

2>	set the timeConnFailure to the elapsed time since the execution of the last RRCReconfiguration message including the reconfigurationWithSync;

1>	else if the failure is detected due to Mobility from NR failure as described in 5.4.3.5, set the fields in VarRLF-report as follows:

2>	set the connectionFailureType to hof;

2>	if last MobilityFromNRCommand concerned a failed inter-RAT handover from NR to E-UTRA and if the UE supports Radio Link Failure Report for Inter-RAT MRO EUTRA (NR to EUTRA):

3>	set the eutraFailedPCellId in failedPCellId to the global cell identity and tracking area code, if available, and otherwise to the physical cell identity and carrier frequency of the target PCell of the failed handover;

2>	include nrPreviousCell in previousPCellId and set it to the global cell identity and tracking area code of the PCell where the last MobilityFromNRCommand message was received;

2>	set the timeConnFailure to the elapsed time since the initialization of the handover associated to the last MobilityFromNRCommand message;

1>	else if the failure is detected due to radio link failure as described in 5.3.10.3, set the fields in VarRLF-report as follows:

2>	set the connectionFailureType to rlf;

2>	set the rlf-Cause to the trigger for detecting radio link failure in accordance with clause 5.3.10.4;

2>	set the nrFailedPCellId in failedPCellId to the global cell identity and the tracking area code, if available, and otherwise to the physical cell identity and carrier frequency of the PCell where radio link failure is detected;

2>	if an RRCReconfiguration message including the reconfigurationWithSync was received before the connection failure:

3>	if the last executed RRCReconfiguration message including the reconfigurationWithSync concerned an intra NR handover and it was received while connected to the previous PCell to which the UE was connected before connecting to the PCell where radio link failure is detected; and

3>	if the PCell in which the radio link failure was detected was a result of cell selection and the T311 was not running at the time of PCell selection:

4>	include the nrPreviousCell in previousPCellId and set it to the global cell identity and the tracking area code of the PCell where the last executed RRCReconfiguration message including reconfigurationWithSync was received;

4>	if the last executed RRCReconfiguration message including reconfigurationWithSync was concerning a DAPS handover:

5>	set lastHO-Type to daps;

4>	else if the last executed RRCReconfiguration message including reconfigurationWithSync was concerning a conditional handover:

5>	set lastHO-Type to cho;

4>	set the timeConnFailure to the elapsed time since the execution of the last RRCReconfiguration message including the reconfigurationWithSync;

3>	else if the last RRCReconfiguration message including the reconfigurationWithSync concerned a handover to NR from E-UTRA and if the UE supports Radio Link Failure Report for Inter-RAT MRO EUTRA:

4>	include the eutraPreviousCell in previousPCellId and set it to the global cell identity and the tracking area code of the E-UTRA PCell where the last RRCReconfiguration message including reconfigurationWithSync was received embedded in E-UTRA RRC message MobilityFromEUTRACommand message as specified in TS 36.331 [10] clause 5.4.3.3;

4>	set the timeConnFailure to the elapsed time since reception of the last RRCReconfiguration message including the reconfigurationWithSync embedded in E-UTRA RRC message MobilityFromEUTRACommand message as specified in TS 36.331 [10] clause 5.4.3.3;

2>	if configuration of the conditional handover is available in the MCG VarConditionalReconfig at the moment of declaring the radio link failure:

3>	set timeSinceCHO-Reconfig to the time elapsed between the detection of the radio link failure, and the reception, in the source PCell, of the last conditionalReconfiguration including the condRRCReconfig message;

3>	set choCandidateCellList to include the global cell identity if available, and otherwise to the physical cell identity and carrier frequency of each of all the candidate target cells for conditional handover included in condRRCReconfig within the MCG VarConditionalReconfig at the time of radio link failure, excluding the candidate target cells included in measResulNeighCells;

1>	if connectionFailureType is rlf and the rlf-Cause is set to randomAccessProblem or beamFailureRecoveryFailure; or

1>	if connectionFailureType is hof and if the failed handover is an intra-RAT handover:

2>	set the ra-InformationCommon to include the random-access related information as described in clause 5.7.10.5;

1>	if available, set the locationInfo as in 5.3.3.7.

The UE may discard the radio link failure information or handover failure information, i.e. release the UE variable VarRLF-Report, 48 hours after the radio link failure/handover failure is detected.

NOTE 2:	In this clause, the term 'handover failure' has been used to refer to 'reconfiguration with sync failure'.

### 5.3.11	UE actions upon going to RRC_IDLE

The UE shall:

1>	reset MAC;

1>	set the variable pendingRNA-Update to false, if that is set to true;

1>	if going to RRC_IDLE was triggered by reception of the RRCRelease message including a waitTime:

2>	if T302 is running:

3>	stop timer T302;

2>	start timer T302 with the value set to the waitTime;

2>	inform upper layers that access barring is applicable for all access categories except categories '0' and '2'.

1>	else:

2>	if T302 is running:

3>	stop timer T302;

3>	perform the actions as specified in 5.3.14.4;

1>	if T390 is running:

2>	stop timer T390 for all access categories;

2>	perform the actions as specified in 5.3.14.4;

1>	if the UE is leaving RRC_INACTIVE:

2>	if going to RRC_IDLE was not triggered by reception of the RRCRelease message:

3>	if stored, discard the cell reselection priority information provided by the cellReselectionPriorities;

3>	stop the timer T320, if running;

2>	if T319a is running:

3>	stop timer T319a;

3>	consider SDT procedure is not ongoing;

1>	stop all timers that are running except T302, T320, T325, T330, T331, T400 and T430;

1>	discard the UE Inactive AS context, if any;

1>	release the suspendConfig, if configured;

1>	remove all the entries within the MCG and the SCG VarConditionalReconfig, if any;

1>	for each measId, if the associated reportConfig has a reportType set to condTriggerConfig:

2>	for the associated reportConfigId:

3>	remove the entry with the matching reportConfigId from the reportConfigList within the VarMeasConfig;

2>	if the associated measObjectId is only associated to a reportConfig with reportType set to condTriggerConfig:

3>	remove the entry with the matching measObjectId from the measObjectList within the VarMeasConfig;

2>	remove the entry with the matching measId from the measIdList within the VarMeasConfig;

1>	discard the KgNB key, the S-KgNB key, the S-KeNB key, the KRRCenc key, the KRRCint key, the KUPint key and the KUPenc key, if any;

1>	release all radio resources, including release of the RLC entity, the BAP entity, the MAC configuration and the associated PDCP entity and SDAP for all established RBs (except for broadcast MRBs), BH RLC channels, Uu Relay RLC channels, PC5 Relay RLC channels and SRAP entity;

1>	indicate the release of the RRC connection to upper layers together with the release cause;

1>	inform upper layers about the release of all application layer measurement configurations;

1>	discard any application layer measurement reports which were not yet submitted to lower layers for transmission;

1>	discard any segments of segmented RRC messages stored according to 5.7.6.3;

1>	except if going to RRC_IDLE was triggered by inter-RAT cell reselection while the UE is in RRC_INACTIVE or RRC_IDLE or when selecting an inter-RAT cell while T311 was running or when selecting an E-UTRA cell for EPS fallback for IMS voice as specified in 5.4.3.5:

2>	if the UE is capable of L2 U2N Remote UE:

3>	enter RRC_IDLE, and perform either cell selection as specified in TS 38.304 [20], or relay selection as specified in clause 5.8.15.3, or both;

2>	else:

3>	enter RRC_IDLE and perform cell selection as specified in TS 38.304 [20];

NOTE 1:	Whether to release the PC5 unicast link is left to L2 U2N Remote UE's implementation.

NOTE 2:	It is left to UE implementation whether to stop T430, if running, when going to RRC_IDLE.

### 5.3.12	UE actions upon PUCCH/SRS release request

Upon receiving a PUCCH release request from lower layers, for all bandwidth parts of an indicated serving cell the UE shall:

1>	release PUCCH-CSI-Resources configured in CSI-ReportConfig;

1>	release SchedulingRequestResourceConfig instances configured in PUCCH-Config.

Upon receiving an SRS release request from lower layers, for all bandwidth parts of an indicated serving cell the UE shall:

1>	release SRS-Resource instances configured in SRS-Config.

Upon receiving a positioning SRS configuration for RRC_INACTIVE release request from lower layers, the UE shall:

1>	release the configured srs-PosRRC-Inactive.

### 5.3.13	RRC connection resume

#### 5.3.13.1	General

Figure 5.3.13.1-1: RRC connection resume, successful

Figure 5.3.13.1-2: RRC connection resume fallback to RRC connection establishment, successful

Figure 5.3.13.1-3: RRC connection resume followed by network release, successful

Figure 5.3.13.1-4: RRC connection resume followed by network suspend, successful

Figure 5.3.13.1-5: RRC connection resume, network reject

The purpose of this procedure is to resume a suspended RRC connection, including resuming SRB(s), DRB(s) and multicast MRB(s) or perform an RNA update. This procedure is also used to initiate SDT in RRC_INACTIVE.

#### 5.3.13.1a	Conditions for resuming RRC Connection for NR sidelink communication/discovery/V2X sidelink communication

For NR sidelink communication/discovery an RRC connection is resumed only in the following cases:

1>	if configured by upper layers to transmit NR sidelink communication/discovery and related data is available for transmission:

2>	if the frequency on which the UE is configured to transmit NR sidelink communication is included in sl-FreqInfoList within SIB12 provided by the cell on which the UE camps; and if the valid version of SIB12 does not include sl-TxPoolSelectedNormal for the concerned frequency; or

2>	if the frequency on which the UE is configured to transmit NR sidelink discovery is included in sl-FreqInfoList within SIB12 provided by the cell on which the UE camps; and if the valid version of SIB12 does not include sl-DiscTxPoolSelected or sl-TxPoolSelectedNormal for the concerned frequency;

For L2 U2N Relay UE in RRC_INACTIVE, an RRC connection establishment is resumed in the following cases:

1>	if any message is received from the L2 U2N Remote UE via SL-RLC0 as specified in 9.1.1.4 or SL-RLC1 as specified in 9.2.4;

For V2X sidelink communication an RRC connection resume is initiated only when the conditions specified for V2X sidelink communication in clause 5.3.3.1a of TS 36.331 [10] are met.

NOTE:	Upper layers initiate an RRC connection resume (except if the RRC connection resume is initiated at the L2 U2N Relay UE upon reception of a message from a L2 U2N Remote UE via SL-RLC0 or SL-RLC1). The interaction with NAS is left to UE implementation.

#### 5.3.13.1b	Conditions for initiating SDT

A UE in RRC_INACTIVE initiates the resume procedure for SDT when all of the following conditions are fulfilled:

1>	the upper layers request resumption of RRC connection; and

1>	SIB1 includes sdt-ConfigCommon; and

1>	sdt-Config is configured; and

1>	all the pending data in UL is mapped to the radio bearers configured for SDT; and

1>	for a RedCap UE when RedCap-specific initial downlink BWP includes no CD-SSB, ncd-SSB-RedCapInitialBWP-SDT is configured; and

1>	lower layers indicate that conditions for initiating SDT as specified in TS 38.321 [3] are fulfilled.

NOTE:	How the UE determines that all pending data in UL is mapped to radio bearers configured for SDT is left to UE implementation.

#### 5.3.13.2	Initiation

The UE initiates the procedure when upper layers or AS (when responding to RAN paging, upon triggering RNA updates while the UE is in RRC_INACTIVE, for NR sidelink communication/discovery/V2X sidelink communication as specified in clause 5.3.13.1a) requests the resume of a suspended RRC connection or requests the resume for initiating SDT as specified in clause 5.3.13.1b.

The UE shall ensure having valid and up to date essential system information as specified in clause 5.2.2.2 before initiating this procedure.

Upon initiation of the procedure, the UE shall:

1>	if the resumption of the RRC connection is triggered by response to NG-RAN paging:

2>	select '0' as the Access Category;

2>	perform the unified access control procedure as specified in 5.3.14 using the selected Access Category and one or more Access Identities provided by upper layers;

3>	if the access attempt is barred, the procedure ends;

1>	else if the resumption of the RRC connection is triggered by upper layers:

2>	if the upper layers provide an Access Category and one or more Access Identities:

3>	perform the unified access control procedure as specified in 5.3.14 using the Access Category and Access Identities provided by upper layers;

4>	if the access attempt is barred, the procedure ends;

2>	if the upper layers provide NSAG information and one or more S-NSSAI(s) triggering the access attempt (TS 23.501 [32] and TS 24.501 [23]):

3>	apply the NSAG with highest NSAG priority among the NSAGs that are included in SIB1 (i.e., in FeatureCombination and in RA-PrioritizationSliceInfo), and that are associated with the S-NSSAI(s) triggering the access attempt, in the Random Access procedure (TS 38.321 [3], clause 5.1);

2>	if the resumption occurs after release with redirect with mpsPriorityIndication:

3>	set the resumeCause to mps-PriorityAccess;

2>	else:

3>	set the resumeCause in accordance with the information received from upper layers;

1>	else if the resumption of the RRC connection is triggered due to an RNA update as specified in 5.3.13.8:

2>	if an emergency service is ongoing:

NOTE 1:	How the RRC layer in the UE is aware of an ongoing emergency service is up to UE implementation.

3>	select '2' as the Access Category;

3>	set the resumeCause to emergency;

2>	else:

3>	select '8' as the Access Category;

2>	perform the unified access control procedure as specified in 5.3.14 using the selected Access Category and one or more Access Identities to be applied as specified in TS 24.501 [23];

3>	if the access attempt is barred:

4>	set the variable pendingRNA-Update to true;

4>	the procedure ends;

NOTE 2:	In case the L2 U2N Relay UE initiates RRC connection resume triggered by reception of message from a L2 U2N Remote UE via SL-RLC0 or SL-RLC1 as specified in 5.3.13.1a, the L2 U2N Relay UE sets the resumeCause by implementation, but it can only set the emergency, mps-PriorityAccess, or mcs-PriorityAccess as resumeCause, if the same cause value in the message received from the L2 U2N Remote UE via SL-RLC0.

1>	if the UE is in NE-DC or NR-DC:

2>	if the UE does not support maintaining SCG configuration upon connection resumption:

3>	release the MR-DC related configurations (i.e., as specified in 5.3.5.10) from the UE Inactive AS context, if stored;

1>	if the UE does not support maintaining the MCG SCell configurations upon connection resumption:

2>	release the MCG SCell(s) from the UE Inactive AS context, if stored;

1>	if the UE is acting as L2 U2N Remote UE:

2>	establish a SRAP entity as specified in TS 38.351 [66], if no SRAP entity has been established;

2>	apply the default configuration of SL-RLC1 as defined in 9.2.4 for SRB1;

2>	apply the default PDCP configuration as defined in 9.2.1 for SRB1;

2>	apply the default configuration of SRAP as defined in 9.2.5 for SRB1;

1>	else:

2>	apply the default L1 parameter values as specified in corresponding physical layer specifications, except for the parameters for which values are provided in SIB1;

2>	apply the default SRB1 configuration as specified in 9.2.1;

2>	apply the default MAC Cell Group configuration as specified in 9.2.2;

1>	release delayBudgetReportingConfig from the UE Inactive AS context, if stored;

1>	stop timer T342, if running;

1>	release overheatingAssistanceConfig from the UE Inactive AS context, if stored;

1>	stop timer T345, if running;

1>	release idc-AssistanceConfig from the UE Inactive AS context, if stored;

1>	release drx-PreferenceConfig for all configured cell groups from the UE Inactive AS context, if stored;

1>	stop all instances of timer T346a, if running;

1>	release maxBW-PreferenceConfig and maxBW-PreferenceConfigFR2-2 for all configured cell groups from the UE Inactive AS context, if stored;

1>	stop all instances of timer T346b, if running;

1>	release maxCC-PreferenceConfig for all configured cell groups from the UE Inactive AS context, if stored;

1>	stop all instances of timer T346c, if running;

1>	release maxMIMO-LayerPreferenceConfig and maxMIMO-LayerPreferenceConfigFR2-2 for all configured cell groups from the UE Inactive AS context, if stored;

1>	stop all instances of timer T346d, if running;

1>	release minSchedulingOffsetPreferenceConfig and minSchedulingOffsetPreferenceConfigExt for all configured cell groups from the UE Inactive AS context, if stored;

1>	stop all instances of timer T346e, if running;

1>	release rlm-RelaxationReportingConfig for all configured cell groups from the UE Inactive AS context, if stored;

1>	stop all instances of timer T346j, if running;

1>	release bfd-RelaxationReportingConfig for all configured cell groups from the UE Inactive AS context, if stored;

1>	stop all instances of timer T346k, if running;

1>	release releasePreferenceConfig from the UE Inactive AS context, if stored;

1>	release wlanNameList from the UE Inactive AS context, if stored;

1>	release btNameList from the UE Inactive AS context, if stored;

1>	release sensorNameList from the UE Inactive AS context, if stored;

1>	release obtainCommonLocation from the UE Inactive AS context, if stored;

1>	stop timer T346f, if running;

1>	stop timer T346i, if running;

1>	release referenceTimePreferenceReporting from the UE Inactive AS context, if stored;

1>	release sl-AssistanceConfigNR from the UE Inactive AS context, if stored;

1>	release musim-GapAssistanceConfig from the UE Inactive AS context, if stored and stop timer T346h, if running;

1>	release musim-GapConfig from the UE Inactive AS context, if stored;

1>	release musim-LeaveAssistanceConfig from the UE Inactive AS context, if stored;

1>	release musim-GapPriorityAssistanceConfig from the UE Inactive AS context, if stored;

1>	release musim-CapabilityRestrictionConfig from the UE Inactive AS context, if stored;

1>	release propDelayDiffReportConfig from the UE Inactive AS context, if stored;

1>	release ul-GapFR2-PreferenceConfig, if configured;

1>	release rrm-MeasRelaxationReportingConfig from the UE Inactive AS context, if stored;

1>	if the UE is acting as L2 U2N Remote UE:

2>	apply the specified configuration of SL-RLC0 used for the delivery of RRC message over SRB0 as specified in 9.1.1.4;

2>	apply the SDAP configuration and PDCP configuration as specified in 9.1.1.2 for SRB0;

1>	else:

2>	apply the CCCH configuration as specified in 9.1.1.2;

2>	apply the timeAlignmentTimerCommon included in SIB1;

1>	if sdt-MAC-PHY-CG-Config is configured:

2>	if the resume procedure is initiated in a cell that is different to the PCell in which the UE received the stored sdt-MAC-PHY-CG-Config:

3>	release the stored sdt-MAC-PHY-CG-Config;

3>	instruct the MAC entity to stop the cg-SDT-TimeAlignmentTimer, if it is running;

1>	if ncd-SSB-RedCapInitialBWP-SDT is configured:

2>	if the resume procedure is initiated in a cell that is different to the PCell in which the UE received the stored ncd-SSB-RedCapInitialBWP-SDT:

3>	release the stored ncd-SSB-RedCapInitialBWP-SDT;

1>	if conditions for initiating SDT in accordance with 5.3.13.1b are fulfilled:

2>	consider the resume procedure is initiated for SDT;

2>	start timer T319a when the lower layers first transmit the CCCH message;

2>	consider SDT procedure is ongoing;

1> else:

2>	start timer T319;

2>	instruct the MAC entity to stop the cg-SDT-TimeAlignmentTimer, if it is running;

1>	if ta-Report is configured with value enabled and the UE supports TA reporting:

2>	indicate TA report initiation to lower layers;

1>	set the variable pendingRNA-Update to false;

1>	release successHO-Config from the UE Inactive AS context, if stored;

1>	initiate transmission of the RRCResumeRequest message or RRCResumeRequest1 in accordance with 5.3.13.3.

#### 5.3.13.3	Actions related to transmission of RRCResumeRequest or RRCResumeRequest1 message

The UE shall set the contents of RRCResumeRequest or RRCResumeRequest1 message as follows:

1>	if field useFullResumeID is signalled in SIB1:

2>	select RRCResumeRequest1 as the message to use;

2>	set the resumeIdentity to the stored fullI-RNTI value;

1>	else:

2>	select RRCResumeRequest as the message to use;

2>	set the resumeIdentity to the stored shortI-RNTI value;

1>	restore the RRC configuration, RoHC state, the EHC context(s), the UDC state, the stored QoS flow to DRB mapping rules and the KgNB and KRRCint keys from the stored UE Inactive AS context except for the following:

-	masterCellGroup;

-	mrdc-SecondaryCellGroup, if stored; and

-	pdcp-Config;

1>	set the resumeMAC-I to the 16 least significant bits of the MAC-I calculated:

2>	over the ASN.1 encoded as per clause 8 (i.e., a multiple of 8 bits) VarResumeMAC-Input;

2>	with the KRRCint key in the UE Inactive AS Context and the previously configured integrity protection algorithm; and

2>	with all input bits for COUNT, BEARER and DIRECTION set to binary ones;

1>	derive the KgNB key based on the current KgNB key or the NH, using the nextHopChainingCount value received in the previous RRCRelease message and stored in the UE Inactive AS Context, as specified in TS 33.501 [11];

1>	derive the KRRCenc key, the KRRCint key, the KUPint key and the KUPenc key;

1>	configure lower layers to apply integrity protection for all radio bearers except SRB0 and MRBs using the configured algorithm and the KRRCint key and KUPint key derived in this clause immediately, i.e., integrity protection shall be applied to all subsequent messages received and sent by the UE;

NOTE 1:	Only DRBs with previously configured UP integrity protection shall resume integrity protection.

1>	configure lower layers to apply ciphering for all radio bearers except SRB0 and MRBs and to apply the configured ciphering algorithm, the KRRCenc key and the KUPenc key derived in this clause, i.e. the ciphering configuration shall be applied to all subsequent messages received and sent by the UE;

1>	re-establish PDCP entities for SRB1;

1>	resume SRB1;

1>	if the resume procedure is initiated for SDT:

2>	for each radio bearer that is configured for SDT and for SRB1:

3>	restore the RLC-BearerConfig associated with the RLC bearers of masterCellGroup and pdcp-Config from the UE Inactive AS context;

3>	if the radio bearer is a DRB configured with Ethernet Header Compression:

4>	indicate to lower layer that ethernetHeaderCompression is not configured;

3>	if the radio bearer is a DRB configured with UDC:

4>	indicate to lower layer that uplinkDataCompression is not configured;

3>	if the radio bearer is a DRB configured with ROHC function:

4>	if sdt-DRB-ContinueROHC is set to cell and the resume procedure is initiated in a cell that is the same as the PCell in which the UE received the previous RRCRelease message; or

4>	if sdt-DRB-ContinueROHC is set to rna and the resume procedure is initiated in a cell belonging to the same RNA as the PCell in which the UE received the previous RRCRelease message:

5>	indicate to lower layer that drb-continueROHC is configured;

4>	else:

5>	indicate to lower layer that drb-continueROHC is not configured;

3>	re-establish PDCP entity for the radio bearer that is configured for SDT without triggering PDCP status report;

2>	resume all the radio bearers that are configured for SDT;

1>	submit the selected message RRCResumeRequest or RRCResumeRequest1 for transmission to lower layers.

NOTE 2:	Only DRBs with previously configured UP ciphering shall resume ciphering.

If lower layers indicate an integrity check failure while T319 is running or SDT procedure is ongoing, perform actions specified in 5.3.13.5.

If the UE is a RedCap UE and the RedCap-specific initial downlink BWP is not associated with CD-SSB, the UE may continue cell re-selection related measurements as well as cell re-selection evaluation, otherwise the UE shall continue cell re-selection related measurements as well as cell re-selection evaluation. If the conditions for cell re-selection are fulfilled, the UE shall perform cell re-selection as specified in 5.3.13.6.

NOTE 3:	For L2 U2N Remote UE in RRC_INACTIVE, the cell (re)selection procedure as specified in TS 38.304 [20] and relay (re)selection procedure as specified in 5.8.15.3 are performed independently and it is up to UE implementation to select either a cell or a L2 U2N Relay UE.

#### 5.3.13.4	Reception of the RRCResume by the UE

The UE shall:

1>	stop timer T319, if running;

1>	stop timer T319a, if running and consider SDT procedure is not ongoing;

1>	stop timer T380, if running;

1>	if T331 is running:

2>	stop timer T331;

2>	perform the actions as specified in 5.7.8.3;

1>	if the RRCResume includes the fullConfig:

2>	perform the full configuration procedure as specified in 5.3.5.11;

1>	else:

2>	if the RRCResume does not include the restoreMCG-SCells:

3>	release the MCG SCell(s) from the UE Inactive AS context, if stored;

2>	if the RRCResume does not include the restoreSCG:

3>	release the MR-DC related configurations (i.e., as specified in 5.3.5.10) from the UE Inactive AS context, if stored;

2>	restore the masterCellGroup, mrdc-SecondaryCellGroup, if stored, and pdcp-Config from the UE Inactive AS context;

2>	configure lower layers to consider the restored MCG and SCG SCell(s) (if any) to be in deactivated state;

1>	discard the UE Inactive AS context;

1>	store the used nextHopChainingCount value associated to the current KgNB;

1>	if sdt-MAC-PHY-CG-Config is configured:

2>	instruct the MAC entity to stop the cg-SDT-TimeAlignmentTimer, if it is running;

2>	instruct the MAC entity to start the timeAlignmentTimer associated with the PTAG, if it is not running;

1>	if srs-PosRRC-InactiveConfig is configured:

2>	instruct the MAC entity to stop inactivePosSRS-TimeAlignmentTimer, if it is running;

1>	release the suspendConfig except the ran-NotificationAreaInfo;

1>	if the RRCResume includes the masterCellGroup:

2>	perform the cell group configuration for the received masterCellGroup according to 5.3.5.5;

1>	if the RRCResume includes the mrdc-SecondaryCellGroup:

2>	if the received mrdc-SecondaryCellGroup is set to nr-SCG:

3>	perform the RRC reconfiguration according to 5.3.5.3 for the RRCReconfiguration message included in nr-SCG;

2>	if the received mrdc-SecondaryCellGroup is set to eutra-SCG:

3>	perform the RRC connection reconfiguration as specified in TS 36.331 [10], clause 5.3.5.3 for the RRCConnectionReconfiguration message included in eutra-SCG;

1>	if the RRCResume includes the radioBearerConfig:

2>	perform the radio bearer configuration according to 5.3.5.6;

1>	if the RRCResume message includes the sk-Counter:

2>	perform security key update procedure as specified in 5.3.5.7;

1>	if the RRCResume message includes the radioBearerConfig2:

2>	perform the radio bearer configuration according to 5.3.5.6;

1>	if the RRCResume message includes the needForGapsConfigNR:

2>	if needForGapsConfigNR is set to setup:

3>	consider itself to be configured to provide the measurement gap requirement information of NR target bands;

2>	else:

3>	consider itself not to be configured to provide the measurement gap requirement information of NR target bands;

1>	if the RRCResume message includes the needForGapNCSG-ConfigNR:

2>	if needForGapNCSG-ConfigNR is set to setup:

3>	consider itself to be configured to provide the measurement gap and NCSG requirement information of NR target bands;

2>	else:

3>	consider itself not to be configured to provide the measurement gap and NCSG requirement information of NR target bands;

1>	if the RRCResume message includes the needForGapNCSG-ConfigEUTRA:

2>	if needForGapNCSG-ConfigEUTRA is set to setup:

3>	consider itself to be configured to provide the measurement gap and NCSG requirement information of E-UTRA target bands;

2>	else:

3>	consider itself not to be configured to provide the measurement gap and NCSG requirement information of E-UTRA target bands;

1>	if the RRCResume message includes the appLayerMeasConfig:

2>	perform the application layer measurement configuration procedure as specified in 5.3.5.13d;

1>	if the RRCResume message includes the sl-L2RemoteUE-Config (i.e. the UE is a L2 U2N Remote UE):

2>	perform the L2 U2N Remote UE configuration procedure as specified in 5.3.5.16;

1>	if the RRCResume message includes the sl-ConfigDedicatedNR:

2>	perform the sidelink dedicated configuration procedure as specified in 5.3.5.14;

1>	resume SRB2 (if suspended), SRB3 (if configured), SRB4 (if configured), all DRBs (that are suspended) and multicast MRBs;

NOTE 1:	If the SCG is deactivated, resuming SRB3 and all DRBs does not imply that PDCP or RRC PDUs can be transmitted or received on SCG RLC bearers.

1>	if stored, discard the cell reselection priority information provided by the cellReselectionPriorities or inherited from another RAT;

1>	stop timer T320, if running;

1>	if the RRCResume message includes the measConfig:

2>	perform the measurement configuration procedure as specified in 5.5.2;

1>	resume measurements if suspended;

1>	if T390 is running:

2>	stop timer T390 for all access categories;

2>	perform the actions as specified in 5.3.14.4;

1>	if T302 is running:

2>	stop timer T302;

2>	perform the actions as specified in 5.3.14.4;

1>	enter RRC_CONNECTED;

1>	indicate to upper layers that the suspended RRC connection has been resumed;

1>	stop the cell re-selection procedure;

1>	stop relay reselection procedure if any for L2 U2N Remote UE;

1>	consider the current cell to be the PCell;

1>	set the content of the of RRCResumeComplete message as follows:

2>	if the upper layer provides NAS PDU, set the dedicatedNAS-Message to include the information received from upper layers;

2>	if upper layers provides a PLMN:

3>	if the UE is either allowed or instructed to access the PLMN via a cell for which at least one CAG ID is broadcast:

4>	set the selectedPLMN-Identity from the npn-IdentityInfoList;

3>	else:

4>	set the selectedPLMN-Identity to the PLMN selected by upper layers from the plmn-IdentityInfoList;

2>	if the masterCellGroup contains the reportUplinkTxDirectCurrent:

3>	include the uplinkTxDirectCurrentList for each MCG serving cell with UL;

3>	include uplinkDirectCurrentBWP-SUL for each MCG serving cell configured with SUL carrier, if any, within the uplinkTxDirectCurrentList;

2>	if the masterCellGroup contains the reportUplinkTxDirectCurrentTwoCarrier:

3>	include in the uplinkTxDirectCurrentTwoCarrierList the list of uplink Tx DC locations for the configured uplink carrier aggregation in the MCG;

2>	if the masterCellGroup contains the reportUplinkTxDirectCurrentMoreCarrier:

3>	include in the uplinkTxDirectCurrentMoreCarrierList the list of uplink Tx DC locations for the configured uplink carrier aggregation in the MCG;

2>	if the UE has idle/inactive measurement information concerning cells other than the PCell available in VarMeasIdleReport:

3>	if the idleModeMeasurementReq is included in the RRCResume message:

4>	set the measResultIdleEUTRA in the RRCResumeComplete message to the value of measReportIdleEUTRA in the VarMeasIdleReport, if available;

4>	set the measResultIdleNR in the RRCResumeComplete message to the value of measReportIdleNR in the VarMeasIdleReport, if available;

4>	discard the VarMeasIdleReport upon successful delivery of the RRCResumeComplete message is confirmed by lower layers;

3>	else:

4>	if the SIB1 contains idleModeMeasurementsNR and the UE has NR idle/inactive measurement information concerning cells other than the PCell available in VarMeasIdleReport; or

4>	if the SIB1 contains idleModeMeasurementsEUTRA and the UE has E-UTRA idle/inactive measurement information available in VarMeasIdleReport:

5>	include the idleMeasAvailable;

2>	if the RRCResume message includes mrdc-SecondaryCellGroup set to eutra-SCG:

3>	include in the eutra-SCG-Response the E-UTRA RRCConnectionReconfigurationComplete message in accordance with TS 36.331 [10] clause 5.3.5.3;

2>	if the RRCResume message includes mrdc-SecondaryCellGroup set to nr-SCG:

3>	include in the nr-SCG-Response the SCG RRCReconfigurationComplete message;

2>	if the UE has logged measurements available for NR and if the RPLMN is included in plmn-IdentityList stored in VarLogMeasReport:

3>	include the logMeasAvailable in the RRCResumeComplete message;

3>	if Bluetooth measurement results are included in the logged measurements the UE has available for NR:

4>	include the logMeasAvailableBT in the RRCResumeComplete message;

3>	if WLAN measurement results are included in the logged measurements the UE has available for NR:

4>	include the logMeasAvailableWLAN in the RRCResumeComplete message;

2>	if the sigLoggedMeasType in VarLogMeasReport is included:

3>	if T330 timer is running and the logged measurements configuration is for NR:

4>	set sigLogMeasConfigAvailable to true in the RRCResumeComplete message;

3>	else:

4>	if the UE has logged measurements available for NR:

5>	set sigLogMeasConfigAvailable to false in the RRCResumeComplete message;

2>	if the UE has connection establishment failure or connection resume failure information available in VarConnEstFailReport or VarConnEstFailReportList and if the RPLMN is equal to plmn-Identity stored in VarConnEstFailReport or in at least one of the entries of VarConnEstFailReportList:

3>	include connEstFailInfoAvailable in the RRCResumeComplete message;

2>	if the UE has radio link failure or handover failure information available in VarRLF-Report and if the RPLMN is included in plmn-IdentityList stored in VarRLF-Report; or

2>	if the UE has radio link failure or handover failure information available in VarRLF-Report of TS 36.331 [10] and if the UE is capable of cross-RAT RLF reporting and if the RPLMN is included in plmn-IdentityList stored in VarRLF-Report of TS 36.331 [10]:

3>	include rlf-InfoAvailable in the RRCResumeComplete message;

2>	if the UE has successful handover information available in VarSuccessHO-Report and if the RPLMN is included in plmn-IdentityList stored in VarSuccessHO-Report:

3>	include successHO-InfoAvailable in the RRCResumeComplete message;

2>	if the UE supports storage of mobility history information and the UE has mobility history information available in VarMobilityHistoryReport:

3>	include the mobilityHistoryAvail in the RRCResumeComplete message;

2>	if speedStateReselectionPars is configured in the SIB2:

3>	include the mobilityState in the RRCResumeComplete message and set it to the mobility state (as specified in TS 38.304 [20]) of the UE just prior to entering RRC_CONNECTED state;

2>	if the UE is configured to provide the measurement gap requirement information of NR target bands:

3>	include the NeedForGapsInfoNR and set the contents as follows:

4> include intraFreq-needForGap and set the gap requirement information of intra-frequency measurement for each NR serving cell;

4>	if requestedTargetBandFilterNR is configured, for each supported NR band that is also included in requestedTargetBandFilterNR, include an entry in interFreq-needForGap and set the gap requirement information for that band; otherwise, include an entry in interFreq-needForGap and set the corresponding gap requirement information for each supported NR band;

2>	if the UE is configured to provide the measurement gap and NCSG requirement information of NR target bands:

3>	include the NeedForGapNCSG-InfoNR and set the contents as follows:

4> include intraFreq-needForNCSG and set the gap and NCSG requirement information of intra-frequency measurement for each NR serving cell;

4>	if requestedTargetBandFilterNCSG-NR is configured:

5>	for each supported NR band included in requestedTargetBandFilterNCSG-NR, include an entry in interFreq-needForNCSG and set the NCSG requirement information for that band;

4>	else:

5>	include an entry for each supported NR band in interFreq-needForNCSG and set the corresponding NCSG requirement information;

2>	if the UE is configured to provide the measurement gap and NCSG requirement information of E-UTRA target bands:

3>	include the NeedForGapNCSG-InfoEUTRA and set the contents as follows:

4>	if requestedTargetBandFilterNCSG-EUTRA is configured:

5>	for each supported E-UTRA band included in requestedTargetBandFilterNCSG-EUTRA, include an entry in needForNCSG-EUTRA and set the NCSG requirement information for that band;

4>	else:

5>	include an entry for each supported E-UTRA band in needForNCSG-EUTRA and set the corresponding NCSG requirement information;

2>	if the SIB1 contains musim-CapabilityRestriction and the UE capability is restricted for MUSIM purpose:

3>	may include the musim-CapabilityRestrictionIndication in the RRCResumeComplete message;

1>	submit the RRCResumeComplete message to lower layers for transmission;

1>	the procedure ends.

NOTE 2:	Network only configures at most one of reportUplinkTxDirectCurrent, reportUplinkTxDirectCurrentTwoCarrier or reportUplinkTxDirectCurrentMoreCarrier in one RRC message.

#### 5.3.13.5	Handling of failure to resume RRC Connection

The UE shall:

1>	if timer T319 expires:

2>	if the UE supports multiple CEF report:

3>	if the UE has connection establishment failure information or connection resume failure information available in VarConnEstFailReport and if the RPLMN is equal to plmn-identity stored in VarConnEstFailReport; and

3>	if the cell identity of current cell is not equal to the cell identity stored in measResultFailedCell in VarConnEstFailReport and if the maxCEFReport-r17 has not been reached:

4>	append the VarConnEstFailReport as a new entry in the VarConnEstFailReportList;

2>	if the UE has connection establishment failure information or connection resume failure information available in VarConnEstFailReport and if the RPLMN is not equal to plmn-identity stored in VarConnEstFailReport; or

2>	if the cell identity of current cell is not equal to the cell identity stored in measResultFailedCell in VarConnEstFailReport:

3>	reset the numberOfConnFail to 0;

2>	if the UE has connection establishment failure information or connection resume failure information available in VarConnEstFailReportList and if the RPLMN is not equal to plmn-identity stored in any entry of VarConnEstFailReportList:

3>	clear the content included in VarConnEstFailReportList;

2> clear the content included in VarConnEstFailReport except for the numberOfConnFail, if any;

2>	store the following connection resume failure information in the VarConnEstFailReport by setting its fields as follows:

3>	set the plmn-Identity to the PLMN selected by upper layers (see TS 24.501 [23]) from the PLMN(s) included in the plmn-IdentityInfoList in SIB1;

3>	set the measResultFailedCell to include the global cell identity, tracking area code, the cell level and SS/PBCH block level RSRP, and RSRQ, and SS/PBCH block indexes, of the failed cell based on the available SSB measurements collected up to the moment the UE detected connection resume failure;

3>	if available, set the measResultNeighCells, in order of decreasing ranking-criterion as used for cell re-selection, to include neighbouring cell measurements for at most the following number of neighbouring cells: 6 intra-frequency and 3 inter-frequency neighbours per frequency as well as 3 inter-RAT neighbours, per frequency/ set of frequencies per RAT and according to the following:

4>	for each neighbour cell included, include the optional fields that are available;

NOTE:	The UE includes the latest results of the available measurements as used for cell reselection evaluation, which are performed in accordance with the performance requirements as specified in TS 38.133 [14].

3>	if available, set the locationInfo as in 5.3.3.7;

3>	set perRAInfoList to indicate the performed random access procedure related information as specified in 5.7.10.5;

3>	if numberOfConnFail is smaller than 8:

4>	increment the numberOfConnFail by 1;

2>	perform the actions upon going to RRC_IDLE as specified in 5.3.11 with release cause 'RRC Resume failure'.

1>	else if upon receiving integrity check failure indication from lower layers while T319 is running:

2>	perform the actions upon going to RRC_IDLE as specified in 5.3.11 with release cause 'RRC Resume failure'.

1>	else if indication from the MCG RLC that the maximum number of retransmissions has been reached is received while SDT procedure is ongoing; or

1>	if random access problem indication is received from MCG MAC while SDT procedure is ongoing; or

1>	if the lower layers indicate that cg-SDT-TimeAlignmentTimer or the configuredGrantTimer expired before receiving network response for the UL CG-SDT transmission with CCCH message while SDT procedure is ongoing; or

1>	if integrity check failure indication is received from lower layers while SDT procedure is ongoing; or

1>	if T319a expires:

2>	consider SDT procedure is not ongoing;

2>	perform the actions upon going to RRC_IDLE as specified in 5.3.11 with release cause 'RRC Resume failure'.

The UE may discard the connection resume failure or connection establishment failure information, i.e. release the UE variable VarConnEstFailReport and the UE variable VarConnEstFailReportList, 48 hours after the last connection resume failure is detected.

The L2 U2N Relay UE either indicates to upper layers (to trigger PC5 unicast link release) or sends Notification message to the connected L2 U2N Remote UE(s) in accordance with 5.8.9.10.

#### 5.3.13.6	Cell re-selection or cell selection or L2 U2N relay (re)selection while T390, T319 or T302 is running or SDT procedure is ongoing (UE in RRC_INACTIVE) or SRS transmission in RRC_INACTIVE is configured

The UE shall:

1>	if cell reselection occurs while T319 or T302 is running or while SDT procedure is ongoing; or

1>	if relay reselection occurs while T319 is running; or

1>	if cell changes due to relay reselection while T302 is running:

2>	perform the actions upon going to RRC_IDLE as specified in 5.3.11 with release cause 'RRC Resume failure';

1>	else if cell selection or reselection occurs while T390 is running, or cell change due to relay selection or reselection occurs while T390 is running:

2>	stop T390 for all access categories;

2>	perform the actions as specified in 5.3.14.4.

1>	else if cell reselection occurs when srs-PosRRC-Inactive is configured:

2>	indicate to the lower layer to stop inactivePosSRS-TimeAlignmentTimer;

2>	release the srs-PosRRC-Inactive.

#### 5.3.13.7	Reception of the RRCSetup by the UE

The UE shall:

1>	perform the RRC connection setup procedure as specified in 5.3.3.4.

#### 5.3.13.8	RNA update

In RRC_INACTIVE state, the UE shall:

1>	if T380 expires; or

1>	if RNA Update is triggered at reception of SIB1, as specified in 5.2.2.4.2:

2>	if T319 is not running or SDT procedure is not ongoing:

3>	initiate RRC connection resume procedure in 5.3.13.2 with resumeCause set to rna-Update;

1>	if barring is alleviated for Access Category '8' or Access Category '2', as specified in 5.3.14.4:

2>	if upper layers do not request RRC the resumption of an RRC connection, and

2>	if the variable pendingRNA-Update is set to true:

3>	initiate RRC connection resume procedure in 5.3.13.2 with resumeCause value set to rna-Update.

If the UE in RRC_INACTIVE state fails to find a suitable cell and camps on the acceptable cell to obtain limited service as defined in TS 38.304 [20], the UE shall:

1>	perform the actions upon going to RRC_IDLE as specified in 5.3.11 with release cause 'other'.

NOTE:	It is left to UE implementation how to behave when T380 expires while the UE is camped neither on a suitable nor on an acceptable cell.

#### 5.3.13.9	Reception of the RRCRelease by the UE

The UE shall:

1>	perform the actions as specified in 5.3.8.

#### 5.3.13.10	Reception of the RRCReject by the UE

The UE shall:

1>	perform the actions as specified in 5.3.15.

#### 5.3.13.11	Inability to comply with RRCResume

The UE shall:

1>	if the UE is unable to comply with (part of) the configuration included in the RRCResume message;

2>	perform the actions upon going to RRC_IDLE as specified in 5.3.11 with release cause ′RRC Resume failure′.

NOTE 1:	The UE may apply above failure handling also in case the RRCResume message causes a protocol error for which the generic error handling as defined in 10 specifies that the UE shall ignore the message.

NOTE 2:	If the UE is unable to comply with part of the configuration, it does not apply any part of the configuration, i.e. there is no partial success/failure.

#### 5.3.13.12	Inter RAT cell reselection

Upon reselecting to an inter-RAT cell, the UE shall:

1>	perform the actions upon going to RRC_IDLE as specified in 5.3.11, with release cause 'other'.

### 5.3.14	Unified Access Control

#### 5.3.14.1	General

The purpose of this procedure is to perform access barring check for an access attempt associated with a given Access Category and one or more Access Identities upon request from upper layers according to TS 24.501 [23] or the RRC layer. This procedure does not apply to IAB-MT. This procedure does not apply to L2 U2N Relay UE initiating RRC connection establishment or RRC connection resume upon reception of any message from a L2 U2N remote UE via SL-RLC0 or SL-RLC1 in accordance to 5.3.3.1a or 5.3.13.1a.

After a PCell change in RRC_CONNECTED the UE shall defer access barring checks until it has obtained SIB1 (as specified in 5.2.2.2) from the target cell.

#### 5.3.14.2	Initiation

Upon initiation of the procedure, the UE shall:

1>	if timer T390 is running for the Access Category:

2>	consider the access attempt as barred;

1>	else if timer T302 is running and the Access Category is neither '2' nor '0':

2>	consider the access attempt as barred;

1>	else:

2>	if the Access Category is '0':

3>	consider the access attempt as allowed;

2>	else:

3>	if SIB1 includes uac-BarringPerPLMN-List that contains a UAC-BarringPerPLMN for the selected PLMN or SNPN:

4>	if the procedure in 5.2.2.4.2 for a selected PLMN resulted in use of information in npn-IdentityInfoList and UAC-BarringPerPLMN has an entry with the plmn-IdentityIndex corresponding to used information in this list:

5>	select the UAC-BarringPerPLMN entry with the plmn-IdentityIndex corresponding to used information in the npn-IdentityInfoList;

4>	else:

5>	select the UAC-BarringPerPLMN entry with the plmn-IdentityIndex corresponding to the selected PLMN and the PLMN-IdentityInfo, if any, or the selected SNPN and the npn-IdentityInfoList;

3>	if any UAC-BarringPerPLMN entry is selected:

4>	in the remainder of this procedure, use the selected UAC-BarringPerPLMN entry (i.e. presence or absence of access barring parameters in this entry) irrespective of the uac-BarringForCommon included in SIB1;

3>	else if SIB1 includes uac-BarringForCommon:

4>	in the remainder of this procedure use the uac-BarringForCommon (i.e. presence or absence of these parameters) included in SIB1;

3>	else:

4>	consider the access attempt as allowed;

3>	if uac-BarringForCommon is applicable or the uac-ACBarringListType indicates that uac-ExplicitACBarringList is used:

4>	if the corresponding UAC-BarringPerCatList contains a UAC-BarringPerCat entry corresponding to the Access Category:

5>	select the UAC-BarringPerCat entry;

5>	if the uac-BarringInfoSetList contains a UAC-BarringInfoSet entry corresponding to the selected uac-barringInfoSetIndex in the UAC-BarringPerCat:

6>	select the UAC-BarringInfoSet entry;

6>	perform access barring check for the Access Category as specified in 5.3.14.5, using the selected UAC-BarringInfoSet as "UAC barring parameter";

5>	else:

6>	consider the access attempt as allowed;

4>	else:

5>	consider the access attempt as allowed;

3>	else if the uac-ACBarringListType indicates that uac-ImplicitACBarringList is used:

4>	select the uac-BarringInfoSetIndex corresponding to the Access Category in the uac-ImplicitACBarringList;

4>	if the uac-BarringInfoSetList contains the UAC-BarringInfoSet entry corresponding to the selected uac-BarringInfoSetIndex:

5>	select the UAC-BarringInfoSet entry;

5>	perform access barring check for the Access Category as specified in 5.3.14.5, using the selected UAC-BarringInfoSet as "UAC barring parameter";

4>	else:

5>	consider the access attempt as allowed;

3>	else:

4>	consider the access attempt as allowed;

1>	if the access barring check was requested by upper layers:

2>	if the access attempt is considered as barred:

3>	if timer T302 is running:

4>	if timer T390 is running for Access Category '2':

5>	inform the upper layer that access barring is applicable for all access categories except categories '0', upon which the procedure ends;

4>	else

5>	inform the upper layer that access barring is applicable for all access categories except categories '0' and '2', upon which the procedure ends;

3>	else:

4>	inform upper layers that the access attempt for the Access Category is barred, upon which the procedure ends;

2>	else:

3>	inform upper layers that the access attempt for the Access Category is allowed, upon which the procedure ends;

1>	else:

2>	the procedure ends.

#### 5.3.14.3	Void

#### 5.3.14.4	T302, T390 expiry or stop (Barring alleviation)

The UE shall:

1>	if timer T302 expires or is stopped:

2>	for each Access Category for which T390 is not running:

3>	consider the barring for this Access Category to be alleviated:

1>	else if timer T390 corresponding to an Access Category other than '2' expires or is stopped, and if timer T302 is not running:

2>	consider the barring for this Access Category to be alleviated;

1>	else if timer T390 corresponding to the Access Category '2' expires or is stopped:

2>	consider the barring for this Access Category to be alleviated;

1>	when barring for an Access Category is considered being alleviated:

2>	if the Access Category was informed to upper layers as barred:

3>	inform upper layers about barring alleviation for the Access Category.

2>	if barring is alleviated for Access Category '8'; or

2>	if barring is alleviated for Access Category '2':

3>	perform actions specified in 5.3.13.8;

#### 5.3.14.5	Access barring check

The UE shall:

1>	if one or more Access Identities equal to 1, 2, 11, 12, 13, 14, or 15 are indicated according to TS 24.501 [23], and

1>	if for at least one of these Access Identities the corresponding bit in the uac-BarringForAccessIdentity contained in "UAC barring parameter" is set to zero:

2>	consider the access attempt as allowed;

1>	else:

2>	if the establishment of the RRC connection is the result of release with redirect with mpsPriorityIndication (either in NR or E-UTRAN); and

2>	if the bit corresponding to Access Identity 1 in the uac-BarringForAccessIdentity contained in the "UAC barring parameter" is set to zero:

3>	consider the access attempt as allowed;

2>	else if Access Identity 3 is indicated:

3>	draw a random number 'rand' uniformly distributed in the range: 0 ≤ rand < 1;

3>	if 'rand' is lower than the value indicated by uac-BarringFactorForAI3 included in "UAC barring parameter":

4>	consider the access attempt as allowed;

3>	else:

4>	consider the access attempt as barred;

2>	else:

3>	draw a random number 'rand' uniformly distributed in the range: 0 ≤ rand < 1;

3>	if 'rand' is lower than the value indicated by uac-BarringFactor included in "UAC barring parameter":

4>	consider the access attempt as allowed;

3>	else:

4>	consider the access attempt as barred;

1>	if the access attempt is considered as barred:

2>	draw a random number 'rand' that is uniformly distributed in the range 0 ≤ rand < 1;

2>	start timer T390 for the Access Category with the timer value calculated as follows, using the uac-BarringTime included in "UAC barring parameter":

T390 = (0.7+ 0.6 * rand) * uac-BarringTime.

### 5.3.15	RRC connection reject

#### 5.3.15.1	Initiation

The UE initiates the procedure upon the reception of RRCReject when the UE tries to establish or resume an RRC connection.

#### 5.3.15.2	Reception of the RRCReject by the UE

The UE shall:

1>	stop timer T300, if running;

1>	stop timer T319, if running;

1>	stop timer T319a, if running and consider SDT procedure is not ongoing;

1>	stop timer T302, if running;

1>	reset MAC and release the default MAC Cell Group configuration;

1>	if waitTime is configured in the RRCReject:

2>	start timer T302, with the timer value set to the waitTime;

1>	if RRCReject is received in response to a request from upper layers:

2>	inform the upper layer that access barring is applicable for all access categories except categories '0' and '2';

1>	if RRCReject is received in response to an RRCSetupRequest:

2>	inform upper layers about the failure to setup the RRC connection, upon which the procedure ends;

1>	else if RRCReject is received in response to an RRCResumeRequest or an RRCResumeRequest1:

2>	if resume is triggered by upper layers:

3>	inform upper layers about the failure to resume the RRC connection;

2>	if resume is triggered due to an RNA update; or

2>	if resume is triggered for SDT and T380 has expired:

3>	set the variable pendingRNA-Update to true;

2>	discard the current KgNB key, the KRRCenc key, the KRRCint key, the KUPint key and the KUPenc key derived in accordance with 5.3.13.3;

2>	if resume is triggered for SDT:

3>	for SRB2, if it is resumed and for SRB1:

4>	trigger the PDCP entity to perform SDU discard as specified in TS 38.323 [5];

4>	re-establish the RLC entity as specified in TS 38.322 [4];

3>	for each DRB that is not suspended:

4>	indicate PDCP suspend to lower layers;

4>	re-establish the RLC entity as specified in TS 38.322 [4];

2>	suspend SRB1 and the radio bearers configured for SDT, if any;

2>	the procedure ends.

Upon L2 U2N Relay UE receives RRCReject, it either indicates to upper layers (to trigger PC5 unicast link release) or sends Notification message to the connected L2 U2N Remote UE(s) in accordance with 5.8.9.10.

The RRC_INACTIVE UE shall continue to monitor paging while the timer T302 is running.

NOTE:	If timer T331 is running, the UE continues to perform idle/inactive measurements according to 5.7.8.

---------------------------------------------------------Skip Unchanged----------------------------------------------------------

## 5.5	Measurements

### 5.5.1	Introduction

The network may configure an RRC_CONNECTED UE to perform measurements. The network may configure the UE to report them in accordance with the measurement configuration or perform conditional reconfiguration evaluation in accordance with the conditional reconfiguration. The measurement configuration is provided by means of dedicated signalling i.e. using the RRCReconfiguration or RRCResume.

The network may configure the UE to perform the following types of measurements:

-	NR measurements;

-	Inter-RAT measurements of E-UTRA frequencies;

-	Inter-RAT measurements of UTRA-FDD frequencies;

-	NR sidelink measurements of L2 U2N Relay UEs.

The network may configure the UE to report the following measurement information based on SS/PBCH block(s):

-	Measurement results per SS/PBCH block;

-	Measurement results per cell based on SS/PBCH block(s);

-	SS/PBCH block(s) indexes.

The network may configure the UE to report the following measurement information based on CSI-RS resources:

-	Measurement results per CSI-RS resource;

-	Measurement results per cell based on CSI-RS resource(s);

-	CSI-RS resource measurement identifiers.

The network may configure the UE to perform the following types of measurements for NR sidelink and V2X sidelink:

-	CBR measurements.

The network may configure the UE to report the following CLI measurement information based on SRS resources:

-	Measurement results per SRS resource;

-	SRS resource(s) indexes.

The network may configure the UE to report the following CLI measurement information based on CLI-RSSI resources:

-	Measurement results per CLI-RSSI resource;

-	CLI-RSSI resource(s) indexes.

The network may configure the UE to report the following Rx-Tx time difference measurement information based on CSI-RS for tracking or PRS:

-	UE Rx-Tx time difference measurement result.

The measurement configuration includes the following parameters:

1.	Measurement objects: A list of objects on which the UE shall perform the measurements.

-	For intra-frequency and inter-frequency measurements a measurement object indicates the frequency/time location and subcarrier spacing of reference signals to be measured. Associated with this measurement object, the network may configure a list of cell specific offsets, a list of 'exclude-listed' cells and a list of 'allow-listed' cells. Exclude-listed cells are not applicable in event evaluation or measurement reporting. Allow-listed cells are the only ones applicable in event evaluation or measurement reporting.

-	The measObjectId of the MO which corresponds to each serving cell is indicated by servingCellMO within the serving cell configuration.

-	For inter-RAT E-UTRA measurements a measurement object is a single E-UTRA carrier frequency. Associated with this E-UTRA carrier frequency, the network can configure a list of cell specific offsets and a list of 'exclude-listed' cells. Exclude-listed cells are not applicable in event evaluation or measurement reporting.

-	For inter-RAT UTRA-FDD measurements a measurement object is a set of cells on a single UTRA-FDD carrier frequency.

-	For NR sidelink measurements of L2 U2N Relay UEs, a measurement object is a single NR sidelink frequency to be measured.

-	For CBR measurement of NR sidelink communication, a measurement object is a set of transmission resource pool(s) on a single carrier frequency for NR sidelink communication.

-	For CBR measurement of NR sidelink discovery, a measurement object is a set of discovery dedicated resource pool(s) or transmission resource pool(s) also used for NR sidelink discovery on a single carrier frequency for NR sidelink discovery.

-	For CLI measurements a measurement object indicates the frequency/time location of SRS resources and/or CLI-RSSI resources, and subcarrier spacing of SRS resources to be measured.

2.	Reporting configurations: A list of reporting configurations where there can be one or multiple reporting configurations per measurement object. Each measurement reporting configuration consists of the following:

-	Reporting criterion: The criterion that triggers the UE to send a measurement report. This can either be periodical or a single event description.

-	RS type: The RS that the UE uses for beam and cell measurement results (SS/PBCH block or CSI-RS).

-	Reporting format: The quantities per cell and per beam that the UE includes in the measurement report (e.g. RSRP) and other associated information such as the maximum number of cells and the maximum number beams per cell to report.

In case of conditional reconfiguration, each configuration consists of the following:

-	Execution criteria: The criteria the UE uses for conditional reconfiguration execution.

-	RS type: The RS that the UE uses for obtaining beam and cell measurement results (SS/PBCH block-based or CSI-RS-based), used for evaluating conditional reconfiguration execution condition.

3.	Measurement identities: For measurement reporting, a list of measurement identities where each measurement identity links one measurement object with one reporting configuration. By configuring multiple measurement identities, it is possible to link more than one measurement object to the same reporting configuration, as well as to link more than one reporting configuration to the same measurement object. The measurement identity is also included in the measurement report that triggered the reporting, serving as a reference to the network. For conditional reconfiguration triggering, one measurement identity links to exactly one conditional reconfiguration trigger configuration. And up to 2 measurement identities can be linked to one conditional reconfiguration execution condition.

4.	Quantity configurations: The quantity configuration defines the measurement filtering configuration used for all event evaluation and related reporting, and for periodical reporting of that measurement. For NR measurements, the network may configure up to 2 quantity configurations with a reference in the NR measurement object to the configuration that is to be used. In each configuration, different filter coefficients can be configured for different measurement quantities, for different RS types, and for measurements per cell and per beam.

5.	Measurement gaps: Periods that the UE may use to perform measurements.

A UE in RRC_CONNECTED maintains a measurement object list, a reporting configuration list, and a measurement identities list according to signalling and procedures in this specification. The measurement object list possibly includes NR measurement object(s), CLI measurement object(s), inter-RAT objects, and L2 U2N Relay objects. Similarly, the reporting configuration list includes NR, inter-RAT, and L2 U2N Relay reporting configurations. Any measurement object can be linked to any reporting configuration of the same RAT type. Some reporting configurations may not be linked to a measurement object. Likewise, some measurement objects may not be linked to a reporting configuration.

The measurement procedures distinguish the following types of cells:

1.	The NR serving cell(s) – these are the SpCell and one or more SCells.

2.	Listed cells – these are cells listed within the measurement object(s).

3.	Detected cells – these are cells that are not listed within the measurement object(s) but are detected by the UE on the SSB frequency(ies) and subcarrier spacing(s) indicated by the measurement object(s).

For NR measurement object(s), the UE measures and reports on the serving cell(s)/serving Relay UE (for L2 U2N Remote UE), listed cells and/or detected cells. For inter-RAT measurements object(s) of E-UTRA, the UE measures and reports on listed cells and detected cells and, for RSSI and channel occupancy measurements, the UE measures and reports on the configured resources on the indicated frequency. For inter-RAT measurements object(s) of UTRA-FDD, the UE measures and reports on listed cells. For CLI measurement object(s), the UE measures and reports on configured measurement resources (i.e. SRS resources and/or CLI-RSSI resources). For L2 U2N Relay object(s), the UE measures and reports on the serving NR cell(s), as well as the discovered L2 U2N Relay UEs.

Whenever the procedural specification, other than contained in clause 5.5.2, refers to a field it concerns a field included in the VarMeasConfig unless explicitly stated otherwise i.e. only the measurement configuration procedure covers the direct UE action related to the received measConfig.

In NR-DC, the UE may receive two independent measConfig:

-	a measConfig, associated with MCG, that is included in the RRCReconfiguration message received via SRB1; and

-	a measConfig, associated with SCG, that is included in the RRCReconfiguration message received via SRB3, or, alternatively, included within a RRCReconfiguration message embedded in a RRCReconfiguration message received via SRB1.

In this case, the UE maintains two independent VarMeasConfig and VarMeasReportList, one associated with each measConfig, and independently performs all the procedures in clause 5.5 for each measConfig and the associated VarMeasConfig and VarMeasReportList, unless explicitly stated otherwise.

The configurations related to CBR measurements are only included in the measConfig associated with MCG.

The configurations related to Rx-Tx time difference measurement are only included in the measConfig associated with MCG.

### 5.5.2	Measurement configuration

#### 5.5.2.1	General

The network applies the procedure as follows:

-	to ensure that, whenever the UE has a measConfig associated with a CG, it includes a measObject for the SpCell and for each NR SCell of the CG to be measured;

-	to configure at most one measurement identity across all CGs using a reporting configuration with the reportType set to reportCGI;

-	to configure at most one measurement identity per the node hosting PDCP entity using a reporting configuration with the ul-DelayValueConfig;

-	to configure at most one measurement identity per the node hosting PDCP entity using a reporting configuration with the ul-ExcessDelayConfig;

-	to ensure that, in the measConfig associated with a CG:

-	for all SSB based measurements there is at most one measurement object with the same ssbFrequency;

-	an smtc1 included in any measurement object with the same ssbFrequency has the same value and that an smtc2 included in any measurement object with the same ssbFrequency has the same value and that an smtc3list included in any measurement object with the same ssbFrequency has the same value and that an smtc4list included in any measurement object with the same ssbFrequency has the same value;

-	to ensure that all measurement objects configured in this specification and in TS 36.331 [10] with the same ssbFrequency have the same ssbSubcarrierSpacing;

-	to ensure that, if a measurement object associated with the MCG has the same ssbFrequency as a measurement object associated with the SCG:

-	for that ssbFrequency, the measurement window according to the smtc1 configured by the MCG includes the measurement window according to the smtc1 configured by the SCG, or vice-versa, with an accuracy of the maximum receive timing difference specified in TS 38.133 [14].

-	if both measurement objects are used for RSSI measurements, bits in measurementSlots in both objects corresponding to the same slot are set to the same value. Also, the endSymbol is the same in both objects.

-	to ensure that, if a measurement object has the same ssbFrequency as a measurement object configured in TS 36.331 [10]:

-	for that ssbFrequency, the measurement window according to the smtc configured in TS 36.331 [10] includes the measurement window according to the smtc1 configured in TS 38.331, or vice-versa, with an accuracy of the maximum receive timing difference specified in TS 38.133 [14].

-	if both measurement objects are used for RSSI measurements, bits in measurementSlots in both objects corresponding to the same slot are set to the same value. Also, the endSymbol is the same in both objects.

-	when the UE is in NE-DC, NR-DC, or NR standalone, to configure at most one measurement identity across all CGs using a reporting configuration with the reportType set to reportSFTD;

For CSI-RS resources, the network applies the procedure as follows:

-	to ensure that all CSI-RS resources configured in each measurement object have the same center frequency, (startPRB+floor(nrofPRBs/2))

-	to ensure that the total number of CSI-RS resources configured in each measurement object does not exceed the maximum number specified in TS 38.214 [19].

The UE shall:

1>	if the received measConfig includes the measObjectToRemoveList:

2>	perform the measurement object removal procedure as specified in 5.5.2.4;

1>	if the received measConfig includes the measObjectToAddModList:

2>	perform the measurement object addition/modification procedure as specified in 5.5.2.5;

1>	if the received measConfig includes the reportConfigToRemoveList:

2>	perform the reporting configuration removal procedure as specified in 5.5.2.6;

1>	if the received measConfig includes the reportConfigToAddModList:

2>	perform the reporting configuration addition/modification procedure as specified in 5.5.2.7;

1>	if the received measConfig includes the quantityConfig:

2>	perform the quantity configuration procedure as specified in 5.5.2.8;

1>	if the received measConfig includes the measIdToRemoveList:

2>	perform the measurement identity removal procedure as specified in 5.5.2.2;

1>	if the received measConfig includes the measIdToAddModList:

2>	perform the measurement identity addition/modification procedure as specified in 5.5.2.3;

1>	if the received measConfig includes the measGapConfig:

2>	perform the measurement gap configuration procedure as specified in 5.5.2.9;

1>	if the received measConfig includes the measGapSharingConfig:

2>	perform the measurement gap sharing configuration procedure as specified in 5.5.2.11;

1>	if the received measConfig includes the s-MeasureConfig:

2>	if s-MeasureConfig is set to ssb-RSRP, set parameter ssb-RSRP of s-MeasureConfig within VarMeasConfig to the threshold value of the RSRP indicated by the received value of s-MeasureConfig which is derived as specified in 6.3.2;

2>	else, set parameter csi-RSRP of s-MeasureConfig within VarMeasConfig to the threshold value of the RSRP indicated by the received value of s-MeasureConfig which is derived as specified in 6.3.2.

#### 5.5.2.2	Measurement identity removal

The UE shall:

1>	for each measId included in the received measIdToRemoveList that is part of the current UE configuration in VarMeasConfig:

2>	remove the entry with the matching measId from the measIdList within the VarMeasConfig;

2>	remove the measurement reporting entry for this measId from the VarMeasReportList, if included;

2>	stop the periodical reporting timer or timer T321 or timer T322, whichever one is running, and reset the associated information (e.g. timeToTrigger) for this measId.

NOTE:	The UE does not consider the message as erroneous if the measIdToRemoveList includes any measId value that is not part of the current UE configuration.

#### 5.5.2.3	Measurement identity addition/modification

The network applies the procedure as follows:

-	configure a measId only if the corresponding measurement object, the corresponding reporting configuration and the corresponding quantity configuration, are configured.

The UE shall:

1>	for each measId included in the received measIdToAddModList:

2>	if an entry with the matching measId exists in the measIdList within the VarMeasConfig:

3>	replace the entry with the value received for this measId;

2>	else:

3>	add a new entry for this measId within the VarMeasConfig;

2>	remove the measurement reporting entry for this measId from the VarMeasReportList, if included;

2>	stop the periodical reporting timer or timer T321 or timer T322, whichever one is running, and reset the associated information (e.g. timeToTrigger) for this measId;

NOTE 1:	If the measId associated with reportConfig for conditional reconfiguration is modified, the conditions are considered to be not fulfilled as specified in 5.3.5.13.4.

2>	if the reportType is set to reportCGI in the reportConfig associated with this measId:

3>	if the measObject associated with this measId concerns E-UTRA:

4>	if the useAutonomousGaps is included in the reportConfig associated with this measId:

5>	start timer T321 with the timer value set to 200 ms for this measId;

4>	else:

5>	start timer T321 with the timer value set to 1 second for this measId;

3>	if the measObject associated with this measId concerns NR:

4>	if the measObject associated with this measId concerns FR1:

5>	if the useAutonomousGaps is included in the reportConfig associated with this measId:

6>	if the UE is a RedCap UE with 1 Rx branch

7>	start timer T321 with the timer value set to 3 seconds for this measId;

6>	else

7>	start timer T321 with the timer value set to 2 seconds for this measId;

5>	else:

6>	start timer T321 with the timer value set to 2 seconds for this measId;

4>	if the measObject associated with this measId concerns FR2:

5>	if the useAutonomousGaps is included in the reportConfig associated with this measId:

6>	if the UE is a RedCap UE with 1 Rx branch

7>	start timer T321 with the timer value set to 6 seconds for this measId;

6>	else

7>	start timer T321 with the timer value set to 5 seconds for this measId;

5>	else:

6>	start timer T321 with the timer value set to 16 seconds for this measId.

2>	if the reportType is set to reportSFTD in the reportConfigNR associated with this measId and the drx-SFTD-NeighMeas is included:

3>	if the measObject associated with this measId concerns FR1:

4>	start timer T322 with the timer value set to 3 seconds for this measId;

3>	if the measObject associated with this measId concerns FR2:

4>	start timer T322 with the timer value set to 24 seconds for this measId.

#### 5.5.2.4	Measurement object removal

The UE shall:

1>	for each measObjectId included in the received measObjectToRemoveList that is part of measObjectList in VarMeasConfig:

2>	remove the entry with the matching measObjectId from the measObjectList within the VarMeasConfig;

2>	remove all measId associated with this measObjectId from the measIdList within the VarMeasConfig, if any;

2>	if a measId is removed from the measIdList:

3>	remove the measurement reporting entry for this measId from the VarMeasReportList, if included;

3>	stop the periodical reporting timer or timer T321 or timer T322, whichever is running, and reset the associated information (e.g. timeToTrigger) for this measId.

NOTE:	The UE does not consider the message as erroneous if the measObjectToRemoveList includes any measObjectId value that is not part of the current UE configuration.

#### 5.5.2.5	Measurement object addition/modification

The UE shall:

1>	for each measObjectId included in the received measObjectToAddModList:

2>	if an entry with the matching measObjectId exists in the measObjectList within the VarMeasConfig, for this entry:

3>	reconfigure the entry with the value received for this measObject, except for the fields cellsToAddModList, excludedCellsToAddModList, allowedCellsToAddModList, cellsToRemoveList, excludedCellsToRemoveList, allowedCellsToRemoveList, tx-PoolMeasToRemoveList, tx-PoolMeasToAddModList, ssb-PositionQCL-CellsToRemoveList, ssb-PositionQCL-CellsToAddModList, cca-CellsToRemoveList, and cca-CellsToAddModList;

3>	if the received measObject includes the cellsToRemoveList:

4>	for each physCellId included in the cellsToRemoveList:

5>	remove the entry with the matching physCellId from the cellsToAddModList;

3>	if the received measObject includes the cellsToAddModList:

4>	for each physCellId value included in the cellsToAddModList:

5>	if an entry with the matching physCellId exists in the cellsToAddModList:

6>	replace the entry with the value received for this physCellId;

5>	else:

6>	add a new entry for the received physCellId to the cellsToAddModList;

3>	if the received measObject includes the excludedCellsToRemoveList:

4>	for each pci-RangeIndex included in the excludedCellsToRemoveList:

5>	remove the entry with the matching pci-RangeIndex from the excludedCellsToAddModList;

NOTE 1:	For each pci-RangeIndex included in the excludedCellsToRemoveList that concerns overlapping ranges of cells, a cell is removed from the exclude-list of cells only if all PCI ranges containing it are removed.

3>	if the received measObject includes the excludedCellsToAddModList:

4>	for each pci-RangeIndex included in the excludedCellsToAddModList:

5>	if an entry with the matching pci-RangeIndex is included in the excludedCellsToAddModList:

6>	replace the entry with the value received for this pci-RangeIndex;

5>	else:

6>	add a new entry for the received pci-RangeIndex to the excludedCellsToAddModList;

3>	if the received measObject includes the allowedCellsToRemoveList:

4>	for each pci-RangeIndex included in the allowedCellsToRemoveList:

5>	remove the entry with the matching pci-RangeIndex from the allowedCellsToAddModList;

NOTE2:	For each pci-RangeIndex included in the allowedCellsToRemoveList that concerns overlapping ranges of cells, a cell is removed from the allow-list of cells only if all PCI ranges containing it are removed.

3>	if the received measObject includes the allowedCellsToAddModList:

4>	for each pci-RangeIndex included in the allowedCellsToAddModList:

5>	if an entry with the matching pci-RangeIndex is included in the allowedCellsToAddModList:

6>	replace the entry with the value received for this pci-RangeIndex;

5>	else:

6>	add a new entry for the received pci-RangeIndex to the allowedCellsToAddModList

3>	for each measId associated with this measObjectId in the measIdList within the VarMeasConfig, if any:

4>	remove the measurement reporting entry for this measId from the VarMeasReportList, if included;

4>	stop the periodical reporting timer or timer T321 or timer T322, whichever one is running, and reset the associated information (e.g. timeToTrigger) for this measId;

3>	if the received measObject includes the tx-PoolMeasToRemoveList:

4>	for each transmission resource pool indicated in tx-PoolMeasToRemoveList:

5>	remove the entry with the matching identity of the transmission resource pool from the tx-PoolMeasToAddModList;

3>	if the received measObject includes the tx-PoolMeasToAddModList:

4>	for each transmission resource pool indicated in tx-PoolMeasToAddModList:

5>	if an entry with the matching identity of the transmission resource pool exists in the tx-PoolMeasToAddModList:

6>	replace the entry with the value received for this transmission resource pool;

5>	else:

6>	add a new entry for the received identity of the transmission resource pool to the tx-PoolMeasToAddModList;

3>	if the received measObject includes the ssb-PositionQCL-CellsToRemoveList:

4>	for each physCellId included in the ssb-PositionQCL-CellsToRemoveList:

5>	remove the entry with the matching physCellId from the ssb-PositionQCL-CellsToAddModList;

3>	if the received measObject includes the ssb-PositionQCL-CellsToAddModList:

4>	for each physCellId included in the ssb-PositionQCL-CellsToAddModList:

5>	if an entry with the matching physCellId exists in the ssb-PositionQCL-CellsToAddModList:

6>	replace the entry with the value received for this physCellId;

5>	else:

6>	add a new entry for the received physCellId to the ssb-PositionQCL-CellsToAddModList;

3>	if the received measObject includes the cca-CellsToRemoveList:

4>	for each physCellId included in the cca-CellsToRemoveList:

5>	remove the entry with the matching physCellId from the cca-CellsToAddModList;

3>	if the received measObject includes the cca-CellsToAddModList:

4>	for each physCellId included in the cca-CellsToAddModList:

5>	if an entry with the matching physCellId exists in the cca-CellsToAddModList:

6>	replace the entry with the value received for this physCellId;

5>	else:

6>	add a new entry for the received physCellId to the cca-CellsToAddModList;

2>	else:

3>	add a new entry for the received measObject to the measObjectList within VarMeasConfig.

#### 5.5.2.6	Reporting configuration removal

The UE shall:

1>	for each reportConfigId included in the received reportConfigToRemoveList that is part of the current UE configuration in VarMeasConfig:

2>	remove the entry with the matching reportConfigId from the reportConfigList within the VarMeasConfig;

2>	remove all measId associated with the reportConfigId from the measIdList within the VarMeasConfig, if any;

2>	if a measId is removed from the measIdList:

3>	remove the measurement reporting entry for this measId from the VarMeasReportList, if included;

3>	stop the periodical reporting timer or timer T321 or timer T322, whichever one is running, and reset the associated information (e.g. timeToTrigger) for this measId.

NOTE:	The UE does not consider the message as erroneous if the reportConfigToRemoveList includes any reportConfigId value that is not part of the current UE configuration.

#### 5.5.2.7	Reporting configuration addition/modification

The UE shall:

1>	for each reportConfigId included in the received reportConfigToAddModList:

2>	if an entry with the matching reportConfigId exists in the reportConfigList within the VarMeasConfig, for this entry:

3>	reconfigure the entry with the value received for this reportConfig;

3>	for each measId associated with this reportConfigId included in the measIdList within the VarMeasConfig, if any:

4>	remove the measurement reporting entry for this measId from the VarMeasReportList, if included;

4>	stop the periodical reporting timer or timer T321 or timer T322, whichever one is running, and reset the associated information (e.g. timeToTrigger) for this measId;

2>	else:

3>	add a new entry for the received reportConfig to the reportConfigList within the VarMeasConfig.

#### 5.5.2.8	Quantity configuration

The UE shall:

1>	for each RAT for which the received quantityConfig includes parameter(s):

2>	set the corresponding parameter(s) in quantityConfig within VarMeasConfig to the value of the received quantityConfig parameter(s);

1>	for each measId included in the measIdList within VarMeasConfig:

2>	remove the measurement reporting entry for this measId from the VarMeasReportList, if included;

2>	stop the periodical reporting timer or timer T321 or timer T322, whichever one is running, and reset the associated information (e.g. timeToTrigger) for this measId.

#### 5.5.2.9	Measurement gap configuration

The UE shall:

1>	if gapFR1 is set to setup:

2>	if an FR1 measurement gap configuration configured by gapFR1 is already setup, release the FR1 measurement gap configuration;

2>	setup the FR1 measurement gap configuration indicated by the gapFR1 in accordance with the received gapOffset, i.e., the first subframe of each gap occurs at an SFN and subframe meeting the following condition:

SFN mod T = FLOOR(gapOffset/10);

subframe = gapOffset mod 10;

with T = MGRP/10 as defined in TS 38.133 [14];

2>	apply the specified timing advance mgta to the gap occurrences calculated above (i.e. the UE starts the measurement mgta ms before the gap subframe occurrences);

1>	else if gapFR1 is set to release:

2>	release the FR1 measurement gap configuration configured by gapFR1;

1>	if gapFR2 is set to setup:

2>	if an FR2 measurement gap configuration configured by gapFR2 is already setup, release the FR2 measurement gap configuration;

2>	setup the FR2 measurement gap configuration indicated by the gapFR2 in accordance with the received gapOffset, i.e., the first subframe of each gap occurs at an SFN and subframe meeting the following condition:

SFN mod T = FLOOR(gapOffset/10);

subframe = gapOffset mod 10;

with T = MGRP/10 as defined in TS 38.133 [14];

2>	apply the specified timing advance mgta to the gap occurrences calculated above (i.e. the UE starts the measurement mgta ms before the gap subframe occurrences);

1>	else if gapFR2 is set to release:

2>	release the FR2 measurement gap configuration configured by gapFR2;

1>	if gapUE is set to setup:

2>	if a per UE measurement gap configuration configured by gapUE is already setup, release the per UE measurement gap configuration;

2>	setup the per UE measurement gap configuration indicated by the gapUE in accordance with the received gapOffset, i.e., the first subframe of each gap occurs at an SFN and subframe meeting the following condition:

SFN mod T = FLOOR(gapOffset/10);

subframe = gapOffset mod 10;

with T = MGRP/10 as defined in TS 38.133 [14];

2>	apply the specified timing advance mgta to the gap occurrences calculated above (i.e. the UE starts the measurement mgta ms before the gap subframe occurrences);

1>	else if gapUE is set to release:

2>	release the per UE measurement gap configuration configured by gapUE.

1>	for each measGapId included in the received gapToReleaseList:

2>	release the measurement gap configuration associated with the measGapId;

1>	for each measPosPreConfigGapId included in the received posMeasGapPreConfigToReleaseList:

2>	release the measurement gap configuration associated with the measPosPreConfigGapId;

1>	for each GapConfig received in gapToAddModList:

2>	setup measurement gap configuration indicated by the GapConfig in accordance with the received gapOffset, i.e., the first subframe of each gap occurs at an SFN and subframe meeting the following condition:

SFN mod T = FLOOR(gapOffset/10);

subframe = gapOffset mod 10;

with T = MGRP/10 as defined in TS 38.133 [14];

2>	apply the specified timing advance mgta to the gap occurrences calculated above (i.e. the UE starts the measurement mgta ms before the gap subframe occurrences);

2>	apply the measurement gap as per UE measurement gap, FR1 measurement gap, or FR2 measurement gap according to the gapType indicated by the GapConfig;

2>	associate the measurement gap with the measGapId indicated by the GapConfig;

2>	if gapSharing in the GapConfig is present:

3>	setup the gap sharing configuration for the measurement gap in accordance with the received gapSharing as defined in TS 38.133 [14];

2>	else:

3>	release the gap sharing configuration (if configured) for the measurement gap;

1>	for each PosGapConfig received in PosMeasGapPreConfigToAddModList:

2>	if a measurement gap configuration associated with the measPosPreConfigGapId indicated by the PosGapConfig is already setup:

3>	release the measurement gap configuration;

2>	setup measurement gap configuration indicated by the PosGapConfig in accordance with the received gapOffset, i.e., the first subframe of each gap occurs at an SFN and subframe meeting the following condition:

SFN mod T = FLOOR(gapOffset/10);

subframe = gapOffset mod 10;

with T = MGRP/10 as defined in TS 38.133 [14];

2>	apply the specified timing advance mgta to the gap occurrences calculated above (i.e. the UE starts the measurement mgta ms before the gap subframe occurrences);

2>	configure the measurement gap as indicated by gapType;

1>	for each FR1, FR2, and per UE measurement gap that is setup:

2>	if the measurement gap is configured by GapConfig and preConfigInd-r17 in the corresponding GapConfig is present:

3>	determine whether the measurement gap is activated or not according to TS 38.133 [14];

2>	else if the measurement gap is configured by PosGapConfig:

3>	consider the measurement gap to be deactivated;

2>	else:

3>	consider the measurement gap to be activated.

NOTE 1:	For FR2 gap configuration with synchronous CA, for the UE in NE-DC or NR-DC, the SFN and subframe of the serving cell indicated by the refServCellIndicator is used in the gap calculation. Otherwise, the SFN and subframe of a serving cell on FR2 frequency is used in the gap calculation

NOTE 2:	For FR1 gap or per UE gap configuration, for the UE in NE-DC or NR-DC, the SFN and subframe of the serving cell indicated by the refServCellIndicator in is used in the gap calculation. Otherwise, the SFN and subframe of the PCell is used in the gap calculation.

NOTE 3:	For FR2 gap configuration with asynchronous CA, for the UE in NE-DC or NR-DC, the SFN and subframe of the serving cell indicated by the refServCellIndicator and refFR2ServCellAsyncCA is used in the gap calculation. Otherwise, the SFN and subframe of a serving cell on FR2 frequency indicated by the refFR2ServCellAsyncCA is used in the gap calculation

#### 5.5.2.10	Reference signal measurement timing configuration

The UE shall setup the first SS/PBCH block measurement timing configuration (SMTC) in accordance with the received periodicityAndOffset parameter (providing Periodicity and Offset value for the following condition) in the smtc1 configuration. The first subframe of each SMTC occasion occurs at an SFN and subframe of the NR SpCell meeting the following condition:

SFN mod T = (FLOOR (Offset/10));

if the Periodicity is larger than sf5:

subframe = Offset mod 10;

else:

subframe = Offset or (Offset +5);

with T = CEIL(Periodicity/10).

If smtc2 is present, for cells indicated in the pci-List parameter in smtc2 in the same MeasObjectNR, the UE shall setup an additional SS/PBCH block measurement timing configuration (SMTC) in accordance with the received periodicity parameter in the smtc2 configuration and use the Offset (derived from parameter periodicityAndOffset) and duration parameter from the smtc1 configuration. The first subframe of each SMTC occasion occurs at an SFN and subframe of the NR SpCell meeting the above condition.

If smtc2-LP is present, for cells indicated in the pci-List parameter in smtc2-LP in the same frequency (for intra frequency cell reselection) or different frequency (for inter frequency cell reselection), the UE shall setup an additional SS/PBCH block measurement timing configuration (SMTC) in accordance with the received periodicity parameter in the smtc2-LP configuration and use the Offset (derived from parameter periodicityAndOffset) and duration parameter from the smtc configuration for that frequency. The first subframe of each SMTC occasion occurs at an SFN and subframe of the NR SpCell or serving cell (for cell reselection) meeting the above condition.

If smtc3list is present, for cells indicated in the pci-List parameter in each SSB-MTC3 element of the list in the same MeasObjectNR, the IAB-MT shall setup an additional SS block measurement timing configuration in accordance with the received periodicityAndOffset parameter (using same condition as smtc1 to identify the SFN and the subframe for SMTC occasion) in each SSB-MTC3 configuration and use the duration and ssb-ToMeasure parameters from each SSB-MTC3 configuration.

If smtc4list is present, for cells indicated in the pci-List parameter in each SSB-MTC4 element of the list in the same MeasObjectNR, the UE shall setup an additional SS/PBCH block measurement timing configuration (SMTC) in accordance with the received Offset parameter in the smtc4 configuration and use the periodicity (derived from parameter periodicityAndOffset) and duration parameter from the smtc1 configuration. The first subframe of each SMTC occasion occurs at an SFN and subframe of the NR SpCell meeting the above condition.

On the indicated ssbFrequency, the UE shall not consider SS/PBCH block transmission in subframes outside the SMTC occasion for RRM measurements based on SS/PBCH blocks and for RRM measurements based on CSI-RS except for SFTD measurement (see TS 38.133 [14], clause 9.3.8).

#### 5.5.2.10a	RSSI measurement timing configuration

The UE shall setup the RSSI measurement timing configuration (RMTC) in accordance with the received rmtc-Periodicity and, if configured, with rmtc-SubframeOffset i.e. the first symbol of each RMTC occasion occurs at first symbol of an SFN and subframe of the NR SpCell meeting the following condition:

SFN mod T = FLOOR(rmtc-SubframeOffset/10);

subframe = rmtc-SubframeOffset mod 10;

with T = rmtc-Periodicity/10;

On the frequency configured by rmtc-Frequency, the UE shall not consider RSSI measurements outside the configured RMTC occasion which lasts for measDurationSymbols for RSSI and channel occupancy measurements.

The UE derives the RSSI measurement duration from a combination of measDurationSymbols and ref-SCS-CP. At least for RSSI measurement confined within the active DL BWP, the UE performs RSSI measurement using the numerology of the active DL BWP during the derived measurement duration. Otherwise, the numerology used by the UE for measurements is up to UE implementation. If configured, the UE performs RSSI measurements on a bandwidth in accordance with the received rmtc-Bandwidth. If configured, the UE performs RSSI measurements according to the TCI state configured by tci-StateId in the reference BWP configured by ref-BWPId of the reference serving cell configured by ref-ServCellId (see TS 38.133 [14], clause 9.2A.7 and clause 9.3A.8).If the UE has no serving cell in FR2-2 and configured with inter-frequency RSSI measurement in FR2-2, it is up to the UE implementation how to determine the spatial domain filter for the inter-frequency RSSI measurement in FR2-2.

#### 5.5.2.11	Measurement gap sharing configuration

The UE shall:

1>	if gapSharingFR1 is set to setup:

2>	if an FR1 measurement gap sharing configuration configured by gapSharingFR1 is already setup:

3>	release the FR1 measurement gap sharing configuration configured by gapSharingFR1;

2>	setup the FR1 measurement gap sharing configuration indicated by the measGapSharingConfig in accordance with the received gapSharingFR1 as defined in TS 38.133 [14];

1>	else if gapSharingFR1 is set to release:

2>	release the FR1 measurement gap sharing configuration configured by gapSharingFR1;

1>	if gapSharingFR2 is set to setup:

2>	if an FR2 measurement gap sharing configuration configured by gapSharingFR2 is already setup:

3>	release the FR2 measurement gap sharing configuration configured by gapSharingFR2;

2>	setup the FR2 measurement gap sharing configuration indicated by the measGapSharingConfig in accordance with the received gapSharingFR2 as defined in TS 38.133 [14];

1>	else if gapSharingFR2 is set to release:

2>	release the FR2 measurement gap sharing configuration configured by gapSharingFR2.

1>	if gapSharingUE is set to setup:

2>	if a per UE measurement gap sharing configuration configured by gapSharingUE is already setup:

3>	release the per UE measurement gap sharing configuration configured by gapSharingUE;

2>	setup the per UE measurement gap sharing configuration indicated by the measGapSharingConfig in accordance with the received gapSharingUE as defined in TS 38.133 [14];

1>	else if gapSharingUE is set to release:

2>	release the per UE measurement gap sharing configuration configured by gapSharingUE.

### 5.5.3	Performing measurements

#### 5.5.3.1	General

An RRC_CONNECTED UE shall derive cell measurement results by measuring one or multiple beams associated per cell as configured by the network, as described in 5.5.3.3. For all cell measurement results, except for RSSI, and CLI measurement results in RRC_CONNECTED, the UE applies the layer 3 filtering as specified in 5.5.3.2, before using the measured results for evaluation of reporting criteria, measurement reporting or the criteria to trigger conditional reconfiguration execution. For cell measurements, the network can configure RSRP, RSRQ, SINR, RSCP or EcN0 as trigger quantity. For CLI measurements, the network can configure SRS-RSRP or CLI-RSSI as trigger quantity. For cell and beam measurements, reporting quantities can be any combination of quantities (i.e. only RSRP; only RSRQ; only SINR; RSRP and RSRQ; RSRP and SINR; RSRQ and SINR; RSRP, RSRQ and SINR; only RSCP; only EcN0; RSCP and EcN0), irrespective of the trigger quantity, and for CLI measurements, reporting quantities can be either SRS-RSRP or CLI-RSSI. For conditional reconfiguration execution, the network can configure up to 2 quantities, both using same RS type. The UE does not apply the layer 3 filtering as specified in 5.5.3.2 to derive the CBR measurements. The UE does not apply the layer 3 filtering as specified in 5.5.3.2 to derive the Rx-Tx time difference measurements.

The network may also configure the UE to report measurement information per beam (which can either be measurement results per beam with respective beam identifier(s) or only beam identifier(s)), derived as described in 5.5.3.3a. If beam measurement information is configured to be included in measurement reports, the UE applies the layer 3 beam filtering as specified in 5.5.3.2. On the other hand, the exact L1 filtering of beam measurements used to derive cell measurement results is implementation dependent.

The UE shall:

1>	whenever the UE has a measConfig, perform RSRP and RSRQ measurements for each serving cell for which servingCellMO is configured as follows:

2>	if the reportConfig associated with at least one measId included in the measIdList within VarMeasConfig contains an rsType set to ssb and ssb-ConfigMobility is configured in the measObject indicated by the servingCellMO:

3>	if the reportConfig associated with at least one measId included in the measIdList within VarMeasConfig contains a reportQuantityRS-Indexes and maxNrofRS-IndexesToReport and contains an rsType set to ssb:

4>	derive layer 3 filtered RSRP and RSRQ per beam for the serving cell based on SS/PBCH block, as described in 5.5.3.3a;

3>	derive serving cell measurement results based on SS/PBCH block, as described in 5.5.3.3;

2>	if the reportConfig associated with at least one measId included in the measIdList within VarMeasConfig contains an rsType set to csi-rs and CSI-RS-ResourceConfigMobility is configured in the measObject indicated by the servingCellMO:

3>	if the reportConfig associated with at least one measId included in the measIdList within VarMeasConfig contains a reportQuantityRS-Indexes and maxNrofRS-IndexesToReport and contains an rsType set to csi-rs:

4>	derive layer 3 filtered RSRP and RSRQ per beam for the serving cell based on CSI-RS, as described in 5.5.3.3a;

3>	derive serving cell measurement results based on CSI-RS, as described in 5.5.3.3;

1>	for each serving cell for which servingCellMO is configured, if the reportConfig associated with at least one measId included in the measIdList within VarMeasConfig contains SINR as trigger quantity and/or reporting quantity:

2>	if the reportConfig contains rsType set to ssb and ssb-ConfigMobility is configured in the servingCellMO:

3>	if the reportConfigcontains a reportQuantityRS-Indexes and maxNrofRS-IndexesToReport:

4>	derive layer 3 filtered SINR per beam for the serving cell based on SS/PBCH block, as described in 5.5.3.3a;

3>	derive serving cell SINR based on SS/PBCH block, as described in 5.5.3.3;

2>	if the reportConfig contains rsType set to csi-rs and CSI-RS-ResourceConfigMobility is configured in the servingCellMO:

3>	if the reportConfigcontains a reportQuantityRS-Indexes and maxNrofRS-IndexesToReport:

4>	derive layer 3 filtered SINR per beam for the serving cell based on CSI-RS, as described in 5.5.3.3a;

3>	derive serving cell SINR based on CSI-RS, as described in 5.5.3.3;

1>	for each measId included in the measIdList within VarMeasConfig:

2>	if the reportType for the associated reportConfig is set to reportCGI and timer T321 is running:

3>	if useAutonomousGaps is configured for the associated reportConfig:

4>	perform the corresponding measurements on the frequency and RAT indicated in the associated measObject using autonomous gaps as necessary;

3>	else:

4>	perform the corresponding measurements on the frequency and RAT indicated in the associated measObject using available idle periods;

3>	if the cell indicated by reportCGI field for the associated measObject is an NR cell and that indicated cell is broadcasting SIB1 (see TS 38.213 [13], clause 13):

4>	try to acquire SIB1 in the concerned cell;

3>	if the cell indicated by reportCGI field is an E-UTRA cell:

4>	try to acquire SystemInformationBlockType1 in the concerned cell;

2>	if the ul-DelayValueConfig is configured for the associated reportConfig:

3>	ignore the measObject;

3>	for each of the configured DRBs, configure the PDCP layer to perform corresponding average UL PDCP packet delay measurement per DRB;

2>	if the ul-ExcessDelayConfig is configured for the associated reportConfig:

3>	ignore the measObject;

3>	for each of the configured DRBs, configure the PDCP layer to perform corresponding UL PDCP Excess Packet Delay delay measurement according to the configured threshold per DRB;

2>	if the reportType for the associated reportConfig is periodical, eventTriggered; or

2>	if the reportType for the associated reportConfig is condTriggerConfig, the measId is within the MCG measConfig and is indicated in the condExecutionCond associated to a condReconfigId in the MCG VarConditionalReconfig (for CHO, CPA or MN-initiated inter-SN CPC in NR-DC); or

2>	if the reportType for the associated reportConfig is condTriggerConfig, the measId is within the SCG VarMeasConfig and is indicated in the condExecutionCond associated to a condReconfigId in the SCG VarConditionalReconfig (for intra-SN CPC); or

2>	if the reportType for the associated reportConfig is condTriggerConfig, the measId is within the SCG VarMeasConfig and is indicated in the condExecutionCondSCG associated to a condReconfigId in the MCG VarConditionalReconfig (for SN-initiated inter-SN CPC in NR-DC); or

2>	if the reportType for the associated reportConfig is condTriggerConfig, the measId is within the SCG VarMeasConfig and is indicated in the triggerConditionSN associated to a condReconfigurationId in VarConditionalReconfiguration as specified in TS 36.331 [10] (for SN-initiated inter-SN CPC in EN-DC):

3>	if a measurement gap configuration is setup, or

3>	if the UE does not require measurement gaps to perform the concerned measurements:

4>	if s-MeasureConfig is not configured, or

4>	if s-MeasureConfig is set to ssb-RSRP and the NR SpCell RSRP based on SS/PBCH block, after layer 3 filtering, is lower than ssb-RSRP, or

4>	if s-MeasureConfig is set to csi-RSRP and the NR SpCell RSRP based on CSI-RS, after layer 3 filtering, is lower than csi-RSRP:

5>	if the measObject is associated to NR and the rsType is set to csi-rs:

6>	if reportQuantityRS-Indexes and maxNrofRS-IndexesToReport for the associated reportConfig are configured:

7>	derive layer 3 filtered beam measurements only based on CSI-RS for each measurement quantity indicated in reportQuantityRS-Indexes, as described in 5.5.3.3a;

6>	derive cell measurement results based on CSI-RS for the trigger quantity and each measurement quantity indicated in reportQuantityCell using parameters from the associated measObject, as described in 5.5.3.3;

5>	if the measObject is associated to NR and the rsType is set to ssb:

6>	if reportQuantityRS-Indexes and maxNrofRS-IndexesToReport for the associated reportConfig are configured:

7>	derive layer 3 beam measurements only based on SS/PBCH block for each measurement quantity indicated in reportQuantityRS-Indexes, as described in 5.5.3.3a;

6>	derive cell measurement results based on SS/PBCH block for the trigger quantity and each measurement quantity indicated in reportQuantityCell using parameters from the associated measObject, as described in 5.5.3.3;

5>	if the measObject is associated to E-UTRA:

6>	perform the corresponding measurements associated to neighbouring cells on the frequencies indicated in the concerned measObject, as described in 5.5.3.2;

5>	if the measObject is associated to UTRA-FDD:

6>	perform the corresponding measurements associated to neighbouring cells on the frequencies indicated in the concerned measObject, as described in 5.5.3.2;

5>	if the measObject is associated to L2 U2N Relay UE:

6>	perform the corresponding measurements associated to candidate Relay UEs on the frequencies indicated in the concerned measObject, as described in 5.5.3.4;

4>	if the measRSSI-ReportConfig is configured in the associated reportConfig:

5>	perform the RSSI and channel occupancy measurements on the frequency indicated in the associated measObject;

NOTE 0:	The network avoids configuring UEs supporting only CHO and/or Rel-16 CPC with measurements not referred to by any execution condition.

2>	if the reportType for the associated reportConfig is set to reportSFTD and the numberOfReportsSent as defined within the VarMeasReportList for this measId is less than one:

3>	if the reportSFTD-Meas is set to true:

4>	if the measObject is associated to E-UTRA:

5>	perform SFTD measurements between the PCell and the E-UTRA PSCell;

5>	if the reportRSRP is set to true;

6>	perform RSRP measurements for the E-UTRA PSCell;

4>	else if the measObject is associated to NR:

5>	perform SFTD measurements between the PCell and the NR PSCell;

5>	if the reportRSRP is set to true;

6>	perform RSRP measurements for the NR PSCell based on SSB;

3>	else if the reportSFTD-NeighMeas is included:

4>	if the measObject is associated to NR:

5>	if the drx-SFTD-NeighMeas is included:

6>	perform SFTD measurements between the PCell and the NR neighbouring cell(s) detected based on parameters in the associated measObject using available idle periods;

5>	else:

6>	perform SFTD measurements between the PCell and the NR neighbouring cell(s) detected based on parameters in the associated measObject;

5>	if the reportRSRP is set to true:

6>	perform RSRP measurements based on SSB for the NR neighbouring cell(s) detected based on parameters in the associated measObject;

2>	if the reportType for the associated reportConfig is cli-Periodical or cli-EventTriggered:

3>	perform the corresponding measurements associated to CLI measurement resources indicated in the concerned measObjectCLI;

2>	perform the evaluation of reporting criteria as specified in 5.5.4, except if reportConfig is condTriggerConfig.

The UE acting as a L2 U2N Remote UE whenever configured with measConfig shall:

1>	perform the corresponding measurements associated to the serving L2 U2N Relay UE, as described in 5.5.3.4;

NOTE 1:	The evaluation of conditional reconfiguration execution criteria is specified in 5.3.5.13.

The UE capable of Rx-Tx time difference measurement when configured with measObjectRxTxDiff shall:

1>	perform the corresponding Rx-Tx time difference measurements associated with downlink reference signals indicated in the concerned measObjectRxTxDiff.

The UE capable of CBR measurement when configured to transmit NR sidelink communication/discovery shall:

1>	If the frequency used for NR sidelink communication/discovery is included in sl-FreqInfoToAddModList in sl-ConfigDedicatedNR within RRCReconfiguration message or included in sl-ConfigCommonNR within SIB12:

2>	if the UE is in RRC_IDLE or in RRC_INACTIVE:

3>	if configured with NR sidelink communication and the cell chosen for NR sidelink communication provides SIB12 which includes sl-TxPoolSelectedNormal or sl-TxPoolExceptional for the concerned frequency; or

3>	if configured with NR sidelink discovery and the cell chosen for NR sidelink discovery provides SIB12 which includes sl-TxPoolSelectedNormal or sl-TxPoolExceptional but does not include sl-DiscTxPoolSelected for the concerned frequency:

4>	perform CBR measurement on pool(s) in sl-TxPoolSelectedNormal or sl-TxPoolExceptional for the concerned frequency in SIB12;

3>	if configured with NR sidelink discovery and the cell chosen for NR sidelink discovery provides SIB12 which includes sl-DiscTxPoolSelected for the concerned frequency:

4>	perform CBR measurement on pools in sl-DiscTxPoolSelected and sl-TxPoolExceptional for the concerned frequency in SIB12;

2>	if the UE is in RRC_CONNECTED:

3>	if tx-PoolMeasToAddModList is included in VarMeasConfig:

4>	perform CBR measurements on each transmission resource pool indicated in the tx-PoolMeasToAddModList;

3>	if sl-DiscTxPoolSelected, sl-TxPoolSelectedNormal, sl-TxPoolScheduling or sl-TxPoolExceptional is included in sl-ConfigDedicatedNR for the concerned frequency within RRCReconfiguration:

4>	perform CBR measurement on pool(s) in sl-DiscTxPoolSelected, sl-TxPoolSelectedNormal, sl-TxPoolScheduling and sl-TxPoolExceptional if included in sl-ConfigDedicatedNR for the concerned frequency within RRCReconfiguration;

3>	else:

4>	if configured with NR sidelink communication and the cell chosen for NR sidelink communication provides SIB12 which includes sl-TxPoolSelectedNormal or sl-TxPoolExceptional for the concerned frequency; or

4>	if configured with NR sidelink discovery and the cell chosen for NR sidelink discovery provides SIB12 which includes sl-TxPoolSelectedNormal or sl-TxPoolExceptional but does not provide sl-DiscTxPoolSelected for the concerned frequency:

5>	perform CBR measurement on pool(s) in sl-TxPoolSelectedNormal or sl-TxPoolExceptional for the concerned frequency in SIB12;

4>	if configured with NR sidelink discovery and the cell chosen for NR sidelink discovery provides SIB12 which includes sl-DiscTxPoolSelected for the concerned frequency:

5>	perform CBR measurement on pools in sl-DiscTxPoolSelected and sl-TxPoolExceptional for the concerned frequency in SIB12;

1>	else:

2>	if configured with NR sidelink communication and sl-TxPoolSelectedNormal is included in SidelinkPreconfigNR for the concerned frequency; or

2>	if configured with NR sidelink discovery and sl-TxPoolSelectedNormal is included in SidelinkPreconfigNR but sl-DiscTxPoolSelected is not included in SidelinkPreconfigNR for the concerned frequency:

3>	perform CBR measurement on pool(s) in sl-TxPoolSelectedNormal in SidelinkPreconfigNR for the concerned frequency.

2>	if configured with NR sidelink discovery and sl-DiscTxPoolSelected is included in SidelinkPreconfigNR for the concerned frequency:

3>	perform CBR measurement on pools in sl-DiscTxPoolSelected if included in SidelinkPreconfigNR.

NOTE 2:	In case the configurations for NR sidelink communication and CBR measurement are acquired via the E-UTRA, configurations for NR sidelink communication in SIB12, sl-ConfigDedicatedNR within RRCReconfiguration used in this clause are provided by the configurations in SystemInformationBlockType28, sl-ConfigDedicatedForNR within RRCConnectionReconfiguration as specified in TS 36.331[10], respectively.

NOTE 3:	If a UE that is configured by upper layers to transmit V2X sidelink communication is configured by NR with transmission resource pool(s) and the measurement objects concerning V2X sidelink communication (i.e. by sl-ConfigDedicatedEUTRA-Info), it shall perform CBR measurement as specified in clause 5.5.3 of TS 36.331 [10], based on the transmission resource pool(s) and the measurement object(s) concerning V2X sidelink communication configured by NR.

NOTE 4:	For V2X sidelink communication, each of the CBR measurement results is associated with a resource pool, as indicated by the poolReportId (see TS 36.331 [10]), that refers to a pool as included in sl-ConfigDedicatedEUTRA-Info or SIB13.

#### 5.5.3.2	Layer 3 filtering

The UE shall:

1>	for each cell measurement quantity, each beam measurement quantity, each sidelink measurement quantity as needed in clause 5.8.10, for each CLI measurement quantity that the UE performs measurements according to 5.5.3.1, and for each candidate L2 U2N Relay UE measurement quantity according to 5.5.3.4:

2>	filter the measured result, before using for evaluation of reporting criteria or for measurement reporting, by the following formula:

Fn = (1 – a)*Fn-1 + a*Mn

where

Mn is the latest received measurement result from the physical layer;

Fn is the updated filtered measurement result, that is used for evaluation of reporting criteria or for measurement reporting;

Fn-1 is the old filtered measurement result, where F0 is set to M1 when the first measurement result from the physical layer is received; and for MeasObjectNR, a = 1/2(ki/4), where ki is the filterCoefficient for the corresponding measurement quantity of the i:th QuantityConfigNR in quantityConfigNR-List, and i is indicated by quantityConfigIndex in MeasObjectNR; for other measurements, a = 1/2(k/4), where k is the filterCoefficient for the corresponding measurement quantity received by the quantityConfig; for UTRA-FDD, a = 1/2(k/4), where k is the filterCoefficient for the corresponding measurement quantity received by quantityConfigUTRA-FDD in the QuantityConfig;

2>	adapt the filter such that the time characteristics of the filter are preserved at different input rates, observing that the filterCoefficient k assumes a sample rate equal to X ms; The value of X is equivalent to one intra-frequency L1 measurement period as defined in TS 38.133 [14] assuming non-DRX operation, and depends on frequency range.

NOTE 1:	If k is set to 0, no layer 3 filtering is applicable.

NOTE 2:	The filtering is performed in the same domain as used for evaluation of reporting criteria or for measurement reporting, i.e., logarithmic filtering for logarithmic measurements.

NOTE 3:	The filter input rate is implementation dependent, to fulfil the performance requirements set in TS 38.133 [14]. For further details about the physical layer measurements, see TS 38.133 [14].

NOTE 4:	For CLI-RSSI measurement, it is up to UE implementation whether to reset filtering upon BWP switch.

#### 5.5.3.3	Derivation of cell measurement results

The network may configure the UE in RRC_CONNECTED to derive RSRP, RSRQ and SINR measurement results per cell associated to NR measurement objects based on parameters configured in the measObject (e.g. maximum number of beams to be averaged and beam consolidation thresholds) and in the reportConfig (rsType to be measured, SS/PBCH block or CSI-RS).

The network may configure the UE in RRC_IDLE or in RRC_INACTIVE to derive RSRP and RSRQ measurement results per cell associated to NR carriers based on parameters configured in measIdleCarrierListNR within VarMeasIdleConfig for measurements performed according to 5.7.8.2a.

The UE shall:

1>	for each cell measurement quantity to be derived based on SS/PBCH block:

2>	if nrofSS-BlocksToAverage is not configured in the associated measObject in RRC_CONNECTED or in the associated entry in measIdleCarrierListNR within VarMeasIdleConfig in RRC_IDLE/RRC_INACTIVE; or

2>	if absThreshSS-BlocksConsolidation is not configured in the associated measObject in RRC_CONNECTED or in the associated entry in measIdleCarrierListNR within VarMeasIdleConfig in RRC_IDLE/RRC_INACTIVE; or

2>	if the highest beam measurement quantity value is below or equal to absThreshSS-BlocksConsolidation:

3>	derive each cell measurement quantity based on SS/PBCH block as the highest beam measurement quantity value, where each beam measurement quantity is described in TS 38.215 [9];

2>	else:

3>	derive each cell measurement quantity based on SS/PBCH block as the linear power scale average of the highest beam measurement quantity values above absThreshSS-BlocksConsolidation where the total number of averaged beams shall not exceed nrofSS-BlocksToAverage, and where each beam measurement quantity is described in TS 38.215 [9];

2>	if in RRC_CONNECTED, apply layer 3 cell filtering as described in 5.5.3.2;

1>	for each cell measurement quantity to be derived based on CSI-RS:

2>	consider a CSI-RS resource to be applicable for deriving cell measurements when the concerned CSI-RS resource is included in the csi-rs-CellMobility including the physCellId of the cell in theCSI-RS-ResourceConfigMobility in the associated measObject;

2>	if nrofCSI-RS-ResourcesToAverage in the associated measObject is not configured; or

2>	if absThreshCSI-RS-Consolidation in the associated measObject is not configured; or

2>	if the highest beam measurement quantity value is below or equal to absThreshCSI-RS-Consolidation:

3>	derive each cell measurement quantity based on applicable CSI-RS resources for the cell as the highest beam measurement quantity value, where each beam measurement quantity is described in TS 38.215 [9];

2>	else:

3>	derive each cell measurement quantity based on CSI-RS as the linear power scale average of the highest beam measurement quantity values above absThreshCSI-RS-Consolidation where the total number of averaged beams shall not exceed nrofCSI-RS-ResourcesToAverage;

2>	apply layer 3 cell filtering as described in 5.5.3.2.

#### 5.5.3.3a	Derivation of layer 3 beam filtered measurement

The UE shall:

1>	for each layer 3 beam filtered measurement quantity to be derived based on SS/PBCH block;

2>	derive each configured beam measurement quantity based on SS/PBCH block as described in TS 38.215[9], and apply layer 3 beam filtering as described in 5.5.3.2;

1>	for each layer 3 beam filtered measurement quantity to be derived based on CSI-RS;

2>	derive each configured beam measurement quantity based on CSI-RS as described in TS 38.215 [9], and apply layer 3 beam filtering as described in 5.5.3.2.

#### 5.5.3.4	Derivation of L2 U2N Relay UE measurement results

A UE may be configured by network to derive NR sidelink measurement results of serving L2 U2N Relay UE or candidate L2 U2N Relay UEs associated to the measurement objects configured in the measObjectRelay.

The UE shall:

1>	for each L2 U2N Relay UE measurement quantity to be derived:

2>	derive the corresponding measurement quantity based on DMRS as described in TS 38.215 [9];

2>	apply layer 3 filtering as described in 5.5.3.2;

### 5.5.4	Measurement report triggering

#### 5.5.4.1	General

If AS security has been activated successfully, the UE shall:

1>	for each measId included in the measIdList within VarMeasConfig:

2>	if the corresponding reportConfig includes a reportType set to eventTriggered or periodical:

3>	if the corresponding measObject concerns NR:

4>	if the corresponding reportConfig includes measRSSI-ReportConfig:

5>	consider the resource indicated by the rmtc-Config on the associated frequency to be applicable;

4>	if the eventA1 or eventA2 is configured in the corresponding reportConfig:

5>	consider only the serving cell to be applicable;

4>	if the eventA3 or eventA5 is configured in the corresponding reportConfig:

5>	if a serving cell is associated with a measObjectNR and neighbours are associated with another measObjectNR, consider any serving cell associated with the other measObjectNR to be a neighbouring cell as well;

4>	if the eventX2 is configured in the corresponding reportConfig:

5>	consider only the serving L2 U2N Relay UE to be applicable;

4>	if corresponding reportConfig includes reportType set to periodical; or

4>	for measurement events other than eventA1, eventA2, eventD1 or eventX2:

5>	if useAllowedCellList is set to true:

6>	consider any neighbouring cell detected based on parameters in the associated measObjectNR to be applicable when the concerned cell is included in the allowedCellsToAddModList defined within the VarMeasConfig for this measId;

5>	else:

6>	consider any neighbouring cell detected based on parameters in the associated measObjectNR to be applicable when the concerned cell is not included in the excludedCellsToAddModList defined within the VarMeasConfig for this measId;

3>	else if the corresponding measObject concerns E-UTRA:

4>	if eventB1 or eventB2 is configured in the corresponding reportConfig:

5>	consider a serving cell, if any, on the associated E-UTRA frequency as neighbour cell;

4>	consider any neighbouring cell detected on the associated frequency to be applicable when the concerned cell is not included in the excludedCellsToAddModListEUTRAN defined within the VarMeasConfig for this measId;

3>	else if the corresponding measObject concerns UTRA-FDD:

4>	if eventB1-UTRA-FDD or eventB2-UTRA-FDD is configured in the corresponding reportConfig; or

4>	if corresponding reportConfig includes reportType set to periodical:

5>	consider a neighbouring cell on the associated frequency to be applicable when the concerned cell is included in the cellsToAddModList defined within the VarMeasConfig for this measId;

3>	else if the corresponding measObject concerns L2 U2N Relay UE:

4>	if eventY1-Relay or eventY2-Relay is configured in the corresponding reportConfig; or

4>	if corresponding reportConfig includes reportType set to periodical:

5>	consider any L2 U2N Relay UE fulfilling upper layer criteria detected on the associated frequency to be applicable for this measId;

2>	else if the corresponding reportConfig includes a reportType set to reportCGI:

3>	consider the cell detected on the associated measObject which has a physical cell identity matching the value of the cellForWhichToReportCGI included in the corresponding reportConfig within the VarMeasConfig to be applicable;

2>	else if the corresponding reportConfig includes a reportType set to reportSFTD:

3>	if the corresponding measObject concerns NR:

4>	if the reportSFTD-Meas is set to true:

5>	consider the NR PSCell to be applicable;

4>	else if the reportSFTD-NeighMeas is included:

5>	if cellsForWhichToReportSFTD is configured in the corresponding reportConfig:

6>	consider any NR neighbouring cell detected on the associated measObjectNR which has a physical cell identity that is included in the cellsForWhichToReportSFTD to be applicable;

5>	else:

6>	consider up to 3 strongest NR neighbouring cells detected based on parameters in the associated measObjectNR to be applicable when the concerned cells are not included in the excludedCellsToAddModList defined within the VarMeasConfig for this measId;

3>	else if the corresponding measObject concerns E-UTRA:

4>	if the reportSFTD-Meas is set to true:

5>	consider the E-UTRA PSCell to be applicable;

2>	else if the corresponding reportConfig includes a reportType set to cli-Periodical or cli-EventTriggered:

3>	consider all CLI measurement resources included in the corresponding measObject to be applicable;

2>	else if the corresponding reportConfig includes a reportType set to rxTxPeriodical:

3>	consider all Rx-Tx time difference measurement resources included in the corresponding measObject to be applicable;

2>	if the corresponding reportConfig concerns the reporting for NR sidelink communication/discovery (i.e. reportConfigNR-SL):

3>	consider the transmission resource pools indicated by the tx-PoolMeasToAddModList defined within the VarMeasConfig for this measId to be applicable;

2>	if the reportType is set to eventTriggered and if the entry condition applicable for this event, i.e. the event corresponding with the eventId of the corresponding reportConfig within VarMeasConfig, is fulfilled for one or more applicable cells for all measurements after layer 3 filtering taken during timeToTrigger defined for this event within the VarMeasConfig, while the VarMeasReportList does not include a measurement reporting entry for this measId (a first cell triggers the event):

3>	include a measurement reporting entry within the VarMeasReportList for this measId;

3>	set the numberOfReportsSent defined within the VarMeasReportList for this measId to 0;

3>	include the concerned cell(s) in the cellsTriggeredList defined within the VarMeasReportList for this measId;

3>	if useT312 is set to true in reportConfig for this event:

4>	if T310 for the corresponding SpCell is running; and

4>	if T312 is not running for corresponding SpCell:

5>	start timer T312 for the corresponding SpCell with the value of T312 configured in the corresponding measObjectNR;

3>	initiate the measurement reporting procedure, as specified in 5.5.5;

2>	else if the reportType is set to eventTriggered and if the entry condition applicable for this event, i.e. the event corresponding with the eventId of the corresponding reportConfig within VarMeasConfig, is fulfilled for one or more applicable cells not included in the cellsTriggeredList for all measurements after layer 3 filtering taken during timeToTrigger defined for this event within the VarMeasConfig (a subsequent cell triggers the event):

3>	set the numberOfReportsSent defined within the VarMeasReportList for this measId to 0;

3>	include the concerned cell(s) in the cellsTriggeredList defined within the VarMeasReportList for this measId;

3>	if useT312 is set to true in reportConfig for this event:

4>	if T310 for the corresponding SpCell is running; and

4>	if T312 is not running for corresponding SpCell:

5>	start timer T312 for the corresponding SpCell with the value of T312 configured in the corresponding measObjectNR;

3>	initiate the measurement reporting procedure, as specified in 5.5.5;

2>	if the reportType is set to eventTriggered and if the leaving condition applicable for this event is fulfilled for one or more of the cells included in the cellsTriggeredList defined within the VarMeasReportList for this measId for all measurements after layer 3 filtering taken during timeToTrigger defined within the VarMeasConfig for this event:

3>	remove the concerned cell(s) in the cellsTriggeredList defined within the VarMeasReportList for this measId;

3>	if reportOnLeave is set to true for the corresponding reporting configuration:

4>	initiate the measurement reporting procedure, as specified in 5.5.5;

3>	if the cellsTriggeredList defined within the VarMeasReportList for this measId is empty:

4>	remove the measurement reporting entry within the VarMeasReportList for this measId;

4>	stop the periodical reporting timer for this measId, if running;

2>	if the reportType is set to eventTriggered and if the entry condition applicable for this event, i.e. the event corresponding with the eventId of the corresponding reportConfig within VarMeasConfig, is fulfilled for one or more applicable L2 U2N Relay UEs for all measurements after layer 3 filtering taken during timeToTrigger defined for this event within the VarMeasConfig, while the VarMeasReportList does not include a measurement reporting entry for this measId (a first L2 U2N Relay UE triggers the event):

3>	include a measurement reporting entry within the VarMeasReportList for this measId;

3>	set the numberOfReportsSent defined within the VarMeasReportList for this measId to 0;

3>	include the concerned L2 U2N Relay UE(s) in the relaysTriggeredList defined within the VarMeasReportList for this measId;

3>	initiate the measurement reporting procedure, as specified in 5.5.5;

2>	else if the reportType is set to eventTriggered and if the entry condition applicable for this event, i.e. the event corresponding with the eventId of the corresponding reportConfig within VarMeasConfig, is fulfilled for one or more applicable L2 U2N Relay UEs not included in the relaysTriggeredList for all measurements after layer 3 filtering taken during timeToTrigger defined for this event within the VarMeasConfig (a subsequent L2 U2N Relay UE triggers the event):

3>	set the numberOfReportsSent defined within the VarMeasReportList for this measId to 0;

3>	include the concerned L2 U2N Relay UE(s) in the relaysTriggeredList defined within the VarMeasReportList for this measId;

3>	initiate the measurement reporting procedure, as specified in 5.5.5;

2>	else if the reportType is set to eventTriggered and if the leaving condition applicable for this event is fulfilled for one or more of the L2 U2N Relay UEs included in the relaysTriggeredList defined within the VarMeasReportList for this measId for all measurements after layer 3 filtering taken during timeToTrigger defined within the VarMeasConfig for this event:

3>	remove the concerned L2 U2N Relay UE(s) in the relaysTriggeredList defined within the VarMeasReportList for this measId;

3>	if reportOnLeave is set to true for the corresponding reporting configuration:

4>	initiate the measurement reporting procedure, as specified in 5.5.5;

3>	if the relaysTriggeredList defined within the VarMeasReportList for this measId is empty:

4>	remove the measurement reporting entry within the VarMeasReportList for this measId;

4>	stop the periodical reporting timer for this measId, if running;

2>	else if the reportType is set to eventTriggered and if the entry condition applicable for this event, i.e. the event corresponding with the eventId of the corresponding reportConfig within VarMeasConfig, is fulfilled for one or more applicable transmission resource pools for all measurements taken during timeToTrigger defined for this event within the VarMeasConfig, while the VarMeasReportList does not include an measurement reporting entry for this measId (a first transmission resource pool triggers the event):

3>	include a measurement reporting entry within the VarMeasReportList for this measId;

3>	set the numberOfReportsSent defined within the VarMeasReportList for this measId to 0;

3>	include the concerned transmission resource pool(s) in the poolsTriggeredList defined within the VarMeasReportList for this measId;

3>	initiate the measurement reporting procedure, as specified in 5.5.5;

2>	else if the reportType is set to eventTriggered and if the entry condition applicable for this event, i.e. the event corresponding with the eventId of the corresponding reportConfig within VarMeasConfig, is fulfilled for one or more applicable transmission resource pools not included in the poolsTriggeredList for all measurements taken during timeToTrigger defined for this event within the VarMeasConfig (a subsequent transmission resource pool triggers the event):

3>	set the numberOfReportsSent defined within the VarMeasReportList for this measId to 0;

3>	include the concerned transmission resource pool(s) in the poolsTriggeredList defined within the VarMeasReportList for this measId;

3>	initiate the measurement reporting procedure, as specified in 5.5.5;

2>	if the reportType is set to eventTriggered and if the leaving condition applicable for this event is fulfilled for one or more applicable transmission resource pools included in the poolsTriggeredList defined within the VarMeasReportList for this measId for all measurements taken during timeToTrigger defined within the VarMeasConfig for this event:

3>	remove the concerned transmission resource pool(s) in the poolsTriggeredList defined within the VarMeasReportList for this measId;

3>	if the poolsTriggeredList defined within the VarMeasReportList for this measId is empty:

4>	remove the measurement reporting entry within the VarMeasReportList for this measId;

4>	stop the periodical reporting timer for this measId, if running

2>	else if the reportType is set to eventTriggered and if the eventId is set to eventD1 and if the entering condition applicable for this event, i.e. the event corresponding with the eventId of the corresponding reportConfig within VarMeasConfig, is fulfilled during timeToTrigger defined for this event within the VarMeasConfig, while the VarMeasReportList does not include a measurement reporting entry for this measId:

3>	include a measurement reporting entry within the VarMeasReportList for this measId;

3>	set the numberOfReportsSent defined within the VarMeasReportList for this measId to 0;

3>	initiate the measurement reporting procedure, as specified in 5.5.5;

2>	else if the reportType is set to eventTriggered and if the eventId is set to eventD1 and if the leaving condition applicable for this event is fulfilled for the associated VarMeasReport within the VarMeasReportList for this measId during timeToTrigger defined within the VarMeasConfig for this event:

3>	if reportOnLeave is set to true for the corresponding reporting configuration:

4>	initiate the measurement reporting procedure, as specified in 5.5.5;

3>	remove the measurement reporting entry within the VarMeasReportList for this measId;

3>	stop the periodical reporting timer for this measId, if running;

NOTE 1:	Void.

2>	if reportType is set to periodical and if a (first) measurement result is available:

3>	include a measurement reporting entry within the VarMeasReportList for this measId;

3>	set the numberOfReportsSent defined within the VarMeasReportList for this measId to 0;

3>	if the corresponding reportConfig includes measRSSI-ReportConfig:

4>	initiate the measurement reporting procedure as specified in 5.5.5 immediately when RSSI sample values are reported by the physical layer after the first L1 measurement duration;

3>	else if the corresponding reportConfig includes the ul-DelayValueConfig:

4>	initiate the measurement reporting procedure, as specified in 5.5.5, immediately after a first measurement result is provided from lower layers of the associated DRB identity;

3>	else if the corresponding reportConfig includes the ul-ExcessDelayConfig:

4>	initiate the measurement reporting procedure, as specified in 5.5.5, immediately after a first measurement result is provided from lower layers of the associated DRB identity(ies) according to the configured threshold per DRB identity(ies);

3>	else if the reportAmount exceeds 1:

4>	initiate the measurement reporting procedure, as specified in 5.5.5, immediately after the quantity to be reported becomes available for the NR SpCell or for the serving L2 U2N Relay UE (if the UE is a L2 U2N Remote UE);

3>	else (i.e. the reportAmount is equal to 1):

4>	initiate the measurement reporting procedure, as specified in 5.5.5, immediately after the quantity to be reported becomes available for the NR SpCell and for the strongest cell among the applicable cells, or for the NR SpCell and for the strongest L2 U2N Relay UEs among the applicable L2 U2N Relay UEs; or initiate the measurement reporting procedure, as specified in 5.5.5, immediately after the quantity to be reported becomes available for the serving L2 U2N Relay UE and for the strongest cell among the applicable cells (if the UE is a L2 U2N Remote UE);

2>	if, in case the corresponding reportConfig concerns the reporting for NR sidelink communication/discovery, reportType is set to periodical and if a (first) measurement result is available:

3>	include a measurement reporting entry within the VarMeasReportList for this measId;

3>	set the numberOfReportsSent defined within the VarMeasReportList for this measId to 0;

3>	initiate the measurement reporting procedure, as specified in 5.5.5, immediately after the quantity to be reported becomes available for the NR SpCell and CBR measurement results become available;

2>	if the reportType is set to cli-EventTriggered and if the entry condition applicable for this event, i.e. the event corresponding with the eventId of the corresponding reportConfig within VarMeasConfig, is fulfilled for one or more applicable CLI measurement resources for all measurements after layer 3 filtering taken during timeToTrigger defined for this event within the VarMeasConfig, while the VarMeasReportList does not include a measurement reporting entry for this measId (a first CLI measurement resource triggers the event):

3>	include a measurement reporting entry within the VarMeasReportList for this measId;

3>	set the numberOfReportsSent defined within the VarMeasReportList for this measId to 0;

3>	include the concerned CLI measurement resource(s) in the cli-TriggeredList defined within the VarMeasReportList for this measId;

3>	initiate the measurement reporting procedure, as specified in 5.5.5;

2>	else if the reportType is set to cli-EventTriggered and if the entry condition applicable for this event, i.e. the event corresponding with the eventId of the corresponding reportConfig within VarMeasConfig, is fulfilled for one or more CLI measurement resources not included in the cli-TriggeredList for all measurements after layer 3 filtering taken during timeToTrigger defined for this event within the VarMeasConfig (a subsequent CLI measurement resource triggers the event):

3>	set the numberOfReportsSent defined within the VarMeasReportList for this measId to 0;

3>	include the concerned CLI measurement resource(s) in the cli-TriggeredList defined within the VarMeasReportList for this measId;

3>	initiate the measurement reporting procedure, as specified in 5.5.5;

2>	if the reportType is set to cli-EventTriggered and if the leaving condition applicable for this event is fulfilled for one or more of the CLI measurement resources included in the cli-TriggeredList defined within the VarMeasReportList for this measId for all measurements after layer 3 filtering taken during timeToTrigger defined within the VarMeasConfig for this event:

3>	remove the concerned CLI measurement resource(s) in the cli-TriggeredList defined within the VarMeasReportList for this measId;

3>	if reportOnLeave is set to true for the corresponding reporting configuration:

4>	initiate the measurement reporting procedure, as specified in 5.5.5;

3>	if the cli-TriggeredList defined within the VarMeasReportList for this measId is empty:

4>	remove the measurement reporting entry within the VarMeasReportList for this measId;

4>	stop the periodical reporting timer for this measId, if running;

2>	if reportType is set to cli-Periodical and if a (first) measurement result is available:

3>	include a measurement reporting entry within the VarMeasReportList for this measId;

3>	set the numberOfReportsSent defined within the VarMeasReportList for this measId to 0;

3>	initiate the measurement reporting procedure, as specified in 5.5.5, immediately after the quantity to be reported becomes available for at least one CLI measurement resource;

2>	if reportType is set to rxTxPeriodical and if a (first) measurement result is available:

3>	include a measurement reporting entry within the VarMeasReportList for this measId;

3>	set the numberOfReportsSent defined within the VarMeasReportList for this measId to 0;

3>	initiate the measurement reporting procedure, as specified in 5.5.5;

2>	upon expiry of the periodical reporting timer for this measId:

3>	initiate the measurement reporting procedure, as specified in 5.5.5.

2>	if the corresponding reportConfig includes a reportType is set to reportSFTD:

3>	if the corresponding measObject concerns NR:

4>	if the drx-SFTD-NeighMeas is included:

5>	if the quantity to be reported becomes available for each requested pair of PCell and NR cell:

6>	stop timer T322;

6>	initiate the measurement reporting procedure, as specified in 5.5.5;

4>	else

5>	initiate the measurement reporting procedure, as specified in 5.5.5, immediately after the quantity to be reported becomes available for each requested pair of PCell and NR cell or the maximal measurement reporting delay as specified in TS 38.133 [14];

3>	else if the corresponding measObject concerns E-UTRA:

4>	initiate the measurement reporting procedure, as specified in 5.5.5, immediately after the quantity to be reported becomes available for the pair of PCell and E-UTRA PSCell or the maximal measurement reporting delay as specified in TS 38.133 [14];

2>	if reportType is set to reportCGI:

3>	if the UE acquired the SIB1 or SystemInformationBlockType1 for the requested cell; or

3>	if the UE detects that the requested NR cell is not transmitting SIB1 (see TS 38.213 [13], clause 13):

4>	stop timer T321;

4>	include a measurement reporting entry within the VarMeasReportList for this measId;

4>	set the numberOfReportsSent defined within the VarMeasReportList for this measId to 0;

4>	initiate the measurement reporting procedure, as specified in 5.5.5;

2>	upon the expiry of T321 for this measId:

3>	include a measurement reporting entry within the VarMeasReportList for this measId;

3>	set the numberOfReportsSent defined within the VarMeasReportList for this measId to 0;

3>	initiate the measurement reporting procedure, as specified in 5.5.5.

2>	upon the expiry of T322 for this measId:

3>	initiate the measurement reporting procedure, as specified in 5.5.5.

#### 5.5.4.2	Event A1 (Serving becomes better than threshold)

The UE shall:

1>	consider the entering condition for this event to be satisfied when condition A1-1, as specified below, is fulfilled;

1>	consider the leaving condition for this event to be satisfied when condition A1-2, as specified below, is fulfilled;

1>	for this measurement, consider the NR serving cell corresponding to the associated measObjectNR associated with this event.

Inequality A1-1 (Entering condition)

Ms – Hys > Thresh

Inequality A1-2 (Leaving condition)

Ms + Hys < Thresh

The variables in the formula are defined as follows:

Ms is the measurement result of the serving cell, not taking into account any offsets.

Hys is the hysteresis parameter for this event (i.e. hysteresis as defined within reportConfigNR for this event).

Thresh is the threshold parameter for this event (i.e. a1-Threshold as defined within reportConfigNR for this event).

Ms is expressed in dBm in case of RSRP, or in dB in case of RSRQ and RS-SINR.

Hys is expressed in dB.

Thresh is expressed in the same unit as Ms.

#### 5.5.4.3	Event A2 (Serving becomes worse than threshold)

The UE shall:

1>	consider the entering condition for this event to be satisfied when condition A2-1, as specified below, is fulfilled;

1>	consider the leaving condition for this event to be satisfied when condition A2-2, as specified below, is fulfilled;

1>	for this measurement, consider the serving cell indicated by the measObjectNR associated to this event.

Inequality A2-1 (Entering condition)

Ms + Hys < Thresh

Inequality A2-2 (Leaving condition)

Ms – Hys > Thresh

The variables in the formula are defined as follows:

Ms is the measurement result of the serving cell, not taking into account any offsets.

Hys is the hysteresis parameter for this event (i.e. hysteresis as defined within reportConfigNR for this event).

Thresh is the threshold parameter for this event (i.e. a2-Threshold as defined within reportConfigNR for this event).

Ms is expressed in dBm in case of RSRP, or in dB in case of RSRQ and RS-SINR.

Hys is expressed in dB.

Thresh is expressed in the same unit as Ms.

#### 5.5.4.4	Event A3 (Neighbour becomes offset better than SpCell)

The UE shall:

1>	consider the entering condition for this event to be satisfied when condition A3-1, as specified below, is fulfilled;

1>	consider the leaving condition for this event to be satisfied when condition A3-2, as specified below, is fulfilled;

1>	use the SpCell for Mp, Ofp and Ocp.

NOTE 1:	The cell(s) that triggers the event has reference signals indicated in the measObjectNR associated to this event which may be different from the NR SpCell measObjectNR.

Inequality A3-1 (Entering condition)

Mn + Ofn + Ocn – Hys > Mp + Ofp + Ocp + Off

Inequality A3-2 (Leaving condition)

Mn + Ofn + Ocn + Hys < Mp + Ofp + Ocp + Off

The variables in the formula are defined as follows:

Mn is the measurement result of the neighbouring cell, not taking into account any offsets.

Ofn is the measurement object specific offset of the reference signal of the neighbour cell (i.e. offsetMO as defined within measObjectNR corresponding to the neighbour cell).

Ocn is the cell specific offset of the neighbour cell (i.e. cellIndividualOffset as defined within measObjectNR corresponding to the frequency of the neighbour cell), and set to zero if not configured for the neighbour cell.

Mp is the measurement result of the SpCell, not taking into account any offsets.

Ofp is the measurement object specific offset of the SpCell (i.e. offsetMO as defined within measObjectNR corresponding to the SpCell).

Ocp is the cell specific offset of the SpCell (i.e. cellIndividualOffset as defined within measObjectNR corresponding to the SpCell), and is set to zero if not configured for the SpCell.

Hys is the hysteresis parameter for this event (i.e. hysteresis as defined within reportConfigNR for this event).

Off is the offset parameter for this event (i.e. a3-Offset as defined within reportConfigNR for this event).

Mn, Mp are expressed in dBm in case of RSRP, or in dB in case of RSRQ and RS-SINR.

Ofn, Ocn, Ofp, Ocp, Hys, Off are expressed in dB.

NOTE 2:	The definition of Event A3 also applies to CondEvent A3.

#### 5.5.4.5	Event A4 (Neighbour becomes better than threshold)

The UE shall:

1>	consider the entering condition for this event to be satisfied when condition A4-1, as specified below, is fulfilled;

1>	consider the leaving condition for this event to be satisfied when condition A4-2, as specified below, is fulfilled.

Inequality A4-1 (Entering condition)

Mn + Ofn + Ocn – Hys > Thresh

Inequality A4-2 (Leaving condition)

Mn + Ofn + Ocn + Hys < Thresh

The variables in the formula are defined as follows:

Mn is the measurement result of the neighbouring cell, not taking into account any offsets.

Ofn is the measurement object specific offset of the neighbour cell (i.e. offsetMO as defined within measObjectNR corresponding to the neighbour cell).

Ocn is the measurement object specific offset of the neighbour cell (i.e. cellIndividualOffset as defined within measObjectNR corresponding to the neighbour cell), and set to zero if not configured for the neighbour cell.

Hys is the hysteresis parameter for this event (i.e. hysteresis as defined within reportConfigNR for this event).

Thresh is the threshold parameter for this event (i.e. a4-Threshold as defined within reportConfigNR for this event).

Mn is expressed in dBm in case of RSRP, or in dB in case of RSRQ and RS-SINR.

Ofn, Ocn, Hys are expressed in dB.

Thresh is expressed in the same unit as Mn.

NOTE:	The definition of Event A4 also applies to CondEvent A4.

#### 5.5.4.6	Event A5 (SpCell becomes worse than threshold1 and neighbour becomes better than threshold2)

The UE shall:

1>	consider the entering condition for this event to be satisfied when both condition A5-1 and condition A5-2, as specified below, are fulfilled;

1>	consider the leaving condition for this event to be satisfied when condition A5-3 or condition A5-4, i.e. at least one of the two, as specified below, is fulfilled;

1>	use the SpCell for Mp.

NOTE 1:	The parameters of the reference signal(s) of the cell(s) that triggers the event are indicated in the measObjectNR associated to the event which may be different from the measObjectNR of the NR SpCell.

Inequality A5-1 (Entering condition 1)

Mp + Hys < Thresh1

Inequality A5-2 (Entering condition 2)

Mn + Ofn + Ocn – Hys > Thresh2

Inequality A5-3 (Leaving condition 1)

Mp – Hys > Thresh1

Inequality A5-4 (Leaving condition 2)

Mn + Ofn + Ocn + Hys < Thresh2

The variables in the formula are defined as follows:

Mp is the measurement result of the NR SpCell, not taking into account any offsets.

Mn is the measurement result of the neighbouring cell, not taking into account any offsets.

Ofn is the measurement object specific offset of the neighbour cell (i.e. offsetMO as defined within measObjectNR corresponding to the neighbour cell).

Ocn is the cell specific offset of the neighbour cell (i.e. cellIndividualOffset as defined within measObjectNR corresponding to the neighbour cell), and set to zero if not configured for the neighbour cell.

Hys is the hysteresis parameter for this event (i.e. hysteresis as defined within reportConfigNR for this event).

Thresh1 is the threshold parameter for this event (i.e. a5-Threshold1 as defined within reportConfigNR for this event).

Thresh2 is the threshold parameter for this event (i.e. a5-Threshold2 as defined within reportConfigNR for this event).

Mn, Mp are expressed in dBm in case of RSRP, or in dB in case of RSRQ and RS-SINR.

Ofn, Ocn, Hys are expressed in dB.

Thresh1is expressed in the same unit as Mp.

Thresh2 is expressed in the same unit as Mn.

NOTE 2:	The definition of Event A5 also applies to CondEvent A5.

#### 5.5.4.7	Event A6 (Neighbour becomes offset better than SCell)

The UE shall:

1>	consider the entering condition for this event to be satisfied when condition A6-1, as specified below, is fulfilled;

1>	consider the leaving condition for this event to be satisfied when condition A6-2, as specified below, is fulfilled;

1>	for this measurement, consider the (secondary) cell corresponding to the measObjectNR associated to this event to be the serving cell.

NOTE:	The reference signal(s) of the neighbour(s) and the reference signal(s) of the SCell are both indicated in the associated measObjectNR.

Inequality A6-1 (Entering condition)

Mn + Ocn – Hys > Ms + Ocs + Off

Inequality A6-2 (Leaving condition)

Mn + Ocn + Hys < Ms + Ocs + Off

The variables in the formula are defined as follows:

Mn is the measurement result of the neighbouring cell, not taking into account any offsets.

Ocn is the cell specific offset of the neighbour cell (i.e. cellIndividualOffset as defined within the associated measObjectNR), and set to zero if not configured for the neighbour cell.

Ms is the measurement result of the serving cell, not taking into account any offsets.

Ocs is the cell specific offset of the serving cell (i.e. cellIndividualOffset as defined within the associated measObjectNR), and is set to zero if not configured for the serving cell.

Hys is the hysteresis parameter for this event (i.e. hysteresis as defined within reportConfigNR for this event).

Off is the offset parameter for this event (i.e. a6-Offset as defined within reportConfigNR for this event).

Mn, Ms are expressed in dBm in case of RSRP, or in dB in case of RSRQ and RS-SINR.

Ocn, Ocs, Hys, Off are expressed in dB.

#### 5.5.4.8	Event B1 (Inter RAT neighbour becomes better than threshold)

The UE shall:

1>	consider the entering condition for this event to be satisfied when condition B1-1, as specified below, is fulfilled;

1>	consider the leaving condition for this event to be satisfied when condition B1-2, as specified below, is fulfilled.

Inequality B1-1 (Entering condition)

Mn + Ofn + Ocn – Hys > Thresh

Inequality B1-2 (Leaving condition)

Mn + Ofn + Ocn + Hys < Thresh

The variables in the formula are defined as follows:

Mn is the measurement result of the inter-RAT neighbour cell, not taking into account any offsets.

Ofn is the measurement object specific offset of the frequency of the inter-RAT neighbour cell (i.e. eutra-Q-OffsetRange as defined within the measObjectEUTRA corresponding to the frequency of the neighbour inter-RAT cell, utra-FDD-Q-OffsetRange as defined within the measObjectUTRA-FDD corresponding to the frequency of the neighbour inter-RAT cell).

Ocn is the cell specific offset of the inter-RAT neighbour cell (i.e. cellIndividualOffset as defined within the measObjectEUTRA corresponding to the neighbour inter-RAT cell), and set to zero if not configured for the neighbour cell.

Hys is the hysteresis parameter for this event (i.e. hysteresis as defined within reportConfigInterRAT for this event).

Thresh is the threshold parameter for this event (i.e. b1-ThresholdEUTRA as defined within reportConfigInterRAT for this event, b1-ThresholdUTRA-FDD as defined for UTRA-FDD within reportConfigInterRAT for this event).

Mn is expressed in dBm or in dB, depending on the measurement quantity of the inter-RAT neighbour cell.

Ofn, Ocn, Hys are expressed in dB.

Thresh is expressed in the same unit as Mn.

#### 5.5.4.9	Event B2 (PCell becomes worse than threshold1 and inter RAT neighbour becomes better than threshold2)

The UE shall:

1>	consider the entering condition for this event to be satisfied when both condition B2-1 and condition B2-2, as specified below, are fulfilled;

1>	consider the leaving condition for this event to be satisfied when condition B2-3 or condition B2-4, i.e. at least one of the two, as specified below, is fulfilled;

Inequality B2-1 (Entering condition 1)

Mp + Hys < Thresh1

Inequality B2-2 (Entering condition 2)

Mn + Ofn + Ocn – Hys > Thresh2

Inequality B2-3 (Leaving condition 1)

Mp – Hys > Thresh1

Inequality B2-4 (Leaving condition 2)

Mn + Ofn + Ocn + Hys < Thresh2

The variables in the formula are defined as follows:

Mp is the measurement result of the PCell, not taking into account any offsets.

Mn is the measurement result of the inter-RAT neighbour cell, not taking into account any offsets.

Ofn is the measurement object specific offset of the frequency of the inter-RAT neighbour cell (i.e. eutra-Q-OffsetRange as defined within the measObjectEUTRA corresponding to the frequency of the inter-RAT neighbour cell, utra-FDD-Q-OffsetRange as defined within the measObjectUTRA-FDD corresponding to the frequency of the neighbour inter-RAT cell).

Ocn is the cell specific offset of the inter-RAT neighbour cell (i.e. cellIndividualOffset as defined within the measObjectEUTRA corresponding to the neighbour inter-RAT cell), and set to zero if not configured for the neighbour cell.

Hys is the hysteresis parameter for this event (i.e. hysteresis as defined within reportConfigInterRAT for this event).

Thresh1 is the threshold parameter for this event (i.e. b2-Threshold1 as defined within reportConfigInterRAT for this event).

Thresh2 is the threshold parameter for this event (i.e. b2-Threshold2EUTRA as defined within reportConfigInterRAT for this event, b2-Threshold2UTRA-FDD as defined for UTRA-FDD within reportConfigInterRAT for this event).

Mp is expressed in dBm in case of RSRP, or in dB in case of RSRQ and SINR.

Mn is expressed in dBm or dB, depending on the measurement quantity of the inter-RAT neighbour cell.

Ofn, Ocn, Hys are expressed in dB.

Thresh1 is expressed in the same unit as Mp.

Thresh2 is expressed in the same unit as Mn.

#### 5.5.4.10	Event I1 (Interference becomes higher than threshold)

The UE shall:

1>	consider the entering condition for this event to be satisfied when condition I1-1, as specified below, is fulfilled;

1>	consider the leaving condition for this event to be satisfied when condition I1-2, as specified below, is fulfilled.

Inequality I1-1 (Entering condition)

Mi – Hys > Thresh

Inequality I1-2 (Leaving condition)

Mi+ Hys < Thresh

The variables in the formula are defined as follows:

Mi is the measurement result of the interference, not taking into account any offsets.

Hys is the hysteresis parameter for this event (i.e. hysteresis as defined within reportConfigNR for this event).

Thresh is the threshold parameter for this event (i.e. i1-Threshold as defined within reportConfigNR for this event).

Mi, Thresh are expressed in dBm.

Hys is expressed in dB.

#### 5.5.4.11	Event C1 (The NR sidelink channel busy ratio is above a threshold)

The UE shall:

1>	consider the entering condition for this event to be satisfied when condition C1-1, as specified below, is fulfilled;

1>	consider the leaving condition for this event to be satisfied when condition C1-2, as specified below, is fulfilled;

Inequality C1-1 (Entering condition)

Inequality C1-2 (Leaving condition)

The variables in the formula are defined as follows:

Ms is the measurement result of channel busy ratio of the transmission resource pool, not taking into account any offsets.

Hys is the hysteresis parameter for this event (i.e. hysteresis as defined within reportConfigNR-SL for this event).

Thresh is the threshold parameter for this event (i.e. c1-Threshold as defined within reportConfigNR-SL for this event).

Ms is expressed in decimal from 0 to 1 in steps of 0.01.

Hys is expressed is in the same unit as Ms.

Thresh is expressed in the same unit as Ms.

#### 5.5.4.12	Event C2 (The NR sidelink channel busy ratio is below a threshold)

The UE shall:

1>	consider the entering condition for this event to be satisfied when condition C2-1, as specified below, is fulfilled;

1>	consider the leaving condition for this event to be satisfied when condition C2-2, as specified below, is fulfilled;

Inequality C2-1 (Entering condition)

Inequality C2-2 (Leaving condition)

The variables in the formula are defined as follows:

Ms is the measurement result of channel busy ratio of the transmission resource pool, not taking into account any offsets.

Hys is the hysteresis parameter for this event (i.e. hysteresis as defined within reportConfigNR-SL for this event).

Thresh is the threshold parameter for this event (i.e. c2-Threshold as defined within reportConfigNR-SL for this event).

Ms is expressed in decimal from 0 to 1 in steps of 0.01.

Hys is expressed is in the same unit as Ms.

Thresh is expressed in the same unit as Ms.

#### 5.5.4.13	Void

#### 5.5.4.14	Void

#### 5.5.4.15	Event D1 (Distance between UE and referenceLocation1 is above threshold1 and distance between UE and referenceLocation2 is below threshold2)

The UE shall:

1>	consider the entering condition for this event to be satisfied when both condition D1-1 and condition D1-2, as specified below, are fulfilled;

1>	consider the leaving condition for this event to be satisfied when condition D1-3 or condition D1-4, i.e. at least one of the two, as specified below, are fulfilled;

Inequality D1-1 (Entering condition 1)

Ml1-Hys>Thresh1

Inequality D1-2 (Entering condition 2)

Ml2+Hys<Thresh2

Inequality D1-3 (Leaving condition 1)

Ml1+Hys<Thresh1

Inequality D1-4 (Leaving condition 2)

Ml2-Hys>Thresh2

The variables in the formula are defined as follows:

Ml1 is the UE location, represented by the distance between UE and a reference location parameter for this event (i.e. referenceLocation1 as defined within reportConfigNR for this event), not taking into account any offsets.

Ml2 is the UE location, represented by the distance between UE and a reference location parameter for this event (i.e. referenceLocation2 as defined within reportConfigNR for this event), not taking into account any offsets.

Hys is the hysteresis parameter for this event (i.e. hysteresis as defined within reportConfigNR for this event).

Thresh1 is the threshold for this event defined as a distance, configured with parameter distanceThreshFromReference1, from a reference location configured with parameter referenceLocation1 within reportConfigNR for this event.

Thresh2 is the threshold for this event defined as a distance, configured with parameter distanceThreshFromReference2, from a reference location configured with parameter referenceLocation2 within reportConfigNR for this event.

Ml1 is expressed in meters.

Ml2 is expressed in the same unit as Ml1.

Hys is expressed in the same unit as Ml1.

Thresh1 is expressed in the same unit as Ml1.

Thresh2 is expressed in the same unit as Ml1.

NOTE:	The definition of Event D1 also applies to CondEvent D1.

#### 5.5.4.16	CondEvent T1 (Time measured at UE is within a duration from threshold)

The UE shall:

1>	consider the entering condition for this event to be satisfied when condition T1-1, as specified below, is fulfilled;

1>	consider the leaving condition for this event to be satisfied when condition T1-2, as specified below, is fulfilled;

Inequality T1-1 (Entering condition)

Mt>Thresh1

Inequality T1-2 (Leaving condition)

Mt>Thresh1+Duration

The variables in the formula are defined as follows:

Mt is the time measured at UE.

Thresh1 is the threshold parameter for this event (i.e. t1-Threshold as defined within reportConfigNR for this event).

Duration is the duration parameter for this event (i.e. duration as defined within reportConfigNR for this event).

Mt is expressed in ms.

Thresh1 is expressed in the same unit as Mt.

Duration is expressed in the same unit as Mt.

#### 5.5.4.17	Event X1 (Serving L2 U2N Relay UE becomes worse than threshold1 and NR Cell becomes better than threshold2)

The UE shall:

1>	consider the entering condition for this event to be satisfied when both condition X1-1 and condition X1-2, as specified below, are fulfilled;

1>	consider the leaving condition for this event to be satisfied when condition X1-3 or condition X1-4, i.e. at least one of the two, as specified below, is fulfilled;

Inequality X1-1 (Entering condition 1)

Mr + Hys < Thresh1

Inequality X1-2 (Entering condition 2)

Mn + Ofn + Ocn – Hys > Thresh2

Inequality X1-3 (Leaving condition 1)

Mr – Hys > Thresh1

Inequality X1-4 (Leaving condition 2)

Mn + Ofn + Ocn + Hys < Thresh2

The variables in the formula are defined as follows:

Mr is the measurement result of the serving L2 U2N Relay UE, not taking into account any offsets.

Mn is the measurement result of the NR cell, not taking into account any offsets.

Ofn is the measurement object specific offset of the reference signal of the NR cell (i.e. offsetMO as defined within measObjectNR corresponding to the NR cell).

Ocn is the cell specific offset of the NR cell (i.e. cellIndividualOffset as defined within measObjectNR corresponding to the frequency of the NR cell), and set to zero if not configured for the cell.

Hys is the hysteresis parameter for this event.

Thresh1 is the threshold parameter for this event (i.e. x1-Threshold1-Relay as defined within reportConfigNR for this event).

Thresh2 is the threshold parameter for this event (i.e. x1-Threshold2 as defined within reportConfigNR for this event).

Mr is expressed in dBm.

Mn is expressed in dBm in case of RSRP, or in dB in case of RSRQ and RS-SINR.

Ofn, Ocn, Hys are expressed in dB.

Thresh1 is expressed in the same unit as Mr.

Thresh2 is expressed in the same unit as Mn.

#### 5.5.4.18	Event X2 (Serving L2 U2N Relay UE becomes worse than threshold)

The UE shall:

1>	consider the entering condition for this event to be satisfied when condition X2-1, as specified below, is fulfilled;

1>	consider the leaving condition for this event to be satisfied when condition X2-2, as specified below, is fulfilled;

Inequality X2-1 (Entering condition)

Mr + Hys < Thresh

Inequality X2-2 (Leaving condition)

Mr – Hys > Thresh

The variables in the formula are defined as follows:

Mr is the measurement result of the serving L2 U2N Relay UE, not taking into account any offsets.

Hys is the hysteresis parameter for this event.

Thresh is the threshold parameter for this event (i.e. x2-Threshold-Relay as defined within reportConfigNR for this event).

Mr is expressed in dBm.

Hys are expressed in dB.

Thresh is expressed in the same unit as Mr.

#### 5.5.4.19	Event Y1 (PCell becomes worse than threshold1 and candidate L2 U2N Relay UE becomes better than threshold2)

The UE shall:

1>	consider the entering condition for this event to be satisfied when both condition Y1-1 and condition Y1-2, as specified below, are fulfilled;

1>	consider the leaving condition for this event to be satisfied when condition Y1-3 or condition Y1-4, i.e. at least one of the two, as specified below, is fulfilled;

Inequality Y1-1 (Entering condition 1)

Mp + Hys < Thresh1

Inequality Y1-2 (Entering condition 2)

Mr– Hys > Thresh2

Inequality Y1-3 (Leaving condition 1)

Mp – Hys > Thresh1

Inequality Y1-4 (Leaving condition 2)

Mr + Hys < Thresh2

The variables in the formula are defined as follows:

Mp is the measurement result of the PCell, not taking into account any offsets.

Mr is the measurement result of the candidate L2 U2N Relay UE, not taking into account any offsets.

Hys is the hysteresis parameter for this event (i.e. hysteresis as defined within reportConfigInterRAT for this event).

Thresh1 is the threshold parameter for this event (i.e. y1-Threshold1 as defined within reportConfigInterRAT for this event).

Thresh2 is the threshold parameter for this event (i.e. y1-Threshold2-Relay as defined within reportConfigInterRAT for this event).

Mp is expressed in dBm in case of RSRP, or in dB in case of RSRQ and SINR.

Mr is expressed in dBm or dB, depending on the measurement quantity of candidate L2 U2N Relay UE.

Hys are expressed in dB.

Thresh1 is expressed in the same unit as Mp.

Thresh2 is expressed in the same unit as Mr.

#### 5.5.4.20	Event Y2 (Candidate L2 U2N Relay UE becomes better than threshold)

The UE shall:

1>	consider the entering condition for this event to be satisfied when condition Y2-1, as specified below, is fulfilled;

1>	consider the leaving condition for this event to be satisfied when condition Y2-2, as specified below, is fulfilled;

Inequality Y2-1 (Entering condition)

Mr– Hys > Thresh2

Inequality Y2-2 (Leaving condition)

Mr + Hys < Thresh2

The variables in the formula are defined as follows:

Mr is the measurement result of the candidate L2 U2N Relay UE, not taking into account any offsets.

Hys is the hysteresis parameter for this event (i.e. hysteresis as defined within reportConfigInterRAT for this event).

Thresh is the threshold parameter for this event (i.e. y2-Threshold-Relay as defined within reportConfigInterRAT for this event).

Mr is expressed in dBm or dB, depending on the measurement quantity of candidate L2 U2N Relay UE.

Hys are expressed in dB.

Thresh is expressed in the same unit as Mr.

### 5.5.5	Measurement reporting

#### 5.5.5.1	General

Figure 5.5.5.1-1: Measurement reporting

The purpose of this procedure is to transfer measurement results from the UE to the network. The UE shall initiate this procedure only after successful AS security activation.

For the measId for which the measurement reporting procedure was triggered, the UE shall set the measResults within the MeasurementReport message as follows:

1>	set the measId to the measurement identity that triggered the measurement reporting;

1>	for each serving cell configured with servingCellMO:

2>	if the reportConfig associated with the measId that triggered the measurement reporting includes rsType:

3>	if the serving cell measurements based on the rsType included in the reportConfig that triggered the measurement report are available:

4>	set the measResultServingCell within measResultServingMOList to include RSRP, RSRQ and the available SINR of the serving cell, derived based on the rsType included in the reportConfig that triggered the measurement report;

2>	else:

3>	if SSB based serving cell measurements are available:

4>	set the measResultServingCell within measResultServingMOList to include RSRP, RSRQ and the available SINR of the serving cell, derived based on SSB;

3>	else if CSI-RS based serving cell measurements are available:

4>	set the measResultServingCell within measResultServingMOList to include RSRP, RSRQ and the available SINR of the serving cell, derived based on CSI-RS;

1>	set the servCellId within measResultServingMOList to include each NR serving cell that is configured with servingCellMO, if any;

1>	if the reportConfig associated with the measId that triggered the measurement reporting includes reportQuantityRS-Indexes and maxNrofRS-IndexesToReport:

2>	for each serving cell configured with servingCellMO, include beam measurement information according to the associated reportConfig as described in 5.5.5.2;

1>	if the reportConfig associated with the measId that triggered the measurement reporting includes reportAddNeighMeas:

2>	for each measObjectId referenced in the measIdList which is also referenced with servingCellMO, other than the measObjectId corresponding with the measId that triggered the measurement reporting:

3>	if the measObjectNR indicated by the servingCellMO includes the RS resource configuration corresponding to the rsType indicated in the reportConfig:

4>	set the measResultBestNeighCell within measResultServingMOList to include the physCellId and the available measurement quantities based on the reportQuantityCell and rsType indicated in reportConfig of the non-serving cell corresponding to the concerned measObjectNR with the highest measured RSRP if RSRP measurement results are available for cells corresponding to this measObjectNR, otherwise with the highest measured RSRQ if RSRQ measurement results are available for cells corresponding to this measObjectNR, otherwise with the highest measured SINR;

4>	if the reportConfig associated with the measId that triggered the measurement reporting includes reportQuantityRS-Indexes and maxNrofRS-IndexesToReport:

5>	for each best non-serving cell included in the measurement report:

6>	include beam measurement information according to the associated reportConfig as described in 5.5.5.2;

1>	if the reportConfig associated with the measId that triggered the measurement reporting is set to eventTriggered and eventID is set to eventA3, or eventA4, or eventA5, or eventB1, or eventB2:

2>	if the UE is in NE-DC and the measurement configuration that triggered this measurement report is associated with the MCG:

3>	set the measResultServFreqListEUTRA-SCG to include an entry for each E-UTRA SCG serving frequency with the following:

4>	include carrierFreq of the E-UTRA serving frequency;

4>	set the measResultServingCell to include the available measurement quantities that the UE is configured to measure by the measurement configuration associated with the SCG;

4>	if reportConfig associated with the measId that triggered the measurement reporting includes reportAddNeighMeas:

5>	set the measResultServFreqListEUTRA-SCG to include within measResultBestNeighCell the quantities of the best non-serving cell, based on RSRP, on the concerned serving frequency;

1>	if reportConfig associated with the measId that triggered the measurement reporting is set to eventTriggered and eventID is set to eventA3, or eventA4, or eventA5:

2>	if the UE is in NR-DC and the measurement configuration that triggered this measurement report is associated with the MCG:

3>	set the measResultServFreqListNR-SCG to include for each NR SCG serving cell that is configured with servingCellMO, if any, the following:

4>	if the reportConfig associated with the measId that triggered the measurement reporting includes rsType:

5>	if the serving cell measurements based on the rsType included in the reportConfig that triggered the measurement report are available according to the measurement configuration associated with the SCG:

6>	set the measResultServingCell within measResultServFreqListNR-SCG to include RSRP, RSRQ and the available SINR of the serving cell, derived based on the rsType included in the reportConfig that triggered the measurement report;

4>	else:

5>	if SSB based serving cell measurements are available according to the measurement configuration associated with the SCG:

6>	set the measResultServingCell within measResultServFreqListNR-SCG to include RSRP, RSRQ and the available SINR of the serving cell, derived based on SSB;

5>	else if CSI-RS based serving cell measurements are available according to the measurement configuration associated with the SCG:

6>	set the measResultServingCell within measResultServFreqListNR-SCG to include RSRP, RSRQ and the available SINR of the serving cell, derived based on CSI-RS;

4>	if results for the serving cell derived based on SSB are included:

5>	include the ssbFrequency to the value indicated by ssbFrequency as included in the MeasObjectNR of the serving cell;

4>	if results for the serving cell derived based on CSI-RS are included:

5>	include the refFreqCSI-RS to the value indicated by refFreqCSI-RS as included in the MeasObjectNR of the serving cell;

4>	if the reportConfig associated with the measId that triggered the measurement reporting includes reportQuantityRS-Indexes and maxNrofRS-IndexesToReport:

5>	for each serving cell configured with servingCellMO, include beam measurement information according to the associated reportConfig as described in 5.5.5.2, where availability is considered according to the measurement configuration associated with the SCG;

4>	if reportConfig associated with the measId that triggered the measurement reporting includes reportAddNeighMeas:

5>	if the measObjectNR indicated by the servingCellMO includes the RS resource configuration corresponding to the rsType indicated in the reportConfig:

6>	set the measResultNeighCellListNR within measResultServFreqListNR-SCG to include one entry with the physCellId and the available measurement quantities based on the reportQuantityCell and rsType indicated in reportConfig of the non-serving cell corresponding to the concerned measObjectNR with the highest measured RSRP if RSRP measurement results are available for cells corresponding to this measObjectNR, otherwise with the highest measured RSRQ if RSRQ measurement results are available for cells corresponding to this measObjectNR, otherwise with the highest measured SINR, where availability is considered according to the measurement configuration associated with the SCG;

7>	if the reportConfig associated with the measId that triggered the measurement reporting includes reportQuantityRS-Indexes and maxNrofRS-IndexesToReport:

8>	for each best non-serving cell included in the measurement report:

9>	include beam measurement information according to the associated reportConfig as described in 5.5.5.2, where availability is considered according to the measurement configuration associated with the SCG;

1>	if the measRSSI-ReportConfig is configured within the corresponding reportConfig for this measId:

2>	set the rssi-Result to the linear average of sample value(s) provided by lower layers in the reportInterval;

2>	set the channelOccupancy to the rounded percentage of sample values which are beyond the channelOccupancyThreshold within all the sample values in the reportInterval;

1>	if the UE is acting as L2 U2N Remote UE:

2>	set the sl-MeasResultServingRelay in accordance with the following:

3>	set the cellIdentity to include the cellAccessRelatedInfo contained in the discovery message received from the serving L2 U2N Relay UE;

3>	set the sl-RelayUE-Identity to include the Source L2 ID of the serving L2 U2N Relay;

3>	set the sl-MeasResult to include the SL-RSRP of the serving L2 U2N Relay UE;

NOTE 1:	In case of no data transmission from L2 U2N Relay UE to L2 U2N Remote UE, it is left to UE implementation whether to use SL-RSRP or SD-RSRP when setting the sl-MeasResultServingRelay of the serving L2 U2N Relay UE.

1>	if there is at least one applicable neighbouring cell or candidate L2 U2N Relay UE to report:

2>	if the reportType is set to eventTriggered or periodical:

3>	if the measurement report concerns the candidate L2 U2N Relay UE:

4>	set the sl-MeasResultsCandRelay in measResultNeighCells to include the best candidate L2 U2N Relay UEs up to maxReportCells in accordance with the following:

5>	if the reportType is set to eventTriggered:

6>	include the L2 U2N Relay UEs included in the relaysTriggeredList as defined within the VarMeasReportList for this measId;

5>	else:

6>	include the applicable L2 U2N Relay UEs for which the new measurement results became available since the last periodical reporting or since the measurement was initiated or reset;

5>	for each L2 U2N Relay UE that is included in the sl-MeasResultsCandRelay:

6>	set the cellIdentity to include the cellAccessRelatedInfo contained in the discovery message received from the concerned L2 U2N Relay UE;

6>	set the sl-RelayUE-Identity to include the Source L2 ID of the concerned L2 U2N Relay UE;

6>	set the sl-MeasResult to include the SD-RSRP of the concerned L2 U2N Relay UE;

5>	for each included L2 U2N Relay UE, include the layer 3 filtered measured results in accordance with the reportConfig for this measId, ordered as follows:

6>	set the sl-MeasResult to include the quantity(ies) indicated in the reportQuantityRelay within the concerned reportConfigRelay in decreasing order of the sorting quantity, determined as specified in 5.5.5.3, i.e. the best L2 U2N Relay UE is included first;

3>	else:

4>	set the measResultNeighCells to include the best neighbouring cells up to maxReportCells in accordance with the following:

5>	if the reportType is set to eventTriggered and eventId is not set to eventD1:

6>	include the cells included in the cellsTriggeredList as defined within the VarMeasReportList for this measId;

5>	else:

6>	include the applicable cells for which the new measurement results became available since the last periodical reporting or since the measurement was initiated or reset;

5>	for each cell that is included in the measResultNeighCells, include the physCellId;

5>	if the reportType is set to eventTriggered or periodical:

6>	for each included cell, include the layer 3 filtered measured results in accordance with the reportConfig for this measId, ordered as follows:

7>	if the measObject associated with this measId concerns NR:

8>	if rsType in the associated reportConfig is set to ssb:

9>	set resultsSSB-Cell within the measResult to include the SS/PBCH block based quantity(ies) indicated in the reportQuantityCell within the concerned reportConfig, in decreasing order of the sorting quantity, determined as specified in 5.5.5.3, i.e. the best cell is included first;

9>	if reportQuantityRS-Indexes and maxNrofRS-IndexesToReport are configured, include beam measurement information as described in 5.5.5.2;

8>	else if rsType in the associated reportConfig is set to csi-rs:

9>	set resultsCSI-RS-Cell within the measResult to include the CSI-RS based quantity(ies) indicated in the reportQuantityCell within the concerned reportConfig, in decreasing order of the sorting quantity, determined as specified in 5.5.5.3, i.e. the best cell is included first;

9>	if reportQuantityRS-Indexes and maxNrofRS-IndexesToReport are configured, include beam measurement information as described in 5.5.5.2;

7>	if the measObject associated with this measId concerns E-UTRA:

8>	set the measResult to include the quantity(ies) indicated in the reportQuantity within the concerned reportConfigInterRAT in decreasing order of the sorting quantity, determined as specified in 5.5.5.3, i.e. the best cell is included first;

7>	if the measObject associated with this measId concerns UTRA-FDD and if ReportConfigInterRAT includes the reportQuantityUTRA-FDD:

8>	set the measResult to include the quantity(ies) indicated in the reportQuantityUTRA-FDD within the concerned reportConfigInterRAT in decreasing order of the sorting quantity, determined as specified in 5.5.5.3, i.e. the best cell is included first;

2>	else:

3>	if the cell indicated by cellForWhichToReportCGI is an NR cell:

4>	if plmn-IdentityInfoList of the cgi-Info for the concerned cell has been obtained:

5>	include the plmn-IdentityInfoList including plmn-IdentityList, trackingAreaCode (if available), trackingAreaList (if available), ranac (if available), cellIdentity and cellReservedForOperatorUse for each entry of the plmn-IdentityInfoList;

5>	include frequencyBandList if available;

5>	for each PLMN-IdentityInfo in plmn-IdentityInfoList:

6>	if the gNB-ID-Length is broadcast:

7>	include gNB-ID-Length;

4>	if nr-CGI-Reporting-NPN is supported by the UE and npn-IdentityInfoList of the cgi-Info for the concerned cell has been obtained:

5>	include the npn-IdentityInfoList including npn-IdentityList, trackingAreaCode, ranac (if available), cellIdentity and cellReservedForOperatorUse for each entry of the npn-IdentityInfoList;

5>	for each NPN-IdentityInfo in NPN-IdentityInfoList:

6>	if the gNB-ID-Length is broadcast:

7>	include gNB-ID-Length;

5>	include cellReservedForOtherUse if available;

4>	else if MIB indicates the SIB1 is not broadcast:

5>	include the noSIB1 including the ssb-SubcarrierOffset and pdcch-ConfigSIB1 obtained from MIB of the concerned cell;

3>	if the cell indicated by cellForWhichToReportCGI is an E-UTRA cell:

4>	if all mandatory fields of the cgi-Info-EPC for the concerned cell have been obtained:

5>	include in the cgi-Info-EPC the fields broadcasted in E-UTRA SystemInformationBlockType1 associated to EPC;

4>	if the UE is E-UTRA/5GC capable and all mandatory fields of the cgi-Info-5GC for the concerned cell have been obtained:

5>	include in the cgi-Info-5GC the fields broadcasted in E-UTRA SystemInformationBlockType1 associated to 5GC;

4>	if the mandatory present fields of the cgi-Info for the cell indicated by the cellForWhichToReportCGI in the associated measObject have been obtained:

5>	include the freqBandIndicator;

5>	if the cell broadcasts the multiBandInfoList, include the multiBandInfoList;

5>	if the cell broadcasts the freqBandIndicatorPriority, include the freqBandIndicatorPriority;

1>	if the corresponding measObject concerns NR:

2>	if the reportSFTD-Meas is set to true within the corresponding reportConfigNR for this measId:

3>	set the measResultSFTD-NR in accordance with the following:

4>	set sfn-OffsetResult and frameBoundaryOffsetResult to the measurement results provided by lower layers;

4>	if the reportRSRP is set to true;

5>	set rsrp-Result to the RSRP of the NR PSCell derived based on SSB;

2>	else if the reportSFTD-NeighMeas is included within the corresponding reportConfigNR for this measId:

3>	for each applicable cell which measurement results are available, include an entry in the measResultCellListSFTD-NR and set the contents as follows:

4>	set physCellId to the physical cell identity of the concerned NR neighbour cell.

4>	set sfn-OffsetResult and frameBoundaryOffsetResult to the measurement results provided by lower layers;

4>	if the reportRSRP is set to true:

5>	set rsrp-Result to the RSRP of the concerned cell derived based on SSB;

1>	else if the corresponding measObject concerns E-UTRA:

2>	if the reportSFTD-Meas is set to true within the corresponding reportConfigInterRAT for this measId:

3>	set the measResultSFTD-EUTRA in accordance with the following:

4>	set sfn-OffsetResult and frameBoundaryOffsetResult to the measurement results provided by lower layers;

4>	if the reportRSRP is set to true;

5>	set rsrpResult-EUTRA to the RSRP of the EUTRA PSCell;

1>	if average uplink PDCP delay values are available:

2>	set the ul-PDCP-DelayValueResultList to include the corresponding average uplink PDCP delay values;

1>	if PDCP excess delay measurements are available:

2>	set the ul-PDCP-ExcessDelayResultList to include the corresponding PDCP excess delay measurements;

1>	if the includeCommonLocationInfo is configured in the corresponding reportConfig for this measId and detailed location information that has not been reported is available, set the content of commonLocationInfo of the locationInfo as follows:

2>	include the locationTimestamp;

2>	include the locationCoordinate, if available;

2>	include the velocityEstimate, if available;

2>	include the locationError, if available;

2>	include the locationSource, if available;

2>	if available, include the gnss-TOD-msec,

1>	if the coarseLocationRequest is set to true in the corresponding reportConfig for this measId:

2>	include coarseLocationInfo, if available;

1>	if the includeWLAN-Meas is configured in the corresponding reportConfig for this measId, set the wlan-LocationInfo of the locationInfo in the measResults as follows:

2>	if available, include the LogMeasResultWLAN, in order of decreasing RSSI for WLAN APs;

1>	if the includeBT-Meas is configured in the corresponding reportConfig for this measId, set the BT-LocationInfo of the locationInfo in the measResults as follows:

2>	if available, include the LogMeasResultBT, in order of decreasing RSSI for Bluetooth beacons;

1>	if the includeSensor-Meas is configured in the corresponding reportConfig for this measId, set the sensor-LocationInfo of the locationInfo in the measResults as follows:

2>	if available, include the sensor-MeasurementInformation;

2>	if available, include the sensor-MotionInformation;

1>	if there is at least one applicable transmission resource pool for NR sidelink communication/discovery (for measResultsSL):

2>	set the measResultsListSL to include the CBR measurement results in accordance with the following:

3>	if the reportType is set to eventTriggered:

4>	include the transmission resource pools included in the poolsTriggeredList as defined within the VarMeasReportList for this measId;

3>	else:

4>	include the applicable transmission resource pools for which the new measurement results became available since the last periodical reporting or since the measurement was initiated or reset;

3>	if the corresponding measObject concerns NR sidelink communication/discovery, then for each transmission resource pool to be reported:

4>	set the sl-poolReportIdentity to the identity of this transmission resource pool;

4>	set the sl-CBR-ResultsNR to the CBR measurement results on PSSCH and PSCCH of this transmission resource pool provided by lower layers, if available;

NOTE 1:	Void.

1>	if there is at least one applicable CLI measurement resource to report:

2>	if the reportType is set to cli-EventTriggered or cli-Periodical:

3>	set the measResultCLI to include the most interfering SRS resources or most interfering CLI-RSSI resources up to maxReportCLI in accordance with the following:

4>	if the reportType is set to cli-EventTriggered:

5>	if trigger quantity is set to srs-RSRP i.e. i1-Threshold is set to srs-RSRP:

6>	include the SRS resource included in the cli-TriggeredList as defined within the VarMeasReportList for this measId;

5>	if trigger quantity is set to cli-RSSI i.e. i1-Threshold is set to cli-RSSI:

6>	include the CLI-RSSI resource included in the cli-TriggeredList as defined within the VarMeasReportList for this measId;

4>	else:

5>	if reportQuantityCLI is set to srs-rsrp:

6>	include the applicable SRS resources for which the new measurement results became available since the last periodical reporting or since the measurement was initiated or reset;

5>	else:

6>	include the applicable CLI-RSSI resources for which the new measurement results became available since the last periodical reporting or since the measurement was initiated or reset;

4>	for each SRS resource that is included in the measResultCLI:

5>	include the srs-ResourceId;

5>	set srs-RSRP-Result to include the layer 3 filtered measured results in decreasing order, i.e. the most interfering SRS resource is included first;

4>	for each CLI-RSSI resource that is included in the measResultCLI:

5>	include the rssi-ResourceId;

5>	set cli-RSSI-Result to include the layer 3 filtered measured results in decreasing order, i.e. the most interfering CLI-RSSI resource is included first;

1>	if there is at least one applicable UE Rx-Tx time difference measurement to report:

2>	set measResultRxTxTimeDiff to the latest measurement result;

1>	increment the numberOfReportsSent as defined within the VarMeasReportList for this measId by 1;

1>	stop the periodical reporting timer, if running;

1>	if the numberOfReportsSent as defined within the VarMeasReportList for this measId is less than the reportAmount as defined within the corresponding reportConfig for this measId:

2>	start the periodical reporting timer with the value of reportInterval as defined within the corresponding reportConfig for this measId;

1>	else:

2>	if the reportType is set to periodical or cli-Periodical or rxTxPeriodical:

3>	remove the entry within the VarMeasReportList for this measId;

3>	remove this measId from the measIdList within VarMeasConfig;

1>	if the measurement reporting was configured by a sl-ConfigDedicatedNR received within the RRCConnectionReconfiguration:

2>	submit the MeasurementReport message to lower layers for transmission via SRB1, embedded in E-UTRA RRC message ULInformationTransferIRAT as specified TS 36.331 [10], clause 5.6.28;

1>	else if the UE is in (NG)EN-DC:

2>	if SRB3 is configured and the SCG is not deactivated:

3>	submit the MeasurementReport message via SRB3 to lower layers for transmission, upon which the procedure ends;

2>	else:

3>	submit the MeasurementReport message via E-UTRA embedded in E-UTRA RRC message ULInformationTransferMRDC as specified in TS 36.331 [10].

1>	else if the UE is in NR-DC:

2>	if the measurement configuration that triggered this measurement report is associated with the SCG:

3>	if SRB3 is configured and the SCG is not deactivated:

4>	submit the MeasurementReport message via SRB3 to lower layers for transmission, upon which the procedure ends;

3>	else:

4>	submit the MeasurementReport message via SRB1 embedded in NR RRC message ULInformationTransferMRDC as specified in 5.7.2a.3;

2>	else:

3>	submit the MeasurementReport message via SRB1 to lower layers for transmission, upon which the procedure ends;

1>	else:

2>	submit the MeasurementReport message to lower layers for transmission, upon which the procedure ends.

#### 5.5.5.2	Reporting of beam measurement information

For beam measurement information to be included in a measurement report the UE shall:

1>	if reportType is set to eventTriggered:

2>	consider the trigger quantity as the sorting quantity if available, otherwise RSRP as sorting quantity if available, otherwise RSRQ as sorting quantity if available, otherwise SINR as sorting quantity;

1>	if reportType is set to periodical:

2>	if a single reporting quantity is set to true in reportQuantityRS-Indexes;

3>	consider the configured single quantity as the sorting quantity;

2>	else:

3>	if rsrp is set to true;

4>	consider RSRP as the sorting quantity;

3>	else:

4>	consider RSRQ as the sorting quantity;

1>	set rsIndexResults to include up to maxNrofRS-IndexesToReport SS/PBCH block indexes or CSI-RS indexes in order of decreasing sorting quantity as follows:

2>	if the measurement information to be included is based on SS/PBCH block:

3>	include within resultsSSB-Indexes the index associated to the best beam for that SS/PBCH block sorting quantity and if absThreshSS-BlocksConsolidation is included in the VarMeasConfig for the measObject associated to the cell for which beams are to be reported, the remaining beams whose sorting quantity is above absThreshSS-BlocksConsolidation;

3>	if includeBeamMeasurements is set to true, include the SS/PBCH based measurement results for the quantities in reportQuantityRS-Indexes for each SS/PBCH block index;

2>	else if the beam measurement information to be included is based on CSI-RS:

3>	include within resultsCSI-RS-Indexes the index associated to the best beam for that CSI-RS sorting quantity and, if absThreshCSI-RS-Consolidation is included in the VarMeasConfig for the measObject associated to the cell for which beams are to be reported, the remaining beams whose sorting quantity is above absThreshCSI-RS-Consolidation;

3>	if includeBeamMeasurements is set to true, include the CSI-RS based measurement results for the quantities in reportQuantityRS-Indexes for each CSI-RS index.

#### 5.5.5.3	Sorting of cell measurement results

The UE shall determine the sorting quantity according to parameters of the reportConfig associated with the measId that triggered the reporting:

1>	if the reportType is set to eventTriggered:

2>	for an NR cell, consider the quantity used in the aN-Threshold (for eventA1, eventA2 and eventA4) or in the a5-Threshold2 (for eventA5) or in the aN-Offset (for eventA3 and eventA6) as the sorting quantity;

2>	for an E-UTRA cell, consider the quantity used in the bN-ThresholdEUTRA as the sorting quantity;

2>	for an UTRA-FDD cell, consider the quantity used in the bN-ThresholdUTRA-FDD as the sorting quantity;

2>	for a candidate L2 U2N Relay UE, consider the yN-Threshold2-Relay as the sorting quantity;

1>	if the reportType is set to periodical:

2>	determine the sorting quantity according to reportQuantityCell for an NR cell, and according to reportQuantity for an E-UTRA cell, as below:

3>	if a single quantity is set to true:

4>	consider this quantity as the sorting quantity;

3>	else:

4>	if rsrp is set to true;

5>	consider RSRP as the sorting quantity;

4>	else:

5>	consider RSRQ as the sorting quantity;

2>	determine the sorting quantity according to reportQuantityUTRA-FDD for UTRA-FDD cell, as below:

3>	if a single quantity is set to true:

4>	consider this quantity as the sorting quantity;

3>	else:

4>	consider RSCP as the sorting quantity.

2>	for a candidate L2 U2N Relay UE, consider the reportQuantityRelay as the sorting quantity;

### 5.5.6	Location measurement indication

#### 5.5.6.1	General

Figure 5.5.5.1-1: Location measurement indication

The purpose of this procedure is to indicate to the network that the UE is going to start/stop location related measurements towards E-UTRA or NR (eutra-RSTD, nr-RSTD, nr-UE-RxTxTimeDiff, nr-PRS-RSRP) which require measurement gaps or start/stop detection of subframe and slot timing towards E-UTRA (eutra-FineTimingDetection) which requires measurement gaps. UE shall initiate this procedure only after successful AS security activation.

NOTE:	It is a network decision to configure the measurement gap.

#### 5.5.6.2	Initiation

The UE shall:

1>	if and only if upper layers indicate to start performing location measurements towards E-UTRA or NR or start subframe and slot timing detection towards E-UTRA, and the UE requires measurement gaps for these operations while measurement gaps are either not configured or not sufficient:

2>	if preconfigured measurement gaps for positioning and posMG-Request are configured and the UE considers that at least one of the preconfigured measurement gaps for positioning is sufficient for the location measurement when activated:

3>	trigger the lower layers to initiate the measurement gap activation request using UL MAC CE as specified in TS 38.321 [3];

2> else:

3>	initiate the procedure to indicate start as specified in clause 5.5.6.3;

NOTE 1:	The UE verifies the measurement gap situation only upon receiving the indication from upper layers. If at this point in time sufficient gaps are available, the UE does not initiate the procedure. Unless it receives a new indication from upper layers, the UE is only allowed to further repeat the procedure in the same PCell once per frequency of the target RAT if the provided measurement gaps are insufficient.

NOTE 1a:	When indication is received from upper layers for performing location measurement and there is pre-configured measurement gap configured (not preconfigured measurement gap for positioning), the UE considers this preconfigured measurement gap to be not sufficient if the measurement gap is not considered to be always activated according to clause 9.1.7.2 of TS 38.133 [14].

1>	if and only if upper layers indicate to stop performing location measurements towards E-UTRA or NR or stop subframe and slot timing detection towards E-UTRA:

2>	if there is no activated preconfigured measurement gap for positioning:

3>	if there is previously triggered UL MAC CE transmission for the measurement gap activation for positioning:

4>	indicate to the lower layers to cancel the triggered UL MAC CE transmission for the measurement gap activation as specified in TS 38.321 [3];

3>	else:

4>	initiate the procedure to indicate stop as specified in 5.5.6.3.

2>	else if there is activated preconfigured measurement gap for positioning:

3>	trigger the lower layers to deactivate all the activated measurement gap(s) for positioning as specified in TS 38.321 [3].

NOTE 2:	The UE may initiate the procedure to indicate stop even if it did not previously initiate the procedure to indicate start.

#### 5.5.6.3	Actions related to transmission of LocationMeasurementIndication message

The UE shall set the contents of LocationMeasurementIndication message as follows:

1>	if the procedure is initiated to indicate start of location related measurements:

2>	if the procedure is initiated for RSTD measurements towards E-UTRA:

3>	set the measurementIndication to the eutra-RSTD according to the information received from upper layers;

2>	else if the procedure is initiated for positioning measurement towards NR:

3>	set the measurementIndication to the nr-PRS-Measurement according to the information received from upper layers;

1>	else if the procedure is initiated to indicate stop of location related measurements:

2>	set the measurementIndication to the value release;

1>	if the procedure is initiated to indicate start of subframe and slot timing detection towards E-UTRA:

2>	set the measurementIndication to the value eutra-FineTimingDetection;

1>	else if the procedure is initiated to indicate stop of subframe and slot timing detection towards E-UTRA:

2>	set the measurementIndication to the value release;

- submit the LocationMeasurementIndication message to lower layers for transmission, upon which the procedure ends.

---------------------------------------------------------Skip Unchanged----------------------------------------------------------

## 5.6	UE capabilities

### 5.6.1	UE capability transfer

#### 5.6.1.1	General

This clause describes how the UE compiles and transfers its UE capability information upon receiving a UECapabilityEnquiry from the network.

Figure 5.6.1.1-1: UE capability transfer

#### 5.6.1.2	Initiation

The network initiates the procedure to a UE in RRC_CONNECTED when it needs (additional) UE radio access capability information. The network should retrieve UE capabilities only after AS security activation. Network does not forward UE capabilities that were retrieved before AS security activation to the CN.

#### 5.6.1.3	Reception of the UECapabilityEnquiry by the UE

The UE shall set the contents of UECapabilityInformation message as follows:

1>	if the ue-CapabilityRAT-RequestList contains a UE-CapabilityRAT-Request with rat-Type set to nr:

2>	include in the ue-CapabilityRAT-ContainerList a UE-CapabilityRAT-Container of the type UE-NR-Capability and with the rat-Type set to nr;

2>	include the supportedBandCombinationList, featureSets and featureSetCombinations as specified in clause 5.6.1.4;

1>	if the ue-CapabilityRAT-RequestList contains a UE-CapabilityRAT-Request with rat-Type set to eutra-nr:

2> if the UE supports (NG)EN-DC or NE-DC:

3>	include in the ue-CapabilityRAT-ContainerList a UE-CapabilityRAT-Container of the type UE-MRDC-Capability and with the rat-Type set to eutra-nr;

3>	include the supportedBandCombinationList and featureSetCombinations as specified in clause 5.6.1.4;

1>	if the ue-CapabilityRAT-RequestList contains a UE-CapabilityRAT-Request with rat-Type set to eutra:

2>	if the UE supports E-UTRA:

3>	include in the ue-CapabilityRAT-ContainerList a ue-CapabilityRAT-Container of the type UE-EUTRA-Capability and with the rat-Type set to eutra as specified in TS 36.331 [10], clause 5.6.3.3, according to the capabilityRequestFilter, if received;

1>	if the ue-CapabilityRAT-RequestList contains a UE-CapabilityRAT-Request with rat-Type set to utra-fdd:

2>	if the UE supports UTRA-FDD:

3>	include the UE radio access capabilities for UTRA-FDD within a ue-CapabilityRAT-Container and with the rat-Type set to utra-fdd;

1>	if the RRC message segmentation is enabled based on the field rrc-SegAllowed received, and the encoded RRC message is larger than the maximum supported size of a PDCP SDU specified in TS 38.323 [5]:

2>	initiate the UL message segment transfer procedure as specified in clause 5.7.7;

1>	else:

2>	submit the UECapabilityInformation message to lower layers for transmission, upon which the procedure ends.

#### 5.6.1.4	Setting band combinations, feature set combinations and feature sets supported by the UE

The UE invokes the procedures in this clause if the NR or E-UTRA network requests UE capabilities for nr, eutra-nr or eutra. This procedure is invoked once per requested rat-Type (see clause 5.6.1.3 for capability enquiry by the NR network; see TS 36.331 [10], clause 5.6.3.3 for capability enquiry by the E-UTRA network). The UE shall ensure that the feature set IDs are consistent across feature sets, feature set combinations and band combinations in all three UE capability containers that the network queries with the same fields with the same values, i.e. UE-CapabilityRequestFilterNR, UE-CapabilityRequestFilterCommon and fields in UECapabilityEnquiry message (i.e. requestedFreqBandsNR-MRDC, requestedCapabilityNR, eutra-nr-only flag, and requestedCapabilityCommon) as defined in TS 36.331, where applicable.

NOTE 1:	Capability enquiry without frequencyBandListFilter is not supported.

NOTE 2:	In (NG)EN-DC, the gNB needs the capabilities for RAT types nr and eutra-nr and it uses the featureSets in the UE-NR-Capability together with the featureSetCombinations in the UE-MRDC-Capability to determine the NR UE capabilities for the supported MRDC band combinations. Similarly, the eNB needs the capabilities for RAT types eutra and eutra-nr and it uses the featureSetsEUTRA in the UE-EUTRA-Capability together with the featureSetCombinations in the UE-MRDC-Capability to determine the E-UTRA UE capabilities for the supported MRDC band combinations. Hence, the IDs used in the featureSets must match the IDs referred to in featureSetCombinations across all three containers. The requirement on consistency implies that there are no undefined feature sets and feature set combinations.

NOTE 3:	If the UE cannot include all feature sets and feature set combinations due to message size or list size constraints, it is up to UE implementation which feature sets and feature set combinations it prioritizes.

The UE shall:

1>	compile a list of "candidate band combinations" according to the filter criteria in capabilityRequestFilterCommon (if included), only consisting of bands included in frequencyBandListFilter, and prioritized in the order of frequencyBandListFilter (i.e. first include band combinations containing the first-listed band, then include remaining band combinations containing the second-listed band, and so on), where for each band in the band combination, the parameters of the band do not exceed maxBandwidthRequestedDL, maxBandwidthRequestedUL, maxCarriersRequestedDL, maxCarriersRequestedUL, ca-BandwidthClassDL-EUTRA or ca-BandwidthClassUL-EUTRA, whichever are received;

1>	for each band combination included in the list of "candidate band combinations":

2>	if the network (E-UTRA) included the eutra-nr-only field, or

2>	if the requested rat-Type is eutra:

3>	remove the NR-only band combination from the list of "candidate band combinations";

NOTE 4:	The (E-UTRA) network may request capabilities for nr but indicate with the eutra-nr-only flag that the UE shall not include any NR band combinations in the UE-NR-Capability. In this case the procedural text above removes all NR-only band combinations from the candidate list and thereby also avoids inclusion of corresponding feature set combinations and feature sets below.

2>	if it is regarded as a fallback band combination with the same capabilities of another band combination included in the list of "candidate band combinations", and

2>	if this fallback band combination is generated by releasing at least one SCell or uplink configuration of SCell or SUL according to TS 38.306 [26]:

3>	remove the band combination from the list of "candidate band combinations";

NOTE 5:	Even if the network requests (only) capabilities for nr, it may include E-UTRA band numbers in the frequencyBandListFilter to ensure that the UE includes all necessary feature sets needed for subsequently requested eutra-nr capabilities. At this point of the procedure the list of "candidate band combinations" contains all NR- and/or E-UTRA-NR band combinations that match the filter (frequencyBandListFilter) provided by the NW and that match the eutra-nr-only flag (if RAT-Type nr is requested by E-UTRA). In the following, this candidate list is used to derive the band combinations, feature set combinations and feature sets to be reported in the requested capability container.

1>	if the requested rat-Type is nr:

2>	include into supportedBandCombinationList as many NR-only band combinations as possible from the list of "candidate band combinations", starting from the first entry;

3>	if srs-SwitchingTimeRequest is received:

4>	if SRS carrier switching is supported;

5>	include srs-SwitchingTimesListNR for each band combination;

4>	set srs-SwitchingTimeRequested to true;

2>	include, into featureSetCombinations, the feature set combinations referenced from the supported band combinations as included in supportedBandCombinationList according to the previous;

2>	compile a list of "candidate feature set combinations" referenced from the list of "candidate band combinations" excluding entries (rows in feature set combinations) with same or lower capabilities;

2>	if uplinkTxSwitchRequest is received:

3>	include into supportedBandCombinationList-UplinkTxSwitch as many NR-only band combinations that supported UL TX switching as possible from the list of "candidate band combinations", starting from the first entry;

4>	if srs-SwitchingTimeRequest is received:

5>	if SRS carrier switching is supported;

6>	include srs-SwitchingTimesListNR for each band combination;

5>	set srs-SwitchingTimeRequested to true;

3>	include, into featureSetCombinations, the feature set combinations referenced from the supported band combinations as included in supportedBandCombinationList-UplinkTxSwitch according to the previous;

NOTE 6:	This list of "candidate feature set combinations" contains the feature set combinations used for NR-only as well as E-UTRA-NR band combinations. It is used to derive a list of NR feature sets referred to from the feature set combinations in the UE-NR-Capability and from the feature set combinations in a UE-MRDC-Capability container.

2>	if sidelinkRequest is received:

3>	for a sidelink band combination the UE included in supportedBandCombinationListSidelinkEUTRA-NR, supportedBandCombinationListSL-RelayDiscovery or supportedBandCombinationListSL-NonRelayDiscovery:

4>	if the UE supports partial sensing for a band of the sidelink band combination, include the partial sensing capabilities for the band using the sl-TransmissionMode2-PartialSensing-r17;

3>	set sidelinkRequested to true;

2>	include into featureSets the feature sets referenced from the "candidate feature set combinations" and may exclude the feature sets with the parameters that exceed any of maxBandwidthRequestedDL, maxBandwidthRequestedUL, maxCarriersRequestedDL or maxCarriersRequestedUL, whichever are received;

1>	else, if the requested rat-Type is eutra-nr:

2>	include into supportedBandCombinationList and/or supportedBandCombinationListNEDC-Only as many E-UTRA-NR band combinations as possible from the list of "candidate band combinations", starting from the first entry;

3>	if srs-SwitchingTimeRequest is received:

4>	if SRS carrier switching is supported;

5>	include srs-SwitchingTimesListNR and srs-SwitchingTimesListEUTRA for each band combination;

4>	set srs-SwitchingTimeRequested to true;

2>	include, into featureSetCombinations, the feature set combinations referenced from the supported band combinations as included in supportedBandCombinationList according to the previous;

2>	if uplinkTxSwitchRequest is received:

3>	include into supportedBandCombinationList-UplinkTxSwitch as many E-UTRA-NR band combinations that supported UL TX switching as possible from the list of "candidate band combinations", starting from the first entry;

4>	if srs-SwitchingTimeRequest is received:

5>	if SRS carrier switching is supported;

6>	include srs-SwitchingTimesListNR and srs-SwitchingTimesListEUTRA for each band combination;

5>	set srs-SwitchingTimeRequested to true;

3>	include, into featureSetCombinations, the feature set combinations referenced from the supported band combinations as included in supportedBandCombinationList-UplinkTxSwitch according to the previous;

1>	else (if the requested rat-Type is eutra):

2>	compile a list of "candidate feature set combinations" referenced from the list of "candidate band combinations" excluding entries (rows in feature set combinations) with same or lower capabilities;

NOTE 7:	This list of "candidate feature set combinations" contains the feature set combinations used for E-UTRA-NR band combinations. It is used to derive a list of E-UTRA feature sets referred to from the feature set combinations in a UE-MRDC-Capability container.

2>	include into featureSetsEUTRA (in the UE-EUTRA-Capability) the feature sets referenced from the "candidate feature set combinations" and may exclude the feature sets with the parameters that exceed ca-BandwidthClassDL-EUTRA or ca-BandwidthClassUL-EUTRA, whichever are received;

1>	include the received frequencyBandListFilter in the field appliedFreqBandListFilter of the requested UE capability, except if the requested rat-Type is nr and the network included the eutra-nr-only field;

1>	if the network included ue-CapabilityEnquiryExt:

2>	include the received ue-CapabilityEnquiryExt in the field receivedFilters;

---------------------------------------------------------Skip Unchanged----------------------------------------------------------

## 5.7	Other

---------------------------------------------------------Skip Unchanged----------------------------------------------------------

### 5.7.4	UE Assistance Information

#### 5.7.4.1	General

Figure 5.7.4.1-1: UE Assistance Information

The purpose of this procedure is for the UE to inform the network of:

-	its delay budget report carrying desired increment/decrement in the connected mode DRX cycle length, or;

-	its overheating assistance information, or;

-	its IDC assistance information, or;

-	its preference on DRX parameters for power saving, or;

-	its preference on the maximum aggregated bandwidth for power saving, or;

-	its preference on the maximum number of secondary component carriers for power saving, or;

-	its preference on the maximum number of MIMO layers for power saving, or;

-	its preference on the minimum scheduling offset for cross-slot scheduling for power saving, or;

-	its preference on the RRC state, or;

-	configured grant assistance information for NR sidelink communication, or;

-	its preference in being provisioned with reference time information, or;

-	its preference for FR2 UL gap, or;

-	its preference to transition out of RRC_CONNECTED state for MUSIM operation, or;

-	its preference on the MUSIM gaps, or;

-	its preference on the MUSIM gap priority, or;

-	its preference on the MUSIM temporary capability restriction, or;

-	its relaxation state for RLM measurements, or;

-	its relaxation state for BFD measurements, or;

-	availability of data and/or signalling mapped to radio bearers which are not configured for SDT, or;

-	its preference for the SCG to be deactivated, or;

-	indicate that the UE has uplink data to transmit for a DRB for which there is no MCG RLC bearer while the SCG is deactivated, or;

-	change of its fulfilment status for RRM measurement relaxation criterion, or;

-	service link (specified in TS 38.300 [2]) propagation delay difference between serving cell and neighbour cell(s).

#### 5.7.4.2	Initiation

A UE capable of providing delay budget report in RRC_CONNECTED may initiate the procedure in several cases, including upon being configured to provide delay budget report and upon change of delay budget preference.

A UE capable of providing overheating assistance information in RRC_CONNECTED may initiate the procedure if it was configured to do so, upon detecting internal overheating, or upon detecting that it is no longer experiencing an overheating condition.

A UE capable of providing IDC assistance information in RRC_CONNECTED may initiate the procedure if it was configured to do so, upon detecting IDC problem if the UE did not transmit an IDC assistance information since it was configured to provide IDC indications, or upon change of IDC problem information.

A UE capable of providing its preference on DRX parameters of a cell group for power saving in RRC_CONNECTED may initiate the procedure in several cases, if it was configured to do so, including upon having a preference on DRX parameters and upon change of its preference on DRX parameters.

A UE capable of providing its preference on the maximum aggregated bandwidth of a cell group for power saving in RRC_CONNECTED may initiate the procedure in several cases, if it was configured to do so, including upon having a maximum aggregated bandwidth preference and upon change of its maximum aggregated bandwidth preference.

A UE capable of providing its preference on the maximum number of secondary component carriers of a cell group for power saving in RRC_CONNECTED may initiate the procedure in several cases, if it was configured to do so, including upon having a maximum number of secondary component carriers preference and upon change of its maximum number of secondary component carriers preference.

A UE capable of providing its preference on the maximum number of MIMO layers of a cell group for power saving in RRC_CONNECTED may initiate the procedure in several cases, if it was configured to do so, including upon having a maximum number of MIMO layers preference and upon change of its maximum number of MIMO layers preference.

A UE capable of providing its preference on the minimum scheduling offset for cross-slot scheduling of a cell group for power saving in RRC_CONNECTED may initiate the procedure in several cases, if it was configured to do so, including upon having a minimum scheduling offset preference and upon change of its minimum scheduling offset preference.

A UE capable of providing assistance information to transition out of RRC_CONNECTED state may initiate the procedure if it was configured to do so, upon determining that it prefers to transition out of RRC_CONNECTED state, or upon change of its preferred RRC state.

A UE capable of providing configured grant assistance information for NR sidelink communication in RRC_CONNECTED may initiate the procedure in several cases, including upon being configured to provide traffic pattern information and upon change of traffic patterns.

A UE capable of providing an indication of its preference in being provisioned with reference time information may initiate the procedure upon being configured to provide this indication, or if it was configured to provide this indication and upon change of its preference.

A UE capable of providing an indication of its preference in FR2 UL gap may initiate the procedure if it was configured to do so, upon detecting the need of FR2 UL gap activation/deactivation.

A UE capable of providing MUSIM assistance information for gap preference may initiate the procedure if it was configured to do so, upon determining it needs the gaps, or upon change of the gap preference information.

A UE capable of providing MUSIM assistance information for leave indication may initiate the procedure if it was configured to do so upon determining that it needs to leave RRC_CONNECTED state.

A UE capable of providing MUSIM assistance information for gap priority preference may initiate the procedure if it was configured to do so, upon determining it has gap priority preference information.

A UE capable of providing MUSIM assistance information for temporary capability restriction may initiate the procedure if it was configured to do so, upon determining it has temporary capability restriction or upon determining the removal of the capability restriction.

A UE capable of relaxing its RLM measurements of a cell group in RRC_CONNECTED state shall initiate the procedure for providing an indication of its relaxation state for RLM measurements upon being configured to do so, and upon change of its relaxation state for RLM measurements in RRC_CONNECTED state.

A UE capable of relaxing its BFD measurements in serving cells of a cell group in RRC_CONNECTED shall initiate the procedure for providing an indication of its relaxation state for BFD measurements upon being configured to do so, and upon change of its relaxation state for BFD measurements in RRC_CONNECTED state.

A UE capable of SDT initiates this procedure when data and/or signalling mapped to radio bearers that are not configured for SDT becomes available during SDT (i.e. while SDT procedure is ongoing).

A UE capable of providing its preference for SCG deactivation may initiate the procedure if it was configured to do so, upon determining that it prefers or does no more prefer the SCG to be deactivated.

A UE that has uplink data to transmit for a DRB for which there is no MCG RLC bearer while the SCG is deactivated shall initiate the procedure.

A UE capable of providing an indication of fulfilment of the RRM measurement relaxation criterion in connected mode may initiate the procedure if it was configured to do so, upon change of its fulfilment status for RRM measurement relaxation criterion for connected mode.

A UE capable of providing service link propagation delay difference between serving cell and neighbour cell(s) shall initiate the procedure upon being configured to do so, and upon determining that service link propagation delay difference between serving cell and a neighbour cell has changed more than threshPropDelayDiff compared with the last reported value.

Upon initiating the procedure, the UE shall:

1>	if configured to provide delay budget report:

2>	if the UE did not transmit a UEAssistanceInformation message with delayBudgetReport since it was configured to provide delay budget report; or

2>	if the current delay budget is different from the one indicated in the last transmission of the UEAssistanceInformation message including delayBudgetReport and timer T342 is not running:

3>	start or restart timer T342 with the timer value set to the delayBudgetReportingProhibitTimer;

3>	initiate transmission of the UEAssistanceInformation message in accordance with 5.7.4.3 to provide a delay budget report;

1>	if configured to provide overheating assistance information:

2>	if the overheating condition has been detected and T345 is not running; or

2>	if the current overheating assistance information is different from the one indicated in the last transmission of the UEAssistanceInformation message including overheatingAssistance and timer T345 is not running:

3>	start timer T345 with the timer value set to the overheatingIndicationProhibitTimer;

3>	initiate transmission of the UEAssistanceInformation message in accordance with 5.7.4.3 to provide overheating assistance information;

1>	if configured to provide IDC assistance information:

2>	if the UE did not transmit a UEAssistanceInformation message with idc-Assistance since it was configured to provide IDC assistance information:

3>	if on one or more frequencies included in candidateServingFreqListNR, the UE is experiencing IDC problems that it cannot solve by itself; or

3>	if on one or more supported UL CA combination comprising of carrier frequencies included in candidateServingFreqListNR, the UE is experiencing IDC problems that it cannot solve by itself:

4>	initiate transmission of the UEAssistanceInformation message in accordance with 5.7.4.3 to provide IDC assistance information;

2>	else if the current IDC assistance information is different from the one indicated in the last transmission of the UEAssistanceInformation message:

3>	initiate transmission of the UEAssistanceInformation message in accordance with 5.7.4.3 to provide IDC assistance information;

NOTE 1:	The term "IDC problems" refers to interference issues applicable across several subframes/slots where not necessarily all the subframes/slots are affected.

NOTE 2:	For the frequencies on which a serving cell or serving cells is configured that is activated, IDC problems consist of interference issues that the UE cannot solve by itself, during either active data exchange or upcoming data activity which is expected in up to a few hundred milliseconds.
For frequencies on which a SCell or SCells is configured that is deactivated, reporting IDC problems indicates an anticipation that the activation of the SCell or SCells would result in interference issues that the UE would not be able to solve by itself.
For a non-serving frequency, reporting IDC problems indicates an anticipation that if the non-serving frequency or frequencies became a serving frequency or serving frequencies then this would result in interference issues that the UE would not be able to solve by itself.

1>	if configured to provide its preference on DRX parameters of a cell group for power saving:

2>	if the UE has a preference on DRX parameters of the cell group and the UE did not transmit a UEAssistanceInformation message with drx-Preference for the cell group since it was configured to provide its preference on DRX parameters of the cell group for power saving; or

2>	if the current drx-Preference information for the cell group is different from the one indicated in the last transmission of the UEAssistanceInformation message including drx-Preference for the cell group and timer T346a associated with the cell group is not running:

3>	start the timer T346a with the timer value set to the drx-PreferenceProhibitTimer of the cell group;

3>	initiate transmission of the UEAssistanceInformation message in accordance with 5.7.4.3 to provide the current drx-Preference;

1>	if configured to provide its preference on the maximum aggregated bandwidth of a cell group for power saving:

2>	if the UE has a preference on the maximum aggregated bandwidth of the cell group and the UE did not transmit a UEAssistanceInformation message with maxBW-Preference and/or maxBW-PreferenceFR2-2 for the cell group since it was configured to provide its preference on the maximum aggregated bandwidth of the cell group for power saving; or

2>	if the current maxBW-Preference information for the cell group is different from the one indicated in the last transmission of the UEAssistanceInformation message including maxBW-Preference and/or maxBW-PreferenceFR2-2for the cell group and timer T346b associated with the cell group is not running:

3>	start the timer T346b with the timer value set to the maxBW-PreferenceProhibitTimer of the cell group;

3>	initiate transmission of the UEAssistanceInformation message in accordance with 5.7.4.3 to provide the current maxBW-Preference and/or maxBW-PreferenceFR2-2;

1>	if configured to provide its preference on the maximum number of secondary component carriers of a cell group for power saving:

2>	if the UE has a preference on the maximum number of secondary component carriers of the cell group and the UE did not transmit a UEAssistanceInformation message with maxCC-Preference for the cell group since it was configured to provide its preference on the maximum number of secondary component carriers of the cell group for power saving; or

2>	if the current maxCC-Preference information for the cell group is different from the one indicated in the last transmission of the UEAssistanceInformation message including maxCC-Preference for the cell group and timer T346c associated with the cell group is not running:

3>	start the timer T346c with the timer value set to the maxCC-PreferenceProhibitTimer of the cell group;

3>	initiate transmission of the UEAssistanceInformation message in accordance with 5.7.4.3 to provide the current maxCC-Preference;

1>	if configured to provide its preference on the maximum number of MIMO layers of a cell group for power saving:

2>	if the UE has a preference on the maximum number of MIMO layers of the cell group and the UE did not transmit a UEAssistanceInformation message with maxMIMO-LayerPreference and/or maxMIMO-LayerPreferenceFR2-2 for the cell group since it was configured to provide its preference on the maximum number of MIMO layers of the cell group for power saving; or

2>	if the current maxMIMO-LayerPreference information for the cell group is different from the one indicated in the last transmission of the UEAssistanceInformation message including maxMIMO-LayerPreference and/or maxMIMO-LayerPreferenceFR2-2 for the cell group and timer T346d associated with the cell group is not running:

3>	start the timer T346d with the timer value set to the maxMIMO-LayerPreferenceProhibitTimer of the cell group;

3>	initiate transmission of the UEAssistanceInformation message in accordance with 5.7.4.3 to provide the current maxMIMO-LayerPreference and/or maxMIMO-LayerPreferenceFR2-2;

1>	if configured to provide its preference on the minimum scheduling offset for cross-slot scheduling of a cell group for power saving:

2>	if the UE has a preference on the minimum scheduling offset for cross-slot scheduling of the cell group and the UE did not transmit a UEAssistanceInformation message with minSchedulingOffsetPreference and/or minSchedulingOffsetPreferenceExt for the cell group since it was configured to provide its preference on the minimum scheduling offset for cross-slot scheduling of the cell group for power saving; or

2>	if the current minSchedulingOffsetPreference and/or minSchedulingOffsetPreferenceExt information for the cell group is different from the one indicated in the last transmission of the UEAssistanceInformation message including minSchedulingOffsetPreference and/or minSchedulingOffsetPreferenceExt for the cell group and timer T346e associated with the cell group is not running:

3>	start the timer T346e with the timer value set to the minSchedulingOffsetPreferenceProhibitTimer of the cell group;

3>	initiate transmission of the UEAssistanceInformation message in accordance with 5.7.4.3 to provide the current minSchedulingOffsetPreference and/or minSchedulingOffsetPreferenceExt;

1>	if configured to provide its release preference and timer T346f is not running:

2>	if the UE determines that it would prefer to transition out of RRC_CONNECTED state; or

2>	if the UE is configured with connectedReporting and the UE determines that it would prefer to revert an earlier indication to transition out of RRC_CONNECTED state:

3>	start timer T346f with the timer value set to the releasePreferenceProhibitTimer;

3>	initiate transmission of the UEAssistanceInformation message in accordance with 5.7.4.3 to provide the release preference;

1>	if configured to provide configured grant assistance information for NR sidelink communication:

2>	initiate transmission of the UEAssistanceInformation message in accordance with 5.7.4.3 to provide configured grant assistance information for NR sidelink communication;

1>	if configured to provide preference in being provisioned with reference time information:

2>	if the UE did not transmit a UEAssistanceInformation message with referenceTimeInfoPreference since it was configured to provide preference; or

2>	if the UE's preference changed from the last time UE initiated transmission of the UEAssistanceInformation message including referenceTimeInfoPreference:

3>	initiate transmission of the UEAssistanceInformation message in accordance with 5.7.4.3 to provide preference in being provisioned with reference time information.

1>	if configured to provide its preference on FR2 UL gap:

2>	if the UE did not transmit a UEAssistanceInformation message with ul-GapFR2-Preference since it was configured to provide its preference on FR2 UL gap information:

3>	if the UE has a preference on FR2 UL gap activation/deactivation:

4>	initiate transmission of the UEAssistanceInformation message in accordance with 5.7.4.3 to provide FR2 UL gap preference;

2>	else if the current FR2 UL gap preference is different from the one indicated in the last transmission of the UEAssistanceInformation message:

3>	initiate transmission of the UEAssistanceInformation message in accordance with 5.7.4.3 to provide FR2 UL gap preference.

1>	if configured to provide MUSIM assistance information for leaving RRC_CONNECTED:

2>	if the UE needs to leave RRC_CONNECTED state and the timer T346g is not running:

3>	initiate transmission of the UEAssistanceInformation message in accordance with 5.7.4.3 to provide MUSIM assistance information for leaving RRC_CONNECTED;

3>	start the timer T346g with the timer value set to the musim-LeaveWithoutResponseTimer;

1>	if configured to provide MUSIM assistance information for gap preference:

2>	if the UE has a preference on the MUSIM gap(s) and the UE did not transmit a UEAssistanceInformation message with musim-GapPreferenceList since it was configured to provide MUSIM assistance information for gap preference; or

2>	if the current musim-GapPreferenceList is different from the one indicated in the last transmission of the UEAssistanceInformation message including musim-GapPreferenceList and the timer T346h is not running:

3>	initiate transmission of the UEAssistanceInformation message in accordance with 5.7.4.3 to provide the current musim-GapPreferenceList;

3>	start or restart the timer T346h with the timer value set to the musim-GapProhibitTimer.

NOTE 3:	The UE does not need to initiate transmission of the UEAssistanceInformation message if the difference between the current musim-GapPreferenceList and the last transmission of the UEAssistanceInformation message including musim-GapPreferenceList is only due to removal of an ended aperiodic gap.

1>	if configured to provide MUSIM assistance information for gap priority preference:

2>	if the UE has a preference on the MUSIM gap(s) priority and the UE did not transmit a UEAssistanceInformation message with musim-GapPriorityPreferenceList since it was configured to provide MUSIM assistance information for gap priority preference; or

2>	if the current musim-GapPriorityPreferenceList is different from the one indicated in the last transmission of the UEAssistanceInformation message including musim-GapPriorityPreferenceList:

3>	initiate transmission of the UEAssistanceInformation message in accordance with 5.7.4.3 to provide the current musim-GapPriorityPreferenceList;

Editor note:	FFS whether prohibit timer is needed for the signaling of MUSIM gap(s) priority preference.

1>	if configured to provide MUSIM assistance information for temporary capability restriction:

2>	if the current musim-CapRestriction is different from the one indicated in the last transmission of the UEAssistanceInformation message including musim-CapRestriction for affected capability for serving cell(s) or serving cell(s) release or SCG release and timer T3xx is not running:

3>	initiate transmission of the UEAssistanceInformation message in accordance with 5.7.4.3 to provide the current musim-Cell-SCG-ToRelease and/or musim-CellToAffectList;

3>	start the timer T3xx with the timer value set to the musim-WaitTimer.

2>	if the current musim-CapRestriction is different from the one indicated in the last transmission of the UEAssistanceInformation message including musim-CapRestriction for constraint band combination(s)and timer T3xy is not running:

3>	initiate transmission of the UEAssistanceInformation message in accordance with 5.7.4.3 to provide the current musim-AffectedBandCombList and/or musim-ForbiddenBandCombList;

3>	start or restart the timer T3xy with the timer value set to the musim-ProhibitTimer.

2>	if the current musim-CapRestriction is different from the one indicated in the last transmission of the UEAssistanceInformation message including musim-CapRestriction to provide the measurement gap requirement of NR target bands:

3>	initiate transmission of the UEAssistanceInformation message in accordance with 5.7.4.3 to provide the current MUSIM assistance information for temporary capability restriction;

2>else if the UE has a preference on the temporary capability restriction and the UE did not transmit a UEAssistanceInformation message with musim-CapRestriction since it was configured to provide MUSIM assistance information for temporary capability restriction:

3>	initiate transmission of the UEAssistanceInformation message in accordance with 5.7.4.3 to provide the current MUSIM assistance information for temporary capability restriction;

2>if the UE has a preference on the measurement gap requirement information and the UE did not transmit a UEAssistanceInformation message with measurement gap requirement information or RRCReconfigurationComplete message with measurement gap requirement information since it was configured to provide its preference on the measurement gap requirement information MUSIM; or

2>	if the current musim-NeedForGapsInfoNR is different from the one indicated in the last transmission of the UEAssistanceInformation message or RRCReconfigurationComplete message including musim-CapRestriction to provide the measurement gap requirement of NR target bands:

3>	initiate transmission of the UEAssistanceInformation message in accordance with 5.7.4.3 to provide the current musim-NeedForGapsInfoNR;

1>	if configured to provide the relaxation state of RLM measurements of a cell group and RLM measurement of the cell group is not stopped:

2>	if the UE did not transmit a UEAssistanceInformation message with rlm-MeasRelaxationState since it was configured to provide the relaxation state of RLM measurements for the cell group; or

2>	if the relaxation state of RLM measurements for the cell group is currently different from the relaxation state reported in the last transmission of the UEAssistanceInformation message including rlm-MeasRelaxationState of the cell group and timer T346j associated with the cell group is not running:

3>	start timer T346j with the timer value set to the rlm-RelaxtionReportingProhibitTimer;

3>	initiate transmission of the UEAssistanceInformation message in accordance with 5.7.4.3 to provide the relaxation state of RLM measurements of the cell group;

1>	if configured to provide the relaxation state of BFD measurements of serving cells of a cell group and BFD measurement of the cell group is not stopped:

2>	if the UE did not transmit a UEAssistanceInformation message with bfd-MeasRelaxationState since it was configured to provide the relaxation state of BFD measurements for the cell group; or

2>	if the relaxation state of BFD measurements in any serving cell of the cell group is currently different from the relaxation state reported in the last transmission of the UEAssistanceInformation message including bfd-MeasRelaxationState of the cell group and timer T346k associated with the cell group is not running:

3>	start timer T346k with the timer value set to the bfd-RelaxtionReportingProhibitTimer;

3>	initiate transmission of the UEAssistanceInformation message in accordance with 5.7.4.3 to provide the relaxation state of BFD measurements of serving cells of the cell group.

1>	if data and/or signalling mapped to radio bearers not configured for SDT becomes available during SDT (i.e. while SDT procedure is ongoing):

2>	if the UE did not transmit a UEAssistanceInformation message with nonSDT-DataIndication since the initiation of the current resume procedure for SDT:

3>	initiate transmission of the UEAssistanceInformation message in accordance with 5.7.4.3 to provide nonSDT-DataIndication.

1>	if configured to provide its preference for SCG deactivation and timer T346i is not running;

2>	if the UE prefers the SCG to be deactivated and did not transmit a UEAssistanceInformation message with scg-DeactivationPreference since it was configured to provide its SCG deactivation preference; or

2>	if the UE preference for SCG deactivation is different from the last indicated scg-DeactivationPreference:

3>	start timer T346i with the timer value set to the scg-DeactivationPreferenceProhibitTimer;

3>	initiate transmission of the UEAssistanceInformation message in accordance with 5.7.4.3 to provide the UE preference for SCG deactivation;

1>	if the SCG is deactivated, and,

1>	the UE has uplink data to send for an SCG RLC entity while the UE previously did not have any uplink data to send for any SCG RLC entity:

2>	initiate transmission of the UEAssistanceInformation message in accordance with 5.7.4.3 to indicate that the UE has uplink data to send for a DRB whose DRB-Identity is not included in any RLC-BearerConfig in the CellGroupConfig associated with the MCG.

1>	if configured to send indications of RRM measurement relaxation criterion fulfilment:

2>	if the criterion in 5.7.4.4 is met for a period of TSearchDeltaP-StationaryConnected:

3>	if the UE did not transmit a UEAssistanceInformation message with rrm-MeasRelaxationFulfilment as true since it was configured to provide indications of RRM measurement relaxation criterion fulfilment; or

3>	the last UEAssistanceInformation message indicated the criterion in 5.7.4.4 is not fulfilled with rrm-MeasRelaxationFulfilment as false:

4>	initiate transmission of the UEAssistanceInformation message in accordance with 5.7.4.3 to indicate that the criterion for RRM measurement relaxation for connected mode is fulfilled;

2>	else:

3>	if the last UEAssistanceInformation message indicated fulfilment of the criterion in 5.7.4.4 with rrm-MeasRelaxationFulfilment as true:

4>	initiate transmission of the UEAssistanceInformation message in accordance with 5.7.4.3 to indicate that the criterion for RRM measurement relaxation for connected mode is not fulfilled.

1>	if configured to provide service link propagation delay difference between serving cell and neighbour cell(s);

2>	if the UE did not transmit a UEAssistanceInformation message with propagationDelayDifference since it was configured to provide service link propagation delay difference between serving cell and neighbour cell(s); or

2>	for any neighbour cell in neighCellInfoList, if the service link propagation delay difference between serving cell and the neighbour cell has changed more than threshPropDelayDiff since the last transmission of the UEAssistanceInformation message including propagationDelayDifference:

3>	initiate transmission of the UEAssistanceInformation message in accordance with 5.7.4.3 to provide service link propagation delay difference between serving cell and each neighbour cell included in the neighCellInfoList;

#### 5.7.4.3	Actions related to transmission of UEAssistanceInformation message

The UE shall set the contents of the UEAssistanceInformation message as follows:

1>	if transmission of the UEAssistanceInformation message is initiated to provide a delay budget report according to 5.7.4.2 or 5.3.5.3;

2>	set delayBudgetReport to type1 according to a desired value;

1>	if transmission of the UEAssistanceInformation message is initiated to provide overheating assistance information according to 5.7.4.2 or 5.3.5.3;

2>	if the UE experiences internal overheating:

3>	if the UE prefers to temporarily reduce the number of maximum secondary component carriers:

4>	include reducedMaxCCs in the OverheatingAssistance IE;

4>	set reducedCCsDL to the number of maximum SCells the UE prefers to be temporarily configured in downlink;

4>	set reducedCCsUL to the number of maximum SCells the UE prefers to be temporarily configured in uplink;

3>	if the UE prefers to temporarily reduce maximum aggregated bandwidth of FR1:

4>	include reducedMaxBW-FR1 in the OverheatingAssistance IE;

4>	set reducedBW-DL to the maximum aggregated bandwidth the UE prefers to be temporarily configured across all downlink carriers of FR1;

4>	set reducedBW-UL to the maximum aggregated bandwidth the UE prefers to be temporarily configured across all uplink carriers of FR1;

3>	if the UE prefers to temporarily reduce maximum aggregated bandwidth of FR2-1:

4>	include reducedMaxBW-FR2 in the OverheatingAssistance IE;

4>	set reducedBW-DL to the maximum aggregated bandwidth the UE prefers to be temporarily configured across all downlink carriers of FR2-1;

4>	set reducedBW-UL to the maximum aggregated bandwidth the UE prefers to be temporarily configured across all uplink carriers of FR2-1;

3>	if the UE prefers to temporarily reduce maximum aggregated bandwidth of FR2-2:

4>	include reducedMaxBW-FR2-2 in the OverheatingAssistance IE;

4>	set reducedBW-FR2-2-DL to the maximum aggregated bandwidth the UE prefers to be temporarily configured across all downlink carriers of FR2-2;

4>	set reducedBW-FR2-2-UL to the maximum aggregated bandwidth the UE prefers to be temporarily configured across all uplink carriers of FR2-2;

3>	if the UE prefers to temporarily reduce the number of maximum MIMO layers of each serving cell operating on FR1:

4>	include reducedMaxMIMO-LayersFR1 in the OverheatingAssistance IE;

4>	set reducedMIMO-LayersFR1-DL to the number of maximum MIMO layers of each serving cell operating on FR1 the UE prefers to be temporarily configured in downlink;

4>	set reducedMIMO-LayersFR1-UL to the number of maximum MIMO layers of each serving cell operating on FR1 the UE prefers to be temporarily configured in uplink;

3>	if the UE prefers to temporarily reduce the number of maximum MIMO layers of each serving cell operating on FR2-1:

4>	include reducedMaxMIMO-LayersFR2 in the OverheatingAssistance IE;

4>	set reducedMIMO-LayersFR2-DL to the number of maximum MIMO layers of each serving cell operating on FR2-1 the UE prefers to be temporarily configured in downlink;

4>	set reducedMIMO-LayersFR2-UL to the number of maximum MIMO layers of each serving cell operating on FR2-1 the UE prefers to be temporarily configured in uplink;

3>	if the UE prefers to temporarily reduce the number of maximum MIMO layers of each serving cell operating on FR2-2:

4>	include reducedMaxMIMO-LayersFR2-2 in the OverheatingAssistance IE;

4>	set reducedMIMO-LayersFR2-2-DL to the number of maximum MIMO layers of each serving cell operating on FR2 the UE prefers to be temporarily configured in downlink;

4>	set reducedMIMO-LayersFR2-2-UL to the number of maximum MIMO layers of each serving cell operating on FR2 the UE prefers to be temporarily configured in uplink;

2>	else (if the UE no longer experiences an overheating condition):

3>	do not include reducedMaxCCs, reducedMaxBW-FR1, reducedMaxBW-FR2, reducedMaxBW-FR2-2, reducedMaxMIMO-LayersFR1, reducedMaxMIMO-LayersFR2 or reducedMaxMIMO-LayersFR2-2 in OverheatingAssistance IE;

1>	if transmission of the UEAssistanceInformation message is initiated to provide IDC assistance information according to 5.7.4.2 or 5.3.5.3:

2>	if there is at least one carrier frequency included in candidateServingFreqListNR, the UE is experiencing IDC problems that it cannot solve by itself:

3>	include the field affectedCarrierFreqList with an entry for each affected carrier frequency included in candidateServingFreqListNR;

3>	for each carrier frequency included in the field affectedCarrierFreqList, include interferenceDirection and set it accordingly;

2>	if there is at least one supported UL CA combination comprising of carrier frequencies included in candidateServingFreqListNR, the UE is experiencing IDC problems that it cannot solve by itself:

3>	include victimSystemType for each UL CA combination included in affectedCarrierFreqCombList;

3>	if the UE sets victimSystemType to wlan or bluetooth:

4>	include affectedCarrierFreqCombList with an entry for each supported UL CA combination comprising of carrier frequencies included in candidateServingFreqListNR, that is affected by IDC problems;

3>	else:

4>	optionally include affectedCarrierFreqCombList with an entry for each supported UL CA combination comprising of carrier frequencies included in candidateServingFreqListNR, that is affected by IDC problems;

NOTE 1:	When sending an UEAssistanceInformation message to inform the IDC problems, the UE includes all IDC assistance information (rather than providing e.g. the changed part(s) of the IDC assistance information).

NOTE 2:	Upon not anymore experiencing a particular IDC problem that the UE previously reported, the UE provides an IDC indication with the modified contents of the UEAssistanceInformation message (e.g. by not including the IDC assistance information in the idc-Assistance field).

1>	if transmission of the UEAssistanceInformation message is initiated to provide drx-Preference of a cell group for power saving according to 5.7.4.2 or 5.3.5.3:

2>	include drx-Preference in the UEAssistanceInformation message;

2>	if the UE has a preference on DRX parameters for the cell group:

3>	if the UE has a preference for the long DRX cycle:

4>	include preferredDRX-LongCycle in the DRX-Preference IE and set it to the preferred value;

3>	if the UE has a preference for the DRX inactivity timer:

4>	include preferredDRX-InactivityTimer in the DRX-Preference IE and set it to the preferred value;

3>	if the UE has a preference for the short DRX cycle:

4>	include preferredDRX-ShortCycle in the DRX-Preference IE and set it to the preferred value;

3>	if the UE has a preference for the short DRX timer:

4>	include preferredDRX-ShortCycleTimer in the DRX-Preference IE and set it to the preferred value;

2>	else (if the UE has no preference on DRX parameters for the cell group):

3>	do not include preferredDRX-LongCycle, preferredDRX-InactivityTimer, preferredDRX-ShortCycle and preferredDRX-ShortCycleTimer in the DRX-Preference IE;

1>	if transmission of the UEAssistanceInformation message is initiated to provide maxBW-Preference of a cell group for power saving according to 5.7.4.2 or 5.3.5.3:

2>	include maxBW-Preference in the UEAssistanceInformation message;

2>	if the UE has a preference on the maximum aggregated bandwidth for the cell group:

3>	if the UE prefers to reduce the maximum aggregated bandwidth of FR1:

4>	include reducedMaxBW-FR1 in the MaxBW-Preference IE;

4>	set reducedBW-DL to the maximum aggregated bandwidth the UE desires to have configured across all downlink carriers of FR1 in the cell group;

4>	set reducedBW-UL to the maximum aggregated bandwidth the UE desires to have configured across all uplink carriers of FR1 in the cell group;

3>	if the UE prefers to reduce the maximum aggregated bandwidth of FR2-1:

4>	include reducedMaxBW-FR2 in the MaxBW-Preference IE;

4>	set reducedBW-DL to the maximum aggregated bandwidth the UE desires to have configured across all downlink carriers of FR2-1 in the cell group;

4>	set reducedBW-UL to the maximum aggregated bandwidth the UE desires to have configured across all uplink carriers of FR2-1 in the cell group;

2>	else (if the UE has no preference on the maximum aggregated bandwidth for the cell group):

3>	do not include reducedMaxBW-FR1 and reducedMaxBW-FR2 in the MaxBW-Preference IE;

1>	if transmission of the UEAssistanceInformation message is initiated to provide maxBW-PreferenceFR2-2 of a cell group for power saving according to 5.7.4.2 or 5.3.5.3:

2>	include maxBW-PreferenceFR2-2 in the UEAssistanceInformation message;

3>	if the UE prefers to reduce the maximum aggregated bandwidth of FR2-2:

4>	include reducedMaxBW-FR2-2 in the MaxBW-PreferenceFR2-2 IE;

4>	set reducedBW-FR2-2-DL to the maximum aggregated bandwidth the UE desires to have configured across all downlink carriers of FR2-2 in the cell group;

4>	set reducedBW-FR2-2-UL to the maximum aggregated bandwidth the UE desires to have configured across all uplink carriers of FR2-2 in the cell group;

2>	else (if the UE has no preference on the maximum aggregated bandwidth for the cell group):

3>	do not include reducedMaxBW-FR2-2 in the MaxBW-PreferenceFR2-2 IE;

1>	if transmission of the UEAssistanceInformation message is initiated to provide maxCC-Preference of a cell group for power saving according to 5.7.4.2 or 5.3.5.3:

2>	include maxCC-Preference in the UEAssistanceInformation message;

2>	if the UE has a preference on the maximum number of secondary component carriers for the cell group:

3>	include reducedMaxCCs in the MaxCC-Preference IE;

3>	set reducedCCsDL to the number of maximum SCells the UE desires to have configured in downlink in the cell group;

3>	set reducedCCsUL to the number of maximum SCells the UE desires to have configured in uplink in the cell group;

2>	else (if the UE has no preference on the maximum number of secondary component carriers for the cell group):

3>	do not include reducedMaxCCs in the MaxCC-Preference IE;

NOTE 3:	The UE can implicitly indicate a preference for NR SCG release by reporting the maximum aggregated bandwidth preference for power saving of the cell group, if configured, as zero for both FR1 and FR2, and by reporting the maximum number of secondary component carriers for power saving of the cell group, if configured, as zero for both uplink and downlink.

1>	if transmission of the UEAssistanceInformation message is initiated to provide maxMIMO-LayerPreference of a cell group for power saving according to 5.7.4.2 or 5.3.5.3:

2>	include maxMIMO-LayerPreference in the UEAssistanceInformation message;

2>	if the UE has a preference on the maximum number of MIMO layers for the cell group:

3>	if the UE prefers to reduce the number of maximum MIMO layers of each serving cell operating on FR1:

4>	include reducedMaxMIMO-LayersFR1 in the MaxMIMO-LayerPreference IE;

4>	set reducedMIMO-LayersFR1-DL to the preferred maximum number of downlink MIMO layers of each BWP of each FR1 serving cell that the UE operates on in the cell group;

4>	set reducedMIMO-LayersFR1-UL to the preferred maximum number of uplink MIMO layers of each FR1 serving cell that the UE operates on in the cell group;

3>	if the UE prefers to reduce the number of maximum MIMO layers of each serving cell operating on FR2-1:

4>	include reducedMaxMIMO-LayersFR2 in the MaxMIMO-LayerPreference IE;

4>	set reducedMIMO-LayersFR2-DL to the preferred maximum number of downlink MIMO layers of each BWP of each FR2-1 serving cell that the UE operates on in the cell group;

4>	set reducedMIMO-LayersFR2-UL to the preferred maximum number of uplink MIMO layers of each FR2-1 serving cell that the UE operates on in the cell group;

2>	else (if the UE has no preference on the maximum number of MIMO layers for the cell group):

3>	do not include reducedMaxMIMO-LayersFR1 and reducedMaxMIMO-LayersFR2 in the MaxMIMO-LayerPreference IE;

1>	if transmission of the UEAssistanceInformation message is initiated to provide maxMIMO LayerPreferenceFR2 2 of a cell group for power saving according to 5.7.4.2 or 5.3.5.3:

2>	include maxMIMO-LayerPreferenceFR2-2 in the UEAssistanceInformation message;

2>	if the UE has a preference on the maximum number of MIMO layers for the cell group for FR2-2:

3>	if the UE prefers to reduce the number of maximum MIMO layers of each serving cell operating on FR2 2:

4>	include reducedMaxMIMO-LayersFR2-2 in the MaxMIMO-LayerPreferenceFR2 2 IE;

4>	set reducedMIMO-LayersFR2-2-DL to the preferred maximum number of downlink MIMO layers of each BWP of each FR2-2 serving cell that the UE operates on in the cell group;

4>	set reducedMIMO-LayersFR2-2-UL to the preferred maximum number of uplink MIMO layers of each FR2-2 serving cell that the UE operates on in the cell group;

2>	else (if the UE has no preference on the maximum number of MIMO layers for the cell group):

3>	do not include reducedMaxMIMO-LayersFR2-2 in the MaxMIMO-LayerPreferenceFR2-2 IE;

1>	if transmission of the UEAssistanceInformation message is initiated to provide minSchedulingOffsetPreference of a cell group for power saving according to 5.7.4.2 or 5.3.5.3:

2>	include minSchedulingOffsetPreference in the UEAssistanceInformation message;

2>	if the UE has a preference on the minimum scheduling offset for cross-slot scheduling for the cell group:

3>	if the UE has a preference for the value of K0 (TS 38.214 [19], clause 5.1.2.1) for cross-slot scheduling with 15 kHz SCS:

4>	include preferredK0-SCS-15kHz in the MinSchedulingOffsetPreference IE and set it to the desired value of K0;

3>	if the UE has a preference for the value of K0 for cross-slot scheduling with 30 kHz SCS:

4>	include preferredK0-SCS-30kHz in the MinSchedulingOffsetPreference IE and set it to the desired value of K0;

3>	if the UE has a preference for the value of K0 for cross-slot scheduling with 60 kHz SCS:

4>	include preferredK0-SCS-60kHz in the MinSchedulingOffsetPreference IE and set it to the desired value of K0;

3>	if the UE has a preference for the value of K0 for cross-slot scheduling with 120 kHz SCS:

4>	include preferredK0-SCS-120kHz in the MinSchedulingOffsetPreference IE and set it to the desired value of K0;

3>	if the UE has a preference for the value of K2 (TS 38.214 [19], clause 6.1.2.1) for cross-slot scheduling with 15 kHz SCS:

4>	include preferredK2-SCS-15kHz in the MinSchedulingOffsetPreference IE and set it to the desired value of K2;

3>	if the UE has a preference for the value of K2 for cross-slot scheduling with 30 kHz SCS:

4>	include preferredK2-SCS-30kHz in the MinSchedulingOffsetPreference IE and set it to the desired value of K2;

3>	if the UE has a preference for the value of K2 for cross-slot scheduling with 60 kHz SCS:

4>	include preferredK2-SCS-60kHz in the MinSchedulingOffsetPreference IE and set it to the desired value of K2;

3>	if the UE has a preference for the value of K2 for cross-slot scheduling with 120 kHz SCS:

4>	include preferredK2-SCS-120kHz in the MinSchedulingOffsetPreference IE and set it to the desired value of K2;

2>	else (if the UE has no preference on the minimum scheduling offset for cross-slot scheduling for the cell group):

3>	do not include preferredK0 and preferredK2 in the MinSchedulingOffsetPreference IE;

1>	if transmission of the UEAssistanceInformation message is initiated to provide minSchedulingOffsetPreferenceExt of a cell group for power saving according to 5.7.4.2 or 5.3.5.3:

2>	include minSchedulingOffsetPreferenceExt in the UEAssistanceInformation message;

2>	if the UE has a preference on the minimum scheduling offset for cross-slot scheduling for the cell group for FR2-2:

3>	include minSchedulingOffsetPreferenceExt in the UEAssistanceInformation message;

4>	if the UE has a preference for the value of K0 (TS 38.214 [19], clause 5.1.2.1) for cross-slot scheduling with 480 kHz SCS:

5>	include preferredK0-SCS-480kHz in the minSchedulingOffsetPreferenceExt IE and set it to the desired value of K0;

4>	if the UE has a preference for the value of K0 for cross-slot scheduling with 960 kHz SCS:

5>	include preferredK0-SCS-960kHz in the minSchedulingOffsetPreferenceExt IE and set it to the desired value of K0;

4>	if the UE has a preference for the value of K2 for cross-slot scheduling with 480 kHz SCS:

5>	include preferredK2-SCS-480kHz in the minSchedulingOffsetPreferenceExt IE and set it to the desired value of K2;

4>	if the UE has a preference for the value of K2 for cross-slot scheduling with 960 kHz SCS:

5>	include preferredK2-SCS-960kHz in the minSchedulingOffsetPreferenceExt IE and set it to the desired value of K2;

3>	else (if the UE has no preference on the minimum scheduling offset for cross-slot scheduling for the cell group):

4>	do not include preferredK0 and preferredK2 in the minSchedulingOffsetPreferenceExt IE;

1>	if transmission of the UEAssistanceInformation message is initiated to provide a release preference according to 5.7.4.2 or 5.3.5.3:

2>	include releasePreference in the UEAssistanceInformation message;

2>	set preferredRRC-State to the desired RRC state on transmission of the UEAssistanceInformation message;

1>	if transmission of the UEAssistanceInformation message is initiated to provide an indication of preference in being provisioned with reference time information according to 5.7.4.2 or 5.3.5.3:

2>	if the UE has a preference in being provisioned with reference time information:

3>	set referenceTimeInfoPreference to true;

2>	else:

3>	set referenceTimeInfoPreference to false.

1>	if transmission of the UEAssistanceInformation message is initiated to provide preference on FR2 UL gap according to 5.7.4.2 or 5.3.5.3:

2>	if the UE has a preference for FR2 UL gap configuration:

3>	set ul-GapFR2-PatternPreference to the preferred FR2 UL gap pattern;

2>	else (if the UE has no preference for the FR2 UL gap configuration):

3>	do not include ul-GapFR2-PatternPreference in the UL-GapFR2-Preference IE.

1>	if transmission of the UEAssistanceInformation message is initiated to provide MUSIM assistance information according to 5.7.4.2 or 5.3.5.3:

2>	if the UE has a preference for MUSIM periodic gap(s):

3>	include musim-GapPreferenceList with an entry for each periodic gap the UE prefers to be configured;

4>	set musim-GapLength and musim-GapRepetitionAndOffset in the musim-GapInfo IE to the values of the length and the repetition/offset of the gap(s), respectively, the UE prefers to be configured with;

2>	if the UE has a preference for MUSIM aperiodic gap:

3>	include the field musim-GapPreferenceList, with one entry for the aperiodic gap the UE prefers to be configured;

4>	include musim-GapLength in the musim-GapInfo IE and set it to the values of the length of the gap the UE prefers to be configured with;

4>	optionally include musim-Starting-SFN-AndSubframe in the musim-GapInfo IE and set it to the starting SFN/subframe of the gap the UE prefers to be configured with;

2>	if the UE has no longer preference for the periodic/aperiodic gaps:

3>	do not include musim-GapPreferenceList in the musim-Assistance IE;

2>	if UE has a preference to leave RRC_CONNECTED state:

3>	set musim-PreferredRRC-State to the preferred RRC state.

2>	if UE has a preference for MUSIM gap priority:

3>	include the musim-GapPriorityPreferenceList the UE prefers to be configured;

3>	if the UE has preference to keep all collided MUSIM gaps for requested aperiodic and periodic MUSIM gap(s):

4>	include the musim-GapPriorityKeep;

2>	if UE has a preference for temporary capability restriction:

3>	if UE has a preference for serving cell(s) and/or SCG to release:

4>	include the musim-Cell-SCG-ToRelease;

5>	set musim-CellToRelease to include the serving cell(s) the UE prefers to be released ;

5>	set scg-ReleasePreference to scgReleasePreferred if the UE prefers the SCG to be released;

3>	if UE has a preference to indicate the affected capabilities for the serving cells:

4>	include the musim-CellToAffectList the UE prefers not to be configured;

5>	include the musim-ServCellIndex and the musim-MIMO-Layers-DL/ musim-MIMO-Layers-UL for the corresponding serving cell with capability affected;

3>if UE has a preference to indicate the affected band combinations and if there is at least one affected band combination comprising of at least one band that is indicated in musim-candidateBandList:

4> include the musim-AffectededBandCombList the UE prefer to be configured;

5> include the bandEntryIndex for each band for each affected band combination;

5> include the musim-BandToAffectList for the bands that indicated in musim-candidateBandList with affected capability together for the affected band combination;

3>	if UE has a preference to indicate the forbidden band combination and if there is at least one forbidden band combination comprising of at least one band that is indicated in musim-candidateBandList:

4>include the musim-ForbiddenBandCombList the UE prefers not to be configured;

5>	include the musim-ForbiddenBandComb for the corresponding band combinations;

5> include the bandEntryIndex for each band for each forbidden band combination;

3>if UE has a preference for measurement gap requirement:

4> include the musim-NeedForGapsInfoNR to provide the measurement gap requirement information of NR target bands the UE prefer to be configured;

2>	if the UE has no longer preference for temporary capability restriction:

3>	do not include the corresponding temporary capability restriction preference in the musim-CapRestriction in the musim-Assistance IE;

Editor’s note: Detailed signaling on how UE can indicate impacted musim-BandCombinationInfo for the proactive reporting is FFS.

Editor’s note: Detail on the musim-candidateBandList is FFS. E.g. define a new musim-candidateBandList or reuse frequencyBandListFilter in the UECapabilityEnquiry

Editor’s note: The UL/DL supported bandwidth restriction (if supported) shall work for the MUSIM-BandToAffect, and the granularity is FFS.

1>	if transmission of the UEAssistanceInformation message is initiated to provide the relaxation state of RLM measurements of a cell group according to 5.7.4.2:

2>	if the UE performs RLM measurement relaxation on the cell group according to TS 38.133 [14]:

3>	set the rlm-MeasRelaxationState to true;

2>	else:

3>	set the rlm-MeasRelaxationState to false;

1>	if transmission of the UEAssistanceInformation message is initiated to provide the relaxation state of BFD measurements of a cell group:

2>	for each serving cell of the cell group:

3>	if the UE performs BFD measurement relaxation on this serving cell according to TS 38.133 [14]:

4>	set the n-th bit of bfd-MeasRelaxationState to '1', where n is equal to the servCellIndex value + 1 of the serving cell;

3>	else:

4>	set the n-th bit of bfd-MeasRelaxationState to '0', where n is equal to the servCellIndex value + 1 of the serving cell.

1>	if transmission of the UEAssistanceInformation message is initiated to indicate availability of data mapped to radio bearers not configured for SDT according to 5.7.4.2:

2>	include the nonSDT-DataIndication in the UEAssistanceInformation message;

2>	include and set the resumeCause according to the information received from the upper layers, if provided.

1>	if transmission of the UEAssistanceInformation message is initiated to provide an indication of preference for SCG deactivation according to 5.7.4.2:

2>	include scg-DeactivationPreference in the UEAssistanceInformation message;

2>	set the scg-DeactivationPreference to scgDeactivationPreferred if the UE prefers the SCG to be deactivated, otherwise set it to noPreference;

1>	if transmission of the UEAssistanceInformation message is initiated to provide an indication that the UE has uplink data related to a deactivated SCG according to 5.7.4.2:

2>	include uplinkData in the UEAssistanceInformation message.

1>	if transmission of the UEAssistanceInformation message is initiated to provide an indication about whether the criterion for RRM relaxation for connected mode is fulfilled or not fulfilled:

2>	if the criterion for RRM measurement relaxation for connected mode is fulfilled:

3>	set the rrm-MeasRelaxationFulfilment to true;

2>	else:

3>	set the rrm-MeasRelaxationFulfilment to false.

1>	if transmission of the UEAssistanceInformation message is initiated to provide the service link propagation delay difference between serving cell and neighbour cell(s) according to 5.7.4.2;

2>	include the propagationDelayDifference for each neighbour cell in the neighCellInfoList;

The UE shall set the contents of the UEAssistanceInformation message for configured grant assistance information for NR sidelink communication:

1>	if configured to provide configured grant assistance information for NR sidelink communication:

2>	include the sl-UE-AssistanceInformationNR;

NOTE 4:	It is up to UE implementation when and how to trigger configured grant assistance information for NR sidelink communication.

The UE shall:

1>	if the procedure was triggered to provide configured grant assistance information for NR sidelink communication by an NR RRCReconfiguration message that was embedded within an E-UTRA RRCConnectionReconfiguration:

2>	submit the UEAssistanceInformation to lower layers via SRB1, embedded in E-UTRA RRC message ULInformationTransferIRAT as specified in TS 36.331 [10], clause 5.6.28;

1>	else if the procedure was triggered to provide UE preference for SCG deactivation or to indicate that the UE with a deactivate SCG has uplink data to send on a DRB for which there is no MCG RLC bearer:

2>	submit the UEAssistanceInformation via SRB1 to lower layers for transmission;

1>	else if the UE is in (NG)EN-DC:

2>	if SRB3 is configured and the SCG is not deactivated:

3>	submit the UEAssistanceInformation message via SRB3 to lower layers for transmission;

2>	else:

3>	submit the UEAssistanceInformation message via the E-UTRA MCG embedded in E-UTRA RRC message ULInformationTransferMRDC as specified in TS 36.331 [10].

1>	else if the UE is in NR-DC:

2>	if the UE assistance configuration that triggered this UE assistance information is associated with the SCG:

3>	if SRB3 is configured and the SCG is not deactivated:

4>	submit the UEAssistanceInformation message via SRB3 to lower layers for transmission;

3>	else:

4>	submit the UEAssistanceInformation message via the NR MCG embedded in NR RRC message ULInformationTransferMRDC as specified in 5.7.2a.3;

2>	else:

3>	submit the UEAssistanceInformation message via SRB1 to lower layers for transmission;

1>	else:

2>	submit the UEAssistanceInformation message to lower layers for transmission.

# 6	Protocol data units, formats and parameters (ASN.1)

## 6.1	General

### 6.1.1	Introduction

The contents of each RRC message is specified in clause 6.2 using ASN.1 to specify the message syntax and using tables when needed to provide further detailed information about the fields specified in the message syntax. The syntax of the information elements that are defined as stand-alone abstract types is further specified in a similar manner in clause 6.3.

Usage of the text "Network always configures the UE with a value for this field" in the field description indicates that the network has to provide a value for the field in this or in a previous message based on delta configuration (for an optional field with Need M). It does not imply a mandatory presence of the field.

### 6.1.2	Need codes and conditions for optional fields

The need for fields to be present in a message or an abstract type, i.e., the ASN.1 fields that are specified as OPTIONAL in the abstract notation (ASN.1), is specified by means of comment text tags attached to the OPTIONAL statement in the abstract syntax. All comment text tags are available for use in the downlink direction for RRC message and in the sidelink for PC5 RRC message. The meaning of each tag is specified in table 6.1.2-1.

If conditions are used, a conditional presence table is provided for the message or information element specifying the need of the field for each condition case. The table also specifies whether UE maintains or releases the value in case the field is absent. The conditions clarify what the UE may expect regarding the setting of the message by the network for the RRC message or by the peer UE in the sidelink RRC message. Violation of conditions is regarded as invalid network behaviour when transmitting downlink RRC message or invalid UE behavior when transmitting PC5 RRC message, which the UE is not required to cope with. Hence the general error handling defined in 10.4 does not apply in case a field is absent although it is mandatory according to the CondC or CondM condition.

For guidelines on the use of need codes and conditions, see Annex A.6 and A.7.

Table 6.1.2-1: Meaning of abbreviations used to specify the need for fields to be present

| Abbreviation | Meaning |
| --- | --- |
| Cond conditionTag | Conditionally present Presence of the field is specified in a tabular form following the ASN.1 segment. |
| CondC conditionTag | Configuration condition Presence of the field is conditional to other configuration settings. |
| CondM conditionTag | Message condition Presence of the field is conditional to other fields included in the message. |
| Need S | Specified Used for (configuration) fields, whose field description or procedure specifies the UE behavior performed upon receiving a message with the field absent (and not if field description or procedure specifies the UE behavior when field is not configured). |
| Need M | Maintain Used for (configuration) fields that are stored by the UE i.e. not one-shot. Upon receiving a message with the field absent, the UE maintains the current value. |
| Need N | No action (one-shot configuration that is not maintained) Used for (configuration) fields that are not stored and whose presence causes a one-time action by the UE. Upon receiving message with the field absent, the UE takes no action. |
| Need R | Release Used for (configuration) fields that are stored by the UE i.e. not one-shot. Upon receiving a message with the field absent, the UE releases the current value. |

NOTE:	In this version of the specification, the condition tags CondC and CondM are not used.

Any field with Need M or Need N in system information shall be interpreted as Need R.

The need code used within a CondX definition only applies for the case (part of the condition) where it is defined: A condition may have different need codes for different parts of the condition. In particular, the CondX definition may contain the following "otherwise the field is absent" parts:

-	"Otherwise, the field is absent": The field is not relevant or should not be configured when this part of the condition applies. In particular, the UE behaviour is not defined when the field is configured via another part of the condition and is reconfigured to this part of the condition. A need code is not provided when the transition from another part of the condition to this part of the condition is not supported, when the field clearly is a one-shot or there is no difference whether UE maintains or releases the value (e.g., in case the field is mandatory present according to the other part of the condition).

-	"Otherwise, the field is absent, Need R": The field is released if absent when this part of the condition applies. This handles UE behaviour in case the field is configured via another part of the condition and this part of the condition applies (which means that network when transmitting downlink RRC message or peer UE transmitting PC5 RRC message can assume UE releases the field if this part of the condition is valid).

-	"Otherwise, the field is absent, Need M": The UE retains the field if it was already configured when this part of the condition applies. This means the network when transmitting downlink RRC message or the peer UE when transmitting PC5 RRC message cannot release the field, but UE retains the previously configured value.

Use of different Need codes in different parts of a condition should be avoided.

For downlink RRC message and sidelink PC5 RRC messages, the need codes, conditions and ASN.1 defaults specified for a particular (child) field only apply in case the (parent) field including the particular field is present. Thus, if the parent is absent the UE shall not release the field unless the absence of the parent field implies that.

For (parent) fields without need codes in downlink RRC messages or sidelink PC5 RRC message, if the parent field is absent, UE shall follow the need codes of the child fields. Thus, if parent field is absent, the need code of each child field is followed (i.e. Need R child fields are released, Need M child fields are not modified and the actions for Need S child fields depend on the specified conditions of each field). Examples of (parent) fields in downlink RRC messages and sidelink PC5 RRC message without need codes where this rule applies are:

-	nonCriticalExtension fields at the end of a message using empty SEQUENCE extension mechanism,

-	groups of non-critical extensions using double brackets (referred to as extension groups), and

-	non-critical extensions at the end of a message or at the end of a structure, contained in a BIT STRING or OCTET STRING (referred to as parent extension fields).

The handling of need codes as specified in the previous is illustrated by means of an example, as shown in the following ASN.1.

-- /example/ ASN1START

RRCMessage-IEs ::=                SEQUENCE {

field1                            InformationElement1            OPTIONAL,  -- Need M

field2                            InformationElement2            OPTIONAL,  -- Need R

nonCriticalExtension              RRCMessage-v1570-IEs           OPTIONAL

}

RRCMessage-1570-IEs ::=           SEQUENCE {

field3                            InformationElement3            OPTIONAL,  -- Need M

nonCriticalExtension              RRCMessage-v1640-IEs           OPTIONAL

}

RRCMessage-v1640-IEs ::=          SEQUENCE {

field4                            InformationElement4            OPTIONAL,  -- Need R

nonCriticalExtension              SEQUENCE {}                    OPTIONAL

}

InformationElement1 ::=           SEQUENCE {

field11                           InformationElement11           OPTIONAL,  -- Need M

field12                           InformationElement12           OPTIONAL,  -- Need R

...,

[[

field13                           InformationElement13           OPTIONAL,  -- Need R

field14                           InformationElement14           OPTIONAL   -- Need M

]]

}

InformationElement2 ::=           SEQUENCE {

field21                           InformationElement11           OPTIONAL,  -- Need M

...

}

-- ASN1STOP

The handling of need codes as specified in the previous implies that:

-	if field1 in RRCMessage-IEs is absent, UE does not modify any child fields configured within field1 (regardless of their need codes);

-	if field2 in RRCMessage-IEs is absent, UE releases the field2 (and also its child field field21);

-	if field1 or field2 in RRCMessage-IEs is present, UE retains or releases their child fields according to the child field presence conditions;

-	if field1 in RRCMessage-IEs is present but the extension group containing field13 and field14 is absent, the UE releases field13 but does not modify field14;

-	if nonCriticalExtension defined by IE RRCMessage-v1570-IEs is absent, the UE does not modify field3 but releases field4;

### 6.1.3	General rules

In the ASN.1 of this specification, the first bit of a bit string refers to the leftmost bit, unless stated otherwise.

Upon reception of a list not using ToAddModList and ToReleaseList structure, the UE shall delete all entries of the list currently in the UE configuration before applying the received list and shall consider each entry as newly created. This applies also to lists whose size is extended (i.e. with a second list structure in the ASN.1 comprising additional entries), unless otherwise specified. This implies that Need M should not be used for fields in the entries of these lists; if used, UE will handle such fields equivalent to a Need R.

## 6.2	RRC messages

### 6.2.1	General message structure

#### –	NR-RRC-Definitions

This ASN.1 segment is the start of the NR RRC PDU definitions.

-- ASN1START

-- TAG-NR-RRC-DEFINITIONS-START

NR-RRC-Definitions DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

-- TAG-NR-RRC-DEFINITIONS-STOP

-- ASN1STOP

#### –	BCCH-BCH-Message

The BCCH-BCH-Message class is the set of RRC messages that may be sent from the network to the UE via BCH on the BCCH logical channel.

-- ASN1START

-- TAG-BCCH-BCH-MESSAGE-START

BCCH-BCH-Message ::=            SEQUENCE {

message                         BCCH-BCH-MessageType

}

BCCH-BCH-MessageType ::=        CHOICE {

mib                             MIB,

messageClassExtension           SEQUENCE {}

}

-- TAG-BCCH-BCH-MESSAGE-STOP

-- ASN1STOP

#### –	BCCH-DL-SCH-Message

The BCCH-DL-SCH-Message class is the set of RRC messages that may be sent from the network to the UE via DL-SCH on the BCCH logical channel.

-- ASN1START

-- TAG-BCCH-DL-SCH-MESSAGE-START

BCCH-DL-SCH-Message ::=         SEQUENCE {

message                         BCCH-DL-SCH-MessageType

}

BCCH-DL-SCH-MessageType ::=     CHOICE {

c1                              CHOICE {

systemInformation               SystemInformation,

systemInformationBlockType1     SIB1

},

messageClassExtension           SEQUENCE {}

}

-- TAG-BCCH-DL-SCH-MESSAGE-STOP

-- ASN1STOP

#### –	DL-CCCH-Message

The DL-CCCH-Message class is the set of RRC messages that may be sent from the Network to the UE on the downlink CCCH logical channel.

-- ASN1START

-- TAG-DL-CCCH-MESSAGE-START

DL-CCCH-Message ::=             SEQUENCE {

message                         DL-CCCH-MessageType

}

DL-CCCH-MessageType ::=         CHOICE {

c1                              CHOICE {

rrcReject                       RRCReject,

rrcSetup                        RRCSetup,

spare2                          NULL,

spare1                          NULL

},

messageClassExtension           SEQUENCE {}

}

-- TAG-DL-CCCH-MESSAGE-STOP

-- ASN1STOP

#### –	DL-DCCH-Message

The DL-DCCH-Message class is the set of RRC messages that may be sent from the network to the UE on the downlink DCCH logical channel.

-- ASN1START

-- TAG-DL-DCCH-MESSAGE-START

DL-DCCH-Message ::=                  SEQUENCE {

message                             DL-DCCH-MessageType

}

DL-DCCH-MessageType ::=             CHOICE {

c1                                  CHOICE {

rrcReconfiguration                  RRCReconfiguration,

rrcResume                           RRCResume,

rrcRelease                          RRCRelease,

rrcReestablishment                  RRCReestablishment,

securityModeCommand                 SecurityModeCommand,

dlInformationTransfer               DLInformationTransfer,

ueCapabilityEnquiry                 UECapabilityEnquiry,

counterCheck                        CounterCheck,

mobilityFromNRCommand               MobilityFromNRCommand,

dlDedicatedMessageSegment-r16       DLDedicatedMessageSegment-r16,

ueInformationRequest-r16            UEInformationRequest-r16,

dlInformationTransferMRDC-r16       DLInformationTransferMRDC-r16,

loggedMeasurementConfiguration-r16  LoggedMeasurementConfiguration-r16,

spare3 NULL, spare2 NULL, spare1 NULL

},

messageClassExtension   SEQUENCE {}

}

-- TAG-DL-DCCH-MESSAGE-STOP

-- ASN1STOP

#### –	MCCH-Message

The MCCH-Message class is the set of RRC messages that may be sent from the network to the UE on the MCCH logical channel.

-- ASN1START

-- TAG-MCCH-MESSAGE-START

MCCH-Message-r17 ::= SEQUENCE {

message              MCCH-MessageType-r17

}

MCCH-MessageType-r17 ::= CHOICE {

c1                       CHOICE {

mbsBroadcastConfiguration-r17     MBSBroadcastConfiguration-r17,

spare1                            NULL

},

messageClassExtension   SEQUENCE {}

}

-- TAG-MCCH-MESSAGE-STOP

-- ASN1STOP

#### –	PCCH-Message

The PCCH-Message class is the set of RRC messages that may be sent from the Network to the UE on the PCCH logical channel.

-- ASN1START

-- TAG-PCCH-PCH-MESSAGE-START

PCCH-Message ::=                SEQUENCE {

message                         PCCH-MessageType

}

PCCH-MessageType ::=            CHOICE {

c1                              CHOICE {

paging                          Paging,

spare1  NULL

},

messageClassExtension       SEQUENCE {}

}

-- TAG-PCCH-PCH-MESSAGE-STOP

-- ASN1STOP

#### –	UL-CCCH-Message

The UL-CCCH-Message class is the set of 48-bits RRC messages that may be sent from the UE to the Network on the uplink CCCH logical channel.

-- ASN1START

-- TAG-UL-CCCH-MESSAGE-START

UL-CCCH-Message ::=             SEQUENCE {

message                         UL-CCCH-MessageType

}

UL-CCCH-MessageType ::=         CHOICE {

c1                              CHOICE {

rrcSetupRequest                 RRCSetupRequest,

rrcResumeRequest                RRCResumeRequest,

rrcReestablishmentRequest       RRCReestablishmentRequest,

rrcSystemInfoRequest            RRCSystemInfoRequest

},

messageClassExtension           SEQUENCE {}

}

-- TAG-UL-CCCH-MESSAGE-STOP

-- ASN1STOP

#### –	UL-CCCH1-Message

The UL-CCCH1-Message class is the set of 64-bits RRC messages that may be sent from the UE to the Network on the uplink CCCH1 logical channel.

-- ASN1START

-- TAG-UL-CCCH1-MESSAGE-START

UL-CCCH1-Message ::=            SEQUENCE {

message                         UL-CCCH1-MessageType

}

UL-CCCH1-MessageType ::=        CHOICE {

c1                              CHOICE {

rrcResumeRequest1               RRCResumeRequest1,

spare3 NULL,

spare2 NULL,

spare1 NULL

},

messageClassExtension SEQUENCE {}

}

-- TAG-UL-CCCH1-MESSAGE-STOP

-- ASN1STOP

#### –	UL-DCCH-Message

The UL-DCCH-Message class is the set of RRC messages that may be sent from the UE to the network on the uplink DCCH logical channel.

-- ASN1START

-- TAG-UL-DCCH-MESSAGE-START

UL-DCCH-Message ::=             SEQUENCE {

message                         UL-DCCH-MessageType

}

UL-DCCH-MessageType ::=         CHOICE {

c1                              CHOICE {

measurementReport               MeasurementReport,

rrcReconfigurationComplete      RRCReconfigurationComplete,

rrcSetupComplete                RRCSetupComplete,

rrcReestablishmentComplete      RRCReestablishmentComplete,

rrcResumeComplete               RRCResumeComplete,

securityModeComplete            SecurityModeComplete,

securityModeFailure             SecurityModeFailure,

ulInformationTransfer           ULInformationTransfer,

locationMeasurementIndication   LocationMeasurementIndication,

ueCapabilityInformation         UECapabilityInformation,

counterCheckResponse            CounterCheckResponse,

ueAssistanceInformation         UEAssistanceInformation,

failureInformation              FailureInformation,

ulInformationTransferMRDC       ULInformationTransferMRDC,

scgFailureInformation           SCGFailureInformation,

scgFailureInformationEUTRA      SCGFailureInformationEUTRA

},

messageClassExtension           CHOICE {

c2                              CHOICE {

ulDedicatedMessageSegment-r16    ULDedicatedMessageSegment-r16,

dedicatedSIBRequest-r16         DedicatedSIBRequest-r16,

mcgFailureInformation-r16       MCGFailureInformation-r16,

ueInformationResponse-r16       UEInformationResponse-r16,

sidelinkUEInformationNR-r16     SidelinkUEInformationNR-r16,

ulInformationTransferIRAT-r16   ULInformationTransferIRAT-r16,

iabOtherInformation-r16         IABOtherInformation-r16,

mbsInterestIndication-r17       MBSInterestIndication-r17,

uePositioningAssistanceInfo-r17 UEPositioningAssistanceInfo-r17,

measurementReportAppLayer-r17   MeasurementReportAppLayer-r17,

spare6 NULL, spare5 NULL, spare4 NULL, spare3 NULL, spare2 NULL, spare1 NULL

},

messageClassExtensionFuture-r16    SEQUENCE {}

}

}

-- TAG-UL-DCCH-MESSAGE-STOP

-- ASN1STOP

### 6.2.2	Message definitions

---------------------------------------------------------Skip Unchanged----------------------------------------------------------

#### –	MeasurementReport

The MeasurementReport message is used for the indication of measurement results.

Signalling radio bearer: SRB1, SRB3

RLC-SAP: AM

Logical channel: DCCH

Direction: UE to Network

MeasurementReport message

-- ASN1START

-- TAG-MEASUREMENTREPORT-START

MeasurementReport ::=               SEQUENCE {

criticalExtensions                  CHOICE {

measurementReport                   MeasurementReport-IEs,

criticalExtensionsFuture            SEQUENCE {}

}

}

MeasurementReport-IEs ::=           SEQUENCE {

measResults                         MeasResults,

lateNonCriticalExtension                OCTET STRING                                                            OPTIONAL,

nonCriticalExtension                    SEQUENCE{}                                                              OPTIONAL

}

-- TAG-MEASUREMENTREPORT-STOP

-- ASN1STOP

---------------------------------------------------------Skip Unchanged----------------------------------------------------------

#### –	Paging

The Paging message is used for the notification of one or more UEs.

Signalling radio bearer: N/A

RLC-SAP: TM

Logical channel: PCCH

Direction: Network to UE

Paging message

-- ASN1START

-- TAG-PAGING-START

Paging ::=                          SEQUENCE {

pagingRecordList                    PagingRecordList                                                        OPTIONAL, -- Need N

lateNonCriticalExtension            OCTET STRING                                                            OPTIONAL,

nonCriticalExtension                Paging-v1700-IEs                                                        OPTIONAL

}

Paging-v1700-IEs ::=                SEQUENCE {

pagingRecordList-v1700              PagingRecordList-v1700                                                  OPTIONAL, -- Need N

pagingGroupList-r17                 PagingGroupList-r17                                                     OPTIONAL, -- Need N

nonCriticalExtension                SEQUENCE {}                                                             OPTIONAL

}

PagingRecordList ::=                SEQUENCE (SIZE(1..maxNrofPageRec)) OF PagingRecord

PagingRecordList-v1700 ::=          SEQUENCE (SIZE(1..maxNrofPageRec)) OF PagingRecord-v1700

PagingGroupList-r17 ::=             SEQUENCE (SIZE(1..maxNrofPageGroup-r17)) OF TMGI-r17

PagingRecord ::=                    SEQUENCE {

ue-Identity                         PagingUE-Identity,

accessType                          ENUMERATED {non3GPP}    OPTIONAL,   -- Need N

...

}

PagingRecord-v1700 ::=              SEQUENCE {

pagingCause-r17                     ENUMERATED {voice}      OPTIONAL    -- Need N

}

PagingUE-Identity ::=               CHOICE {

ng-5G-S-TMSI                        NG-5G-S-TMSI,

fullI-RNTI                          I-RNTI-Value,

...

}

-- TAG-PAGING-STOP

-- ASN1STOP

PagingRecord field descriptions

accessType Indicates whether the Paging message is originated due to the PDU sessions from the non-3GPP access.

pagingRecordList If the network includes pagingRecordList-v1700, it includes the same number of entries, and listed in the same order, as in pagingRecordList (i.e. without suffix).

pagingCause Indicates whether the Paging message is originated due to IMS voice. If this field is present, it implies that the corresponding paging entry is for IMS voice. If upper layers indicate the support of paging cause and if this field is not present but pagingRecordList-v1700 is present, it implies that the corresponding paging entry is for a service other than IMS voice. Otherwise, paging cause is undetermined.

#### –	RRCReestablishment

The RRCReestablishment message is used to re-establish SRB1.

Signalling radio bearer: SRB1

RLC-SAP: AM

Logical channel: DCCH

Direction: Network to UE

RRCReestablishment message

-- ASN1START

-- TAG-RRCREESTABLISHMENT-START

RRCReestablishment ::=              SEQUENCE {

rrc-TransactionIdentifier           RRC-TransactionIdentifier,

criticalExtensions                  CHOICE {

rrcReestablishment                  RRCReestablishment-IEs,

criticalExtensionsFuture            SEQUENCE {}

}

}

RRCReestablishment-IEs ::=          SEQUENCE {

nextHopChainingCount                NextHopChainingCount,

lateNonCriticalExtension            OCTET STRING                        OPTIONAL,

nonCriticalExtension                RRCReestablishment-v1700-IEs         OPTIONAL

}

RRCReestablishment-v1700-IEs ::=    SEQUENCE {

sl-L2RemoteUE-Config-r17            SetupRelease {SL-L2RemoteUE-Config-r17}    OPTIONAL, -- Cond L2RemoteUE

nonCriticalExtension                SEQUENCE {}                                OPTIONAL

}

-- TAG-RRCREESTABLISHMENT-STOP

-- ASN1STOP

RRCReestablishment-IEs field descriptions

sl-L2RemoteUE-Config Contains dedicated configurations used for L2 U2N relay related operation. The network configures only the SRAP configuration used for the SRB1 and local UE ID.

| Conditional Presence | Explanation |
| --- | --- |
| L2RemoteUE | The field is mandatory present for L2 U2N Remote UE; otherwise it is absent. |

#### –	RRCReestablishmentComplete

The RRCReestablishmentComplete message is used to confirm the successful completion of an RRC connection re-establishment.

Signalling radio bearer: SRB1

RLC-SAP: AM

Logical channel: DCCH

Direction: UE to Network

RRCReestablishmentComplete message

-- ASN1START

-- TAG-RRCREESTABLISHMENTCOMPLETE-START

RRCReestablishmentComplete ::=              SEQUENCE {

rrc-TransactionIdentifier                   RRC-TransactionIdentifier,

criticalExtensions                          CHOICE {

rrcReestablishmentComplete                  RRCReestablishmentComplete-IEs,

criticalExtensionsFuture                    SEQUENCE {}

}

}

RRCReestablishmentComplete-IEs ::=          SEQUENCE {

lateNonCriticalExtension                    OCTET STRING                            OPTIONAL,

nonCriticalExtension                        RRCReestablishmentComplete-v1610-IEs    OPTIONAL

}

RRCReestablishmentComplete-v1610-IEs ::=    SEQUENCE {

ue-MeasurementsAvailable-r16                UE-MeasurementsAvailable-r16    OPTIONAL,

nonCriticalExtension                        SEQUENCE {}                     OPTIONAL

}

-- TAG-RRCREESTABLISHMENTCOMPLETE-STOP

-- ASN1STOP

#### –	RRCReestablishmentRequest

The RRCReestablishmentRequest message is used to request the reestablishment of an RRC connection.

Signalling radio bearer: SRB0

RLC-SAP: TM

Logical channel: CCCH

Direction: UE to Network

RRCReestablishmentRequest message

-- ASN1START

-- TAG-RRCREESTABLISHMENTREQUEST-START

RRCReestablishmentRequest ::=       SEQUENCE {

rrcReestablishmentRequest           RRCReestablishmentRequest-IEs

}

RRCReestablishmentRequest-IEs ::=   SEQUENCE {

ue-Identity                         ReestabUE-Identity,

reestablishmentCause                ReestablishmentCause,

spare                               BIT STRING (SIZE (1))

}

ReestabUE-Identity ::=              SEQUENCE {

c-RNTI                              RNTI-Value,

physCellId                          PhysCellId,

shortMAC-I                          ShortMAC-I

}

ReestablishmentCause ::=            ENUMERATED {reconfigurationFailure, handoverFailure, otherFailure, spare1}

-- TAG-RRCREESTABLISHMENTREQUEST-STOP

-- ASN1STOP

ReestabUE-Identity field descriptions

physCellId The Physical Cell Identity of the PCell the UE was connected to prior to the failure.

RRCReestablishmentRequest-IEs field descriptions

reestablishmentCause Indicates the failure cause that triggered the re-establishment procedure. gNB is not expected to reject a RRCReestablishmentRequest due to unknown cause value being used by the UE.

ue-Identity UE identity included to retrieve UE context and to facilitate contention resolution by lower layers.

#### –	RRCReconfiguration

The RRCReconfiguration message is the command to modify an RRC connection. It may convey information for measurement configuration, mobility control, radio resource configuration (including RBs, MAC main configuration and physical channel configuration) and AS security configuration.

Signalling radio bearer: SRB1 or SRB3

RLC-SAP: AM

Logical channel: DCCH

Direction: Network to UE

RRCReconfiguration message

-- ASN1START

-- TAG-RRCRECONFIGURATION-START

RRCReconfiguration ::=                  SEQUENCE {

rrc-TransactionIdentifier               RRC-TransactionIdentifier,

criticalExtensions                      CHOICE {

rrcReconfiguration                      RRCReconfiguration-IEs,

criticalExtensionsFuture                SEQUENCE {}

}

}

RRCReconfiguration-IEs ::=              SEQUENCE {

radioBearerConfig                       RadioBearerConfig                                                      OPTIONAL, -- Need M

secondaryCellGroup                      OCTET STRING (CONTAINING CellGroupConfig)                              OPTIONAL, -- Cond SCG

measConfig                              MeasConfig                                                             OPTIONAL, -- Need M

lateNonCriticalExtension                OCTET STRING                                                           OPTIONAL,

nonCriticalExtension                    RRCReconfiguration-v1530-IEs                                           OPTIONAL

}

RRCReconfiguration-v1530-IEs ::=            SEQUENCE {

masterCellGroup                         OCTET STRING (CONTAINING CellGroupConfig)                              OPTIONAL, -- Need M

fullConfig                              ENUMERATED {true}                                                      OPTIONAL, -- Cond FullConfig

dedicatedNAS-MessageList                SEQUENCE (SIZE(1..maxDRB)) OF DedicatedNAS-Message                     OPTIONAL, -- Cond nonHO

masterKeyUpdate                         MasterKeyUpdate                                                        OPTIONAL, -- Cond MasterKeyChange

dedicatedSIB1-Delivery                  OCTET STRING (CONTAINING SIB1)                                         OPTIONAL, -- Need N

dedicatedSystemInformationDelivery      OCTET STRING (CONTAINING SystemInformation)                            OPTIONAL, -- Need N

otherConfig                             OtherConfig                                                            OPTIONAL, -- Need M

nonCriticalExtension                    RRCReconfiguration-v1540-IEs                                           OPTIONAL

}

RRCReconfiguration-v1540-IEs ::=        SEQUENCE {

otherConfig-v1540                       OtherConfig-v1540                                                      OPTIONAL, -- Need M

nonCriticalExtension                    RRCReconfiguration-v1560-IEs                                           OPTIONAL

}

RRCReconfiguration-v1560-IEs ::=         SEQUENCE {

mrdc-SecondaryCellGroupConfig            SetupRelease { MRDC-SecondaryCellGroupConfig }                        OPTIONAL,   -- Need M

radioBearerConfig2                       OCTET STRING (CONTAINING RadioBearerConfig)                           OPTIONAL,   -- Need M

sk-Counter                               SK-Counter                                                            OPTIONAL,   -- Need N

nonCriticalExtension                     RRCReconfiguration-v1610-IEs                                          OPTIONAL

}

RRCReconfiguration-v1610-IEs ::=        SEQUENCE {

otherConfig-v1610                       OtherConfig-v1610                                                    OPTIONAL, -- Need M

bap-Config-r16                          SetupRelease { BAP-Config-r16 }                                      OPTIONAL, -- Need M

iab-IP-AddressConfigurationList-r16     IAB-IP-AddressConfigurationList-r16                                  OPTIONAL, -- Need M

conditionalReconfiguration-r16          ConditionalReconfiguration-r16                                       OPTIONAL, -- Need M

daps-SourceRelease-r16                  ENUMERATED{true}                                                     OPTIONAL, -- Need N

t316-r16                                SetupRelease {T316-r16}                                              OPTIONAL, -- Need M

needForGapsConfigNR-r16                 SetupRelease {NeedForGapsConfigNR-r16}                               OPTIONAL, -- Need M

onDemandSIB-Request-r16                 SetupRelease { OnDemandSIB-Request-r16 }                             OPTIONAL, -- Need M

dedicatedPosSysInfoDelivery-r16         OCTET STRING (CONTAINING PosSystemInformation-r16-IEs)               OPTIONAL, -- Need N

sl-ConfigDedicatedNR-r16                SetupRelease {SL-ConfigDedicatedNR-r16}                              OPTIONAL, -- Need M

sl-ConfigDedicatedEUTRA-Info-r16        SetupRelease {SL-ConfigDedicatedEUTRA-Info-r16}                      OPTIONAL, -- Need M

targetCellSMTC-SCG-r16                  SSB-MTC                                                              OPTIONAL, -- Need S

nonCriticalExtension                    RRCReconfiguration-v1700-IEs                                         OPTIONAL

}

RRCReconfiguration-v1700-IEs ::=        SEQUENCE {

otherConfig-v1700                       OtherConfig-v1700                                              OPTIONAL, -- Need M

sl-L2RelayUE-Config-r17                 SetupRelease { SL-L2RelayUE-Config-r17 }                       OPTIONAL, -- Need M

sl-L2RemoteUE-Config-r17                SetupRelease { SL-L2RemoteUE-Config-r17 }                      OPTIONAL, -- Need M

dedicatedPagingDelivery-r17             OCTET STRING (CONTAINING Paging)                               OPTIONAL, -- Cond PagingRelay

needForGapNCSG-ConfigNR-r17             SetupRelease {NeedForGapNCSG-ConfigNR-r17}                     OPTIONAL, -- Need M

needForGapNCSG-ConfigEUTRA-r17          SetupRelease {NeedForGapNCSG-ConfigEUTRA-r17}                  OPTIONAL, -- Need M

musim-GapConfig-r17                     SetupRelease {MUSIM-GapConfig-r17}                             OPTIONAL, -- Need M

ul-GapFR2-Config-r17                    SetupRelease { UL-GapFR2-Config-r17 }                          OPTIONAL, -- Need M

scg-State-r17                           ENUMERATED { deactivated }                                     OPTIONAL, -- Need N

appLayerMeasConfig-r17                  AppLayerMeasConfig-r17                                         OPTIONAL, -- Need M

ue-TxTEG-RequestUL-TDOA-Config-r17      SetupRelease {UE-TxTEG-RequestUL-TDOA-Config-r17}              OPTIONAL,  -- Need M

nonCriticalExtension                    RRCReconfiguration-v18xy-IEs                                    OPTIONAL

}

RRCReconfiguration-v18xy-IEs ::=        SEQUENCE {

otherConfig-v18xy                       OtherConfig-v18xy                                              OPTIONAL, -- Need M

nonCriticalExtension                    SEQUENCE {}                                                    OPTIONAL

}

MRDC-SecondaryCellGroupConfig ::=       SEQUENCE {

mrdc-ReleaseAndAdd                      ENUMERATED {true}                                                     OPTIONAL,   -- Need N

mrdc-SecondaryCellGroup                 CHOICE {

nr-SCG                                  OCTET STRING  (CONTAINING RRCReconfiguration),

eutra-SCG                               OCTET STRING

}

}

BAP-Config-r16 ::=                      SEQUENCE {

bap-Address-r16                         BIT STRING (SIZE (10))                                    OPTIONAL, -- Need M

defaultUL-BAP-RoutingID-r16             BAP-RoutingID-r16                                         OPTIONAL, -- Need M

defaultUL-BH-RLC-Channel-r16            BH-RLC-ChannelID-r16                                      OPTIONAL, -- Need M

flowControlFeedbackType-r16             ENUMERATED {perBH-RLC-Channel, perRoutingID, both}        OPTIONAL, -- Need R

...

}

MasterKeyUpdate ::=                 SEQUENCE {

keySetChangeIndicator           BOOLEAN,

nextHopChainingCount            NextHopChainingCount,

nas-Container                   OCTET STRING                                                     OPTIONAL,    -- Cond securityNASC

...

}

OnDemandSIB-Request-r16 ::=                  SEQUENCE {

onDemandSIB-RequestProhibitTimer-r16         ENUMERATED {s0, s0dot5, s1, s2, s5, s10, s20, s30}

}

T316-r16 ::=         ENUMERATED {ms50, ms100, ms200, ms300, ms400, ms500, ms600, ms1000, ms1500, ms2000}

IAB-IP-AddressConfigurationList-r16 ::= SEQUENCE {

iab-IP-AddressToAddModList-r16      SEQUENCE (SIZE(1..maxIAB-IP-Address-r16)) OF IAB-IP-AddressConfiguration-r16 OPTIONAL, -- Need N

iab-IP-AddressToReleaseList-r16     SEQUENCE (SIZE(1..maxIAB-IP-Address-r16)) OF IAB-IP-AddressIndex-r16         OPTIONAL, -- Need N

...

}

IAB-IP-AddressConfiguration-r16 ::=     SEQUENCE {

iab-IP-AddressIndex-r16                 IAB-IP-AddressIndex-r16,

iab-IP-Address-r16                      IAB-IP-Address-r16                                                OPTIONAL,  -- Need M

iab-IP-Usage-r16                        IAB-IP-Usage-r16                                                  OPTIONAL,  -- Need M

iab-donor-DU-BAP-Address-r16            BIT STRING (SIZE(10))                                             OPTIONAL,  -- Need M

...

}

SL-ConfigDedicatedEUTRA-Info-r16 ::=            SEQUENCE {

sl-ConfigDedicatedEUTRA-r16                    OCTET STRING                                              OPTIONAL,  -- Need M

sl-TimeOffsetEUTRA-List-r16                    SEQUENCE (SIZE (8)) OF SL-TimeOffsetEUTRA-r16             OPTIONAL    -- Need M

}

SL-TimeOffsetEUTRA-r16 ::=        ENUMERATED {ms0, ms0dot25, ms0dot5, ms0dot625, ms0dot75, ms1, ms1dot25, ms1dot5, ms1dot75,

ms2, ms2dot5, ms3, ms4, ms5, ms6, ms8, ms10, ms20}

UE-TxTEG-RequestUL-TDOA-Config-r17 ::=  CHOICE {

oneShot-r17                             NULL,

periodicReporting-r17                   ENUMERATED { ms160, ms320, ms1280, ms2560, ms61440, ms81920, ms368640, ms737280 }

}

-- TAG-RRCRECONFIGURATION-STOP

-- ASN1STOP

RRCReconfiguration-IEs field descriptions

appLayerMeasConfig This field is used to configure application layer measurements. This field is absent when the UE is configured to operate with shared spectrum channel access or if sl-L2RemoteUE-Config-r17 is configured or not released.

bap-Config This field is used to configure the BAP entity for IAB nodes.

bap-Address Indicates the BAP address of an IAB-node. The BAP address of an IAB-node cannot be changed once configured for the cell group to the BAP entity.

conditionalReconfiguration Configuration of candidate target SpCell(s) and execution condition(s) for conditional handover, conditional PSCell addition or conditional PSCell change. The field is absent if any DAPS bearer is configured or if the masterCellGroup includes ReconfigurationWithSync or if the sl-L2RemoteUE-Config or sl-L2RelayUE-Config is configured. For conditional PSCell change, the field is absent if the secondaryCellGroup includes ReconfigurationWithSync. The RRCReconfiguration message contained in DLInformationTransferMRDC cannot contain the field conditionalReconfiguration for conditional PSCell change or for conditional PSCell addition.

daps-SourceRelease Indicates to UE that the source cell part of DAPS operation is to be stopped and the source cell part of DAPS configuration is to be released.

dedicatedNAS-MessageList This field is used to transfer UE specific NAS layer information between the network and the UE. The RRC layer is transparent for each PDU in the list.

dedicatedPagingDelivery This field is used to transfer Paging message for the associated L2 U2N Remote UE to the L2 U2N Relay UE in RRC_CONNECTED.

dedicatedPosSysInfoDelivery This field is used to transfer SIBPos to the UE in RRC_CONNECTED.

dedicatedSIB1-Delivery This field is used to transfer SIB1 to the UE (including L2 U2N Remote UE). The field has the same values as the corresponding configuration in servingCellConfigCommon.

dedicatedSystemInformationDelivery This field is used to transfer SIB6, SIB7, SIB8, SIB19, SIB21 to the UE with an active BWP with no common search space configured or the L2 U2N Remote UE in RRC_CONNECTED. For UEs in RRC_CONNECTED (including L2 U2N Remote UE), this field is also used to transfer the SIBs requested on-demand.

defaultUL-BAP-RoutingID This field is used for IAB-node to configure the default uplink Routing ID, which is used by IAB-node during IAB-node bootstrapping, migration, IAB-MT RRC resume and IAB-MT RRC re-establishment for F1-C and non-F1 traffic. The defaultUL-BAP-RoutingID can be (re-)configured when IAB-node IP address for F1-C related traffic changes. This field is mandatory only for IAB-node bootstrapping.

defaultUL-BH-RLC-Channel This field is used for IAB-nodes to configure the default uplink BH RLC channel, which is used by IAB-node during IAB-node bootstrapping, migration, IAB-MT RRC resume and IAB-MT RRC re-establishment for F1-C and non-F1 traffic. The defaultUL-BH-RLC-Channel can be (re-)configured when IAB-node IP address for F1-C related traffic changes, and the new IP address is anchored at a different IAB-donor-DU. This field is mandatory for IAB-node bootstrapping. If the IAB-MT is operating in EN-DC, the default uplink BH RLC channel is referring to an RLC channel on the SCG; Otherwise, it is referring to an RLC channel either on the MCG or on the SCG depending on whether the MN or the SN configures this field.

flowControlFeedbackType This field is only used for IAB-node that support hop-by-hop flow control to configure the type of flow control feedback. Value perBH-RLC-Channel indicates that the IAB-node shall provide flow control feedback per BH RLC channel, value perRoutingID indicates that the IAB-node shall provide flow control feedback per routing ID, and value both indicates that the IAB-node shall provide flow control feedback both per BH RLC channel and per routing ID.

fullConfig Indicates that the full configuration option is applicable for the RRCReconfiguration message for intra-system intra-RAT HO. For inter-RAT HO from E-UTRA to NR, fullConfig indicates whether or not delta signalling of SDAP/PDCP from source RAT is applicable. This field is absent if any DAPS bearer is configured or when the RRCReconfiguration message is transmitted on SRB3, and in an RRCReconfiguration message for SCG contained in another RRCReconfiguration message (or RRCConnectionReconfiguration message, see TS 36.331 [10]) transmitted on SRB1.

iab-IP-Address This field is used to provide the IP address information for IAB-node.

iab-IP-AddressIndex This field is used to identify a configuration of an IP address.

iab-IP-AddressToAddModList List of IP addresses allocated for IAB-node to be added and modified.

iab-IP-AddressToReleaseList List of IP address allocated for IAB-node to be released.

iab-IP-Usage This field is used to indicate the usage of the assigned IP address. If this field is not configured, the assigned IP address is used for all traffic.

iab-donor-DU-BAP-Address This field is used to indicate the BAP address of the IAB-donor-DU where the IP address is anchored.

keySetChangeIndicator Indicates whether UE shall derive a new KgNB. If reconfigurationWithSync is included, value true indicates that a KgNB key is derived from a KAMF key taken into use through the latest successful NAS SMC procedure, or N2 handover procedure with KAMF change, as described in TS 33.501 [11] for KgNB re-keying. Value false indicates that the new KgNB key is obtained from the current KgNB key or from the NH as described in TS 33.501 [11].

masterCellGroup Configuration of master cell group.

mrdc-ReleaseAndAdd This field indicates that the current SCG configuration is released and a new SCG is added at the same time.

mrdc-SecondaryCellGroup Includes an RRC message for SCG configuration in NR-DC or NE-DC. For NR-DC (nr-SCG), mrdc-SecondaryCellGroup contains the RRCReconfiguration message as generated (entirely) by SN gNB. In this version of the specification, the RRC message can only include fields secondaryCellGroup, otherConfig, conditionalReconfiguration, measConfig, bap-Config and IAB-IP-AddressConfigurationList. For NE-DC (eutra-SCG), mrdc-SecondaryCellGroup includes the E-UTRA RRCConnectionReconfiguration message as specified in TS 36.331 [10]. In this version of the specification, the E-UTRA RRC message can only include the field scg-Configuration.

musim-GapConfig Indicates the MUSIM gap configuration and controls setup/release of MUSIM gaps. In this version of the specification, the network does not configure MUSIM gap together with concurrent measurement gap or preconfigured measurement gap for positioning.

nas-Container This field is used to transfer UE specific NAS layer information between the network and the UE. The RRC layer is transparent for this field, although it affects activation of AS security after inter-system handover to NR. The content is defined in TS 24.501 [23].

needForGapsConfigNR Configuration for the UE to report measurement gap requirement information of NR target bands in the RRCReconfigurationComplete and RRCResumeComplete message.

needForGapNCSG-ConfigEUTRA Configuration for the UE to report measurement gap and NCSG requirement information of E-UTRA target bands in the RRCReconfigurationComplete and RRCResumeComplete message.

needForGapNCSG-ConfigNR Configuration for the UE to report measurement gap and NCSG requirement information of NR target bands in the RRCReconfigurationComplete and RRCResumeComplete message.

nextHopChainingCount Parameter NCC: See TS 33.501 [11]

onDemandSIB-Request If the field is present, the UE is allowed to request SIB(s) on-demand while in RRC_CONNECTED according to clause 5.2.2.3.5.

onDemandSIB-RequestProhibitTimer Prohibit timer for requesting SIB(s) on-demand while in RRC_CONNECTED according to clause 5.2.2.3.5. Value in seconds. Value s0 means prohibit timer is set to 0 seconds, value s0dot5 means prohibit timer is set to 0.5 seconds, value s1 means prohibit timer is set to 1 second and so on.

otherConfig Contains configuration related to other configurations. When configured for the SCG, only fields drx-PreferenceConfig, maxBW-PreferenceConfig, maxBW-PreferenceConfigFR2-2, maxCC-PreferenceConfig, maxMIMO-LayerPreferenceConfig, maxMIMO-LayerPreferenceConfigFR2-2, minSchedulingOffsetPreferenceConfig, minSchedulingOffsetPreferenceConfigExt, rlm-RelaxationReportingConfig, bfd-RelaxationReportingConfig, btNameList, wlanNameList, sensorNameList and obtainCommonLocation can be included.

radioBearerConfig Configuration of Radio Bearers (DRBs, SRBs, multicast MRBs) including SDAP/PDCP. In (NG)EN-DC this field may only be present if the RRCReconfiguration is transmitted over SRB3. SRB4 should not be configured if sl-L2RemoteUE-Config-r17 is configured or not released.

radioBearerConfig2 Configuration of Radio Bearers (DRBs, SRBs) including SDAP/PDCP. This field can only be used if the UE supports NR-DC or NE-DC.

scg-State Indicates that the SCG is in deactivated state. This field is not used - in an RRCReconfiguration message received: - within mrdc-SecondaryCellGroup, or - in an E-UTRA RRCConnectionReconfiguration message, or - in an E-UTRA RRCConnectionResume message or - in an RRCReconfiguration message received via SRB3, except if the RRCReconfiguration message is included in DLInformationTransferMRDC. The field is absent if CPA or CPC is configured for the UE, or if the RRCReconfiguration message is contained in CondRRCReconfig.

sl-L2RelayUE-Config Contains L2 U2N relay operation related configurations used by a UE acting as or to be acting as a L2 U2N Relay UE. The field is absent if conditionalReconfiguration is configured for CHO.

sl-L2RemoteUE-Config Contains L2 U2N relay operation related configurations used by a UE acting as or to be acting as a L2 U2N Remote UE. The field is absent if conditionalReconfiguration is configured for CHO, or if appLayerMeasConfig or SRB4 is configured/not released.

secondaryCellGroup Configuration of secondary cell group ((NG)EN-DC or NR-DC).

sk-Counter A counter used upon initial configuration of S-KgNB or S-KeNB, as well as upon refresh of S-KgNB or S-KeNB. This field is always included either upon initial configuration of an NR SCG or upon configuration of the first RB with keyToUse set to secondary, whichever happens first. This field is absent if there is neither any NR SCG nor any RB with keyToUse set to secondary.

sl-ConfigDedicatedNR This field is used to provide the dedicated configurations for NR sidelink communication/discovery.

sl-ConfigDedicatedEUTRA-Info This field includes the E-UTRA RRCConnectionReconfiguration as specified in TS 36.331 [10]. In this version of the specification, the E-UTRA RRCConnectionReconfiguration can only includes sidelink related fields for V2X sidelink communication, i.e. sl-V2X-ConfigDedicated, sl-V2X-SPS-Config, measConfig and/or otherConfig.

sl-TimeOffsetEUTRA This field indicates the possible time offset to (de)activation of V2X sidelink transmission after receiving DCI format 3_1 used for scheduling V2X sidelink communication. Value ms0dpt75 corresponds to 0.75ms, ms1 corresponds to 1ms and so on. The network includes this field only when sl-ConfigDedicatedEUTRA is configured.

targetCellSMTC-SCG The SSB periodicity/offset/duration configuration of target cell for NR PSCell addition and SN change. When UE receives this field, UE applies the configuration based on the timing reference of NR PCell for PSCell addition and PSCell change for the case of no reconfiguration with sync of MCG, and UE applies the configuration based on the timing reference of target NR PCell for the case of reconfiguration with sync of MCG. If both this field and the smtc in secondaryCellGroup -> SpCellConfig -> reconfigurationWithSync are absent, the UE uses the SMTC in the measObjectNR having the same SSB frequency and subcarrier spacing, as configured before the reception of the RRC message.

t316 Indicates the value for timer T316 as described in clause 7.1. Value ms50 corresponds to 50 ms, value ms100 corresponds to 100 ms and so on. This field can be configured only if the UE is configured with split SRB1 or SRB3.

ue-TxTEG-RequestUL-TDOA-Config Configures the periodicity of UE reporting for the association between Tx TEG and SRS Positioning resources. When configured with oneShot UE reports the association only one time. When configured with periodicReporting UE reports the association periodically and the periodicReporting indicates the periodicity. Value ms160 corresponds to 160ms, value ms320 corresponds to 320ms and so on.

ul-GapFR2-Config Indicates the FR2 UL gap configuration to UE. In EN-DC and NGEN-DC, the SN decides and configures the FR2 UL gap pattern. In NE-DC, the MN decides and configures the FR2 UL gap pattern. In NR-DC without FR2-FR2 band combination, the network entity which is configured with FR2 serving cell(s) decides and configures the FR2 UL gap pattern.

| Conditional Presence | Explanation |
| --- | --- |
| nonHO | The field is absent in case of reconfiguration with sync within NR or to NR; otherwise it is optionally present, need N. |
| securityNASC | This field is mandatory present in case of inter system handover. Otherwise the field is optionally present, need N. |
| MasterKeyChange | This field is mandatory present in case masterCellGroup includes ReconfigurationWithSync and RadioBearerConfig includes SecurityConfig with SecurityAlgorithmConfig, indicating a change of the AS security algorithms associated to the master key. If ReconfigurationWithSync is included for other cases, this field is optionally present, need N. Otherwise the field is absent. |
| FullConfig | The field is mandatory present in case of inter-system handover from E-UTRA/EPC to NR. It is optionally present, Need N, during reconfiguration with sync and also in first reconfiguration after reestablishment; or for intra-system handover from E-UTRA/5GC to NR. It is absent otherwise. |
| SCG | The field is mandatory present in: - an RRCReconfiguration message contained in an RRCResume message (or in an RRCConnectionResume message, see TS 36.331 [10]), - an RRCReconfiguration message contained in an RRCConnectionReconfiguration message, see TS 36.331 [10], which is contained in DLInformationTransferMRDC transmitted on SRB3 (as a response to ULInformationTransferMRDC including an MCGFailureInformation). The field is optional present, Need M, in: - an RRCReconfiguration message transmitted on SRB3, - an RRCReconfiguration message contained in another RRCReconfiguration message (or in an RRCConnectionReconfiguration message, see TS 36.331 [10]) transmitted on SRB1 - an RRCReconfiguration message contained in another RRCReconfiguration message which is contained in DLInformationTransferMRDC transmitted on SRB3 (as a response to ULInformationTransferMRDC including an MCGFailureInformation) Otherwise, the field is absent |
| PagingRelay | For L2 U2N Relay UE, the field is optionally present, Need N. Otherwise, it is absent. |

#### –	RRCReconfigurationComplete

The RRCReconfigurationComplete message is used to confirm the successful completion of an RRC connection reconfiguration.

Signalling radio bearer: SRB1 or SRB3

RLC-SAP: AM

Logical channel: DCCH

Direction: UE to Network

RRCReconfigurationComplete message

-- ASN1START

-- TAG-RRCRECONFIGURATIONCOMPLETE-START

RRCReconfigurationComplete ::=              SEQUENCE {

rrc-TransactionIdentifier                   RRC-TransactionIdentifier,

criticalExtensions                          CHOICE {

rrcReconfigurationComplete                  RRCReconfigurationComplete-IEs,

criticalExtensionsFuture                    SEQUENCE {}

}

}

RRCReconfigurationComplete-IEs ::=          SEQUENCE {

lateNonCriticalExtension                    OCTET STRING                                                            OPTIONAL,

nonCriticalExtension                        RRCReconfigurationComplete-v1530-IEs                                    OPTIONAL

}

RRCReconfigurationComplete-v1530-IEs ::=    SEQUENCE {

uplinkTxDirectCurrentList                   UplinkTxDirectCurrentList                                               OPTIONAL,

nonCriticalExtension                        RRCReconfigurationComplete-v1560-IEs                                    OPTIONAL

}

RRCReconfigurationComplete-v1560-IEs ::=    SEQUENCE {

scg-Response                                CHOICE {

nr-SCG-Response                             OCTET STRING (CONTAINING RRCReconfigurationComplete),

eutra-SCG-Response                          OCTET STRING

}                                                                                                                       OPTIONAL,

nonCriticalExtension                        RRCReconfigurationComplete-v1610-IEs                                    OPTIONAL

}

RRCReconfigurationComplete-v1610-IEs ::=    SEQUENCE {

ue-MeasurementsAvailable-r16                UE-MeasurementsAvailable-r16                                            OPTIONAL,

needForGapsInfoNR-r16                       NeedForGapsInfoNR-r16                                                   OPTIONAL,

nonCriticalExtension                        RRCReconfigurationComplete-v1640-IEs                                    OPTIONAL

}

RRCReconfigurationComplete-v1640-IEs ::=    SEQUENCE {

uplinkTxDirectCurrentTwoCarrierList-r16     UplinkTxDirectCurrentTwoCarrierList-r16                                 OPTIONAL,

nonCriticalExtension                        RRCReconfigurationComplete-v1700-IEs                                    OPTIONAL

}

RRCReconfigurationComplete-v1700-IEs ::=    SEQUENCE {

needForGapNCSG-InfoNR-r17                   NeedForGapNCSG-InfoNR-r17                                               OPTIONAL,

needForGapNCSG-InfoEUTRA-r17                NeedForGapNCSG-InfoEUTRA-r17                                            OPTIONAL,

selectedCondRRCReconfig-r17                 CondReconfigId-r16                                                      OPTIONAL,

nonCriticalExtension                        RRCReconfigurationComplete-v1720-IEs                                    OPTIONAL

}

RRCReconfigurationComplete-v1720-IEs ::=    SEQUENCE {

uplinkTxDirectCurrentMoreCarrierList-r17    UplinkTxDirectCurrentMoreCarrierList-r17                                OPTIONAL,

nonCriticalExtension                        SEQUENCE {}                                                             OPTIONAL

}

-- TAG-RRCRECONFIGURATIONCOMPLETE-STOP

-- ASN1STOP

RRCReconfigurationComplete-IEs field descriptions

needForGapsInfoNR This field is used to indicate the measurement gap requirement information of the UE for NR target bands.

needForGapNCSG-InfoEUTRA This field is used to indicate the measurement gap and NCSG requirement information of the UE for E-UTRA target bands.

needForGapNCSG-InfoNR This field is used to indicate the measurement gap and NCSG requirement information of the UE for NR target bands.

scg-Response In case of NR-DC (nr-SCG-Response), this field includes the RRCReconfigurationComplete message. In case of NE-DC (eutra-SCG-Response), this field includes the E-UTRA RRCConnectionReconfigurationComplete message as specified in TS 36.331 [10].

selectedCondRRCReconfig This field indicates the ID of the selected conditional reconfiguration the UE applied upon the execution of CPA or inter-SN CPC.

uplinkTxDirectCurrentList The Tx Direct Current locations for the configured serving cells and BWPs if requested by the NW (see reportUplinkTxDirectCurrent in CellGroupConfig).

uplinkTxDirectCurrentMoreCarrierList The Tx Direct Current locations for the configured intra-band CA requested by reportUplinkTxDirectCurrentMoreCarrier-r17.

uplinkTxDirectCurrentTwoCarrierList The Tx Direct Current locations for the configured uplink intra-band CA with two carriers if requested by the NW (see reportUplinkTxDirectCurrentTwoCarrier-r16 in CellGroupConfig).

#### –	RRCReject

The RRCReject message is used to reject an RRC connection establishment or an RRC connection resumption.

Signalling radio bearer: SRB0

RLC-SAP: TM

Logical channel: CCCH

Direction: Network to UE

RRCReject message

-- ASN1START

-- TAG-RRCREJECT-START

RRCReject ::=                       SEQUENCE {

criticalExtensions                  CHOICE {

rrcReject                           RRCReject-IEs,

criticalExtensionsFuture            SEQUENCE {}

}

}

RRCReject-IEs ::=                   SEQUENCE {

waitTime                            RejectWaitTime                                                          OPTIONAL,   -- Need N

lateNonCriticalExtension            OCTET STRING                                                            OPTIONAL,

nonCriticalExtension                SEQUENCE{}                                                              OPTIONAL

}

-- TAG-RRCREJECT-STOP

-- ASN1STOP

RRCReject-IEs field descriptions

waitTime Wait time value in seconds. The field is always included.

#### –	RRCRelease

The RRCRelease message is used to command the release of an RRC connection or the suspension of the RRC connection.

Signalling radio bearer: SRB1

RLC-SAP: AM

Logical channel: DCCH

Direction: Network to UE

RRCRelease message

-- ASN1START

-- TAG-RRCRELEASE-START

RRCRelease ::=                      SEQUENCE {

rrc-TransactionIdentifier           RRC-TransactionIdentifier,

criticalExtensions                  CHOICE {

rrcRelease                          RRCRelease-IEs,

criticalExtensionsFuture            SEQUENCE {}

}

}

RRCRelease-IEs ::=                  SEQUENCE {

redirectedCarrierInfo               RedirectedCarrierInfo                                                       OPTIONAL,   -- Need N

cellReselectionPriorities           CellReselectionPriorities                                                   OPTIONAL,   -- Need R

suspendConfig                       SuspendConfig                                                               OPTIONAL,   -- Need R

deprioritisationReq                 SEQUENCE {

deprioritisationType                ENUMERATED {frequency, nr},

deprioritisationTimer               ENUMERATED {min5, min10, min15, min30}

}                                                                                                               OPTIONAL,   -- Need N

lateNonCriticalExtension                OCTET STRING                                                        OPTIONAL,

nonCriticalExtension                    RRCRelease-v1540-IEs                                                OPTIONAL

}

RRCRelease-v1540-IEs ::=            SEQUENCE {

waitTime                           RejectWaitTime                OPTIONAL, -- Need N

nonCriticalExtension               RRCRelease-v1610-IEs          OPTIONAL

}

RRCRelease-v1610-IEs ::=            SEQUENCE {

voiceFallbackIndication-r16        ENUMERATED {true}                             OPTIONAL, -- Need N

measIdleConfig-r16                 SetupRelease {MeasIdleConfigDedicated-r16}    OPTIONAL, -- Need M

nonCriticalExtension               RRCRelease-v1650-IEs                          OPTIONAL

}

RRCRelease-v1650-IEs ::=            SEQUENCE {

mpsPriorityIndication-r16          ENUMERATED {true}                             OPTIONAL, -- Cond Redirection2

nonCriticalExtension               RRCRelease-v1710-IEs                          OPTIONAL

}

RRCRelease-v1710-IEs ::=            SEQUENCE {

noLastCellUpdate-r17                ENUMERATED {true}                            OPTIONAL,   -- Need S

nonCriticalExtension                SEQUENCE {}                                  OPTIONAL

}

RedirectedCarrierInfo ::=           CHOICE {

nr                                  CarrierInfoNR,

eutra                               RedirectedCarrierInfo-EUTRA,

...

}

RedirectedCarrierInfo-EUTRA ::=     SEQUENCE {

eutraFrequency                      ARFCN-ValueEUTRA,

cnType                              ENUMERATED {epc,fiveGC}                                             OPTIONAL    -- Need N

}

CarrierInfoNR ::=                   SEQUENCE {

carrierFreq                         ARFCN-ValueNR,

ssbSubcarrierSpacing                SubcarrierSpacing,

smtc                                SSB-MTC                                                             OPTIONAL,      -- Need S

...

}

SuspendConfig ::=                   SEQUENCE {

fullI-RNTI                          I-RNTI-Value,

shortI-RNTI                         ShortI-RNTI-Value,

ran-PagingCycle                     PagingCycle,

ran-NotificationAreaInfo            RAN-NotificationAreaInfo                                            OPTIONAL,   -- Need M

t380                                PeriodicRNAU-TimerValue                                             OPTIONAL,   -- Need R

nextHopChainingCount                NextHopChainingCount,

...,

[[

sl-UEIdentityRemote-r17             RNTI-Value                                                          OPTIONAL, -- Cond L2RemoteUE

sdt-Config-r17                      SetupRelease { SDT-Config-r17 }                                     OPTIONAL,   -- Need M

srs-PosRRC-Inactive-r17             SetupRelease { SRS-PosRRC-Inactive-r17 }                            OPTIONAL,   -- Need M

ran-ExtendedPagingCycle-r17         ExtendedPagingCycle-r17                                             OPTIONAL    -- Cond RANPaging

]],

[[

ncd-SSB-RedCapInitialBWP-SDT-r17    SetupRelease {NonCellDefiningSSB-r17}                               OPTIONAL    -- Need M

]]

}

PeriodicRNAU-TimerValue ::=         ENUMERATED { min5, min10, min20, min30, min60, min120, min360, min720}

CellReselectionPriorities ::=       SEQUENCE {

freqPriorityListEUTRA               FreqPriorityListEUTRA                                               OPTIONAL,       -- Need M

freqPriorityListNR                  FreqPriorityListNR                                                  OPTIONAL,       -- Need M

t320                                ENUMERATED {min5, min10, min20, min30, min60, min120, min180, spare1} OPTIONAL,     -- Need R

...,

[[

freqPriorityListDedicatedSlicing-r17 FreqPriorityListDedicatedSlicing-r17                               OPTIONAL        -- Need M

]]

}

PagingCycle ::=                     ENUMERATED {rf32, rf64, rf128, rf256}

ExtendedPagingCycle-r17 ::=         ENUMERATED {rf256, rf512, rf1024, spare1}

FreqPriorityListEUTRA ::=           SEQUENCE (SIZE (1..maxFreq)) OF FreqPriorityEUTRA

FreqPriorityListNR ::=              SEQUENCE (SIZE (1..maxFreq)) OF FreqPriorityNR

FreqPriorityEUTRA ::=               SEQUENCE {

carrierFreq                         ARFCN-ValueEUTRA,

cellReselectionPriority             CellReselectionPriority,

cellReselectionSubPriority          CellReselectionSubPriority                                          OPTIONAL        -- Need R

}

FreqPriorityNR ::=                  SEQUENCE {

carrierFreq                         ARFCN-ValueNR,

cellReselectionPriority             CellReselectionPriority,

cellReselectionSubPriority          CellReselectionSubPriority                                          OPTIONAL        -- Need R

}

RAN-NotificationAreaInfo ::=        CHOICE {

cellList                            PLMN-RAN-AreaCellList,

ran-AreaConfigList                  PLMN-RAN-AreaConfigList,

...

}

PLMN-RAN-AreaCellList ::=           SEQUENCE (SIZE (1.. maxPLMNIdentities)) OF PLMN-RAN-AreaCell

PLMN-RAN-AreaCell ::=               SEQUENCE {

plmn-Identity                       PLMN-Identity                                                       OPTIONAL,   -- Need S

ran-AreaCells                       SEQUENCE (SIZE (1..32)) OF  CellIdentity

}

PLMN-RAN-AreaConfigList ::=         SEQUENCE (SIZE (1..maxPLMNIdentities)) OF PLMN-RAN-AreaConfig

PLMN-RAN-AreaConfig ::=             SEQUENCE {

plmn-Identity                       PLMN-Identity                                                       OPTIONAL,   -- Need S

ran-Area                            SEQUENCE (SIZE (1..16)) OF  RAN-AreaConfig

}

RAN-AreaConfig ::=                  SEQUENCE {

trackingAreaCode                    TrackingAreaCode,

ran-AreaCodeList                    SEQUENCE (SIZE (1..32)) OF  RAN-AreaCode                            OPTIONAL    -- Need R

}

SDT-Config-r17 ::=                  SEQUENCE {

sdt-DRB-List-r17                    SEQUENCE (SIZE (0..maxDRB)) OF DRB-Identity                         OPTIONAL,   -- Need M

sdt-SRB2-Indication-r17             ENUMERATED {allowed}                                                OPTIONAL,   -- Need R

sdt-MAC-PHY-CG-Config-r17           SetupRelease {SDT-CG-Config-r17}                                    OPTIONAL,   -- Need M

sdt-DRB-ContinueROHC-r17            ENUMERATED { cell, rna }                                            OPTIONAL    -- Need S

}

SDT-CG-Config-r17 ::= OCTET STRING (CONTAINING SDT-MAC-PHY-CG-Config-r17)

SDT-MAC-PHY-CG-Config-r17 ::=       SEQUENCE {

-- CG-SDT specific configuration

cg-SDT-ConfigLCH-RestrictionToAddModList-r17 SEQUENCE (SIZE(1..maxLC-ID)) OF  CG-SDT-ConfigLCH-Restriction-r17 OPTIONAL,   -- Need N

cg-SDT-ConfigLCH-RestrictionToReleaseList-r17 SEQUENCE (SIZE(1..maxLC-ID)) OF  LogicalChannelIdentity  OPTIONAL,   -- Need N

cg-SDT-ConfigInitialBWP-NUL-r17       SetupRelease {BWP-UplinkDedicatedSDT-r17}                     OPTIONAL,   -- Need M

cg-SDT-ConfigInitialBWP-SUL-r17       SetupRelease {BWP-UplinkDedicatedSDT-r17}                     OPTIONAL,   -- Need M

cg-SDT-ConfigInitialBWP-DL-r17        BWP-DownlinkDedicatedSDT-r17                                  OPTIONAL,   -- Need M

cg-SDT-TimeAlignmentTimer-r17           TimeAlignmentTimer                                              OPTIONAL,   -- Need M

cg-SDT-RSRP-ThresholdSSB-r17            RSRP-Range                                                      OPTIONAL,   -- Need M

cg-SDT-TA-ValidationConfig-r17          SetupRelease { CG-SDT-TA-ValidationConfig-r17 }                 OPTIONAL,   -- Need M

cg-SDT-CS-RNTI-r17                      RNTI-Value                                                      OPTIONAL,   -- Need M

...

}

CG-SDT-TA-ValidationConfig-r17 ::=  SEQUENCE {

cg-SDT-RSRP-ChangeThreshold-r17     ENUMERATED { dB2, dB4, dB6, dB8, dB10, dB14, dB18, dB22,

dB26, dB30, dB34, spare5, spare4, spare3, spare2, spare1}

}

BWP-DownlinkDedicatedSDT-r17 ::=    SEQUENCE {

pdcch-Config-r17                    SetupRelease { PDCCH-Config }                                       OPTIONAL,   -- Need M

pdsch-Config-r17                    SetupRelease { PDSCH-Config }                                       OPTIONAL,   -- Need M

...

}

BWP-UplinkDedicatedSDT-r17 ::=      SEQUENCE {

pusch-Config-r17                    SetupRelease { PUSCH-Config }                                       OPTIONAL,   -- Need M

configuredGrantConfigToAddModList-r17                 ConfiguredGrantConfigToAddModList-r16             OPTIONAL,   -- Need N

configuredGrantConfigToReleaseList-r17                ConfiguredGrantConfigToReleaseList-r16            OPTIONAL,   -- Need N

...

}

CG-SDT-ConfigLCH-Restriction-r17 ::= SEQUENCE {

logicalChannelIdentity-r17          LogicalChannelIdentity,

configuredGrantType1Allowed-r17     ENUMERATED {true}                                                   OPTIONAL,   -- Need R

allowedCG-List-r17                  SEQUENCE (SIZE (0.. maxNrofConfiguredGrantConfigMAC-1-r16)) OF ConfiguredGrantConfigIndexMAC-r16

OPTIONAL    -- Need R

}

SRS-PosRRC-Inactive-r17 ::= OCTET STRING (CONTAINING SRS-PosRRC-InactiveConfig-r17)

SRS-PosRRC-InactiveConfig-r17 ::=       SEQUENCE {

srs-PosConfigNUL-r17                    SRS-PosConfig-r17                                                   OPTIONAL,    -- Need R

srs-PosConfigSUL-r17                    SRS-PosConfig-r17                                                   OPTIONAL,    -- Need R

bwp-NUL-r17                             BWP                                                                 OPTIONAL,    -- Need S

bwp-SUL-r17                             BWP                                                                 OPTIONAL,    -- Need S

inactivePosSRS-TimeAlignmentTimer-r17   TimeAlignmentTimer                                                  OPTIONAL,    -- Need M

inactivePosSRS-RSRP-ChangeThreshold-r17 RSRP-ChangeThreshold-r17                                            OPTIONAL     -- Need M

}

RSRP-ChangeThreshold-r17 ::= ENUMERATED {dB4, dB6, dB8, dB10, dB14, dB18, dB22, dB26, dB30, dB34, spare6, spare5, spare4, spare3, spare2, spare1}

SRS-PosConfig-r17 ::=               SEQUENCE {

srs-PosResourceSetToReleaseList-r17 SEQUENCE (SIZE(1..maxNrofSRS-PosResourceSets-r16)) OF SRS-PosResourceSetId-r16 OPTIONAL,-- Need N

srs-PosResourceSetToAddModList-r17  SEQUENCE (SIZE(1..maxNrofSRS-PosResourceSets-r16)) OF SRS-PosResourceSet-r16  OPTIONAL,-- Need N

srs-PosResourceToReleaseList-r17    SEQUENCE (SIZE(1..maxNrofSRS-PosResources-r16)) OF SRS-PosResourceId-r16      OPTIONAL,-- Need N

srs-PosResourceToAddModList-r17     SEQUENCE (SIZE(1..maxNrofSRS-PosResources-r16)) OF SRS-PosResource-r16        OPTIONAL -- Need N

}

-- TAG-RRCRELEASE-STOP

-- ASN1STOP

RRCRelease-IEs field descriptions

cellReselectionPriorities Dedicated priorities to be used for cell reselection as specified in TS 38.304 [20]. The maximum number of NR carrier frequencies that the network can configure through FreqPriorityListNR and FreqPriorityListDedicatedSlicing together is eight. If the same frequency is configured in both FreqPriorityListNR and FreqPriorityListDedicatedSlicing, the frequency is only counted once.

cnType Indicate that the UE is redirected to EPC or 5GC.

deprioritisationReq Indicates whether the current frequency or RAT is to be de-prioritised.

deprioritisationTimer Indicates the period for which either the current carrier frequency or NR is deprioritised. Value minN corresponds to N minutes.

measIdleConfig Indicates measurement configuration to be stored and used by the UE while in RRC_IDLE or RRC_INACTIVE.

mpsPriorityIndication Indicates the UE can set the establishment cause to mps-PriorityAccess for a new connection following a redirect to NR. If the target RAT is E-UTRA, see TS 36.331 [10]. The gNB sets the indication only for UEs authorized to receive MPS treatment as indicated by ARP and/or QoS characteristics at the gNB, and it is applicable only for this instance of release with redirection to carrier/RAT included in the redirectedCarrierInfo field in the RRCRelease message.

noLastCellUpdate Presence of the field indicates that the last used cell for PEI shall not be updated. When the field is absent, the PEI-capable UE shall update its last used cell with the current cell. The UE shall not update its last used cell with the current cell if the AS security is not activated.

srs-PosRRC-InactiveConfig SRS for positioning configuration during RRC_INACTIVE state.

suspendConfig Indicates configuration for the RRC_INACTIVE state. The network does not configure suspendConfig when the network redirect the UE to an inter-RAT carrier frequency or if the UE is configured with a DAPS bearer.

redirectedCarrierInfo Indicates a carrier frequency (downlink for FDD) and is used to redirect the UE to an NR or an inter-RAT carrier frequency, by means of cell selection at transition to RRC_IDLE or RRC_INACTIVE as specified in TS 38.304 [20]. Based on UE capability, the network may include redirectedCarrierInfo in RRCRelease message with suspendConfig if this message is sent in response to an RRCResumeRequest or an RRCResumeRequest1 which is triggered by the NAS layer (see 5.3.1.4 in TS 24.501 [23]).

voiceFallbackIndication Indicates the RRC release is triggered by EPS fallback for IMS voice as specified in TS 23.502 [43].

CarrierInfoNR field descriptions

carrierFreq Indicates the redirected NR frequency.

ssbSubcarrierSpacing Subcarrier spacing of SSB in the redirected SSB frequency. Only the following values are applicable depending on the used frequency: FR1: 15 or 30 kHz FR2-1: 120 or 240 kHz FR2-2: 120, 480, or 960 kHz

smtc The SSB periodicity/offset/duration configuration for the redirected SSB frequency. It is based on timing reference of PCell. If the field is absent, the UE uses the SMTC configured in the measObjectNR having the same SSB frequency and subcarrier spacing.

RAN-NotificationAreaInfo field descriptions

cellList A list of cells configured as RAN area.

ran-AreaConfigList A list of RAN area codes or RA code(s) as RAN area.

PLMN-RAN-AreaConfig field descriptions

plmn-Identity PLMN Identity to which the cells in ran-Area belong. If the field is absent the UE not in SNPN access mode uses the ID of the registered PLMN. This field is not included for UE in SNPN access mode (for UE in SNPN access mode the ran-Area always belongs to the registered SNPN).

ran-AreaCodeList The total number of RAN-AreaCodes of all PLMNs does not exceed 32.

ran-Area Indicates whether TA code(s) or RAN area code(s) are used for the RAN notification area. The network uses only TA code(s) or both TA code(s) and RAN area code(s) to configure a UE. The total number of TACs across all PLMNs does not exceed 16.

PLMN-RAN-AreaCell field descriptions

plmn-Identity PLMN Identity to which the cells in ran-AreaCells belong. If the field is absent the UE not in SNPN access mode uses the ID of the registered PLMN. This field is not included for UE in SNPN access mode (for UE in SNPN access mode the ran-AreaCells always belongs to the registered SNPN).

ran-AreaCells The total number of cells of all PLMNs does not exceed 32.

SDT-Config field descriptions

sdt-DRB-ContinueROHC Indicates whether the PDCP entity of the radio bearers configured for SDT continues or resets the ROHC header compression protocol during PDCP re-establishment during SDT procedure, as specified in TS 38.323 [5]. Value cell indicates that ROHC header compression continues when the UE resumes for SDT in the same cell as the PCell when the RRCRelease message was received. Value rna indicates that ROHC header compression continues when the UE resumes for SDT in a cell belonging to the same RNA as the PCell where the RRCRelease message was received. If the field is absent, the UE releases any stored value for this field and the PDCP entity of the radio bearers configured for SDT always resets the ROHC header compression protocol during PDCP re-establishment when SDT procedure is initiated, as specified in TS 38.323 [5].

sdt-DRB-List Indicates the ID(s) of the DRB(s) that are configured for SDT. If size of the sequence is zero, then the UE assumes that none of the DRBs are configured for SDT. The network only configures MN terminated MCG bearers for SDT.

sdt-SRB2-Indication Indiates whether SRB2 is configured for SDT or not.

SDT-MAC-PHY-CG-Config field descriptions

cg-SDT-ConfigInitialBWP-DL Downlink BWP configuration for CG-SDT. If a UE is a RedCap UE and if the initialDownlinkBWP-RedCap is configured in downlinkConfigCommon in SIB1, this field is configured for initialDownlinkBWP-RedCap, otherwise it is configured for initialDownlinkBWP.

cg-SDT-ConfigInitialBWP-NUL UL BWP configuration for CG-SDT on NUL carrier. If a UE is a RedCap UE and if the initialUplinkBWP-RedCap is configured in uplinkConfigCommon in SIB1, this field is configured for initialUplinkBWP-RedCap, otherwise it is configured for initialUplinkBWP for NUL.

cg-SDT-ConfigInitialBWP-SUL UL BWP configuration for CG-SDT on SUL carrier configured for the initialUplinkBWP for SUL.

cg-SDT-CS-RNTI The CS-RNTI value for CG-SDT as specified in TS 38.321 [3].

cg-SDT-RSRP-ThresholdSSB An RSRP threshold configured for SSB selection for CG-SDT as specified in TS 38.321 [3].

cg-SDT-TA-ValidationConfig Configuration for the RSRP based TA validation. If this field is not configured, then the UE does not perform RSRP based TA validation.

cg-SDT-timeAlignmentTimer TAT value for CG-SDT as specified in TS 38.321 [3]. The network always configures this field when sdt-MAC-PHY-CG-Config is configured.

CG-SDT-ConfigLCH-Restriction field descriptions

allowedCG-List This restriction applies only when the UL grant is a configured grant for CG-SDT. If present, UL MAC SDUs from this logical channel can only be mapped to the indicated CG-SDT configured grant configuration. If the size of the sequence is zero, then UL MAC SDUs from this logical channel cannot be mapped to any CG-SDT configured grant configurations. If the field is not present, UL MAC SDUs from this logical channel can be mapped to any CG-SDT configured grant configurations. If the field configuredGrantType1Allowed is present, only those CG-SDT configured grant type 1 configurations indicated in this sequence are allowed for use by this logical channel; otherwise, this sequence shall not include any CG-SDT configured grant type 1 configuration. Corresponds to "allowedCG-List" as specified in TS 38.321 [3].

configuredGrantType1Allowed If present, or if the capability lcp-Restriction as specified in TS 38.306 [26] is not supported, UL MAC SDUs from this logical channel can be transmitted on a configured grant type 1 for CG-SDT. Otherwise, UL MAC SDUs from this logical channel cannot be transmitted on a configured grant type 1 for CG-SDT. Corresponds to "configuredGrantType1Allowed" in TS 38.321 [3].

logicalChannelIdentity ID used commonly for the MAC logical channel and for the RLC bearer associated with a servedRadioBearer configured for SDT.

CG-SDT-TA-ValidationConfig field descriptions

cg-SDT-RSRP-ChangeThreshold The RSRP threshold for TA validation for CG-SDT as specified in TS 38.321 [3]. Value dB2 corresponds to 2 dB, value dB4 corresponds to 4 dB and so on.

SRS-PosRRC-InactiveConfig field descriptions

bwp-NUL BWP configuration for SRS for Positioning during the RRC_INACTIVE state in Normal Uplink Carrier. If the field is absent UE is configured with an SRS for Positioning associated with the initial UL BWP and transmitted, during the RRC_INACTIVE state, inside the initial UL BWP with the same CP and SCS as configured for initial UL BWP.

bwp-SUL BWP configuration for SRS for Positioning during the RRC_INACTIVE state in Supplementary Uplink Carrier. If the field is absent UE is configured with an SRS for Positioning associated with the initial UL BWP and transmitted, during the RRC_INACTIVE state, inside the initial UL BWP with the same CP and SCS as configured for initial UL BWP.

inactivePosSRS-RSRP-ChangeThreshold RSRP threshold for the increase/decrease of RSRP for time alignment validation as specified in TS 38.321 [3].

inactivePosSRS-TimeAlignmentTimer TAT value for SRS for positioning transmission during RRC_INACTIVE state as specified in TS 38.321 [3]. The network always configures this field when srs-PosRRC-Inactive is configured.

srs-PosConfigNUL SRS for Positioning configuration in RRC_INACTIVE state in Normal Uplink Carrier.

srs-PosConfigSUL SRS for Positioning configuration in RRC_INACTIVE state in Supplementary Uplink Carrier.

| SuspendConfig field descriptions |  |
| --- | --- |
| ncd-SSB-RedCapInitialBWP-SDT Indicates that the UE uses the RedCap-specific initial DL BWP associated with the NCD-SSB for SDT. The network configures this field if a RedCap UE is configured with SDT in the RedCap-specific initial DL BWP not associated with CD-SSB. If configured, the NCD-SSB indicated by this field can only be used during the SDT procedure for CG-SDT or RA-SDT. |  |
| ran-ExtendedPagingCycle The extended DRX (eDRX) cycle for RAN-initiated paging to be applied by the UE. Value rf256 corresponds to 256 radio frames, value rf512 corresponds to 512 radio frames and so on. Value of the field indicates an eDRX cycle which is shorter or equal to the IDLE mode eDRX cycle configured for the UE. |
| ran-NotificationAreaInfo Network ensures that the UE in RRC_INACTIVE always has a valid ran-NotificationAreaInfo. |  |
| ran-PagingCycle Refers to the UE specific cycle for RAN-initiated paging. Value rf32 corresponds to 32 radio frames, value rf64 corresponds to 64 radio frames and so on. |  |
| sl-UEIdentityRemote Indicates the C-RNTI to the L2 U2N Remote UE. |  |
| t380 Refers to the timer that triggers the periodic RNAU procedure in UE. Value min5 corresponds to 5 minutes, value min10 corresponds to 10 minutes and so on. |  |

| Conditional Presence | Explanation |
| --- | --- |
| L2RemoteUE | The field is mandatory present for L2 U2N Remote UE's RNAU; otherwise it is absent. |
| RANPaging | This field is optionally present, Need R, if the UE is configured with IDLE eDRX, see TS 24.501 [23]; otherwise the field is not present. |
| Redirection2 | The field is optionally present, Need R, if redirectedCarrierInfo is included; otherwise the field is not present. |

#### –	RRCResume

The RRCResume message is used to resume the suspended RRC connection.

Signalling radio bearer: SRB1

RLC-SAP: AM

Logical channel: DCCH

Direction: Network to UE

RRCResume message

-- ASN1START

-- TAG-RRCRESUME-START

RRCResume ::=                       SEQUENCE {

rrc-TransactionIdentifier           RRC-TransactionIdentifier,

criticalExtensions                  CHOICE {

rrcResume                           RRCResume-IEs,

criticalExtensionsFuture            SEQUENCE {}

}

}

RRCResume-IEs ::=                   SEQUENCE {

radioBearerConfig                   RadioBearerConfig                                               OPTIONAL, -- Need M

masterCellGroup                     OCTET STRING (CONTAINING CellGroupConfig)                       OPTIONAL, -- Need M

measConfig                          MeasConfig                                                      OPTIONAL, -- Need M

fullConfig                          ENUMERATED {true}                                               OPTIONAL, -- Need N

lateNonCriticalExtension            OCTET STRING                                                    OPTIONAL,

nonCriticalExtension                RRCResume-v1560-IEs                                             OPTIONAL

}

RRCResume-v1560-IEs ::=             SEQUENCE {

radioBearerConfig2                  OCTET STRING (CONTAINING RadioBearerConfig)                     OPTIONAL, -- Need M

sk-Counter                          SK-Counter                                                      OPTIONAL, -- Need N

nonCriticalExtension                RRCResume-v1610-IEs                                             OPTIONAL

}

RRCResume-v1610-IEs ::=             SEQUENCE {

idleModeMeasurementReq-r16          ENUMERATED {true}                                               OPTIONAL, -- Need N

restoreMCG-SCells-r16               ENUMERATED {true}                                               OPTIONAL, -- Need N

restoreSCG-r16                      ENUMERATED {true}                                               OPTIONAL, -- Need N

mrdc-SecondaryCellGroup-r16         CHOICE {

nr-SCG-r16                          OCTET STRING (CONTAINING RRCReconfiguration),

eutra-SCG-r16                       OCTET STRING

}                                                                                                   OPTIONAL, -- Cond RestoreSCG

needForGapsConfigNR-r16             SetupRelease {NeedForGapsConfigNR-r16}                          OPTIONAL, -- Need M

nonCriticalExtension                RRCResume-v1700-IEs                                             OPTIONAL

}

RRCResume-v1700-IEs ::=             SEQUENCE {

sl-ConfigDedicatedNR-r17            SetupRelease {SL-ConfigDedicatedNR-r16}                         OPTIONAL, -- Cond L2RemoteUE

sl-L2RemoteUE-Config-r17            SetupRelease {SL-L2RemoteUE-Config-r17}                         OPTIONAL, -- Cond L2RemoteUE

needForGapNCSG-ConfigNR-r17         SetupRelease {NeedForGapNCSG-ConfigNR-r17}                      OPTIONAL, -- Need M

needForGapNCSG-ConfigEUTRA-r17      SetupRelease {NeedForGapNCSG-ConfigEUTRA-r17}                   OPTIONAL, -- Need M

scg-State-r17                       ENUMERATED {deactivated}                                        OPTIONAL, -- Need N

appLayerMeasConfig-r17              AppLayerMeasConfig-r17                                          OPTIONAL, -- Need M

nonCriticalExtension                SEQUENCE {}                                                     OPTIONAL

}

-- TAG-RRCRESUME-STOP

-- ASN1STOP

RRCResume-IEs field descriptions

appLayerMeasConfig This field is used to configure application layer measurements. This field is absent when the UE is configured to operate with shared spectrum channel access.

idleModeMeasurementReq This field indicates that the UE shall report the idle/inactive measurements, if available, to the network in the RRCResumeComplete message

masterCellGroup Configuration of the master cell group.

mrdc-SecondaryCellGroup Includes an RRC message for SCG configuration in NR-DC or NE-DC. For NR-DC (nr-SCG), mrdc-SecondaryCellGroup contains the RRCReconfiguration message as generated (entirely) by SN gNB. In this version of the specification, the RRC message can only include fields secondaryCellGroup (with at least reconfigurationWithSync), otherConfig and measConfig. For NE-DC (eutra-SCG), mrdc-SecondaryCellGroup includes the E-UTRA RRCConnectionReconfiguration message as specified in TS 36.331 [10]. In this version of the specification, the E-UTRA RRC message only include the field scg-Configuration with at least mobilityControlInfoSCG.

needForGapsConfigNR Configuration for the UE to report measurement gap requirement information of NR target bands in the RRCReconfigurationComplete and RRCResumeComplete message.

needForGapNCSG-ConfigEUTRA Configuration for the UE to report measurement gap and NCSG requirement information of E-UTRA target bands in the RRCReconfigurationComplete and RRCResumeComplete message.

needForGapNCSG-ConfigNR Configuration for the UE to report measurement gap and NCSG requirement information of NR target bands in the RRCReconfigurationComplete and RRCResumeComplete message.

radioBearerConfig Configuration of Radio Bearers (DRBs, SRBs, multicast MRBs) including SDAP/PDCP.

radioBearerConfig2 Configuration of Radio Bearers (DRBs, SRBs) including SDAP/PDCP. This field can only be used if the UE supports NR-DC or NE-DC.

restoreMCG-SCells Indicates that the UE shall restore the MCG SCells from the UE Inactive AS Context, if stored.

restoreSCG Indicates that the UE shall restore the SCG configurations from the UE Inactive AS Context, if stored.

scg-State Indicates that the SCG is in deactivated state.

sk-Counter A counter used to derive S-KgNB or S-KeNB based on the newly derived KgNB during RRC Resume. The field is only included when there is one or more RB with keyToUse set to secondary or mrdc-SecondaryCellGroup is included.

sl-ConfigDedicatedNR This field is used to provide the dedicated configurations for NR sidelink communication/discovery used by L2 U2N Remote UE.

sl-L2RemoteUE-Config Contains L2 U2N relay operation related configurations used by L2 U2N Remote UE. The field is absent if appLayerMeasConfig or SRB4 is configured/not released.

| Conditional Presence | Explanation |
| --- | --- |
| L2RemoteUE | The field is mandatory present for L2 U2N Remote UE; otherwise it is absent. |
| RestoreSCG | The field is mandatory present if restoreSCG is included. It is optionally present, Need M, otherwise. |

#### –	RRCResumeComplete

The RRCResumeComplete message is used to confirm the successful completion of an RRC connection resumption.

Signalling radio bearer: SRB1

RLC-SAP: AM

Logical channel: DCCH

Direction: UE to Network

RRCResumeComplete message

-- ASN1START

-- TAG-RRCRESUMECOMPLETE-START

RRCResumeComplete ::=                   SEQUENCE {

rrc-TransactionIdentifier               RRC-TransactionIdentifier,

criticalExtensions                      CHOICE {

rrcResumeComplete                       RRCResumeComplete-IEs,

criticalExtensionsFuture                SEQUENCE {}

}

}

RRCResumeComplete-IEs ::=               SEQUENCE {

dedicatedNAS-Message                    DedicatedNAS-Message                                                    OPTIONAL,

selectedPLMN-Identity                   INTEGER (1..maxPLMN)                                                    OPTIONAL,

uplinkTxDirectCurrentList               UplinkTxDirectCurrentList                                               OPTIONAL,

lateNonCriticalExtension                OCTET STRING                                                            OPTIONAL,

nonCriticalExtension                    RRCResumeComplete-v1610-IEs                                             OPTIONAL

}

RRCResumeComplete-v1610-IEs ::=         SEQUENCE {

idleMeasAvailable-r16                   ENUMERATED {true}                                                       OPTIONAL,

measResultIdleEUTRA-r16                 MeasResultIdleEUTRA-r16                                                 OPTIONAL,

measResultIdleNR-r16                    MeasResultIdleNR-r16                                                    OPTIONAL,

scg-Response-r16                        CHOICE {

nr-SCG-Response                         OCTET STRING (CONTAINING RRCReconfigurationComplete),

eutra-SCG-Response                      OCTET STRING

}                                                                                                               OPTIONAL,

ue-MeasurementsAvailable-r16            UE-MeasurementsAvailable-r16                                            OPTIONAL,

mobilityHistoryAvail-r16                ENUMERATED {true}                                                       OPTIONAL,

mobilityState-r16                       ENUMERATED {normal, medium, high, spare}                                OPTIONAL,

needForGapsInfoNR-r16                   NeedForGapsInfoNR-r16                                                   OPTIONAL,

nonCriticalExtension                    RRCResumeComplete-v1640-IEs                                             OPTIONAL

}

RRCResumeComplete-v1640-IEs ::=         SEQUENCE {

uplinkTxDirectCurrentTwoCarrierList-r16 UplinkTxDirectCurrentTwoCarrierList-r16                                 OPTIONAL,

nonCriticalExtension                    RRCResumeComplete-v1700-IEs                                             OPTIONAL

}

RRCResumeComplete-v1700-IEs ::=         SEQUENCE {

needForGapNCSG-InfoNR-r17               NeedForGapNCSG-InfoNR-r17                                               OPTIONAL,

needForGapNCSG-InfoEUTRA-r17            NeedForGapNCSG-InfoEUTRA-r17                                            OPTIONAL,

nonCriticalExtension                    RRCResumeComplete-v1720-IEs                                             OPTIONAL

}

RRCResumeComplete-v1720-IEs ::=         SEQUENCE {

uplinkTxDirectCurrentMoreCarrierList-r17 UplinkTxDirectCurrentMoreCarrierList-r17                               OPTIONAL,

nonCriticalExtension                     RRCResumeComplete-v18xy                                                            OPTIONAL

}

RRCResumeComplete-v18xy-IEs ::=      SEQUENCE {

musim-CapabilityRestrictionIndication-r18     ENUMERATED {true}                               OPTIONAL,

nonCriticalExtension                SEQUENCE{}                                      OPTIONAL

}

-- TAG-RRCRESUMECOMPLETE-STOP

-- ASN1STOP

RRCResumeComplete-IEs field descriptions

idleMeasAvailable Indication that the UE has idle/inactive measurement report available.

measResultIdleEUTRA EUTRA measurement results performed during RRC_INACTIVE.

measResultIdleNR NR measurement results performed during RRC_INACTIVE.

musim-CapabilityRestrictionIndication This field indicates the UE temporary capability restriction due to MUSIM operation.

needForGapsInfoNR This field is used to indicate the measurement gap requirement information of the UE for NR target bands.

needForGapNCSG-InfoEUTRA This field is used to indicate the measurement gap and NCSG requirement information of the UE for E-UTRA target bands

needForGapNCSG-InfoNR This field is used to indicate the measurement gap and NCSG requirement information of the UE for NR target bands

selectedPLMN-Identity Index of the PLMN selected by the UE from the plmn-IdentityInfoList or npn-IdentityInfoList fields included in SIB1.

uplinkTxDirectCurrentList The Tx Direct Current locations for the configured serving cells and BWPs if requested by the NW (see reportUplinkTxDirectCurrent in CellGroupConfig).

uplinkTxDirectCurrentMoreCarrierList The Tx Direct Current locations for the configured intra-band CA requested by reportUplinkTxDirectCurrentMoreCarrier-r17.

uplinkTxDirectCurrentTwoCarrierList The Tx Direct Current locations for the configured uplink intra-band CA with two carriers if requested by the NW (see reportUplinkTxDirectCurrentTwoCarrier-r16 in CellGroupConfig).

#### –	RRCResumeRequest

The RRCResumeRequest message is used to request the resumption of a suspended RRC connection or perform an RNA update.

Signalling radio bearer: SRB0

RLC-SAP: TM

Logical channel: CCCH

Direction: UE to Network

RRCResumeRequest message

-- ASN1START

-- TAG-RRCRESUMEREQUEST-START

RRCResumeRequest ::=            SEQUENCE {

rrcResumeRequest            RRCResumeRequest-IEs

}

RRCResumeRequest-IEs ::=        SEQUENCE {

resumeIdentity                  ShortI-RNTI-Value,

resumeMAC-I                     BIT STRING (SIZE (16)),

resumeCause                     ResumeCause,

spare                           BIT STRING (SIZE (1))

}

-- TAG-RRCRESUMEREQUEST-STOP

-- ASN1STOP

RRCResumeRequest-IEs field descriptions

resumeCause Provides the resume cause for the RRC connection resume request as provided by the upper layers or RRC. The network is not expected to reject an RRCResumeRequest due to unknown cause value being used by the UE.

resumeIdentity UE identity to facilitate UE context retrieval at gNB.

resumeMAC-I Authentication token to facilitate UE authentication at gNB. The 16 least significant bits of the MAC-I calculated using the AS security configuration as specified in 5.3.13.3.

#### –	RRCResumeRequest1

The RRCResumeRequest1 message is used to request the resumption of a suspended RRC connection or perform an RNA update.

Signalling radio bearer: SRB0

RLC-SAP: TM

Logical channel: CCCH1

Direction: UE to Network

RRCResumeRequest1 message

-- ASN1START

-- TAG-RRCRESUMEREQUEST1-START

RRCResumeRequest1 ::=         SEQUENCE {

rrcResumeRequest1          RRCResumeRequest1-IEs

}

RRCResumeRequest1-IEs ::=    SEQUENCE {

resumeIdentity               I-RNTI-Value,

resumeMAC-I                  BIT STRING (SIZE (16)),

resumeCause                  ResumeCause,

spare                        BIT STRING (SIZE (1))

}

-- TAG-RRCRESUMEREQUEST1-STOP

-- ASN1STOP

RRCResumeRequest1-IEs field descriptions

resumeCause Provides the resume cause for the RRCResumeRequest1 as provided by the upper layers or RRC. A gNB is not expected to reject an RRCResumeRequest1 due to unknown cause value being used by the UE.

resumeIdentity UE identity to facilitate UE context retrieval at gNB.

resumeMAC-I Authentication token to facilitate UE authentication at gNB. The 16 least significant bits of the MAC-I calculated using the AS security configuration as specified in 5.3.13.3.

#### –	RRCSetup

The RRCSetup message is used to establish SRB1.

Signalling radio bearer: SRB0

RLC-SAP: TM

Logical channel: CCCH

Direction: Network to UE

RRCSetup message

-- ASN1START

-- TAG-RRCSETUP-START

RRCSetup ::=                        SEQUENCE {

rrc-TransactionIdentifier           RRC-TransactionIdentifier,

criticalExtensions                  CHOICE {

rrcSetup                            RRCSetup-IEs,

criticalExtensionsFuture            SEQUENCE {}

}

}

RRCSetup-IEs ::=                    SEQUENCE {

radioBearerConfig                   RadioBearerConfig,

masterCellGroup                     OCTET STRING (CONTAINING CellGroupConfig),

lateNonCriticalExtension            OCTET STRING                                                            OPTIONAL,

nonCriticalExtension                RRCSetup-v1700-IEs                                                      OPTIONAL

}

RRCSetup-v1700-IEs ::=              SEQUENCE {

sl-ConfigDedicatedNR-r17            SL-ConfigDedicatedNR-r16                           OPTIONAL, -- Cond L2RemoteUE

sl-L2RemoteUE-Config-r17            SL-L2RemoteUE-Config-r17                           OPTIONAL, -- Cond L2RemoteUE

nonCriticalExtension                SEQUENCE {}                                        OPTIONAL

}

-- TAG-RRCSETUP-STOP

-- ASN1STOP

RRCSetup-IEs field descriptions

masterCellGroup The network configures only the RLC bearer for the SRB1, mac-CellGroupConfig, physicalCellGroupConfig and spCellConfig.

radioBearerConfig Only SRB1 can be configured in RRC setup.

sl-ConfigDedicatedNR Contains dedicated configurations for NR sidelink communication. The network configures only the PC5 Relay RLC channel and sl-PHY-MAC-RLC-Config used for the SRB1.

sl-L2RemoteUE-Config Contains dedicated configurations used for L2 U2N relay related operation. The network configures only the SRAP configuration used for the SRB1 and local UE ID.

| Conditional Presence | Explanation |
| --- | --- |
| L2RemoteUE | The field is mandatory present for L2 U2N Remote UE; otherwise it is absent. |

#### –	RRCSetupComplete

The RRCSetupComplete message is used to confirm the successful completion of an RRC connection establishment.

Signalling radio bearer: SRB1

RLC-SAP: AM

Logical channel: DCCH

Direction: UE to Network

RRCSetupComplete message

-- ASN1START

-- TAG-RRCSETUPCOMPLETE-START

RRCSetupComplete ::=                SEQUENCE {

rrc-TransactionIdentifier           RRC-TransactionIdentifier,

criticalExtensions                  CHOICE {

rrcSetupComplete                    RRCSetupComplete-IEs,

criticalExtensionsFuture            SEQUENCE {}

}

}

RRCSetupComplete-IEs ::=            SEQUENCE {

selectedPLMN-Identity               INTEGER (1..maxPLMN),

registeredAMF                       RegisteredAMF                                   OPTIONAL,

guami-Type                          ENUMERATED {native, mapped}                     OPTIONAL,

s-NSSAI-List                        SEQUENCE (SIZE (1..maxNrofS-NSSAI)) OF S-NSSAI  OPTIONAL,

dedicatedNAS-Message                DedicatedNAS-Message,

ng-5G-S-TMSI-Value                  CHOICE {

ng-5G-S-TMSI                        NG-5G-S-TMSI,

ng-5G-S-TMSI-Part2                  BIT STRING (SIZE (9))

}                                                                                   OPTIONAL,

lateNonCriticalExtension            OCTET STRING                                    OPTIONAL,

nonCriticalExtension                RRCSetupComplete-v1610-IEs                      OPTIONAL

}

RRCSetupComplete-v1610-IEs ::=      SEQUENCE {

iab-NodeIndication-r16              ENUMERATED {true}                               OPTIONAL,

idleMeasAvailable-r16               ENUMERATED {true}                               OPTIONAL,

ue-MeasurementsAvailable-r16        UE-MeasurementsAvailable-r16                    OPTIONAL,

mobilityHistoryAvail-r16            ENUMERATED {true}                               OPTIONAL,

mobilityState-r16                   ENUMERATED {normal, medium, high, spare}        OPTIONAL,

nonCriticalExtension                RRCSetupComplete-v1690-IEs                      OPTIONAL

}

RRCSetupComplete-v1690-IEs ::=      SEQUENCE {

ul-RRC-Segmentation-r16             ENUMERATED {true}                               OPTIONAL,

nonCriticalExtension                RRCSetupComplete-v1700-IEs                      OPTIONAL

}

RRCSetupComplete-v1700-IEs ::=      SEQUENCE {

onboardingRequest-r17               ENUMERATED {true}                               OPTIONAL,

nonCriticalExtension                RRCSetupComplete-v18xy-IEs                                      OPTIONAL

}

RRCSetupComplete-v18xy-IEs ::=      SEQUENCE {

musim-CapabilityRestrictionIndication-r18     ENUMERATED {true}                               OPTIONAL,

nonCriticalExtension                SEQUENCE{}                                      OPTIONAL

}

RegisteredAMF ::=                   SEQUENCE {

plmn-Identity                       PLMN-Identity                                   OPTIONAL,

amf-Identifier                      AMF-Identifier

}

-- TAG-RRCSETUPCOMPLETE-STOP

-- ASN1STOP

RRCSetupComplete-IEs field descriptions

guami-Type This field is used to indicate whether the GUAMI included is native (derived from native 5G-GUTI) or mapped (from EPS, derived from EPS GUTI) as specified in TS 24.501 [23].

iab-NodeIndication This field is used to indicate that the connection is being established by an IAB-node as specified in TS 38.300 [2].

idleMeasAvailable Indication that the UE has idle/inactive measurement report available.

mobilityState This field indicates the UE mobility state (as defined in TS 38.304 [20], clause 5.2.4.3) just prior to UE going into RRC_CONNECTED state. The UE indicates the value of medium and high when being in Medium-mobility and High-mobility states respectively. Otherwise the UE indicates the value normal.

musim-CapabilityRestrictionIndication This field indicates the UE temporary capability restriction due to MUSIM operation.

ng-5G-S-TMSI-Part2 The leftmost 9 bits of 5G-S-TMSI.

onboardingRequest This field indicates that the connection is being established for UE onboarding in the selected onboarding SNPN, see TS 23.501 [32].

registeredAMF This field is used to transfer the GUAMI of the AMF where the UE is registered, as provided by upper layers, see TS 23.003 [21].

selectedPLMN-Identity Index of the PLMN or SNPN selected by the UE from the plmn-IdentityInfoList or npn-IdentityInfoList fields included in SIB1.

ul-RRC-Segmentation This field indicates the UE supports uplink RRC segmentation of UECapabilityInformation.

#### –	RRCSetupRequest

The RRCSetupRequest message is used to request the establishment of an RRC connection.

Signalling radio bearer: SRB0

RLC-SAP: TM

Logical channel: CCCH

Direction: UE to Network

RRCSetupRequest message

-- ASN1START

-- TAG-RRCSETUPREQUEST-START

RRCSetupRequest ::=                 SEQUENCE {

rrcSetupRequest                     RRCSetupRequest-IEs

}

RRCSetupRequest-IEs ::=             SEQUENCE {

ue-Identity                         InitialUE-Identity,

establishmentCause                  EstablishmentCause,

spare                               BIT STRING (SIZE (1))

}

InitialUE-Identity ::=              CHOICE {

ng-5G-S-TMSI-Part1                  BIT STRING (SIZE (39)),

randomValue                         BIT STRING (SIZE (39))

}

EstablishmentCause ::=              ENUMERATED {

emergency, highPriorityAccess, mt-Access, mo-Signalling,

mo-Data, mo-VoiceCall, mo-VideoCall, mo-SMS, mps-PriorityAccess, mcs-PriorityAccess,

spare6, spare5, spare4, spare3, spare2, spare1}

-- TAG-RRCSETUPREQUEST-STOP

-- ASN1STOP

RRCSetupRequest-IEs field descriptions

establishmentCause Provides the establishment cause for the RRCSetupRequest in accordance with the information received from upper layers. gNB is not expected to reject an RRCSetupRequest due to unknown cause value being used by the UE.

ue-Identity UE identity included to facilitate contention resolution by lower layers.

InitialUE-Identity field descriptions

ng-5G-S-TMSI-Part1 The rightmost 39 bits of 5G-S-TMSI.

randomValue Integer value in the range 0 to 239 – 1.

#### –	RRCSystemInfoRequest

The RRCSystemInfoRequest message is used to request SI message(s) required by the UE as specified in clause 5.2.2.3.3 and 5.2.2.3.3a.

Signalling radio bearer: SRB0

RLC-SAP: TM

Logical channel: CCCH

Direction: UE to Network

RRCSystemInfoRequest message

-- ASN1START

-- TAG-RRCSYSTEMINFOREQUEST-START

RRCSystemInfoRequest ::=            SEQUENCE {

criticalExtensions                  CHOICE {

rrcSystemInfoRequest                RRCSystemInfoRequest-IEs,

criticalExtensionsFuture-r16        CHOICE {

rrcPosSystemInfoRequest-r16         RRC-PosSystemInfoRequest-r16-IEs,

criticalExtensionsFuture            SEQUENCE {}

}

}

}

RRCSystemInfoRequest-IEs ::=    SEQUENCE {

requested-SI-List                   BIT STRING (SIZE (maxSI-Message)),  --32bits

spare                               BIT STRING (SIZE (12))

}

RRC-PosSystemInfoRequest-r16-IEs ::=  SEQUENCE {

requestedPosSI-List                   BIT STRING (SIZE (maxSI-Message)),  --32bits

spare                                 BIT STRING (SIZE (11))

}

-- TAG-RRCSYSTEMINFOREQUEST-STOP

-- ASN1STOP

RRCSystemInfoRequest-IEs field descriptions

requested-SI-List Contains a list of requested SI messages which are configured by schedulingInfoList in si-SchedulingInfo and schedulingInfoList2 in si-SchedulingInfo-v1700 (if present) in SIB1. If si-SchedulingInfo-v1700 is not present: - According to the order of entry in the list of SI messages configured by schedulingInfoList in si-SchedulingInfo in SIB1, first bit corresponds to first/leftmost listed SI message, second bit corresponds to second listed SI message, and so on. If si-SchedulingInfo-v1700 is present: - The UE generates a list of concatenated SI messages by appending the SI messages containing type1 SIB configured by schedulingInfoList2 in si-SchedulingInfo-v1700 to the SI messages configured by schedulingInfoList in si-SchedulingInfo. - According to the order of entry in the list of concatenated SI messages, first bit corresponds to first/leftmost listed SI message, second bit corresponds to second listed SI message, and so on.

requestedPosSI-List Contains a list of requested SI messages which are configured by posSchedulingInfoList in posSI-SchedulingInfo and schedulingInfoList2 in si-SchedulingInfo-v1700 (if present) in SIB1. If si-SchedulingInfo-v1700 is not present: - According to the order of entry in the list of SI messages configured by posSchedulingInfoList in posSI-SchedulingInfo in SIB1, first bit corresponds to first/leftmost listed SI message, second bit corresponds to second listed SI message, and so on. If si-SchedulingInfo-v1700 is present: - The UE creates a list of concatenated SI messages by appending the SI messages containing type2 SIB configured by schedulingInfoList2 in si-SchedulingInfo-v1700 to the SI messages configured by posSchedulingInfoList in posSI-SchedulingInfo. - According to the order of entry in the list of concatenated SI messages, first bit corresponds to first/leftmost listed SI message, second bit corresponds to second listed SI message, and so on.

---------------------------------------------------------Skip Unchanged----------------------------------------------------------

#### –	SIB1

SIB1 contains information relevant when evaluating if a UE is allowed to access a cell and defines the scheduling of other system information. It also contains radio resource configuration information that is common for all UEs and barring information applied to the unified access control.

Signalling radio bearer: N/A

RLC-SAP: TM

Logical channels: BCCH

Direction: Network to UE

SIB1 message

-- ASN1START

-- TAG-SIB1-START

SIB1 ::=        SEQUENCE {

cellSelectionInfo                   SEQUENCE {

q-RxLevMin                          Q-RxLevMin,

q-RxLevMinOffset                    INTEGER (1..8)                                              OPTIONAL,   -- Need S

q-RxLevMinSUL                       Q-RxLevMin                                                  OPTIONAL,   -- Need R

q-QualMin                           Q-QualMin                                                   OPTIONAL,   -- Need S

q-QualMinOffset                     INTEGER (1..8)                                              OPTIONAL    -- Need S

}                                                                                                   OPTIONAL,   -- Cond Standalone

cellAccessRelatedInfo               CellAccessRelatedInfo,

connEstFailureControl               ConnEstFailureControl                                           OPTIONAL,   -- Need R

si-SchedulingInfo                   SI-SchedulingInfo                                               OPTIONAL,   -- Need R

servingCellConfigCommon             ServingCellConfigCommonSIB                                      OPTIONAL,   -- Need R

ims-EmergencySupport                ENUMERATED {true}                                               OPTIONAL,   -- Need R

eCallOverIMS-Support                ENUMERATED {true}                                               OPTIONAL,   -- Need R

ue-TimersAndConstants               UE-TimersAndConstants                                           OPTIONAL,   -- Need R

uac-BarringInfo                     SEQUENCE {

uac-BarringForCommon                UAC-BarringPerCatList                                           OPTIONAL,   -- Need S

uac-BarringPerPLMN-List             UAC-BarringPerPLMN-List                                         OPTIONAL,   -- Need S

uac-BarringInfoSetList              UAC-BarringInfoSetList,

uac-AccessCategory1-SelectionAssistanceInfo CHOICE {

plmnCommon                           UAC-AccessCategory1-SelectionAssistanceInfo,

individualPLMNList                   SEQUENCE (SIZE (2..maxPLMN)) OF UAC-AccessCategory1-SelectionAssistanceInfo

}                                                                                                   OPTIONAL    -- Need S

}                                                                                                   OPTIONAL,   -- Need R

useFullResumeID                     ENUMERATED {true}                                               OPTIONAL,   -- Need R

lateNonCriticalExtension            OCTET STRING                                                    OPTIONAL,

nonCriticalExtension                SIB1-v1610-IEs                                                  OPTIONAL

}

SIB1-v1610-IEs ::=               SEQUENCE {

idleModeMeasurementsEUTRA-r16    ENUMERATED{true}                                                   OPTIONAL,  -- Need R

idleModeMeasurementsNR-r16       ENUMERATED{true}                                                   OPTIONAL,  -- Need R

posSI-SchedulingInfo-r16         PosSI-SchedulingInfo-r16                                           OPTIONAL,  -- Need R

nonCriticalExtension             SIB1-v1630-IEs                                                     OPTIONAL

}

SIB1-v1630-IEs ::=               SEQUENCE {

uac-BarringInfo-v1630            SEQUENCE {

uac-AC1-SelectAssistInfo-r16     SEQUENCE (SIZE (2..maxPLMN)) OF UAC-AC1-SelectAssistInfo-r16

}                                                                                                   OPTIONAL,  -- Need R

nonCriticalExtension             SIB1-v1700-IEs                                                     OPTIONAL

}

SIB1-v1700-IEs ::=               SEQUENCE {

hsdn-Cell-r17                        ENUMERATED {true}                                              OPTIONAL,  -- Need R

uac-BarringInfo-v1700                SEQUENCE {

uac-BarringInfoSetList-v1700         UAC-BarringInfoSetList-v1700

}                                                                                                   OPTIONAL,  -- Cond MINT

sdt-ConfigCommon-r17                 SDT-ConfigCommonSIB-r17                                        OPTIONAL,  -- Need R

redCap-ConfigCommon-r17              RedCap-ConfigCommonSIB-r17                                     OPTIONAL,  -- Need R

featurePriorities-r17        SEQUENCE {

redCapPriority-r17           FeaturePriority-r17                                                OPTIONAL,  -- Need R

slicingPriority-r17          FeaturePriority-r17                                                OPTIONAL,  -- Need R

msg3-Repetitions-Priority-r17 FeaturePriority-r17                                               OPTIONAL,  -- Need R

sdt-Priority-r17             FeaturePriority-r17                                                OPTIONAL   -- Need R

}                                                                                                   OPTIONAL,  -- Need R

si-SchedulingInfo-v1700      SI-SchedulingInfo-v1700                                                OPTIONAL,  -- Need R

hyperSFN-r17                 BIT STRING (SIZE (10))                                                 OPTIONAL,  -- Need R

eDRX-AllowedIdle-r17         ENUMERATED {true}                                                      OPTIONAL,  -- Need R

eDRX-AllowedInactive-r17     ENUMERATED {true}                                                      OPTIONAL,  -- Cond EDRX-RC

intraFreqReselectionRedCap-r17 ENUMERATED {allowed, notAllowed}                                     OPTIONAL,  -- Need S

cellBarredNTN-r17            ENUMERATED {barred, notBarred}                                         OPTIONAL,  -- Need S

nonCriticalExtension         SIB1-v1740-IEs                                                         OPTIONAL

}

SIB1-v1740-IEs ::=               SEQUENCE {

si-SchedulingInfo-v1740          SI-SchedulingInfo-v1740                                            OPTIONAL,  -- Need R

nonCriticalExtension             SIB1-v18xy-IEs                                                        OPTIONAL

}

SIB1-v18xy-IEs ::=               SEQUENCE {

musim-CapabilityRestriction-r18          ENUMERATED {true}                                          OPTIONAL,  -- Need R

nonCriticalExtension             SEQUENCE {}                                                        OPTIONAL

}

UAC-AccessCategory1-SelectionAssistanceInfo ::=    ENUMERATED {a, b, c}

UAC-AC1-SelectAssistInfo-r16 ::=     ENUMERATED {a, b, c, notConfigured}

SDT-ConfigCommonSIB-r17 ::=          SEQUENCE {

sdt-RSRP-Threshold-r17               RSRP-Range                                                            OPTIONAL, -- Need R

sdt-LogicalChannelSR-DelayTimer-r17  ENUMERATED { sf20, sf40, sf64, sf128, sf512, sf1024, sf2560, spare1}  OPTIONAL, -- Need R

sdt-DataVolumeThreshold-r17          ENUMERATED {byte32, byte100, byte200, byte400, byte600, byte800, byte1000, byte2000, byte4000,

byte8000, byte9000, byte10000, byte12000, byte24000, byte48000, byte96000},

t319a-r17                            ENUMERATED { ms100, ms200, ms300, ms400, ms600, ms1000, ms2000,

ms3000, ms4000, spare7, spare6, spare5, spare4, spare3, spare2, spare1}

}

RedCap-ConfigCommonSIB-r17 ::= SEQUENCE {

halfDuplexRedCapAllowed-r17    ENUMERATED {true}                                                    OPTIONAL,  -- Need R

cellBarredRedCap-r17           SEQUENCE {

cellBarredRedCap1Rx-r17        ENUMERATED {barred, notBarred},

cellBarredRedCap2Rx-r17        ENUMERATED {barred, notBarred}

}                                                                                                   OPTIONAL,  -- Need R

...

}

FeaturePriority-r17 ::= INTEGER (0..7)

-- TAG-SIB1-STOP

-- ASN1STOP

SIB1 field descriptions

cellBarredNTN Value barred means that the cell is barred for connectivity to NTN, as defined in TS 38.304 [20]. Value notBarred means that the cell is allowed for connectivity to NTN. If not present, the UE considers the cell is not allowed for connectivity to NTN, as defined in TS 38.304 [20]. This field is only applicable to NTN-capable UEs.

cellBarredRedCap1Rx Value barred means that the cell is barred for a RedCap UE with 1 Rx branch, as defined in TS 38.304 [20]. This field is ignored by non-RedCap UEs.

cellBarredRedCap2Rx Value barred means that the cell is barred for a RedCap UE with 2 Rx branches, as defined in TS 38.304 [20]. This field is ignored by non-RedCap UEs.

cellSelectionInfo Parameters for cell selection related to the serving cell.

eCallOverIMS-Support Indicates whether the cell supports eCall over IMS services as defined in TS 23.501 [32]. If absent, eCall over IMS is not supported by the network in the cell.

eDRX-AllowedIdle The presence of this field indicates that extended DRX for CN paging is allowed in the cell for UEs in RRC_IDLE or RRC_INACTIVE. The UE shall stop using extended DRX for CN paging in RRC_IDLE or RRC_INACTIVE if eDRX-AllowedIdle is not present.

eDRX-AllowedInactive The presence of this field indicates that extended DRX for RAN paging is allowed in the cell for UEs in RRC_INACTIVE. The UE shall stop using extended DRX for RAN paging in RRC_INACTIVE if eDRX-AllowedInactive is not present.

featurePriorities Indicates priorities for features, such as RedCap, Slicing, SDT and MSG3-Repetitions for Coverage Enhancements. These priorities are used to determine which FeatureCombinationPreambles the UE shall use when a feature maps to more than one FeatureCombinationPreambles, as specified in TS 38.321 [3]. A lower value means a higher priority. The network does not signal the same priority for more than one feature. The network signals a priority for all feature that map to at least one FeatureCombinationPreambles.

halfDuplexRedCap-Allowed The presence of this field indicates that the cell supports half-duplex FDD RedCap UEs.

hsdn-Cell This field indicates this is a HSDN cell as specified in TS 38.304 [20].

hyperSFN Indicates hyper SFN which increments by one when the SFN wraps around. This field is excluded when determining changes in system information, i.e. changes of hyper SFN should not result in system information change notifications.

idleModeMeasurementsEUTRA This field indicates that a UE that is configured for EUTRA idle/inactive measurements shall perform the measurements while camping in this cell and report availability of these measurements when establishing or resuming a connection in this cell. If absent, a UE is not required to perform EUTRA idle/inactive measurements.

idleModeMeasurementsNR This field indicates that a UE that is configured for NR idle/inactive measurements shall perform the measurements while camping in this cell and report availability of these measurements when establishing or resuming a connection in this cell. If absent, a UE is not required to perform NR idle/inactive measurements.

ims-EmergencySupport Indicates whether the cell supports IMS emergency bearer services for UEs in limited service mode. If absent, IMS emergency call is not supported by the network in the cell for UEs in limited service mode.

intraFreqReselectionRedCap Controls cell selection/reselection to intra-frequency cells for RedCap UEs when this cell is barred, or treated as barred by the RedCap UE, as specified in TS 38.304 [20]. If not present, a RedCap UE treats the cell as barred, i.e.,the UE considers that the cell does not support RedCap.

musim-CapabilityRestriction Indicates the UE is allowed to sends indication of MUSIM temporary capability restriction during RRC setup and RRC resume procedure.

q-QualMin Parameter "Qqualmin" in TS 38.304 [20], applicable for serving cell. If the field is absent, the UE applies the (default) value of negative infinity for Qqualmin.

q-QualMinOffset Parameter "Qqualminoffset" in TS 38.304 [20]. Actual value Qqualminoffset = field value [dB]. If the field is absent, the UE applies the (default) value of 0 dB for Qqualminoffset. Affects the minimum required quality level in the cell.

q-RxLevMin Parameter "Qrxlevmin" in TS 38.304 [20], applicable for serving cell.

q-RxLevMinOffset Parameter "Qrxlevminoffset" in TS 38.304 [20]. Actual value Qrxlevminoffset = field value * 2 [dB]. If absent, the UE applies the (default) value of 0 dB for Qrxlevminoffset. Affects the minimum required Rx level in the cell.

q-RxLevMinSUL Parameter "Qrxlevmin" in TS 38.304 [20], applicable for serving cell.

sdt-RSRP-Threshold RSRP threshold used to determine whether SDT procedure can be initiated, as specified in TS 38.321 [3].

sdt-DataVolumeThreshold Data volume threshold used to determine whether SDT can be initiated, as specified in TS 38.321 [3]. Value byte32 corresponds to 32 bytes, value byte100 corresponds to 100 bytes, and so on.

sdt-LogicalChannelSR-DelayTimer The value of logicalChannelSR-DelayTimer applied during SDT for logical channels configured with SDT, as specified in TS 38.321 [3]. Value in number of subframes. Value sf20 corresponds to 20 subframes, sf40 corresponds to 40 subframes, and so on. If this field is not configured, then logicalChannelSR-DelayTimer is not applied for SDT logical channels.

servingCellConfigCommon Configuration of the serving cell.

t319a Initial value of the timer T319a used for detection of SDT failure. Value ms100 corresponds to 100 milliseconds, value ms200 corresponds to 200 milliseconds and so on.

uac-AccessCategory1-SelectionAssistanceInfo Information used to determine whether Access Category 1 applies to the UE, as defined in TS 22.261 [25]. If plmnCommon is chosen, the UAC-AccessCategory1-SelectionAssistanceInfo is applicable to all the PLMNs and SNPNs in plmn-IdentityInfoList and npn-IdentityInfoList. If individualPLMNList is chosen, the 1st entry in the list corresponds to the first network within all of the PLMNs and SNPNs across the plmn-IdentityList and the npn-IdentityInfoList, the 2nd entry in the list corresponds to the second network within all of the PLMNs and SNPNs across the plmn-IdentityList and the npn-IdentityInfoList and so on. If uac-AC1-SelectAssistInfo-r16 is present, the UE shall ignore the uac-AccessCategory1-SelectionAssistanceInfo.

uac-AC1-SelectAssistInfo Information used to determine whether Access Category 1 applies to the UE, as defined in TS 22.261 [25]. The 1st entry in the list corresponds to the first network within all of the PLMNs and SNPNs across the plmn-IdentityList and npn-IdentityInfoList, the 2nd entry in the list corresponds to the second network within all of the PLMNs and SNPNs across the plmn-IdentityList and the npn-IdentityInfoList and so on. Value notConfigured indicates that Access Category1 is not configured for the corresponding PLMN/SNPN.

uac-BarringForCommon Common access control parameters for each access category. Common values are used for all PLMNs/SNPNs, unless overwritten by the PLMN/SNPN specific configuration provided in uac-BarringPerPLMN-List. The parameters are specified by providing an index to the set of configurations (uac-BarringInfoSetList). UE behaviour upon absence of this field is specified in clause 5.3.14.2.

ue-TimersAndConstants Timer and constant values to be used by the UE. The cell operating as PCell always provides this field.

useFullResumeID Indicates which resume identifier and Resume request message should be used. UE uses fullI-RNTI and RRCResumeRequest1 if the field is present, or shortI-RNTI and RRCResumeRequest if the field is absent.

| Conditional Presence | Explanation |
| --- | --- |
| EDRX-RC | The field is optionally present, Need R, in a cell that enables eDRX-AllowedIdle, otherwise it is absent. |
| MINT | The field is optionally present, Need R, in a cell that provides a configuration for disaster roaming, otherwise it is absent, Need R. |
| Standalone | The field is mandatory present in a cell that supports standalone operation, otherwise it is absent. |

---------------------------------------------------------Skip Unchanged----------------------------------------------------------

#### –	UEAssistanceInformation

The UEAssistanceInformation message is used for the indication of UE assistance information to the network.

Signalling radio bearer: SRB1, SRB3

RLC-SAP: AM

Logical channel: DCCH

Direction: UE to Network

UEAssistanceInformation message

-- ASN1START

-- TAG-UEASSISTANCEINFORMATION-START

UEAssistanceInformation ::=         SEQUENCE {

criticalExtensions                  CHOICE {

ueAssistanceInformation             UEAssistanceInformation-IEs,

criticalExtensionsFuture            SEQUENCE {}

}

}

UEAssistanceInformation-IEs ::=     SEQUENCE {

delayBudgetReport                   DelayBudgetReport                   OPTIONAL,

lateNonCriticalExtension            OCTET STRING                        OPTIONAL,

nonCriticalExtension                UEAssistanceInformation-v1540-IEs   OPTIONAL

}

DelayBudgetReport::=                CHOICE {

type1                               ENUMERATED {

msMinus1280, msMinus640, msMinus320, msMinus160,msMinus80, msMinus60, msMinus40,

msMinus20, ms0, ms20,ms40, ms60, ms80, ms160, ms320, ms640, ms1280},

...

}

UEAssistanceInformation-v1540-IEs ::= SEQUENCE {

overheatingAssistance               OverheatingAssistance               OPTIONAL,

nonCriticalExtension                UEAssistanceInformation-v1610-IEs   OPTIONAL

}

OverheatingAssistance ::=           SEQUENCE {

reducedMaxCCs                       ReducedMaxCCs-r16                   OPTIONAL,

reducedMaxBW-FR1                    ReducedMaxBW-FRx-r16                OPTIONAL,

reducedMaxBW-FR2                    ReducedMaxBW-FRx-r16                OPTIONAL,

reducedMaxMIMO-LayersFR1            SEQUENCE {

reducedMIMO-LayersFR1-DL            MIMO-LayersDL,

reducedMIMO-LayersFR1-UL            MIMO-LayersUL

} OPTIONAL,

reducedMaxMIMO-LayersFR2            SEQUENCE {

reducedMIMO-LayersFR2-DL            MIMO-LayersDL,

reducedMIMO-LayersFR2-UL            MIMO-LayersUL

} OPTIONAL

}

OverheatingAssistance-r17 ::=       SEQUENCE {

reducedMaxBW-FR2-2-r17              SEQUENCE {

reducedBW-FR2-2-DL-r17              ReducedAggregatedBandwidth-r17,

reducedBW-FR2-2-UL-r17              ReducedAggregatedBandwidth-r17

} OPTIONAL,

reducedMaxMIMO-LayersFR2-2          SEQUENCE {

reducedMIMO-LayersFR2-2-DL          MIMO-LayersDL,

reducedMIMO-LayersFR2-2-UL          MIMO-LayersUL

} OPTIONAL

}

ReducedAggregatedBandwidth ::= ENUMERATED {mhz0, mhz10, mhz20, mhz30, mhz40, mhz50, mhz60, mhz80, mhz100, mhz200, mhz300, mhz400}

ReducedAggregatedBandwidth-r17 ::= ENUMERATED {mhz0, mhz100, mhz200, mhz400, mhz800, mhz1200, mhz1600, mhz2000}

UEAssistanceInformation-v1610-IEs ::= SEQUENCE {

idc-Assistance-r16                  IDC-Assistance-r16                  OPTIONAL,

drx-Preference-r16                  DRX-Preference-r16                  OPTIONAL,

maxBW-Preference-r16                MaxBW-Preference-r16                OPTIONAL,

maxCC-Preference-r16                MaxCC-Preference-r16                OPTIONAL,

maxMIMO-LayerPreference-r16         MaxMIMO-LayerPreference-r16         OPTIONAL,

minSchedulingOffsetPreference-r16   MinSchedulingOffsetPreference-r16   OPTIONAL,

releasePreference-r16               ReleasePreference-r16               OPTIONAL,

sl-UE-AssistanceInformationNR-r16   SL-UE-AssistanceInformationNR-r16   OPTIONAL,

referenceTimeInfoPreference-r16     BOOLEAN                             OPTIONAL,

nonCriticalExtension                UEAssistanceInformation-v1700-IEs   OPTIONAL

}

UEAssistanceInformation-v1700-IEs ::= SEQUENCE {

ul-GapFR2-Preference-r17              UL-GapFR2-Preference-r17              OPTIONAL,

musim-Assistance-r17                  MUSIM-Assistance-r17                  OPTIONAL,

overheatingAssistance-r17             OverheatingAssistance-r17             OPTIONAL,

maxBW-PreferenceFR2-2-r17             MaxBW-PreferenceFR2-2-r17             OPTIONAL,

maxMIMO-LayerPreferenceFR2-2-r17      MaxMIMO-LayerPreferenceFR2-2-r17      OPTIONAL,

minSchedulingOffsetPreferenceExt-r17  MinSchedulingOffsetPreferenceExt-r17  OPTIONAL,

rlm-MeasRelaxationState-r17           BOOLEAN                               OPTIONAL,

bfd-MeasRelaxationState-r17           BIT STRING (SIZE (1..maxNrofServingCells)) OPTIONAL,

nonSDT-DataIndication-r17             SEQUENCE {

resumeCause-r17                       ResumeCause                       OPTIONAL

}                                                                           OPTIONAL,

scg-DeactivationPreference-r17        ENUMERATED { scgDeactivationPreferred, noPreference }    OPTIONAL,

uplinkData-r17                        ENUMERATED { true }                   OPTIONAL,

rrm-MeasRelaxationFulfilment-r17      BOOLEAN                               OPTIONAL,

propagationDelayDifference-r17        PropagationDelayDifference-r17        OPTIONAL,

nonCriticalExtension                  UEAssistanceInformation-v18xy-IEs     OPTIONAL

}

UEAssistanceInformation-v18xy-IEs ::= SEQUENCE {

musim-Assistance-v18xy                  MUSIM-Assistance-v18xy                  OPTIONAL,

nonCriticalExtension                  SEQUENCE {}                           OPTIONAL

}

IDC-Assistance-r16 ::=                  SEQUENCE {

affectedCarrierFreqList-r16             AffectedCarrierFreqList-r16               OPTIONAL,

affectedCarrierFreqCombList-r16         AffectedCarrierFreqCombList-r16           OPTIONAL,

...

}

AffectedCarrierFreqList-r16 ::= SEQUENCE (SIZE (1.. maxFreqIDC-r16)) OF AffectedCarrierFreq-r16

AffectedCarrierFreq-r16 ::=     SEQUENCE {

carrierFreq-r16                 ARFCN-ValueNR,

interferenceDirection-r16       ENUMERATED {nr, other, both, spare}

}

AffectedCarrierFreqCombList-r16 ::= SEQUENCE (SIZE (1..maxCombIDC-r16)) OF AffectedCarrierFreqComb-r16

AffectedCarrierFreqComb-r16 ::=     SEQUENCE {

affectedCarrierFreqComb-r16         SEQUENCE (SIZE (2..maxNrofServingCells)) OF  ARFCN-ValueNR    OPTIONAL,

victimSystemType-r16                VictimSystemType-r16

}

VictimSystemType-r16 ::=    SEQUENCE {

gps-r16                     ENUMERATED {true}        OPTIONAL,

glonass-r16                 ENUMERATED {true}        OPTIONAL,

bds-r16                     ENUMERATED {true}        OPTIONAL,

galileo-r16                 ENUMERATED {true}        OPTIONAL,

navIC-r16                   ENUMERATED {true}        OPTIONAL,

wlan-r16                    ENUMERATED {true}        OPTIONAL,

bluetooth-r16               ENUMERATED {true}        OPTIONAL,

...

}

DRX-Preference-r16 ::=              SEQUENCE {

preferredDRX-InactivityTimer-r16    ENUMERATED {

ms0, ms1, ms2, ms3, ms4, ms5, ms6, ms8, ms10, ms20, ms30, ms40, ms50, ms60, ms80,

ms100, ms200, ms300, ms500, ms750, ms1280, ms1920, ms2560, spare9, spare8,

spare7, spare6, spare5, spare4, spare3, spare2, spare1} OPTIONAL,

preferredDRX-LongCycle-r16          ENUMERATED {

ms10, ms20, ms32, ms40, ms60, ms64, ms70, ms80, ms128, ms160, ms256, ms320, ms512,

ms640, ms1024, ms1280, ms2048, ms2560, ms5120, ms10240, spare12, spare11, spare10,

spare9, spare8, spare7, spare6, spare5, spare4, spare3, spare2, spare1 } OPTIONAL,

preferredDRX-ShortCycle-r16         ENUMERATED {

ms2, ms3, ms4, ms5, ms6, ms7, ms8, ms10, ms14, ms16, ms20, ms30, ms32,

ms35, ms40, ms64, ms80, ms128, ms160, ms256, ms320, ms512, ms640, spare9,

spare8, spare7, spare6, spare5, spare4, spare3, spare2, spare1 } OPTIONAL,

preferredDRX-ShortCycleTimer-r16    INTEGER (1..16)    OPTIONAL

}

MaxBW-Preference-r16 ::=            SEQUENCE {

reducedMaxBW-FR1-r16                ReducedMaxBW-FRx-r16                     OPTIONAL,

reducedMaxBW-FR2-r16                ReducedMaxBW-FRx-r16                     OPTIONAL

}

MaxBW-PreferenceFR2-2-r17 ::=       SEQUENCE {

reducedMaxBW-FR2-2-r17              SEQUENCE {

reducedBW-FR2-2-DL-r17              ReducedAggregatedBandwidth-r17       OPTIONAL,

reducedBW-FR2-2-UL-r17              ReducedAggregatedBandwidth-r17       OPTIONAL

} OPTIONAL

}

MaxCC-Preference-r16 ::=            SEQUENCE {

reducedMaxCCs-r16                   ReducedMaxCCs-r16                        OPTIONAL

}

MaxMIMO-LayerPreference-r16 ::=     SEQUENCE {

reducedMaxMIMO-LayersFR1-r16        SEQUENCE {

reducedMIMO-LayersFR1-DL-r16        INTEGER (1..8),

reducedMIMO-LayersFR1-UL-r16        INTEGER (1..4)

} OPTIONAL,

reducedMaxMIMO-LayersFR2-r16        SEQUENCE {

reducedMIMO-LayersFR2-DL-r16        INTEGER (1..8),

reducedMIMO-LayersFR2-UL-r16        INTEGER (1..4)

} OPTIONAL

}

MaxMIMO-LayerPreferenceFR2-2-r17 ::=    SEQUENCE {

reducedMaxMIMO-LayersFR2-2-r17          SEQUENCE {

reducedMIMO-LayersFR2-2-DL-r17          INTEGER (1..8),

reducedMIMO-LayersFR2-2-UL-r17          INTEGER (1..4)

} OPTIONAL

}

MinSchedulingOffsetPreference-r16 ::= SEQUENCE {

preferredK0-r16                       SEQUENCE {

preferredK0-SCS-15kHz-r16             ENUMERATED {sl1, sl2, sl4, sl6}              OPTIONAL,

preferredK0-SCS-30kHz-r16             ENUMERATED {sl1, sl2, sl4, sl6}              OPTIONAL,

preferredK0-SCS-60kHz-r16             ENUMERATED {sl2, sl4, sl8, sl12}             OPTIONAL,

preferredK0-SCS-120kHz-r16            ENUMERATED {sl2, sl4, sl8, sl12}             OPTIONAL

}                                                                                  OPTIONAL,

preferredK2-r16                       SEQUENCE {

preferredK2-SCS-15kHz-r16             ENUMERATED {sl1, sl2, sl4, sl6}             OPTIONAL,

preferredK2-SCS-30kHz-r16             ENUMERATED {sl1, sl2, sl4, sl6}             OPTIONAL,

preferredK2-SCS-60kHz-r16             ENUMERATED {sl2, sl4, sl8, sl12}            OPTIONAL,

preferredK2-SCS-120kHz-r16            ENUMERATED {sl2, sl4, sl8, sl12}            OPTIONAL

}                                                                                 OPTIONAL

}

MinSchedulingOffsetPreferenceExt-r17 ::=  SEQUENCE {

preferredK0-r17                           SEQUENCE {

preferredK0-SCS-480kHz-r17                ENUMERATED {sl8, sl16, sl32, sl48}      OPTIONAL,

preferredK0-SCS-960kHz-r17                ENUMERATED {sl8, sl16, sl32, sl48}      OPTIONAL

}                                                                                     OPTIONAL,

preferredK2-r17                           SEQUENCE {

preferredK2-SCS-480kHz-r17                ENUMERATED {sl8, sl16, sl32, sl48}      OPTIONAL,

preferredK2-SCS-960kHz-r17                ENUMERATED {sl8, sl16, sl32, sl48}      OPTIONAL

}                                                                                     OPTIONAL

}

MUSIM-Assistance-r17 ::=              SEQUENCE {

musim-PreferredRRC-State-r17          ENUMERATED {idle, inactive, outOfConnected}     OPTIONAL,

musim-GapPreferenceList-r17           MUSIM-GapPreferenceList-r17                     OPTIONAL

}

MUSIM-GapPreferenceList-r17 ::= SEQUENCE (SIZE (1..4)) OF MUSIM-GapInfo-r17

MUSIM-Assistance-v18xy ::=              SEQUENCE {

musim-GapPriorityPreferenceList-r18           MUSIM-GapPriorityPreferenceList-r18     OPTIONAL,

musim-GapPriorityKeep-r18                     ENUMERATED {true}     OPTIONAL,

musim-CapRestriction-r18                      MUSIM-CapRestriction-r18     OPTIONAL,

musim-NeedForGapsInfoNR-r18                   NeedForGapsInfoNR-r16       OPTIONAL

}

MUSIM-GapPriorityPreferenceList-r18 ::= SEQUENCE (SIZE (1..3)) OF GapPriority-r17

MUSIM-CapRestriction-r18 ::=                SEQUENCE {

musim-Cell-SCG-ToRelease-r18              MUSIM-Cell-SCG-ToRelease-r18     OPTIONAL,

musim-CellToAffectList-r18                MUSIM-CellToAffectList-r18     OPTIONAL,

musim-AffectedBandCombList-r18            MUSIM-AffectedBandCombList-r18     OPTIONAL,

musim-ForbiddenBandCombList-r18           MUSIM-ForbiddenBandCombList-r18     OPTIONAL

}

MUSIM-Cell-SCG-ToRelease-r18 ::=              SEQUENCE {

musim-CellToRelease-r18           MUSIM-CellToRelease-r18     OPTIONAL,

scg-ReleasePreference-r18         ENUMERATED { scgReleasePreferred }     OPTIONAL

}

MUSIM-CellToRelease-r18 ::=         SEQUENCE (SIZE (1..maxNrofSCells)) OF SCellIndex

MUSIM-CellToAffectList-r18::=             SEQUENCE (SIZE (1..maxNrofSCells)) OF MUSIM-CellToAffect-r18                  OPTIONAL   -- Need N

MUSIM-CellToAffect-r18 ::=           SEQUENCE {

musim-SCellIndex-r18                  SCellIndex,

musim-MIMO-Layers-DL-r18              INTEGER (1..8)          OPTIONAL,

musim-MIMO-Layers-UL-r18              INTEGER (1..4)          OPTIONAL

}

MUSIM-AffectedBandCombList-r18  ::=         SEQUENCE (SIZE (1..maxBandComb)) OF MUSIM-AffectedBandComb-r18

MUSIM-AffectedBandComb-r18  ::=             SEQUENCE (SIZE (1.. maxSimultaneousBands)) OF MUSIM-CapabilityRestrictedBandParameters OPTIONAL

MUSIM-CapabilityRestrictedBandParameters               SEQUENCE {

bandEntryIndex       INTEGER(1..maxSimultaneousBands),

musim-capabilityRestricted-r18   SEQUENCE {

musim-MIMO-Layers-DL-r18              INTEGER (1..8)              OPTIONAL,

musim-MIMO-Layers-UL-r18              INTEGER (1..4)             OPTIONAL

}                                                                    OPTIONAL

}

MUSIM-ForbiddenBandCombList-r18::=         SEQUENCE (SIZE (1.. maxBandComb)) OF MUSIM-ForbiddenBandComb-r18                  OPTIONAL   -- Need N

MUSIM-ForbiddenBandComb-r18 ::=              SEQUENCE (SIZE (1.. maxSimultaneousBands)) OF BandEntryIndex

BandEntryIndex       ::= INTEGER(1..maxSimultaneousBands)

Editor’s note: The UL/DL supported bandwidth restriction (if supported) shall work for the MUSIM-BandToAffect-r18, and the granularity is FFS.

ReleasePreference-r16 ::=           SEQUENCE {

preferredRRC-State-r16              ENUMERATED {idle, inactive, connected, outOfConnected}

}

ReducedMaxBW-FRx-r16 ::=            SEQUENCE {

reducedBW-DL-r16                    ReducedAggregatedBandwidth,

reducedBW-UL-r16                    ReducedAggregatedBandwidth

}

ReducedMaxCCs-r16 ::=               SEQUENCE {

reducedCCsDL-r16                    INTEGER (0..31),

reducedCCsUL-r16                    INTEGER (0..31)

}

SL-UE-AssistanceInformationNR-r16 ::= SEQUENCE (SIZE (1..maxNrofTrafficPattern-r16)) OF SL-TrafficPatternInfo-r16

SL-TrafficPatternInfo-r16::=          SEQUENCE {

trafficPeriodicity-r16                ENUMERATED {ms20, ms50, ms100, ms200, ms300, ms400, ms500, ms600, ms700, ms800, ms900, ms1000},

timingOffset-r16                      INTEGER (0..10239),

messageSize-r16                       BIT STRING (SIZE (8)),

sl-QoS-FlowIdentity-r16               SL-QoS-FlowIdentity-r16

}

UL-GapFR2-Preference-r17::=           SEQUENCE {

ul-GapFR2-PatternPreference-r17       INTEGER (0..3)                     OPTIONAL

}

PropagationDelayDifference-r17 ::=  SEQUENCE (SIZE (1..4)) OF INTEGER (-270..270)

-- TAG-UEASSISTANCEINFORMATION-STOP

-- ASN1STOP

Editor's note: The value range for ReducedAggregatedBandwidth-r17 needs RAN4 confirmation

Editor's note: The value range for preferred K0/K2 for SCS 960 kHz needs RAN1 confirmation

UEAssistanceInformation field descriptions

affectedCarrierFreqList Indicates a list of NR carrier frequencies that are affected by IDC problem.

affectedCarrierFreqCombList Indicates a list of NR carrier frequencie combinations that are affected by IDC problems due to Inter-Modulation Distortion and harmonics from NR when configured with UL CA.

bfd-MeasRelaxationState Indicates the relaxation state of BFD measurements. Each bit corresponds to a serving cell of the cell group. A serving cell is mapped to the (servCellIndex+1)-th bit, starting from MSB. A bit that is set to 1 indicates that the UE is performing BFD measurements relaxation on the serving cell mapped on the bit. A bit that is set to 0 indicates that the UE is not performing BFD measurements relaxation on the serving cell mapped on the bit. If a serving cell is not configured to the UE, the corresponding bit is set to 0.

delayBudgetReport Indicates the UE-preferred adjustment to connected mode DRX.

interferenceDirection Indicates the direction of IDC interference. Value nr indicates that only NR is victim of IDC interference, value other indicates that only another radio is victim of IDC interference and value both indicates that both NR and another radio are victims of IDC interference. The other radio refers to either the ISM radio or GNSS (see TR 36.816 [44]).

minSchedulingOffsetPreference Indicates the UE's preferences on minimumSchedulingOffset of cross-slot scheduling for power saving.

minSchedulingOffsetPreferenceExt Indicates the UE's preferences on minimumSchedulingOffset of cross-slot scheduling for power saving for SCS 480 kHz and/or 960 kHz.

musim-Cell-SCG-ToReleasedList Indicates the UE's preference on serving cell(s) and/or SCG to be released for MUSIM purpose.

musim-CellToAffectList-r18 Indicates the UE’s preference on the temporary capability restriction on the serving cell(s) for MUSIM purpose.

musim-AffectedBandCombList Indicates the UE’s preference on the affected band combinations for MUSIM purpose.

musim-BandToAffectList Indicates the UE’s preference on the temporary capability restriction on the affected bands for MUSIM purpose.

musim-ForbiddenBandCombList Indicates the UE’s preference on the forbidden band(s) and or band combination for MUSIM purpose.

musim-GapPreferenceList Indicates the UE's MUSIM gap preference and related MUSIM gap configuration, as defined in TS 38.133 [14] clause 9.1.10.

musim-GapPriorityKeep Indicates the UE's preference to keep all collided gaps for requested MUSIM gaps. If the field is absent, the collided MUSIM gaps with lower priority shall be dropped.

musim-GapPriorityPreferenceList Indicates the UE's MUSIM gap priority preference for periodic MUSIM gaps as specified in TS 38.133. If the UE includes musim-GapPriorityPreferenceList-r18, it includes the same number of entries, and listed in the same order for periodic gaps, as in musim-GapPreferenceList-r17.

musim-needForGapsInfoNR This field is used to indicate the measurement gap requirement information of the UE for NR target bands.

musim-PreferredRRC-State Indicates the UE's preferred RRC state when leaving RRC_CONNECTED.

nonSDT-DataIndication Informs the network about the arrival of data and/or signaling mapped to radio bearers not configured for SDT while SDT procedure is ongoing.

preferredDRX-InactivityTimer Indicates the UE's preferred DRX inactivity timer length for power saving. Value in ms (milliSecond). ms0 corresponds to 0, ms1 corresponds to 1 ms, ms2 corresponds to 2 ms, and so on. If the field is absent from the DRX-Preference IE, it is interpreted as the UE having no preference for the DRX inactivity timer. If secondary DRX group is configured, the preferredDRX-InactivityTimer only applies to the default DRX group.

preferredDRX-LongCycle Indicates the UE's preferred long DRX cycle length for power saving. Value in ms. ms10 corresponds to 10ms, ms20 corresponds to 20 ms, ms32 corresponds to 32 ms, and so on. If preferredDRX-ShortCycle is provided, the value of preferredDRX-LongCycle shall be a multiple of the preferredDRX-ShortCycle value. If the field is absent from the DRX-Preference IE, it is interpreted as the UE having no preference for the long DRX cycle.

preferredDRX-ShortCycle Indicates the UE's preferred short DRX cycle length for power saving. Value in ms. ms2 corresponds to 2ms, ms3 corresponds to 3 ms, ms4 corresponds to 4 ms, and so on. If the field is absent from the DRX-Preference IE, it is interpreted as the UE having no preference for the short DRX cycle.

preferredDRX-ShortCycleTimer Indicates the UE's preferred short DRX cycle timer for power saving. Value in multiples of preferredDRX-ShortCycle. A value of 1 corresponds to preferredDRX-ShortCycle, a value of 2 corresponds to 2 * preferredDRX-ShortCycle and so on. If the field is absent from the DRX-Preference IE, it is interpreted as the UE having no preference for the short DRX cycle timer. A preference for the short DRX cycle is indicated when a preference for the short DRX cycle timer is indicated.

preferredK0 Indicates the UE's preferred value of k0 (slot offset between DCI and its scheduled PDSCH - see TS 38.214 [19], clause 5.1.2.1) for cross-slot scheduling for power saving. Value is defined for each subcarrier spacing (numerology) in units of slots. sl1 corresponds to 1 slot, sl2 corresponds to 2 slots, sl4 corresponds to 4 slots, and so on. If a value for a subcarrier spacing is absent, it is interpreted as the UE having no preference on k0 for cross-slot scheduling for that subcarrier spacing. If the field is absent from the MinSchedulingOffsetPreference IE, it is interpreted as the UE having no preference on k0 for cross-slot scheduling.

preferredK2 Indicates the UE's preferred value of k2 (slot offset between DCI and its scheduled PUSCH - see TS 38.214 [19], clause 6.1.2.1) for cross-slot scheduling for power saving. Value is defined for each subcarrier spacing (numerology) in units of slots. sl1 corresponds to 1 slot, sl2 corresponds to 2 slots, sl4 corresponds to 4 slots, and so on. If a value for a subcarrier spacing is absent, it is interpreted as the UE having no preference on k2 for cross-slot scheduling for that subcarrier spacing. If the field is absent from the MinSchedulingOffsetPreference IE, it is interpreted as the UE having no preference on k2 for cross-slot scheduling.

preferredRRC-State Indicates the UE's preferred RRC state. The value idle is indicated if the UE prefers to be released from RRC_CONNECTED and transition to RRC_IDLE. The value inactive is indicated if the UE prefers to be released from RRC_CONNECTED and transition to RRC_INACTIVE. The value connected is indicated if the UE prefers to revert an earlier indication to leave RRC_CONNECTED state. The value outOfConnected is indicated if the UE prefers to be released from RRC_CONNECTED and has no preferred RRC state to transition to. The value connected can only be indicated if the UE is configured with connectedReporting.

propagationDelayDifference Indicates the service link propagation delay difference between serving cell and each neighbour cell included in neighCellInfoList, defined as neighbour cell's service link propagation delay minus serving cell's service link propagation delay, in number of ms. First entry in propagationDelayDifference corresponds to first entry in neighCellInfoList, second entry in propagationDelayDifference corresponds to second entry in neighCellInfoList, and so on.

reducedBW-FR1 Indicates the UE's preference on reduced configuration corresponding to the maximum aggregated bandwidth across all downlink carrier(s) and across all uplink carrier(s) of FR1, to address overheating or power saving. This field is allowed to be reported only when UE is configured with serving cell(s) operating on FR1. The aggregated bandwidth across all downlink carrier(s) of FR1 is the sum of bandwidth of active downlink BWP(s) across all activated downlink carrier(s) of FR1. The aggregated bandwidth across all uplink carrier(s) of FR1 is the sum of bandwidth of active uplink BWP(s) across all activated uplink carrier(s) of FR1. If the field is absent from the MaxBW-Preference IE or the OverheatingAssistance IE, it is interpreted as the UE having no preference on the maximum aggregated bandwidth of FR1. When indicated to address overheating, this maximum aggregated bandwidth includes carrier(s) of FR1 of both the NR MCG and the SCG. This maximum aggregated bandwidth only includes carriers of FR1 of the SCG in (NG)EN-DC. Value mhz0 is not used when indicated to address overheating. When indicated to address power saving, this maximum aggregated bandwidth includes carrier(s) of FR1 of the cell group that this UE assistance information is associated with. The aggregated bandwidth can only range up to the current active configuration when indicated to address power savings.

reducedBW-FR2 Indicates the UE's preference on reduced configuration corresponding to the maximum aggregated bandwidth across all downlink carrier(s) and across all uplink carrier(s) of FR2-1, to address overheating or power saving. This field is allowed to be reported only when UE is configured with serving cell(s) operating on FR2-1. The aggregated bandwidth across all downlink carrier(s) of FR2-1 is the sum of bandwidth of active downlink BWP(s) across all activated downlink carrier(s) of FR2-1. The aggregated bandwidth across all uplink carrier(s) of FR2-1 is the sum of bandwidth of active uplink BWP(s) across all activated uplink carrier(s) of FR2-1. If the field is absent from the MaxBW-Preference IE or the OverheatingAssistance IE, it is interpreted as the UE having no preference on the maximum aggregated bandwidth of FR2-1. When indicated to address overheating, this maximum aggregated bandwidth includes carrier(s) of FR2-1 of both the NR MCG and the NR SCG. This maximum aggregated bandwidth only includes carriers of FR2-1 of the SCG in (NG)EN-DC. When indicated to address power saving, this maximum aggregated bandwidth includes carrier(s) of FR2-1 of the cell group that this UE assistance information is associated with. The aggregated bandwidth can only range up to the current active configuration when indicated to address power savings.

reducedMaxBW-FR2-2 Indicates the UE's preference on reduced configuration corresponding to the maximum aggregated bandwidth across all downlink carrier(s) and across all uplink carrier(s) of FR2-2, to address overheating or power saving. This field is allowed to be reported only when UE is configured with serving cell(s) operating on FR2-2. The aggregated bandwidth across all downlink carrier(s) of FR2-2 is the sum of bandwidth of active downlink BWP(s) across all activated downlink carrier(s) of FR2-2. The aggregated bandwidth across all uplink carrier(s) of FR2-2 is the sum of bandwidth of active uplink BWP(s) across all activated uplink carrier(s) of FR2-2. If the field is absent from the MaxBW-PreferenceFR2-2 IE or the OverheatingAssistance IE, it is interpreted as the UE having no preference on the maximum aggregated bandwidth of FR2-2. When indicated to address overheating, this maximum aggregated bandwidth includes carrier(s) of FR2-2 of both the NR MCG and the NR SCG. This maximum aggregated bandwidth only includes carriers of FR2-2 of the SCG in (NG)EN-DC. When indicated to address power saving, this maximum aggregated bandwidth includes carrier(s) of FR2-2 of the cell group that this UE assistance information is associated with. The aggregated bandwidth can only range up to the current active configuration when indicated to address power savings.

reducedCCsDL Indicates the UE's preference on reduced configuration corresponding to the maximum number of downlink SCells indicated by the field, to address overheating or power saving. When indicated to address overheating, this maximum number includes both SCells of the NR MCG and PSCell/SCells of the SCG. This maximum number only includes PSCell/SCells of the SCG in (NG)EN-DC. When indicated to address power saving, this maximum number includes PSCell/SCells of the cell group that this UE assistance information is associated with. The maximum number of downlink SCells can only range up to the current active configuration when indicated to address power savings.

reducedCCsUL Indicates the UE's preference on reduced configuration corresponding to the maximum number of uplink SCells indicated by the field, to address overheating or power saving. When indicated to address overheating, this maximum number includes both SCells of the NR MCG and PSCell/SCells of the SCG. This maximum number only includes PSCell/SCells of the SCG in (NG)EN-DC. When indicated to address power saving, this maximum number includes PSCell/SCells of the cell group that this UE assistance information is associated with. The maximum number of uplink SCells can only range up to the current active configuration when indicated to address power savings.

reducedMIMO-LayersFR1-DL Indicates the UE's preference on reduced configuration corresponding to the maximum number of downlink MIMO layers of each serving cell operating on FR1 indicated by the field, to address overheating or power saving. This field is allowed to be reported only when UE is configured with serving cells operating on FR1. The maximum number of downlink MIMO layers can only range up to the maximum number of MIMO layers configured across all activated downlink carrier(s) of FR1 in the cell group when indicated to address power savings.

reducedMIMO-LayersFR1-UL Indicates the UE's preference on reduced configuration corresponding to the maximum number of uplink MIMO layers of each serving cell operating on FR1 indicated by the field, to address overheating or power saving. This field is allowed to be reported only when UE is configured with serving cells operating on FR1. The maximum number of uplink MIMO layers can only range up to the maximum number of MIMO layers configured across all activated uplink carrier(s) of FR1 in the cell group when indicated to address power savings.

reducedMIMO-LayersFR2-DL Indicates the UE's preference on reduced configuration corresponding to the maximum number of downlink MIMO layers of each serving cell operating on FR2-1 indicated by the field, to address overheating or power saving. This field is allowed to be reported only when UE is configured with serving cells operating on FR2-1. The maximum number of downlink MIMO layers can only range up to the maximum number of MIMO layers configured across all activated downlink carrier(s) of FR2-1 in the cell group when indicated to address power savings.

reducedMIMO-LayersFR2-UL Indicates the UE's preference on reduced configuration corresponding to the maximum number of uplink MIMO layers of each serving cell operating on FR2-1 indicated by the field, to address overheating or power saving. This field is allowed to be reported only when UE is configured with serving cells operating on FR2-1. The maximum number of uplink MIMO layers can only range up to the maximum number of MIMO layers configured across all activated uplink carrier(s) of FR2-1 in the cell group when indicated to address power savings.

reducedMIMO-LayersFR2-2-DL Indicates the UE's preference on reduced configuration corresponding to the maximum number of downlink MIMO layers of each serving cell operating on FR2-2 indicated by the field, to address overheating or power saving. This field is allowed to be reported only when UE is configured with serving cells operating on FR2-2. The maximum number of downlink MIMO layers can only range up to the maximum number of MIMO layers configured across all activated downlink carrier(s) of FR2-2 in the cell group when indicated to address power savings.

reducedMIMO-LayersFR2-2-UL Indicates the UE's preference on reduced configuration corresponding to the maximum number of uplink MIMO layers of each serving cell operating on FR2-2 indicated by the field, to address overheating or power saving. This field is allowed to be reported only when UE is configured with serving cells operating on FR2-2. The maximum number of uplink MIMO layers can only range up to the maximum number of MIMO layers configured across all activated uplink carrier(s) of FR2-2 in the cell group when indicated to address power savings.

referenceTimeInfoPreference Indicates whether the UE prefers being provisioned with the timing information specified in the IE ReferenceTimeInfo.

resumeCause Provides the resume cause based on the information received from the upper layers.

rlm-MeasRelaxationState Indicates the relaxation state of RLM measurements. Value true indicates that the UE is performing relaxation of RLM measurements, and value false indicates that the UE is not performing relaxation of RLM measurements.

rrm-MeasRelaxationFulfilment Indicates whether the UE fulfils the relaxed measurement criterion for stationary UE in 5.7.4.4. Value true indicates that the UE fulfils the criterion, and value false indicates that the UE does not fulfil the criterion.

sl-QoS-FlowIdentity This identity uniquely identifies one sidelink QoS flow between the UE and the network in the scope of UE, which is unique for different destination and cast type.

sl-UE-AssistanceInformationNR Indicates the traffic characteristic of sidelink logical channel(s), specified in the IE SL-TrafficPatternInfo, that are setup for NR sidelink communication.

type1 Indicates the preferred amount of increment/decrement to the long DRX cycle length with respect to the current configuration. Value in number of milliseconds. Value ms40 corresponds to 40 milliseconds, msMinus40 corresponds to -40 milliseconds and so on.

ul-GapFR2-PatternPreference Indicates the UE's preference on FR2 UL gap pattern as defined in TS 38.133 [14].

victimSystemType Indicate the list of victim system types to which IDC interference is caused from NR when configured with UL CA. Value gps, glonass, bds, galileo and navIC indicates the type of GNSS. Value wlan indicates WLAN and value bluetooth indicates Bluetooth.

SL-TrafficPatternInfo field descriptions

messageSize Indicates the maximum TB size based on the observed traffic pattern. The value refers to the index of TS 38.321 [3], table 6.1.3.1-2.

timingOffset This field indicates the estimated timing for a packet arrival in a sidelink logical channel. Specifically, the value indicates the timing offset with respect to subframe#0 of SFN#0 in milliseconds.

trafficPeriodicity This field indicates the estimated data arrival periodicity in a sidelink logical channel. Value ms20 corresponds to 20 ms, ms50 corresponds to 50 ms and so on.

#### –	UECapabilityEnquiry

The UECapabilityEnquiry message is used to request UE radio access capabilities for NR as well as for other RATs.

Signalling radio bearer: SRB1

RLC-SAP: AM

Logical channel: DCCH

Direction: Network to UE

UECapabilityEnquiry message

-- ASN1START

-- TAG-UECAPABILITYENQUIRY-START

UECapabilityEnquiry ::=             SEQUENCE {

rrc-TransactionIdentifier           RRC-TransactionIdentifier,

criticalExtensions                  CHOICE {

ueCapabilityEnquiry                 UECapabilityEnquiry-IEs,

criticalExtensionsFuture            SEQUENCE {}

}

}

UECapabilityEnquiry-IEs ::=         SEQUENCE {

ue-CapabilityRAT-RequestList        UE-CapabilityRAT-RequestList,

lateNonCriticalExtension            OCTET STRING                                                            OPTIONAL,

ue-CapabilityEnquiryExt             OCTET STRING (CONTAINING UECapabilityEnquiry-v1560-IEs)                 OPTIONAL --  Need N

}

UECapabilityEnquiry-v1560-IEs ::=   SEQUENCE {

capabilityRequestFilterCommon       UE-CapabilityRequestFilterCommon                                        OPTIONAL, -- Need N

nonCriticalExtension                UECapabilityEnquiry-v1610-IEs                                           OPTIONAL

}

UECapabilityEnquiry-v1610-IEs ::=   SEQUENCE {

rrc-SegAllowed-r16                  ENUMERATED {enabled}           OPTIONAL, -- Need N

nonCriticalExtension                SEQUENCE {}                    OPTIONAL

}

-- TAG-UECAPABILITYENQUIRY-STOP

-- ASN1STOP

#### –	UECapabilityInformation

The IE UECapabilityInformation message is used to transfer UE radio access capabilities requested by the network.

Signalling radio bearer: SRB1

RLC-SAP: AM

Logical channel: DCCH

Direction: UE to Network

UECapabilityInformation message

-- ASN1START

-- TAG-UECAPABILITYINFORMATION-START

UECapabilityInformation ::=         SEQUENCE {

rrc-TransactionIdentifier           RRC-TransactionIdentifier,

criticalExtensions                  CHOICE {

ueCapabilityInformation             UECapabilityInformation-IEs,

criticalExtensionsFuture            SEQUENCE {}

}

}

UECapabilityInformation-IEs ::=     SEQUENCE {

ue-CapabilityRAT-ContainerList      UE-CapabilityRAT-ContainerList                                          OPTIONAL,

lateNonCriticalExtension            OCTET STRING                                                            OPTIONAL,

nonCriticalExtension                SEQUENCE{}                                                              OPTIONAL

}

-- TAG-UECAPABILITYINFORMATION-STOP

-- ASN1STOP

---------------------------------------------------------Skip Unchanged----------------------------------------------------------

## 6.3	RRC information elements

### 6.3.0	Parameterized types

#### –	SetupRelease

SetupRelease allows the ElementTypeParam to be used as the referenced data type for the setup and release entries. See A.3.8 for guidelines.

-- ASN1START

-- TAG-SETUPRELEASE-START

SetupRelease { ElementTypeParam } ::= CHOICE {

release         NULL,

setup           ElementTypeParam

}

-- TAG-SETUPRELEASE-STOP

-- ASN1STOP

---------------------------------------------------------Skip Unchanged----------------------------------------------------------

### 6.3.2	Radio resource control information elements

---------------------------------------------------------Skip Unchanged----------------------------------------------------------

#### –	GapPriority

The IE GapPriority is used to identify the priority of a gap configuration.

GapPriority information element

-- ASN1START

-- TAG-GAPPRIORITY-START

GapPriority-r17 ::=                       INTEGER (1..maxNrOfGapPri-r17)

-- TAG-GAPPRIORITY-STOP

-- ASN1STOP

---------------------------------------------------------Skip Unchanged----------------------------------------------------------

#### –	MeasGapId

The IE MeasGapId used to identify a per UE or per FR measurement gap configuration.

MeasGapId information element

-- ASN1START

-- TAG-MEASGAPID-START

MeasGapId-r17 ::=                       INTEGER (1..maxNrofGapId-r17)

-- TAG-MEASGAPID-STOP

-- ASN1STOP

#### –	MeasGapSharingConfig

The IE MeasGapSharingConfig specifies the measurement gap sharing scheme and controls setup/ release of measurement gap sharing.

MeasGapSharingConfig information element

-- ASN1START

-- TAG-MEASGAPSHARINGCONFIG-START

MeasGapSharingConfig ::=        SEQUENCE {

gapSharingFR2                   SetupRelease { MeasGapSharingScheme }       OPTIONAL,   -- Need M

...,

[[

gapSharingFR1                   SetupRelease { MeasGapSharingScheme }       OPTIONAL,   --Need M

gapSharingUE                    SetupRelease { MeasGapSharingScheme }       OPTIONAL    --Need M

]]

}

MeasGapSharingScheme::=         ENUMERATED {scheme00, scheme01, scheme10, scheme11}

-- TAG-MEASGAPSHARINGCONFIG-STOP

-- ASN1STOP

MeasGapSharingConfig field descriptions

gapSharingFR1 Indicates the measurement gap sharing scheme that applies to the gap set via gapFR1. In (NG)EN-DC, gapSharingFR1 cannot be set up by NR RRC (i.e. only LTE RRC can configure FR1 gap sharing). In NE-DC, gapSharingFR1 can only be set up by NR RRC (i.e. LTE RRC cannot configure FR1 gap sharing). In NR-DC, gapSharingFR1 can only be set up in the measConfig associated with MCG. gapSharingFR1 can not be configured together with gapSharingUE. For the applicability of the different gap sharing schemes, see TS 38.133 [14]. Value scheme00 corresponds to scheme "00", value scheme01 corresponds to scheme "01", and so on.

gapSharingFR2 Indicates the measurement gap sharing scheme that applies to the gap set via gapFR2. In (NG)EN-DC or NE-DC, gapSharingFR2 can only be set up by NR RRC (i.e. LTE RRC cannot configure FR2 gap sharing). In NR-DC, gapSharingFR2 can only be set up by MCG in the measConfig associated with MCG. gapSharingFR2 cannot be configured together with gapSharingUE. For applicability of the different gap sharing schemes, see TS 38.133 [14]. Value scheme00 corresponds to scheme "00", value scheme01 corresponds to scheme "01", and so on.

gapSharingUE Indicates the measurement gap sharing scheme that applies to the gap set via gapUE. In (NG)EN-DC, gapSharingUE cannot be set up by NR RRC (i.e. only LTE RRC can configure per UE gap sharing). In NE-DC, gapSharingUE can only be set up by NR RRC (i.e. LTE RRC cannot configure per UE gap sharing). In NR-DC, gapSharingUE can only be set up in the measConfig associated with MCG. If gapSharingUE is configured, then neither gapSharingFR1 nor gapSharingFR2 can be configured. For the applicability of the different gap sharing schemes, see TS 38.133 [14]. Value scheme00 corresponds to scheme "00", value scheme01 corresponds to scheme "01", and so on.

---------------------------------------------------------Skip Unchanged----------------------------------------------------------

#### –	MUSIM-GapConfig

The IE MUSIM-GapConfig specifies the MUSIM gap configuration and controls setup/release of MUSIM gaps.

MUSIM-GapConfig information element

-- ASN1START

-- TAG-MUSIM-GAPCONFIG-START

MUSIM-GapConfig-r17 ::=                  SEQUENCE {

musim-GapToReleaseList-r17       SEQUENCE (SIZE (1..3)) OF MUSIM-GapId-r17               OPTIONAL, -- Need N

musim-GapToAddModList-r17        SEQUENCE (SIZE (1..3)) OF MUSIM-Gap-r17             OPTIONAL, -- Need N

musim-AperiodicGap-r17           MUSIM-GapInfo-r17                OPTIONAL, -- Need N

...,

[[

musim-GapPriorityToAddModList-r18        SEQUENCE (SIZE (1..3)) OF GapPriority-r17             OPTIONAL

]]

}

MUSIM-Gap-r17 ::=          SEQUENCE {

musim-GapId-r17                        MUSIM-GapId-r17,

musim-GapInfo-r17                      MUSIM-GapInfo-r17

}

-- TAG-MUSIM-GAPCONFIG-STOP

-- ASN1STOP

MUSIM-GapConfig field descriptions

musim-AperiodicGap Indicates the MUSIM aperiodic gap as specified in TS 38.133 [14] clause 9.1.10. If UE indicates the musim-Starting-SFN-AndSubframe when requesting aperiodic gap the network can only configure the aperiodic gap with the same start point or no aperiodic gap. If the field musim-Starting-SFN-AndSubframe is absent for aperiodic gap, network can configure any timing as the starting point for aperiodic gap or configure no aperiodic gap.

musim-GapInfo Indicates the values for musim-GapLength and musim-GapRepetitionAndOffset. When network provides periodic gap, network always signals the musim-GapLength and musim-GapRepetitionAndOffset as indicated by the UE's preferred MUSIM gap configuration.

musim-GapPriorityToAddModList Indicate the priority of MUSIM periodic gap. If the network includes musim-GapPriorityToAddModList-r18, it includes the same number of entries, and listed in the same order, as in musim-GapToAddModList-r17. For the priority of MUSIM aperiodic gap, the MUSIM aperiodic gap is always kept (not dropped) from UE perspective in case of collisions with other gaps (i.e. all gaps including MUSIM gaps, measurement gap,, etc).The gap priority level of MUSIM aperiodic gap is not explicitly configured by the NW.

musim-GapToAddModList List of MUSIM periodic gap patterns to add or modify.

musim-GapToReleaseList List of MUSIM periodic gap patterns to release.

#### –	MUSIM-GapId

The IE MUSIM-GapId is used to identify UE periodic MUSIM gap(s) to add, modify or release.

MUSIM-GapId information element

-- ASN1START

-- TAG-MUSIM-GAPID-START

MUSIM-GapId-r17 ::=                  INTEGER (0..2)

-- TAG-MUSIM-GAPID-STOP

-- ASN1STOP

#### –	MUSIM-GapInfo

The IE MUSIM-GapInfo is used to indicate MUSIM gap parameters.

MUSIM-GapInfo information element

-- ASN1START

-- TAG-MUSIM-GAPINFO-START

MUSIM-GapInfo-r17 ::=               SEQUENCE {

musim-Starting-SFN-AndSubframe-r17  MUSIM-Starting-SFN-AndSubframe-r17             OPTIONAL, -- Cond aperiodic

musim-GapLength-r17                 ENUMERATED {ms3, ms4, ms6, ms10, ms20}         OPTIONAL, -- Cond gapSetup

musim-GapRepetitionAndOffset-r17    CHOICE {

ms20-r17                            INTEGER (0..19),

ms40-r17                            INTEGER (0..39),

ms80-r17                            INTEGER (0..79),

ms160-r17                           INTEGER (0..159),

ms320-r17                           INTEGER (0..319),

ms640-r17                           INTEGER (0..639),

ms1280-r17                          INTEGER (0..1279),

ms2560-r17                          INTEGER (0..2559),

ms5120-r17                          INTEGER (0..5119),

...

}                                                                                  OPTIONAL -- Cond periodic

}

MUSIM-Starting-SFN-AndSubframe-r17 ::= SEQUENCE {

starting-SFN-r17                       INTEGER (0..1023),

startingSubframe-r17                   INTEGER (0..9)

}

-- TAG-MUSIM-GAPINFO-STOP

-- ASN1STOP

MUSIM-GapInfo field descriptions

musim-GapLength Indicates the length of the UE's MUSIM gap as specified in TS 38.133 [14] clause 9.1.10. This field is mandatory present for both periodic gap and aperiodic gap preference indication.

musim-GapRepetitionAndOffset Indicates the gap repetition period in ms and gap offset in number of subframes for the periodic MUSIM gap as specified in TS 38.133 [14] clause 9.1.10. This field is mandatory present for the periodic MUSIM gap preference indication.

musim-Starting-SFN-AndSubframe Indicates gap starting position for the aperiodic MUSIM gap. This field is optionally present for the aperiodic MUSIM gap preference indication.

starting-SFN Indicates gap starting SFN number for the aperiodic MUSIM gap.

startingSubframe Indicates gap starting subframe number for the aperiodic MUSIM gap.

| Conditional Presence | Explanation |
| --- | --- |
| aperiodic | This field is mandatory present in case of aperiodic MUSIM gap configuration. Otherwise it is absent. |
| gapSetup | The field is mandatory present upon configuration of a new MUSIM gap. The field is optionally present, Need M, otherwise. |
| periodic | This field is mandatory present in case of periodic MUSIM gap configuration. Otherwise it is absent. |

#### –	NeedForGapsConfigNR

The IE NeedForGapsConfigNR contains configuration related to the reporting of measurement gap requirement information.

NeedForGapsConfigNR information element

-- ASN1START

-- TAG-NeedForGapsConfigNR-START

NeedForGapsConfigNR-r16 ::=        SEQUENCE {

requestedTargetBandFilterNR-r16       SEQUENCE (SIZE (1..maxBands)) OF FreqBandIndicatorNR               OPTIONAL          -- Need R

}

-- TAG-NeedForGapsConfigNR-STOP

-- ASN1STOP

NeedForGapsConfigNR field descriptions

requestedTargetBandFilterNR Indicates the target NR bands that the UE is requested to report the gap requirement information.

#### –	NeedForGapsInfoNR

The IE NeedForGapsInfoNR indicates whether measurement gap is required for the UE to perform SSB based measurements on an NR target band while NR-DC or NE-DC is not configured.

NeedForGapsInfoNR information element

-- ASN1START

-- TAG-NeedForGapsInfoNR-START

NeedForGapsInfoNR-r16 ::=        SEQUENCE {

intraFreq-needForGap-r16      NeedForGapsIntraFreqList-r16,

interFreq-needForGap-r16      NeedForGapsBandListNR-r16

}

NeedForGapsIntraFreqList-r16 ::=          SEQUENCE (SIZE (1.. maxNrofServingCells)) OF NeedForGapsIntraFreq-r16

NeedForGapsBandListNR-r16 ::=             SEQUENCE (SIZE (1..maxBands)) OF NeedForGapsNR-r16

NeedForGapsIntraFreq-r16  ::=                 SEQUENCE {

servCellId-r16                               ServCellIndex,

gapIndicationIntra-r16                       ENUMERATED {gap, no-gap}

}

NeedForGapsNR-r16  ::=                        SEQUENCE {

bandNR-r16                                   FreqBandIndicatorNR,

gapIndication-r16                            ENUMERATED {gap, no-gap}

}

-- TAG-NeedForGapsInfoNR-STOP

-- ASN1STOP

NeedForGapsInfoNR field descriptions

intraFreq-needForGap Indicates the measurement gap requirement information for NR intra-frequency measurement.

interFreq-needForGap Indicates the measurement gap requirement information for NR inter-frequency measurement.

NeedForGapsIntraFreq field descriptions

servCellId Indicates the serving cell which contains the target SSB (associated with the initial DL BWP) to be measured.

gapIndicationIntra Indicates whether measurement gap is required for the UE to perform intra-frequency SSB based measurements on the concerned serving cell. Value gap indicates that a measurement gap is needed if any of the UE configured BWPs (except the BWP(s) configured with servingCellMO associated with NCD-SSB) do not contain the frequency domain resources of the SSB associated to the initial DL BWP (CD-SSB). Value no-gap indicates a measurement gap is not needed to measure the SSB associated to the initial DL BWP (CD-SSB) for all configured BWPs (except the BWP(s) configured with servingCellMO associated with NCD-SSB), no matter the SSB is within the configured BWP or not.

NeedForGapsNR field descriptions

bandNR Indicates the NR target band to be measured.

gapIndication Indicates whether measurement gap is required for the UE to perform SSB based measurements on the concerned NR target band while NR-DC or NE-DC is not configured. The UE determines this information based on the resultant configuration of the RRCReconfiguration or RRCResume message that triggers this response. Value gap indicates that a measurement gap is needed, value no-gap indicates a measurement gap is not needed.

#### –	NeedForGapNCSG-ConfigEUTRA

The IE NeedForGapNCSG-ConfigEUTRA contains configuration related to the reporting of measurement gap and NCSG requirement information.

NeedForGapNCSG-ConfigEUTRA information element

-- ASN1START

-- TAG-NeedForGapNCSG-ConfigEUTRA-START

NeedForGapNCSG-ConfigEUTRA-r17 ::=        SEQUENCE {

requestedTargetBandFilterNCSG-EUTRA-r17   SEQUENCE (SIZE (1..maxBandsEUTRA)) OF FreqBandIndicatorEUTRA     OPTIONAL    -- Need R

}

-- TAG-NeedForGapNCSG-ConfigEUTRA-STOP

-- ASN1STOP

NeedForGapNCSG-ConfigEUTRA field descriptions

requestedTargetBandFilterNCSG-EUTRA Indicates the target E-UTRA bands that the UE is requested to report the measurement gap and NCSG requirement information.

#### –	NeedForGapNCSG-ConfigNR

The IE NeedForGapNCSG-ConfigNR contains configuration related to the reporting of measurement gap and NCSG requirement information.

NeedForGapNCSG-ConfigNR information element

-- ASN1START

-- TAG-NEEDFORGAPNCSG-CONFIGNR-START

NeedForGapNCSG-ConfigNR-r17 ::=        SEQUENCE {

requestedTargetBandFilterNCSG-NR-r17   SEQUENCE (SIZE (1..maxBands)) OF FreqBandIndicatorNR          OPTIONAL          -- Need R

}

-- TAG-NEEDFORGAPNCSG-CONFIGNR-STOP

-- ASN1STOP

NeedForGapNCSG-ConfigNR field descriptions

requestedTargetBandFilterNCSG-NR Indicates the target NR bands that the UE is requested to report the measurement gap and NCSG requirement information.

#### –	NeedForGapNCSG-InfoEUTRA

The IE NeedForGapNCSG-InfoEUTRA indicates whether measurement gap or NCSG is required for the UE to perform measurements on an E-UTRA target band while NR-DC or NE-DC is not configured.

NeedForGapNCSG-InfoEUTRA information element

-- ASN1START

-- TAG-NEEDFORGAPNCSG-INFOEUTRA-START

NeedForGapNCSG-InfoEUTRA-r17 ::=   SEQUENCE {

needForNCSG-EUTRA-r17              SEQUENCE (SIZE (1..maxBandsEUTRA)) OF NeedForNCSG-EUTRA-r17

}

NeedForNCSG-EUTRA-r17  ::=         SEQUENCE {

bandEUTRA-r17                      FreqBandIndicatorEUTRA,

gapIndication-r17                  ENUMERATED {gap, ncsg, nogap-noncsg}

}

-- TAG-NEEDFORGAPNCSG-INFOEUTRA-STOP

-- ASN1STOP

NeedForGapNCSG-InfoEUTRA field descriptions

needForNCSG-EUTRA Indicates the measurement gap and NCSG requirement information for E-UTRA measurement.

NeedForNCSG-EUTRA field descriptions

bandEUTRA Indicates the E-UTRA target band to be measured.

gapIndication Indicates whether measurement gap or NCSG is required for the UE to perform measurements on the concerned E-UTRA target band while NR-DC or NE-DC is not configured. The UE determines this information based on the resultant configuration of the RRCReconfiguration message or RRCResume message that triggers this response. Value gap indicates that a measurement gap is needed, value ncsg indicates that NCSG is needed, value nogap-noncsg indicates neither a measurement gap nor a NCSG is needed.

#### –	NeedForGapNCSG-InfoNR

The IE NeedForGapNCSG-InfoNR indicates whether measurement gap or NCSG is required for the UE to perform SSB based measurements on an NR target band while NR-DC or NE-DC is not configured.

NeedForGapNCSG-InfoNR information element

-- ASN1START

-- TAG-NEEDFORGAPNCSG-INFONR-START

NeedForGapNCSG-InfoNR-r17 ::=     SEQUENCE {

intraFreq-needForNCSG-r17         NeedForNCSG-IntraFreqList-r17,

interFreq-needForNCSG-r17         NeedForNCSG-BandListNR-r17

}

NeedForNCSG-IntraFreqList-r17 ::= SEQUENCE (SIZE (1.. maxNrofServingCells)) OF NeedForNCSG-IntraFreq-r17

NeedForNCSG-BandListNR-r17 ::=    SEQUENCE (SIZE (1..maxBands)) OF NeedForNCSG-NR-r17

NeedForNCSG-IntraFreq-r17  ::=    SEQUENCE {

servCellId-r17                    ServCellIndex,

gapIndicationIntra-r17            ENUMERATED {gap, ncsg, nogap-noncsg}

}

NeedForNCSG-NR-r17  ::=           SEQUENCE {

bandNR-r17                        FreqBandIndicatorNR,

gapIndication-r17                 ENUMERATED {gap, ncsg, nogap-noncsg}

}

-- TAG-NEEDFORGAPNCSG-INFONR-STOP

-- ASN1STOP

NeedForGapNCSG-InfoNR field descriptions

intraFreq-needForNCSG Indicates the measurement gap and NCSG requirement information for NR intra-frequency measurement.

interFreq-needForNCSG Indicates the measurement gap and NCSG requirement information for NR inter-frequency measurement.

NeedForNCSG-IntraFreq field descriptions

servCellId Indicates the serving cell which contains the target SSB (associated with the initial DL BWP) to be measured.

gapIndicationIntra Indicates whether measurement gap or NCSG is required for the UE to perform intra-frequency SSB based measurements on the concerned serving cell. Value gap indicates that a measurement gap is needed if any of the UE configured BWPs (except the BWP(s) configured with servingCellMO associated with NCD-SSB) do not contain the frequency domain resources of the SSB associated to the initial DL BWP (CD-SSB). Value ncsg indicates that a NCSG is needed if any of the UE configured BWPs do not contain the frequency domain resources of the SSB associated to the initial DL BWP. Value nogap-noncsg indicates that neither a measurement gap nor a NCSG is needed to measure the SSB associated to the initial DL BWP (CD-SSB) for all configured BWPs (except the BWP(s) configured with servingCellMO associated with NCD-SSB), no matter the SSB is within the configured BWP or not.

NeedForNCSG-NR field descriptions

bandNR Indicates the NR target band to be measured.

gapIndication Indicates whether measurement gap or NCSG is required for the UE to perform SSB based measurements on the concerned NR target band while NR-DC or NE-DC is not configured. The UE determines this information based on the resultant configuration of the RRCReconfiguration or RRCResume message that triggers this response. Value gap indicates that a measurement gap is needed, value ncsg indicates that a NCSG is needed, and value nogap-noncsg indicates neither a measurement gap nor a NCSG is needed.

---------------------------------------------------------Skip Unchanged----------------------------------------------------------

### 6.3.3	UE capability information elements

---------------------------------------------------------Skip Unchanged----------------------------------------------------------

#### –	AccessStratumRelease

The IE AccessStratumRelease indicates the release supported by the UE.

AccessStratumRelease information element

-- ASN1START

-- TAG-ACCESSSTRATUMRELEASE-START

AccessStratumRelease ::= ENUMERATED {

rel15, rel16, rel17, spare5, spare4, spare3, spare2, spare1, ... }

-- TAG-ACCESSSTRATUMRELEASE-STOP

-- ASN1STOP

#### –	AppLayerMeasParameters

The IE AppLayerMeasParameters is used to convey the capabilities supported by the UE for application layer measurements.

AppLayerMeasParameters information element

-- ASN1START

-- TAG-APPLAYERMEASPARAMETERS-START

AppLayerMeasParameters-r17 ::=            SEQUENCE {

qoe-Streaming-MeasReport-r17              ENUMERATED {supported}                                             OPTIONAL,

qoe-MTSI-MeasReport-r17                   ENUMERATED {supported}                                             OPTIONAL,

qoe-VR-MeasReport-r17                     ENUMERATED {supported}                                             OPTIONAL,

ran-VisibleQoE-Streaming-MeasReport-r17   ENUMERATED {supported}                                             OPTIONAL,

ran-VisibleQoE-VR-MeasReport-r17          ENUMERATED {supported}                                             OPTIONAL,

ul-MeasurementReportAppLayer-Seg-r17      ENUMERATED {supported}                                             OPTIONAL,

...

}

-- TAG-APPLAYERMEASPARAMETERS-STOP

-- ASN1STOP

#### –	BandCombinationList

The IE BandCombinationList contains a list of NR CA, NR non-CA and/or MR-DC band combinations (also including DL only or UL only band).

BandCombinationList information element

-- ASN1START

-- TAG-BANDCOMBINATIONLIST-START

BandCombinationList ::=             SEQUENCE (SIZE (1..maxBandComb)) OF BandCombination

BandCombinationList-v1540 ::=       SEQUENCE (SIZE (1..maxBandComb)) OF BandCombination-v1540

BandCombinationList-v1550 ::=       SEQUENCE (SIZE (1..maxBandComb)) OF BandCombination-v1550

BandCombinationList-v1560 ::=       SEQUENCE (SIZE (1..maxBandComb)) OF BandCombination-v1560

BandCombinationList-v1570 ::=       SEQUENCE (SIZE (1..maxBandComb)) OF BandCombination-v1570

BandCombinationList-v1580 ::=       SEQUENCE (SIZE (1..maxBandComb)) OF BandCombination-v1580

BandCombinationList-v1590 ::=       SEQUENCE (SIZE (1..maxBandComb)) OF BandCombination-v1590

BandCombinationList-v15g0 ::=       SEQUENCE (SIZE (1..maxBandComb)) OF BandCombination-v15g0

BandCombinationList-v1610 ::=       SEQUENCE (SIZE (1..maxBandComb)) OF BandCombination-v1610

BandCombinationList-v1630 ::=       SEQUENCE (SIZE (1..maxBandComb)) OF BandCombination-v1630

BandCombinationList-v1640 ::=       SEQUENCE (SIZE (1..maxBandComb)) OF BandCombination-v1640

BandCombinationList-v1650 ::=       SEQUENCE (SIZE (1..maxBandComb)) OF BandCombination-v1650

BandCombinationList-v1680 ::=       SEQUENCE (SIZE (1..maxBandComb)) OF BandCombination-v1680

BandCombinationList-v1690 ::=       SEQUENCE (SIZE (1..maxBandComb)) OF BandCombination-v1690

BandCombinationList-v16a0 ::=       SEQUENCE (SIZE (1..maxBandComb)) OF BandCombination-v16a0

BandCombinationList-v1700 ::=       SEQUENCE (SIZE (1..maxBandComb)) OF BandCombination-v1700

BandCombinationList-v1720 ::=       SEQUENCE (SIZE (1..maxBandComb)) OF BandCombination-v1720

BandCombinationList-v1730 ::=       SEQUENCE (SIZE (1..maxBandComb)) OF BandCombination-v1730

BandCombinationList-v1740 ::=       SEQUENCE (SIZE (1..maxBandComb)) OF BandCombination-v1740

BandCombinationList-UplinkTxSwitch-r16 ::= SEQUENCE (SIZE (1..maxBandComb)) OF BandCombination-UplinkTxSwitch-r16

BandCombinationList-UplinkTxSwitch-v1630 ::= SEQUENCE (SIZE (1..maxBandComb)) OF BandCombination-UplinkTxSwitch-v1630

BandCombinationList-UplinkTxSwitch-v1640 ::= SEQUENCE (SIZE (1..maxBandComb)) OF BandCombination-UplinkTxSwitch-v1640

BandCombinationList-UplinkTxSwitch-v1650 ::= SEQUENCE (SIZE (1..maxBandComb)) OF BandCombination-UplinkTxSwitch-v1650

BandCombinationList-UplinkTxSwitch-v1670 ::= SEQUENCE (SIZE (1..maxBandComb)) OF BandCombination-UplinkTxSwitch-v1670

BandCombinationList-UplinkTxSwitch-v1690 ::= SEQUENCE (SIZE (1..maxBandComb)) OF BandCombination-UplinkTxSwitch-v1690

BandCombinationList-UplinkTxSwitch-v16a0 ::= SEQUENCE (SIZE (1..maxBandComb)) OF BandCombination-UplinkTxSwitch-v16a0

BandCombinationList-UplinkTxSwitch-v1700 ::= SEQUENCE (SIZE (1..maxBandComb)) OF BandCombination-UplinkTxSwitch-v1700

BandCombinationList-UplinkTxSwitch-v1720 ::= SEQUENCE (SIZE (1..maxBandComb)) OF BandCombination-UplinkTxSwitch-v1720

BandCombinationList-UplinkTxSwitch-v1730 ::= SEQUENCE (SIZE (1..maxBandComb)) OF BandCombination-UplinkTxSwitch-v1730

BandCombinationList-UplinkTxSwitch-v1740 ::= SEQUENCE (SIZE (1..maxBandComb)) OF BandCombination-UplinkTxSwitch-v1740

BandCombination ::=                 SEQUENCE {

bandList                            SEQUENCE (SIZE (1..maxSimultaneousBands)) OF BandParameters,

featureSetCombination               FeatureSetCombinationId,

ca-ParametersEUTRA                  CA-ParametersEUTRA                          OPTIONAL,

ca-ParametersNR                     CA-ParametersNR                             OPTIONAL,

mrdc-Parameters                     MRDC-Parameters                             OPTIONAL,

supportedBandwidthCombinationSet    BIT STRING (SIZE (1..32))                   OPTIONAL,

powerClass-v1530                    ENUMERATED {pc2}                            OPTIONAL

}

BandCombination-v1540::=            SEQUENCE {

bandList-v1540                      SEQUENCE (SIZE (1..maxSimultaneousBands)) OF BandParameters-v1540,

ca-ParametersNR-v1540               CA-ParametersNR-v1540                       OPTIONAL

}

BandCombination-v1550 ::=           SEQUENCE {

ca-ParametersNR-v1550               CA-ParametersNR-v1550

}

BandCombination-v1560::=            SEQUENCE {

ne-DC-BC                                ENUMERATED {supported}                 OPTIONAL,

ca-ParametersNRDC                       CA-ParametersNRDC                      OPTIONAL,

ca-ParametersEUTRA-v1560                CA-ParametersEUTRA-v1560               OPTIONAL,

ca-ParametersNR-v1560                   CA-ParametersNR-v1560                  OPTIONAL

}

BandCombination-v1570 ::=           SEQUENCE {

ca-ParametersEUTRA-v1570            CA-ParametersEUTRA-v1570

}

BandCombination-v1580 ::=           SEQUENCE {

mrdc-Parameters-v1580               MRDC-Parameters-v1580

}

BandCombination-v1590::=            SEQUENCE {

supportedBandwidthCombinationSetIntraENDC  BIT STRING (SIZE (1..32))           OPTIONAL,

mrdc-Parameters-v1590                      MRDC-Parameters-v1590

}

BandCombination-v15g0::=            SEQUENCE {

ca-ParametersNR-v15g0               CA-ParametersNR-v15g0                      OPTIONAL,

ca-ParametersNRDC-v15g0             CA-ParametersNRDC-v15g0                    OPTIONAL,

mrdc-Parameters-v15g0               MRDC-Parameters-v15g0                      OPTIONAL

}

BandCombination-v1610 ::=          SEQUENCE {

bandList-v1610                      SEQUENCE (SIZE (1..maxSimultaneousBands)) OF BandParameters-v1610  OPTIONAL,

ca-ParametersNR-v1610               CA-ParametersNR-v1610                  OPTIONAL,

ca-ParametersNRDC-v1610             CA-ParametersNRDC-v1610                OPTIONAL,

powerClass-v1610                    ENUMERATED {pc1dot5}                   OPTIONAL,

powerClassNRPart-r16                ENUMERATED {pc1, pc2, pc3, pc5}        OPTIONAL,

featureSetCombinationDAPS-r16       FeatureSetCombinationId                OPTIONAL,

mrdc-Parameters-v1620               MRDC-Parameters-v1620                  OPTIONAL

}

BandCombination-v1630 ::=                   SEQUENCE {

ca-ParametersNR-v1630                       CA-ParametersNR-v1630                                             OPTIONAL,

ca-ParametersNRDC-v1630                     CA-ParametersNRDC-v1630                                           OPTIONAL,

mrdc-Parameters-v1630                       MRDC-Parameters-v1630                                             OPTIONAL,

supportedTxBandCombListPerBC-Sidelink-r16   BIT STRING (SIZE (1..maxBandComb))                                OPTIONAL,

supportedRxBandCombListPerBC-Sidelink-r16   BIT STRING (SIZE (1..maxBandComb))                                OPTIONAL,

scalingFactorTxSidelink-r16                 SEQUENCE (SIZE (1..maxBandComb)) OF ScalingFactorSidelink-r16     OPTIONAL,

scalingFactorRxSidelink-r16                 SEQUENCE (SIZE (1..maxBandComb)) OF ScalingFactorSidelink-r16     OPTIONAL

}

BandCombination-v1640 ::=                   SEQUENCE {

ca-ParametersNR-v1640                       CA-ParametersNR-v1640                                             OPTIONAL,

ca-ParametersNRDC-v1640                     CA-ParametersNRDC-v1640                                           OPTIONAL

}

BandCombination-v1650 ::=          SEQUENCE {

ca-ParametersNRDC-v1650             CA-ParametersNRDC-v1650                 OPTIONAL

}

BandCombination-v1680 ::=          SEQUENCE {

intrabandConcurrentOperationPowerClass-r16 SEQUENCE (SIZE (1..maxBandComb)) OF IntraBandPowerClass-r16     OPTIONAL

}

BandCombination-v1690 ::=          SEQUENCE {

ca-ParametersNR-v1690              CA-ParametersNR-v1690                 OPTIONAL

}

BandCombination-v16a0 ::=          SEQUENCE {

ca-ParametersNR-v16a0              CA-ParametersNR-v16a0                    OPTIONAL,

ca-ParametersNRDC-v16a0            CA-ParametersNRDC-v16a0                  OPTIONAL

}

BandCombination-v1700 ::=          SEQUENCE {

ca-ParametersNR-v1700              CA-ParametersNR-v1700                    OPTIONAL,

ca-ParametersNRDC-v1700            CA-ParametersNRDC-v1700                  OPTIONAL,

mrdc-Parameters-v1700              MRDC-Parameters-v1700                    OPTIONAL,

bandList-v1710                      SEQUENCE (SIZE (1..maxSimultaneousBands)) OF BandParameters-v1710  OPTIONAL,

supportedBandCombListPerBC-SL-RelayDiscovery-r17      BIT STRING (SIZE (1..maxBandComb))               OPTIONAL,

supportedBandCombListPerBC-SL-NonRelayDiscovery-r17   BIT STRING (SIZE (1..maxBandComb))               OPTIONAL

}

BandCombination-v1720 ::=          SEQUENCE {

ca-ParametersNR-v1720              CA-ParametersNR-v1720                    OPTIONAL,

ca-ParametersNRDC-v1720            CA-ParametersNRDC-v1720                  OPTIONAL

}

BandCombination-v1730 ::=          SEQUENCE {

ca-ParametersNR-v1730              CA-ParametersNR-v1730                    OPTIONAL,

ca-ParametersNRDC-v1730            CA-ParametersNRDC-v1730                  OPTIONAL,

bandList-v1730                     SEQUENCE (SIZE (1..maxSimultaneousBands)) OF BandParameters-v1730  OPTIONAL

}

BandCombination-v1740 ::=          SEQUENCE {

ca-ParametersNR-v1740              CA-ParametersNR-v1740                    OPTIONAL

}

BandCombination-UplinkTxSwitch-r16 ::= SEQUENCE {

bandCombination-r16                 BandCombination,

bandCombination-v1540               BandCombination-v1540                      OPTIONAL,

bandCombination-v1560               BandCombination-v1560                      OPTIONAL,

bandCombination-v1570               BandCombination-v1570                      OPTIONAL,

bandCombination-v1580               BandCombination-v1580                      OPTIONAL,

bandCombination-v1590               BandCombination-v1590                      OPTIONAL,

bandCombination-v1610               BandCombination-v1610                      OPTIONAL,

supportedBandPairListNR-r16         SEQUENCE (SIZE (1..maxULTxSwitchingBandPairs)) OF ULTxSwitchingBandPair-r16,

uplinkTxSwitching-OptionSupport-r16 ENUMERATED {switchedUL, dualUL, both}      OPTIONAL,

uplinkTxSwitching-PowerBoosting-r16 ENUMERATED {supported}                     OPTIONAL,

...,

[[

-- R4 16-5 UL-MIMO coherence capability for dynamic Tx switching between 3CC 1Tx-2Tx switching

uplinkTxSwitching-PUSCH-TransCoherence-r16     ENUMERATED {nonCoherent, fullCoherent}   OPTIONAL

]]

}

BandCombination-UplinkTxSwitch-v1630 ::=    SEQUENCE {

bandCombination-v1630                       BandCombination-v1630              OPTIONAL

}

BandCombination-UplinkTxSwitch-v1640 ::=    SEQUENCE {

bandCombination-v1640                       BandCombination-v1640              OPTIONAL

}

BandCombination-UplinkTxSwitch-v1650 ::= SEQUENCE {

bandCombination-v1650               BandCombination-v1650                      OPTIONAL

}

BandCombination-UplinkTxSwitch-v1670 ::= SEQUENCE {

bandCombination-v15g0                    BandCombination-v15g0                 OPTIONAL

}

BandCombination-UplinkTxSwitch-v1690 ::=  SEQUENCE {

bandCombination-v1690                     BandCombination-v1690                OPTIONAL

}

BandCombination-UplinkTxSwitch-v16a0 ::= SEQUENCE {

bandCombination-v16a0                    BandCombination-v16a0                 OPTIONAL

}

BandCombination-UplinkTxSwitch-v1700 ::= SEQUENCE {

bandCombination-v1700                    BandCombination-v1700                      OPTIONAL,

-- R4 16-1/16-2/16-3 Dynamic Tx switching between 2CC/3CC 2Tx-2Tx/1Tx-2Tx switching

supportedBandPairListNR-v1700            SEQUENCE (SIZE (1..maxULTxSwitchingBandPairs)) OF ULTxSwitchingBandPair-v1700  OPTIONAL,

-- R4 16-6: UL-MIMO coherence capability for dynamic Tx switching between 2Tx-2Tx switching

uplinkTxSwitchingBandParametersList-v1700 SEQUENCE (SIZE (1.. maxSimultaneousBands)) OF UplinkTxSwitchingBandParameters-v1700  OPTIONAL

}

BandCombination-UplinkTxSwitch-v1720 ::= SEQUENCE {

bandCombination-v1720                    BandCombination-v1720                 OPTIONAL,

uplinkTxSwitching-OptionSupport2T2T-r17  ENUMERATED {switchedUL, dualUL, both} OPTIONAL

}

BandCombination-UplinkTxSwitch-v1730 ::= SEQUENCE {

bandCombination-v1730                    BandCombination-v1730                 OPTIONAL

}

BandCombination-UplinkTxSwitch-v1740 ::= SEQUENCE {

bandCombination-v1740                    BandCombination-v1740                 OPTIONAL

}

ULTxSwitchingBandPair-r16 ::=       SEQUENCE {

bandIndexUL1-r16                    INTEGER(1..maxSimultaneousBands),

bandIndexUL2-r16                    INTEGER(1..maxSimultaneousBands),

uplinkTxSwitchingPeriod-r16         ENUMERATED {n35us, n140us, n210us},

uplinkTxSwitching-DL-Interruption-r16 BIT STRING (SIZE(1..maxSimultaneousBands)) OPTIONAL

}

ULTxSwitchingBandPair-v1700 ::=     SEQUENCE {

uplinkTxSwitchingPeriod2T2T-r17     ENUMERATED {n35us, n140us, n210us}     OPTIONAL

}

UplinkTxSwitchingBandParameters-v1700 ::=       SEQUENCE {

bandIndex-r17                                   INTEGER(1..maxSimultaneousBands),

uplinkTxSwitching2T2T-PUSCH-TransCoherence-r17  ENUMERATED {nonCoherent, fullCoherent}            OPTIONAL

}

BandParameters ::=                      CHOICE {

eutra                               SEQUENCE {

bandEUTRA                           FreqBandIndicatorEUTRA,

ca-BandwidthClassDL-EUTRA           CA-BandwidthClassEUTRA                 OPTIONAL,

ca-BandwidthClassUL-EUTRA           CA-BandwidthClassEUTRA                 OPTIONAL

},

nr                                  SEQUENCE {

bandNR                              FreqBandIndicatorNR,

ca-BandwidthClassDL-NR              CA-BandwidthClassNR                    OPTIONAL,

ca-BandwidthClassUL-NR              CA-BandwidthClassNR                    OPTIONAL

}

}

BandParameters-v1540 ::=            SEQUENCE {

srs-CarrierSwitch                   CHOICE {

nr                                  SEQUENCE {

srs-SwitchingTimesListNR            SEQUENCE (SIZE (1..maxSimultaneousBands)) OF SRS-SwitchingTimeNR

},

eutra                               SEQUENCE {

srs-SwitchingTimesListEUTRA         SEQUENCE (SIZE (1..maxSimultaneousBands)) OF SRS-SwitchingTimeEUTRA

}

}                                                                              OPTIONAL,

srs-TxSwitch                    SEQUENCE {

supportedSRS-TxPortSwitch       ENUMERATED {t1r2, t1r4, t2r4, t1r4-t2r4, t1r1, t2r2, t4r4, notSupported},

txSwitchImpactToRx              INTEGER (1..32)                            OPTIONAL,

txSwitchWithAnotherBand         INTEGER (1..32)                            OPTIONAL

}                                                                              OPTIONAL

}

BandParameters-v1610 ::=         SEQUENCE {

srs-TxSwitch-v1610               SEQUENCE {

supportedSRS-TxPortSwitch-v1610  ENUMERATED {t1r1-t1r2, t1r1-t1r2-t1r4, t1r1-t1r2-t2r2-t2r4, t1r1-t1r2-t2r2-t1r4-t2r4,

t1r1-t2r2, t1r1-t2r2-t4r4}

}                                                                              OPTIONAL

}

BandParameters-v1710 ::=         SEQUENCE {

-- R1 23-8-3	SRS Antenna switching for >4Rx

srs-AntennaSwitchingBeyond4RX-r17                     SEQUENCE {

-- 1. Support of SRS antenna switching xTyR with y>4

supportedSRS-TxPortSwitchBeyond4Rx-r17                BIT STRING (SIZE (11)),

-- 2. Report the entry number of the first-listed band with UL in the band combination that affects this DL

entryNumberAffectBeyond4Rx-r17                        INTEGER (1..32)      OPTIONAL,

-- 3. Report the entry number of the first-listed band with UL in the band combination that switches together with this UL

entryNumberSwitchBeyond4Rx-r17                        INTEGER (1..32)      OPTIONAL

}                                                                              OPTIONAL

}

BandParameters-v1730 ::= SEQUENCE {

-- R1 39-3-2	Affected bands for inter-band CA during SRS carrier switching

srs-SwitchingAffectedBandsListNR-r17    SEQUENCE (SIZE (1..maxSimultaneousBands)) OF SRS-SwitchingAffectedBandsNR-r17

}

ScalingFactorSidelink-r16 ::=       ENUMERATED {f0p4, f0p75, f0p8, f1}

IntraBandPowerClass-r16 ::=         ENUMERATED {pc2, pc3, spare6, spare5, spare4, spare3, spare2, spare1}

SRS-SwitchingAffectedBandsNR-r17 ::= BIT STRING (SIZE (1..maxSimultaneousBands))

-- TAG-BANDCOMBINATIONLIST-STOP

-- ASN1STOP

| BandCombination field descriptions |
| --- |
| BandCombinationList-v1540, BandCombinationList-v1550, BandCombinationList-v1560, BandCombinationList-v1570, BandCombinationList-v1580, BandCombinationList-v1590, BandCombinationList-v15g0, BandCombinationList-v1610, BandCombinationList-v1630, BandCombinationList-v1640, BandCombinationList-v1650, BandCombinationList-v1680, BandCombinationList-v1690, BandCombinationList-v16a0, BandCombinationList-v1700, BandCombinationList-v1720, BandCombinationList-v1730 The UE shall include the same number of entries, and listed in the same order, as in BandCombinationList (without suffix). If the field is included in supportedBandCombinationListNEDC-Only-v1610, the UE shall include the same number of entries, and listed in the same order, as in BandCombinationList of supportedBandCombinationListNEDC-Only (without suffix) field. If the field is included in supportedBandCombinationListNEDC-Only-v15a0, the UE shall include the same number of entries, and listed in the same order, as in BandCombinationList (without suffix) of supportedBandCombinationListNEDC-Only (without suffix) field. |
| BandCombinationList-UplinkTxSwitch-r16, BandCombinationList-UplinkTxSwitch-v1630, BandCombinationList-UplinkTxSwitch-v1640, BandCombinationList-UplinkTxSwitch-v1650, BandCombinationList-UplinkTxSwitch-v1690, BandCombinationList-UplinkTxSwitch-v16a0, BandCombinationList-UplinkTxSwitch-v1700, BandCombinationList-UplinkTxSwitch-v1720, BandCombinationList-UplinkTxSwitch-v1730 The UE shall include the same number of entries, and listed in the same order, as in BandCombinationList-UplinkTxSwitch-r16. For the field of supportedBandCombinationList-UplinkTxSwitch-v1700, if the UE does not support 2Tx-2Tx switching for a given band combination, the field of supportedBandPairListNR-v1700 in the corresponding entry is absent. |
| ca-ParametersNRDC If the field is included for a band combination in the NR capability container, the field indicates support of NR-DC. Otherwise, the field is absent. |
| featureSetCombinationDAPS If this field is present for a band combination, it reports the feature set combination supported for the band combination when any DAPS bearer is configured. |
| ne-DC-BC If the field is included for a band combination in the MR-DC capability container, the field indicates support of NE-DC. Otherwise, the field is absent. |
| supportedBandPairListNR-r16, supportedBandPairListNR-v1700 Indicates a list of band pair supporting UL Tx switching as defined in TS 38.101-1 [15] for a given band combination. A UE supporting 2Tx-2Tx switching should include both of supportedBandPairListNR-r16 and supportedBandPairListNR-v1700. And the UE shall include the same number of entries listed in the same order as in supportedBandPairListNR-r16. If the UE does not support 2Tx-2Tx switching for a given band pair, the field of uplinkTxSwitchingPeriod2T2T in the corresponding entry is absent. |
| srs-SwitchingTimesListNR Indicates, for a particular pair of NR bands, the RF retuning time when switching between a NR carrier corresponding to this band entry and another (PUSCH-less) NR carrier corresponding to the band entry in the order indicated below: - For the first NR band, the UE shall include the same number of entries for NR bands as in bandList, i.e. first entry corresponds to first NR band in bandList and so on, - For the second NR band, the UE shall include one entry less, i.e. first entry corresponds to the second NR band in bandList and so on - And so on |
| srs-SwitchingTimesListEUTRA Indicates, for a particular pair of E-UTRA bands, the RF retuning time when switching between an E-UTRA carrier corresponding to this band entry and another (PUSCH-less) E-UTRA carrier corresponding to the band entry in the order indicated below: - For the first E-UTRA band, the UE shall include the same number of entries for E-UTRA bands as in bandList, i.e. first entry corresponds to first E-UTRA band in bandList and so on, - For the second E-UTRA band, the UE shall include one entry less, i.e. first entry corresponds to the second E-UTRA band in bandList and so on - And so on |
| srs-TxSwitch Indicates supported SRS antenna switch capability for the associated band. If the UE indicates support of SRS-SwitchingTimeNR, the UE is allowed to set this field for a band with associated FeatureSetUplinkId set to 0 for SRS carrier switching. |  |
| uplinkTxSwitchingBandParametersList-v1700 Indicates a list of per band per band combination capabilities for UL Tx switching. |  |

#### –	BandCombinationListSidelinkEUTRA-NR

The IE BandCombinationListSidelinkEUTRA-NR contains a list of V2X sidelink and NR sidelink band combinations.

BandCombinationListSidelinkEUTRA-NR information element

-- ASN1START

-- TAG-BANDCOMBINATIONLISTSIDELINKEUTRANR-START

BandCombinationListSidelinkEUTRA-NR-r16 ::= SEQUENCE (SIZE (1..maxBandComb)) OF BandCombinationParametersSidelinkEUTRA-NR-r16

BandCombinationListSidelinkEUTRA-NR-v1630 ::= SEQUENCE (SIZE (1..maxBandComb)) OF BandCombinationParametersSidelinkEUTRA-NR-v1630

BandCombinationListSidelinkEUTRA-NR-v1710 ::= SEQUENCE (SIZE (1..maxBandComb)) OF BandCombinationParametersSidelinkEUTRA-NR-v1710

BandCombinationParametersSidelinkEUTRA-NR-r16 ::= SEQUENCE (SIZE (1..maxSimultaneousBands)) OF BandParametersSidelinkEUTRA-NR-r16

BandCombinationParametersSidelinkEUTRA-NR-v1630 ::= SEQUENCE (SIZE (1..maxSimultaneousBands)) OF BandParametersSidelinkEUTRA-NR-v1630

BandCombinationParametersSidelinkEUTRA-NR-v1710 ::= SEQUENCE (SIZE (1..maxSimultaneousBands)) OF BandParametersSidelinkEUTRA-NR-v1710

BandParametersSidelinkEUTRA-NR-r16 ::= CHOICE {

eutra                                  SEQUENCE {

bandParametersSidelinkEUTRA1-r16       OCTET STRING                         OPTIONAL,

bandParametersSidelinkEUTRA2-r16       OCTET STRING                         OPTIONAL

},

nr                                     SEQUENCE {

bandParametersSidelinkNR-r16           BandParametersSidelink-r16

}

}

BandParametersSidelinkEUTRA-NR-v1630 ::= CHOICE {

eutra                                    NULL,

nr                                       SEQUENCE {

tx-Sidelink-r16                          ENUMERATED {supported}                          OPTIONAL,

rx-Sidelink-r16                          ENUMERATED {supported}                          OPTIONAL,

sl-CrossCarrierScheduling-r16            ENUMERATED {supported}                          OPTIONAL

}

}

BandParametersSidelinkEUTRA-NR-v1710 ::= CHOICE {

eutra                                    NULL,

nr                                       SEQUENCE {

--32-4

sl-TransmissionMode2-PartialSensing-r17  SEQUENCE {

harq-TxProcessModeTwoSidelink-r17        ENUMERATED {n8, n16},

scs-CP-PatternTxSidelinkModeTwo-r17      CHOICE {

fr1-r17                                  SEQUENCE {

scs-15kHz-r17                            BIT STRING (SIZE (16))                OPTIONAL,

scs-30kHz-r17                            BIT STRING (SIZE (16))                OPTIONAL,

scs-60kHz-r17                            BIT STRING (SIZE (16))                OPTIONAL

},

fr2-r17                                  SEQUENCE {

scs-60kHz-r17                            BIT STRING (SIZE (16))                OPTIONAL,

scs-120kHz-r17                           BIT STRING (SIZE (16))                OPTIONAL

}

}                                                                                      OPTIONAL,

extendedCP-Mode2PartialSensing-r17           ENUMERATED {supported}                    OPTIONAL,

dl-openLoopPC-Sidelink-r17                   ENUMERATED {supported}                    OPTIONAL

}                                                                                          OPTIONAL,

--32-2a:  Receiving NR sidelink of PSFCH

rx-sidelinkPSFCH-r17                     ENUMERATED {n5, n15, n25, n32, n35, n45, n50, n64} OPTIONAL,

--32-5a-1

tx-IUC-Scheme1-Mode2Sidelink-r17         ENUMERATED {supported}                            OPTIONAL,

--32-5b-1

tx-IUC-Scheme2-Mode2Sidelink-r17         ENUMERATED {n4, n8, n16}                          OPTIONAL

}

}

BandParametersSidelink-r16 ::= SEQUENCE {

freqBandSidelink-r16           FreqBandIndicatorNR

}

-- TAG-BANDCOMBINATIONLISTSIDELINKEUTRANR-STOP

-- ASN1STOP

BandParametersSidelinkEUTRA-NR field descriptions

bandParametersSidelinkEUTRA1, bandParametersSidelinkEUTRA2 This field includes the V2X-BandParameters-r14 and V2X-BandParameters-v1530 IE as specified in 36.331 [10]. It is used for reporting the per-band capability for V2X sidelink communication.

#### –	BandCombinationListSL-Discovery

The IE BandCombinationListSL-Discovery contains a list of NR Sidelink discovery band combinations.

BandCombinationListSidelinkSL-Discovery information element

-- ASN1START

-- TAG-BANDCOMBINATIONLISTSLDISCOVERY-START

BandCombinationListSL-Discovery-r17 ::= SEQUENCE (SIZE (1..maxSimultaneousBands)) OF BandParametersSidelinkDiscovery-r17

BandParametersSidelinkDiscovery-r17 ::= SEQUENCE {

sl-CrossCarrierScheduling-r17            ENUMERATED {supported}                            OPTIONAL,

--R1 32-4: Transmitting NR sidelink mode 2 with partial sensing

sl-TransmissionMode2-PartialSensing-r17  SEQUENCE {

harq-TxProcessModeTwoSidelink-r17        ENUMERATED {n8, n16},

scs-CP-PatternTxSidelinkModeTwo-r17      CHOICE {

fr1-r17                                  SEQUENCE {

scs-15kHz-r17                            BIT STRING (SIZE (16))                OPTIONAL,

scs-30kHz-r17                            BIT STRING (SIZE (16))                OPTIONAL,

scs-60kHz-r17                            BIT STRING (SIZE (16))                OPTIONAL

},

fr2-r17                                  SEQUENCE {

scs-60kHz-r17                            BIT STRING (SIZE (16))                OPTIONAL,

scs-120kHz-r17                           BIT STRING (SIZE (16))                OPTIONAL

}

}                                                                                      OPTIONAL,

extendedCP-Mode2PartialSensing-r17           ENUMERATED {supported}                    OPTIONAL,

dl-openLoopPC-Sidelink-r17                   ENUMERATED {supported}                    OPTIONAL

}                                                                                          OPTIONAL,

--R1 32-5a-1: Transmitting Inter-UE coordination scheme 1 in NR sidelink mode 2

tx-IUC-Scheme1-Mode2Sidelink-r17         ENUMERATED {supported}                            OPTIONAL

}

-- TAG-BANDCOMBINATIONLISTSLDISCOVERY-STOP

-- ASN1STOP

#### –	CA-BandwidthClassEUTRA

The IE CA-BandwidthClassEUTRA indicates the E-UTRA CA bandwidth class as defined in TS 36.101 [22], table 5.6A-1.

CA-BandwidthClassEUTRA information element

-- ASN1START

-- TAG-CA-BANDWIDTHCLASSEUTRA-START

CA-BandwidthClassEUTRA ::=          ENUMERATED {a, b, c, d, e, f, ...}

-- TAG-CA-BANDWIDTHCLASSEUTRA-STOP

-- ASN1STOP

#### –	CA-BandwidthClassNR

The IE CA-BandwidthClassNR indicates the NR CA bandwidth class as defined in TS 38.101-1 [15], table 5.3A.5-1 and TS 38.101-2 [39], table 5.3A.4-1.

CA-BandwidthClassNR information element

-- ASN1START

-- TAG-CA-BANDWIDTHCLASSNR-START

-- R4 17-6: new CA BW Classes R2~R12

CA-BandwidthClassNR ::=             ENUMERATED {a, b, c, d, e, f, g, h, i, j, k, l, m, n, o, p, q, ...,r2-v1730, r3-v1730, r4-v1730, r5-v1730, r6-v1730, r7-v1730, r8-v1730, r9-v1730, r10-v1730, r11-v1730, r12-v1730 }

-- TAG-CA-BANDWIDTHCLASSNR-STOP

-- ASN1STOP

#### –	CA-ParametersEUTRA

The IE CA-ParametersEUTRA contains the E-UTRA part of band combination parameters for a given MR-DC band combination.

NOTE:	If additional E-UTRA band combination parameters are defined in TS 36.331 [10], which are supported for MR-DC, they will be defined here as well.

CA-ParametersEUTRA information element

-- ASN1START

-- TAG-CA-PARAMETERSEUTRA-START

CA-ParametersEUTRA ::=                          SEQUENCE {

multipleTimingAdvance                           ENUMERATED {supported}                          OPTIONAL,

simultaneousRx-Tx                               ENUMERATED {supported}                          OPTIONAL,

supportedNAICS-2CRS-AP                          BIT STRING (SIZE (1..8))                        OPTIONAL,

additionalRx-Tx-PerformanceReq                  ENUMERATED {supported}                          OPTIONAL,

ue-CA-PowerClass-N                              ENUMERATED {class2}                             OPTIONAL,

supportedBandwidthCombinationSetEUTRA-v1530     BIT STRING (SIZE (1..32))                       OPTIONAL,

...

}

CA-ParametersEUTRA-v1560 ::=                    SEQUENCE {

fd-MIMO-TotalWeightedLayers                     INTEGER (2..128)                                OPTIONAL

}

CA-ParametersEUTRA-v1570 ::=                    SEQUENCE {

dl-1024QAM-TotalWeightedLayers                  INTEGER (0..10)                                 OPTIONAL

}

-- TAG-CA-PARAMETERSEUTRA-STOP

-- ASN1STOP

#### –	CA-ParametersNR

The IE CA-ParametersNR contains carrier aggregation and inter-frequency DAPS handover related capabilities that are defined per band combination.

CA-ParametersNR information element

-- ASN1START

-- TAG-CA-PARAMETERSNR-START

CA-ParametersNR ::=                 SEQUENCE {

dummy                                         ENUMERATED {supported}      OPTIONAL,

parallelTxSRS-PUCCH-PUSCH                     ENUMERATED {supported}      OPTIONAL,

parallelTxPRACH-SRS-PUCCH-PUSCH               ENUMERATED {supported}      OPTIONAL,

simultaneousRxTxInterBandCA                   ENUMERATED {supported}      OPTIONAL,

simultaneousRxTxSUL                           ENUMERATED {supported}      OPTIONAL,

diffNumerologyAcrossPUCCH-Group               ENUMERATED {supported}      OPTIONAL,

diffNumerologyWithinPUCCH-GroupSmallerSCS     ENUMERATED {supported}      OPTIONAL,

supportedNumberTAG                            ENUMERATED {n2, n3, n4}     OPTIONAL,

...

}

CA-ParametersNR-v1540 ::=           SEQUENCE {

simultaneousSRS-AssocCSI-RS-AllCC                       INTEGER (5..32)         OPTIONAL,

csi-RS-IM-ReceptionForFeedbackPerBandComb               SEQUENCE {

maxNumberSimultaneousNZP-CSI-RS-ActBWP-AllCC            INTEGER (1..64)     OPTIONAL,

totalNumberPortsSimultaneousNZP-CSI-RS-ActBWP-AllCC     INTEGER (2..256)    OPTIONAL

}                                                                               OPTIONAL,

simultaneousCSI-ReportsAllCC                            INTEGER (5..32)         OPTIONAL,

dualPA-Architecture                                     ENUMERATED {supported}  OPTIONAL

}

CA-ParametersNR-v1550 ::=           SEQUENCE {

dummy                               ENUMERATED {supported}                      OPTIONAL

}

CA-ParametersNR-v1560 ::=           SEQUENCE {

diffNumerologyWithinPUCCH-GroupLargerSCS      ENUMERATED {supported}            OPTIONAL

}

CA-ParametersNR-v15g0 ::=           SEQUENCE {

simultaneousRxTxInterBandCAPerBandPair        SimultaneousRxTxPerBandPair       OPTIONAL,

simultaneousRxTxSULPerBandPair                SimultaneousRxTxPerBandPair       OPTIONAL

}

CA-ParametersNR-v1610 ::=           SEQUENCE {

-- R1 9-3: Parallel MsgA and SRS/PUCCH/PUSCH transmissions across CCs in inter-band CA

parallelTxMsgA-SRS-PUCCH-PUSCH-r16                ENUMERATED {supported}        OPTIONAL,

-- R1 9-4: MsgA operation in a band combination including SUL

msgA-SUL-r16                                      ENUMERATED {supported}        OPTIONAL,

-- R1 10-9c: Joint search space group switching across multiple cells

jointSearchSpaceSwitchAcrossCells-r16             ENUMERATED {supported}        OPTIONAL,

-- R1 14-5: Half-duplex UE behaviour in TDD CA for same SCS

half-DuplexTDD-CA-SameSCS-r16                     ENUMERATED {supported}        OPTIONAL,

-- R1 18-4: SCell dormancy within active time

scellDormancyWithinActiveTime-r16                 ENUMERATED {supported}        OPTIONAL,

-- R1 18-4a: SCell dormancy outside active time

scellDormancyOutsideActiveTime-r16                ENUMERATED {supported}        OPTIONAL,

-- R1 18-6: Cross-carrier A-CSI RS triggering with different SCS

crossCarrierA-CSI-trigDiffSCS-r16                 ENUMERATED {higherA-CSI-SCS,lowerA-CSI-SCS,both}   OPTIONAL,

-- R1 18-6a: Default QCL assumption for cross-carrier A-CSI-RS triggering

defaultQCL-CrossCarrierA-CSI-Trig-r16             ENUMERATED {diffOnly, both}   OPTIONAL,

-- R1 18-7: CA with non-aligned frame boundaries for inter-band CA

interCA-NonAlignedFrame-r16                       ENUMERATED {supported}        OPTIONAL,

simul-SRS-Trans-BC-r16                            ENUMERATED {n2}               OPTIONAL,

interFreqDAPS-r16                                 SEQUENCE {

interFreqAsyncDAPS-r16                            ENUMERATED {supported}    OPTIONAL,

interFreqDiffSCS-DAPS-r16                         ENUMERATED {supported}    OPTIONAL,

interFreqMultiUL-TransmissionDAPS-r16             ENUMERATED {supported}    OPTIONAL,

interFreqSemiStaticPowerSharingDAPS-Mode1-r16     ENUMERATED {supported}    OPTIONAL,

interFreqSemiStaticPowerSharingDAPS-Mode2-r16     ENUMERATED {supported}    OPTIONAL,

interFreqDynamicPowerSharingDAPS-r16              ENUMERATED {short, long}  OPTIONAL,

interFreqUL-TransCancellationDAPS-r16             ENUMERATED {supported}    OPTIONAL

}                                                                               OPTIONAL,

codebookParametersPerBC-r16                       CodebookParameters-v1610      OPTIONAL,

-- R1 16-2a-10 Value of R for BD/CCE

blindDetectFactor-r16                             INTEGER (1..2)                OPTIONAL,

-- R1 11-2a: Capability on the number of CCs for monitoring a maximum number of BDs and non-overlapped CCEs per span when configured

-- with DL CA with Rel-16 PDCCH monitoring capability on all the serving cells

pdcch-MonitoringCA-r16                            SEQUENCE {

maxNumberOfMonitoringCC-r16                       INTEGER (2..16),

supportedSpanArrangement-r16                      ENUMERATED {alignedOnly, alignedAndNonAligned}

}                                                                               OPTIONAL,

-- R1 11-2c: Number of carriers for CCE/BD scaling with DL CA with mix of Rel. 16 and Rel. 15 PDCCH monitoring capabilities on

-- different carriers

pdcch-BlindDetectionCA-Mixed-r16                  SEQUENCE {

pdcch-BlindDetectionCA1-r16                       INTEGER (1..15),

pdcch-BlindDetectionCA2-r16                       INTEGER (1..15),

supportedSpanArrangement-r16                      ENUMERATED {alignedOnly, alignedAndNonAligned}

}                                                                               OPTIONAL,

-- R1 11-2d: Capability on the number of CCs for monitoring a maximum number of BDs and non-overlapped CCEs per span for MCG and for

-- SCG when configured for NR-DC operation with Rel-16 PDCCH monitoring capability on all the serving cells

pdcch-BlindDetectionMCG-UE-r16                    INTEGER (1..14)               OPTIONAL,

pdcch-BlindDetectionSCG-UE-r16                    INTEGER (1..14)               OPTIONAL,

-- R1 11-2e: Number of carriers for CCE/BD scaling for MCG and for SCG when configured for NR-DC operation with mix of Rel. 16 and

-- Rel. 15 PDCCH monitoring capabilities on different carriers

pdcch-BlindDetectionMCG-UE-Mixed-r16              SEQUENCE {

pdcch-BlindDetectionMCG-UE1-r16                   INTEGER (0..15),

pdcch-BlindDetectionMCG-UE2-r16                   INTEGER (0..15)

}                                                                               OPTIONAL,

pdcch-BlindDetectionSCG-UE-Mixed-r16              SEQUENCE {

pdcch-BlindDetectionSCG-UE1-r16                   INTEGER (0..15),

pdcch-BlindDetectionSCG-UE2-r16                   INTEGER (0..15)

}                                                                               OPTIONAL,

-- R1 18-5 cross-carrier scheduling with different SCS in DL CA

crossCarrierSchedulingDL-DiffSCS-r16              ENUMERATED {low-to-high, high-to-low, both} OPTIONAL,

-- R1 18-5a Default QCL assumption for cross-carrier scheduling

crossCarrierSchedulingDefaultQCL-r16              ENUMERATED {diff-only, both}  OPTIONAL,

-- R1 18-5b cross-carrier scheduling with different SCS in UL CA

crossCarrierSchedulingUL-DiffSCS-r16              ENUMERATED {low-to-high, high-to-low, both} OPTIONAL,

-- R1 13.19a Simultaneous positioning SRS and MIMO SRS transmission for a given BC

simul-SRS-MIMO-Trans-BC-r16                       ENUMERATED {n2}               OPTIONAL,

-- R1 16-3a, 16-3a-1, 16-3b, 16-3b-1: New Individual Codebook

codebookParametersAdditionPerBC-r16               CodebookParametersAdditionPerBC-r16         OPTIONAL,

-- R1 16-8: Mixed codebook

codebookComboParametersAdditionPerBC-r16          CodebookComboParametersAdditionPerBC-r16    OPTIONAL

}

CA-ParametersNR-v1630 ::= SEQUENCE {

-- R1 22-5b: Simultaneous transmission of SRS for antenna switching and SRS for CB/NCB /BM for inter-band UL CA

-- R1 22-5d: Simultaneous transmission of SRS for antenna switching for inter-band UL CA

simulTX-SRS-AntSwitchingInterBandUL-CA-r16        SimulSRS-ForAntennaSwitching-r16            OPTIONAL,

-- R4 8-5: supported beam management type for inter-band CA

beamManagementType-r16                            ENUMERATED {ibm, dummy}                       OPTIONAL,

-- R4 7-3a: UL frequency separation class with aggregate BW and Gap BW

intraBandFreqSeparationUL-AggBW-GapBW-r16         ENUMERATED {classI, classII, classIII}      OPTIONAL,

-- RAN 89: Case B in case of Inter-band CA with non-aligned frame boundaries

interCA-NonAlignedFrame-B-r16                     ENUMERATED {supported}                      OPTIONAL

}

CA-ParametersNR-v1640 ::= SEQUENCE {

-- R4 7-5: Support of reporting UL Tx DC locations for uplink intra-band CA.

uplinkTxDC-TwoCarrierReport-r16                               ENUMERATED {supported}          OPTIONAL,

-- RAN 22-6: Support of up to 3 different numerologies in the same NR PUCCH group for NR part of EN-DC, NGEN-DC, NE-DC and NR-CA

-- where UE is not configured with two NR PUCCH groups

maxUpTo3Diff-NumerologiesConfigSinglePUCCH-grp-r16            PUCCH-Grp-CarrierTypes-r16      OPTIONAL,

-- RAN 22-6a: Support of up to 4 different numerologies in the same NR PUCCH group for NR part of EN-DC, NGEN-DC, NE-DC and NR-CA

-- where UE is not configured with two NR PUCCH groups

maxUpTo4Diff-NumerologiesConfigSinglePUCCH-grp-r16            PUCCH-Grp-CarrierTypes-r16      OPTIONAL,

-- RAN 22-7: Support two PUCCH groups for NR-CA with 3 or more bands with at least two carrier types

twoPUCCH-Grp-ConfigurationsList-r16 SEQUENCE (SIZE (1..maxTwoPUCCH-Grp-ConfigList-r16)) OF TwoPUCCH-Grp-Configurations-r16 OPTIONAL,

-- R1 22-7a: Different numerology across NR PUCCH groups

diffNumerologyAcrossPUCCH-Group-CarrierTypes-r16              ENUMERATED {supported}          OPTIONAL,

-- R1 22-7b: Different numerologies across NR carriers within the same NR PUCCH group, with PUCCH on a carrier of smaller SCS

diffNumerologyWithinPUCCH-GroupSmallerSCS-CarrierTypes-r16    ENUMERATED {supported}          OPTIONAL,

-- R1 22-7c: Different numerologies across NR carriers within the same NR PUCCH group, with PUCCH on a carrier of larger SCS

diffNumerologyWithinPUCCH-GroupLargerSCS-CarrierTypes-r16     ENUMERATED {supported}          OPTIONAL,

-- R1 11-2f: add the replicated FGs of 11-2a/c with restriction for non-aligned span case

-- with DL CA with Rel-16 PDCCH monitoring capability on all the serving cells

pdcch-MonitoringCA-NonAlignedSpan-r16                         INTEGER (2..16)                 OPTIONAL,

-- R1 11-2g: add the replicated FGs of 11-2a/c with restriction for non-aligned span case

pdcch-BlindDetectionCA-Mixed-NonAlignedSpan-r16               SEQUENCE {

pdcch-BlindDetectionCA1-r16                                   INTEGER (1..15),

pdcch-BlindDetectionCA2-r16                                   INTEGER (1..15)

}                                                                                             OPTIONAL

}

CA-ParametersNR-v1690 ::= SEQUENCE {

csi-ReportingCrossPUCCH-Grp-r16          SEQUENCE {

computationTimeForA-CSI-r16              ENUMERATED {sameAsNoCross, relaxed},

additionalSymbols-r16                    SEQUENCE {

scs-15kHz-additionalSymbols-r16          ENUMERATED {s14, s28}            OPTIONAL,

scs-30kHz-additionalSymbols-r16          ENUMERATED {s14, s28}            OPTIONAL,

scs-60kHz-additionalSymbols-r16          ENUMERATED {s14, s28, s56}       OPTIONAL,

scs-120kHz-additionalSymbols-r16         ENUMERATED {s14, s28, s56}       OPTIONAL

}                                                                             OPTIONAL,

sp-CSI-ReportingOnPUCCH-r16              ENUMERATED {supported}               OPTIONAL,

sp-CSI-ReportingOnPUSCH-r16              ENUMERATED {supported}               OPTIONAL,

carrierTypePairList-r16                  SEQUENCE (SIZE (1..maxCarrierTypePairList-r16)) OF CarrierTypePair-r16

}                                                                                 OPTIONAL

}

CA-ParametersNR-v16a0 ::= SEQUENCE {

pdcch-BlindDetectionMixedList-r16    SEQUENCE(SIZE(1..maxNrofPdcch-BlindDetectionMixed-1-r16)) OF PDCCH-BlindDetectionMixedList-r16

}

CA-ParametersNR-v1700 ::= SEQUENCE {

-- R1 23-9-1: Basic Features of Further Enhanced Port-Selection Type II Codebook (FeType-II) per band combination information

codebookParametersfetype2PerBC-r17               CodebookParametersfetype2PerBC-r17           OPTIONAL,

-- R4 18-4: Support of enhanced Demodulation requirements for CA in HST SFN FR1

demodulationEnhancementCA-r17                    ENUMERATED {supported}                       OPTIONAL,

-- R4 20-1: Maximum uplink duty cycle for NR inter-band CA power class 2

maxUplinkDutyCycle-interBandCA-PC2-r17           ENUMERATED {n50, n60, n70, n80, n90, n100}   OPTIONAL,

-- R4 20-2: Maximum uplink duty cycle for NR SUL combination power class 2

maxUplinkDutyCycle-SULcombination-PC2-r17        ENUMERATED {n50, n60, n70, n80, n90, n100}   OPTIONAL,

beamManagementType-CBM-r17                       ENUMERATED {supported}                       OPTIONAL,

-- R1 25-18: Parallel PUCCH and PUSCH transmission across CCs in inter-band CA

parallelTxPUCCH-PUSCH-r17                        ENUMERATED {supported}      OPTIONAL,

-- R1 23-9-5	Active CSI-RS resources and ports for mixed codebook types in any slot per band combination

codebookComboParameterMixedTypePerBC-r17         CodebookComboParameterMixedTypePerBC-r17     OPTIONAL,

-- R1 23-7-1	Basic Features of CSI Enhancement for Multi-TRP

mTRP-CSI-EnhancementPerBC-r17                    SEQUENCE {

maxNumNZP-CSI-RS-r17                             INTEGER (2..8),

cSI-Report-mode-r17                              ENUMERATED {mode1, mode2, both},

supportedComboAcrossCCs-r17                      SEQUENCE (SIZE (1..16)) OF CSI-MultiTRP-SupportedCombinations-r17,

codebookMode-NCJT-r17	ENUMERATED{mode1,mode1And2}

}                                                                                             OPTIONAL,

-- R1 23-7-1b	Active CSI-RS resources and ports in the presence of multi-TRP CSI

codebookComboParameterMultiTRP-PerBC-r17         CodebookComboParameterMultiTRP-PerBC-r17     OPTIONAL,

-- R1 24-8b: 32 DL HARQ processes for FR 2-2 - maximum number of component carriers

maxCC-32-DL-HARQ-ProcessFR2-2-r17                ENUMERATED {n1, n2, n3, n4, n6, n8, n16, n32} OPTIONAL,

-- R1 24-9b: 32 UL HARQ processes for FR 2-2 - maximum number of component carriers

maxCC-32-UL-HARQ-ProcessFR2-2-r17                ENUMERATED {n1, n2, n3, n4, n5, n8, n16, n32}  OPTIONAL,

-- R1 34-2: Cross-carrier scheduling from SCell to PCell/PSCell (Type B)

crossCarrierSchedulingSCell-SpCellTypeB-r17      CrossCarrierSchedulingSCell-SpCell-r17       OPTIONAL,

-- R1 34-1: Cross-carrier scheduling from SCell to PCell/PSCell with search space restrictions (Type A)

crossCarrierSchedulingSCell-SpCellTypeA-r17      CrossCarrierSchedulingSCell-SpCell-r17       OPTIONAL,

-- R1 34-1a: DCI formats on PCell/PSCell USS set(s) support

dci-FormatsPCellPSCellUSS-Sets-r17               ENUMERATED {supported}                       OPTIONAL,

-- R1 34-3: Disabling scaling factor alpha when sSCell is deactivated

disablingScalingFactorDeactSCell-r17             ENUMERATED {supported}                       OPTIONAL,

-- R1 34-4: Disabling scaling factor alpha when sSCell is deactivated

disablingScalingFactorDormantSCell-r17           ENUMERATED {supported}                       OPTIONAL,

-- R1 34-5: Non-aligned frame boundaries between PCell/PSCell and sSCell

non-AlignedFrameBoundaries-r17 SEQUENCE {

scs15kHz-15kHz-r17                           BIT STRING (SIZE (1..496))                   OPTIONAL,

scs15kHz-30kHz-r17                           BIT STRING (SIZE (1..496))                   OPTIONAL,

scs15kHz-60kHz-r17                           BIT STRING (SIZE (1..496))                   OPTIONAL,

scs30kHz-30kHz-r17                           BIT STRING (SIZE (1..496))                   OPTIONAL,

scs30kHz-60kHz-r17                           BIT STRING (SIZE (1..496))                   OPTIONAL,

scs60kHz-60kHz-r17                           BIT STRING (SIZE (1..496))                   OPTIONAL

}                                                                                             OPTIONAL

}

CA-ParametersNR-v1720 ::= SEQUENCE {

-- R1 39-1: Parallel SRS and PUCCH/PUSCH transmission across CCs in intra-band non-contiguous CA

parallelTxSRS-PUCCH-PUSCH-intraBand-r17          ENUMERATED {supported}                       OPTIONAL,

-- R1 39-2: Parallel PRACH and SRS/PUCCH/PUSCH transmissions across CCs in intra-band non-contiguous CA

parallelTxPRACH-SRS-PUCCH-PUSCH-intraBand-r17    ENUMERATED {supported}                       OPTIONAL,

-- R1 25-9: Semi-static PUCCH cell switching for a single PUCCH group only

semiStaticPUCCH-CellSwitchSingleGroup-r17        SEQUENCE {

pucch-Group-r17                                ENUMERATED {primaryGroupOnly, secondaryGroupOnly, eitherPrimaryOrSecondaryGroup},

pucch-Group-Config-r17                           PUCCH-Group-Config-r17

}                                                                                             OPTIONAL,

-- R1 25-9a: Semi-static PUCCH cell switching for two PUCCH groups

semiStaticPUCCH-CellSwitchTwoGroups-r17    SEQUENCE (SIZE (1..maxTwoPUCCH-Grp-ConfigList-r17)) OF TwoPUCCH-Grp-Configurations-r17 OPTIONAL,

-- R1 25-10: PUCCH cell switching based on dynamic indication for same length of overlapping PUCCH slots/sub-slots for a single

-- PUCCH group only

dynamicPUCCH-CellSwitchSameLengthSingleGroup-r17 SEQUENCE {

pucch-Group-r17                                  ENUMERATED {primaryGroupOnly, secondaryGroupOnly, eitherPrimaryOrSecondaryGroup},

pucch-Group-Config-r17                       PUCCH-Group-Config-r17

}                                                                                             OPTIONAL,

-- R1 25-10a: PUCCH cell switching based on dynamic indication for different length of overlapping PUCCH slots/sub-slots

-- for a single PUCCH group only

dynamicPUCCH-CellSwitchDiffLengthSingleGroup-r17 SEQUENCE {

pucch-Group-r17                                  ENUMERATED {primaryGroupOnly, secondaryGroupOnly, eitherPrimaryOrSecondaryGroup},

pucch-Group-Config-r17                           PUCCH-Group-Config-r17

}                                                                                             OPTIONAL,

-- R1 25-10b: PUCCH cell switching based on dynamic indication for same length of overlapping PUCCH slots/sub-slots for two PUCCH

-- groups

dynamicPUCCH-CellSwitchSameLengthTwoGroups-r17   SEQUENCE (SIZE (1..maxTwoPUCCH-Grp-ConfigList-r17)) OF TwoPUCCH-Grp-Configurations-r17

OPTIONAL,

-- R1 25-10c: PUCCH cell switching based on dynamic indication for different length of overlapping PUCCH slots/sub-slots for two

-- PUCCH groups

dynamicPUCCH-CellSwitchDiffLengthTwoGroups-r17   SEQUENCE (SIZE (1..maxTwoPUCCH-Grp-ConfigList-r17)) OF TwoPUCCH-Grp-Configurations-r17

OPTIONAL,

-- R1 33-2a: ACK/NACK based HARQ-ACK feedback and RRC-based enabling/disabling ACK/NACK-based

-- feedback for dynamic scheduling for multicast

ack-NACK-FeedbackForMulticast-r17                ENUMERATED {supported}                       OPTIONAL,

-- R1 33-2d: PTP retransmission for multicast dynamic scheduling

ptp-Retx-Multicast-r17                           ENUMERATED {supported}                       OPTIONAL,

-- R1 33-4: NACK-only based HARQ-ACK feedback for RRC-based enabling/disabling multicast with ACK/NACK transforming

nack-OnlyFeedbackForMulticast-r17                ENUMERATED {supported}                       OPTIONAL,

-- R1 33-4a: NACK-only based HARQ-ACK feedback for multicast corresponding to a specific sequence or a PUCCH transmission

nack-OnlyFeedbackSpecificResourceForMulticast-r17 ENUMERATED {supported}                      OPTIONAL,

-- R1 33-5-1a: ACK/NACK based HARQ-ACK feedback and RRC-based enabling/disabling ACK/NACK-based feedback

-- for SPS group-common PDSCH for multicast

ack-NACK-FeedbackForSPS-Multicast-r17            ENUMERATED {supported}                       OPTIONAL,

-- R1 33-5-1d: PTP retransmission for SPS group-common PDSCH for multicast

ptp-Retx-SPS-Multicast-r17                       ENUMERATED {supported}                       OPTIONAL,

-- R4 26-1: Higher Power Limit CA DC

higherPowerLimit-r17                             ENUMERATED {supported}                       OPTIONAL,

-- R1 39-4: Parallel MsgA and SRS/PUCCH/PUSCH transmissions across CCs in intra-band non-contiguous CA

parallelTxMsgA-SRS-PUCCH-PUSCH-intraBand-r17     ENUMERATED {supported}                       OPTIONAL,

-- R1 24-11a: Capability on the number of CCs for monitoring a maximum number of BDs and non-overlapped CCEs per span when

-- configured with DL CA with Rel-17 PDCCH monitoring capability on all the serving cells

pdcch-MonitoringCA-r17                           INTEGER (4..16)                              OPTIONAL,

-- R1 24-11f: Capability on the number of CCs for monitoring a maximum number of BDs and non-overlapped CCEs for MCG and for SCG

-- when configured for NR-DC operation with Rel-17 PDCCH monitoring capability on all the serving cells

pdcch-BlindDetectionMCG-SCG-List-r17             SEQUENCE(SIZE(1..maxNrofPdcch-BlindDetection-r17)) OF PDCCH-BlindDetectionMCG-SCG-r17

OPTIONAL,

-- R1 24-11c: Number of carriers for CCE/BD scaling with DL CA with mix of Rel. 17 and Rel. 15 PDCCH monitoring capabilities on

-- different Carriers

-- R1 24-11g: Number of carriers for CCE/BD scaling for MCG and for SCG when configured for NR-DC operation with mix of Rel. 17 and

-- Rel. 15 PDCCH monitoring capabilities on different carriers

pdcch-BlindDetectionMixedList1-r17               SEQUENCE(SIZE(1..maxNrofPdcch-BlindDetection-r17)) OF PDCCH-BlindDetectionMixed-r17

OPTIONAL,

-- R1 24-11d: Number of carriers for CCE/BD scaling with DL CA with mix of Rel. 17 and Rel. 16 PDCCH monitoring capabilities on

-- different Carriers

-- R1 24-11h: Number of carriers for CCE/BD scaling for MCG and for SCG when configured for NR-DC operation with mix of Rel. 17 and

-- Rel. 16 PDCCH monitoring capabilities on different carriers

pdcch-BlindDetectionMixedList2-r17               SEQUENCE(SIZE(1..maxNrofPdcch-BlindDetection-r17)) OF PDCCH-BlindDetectionMixed-r17

OPTIONAL,

-- R1 24-11e: Number of carriers for CCE/BD scaling with DL CA with mix of Rel. 17, Rel. 16 and Rel. 15 PDCCH monitoring

-- capabilities on different carriers

-- R1 24-11i: Number of carriers for CCE/BD scaling for MCG and for SCG when configured for NR-DC operation with mix of Rel. 17,

-- Rel. 16 and Rel. 15 PDCCH monitoring capabilities on different carriers

pdcch-BlindDetectionMixedList3-r17               SEQUENCE(SIZE(1..maxNrofPdcch-BlindDetection-r17)) OF PDCCH-BlindDetectionMixed1-r17

OPTIONAL

}

CA-ParametersNR-v1730 ::= SEQUENCE {

-- R1 30-4a: DM-RS bundling for PUSCH repetition type A (per BC)

dmrs-BundlingPUSCH-RepTypeAPerBC-r17                   ENUMERATED {supported}                         OPTIONAL,

-- R1 30-4b: DM-RS bundling for PUSCH repetition type B(per BC)

dmrs-BundlingPUSCH-RepTypeBPerBC-r17                   ENUMERATED {supported}                         OPTIONAL,

-- R1 30-4c: DM-RS bundling for TB processing over multi-slot PUSCH(per BC)

dmrs-BundlingPUSCH-multiSlotPerBC-r17                  ENUMERATED {supported}                         OPTIONAL,

-- R1 30-4d: DMRS bundling for PUCCH repetitions(per BC)

dmrs-BundlingPUCCH-RepPerBC-r17                        ENUMERATED {supported}                         OPTIONAL,

-- R1 30-4g: Restart DM-RS bundling (per BC)

dmrs-BundlingRestartPerBC-r17                          ENUMERATED {supported}                         OPTIONAL,

-- R1 30-4h: DM-RS bundling for non-back-to-back transmission (per BC)

dmrs-BundlingNonBackToBackTX-PerBC-r17                 ENUMERATED {supported}                         OPTIONAL,

-- R1 39-3-1: Stay on the target CC for SRS carrier switching

stayOnTargetCC-SRS-CarrierSwitch-r17                   ENUMERATED {supported}                         OPTIONAL,

-- R1 33-3-3a: FDM-ed Type-1 and Type-2 HARQ-ACK codebooks for multiplexing HARQ-ACK for unicast and HARQ-ACK for multicast

fdm-CodebookForMux-UnicastMulticastHARQ-ACK-r17        ENUMERATED {supported}                         OPTIONAL,

-- R1 33-3-3b: Mode 2 TDM-ed Type-1 and Type-2 HARQ-ACK codebook for multiplexing HARQ-ACK for unicast and HARQ-ACK for multicast

mode2-TDM-CodebookForMux-UnicastMulticastHARQ-ACK-r17  ENUMERATED {supported}                         OPTIONAL,

-- R1 33-3-4: Mode 1 for type1 codebook generation

mode1-ForType1-CodebookGeneration-r17                  ENUMERATED {supported}                         OPTIONAL,

-- R1 33-5-1j: NACK-only based HARQ-ACK feedback for multicast corresponding to a specific sequence or a PUCCH transmission

-- for SPS group-commmon PDSCH for multicast

nack-OnlyFeedbackSpecificResourceForSPS-Multicast-r17  ENUMERATED {supported}                         OPTIONAL,

-- R1 33-8-2: Up to 2 PUCCH resources configuration for multicast feedback for dynamically scheduled multicast

multiPUCCH-ConfigForMulticast-r17                      ENUMERATED {supported}                         OPTIONAL,

-- R1 33-8-3: PUCCH resource configuration for multicast feedback for SPS GC-PDSCH

pucch-ConfigForSPS-Multicast-r17                       ENUMERATED {supported}                         OPTIONAL,

-- The following parameter is associated with R1 33-2a, R1 33-3-3a, and R1 33-3-3b, and is not a RAN1 FG.

maxNumberG-RNTI-HARQ-ACK-Codebook-r17                  INTEGER (1..4)                                 OPTIONAL,

-- R1 33-3-5: Feedback multiplexing for unicast PDSCH and group-common PDSCH for multicast with same priority and different codebook

-- type

mux-HARQ-ACK-UnicastMulticast-r17                      ENUMERATED {supported}                         OPTIONAL

}

CA-ParametersNR-v1740 ::= SEQUENCE {

-- R1 33-5-1f: NACK-only based HARQ-ACK feedback for multicast RRC-based enabling/disabling NACK-only based feedback

-- for SPS group-common PDSCH for multicast

nack-OnlyFeedbackForSPS-Multicast-r17                  ENUMERATED {supported}                         OPTIONAL,

-- R1 33-8-1: PUCCH resource configuration for multicast feedback for dynamically scheduled multicast

singlePUCCH-ConfigForMulticast-r17                     ENUMERATED {supported}                         OPTIONAL

}

CrossCarrierSchedulingSCell-SpCell-r17 ::= SEQUENCE {

supportedSCS-Combinations-r17              SEQUENCE {

scs15kHz-15kHz-r17                         ENUMERATED {supported}                         OPTIONAL,

scs15kHz-30kHz-r17                         ENUMERATED {supported}                         OPTIONAL,

scs15kHz-60kHz-r17                         ENUMERATED {supported}                         OPTIONAL,

scs30kHz-30kHz-r17                         BIT STRING (SIZE (1..496))                     OPTIONAL,

scs30kHz-60kHz-r17                         BIT STRING (SIZE (1..496))                     OPTIONAL,

scs60kHz-60kHz-r17                         BIT STRING (SIZE (1..496))                     OPTIONAL

},

pdcch-MonitoringOccasion-r17               ENUMERATED {val1, val2}

}

PDCCH-BlindDetectionMixedList-r16::=       SEQUENCE {

pdcch-BlindDetectionCA-MixedExt-r16        CHOICE {

pdcch-BlindDetectionCA-Mixed-v16a0                PDCCH-BlindDetectionCA-MixedExt-r16,

pdcch-BlindDetectionCA-Mixed-NonAlignedSpan-v16a0 PDCCH-BlindDetectionCA-MixedExt-r16

}                                                                                             OPTIONAL,

pdcch-BlindDetectionCG-UE-MixedExt-r16     SEQUENCE{

pdcch-BlindDetectionMCG-UE-Mixed-v16a0                PDCCH-BlindDetectionCG-UE-MixedExt-r16,

pdcch-BlindDetectionSCG-UE-Mixed-v16a0            PDCCH-BlindDetectionCG-UE-MixedExt-r16

}                                                                                             OPTIONAL

}

PDCCH-BlindDetectionCA-MixedExt-r16 ::=    SEQUENCE {

pdcch-BlindDetectionCA1-r16                INTEGER (1..15),

pdcch-BlindDetectionCA2-r16                INTEGER (1..15)

}

PDCCH-BlindDetectionCG-UE-MixedExt-r16 ::= SEQUENCE {

pdcch-BlindDetectionCG-UE1-r16             INTEGER (0..15),

pdcch-BlindDetectionCG-UE2-r16             INTEGER (0..15)

}

PDCCH-BlindDetectionMCG-SCG-r17 ::=        SEQUENCE {

pdcch-BlindDetectionMCG-UE-r17             INTEGER (1..15),

pdcch-BlindDetectionSCG-UE-r17             INTEGER (1..15)

}

PDCCH-BlindDetectionMixed-r17::=           SEQUENCE {

pdcch-BlindDetectionCA-Mixed-r17           PDCCH-BlindDetectionCA-Mixed-r17                   OPTIONAL,

pdcch-BlindDetectionCG-UE-Mixed-r17        SEQUENCE{

pdcch-BlindDetectionMCG-UE-Mixed-v17       PDCCH-BlindDetectionCG-UE-Mixed-r17,

pdcch-BlindDetectionSCG-UE-Mixed-v17       PDCCH-BlindDetectionCG-UE-Mixed-r17

}                                                                                             OPTIONAL

}

PDCCH-BlindDetectionCG-UE-Mixed-r17 ::=    SEQUENCE {

pdcch-BlindDetectionCG-UE1-r17             INTEGER (0..15),

pdcch-BlindDetectionCG-UE2-r17             INTEGER (0..15)

}

PDCCH-BlindDetectionCA-Mixed-r17 ::=       SEQUENCE {

pdcch-BlindDetectionCA1-r17                INTEGER (1..15)                                    OPTIONAL,

pdcch-BlindDetectionCA2-r17                INTEGER (1..15)                                    OPTIONAL

}

PDCCH-BlindDetectionMixed1-r17::=          SEQUENCE {

pdcch-BlindDetectionCA-Mixed1-r17          PDCCH-BlindDetectionCA-Mixed1-r17                  OPTIONAL,

pdcch-BlindDetectionCG-UE-Mixed1-r17       SEQUENCE{

pdcch-BlindDetectionMCG-UE-Mixed1-v17      PDCCH-BlindDetectionCG-UE-Mixed1-r17,

pdcch-BlindDetectionSCG-UE-Mixed1-v17      PDCCH-BlindDetectionCG-UE-Mixed1-r17

}                                                                                             OPTIONAL

}

PDCCH-BlindDetectionCG-UE-Mixed1-r17 ::=   SEQUENCE {

pdcch-BlindDetectionCG-UE1-r17             INTEGER (0..15),

pdcch-BlindDetectionCG-UE2-r17             INTEGER (0..15),

pdcch-BlindDetectionCG-UE3-r17             INTEGER (0..15)

}

PDCCH-BlindDetectionCA-Mixed1-r17 ::=      SEQUENCE {

pdcch-BlindDetectionCA1-r17                INTEGER (1..15)                                    OPTIONAL,

pdcch-BlindDetectionCA2-r17                INTEGER (1..15)                                    OPTIONAL,

pdcch-BlindDetectionCA3-r17                INTEGER (1..15)                                    OPTIONAL

}

SimulSRS-ForAntennaSwitching-r16 ::= SEQUENCE {

supportSRS-xTyR-xLessThanY-r16       ENUMERATED {supported}                     OPTIONAL,

supportSRS-xTyR-xEqualToY-r16        ENUMERATED {supported}                     OPTIONAL,

supportSRS-AntennaSwitching-r16      ENUMERATED {supported}                     OPTIONAL

}

TwoPUCCH-Grp-Configurations-r16 ::=  SEQUENCE {

pucch-PrimaryGroupMapping-r16        TwoPUCCH-Grp-ConfigParams-r16,

pucch-SecondaryGroupMapping-r16      TwoPUCCH-Grp-ConfigParams-r16

}

TwoPUCCH-Grp-Configurations-r17 ::=  SEQUENCE {

primaryPUCCH-GroupConfig-r17         PUCCH-Group-Config-r17,

secondaryPUCCH-GroupConfig-r17       PUCCH-Group-Config-r17

}

TwoPUCCH-Grp-ConfigParams-r16 ::=    SEQUENCE {

pucch-GroupMapping-r16               PUCCH-Grp-CarrierTypes-r16,

pucch-TX-r16                         PUCCH-Grp-CarrierTypes-r16

}

CarrierTypePair-r16 ::=             SEQUENCE {

carrierForCSI-Measurement-r16       PUCCH-Grp-CarrierTypes-r16,

carrierForCSI-Reporting-r16         PUCCH-Grp-CarrierTypes-r16

}

PUCCH-Grp-CarrierTypes-r16 ::=       SEQUENCE {

fr1-NonSharedTDD-r16                 ENUMERATED {supported}                     OPTIONAL,

fr1-SharedTDD-r16                    ENUMERATED {supported}                     OPTIONAL,

fr1-NonSharedFDD-r16                 ENUMERATED {supported}                     OPTIONAL,

fr2-r16                              ENUMERATED {supported}                     OPTIONAL

}

PUCCH-Group-Config-r17 ::=           SEQUENCE {

fr1-FR1-NonSharedTDD-r17             ENUMERATED {supported}                     OPTIONAL,

fr2-FR2-NonSharedTDD-r17             ENUMERATED {supported}                     OPTIONAL,

fr1-FR2-NonSharedTDD-r17             ENUMERATED {supported}                     OPTIONAL

}

-- TAG-CA-PARAMETERSNR-STOP

-- ASN1STOP

CA-ParametersNR field description

codebookParametersPerBC For a given supported band combination, this field indicates the alternative list of SupportedCSI-RS-Resource supported for each codebook type, amongst the supported CSI-RS resources included in codebookParametersPerBand in MIMO-ParametersPerBand.

#### –	CA-ParametersNRDC

The IE CA-ParametersNRDC contains dual connectivity related capabilities that are defined per band combination.

CA-ParametersNRDC information element

-- ASN1START

-- TAG-CA-PARAMETERS-NRDC-START

CA-ParametersNRDC ::=                         SEQUENCE {

ca-ParametersNR-ForDC                       CA-ParametersNR                              OPTIONAL,

ca-ParametersNR-ForDC-v1540                 CA-ParametersNR-v1540                        OPTIONAL,

ca-ParametersNR-ForDC-v1550                 CA-ParametersNR-v1550                        OPTIONAL,

ca-ParametersNR-ForDC-v1560                 CA-ParametersNR-v1560                        OPTIONAL,

featureSetCombinationDC                     FeatureSetCombinationId                      OPTIONAL

}

CA-ParametersNRDC-v15g0 ::=                  SEQUENCE {

ca-ParametersNR-ForDC-v15g0                   CA-ParametersNR-v15g0                        OPTIONAL

}

CA-ParametersNRDC-v1610 ::= SEQUENCE {

-- R1 18-1: Semi-static power sharing mode1 between MCG and SCG cells of same FR for NR dual connectivity

intraFR-NR-DC-PwrSharingMode1-r16        ENUMERATED {supported}         OPTIONAL,

-- R1 18-1a: Semi-static power sharing mode 2 between MCG and SCG cells of same FR for NR dual connectivity

intraFR-NR-DC-PwrSharingMode2-r16        ENUMERATED {supported}         OPTIONAL,

-- R1 18-1b: Dynamic power sharing between MCG and SCG cells of same FR for NR dual connectivity

intraFR-NR-DC-DynamicPwrSharing-r16      ENUMERATED {short, long}       OPTIONAL,

asyncNRDC-r16                            ENUMERATED {supported}         OPTIONAL

}

CA-ParametersNRDC-v1630 ::=                         SEQUENCE {

ca-ParametersNR-ForDC-v1610                 CA-ParametersNR-v1610                        OPTIONAL,

ca-ParametersNR-ForDC-v1630                 CA-ParametersNR-v1630                        OPTIONAL

}

CA-ParametersNRDC-v1640 ::=                  SEQUENCE {

ca-ParametersNR-ForDC-v1640                  CA-ParametersNR-v1640                        OPTIONAL

}

CA-ParametersNRDC-v1650 ::=                  SEQUENCE {

supportedCellGrouping-r16                    BIT STRING (SIZE (1..maxCellGroupings-r16))  OPTIONAL

}

CA-ParametersNRDC-v16a0 ::=                  SEQUENCE {

ca-ParametersNR-ForDC-v16a0                  CA-ParametersNR-v16a0                        OPTIONAL

}

CA-ParametersNRDC-v1700 ::=                   SEQUENCE {

-- R1 31-9: Indicates the support of simultaneous transmission and reception of an IAB-node from multiple parent nodes

simultaneousRxTx-IAB-MultipleParents-r17     ENUMERATED {supported}                       OPTIONAL,

condPSCellAdditionNRDC-r17                   ENUMERATED {supported}                       OPTIONAL,

scg-ActivationDeactivationNRDC-r17           ENUMERATED {supported}                       OPTIONAL,

scg-ActivationDeactivationResumeNRDC-r17     ENUMERATED {supported}                       OPTIONAL,

beamManagementType-CBM-r17                   ENUMERATED {supported}                       OPTIONAL

}

CA-ParametersNRDC-v1720 ::=                  SEQUENCE {

ca-ParametersNR-ForDC-v1700                  CA-ParametersNR-v1700                        OPTIONAL,

ca-ParametersNR-ForDC-v1720                  CA-ParametersNR-v1720                        OPTIONAL

}

CA-ParametersNRDC-v1730 ::=                  SEQUENCE {

ca-ParametersNR-ForDC-v1730                   CA-ParametersNR-v1730                        OPTIONAL

}

-- TAG-CA-PARAMETERS-NRDC-STOP

-- ASN1STOP

CA-ParametersNRDC field descriptions

ca-ParametersNR-forDC (with and without suffix) If this field is present for a band combination, it reports the UE capabilities when NR-DC is configured with the band combination. If a version of this field (i.e., with or without suffix) is absent for a band combination, the corresponding ca-ParametersNR field version in BandCombination is applicable to the UE configured with NR-DC for the band combination. If a version of this field (i.e., with or without suffix) is present for a band combination but does not contain any parameters, the UE does not support the corresponding field version when configured with NR-DC for the band combination.

featureSetCombinationDC If this field is present for a band combination, it reports the feature set combination supported for the band combination when NR-DC is configured. If this field is absent for a band combination, the featureSetCombination in BandCombination (without suffix) is applicable to the UE configured with NR-DC for the band combination.

#### –	CarrierAggregationVariant

The IE CarrierAggregationVariant informs the network about supported "placement" of the SpCell in an NR cell group.

CarrierAggregationVariant information element

-- ASN1START

-- TAG-CARRIERAGGREGATIONVARIANT-START

CarrierAggregationVariant ::=          SEQUENCE {

fr1fdd-FR1TDD-CA-SpCellOnFR1FDD         ENUMERATED {supported}                      OPTIONAL,

fr1fdd-FR1TDD-CA-SpCellOnFR1TDD         ENUMERATED {supported}                      OPTIONAL,

fr1fdd-FR2TDD-CA-SpCellOnFR1FDD         ENUMERATED {supported}                      OPTIONAL,

fr1fdd-FR2TDD-CA-SpCellOnFR2TDD         ENUMERATED {supported}                      OPTIONAL,

fr1tdd-FR2TDD-CA-SpCellOnFR1TDD         ENUMERATED {supported}                      OPTIONAL,

fr1tdd-FR2TDD-CA-SpCellOnFR2TDD         ENUMERATED {supported}                      OPTIONAL,

fr1fdd-FR1TDD-FR2TDD-CA-SpCellOnFR1FDD  ENUMERATED {supported}                      OPTIONAL,

fr1fdd-FR1TDD-FR2TDD-CA-SpCellOnFR1TDD  ENUMERATED {supported}                      OPTIONAL,

fr1fdd-FR1TDD-FR2TDD-CA-SpCellOnFR2TDD  ENUMERATED {supported}                      OPTIONAL

}

-- TAG-CARRIERAGGREGATIONVARIANT-STOP

-- ASN1STOP

#### –	CodebookParameters

The IE CodebookParameters is used to convey codebook related parameters.

CodebookParameters information element

-- ASN1START

-- TAG-CODEBOOKPARAMETERS-START

CodebookParameters ::=             SEQUENCE {

type1                                  SEQUENCE {

singlePanel                           SEQUENCE {

supportedCSI-RS-ResourceList      SEQUENCE (SIZE (1.. maxNrofCSI-RS-Resources)) OF SupportedCSI-RS-Resource,

modes                                  ENUMERATED {mode1, mode1andMode2},

maxNumberCSI-RS-PerResourceSet    INTEGER (1..8)

},

multiPanel                            SEQUENCE {

supportedCSI-RS-ResourceList      SEQUENCE (SIZE (1.. maxNrofCSI-RS-Resources)) OF SupportedCSI-RS-Resource,

modes                                  ENUMERATED {mode1, mode2, both},

nrofPanels                            ENUMERATED {n2, n4},

maxNumberCSI-RS-PerResourceSet    INTEGER (1..8)

}                                                                                                               OPTIONAL

},

type2                                  SEQUENCE {

supportedCSI-RS-ResourceList        SEQUENCE (SIZE (1.. maxNrofCSI-RS-Resources)) OF SupportedCSI-RS-Resource,

parameterLx                           INTEGER (2..4),

amplitudeScalingType                 ENUMERATED {wideband, widebandAndSubband},

amplitudeSubsetRestriction          ENUMERATED {supported}              OPTIONAL

}                                                                                                                   OPTIONAL,

type2-PortSelection                  SEQUENCE {

supportedCSI-RS-ResourceList        SEQUENCE (SIZE (1.. maxNrofCSI-RS-Resources)) OF SupportedCSI-RS-Resource,

parameterLx                           INTEGER (2..4),

amplitudeScalingType                 ENUMERATED {wideband, widebandAndSubband}

}                                                                                                                   OPTIONAL

}

CodebookParameters-v1610 ::=        SEQUENCE {

supportedCSI-RS-ResourceListAlt-r16  SEQUENCE {

type1-SinglePanel-r16                SEQUENCE (SIZE (1..maxNrofCSI-RS-Resources)) OF INTEGER (0..maxNrofCSI-RS-ResourcesAlt-1-r16)  OPTIONAL,

type1-MultiPanel-r16                 SEQUENCE (SIZE (1..maxNrofCSI-RS-Resources)) OF INTEGER (0..maxNrofCSI-RS-ResourcesAlt-1-r16)  OPTIONAL,

type2-r16                            SEQUENCE (SIZE (1..maxNrofCSI-RS-Resources)) OF INTEGER (0..maxNrofCSI-RS-ResourcesAlt-1-r16)  OPTIONAL,

type2-PortSelection-r16              SEQUENCE (SIZE (1..maxNrofCSI-RS-Resources)) OF INTEGER (0..maxNrofCSI-RS-ResourcesAlt-1-r16)  OPTIONAL

}                                                                                                                                       OPTIONAL

}

CodebookParametersAddition-r16 ::=      SEQUENCE {

etype2-r16                             SEQUENCE {

-- R1 16-3a Regular eType 2 R=1

etype2R1-r16                           SEQUENCE {

supportedCSI-RS-ResourceListAdd-r16    SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesExt-r16)) OF

INTEGER (0..maxNrofCSI-RS-ResourcesAlt-1-r16)

},

-- R1 16-3a-1 Regular eType 2 R=2

etype2R2-r16                           SEQUENCE {

supportedCSI-RS-ResourceListAdd-r16    SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesExt-r16)) OF

INTEGER (0..maxNrofCSI-RS-ResourcesAlt-1-r16)

}                                                                  OPTIONAL,

-- R1 16-3a-2: Support of parameter combinations 7-8

paramComb7-8-r16                       ENUMERATED {supported}      OPTIONAL,

-- R1 16-3a-3: Support of rank 3,4

rank3-4-r16                            ENUMERATED {supported}      OPTIONAL,

-- R1 16-3a-4: CBSR with soft amplitude restriction

amplitudeSubsetRestriction-r16         ENUMERATED {supported}      OPTIONAL

}                                                                      OPTIONAL,

etype2-PS-r16                          SEQUENCE {

-- R1 16-3b Regular eType 2 R=1 PortSelection

etype2R1-PortSelection-r16             SEQUENCE {

supportedCSI-RS-ResourceListAdd-r16    SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesExt-r16)) OF

INTEGER (0..maxNrofCSI-RS-ResourcesAlt-1-r16)

},

-- R1 16-3b-1 Regular eType 2 R=2 PortSelection

etype2R2-PortSelection-r16             SEQUENCE {

supportedCSI-RS-ResourceListAdd-r16    SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesExt-r16)) OF

INTEGER (0..maxNrofCSI-RS-ResourcesAlt-1-r16)

}                                                                  OPTIONAL,

-- R1 16-3b-2: Support of rank 3,4

rank3-4-r16                            ENUMERATED {supported}      OPTIONAL

}                                                                      OPTIONAL

}

CodebookComboParametersAddition-r16 ::= SEQUENCE {

-- R1 16-8 Mixed codebook types

type1SP-Type2-null-r16                 SEQUENCE {

supportedCSI-RS-ResourceListAdd-r16    SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesExt-r16)) OF INTEGER (0..maxNrofCSI-RS-ResourcesAlt-1-r16)

}                                                          OPTIONAL,

type1SP-Type2PS-null-r16               SEQUENCE {

supportedCSI-RS-ResourceListAdd-r16    SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesExt-r16)) OF INTEGER (0..maxNrofCSI-RS-ResourcesAlt-1-r16)

}                                                          OPTIONAL,

type1SP-eType2R1-null-r16              SEQUENCE {

supportedCSI-RS-ResourceListAdd-r16    SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesExt-r16)) OF INTEGER (0..maxNrofCSI-RS-ResourcesAlt-1-r16)

}                                                          OPTIONAL,

type1SP-eType2R2-null-r16              SEQUENCE {

supportedCSI-RS-ResourceListAdd-r16    SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesExt-r16)) OF INTEGER (0..maxNrofCSI-RS-ResourcesAlt-1-r16)

}                                                          OPTIONAL,

type1SP-eType2R1PS-null-r16            SEQUENCE {

supportedCSI-RS-ResourceListAdd-r16    SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesExt-r16)) OF INTEGER (0..maxNrofCSI-RS-ResourcesAlt-1-r16)

}                                                          OPTIONAL,

type1SP-eType2R2PS-null-r16            SEQUENCE {

supportedCSI-RS-ResourceListAdd-r16    SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesExt-r16)) OF INTEGER (0..maxNrofCSI-RS-ResourcesAlt-1-r16)

}                                                          OPTIONAL,

type1SP-Type2-Type2PS-r16              SEQUENCE {

supportedCSI-RS-ResourceListAdd-r16    SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesExt-r16)) OF INTEGER (0..maxNrofCSI-RS-ResourcesAlt-1-r16)

}                                                          OPTIONAL,

type1MP-Type2-null-r16                 SEQUENCE {

supportedCSI-RS-ResourceListAdd-r16        SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesExt-r16)) OF INTEGER (0..maxNrofCSI-RS-ResourcesAlt-1-r16)

}                                                          OPTIONAL,

type1MP-Type2PS-null-r16               SEQUENCE {

supportedCSI-RS-ResourceListAdd-r16    SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesExt-r16)) OF INTEGER (0..maxNrofCSI-RS-ResourcesAlt-1-r16)

}                                                          OPTIONAL,

type1MP-eType2R1-null-r16              SEQUENCE {

supportedCSI-RS-ResourceListAdd-r16    SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesExt-r16)) OF INTEGER (0..maxNrofCSI-RS-ResourcesAlt-1-r16)

}                                                          OPTIONAL,

type1MP-eType2R2-null-r16              SEQUENCE {

supportedCSI-RS-ResourceListAdd-r16    SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesExt-r16)) OF INTEGER (0..maxNrofCSI-RS-ResourcesAlt-1-r16)

}                                                          OPTIONAL,

type1MP-eType2R1PS-null-r16            SEQUENCE {

supportedCSI-RS-ResourceListAdd-r16    SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesExt-r16)) OF INTEGER (0..maxNrofCSI-RS-ResourcesAlt-1-r16)

}                                                          OPTIONAL,

type1MP-eType2R2PS-null-r16            SEQUENCE {

supportedCSI-RS-ResourceListAdd-r16    SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesExt-r16)) OF INTEGER (0..maxNrofCSI-RS-ResourcesAlt-1-r16)

}                                                          OPTIONAL,

type1MP-Type2-Type2PS-r16              SEQUENCE {

supportedCSI-RS-ResourceListAdd-r16    SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesExt-r16)) OF INTEGER (0..maxNrofCSI-RS-ResourcesAlt-1-r16)

}                                                          OPTIONAL

}

CodebookParametersfetype2-r17 ::= SEQUENCE {

-- R1 23-9-1  Basic Features of Further Enhanced Port-Selection Type II Codebook (FeType-II)

fetype2basic-r17        SEQUENCE (SIZE (1.. maxNrofCSI-RS-ResourcesExt-r16)) OF INTEGER (0..maxNrofCSI-RS-ResourcesAlt-1-r16),

-- R1 23-9-2  Support of M=2 and R=1 for FeType-II

fetype2R1-r17           SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesExt-r17)) OF INTEGER (0.. maxNrofCSI-RS-ResourcesAlt-1-r16)

OPTIONAL,

-- R1 23-9-4  Support of R = 2 for FeType-II

fetype2R2-r17           SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesExt-r17)) OF INTEGER (0.. maxNrofCSI-RS-ResourcesAlt-1-r16)

OPTIONAL,

-- R1 23-9-3  Support of rank 3, 4 for FeType-II

fetype2Rank3Rank4-r17   ENUMERATED {supported}     OPTIONAL

}

CodebookComboParameterMixedType-r17 ::= SEQUENCE {

-- R1 23-9-5 Active CSI-RS resources and ports for mixed codebook types in any slot

type1SP-feType2PS-null-r17              SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesExt-r16)) OF INTEGER (0..maxNrofCSI-RS-ResourcesAlt-1-r16)

OPTIONAL,

type1SP-feType2PS-M2R1-null-r17         SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesExt-r16)) OF INTEGER (0..maxNrofCSI-RS-ResourcesAlt-1-r16)

OPTIONAL,

type1SP-feType2PS-M2R2-null-r1          SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesExt-r16)) OF INTEGER (0..maxNrofCSI-RS-ResourcesAlt-1-r16)

OPTIONAL,

type1SP-Type2-feType2-PS-M1-r17         SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesExt-r16)) OF INTEGER (0..maxNrofCSI-RS-ResourcesAlt-1-r16)

OPTIONAL,

type1SP-Type2-feType2-PS-M2R1-r17       SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesExt-r16)) OF INTEGER (0..maxNrofCSI-RS-ResourcesAlt-1-r16)

OPTIONAL,

type1SP-eType2R1-feType2-PS-M1-r17      SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesExt-r16)) OF INTEGER (0..maxNrofCSI-RS-ResourcesAlt-1-r16)

OPTIONAL,

type1SP-eType2R1-feType2-PS-M2R1-r17    SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesExt-r16)) OF INTEGER (0..maxNrofCSI-RS-ResourcesAlt-1-r16)

OPTIONAL,

type1MP-feType2PS-null-r17              SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesExt-r16)) OF INTEGER (0..maxNrofCSI-RS-ResourcesAlt-1-r16)

OPTIONAL,

type1MP-feType2PS-M2R1-null-r17         SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesExt-r16)) OF INTEGER (0..maxNrofCSI-RS-ResourcesAlt-1-r16)

OPTIONAL,

type1MP-feType2PS-M2R2-null-r17         SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesExt-r16)) OF INTEGER (0..maxNrofCSI-RS-ResourcesAlt-1-r16)

OPTIONAL,

type1MP-Type2-feType2-PS-M1-r17         SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesExt-r16)) OF INTEGER (0..maxNrofCSI-RS-ResourcesAlt-1-r16)

OPTIONAL,

type1MP-Type2-feType2-PS-M2R1-r17       SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesExt-r16)) OF INTEGER (0..maxNrofCSI-RS-ResourcesAlt-1-r16)

OPTIONAL,

type1MP-eType2R1-feType2-PS-M1-r17      SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesExt-r16)) OF INTEGER (0..maxNrofCSI-RS-ResourcesAlt-1-r16)

OPTIONAL,

type1MP-eType2R1-feType2-PS-M2R1-r17    SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesExt-r16)) OF INTEGER (0..maxNrofCSI-RS-ResourcesAlt-1-r16)

OPTIONAL

}

CodebookComboParameterMultiTRP-r17::= SEQUENCE {

-- R1 23-7-1b	Active CSI-RS resources and ports in the presence of multi-TRP CSI

--  {Codebook 2, Codebook 3} =(NULL, NULL}

nCJT-null-null                        SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesExt-r16)) OF INTEGER (0..maxNrofCSI-RS-ResourcesAlt-1-r16)

OPTIONAL,

nCJT1SP-null-null                     SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesExt-r16)) OF INTEGER (0..maxNrofCSI-RS-ResourcesAlt-1-r16)

OPTIONAL,

--    {Codebook 2, Codebook 3} = {( {"Rel 16 combinations in FG 16-8"}

nCJT-Type2-null-r16                   SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesExt-r16)) OF INTEGER (0..maxNrofCSI-RS-ResourcesAlt-1-r16)

OPTIONAL,

nCJT-Type2PS-null-r16                 SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesExt-r16)) OF INTEGER (0..maxNrofCSI-RS-ResourcesAlt-1-r16)

OPTIONAL,

nCJT-eType2R1-null-r16                SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesExt-r16)) OF INTEGER (0..maxNrofCSI-RS-ResourcesAlt-1-r16)

OPTIONAL,

nCJT-eType2R2-null-r16                SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesExt-r16)) OF INTEGER (0..maxNrofCSI-RS-ResourcesAlt-1-r16)

OPTIONAL,

nCJT-eType2R1PS-null-r16              SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesExt-r16)) OF INTEGER (0..maxNrofCSI-RS-ResourcesAlt-1-r16)

OPTIONAL,

nCJT-eType2R2PS-null-r16              SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesExt-r16)) OF INTEGER (0..maxNrofCSI-RS-ResourcesAlt-1-r16)

OPTIONAL,

nCJT-Type2-Type2PS-r16                SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesExt-r16)) OF INTEGER (0..maxNrofCSI-RS-ResourcesAlt-1-r16)

OPTIONAL,

nCJT1SP-Type2-null-r16                SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesExt-r16)) OF INTEGER (0..maxNrofCSI-RS-ResourcesAlt-1-r16)

OPTIONAL,

nCJT1SP-Type2PS-null-r16              SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesExt-r16)) OF INTEGER (0..maxNrofCSI-RS-ResourcesAlt-1-r16)

OPTIONAL,

nCJT1SP-eType2R1-null-r16             SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesExt-r16)) OF INTEGER (0..maxNrofCSI-RS-ResourcesAlt-1-r16)

OPTIONAL,

nCJT1SP-eType2R2-null-r16             SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesExt-r16)) OF INTEGER (0..maxNrofCSI-RS-ResourcesAlt-1-r16)

OPTIONAL,

nCJT1SP-eType2R1PS-null-r16           SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesExt-r16)) OF INTEGER (0..maxNrofCSI-RS-ResourcesAlt-1-r16)

OPTIONAL,

nCJT1SP-eType2R2PS-null-r16           SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesExt-r16)) OF INTEGER (0..maxNrofCSI-RS-ResourcesAlt-1-r16)

OPTIONAL,

nCJT1SP-Type2-Type2PS-r16             SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesExt-r16)) OF INTEGER (0..maxNrofCSI-RS-ResourcesAlt-1-r16)

OPTIONAL,

-- {Codebook 2, Codebook 3} = {"New Rel17 combinations in FG 23-9-5"}

nCJT-feType2PS-null-r17               SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesExt-r16)) OF INTEGER (0..maxNrofCSI-RS-ResourcesAlt-1-r16)

OPTIONAL,

nCJT-feType2PS-M2R1-null-r17          SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesExt-r16)) OF INTEGER (0..maxNrofCSI-RS-ResourcesAlt-1-r16)

OPTIONAL,

nCJT-feType2PS-M2R2-null-r17          SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesExt-r16)) OF INTEGER (0..maxNrofCSI-RS-ResourcesAlt-1-r16)

OPTIONAL,

nCJT-Type2-feType2-PS-M1-r17          SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesExt-r16)) OF INTEGER (0..maxNrofCSI-RS-ResourcesAlt-1-r16)

OPTIONAL,

nCJT-Type2-feType2-PS-M2R1-r17        SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesExt-r16)) OF INTEGER (0..maxNrofCSI-RS-ResourcesAlt-1-r16)

OPTIONAL,

nCJT-eType2R1-feType2-PS-M1-r17       SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesExt-r16)) OF INTEGER (0..maxNrofCSI-RS-ResourcesAlt-1-r16)

OPTIONAL,

nCJT-eType2R1-feType2-PS-M2R1-r17     SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesExt-r16)) OF INTEGER (0..maxNrofCSI-RS-ResourcesAlt-1-r16)

OPTIONAL,

nCJT1SP-feType2PS-null-r17            SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesExt-r16)) OF INTEGER (0..maxNrofCSI-RS-ResourcesAlt-1-r16)

OPTIONAL,

nCJT1SP-feType2PS-M2R1-null-r17       SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesExt-r16)) OF INTEGER (0..maxNrofCSI-RS-ResourcesAlt-1-r16)

OPTIONAL,

nCJT1SP-feType2PS-M2R2-null-r1        SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesExt-r16)) OF INTEGER (0..maxNrofCSI-RS-ResourcesAlt-1-r16)

OPTIONAL,

nCJT1SP-Type2-feType2-PS-M1-r17       SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesExt-r16)) OF INTEGER (0..maxNrofCSI-RS-ResourcesAlt-1-r16)

OPTIONAL,

nCJT1SP-Type2-feType2-PS-M2R1-r17     SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesExt-r16)) OF INTEGER (0..maxNrofCSI-RS-ResourcesAlt-1-r16)

OPTIONAL,

nCJT1SP-eType2R1-feType2-PS-M1-r17    SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesExt-r16)) OF INTEGER (0..maxNrofCSI-RS-ResourcesAlt-1-r16)

OPTIONAL,

nCJT1SP-eType2R1-feType2-PS-M2R1-r17  SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesExt-r16)) OF INTEGER (0..maxNrofCSI-RS-ResourcesAlt-1-r16)

OPTIONAL

}

CodebookParametersAdditionPerBC-r16::=  SEQUENCE {

-- R1 16-3a Regular eType 2 R=1

etype2R1-r16                    SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesExt-r16)) OF INTEGER (0..maxNrofCSI-RS-ResourcesAlt-1-r16)

OPTIONAL,

-- R1 16-3a-1 Regular eType 2 R=2

etype2R2-r16                    SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesExt-r16)) OF INTEGER (0..maxNrofCSI-RS-ResourcesAlt-1-r16)

OPTIONAL,

-- R1 16-3b Regular eType 2 R=1 PortSelection

etype2R1-PortSelection-r16      SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesExt-r16)) OF INTEGER (0..maxNrofCSI-RS-ResourcesAlt-1-r16)

OPTIONAL,

-- R1 16-3b-1 Regular eType 2 R=2 PortSelection

etype2R2-PortSelection-r16      SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesExt-r16)) OF INTEGER (0..maxNrofCSI-RS-ResourcesAlt-1-r16)

OPTIONAL

}

CodebookComboParametersAdditionPerBC-r16::= SEQUENCE {

-- R1 16-8 Mixed codebook types

type1SP-Type2-null-r16          SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesExt-r16)) OF INTEGER (0..maxNrofCSI-RS-ResourcesAlt-1-r16)

OPTIONAL,

type1SP-Type2PS-null-r16        SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesExt-r16)) OF INTEGER (0..maxNrofCSI-RS-ResourcesAlt-1-r16)

OPTIONAL,

type1SP-eType2R1-null-r16       SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesExt-r16)) OF INTEGER (0..maxNrofCSI-RS-ResourcesAlt-1-r16)

OPTIONAL,

type1SP-eType2R2-null-r16       SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesExt-r16)) OF INTEGER (0..maxNrofCSI-RS-ResourcesAlt-1-r16)

OPTIONAL,

type1SP-eType2R1PS-null-r16     SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesExt-r16)) OF INTEGER (0..maxNrofCSI-RS-ResourcesAlt-1-r16)

OPTIONAL,

type1SP-eType2R2PS-null-r16     SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesExt-r16)) OF INTEGER (0..maxNrofCSI-RS-ResourcesAlt-1-r16)

OPTIONAL,

type1SP-Type2-Type2PS-r16       SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesExt-r16)) OF INTEGER (0..maxNrofCSI-RS-ResourcesAlt-1-r16)

OPTIONAL,

type1MP-Type2-null-r16          SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesExt-r16)) OF INTEGER (0..maxNrofCSI-RS-ResourcesAlt-1-r16)

OPTIONAL,

type1MP-Type2PS-null-r16        SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesExt-r16)) OF INTEGER (0..maxNrofCSI-RS-ResourcesAlt-1-r16)

OPTIONAL,

type1MP-eType2R1-null-r16       SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesExt-r16)) OF INTEGER (0..maxNrofCSI-RS-ResourcesAlt-1-r16)

OPTIONAL,

type1MP-eType2R2-null-r16       SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesExt-r16)) OF INTEGER (0..maxNrofCSI-RS-ResourcesAlt-1-r16)

OPTIONAL,

type1MP-eType2R1PS-null-r16     SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesExt-r16)) OF INTEGER (0..maxNrofCSI-RS-ResourcesAlt-1-r16)

OPTIONAL,

type1MP-eType2R2PS-null-r16     SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesExt-r16)) OF INTEGER (0..maxNrofCSI-RS-ResourcesAlt-1-r16)

OPTIONAL,

type1MP-Type2-Type2PS-r16       SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesExt-r16)) OF INTEGER (0..maxNrofCSI-RS-ResourcesAlt-1-r16)

OPTIONAL

}

CodebookParametersfetype2PerBC-r17 ::= SEQUENCE {

-- R1 23-9-1	Basic Features of Further Enhanced Port-Selection Type II Codebook (FeType-II)

fetype2basic-r17    SEQUENCE (SIZE (1.. maxNrofCSI-RS-ResourcesExt-r16)) OF INTEGER (0..maxNrofCSI-RS-ResourcesAlt-1-r16),

-- R1 23-9-2	Support of M=2 and R=1 for FeType-II

fetype2R1-r17       SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesExt-r17)) OF INTEGER (0.. maxNrofCSI-RS-ResourcesAlt-1-r16)

OPTIONAL,

-- R1 23-9-4	Support of R = 2 for FeType-II

fetype2R2-r17       SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesExt-r17)) OF INTEGER (0.. maxNrofCSI-RS-ResourcesAlt-1-r16)

OPTIONAL

}

CodebookComboParameterMixedTypePerBC-r17 ::= SEQUENCE {

-- R1 23-9-5 Active CSI-RS resources and ports for mixed codebook types in any slot

type1SP-feType2PS-null-r17      SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesExt-r16)) OF INTEGER (0..maxNrofCSI-RS-ResourcesAlt-1-r16)

OPTIONAL,

type1SP-feType2PS-M2R1-null-r17 SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesExt-r16)) OF INTEGER (0..maxNrofCSI-RS-ResourcesAlt-1-r16)

OPTIONAL,

type1SP-feType2PS-M2R2-null-r17 SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesExt-r16)) OF INTEGER (0..maxNrofCSI-RS-ResourcesAlt-1-r16)

OPTIONAL,

type1SP-Type2-feType2-PS-M1-r17 SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesExt-r16)) OF INTEGER (0..maxNrofCSI-RS-ResourcesAlt-1-r16)

OPTIONAL,

type1SP-Type2-feType2-PS-M2R1-r17 SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesExt-r16)) OF INTEGER (0..maxNrofCSI-RS-ResourcesAlt-1-r16)

OPTIONAL,

type1SP-eType2R1-feType2-PS-M1-r17 SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesExt-r16)) OF INTEGER (0..maxNrofCSI-RS-ResourcesAlt-1-r16)

OPTIONAL,

type1SP-eType2R1-feType2-PS-M2R1-r17 SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesExt-r16)) OF INTEGER (0..maxNrofCSI-RS-ResourcesAlt-1-r16)

OPTIONAL,

type1MP-feType2PS-null-r17      SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesExt-r16)) OF INTEGER (0..maxNrofCSI-RS-ResourcesAlt-1-r16)

OPTIONAL,

type1MP-feType2PS-M2R1-null-r17 SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesExt-r16)) OF INTEGER (0..maxNrofCSI-RS-ResourcesAlt-1-r16)

OPTIONAL,

type1MP-feType2PS-M2R2-null-r17 SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesExt-r16)) OF INTEGER (0..maxNrofCSI-RS-ResourcesAlt-1-r16)

OPTIONAL,

type1MP-Type2-feType2-PS-M1-r17 SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesExt-r16)) OF INTEGER (0..maxNrofCSI-RS-ResourcesAlt-1-r16)

OPTIONAL,

type1MP-Type2-feType2-PS-M2R1-r17 SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesExt-r16)) OF INTEGER (0..maxNrofCSI-RS-ResourcesAlt-1-r16)

OPTIONAL,

type1MP-eType2R1-feType2-PS-M1-r17 SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesExt-r16)) OF INTEGER (0..maxNrofCSI-RS-ResourcesAlt-1-r16)

OPTIONAL,

type1MP-eType2R1-feType2-PS-M2R1-r17 SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesExt-r16)) OF INTEGER (0..maxNrofCSI-RS-ResourcesAlt-1-r16)

OPTIONAL

}

CodebookComboParameterMultiTRP-PerBC-r17::= SEQUENCE {

-- R1 23-7-1b	Active CSI-RS resources and ports in the presence of multi-TRP CSI

--  {Codebook 2, Codebook 3} =(NULL, NULL}

nCJT-null-null               SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesExt-r16)) OF INTEGER (0..maxNrofCSI-RS-ResourcesAlt-1-r16)

OPTIONAL,

nCJT1SP-null-null            SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesExt-r16)) OF INTEGER (0..maxNrofCSI-RS-ResourcesAlt-1-r16)

OPTIONAL,

--    {Codebook 2, Codebook 3} = {( {"Rel 16 combinations in FG 16-8"}

nCJT-Type2-null-r16          SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesExt-r16)) OF INTEGER (0..maxNrofCSI-RS-ResourcesAlt-1-r16)

OPTIONAL,

nCJT-Type2PS-null-r16        SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesExt-r16)) OF INTEGER (0..maxNrofCSI-RS-ResourcesAlt-1-r16)

OPTIONAL,

nCJT-eType2R1-null-r16       SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesExt-r16)) OF INTEGER (0..maxNrofCSI-RS-ResourcesAlt-1-r16)

OPTIONAL,

nCJT-eType2R2-null-r16       SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesExt-r16)) OF INTEGER (0..maxNrofCSI-RS-ResourcesAlt-1-r16)

OPTIONAL,

nCJT-eType2R1PS-null-r16     SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesExt-r16)) OF INTEGER (0..maxNrofCSI-RS-ResourcesAlt-1-r16)

OPTIONAL,

nCJT-eType2R2PS-null-r16     SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesExt-r16)) OF INTEGER (0..maxNrofCSI-RS-ResourcesAlt-1-r16)

OPTIONAL,

nCJT-Type2-Type2PS-r16       SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesExt-r16)) OF INTEGER (0..maxNrofCSI-RS-ResourcesAlt-1-r16)

OPTIONAL,

nCJT1SP-Type2-null-r16       SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesExt-r16)) OF INTEGER (0..maxNrofCSI-RS-ResourcesAlt-1-r16)

OPTIONAL,

nCJT1SP-Type2PS-null-r16     SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesExt-r16)) OF INTEGER (0..maxNrofCSI-RS-ResourcesAlt-1-r16)

OPTIONAL,

nCJT1SP-eType2R1-null-r16    SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesExt-r16)) OF INTEGER (0..maxNrofCSI-RS-ResourcesAlt-1-r16)

OPTIONAL,

nCJT1SP-eType2R2-null-r16    SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesExt-r16)) OF INTEGER (0..maxNrofCSI-RS-ResourcesAlt-1-r16)

OPTIONAL,

nCJT1SP-eType2R1PS-null-r16  SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesExt-r16)) OF INTEGER (0..maxNrofCSI-RS-ResourcesAlt-1-r16)

OPTIONAL,

nCJT1SP-eType2R2PS-null-r16  SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesExt-r16)) OF INTEGER (0..maxNrofCSI-RS-ResourcesAlt-1-r16)

OPTIONAL,

nCJT1SP-Type2-Type2PS-r16    SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesExt-r16)) OF INTEGER (0..maxNrofCSI-RS-ResourcesAlt-1-r16)

OPTIONAL,

-- {Codebook 2, Codebook 3} = {"New Rel17 combinations in FG 23-9-5"}

nCJT-feType2PS-null-r17         SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesExt-r16)) OF INTEGER (0..maxNrofCSI-RS-ResourcesAlt-1-r16)

OPTIONAL,

nCJT-feType2PS-M2R1-null-r17    SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesExt-r16)) OF INTEGER (0..maxNrofCSI-RS-ResourcesAlt-1-r16)

OPTIONAL,

nCJT-feType2PS-M2R2-null-r17    SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesExt-r16)) OF INTEGER (0..maxNrofCSI-RS-ResourcesAlt-1-r16)

OPTIONAL,

nCJT-Type2-feType2-PS-M1-r17    SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesExt-r16)) OF INTEGER (0..maxNrofCSI-RS-ResourcesAlt-1-r16)

OPTIONAL,

nCJT-Type2-feType2-PS-M2R1-r17  SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesExt-r16)) OF INTEGER (0..maxNrofCSI-RS-ResourcesAlt-1-r16)

OPTIONAL,

nCJT-eType2R1-feType2-PS-M1-r17 SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesExt-r16)) OF INTEGER (0..maxNrofCSI-RS-ResourcesAlt-1-r16)

OPTIONAL,

nCJT-eType2R1-feType2-PS-M2R1-r17 SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesExt-r16)) OF INTEGER (0..maxNrofCSI-RS-ResourcesAlt-1-r16)

OPTIONAL,

nCJT1SP-feType2PS-null-r17      SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesExt-r16)) OF INTEGER (0..maxNrofCSI-RS-ResourcesAlt-1-r16)

OPTIONAL,

nCJT1SP-feType2PS-M2R1-null-r17 SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesExt-r16)) OF INTEGER (0..maxNrofCSI-RS-ResourcesAlt-1-r16)

OPTIONAL,

nCJT1SP-feType2PS-M2R2-null-r1  SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesExt-r16)) OF INTEGER (0..maxNrofCSI-RS-ResourcesAlt-1-r16)

OPTIONAL,

nCJT1SP-Type2-feType2-PS-M1-r17 SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesExt-r16)) OF INTEGER (0..maxNrofCSI-RS-ResourcesAlt-1-r16)

OPTIONAL,

nCJT1SP-Type2-feType2-PS-M2R1-r17 SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesExt-r16)) OF INTEGER (0..maxNrofCSI-RS-ResourcesAlt-1-r16)

OPTIONAL,

nCJT1SP-eType2R1-feType2-PS-M1-r17 SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesExt-r16)) OF INTEGER (0..maxNrofCSI-RS-ResourcesAlt-1-r16)

OPTIONAL,

nCJT1SP-eType2R1-feType2-PS-M2R1-r17 SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesExt-r16)) OF INTEGER (0..maxNrofCSI-RS-ResourcesAlt-1-r16)

OPTIONAL

}

CodebookVariantsList-r16 ::= SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesAlt-r16)) OF SupportedCSI-RS-Resource

SupportedCSI-RS-Resource ::=     SEQUENCE {

maxNumberTxPortsPerResource      ENUMERATED {p2, p4, p8, p12, p16, p24, p32},

maxNumberResourcesPerBand        INTEGER (1..64),

totalNumberTxPortsPerBand        INTEGER (2..256)

}

-- TAG-CODEBOOKPARAMETERS-STOP

-- ASN1STOP

CodebookParameters field descriptions

supportedCSI-RS-ResourceListAlt This field indicates the alternative list of SupportedCSI-RS-Resource supported for each codebook type. The supported CSI-RS resource is indicated by an integer value which pinpoints SupportedCSI-RS-Resource defined in CodebookVariantsList. The value 0 corresponds to the first entry of CodebookVariantsList. The value 1 corresponds to the second entry of CodebookVariantsList, and so on. For each codebook type, the field shall be included in both codebookParametersPerBC (but optional for single CC) and codebookParametersPerBand.

#### –	FeatureSetCombination

The IE FeatureSetCombination is a two-dimensional matrix of FeatureSet entries.

Each FeatureSetsPerBand contains a list of feature sets applicable to the carrier(s) of one band entry of the associated band combination. Across the associated bands, the UE shall support the combination of FeatureSets at the same position in the FeatureSetsPerBand. All FeatureSetsPerBand in one FeatureSetCombination must have the same number of entries.

The number of FeatureSetsPerBand in the FeatureSetCombination must be equal to the number of band entries in an associated band combination. The first FeatureSetPerBand applies to the first band entry of the band combination, and so on.

Each FeatureSet contains either a pair of NR or E-UTRA feature set IDs for UL and DL.

In case of NR, the actual feature sets for UL and DL are defined in the FeatureSets IE and referred to from here by their ID, i.e., their position in the featureSetsUplink / featureSetsDownlink list in the FeatureSet IE.

In case of E-UTRA, the feature sets referred to from this list are defined in TS 36.331 [10] and conveyed as part of the UE-EUTRA-Capability container.

The FeatureSetUplink and FeatureSetDownlink referred to from the FeatureSet comprise, among other information, a set of FeatureSetUplinkPerCC-Ids and FeatureSetDownlinkPerCC-Ids. The number of these per-CC IDs determines the number of carriers that the UE is able to aggregate contiguously in frequency domain in the corresponding band. The number of carriers supported by the UE is also restricted by the bandwidth class indicated in the associated BandCombination, if present.

In feature set combinations the UE shall exclude entries with same or lower capabilities, since the network may anyway assume that the UE supports those.

NOTE 1:	The UE may advertise fallback band-combinations in which it supports additional functionality explicitly in two ways: Either by setting FeatureSet IDs to zero (inter-band and intra-band non-contiguous fallback) and by reducing the number of FeatureSet-PerCC Ids in a Feature Set (intra-band contiguous fallback). Or by separate BandCombination entries with associated FeatureSetCombinations.

NOTE 2:	The UE may advertise a FeatureSetCombination containing only fallback band combinations. That means, in a FeatureSetCombination, each group of FeatureSets across the bands may contain at least one pair of FeatureSetUplinkId and FeatureSetDownlinkId which is set to 0/0.

NOTE 3:	The Network configures serving cell(s) and BWP(s) configuration to comply with capabilities derived from the combination of FeatureSets at the same position in the FeatureSetsPerBand, regardless of activated/deactivated serving cell(s) and BWP(s).

FeatureSetCombination information element

-- ASN1START

-- TAG-FEATURESETCOMBINATION-START

FeatureSetCombination ::=       SEQUENCE (SIZE (1..maxSimultaneousBands)) OF FeatureSetsPerBand

FeatureSetsPerBand ::=          SEQUENCE (SIZE (1..maxFeatureSetsPerBand)) OF FeatureSet

FeatureSet ::=                  CHOICE {

eutra                           SEQUENCE {

downlinkSetEUTRA                FeatureSetEUTRA-DownlinkId,

uplinkSetEUTRA                  FeatureSetEUTRA-UplinkId

},

nr                              SEQUENCE {

downlinkSetNR                   FeatureSetDownlinkId,

uplinkSetNR                     FeatureSetUplinkId

}

}

-- TAG-FEATURESETCOMBINATION-STOP

-- ASN1STOP

#### –	FeatureSetCombinationId

The IE FeatureSetCombinationId identifies a FeatureSetCombination. The FeatureSetCombinationId of a FeatureSetCombination is the position of the FeatureSetCombination in the featureSetCombinations list (in UE-NR-Capability or UE-MRDC-Capability). The FeatureSetCombinationId = 0 refers to the first entry in the featureSetCombinations list (in UE-NR-Capability or UE-MRDC-Capability).

NOTE:	The FeatureSetCombinationId = 1024 is not used due to the maximum entry number of featureSetCombinations.

FeatureSetCombinationId information element

-- ASN1START

-- TAG-FEATURESETCOMBINATIONID-START

FeatureSetCombinationId ::=         INTEGER (0.. maxFeatureSetCombinations)

-- TAG-FEATURESETCOMBINATIONID-STOP

-- ASN1STOP

#### –	FeatureSetDownlink

The IE FeatureSetDownlink indicates a set of features that the UE supports on the carriers corresponding to one band entry in a band combination.

FeatureSetDownlink information element

-- ASN1START

-- TAG-FEATURESETDOWNLINK-START

FeatureSetDownlink ::=                  SEQUENCE {

featureSetListPerDownlinkCC             SEQUENCE (SIZE (1..maxNrofServingCells)) OF FeatureSetDownlinkPerCC-Id,

intraBandFreqSeparationDL               FreqSeparationClass                                                     OPTIONAL,

scalingFactor                           ENUMERATED {f0p4, f0p75, f0p8}                                          OPTIONAL,

dummy8                                  ENUMERATED {supported}                                                  OPTIONAL,

scellWithoutSSB                         ENUMERATED {supported}                                                  OPTIONAL,

csi-RS-MeasSCellWithoutSSB              ENUMERATED {supported}                                                  OPTIONAL,

dummy1                                  ENUMERATED {supported}                                                  OPTIONAL,

type1-3-CSS                             ENUMERATED {supported}                                                  OPTIONAL,

pdcch-MonitoringAnyOccasions            ENUMERATED {withoutDCI-Gap, withDCI-Gap}                                OPTIONAL,

dummy2                                  ENUMERATED {supported}                                                  OPTIONAL,

ue-SpecificUL-DL-Assignment             ENUMERATED {supported}                                                  OPTIONAL,

searchSpaceSharingCA-DL                 ENUMERATED {supported}                                                  OPTIONAL,

timeDurationForQCL                      SEQUENCE {

scs-60kHz                           ENUMERATED {s7, s14, s28}                                               OPTIONAL,

scs-120kHz                          ENUMERATED {s14, s28}                                                   OPTIONAL

}                                                                                                           OPTIONAL,

pdsch-ProcessingType1-DifferentTB-PerSlot SEQUENCE {

scs-15kHz                               ENUMERATED {upto2, upto4, upto7}                                    OPTIONAL,

scs-30kHz                               ENUMERATED {upto2, upto4, upto7}                                    OPTIONAL,

scs-60kHz                               ENUMERATED {upto2, upto4, upto7}                                    OPTIONAL,

scs-120kHz                              ENUMERATED {upto2, upto4, upto7}                                    OPTIONAL

}                                                                                                           OPTIONAL,

dummy3                                  DummyA                                                                  OPTIONAL,

dummy4                                  SEQUENCE (SIZE (1.. maxNrofCodebooks)) OF DummyB                        OPTIONAL,

dummy5                                  SEQUENCE (SIZE (1.. maxNrofCodebooks)) OF DummyC                        OPTIONAL,

dummy6                                  SEQUENCE (SIZE (1.. maxNrofCodebooks)) OF DummyD                        OPTIONAL,

dummy7                                  SEQUENCE (SIZE (1.. maxNrofCodebooks)) OF DummyE                        OPTIONAL

}

FeatureSetDownlink-v1540 ::= SEQUENCE {

oneFL-DMRS-TwoAdditionalDMRS-DL         ENUMERATED {supported}                       OPTIONAL,

additionalDMRS-DL-Alt                   ENUMERATED {supported}                       OPTIONAL,

twoFL-DMRS-TwoAdditionalDMRS-DL         ENUMERATED {supported}                       OPTIONAL,

oneFL-DMRS-ThreeAdditionalDMRS-DL       ENUMERATED {supported}                       OPTIONAL,

pdcch-MonitoringAnyOccasionsWithSpanGap SEQUENCE {

scs-15kHz                               ENUMERATED {set1, set2, set3}                OPTIONAL,

scs-30kHz                               ENUMERATED {set1, set2, set3}                OPTIONAL,

scs-60kHz                               ENUMERATED {set1, set2, set3}                OPTIONAL,

scs-120kHz                              ENUMERATED {set1, set2, set3}                OPTIONAL

}                                                                                    OPTIONAL,

pdsch-SeparationWithGap                 ENUMERATED {supported}                       OPTIONAL,

pdsch-ProcessingType2                   SEQUENCE {

scs-15kHz                               ProcessingParameters                         OPTIONAL,

scs-30kHz                               ProcessingParameters                         OPTIONAL,

scs-60kHz                               ProcessingParameters                         OPTIONAL

} OPTIONAL,

pdsch-ProcessingType2-Limited           SEQUENCE {

differentTB-PerSlot-SCS-30kHz           ENUMERATED {upto1, upto2, upto4, upto7}

} OPTIONAL,

dl-MCS-TableAlt-DynamicIndication       ENUMERATED {supported}                       OPTIONAL

}

FeatureSetDownlink-v15a0 ::= SEQUENCE {

supportedSRS-Resources              SRS-Resources                                    OPTIONAL

}

FeatureSetDownlink-v1610 ::=   SEQUENCE {

-- R1 22-4e/4f/4g/4h: CBG based reception for DL with unicast PDSCH(s) per slot per CC with UE processing time Capability 1

cbgPDSCH-ProcessingType1-DifferentTB-PerSlot-r16   SEQUENCE {

scs-15kHz-r16        ENUMERATED {one, upto2, upto4, upto7} OPTIONAL,

scs-30kHz-r16        ENUMERATED {one, upto2, upto4, upto7} OPTIONAL,

scs-60kHz-r16        ENUMERATED {one, upto2, upto4, upto7} OPTIONAL,

scs-120kHz-r16       ENUMERATED {one, upto2, upto4, upto7} OPTIONAL

} OPTIONAL,

-- R1 22-3e/3f/3g/3h: CBG based reception for DL with unicast PDSCH(s) per slot per CC with UE processing time Capability 2

cbgPDSCH-ProcessingType2-DifferentTB-PerSlot-r16   SEQUENCE {

scs-15kHz-r16        ENUMERATED {one, upto2, upto4, upto7} OPTIONAL,

scs-30kHz-r16        ENUMERATED {one, upto2, upto4, upto7} OPTIONAL,

scs-60kHz-r16        ENUMERATED {one, upto2, upto4, upto7} OPTIONAL,

scs-120kHz-r16       ENUMERATED {one, upto2, upto4, upto7} OPTIONAL

} OPTIONAL,

intraFreqDAPS-r16                  SEQUENCE {

intraFreqDiffSCS-DAPS-r16          ENUMERATED {supported}            OPTIONAL,

intraFreqAsyncDAPS-r16             ENUMERATED {supported}            OPTIONAL

}                                                                        OPTIONAL,

intraBandFreqSeparationDL-v1620    FreqSeparationClassDL-v1620           OPTIONAL,

intraBandFreqSeparationDL-Only-r16 FreqSeparationClassDL-Only-r16        OPTIONAL,

-- R1 11-2: Rel-16 PDCCH monitoring capability

pdcch-Monitoring-r16               SEQUENCE {

pdsch-ProcessingType1-r16          SEQUENCE {

scs-15kHz-r16                      PDCCH-MonitoringOccasions-r16 OPTIONAL,

scs-30kHz-r16                      PDCCH-MonitoringOccasions-r16 OPTIONAL

}                                                                    OPTIONAL,

pdsch-ProcessingType2-r16      SEQUENCE {

scs-15kHz-r16                  PDCCH-MonitoringOccasions-r16     OPTIONAL,

scs-30kHz-r16                  PDCCH-MonitoringOccasions-r16     OPTIONAL

}                                                                    OPTIONAL

}                                                                        OPTIONAL,

-- R1 11-2b: Mix of Rel. 16 PDCCH monitoring capability and Rel. 15 PDCCH monitoring capability on different carriers

pdcch-MonitoringMixed-r16          ENUMERATED {supported}                OPTIONAL,

-- R1 18-5c: Processing up to X unicast DCI scheduling for DL per scheduled CC

crossCarrierSchedulingProcessing-DiffSCS-r16  SEQUENCE {

scs-15kHz-120kHz-r16               ENUMERATED {n1,n2,n4}             OPTIONAL,

scs-15kHz-60kHz-r16                ENUMERATED {n1,n2,n4}             OPTIONAL,

scs-30kHz-120kHz-r16               ENUMERATED {n1,n2,n4}             OPTIONAL,

scs-15kHz-30kHz-r16                ENUMERATED {n2}                   OPTIONAL,

scs-30kHz-60kHz-r16                ENUMERATED {n2}                   OPTIONAL,

scs-60kHz-120kHz-r16               ENUMERATED {n2}                   OPTIONAL

}                                                                        OPTIONAL,

-- R1 16-2b-1: Support of single-DCI based SDM scheme

singleDCI-SDM-scheme-r16           ENUMERATED {supported}                OPTIONAL

}

FeatureSetDownlink-v1700 ::= SEQUENCE {

-- R1 36-2: Scaling factor to be applied to 1024QAM for FR1

scalingFactor-1024QAM-FR1-r17 ENUMERATED {f0p4, f0p75, f0p8}             OPTIONAL,

-- R1 24 feature for existing UE cap to include new SCS

timeDurationForQCL-v1710     SEQUENCE {

scs-480kHz                   ENUMERATED {s56, s112}                  OPTIONAL,

scs-960kHz                   ENUMERATED {s112, s224}                 OPTIONAL

}                                                                        OPTIONAL,

-- R1 23-6-1	SFN scheme A (scheme 1) for PDSCH and PDCCH

sfn-SchemeA-r17                  ENUMERATED {supported}                  OPTIONAL,

-- R1 23-6-1-1	SFN scheme A (scheme 1) for PDCCH only

sfn-SchemeA-PDCCH-only-r17       ENUMERATED {supported}                  OPTIONAL,

-- R1 23-6-1a	Dynamic switching - scheme A

sfn-SchemeA-DynamicSwitching-r17 ENUMERATED {supported}                  OPTIONAL,

-- R1 23-6-1b	SFN scheme A (scheme 1) for PDSCH only

sfn-SchemeA-PDSCH-only-r17       ENUMERATED {supported}                  OPTIONAL,

-- R1 23-6-2	SFN scheme B (TRP based pre-compensation) for PDSCH and PDCCH

sfn-SchemeB-r17                  ENUMERATED {supported}                  OPTIONAL,

-- R1 23-6-2a	Dynamic switching - scheme B

sfn-SchemeB-DynamicSwitching-r17 ENUMERATED {supported}                  OPTIONAL,

-- R1 23-6-2b	SFN scheme B (TRP based pre-compensation) for PDSCH only

sfn-SchemeB-PDSCH-only-r17       ENUMERATED {supported}                  OPTIONAL,

-- R1 23-2-1d	PDCCH repetition for Case 2 PDCCH monitoring with a span gap

mTRP-PDCCH-Case2-1SpanGap-r17    SEQUENCE {

scs-15kHz-r17                    PDCCH-RepetitionParameters-r17      OPTIONAL,

scs-30kHz-r17                    PDCCH-RepetitionParameters-r17      OPTIONAL,

scs-60kHz-r17                    PDCCH-RepetitionParameters-r17      OPTIONAL,

scs-120kHz-r17                   PDCCH-RepetitionParameters-r17      OPTIONAL

}                                                                        OPTIONAL,

-- R1 23-2-1e	PDCCH repetition for Rel-16 PDCCH monitoring

mTRP-PDCCH-legacyMonitoring-r17  SEQUENCE {

scs-15kHz-r17                    PDCCH-RepetitionParameters-r17      OPTIONAL,

scs-30kHz-r17                    PDCCH-RepetitionParameters-r17      OPTIONAL

}                                                                        OPTIONAL,

-- R1  23-2-4	Simultaneous configuration of PDCCH repetition and multi-DCI based multi-TRP

mTRP-PDCCH-multiDCI-multiTRP-r17 ENUMERATED {supported}                  OPTIONAL,

-- R1 33-2:	Dynamic scheduling for multicast for PCell

dynamicMulticastPCell-r17        ENUMERATED {supported}                  OPTIONAL,

-- R1 23-2-1	PDCCH repetition

mTRP-PDCCH-Repetition-r17        SEQUENCE {

numBD-twoPDCCH-r17               INTEGER (2..3),

maxNumOverlaps-r17               ENUMERATED {n1,n2,n3,n5,n10,n20,n40}

}                                                                        OPTIONAL

}

FeatureSetDownlink-v1720 ::=                SEQUENCE {

-- R1 25-19: RTT-based Propagation delay compensation based on CSI-RS for tracking and SRS

rtt-BasedPDC-CSI-RS-ForTracking-r17         ENUMERATED {supported}                                                   OPTIONAL,

-- R1 25-19a: RTT-based Propagation delay compensation based on DL PRS for RTT-based PDC and SRS

rtt-BasedPDC-PRS-r17                        SEQUENCE {

maxNumberPRS-Resource-r17                   ENUMERATED {n1, n2, n4, n8, n16, n32, n64},

maxNumberPRS-ResourceProcessedPerSlot-r17   SEQUENCE {

scs-15kHz-r17                               ENUMERATED {n1, n2, n4, n6, n8, n12, n16, n24, n32, n48, n64}    OPTIONAL,

scs-30kHz-r17                               ENUMERATED {n1, n2, n4, n6, n8, n12, n16, n24, n32, n48, n64}    OPTIONAL,

scs-60kHz-r17                               ENUMERATED {n1, n2, n4, n6, n8, n12, n16, n24, n32, n48, n64}    OPTIONAL,

scs-120kHz-r17                              ENUMERATED {n1, n2, n4, n6, n8, n12, n16, n24, n32, n48, n64}    OPTIONAL

}

}                                                                                                                    OPTIONAL,

-- R1 33-5-1: SPS group-common PDSCH for multicast on PCell

sps-Multicast-r17                           ENUMERATED {supported}                                                   OPTIONAL

}

FeatureSetDownlink-v1730 ::=                SEQUENCE {

-- R1 25-19b: Support of PRS as spatial relation RS for SRS

prs-AsSpatialRelationRS-For-SRS-r17         ENUMERATED {supported}                                                   OPTIONAL

}

PDCCH-MonitoringOccasions-r16 ::= SEQUENCE {

period7span3-r16                  ENUMERATED {supported}                 OPTIONAL,

period4span3-r16                  ENUMERATED {supported}                 OPTIONAL,

period2span2-r16                  ENUMERATED {supported}                 OPTIONAL

}

PDCCH-RepetitionParameters-r17 ::= SEQUENCE {

supportedMode-r17                  ENUMERATED {intra-span, inter-span, both},

limitX-PerCC-r17                   ENUMERATED {n4, n8, n16, n32, n44, n64, nolimit}                      OPTIONAL,

limitX-AcrossCC-r17                ENUMERATED {n4, n8, n16, n32, n44, n64, n128, n256, n512, nolimit}    OPTIONAL

}

DummyA ::=      SEQUENCE {

maxNumberNZP-CSI-RS-PerCC                   INTEGER (1..32),

maxNumberPortsAcrossNZP-CSI-RS-PerCC        ENUMERATED {p2, p4, p8, p12, p16, p24, p32, p40, p48, p56, p64, p72, p80,

p88, p96, p104, p112, p120, p128, p136, p144, p152, p160, p168,

p176, p184, p192, p200, p208, p216, p224, p232, p240, p248, p256},

maxNumberCS-IM-PerCC                        ENUMERATED {n1, n2, n4, n8, n16, n32},

maxNumberSimultaneousCSI-RS-ActBWP-AllCC    ENUMERATED {n5, n6, n7, n8, n9, n10, n12, n14, n16, n18, n20, n22, n24, n26,

n28, n30, n32, n34, n36, n38, n40, n42, n44, n46, n48, n50, n52,

n54, n56, n58, n60, n62, n64},

totalNumberPortsSimultaneousCSI-RS-ActBWP-AllCC ENUMERATED {p8, p12, p16, p24, p32, p40, p48, p56, p64, p72, p80,

p88, p96, p104, p112, p120, p128, p136, p144, p152, p160, p168,

p176, p184, p192, p200, p208, p216, p224, p232, p240, p248, p256}

}

DummyB ::=       SEQUENCE {

maxNumberTxPortsPerResource         ENUMERATED {p2, p4, p8, p12, p16, p24, p32},

maxNumberResources                  INTEGER (1..64),

totalNumberTxPorts                  INTEGER (2..256),

supportedCodebookMode               ENUMERATED {mode1, mode1AndMode2},

maxNumberCSI-RS-PerResourceSet      INTEGER (1..8)

}

DummyC ::=        SEQUENCE {

maxNumberTxPortsPerResource         ENUMERATED {p8, p16, p32},

maxNumberResources                  INTEGER (1..64),

totalNumberTxPorts                  INTEGER (2..256),

supportedCodebookMode               ENUMERATED {mode1, mode2, both},

supportedNumberPanels               ENUMERATED {n2, n4},

maxNumberCSI-RS-PerResourceSet      INTEGER (1..8)

}

DummyD ::=                 SEQUENCE {

maxNumberTxPortsPerResource         ENUMERATED {p4, p8, p12, p16, p24, p32},

maxNumberResources                  INTEGER (1..64),

totalNumberTxPorts                  INTEGER (2..256),

parameterLx                         INTEGER (2..4),

amplitudeScalingType                ENUMERATED {wideband, widebandAndSubband},

amplitudeSubsetRestriction          ENUMERATED {supported}                          OPTIONAL,

maxNumberCSI-RS-PerResourceSet      INTEGER (1..8)

}

DummyE ::=    SEQUENCE {

maxNumberTxPortsPerResource         ENUMERATED {p4, p8, p12, p16, p24, p32},

maxNumberResources                  INTEGER (1..64),

totalNumberTxPorts                  INTEGER (2..256),

parameterLx                         INTEGER (2..4),

amplitudeScalingType                ENUMERATED {wideband, widebandAndSubband},

maxNumberCSI-RS-PerResourceSet      INTEGER (1..8)

}

-- TAG-FEATURESETDOWNLINK-STOP

-- ASN1STOP

FeatureSetDownlink field descriptions

featureSetListPerDownlinkCC Indicates which features the UE supports on the individual DL carriers of the feature set (and hence of a band entry that refer to the feature set). The UE shall hence include at least as many FeatureSetDownlinkPerCC-Id in this list as the number of carriers it supports according to the ca-BandwidthClassDL, except if indicating additional functionality by reducing the number of FeatureSetDownlinkPerCC-Id in the feature set (see NOTE 1 in FeatureSetCombination IE description). The order of the elements in this list is not relevant, i.e., the network may configure any of the carriers in accordance with any of the FeatureSetDownlinkPerCC-Id in this list.

supportedSRS-Resources Indicates supported SRS resources for SRS carrier switching to the band associated with this FeatureSetDownlink. The UE is only allowed to set this field for a band with associated FeatureSetUplinkId set to 0.

#### –	FeatureSetDownlinkId

The IE FeatureSetDownlinkId identifies a downlink feature set. The FeatureSetDownlinkId of a FeatureSetDownlink is the index position of the FeatureSetDownlink in the featureSetsDownlink list in the FeatureSets IE. The first element in that list is referred to by FeatureSetDownlinkId = 1. The FeatureSetDownlinkId=0 is not used by an actual FeatureSetDownlink but means that the UE does not support a carrier in this band of a band combination.

FeatureSetDownlinkId information element

-- ASN1START

-- TAG-FEATURESETDOWNLINKID-START

FeatureSetDownlinkId ::=            INTEGER (0..maxDownlinkFeatureSets)

-- TAG-FEATURESETDOWNLINKID-STOP

-- ASN1STOP

#### –	FeatureSetDownlinkPerCC

The IE FeatureSetDownlinkPerCC indicates a set of features that the UE supports on the corresponding carrier of one band entry of a band combination.

FeatureSetDownlinkPerCC information element

-- ASN1START

-- TAG-FEATURESETDOWNLINKPERCC-START

FeatureSetDownlinkPerCC ::=         SEQUENCE {

supportedSubcarrierSpacingDL        SubcarrierSpacing,

supportedBandwidthDL                SupportedBandwidth,

channelBW-90mhz                     ENUMERATED {supported}                                                  OPTIONAL,

maxNumberMIMO-LayersPDSCH           MIMO-LayersDL                                                           OPTIONAL,

supportedModulationOrderDL          ModulationOrder                                                         OPTIONAL

}

FeatureSetDownlinkPerCC-v1620 ::=   SEQUENCE {

-- R1 16-2a: Mulit-DCI based multi-TRP

multiDCI-MultiTRP-r16               MultiDCI-MultiTRP-r16                                                   OPTIONAL,

-- R1 16-2b-3: Support of single-DCI based FDMSchemeB

supportFDM-SchemeB-r16              ENUMERATED {supported}                                                  OPTIONAL

}

FeatureSetDownlinkPerCC-v1700 ::=   SEQUENCE {

supportedMinBandwidthDL-r17             SupportedBandwidth-v1700                                                OPTIONAL,

broadcastSCell-r17                     ENUMERATED {supported}                                                  OPTIONAL,

-- R1 33-2g: MIMO layers for multicast PDSCH

maxNumberMIMO-LayersMulticastPDSCH-r17  ENUMERATED {n2, n4, n8}                                                 OPTIONAL,

-- R1 33-2h: Dynamic scheduling for multicast for SCell

dynamicMulticastSCell-r17               ENUMERATED {supported}                                                  OPTIONAL,

supportedBandwidthDL-v1710              SupportedBandwidth-v1700                                                OPTIONAL,

-- R4 24-1/24-2/24-3/24-4/24-5

supportedCRS-InterfMitigation-r17       CRS-InterfMitigation-r17                                                OPTIONAL

}

FeatureSetDownlinkPerCC-v1720 ::=   SEQUENCE {

-- R1 33-2j: Supported maximum modulation order used for maximum data rate calculation for multicast PDSCH

maxModulationOrderForMulticastDataRateCalculation-r17  ENUMERATED {qam64, qam256, qam1024}                  OPTIONAL,

-- R1 33-1-2: FDM-ed unicast PDSCH and group-common PDSCH for broadcast

fdm-BroadcastUnicast-r17            ENUMERATED {supported}                                                  OPTIONAL,

-- R1 33-3-2: FDM-ed unicast PDSCH and one group-common PDSCH for multicast

fdm-MulticastUnicast-r17            ENUMERATED {supported}                                                  OPTIONAL

}

FeatureSetDownlinkPerCC-v1730 ::=           SEQUENCE {

-- R1 33-3-3: Intra-slot TDM-ed unicast PDSCH and group-common PDSCH

intraSlotTDM-UnicastGroupCommonPDSCH-r17    ENUMERATED {yes, no}                    OPTIONAL,

-- R1 33-5-3: One SPS group-common PDSCH configuration for multicast for SCell

sps-MulticastSCell-r17                      ENUMERATED {supported}                  OPTIONAL,

-- R1 33-5-4: Up to 8 SPS group-common PDSCH configurations per CFR for multicast for SCell

sps-MulticastSCellMultiConfig-r17           INTEGER (1..8)                          OPTIONAL,

-- R1 33-1-1: Dynamic slot-level repetition for broadcast MTCH

dci-BroadcastWith16Repetitions-r17          ENUMERATED {supported}                  OPTIONAL

}

MultiDCI-MultiTRP-r16 ::=           SEQUENCE {

maxNumberCORESET-r16                ENUMERATED {n2, n3, n4, n5},

maxNumberCORESETPerPoolIndex-r16    INTEGER (1..3),

maxNumberUnicastPDSCH-PerPool-r16   ENUMERATED {n1, n2, n3, n4, n7}

}

CRS-InterfMitigation-r17 ::=        SEQUENCE {

-- R4 24-1 CRS-IM (Interference Mitigation) in DSS scenario

crs-IM-DSS-15kHzSCS-r17             ENUMERATED {supported}                                                  OPTIONAL,

-- R4 24-2 CRS-IM in non-DSS and 15 kHz NR SCS scenario, without the assistance of network signaling on LTE channel bandwidth

crs-IM-nonDSS-15kHzSCS-r17          ENUMERATED {supported}                                                  OPTIONAL,

-- R4 24-3 CRS-IM in non-DSS and 15 kHz NR SCS scenario, with the assistance of network signaling on LTE channel bandwidth

crs-IM-nonDSS-NWA-15kHzSCS-r17      ENUMERATED {supported}                                                  OPTIONAL,

-- R4 24-4 CRS-IM in non-DSS and 30 kHz NR SCS scenario, without the assistance of network signaling on LTE channel bandwidth

crs-IM-nonDSS-30kHzSCS-r17          ENUMERATED {supported}                                                  OPTIONAL,

-- R4 24-5 CRS-IM in non-DSS and 30 kHz NR SCS scenario, with the assistance of network signaling on LTE channel bandwidth

crs-IM-nonDSS-NWA-30kHzSCS-r17      ENUMERATED {supported}                                                  OPTIONAL

}

-- TAG-FEATURESETDOWNLINKPERCC-STOP

-- ASN1STOP

#### –	FeatureSetDownlinkPerCC-Id

The IE FeatureSetDownlinkPerCC-Id identifies a set of features applicable to one carrier of a feature set. The FeatureSetDownlinkPerCC-Id of a FeatureSetDownlinkPerCC is the index position of the FeatureSetDownlinkPerCC in the featureSetsDownlinkPerCC. The first element in the list is referred to by FeatureSetDownlinkPerCC-Id = 1, and so on.

FeatureSetDownlinkPerCC-Id information element

-- ASN1START

-- TAG-FEATURESETDOWNLINKPERCC-ID-START

FeatureSetDownlinkPerCC-Id ::=      INTEGER (1..maxPerCC-FeatureSets)

-- TAG-FEATURESETDOWNLINKPERCC-ID-STOP

-- ASN1STOP

#### –	FeatureSetEUTRA-DownlinkId

The IE FeatureSetEUTRA-DownlinkId identifies a downlink feature set in E-UTRA list (see TS 36.331 [10]. The first element in that list is referred to by FeatureSetEUTRA-DownlinkId = 1. The FeatureSetEUTRA-DownlinkId=0 is used when the UE does not support a carrier in this band of a band combination.

FeatureSetEUTRA-DownlinkId information element

-- ASN1START

-- TAG-FEATURESETEUTRADOWNLINKID-START

FeatureSetEUTRA-DownlinkId ::=      INTEGER (0..maxEUTRA-DL-FeatureSets)

-- TAG-FEATURESETEUTRADOWNLINKID-STOP

-- ASN1STOP

#### –	FeatureSetEUTRA-UplinkId

The IE FeatureSetEUTRA-UplinkId identifies an uplink feature set in E-UTRA list (see TS 36.331 [10]. The first element in that list is referred to by FeatureSetEUTRA-UplinkId = 1. The FeatureSetEUTRA-UplinkId =0 is used when the UE does not support a carrier in this band of a band combination.

FeatureSetEUTRA-UplinkId information element

-- ASN1START

-- TAG-FEATURESETEUTRAUPLINKID-START

FeatureSetEUTRA-UplinkId ::=                    INTEGER (0..maxEUTRA-UL-FeatureSets)

-- TAG-FEATURESETEUTRAUPLINKID-STOP

-- ASN1STOP

#### –	FeatureSets

The IE FeatureSets is used to provide pools of downlink and uplink features sets. A FeatureSetCombination refers to the IDs of the feature set(s) that the UE supports in that FeatureSetCombination. The BandCombination entries in the BandCombinationList then indicate the ID of the FeatureSetCombination that the UE supports for that band combination.

The entries in the lists in this IE are identified by their index position. For example, the FeatureSetUplinkPerCC-Id = 4 identifies the 4th element in the featureSetsUplinkPerCC list.

NOTE:	When feature sets (per CC) IEs require extension in future versions of the specification, new versions of the FeatureSetDownlink, FeatureSetUplink, FeatureSets, FeatureSetDownlinkPerCC and/or FeatureSetUplinkPerCC will be created and instantiated in corresponding new lists in the FeatureSets IE. For example, if new capability bits are to be added to the FeatureSetDownlink, they will instead be defined in a new FeatureSetDownlink-rxy which will be instantiated in a new featureSetDownlinkList-rxy list. If a UE indicates in a FeatureSetCombination that it supports the FeatureSetDownlink with ID #5, it implies that it supports both the features in FeatureSetDownlink #5 and FeatureSetDownlink-rxy #5 (if present). The number of entries in the new list(s) shall be the same as in the original list(s).

FeatureSets information element

-- ASN1START

-- TAG-FEATURESETS-START

FeatureSets ::=    SEQUENCE {

featureSetsDownlink                 SEQUENCE (SIZE (1..maxDownlinkFeatureSets)) OF FeatureSetDownlink               OPTIONAL,

featureSetsDownlinkPerCC            SEQUENCE (SIZE (1..maxPerCC-FeatureSets)) OF FeatureSetDownlinkPerCC            OPTIONAL,

featureSetsUplink                   SEQUENCE (SIZE (1..maxUplinkFeatureSets)) OF FeatureSetUplink                   OPTIONAL,

featureSetsUplinkPerCC              SEQUENCE (SIZE (1..maxPerCC-FeatureSets)) OF FeatureSetUplinkPerCC              OPTIONAL,

...,

[[

featureSetsDownlink-v1540           SEQUENCE (SIZE (1..maxDownlinkFeatureSets)) OF FeatureSetDownlink-v1540         OPTIONAL,

featureSetsUplink-v1540             SEQUENCE (SIZE (1..maxUplinkFeatureSets)) OF FeatureSetUplink-v1540             OPTIONAL,

featureSetsUplinkPerCC-v1540        SEQUENCE (SIZE (1..maxPerCC-FeatureSets)) OF FeatureSetUplinkPerCC-v1540        OPTIONAL

]],

[[

featureSetsDownlink-v15a0           SEQUENCE (SIZE (1..maxDownlinkFeatureSets)) OF FeatureSetDownlink-v15a0         OPTIONAL

]],

[[

featureSetsDownlink-v1610           SEQUENCE (SIZE (1..maxDownlinkFeatureSets)) OF FeatureSetDownlink-v1610         OPTIONAL,

featureSetsUplink-v1610             SEQUENCE (SIZE (1..maxUplinkFeatureSets)) OF FeatureSetUplink-v1610             OPTIONAL,

featureSetDownlinkPerCC-v1620       SEQUENCE (SIZE (1..maxPerCC-FeatureSets)) OF FeatureSetDownlinkPerCC-v1620      OPTIONAL

]],

[[

featureSetsUplink-v1630             SEQUENCE (SIZE (1..maxUplinkFeatureSets)) OF FeatureSetUplink-v1630             OPTIONAL

]],

[[

featureSetsUplink-v1640             SEQUENCE (SIZE (1..maxUplinkFeatureSets)) OF FeatureSetUplink-v1640             OPTIONAL

]],

[[

featureSetsDownlink-v1700           SEQUENCE (SIZE (1..maxDownlinkFeatureSets)) OF FeatureSetDownlink-v1700         OPTIONAL,

featureSetsDownlinkPerCC-v1700      SEQUENCE (SIZE (1..maxPerCC-FeatureSets)) OF FeatureSetDownlinkPerCC-v1700      OPTIONAL,

featureSetsUplink-v1710             SEQUENCE (SIZE (1..maxUplinkFeatureSets)) OF FeatureSetUplink-v1710             OPTIONAL,

featureSetsUplinkPerCC-v1700        SEQUENCE (SIZE (1..maxPerCC-FeatureSets)) OF FeatureSetUplinkPerCC-v1700        OPTIONAL

]],

[[

featureSetsDownlink-v1720           SEQUENCE (SIZE (1..maxDownlinkFeatureSets)) OF FeatureSetDownlink-v1720         OPTIONAL,

featureSetsDownlinkPerCC-v1720      SEQUENCE (SIZE (1..maxPerCC-FeatureSets)) OF FeatureSetDownlinkPerCC-v1720      OPTIONAL,

featureSetsUplink-v1720             SEQUENCE (SIZE (1..maxUplinkFeatureSets)) OF FeatureSetUplink-v1720             OPTIONAL

]],

[[

featureSetsDownlink-v1730           SEQUENCE (SIZE (1..maxDownlinkFeatureSets)) OF FeatureSetDownlink-v1730         OPTIONAL,

featureSetsDownlinkPerCC-v1730      SEQUENCE (SIZE (1..maxPerCC-FeatureSets)) OF FeatureSetDownlinkPerCC-v1730      OPTIONAL

]]

}

-- TAG-FEATURESETS-STOP

-- ASN1STOP

#### –	FeatureSetUplink

The IE FeatureSetUplink is used to indicate the features that the UE supports on the carriers corresponding to one band entry in a band combination.

FeatureSetUplink information element

-- ASN1START

-- TAG-FEATURESETUPLINK-START

FeatureSetUplink ::=                SEQUENCE {

featureSetListPerUplinkCC           SEQUENCE (SIZE (1.. maxNrofServingCells)) OF FeatureSetUplinkPerCC-Id,

scalingFactor                       ENUMERATED {f0p4, f0p75, f0p8}                                          OPTIONAL,

dummy3                              ENUMERATED {supported}                                                  OPTIONAL,

intraBandFreqSeparationUL           FreqSeparationClass                                                     OPTIONAL,

searchSpaceSharingCA-UL             ENUMERATED {supported}                                                  OPTIONAL,

dummy1                              DummyI                                                                  OPTIONAL,

supportedSRS-Resources              SRS-Resources                                                           OPTIONAL,

twoPUCCH-Group                      ENUMERATED {supported}                                                  OPTIONAL,

dynamicSwitchSUL                    ENUMERATED {supported}                                                  OPTIONAL,

simultaneousTxSUL-NonSUL            ENUMERATED {supported}                                                  OPTIONAL,

pusch-ProcessingType1-DifferentTB-PerSlot SEQUENCE {

scs-15kHz                                 ENUMERATED {upto2, upto4, upto7}                                  OPTIONAL,

scs-30kHz                                 ENUMERATED {upto2, upto4, upto7}                                  OPTIONAL,

scs-60kHz                                 ENUMERATED {upto2, upto4, upto7}                                  OPTIONAL,

scs-120kHz                                ENUMERATED {upto2, upto4, upto7}                                  OPTIONAL

}                                                                                                           OPTIONAL,

dummy2                               DummyF                                                                 OPTIONAL

}

FeatureSetUplink-v1540 ::=           SEQUENCE {

zeroSlotOffsetAperiodicSRS           ENUMERATED {supported}                     OPTIONAL,

pa-PhaseDiscontinuityImpacts         ENUMERATED {supported}                     OPTIONAL,

pusch-SeparationWithGap              ENUMERATED {supported}                     OPTIONAL,

pusch-ProcessingType2                SEQUENCE {

scs-15kHz                            ProcessingParameters                       OPTIONAL,

scs-30kHz                            ProcessingParameters                       OPTIONAL,

scs-60kHz                            ProcessingParameters                       OPTIONAL

}                                                                               OPTIONAL,

ul-MCS-TableAlt-DynamicIndication    ENUMERATED {supported}                     OPTIONAL

}

FeatureSetUplink-v1610 ::=       SEQUENCE {

-- R1 11-5: PUsCH repetition Type B

pusch-RepetitionTypeB-r16        SEQUENCE {

maxNumberPUSCH-Tx-r16            ENUMERATED {n2, n3, n4, n7, n8, n12},

hoppingScheme-r16                ENUMERATED {interSlotHopping, interRepetitionHopping, both}

}                                                                              OPTIONAL,

-- R1 11-7: UL cancelation scheme for self-carrier

ul-CancellationSelfCarrier-r16       ENUMERATED {supported}                    OPTIONAL,

-- R1 11-7a: UL cancelation scheme for cross-carrier

ul-CancellationCrossCarrier-r16      ENUMERATED {supported}                    OPTIONAL,

-- R1 16-5c: The maximum number of SRS resources in one SRS resource set with usage set to 'codebook' for Mode 2

ul-FullPwrMode2-MaxSRS-ResInSet-r16  ENUMERATED {n1, n2, n4}                   OPTIONAL,

-- R1 22-4a/4b/4c/4d: CBG based transmission for UL with unicast PUSCH(s) per slot per CC with UE processing time Capability 1

cbgPUSCH-ProcessingType1-DifferentTB-PerSlot-r16    SEQUENCE {

scs-15kHz-r16        ENUMERATED {one-pusch, upto2, upto4, upto7}               OPTIONAL,

scs-30kHz-r16        ENUMERATED {one-pusch, upto2, upto4, upto7}               OPTIONAL,

scs-60kHz-r16        ENUMERATED {one-pusch, upto2, upto4, upto7}               OPTIONAL,

scs-120kHz-r16       ENUMERATED {one-pusch, upto2, upto4, upto7}               OPTIONAL

} OPTIONAL,

-- R1 22-3a/3b/3c/3d: CBG based transmission for UL with unicast PUSCH(s) per slot per CC with UE processing time Capability 2

cbgPUSCH-ProcessingType2-DifferentTB-PerSlot-r16    SEQUENCE {

scs-15kHz-r16        ENUMERATED {one-pusch, upto2, upto4, upto7}               OPTIONAL,

scs-30kHz-r16        ENUMERATED {one-pusch, upto2, upto4, upto7}               OPTIONAL,

scs-60kHz-r16        ENUMERATED {one-pusch, upto2, upto4, upto7}               OPTIONAL,

scs-120kHz-r16       ENUMERATED {one-pusch, upto2, upto4, upto7}               OPTIONAL

} OPTIONAL,

supportedSRS-PosResources-r16              SRS-AllPosResources-r16             OPTIONAL,

intraFreqDAPS-UL-r16                             SEQUENCE {

dummy                                            ENUMERATED {supported}    OPTIONAL,

intraFreqTwoTAGs-DAPS-r16                        ENUMERATED {supported}    OPTIONAL,

dummy1                                           ENUMERATED {supported}    OPTIONAL,

dummy2                                           ENUMERATED {supported}    OPTIONAL,

dummy3                                           ENUMERATED {short, long}  OPTIONAL

}                                                                              OPTIONAL,

intraBandFreqSeparationUL-v1620                  FreqSeparationClassUL-v1620   OPTIONAL,

-- R1 11-3: More than one PUCCH for HARQ-ACK transmission within a slot

multiPUCCH-r16                        SEQUENCE {

sub-SlotConfig-NCP-r16                ENUMERATED {set1, set2}              OPTIONAL,

sub-SlotConfig-ECP-r16                ENUMERATED {set1, set2}              OPTIONAL

}                                                                              OPTIONAL,

-- R1 11-3c: 2 PUCCH of format 0 or 2 for a single 7*2-symbol subslot based HARQ-ACK codebook

twoPUCCH-Type1-r16                    ENUMERATED {supported}                   OPTIONAL,

-- R1 11-3d: 2 PUCCH of format 0 or 2 for a single 2*7-symbol subslot based HARQ-ACK codebook

twoPUCCH-Type2-r16                    ENUMERATED {supported}                   OPTIONAL,

-- R1 11-3e: 1 PUCCH format 0 or 2 and 1 PUCCH format 1, 3 or 4 in the same subslot for a single 2*7-symbol HARQ-ACK codebooks

twoPUCCH-Type3-r16                    ENUMERATED {supported}                   OPTIONAL,

-- R1 11-3f: 2 PUCCH transmissions in the same subslot for a single 2*7-symbol HARQ-ACK codebooks which are not covered by 11-3d and

-- 11-3e

twoPUCCH-Type4-r16                    ENUMERATED {supported}                   OPTIONAL,

-- R1 11-3g: SR/HARQ-ACK multiplexing once per subslot using a PUCCH (or HARQ-ACK piggybacked on a PUSCH) when SR/HARQ-ACK

-- are supposed to be sent with different starting symbols in a subslot

mux-SR-HARQ-ACK-r16                   ENUMERATED {supported}                   OPTIONAL,

dummy1                                ENUMERATED {supported}                   OPTIONAL,

dummy2                                ENUMERATED {supported}                   OPTIONAL,

-- R1 11-4c: 2 PUCCH of format 0 or 2 for two HARQ-ACK codebooks with one 7*2-symbol sub-slot based HARQ-ACK codebook

twoPUCCH-Type5-r16                    ENUMERATED {supported}                   OPTIONAL,

-- R1 11-4d: 2 PUCCH of format 0 or 2 in consecutive symbols for two HARQ-ACK codebooks with one 2*7-symbol sub-slot based HARQ-ACK

-- codebook

twoPUCCH-Type6-r16                    ENUMERATED {supported}                   OPTIONAL,

-- R1 11-4e: 2 PUCCH of format 0 or 2 for two subslot based HARQ-ACK codebooks

twoPUCCH-Type7-r16                    ENUMERATED {supported}                   OPTIONAL,

-- R1 11-4f: 1 PUCCH format 0 or 2 and 1 PUCCH format 1, 3 or 4 in the same subslot for HARQ-ACK codebooks with one 2*7-symbol

-- subslot based HARQ-ACK codebook

twoPUCCH-Type8-r16                    ENUMERATED {supported}                   OPTIONAL,

-- R1 11-4g: 1 PUCCH format 0 or 2 and 1 PUCCH format 1, 3 or 4 in the same subslot for two subslot based HARQ-ACK codebooks

twoPUCCH-Type9-r16                    ENUMERATED {supported}                   OPTIONAL,

-- R1 11-4h: 2 PUCCH transmissions in the same subslot for two HARQ-ACK codebooks with one 2*7-symbol subslot which are not covered

-- by 11-4c and 11-4e

twoPUCCH-Type10-r16                   ENUMERATED {supported}                   OPTIONAL,

-- R1 11-4i: 2 PUCCH transmissions in the same subslot for two subslot based HARQ-ACK codebooks which are not covered by 11-4d and

-- 11-4f

twoPUCCH-Type11-r16                   ENUMERATED {supported}                   OPTIONAL,

-- R1 12-1: UL intra-UE multiplexing/prioritization of overlapping channel/signals with two priority levels in physical layer

ul-IntraUE-Mux-r16                    SEQUENCE {

pusch-PreparationLowPriority-r16      ENUMERATED {sym0, sym1, sym2},

pusch-PreparationHighPriority-r16     ENUMERATED {sym0, sym1, sym2}

}                                                                              OPTIONAL,

-- R1 16-5a: Supported UL full power transmission mode of fullpower

ul-FullPwrMode-r16                    ENUMERATED {supported}                   OPTIONAL,

-- R1 18-5d: Processing up to X unicast DCI scheduling for UL per scheduled CC

crossCarrierSchedulingProcessing-DiffSCS-r16    SEQUENCE {

scs-15kHz-120kHz-r16                  ENUMERATED {n1,n2,n4}                OPTIONAL,

scs-15kHz-60kHz-r16                   ENUMERATED {n1,n2,n4}                OPTIONAL,

scs-30kHz-120kHz-r16                  ENUMERATED {n1,n2,n4}                OPTIONAL,

scs-15kHz-30kHz-r16                   ENUMERATED {n2}                      OPTIONAL,

scs-30kHz-60kHz-r16                   ENUMERATED {n2}                      OPTIONAL,

scs-60kHz-120kHz-r16                  ENUMERATED {n2}                      OPTIONAL

}                                                                              OPTIONAL,

-- R1 16-5b: Supported UL full power transmission mode of fullpowerMode1

ul-FullPwrMode1-r16                   ENUMERATED {supported}                   OPTIONAL,

-- R1 16-5c-2: Ports configuration for Mode 2

ul-FullPwrMode2-SRSConfig-diffNumSRSPorts-r16  ENUMERATED {p1-2, p1-4, p1-2-4} OPTIONAL,

-- R1 16-5c-3: TPMI group for Mode 2

ul-FullPwrMode2-TPMIGroup-r16         SEQUENCE {

twoPorts-r16                          BIT STRING(SIZE(2))                      OPTIONAL,

fourPortsNonCoherent-r16              ENUMERATED{g0, g1, g2, g3}               OPTIONAL,

fourPortsPartialCoherent-r16          ENUMERATED{g0, g1, g2, g3, g4, g5, g6}   OPTIONAL

}                                                                                  OPTIONAL

}

FeatureSetUplink-v1630 ::=       SEQUENCE {

-- R1 22-8: For SRS for CB PUSCH and antenna switching on FR1 with symbol level offset for aperiodic SRS transmission

offsetSRS-CB-PUSCH-Ant-Switch-fr1-r16                       ENUMERATED {supported}                   OPTIONAL,

-- R1 22-8a: PDCCH monitoring on any span of up to 3 consecutive OFDM symbols of a slot and constrained timeline for SRS for CB

-- PUSCH and antenna switching on FR1

offsetSRS-CB-PUSCH-PDCCH-MonitorSingleOcc-fr1-r16           ENUMERATED {supported}                   OPTIONAL,

-- R1 22-8b: For type 1 CSS with dedicated RRC configuration, type 3 CSS, and UE-SS, monitoring occasion can be any OFDM symbol(s)

-- of a slot for Case 2 and constrained timeline for SRS for CB PUSCH and antenna switching on FR1

offsetSRS-CB-PUSCH-PDCCH-MonitorAnyOccWithoutGap-fr1-r16    ENUMERATED {supported}                   OPTIONAL,

-- R1 22-8c: For type 1 CSS with dedicated RRC configuration, type 3 CSS, and UE-SS, monitoring occasion can be any OFDM symbol(s)

-- of a slot for Case 2 with a DCI gap and constrained timeline for SRS for CB PUSCH and antenna switching on FR1

offsetSRS-CB-PUSCH-PDCCH-MonitorAnyOccWithGap-fr1-r16       ENUMERATED {supported}                   OPTIONAL,

dummy                                                       ENUMERATED {supported}                   OPTIONAL,

-- R1 22-9: Cancellation of PUCCH, PUSCH or PRACH with a DCI scheduling a PDSCH or CSI-RS or a DCI format 2_0 for SFI

partialCancellationPUCCH-PUSCH-PRACH-TX-r16                 ENUMERATED {supported}                   OPTIONAL

}

FeatureSetUplink-v1640 ::=              SEQUENCE {

-- R1 11-4: Two HARQ-ACK codebooks with up to one sub-slot based HARQ-ACK codebook (i.e. slot-based + slot-based, or slot-based +

-- sub-slot based) simultaneously constructed for supporting HARQ-ACK codebooks with different priorities at a UE

twoHARQ-ACK-Codebook-type1-r16          SubSlot-Config-r16      OPTIONAL,

-- R1 11-4a: Two sub-slot based HARQ-ACK codebooks simultaneously constructed for supporting HARQ-ACK codebooks with different

-- priorities at a UE

twoHARQ-ACK-Codebook-type2-r16          SubSlot-Config-r16      OPTIONAL,

-- R1 22-8d: All PDCCH monitoring occasion can be any OFDM symbol(s) of a slot for Case 2 with a span gap and constrained timeline

-- for SRS for CB PUSCH and antenna switching on FR1

offsetSRS-CB-PUSCH-PDCCH-MonitorAnyOccWithSpanGap-fr1-r16 SEQUENCE {

scs-15kHz-r16                                 ENUMERATED {set1, set2, set3}                             OPTIONAL,

scs-30kHz-r16                                 ENUMERATED {set1, set2, set3}                             OPTIONAL,

scs-60kHz-r16                                 ENUMERATED {set1, set2, set3}                             OPTIONAL

}                                                                                                           OPTIONAL

}

FeatureSetUplink-v1710 ::= SEQUENCE {

-- R1 23-3-1	Multi-TRP PUSCH repetition (type A) -codebook based

mTRP-PUSCH-TypeA-CB-r17                ENUMERATED {n1,n2,n4}                              OPTIONAL,

-- R1 23-3-1-2	Multi-TRP PUSCH repetition (type A) - non-codebook based

mTRP-PUSCH-RepetitionTypeA-r17         ENUMERATED {n1,n2,n3,n4}                           OPTIONAL,

-- R1 23-3-3	Multi-TRP PUCCH repetition-intra-slot

mTRP-PUCCH-IntraSlot-r17               ENUMERATED {pf0-2, pf1-3-4, pf0-4}                 OPTIONAL,

-- R1 23-8-4	Maximum 2 SP and 1 periodic SRS sets for antenna switching

srs-AntennaSwitching2SP-1Periodic-r17  ENUMERATED {supported}                             OPTIONAL,

-- R1 23-8-9	Extension of aperiodic SRS configuration for 1T4R, 1T2R and 2T4R

srs-ExtensionAperiodicSRS-r17          ENUMERATED {supported}                             OPTIONAL,

-- R1 23-8-10	1 aperiodic SRS resource set for 1T4R

srs-OneAP-SRS-r17                      ENUMERATED {supported}                             OPTIONAL,

-- R4 16-8 UE power class per band per band combination

ue-PowerClassPerBandPerBC-r17          ENUMERATED {pc1dot5, pc2, pc3}                     OPTIONAL,

-- R4 17-8 UL transmission in FR2 bands within an UL gap when the UL gap is activated

tx-Support-UL-GapFR2-r17               ENUMERATED {supported}                             OPTIONAL

}

FeatureSetUplink-v1720 ::= SEQUENCE {

-- R1 25-3: Repetitions for PUCCH format 0, 1, 2, 3 and 4 over multiple PUCCH subslots with configured K = 2, 4, 8

pucch-Repetition-F0-1-2-3-4-RRC-Config-r17         ENUMERATED {supported}                 OPTIONAL,

-- R1 25-3a: Repetitions for PUCCH format 0, 1, 2, 3 and 4 over multiple PUCCH subslots using dynamic repetition indication

pucch-Repetition-F0-1-2-3-4-DynamicIndication-r17  ENUMERATED {supported}                 OPTIONAL,

-- R1 25-3b: Inter-subslot frequency hopping for PUCCH repetitions

interSubslotFreqHopping-PUCCH-r17                  ENUMERATED {supported}                 OPTIONAL,

-- R1 25-8: Semi-static HARQ-ACK codebook for sub-slot PUCCH

semiStaticHARQ-ACK-CodebookSub-SlotPUCCH-r17       ENUMERATED {supported}                 OPTIONAL,

-- R1 25-14: PHY prioritization of overlapping low-priority DG-PUSCH and high-priority CG-PUSCH

phy-PrioritizationLowPriorityDG-HighPriorityCG-r17 INTEGER(1..16)                         OPTIONAL,

-- R1 25-15: PHY prioritization of overlapping high-priority DG-PUSCH and low-priority CG-PUSCH

phy-PrioritizationHighPriorityDG-LowPriorityCG-r17 SEQUENCE {

pusch-PreparationLowPriority-r17                   ENUMERATED{sym0, sym1, sym2},

additionalCancellationTime-r17                     SEQUENCE {

scs-15kHz-r17                                      ENUMERATED{sym0, sym1, sym2}   OPTIONAL,

scs-30kHz-r17                                      ENUMERATED{sym0, sym1, sym2, sym3, sym4}    OPTIONAL,

scs-60kHz-r17                                      ENUMERATED{sym0, sym1, sym2, sym3, sym4, sym5, sym6, sym7, sym8} OPTIONAL,

scs-120kHz-r17                                     ENUMERATED{sym0, sym1, sym2, sym3, sym4, sym5, sym6, sym7, sym8, sym9,

sym10, sym11, sym12, sym13, sym14, sym15, sym16}    OPTIONAL

},

maxNumberCarriers-r17                              INTEGER(1..16)

}                                                                                         OPTIONAL,

-- R4 17-5 Support of UL DC location(s) report

extendedDC-LocationReport-r17                      ENUMERATED {supported}                 OPTIONAL

}

SubSlot-Config-r16 ::=                  SEQUENCE {

sub-SlotConfig-NCP-r16                  ENUMERATED {n4,n5,n6,n7}              OPTIONAL,

sub-SlotConfig-ECP-r16                  ENUMERATED {n4,n5,n6}                 OPTIONAL

}

SRS-AllPosResources-r16 ::=               SEQUENCE {

srs-PosResources-r16                      SRS-PosResources-r16,

srs-PosResourceAP-r16                     SRS-PosResourceAP-r16                OPTIONAL,

srs-PosResourceSP-r16                     SRS-PosResourceSP-r16                OPTIONAL

}

SRS-PosResources-r16 ::=                       SEQUENCE {

maxNumberSRS-PosResourceSetPerBWP-r16                ENUMERATED {n1, n2, n4, n8, n12, n16},

maxNumberSRS-PosResourcesPerBWP-r16                  ENUMERATED {n1, n2, n4, n8, n16, n32, n64},

maxNumberSRS-ResourcesPerBWP-PerSlot-r16             ENUMERATED {n1, n2, n3, n4, n5, n6, n8, n10, n12, n14},

maxNumberPeriodicSRS-PosResourcesPerBWP-r16          ENUMERATED {n1, n2, n4, n8, n16, n32, n64},

maxNumberPeriodicSRS-PosResourcesPerBWP-PerSlot-r16  ENUMERATED {n1, n2, n3, n4, n5, n6, n8, n10, n12, n14}

}

SRS-PosResourceAP-r16 ::=                SEQUENCE {

maxNumberAP-SRS-PosResourcesPerBWP-r16         ENUMERATED {n1, n2, n4, n8, n16, n32, n64},

maxNumberAP-SRS-PosResourcesPerBWP-PerSlot-r16 ENUMERATED {n1, n2, n3, n4, n5, n6, n8, n10, n12, n14}

}

SRS-PosResourceSP-r16 ::=                       SEQUENCE {

maxNumberSP-SRS-PosResourcesPerBWP-r16               ENUMERATED {n1, n2, n4, n8, n16, n32, n64},

maxNumberSP-SRS-PosResourcesPerBWP-PerSlot-r16       ENUMERATED {n1, n2, n3, n4, n5, n6, n8, n10, n12, n14}

}

SRS-Resources ::=                           SEQUENCE {

maxNumberAperiodicSRS-PerBWP                ENUMERATED {n1, n2, n4, n8, n16},

maxNumberAperiodicSRS-PerBWP-PerSlot        INTEGER (1..6),

maxNumberPeriodicSRS-PerBWP                 ENUMERATED {n1, n2, n4, n8, n16},

maxNumberPeriodicSRS-PerBWP-PerSlot         INTEGER (1..6),

maxNumberSemiPersistentSRS-PerBWP           ENUMERATED {n1, n2, n4, n8, n16},

maxNumberSemiPersistentSRS-PerBWP-PerSlot   INTEGER (1..6),

maxNumberSRS-Ports-PerResource              ENUMERATED {n1, n2, n4}

}

DummyF ::=                                  SEQUENCE {

maxNumberPeriodicCSI-ReportPerBWP           INTEGER (1..4),

maxNumberAperiodicCSI-ReportPerBWP          INTEGER (1..4),

maxNumberSemiPersistentCSI-ReportPerBWP     INTEGER (0..4),

simultaneousCSI-ReportsAllCC                INTEGER (5..32)

}

-- TAG-FEATURESETUPLINK-STOP

-- ASN1STOP

FeatureSetUplink field descriptions

featureSetListPerUplinkCC Indicates which features the UE supports on the individual UL carriers of the feature set (and hence of a band entry that refers to the feature set). The UE shall hence include at least as many FeatureSetUplinkPerCC-Id in this list as the number of carriers it supports according to the ca-BandwidthClassUL, except if indicating additional functionality by reducing the number of FeatureSetUplinkPerCC-Id in the feature set (see NOTE 1 in FeatureSetCombination IE description). The order of the elements in this list is not relevant, i.e., the network may configure any of the carriers in accordance with any of the FeatureSetUplinkPerCC-Id in this list.

#### –	FeatureSetUplinkId

The IE FeatureSetUplinkId identifies an uplink feature set. The FeatureSetUplinkId of a FeatureSetUplink is the index position of the FeatureSetUplink in the featureSetsUplink list in the FeatureSets IE. The first element in the list is referred to by FeatureSetUplinkId = 1, and so on. The FeatureSetUplinkId =0 is not used by an actual FeatureSetUplink but means that the UE does not support a carrier in this band of a band combination.

FeatureSetUplinkId information element

-- ASN1START

-- TAG-FEATURESETUPLINKID-START

FeatureSetUplinkId ::=                  INTEGER (0..maxUplinkFeatureSets)

-- TAG-FEATURESETUPLINKID-STOP

-- ASN1STOP

#### –	FeatureSetUplinkPerCC

The IE FeatureSetUplinkPerCC indicates a set of features that the UE supports on the corresponding carrier of one band entry of a band combination.

FeatureSetUplinkPerCC information element

-- ASN1START

-- TAG-FEATURESETUPLINKPERCC-START

FeatureSetUplinkPerCC ::=               SEQUENCE {

supportedSubcarrierSpacingUL            SubcarrierSpacing,

supportedBandwidthUL                    SupportedBandwidth,

channelBW-90mhz                         ENUMERATED {supported}                      OPTIONAL,

mimo-CB-PUSCH                           SEQUENCE {

maxNumberMIMO-LayersCB-PUSCH            MIMO-LayersUL                               OPTIONAL,

maxNumberSRS-ResourcePerSet             INTEGER (1..2)

}                                                                                   OPTIONAL,

maxNumberMIMO-LayersNonCB-PUSCH         MIMO-LayersUL                               OPTIONAL,

supportedModulationOrderUL              ModulationOrder                             OPTIONAL

}

FeatureSetUplinkPerCC-v1540 ::=       SEQUENCE {

mimo-NonCB-PUSCH                      SEQUENCE {

maxNumberSRS-ResourcePerSet           INTEGER (1..4),

maxNumberSimultaneousSRS-ResourceTx   INTEGER (1..4)

} OPTIONAL

}

FeatureSetUplinkPerCC-v1700 ::=   SEQUENCE {

supportedMinBandwidthUL-r17       SupportedBandwidth-v1700                          OPTIONAL,

-- R1 23-3-1-3	FeMIMO: Multi-TRP PUSCH repetition (type B) - non-codebook based

mTRP-PUSCH-RepetitionTypeB-r17    ENUMERATED {n1,n2,n3,n4}                          OPTIONAL,

-- R1 23-3-1-1 -codebook based Multi-TRP PUSCH repetition (type B)

mTRP-PUSCH-TypeB-CB-r17           ENUMERATED {n1,n2,n4}                             OPTIONAL,

supportedBandwidthUL-v1710        SupportedBandwidth-v1700                          OPTIONAL

}

-- TAG-FEATURESETUPLINKPERCC-STOP

-- ASN1STOP

#### –	FeatureSetUplinkPerCC-Id

The IE FeatureSetUplinkPerCC-Id identifies a set of features applicable to one carrier of a feature set. The FeatureSetUplinkPerCC-Id of a FeatureSetUplinkPerCC is the index position of the FeatureSetUplinkPerCC in the featureSetsUplinkPerCC. The first element in the list is referred to by FeatureSetUplinkPerCC-Id = 1, and so on.

FeatureSetUplinkPerCC-Id information element

-- ASN1START

-- TAG-FEATURESETUPLINKPERCC-ID-START

FeatureSetUplinkPerCC-Id ::=            INTEGER (1..maxPerCC-FeatureSets)

-- TAG-FEATURESETUPLINKPERCC-ID-STOP

-- ASN1STOP

#### –	FreqBandIndicatorEUTRA

-- ASN1START

-- TAG-FREQBANDINDICATOREUTRA-START

FreqBandIndicatorEUTRA ::=  INTEGER (1..maxBandsEUTRA)

-- TAG-FREQBANDINDICATOREUTRA-STOP

-- ASN1STOP

#### –	FreqBandList

The IE FreqBandList is used by the network to request NR CA, NR non-CA and/or MR-DC band combinations for specific NR and/or E-UTRA frequency bands and/or up to a specific number of carriers and/or up to specific aggregated bandwidth. This is also used to request feature sets (for NR) and feature set combinations (for NR and MR-DC). For NR sidelink communication, this is used by the initiating UE to request sidelink UE radio access capabilities from the peer UE.

FreqBandList information element

-- ASN1START

-- TAG-FREQBANDLIST-START

FreqBandList ::=                SEQUENCE (SIZE (1..maxBandsMRDC)) OF FreqBandInformation

FreqBandInformation ::=         CHOICE {

bandInformationEUTRA            FreqBandInformationEUTRA,

bandInformationNR               FreqBandInformationNR

}

FreqBandInformationEUTRA ::=    SEQUENCE {

bandEUTRA                       FreqBandIndicatorEUTRA,

ca-BandwidthClassDL-EUTRA       CA-BandwidthClassEUTRA                  OPTIONAL,   -- Need N

ca-BandwidthClassUL-EUTRA       CA-BandwidthClassEUTRA                  OPTIONAL    -- Need N

}

FreqBandInformationNR ::=       SEQUENCE {

bandNR                          FreqBandIndicatorNR,

maxBandwidthRequestedDL         AggregatedBandwidth                     OPTIONAL,   -- Need N

maxBandwidthRequestedUL         AggregatedBandwidth                     OPTIONAL,   -- Need N

maxCarriersRequestedDL          INTEGER (1..maxNrofServingCells)        OPTIONAL,   -- Need N

maxCarriersRequestedUL          INTEGER (1..maxNrofServingCells)        OPTIONAL    -- Need N

}

AggregatedBandwidth ::=         ENUMERATED {mhz50, mhz100, mhz150, mhz200, mhz250, mhz300, mhz350,

mhz400, mhz450, mhz500, mhz550, mhz600, mhz650, mhz700, mhz750, mhz800}

-- TAG-FREQBANDLIST-STOP

-- ASN1STOP

#### –	FreqSeparationClass

The IE FreqSeparationClass is used for an intra-band non-contiguous CA band combination to indicate frequency separation between lower edge of lowest CC and upper edge of highest CC in a frequency band.

FreqSeparationClass information element

-- ASN1START

-- TAG-FREQSEPARATIONCLASS-START

FreqSeparationClass ::= ENUMERATED { mhz800, mhz1200, mhz1400, ..., mhz400-v1650, mhz600-v1650}

FreqSeparationClassDL-v1620 ::= ENUMERATED {mhz1000, mhz1600, mhz1800, mhz2000, mhz2200, mhz2400}

FreqSeparationClassUL-v1620 ::= ENUMERATED {mhz1000}

-- TAG-FREQSEPARATIONCLASS-STOP

-- ASN1STOP

#### –	FreqSeparationClassDL-Only

The IE FreqSeparationClassDL-Only is used to indicate the frequency separation between lower edge of lowest CC and upper edge of highest CC of DL only frequency spectrum in a frequency band.

FreqSeparationClassDL-Only information element

-- ASN1START

-- TAG-FREQSEPARATIONCLASSDL-Only-START

FreqSeparationClassDL-Only-r16 ::= ENUMERATED {mhz200, mhz400, mhz600, mhz800, mhz1000, mhz1200}

-- TAG-FREQSEPARATIONCLASSDL-Only-STOP

-- ASN1STOP

#### –	FR2-2-AccessParamsPerBand

The IE FR2-2-AccessParamsPerBand is used to convey FR2-2 related parameters specific for a certain frequency band (not per feature set or band combination).

FR2-2-AccessParamsPerBand information element

-- ASN1START

-- TAG-FR2-2-ACCESSPARAMSPERBAND-START

FR2-2-AccessParamsPerBand-r17 ::=       SEQUENCE {

-- R1 24-1: Basic FR2-2 DL support

dl-FR2-2-SCS-120kHz-r17                 ENUMERATED {supported}            OPTIONAL,

-- R1 24-1a: Basic FR2-2 UL support

ul-FR2-2-SCS-120kHz-r17                 ENUMERATED {supported}            OPTIONAL,

-- R1 24-2: 120KHz SSB support for initial access in FR2-2

initialAccessSSB-120kHz-r17             ENUMERATED {supported}            OPTIONAL,

-- R1 24-1b: Wideband PRACH for 120 kHz in FR2-2

widebandPRACH-SCS-120kHz-r17            ENUMERATED {supported}            OPTIONAL,

-- R1 24-1c: Multi-RB support PUCCH format 0/1/4 for 120 kHz in FR2-2

multiRB-PUCCH-SCS-120kHz-r17            ENUMERATED {supported}            OPTIONAL,

-- R1 24-1d: Multiple PDSCH scheduling by single DCI for 120kHz in FR2-2

multiPDSCH-SingleDCI-FR2-2-SCS-120kHz-r17 ENUMERATED {supported}          OPTIONAL,

-- R1 24-1e: Multiple PUSCH scheduling by single DCI for 120kHz in FR2-2

multiPUSCH-SingleDCI-FR2-2-SCS-120kHz-r17 ENUMERATED {supported}          OPTIONAL,

-- R1 24-4: 480KHz SCS support for DL

dl-FR2-2-SCS-480kHz-r17                 ENUMERATED {supported}            OPTIONAL,

-- R1 24-4a: 480KHz SCS support for UL

ul-FR2-2-SCS-480kHz-r17                 ENUMERATED {supported}            OPTIONAL,

-- R1 24-3: 480KHz SSB support for initial access in FR2-2

initialAccessSSB-480kHz-r17             ENUMERATED {supported}            OPTIONAL,

-- R1 24-4b: Wideband PRACH for 480 kHz in FR2-2

widebandPRACH-SCS-480kHz-r17            ENUMERATED {supported}            OPTIONAL,

-- R1 24-4c: Multi-RB support PUCCH format 0/1/4 for 480 kHz in FR2-2

multiRB-PUCCH-SCS-480kHz-r17            ENUMERATED {supported}            OPTIONAL,

-- R1 24-4f: Enhanced PDCCH monitoring for 480KHz in FR2-2

enhancedPDCCH-monitoringSCS-480kHz-r17  ENUMERATED {supported}            OPTIONAL,

-- R1 24-5: 960KHz SCS support for DL

dl-FR2-2-SCS-960kHz-r17                 ENUMERATED {supported}            OPTIONAL,

-- R1 24-5a: 960KHz SCS support for UL

ul-FR2-2-SCS-960kHz-r17                 ENUMERATED {supported}            OPTIONAL,

-- R1 24-5c: Multi-RB support PUCCH format 0/1/4 for 960 kHz in FR2-2

multiRB-PUCCH-SCS-960kHz-r17            ENUMERATED {supported}            OPTIONAL,

-- R1 24-5f: Enhanced PDCCH monitoring for 960KHz in FR2-2

enhancedPDCCH-monitoringSCS-960kHz-r17  SEQUENCE {

pdcch-monitoring4-1-r17                 ENUMERATED {supported}        OPTIONAL,

pdcch-monitoring4-2-r17                 ENUMERATED {supported}        OPTIONAL,

pdcch-monitoring8-4-r17                 ENUMERATED {supported}        OPTIONAL

}                                                                         OPTIONAL,

-- R1 24-6: Type 1 channel access procedure in uplink for FR2-2 with shared spectrum channel access

type1-ChannelAccess-FR2-2-r17           ENUMERATED {supported}            OPTIONAL,

-- R1 24-7: Type 2 channel access procedure in uplink for FR2-2 with shared spectrum channel access

type2-ChannelAccess-FR2-2-r17           ENUMERATED {supported}            OPTIONAL,

-- R1 24-10: Reduced beam switching time delay

reduced-BeamSwitchTiming-FR2-2-r17      ENUMERATED {supported}            OPTIONAL,

-- R1 24-8: 32 DL HARQ processes for FR 2-2

support32-DL-HARQ-ProcessPerSCS-r17     SEQUENCE {

scs-120kHz-r17                          ENUMERATED {supported}        OPTIONAL,

scs-480kHz-r17                          ENUMERATED {supported}        OPTIONAL,

scs-960kHz-r17                          ENUMERATED {supported}        OPTIONAL

}                                                                         OPTIONAL,

-- R1 24-9: 32 UL HARQ processes for FR 2-2

support32-UL-HARQ-ProcessPerSCS-r17		SEQUENCE {

scs-120kHz-r17                          ENUMERATED {supported}        OPTIONAL,

scs-480kHz-r17                          ENUMERATED {supported}        OPTIONAL,

scs-960kHz-r17                          ENUMERATED {supported}        OPTIONAL

}                                                                         OPTIONAL,

...,

[[

-- R4 15-1: 64QAM for PUSCH for FR2-2

modulation64-QAM-PUSCH-FR2-2-r17            ENUMERATED {supported}        OPTIONAL

]]

}

-- TAG-FR2-2-ACCESSPARAMSPERBAND-STOP

-- ASN1STOP

#### –	HighSpeedParameters

The IE HighSpeedParameters is used to convey capabilities related to high speed scenarios.

HighSpeedParameters information element

-- ASN1START

-- TAG-HIGHSPEEDPARAMETERS-START

HighSpeedParameters-r16 ::= SEQUENCE {

measurementEnhancement-r16       ENUMERATED {supported}   OPTIONAL,

demodulationEnhancement-r16      ENUMERATED {supported}   OPTIONAL

}

HighSpeedParameters-v1650 ::= CHOICE {

intraNR-MeasurementEnhancement-r16       ENUMERATED {supported},

interRAT-MeasurementEnhancement-r16      ENUMERATED {supported}

}

HighSpeedParameters-v1700 ::= SEQUENCE {

-- R4 18-1: Enhanced RRM requirements specified for CA for FR1 HST

measurementEnhancementCA-r17            ENUMERATED {supported}   OPTIONAL,

-- R4 18-2: Enhanced RRM requirements specified for inter-frequency measurement in connected mode for FR1 HST

measurementEnhancementInterFreq-r17     ENUMERATED {supported}   OPTIONAL

}

-- TAG-HIGHSPEEDPARAMETERS-STOP

-- ASN1STOP

#### –	IMS-Parameters

The IE IMS-Parameters is used to convey capabilities related to IMS.

IMS-Parameters information element

-- ASN1START

-- TAG-IMS-PARAMETERS-START

IMS-Parameters ::=         SEQUENCE {

ims-ParametersCommon       IMS-ParametersCommon                  OPTIONAL,

ims-ParametersFRX-Diff     IMS-ParametersFRX-Diff                OPTIONAL,

...

}

IMS-Parameters-v1700 ::=   SEQUENCE {

ims-ParametersFR2-2-r17    IMS-ParametersFR2-2-r17               OPTIONAL

}

IMS-ParametersCommon ::=   SEQUENCE {

voiceOverEUTRA-5GC                  ENUMERATED {supported}                OPTIONAL,

...,

[[

voiceOverSCG-BearerEUTRA-5GC        ENUMERATED {supported}                OPTIONAL

]],

[[

voiceFallbackIndicationEPS-r16       ENUMERATED {supported}                   OPTIONAL

]]

}

IMS-ParametersFRX-Diff ::= SEQUENCE {

voiceOverNR                ENUMERATED {supported}                OPTIONAL,

...

}

IMS-ParametersFR2-2-r17 ::= SEQUENCE {

voiceOverNR-r17             ENUMERATED {supported}               OPTIONAL,

...

}

-- TAG-IMS-PARAMETERS-STOP

-- ASN1STOP

#### –	InterRAT-Parameters

The IE InterRAT-Parameters is used convey UE capabilities related to the other RATs.

InterRAT-Parameters information element

-- ASN1START

-- TAG-INTERRAT-PARAMETERS-START

InterRAT-Parameters ::=             SEQUENCE {

eutra                               EUTRA-Parameters                OPTIONAL,

...,

[[

utra-FDD-r16                        UTRA-FDD-Parameters-r16         OPTIONAL

]]

}

EUTRA-Parameters ::=                SEQUENCE {

supportedBandListEUTRA          SEQUENCE (SIZE (1..maxBandsEUTRA)) OF FreqBandIndicatorEUTRA,

eutra-ParametersCommon              EUTRA-ParametersCommon                                      OPTIONAL,

eutra-ParametersXDD-Diff            EUTRA-ParametersXDD-Diff                                    OPTIONAL,

...

}

EUTRA-ParametersCommon ::=      SEQUENCE {

mfbi-EUTRA                          ENUMERATED {supported}          OPTIONAL,

modifiedMPR-BehaviorEUTRA           BIT STRING (SIZE (32))          OPTIONAL,

multiNS-Pmax-EUTRA                  ENUMERATED {supported}          OPTIONAL,

rs-SINR-MeasEUTRA                   ENUMERATED {supported}          OPTIONAL,

...,

[[

ne-DC                               ENUMERATED {supported}          OPTIONAL

]],

[[

nr-HO-ToEN-DC-r16                   ENUMERATED {supported}          OPTIONAL

]]

}

EUTRA-ParametersXDD-Diff ::=        SEQUENCE {

rsrqMeasWidebandEUTRA               ENUMERATED {supported}          OPTIONAL,

...

}

UTRA-FDD-Parameters-r16 ::=                SEQUENCE {

supportedBandListUTRA-FDD-r16              SEQUENCE (SIZE (1..maxBandsUTRA-FDD-r16)) OF SupportedBandUTRA-FDD-r16,

...

}

SupportedBandUTRA-FDD-r16 ::=           ENUMERATED {

bandI, bandII, bandIII, bandIV, bandV, bandVI,

bandVII, bandVIII, bandIX, bandX, bandXI,

bandXII, bandXIII, bandXIV, bandXV, bandXVI,

bandXVII, bandXVIII, bandXIX, bandXX,

bandXXI, bandXXII, bandXXIII, bandXXIV,

bandXXV, bandXXVI, bandXXVII, bandXXVIII,

bandXXIX, bandXXX, bandXXXI, bandXXXII}

-- TAG-INTERRAT-PARAMETERS-STOP

-- ASN1STOP

#### –	MAC-Parameters

The IE MAC-Parameters is used to convey capabilities related to MAC.

MAC-Parameters information element

-- ASN1START

-- TAG-MAC-PARAMETERS-START

MAC-Parameters ::= SEQUENCE {

mac-ParametersCommon            MAC-ParametersCommon        OPTIONAL,

mac-ParametersXDD-Diff          MAC-ParametersXDD-Diff      OPTIONAL

}

MAC-Parameters-v1610 ::= SEQUENCE {

mac-ParametersFRX-Diff-r16      MAC-ParametersFRX-Diff-r16  OPTIONAL

}

MAC-Parameters-v1700 ::= SEQUENCE {

mac-ParametersFR2-2-r17         MAC-ParametersFR2-2-r17     OPTIONAL

}

MAC-ParametersCommon ::=    SEQUENCE {

lcp-Restriction                         ENUMERATED {supported}      OPTIONAL,

dummy                                   ENUMERATED {supported}      OPTIONAL,

lch-ToSCellRestriction                  ENUMERATED {supported}      OPTIONAL,

...,

[[

recommendedBitRate                      ENUMERATED {supported}      OPTIONAL,

recommendedBitRateQuery                 ENUMERATED {supported}      OPTIONAL

]],

[[

recommendedBitRateMultiplier-r16         ENUMERATED {supported}     OPTIONAL,

preEmptiveBSR-r16                        ENUMERATED {supported}     OPTIONAL,

autonomousTransmission-r16               ENUMERATED {supported}     OPTIONAL,

lch-PriorityBasedPrioritization-r16      ENUMERATED {supported}     OPTIONAL,

lch-ToConfiguredGrantMapping-r16         ENUMERATED {supported}     OPTIONAL,

lch-ToGrantPriorityRestriction-r16       ENUMERATED {supported}     OPTIONAL,

singlePHR-P-r16                          ENUMERATED {supported}     OPTIONAL,

ul-LBT-FailureDetectionRecovery-r16      ENUMERATED {supported}     OPTIONAL,

-- R4 8-1: MPE

tdd-MPE-P-MPR-Reporting-r16              ENUMERATED {supported}     OPTIONAL,

lcid-ExtensionIAB-r16                    ENUMERATED {supported}     OPTIONAL

]],

[[

spCell-BFR-CBRA-r16                      ENUMERATED {supported}     OPTIONAL

]],

[[

srs-ResourceId-Ext-r16                   ENUMERATED {supported}     OPTIONAL

]],

[[

enhancedUuDRX-forSidelink-r17            ENUMERATED {supported}     OPTIONAL,

--27-10: Support of UL MAC CE based MG activation request for PRS measurements

mg-ActivationRequestPRS-Meas-r17         ENUMERATED {supported}     OPTIONAL,

--27-11: Support of DL MAC CE based MG activation request for PRS measurements

mg-ActivationCommPRS-Meas-r17            ENUMERATED {supported}     OPTIONAL,

intraCG-Prioritization-r17               ENUMERATED {supported}     OPTIONAL,

jointPrioritizationCG-Retx-Timer-r17     ENUMERATED {supported}     OPTIONAL,

survivalTime-r17                         ENUMERATED {supported}     OPTIONAL,

lcg-ExtensionIAB-r17                     ENUMERATED {supported}     OPTIONAL,

harq-FeedbackDisabled-r17                ENUMERATED {supported}     OPTIONAL,

uplink-Harq-ModeB-r17                    ENUMERATED {supported}     OPTIONAL,

sr-TriggeredBy-TA-Report-r17             ENUMERATED {supported}     OPTIONAL,

extendedDRX-CycleInactive-r17            ENUMERATED {supported}     OPTIONAL,

simultaneousSR-PUSCH-DiffPUCCH-groups-r17 ENUMERATED {supported}    OPTIONAL,

lastTransmissionUL-r17                   ENUMERATED {supported}     OPTIONAL

]]

}

MAC-ParametersFRX-Diff-r16 ::=  SEQUENCE {

directMCG-SCellActivation-r16           ENUMERATED {supported}      OPTIONAL,

directMCG-SCellActivationResume-r16     ENUMERATED {supported}      OPTIONAL,

directSCG-SCellActivation-r16           ENUMERATED {supported}      OPTIONAL,

directSCG-SCellActivationResume-r16     ENUMERATED {supported}      OPTIONAL,

-- R1 19-1: DRX Adaptation

drx-Adaptation-r16          SEQUENCE {

non-SharedSpectrumChAccess-r16      MinTimeGap-r16              OPTIONAL,

sharedSpectrumChAccess-r16          MinTimeGap-r16              OPTIONAL

}                                                                   OPTIONAL,

...

}

MAC-ParametersFR2-2-r17 ::=  SEQUENCE {

directMCG-SCellActivation-r17           ENUMERATED {supported}      OPTIONAL,

directMCG-SCellActivationResume-r17     ENUMERATED {supported}      OPTIONAL,

directSCG-SCellActivation-r17           ENUMERATED {supported}      OPTIONAL,

directSCG-SCellActivationResume-r17     ENUMERATED {supported}      OPTIONAL,

drx-Adaptation-r17       SEQUENCE {

non-SharedSpectrumChAccess-r17      MinTimeGapFR2-2-r17         OPTIONAL,

sharedSpectrumChAccess-r17          MinTimeGapFR2-2-r17         OPTIONAL

}                                                                   OPTIONAL,

...

}

MAC-ParametersXDD-Diff ::=  SEQUENCE {

skipUplinkTxDynamic                     ENUMERATED {supported}     OPTIONAL,

logicalChannelSR-DelayTimer             ENUMERATED {supported}     OPTIONAL,

longDRX-Cycle                           ENUMERATED {supported}     OPTIONAL,

shortDRX-Cycle                          ENUMERATED {supported}     OPTIONAL,

multipleSR-Configurations               ENUMERATED {supported}     OPTIONAL,

multipleConfiguredGrants                ENUMERATED {supported}     OPTIONAL,

...,

[[

secondaryDRX-Group-r16                  ENUMERATED {supported}     OPTIONAL

]],

[[

enhancedSkipUplinkTxDynamic-r16         ENUMERATED {supported}     OPTIONAL,

enhancedSkipUplinkTxConfigured-r16      ENUMERATED {supported}     OPTIONAL

]]

}

MinTimeGap-r16 ::=    SEQUENCE {

scs-15kHz-r16                         ENUMERATED {sl1, sl3}        OPTIONAL,

scs-30kHz-r16                         ENUMERATED {sl1, sl6}        OPTIONAL,

scs-60kHz-r16                         ENUMERATED {sl1, sl12}       OPTIONAL,

scs-120kHz-r16                        ENUMERATED {sl2, sl24}       OPTIONAL

}

MinTimeGapFR2-2-r17 ::= SEQUENCE {

scs-120kHz-r17                        ENUMERATED {sl2, sl24}       OPTIONAL,

scs-480kHz-r17                        ENUMERATED {sl8, sl96}       OPTIONAL,

scs-960kHz-r17                        ENUMERATED {sl16, sl192}     OPTIONAL

}

-- TAG-MAC-PARAMETERS-STOP

-- ASN1STOP

#### –	MeasAndMobParameters

The IE MeasAndMobParameters is used to convey UE capabilities related to measurements for radio resource management (RRM), radio link monitoring (RLM) and mobility (e.g. handover).

MeasAndMobParameters information element

-- ASN1START

-- TAG-MEASANDMOBPARAMETERS-START

MeasAndMobParameters ::=                    SEQUENCE {

measAndMobParametersCommon              MeasAndMobParametersCommon              OPTIONAL,

measAndMobParametersXDD-Diff                MeasAndMobParametersXDD-Diff        OPTIONAL,

measAndMobParametersFRX-Diff                MeasAndMobParametersFRX-Diff        OPTIONAL

}

MeasAndMobParameters-v1700 ::=          SEQUENCE {

measAndMobParametersFR2-2-r17           MeasAndMobParametersFR2-2-r17           OPTIONAL

}

MeasAndMobParametersCommon ::=          SEQUENCE {

supportedGapPattern                     BIT STRING (SIZE (22))                  OPTIONAL,

ssb-RLM                                 ENUMERATED {supported}                  OPTIONAL,

ssb-AndCSI-RS-RLM                       ENUMERATED {supported}                  OPTIONAL,

...,

[[

eventB-MeasAndReport                    ENUMERATED {supported}                  OPTIONAL,

handoverFDD-TDD                         ENUMERATED {supported}                  OPTIONAL,

eutra-CGI-Reporting                     ENUMERATED {supported}                  OPTIONAL,

nr-CGI-Reporting                        ENUMERATED {supported}                  OPTIONAL

]],

[[

independentGapConfig                    ENUMERATED {supported}                  OPTIONAL,

periodicEUTRA-MeasAndReport             ENUMERATED {supported}                  OPTIONAL,

handoverFR1-FR2                         ENUMERATED {supported}                  OPTIONAL,

maxNumberCSI-RS-RRM-RS-SINR             ENUMERATED {n4, n8, n16, n32, n64, n96} OPTIONAL

]],

[[

nr-CGI-Reporting-ENDC                   ENUMERATED {supported}                  OPTIONAL

]],

[[

eutra-CGI-Reporting-NEDC                ENUMERATED {supported}                  OPTIONAL,

eutra-CGI-Reporting-NRDC                ENUMERATED {supported}                  OPTIONAL,

nr-CGI-Reporting-NEDC                   ENUMERATED {supported}                  OPTIONAL,

nr-CGI-Reporting-NRDC                   ENUMERATED {supported}                  OPTIONAL

]],

[[

reportAddNeighMeasForPeriodic-r16       ENUMERATED {supported}                  OPTIONAL,

condHandoverParametersCommon-r16        SEQUENCE {

condHandoverFDD-TDD-r16                  ENUMERATED {supported}              OPTIONAL,

condHandoverFR1-FR2-r16                  ENUMERATED {supported}              OPTIONAL

}                                                                               OPTIONAL,

nr-NeedForGap-Reporting-r16             ENUMERATED {supported}                  OPTIONAL,

supportedGapPattern-NRonly-r16          BIT STRING (SIZE (10))                  OPTIONAL,

supportedGapPattern-NRonly-NEDC-r16     ENUMERATED {supported}                  OPTIONAL,

maxNumberCLI-RSSI-r16                   ENUMERATED {n8, n16, n32, n64}          OPTIONAL,

maxNumberCLI-SRS-RSRP-r16               ENUMERATED {n4, n8, n16, n32}           OPTIONAL,

maxNumberPerSlotCLI-SRS-RSRP-r16        ENUMERATED {n2, n4, n8}                 OPTIONAL,

mfbi-IAB-r16                            ENUMERATED {supported}                  OPTIONAL,

dummy                                   ENUMERATED {supported}                  OPTIONAL,

nr-CGI-Reporting-NPN-r16                ENUMERATED {supported}                  OPTIONAL,

idleInactiveEUTRA-MeasReport-r16        ENUMERATED {supported}                  OPTIONAL,

idleInactive-ValidityArea-r16           ENUMERATED {supported}                  OPTIONAL,

eutra-AutonomousGaps-r16                ENUMERATED {supported}                  OPTIONAL,

eutra-AutonomousGaps-NEDC-r16           ENUMERATED {supported}                  OPTIONAL,

eutra-AutonomousGaps-NRDC-r16           ENUMERATED {supported}                  OPTIONAL,

pcellT312-r16                           ENUMERATED {supported}                  OPTIONAL,

supportedGapPattern-r16                 BIT STRING (SIZE (2))                   OPTIONAL

]],

[[

-- R4 19-2 Concurrent measurement gaps

concurrentMeasGap-r17                   CHOICE {

concurrentPerUE-OnlyMeasGap-r17         ENUMERATED {supported},

concurrentPerUE-PerFRCombMeasGap-r17    ENUMERATED {supported}

}                                                                               OPTIONAL,

-- R4 19-1 Network controlled small gap (NCSG)

nr-NeedForGapNCSG-Reporting-r17         ENUMERATED {supported}                  OPTIONAL,

eutra-NeedForGapNCSG-Reporting-r17      ENUMERATED {supported}                  OPTIONAL,

-- R4 19-1-1 per FR Network controlled small gap (NCSG)

ncsg-MeasGapPerFR-r17                   ENUMERATED {supported}                  OPTIONAL,

-- R4 19-1-2 Network controlled small gap (NCSG) supported patterns

ncsg-MeasGapPatterns-r17                BIT STRING (SIZE(24))                   OPTIONAL,

-- R4 19-1-3 Network controlled small gap (NCSG) supported NR-only patterns

ncsg-MeasGapNR-Patterns-r17             BIT STRING (SIZE(24))                   OPTIONAL,

-- R4 19-3-2 pre-configured measurement gap

preconfiguredUE-AutonomousMeasGap-r17   ENUMERATED {supported}                  OPTIONAL,

-- R4 19-3-1 pre-configured measurement gap

preconfiguredNW-ControlledMeasGap-r17   ENUMERATED {supported}                  OPTIONAL,

handoverFR1-FR2-2-r17                   ENUMERATED {supported}                  OPTIONAL,

handoverFR2-1-FR2-2-r17                 ENUMERATED {supported}                  OPTIONAL,

-- RAN4 14-1: per-FR MG for PRS measurement

independentGapConfigPRS-r17             ENUMERATED {supported}                  OPTIONAL,

rrm-RelaxationRRC-ConnectedRedCap-r17   ENUMERATED {supported}                  OPTIONAL,

-- R4 25-3: Parallel measurements with multiple measurement gaps

parallelMeasurementGap-r17              ENUMERATED {n2}                         OPTIONAL,

condHandoverWithSCG-NRDC-r17            ENUMERATED {supported}                  OPTIONAL,

gNB-ID-LengthReporting-r17              ENUMERATED {supported}                  OPTIONAL,

gNB-ID-LengthReporting-ENDC-r17         ENUMERATED {supported}                  OPTIONAL,

gNB-ID-LengthReporting-NEDC-r17         ENUMERATED {supported}                  OPTIONAL,

gNB-ID-LengthReporting-NRDC-r17         ENUMERATED {supported}                  OPTIONAL,

gNB-ID-LengthReporting-NPN-r17          ENUMERATED {supported}                  OPTIONAL

]],

[[

-- R4 25-1: Parallel measurements on multiple SMTC-s for a single frequency carrier

parallelSMTC-r17                        ENUMERATED {n4}                         OPTIONAL,

-- R4 19-2-1 Concurrent measurement gaps for EUTRA

concurrentMeasGapEUTRA-r17              ENUMERATED {supported}                  OPTIONAL,

serviceLinkPropDelayDiffReporting-r17   ENUMERATED {supported}                  OPTIONAL,

-- R4 19-1-4 Network controlled small gap (NCSG) performing measurement based on flag deriveSSB-IndexFromCellInter

ncsg-SymbolLevelScheduleRestrictionInter-r17  ENUMERATED {supported}            OPTIONAL

]],

[[

eventD1-MeasReportTrigger-r17           ENUMERATED {supported}                  OPTIONAL,

independentGapConfig-maxCC-r17          SEQUENCE {

fr1-Only-r17                            INTEGER (1..32)                     OPTIONAL,

fr2-Only-r17                            INTEGER (1..32)                     OPTIONAL,

fr1-AndFR2-r17                          INTEGER (1..32)                     OPTIONAL

}                                                                               OPTIONAL

]],

[[

interSatMeas-r17                            ENUMERATED {supported}              OPTIONAL,

deriveSSB-IndexFromCellInterNon-NCSG-r17    ENUMERATED {supported}              OPTIONAL

]]

}

MeasAndMobParametersXDD-Diff ::=        SEQUENCE {

intraAndInterF-MeasAndReport            ENUMERATED {supported}                  OPTIONAL,

eventA-MeasAndReport                    ENUMERATED {supported}                  OPTIONAL,

...,

[[

handoverInterF                          ENUMERATED {supported}                  OPTIONAL,

handoverLTE-EPC                         ENUMERATED {supported}                  OPTIONAL,

handoverLTE-5GC                         ENUMERATED {supported}                  OPTIONAL

]],

[[

sftd-MeasNR-Neigh                       ENUMERATED {supported}                  OPTIONAL,

sftd-MeasNR-Neigh-DRX                   ENUMERATED {supported}                  OPTIONAL

]],

[[

dummy                                   ENUMERATED {supported}                  OPTIONAL

]]

}

MeasAndMobParametersFRX-Diff ::=            SEQUENCE {

ss-SINR-Meas                                ENUMERATED {supported}              OPTIONAL,

csi-RSRP-AndRSRQ-MeasWithSSB                ENUMERATED {supported}              OPTIONAL,

csi-RSRP-AndRSRQ-MeasWithoutSSB             ENUMERATED {supported}              OPTIONAL,

csi-SINR-Meas                               ENUMERATED {supported}              OPTIONAL,

csi-RS-RLM                                  ENUMERATED {supported}              OPTIONAL,

...,

[[

handoverInterF                              ENUMERATED {supported}              OPTIONAL,

handoverLTE-EPC                             ENUMERATED {supported}              OPTIONAL,

handoverLTE-5GC                             ENUMERATED {supported}              OPTIONAL

]],

[[

maxNumberResource-CSI-RS-RLM                ENUMERATED {n2, n4, n6, n8}         OPTIONAL

]],

[[

simultaneousRxDataSSB-DiffNumerology        ENUMERATED {supported}              OPTIONAL

]],

[[

nr-AutonomousGaps-r16                       ENUMERATED {supported}              OPTIONAL,

nr-AutonomousGaps-ENDC-r16                  ENUMERATED {supported}              OPTIONAL,

nr-AutonomousGaps-NEDC-r16                  ENUMERATED {supported}              OPTIONAL,

nr-AutonomousGaps-NRDC-r16                  ENUMERATED {supported}              OPTIONAL,

dummy                                       ENUMERATED {supported}              OPTIONAL,

cli-RSSI-Meas-r16                           ENUMERATED {supported}              OPTIONAL,

cli-SRS-RSRP-Meas-r16                       ENUMERATED {supported}              OPTIONAL,

interFrequencyMeas-NoGap-r16                ENUMERATED {supported}              OPTIONAL,

simultaneousRxDataSSB-DiffNumerology-Inter-r16  ENUMERATED {supported}          OPTIONAL,

idleInactiveNR-MeasReport-r16               ENUMERATED {supported}              OPTIONAL,

-- R4 6-2: Support of beam level Early Measurement Reporting

idleInactiveNR-MeasBeamReport-r16           ENUMERATED {supported}              OPTIONAL

]],

[[

increasedNumberofCSIRSPerMO-r16             ENUMERATED {supported}              OPTIONAL

]]

}

MeasAndMobParametersFR2-2-r17 ::=           SEQUENCE {

handoverInterF-r17                          ENUMERATED {supported}              OPTIONAL,

handoverLTE-EPC-r17                         ENUMERATED {supported}              OPTIONAL,

handoverLTE-5GC-r17                         ENUMERATED {supported}              OPTIONAL,

idleInactiveNR-MeasReport-r17               ENUMERATED {supported}              OPTIONAL,

...

}

-- TAG-MEASANDMOBPARAMETERS-STOP

-- ASN1STOP

#### –	MeasAndMobParametersMRDC

The IE MeasAndMobParametersMRDC is used to convey capability parameters related to RRM measurements and RRC mobility.

MeasAndMobParametersMRDC information element

-- ASN1START

-- TAG-MEASANDMOBPARAMETERSMRDC-START

MeasAndMobParametersMRDC ::=            SEQUENCE {

measAndMobParametersMRDC-Common         MeasAndMobParametersMRDC-Common                 OPTIONAL,

measAndMobParametersMRDC-XDD-Diff       MeasAndMobParametersMRDC-XDD-Diff               OPTIONAL,

measAndMobParametersMRDC-FRX-Diff       MeasAndMobParametersMRDC-FRX-Diff               OPTIONAL

}

MeasAndMobParametersMRDC-v1560 ::=      SEQUENCE {

measAndMobParametersMRDC-XDD-Diff-v1560    MeasAndMobParametersMRDC-XDD-Diff-v1560      OPTIONAL

}

MeasAndMobParametersMRDC-v1610 ::=      SEQUENCE {

measAndMobParametersMRDC-Common-v1610      MeasAndMobParametersMRDC-Common-v1610        OPTIONAL,

interNR-MeasEUTRA-IAB-r16                  ENUMERATED {supported}                       OPTIONAL

}

MeasAndMobParametersMRDC-v1700 ::=      SEQUENCE {

measAndMobParametersMRDC-Common-v1700      MeasAndMobParametersMRDC-Common-v1700        OPTIONAL

}

MeasAndMobParametersMRDC-v1730 ::=      SEQUENCE {

measAndMobParametersMRDC-Common-v1730   MeasAndMobParametersMRDC-Common-v1730           OPTIONAL

}

MeasAndMobParametersMRDC-Common ::=     SEQUENCE {

independentGapConfig                    ENUMERATED {supported}                          OPTIONAL

}

MeasAndMobParametersMRDC-Common-v1610 ::=   SEQUENCE {

condPSCellChangeParametersCommon-r16        SEQUENCE {

condPSCellChangeFDD-TDD-r16                 ENUMERATED {supported}                  OPTIONAL,

condPSCellChangeFR1-FR2-r16                 ENUMERATED {supported}                  OPTIONAL

}                                                                                       OPTIONAL,

pscellT312-r16                              ENUMERATED {supported}                      OPTIONAL

}

MeasAndMobParametersMRDC-Common-v1700 ::=   SEQUENCE {

condPSCellChangeParameters-r17              SEQUENCE {

inter-SN-condPSCellChangeFDD-TDD-NRDC-r17       ENUMERATED {supported}              OPTIONAL,

inter-SN-condPSCellChangeFR1-FR2-NRDC-r17       ENUMERATED {supported}              OPTIONAL,

inter-SN-condPSCellChangeFDD-TDD-ENDC-r17       ENUMERATED {supported}              OPTIONAL,

inter-SN-condPSCellChangeFR1-FR2-ENDC-r17       ENUMERATED {supported}              OPTIONAL,

mn-InitiatedCondPSCellChange-FR1FDD-ENDC-r17    ENUMERATED {supported}              OPTIONAL,

mn-InitiatedCondPSCellChange-FR1TDD-ENDC-r17    ENUMERATED {supported}              OPTIONAL,

mn-InitiatedCondPSCellChange-FR2TDD-ENDC-r17    ENUMERATED {supported}              OPTIONAL,

sn-InitiatedCondPSCellChange-FR1FDD-ENDC-r17    ENUMERATED {supported}              OPTIONAL,

sn-InitiatedCondPSCellChange-FR1TDD-ENDC-r17    ENUMERATED {supported}              OPTIONAL,

sn-InitiatedCondPSCellChange-FR2TDD-ENDC-r17    ENUMERATED {supported}              OPTIONAL

}                                                                                       OPTIONAL,

condHandoverWithSCG-ENDC-r17                        ENUMERATED {supported}              OPTIONAL,

condHandoverWithSCG-NEDC-r17                        ENUMERATED {supported}              OPTIONAL

}

MeasAndMobParametersMRDC-Common-v1730 ::= SEQUENCE {

independentGapConfig-maxCC-r17          SEQUENCE {

fr1-Only-r17                            INTEGER (1..32)                             OPTIONAL,

fr2-Only-r17                            INTEGER (1..32)                             OPTIONAL,

fr1-AndFR2-r17                          INTEGER (1..32)                             OPTIONAL

}

}

MeasAndMobParametersMRDC-XDD-Diff ::=   SEQUENCE {

sftd-MeasPSCell                         ENUMERATED {supported}                          OPTIONAL,

sftd-MeasNR-Cell                        ENUMERATED {supported}                          OPTIONAL

}

MeasAndMobParametersMRDC-XDD-Diff-v1560 ::=    SEQUENCE {

sftd-MeasPSCell-NEDC                           ENUMERATED {supported}                   OPTIONAL

}

MeasAndMobParametersMRDC-FRX-Diff ::=          SEQUENCE {

simultaneousRxDataSSB-DiffNumerology           ENUMERATED {supported}                   OPTIONAL

}

-- TAG-MEASANDMOBPARAMETERSMRDC-STOP

-- ASN1STOP

#### –	MIMO-Layers

The IE MIMO-Layers is used to convey the number of supported MIMO layers.

MIMO-Layers information element

-- ASN1START

-- TAG-MIMO-LAYERS-START

MIMO-LayersDL ::=   ENUMERATED {twoLayers, fourLayers, eightLayers}

MIMO-LayersUL ::=   ENUMERATED {oneLayer, twoLayers, fourLayers}

-- TAG-MIMO-LAYERS-STOP

-- ASN1STOP

#### –	MIMO-ParametersPerBand

The IE MIMO-ParametersPerBand is used to convey MIMO related parameters specific for a certain band (not per feature set or band combination).

MIMO-ParametersPerBand information element

-- ASN1START

-- TAG-MIMO-PARAMETERSPERBAND-START

MIMO-ParametersPerBand ::=          SEQUENCE {

tci-StatePDSCH                      SEQUENCE {

maxNumberConfiguredTCI-StatesPerCC  ENUMERATED {n4, n8, n16, n32, n64, n128}                                   OPTIONAL,

maxNumberActiveTCI-PerBWP           ENUMERATED {n1, n2, n4, n8}                                                OPTIONAL

}                                                                                                              OPTIONAL,

additionalActiveTCI-StatePDCCH              ENUMERATED {supported}                                             OPTIONAL,

pusch-TransCoherence                        ENUMERATED {nonCoherent, partialCoherent, fullCoherent}            OPTIONAL,

beamCorrespondenceWithoutUL-BeamSweeping    ENUMERATED {supported}                                             OPTIONAL,

periodicBeamReport                          ENUMERATED {supported}                                             OPTIONAL,

aperiodicBeamReport                         ENUMERATED {supported}                                             OPTIONAL,

sp-BeamReportPUCCH                          ENUMERATED {supported}                                             OPTIONAL,

sp-BeamReportPUSCH                          ENUMERATED {supported}                                             OPTIONAL,

dummy1                                      DummyG                                                             OPTIONAL,

maxNumberRxBeam                             INTEGER (2..8)                                                     OPTIONAL,

maxNumberRxTxBeamSwitchDL                   SEQUENCE {

scs-15kHz                                   ENUMERATED {n4, n7, n14}                                           OPTIONAL,

scs-30kHz                                   ENUMERATED {n4, n7, n14}                                           OPTIONAL,

scs-60kHz                                   ENUMERATED {n4, n7, n14}                                           OPTIONAL,

scs-120kHz                                  ENUMERATED {n4, n7, n14}                                           OPTIONAL,

scs-240kHz                                  ENUMERATED {n4, n7, n14}                                           OPTIONAL

}                                                                                                              OPTIONAL,

maxNumberNonGroupBeamReporting              ENUMERATED {n1, n2, n4}                                            OPTIONAL,

groupBeamReporting                          ENUMERATED {supported}                                             OPTIONAL,

uplinkBeamManagement                        SEQUENCE {

maxNumberSRS-ResourcePerSet-BM              ENUMERATED {n2, n4, n8, n16},

maxNumberSRS-ResourceSet                    INTEGER (1..8)

}                                                                                                              OPTIONAL,

maxNumberCSI-RS-BFD                 INTEGER (1..64)                                                            OPTIONAL,

maxNumberSSB-BFD                    INTEGER (1..64)                                                            OPTIONAL,

maxNumberCSI-RS-SSB-CBD             INTEGER (1..256)                                                           OPTIONAL,

dummy2                              ENUMERATED {supported}                                                     OPTIONAL,

twoPortsPTRS-UL                     ENUMERATED {supported}                                                     OPTIONAL,

dummy5                              SRS-Resources                                                              OPTIONAL,

dummy3                              INTEGER (1..4)                                                             OPTIONAL,

beamReportTiming                    SEQUENCE {

scs-15kHz                           ENUMERATED {sym2, sym4, sym8}                                              OPTIONAL,

scs-30kHz                           ENUMERATED {sym4, sym8, sym14, sym28}                                      OPTIONAL,

scs-60kHz                           ENUMERATED {sym8, sym14, sym28}                                            OPTIONAL,

scs-120kHz                          ENUMERATED {sym14, sym28, sym56}                                           OPTIONAL

}                                                                                                              OPTIONAL,

ptrs-DensityRecommendationSetDL     SEQUENCE {

scs-15kHz                           PTRS-DensityRecommendationDL                                               OPTIONAL,

scs-30kHz                           PTRS-DensityRecommendationDL                                               OPTIONAL,

scs-60kHz                           PTRS-DensityRecommendationDL                                               OPTIONAL,

scs-120kHz                          PTRS-DensityRecommendationDL                                               OPTIONAL

}                                                                                                              OPTIONAL,

ptrs-DensityRecommendationSetUL     SEQUENCE {

scs-15kHz                           PTRS-DensityRecommendationUL                                               OPTIONAL,

scs-30kHz                           PTRS-DensityRecommendationUL                                               OPTIONAL,

scs-60kHz                           PTRS-DensityRecommendationUL                                               OPTIONAL,

scs-120kHz                          PTRS-DensityRecommendationUL                                               OPTIONAL

}                                                                                                              OPTIONAL,

dummy4                              DummyH                                                                     OPTIONAL,

aperiodicTRS                        ENUMERATED {supported}                                                     OPTIONAL,

...,

[[

dummy6                              ENUMERATED {true}                                                          OPTIONAL,

beamManagementSSB-CSI-RS            BeamManagementSSB-CSI-RS                                                   OPTIONAL,

beamSwitchTiming                    SEQUENCE {

scs-60kHz                           ENUMERATED {sym14, sym28, sym48, sym224, sym336}                           OPTIONAL,

scs-120kHz                          ENUMERATED {sym14, sym28, sym48, sym224, sym336}                           OPTIONAL

}                                                                                                              OPTIONAL,

codebookParameters                  CodebookParameters                                                         OPTIONAL,

csi-RS-IM-ReceptionForFeedback      CSI-RS-IM-ReceptionForFeedback                                             OPTIONAL,

csi-RS-ProcFrameworkForSRS          CSI-RS-ProcFrameworkForSRS                                                 OPTIONAL,

csi-ReportFramework                 CSI-ReportFramework                                                        OPTIONAL,

csi-RS-ForTracking                  CSI-RS-ForTracking                                                         OPTIONAL,

srs-AssocCSI-RS                     SEQUENCE (SIZE (1.. maxNrofCSI-RS-Resources)) OF SupportedCSI-RS-Resource  OPTIONAL,

spatialRelations                    SpatialRelations                                                           OPTIONAL

]],

[[

-- R1 16-2b-0: Support of default QCL assumption with two TCI states

defaultQCL-TwoTCI-r16               ENUMERATED {supported}                                                     OPTIONAL,

codebookParametersPerBand-r16       CodebookParameters-v1610                                                   OPTIONAL,

-- R1 16-1b-3: Support of PUCCH resource groups per BWP for simultaneous spatial relation update

simul-SpatialRelationUpdatePUCCHResGroup-r16    ENUMERATED {supported}                                         OPTIONAL,

-- R1 16-1f: Maximum number of SCells configured for SCell beam failure recovery simultaneously

maxNumberSCellBFR-r16                           ENUMERATED {n1,n2,n4,n8}                                       OPTIONAL,

-- R1 16-2c: Supports simultaneous reception with different Type-D for FR2 only

simultaneousReceptionDiffTypeD-r16              ENUMERATED {supported}                                         OPTIONAL,

-- R1 16-1a-1: SSB/CSI-RS for L1-SINR measurement

ssb-csirs-SINR-measurement-r16      SEQUENCE {

maxNumberSSB-CSIRS-OneTx-CMR-r16    ENUMERATED {n8, n16, n32, n64},

maxNumberCSI-IM-NZP-IMR-res-r16     ENUMERATED {n8, n16, n32, n64},

maxNumberCSIRS-2Tx-res-r16          ENUMERATED {n0, n4, n8, n16, n32, n64},

maxNumberSSB-CSIRS-res-r16          ENUMERATED {n8, n16, n32, n64, n128},

maxNumberCSI-IM-NZP-IMR-res-mem-r16 ENUMERATED {n8, n16, n32, n64, n128},

supportedCSI-RS-Density-CMR-r16     ENUMERATED {one, three, oneAndThree},

maxNumberAperiodicCSI-RS-Res-r16    ENUMERATED {n2, n4, n8, n16, n32, n64},

supportedSINR-meas-r16              ENUMERATED {ssbWithCSI-IM, ssbWithNZP-IMR, csirsWithNZP-IMR, csi-RSWithoutIMR}  OPTIONAL

}                                                                                                              OPTIONAL,

-- R1 16-1a-2: Non-group based L1-SINR reporting

nonGroupSINR-reporting-r16              ENUMERATED {n1, n2, n4}                                                OPTIONAL,

-- R1 16-1a-3: Non-group based L1-SINR reporting

groupSINR-reporting-r16                 ENUMERATED {supported}                                                 OPTIONAL,

multiDCI-multiTRP-Parameters-r16        SEQUENCE {

-- R1 16-2a-0: Overlapping PDSCHs in time and fully overlapping in frequency and time

overlapPDSCHsFullyFreqTime-r16          INTEGER (1..2)                                                     OPTIONAL,

-- R1 16-2a-1: Overlapping PDSCHs in time and partially overlapping in frequency and time

overlapPDSCHsInTimePartiallyFreq-r16    ENUMERATED {supported}                                             OPTIONAL,

-- R1 16-2a-2: Out of order operation for DL

outOfOrderOperationDL-r16               SEQUENCE {

supportPDCCH-ToPDSCH-r16                ENUMERATED {supported}                                         OPTIONAL,

supportPDSCH-ToHARQ-ACK-r16             ENUMERATED {supported}                                         OPTIONAL

}                                                                                                          OPTIONAL,

-- R1 16-2a-3: Out of order operation for UL

outOfOrderOperationUL-r16               ENUMERATED {supported}                                             OPTIONAL,

-- R1 16-2a-5: Separate CRS rate matching

separateCRS-RateMatching-r16            ENUMERATED {supported}                                             OPTIONAL,

-- R1 16-2a-6: Default QCL enhancement for multi-DCI based multi-TRP

defaultQCL-PerCORESETPoolIndex-r16      ENUMERATED {supported}                                             OPTIONAL,

-- R1 16-2a-7: Maximum number of activated TCI states

maxNumberActivatedTCI-States-r16        SEQUENCE {

maxNumberPerCORESET-Pool-r16            ENUMERATED {n1, n2, n4, n8},

maxTotalNumberAcrossCORESET-Pool-r16    ENUMERATED {n2, n4, n8, n16}

}                                                                                                          OPTIONAL

}                                                                                                              OPTIONAL,

singleDCI-SDM-scheme-Parameters-r16         SEQUENCE {

-- R1 16-2b-1b: Single-DCI based SDM scheme - Support of new DMRS port entry

supportNewDMRS-Port-r16                     ENUMERATED {supported1, supported2, supported3}                                        OPTIONAL,

-- R1 16-2b-1a: Support of s-port DL PTRS

supportTwoPortDL-PTRS-r16                   ENUMERATED {supported}                                         OPTIONAL

}                                                                                                              OPTIONAL,

-- R1 16-2b-2: Support of single-DCI based FDMSchemeA

supportFDM-SchemeA-r16                      ENUMERATED {supported}                                             OPTIONAL,

-- R1 16-2b-3a: Single-DCI based FDMSchemeB CW soft combining

supportCodeWordSoftCombining-r16            ENUMERATED {supported}                                             OPTIONAL,

-- R1 16-2b-4: Single-DCI based TDMSchemeA

supportTDM-SchemeA-r16                      ENUMERATED {kb3, kb5, kb10, kb20, noRestriction}                   OPTIONAL,

-- R1 16-2b-5: Single-DCI based inter-slot TDM

supportInter-slotTDM-r16                    SEQUENCE {

supportRepNumPDSCH-TDRA-r16                 ENUMERATED {n2, n3, n4, n5, n6, n7, n8, n16},

maxTBS-Size-r16                             ENUMERATED {kb3, kb5, kb10, kb20, noRestriction},

maxNumberTCI-states-r16                     INTEGER (1..2)

}                                                                                                              OPTIONAL,

-- R1 16-4: Low PAPR DMRS for PDSCH

lowPAPR-DMRS-PDSCH-r16                      ENUMERATED {supported}                                             OPTIONAL,

-- R1 16-6a: Low PAPR DMRS for PUSCH without transform precoding

lowPAPR-DMRS-PUSCHwithoutPrecoding-r16      ENUMERATED {supported}                                             OPTIONAL,

-- R1 16-6b: Low PAPR DMRS for PUCCH

lowPAPR-DMRS-PUCCH-r16                      ENUMERATED {supported}                                             OPTIONAL,

-- R1 16-6c: Low PAPR DMRS for PUSCH with transform precoding & pi/2 BPSK

lowPAPR-DMRS-PUSCHwithPrecoding-r16         ENUMERATED {supported}                                             OPTIONAL,

-- R1 16-7: Extension of the maximum number of configured aperiodic CSI report settings

csi-ReportFrameworkExt-r16                  CSI-ReportFrameworkExt-r16                                         OPTIONAL,

-- R1 16-3a, 16-3a-1, 16-3b, 16-3b-1, 16-8: Individual new codebook types

codebookParametersAddition-r16              CodebookParametersAddition-r16                                     OPTIONAL,

-- R1 16-8: Mixed codebook types

codebookComboParametersAddition-r16         CodebookComboParametersAddition-r16                                OPTIONAL,

-- R4 8-2: SSB based beam correspondence

beamCorrespondenceSSB-based-r16             ENUMERATED {supported}                                             OPTIONAL,

-- R4 8-3: CSI-RS based beam correspondence

beamCorrespondenceCSI-RS-based-r16          ENUMERATED {supported}                                             OPTIONAL,

beamSwitchTiming-r16                        SEQUENCE {

scs-60kHz-r16                               ENUMERATED {sym224, sym336}                                    OPTIONAL,

scs-120kHz-r16                              ENUMERATED {sym224, sym336}                                    OPTIONAL

}                                                                                                              OPTIONAL

]],

[[

-- R1 16-1a-4: Semi-persistent L1-SINR report on PUCCH

semi-PersistentL1-SINR-Report-PUCCH-r16     SEQUENCE {

supportReportFormat1-2OFDM-syms-r16         ENUMERATED {supported}                                     OPTIONAL,

supportReportFormat4-14OFDM-syms-r16        ENUMERATED {supported}                                     OPTIONAL

}                                                                                                          OPTIONAL,

-- R1 16-1a-5: Semi-persistent L1-SINR report on PUSCH

semi-PersistentL1-SINR-Report-PUSCH-r16     ENUMERATED {supported}                                         OPTIONAL

]],

[[

-- R1 16-1h: Support of 64 configured PUCCH spatial relations

spatialRelations-v1640                      SEQUENCE {

maxNumberConfiguredSpatialRelations-v1640   ENUMERATED {n96, n128, n160, n192, n224, n256, n288, n320}

}                                                                                                          OPTIONAL,

-- R1 16-1i: Support of 64 configured candidate beam RSs for BFR

support64CandidateBeamRS-BFR-r16            ENUMERATED {supported}                                         OPTIONAL

]],

[[

-- R1 16-2a-9: Interpretation of maxNumberMIMO-LayersPDSCH for multi-DCI based mTRP

maxMIMO-LayersForMulti-DCI-mTRP-r16         ENUMERATED {supported}                                         OPTIONAL

]],

[[

supportedSINR-meas-v1670                    BIT STRING (SIZE (4))                                          OPTIONAL

]],

[[

-- R1 23-8-5	Increased repetition for SRS

srs-increasedRepetition-r17                 ENUMERATED {supported}                                         OPTIONAL,

-- R1 23-8-6	Partial frequency sounding of SRS

srs-partialFrequencySounding-r17            ENUMERATED {supported}                                         OPTIONAL,

-- R1 23-8-7	Start RB location hopping for partial frequency SRS

srs-startRB-locationHoppingPartial-r17      ENUMERATED {supported}                                         OPTIONAL,

-- R1 23-8-8	Comb-8 SRS

srs-combEight-r17                           ENUMERATED {supported}                                         OPTIONAL,

-- R1 23-9-1	Basic Features of Further Enhanced Port-Selection Type II Codebook (FeType-II) per band information

codebookParametersfetype2-r17               CodebookParametersfetype2-r17                                  OPTIONAL,

-- R1 23-3-1-2a    Two associated CSI-RS resources

mTRP-PUSCH-twoCSI-RS-r17                    ENUMERATED {supported}                                         OPTIONAL,

-- R1 23-3-2    Multi-TRP PUCCH repetition scheme 1 (inter-slot)

mTRP-PUCCH-InterSlot-r17                    ENUMERATED {pf0-2, pf1-3-4, pf0-4}                             OPTIONAL,

-- R1 23-3-2b    Cyclic mapping for multi-TRP PUCCH repetition

mTRP-PUCCH-CyclicMapping-r17                ENUMERATED {supported}                                         OPTIONAL,

-- R1 23-3-2c    Second TPC field for multi-TRP PUCCH repetition

mTRP-PUCCH-SecondTPC-r17                    ENUMERATED {supported}                                         OPTIONAL,

-- R1 23-5-2    MTRP BFR based on two BFD-RS set

mTRP-BFR-twoBFD-RS-Set-r17                  SEQUENCE {

maxBFD-RS-resourcesPerSetPerBWP-r17         ENUMERATED {n1, n2},

maxBFR-r17                                  INTEGER (1..9),

maxBFD-RS-resourcesAcrossSetsPerBWP-r17     ENUMERATED {n2, n3, n4}

}                                                                                                          OPTIONAL,

-- R1 23-5-2a    PUCCH-SR resources for MTRP BFRQ - Max number of PUCCH-SR resources for MTRP BFRQ per cell group

mTRP-BFR-PUCCH-SR-perCG-r17                 ENUMERATED{n1, n2}                                             OPTIONAL,

-- R1 23-5-2b    Association between a BFD-RS resource set on SpCell and a PUCCH SR resource

mTRP-BFR-association-PUCCH-SR-r17           ENUMERATED {supported}                                         OPTIONAL,

-- R1 23-6-3    Simultaneous activation of two TCI states for PDCCH across multiple CCs (HST/URLLC)

sfn-SimulTwoTCI-AcrossMultiCC-r17           ENUMERATED {supported}                                         OPTIONAL,

-- R1 23-6-4    Default DL beam setup for SFN

sfn-DefaultDL-BeamSetup-r17                 ENUMERATED {supported}                                         OPTIONAL,

-- R1 23-6-4a    Default UL beam setup for SFN PDCCH(FR2 only)

sfn-DefaultUL-BeamSetup-r17                 ENUMERATED {supported}                                         OPTIONAL,

-- R1 23-8-1    SRS triggering offset enhancement

srs-TriggeringOffset-r17                    ENUMERATED {n1, n2, n4}                                        OPTIONAL,

-- R1 23-8-2    Triggering SRS only in DCI 0_1/0_2

srs-TriggeringDCI-r17                       ENUMERATED {supported}                                         OPTIONAL,

-- R1 23-9-5    Active CSI-RS resources and ports for mixed codebook types in any slot per band information

codebookComboParameterMixedType-r17         CodebookComboParameterMixedType-r17                            OPTIONAL,

-- R1 23-1-1    Unified TCI [with joint DL/UL TCI update] for intra-cell beam management

unifiedJointTCI-r17                         SEQUENCE{

maxConfiguredJointTCI-r17                   ENUMERATED {n8, n12, n16, n24, n32, n48, n64, n128},

maxActivatedTCIAcrossCC-r17                 ENUMERATED {n1, n2, n4, n8, n16}

}                                                                                                          OPTIONAL,

-- R1  23-1-1b    Unified TCI with joint DL/UL TCI update for intra- and inter-cell beam management with more than one MAC-CE

unifiedJointTCI-multiMAC-CE-r17             SEQUENCE{

minBeamApplicationTime-r17                  ENUMERATED {n1, n2, n4, n7, n14, n28, n42, n56, n70, n84, n98, n112, n224, n336}

OPTIONAL,

maxNumMAC-CE-PerCC                          ENUMERATED {n2, n3, n4, n5, n6, n7, n8}

} OPTIONAL,

-- R1 23-1-1d    Per BWP TCI state pool configuration for CA mode

unifiedJointTCI-perBWP-CA-r17               ENUMERATED {supported}                                         OPTIONAL,

-- R1 23-1-1e    TCI state pool configuration with TCI pool sharing for CA mode

unifiedJointTCI-ListSharingCA-r17           ENUMERATED {n1,n2,n4,n8}                                       OPTIONAL,

-- R1 23-1-1f    Common multi-CC TCI state ID update and activation

unifiedJointTCI-commonMultiCC-r17           ENUMERATED {supported}                                         OPTIONAL,

-- R1 23-1-1g    Beam misalignment between the DL source RS in the TCI state

unifiedJointTCI-BeamAlignDLRS-r17           ENUMERATED {supported}                                         OPTIONAL,

-- R1 23-1-1h    Association between TCI state and UL PC settings for PUCCH, PUSCH, and SRS

unifiedJointTCI-PC-association-r17          ENUMERATED {supported}                                         OPTIONAL,

-- R1 23-1-1i    Indication/configuration of R17 TCI states for aperiodic CSI-RS, PDCCH, PDSCH

unifiedJointTCI-Legacy-r17                  ENUMERATED {supported}                                         OPTIONAL,

-- 23-1-1m    Indication/configuration of R17 TCI states for SRS

unifiedJointTCI-Legacy-SRS-r17              ENUMERATED {supported}                                         OPTIONAL,

-- R1 23-1-1j    Indication/configuration of R17 TCI states for CORESET #0

unifiedJointTCI-Legacy-CORESET0-r17         ENUMERATED {supported}                                         OPTIONAL,

-- R1 23-1-1c    SCell BFR with unified TCI framework  (NOTE; pre-requisite is empty)

unifiedJointTCI-SCellBFR-r17                ENUMERATED {supported}                                         OPTIONAL,

-- R1 23-1-1a    Unified TCI with joint DL/UL TCI update for inter-cell beam management

unifiedJointTCI-InterCell-r17               SEQUENCE{

additionalMAC-CE-PerCC-r17                  ENUMERATED {n0, n1, n2, n4},

additionalMAC-CE-AcrossCC-r17               ENUMERATED {n0, n1, n2, n4}

}                                                                                                          OPTIONAL,

-- R1  23-10-1    Unified TCI with separate DL/UL TCI update for intra-cell beam management

unifiedSeparateTCI-r17                      SEQUENCE{

maxConfiguredDL-TCI-r17                     ENUMERATED {n4, n8, n12, n16, n24, n32, n48, n64, n128},

maxConfiguredUL-TCI-r17                     ENUMERATED {n4, n8, n12, n16, n24, n32, n48, n64},

maxActivatedDL-TCIAcrossCC-r17              ENUMERATED {n1, n2, n4, n8, n16},

maxActivatedUL-TCIAcrossCC-r17              ENUMERATED {n1, n2, n4, n8, n16}

} OPTIONAL,

-- R1  23-10-1b    Unified TCI with separate DL/UL TCI update for intra-cell beam management with more than one MAC-CE

unifiedSeparateTCI-multiMAC-CE-r17          SEQUENCE{

minBeamApplicationTime-r17                  ENUMERATED {n1, n2, n4, n7, n14, n28, n42, n56, n70, n84, n98, n112, n224, n336},

maxActivatedDL-TCIPerCC-r17                 INTEGER (2..8),

maxActivatedUL-TCIPerCC-r17                 INTEGER (2..8)

}                                                                                                          OPTIONAL,

-- R1 23-10-1d    Per BWP DL/UL-TCI state pool configuration for CA mode

unifiedSeparateTCI-perBWP-CA-r17            ENUMERATED {supported}                                         OPTIONAL,

-- R1 23-10-1e    TCI state pool configuration with DL/UL-TCI pool sharing for CA mode

unifiedSeparateTCI-ListSharingCA-r17        SEQUENCE {

maxNumListDL-TCI-r17                        ENUMERATED {n1,n2,n4,n8}                                   OPTIONAL,

maxNumListUL-TCI-r17                        ENUMERATED {n1,n2,n4,n8}                                   OPTIONAL

} OPTIONAL,

-- R1 23-10-1f    Common multi-CC DL/UL-TCI state ID update and activation with separate DL/UL TCI update

unifiedSeparateTCI-commonMultiCC-r17    ENUMERATED {supported}                                             OPTIONAL,

-- 23-10-1m    Unified TCI with separate DL/UL TCI update for inter-cell beam management with more than one MAC-CE

unifiedSeparateTCI-InterCell-r17            SEQUENCE {

k-DL-PerCC-r17                              ENUMERATED {n0, n1, n2, n4},

k-UL-PerCC-r17                              ENUMERATED {n0, n1, n2, n4},

k-DL-AcrossCC-r17                           ENUMERATED {n0, n1, n2, n4},

k-UL-AcrossCC-r17                           ENUMERATED {n0, n1, n2, n4}

}                                                                                                          OPTIONAL,

-- R1  23-1-2    Inter-cell beam measurement and reporting (for inter-cell BM and mTRP)

unifiedJointTCI-mTRP-InterCell-BM-r17       SEQUENCE {

maxNumAdditionalPCI-L1-RSRP-r17             INTEGER (1..7),

maxNumSSB-ResourceL1-RSRP-AcrossCC-r17      ENUMERATED {n1,n2,n4,n8}

}                                                                                                          OPTIONAL,

-- R1  23-1-3    MPE mitigation

mpe-Mitigation-r17                          SEQUENCE {

maxNumP-MPR-RI-pairs-r17                    INTEGER (1..4),

maxNumConfRS-r17                            ENUMERATED {n1, n2, n4, n8, n12, n16, n28, n32, n48, n64}

}                                                                                                          OPTIONAL,

-- R1  23-1-4    UE capability value reporting

srs-PortReport-r17                          SEQUENCE {

capVal1-r17                             ENUMERATED {n1, n2, n4}                                    OPTIONAL,

capVal2-r17                             ENUMERATED {n1, n2, n4}                                    OPTIONAL,

capVal3-r17                             ENUMERATED {n1, n2, n4}                                    OPTIONAL,

capVal4-r17                             ENUMERATED {n1, n2, n4}                                    OPTIONAL

}                                                                                                          OPTIONAL,

-- R1 23-2-1a    Monitoring of individual candidates

mTRP-PDCCH-individual-r17                   ENUMERATED {supported}                                         OPTIONAL,

-- R1 23-2-1b    PDCCH repetition with PDCCH monitoring on any span of up to 3 consecutive OFDM symbols of a slot

mTRP-PDCCH-anySpan-3Symbols-r17             ENUMERATED {supported}                                         OPTIONAL,

-- R1 23-2-2    Two QCL TypeD for CORESET monitoring in PDCCH repetition

mTRP-PDCCH-TwoQCL-TypeD-r17                 ENUMERATED {supported}                                         OPTIONAL,

-- R1 23-3-1-2b    CSI-RS processing framework for SRS with two associated CSI-RS resources

mTRP-PUSCH-CSI-RS-r17                       SEQUENCE {

maxNumPeriodicSRS-r17                       INTEGER (1..8),

maxNumAperiodicSRS-r17                      INTEGER (1..8),

maxNumSP-SRS-r17                            INTEGER (0..8),

numSRS-ResourcePerCC-r17                    INTEGER (1..16),

numSRS-ResourceNonCodebook-r17              INTEGER (1..2)

}                                                                                                          OPTIONAL,

-- R1 23-3-1a    Cyclic mapping for Multi-TRP PUSCH repetition

mTRP-PUSCH-cyclicMapping-r17                ENUMERATED {typeA,typeB,both}                                  OPTIONAL,

-- R1 23-3-1b    Second TPC field for Multi-TRP PUSCH repetition

mTRP-PUSCH-secondTPC-r17                    ENUMERATED {supported}                                         OPTIONAL,

-- R1 23-3-1c     Two PHR reporting

mTRP-PUSCH-twoPHR-Reporting-r17             ENUMERATED {supported}                                         OPTIONAL,

-- R1 23-3-1e    A-CSI report

mTRP-PUSCH-A-CSI-r17                        ENUMERATED {supported}                                         OPTIONAL,

-- R1 23-3-1f    SP-CSI report

mTRP-PUSCH-SP-CSI-r17                       ENUMERATED {supported}                                         OPTIONAL,

-- R1 23-3-1g    CG PUSCH transmission

mTRP-PUSCH-CG-r17                           ENUMERATED {supported}                                         OPTIONAL,

-- R1 23-3-2d    Updating two Spatial relation or two sets of power control parameters for PUCCH group

mTRP-PUCCH-MAC-CE-r17                       ENUMERATED {supported}                                         OPTIONAL,

-- R1 23-3-2e    Maximum number of power control parameter sets configured for multi-TRP PUCCH repetition in FR1

mTRP-PUCCH-maxNum-PC-FR1-r17                INTEGER (3..8)                                                 OPTIONAL,

-- R1 23-4    IntCell-mTRP

mTRP-inter-Cell-r17                         SEQUENCE {

maxNumAdditionalPCI-Case1-r17               INTEGER (1..7),

maxNumAdditionalPCI-Case2-r17               INTEGER (0..7)

}                                                                                                          OPTIONAL,

-- R1 23-5-1    Group based L1-RSRP reporting enhancements

mTRP-GroupBasedL1-RSRP-r17                  SEQUENCE {

maxNumBeamGroups-r17                        INTEGER (1..4),

maxNumRS-WithinSlot-r17                     ENUMERATED {n2,n3,n4,n8,n16,n32,n64},

maxNumRS-AcrossSlot-r17                     ENUMERATED {n8, n16, n32, n64, n128}

}                                                                                                          OPTIONAL,

-- R1 23-5-2c    MAC-CE based update of explicit BFD-RS    mTRP-PUCCH-IntraSlot-r17  => per band

mTRP-BFD-RS-MAC-CE-r17                      ENUMERATED {n4, n8, n12, n16, n32, n48, n64 }                  OPTIONAL,

-- R1 23-7-1    Basic Features of CSI Enhancement for Multi-TRP

mTRP-CSI-EnhancementPerBand-r17             SEQUENCE {

maxNumNZP-CSI-RS-r17                        INTEGER (2..8),

cSI-Report-mode-r17                         ENUMERATED {mode1, mode2, both},

supportedComboAcrossCCs-r17                 SEQUENCE (SIZE (1..16)) OF CSI-MultiTRP-SupportedCombinations-r17,

codebookModeNCJT-r17                        ENUMERATED{mode1,mode1And2}

}                                                                                                          OPTIONAL,

-- R1 23-7-1b    Active CSI-RS resources and ports in the presence of multi-TRP CSI

codebookComboParameterMultiTRP-r17          CodebookComboParameterMultiTRP-r17                             OPTIONAL,

-- R1 23-7-1a    Additional CSI report mode 1

mTRP-CSI-additionalCSI-r17                  ENUMERATED{x1,x2}                                              OPTIONAL,

-- R1 23-7-4    Support of Nmax=2 for Multi-TRP CSI

mTRP-CSI-N-Max2-r17                         ENUMERATED {supported}                                         OPTIONAL,

-- R1 23-7-5    CMR sharing

mTRP-CSI-CMR-r17                            ENUMERATED {supported}                                         OPTIONAL,

-- R1 23-8-11    Partial frequency sounding of SRS for non-frequency hopping case

srs-partialFreqSounding-r17                 ENUMERATED {supported}                                         OPTIONAL,

-- R1-24 feature: Extend beamSwitchTiming for FR2-2

beamSwitchTiming-v1710                      SEQUENCE {

scs-480kHz                                  ENUMERATED {sym56, sym112, sym192, sym896, sym1344}        OPTIONAL,

scs-960kHz                                  ENUMERATED {sym112, sym224, sym384, sym1792, sym2688}      OPTIONAL

}                                                                                                          OPTIONAL,

-- R1-24 feature: Extend beamSwitchTiming-r16 for FR2-2

beamSwitchTiming-r17                        SEQUENCE {

scs-480kHz-r17                              ENUMERATED {sym896, sym1344}                               OPTIONAL,

scs-960kHz-r17                              ENUMERATED {sym1792, sym2688}                              OPTIONAL

}                                                                                                          OPTIONAL,

-- R1-24 feature: Extend beamReportTiming for FR2-2

beamReportTiming-v1710                      SEQUENCE {

scs-480kHz-r17                              ENUMERATED {sym56, sym112, sym224}                         OPTIONAL,

scs-960kHz-r17                              ENUMERATED {sym112, sym224, sym448}                        OPTIONAL

}                                                                                                          OPTIONAL,

-- R1-24 feature:    Extend maximum number of RX/TX beam switch DL for FR2-2

maxNumberRxTxBeamSwitchDL-v1710             SEQUENCE {

scs-480kHz-r17                              ENUMERATED {n2, n4, n7}                                    OPTIONAL,

scs-960kHz-r17                              ENUMERATED {n1, n2, n4, n7}                                OPTIONAL

}                                                                                                          OPTIONAL

]],

[[

-- R1-23-1-4a:	Semi-persistent/aperiodic capability value report

srs-PortReportSP-AP-r17                     ENUMERATED {supported}                                         OPTIONAL,

maxNumberRxBeam-v1720                       INTEGER (9..12)                                                OPTIONAL,

-- R1-23-6-5	Support implicit configuration of RS(s) with two TCI states for beam failure detection

sfn-ImplicitRS-twoTCI-r17                   ENUMERATED {supported}                                         OPTIONAL,

-- R1-23-6-6	QCL-TypeD collision handling with CORESET with 2 TCI states

sfn-QCL-TypeD-Collision-twoTCI-r17          ENUMERATED {supported}                                         OPTIONAL,

-- R1-23-7-1c	Basic Features of CSI Enhancement for Multi-TRP - number of CPUs

mTRP-CSI-numCPU-r17                         ENUMERATED {n2, n3, n4}                                        OPTIONAL

]],

[[

supportRepNumPDSCH-TDRA-DCI-1-2-r17         ENUMERATED {n2, n3, n4, n5, n6, n7, n8, n16}                   OPTIONAL

]]

}

DummyG ::=                          SEQUENCE {

maxNumberSSB-CSI-RS-ResourceOneTx   ENUMERATED {n8, n16, n32, n64},

maxNumberSSB-CSI-RS-ResourceTwoTx   ENUMERATED {n0, n4, n8, n16, n32, n64},

supportedCSI-RS-Density             ENUMERATED {one, three, oneAndThree}

}

BeamManagementSSB-CSI-RS ::=        SEQUENCE {

maxNumberSSB-CSI-RS-ResourceOneTx   ENUMERATED {n0, n8, n16, n32, n64},

maxNumberCSI-RS-Resource            ENUMERATED {n0, n4, n8, n16, n32, n64},

maxNumberCSI-RS-ResourceTwoTx       ENUMERATED {n0, n4, n8, n16, n32, n64},

supportedCSI-RS-Density             ENUMERATED {one, three, oneAndThree}                                       OPTIONAL,

maxNumberAperiodicCSI-RS-Resource   ENUMERATED {n0, n1, n4, n8, n16, n32, n64}

}

DummyH ::=                          SEQUENCE {

burstLength                         INTEGER (1..2),

maxSimultaneousResourceSetsPerCC    INTEGER (1..8),

maxConfiguredResourceSetsPerCC      INTEGER (1..64),

maxConfiguredResourceSetsAllCC      INTEGER (1..128)

}

CSI-RS-ForTracking ::=              SEQUENCE {

maxBurstLength                      INTEGER (1..2),

maxSimultaneousResourceSetsPerCC    INTEGER (1..8),

maxConfiguredResourceSetsPerCC      INTEGER (1..64),

maxConfiguredResourceSetsAllCC      INTEGER (1..256)

}

CSI-RS-IM-ReceptionForFeedback ::=              SEQUENCE {

maxConfigNumberNZP-CSI-RS-PerCC                 INTEGER (1..64),

maxConfigNumberPortsAcrossNZP-CSI-RS-PerCC      INTEGER (2..256),

maxConfigNumberCSI-IM-PerCC                     ENUMERATED {n1, n2, n4, n8, n16, n32},

maxNumberSimultaneousNZP-CSI-RS-PerCC           INTEGER (1..64),

totalNumberPortsSimultaneousNZP-CSI-RS-PerCC    INTEGER (2..256)

}

CSI-RS-ProcFrameworkForSRS ::=                  SEQUENCE {

maxNumberPeriodicSRS-AssocCSI-RS-PerBWP         INTEGER (1..4),

maxNumberAperiodicSRS-AssocCSI-RS-PerBWP        INTEGER (1..4),

maxNumberSP-SRS-AssocCSI-RS-PerBWP              INTEGER (0..4),

simultaneousSRS-AssocCSI-RS-PerCC               INTEGER (1..8)

}

CSI-ReportFramework ::=                         SEQUENCE {

maxNumberPeriodicCSI-PerBWP-ForCSI-Report       INTEGER (1..4),

maxNumberAperiodicCSI-PerBWP-ForCSI-Report      INTEGER (1..4),

maxNumberSemiPersistentCSI-PerBWP-ForCSI-Report INTEGER (0..4),

maxNumberPeriodicCSI-PerBWP-ForBeamReport       INTEGER (1..4),

maxNumberAperiodicCSI-PerBWP-ForBeamReport      INTEGER (1..4),

maxNumberAperiodicCSI-triggeringStatePerCC      ENUMERATED {n3, n7, n15, n31, n63, n128},

maxNumberSemiPersistentCSI-PerBWP-ForBeamReport INTEGER (0..4),

simultaneousCSI-ReportsPerCC                    INTEGER (1..8)

}

CSI-ReportFrameworkExt-r16 ::=                      SEQUENCE {

maxNumberAperiodicCSI-PerBWP-ForCSI-ReportExt-r16   INTEGER (5..8)

}

PTRS-DensityRecommendationDL ::=    SEQUENCE {

frequencyDensity1                   INTEGER (1..276),

frequencyDensity2                   INTEGER (1..276),

timeDensity1                        INTEGER (0..29),

timeDensity2                        INTEGER (0..29),

timeDensity3                        INTEGER (0..29)

}

PTRS-DensityRecommendationUL ::=    SEQUENCE {

frequencyDensity1                   INTEGER (1..276),

frequencyDensity2                   INTEGER (1..276),

timeDensity1                        INTEGER (0..29),

timeDensity2                        INTEGER (0..29),

timeDensity3                        INTEGER (0..29),

sampleDensity1                      INTEGER (1..276),

sampleDensity2                      INTEGER (1..276),

sampleDensity3                      INTEGER (1..276),

sampleDensity4                      INTEGER (1..276),

sampleDensity5                      INTEGER (1..276)

}

SpatialRelations ::=                    SEQUENCE {

maxNumberConfiguredSpatialRelations     ENUMERATED {n4, n8, n16, n32, n64, n96},

maxNumberActiveSpatialRelations         ENUMERATED {n1, n2, n4, n8, n14},

additionalActiveSpatialRelationPUCCH    ENUMERATED {supported}                              OPTIONAL,

maxNumberDL-RS-QCL-TypeD                ENUMERATED {n1, n2, n4, n8, n14}

}

DummyI ::=               SEQUENCE {

supportedSRS-TxPortSwitch           ENUMERATED {t1r2, t1r4, t2r4, t1r4-t2r4, tr-equal},

txSwitchImpactToRx                  ENUMERATED {true}                                       OPTIONAL

}

CSI-MultiTRP-SupportedCombinations-r17 ::= SEQUENCE {

maxNumTx-Ports-r17                         ENUMERATED {n2, n4, n8, n12, n16, n24, n32},

maxTotalNumCMR-r17                         INTEGER (2..64),

maxTotalNumTx-PortsNZP-CSI-RS-r17          INTEGER (2..256)

}

-- TAG-MIMO-PARAMETERSPERBAND-STOP

-- ASN1STOP

MIMO-ParametersPerBand field descriptions

codebookParametersPerBand For a given frequency band, this field this field indicates the alternative list of SupportedCSI-RS-Resource supported for each codebook type. The supported CSI-RS resources indicated by this field are referred by codebookParametersperBC in CA-ParametersNR to indicate the supported CSI-RS resource per band combination.

csi-RS-IM-ReceptionForFeedback/ csi-RS-ProcFrameworkForSRS/ csi-ReportFramework CSI related capabilities which the UE supports on each of the carriers operated on this band. If the network configures the UE with serving cells on both FR1 and FR2 bands these values may be further limited by the corresponding fields in fr1-fr2-Add-UE-NR-Capabilities.

supportNewDMRS-Port Presence of this field set to supported1, supported2 or supported3 indicates that the UE supports the new DMRS port entry {0,2,3}.

#### –	ModulationOrder

The IE ModulationOrder is used to convey the maximum supported modulation order.

ModulationOrder information element

-- ASN1START

-- TAG-MODULATIONORDER-START

ModulationOrder ::= ENUMERATED {bpsk-halfpi, bpsk, qpsk, qam16, qam64, qam256}

-- TAG-MODULATIONORDER-STOP

-- ASN1STOP

#### –	MRDC-Parameters

The IE MRDC-Parameters contains the band combination parameters specific to MR-DC for a given MR-DC band combination.

MRDC-Parameters information element

-- ASN1START

-- TAG-MRDC-PARAMETERS-START

MRDC-Parameters ::= SEQUENCE {

singleUL-Transmission               ENUMERATED {supported}              OPTIONAL,

dynamicPowerSharingENDC             ENUMERATED {supported}              OPTIONAL,

tdm-Pattern                         ENUMERATED {supported}              OPTIONAL,

ul-SharingEUTRA-NR                  ENUMERATED {tdm, fdm, both}         OPTIONAL,

ul-SwitchingTimeEUTRA-NR            ENUMERATED {type1, type2}           OPTIONAL,

simultaneousRxTxInterBandENDC       ENUMERATED {supported}              OPTIONAL,

asyncIntraBandENDC                  ENUMERATED {supported}              OPTIONAL,

...,

[[

dualPA-Architecture                 ENUMERATED {supported}              OPTIONAL,

intraBandENDC-Support               ENUMERATED {non-contiguous, both}   OPTIONAL,

ul-TimingAlignmentEUTRA-NR          ENUMERATED {required}               OPTIONAL

]]

}

MRDC-Parameters-v1580 ::= SEQUENCE {

dynamicPowerSharingNEDC             ENUMERATED {supported}              OPTIONAL

}

MRDC-Parameters-v1590 ::=	SEQUENCE {

interBandContiguousMRDC             ENUMERATED {supported}              OPTIONAL

}

MRDC-Parameters-v15g0 ::=   SEQUENCE {

simultaneousRxTxInterBandENDCPerBandPair   SimultaneousRxTxPerBandPair  OPTIONAL

}

MRDC-Parameters-v1620 ::=    SEQUENCE {

maxUplinkDutyCycle-interBandENDC-TDD-PC2-r16    SEQUENCE{

eutra-TDD-Config0-r16    ENUMERATED {n20, n40, n50, n60, n70, n80, n90, n100}    OPTIONAL,

eutra-TDD-Config1-r16    ENUMERATED {n20, n40, n50, n60, n70, n80, n90, n100}    OPTIONAL,

eutra-TDD-Config2-r16    ENUMERATED {n20, n40, n50, n60, n70, n80, n90, n100}    OPTIONAL,

eutra-TDD-Config3-r16    ENUMERATED {n20, n40, n50, n60, n70, n80, n90, n100}    OPTIONAL,

eutra-TDD-Config4-r16    ENUMERATED {n20, n40, n50, n60, n70, n80, n90, n100}    OPTIONAL,

eutra-TDD-Config5-r16    ENUMERATED {n20, n40, n50, n60, n70, n80, n90, n100}    OPTIONAL,

eutra-TDD-Config6-r16    ENUMERATED {n20, n40, n50, n60, n70, n80, n90, n100}    OPTIONAL

}                                                                                    OPTIONAL,

-- R1 18-2 Single UL TX operation for TDD PCell in EN-DC

tdm-restrictionTDD-endc-r16          ENUMERATED {supported}                          OPTIONAL,

-- R1 18-2a Single UL TX operation for FDD PCell in EN-DC

tdm-restrictionFDD-endc-r16          ENUMERATED {supported}                          OPTIONAL,

--  R1 18-2b Support of HARQ-offset for SUO case1 in EN-DC with LTE TDD PCell for type 1 UE

singleUL-HARQ-offsetTDD-PCell-r16    ENUMERATED {supported}                          OPTIONAL,

--  R1 18-3 Dual Tx transmission for EN-DC with FDD PCell(TDM pattern for dual Tx UE)

tdm-restrictionDualTX-FDD-endc-r16   ENUMERATED {supported}                          OPTIONAL

}

MRDC-Parameters-v1630 ::= SEQUENCE {

-- R4 2-20 Maximum uplink duty cycle for FDD+TDD EN-DC power class 2

maxUplinkDutyCycle-interBandENDC-FDD-TDD-PC2-r16  SEQUENCE {

maxUplinkDutyCycle-FDD-TDD-EN-DC1-r16             ENUMERATED {n30, n40, n50, n60, n70, n80, n90, n100}    OPTIONAL,

maxUplinkDutyCycle-FDD-TDD-EN-DC2-r16             ENUMERATED {n30, n40, n50, n60, n70, n80, n90, n100}    OPTIONAL

}                                                                                                             OPTIONAL,

-- R4 2-19 FDD-FDD or TDD-TDD inter-band MR-DC with overlapping or partially overlapping DL spectrum

interBandMRDC-WithOverlapDL-Bands-r16       ENUMERATED {supported}                   OPTIONAL

}

MRDC-Parameters-v1700 ::=	SEQUENCE {

condPSCellAdditionENDC-r17                  ENUMERATED {supported}                   OPTIONAL,

scg-ActivationDeactivationENDC-r17          ENUMERATED {supported}                   OPTIONAL,

scg-ActivationDeactivationResumeENDC-r17    ENUMERATED {supported}                   OPTIONAL

}

-- TAG-MRDC-PARAMETERS-STOP

-- ASN1STOP

#### –	NRDC-Parameters

The IE NRDC-Parameters contains parameters specific to NR-DC, i.e., which are not applicable to NR SA.

NRDC-Parameters information element

-- ASN1START

-- TAG-NRDC-PARAMETERS-START

NRDC-Parameters ::=                 SEQUENCE {

measAndMobParametersNRDC            MeasAndMobParametersMRDC                    OPTIONAL,

generalParametersNRDC               GeneralParametersMRDC-XDD-Diff              OPTIONAL,

fdd-Add-UE-NRDC-Capabilities        UE-MRDC-CapabilityAddXDD-Mode               OPTIONAL,

tdd-Add-UE-NRDC-Capabilities        UE-MRDC-CapabilityAddXDD-Mode               OPTIONAL,

fr1-Add-UE-NRDC-Capabilities        UE-MRDC-CapabilityAddFRX-Mode               OPTIONAL,

fr2-Add-UE-NRDC-Capabilities        UE-MRDC-CapabilityAddFRX-Mode               OPTIONAL,

dummy2                              OCTET STRING                                OPTIONAL,

dummy                               SEQUENCE {}                                 OPTIONAL

}

NRDC-Parameters-v1570 ::=           SEQUENCE {

sfn-SyncNRDC                        ENUMERATED {supported}                      OPTIONAL

}

NRDC-Parameters-v15c0 ::=           SEQUENCE {

pdcp-DuplicationSplitSRB            ENUMERATED {supported}                      OPTIONAL,

pdcp-DuplicationSplitDRB            ENUMERATED {supported}                      OPTIONAL

}

NRDC-Parameters-v1610 ::=           SEQUENCE {

measAndMobParametersNRDC-v1610      MeasAndMobParametersMRDC-v1610              OPTIONAL

}

NRDC-Parameters-v1700   ::=         SEQUENCE {

f1c-OverNR-RRC-r17                  ENUMERATED {supported}                      OPTIONAL,

measAndMobParametersNRDC-v1700      MeasAndMobParametersMRDC-v1700

}

-- TAG-NRDC-PARAMETERS-STOP

-- ASN1STOP

#### –	NTN-Parameters

The IE NTN-Parameters is used to convey the subset of UE Radio Access Capability Parameters that apply to NTN access when there is a difference compared to TN access.

NTN-Parameters information element

-- ASN1START

-- TAG-NTN-PARAMETERS-START

NTN-Parameters-r17 ::= SEQUENCE {

inactiveStateNTN-r17                ENUMERATED {supported}                                OPTIONAL,

ra-SDT-NTN-r17                      ENUMERATED {supported}                                OPTIONAL,

srb-SDT-NTN-r17                     ENUMERATED {supported}                                OPTIONAL,

measAndMobParametersNTN-r17         MeasAndMobParameters                                  OPTIONAL,

mac-ParametersNTN-r17               MAC-Parameters                                        OPTIONAL,

phy-ParametersNTN-r17               Phy-Parameters                                        OPTIONAL,

fdd-Add-UE-NR-CapabilitiesNTN-r17   UE-NR-CapabilityAddXDD-Mode                           OPTIONAL,

fr1-Add-UE-NR-CapabilitiesNTN-r17   UE-NR-CapabilityAddFRX-Mode                           OPTIONAL,

ue-BasedPerfMeas-ParametersNTN-r17  UE-BasedPerfMeas-Parameters-r16                       OPTIONAL,

son-ParametersNTN-r17               SON-Parameters-r16                                    OPTIONAL

}

-- TAG-NTN-PARAMETERS-STOP

-- ASN1STOP

NTN-Parameters field descriptions

fdd-Add-UE-NR-CapabilitiesNTN NTN related capabilities which the UE supports in NTN differently than in TN. If absent, fdd-Add-UE-NR-Capabilities applies to NTN.

fr1-Add-UE-NR-CapabilitiesNTN NTN related capabilities which the UE supports in NTN differently than in TN. If absent, fr1-Add-UE-NR-Capabilities applies to NTN.

mac-ParametersNTN NTN related capabilities which the UE supports in NTN differently than in TN. If absent, mac-Parameters applies to NTN.

measAndMobParametersNTN NTN related capabilities which the UE supports in NTN differently than in TN. If absent, measAndMobParameters applies to NTN.

phy-ParametersNTN NTN related capabilities which the UE supports in NTN differently than in TN. If absent, phy-Parameters applies to NTN.

son-ParametersNTN NTN related capabilities which the UE supports in NTN differently than in TN. If absent, son-Parameters-r16 applies to NTN.

ue-BasedPerfMeas-ParametersNTN NTN related capabilities which the UE supports in NTN differently than in TN. If absent, ue-BasedPerfMeas-Parameters-r16 applies to NTN.

#### –	OLPC-SRS-Pos

The IE OLPC-SRS-Pos is used to convey OLPC SRS positioning related parameters specific for a certain band.

OLPC-SRS-Pos information element

-- ASN1START

-- TAG-OLPC-SRS-POS-START

OLPC-SRS-Pos-r16 ::=        SEQUENCE {

olpc-SRS-PosBasedOnPRS-Serving-r16         ENUMERATED {supported}               OPTIONAL,

olpc-SRS-PosBasedOnSSB-Neigh-r16           ENUMERATED {supported}               OPTIONAL,

olpc-SRS-PosBasedOnPRS-Neigh-r16           ENUMERATED {supported}               OPTIONAL,

maxNumberPathLossEstimatePerServing-r16    ENUMERATED {n1, n4, n8, n16}         OPTIONAL

}

--TAG-OLPC-SRS-POS-STOP

-- ASN1STOP

#### –	PDCP-Parameters

The IE PDCP-Parameters is used to convey capabilities related to PDCP.

PDCP-Parameters information element

-- ASN1START

-- TAG-PDCP-PARAMETERS-START

PDCP-Parameters ::=         SEQUENCE {

supportedROHC-Profiles      SEQUENCE {

profile0x0000               BOOLEAN,

profile0x0001               BOOLEAN,

profile0x0002               BOOLEAN,

profile0x0003               BOOLEAN,

profile0x0004               BOOLEAN,

profile0x0006               BOOLEAN,

profile0x0101               BOOLEAN,

profile0x0102               BOOLEAN,

profile0x0103               BOOLEAN,

profile0x0104               BOOLEAN

},

maxNumberROHC-ContextSessions       ENUMERATED {cs2, cs4, cs8, cs12, cs16, cs24, cs32, cs48, cs64,

cs128, cs256, cs512, cs1024, cs16384, spare2, spare1},

uplinkOnlyROHC-Profiles             ENUMERATED {supported}      OPTIONAL,

continueROHC-Context                ENUMERATED {supported}      OPTIONAL,

outOfOrderDelivery                  ENUMERATED {supported}      OPTIONAL,

shortSN                             ENUMERATED {supported}      OPTIONAL,

pdcp-DuplicationSRB                 ENUMERATED {supported}      OPTIONAL,

pdcp-DuplicationMCG-OrSCG-DRB       ENUMERATED {supported}      OPTIONAL,

...,

[[

drb-IAB-r16                         ENUMERATED {supported}      OPTIONAL,

non-DRB-IAB-r16                     ENUMERATED {supported}      OPTIONAL,

extendedDiscardTimer-r16            ENUMERATED {supported}      OPTIONAL,

continueEHC-Context-r16             ENUMERATED {supported}      OPTIONAL,

ehc-r16                             ENUMERATED {supported}      OPTIONAL,

maxNumberEHC-Contexts-r16           ENUMERATED {cs2, cs4, cs8, cs16, cs32, cs64, cs128, cs256, cs512,

cs1024, cs2048, cs4096, cs8192, cs16384, cs32768, cs65536}    OPTIONAL,

jointEHC-ROHC-Config-r16            ENUMERATED {supported}      OPTIONAL,

pdcp-DuplicationMoreThanTwoRLC-r16  ENUMERATED {supported}      OPTIONAL

]],

[[

longSN-RedCap-r17                   ENUMERATED {supported}      OPTIONAL,

udc-r17                             SEQUENCE {

standardDictionary-r17              ENUMERATED {supported}  OPTIONAL,

operatorDictionary-r17              SEQUENCE {

versionOfDictionary-r17             INTEGER (0..15),

associatedPLMN-ID-r17               PLMN-Identity

}                                                           OPTIONAL,

continueUDC-r17                     ENUMERATED {supported}  OPTIONAL,

supportOfBufferSize-r17             ENUMERATED {kbyte4, kbyte8}  OPTIONAL

}                                                               OPTIONAL

]]

}

-- TAG-PDCP-PARAMETERS-STOP

-- ASN1STOP

#### –	PDCP-ParametersMRDC

The IE PDCP-ParametersMRDC is used to convey PDCP related capabilities for MR-DC.

PDCP-ParametersMRDC information element

-- ASN1START

-- TAG-PDCP-PARAMETERSMRDC-START

PDCP-ParametersMRDC ::=                 SEQUENCE {

pdcp-DuplicationSplitSRB                ENUMERATED {supported}      OPTIONAL,

pdcp-DuplicationSplitDRB                ENUMERATED {supported}      OPTIONAL

}

PDCP-ParametersMRDC-v1610 ::= SEQUENCE {

scg-DRB-NR-IAB-r16                  ENUMERATED {supported}          OPTIONAL

}

-- TAG-PDCP-PARAMETERSMRDC-STOP

-- ASN1STOP

#### –	Phy-Parameters

The IE Phy-Parameters is used to convey the physical layer capabilities.

Phy-Parameters information element

-- ASN1START

-- TAG-PHY-PARAMETERS-START

Phy-Parameters ::=                  SEQUENCE {

phy-ParametersCommon                Phy-ParametersCommon                        OPTIONAL,

phy-ParametersXDD-Diff              Phy-ParametersXDD-Diff                      OPTIONAL,

phy-ParametersFRX-Diff              Phy-ParametersFRX-Diff                      OPTIONAL,

phy-ParametersFR1                   Phy-ParametersFR1                           OPTIONAL,

phy-ParametersFR2                   Phy-ParametersFR2                           OPTIONAL

}

Phy-Parameters-v16a0 ::=            SEQUENCE {

phy-ParametersCommon-v16a0          Phy-ParametersCommon-v16a0                  OPTIONAL

}

Phy-ParametersCommon ::=            SEQUENCE {

csi-RS-CFRA-ForHO                   ENUMERATED {supported}                      OPTIONAL,

dynamicPRB-BundlingDL               ENUMERATED {supported}                      OPTIONAL,

sp-CSI-ReportPUCCH                  ENUMERATED {supported}                      OPTIONAL,

sp-CSI-ReportPUSCH                  ENUMERATED {supported}                      OPTIONAL,

nzp-CSI-RS-IntefMgmt                ENUMERATED {supported}                      OPTIONAL,

type2-SP-CSI-Feedback-LongPUCCH     ENUMERATED {supported}                      OPTIONAL,

precoderGranularityCORESET          ENUMERATED {supported}                      OPTIONAL,

dynamicHARQ-ACK-Codebook            ENUMERATED {supported}                      OPTIONAL,

semiStaticHARQ-ACK-Codebook         ENUMERATED {supported}                      OPTIONAL,

spatialBundlingHARQ-ACK             ENUMERATED {supported}                      OPTIONAL,

dynamicBetaOffsetInd-HARQ-ACK-CSI   ENUMERATED {supported}                      OPTIONAL,

pucch-Repetition-F1-3-4             ENUMERATED {supported}                      OPTIONAL,

ra-Type0-PUSCH                      ENUMERATED {supported}                      OPTIONAL,

dynamicSwitchRA-Type0-1-PDSCH       ENUMERATED {supported}                      OPTIONAL,

dynamicSwitchRA-Type0-1-PUSCH       ENUMERATED {supported}                      OPTIONAL,

pdsch-MappingTypeA                  ENUMERATED {supported}                      OPTIONAL,

pdsch-MappingTypeB                  ENUMERATED {supported}                      OPTIONAL,

interleavingVRB-ToPRB-PDSCH         ENUMERATED {supported}                      OPTIONAL,

interSlotFreqHopping-PUSCH          ENUMERATED {supported}                      OPTIONAL,

type1-PUSCH-RepetitionMultiSlots    ENUMERATED {supported}                      OPTIONAL,

type2-PUSCH-RepetitionMultiSlots    ENUMERATED {supported}                      OPTIONAL,

pusch-RepetitionMultiSlots          ENUMERATED {supported}                      OPTIONAL,

pdsch-RepetitionMultiSlots          ENUMERATED {supported}                      OPTIONAL,

downlinkSPS                         ENUMERATED {supported}                      OPTIONAL,

configuredUL-GrantType1             ENUMERATED {supported}                      OPTIONAL,

configuredUL-GrantType2             ENUMERATED {supported}                      OPTIONAL,

pre-EmptIndication-DL               ENUMERATED {supported}                      OPTIONAL,

cbg-TransIndication-DL              ENUMERATED {supported}                      OPTIONAL,

cbg-TransIndication-UL              ENUMERATED {supported}                      OPTIONAL,

cbg-FlushIndication-DL              ENUMERATED {supported}                      OPTIONAL,

dynamicHARQ-ACK-CodeB-CBG-Retx-DL   ENUMERATED {supported}                      OPTIONAL,

rateMatchingResrcSetSemi-Static     ENUMERATED {supported}                      OPTIONAL,

rateMatchingResrcSetDynamic         ENUMERATED {supported}                      OPTIONAL,

bwp-SwitchingDelay                  ENUMERATED {type1, type2}                   OPTIONAL,

...,

[[

dummy                               ENUMERATED {supported}                      OPTIONAL

]],

[[

maxNumberSearchSpaces               ENUMERATED {n10}                            OPTIONAL,

rateMatchingCtrlResrcSetDynamic     ENUMERATED {supported}                      OPTIONAL,

maxLayersMIMO-Indication            ENUMERATED {supported}                      OPTIONAL

]],

[[

spCellPlacement                             CarrierAggregationVariant           OPTIONAL

]],

[[

-- R1 9-1: Basic channel structure and procedure of 2-step RACH

twoStepRACH-r16                             ENUMERATED {supported}              OPTIONAL,

-- R1 11-1: Monitoring DCI format 1_2 and DCI format 0_2

dci-Format1-2And0-2-r16                     ENUMERATED {supported}              OPTIONAL,

-- R1 11-1a: Monitoring both DCI format 0_1/1_1 and DCI format 0_2/1_2 in the same search space

monitoringDCI-SameSearchSpace-r16           ENUMERATED {supported}              OPTIONAL,

-- R1 11-10: Type 2 configured grant release by DCI format 0_1

type2-CG-ReleaseDCI-0-1-r16                 ENUMERATED {supported}              OPTIONAL,

-- R1 11-11: Type 2 configured grant release by DCI format 0_2

type2-CG-ReleaseDCI-0-2-r16                 ENUMERATED {supported}              OPTIONAL,

-- R1 12-3: SPS release by DCI format 1_1

sps-ReleaseDCI-1-1-r16                      ENUMERATED {supported}              OPTIONAL,

-- R1 12-3a: SPS release by DCI format 1_2

sps-ReleaseDCI-1-2-r16                      ENUMERATED {supported}              OPTIONAL,

-- R1 14-8: CSI trigger states containing non-active BWP

csi-TriggerStateNon-ActiveBWP-r16           ENUMERATED {supported}              OPTIONAL,

-- R1 20-2: Support up to 4 SMTCs configured for an IAB node MT per frequency location, including IAB-specific SMTC window periodicities

separateSMTC-InterIAB-Support-r16           ENUMERATED {supported}              OPTIONAL,

-- R1 20-3: Support RACH configuration separately from the RACH configuration for UE access, including new IAB-specific offset and scaling factors

separateRACH-IAB-Support-r16                ENUMERATED {supported}              OPTIONAL,

-- R1 20-5a: Support semi-static configuration/indication of UL-Flexible-DL slot formats for IAB-MT resources

ul-flexibleDL-SlotFormatSemiStatic-IAB-r16  ENUMERATED {supported}              OPTIONAL,

-- R1 20-5b: Support dynamic indication of UL-Flexible-DL slot formats for IAB-MT resources

ul-flexibleDL-SlotFormatDynamics-IAB-r16    ENUMERATED {supported}              OPTIONAL,

dft-S-OFDM-WaveformUL-IAB-r16               ENUMERATED {supported}              OPTIONAL,

-- R1 20-6: Support DCI Format 2_5 based indication of soft resource availability to an IAB node

dci-25-AI-RNTI-Support-IAB-r16              ENUMERATED {supported}              OPTIONAL,

-- R1 20-7: Support T_delta reception.

t-DeltaReceptionSupport-IAB-r16             ENUMERATED {supported}              OPTIONAL,

-- R1 20-8: Support of Desired guard symbol reporting and provided guard symbok reception.

guardSymbolReportReception-IAB-r16          ENUMERATED {supported}              OPTIONAL,

-- R1 18-8 HARQ-ACK codebook type and spatial bundling per PUCCH group

harqACK-CB-SpatialBundlingPUCCH-Group-r16   ENUMERATED {supported}              OPTIONAL,

-- R1 19-2: Cross Slot Scheduling

crossSlotScheduling-r16                     SEQUENCE {

non-SharedSpectrumChAccess-r16              ENUMERATED {supported}          OPTIONAL,

sharedSpectrumChAccess-r16                  ENUMERATED {supported}          OPTIONAL

}                                                                               OPTIONAL,

maxNumberSRS-PosPathLossEstimateAllServingCells-r16  ENUMERATED {n1, n4, n8, n16}         OPTIONAL,

extendedCG-Periodicities-r16                ENUMERATED {supported}              OPTIONAL,

extendedSPS-Periodicities-r16               ENUMERATED {supported}              OPTIONAL,

codebookVariantsList-r16                    CodebookVariantsList-r16            OPTIONAL,

-- R1 11-6: PUSCH repetition Type A

pusch-RepetitionTypeA-r16                   SEQUENCE {

sharedSpectrumChAccess-r16                  ENUMERATED {supported}          OPTIONAL,

non-SharedSpectrumChAccess-r16              ENUMERATED {supported}          OPTIONAL

}                                                                               OPTIONAL,

-- R1 11-4b: DL priority indication in DCI with mixed DCI formats

dci-DL-PriorityIndicator-r16                ENUMERATED {supported}              OPTIONAL,

-- R1 12-1a: UL priority indication in DCI with mixed DCI formats

dci-UL-PriorityIndicator-r16                ENUMERATED {supported}              OPTIONAL,

-- R1 16-1e: Maximum number of configured pathloss reference RSs for PUSCH/PUCCH/SRS by RRC for MAC-CE based pathloss reference RS update

maxNumberPathlossRS-Update-r16              ENUMERATED {n4, n8, n16, n32, n64}  OPTIONAL,

-- R1 18-9: Usage of the PDSCH starting time for HARQ-ACK type 2 codebook

type2-HARQ-ACK-Codebook-r16                 ENUMERATED {supported}              OPTIONAL,

-- R1 16-1g-1: Resources for beam management, pathloss measurement, BFD, RLM and new beam identification across frequency ranges

maxTotalResourcesForAcrossFreqRanges-r16    SEQUENCE {

maxNumberResWithinSlotAcrossCC-AcrossFR-r16 ENUMERATED {n2, n4, n8, n12, n16, n32, n64, n128}        OPTIONAL,

maxNumberResAcrossCC-AcrossFR-r16           ENUMERATED {n2, n4, n8, n12, n16, n32, n40, n48, n64, n72, n80, n96, n128, n256}

OPTIONAL

}                                                                               OPTIONAL,

-- R1 16-2a-4: HARQ-ACK for multi-DCI based multi-TRP - separate

harqACK-separateMultiDCI-MultiTRP-r16       SEQUENCE {

maxNumberLongPUCCHs-r16                         ENUMERATED {longAndLong, longAndShort, shortAndShort}    OPTIONAL

}                                                                               OPTIONAL,

-- R1 16-2a-4: HARQ-ACK for multi-DCI based multi-TRP - joint

harqACK-jointMultiDCI-MultiTRP-r16          ENUMERATED {supported}              OPTIONAL,

-- R4 9-1: BWP switching on multiple CCs RRM requirements

bwp-SwitchingMultiCCs-r16                   CHOICE {

type1-r16                                   ENUMERATED {us100, us200},

type2-r16                                   ENUMERATED {us200, us400, us800, us1000}

}                                                                               OPTIONAL

]],

[[

targetSMTC-SCG-r16                          ENUMERATED {supported}              OPTIONAL,

supportRepetitionZeroOffsetRV-r16           ENUMERATED {supported}              OPTIONAL,

-- R1 11-12: in-order CBG-based re-transmission

cbg-TransInOrderPUSCH-UL-r16                ENUMERATED {supported}              OPTIONAL

]],

[[

-- R4 6-3: Dormant BWP switching on multiple CCs RRM requirements

bwp-SwitchingMultiDormancyCCs-r16           CHOICE {

type1-r16                                   ENUMERATED {us100, us200},

type2-r16                                   ENUMERATED {us200, us400, us800, us1000}

}                                                                               OPTIONAL,

-- R1 16-2a-8: Indicates that retransmission scheduled by a different CORESETPoolIndex for multi-DCI multi-TRP is not supported.

supportRetx-Diff-CoresetPool-Multi-DCI-TRP-r16               ENUMERATED {notSupported}          OPTIONAL,

-- R1 22-10: Support of pdcch-MonitoringAnyOccasionsWithSpanGap in case of cross-carrier scheduling with different SCSs

pdcch-MonitoringAnyOccasionsWithSpanGapCrossCarrierSch-r16   ENUMERATED {mode2, mode3}          OPTIONAL

]],

[[

-- R1 16-1j-1: Support of 2 port CSI-RS for new beam identification

newBeamIdentifications2PortCSI-RS-r16       ENUMERATED {supported}              OPTIONAL,

-- R1 16-1j-2: Support of 2 port CSI-RS for pathloss estimation

pathlossEstimation2PortCSI-RS-r16           ENUMERATED {supported}              OPTIONAL

]],

[[

mux-HARQ-ACK-withoutPUCCH-onPUSCH-r16       ENUMERATED {supported}              OPTIONAL

]],

[[

-- R1 31-1: Support of Desired Guard Symbol reporting and provided guard symbol reception.

guardSymbolReportReception-IAB-r17          ENUMERATED {supported}              OPTIONAL,

-- R1 31-2: support of restricted IAB-DU beam reception

restricted-IAB-DU-BeamReception-r17         ENUMERATED {supported}              OPTIONAL,

-- R1 31-3: support of recommended IAB-MT beam transmission for DL and UL beam

recommended-IAB-MT-BeamTransmission-r17     ENUMERATED {supported}              OPTIONAL,

-- R1 31-4: support of case 6 timing alignment indication reception

case6-TimingAlignmentReception-IAB-r17      ENUMERATED {supported}              OPTIONAL,

-- R1 31-5: support of case 7 timing offset indication reception and case 7 timing at parent-node indication reception

case7-TimingAlignmentReception-IAB-r17      ENUMERATED {supported}              OPTIONAL,

-- R1 31-6: support of desired DL Tx power adjustment reporting and DL Tx power adjustment reception

dl-tx-PowerAdjustment-IAB-r17               ENUMERATED {supported}              OPTIONAL,

-- R1 31-7: support of desired IAB-MT PSD range reporting

desired-ul-tx-PowerAdjustment-r17           ENUMERATED {supported}              OPTIONAL,

-- R1 31-8: support of monitoring DCI Format 2_5 scrambled by AI-RNTI for indication of FDM soft resource availability to an IAB node

fdm-SoftResourceAvailability-DynamicIndication-r17  ENUMERATED{supported}       OPTIONAL,

-- R1 31-10: Support of updated T_delta range reception

updated-T-DeltaRangeRecption-r17            ENUMERATED{supported}               OPTIONAL,

-- R1 30-5: Support slot based dynamic PUCCH repetition indication for PUCCH formats 0/1/2/3/4

slotBasedDynamicPUCCH-Rep-r17               ENUMERATED {supported}              OPTIONAL,

-- R1 25-1: Support of HARQ-ACK deferral in case of TDD collision

sps-HARQ-ACK-Deferral-r17                   SEQUENCE {

non-SharedSpectrumChAccess-r17              ENUMERATED {supported}          OPTIONAL,

sharedSpectrumChAccess-r17                  ENUMERATED {supported}          OPTIONAL

}                                                                               OPTIONAL,

-- R1 23-1-1k Maximum number of configured CC lists (per UE)

unifiedJointTCI-commonUpdate-r17            INTEGER (1..4)                      OPTIONAL,

-- R1 23-2-1c PDCCH repetition with a single span of three contiguous OFDM symbols that is within the first four OFDM symbols in a slot

mTRP-PDCCH-singleSpan-r17                   ENUMERATED {supported}              OPTIONAL,

-- R1 27-23: Support of more than one activated PRS processing windows across all active DL BWPs

supportedActivatedPRS-ProcessingWindow-r17  ENUMERATED {n2, n3, n4}             OPTIONAL,

cg-TimeDomainAllocationExtension-r17        ENUMERATED {supported}              OPTIONAL

]],

[[

-- R1 25-20: Propagation delay compensation based on legacy TA procedure for TN and licensed

ta-BasedPDC-TN-NonSharedSpectrumChAccess-r17 ENUMERATED {supported}             OPTIONAL,

-- R1 31-11: Directional Collision Handling in DC operation

directionalCollisionDC-IAB-r17              ENUMERATED {supported}              OPTIONAL

]],

[[

dummy1                                      ENUMERATED {supported}              OPTIONAL,

dummy2                                      ENUMERATED {supported}              OPTIONAL,

dummy3                                      ENUMERATED {supported}              OPTIONAL,

dummy4                                      ENUMERATED {supported}              OPTIONAL,

srs-AdditionalRepetition-r17                ENUMERATED {supported}              OPTIONAL,

pusch-Repetition-CG-SDT-r17                 ENUMERATED {supported}              OPTIONAL

]]

}

Phy-ParametersCommon-v16a0 ::=                  SEQUENCE {

srs-PeriodicityAndOffsetExt-r16                 ENUMERATED {supported}          OPTIONAL

}

Phy-ParametersXDD-Diff ::=          SEQUENCE {

dynamicSFI                          ENUMERATED {supported}                      OPTIONAL,

twoPUCCH-F0-2-ConsecSymbols         ENUMERATED {supported}                      OPTIONAL,

twoDifferentTPC-Loop-PUSCH          ENUMERATED {supported}                      OPTIONAL,

twoDifferentTPC-Loop-PUCCH          ENUMERATED {supported}                      OPTIONAL,

...,

[[

dl-SchedulingOffset-PDSCH-TypeA     ENUMERATED {supported}                      OPTIONAL,

dl-SchedulingOffset-PDSCH-TypeB     ENUMERATED {supported}                      OPTIONAL,

ul-SchedulingOffset                 ENUMERATED {supported}                      OPTIONAL

]]

}

Phy-ParametersFRX-Diff ::=                  SEQUENCE {

dynamicSFI                                  ENUMERATED {supported}                      OPTIONAL,

dummy1                                      BIT STRING (SIZE (2))                       OPTIONAL,

twoFL-DMRS                                  BIT STRING (SIZE (2))                       OPTIONAL,

dummy2                                      BIT STRING (SIZE (2))                       OPTIONAL,

dummy3                                      BIT STRING (SIZE (2))                       OPTIONAL,

supportedDMRS-TypeDL                        ENUMERATED {type1, type1And2}               OPTIONAL,

supportedDMRS-TypeUL                        ENUMERATED {type1, type1And2}               OPTIONAL,

semiOpenLoopCSI                             ENUMERATED {supported}                      OPTIONAL,

csi-ReportWithoutPMI                        ENUMERATED {supported}                      OPTIONAL,

csi-ReportWithoutCQI                        ENUMERATED {supported}                      OPTIONAL,

onePortsPTRS                                BIT STRING (SIZE (2))                       OPTIONAL,

twoPUCCH-F0-2-ConsecSymbols                 ENUMERATED {supported}                      OPTIONAL,

pucch-F2-WithFH                             ENUMERATED {supported}                      OPTIONAL,

pucch-F3-WithFH                             ENUMERATED {supported}                      OPTIONAL,

pucch-F4-WithFH                             ENUMERATED {supported}                      OPTIONAL,

pucch-F0-2WithoutFH                         ENUMERATED {notSupported}                   OPTIONAL,

pucch-F1-3-4WithoutFH                       ENUMERATED {notSupported}                   OPTIONAL,

mux-SR-HARQ-ACK-CSI-PUCCH-MultiPerSlot      ENUMERATED {supported}                      OPTIONAL,

uci-CodeBlockSegmentation                   ENUMERATED {supported}                      OPTIONAL,

onePUCCH-LongAndShortFormat                 ENUMERATED {supported}                      OPTIONAL,

twoPUCCH-AnyOthersInSlot                    ENUMERATED {supported}                      OPTIONAL,

intraSlotFreqHopping-PUSCH                  ENUMERATED {supported}                      OPTIONAL,

pusch-LBRM                                  ENUMERATED {supported}                      OPTIONAL,

pdcch-BlindDetectionCA                      INTEGER (4..16)                             OPTIONAL,

tpc-PUSCH-RNTI                              ENUMERATED {supported}                      OPTIONAL,

tpc-PUCCH-RNTI                              ENUMERATED {supported}                      OPTIONAL,

tpc-SRS-RNTI                                ENUMERATED {supported}                      OPTIONAL,

absoluteTPC-Command                         ENUMERATED {supported}                      OPTIONAL,

twoDifferentTPC-Loop-PUSCH                  ENUMERATED {supported}                      OPTIONAL,

twoDifferentTPC-Loop-PUCCH                  ENUMERATED {supported}                      OPTIONAL,

pusch-HalfPi-BPSK                           ENUMERATED {supported}                      OPTIONAL,

pucch-F3-4-HalfPi-BPSK                      ENUMERATED {supported}                      OPTIONAL,

almostContiguousCP-OFDM-UL                  ENUMERATED {supported}                      OPTIONAL,

sp-CSI-RS                                   ENUMERATED {supported}                      OPTIONAL,

sp-CSI-IM                                   ENUMERATED {supported}                      OPTIONAL,

tdd-MultiDL-UL-SwitchPerSlot                ENUMERATED {supported}                      OPTIONAL,

multipleCORESET                             ENUMERATED {supported}                      OPTIONAL,

...,

[[

csi-RS-IM-ReceptionForFeedback              CSI-RS-IM-ReceptionForFeedback              OPTIONAL,

csi-RS-ProcFrameworkForSRS                  CSI-RS-ProcFrameworkForSRS                  OPTIONAL,

csi-ReportFramework                         CSI-ReportFramework                         OPTIONAL,

mux-SR-HARQ-ACK-CSI-PUCCH-OncePerSlot       SEQUENCE {

sameSymbol                                  ENUMERATED {supported}                      OPTIONAL,

diffSymbol                                  ENUMERATED {supported}                      OPTIONAL

}                                                                                       OPTIONAL,

mux-SR-HARQ-ACK-PUCCH                       ENUMERATED {supported}                      OPTIONAL,

mux-MultipleGroupCtrlCH-Overlap             ENUMERATED {supported}                      OPTIONAL,

dl-SchedulingOffset-PDSCH-TypeA             ENUMERATED {supported}                      OPTIONAL,

dl-SchedulingOffset-PDSCH-TypeB             ENUMERATED {supported}                      OPTIONAL,

ul-SchedulingOffset                         ENUMERATED {supported}                      OPTIONAL,

dl-64QAM-MCS-TableAlt                       ENUMERATED {supported}                      OPTIONAL,

ul-64QAM-MCS-TableAlt                       ENUMERATED {supported}                      OPTIONAL,

cqi-TableAlt                                ENUMERATED {supported}                      OPTIONAL,

oneFL-DMRS-TwoAdditionalDMRS-UL             ENUMERATED {supported}                      OPTIONAL,

twoFL-DMRS-TwoAdditionalDMRS-UL             ENUMERATED {supported}                      OPTIONAL,

oneFL-DMRS-ThreeAdditionalDMRS-UL           ENUMERATED {supported}                      OPTIONAL

]],

[[

pdcch-BlindDetectionNRDC                SEQUENCE {

pdcch-BlindDetectionMCG-UE              INTEGER (1..15),

pdcch-BlindDetectionSCG-UE              INTEGER (1..15)

}                                                                                       OPTIONAL,

mux-HARQ-ACK-PUSCH-DiffSymbol               ENUMERATED {supported}                      OPTIONAL

]],

[[

-- R1 11-1b: Type 1 HARQ-ACK codebook support for relative TDRA for DL

type1-HARQ-ACK-Codebook-r16                 ENUMERATED {supported}                      OPTIONAL,

-- R1 11-8: Enhanced UL power control scheme

enhancedPowerControl-r16                    ENUMERATED {supported}                      OPTIONAL,

-- R1 16-1b-1: TCI state activation across multiple CCs

simultaneousTCI-ActMultipleCC-r16           ENUMERATED {supported}                      OPTIONAL,

-- R1 16-1b-2: Spatial relation update across multiple CCs

simultaneousSpatialRelationMultipleCC-r16   ENUMERATED {supported}                      OPTIONAL,

cli-RSSI-FDM-DL-r16                         ENUMERATED {supported}                      OPTIONAL,

cli-SRS-RSRP-FDM-DL-r16                     ENUMERATED {supported}                      OPTIONAL,

-- R1 19-3: Maximum MIMO Layer Adaptation

maxLayersMIMO-Adaptation-r16                ENUMERATED {supported}                      OPTIONAL,

-- R1 12-5: Configuration of aggregation factor per SPS configuration

aggregationFactorSPS-DL-r16                 ENUMERATED {supported}                      OPTIONAL,

-- R1 16-1g: Resources for beam management, pathloss measurement, BFD, RLM and new beam identification

maxTotalResourcesForOneFreqRange-r16        SEQUENCE {

maxNumberResWithinSlotAcrossCC-OneFR-r16    ENUMERATED {n2, n4, n8, n12, n16, n32, n64, n128}    OPTIONAL,

maxNumberResAcrossCC-OneFR-r16              ENUMERATED {n2, n4, n8, n12, n16, n32, n40, n48, n64, n72, n80, n96, n128, n256}

OPTIONAL

}                                           OPTIONAL,

-- R1 16-7: Extension of the maximum number of configured aperiodic CSI report settings

csi-ReportFrameworkExt-r16                  CSI-ReportFrameworkExt-r16                  OPTIONAL

]],

[[

twoTCI-Act-servingCellInCC-List-r16         ENUMERATED {supported}                      OPTIONAL

]],

[[

-- R1 22-11: Support of 'cri-RI-CQI' report without non-PMI-PortIndication

cri-RI-CQI-WithoutNon-PMI-PortInd-r16       ENUMERATED {supported}                      OPTIONAL

]],

[[

-- R1 25-11: 4-bits subband CQI for TN and licensed

cqi-4-BitsSubbandTN-NonSharedSpectrumChAccess-r17  ENUMERATED {supported}               OPTIONAL

]]

}

Phy-ParametersFR1 ::=                       SEQUENCE {

pdcch-MonitoringSingleOccasion              ENUMERATED {supported}                      OPTIONAL,

scs-60kHz                                   ENUMERATED {supported}                      OPTIONAL,

pdsch-256QAM-FR1                            ENUMERATED {supported}                      OPTIONAL,

pdsch-RE-MappingFR1-PerSymbol               ENUMERATED {n10, n20}                       OPTIONAL,

...,

[[

pdsch-RE-MappingFR1-PerSlot                 ENUMERATED {n16, n32, n48, n64, n80, n96, n112, n128,

n144, n160, n176, n192, n208, n224, n240, n256}         OPTIONAL

]],

[[

-- R1 22-12: PDCCH monitoring with a single span of three contiguous OFDM symbols that is within the first four OFDM symbols in a

-- slot

pdcch-MonitoringSingleSpanFirst4Sym-r16     ENUMERATED {supported}                      OPTIONAL

]]

}

Phy-ParametersFR2 ::=                       SEQUENCE {

dummy                                       ENUMERATED {supported}                                  OPTIONAL,

pdsch-RE-MappingFR2-PerSymbol               ENUMERATED {n6, n20}                                    OPTIONAL,

...,

[[

pCell-FR2                                   ENUMERATED {supported}                                  OPTIONAL,

pdsch-RE-MappingFR2-PerSlot                 ENUMERATED {n16, n32, n48, n64, n80, n96, n112, n128,

n144, n160, n176, n192, n208, n224, n240, n256}     OPTIONAL

]],

[[

-- R1 16-1c: Support of default spatial relation and pathloss reference RS for dedicated-PUCCH/SRS and PUSCH

defaultSpatialRelationPathlossRS-r16        ENUMERATED {supported}                                  OPTIONAL,

-- R1 16-1d: Support of spatial relation update for AP-SRS via MAC CE

spatialRelationUpdateAP-SRS-r16             ENUMERATED {supported}                                  OPTIONAL,

maxNumberSRS-PosSpatialRelationsAllServingCells-r16  ENUMERATED {n0, n1, n2, n4, n8, n16}           OPTIONAL

]]

}

-- TAG-PHY-PARAMETERS-STOP

-- ASN1STOP

Phy-ParametersFRX-Diff field descriptions

csi-RS-IM-ReceptionForFeedback/ csi-RS-ProcFrameworkForSRS/ csi-ReportFramework These fields are optionally present in fr1-fr2-Add-UE-NR-Capabilities in UE-NR-Capability. They shall not be set in any other instance of the IE Phy-ParametersFRX-Diff. If the network configures the UE with serving cells on both FR1 and FR2 bands, these parameters, if present, limit the corresponding parameters in MIMO-ParametersPerBand.

#### –	Phy-ParametersMRDC

The IE Phy-ParametersMRDC is used to convey physical layer capabilities for MR-DC.

Phy-ParametersMRDC information element

-- ASN1START

-- TAG-PHY-PARAMETERSMRDC-START

Phy-ParametersMRDC ::=              SEQUENCE {

naics-Capability-List               SEQUENCE (SIZE (1..maxNrofNAICS-Entries)) OF NAICS-Capability-Entry         OPTIONAL,

...,

[[

spCellPlacement                     CarrierAggregationVariant                                                   OPTIONAL

]],

[[

-- R1 18-3b: Semi-statically configured LTE UL transmissions in all UL subframes not limited to tdm-pattern in case of TDD PCell

tdd-PCellUL-TX-AllUL-Subframe-r16   ENUMERATED {supported}                                                      OPTIONAL,

-- R1 18-3a: Semi-statically configured LTE UL transmissions in all UL subframes not limited to tdm-pattern in case of FDD PCell

fdd-PCellUL-TX-AllUL-Subframe-r16   ENUMERATED {supported}                                                      OPTIONAL

]]

}

NAICS-Capability-Entry ::=          SEQUENCE {

numberOfNAICS-CapableCC             INTEGER(1..5),

numberOfAggregatedPRB               ENUMERATED {n50, n75, n100, n125, n150, n175, n200, n225,

n250, n275, n300, n350, n400, n450, n500, spare},

...

}

-- TAG-PHY-PARAMETERSMRDC-STOP

-- ASN1STOP

PHY-ParametersMRDC field descriptions

naics-Capability-List Indicates that UE in MR-DC supports NAICS as defined in TS 36.331 [10].

#### –	Phy-ParametersSharedSpectrumChAccess

The IE Phy-ParametersSharedSpectrumChAccess is used to convey the physical layer capabilities specific for shared spectrum channel access.

Phy-ParametersSharedSpectrumChAccess information element

-- ASN1START

-- TAG-PHY-PARAMETERSSHAREDSPECTRUMCHACCESS-START

Phy-ParametersSharedSpectrumChAccess-r16 ::=    SEQUENCE {

-- 10-32 (1-2): SS block based SINR measurement (SS-SINR) for unlicensed spectrum

ss-SINR-Meas-r16                                ENUMERATED {supported}                      OPTIONAL,

-- 10-33 (2-32a): Semi-persistent CSI report on PUCCH for unlicensed spectrum

sp-CSI-ReportPUCCH-r16                          ENUMERATED {supported}                      OPTIONAL,

-- 10-33a (2-32b): Semi-persistent CSI report on PUSCH for unlicensed spectrum

sp-CSI-ReportPUSCH-r16                          ENUMERATED {supported}                      OPTIONAL,

-- 10-34 (3-6): Dynamic SFI monitoring for unlicensed spectrum

dynamicSFI-r16                                  ENUMERATED {supported}                      OPTIONAL,

-- 10-35c (4-19c): SR/HARQ-ACK/CSI multiplexing once per slot using a PUCCH (or HARQ-ACK/CSI piggybacked on a PUSCH) when SR/HARQ-

-- ACK/CSI are supposed to be sent with different starting symbols in a slot for unlicensed spectrum

-- 10-35 (4-19): SR/HARQ-ACK/CSI multiplexing once per slot using a PUCCH (or HARQ-ACK/CSI piggybacked on a PUSCH) when SR/HARQ-

-- ACK/CSI are supposed to be sent with the same starting symbol on the PUCCH resources in a slot for unlicensed spectrum

mux-SR-HARQ-ACK-CSI-PUCCH-OncePerSlot-r16       SEQUENCE {

sameSymbol-r16                                  ENUMERATED {supported}                  OPTIONAL,

diffSymbol-r16                                  ENUMERATED {supported}                  OPTIONAL

}                                                                                           OPTIONAL,

-- 10-35a (4-19a): Overlapping PUCCH resources have different starting symbols in a slot for unlicensed spectrum

mux-SR-HARQ-ACK-PUCCH-r16                       ENUMERATED {supported}                      OPTIONAL,

-- 10-35b (4-19b): SR/HARQ-ACK/CSI multiplexing more than once per slot using a PUCCH (or HARQ-ACK/CSI piggybacked on a PUSCH) when

-- SR/HARQ ACK/CSI are supposed to be sent with the same or different starting symbol in a slot for unlicensed spectrum

mux-SR-HARQ-ACK-CSI-PUCCH-MultiPerSlot-r16      ENUMERATED {supported}                      OPTIONAL,

-- 10-36 (4-28): HARQ-ACK multiplexing on PUSCH with different PUCCH/PUSCH starting OFDM symbols for unlicensed spectrum

mux-HARQ-ACK-PUSCH-DiffSymbol-r16               ENUMERATED {supported}                      OPTIONAL,

-- 10-37 (4-23): Repetitions for PUCCH format 1, 3, and 4 over multiple slots with K = 2, 4, 8 for unlicensed spectrum

pucch-Repetition-F1-3-4-r16                     ENUMERATED {supported}                      OPTIONAL,

-- 10-38 (5-14): Type 1 configured PUSCH repetitions over multiple slots for unlicensed spectrum

type1-PUSCH-RepetitionMultiSlots-r16            ENUMERATED {supported}                      OPTIONAL,

-- 10-39 (5-16): Type 2 configured PUSCH repetitions over multiple slots for unlicensed spectrum

type2-PUSCH-RepetitionMultiSlots-r16            ENUMERATED {supported}                      OPTIONAL,

-- 10-40 (5-17): PUSCH repetitions over multiple slots for unlicensed spectrum

pusch-RepetitionMultiSlots-r16                  ENUMERATED {supported}                      OPTIONAL,

-- 10-40a (5-17a): PDSCH repetitions over multiple slots for unlicensed spectrum

pdsch-RepetitionMultiSlots-r16                  ENUMERATED {supported}                      OPTIONAL,

-- 10-41 (5-18): DL SPS

downlinkSPS-r16                                 ENUMERATED {supported}                      OPTIONAL,

-- 10-42 (5-19): Type 1 Configured UL grant

configuredUL-GrantType1-r16                     ENUMERATED {supported}                      OPTIONAL,

-- 10-43 (5-20): Type 2 Configured UL grant

configuredUL-GrantType2-r16                     ENUMERATED {supported}                      OPTIONAL,

-- 10-44 (5-21): Pre-emption indication for DL

pre-EmptIndication-DL-r16                       ENUMERATED {supported}                      OPTIONAL,

...

}

-- TAG-PHY-PARAMETERSSHAREDSPECTRUMCHACCESS-STOP

-- ASN1STOP

#### –	PosSRS-RRC-Inactive-OutsideInitialUL-BWP

The IE PosSRS-RRC-Inactive-OutsideInitialUL-BWP is used to convey the capabilities supported by the UE for Positioning SRS transmission in RRC_INACTIVE state configured outside initial UL BWP.

PosSRS-RRC-Inactive-OutsideInitialUL-BWP information element

-- ASN1START

-- TAG-POSSRS-RRC-INACTIVE-OUTSIDEINITIALUL-BWP-START

PosSRS-RRC-Inactive-OutsideInitialUL-BWP-r17::= SEQUENCE {

-- R1 27-15b: Positioning SRS transmission in RRC_INACTIVE state configured outside initial UL BWP

maxSRSposBandwidthForEachSCS-withinCC-FR1-r17   ENUMERATED {bw5, bw10, bw15, bw20, bw25, bw30, bw35, bw40,

bw45, bw50, bw60, bw70, bw80, bw90, bw100}                    OPTIONAL,

maxSRSposBandwidthForEachSCS-withinCC-FR2-r17   ENUMERATED {bw50, bw100, bw200, bw400}                        OPTIONAL,

maxNumOfSRSposResourceSets-r17                  ENUMERATED {n1, n2, n4, n8, n12, n16}                         OPTIONAL,

maxNumOfPeriodicSRSposResources-r17             ENUMERATED {n1, n2, n4, n8, n16, n32, n64}                    OPTIONAL,

maxNumOfPeriodicSRSposResourcesPerSlot-r17      ENUMERATED {n1, n2, n3, n4, n5, n6, n8, n10, n12, n14}        OPTIONAL,

differentNumerologyBetweenSRSposAndInitialBWP-r17  ENUMERATED {supported}                                     OPTIONAL,

srsPosWithoutRestrictionOnBWP-r17               ENUMERATED {supported}                                        OPTIONAL,

maxNumOfPeriodicAndSemipersistentSRSposResources-r17  ENUMERATED {n1, n2, n4, n8, n16, n32, n64}              OPTIONAL,

maxNumOfPeriodicAndSemipersistentSRSposResourcesPerSlot-r17  ENUMERATED {n1, n2, n3, n4, n5, n6, n8, n10, n12, n14}  OPTIONAL,

differentCenterFreqBetweenSRSposAndInitialBWP-r17  ENUMERATED {supported}                                     OPTIONAL,

switchingTimeSRS-TX-OtherTX-r17                 ENUMERATED {us100, us140, us200, us300, us500}                OPTIONAL,

-- R1 27-15c: Support of positioning SRS transmission in RRC_INACTIVE state outside initial BWP with semi-persistent SRS

maxNumOfSemiPersistentSRSposResources-r17       ENUMERATED {n1, n2, n4, n8, n16, n32, n64}                    OPTIONAL,

maxNumOfSemiPersistentSRSposResourcesPerSlot-r17  ENUMERATED {n1, n2, n3, n4, n5, n6, n8, n10, n12, n14}      OPTIONAL,

...

}

-- TAG-POSSRS-RRC-INACTIVE-OUTSIDEINITIALUL-BWP-STOP

-- ASN1STOP

#### –	PowSav-Parameters

The IE PowSav-Parameters is used to convey the capabilities supported by the UE for the power saving preferences.

PowSav-Parameters information element

-- ASN1START

-- TAG-POWSAV-PARAMETERS-START

PowSav-Parameters-r16 ::=         SEQUENCE {

powSav-ParametersCommon-r16               PowSav-ParametersCommon-r16                                        OPTIONAL,

powSav-ParametersFRX-Diff-r16             PowSav-ParametersFRX-Diff-r16                                      OPTIONAL,

...

}

PowSav-Parameters-v1700 ::=     SEQUENCE {

powSav-ParametersFR2-2-r17      PowSav-ParametersFR2-2-r17                                                   OPTIONAL,

...

}

PowSav-ParametersCommon-r16 ::=    SEQUENCE {

drx-Preference-r16                        ENUMERATED {supported}                                             OPTIONAL,

maxCC-Preference-r16                      ENUMERATED {supported}                                             OPTIONAL,

releasePreference-r16                     ENUMERATED {supported}                                             OPTIONAL,

-- R1 19-4a: UE assistance information

minSchedulingOffsetPreference-r16         ENUMERATED {supported}                                             OPTIONAL,

...

}

PowSav-ParametersFRX-Diff-r16 ::=    SEQUENCE {

maxBW-Preference-r16                      ENUMERATED {supported}                                             OPTIONAL,

maxMIMO-LayerPreference-r16               ENUMERATED {supported}                                             OPTIONAL,

...

}

PowSav-ParametersFR2-2-r17 ::=      SEQUENCE {

maxBW-Preference-r17                      ENUMERATED {supported}                                             OPTIONAL,

maxMIMO-LayerPreference-r17               ENUMERATED {supported}                                             OPTIONAL,

...

}

-- TAG-POWSAV-PARAMETERS-STOP

-- ASN1STOP

#### –	ProcessingParameters

The IE ProcessingParameters is used to indicate PDSCH/PUSCH processing capabilities supported by the UE.

ProcessingParameters information element

-- ASN1START

-- TAG-PROCESSINGPARAMETERS-START

ProcessingParameters ::=        SEQUENCE {

fallback                        ENUMERATED {sc, cap1-only},

differentTB-PerSlot              SEQUENCE {

upto1                          NumberOfCarriers                    OPTIONAL,

upto2                          NumberOfCarriers                    OPTIONAL,

upto4                          NumberOfCarriers                    OPTIONAL,

upto7                          NumberOfCarriers                    OPTIONAL

}                                                                 OPTIONAL

}

NumberOfCarriers ::=    INTEGER (1..16)

-- TAG-PROCESSINGPARAMETERS-STOP

-- ASN1STOP

#### –	PRS-ProcessingCapabilityOutsideMGinPPWperType

The IE PRS-ProcessingCapabilityOutsideMGinPPWperType is used to indicate DL PRS Processing Capability outside MG capabilities supported by the UE.

PRS-ProcessingCapabilityOutsideMGinPPWperType information element

-- ASN1START

-- TAG-PRS-PROCESSINGCAPABILITYOUTSIDEMGINPPWPERType-START

PRS-ProcessingCapabilityOutsideMGinPPWperType-r17 ::= SEQUENCE {

prsProcessingType-r17                                 ENUMERATED {type1A, type1B, type2},

ppw-dl-PRS-BufferType-r17                             ENUMERATED {type1, type2, ...},

ppw-durationOfPRS-Processing-r17                      CHOICE {

ppw-durationOfPRS-Processing1-r17                     SEQUENCE {

ppw-durationOfPRS-ProcessingSymbolsN-r17              ENUMERATED {msDot125, msDot25, msDot5, ms1, ms2, ms4, ms6, ms8, ms12,

ms16, ms20, ms25, ms30, ms32, ms35, ms40, ms45, ms50},

ppw-durationOfPRS-ProcessingSymbolsT-r17              ENUMERATED {ms1, ms2, ms4, ms8, ms16, ms20, ms30, ms40, ms80,

ms160, ms320, ms640, ms1280}

},

ppw-durationOfPRS-Processing2-r17                     SEQUENCE {

ppw-durationOfPRS-ProcessingSymbolsN2-r17             ENUMERATED {msDot125, msDot25, msDot5, ms1, ms2, ms3, ms4, ms5,

ms6, ms8, ms12},

ppw-durationOfPRS-ProcessingSymbolsT2-r17             ENUMERATED {ms4, ms5, ms6, ms8}

}

}                                                                                                                          OPTIONAL,

ppw-maxNumOfDL-PRS-ResProcessedPerSlot-r17            SEQUENCE {

scs15-r17                                             ENUMERATED {n1, n2, n4, n6, n8, n12, n16, n24, n32, n48, n64}    OPTIONAL,

scs30-r17                                             ENUMERATED {n1, n2, n4, n6, n8, n12, n16, n24, n32, n48, n64}    OPTIONAL,

scs60-r17                                             ENUMERATED {n1, n2, n4, n6, n8, n12, n16, n24, n32, n48, n64}    OPTIONAL,

scs120-r17                                            ENUMERATED {n1, n2, n4, n6, n8, n12, n16, n24, n32, n48, n64}    OPTIONAL,

...

},

ppw-maxNumOfDL-Bandwidth-r17                          CHOICE {

fr1-r17                                               ENUMERATED {mhz5, mhz10, mhz20, mhz40,	mhz50, mhz80, mhz100},

fr2-r17                                               ENUMERATED {mhz50, mhz100, mhz200, mhz400}

}                                                                                                                          OPTIONAL

}

-- TAG-PRS-PROCESSINGCAPABILITYOUTSIDEMGINPPWPERType-STOP

-- ASN1STOP

#### –	RAT-Type

The IE RAT-Type is used to indicate the radio access technology (RAT), including NR, of the requested/transferred UE capabilities.

RAT-Type information element

-- ASN1START

-- TAG-RAT-TYPE-START

RAT-Type ::= ENUMERATED {nr, eutra-nr, eutra, utra-fdd-v1610, ...}

-- TAG-RAT-TYPE-STOP

-- ASN1STOP

#### –	RedCapParameters

The IE RedCapParameters is used to indicate the UE capabilities supported by RedCap UEs.

RedCapParameters information element

-- ASN1START

-- TAG-REDCAPPARAMETERS-START

RedCapParameters-r17::=                   SEQUENCE {

-- R1 28-1: RedCap UE

supportOfRedCap-r17                       ENUMERATED {supported}                                      OPTIONAL,

supportOf16DRB-RedCap-r17                 ENUMERATED {supported}                                      OPTIONAL

}

RedCapParameters-v1740::=                 SEQUENCE {

ncd-SSB-ForRedCapInitialBWP-SDT-r17       ENUMERATED {supported}                                      OPTIONAL

}

-- TAG-REDCAPPARAMETERS-STOP

-- ASN1STOP

#### –	RF-Parameters

The IE RF-Parameters is used to convey RF-related capabilities for NR operation.

RF-Parameters information element

-- ASN1START

-- TAG-RF-PARAMETERS-START

RF-Parameters ::=                                   SEQUENCE {

supportedBandListNR                                 SEQUENCE (SIZE (1..maxBands)) OF BandNR,

supportedBandCombinationList                        BandCombinationList                         OPTIONAL,

appliedFreqBandListFilter                           FreqBandList                                OPTIONAL,

...,

[[

supportedBandCombinationList-v1540                  BandCombinationList-v1540                   OPTIONAL,

srs-SwitchingTimeRequested                          ENUMERATED {true}                           OPTIONAL

]],

[[

supportedBandCombinationList-v1550                  BandCombinationList-v1550                   OPTIONAL

]],

[[

supportedBandCombinationList-v1560                  BandCombinationList-v1560                   OPTIONAL

]],

[[

supportedBandCombinationList-v1610                  BandCombinationList-v1610                   OPTIONAL,

supportedBandCombinationListSidelinkEUTRA-NR-r16    BandCombinationListSidelinkEUTRA-NR-r16     OPTIONAL,

supportedBandCombinationList-UplinkTxSwitch-r16     BandCombinationList-UplinkTxSwitch-r16      OPTIONAL

]],

[[

supportedBandCombinationList-v1630                  BandCombinationList-v1630                   OPTIONAL,

supportedBandCombinationListSidelinkEUTRA-NR-v1630  BandCombinationListSidelinkEUTRA-NR-v1630   OPTIONAL,

supportedBandCombinationList-UplinkTxSwitch-v1630   BandCombinationList-UplinkTxSwitch-v1630    OPTIONAL

]],

[[

supportedBandCombinationList-v1640                  BandCombinationList-v1640                   OPTIONAL,

supportedBandCombinationList-UplinkTxSwitch-v1640   BandCombinationList-UplinkTxSwitch-v1640    OPTIONAL

]],

[[

supportedBandCombinationList-v1650                  BandCombinationList-v1650                   OPTIONAL,

supportedBandCombinationList-UplinkTxSwitch-v1650   BandCombinationList-UplinkTxSwitch-v1650    OPTIONAL

]],

[[

extendedBand-n77-r16                                ENUMERATED {supported}                      OPTIONAL

]],

[[

supportedBandCombinationList-UplinkTxSwitch-v1670   BandCombinationList-UplinkTxSwitch-v1670    OPTIONAL

]],

[[

supportedBandCombinationList-v1680                  BandCombinationList-v1680                   OPTIONAL

]],

[[

supportedBandCombinationList-v1690                  BandCombinationList-v1690                   OPTIONAL,

supportedBandCombinationList-UplinkTxSwitch-v1690   BandCombinationList-UplinkTxSwitch-v1690    OPTIONAL

]],

[[

supportedBandCombinationList-v1700                  BandCombinationList-v1700                   OPTIONAL,

supportedBandCombinationList-UplinkTxSwitch-v1700   BandCombinationList-UplinkTxSwitch-v1700    OPTIONAL,

supportedBandCombinationListSL-RelayDiscovery-r17   OCTET STRING                                OPTIONAL,  -- Contains PC5 BandCombinationListSidelinkNR-r16

supportedBandCombinationListSL-NonRelayDiscovery-r17 OCTET STRING                               OPTIONAL,  -- Contains PC5 BandCombinationListSidelinkNR-r16

supportedBandCombinationListSidelinkEUTRA-NR-v1710  BandCombinationListSidelinkEUTRA-NR-v1710   OPTIONAL,

sidelinkRequested-r17                               ENUMERATED {true}                           OPTIONAL,

extendedBand-n77-2-r17                              ENUMERATED {supported}                      OPTIONAL

]],

[[

supportedBandCombinationList-v1720                  BandCombinationList-v1720                   OPTIONAL,

supportedBandCombinationList-UplinkTxSwitch-v1720   BandCombinationList-UplinkTxSwitch-v1720    OPTIONAL

]],

[[

supportedBandCombinationList-v1730                  BandCombinationList-v1730                   OPTIONAL,

supportedBandCombinationList-UplinkTxSwitch-v1730   BandCombinationList-UplinkTxSwitch-v1730    OPTIONAL,

supportedBandCombinationListSL-RelayDiscovery-v1730 BandCombinationListSL-Discovery-r17         OPTIONAL,

supportedBandCombinationListSL-NonRelayDiscovery-v1730 BandCombinationListSL-Discovery-r17      OPTIONAL

]],

[[

supportedBandCombinationList-v1740                  BandCombinationList-v1740                   OPTIONAL,

supportedBandCombinationList-UplinkTxSwitch-v1740   BandCombinationList-UplinkTxSwitch-v1740    OPTIONAL

]]

}

RF-Parameters-v15g0 ::=                   SEQUENCE {

supportedBandCombinationList-v15g0        BandCombinationList-v15g0                   OPTIONAL

}

RF-Parameters-v16a0 ::=                            SEQUENCE {

supportedBandCombinationList-v16a0                 BandCombinationList-v16a0                    OPTIONAL,

supportedBandCombinationList-UplinkTxSwitch-v16a0  BandCombinationList-UplinkTxSwitch-v16a0     OPTIONAL

}

RF-Parameters-v16c0 ::=                            SEQUENCE {

supportedBandListNR-v16c0                          SEQUENCE (SIZE (1..maxBands)) OF BandNR-v16c0

}

BandNR ::=                          SEQUENCE {

bandNR                              FreqBandIndicatorNR,

modifiedMPR-Behaviour               BIT STRING (SIZE (8))                           OPTIONAL,

mimo-ParametersPerBand              MIMO-ParametersPerBand                          OPTIONAL,

extendedCP                          ENUMERATED {supported}                          OPTIONAL,

multipleTCI                         ENUMERATED {supported}                          OPTIONAL,

bwp-WithoutRestriction              ENUMERATED {supported}                          OPTIONAL,

bwp-SameNumerology                  ENUMERATED {upto2, upto4}                       OPTIONAL,

bwp-DiffNumerology                  ENUMERATED {upto4}                              OPTIONAL,

crossCarrierScheduling-SameSCS      ENUMERATED {supported}                          OPTIONAL,

pdsch-256QAM-FR2                    ENUMERATED {supported}                          OPTIONAL,

pusch-256QAM                        ENUMERATED {supported}                          OPTIONAL,

ue-PowerClass                       ENUMERATED {pc1, pc2, pc3, pc4}                 OPTIONAL,

rateMatchingLTE-CRS                 ENUMERATED {supported}                          OPTIONAL,

channelBWs-DL                       CHOICE {

fr1                                 SEQUENCE {

scs-15kHz                           BIT STRING (SIZE (10))                      OPTIONAL,

scs-30kHz                           BIT STRING (SIZE (10))                      OPTIONAL,

scs-60kHz                           BIT STRING (SIZE (10))                      OPTIONAL

},

fr2                                 SEQUENCE {

scs-60kHz                           BIT STRING (SIZE (3))                       OPTIONAL,

scs-120kHz                          BIT STRING (SIZE (3))                       OPTIONAL

}

}                                                                                   OPTIONAL,

channelBWs-UL                       CHOICE {

fr1                                 SEQUENCE {

scs-15kHz                           BIT STRING (SIZE (10))                      OPTIONAL,

scs-30kHz                           BIT STRING (SIZE (10))                      OPTIONAL,

scs-60kHz                           BIT STRING (SIZE (10))                      OPTIONAL

},

fr2                                 SEQUENCE {

scs-60kHz                           BIT STRING (SIZE (3))                       OPTIONAL,

scs-120kHz                          BIT STRING (SIZE (3))                       OPTIONAL

}

}                                                                                   OPTIONAL,

...,

[[

maxUplinkDutyCycle-PC2-FR1                  ENUMERATED {n60, n70, n80, n90, n100}   OPTIONAL

]],

[[

pucch-SpatialRelInfoMAC-CE          ENUMERATED {supported}                          OPTIONAL,

powerBoosting-pi2BPSK               ENUMERATED {supported}                          OPTIONAL

]],

[[

maxUplinkDutyCycle-FR2          ENUMERATED {n15, n20, n25, n30, n40, n50, n60, n70, n80, n90, n100}     OPTIONAL

]],

[[

channelBWs-DL-v1590                 CHOICE {

fr1                                 SEQUENCE {

scs-15kHz                           BIT STRING (SIZE (16))              OPTIONAL,

scs-30kHz                           BIT STRING (SIZE (16))              OPTIONAL,

scs-60kHz                           BIT STRING (SIZE (16))              OPTIONAL

},

fr2                                 SEQUENCE {

scs-60kHz                           BIT STRING (SIZE (8))               OPTIONAL,

scs-120kHz                          BIT STRING (SIZE (8))               OPTIONAL

}

}                                                                               OPTIONAL,

channelBWs-UL-v1590                 CHOICE {

fr1                                 SEQUENCE {

scs-15kHz                           BIT STRING (SIZE (16))              OPTIONAL,

scs-30kHz                           BIT STRING (SIZE (16))              OPTIONAL,

scs-60kHz                           BIT STRING (SIZE (16))              OPTIONAL

},

fr2                                 SEQUENCE {

scs-60kHz                           BIT STRING (SIZE (8))               OPTIONAL,

scs-120kHz                          BIT STRING (SIZE (8))               OPTIONAL

}

}                                                                               OPTIONAL

]],

[[

asymmetricBandwidthCombinationSet     BIT STRING (SIZE (1..32))           OPTIONAL

]],

[[

-- R1 10: NR-unlicensed

sharedSpectrumChAccessParamsPerBand-r16 SharedSpectrumChAccessParamsPerBand-r16 OPTIONAL,

-- R1 11-7b: Independent cancellation of the overlapping PUSCHs in an intra-band UL CA

cancelOverlappingPUSCH-r16              ENUMERATED {supported}                  OPTIONAL,

-- R1 14-1: Multiple LTE-CRS rate matching patterns

multipleRateMatchingEUTRA-CRS-r16       SEQUENCE {

maxNumberPatterns-r16               INTEGER (2..6),

maxNumberNon-OverlapPatterns-r16    INTEGER (1..3)

}                                                                               OPTIONAL,

-- R1 14-1a: Two LTE-CRS overlapping rate matching patterns within a part of NR carrier using 15 kHz overlapping with a LTE carrier

overlapRateMatchingEUTRA-CRS-r16        ENUMERATED {supported}                  OPTIONAL,

-- R1 14-2: PDSCH Type B mapping of length 9 and 10 OFDM symbols

pdsch-MappingTypeB-Alt-r16              ENUMERATED {supported}                  OPTIONAL,

-- R1 14-3: One slot periodic TRS configuration for FR1

oneSlotPeriodicTRS-r16                  ENUMERATED {supported}                  OPTIONAL,

olpc-SRS-Pos-r16                        OLPC-SRS-Pos-r16                        OPTIONAL,

spatialRelationsSRS-Pos-r16             SpatialRelationsSRS-Pos-r16             OPTIONAL,

simulSRS-MIMO-TransWithinBand-r16       ENUMERATED {n2}                         OPTIONAL,

channelBW-DL-IAB-r16                    CHOICE {

fr1-100mhz                              SEQUENCE {

scs-15kHz                               ENUMERATED {supported}          OPTIONAL,

scs-30kHz                               ENUMERATED {supported}          OPTIONAL,

scs-60kHz                               ENUMERATED {supported}          OPTIONAL

},

fr2-200mhz                          SEQUENCE {

scs-60kHz                           ENUMERATED {supported}              OPTIONAL,

scs-120kHz                          ENUMERATED {supported}              OPTIONAL

}

}                                                                               OPTIONAL,

channelBW-UL-IAB-r16                    CHOICE {

fr1-100mhz                              SEQUENCE {

scs-15kHz                               ENUMERATED {supported}          OPTIONAL,

scs-30kHz                               ENUMERATED {supported}          OPTIONAL,

scs-60kHz                               ENUMERATED {supported}          OPTIONAL

},

fr2-200mhz                              SEQUENCE {

scs-60kHz                               ENUMERATED {supported}          OPTIONAL,

scs-120kHz                              ENUMERATED {supported}          OPTIONAL

}

}                                                                               OPTIONAL,

rasterShift7dot5-IAB-r16                ENUMERATED {supported}                  OPTIONAL,

ue-PowerClass-v1610                     ENUMERATED {pc1dot5}                    OPTIONAL,

condHandover-r16                        ENUMERATED {supported}                  OPTIONAL,

condHandoverFailure-r16                 ENUMERATED {supported}                  OPTIONAL,

condHandoverTwoTriggerEvents-r16        ENUMERATED {supported}                  OPTIONAL,

condPSCellChange-r16                    ENUMERATED {supported}                  OPTIONAL,

condPSCellChangeTwoTriggerEvents-r16    ENUMERATED {supported}                  OPTIONAL,

mpr-PowerBoost-FR2-r16                  ENUMERATED {supported}                  OPTIONAL,

-- R1 11-9: Multiple active configured grant configurations for a BWP of a serving cell

activeConfiguredGrant-r16               SEQUENCE {

maxNumberConfigsPerBWP-r16                  ENUMERATED {n1, n2, n4, n8, n12},

maxNumberConfigsAllCC-r16                   INTEGER (2..32)

}                                                                               OPTIONAL,

-- R1 11-9a: Joint release in a DCI for two or more configured grant Type 2 configurations for a given BWP of a serving cell

jointReleaseConfiguredGrantType2-r16    ENUMERATED {supported}                  OPTIONAL,

-- R1 12-2: Multiple SPS configurations

sps-r16                                 SEQUENCE {

maxNumberConfigsPerBWP-r16                  INTEGER (1..8),

maxNumberConfigsAllCC-r16                   INTEGER (2..32)

}                                                                               OPTIONAL,

-- R1 12-2a: Joint release in a DCI for two or more SPS configurations for a given BWP of a serving cell

jointReleaseSPS-r16                     ENUMERATED {supported}                  OPTIONAL,

-- R1 13-19: Simultaneous positioning SRS and MIMO SRS transmission within a band across multiple CCs

simulSRS-TransWithinBand-r16            ENUMERATED {n2}                         OPTIONAL,

trs-AdditionalBandwidth-r16             ENUMERATED {trs-AddBW-Set1, trs-AddBW-Set2}  OPTIONAL,

handoverIntraF-IAB-r16                  ENUMERATED {supported}                  OPTIONAL

]],

[[

-- R1 22-5a: Simultaneous transmission of SRS for antenna switching and SRS for CB/NCB /BM for intra-band UL CA

-- R1 22-5c: Simultaneous transmission of SRS for antenna switching and SRS for antenna switching for intra-band UL CA

simulTX-SRS-AntSwitchingIntraBandUL-CA-r16  SimulSRS-ForAntennaSwitching-r16            OPTIONAL,

-- R1 10: NR-unlicensed

sharedSpectrumChAccessParamsPerBand-v1630   SharedSpectrumChAccessParamsPerBand-v1630   OPTIONAL

]],

[[

handoverUTRA-FDD-r16                      ENUMERATED {supported}                       OPTIONAL,

-- R4 7-4: Report the shorter transient capability supported by the UE: 2, 4 or 7us

enhancedUL-TransientPeriod-r16            ENUMERATED {us2, us4, us7}                   OPTIONAL,

sharedSpectrumChAccessParamsPerBand-v1640 SharedSpectrumChAccessParamsPerBand-v1640    OPTIONAL

]],

[[

type1-PUSCH-RepetitionMultiSlots-v1650    ENUMERATED {supported}                       OPTIONAL,

type2-PUSCH-RepetitionMultiSlots-v1650    ENUMERATED {supported}                       OPTIONAL,

pusch-RepetitionMultiSlots-v1650          ENUMERATED {supported}                       OPTIONAL,

configuredUL-GrantType1-v1650             ENUMERATED {supported}                       OPTIONAL,

configuredUL-GrantType2-v1650             ENUMERATED {supported}                       OPTIONAL,

sharedSpectrumChAccessParamsPerBand-v1650 SharedSpectrumChAccessParamsPerBand-v1650    OPTIONAL

]],

[[

enhancedSkipUplinkTxConfigured-v1660      ENUMERATED {supported}                       OPTIONAL,

enhancedSkipUplinkTxDynamic-v1660         ENUMERATED {supported}                       OPTIONAL

]],

[[

maxUplinkDutyCycle-PC1dot5-MPE-FR1-r16    ENUMERATED {n10, n15, n20, n25, n30, n40, n50, n60, n70, n80, n90, n100}   OPTIONAL,

txDiversity-r16                           ENUMERATED {supported}                       OPTIONAL

]],

[[

-- R1 36-1: Support of 1024QAM for PDSCH for FR1

pdsch-1024QAM-FR1-r17                     ENUMERATED {supported}                       OPTIONAL,

-- R4 22-1 support of FR2 HST operation

ue-PowerClass-v1700                       ENUMERATED {pc5, pc6, pc7}                   OPTIONAL,

-- R1 24: NR extension to 71GHz (FR2-2)

fr2-2-AccessParamsPerBand-r17             FR2-2-AccessParamsPerBand-r17                OPTIONAL,

rlm-Relaxation-r17                        ENUMERATED {supported}                       OPTIONAL,

bfd-Relaxation-r17                        ENUMERATED {supported}                       OPTIONAL,

cg-SDT-r17                                ENUMERATED {supported}                       OPTIONAL,

locationBasedCondHandover-r17             ENUMERATED {supported}                       OPTIONAL,

timeBasedCondHandover-r17                 ENUMERATED {supported}                       OPTIONAL,

eventA4BasedCondHandover-r17              ENUMERATED {supported}                       OPTIONAL,

mn-InitiatedCondPSCellChangeNRDC-r17      ENUMERATED {supported}                       OPTIONAL,

sn-InitiatedCondPSCellChangeNRDC-r17      ENUMERATED {supported}                       OPTIONAL,

-- R1 29-3a: PDCCH skipping

pdcch-SkippingWithoutSSSG-r17             ENUMERATED {supported}                       OPTIONAL,

-- R1 29-3b: 2 search space sets group switching

sssg-Switching-1BitInd-r17                ENUMERATED {supported}                       OPTIONAL,

-- R1 29-3c: 3 search space sets group switching

sssg-Switching-2BitInd-r17                ENUMERATED {supported}                       OPTIONAL,

-- R1 29-3d: 2 search space sets group switching with PDCCH skipping

pdcch-SkippingWithSSSG-r17                ENUMERATED {supported}                       OPTIONAL,

-- R1 29-3e: Support Search space set group switching capability 2 for FR1

searchSpaceSetGrp-switchCap2-r17          ENUMERATED {supported}                       OPTIONAL,

-- R1 26-1: Uplink Time and Frequency pre-compensation and timing relationship enhancements

uplinkPreCompensation-r17                 ENUMERATED {supported}                       OPTIONAL,

-- R1 26-4: UE reporting of information related to TA pre-compensation

uplink-TA-Reporting-r17                   ENUMERATED {supported}                       OPTIONAL,

-- R1 26-5: Increasing the number of HARQ processes

max-HARQ-ProcessNumber-r17                ENUMERATED {u16d32, u32d16, u32d32}          OPTIONAL,

-- R1 26-6: Type-2 HARQ codebook enhancement

type2-HARQ-Codebook-r17                   ENUMERATED {supported}                       OPTIONAL,

-- R1 26-6a: Type-1 HARQ codebook enhancement

type1-HARQ-Codebook-r17                   ENUMERATED {supported}                       OPTIONAL,

-- R1 26-6b: Type-3 HARQ codebook enhancement

type3-HARQ-Codebook-r17                   ENUMERATED {supported}                       OPTIONAL,

-- R1 26-9: UE-specific K_offset

ue-specific-K-Offset-r17                  ENUMERATED {supported}                       OPTIONAL,

-- R1 24-1f: Multiple PDSCH scheduling by single DCI for 120kHz in FR2-1

multiPDSCH-SingleDCI-FR2-1-SCS-120kHz-r17 ENUMERATED {supported}                       OPTIONAL,

-- R1 24-1g: Multiple PUSCH scheduling by single DCI for 120kHz in FR2-1

multiPUSCH-SingleDCI-FR2-1-SCS-120kHz-r17 ENUMERATED {supported}                       OPTIONAL,

-- R4 14-4: Parallel PRS measurements in RRC_INACTIVE state, FR1/FR2 diff

parallelPRS-MeasRRC-Inactive-r17          ENUMERATED {supported}                       OPTIONAL,

-- R1 27-1-2: Support of UE-TxTEGs for UL TDOA

nr-UE-TxTEG-ID-MaxSupport-r17             ENUMERATED {n1, n2, n3, n4, n6, n8}          OPTIONAL,

-- R1 27-17: PRS processing in RRC_INACTIVE

prs-ProcessingRRC-Inactive-r17            ENUMERATED {supported}                       OPTIONAL,

-- R1 27-3-2: DL PRS measurement outside MG and in a PRS processing window

prs-ProcessingWindowType1A-r17            ENUMERATED {option1, option2, option3}       OPTIONAL,

prs-ProcessingWindowType1B-r17            ENUMERATED {option1, option2, option3}       OPTIONAL,

prs-ProcessingWindowType2-r17             ENUMERATED {option1, option2, option3}       OPTIONAL,

-- R1 27-15: Positioning SRS transmission in RRC_INACTIVE state for initial UL BWP

srs-AllPosResourcesRRC-Inactive-r17       SRS-AllPosResourcesRRC-Inactive-r17          OPTIONAL,

-- R1 27-16: OLPC for positioning SRS in RRC_INACTIVE state - gNB

olpc-SRS-PosRRC-Inactive-r17              OLPC-SRS-Pos-r16                             OPTIONAL,

-- R1 27-19: Spatial relation for positioning SRS in RRC_INACTIVE state - gNB

spatialRelationsSRS-PosRRC-Inactive-r17   SpatialRelationsSRS-Pos-r16                  OPTIONAL,

-- R1 30-1: Increased maximum number of PUSCH Type A repetitions

maxNumberPUSCH-TypeA-Repetition-r17       ENUMERATED {supported}                       OPTIONAL,

-- R1 30-2: PUSCH Type A repetitions based on available slots

puschTypeA-RepetitionsAvailSlot-r17       ENUMERATED {supported}                       OPTIONAL,

-- R1 30-3: TB processing over multi-slot PUSCH

tb-ProcessingMultiSlotPUSCH-r17           ENUMERATED {supported}                       OPTIONAL,

-- R1 30-3a: Repetition of TB processing over multi-slot PUSCH

tb-ProcessingRepMultiSlotPUSCH-r17        ENUMERATED {supported}                       OPTIONAL,

-- R1 30-4: The maximum duration for DM-RS bundling

maxDurationDMRS-Bundling-r17              SEQUENCE {

fdd-r17                                   ENUMERATED {n4, n8, n16, n32}            OPTIONAL,

tdd-r17                                   ENUMERATED {n2, n4, n8, n16}             OPTIONAL

}                                                                                      OPTIONAL,

-- R1 30-6: Repetition of PUSCH transmission scheduled by RAR UL grant and DCI format 0_0 with CRC scrambled by TC-RNTI

pusch-RepetitionMsg3-r17                  ENUMERATED {supported}                       OPTIONAL,

sharedSpectrumChAccessParamsPerBand-v1710 SharedSpectrumChAccessParamsPerBand-v1710    OPTIONAL,

-- R4 25-2: Parallel measurements on cells belonging to a different NGSO satellite than a serving satellite without scheduling restrictions

-- on normal operations with the serving cell

parallelMeasurementWithoutRestriction-r17 ENUMERATED {supported}                       OPTIONAL,

-- R4 25-5: Parallel measurements on multiple NGSO satellites within a SMTC

maxNumber-NGSO-SatellitesWithinOneSMTC-r17 ENUMERATED {n1, n2, n3, n4}                 OPTIONAL,

-- R1 26-10: K1 range extension

k1-RangeExtension-r17                     ENUMERATED {supported}                       OPTIONAL,

-- R1 35-1: Aperiodic CSI-RS for tracking for fast SCell activation

aperiodicCSI-RS-FastScellActivation-r17   SEQUENCE {

maxNumberAperiodicCSI-RS-PerCC-r17        ENUMERATED {n8, n16, n32, n48, n64, n128, n255},

maxNumberAperiodicCSI-RS-AcrossCCs-r17    ENUMERATED {n8, n16, n32, n64, n128, n256, n512, n1024}

}                                                                                      OPTIONAL,

-- R1 35-2: Aperiodic CSI-RS bandwidth for tracking for fast SCell activation for 10MHz UE channel bandwidth

aperiodicCSI-RS-AdditionalBandwidth-r17   ENUMERATED {addBW-Set1, addBW-Set2}          OPTIONAL,

-- R1 28-1a: RRC-configured DL BWP without CD-SSB or NCD-SSB

bwp-WithoutCD-SSB-OrNCD-SSB-RedCap-r17    ENUMERATED {supported}                       OPTIONAL,

-- R1 28-3: Half-duplex FDD operation type A for RedCap UE

halfDuplexFDD-TypeA-RedCap-r17            ENUMERATED {supported}                       OPTIONAL,

-- R1 27-15b: Positioning SRS transmission in RRC_INACTIVE state configured outside initial UL BWP

posSRS-RRC-Inactive-OutsideInitialUL-BWP-r17 PosSRS-RRC-Inactive-OutsideInitialUL-BWP-r17 OPTIONAL,

-- R4 15-3 UE support of CBW for 480kHz SCS

channelBWs-DL-SCS-480kHz-FR2-2-r17        BIT STRING (SIZE (8))                        OPTIONAL,

channelBWs-UL-SCS-480kHz-FR2-2-r17        BIT STRING (SIZE (8))                        OPTIONAL,

-- R4 15-4 UE support of CBW for 960kHz SCS

channelBWs-DL-SCS-960kHz-FR2-2-r17        BIT STRING (SIZE (8))                        OPTIONAL,

channelBWs-UL-SCS-960kHz-FR2-2-r17        BIT STRING (SIZE (8))                        OPTIONAL,

-- R4 17-1 UL gap for Tx power management

ul-GapFR2-r17                             ENUMERATED {supported}                       OPTIONAL,

-- R1 25-4: One-shot HARQ ACK feedback triggered by DCI format 1_2

oneShotHARQ-feedbackTriggeredByDCI-1-2-r17 ENUMERATED {supported}                      OPTIONAL,

-- R1 25-5: PHY priority handling for one-shot HARQ ACK feedback

oneShotHARQ-feedbackPhy-Priority-r17      ENUMERATED {supported}                       OPTIONAL,

-- R1 25-6: Enhanced type 3 HARQ-ACK codebook feedback

enhancedType3-HARQ-CodebookFeedback-r17   SEQUENCE {

enhancedType3-HARQ-Codebooks-r17          ENUMERATED {n1, n2, n4, n8},

maxNumberPUCCH-Transmissions-r17          ENUMERATED {n1, n2, n3, n4, n5, n6, n7}

}                                                                                      OPTIONAL,

-- R1 25-7: Triggered HARQ-ACK codebook re-transmission

triggeredHARQ-CodebookRetx-r17              SEQUENCE {

minHARQ-Retx-Offset-r17                     ENUMERATED {n-7, n-5, n-3, n-1, n1},

maxHARQ-Retx-Offset-r17                     ENUMERATED {n4, n6, n8, n10, n12, n14, n16, n18, n20, n22, n24}

}                                                                                      OPTIONAL

]],

[[

-- R4 22-2 support of one shot large UL timing adjustment

ue-OneShotUL-TimingAdj-r17                        ENUMERATED {supported}               OPTIONAL,

-- R1 25-2: Repetitions for PUCCH format 0, and 2 over multiple slots with K = 2, 4, 8

pucch-Repetition-F0-2-r17                         ENUMERATED {supported}               OPTIONAL,

-- R1 25-11a: 4-bits subband CQI for NTN and unlicensed

cqi-4-BitsSubbandNTN-SharedSpectrumChAccess-r17   ENUMERATED {supported}               OPTIONAL,

-- R1 25-16: HARQ-ACK with different priorities multiplexing on a PUCCH/PUSCH

mux-HARQ-ACK-DiffPriorities-r17                   ENUMERATED {supported}               OPTIONAL,

-- R1 25-20a: Propagation delay compensation based on legacy TA procedure for NTN and unlicensed

ta-BasedPDC-NTN-SharedSpectrumChAccess-r17        ENUMERATED {supported}               OPTIONAL,

-- R1 33-2b: DCI-based enabling/disabling ACK/NACK-based feedback for dynamic scheduling for multicast

ack-NACK-FeedbackForMulticastWithDCI-Enabler-r17  ENUMERATED {supported}               OPTIONAL,

-- R1 33-2e: Multiple G-RNTIs for group-common PDSCHs

maxNumberG-RNTI-r17                               INTEGER (2..8)                       OPTIONAL,

-- R1 33-2f: Dynamic multicast with DCI format 4_2

dynamicMulticastDCI-Format4-2-r17                 ENUMERATED {supported}               OPTIONAL,

-- R1 33-2i: Supported maximal modulation order for multicast PDSCH

maxModulationOrderForMulticast-r17                CHOICE {

fr1-r17                                           ENUMERATED {qam256, qam1024},

fr2-r17                                           ENUMERATED {qam64, qam256}

}                                                                                                                          OPTIONAL,

-- R1 33-3-1: Dynamic Slot-level repetition for group-common PDSCH for TN and licensed

dynamicSlotRepetitionMulticastTN-NonSharedSpectrumChAccess-r17  ENUMERATED {n8, n16}                                       OPTIONAL,

-- R1 33-3-1a: Dynamic Slot-level repetition for group-common PDSCH for NTN and unlicensed

dynamicSlotRepetitionMulticastNTN-SharedSpectrumChAccess-r17    ENUMERATED {n8, n16}                                       OPTIONAL,

-- R1 33-4-1: DCI-based enabling/disabling NACK-only based feedback for dynamic scheduling for multicast

nack-OnlyFeedbackForMulticastWithDCI-Enabler-r17                ENUMERATED {supported}                                     OPTIONAL,

-- R1 33-5-1b: DCI-based enabling/disabling ACK/NACK-based feedback for dynamic scheduling for multicast

ack-NACK-FeedbackForSPS-MulticastWithDCI-Enabler-r17            ENUMERATED {supported}                                     OPTIONAL,

-- R1 33-5-1h: Multiple G-CS-RNTIs for SPS group-common PDSCHs

maxNumberG-CS-RNTI-r17                                          INTEGER (2..8)                                             OPTIONAL,

-- R1 33-10: Support group-common PDSCH RE-level rate matching for multicast

re-LevelRateMatchingForMulticast-r17                            ENUMERATED {supported}                                     OPTIONAL,

-- R1 36-1a: Support of 1024QAM for PDSCH with maximum 2 MIMO layers for FR1

pdsch-1024QAM-2MIMO-FR1-r17                                     ENUMERATED {supported}                                     OPTIONAL,

-- R4 14-3 PRS measurement without MG

prs-MeasurementWithoutMG-r17                                    ENUMERATED {cpLength, quarterSymbol, halfSymbol, halfSlot} OPTIONAL,

-- R4 25-7: The number of target LEO satellites the UE can monitor per carrier

maxNumber-LEO-SatellitesPerCarrier-r17                          INTEGER (3..4)                                             OPTIONAL,

-- R1 27-3-3 DL PRS Processing Capability outside MG - buffering capability

prs-ProcessingCapabilityOutsideMGinPPW-r17    SEQUENCE (SIZE(1..3)) OF PRS-ProcessingCapabilityOutsideMGinPPWperType-r17   OPTIONAL,

-- R1 27-15a: Positioning SRS transmission in RRC_INACTIVE state for initial UL BWP with semi-persistent SRS

srs-SemiPersistent-PosResourcesRRC-Inactive-r17                 SEQUENCE {

maxNumOfSemiPersistentSRSposResources-r17                       ENUMERATED {n1, n2, n4, n8, n16, n32, n64},

maxNumOfSemiPersistentSRSposResourcesPerSlot-r17                ENUMERATED {n1, n2, n3, n4, n5, n6, n8, n10, n12, n14}

}                                                                                                                          OPTIONAL,

-- R2: UE support of CBW for 120kHz SCS

channelBWs-DL-SCS-120kHz-FR2-2-r17                              BIT STRING (SIZE (8))                                      OPTIONAL,

channelBWs-UL-SCS-120kHz-FR2-2-r17                              BIT STRING (SIZE (8))                                      OPTIONAL

]],

[[

-- R1 30-4a: DM-RS bundling for PUSCH repetition type A

dmrs-BundlingPUSCH-RepTypeA-r17                                 ENUMERATED {supported}                                     OPTIONAL,

-- R1 30-4b: DM-RS bundling for PUSCH repetition type B

dmrs-BundlingPUSCH-RepTypeB-r17                                 ENUMERATED {supported}                                     OPTIONAL,

-- R1 30-4c: DM-RS bundling for TB processing over multi-slot PUSCH

dmrs-BundlingPUSCH-multiSlot-r17                                ENUMERATED {supported}                                     OPTIONAL,

-- R1 30-4d: DMRS bundling for PUCCH repetitions

dmrs-BundlingPUCCH-Rep-r17                                      ENUMERATED {supported}                                     OPTIONAL,

-- R1 30-4e: Enhanced inter-slot frequency hopping with inter-slot bundling for PUSCH

interSlotFreqHopInterSlotBundlingPUSCH-r17                      ENUMERATED {supported}                                     OPTIONAL,

-- R1 30-4f: Enhanced inter-slot frequency hopping for PUCCH repetitions with DMRS bundling

interSlotFreqHopPUCCH-r17                                       ENUMERATED {supported}                                     OPTIONAL,

-- R1 30-4g: Restart DM-RS bundling

dmrs-BundlingRestart-r17                                        ENUMERATED {supported}                                     OPTIONAL,

-- R1 30-4h: DM-RS bundling for non-back-to-back transmission

dmrs-BundlingNonBackToBackTX-r17                                ENUMERATED {supported}                                     OPTIONAL

]],

[[

-- R1 33-5-1e: Dynamic Slot-level repetition for SPS group-common PDSCH for multicast

maxDynamicSlotRepetitionForSPS-Multicast-r17                    ENUMERATED {n8, n16}                                       OPTIONAL,

-- R1 33-5-1g: DCI-based enabling/disabling NACK-only based feedback for SPS group-common PDSCH for multicast

nack-OnlyFeedbackForSPS-MulticastWithDCI-Enabler-r17            ENUMERATED {supported}                                     OPTIONAL,

-- R1 33-5-1i: Multicast SPS scheduling with DCI format 4_2

sps-MulticastDCI-Format4-2-r17                                  ENUMERATED {supported}                                     OPTIONAL,

-- R1 33-5-2: Multiple SPS group-common PDSCH configuration on PCell

sps-MulticastMultiConfig-r17                                    INTEGER (1..8)                                             OPTIONAL,

-- R1 33-6-1: DL priority indication for multicast in DCI

priorityIndicatorInDCI-Multicast-r17                            ENUMERATED {supported}                                     OPTIONAL,

-- R1 33-6-1a: DL priority configuration for SPS multicast

priorityIndicatorInDCI-SPS-Multicast-r17                        ENUMERATED {supported}                                     OPTIONAL,

-- R1 33-6-2: Two HARQ-ACK codebooks simultaneously constructed for supporting HARQ-ACK codebooks with different priorities

-- for unicast and multicast at a UE

twoHARQ-ACK-CodebookForUnicastAndMulticast-r17                  ENUMERATED {supported}                                     OPTIONAL,

-- R1 33-6-3: More than one PUCCH for HARQ-ACK transmission for multicast or for unicast and multicast within a slot

multiPUCCH-HARQ-ACK-ForMulticastUnicast-r17                     ENUMERATED {supported}                                     OPTIONAL,

-- R1 33-9: Supporting unicast PDCCH to release SPS group-common PDSCH

releaseSPS-MulticastWithCS-RNTI-r17                             ENUMERATED {supported}                                     OPTIONAL

]]

}

BandNR-v16c0 ::=                                                SEQUENCE {

pusch-RepetitionTypeA-v16c0                                     ENUMERATED {supported}                                     OPTIONAL,

...

}

-- TAG-RF-PARAMETERS-STOP

-- ASN1STOP

RF-Parameters field descriptions

appliedFreqBandListFilter In this field the UE mirrors the FreqBandList that the NW provided in the capability enquiry, if any. The UE filtered the band combinations in the supportedBandCombinationList in accordance with this appliedFreqBandListFilter. The UE does not include this field if the UE capability is requested by E-UTRAN and the network request includes the field eutra-nr-only [10].

supportedBandCombinationList A list of band combinations that the UE supports for NR (and NR-DC, if requested). The FeatureSetCombinationId:s in this list refer to the FeatureSetCombination entries in the featureSetCombinations list in the UE-NR-Capability IE. The UE does not include this field if the UE capability is requested by E-UTRAN and the network request includes the field eutra-nr-only [10].

supportedBandCombinationListSidelinkEUTRA-NR A list of band combinations that the UE supports for NR sidelink communication only, for joint NR sidelink communication and V2X sidelink communication, or for V2X sidelink communication only. The UE does not include this field if the UE capability is requested by E-UTRAN (see TS 36.331[10]) and the network request includes the field eutra-nr-only.

supportedBandCombinationListSL-NonRelayDiscovery A list of band combinations that the UE supports for NR sidelink non-relay discovery. The encoding is defined in PC5 BandCombinationListSidelinkNR-r16.

supportedBandCombinationListSL-RelayDiscovery A list of band combinations that the UE supports for NR sidelink relay discovery. The encoding is defined in PC5 BandCombinationListSidelinkNR-r16.

supportedBandCombinationList-UplinkTxSwitch A list of band combinations that the UE supports dynamic uplink Tx switching for NR UL CA and SUL. The FeatureSetCombinationId:s in this list refer to the FeatureSetCombination entries in the featureSetCombinations list in the UE-NR-Capability IE. The UE does not include this field if the UE capability is requested by E-UTRAN and the network request includes the field eutra-nr-only [10].

supportedBandListNR A list of NR bands supported by the UE. If supportedBandListNR-v16c0 is included, the UE shall include the same number of entries, and listed in the same order, as in supportedBandListNR (without suffix).

#### –	RF-ParametersMRDC

The IE RF-ParametersMRDC is used to convey RF related capabilities for MR-DC.

RF-ParametersMRDC information element

-- ASN1START

-- TAG-RF-PARAMETERSMRDC-START

RF-ParametersMRDC ::=                   SEQUENCE {

supportedBandCombinationList            BandCombinationList                             OPTIONAL,

appliedFreqBandListFilter               FreqBandList                                    OPTIONAL,

...,

[[

srs-SwitchingTimeRequested              ENUMERATED {true}                               OPTIONAL,

supportedBandCombinationList-v1540      BandCombinationList-v1540                       OPTIONAL

]],

[[

supportedBandCombinationList-v1550      BandCombinationList-v1550                       OPTIONAL

]],

[[

supportedBandCombinationList-v1560      BandCombinationList-v1560                       OPTIONAL,

supportedBandCombinationListNEDC-Only   BandCombinationList                             OPTIONAL

]],

[[

supportedBandCombinationList-v1570      BandCombinationList-v1570                       OPTIONAL

]],

[[

supportedBandCombinationList-v1580      BandCombinationList-v1580                       OPTIONAL

]],

[[

supportedBandCombinationList-v1590      BandCombinationList-v1590                       OPTIONAL

]],

[[

supportedBandCombinationListNEDC-Only-v15a0    SEQUENCE {

supportedBandCombinationList-v1540      BandCombinationList-v1540                   OPTIONAL,

supportedBandCombinationList-v1560      BandCombinationList-v1560                   OPTIONAL,

supportedBandCombinationList-v1570      BandCombinationList-v1570                   OPTIONAL,

supportedBandCombinationList-v1580      BandCombinationList-v1580                   OPTIONAL,

supportedBandCombinationList-v1590      BandCombinationList-v1590                   OPTIONAL

}                                                                                       OPTIONAL

]],

[[

supportedBandCombinationList-v1610      BandCombinationList-v1610                       OPTIONAL,

supportedBandCombinationListNEDC-Only-v1610   BandCombinationList-v1610                 OPTIONAL,

supportedBandCombinationList-UplinkTxSwitch-r16 BandCombinationList-UplinkTxSwitch-r16  OPTIONAL

]],

[[

supportedBandCombinationList-v1630                  BandCombinationList-v1630                   OPTIONAL,

supportedBandCombinationListNEDC-Only-v1630         BandCombinationList-v1630                   OPTIONAL,

supportedBandCombinationList-UplinkTxSwitch-v1630   BandCombinationList-UplinkTxSwitch-v1630    OPTIONAL

]],

[[

supportedBandCombinationList-v1640                  BandCombinationList-v1640                   OPTIONAL,

supportedBandCombinationListNEDC-Only-v1640         BandCombinationList-v1640                   OPTIONAL,

supportedBandCombinationList-UplinkTxSwitch-v1640   BandCombinationList-UplinkTxSwitch-v1640    OPTIONAL

]],

[[

supportedBandCombinationList-UplinkTxSwitch-v1670   BandCombinationList-UplinkTxSwitch-v1670    OPTIONAL

]],

[[

supportedBandCombinationList-v1700                  BandCombinationList-v1700                   OPTIONAL,

supportedBandCombinationList-UplinkTxSwitch-v1700   BandCombinationList-UplinkTxSwitch-v1700    OPTIONAL

]],

[[

supportedBandCombinationList-v1720                  BandCombinationList-v1720                   OPTIONAL,

supportedBandCombinationListNEDC-Only-v1720         SEQUENCE {

supportedBandCombinationList-v1700                  BandCombinationList-v1700               OPTIONAL,

supportedBandCombinationList-v1720                  BandCombinationList-v1720               OPTIONAL

}                                                                                               OPTIONAL,

supportedBandCombinationList-UplinkTxSwitch-v1720   BandCombinationList-UplinkTxSwitch-v1720    OPTIONAL

]],

[[

supportedBandCombinationList-v1730                  BandCombinationList-v1730                   OPTIONAL,

supportedBandCombinationListNEDC-Only-v1730         BandCombinationList-v1730                   OPTIONAL,

supportedBandCombinationList-UplinkTxSwitch-v1730   BandCombinationList-UplinkTxSwitch-v1730    OPTIONAL

]],

[[

supportedBandCombinationList-v1740                  BandCombinationList-v1740                   OPTIONAL,

supportedBandCombinationListNEDC-Only-v1740         BandCombinationList-v1740                   OPTIONAL,

supportedBandCombinationList-UplinkTxSwitch-v1740   BandCombinationList-UplinkTxSwitch-v1740    OPTIONAL

]]

}

RF-ParametersMRDC-v15g0 ::=                    SEQUENCE {

supportedBandCombinationList-v15g0             BandCombinationList-v15g0        OPTIONAL,

supportedBandCombinationListNEDC-Only-v15g0    BandCombinationList-v15g0        OPTIONAL

}

-- TAG-RF-PARAMETERSMRDC-STOP

-- ASN1STOP

RF-ParametersMRDC field descriptions

appliedFreqBandListFilter In this field the UE mirrors the FreqBandList that the NW provided in the capability enquiry, if any. The UE filtered the band combinations in the supportedBandCombinationList in accordance with this appliedFreqBandListFilter.

supportedBandCombinationList A list of band combinations that the UE supports for (NG)EN-DC, or both (NG)EN-DC and NE-DC. The FeatureSetCombinationId:s in this list refer to the FeatureSetCombination entries in the featureSetCombinations list in the UE-MRDC-Capability IE.

supportedBandCombinationListNEDC-Only, supportedBandCombinationListNEDC-Only-v1610 A list of band combinations that the UE supports only for NE-DC. The FeatureSetCombinationId:s in this list refer to the FeatureSetCombination entries in the featureSetCombinations list in the UE-MRDC-Capability IE.

supportedBandCombinationList-UplinkTxSwitch A list of band combinations that the UE supports dynamic UL Tx switching for (NG)EN-DC. The FeatureSetCombinationId:s in this list refer to the FeatureSetCombination entries in the featureSetCombinations list in the UE-MRDC-Capability IE.

#### –	RLC-Parameters

The IE RLC-Parameters is used to convey capabilities related to RLC.

RLC-Parameters information element

-- ASN1START

-- TAG-RLC-PARAMETERS-START

RLC-Parameters ::= SEQUENCE {

am-WithShortSN                  ENUMERATED {supported}  OPTIONAL,

um-WithShortSN                  ENUMERATED {supported}  OPTIONAL,

um-WithLongSN                   ENUMERATED {supported}  OPTIONAL,

...,

[[

extendedT-PollRetransmit-r16    ENUMERATED {supported}  OPTIONAL,

extendedT-StatusProhibit-r16    ENUMERATED {supported}  OPTIONAL

]],

[[

am-WithLongSN-RedCap-r17        ENUMERATED {supported}  OPTIONAL

]]

}

-- TAG-RLC-PARAMETERS-STOP

-- ASN1STOP

#### –	SDAP-Parameters

The IE SDAP-Parameters is used to convey capabilities related to SDAP.

SDAP-Parameters information element

-- ASN1START

-- TAG-SDAP-PARAMETERS-START

SDAP-Parameters ::= SEQUENCE {

as-ReflectiveQoS                 ENUMERATED {true}               OPTIONAL,

...,

[[

sdap-QOS-IAB-r16              ENUMERATED {supported}       OPTIONAL,

sdapHeaderIAB-r16             ENUMERATED {supported}       OPTIONAL

]]

}

-- TAG-SDAP-PARAMETERS-STOP

-- ASN1STOP

#### –	SidelinkParameters

The IE SidelinkParameters is used to convey capabilities related to NR and V2X sidelink communications.

SidelinkParameters information element

-- ASN1START

-- TAG-SIDELINKPARAMETERS-START

SidelinkParameters-r16 ::=    SEQUENCE {

sidelinkParametersNR-r16                  SidelinkParametersNR-r16                                                  OPTIONAL,

sidelinkParametersEUTRA-r16               SidelinkParametersEUTRA-r16                                               OPTIONAL

}

SidelinkParametersNR-r16 ::= SEQUENCE {

rlc-ParametersSidelink-r16                RLC-ParametersSidelink-r16                                                OPTIONAL,

mac-ParametersSidelink-r16                MAC-ParametersSidelink-r16                                                OPTIONAL,

fdd-Add-UE-Sidelink-Capabilities-r16      UE-SidelinkCapabilityAddXDD-Mode-r16                                      OPTIONAL,

tdd-Add-UE-Sidelink-Capabilities-r16      UE-SidelinkCapabilityAddXDD-Mode-r16                                      OPTIONAL,

supportedBandListSidelink-r16             SEQUENCE (SIZE (1..maxBands)) OF BandSidelink-r16                         OPTIONAL,

...,

[[

relayParameters-r17                       RelayParameters-r17                                                       OPTIONAL

]],

[[

-- R1 32-x: Use of new P0 parameters for open loop power control

p0-OLPC-Sidelink-r17                      ENUMERATED {supported}                                                    OPTIONAL

]]

}

SidelinkParametersEUTRA-r16 ::= SEQUENCE {

sl-ParametersEUTRA1-r16                   OCTET STRING                                                              OPTIONAL,

sl-ParametersEUTRA2-r16                   OCTET STRING                                                              OPTIONAL,

sl-ParametersEUTRA3-r16                   OCTET STRING                                                              OPTIONAL,

supportedBandListSidelinkEUTRA-r16        SEQUENCE (SIZE (1..maxBandsEUTRA)) OF BandSidelinkEUTRA-r16               OPTIONAL,

...

}

RLC-ParametersSidelink-r16 ::= SEQUENCE {

am-WithLongSN-Sidelink-r16                ENUMERATED {supported}                                                    OPTIONAL,

um-WithLongSN-Sidelink-r16                ENUMERATED {supported}                                                    OPTIONAL,

...

}

MAC-ParametersSidelink-r16 ::= SEQUENCE {

mac-ParametersSidelinkCommon-r16          MAC-ParametersSidelinkCommon-r16                                          OPTIONAL,

mac-ParametersSidelinkXDD-Diff-r16        MAC-ParametersSidelinkXDD-Diff-r16                                        OPTIONAL,

...

}

UE-SidelinkCapabilityAddXDD-Mode-r16 ::=  SEQUENCE {

mac-ParametersSidelinkXDD-Diff-r16        MAC-ParametersSidelinkXDD-Diff-r16                                        OPTIONAL

}

MAC-ParametersSidelinkCommon-r16 ::= SEQUENCE {

lcp-RestrictionSidelink-r16               ENUMERATED {supported}                                                    OPTIONAL,

multipleConfiguredGrantsSidelink-r16      ENUMERATED {supported}                                                    OPTIONAL,

...,

[[

drx-OnSidelink-r17                        ENUMERATED {supported}                                                    OPTIONAL

]]

}

MAC-ParametersSidelinkXDD-Diff-r16 ::=  SEQUENCE {

multipleSR-ConfigurationsSidelink-r16     ENUMERATED {supported}                                                    OPTIONAL,

logicalChannelSR-DelayTimerSidelink-r16   ENUMERATED {supported}                                                    OPTIONAL,

...

}

BandSidelinkEUTRA-r16 ::=               SEQUENCE {

freqBandSidelinkEUTRA-r16               FreqBandIndicatorEUTRA,

-- R1 15-7: Transmitting LTE sidelink mode 3 scheduled by NR Uu

gnb-ScheduledMode3SidelinkEUTRA-r16     SEQUENCE {

gnb-ScheduledMode3DelaySidelinkEUTRA-r16 ENUMERATED {ms0, ms0dot25, ms0dot5, ms0dot625, ms0dot75, ms1,

ms1dot25, ms1dot5, ms1dot75, ms2, ms2dot5, ms3, ms4,

ms5, ms6, ms8, ms10, ms20}

}                                                                                                                   OPTIONAL,

-- R1 15-9: Transmitting LTE sidelink mode 4 configured by NR Uu

gnb-ScheduledMode4SidelinkEUTRA-r16     ENUMERATED {supported}                                                      OPTIONAL

}

BandSidelink-r16 ::=  SEQUENCE {

freqBandSidelink-r16                          FreqBandIndicatorNR,

--15-1

sl-Reception-r16                              SEQUENCE {

harq-RxProcessSidelink-r16                    ENUMERATED {n16, n24, n32, n48, n64},

pscch-RxSidelink-r16                          ENUMERATED {value1, value2},

scs-CP-PatternRxSidelink-r16                  CHOICE {

fr1-r16                                       SEQUENCE {

scs-15kHz-r16                                 BIT STRING (SIZE (16))                OPTIONAL,

scs-30kHz-r16                                 BIT STRING (SIZE (16))                OPTIONAL,

scs-60kHz-r16                                 BIT STRING (SIZE (16))                OPTIONAL

},

fr2-r16                                       SEQUENCE {

scs-60kHz-r16                                 BIT STRING (SIZE (16))                OPTIONAL,

scs-120kHz-r16                                BIT STRING (SIZE (16))                OPTIONAL

}

}                                                                                           OPTIONAL,

extendedCP-RxSidelink-r16                     ENUMERATED {supported}                        OPTIONAL

}                                                                                               OPTIONAL,

--15-2

sl-TransmissionMode1-r16                      SEQUENCE {

harq-TxProcessModeOneSidelink-r16             ENUMERATED {n8, n16},

scs-CP-PatternTxSidelinkModeOne-r16           CHOICE {

fr1-r16                                       SEQUENCE {

scs-15kHz-r16                                 BIT STRING (SIZE (16))                OPTIONAL,

scs-30kHz-r16                                 BIT STRING (SIZE (16))                OPTIONAL,

scs-60kHz-r16                                 BIT STRING (SIZE (16))                OPTIONAL

},

fr2-r16                                       SEQUENCE {

scs-60kHz-r16                                 BIT STRING (SIZE (16))                OPTIONAL,

scs-120kHz-r16                                BIT STRING (SIZE (16))                OPTIONAL

}

},

extendedCP-TxSidelink-r16                     ENUMERATED {supported}                        OPTIONAL,

harq-ReportOnPUCCH-r16                        ENUMERATED {supported}                        OPTIONAL

}                                                                                               OPTIONAL,

--15-4

sync-Sidelink-r16                             SEQUENCE {

gNB-Sync-r16                                  ENUMERATED {supported}                        OPTIONAL,

gNB-GNSS-UE-SyncWithPriorityOnGNB-ENB-r16     ENUMERATED {supported}                        OPTIONAL,

gNB-GNSS-UE-SyncWithPriorityOnGNSS-r16        ENUMERATED {supported}                        OPTIONAL

}                                                                                               OPTIONAL,

--15-10

sl-Tx-256QAM-r16                              ENUMERATED {supported}                            OPTIONAL,

--15-11

psfch-FormatZeroSidelink-r16                  SEQUENCE {

psfch-RxNumber                                ENUMERATED {n5, n15, n25, n32, n35, n45, n50, n64},

psfch-TxNumber                                ENUMERATED {n4, n8, n16}

}                                                                                               OPTIONAL,

--15-12

lowSE-64QAM-MCS-TableSidelink-r16             ENUMERATED {supported}                            OPTIONAL,

--15-15

enb-sync-Sidelink-r16                         ENUMERATED {supported}                            OPTIONAL,

...,

[[

--15-3

sl-TransmissionMode2-r16                      SEQUENCE {

harq-TxProcessModeTwoSidelink-r16              ENUMERATED {n8, n16},

scs-CP-PatternTxSidelinkModeTwo-r16           ENUMERATED {supported}                        OPTIONAL,

dl-openLoopPC-Sidelink-r16                    ENUMERATED {supported}                        OPTIONAL

}                                                                                               OPTIONAL,

--15-5

congestionControlSidelink-r16                 SEQUENCE {

cbr-ReportSidelink-r16                        ENUMERATED {supported}                        OPTIONAL,

cbr-CR-TimeLimitSidelink-r16                  ENUMERATED {time1, time2}

}                                                                                               OPTIONAL,

--15-22

fewerSymbolSlotSidelink-r16                   ENUMERATED {supported}                            OPTIONAL,

--15-23

sl-openLoopPC-RSRP-ReportSidelink-r16         ENUMERATED {supported}                            OPTIONAL,

--13-1

sl-Rx-256QAM-r16                              ENUMERATED {supported}                            OPTIONAL

]],

[[

ue-PowerClassSidelink-r16                         ENUMERATED {pc2, pc3, spare6, spare5, spare4, spare3, spare2, spare1}

OPTIONAL

]],

[[

--32-4a

sl-TransmissionMode2-RandomResourceSelection-r17 SEQUENCE {

harq-TxProcessModeTwoSidelink-r17             ENUMERATED {n8, n16},

scs-CP-PatternTxSidelinkModeTwo-r17               CHOICE {

fr1-r17                                           SEQUENCE {

scs-15kHz-r17                                     BIT STRING (SIZE (16))            OPTIONAL,

scs-30kHz-r17                                     BIT STRING (SIZE (16))            OPTIONAL,

scs-60kHz-r17                                     BIT STRING (SIZE (16))            OPTIONAL

},

fr2-r17                                           SEQUENCE {

scs-60kHz-r17                                     BIT STRING (SIZE (16))            OPTIONAL,

scs-120kHz-r17                                    BIT STRING (SIZE (16))            OPTIONAL

}

}                                                                                           OPTIONAL,

extendedCP-Mode2Random-r17                    ENUMERATED {supported}                        OPTIONAL,

dl-openLoopPC-Sidelink-r17                    ENUMERATED {supported}                            OPTIONAL

}                                                                                               OPTIONAL,

--32-4b

sync-Sidelink-v1710                           SEQUENCE {

sync-GNSS-r17                                 ENUMERATED {supported}                        OPTIONAL,

gNB-Sync-r17                                  ENUMERATED {supported}                        OPTIONAL,

gNB-GNSS-UE-SyncWithPriorityOnGNB-ENB-r17     ENUMERATED {supported}                        OPTIONAL,

gNB-GNSS-UE-SyncWithPriorityOnGNSS-r17        ENUMERATED {supported}                        OPTIONAL

}                                                                                               OPTIONAL,

--32-4c

enb-sync-Sidelink-v1710                       ENUMERATED {supported}                            OPTIONAL,

--32-5a-2

rx-IUC-Scheme1-PreferredMode2Sidelink-r17     ENUMERATED {supported}                            OPTIONAL,

--32-5a-3

rx-IUC-Scheme1-NonPreferredMode2Sidelink-r17  ENUMERATED {supported}                            OPTIONAL,

--32-5b-2

rx-IUC-Scheme2-Mode2Sidelink-r17              ENUMERATED {n5, n15, n25, n32, n35, n45, n50, n64} OPTIONAL,

--32-6-1

rx-IUC-Scheme1-SCI-r17                        ENUMERATED {supported}                            OPTIONAL,

--32-6-2

rx-IUC-Scheme1-SCI-ExplicitReq-r17            ENUMERATED {supported}                            OPTIONAL

]]

}

RelayParameters-r17 ::= SEQUENCE {

relayUE-Operation-L2-r17                         ENUMERATED {supported}                                 OPTIONAL,

remoteUE-Operation-L2-r17                        ENUMERATED {supported}                                 OPTIONAL,

remoteUE-PathSwitchToIdleInactiveRelay-r17    ENUMERATED {supported}                                 OPTIONAL,

...

}

-- TAG-SIDELINKPARAMETERS-STOP

-- ASN1STOP

SidelinkParametersEUTRA field descriptions

sl-ParametersEUTRA1, sl-ParametersEUTRA2, sl-ParametersEUTRA3 This field includes IE of SL-Parameters-v1430 (where v2x-eNB-Scheduled-r14 and V2X-SupportedBandCombination-r14 shall not be included), SL-Parameters-v1530 (where V2X-SupportedBandCombination-r1530 shall not be included) and SL-Parameters-v1540 respectively defined in 36.331 [10]. It is used for reporting the per-UE capability for V2X sidelink communication.

#### –	SimultaneousRxTxPerBandPair

The IE SimultaneousRxTxPerBandPair contains the simultaneous Rx/Tx UE capability for each band pair in a band combination.

SimultaneousRxTxPerBandPair information element

-- ASN1START

-- TAG-SIMULTANEOUSRXTXPERBANDPAIR-START

SimultaneousRxTxPerBandPair ::=             BIT STRING (SIZE (3..496))

-- TAG-SIMULTANEOUSRXTXPERBANDPAIR-STOP

-- ASN1STOP

#### –	SON-Parameters

The IE SON-Parameters contains SON related parameters.

SON-Parameters information element

-- ASN1START

-- TAG-SON-PARAMETERS-START

SON-Parameters-r16 ::= SEQUENCE {

rach-Report-r16        ENUMERATED {supported}    OPTIONAL,

...,

[[

rlfReportCHO-r17       ENUMERATED {supported}    OPTIONAL,

rlfReportDAPS-r17      ENUMERATED {supported}    OPTIONAL,

success-HO-Report-r17  ENUMERATED {supported}    OPTIONAL,

twoStepRACH-Report-r17 ENUMERATED {supported}    OPTIONAL,

pscell-MHI-Report-r17  ENUMERATED {supported}    OPTIONAL,

onDemandSI-Report-r17  ENUMERATED {supported}    OPTIONAL

]]

}

-- TAG-SON-PARAMETERS-STOP

-- ASN1STOP

#### –	SpatialRelationsSRS-Pos

The IE SpatialRelationsSRS-Pos is used to convey spatial relation for SRS for positioning related parameters.

SpatialRelationsSRS-Pos information element

-- ASN1START

-- TAG-SPATIALRELATIONSSRS-POS-START

SpatialRelationsSRS-Pos-r16 ::=                    SEQUENCE {

spatialRelation-SRS-PosBasedOnSSB-Serving-r16      ENUMERATED {supported}                OPTIONAL,

spatialRelation-SRS-PosBasedOnCSI-RS-Serving-r16   ENUMERATED {supported}                OPTIONAL,

spatialRelation-SRS-PosBasedOnPRS-Serving-r16      ENUMERATED {supported}                OPTIONAL,

spatialRelation-SRS-PosBasedOnSRS-r16              ENUMERATED {supported}                OPTIONAL,

spatialRelation-SRS-PosBasedOnSSB-Neigh-r16        ENUMERATED {supported}                OPTIONAL,

spatialRelation-SRS-PosBasedOnPRS-Neigh-r16        ENUMERATED {supported}                OPTIONAL

}

--TAG-SPATIALRELATIONSSRS-POS-STOP

-- ASN1STOP

#### –	SRS-AllPosResourcesRRC-Inactive

The IE SRS-AllPosResourcesRRC-Inactive is used to convey SRS positioning related parameters specific for a certain band.

SRS-AllPosResourcesRRC-Inactive information element

-- ASN1START

-- TAG-SRS-ALLPOSRESOURCESRRC-INACTIVE-START

SRS-AllPosResourcesRRC-Inactive-r17 ::=             SEQUENCE {

srs-PosResourcesRRC-Inactive-r17                    SEQUENCE {

-- R1 27-15: Positioning SRS transmission in RRC_INACTIVE state for initial UL BWP

maxNumberSRS-PosResourceSetPerBWP-r17               ENUMERATED {n1, n2, n4, n8, n12, n16},

maxNumberSRS-PosResourcesPerBWP-r17                 ENUMERATED {n1, n2, n4, n8, n16, n32, n64},

maxNumberSRS-ResourcesPerBWP-PerSlot-r17            ENUMERATED {n1, n2, n3, n4, n5, n6, n8, n10, n12, n14},

maxNumberPeriodicSRS-PosResourcesPerBWP-r17         ENUMERATED {n1, n2, n4, n8, n16, n32, n64},

maxNumberPeriodicSRS-PosResourcesPerBWP-PerSlot-r17 ENUMERATED {n1, n2, n3, n4, n5, n6, n8, n10, n12, n14},

dummy1                                              ENUMERATED {n1, n2, n4, n8, n16, n32, n64 },

dummy2                                              ENUMERATED {n1, n2, n3, n4, n5, n6, n8, n10, n12, n14}

}

}

-- TAG-SRS-ALLPOSRESOURCESRRC-INACTIVE-STOP

-- ASN1STOP

SRS-AllPosResourcesRRC-Inactive field descriptions

dummy1, dummy2 The fields are not used in the specification and the network ignores the received values.

#### –	SRS-SwitchingTimeNR

The IE SRS-SwitchingTimeNR is used to indicate the SRS carrier switching time supported by the UE for one NR band pair.

SRS-SwitchingTimeNR information element

-- ASN1START

-- TAG-SRS-SWITCHINGTIMENR-START

SRS-SwitchingTimeNR ::= SEQUENCE {

switchingTimeDL         ENUMERATED {n0us, n30us, n100us, n140us, n200us, n300us, n500us, n900us}  OPTIONAL,

switchingTimeUL         ENUMERATED {n0us, n30us, n100us, n140us, n200us, n300us, n500us, n900us}  OPTIONAL

}

-- TAG-SRS-SWITCHINGTIMENR-STOP

-- ASN1STOP

#### –	SRS-SwitchingTimeEUTRA

The IE SRS-SwitchingTimeEUTRA is used to indicate the SRS carrier switching time supported by the UE for one E-UTRA band pair.

SRS-SwitchingTimeEUTRA information element

-- ASN1START

-- TAG-SRS-SWITCHINGTIMEEUTRA-START

SRS-SwitchingTimeEUTRA ::= SEQUENCE {

switchingTimeDL            ENUMERATED {n0, n0dot5, n1, n1dot5, n2, n2dot5, n3, n3dot5, n4, n4dot5, n5, n5dot5, n6, n6dot5, n7}

OPTIONAL,

switchingTimeUL            ENUMERATED {n0, n0dot5, n1, n1dot5, n2, n2dot5, n3, n3dot5, n4, n4dot5, n5, n5dot5, n6, n6dot5, n7}

OPTIONAL

}

-- TAG-SRS-SWITCHINGTIMEEUTRA-STOP

-- ASN1STOP

#### –	SupportedBandwidth

The IE SupportedBandwidth is used to indicate the channel bandwidth supported by the UE on one carrier of a band of a band combination.

SupportedBandwidth information element

-- ASN1START

-- TAG-SUPPORTEDBANDWIDTH-START

SupportedBandwidth ::=      CHOICE {

fr1                         ENUMERATED {mhz5, mhz10, mhz15, mhz20, mhz25, mhz30, mhz40, mhz50, mhz60, mhz80, mhz100},

fr2                         ENUMERATED {mhz50, mhz100, mhz200, mhz400}

}

SupportedBandwidth-v1700 ::= CHOICE {

fr1-r17    ENUMERATED {mhz5, mhz10, mhz15, mhz20, mhz25, mhz30, mhz35, mhz40, mhz45, mhz50, mhz60, mhz70, mhz80, mhz90, mhz100},

fr2-r17    ENUMERATED {mhz50, mhz100, mhz200, mhz400, mhz800, mhz1600, mhz2000}

}

-- TAG-SUPPORTEDBANDWIDTH-STOP

-- ASN1STOP

#### –	UE-BasedPerfMeas-Parameters

The IE UE-BasedPerfMeas-Parameters contains UE-based performance measurement parameters.

UE-BasedPerfMeas-Parameters information element

-- ASN1START

-- TAG-UE-BASEDPERFMEAS-PARAMETERS-START

UE-BasedPerfMeas-Parameters-r16 ::= SEQUENCE {

barometerMeasReport-r16      ENUMERATED {supported}        OPTIONAL,

immMeasBT-r16                ENUMERATED {supported}        OPTIONAL,

immMeasWLAN-r16              ENUMERATED {supported}        OPTIONAL,

loggedMeasBT-r16             ENUMERATED {supported}        OPTIONAL,

loggedMeasurements-r16       ENUMERATED {supported}        OPTIONAL,

loggedMeasWLAN-r16           ENUMERATED {supported}        OPTIONAL,

orientationMeasReport-r16    ENUMERATED {supported}        OPTIONAL,

speedMeasReport-r16          ENUMERATED {supported}        OPTIONAL,

gnss-Location-r16            ENUMERATED {supported}        OPTIONAL,

ulPDCP-Delay-r16             ENUMERATED {supported}        OPTIONAL,

...,

[[

sigBasedLogMDT-OverrideProtect-r17 ENUMERATED {supported}  OPTIONAL,

multipleCEF-Report-r17             ENUMERATED {supported}  OPTIONAL,

excessPacketDelay-r17              ENUMERATED {supported}  OPTIONAL,

earlyMeasLog-r17                   ENUMERATED {supported}  OPTIONAL

]]

}

-- TAG-UE-BASEDPERFMEAS-PARAMETERS-STOP

-- ASN1STOP

#### –	UE-CapabilityRAT-ContainerList

The IE UE-CapabilityRAT-ContainerList contains a list of radio access technology specific capability containers.

UE-CapabilityRAT-ContainerList information element

-- ASN1START

-- TAG-UE-CAPABILITYRAT-CONTAINERLIST-START

UE-CapabilityRAT-ContainerList ::=    SEQUENCE (SIZE (0..maxRAT-CapabilityContainers)) OF UE-CapabilityRAT-Container

UE-CapabilityRAT-Container ::=        SEQUENCE {

rat-Type                              RAT-Type,

ue-CapabilityRAT-Container            OCTET STRING

}

-- TAG-UE-CAPABILITYRAT-CONTAINERLIST-STOP

-- ASN1STOP

UE-CapabilityRAT-ContainerList field descriptions

ue-CapabilityRAT-Container Container for the UE capabilities of the indicated RAT. The encoding is defined in the specification of each RAT: For rat-Type set to nr: the encoding of UE capabilities is defined in UE-NR-Capability. For rat-Type set to eutra-nr: the encoding of UE capabilities is defined in UE-MRDC-Capability. For rat-Type set to eutra: the encoding of UE capabilities is defined in UE-EUTRA-Capability specified in TS 36.331 [10]. For rat-Type set to utra-fdd: the octet string contains the INTER RAT HANDOVER INFO message defined in TS 25.331 [45].

#### –	UE-CapabilityRAT-RequestList

The IE UE-CapabilityRAT-RequestList is used to request UE capabilities for one or more RATs from the UE.

UE-CapabilityRAT-RequestList information element

-- ASN1START

-- TAG-UE-CAPABILITYRAT-REQUESTLIST-START

UE-CapabilityRAT-RequestList ::=        SEQUENCE (SIZE (1..maxRAT-CapabilityContainers)) OF UE-CapabilityRAT-Request

UE-CapabilityRAT-Request ::=            SEQUENCE {

rat-Type                                RAT-Type,

capabilityRequestFilter                 OCTET STRING                    OPTIONAL,   -- Need N

...

}

-- TAG-UE-CAPABILITYRAT-REQUESTLIST-STOP

-- ASN1STOP

UE-CapabilityRAT-Request field descriptions

capabilityRequestFilter Information by which the network requests the UE to filter the UE capabilities. For rat-Type set to nr or eutra-nr: the encoding of the capabilityRequestFilter is defined in UE-CapabilityRequestFilterNR. For rat-Type set to eutra: the encoding of the capabilityRequestFilter is defined by UECapabilityEnquiry message defined in TS36.331 [10], in which RAT-Type in UE-CapabilityRequest includes only 'eutra'.

rat-Type The RAT type for which the NW requests UE capabilities.

#### –	UE-CapabilityRequestFilterCommon

The IE UE-CapabilityRequestFilterCommon is used to request filtered UE capabilities. The filter is common for all capability containers that are requested.

UE-CapabilityRequestFilterCommon information element

-- ASN1START

-- TAG-UE-CAPABILITYREQUESTFILTERCOMMON-START

UE-CapabilityRequestFilterCommon ::=            SEQUENCE {

mrdc-Request                                SEQUENCE {

omitEN-DC                                   ENUMERATED {true}                      OPTIONAL,    -- Need N

includeNR-DC                                ENUMERATED {true}                      OPTIONAL,    -- Need N

includeNE-DC                                ENUMERATED {true}                      OPTIONAL     -- Need N

}                                                                                  OPTIONAL,        -- Need N

...,

[[

codebookTypeRequest-r16        SEQUENCE {

type1-SinglePanel-r16          ENUMERATED {true}                                    OPTIONAL,    -- Need N

type1-MultiPanel-r16           ENUMERATED {true}                                    OPTIONAL,    -- Need N

type2-r16                      ENUMERATED {true}                                    OPTIONAL,    -- Need N

type2-PortSelection-r16        ENUMERATED {true}                                    OPTIONAL     -- Need N

}                                                                                   OPTIONAL,    -- Need N

uplinkTxSwitchRequest-r16      ENUMERATED {true}                                    OPTIONAL     -- Need N

]],

[[

requestedCellGrouping-r16      SEQUENCE (SIZE (1..maxCellGroupings-r16)) OF CellGrouping-r16    OPTIONAL    -- Cond NRDC

]],

[[

fallbackGroupFiveRequest-r17   ENUMERATED {true}                                    OPTIONAL     -- Need N

]]

}

CellGrouping-r16 ::=    SEQUENCE {

mcg-r16                 SEQUENCE (SIZE (1..maxBands)) OF FreqBandIndicatorNR,

scg-r16                 SEQUENCE (SIZE (1..maxBands)) OF FreqBandIndicatorNR,

mode-r16                ENUMERATED {sync, async}

}

-- TAG-UE-CAPABILITYREQUESTFILTERCOMMON-STOP

-- ASN1STOP

UE-CapabilityRequestFilterCommon field descriptions

codebookTypeRequest Only if this field is present, the UE includes SupportedCSI-RS-Resource supported for the codebook type(s) requested within this field (i.e. type I single/multi-panel, type II and type II port selection) into codebookVariantsList, codebookParametersPerBand and codebookParametersPerBC. If this field is present and none of the codebook types is requested within this field (i.e. empty field), the UE includes SupportedCSI-RS-Resource supported for all codebook types into codebookVariantsList, codebookParametersPerBand and codebookParametersPerBC.

fallbackGroupFiveRequest Only if this field is present, the UE supporting FR2 CA bandwidth class from fallback group 5 shall include band combinations with FR2 CA bandwidth class from fallback group 5, and shall omit band combinations with FR2 CA bandwidth class from fallback group 2 or 3 (see TS 38.101-2 [39]) with same or lower capabilities.

includeNE-DC Only if this field is present, the UE supporting NE-DC shall indicate support for NE-DC in band combinations and include feature set combinations which are applicable to NE-DC. Band combinations supporting both NE-DC and (NG)EN-DC shall be included in supportedBandCombinationList, band combinations supporting only NE-DC shall be included in supportedBandCombinationListNEDC-Only.

includeNR-DC Only if this field is present, the UE supporting NR-DC shall indicate support for NR-DC in band combinations and include feature set combinations which are applicable to NR-DC.

mode The mode of NR-DC operation that the NW is interested in for this cell grouping. The value sync means that the UE only indicates NR-DC support for band combinations for which it supports synchronous NR-DC with the requested cell grouping. The value async means that the UE only indicates NR-DC support for band combinations for which it supports asynchronous NR-DC with the requested cell grouping.

omitEN-DC Only if this field is present, the UE shall omit band combinations and feature set combinations which are only applicable to (NG)EN-DC.

requestedCellGrouping The NR-DC cell groupings that the NW is interested in, i.e., the bands that it might use in an MCG and the bands that it might use in an SCG. Only if this field is present, the UE indicates NR-DC support for band combinations for which it supports the requested cell grouping, i.e., in which it supports at least one of the mcg bands on MCG and at least one of the scg bands on the SCG. In its supportedBandCombinationList, the UE indicates which of its NR-DC band combinations supports which of the requested cell groupings. The first element in this list is referred to by ID#0, the second by ID#1 and so on. If this field is absent, the UE only includes band combinations for which it supports NR-DC with only FR1 bands in MCG and only FR2 bands in SCG. Example 1: requestedCellGrouping is set to mcg=[n1, n7, n41, n66] and scg=[n78, n261]. This assumes that the NW would always use CA among n1, n7, n41 and n66 (depending on which are deployed on a given site) whereas with n78 and/or n261 the NW may need to use DC. With this filter a UE may report a band combination n1A-n7A-n78A for NR-DC only if it supports that serving cells for n1 and n7 are in the MCG and a serving cell for n78 is in the SCG. The UE may also report a band combination n41C-n261M for NR-DC provided that it supports a serving cell for n41 in the MCG and a serving cell for n261 in the SCG. Example 2: One requestedCellGrouping is set to mcg=[n1, n7, n41, n66] and scg=[n78, n261] and another requestedCellGrouping is set to mcg=[n1, n7, n66] and scg=[ n41, n78, n261]. This assumes that the NW uses sometimes CA among n1, n7, n41 and n66 (as in example 1) and sometimes CA among n1, n7 and n66 but DC towards one or several of n41, n78, n261. If a UE supports n1A-n41A-n78A only if n41A and n78A are in the same cell group, this UE may only indicate cell grouping ID#1 (not #0) in its BC.

uplinkTxSwitchRequest Only if this field is present, the UE supporting dynamic UL Tx switching shall indicate support for UL Tx switching in band combinations which are applicable to inter-band UL CA, SUL and (NG)EN-DC.

| Conditional Presence | Explanation |
| --- | --- |
| NRDC | The field is optionally present, Need N, if includeNR-DC is included. It is absent otherwise. |

#### –	UE-CapabilityRequestFilterNR

The IE UE-CapabilityRequestFilterNR is used to request filtered UE capabilities.

UE-CapabilityRequestFilterNR information element

-- ASN1START

-- TAG-UE-CAPABILITYREQUESTFILTERNR-START

UE-CapabilityRequestFilterNR ::=            SEQUENCE {

frequencyBandListFilter                     FreqBandList                          OPTIONAL,   -- Need N

nonCriticalExtension                        UE-CapabilityRequestFilterNR-v1540    OPTIONAL

}

UE-CapabilityRequestFilterNR-v1540 ::=      SEQUENCE {

srs-SwitchingTimeRequest                    ENUMERATED {true}                     OPTIONAL,  -- Need N

nonCriticalExtension                        UE-CapabilityRequestFilterNR-v1710    OPTIONAL

}

UE-CapabilityRequestFilterNR-v1710 ::=      SEQUENCE {

sidelinkRequest-r17                         ENUMERATED {true}                     OPTIONAL,  -- Need N

nonCriticalExtension                        SEQUENCE {}                           OPTIONAL

}

-- TAG-UE-CAPABILITYREQUESTFILTERNR-STOP

-- ASN1STOP

#### –	UE-MRDC-Capability

The IE UE-MRDC-Capability is used to convey the UE Radio Access Capability Parameters for MR-DC, see TS 38.306 [26].

UE-MRDC-Capability information element

-- ASN1START

-- TAG-UE-MRDC-CAPABILITY-START

UE-MRDC-Capability ::=              SEQUENCE {

measAndMobParametersMRDC            MeasAndMobParametersMRDC                                                        OPTIONAL,

phy-ParametersMRDC-v1530            Phy-ParametersMRDC                                                              OPTIONAL,

rf-ParametersMRDC                   RF-ParametersMRDC,

generalParametersMRDC               GeneralParametersMRDC-XDD-Diff                                                  OPTIONAL,

fdd-Add-UE-MRDC-Capabilities        UE-MRDC-CapabilityAddXDD-Mode                                                   OPTIONAL,

tdd-Add-UE-MRDC-Capabilities        UE-MRDC-CapabilityAddXDD-Mode                                                   OPTIONAL,

fr1-Add-UE-MRDC-Capabilities        UE-MRDC-CapabilityAddFRX-Mode                                                   OPTIONAL,

fr2-Add-UE-MRDC-Capabilities        UE-MRDC-CapabilityAddFRX-Mode                                                   OPTIONAL,

featureSetCombinations              SEQUENCE (SIZE (1..maxFeatureSetCombinations)) OF FeatureSetCombination         OPTIONAL,

pdcp-ParametersMRDC-v1530           PDCP-ParametersMRDC                                                             OPTIONAL,

lateNonCriticalExtension            OCTET STRING (CONTAINING UE-MRDC-Capability-v15g0)                              OPTIONAL,

nonCriticalExtension                UE-MRDC-Capability-v1560                                                        OPTIONAL

}

-- Regular non-critical extensions:

UE-MRDC-Capability-v1560 ::=        SEQUENCE {

receivedFilters                     OCTET STRING (CONTAINING UECapabilityEnquiry-v1560-IEs)                         OPTIONAL,

measAndMobParametersMRDC-v1560      MeasAndMobParametersMRDC-v1560                                                  OPTIONAL,

fdd-Add-UE-MRDC-Capabilities-v1560  UE-MRDC-CapabilityAddXDD-Mode-v1560                                             OPTIONAL,

tdd-Add-UE-MRDC-Capabilities-v1560  UE-MRDC-CapabilityAddXDD-Mode-v1560                                             OPTIONAL,

nonCriticalExtension                UE-MRDC-Capability-v1610                                                        OPTIONAL

}

UE-MRDC-Capability-v1610 ::=        SEQUENCE {

measAndMobParametersMRDC-v1610      MeasAndMobParametersMRDC-v1610                                                  OPTIONAL,

generalParametersMRDC-v1610         GeneralParametersMRDC-v1610                                                     OPTIONAL,

pdcp-ParametersMRDC-v1610           PDCP-ParametersMRDC-v1610                                                       OPTIONAL,

nonCriticalExtension                UE-MRDC-Capability-v1700                                                        OPTIONAL

}

UE-MRDC-Capability-v1700 ::=        SEQUENCE {

measAndMobParametersMRDC-v1700      MeasAndMobParametersMRDC-v1700,

nonCriticalExtension                UE-MRDC-Capability-v1730                                                        OPTIONAL

}

UE-MRDC-Capability-v1730 ::=        SEQUENCE {

measAndMobParametersMRDC-v1730      MeasAndMobParametersMRDC-v1730                                                  OPTIONAL,

nonCriticalExtension                SEQUENCE {}                                                                     OPTIONAL

}

-- Late non-critical extensions:

UE-MRDC-Capability-v15g0 ::=        SEQUENCE {

rf-ParametersMRDC-v15g0             RF-ParametersMRDC-v15g0                                                         OPTIONAL,

nonCriticalExtension                SEQUENCE {}                                                                     OPTIONAL

}

UE-MRDC-CapabilityAddXDD-Mode ::=   SEQUENCE {

measAndMobParametersMRDC-XDD-Diff       MeasAndMobParametersMRDC-XDD-Diff                                           OPTIONAL,

generalParametersMRDC-XDD-Diff          GeneralParametersMRDC-XDD-Diff                                              OPTIONAL

}

UE-MRDC-CapabilityAddXDD-Mode-v1560 ::=    SEQUENCE {

measAndMobParametersMRDC-XDD-Diff-v1560    MeasAndMobParametersMRDC-XDD-Diff-v1560                                  OPTIONAL

}

UE-MRDC-CapabilityAddFRX-Mode ::=   SEQUENCE {

measAndMobParametersMRDC-FRX-Diff       MeasAndMobParametersMRDC-FRX-Diff

}

GeneralParametersMRDC-XDD-Diff ::= SEQUENCE {

splitSRB-WithOneUL-Path             ENUMERATED {supported}                                                          OPTIONAL,

splitDRB-withUL-Both-MCG-SCG        ENUMERATED {supported}                                                          OPTIONAL,

srb3                                ENUMERATED {supported}                                                          OPTIONAL,

dummy                               ENUMERATED {supported}                                                          OPTIONAL,

...

}

GeneralParametersMRDC-v1610 ::= SEQUENCE {

f1c-OverEUTRA-r16                   ENUMERATED {supported}                                                          OPTIONAL

}

-- TAG-UE-MRDC-CAPABILITY-STOP

-- ASN1STOP

UE-MRDC-Capability field descriptions

featureSetCombinations A list of FeatureSetCombination:s for supportedBandCombinationList and supportedBandCombinationListNEDC-Only in UE-MRDC-Capability. The FeatureSetDownlink:s and FeatureSetUplink:s referred to from these FeatureSetCombination:s are defined in the featureSets list in UE-NR-Capability.

#### –	UE-NR-Capability

The IE UE-NR-Capability is used to convey the NR UE Radio Access Capability Parameters, see TS 38.306 [26].

UE-NR-Capability information element

-- ASN1START

-- TAG-UE-NR-CAPABILITY-START

UE-NR-Capability ::=            SEQUENCE {

accessStratumRelease            AccessStratumRelease,

pdcp-Parameters                 PDCP-Parameters,

rlc-Parameters                  RLC-Parameters                                                        OPTIONAL,

mac-Parameters                  MAC-Parameters                                                        OPTIONAL,

phy-Parameters                  Phy-Parameters,

rf-Parameters                   RF-Parameters,

measAndMobParameters            MeasAndMobParameters                                                  OPTIONAL,

fdd-Add-UE-NR-Capabilities      UE-NR-CapabilityAddXDD-Mode                                           OPTIONAL,

tdd-Add-UE-NR-Capabilities      UE-NR-CapabilityAddXDD-Mode                                           OPTIONAL,

fr1-Add-UE-NR-Capabilities      UE-NR-CapabilityAddFRX-Mode                                           OPTIONAL,

fr2-Add-UE-NR-Capabilities      UE-NR-CapabilityAddFRX-Mode                                           OPTIONAL,

featureSets                     FeatureSets                                                           OPTIONAL,

featureSetCombinations          SEQUENCE (SIZE (1..maxFeatureSetCombinations)) OF FeatureSetCombination         OPTIONAL,

lateNonCriticalExtension        OCTET STRING (CONTAINING UE-NR-Capability-v15c0)                      OPTIONAL,

nonCriticalExtension            UE-NR-Capability-v1530                                                OPTIONAL

}

-- Regular non-critical Rel-15 extensions:

UE-NR-Capability-v1530 ::=               SEQUENCE {

fdd-Add-UE-NR-Capabilities-v1530         UE-NR-CapabilityAddXDD-Mode-v1530                            OPTIONAL,

tdd-Add-UE-NR-Capabilities-v1530         UE-NR-CapabilityAddXDD-Mode-v1530                            OPTIONAL,

dummy                                    ENUMERATED {supported}                                       OPTIONAL,

interRAT-Parameters                      InterRAT-Parameters                                          OPTIONAL,

inactiveState                            ENUMERATED {supported}                                       OPTIONAL,

delayBudgetReporting                     ENUMERATED {supported}                                       OPTIONAL,

nonCriticalExtension                     UE-NR-Capability-v1540                                       OPTIONAL

}

UE-NR-Capability-v1540 ::=              SEQUENCE {

sdap-Parameters                         SDAP-Parameters                                               OPTIONAL,

overheatingInd                          ENUMERATED {supported}                                        OPTIONAL,

ims-Parameters                          IMS-Parameters                                                OPTIONAL,

fr1-Add-UE-NR-Capabilities-v1540        UE-NR-CapabilityAddFRX-Mode-v1540                             OPTIONAL,

fr2-Add-UE-NR-Capabilities-v1540        UE-NR-CapabilityAddFRX-Mode-v1540                             OPTIONAL,

fr1-fr2-Add-UE-NR-Capabilities          UE-NR-CapabilityAddFRX-Mode                                   OPTIONAL,

nonCriticalExtension                    UE-NR-Capability-v1550                                        OPTIONAL

}

UE-NR-Capability-v1550 ::=               SEQUENCE {

reducedCP-Latency                        ENUMERATED {supported}                                       OPTIONAL,

nonCriticalExtension                     UE-NR-Capability-v1560                                       OPTIONAL

}

UE-NR-Capability-v1560 ::=               SEQUENCE {

nrdc-Parameters                         NRDC-Parameters                                               OPTIONAL,

receivedFilters                         OCTET STRING (CONTAINING UECapabilityEnquiry-v1560-IEs)       OPTIONAL,

nonCriticalExtension                    UE-NR-Capability-v1570                                        OPTIONAL

}

UE-NR-Capability-v1570 ::=               SEQUENCE {

nrdc-Parameters-v1570                   NRDC-Parameters-v1570                                         OPTIONAL,

nonCriticalExtension                    UE-NR-Capability-v1610                                        OPTIONAL

}

-- Late non-critical Rel-15 extensions:

UE-NR-Capability-v15c0 ::=               SEQUENCE {

nrdc-Parameters-v15c0                    NRDC-Parameters-v15c0                                        OPTIONAL,

partialFR2-FallbackRX-Req                ENUMERATED {true}                                            OPTIONAL,

nonCriticalExtension                     UE-NR-Capability-v15g0                                       OPTIONAL

}

UE-NR-Capability-v15g0 ::=               SEQUENCE {

rf-Parameters-v15g0                      RF-Parameters-v15g0                                          OPTIONAL,

nonCriticalExtension                     UE-NR-Capability-v15j0                                       OPTIONAL

}

UE-NR-Capability-v15j0 ::=               SEQUENCE {

-- Following field is only for REL-15 late non-critical extensions

lateNonCriticalExtension                 OCTET STRING                                                 OPTIONAL,

nonCriticalExtension                     UE-NR-Capability-v16a0                                       OPTIONAL

}

-- Regular non-critical Rel-16 extensions:

UE-NR-Capability-v1610 ::=               SEQUENCE {

inDeviceCoexInd-r16                     ENUMERATED {supported}                                        OPTIONAL,

dl-DedicatedMessageSegmentation-r16     ENUMERATED {supported}                                        OPTIONAL,

nrdc-Parameters-v1610                   NRDC-Parameters-v1610                                         OPTIONAL,

powSav-Parameters-r16                   PowSav-Parameters-r16                                         OPTIONAL,

fr1-Add-UE-NR-Capabilities-v1610        UE-NR-CapabilityAddFRX-Mode-v1610                             OPTIONAL,

fr2-Add-UE-NR-Capabilities-v1610        UE-NR-CapabilityAddFRX-Mode-v1610                             OPTIONAL,

bh-RLF-Indication-r16                   ENUMERATED {supported}                                        OPTIONAL,

directSN-AdditionFirstRRC-IAB-r16       ENUMERATED {supported}                                        OPTIONAL,

bap-Parameters-r16                      BAP-Parameters-r16                                            OPTIONAL,

referenceTimeProvision-r16              ENUMERATED {supported}                                        OPTIONAL,

sidelinkParameters-r16                  SidelinkParameters-r16                                        OPTIONAL,

highSpeedParameters-r16                 HighSpeedParameters-r16                                       OPTIONAL,

mac-Parameters-v1610                    MAC-Parameters-v1610                                          OPTIONAL,

mcgRLF-RecoveryViaSCG-r16               ENUMERATED {supported}                                        OPTIONAL,

resumeWithStoredMCG-SCells-r16          ENUMERATED {supported}                                        OPTIONAL,

resumeWithStoredSCG-r16                 ENUMERATED {supported}                                        OPTIONAL,

resumeWithSCG-Config-r16                ENUMERATED {supported}                                        OPTIONAL,

ue-BasedPerfMeas-Parameters-r16         UE-BasedPerfMeas-Parameters-r16                               OPTIONAL,

son-Parameters-r16                      SON-Parameters-r16                                            OPTIONAL,

onDemandSIB-Connected-r16               ENUMERATED {supported}                                        OPTIONAL,

nonCriticalExtension                    UE-NR-Capability-v1640                                        OPTIONAL

}

UE-NR-Capability-v1640 ::=               SEQUENCE {

redirectAtResumeByNAS-r16               ENUMERATED {supported}                                        OPTIONAL,

phy-ParametersSharedSpectrumChAccess-r16  Phy-ParametersSharedSpectrumChAccess-r16                    OPTIONAL,

nonCriticalExtension                    UE-NR-Capability-v1650                                        OPTIONAL

}

UE-NR-Capability-v1650 ::=               SEQUENCE {

mpsPriorityIndication-r16                ENUMERATED {supported}                                       OPTIONAL,

highSpeedParameters-v1650                HighSpeedParameters-v1650                                    OPTIONAL,

nonCriticalExtension                     UE-NR-Capability-v1690                                       OPTIONAL

}

UE-NR-Capability-v1690 ::=               SEQUENCE {

ul-RRC-Segmentation-r16                  ENUMERATED {supported}                                       OPTIONAL,

nonCriticalExtension                     UE-NR-Capability-v1700                                       OPTIONAL

}

-- Late non-critical extensions from Rel-16 onwards:

UE-NR-Capability-v16a0 ::=               SEQUENCE {

phy-Parameters-v16a0                     Phy-Parameters-v16a0                                         OPTIONAL,

rf-Parameters-v16a0                      RF-Parameters-v16a0                                          OPTIONAL,

nonCriticalExtension                     UE-NR-Capability-v16c0                                       OPTIONAL

}

UE-NR-Capability-v16c0 ::=               SEQUENCE {

rf-Parameters-v16c0                      RF-Parameters-v16c0                                          OPTIONAL,

nonCriticalExtension                     SEQUENCE{}                                                   OPTIONAL

}

-- Regular non-critical Rel-17 extensions:

UE-NR-Capability-v1700 ::=               SEQUENCE {

inactiveStatePO-Determination-r17        ENUMERATED {supported}                                       OPTIONAL,

highSpeedParameters-v1700                HighSpeedParameters-v1700                                    OPTIONAL,

powSav-Parameters-v1700                  PowSav-Parameters-v1700                                      OPTIONAL,

mac-Parameters-v1700                     MAC-Parameters-v1700                                         OPTIONAL,

ims-Parameters-v1700                     IMS-Parameters-v1700                                         OPTIONAL,

measAndMobParameters-v1700               MeasAndMobParameters-v1700,

appLayerMeasParameters-r17               AppLayerMeasParameters-r17                                   OPTIONAL,

redCapParameters-r17                     RedCapParameters-r17                                         OPTIONAL,

ra-SDT-r17                               ENUMERATED {supported}                                       OPTIONAL,

srb-SDT-r17                              ENUMERATED {supported}                                       OPTIONAL,

gNB-SideRTT-BasedPDC-r17                 ENUMERATED {supported}                                       OPTIONAL,

bh-RLF-DetectionRecovery-Indication-r17  ENUMERATED {supported}                                       OPTIONAL,

nrdc-Parameters-v1700                    NRDC-Parameters-v1700                                        OPTIONAL,

bap-Parameters-v1700                     BAP-Parameters-v1700                                         OPTIONAL,

musim-GapPreference-r17                  ENUMERATED {supported}                                       OPTIONAL,

musimLeaveConnected-r17                  ENUMERATED {supported}                                       OPTIONAL,

mbs-Parameters-r17                       MBS-Parameters-r17,

nonTerrestrialNetwork-r17                ENUMERATED {supported}                                       OPTIONAL,

ntn-ScenarioSupport-r17                  ENUMERATED {gso, ngso}                                       OPTIONAL,

sliceInfoforCellReselection-r17          ENUMERATED {supported}                                       OPTIONAL,

ue-RadioPagingInfo-r17                   UE-RadioPagingInfo-r17                                       OPTIONAL,

-- R4 17-2 UL gap pattern for Tx power management

ul-GapFR2-Pattern-r17                    BIT STRING (SIZE (4))                                        OPTIONAL,

ntn-Parameters-r17                       NTN-Parameters-r17                                           OPTIONAL,

nonCriticalExtension                     UE-NR-Capability-v1740                                       OPTIONAL

}

UE-NR-Capability-v1740 ::=               SEQUENCE {

redCapParameters-v1740                   RedCapParameters-v1740,

nonCriticalExtension                     SEQUENCE {}                                                  OPTIONAL

}

UE-NR-CapabilityAddXDD-Mode ::=          SEQUENCE {

phy-ParametersXDD-Diff                   Phy-ParametersXDD-Diff                                       OPTIONAL,

mac-ParametersXDD-Diff                   MAC-ParametersXDD-Diff                                       OPTIONAL,

measAndMobParametersXDD-Diff             MeasAndMobParametersXDD-Diff                                 OPTIONAL

}

UE-NR-CapabilityAddXDD-Mode-v1530 ::=    SEQUENCE {

eutra-ParametersXDD-Diff                 EUTRA-ParametersXDD-Diff

}

UE-NR-CapabilityAddFRX-Mode ::=          SEQUENCE {

phy-ParametersFRX-Diff                   Phy-ParametersFRX-Diff                                       OPTIONAL,

measAndMobParametersFRX-Diff             MeasAndMobParametersFRX-Diff                                 OPTIONAL

}

UE-NR-CapabilityAddFRX-Mode-v1540 ::=    SEQUENCE {

ims-ParametersFRX-Diff                   IMS-ParametersFRX-Diff                                       OPTIONAL

}

UE-NR-CapabilityAddFRX-Mode-v1610 ::=    SEQUENCE {

powSav-ParametersFRX-Diff-r16            PowSav-ParametersFRX-Diff-r16                                OPTIONAL,

mac-ParametersFRX-Diff-r16               MAC-ParametersFRX-Diff-r16                                   OPTIONAL

}

BAP-Parameters-r16 ::=                   SEQUENCE {

flowControlBH-RLC-ChannelBased-r16       ENUMERATED {supported}                                       OPTIONAL,

flowControlRouting-ID-Based-r16          ENUMERATED {supported}                                       OPTIONAL

}

BAP-Parameters-v1700 ::=                 SEQUENCE {

bapHeaderRewriting-Rerouting-r17         ENUMERATED {supported}                                       OPTIONAL,

bapHeaderRewriting-Routing-r17           ENUMERATED {supported}                                       OPTIONAL

}

MBS-Parameters-r17 ::=                   SEQUENCE {

maxMRB-Add-r17                           INTEGER (1..16)                                              OPTIONAL

}

-- TAG-UE-NR-CAPABILITY-STOP

-- ASN1STOP

UE-NR-Capability field descriptions

featureSetCombinations A list of FeatureSetCombination:s for supportedBandCombinationList in UE-NR-Capability. The FeatureSetDownlink:s and FeatureSetUplink:s referred to from these FeatureSetCombination:s are defined in the featureSets list in UE-NR-Capability.

UE-NR-Capability-v1540 field descriptions

fr1-fr2-Add-UE-NR-Capabilities This instance of UE-NR-CapabilityAddFRX-Mode does not include any other fields than csi-RS-IM-ReceptionForFeedback/ csi-RS-ProcFrameworkForSRS/ csi-ReportFramework.

#### –	UE-RadioPagingInfo

The IE UE-RadioPagingInfo contains UE capability information needed for paging.

UE-RadioPagingInfo information element

-- ASN1START

-- TAG-UE-RADIOPAGINGINFO-START

UE-RadioPagingInfo-r17 ::=            SEQUENCE {

-- R1 29-1: Paging enhancement

pei-SubgroupingSupportBandList-r17    SEQUENCE (SIZE (1..maxBands)) OF FreqBandIndicatorNR    OPTIONAL,

...

}

-- TAG-UE-RADIOPAGINGINFO-STOP

-- ASN1STOP

#### –	SharedSpectrumChAccessParamsPerBand

The IE SharedSpectrumChAccessParamsPerBand is used to convey shared channel access related parameters specific for a certain frequency band (not per feature set or band combination).

SharedSpectrumChAccessParamsPerBand information element

-- ASN1START

-- TAG-SHAREDSPECTRUMCHACCESSPARAMSPERBAND-START

SharedSpectrumChAccessParamsPerBand-r16 ::=           SEQUENCE {

-- R1 10-1: UL channel access for dynamic channel access mode

ul-DynamicChAccess-r16                              ENUMERATED {supported}            OPTIONAL,

-- R1 10-1a: UL channel access for semi-static channel access mode

ul-Semi-StaticChAccess-r16                          ENUMERATED {supported}            OPTIONAL,

-- R1 10-2: SSB-based RRM for dynamic channel access mode

ssb-RRM-DynamicChAccess-r16                         ENUMERATED {supported}            OPTIONAL,

-- R1 10-2a: SSB-based RRM for semi-static channel access mode

ssb-RRM-Semi-StaticChAccess-r16                     ENUMERATED {supported}            OPTIONAL,

-- R1 10-2b: MIB reading on unlicensed cell

mib-Acquisition-r16                                 ENUMERATED {supported}            OPTIONAL,

-- R1 10-2c: SSB-based RLM for dynamic channel access mode

ssb-RLM-DynamicChAccess-r16                         ENUMERATED {supported}            OPTIONAL,

-- R1 10-2d: SSB-based RLM for semi-static channel access mode

ssb-RLM-Semi-StaticChAccess-r16                     ENUMERATED {supported}            OPTIONAL,

-- R1 10-2e: SIB1 reception on unlicensed cell

sib1-Acquisition-r16                                ENUMERATED {supported}            OPTIONAL,

-- R1 10-2f: Support monitoring of extended RAR window

extRA-ResponseWindow-r16                            ENUMERATED {supported}            OPTIONAL,

-- R1 10-2g: SSB-based BFD/CBD for dynamic channel access mode

ssb-BFD-CBD-dynamicChannelAccess-r16                ENUMERATED {supported}            OPTIONAL,

-- R1 10-2h: SSB-based BFD/CBD for semi-static channel access mode

ssb-BFD-CBD-semi-staticChannelAccess-r16            ENUMERATED {supported}            OPTIONAL,

-- R1 10-2i: CSI-RS-based BFD/CBD for NR-U

csi-RS-BFD-CBD-r16                                  ENUMERATED {supported}            OPTIONAL,

-- R1 10-7: UL channel access for 10 MHz SCell

ul-ChannelBW-SCell-10mhz-r16                        ENUMERATED {supported}            OPTIONAL,

-- R1 10-10: RSSI and channel occupancy measurement and reporting

rssi-ChannelOccupancyReporting-r16                  ENUMERATED {supported}            OPTIONAL,

-- R1 10-11:SRS starting position at any OFDM symbol in a slot

srs-StartAnyOFDM-Symbol-r16                         ENUMERATED {supported}            OPTIONAL,

-- R1 10-20: Support search space set configuration with freqMonitorLocation-r16

searchSpaceFreqMonitorLocation-r16                  INTEGER (1..5)                    OPTIONAL,

-- R1 10-20a: Support coreset configuration with rb-Offset

coreset-RB-Offset-r16                               ENUMERATED {supported}            OPTIONAL,

-- R1 10-23:CGI reading on unlicensed cell for ANR functionality

cgi-Acquisition-r16                                 ENUMERATED {supported}            OPTIONAL,

-- R1 10-25: Enable configured UL transmissions when DCI 2_0 is configured but not detected

configuredUL-Tx-r16                                  ENUMERATED {supported}            OPTIONAL,

-- R1 10-27: Wideband PRACH

prach-Wideband-r16                                  ENUMERATED {supported}            OPTIONAL,

-- R1 10-29: Support available RB set indicator field in DCI 2_0

dci-AvailableRB-Set-r16                             ENUMERATED {supported}            OPTIONAL,

-- R1 10-30: Support channel occupancy duration indicator field in DCI 2_0

dci-ChOccupancyDuration-r16                         ENUMERATED {supported}            OPTIONAL,

-- R1 10-8: Type B PDSCH length {3, 5, 6, 8, 9, 10, 11, 12, 13} without DMRS shift due to CRS collision

typeB-PDSCH-length-r16                              ENUMERATED {supported}            OPTIONAL,

-- R1 10-9: Search space set group switching with explicit DCI 2_0 bit field trigger or with implicit PDCCH decoding with DCI 2_0 monitoring

searchSpaceSwitchWithDCI-r16                        ENUMERATED {supported}            OPTIONAL,

-- R1 10-9b: Search space set group switching with implicit PDCCH decoding without DCI 2_0 monitoring

searchSpaceSwitchWithoutDCI-r16                     ENUMERATED {supported}            OPTIONAL,

-- R1 10-9d: Support Search space set group switching capability 2

searchSpaceSwitchCapability2-r16                    ENUMERATED {supported}            OPTIONAL,

-- R1 10-14: Non-numerical PDSCH to HARQ-ACK timing

non-numericalPDSCH-HARQ-timing-r16                  ENUMERATED {supported}            OPTIONAL,

-- R1 10-15: Enhanced dynamic HARQ codebook

enhancedDynamicHARQ-codebook-r16                    ENUMERATED {supported}            OPTIONAL,

-- R1 10-16: One-shot HARQ ACK feedback

oneShotHARQ-feedback-r16                            ENUMERATED {supported}            OPTIONAL,

-- R1 10-17: Multi-PUSCH UL grant

multiPUSCH-UL-grant-r16                             ENUMERATED {supported}            OPTIONAL,

-- R1 10-26: CSI-RS based RLM for NR-U

csi-RS-RLM-r16                                      ENUMERATED {supported}            OPTIONAL,

dummy                                              ENUMERATED {supported}            OPTIONAL,

-- R1 10-31: Support of P/SP-CSI-RS reception with CSI-RS-ValidationWith-DCI-r16 configured

periodicAndSemi-PersistentCSI-RS-r16                ENUMERATED {supported}            OPTIONAL,

-- R1 10-3: PRB interlace mapping for PUSCH

pusch-PRB-interlace-r16                             ENUMERATED {supported}            OPTIONAL,

-- R1 10-3a: PRB interlace mapping for PUCCH

pucch-F0-F1-PRB-Interlace-r16                       ENUMERATED {supported}            OPTIONAL,

-- R1 10-12: OCC for PRB interlace mapping for PF2 and PF3

occ-PRB-PF2-PF3-r16                                 ENUMERATED {supported}            OPTIONAL,

-- R1 10-13a: Extended CP range of more than one symbol for CG-PUSCH

extCP-rangeCG-PUSCH-r16                             ENUMERATED {supported}            OPTIONAL,

-- R1 10-18: Configured grant with retransmission in CG resources

configuredGrantWithReTx-r16                         ENUMERATED {supported}            OPTIONAL,

-- R1 10-21a: Support using ED threshold given by gNB for UL to DL COT sharing

ed-Threshold-r16                                    ENUMERATED {supported}            OPTIONAL,

-- R1 10-21b: Support UL to DL COT sharing

ul-DL-COT-Sharing-r16                               ENUMERATED {supported}            OPTIONAL,

-- R1 10-24: CG-UCI multiplexing with HARQ ACK

mux-CG-UCI-HARQ-ACK-r16                             ENUMERATED {supported}            OPTIONAL,

-- R1 10-28: Configured grant with Rel-16 enhanced resource configuration

cg-resourceConfig-r16                               ENUMERATED {supported}            OPTIONAL

}

SharedSpectrumChAccessParamsPerBand-v1630 ::=       SEQUENCE {

-- R4 4-1: DL reception in intra-carrier guardband

dl-ReceptionIntraCellGuardband-r16                  ENUMERATED {supported}            OPTIONAL,

-- R4 4-2: DL reception when gNB does not transmit on all RB sets of a carrier as a result of LBT

dl-ReceptionLBT-subsetRB-r16                        ENUMERATED {supported}            OPTIONAL

}

SharedSpectrumChAccessParamsPerBand-v1640 ::=       SEQUENCE {

-- 10-26b(1-4): CSI-RS based RRM measurement with associated SS-block

csi-RSRP-AndRSRQ-MeasWithSSB-r16                     ENUMERATED {supported}              OPTIONAL,

-- 10-26c(1-5): CSI-RS based RRM measurement without associated SS-block

csi-RSRP-AndRSRQ-MeasWithoutSSB-r16                 ENUMERATED {supported}               OPTIONAL,

-- 10-26d(1-6): CSI-RS based RS-SINR measurement

csi-SINR-Meas-r16                                      ENUMERATED {supported}               OPTIONAL,

-- 10-26e(1-8): RLM based on a mix of SS block and CSI-RS signals within active BWP

ssb-AndCSI-RS-RLM-r16                                 ENUMERATED {supported}               OPTIONAL,

-- 10-26f(1-9): CSI-RS based contention free RA for HO

csi-RS-CFRA-ForHO-r16                                 ENUMERATED {supported}               OPTIONAL

}

SharedSpectrumChAccessParamsPerBand-v1650 ::=       SEQUENCE {

-- Extension of R1 10-9 capability to configure up to 16 instead of 4 cells or cell groups, respectively

extendedSearchSpaceSwitchWithDCI-r16                ENUMERATED {supported}               OPTIONAL

}

SharedSpectrumChAccessParamsPerBand-v1710 ::=    SEQUENCE {

-- R1 25-12: UE initiated semi-static channel occupancy with dependent configurations

ul-Semi-StaticChAccessDependentConfig-r17        ENUMERATED {supported}            OPTIONAL,

-- R1 25-13: UE initiated semi-static channel occupancy with independent configurations

ul-Semi-StaticChAccessIndependentConfig-r17      ENUMERATED {supported}            OPTIONAL

}

-- TAG-SHAREDSPECTRUMCHACCESSPARAMSPERBAND-STOP

-- ASN1STOP

---------------------------------------------------------Skip Unchanged----------------------------------------------------------

### 6.3.4	Other information elements

#### –	AbsoluteTimeInfo

The IE AbsoluteTimeInfo indicates an absolute time in a format YY-MM-DD HH:MM:SS and using BCD encoding. The first/ leftmost bit of the bit string contains the most significant bit of the most significant digit of the year and so on.

AbsoluteTimeInfo information element

-- ASN1START

-- TAG-ABSOLUTETIMEINFO-START

AbsoluteTimeInfo-r16 ::= BIT STRING (SIZE (48))

-- TAG-ABSOLUTETIMEINFO-STOP

-- ASN1STOP

#### –	AppLayerMeasConfig

The IE AppLayerMeasConfig indicates configuration of application layer measurements.

AppLayerMeasConfig information element

-- ASN1START

-- TAG-APPLAYERMEASCONFIG-START

AppLayerMeasConfig-r17 ::=           SEQUENCE {

measConfigAppLayerToAddModList-r17   SEQUENCE (SIZE (1..maxNrofAppLayerMeas-r17)) OF MeasConfigAppLayer-r17     OPTIONAL, -- Need N

measConfigAppLayerToReleaseList-r17  SEQUENCE (SIZE (1..maxNrofAppLayerMeas-r17)) OF MeasConfigAppLayerId-r17   OPTIONAL, -- Need N

rrc-SegAllowed-r17                   ENUMERATED {enabled}                                                       OPTIONAL, -- Need R

...

}

MeasConfigAppLayer-r17 ::=           SEQUENCE {

measConfigAppLayerId-r17             MeasConfigAppLayerId-r17,

measConfigAppLayerContainer-r17      OCTET STRING (SIZE (1..8000))                                              OPTIONAL, -- Need N

serviceType-r17                      ENUMERATED {streaming, mtsi, vr, spare5, spare4, spare3, spare2, spare1}   OPTIONAL, -- Need M

pauseReporting-r17                   BOOLEAN                                                                    OPTIONAL, -- Need M

transmissionOfSessionStartStop-r17   BOOLEAN                                                                    OPTIONAL, -- Need M

ran-VisibleParameters-r17            SetupRelease {RAN-VisibleParameters-r17}                                   OPTIONAL, -- Cond ServiceType

...

}

RAN-VisibleParameters-r17 ::=        SEQUENCE {

ran-VisiblePeriodicity-r17           ENUMERATED {ms120, ms240, ms480, ms640, ms1024}                            OPTIONAL, -- Need S

numberOfBufferLevelEntries-r17       INTEGER (1..8)                                                             OPTIONAL, -- Need R

reportPlayoutDelayForMediaStartup-r17 BOOLEAN                                                                   OPTIONAL, -- Need M

...

}

-- TAG-APPLAYERMEASCONFIG-STOP

-- ASN1STOP

AppLayerMeasConfig field descriptions

measConfigAppLayerContainer The field contains configuration of application layer measurements, see Annex L (normative) in TS 26.247 [68], clause 16.5 in TS 26.114 [69] and TS 26.118 [70].

pauseReporting The field indicates whether the transmission of measReportAppLayerContainer is paused or not. Value true indicates the transmission of measReportAppLayerContainer is paused; value false indicates the transmission of measReportAppLayerContainer is not paused.

ran-VisibleParameters The field indicates whether RAN visible application layer measurements shall be reported or not.

rrc-SegAllowed This field indicates that RRC segmentation of MeasurementReportAppLayer is allowed. It may be present only if the UE supports RRC segmentation of the MeasurementReportAppLayer message in UL.

serviceType Indicates the type of application layer measurement. Value streaming indicates Quality of Experience Measurement Collection for streaming services (see TS 26.247 [68]), value mtsi indicates Quality of Experience Measurement Collection for MTSI (see TS 26.114 [69]). value vr indicates Quality of Experience Measurement Collection for VR service (see TS 26.118 [70]). The network always configures serviceType when application layer measurements are initially configured and at fullConfig.

transmissionOfSessionStartStop The field indicates whether the UE shall transmit indications when sessions in the application layer start and stop. The UE transmits a session start indication upon configuration of this field if a session already has started in the application layer.

RAN-VisibleParameters field descriptions

numberOfBufferLevelEntries The field contains the maximum number of buffer level entries that can be reported for RAN visible application layer measurements. This field is also used by application layer to calculate the interval of RAN visible buffer level measurement, which is equal to the periodicity of RAN visible application layer measurements reporting divided by numberOfBufferLevelEntries.

ran-VisiblePeriodicity The field indicates the periodicity of RAN visible application layer measurements reporting. Value ms120 indicates 120 ms, value ms240 indicates 240 ms and so on. If this field is absent, the periodicity of RAN visible application layer reporting is the same as the reporting periodicity indicated in measConfigAppLayerContainer.

reportPlayoutDelayForMediaStartup The field indicates whether the UE shall report Playout Delay for Media Startup for RAN visible application layer measurements.

| Conditional Presence | Explanation |
| --- | --- |
| ServiceType | This field is optionally present, Need M, when serviceType is set to streaming or vr. Otherwise, it is absent. |

#### –	AreaConfiguration

The AreaConfiguration indicates area for which UE is requested to perform measurement logging. If not configured, measurement logging is not restricted to specific cells or tracking areas but applies as long as the RPLMN is contained in plmn-IdentityList stored in VarLogMeasReport.

AreaConfiguration information element

-- ASN1START

-- TAG-AREACONFIGURATION-START

AreaConfiguration-r16 ::=        SEQUENCE {

areaConfig-r16                   AreaConfig-r16,

interFreqTargetList-r16          SEQUENCE(SIZE (1..maxFreq)) OF InterFreqTargetInfo-r16              OPTIONAL  -- Need R

}

AreaConfiguration-v1700 ::=      SEQUENCE {

areaConfig-r17                   AreaConfig-r16                                                      OPTIONAL, -- Need R

interFreqTargetList-r17          SEQUENCE(SIZE (1..maxFreq)) OF InterFreqTargetInfo-r16              OPTIONAL  -- Need R

}

AreaConfig-r16 ::=     CHOICE {

cellGlobalIdList-r16             CellGlobalIdList-r16,

trackingAreaCodeList-r16         TrackingAreaCodeList-r16,

trackingAreaIdentityList-r16     TrackingAreaIdentityList-r16

}

InterFreqTargetInfo-r16    ::=   SEQUENCE {

dl-CarrierFreq-r16               ARFCN-ValueNR,

cellList-r16                     SEQUENCE (SIZE (1..32)) OF  PhysCellId  OPTIONAL      -- Need R

}

CellGlobalIdList-r16 ::=         SEQUENCE (SIZE (1..32)) OF CGI-Info-Logging-r16

TrackingAreaCodeList-r16 ::=     SEQUENCE (SIZE (1..8)) OF TrackingAreaCode

TrackingAreaIdentityList-r16 ::= SEQUENCE (SIZE (1..8)) OF TrackingAreaIdentity-r16

TrackingAreaIdentity-r16 ::=     SEQUENCE {

plmn-Identity-r16                PLMN-Identity,

trackingAreaCode-r16             TrackingAreaCode

}

-- TAG-AREACONFIGURATION-STOP

-- ASN1STOP

AreaConfiguration field descriptions

InterFreqTargetInfo If configured, it indicates the neighbouring frequency and cells for which UE is requested to perform measurement logging. It can include sync raster or non-sync raster frequencies.

#### –	BT-NameList

The IE BT-NameList is used to indicate the names of the Bluetooth beacon which the UE is configured to measure.

BT-NameList information element

-- ASN1START

-- TAG-BTNAMELIST-START

BT-NameList-r16 ::=                SEQUENCE (SIZE (1..maxBT-Name-r16)) OF BT-Name-r16

BT-Name-r16 ::=                    OCTET STRING (SIZE (1..248))

-- TAG-BTNAMELIST-STOP

-- ASN1STOP

BT-NameList field descriptions

bt-Name If configured, the UE only performs Bluetooth measurements according to the names identified. For each name, it refers to LOCAL NAME defined in Bluetooth specification [51].

#### –	DedicatedInfoF1c

The IE DedicatedInfoF1c is used to transfer IAB-DU specific F1-C related information between the network and the IAB node. The carried information consists of F1AP message encapsulated in SCTP/IP or F1-C related (SCTP)/IP packet, see TS 38.472 [64]. The RRC layer is transparent for this information.

DedicatedInfoF1c information element

-- ASN1START

-- TAG-DEDICATEDINFOF1C-START

DedicatedInfoF1c-r17 ::=        OCTET STRING

-- TAG-DEDICATEDINFOF1C-STOP

-- ASN1STOP

#### –	EUTRA-AllowedMeasBandwidth

The IE EUTRA-AllowedMeasBandwidth is used to indicate the maximum allowed measurement bandwidth on a carrier frequency as defined by the parameter Transmission Bandwidth Configuration "NRB" in TS 36.104 [33]. The values mbw6, mbw15, mbw25, mbw50, mbw75, mbw100 indicate 6, 15, 25, 50, 75 and 100 resource blocks, respectively.

EUTRA-AllowedMeasBandwidth information element

-- ASN1START

-- TAG-EUTRA-ALLOWEDMEASBANDWIDTH-START

EUTRA-AllowedMeasBandwidth ::=              ENUMERATED {mbw6, mbw15, mbw25, mbw50, mbw75, mbw100}

-- TAG-EUTRA-ALLOWEDMEASBANDWIDTH-STOP

-- ASN1STOP

#### –	EUTRA-MBSFN-SubframeConfigList

The IE EUTRA-MBSFN-SubframeConfigList is used to define an E-UTRA MBSFN subframe pattern (for the purpose of NR rate matching).

EUTRA-MBSFN-SubframeConfigList information element

-- ASN1START

-- TAG-EUTRA-MBSFN-SUBFRAMECONFIGLIST-START

EUTRA-MBSFN-SubframeConfigList ::= SEQUENCE (SIZE (1..maxMBSFN-Allocations)) OF EUTRA-MBSFN-SubframeConfig

EUTRA-MBSFN-SubframeConfig ::=      SEQUENCE {

radioframeAllocationPeriod          ENUMERATED {n1, n2, n4, n8, n16, n32},

radioframeAllocationOffset          INTEGER (0..7),

subframeAllocation1                 CHOICE {

oneFrame                            BIT STRING (SIZE(6)),

fourFrames                          BIT STRING (SIZE(24))

},

subframeAllocation2                 CHOICE {

oneFrame                            BIT STRING (SIZE(2)),

fourFrames                          BIT STRING (SIZE(8))

}                                                                                                       OPTIONAL,   -- Need R

...

}

-- TAG-EUTRA-MBSFN-SUBFRAMECONFIGLIST-STOP

-- ASN1STOP

EUTRA-MBSFN-SubframeConfig field descriptions

radioframeAllocationOffset Field as defined in MBSFN-SubframeConfig in TS 36.331 [10].

radioframeAllocationPeriod Field as defined in MBSFN-SubframeConfig in TS 36.331 [10], where SFN refers to the SFN of the NR serving cell.

subframeAllocation1 Field as defined in MBSFN-SubframeConfig in TS 36.331 [10], where the UE assumes the duplex mode (FDD or TDD) of the NR cell for which the E-UTRA-MBSFN-SubframeConfig is provided.

subframeAllocation2 Field as defined in MBSFN-SubframeConfig-v1430 in TS 36.331 [10], where the UE assumes the duplex mode (FDD or TDD) of the NR cell for which the E-UTRA-MBSFN-SubframeConfig is provided.

#### –	EUTRA-MultiBandInfoList

The IE EUTRA-MultiBandInfoList indicates the list of frequency bands in addition to the band represented by CarrierFreq for which cell reselection parameters are common, and a list of additionalPmax and additionalSpectrumEmission.

EUTRA-MultiBandInfoList information element

-- ASN1START

-- TAG-EUTRA-MULTIBANDINFOLIST-START

EUTRA-MultiBandInfoList ::=     SEQUENCE (SIZE (1..maxMultiBands)) OF EUTRA-MultiBandInfo

EUTRA-MultiBandInfo ::=         SEQUENCE {

eutra-FreqBandIndicator         FreqBandIndicatorEUTRA,

eutra-NS-PmaxList               EUTRA-NS-PmaxList                           OPTIONAL    -- Need R

}

-- TAG-EUTRA-MULTIBANDINFOLIST-STOP

-- ASN1STOP

#### –	EUTRA-NS-PmaxList

The IE EUTRA-NS-PmaxList concerns a list of additionalPmax and additionalSpectrumEmission, as defined in TS 36.101 [22], table 6.2.4-1 for UEs neither in CE nor BL UEs and TS 36.101 [22], table 6.2.4E-1 for UEs in CE or BL UEs, for a given frequency band.

EUTRA-NS-PmaxList information element

-- ASN1START

-- TAG-EUTRA-NS-PMAXLIST-START

EUTRA-NS-PmaxList ::=               SEQUENCE (SIZE (1..maxEUTRA-NS-Pmax)) OF EUTRA-NS-PmaxValue

EUTRA-NS-PmaxValue ::=              SEQUENCE {

additionalPmax                      INTEGER (-30..33)                       OPTIONAL,   -- Need R

additionalSpectrumEmission          INTEGER (1..288)                        OPTIONAL    -- Need R

}

-- TAG-EUTRA-NS-PMAXLIST-STOP

-- ASN1STOP

#### –	EUTRA-PhysCellId

The IE EUTRA-PhysCellId is used to indicate the physical layer identity of the cell, as defined in TS 36.211 [31].

EUTRA-PhysCellId information element

-- ASN1START

-- TAG-EUTRA-PHYSCELLID-START

EUTRA-PhysCellId ::=                        INTEGER (0..503)

-- TAG-EUTRA-PHYSCELLID-STOP

-- ASN1STOP

#### –	EUTRA-PhysCellIdRange

The IE EUTRA-PhysCellIdRange is used to encode either a single or a range of physical cell identities. The range is encoded by using a start value and by indicating the number of consecutive physical cell identities (including start) in the range. For fields comprising multiple occurrences of EUTRA-PhysCellIdRange, NW may configure overlapping ranges of physical cell identities.

EUTRA-PhysCellIdRange information element

-- ASN1START

-- TAG-EUTRA-PHYSCELLIDRANGE-START

EUTRA-PhysCellIdRange ::=       SEQUENCE {

start                           EUTRA-PhysCellId,

range                           ENUMERATED {n4, n8, n12, n16, n24, n32, n48, n64, n84, n96,

n128, n168, n252, n504, spare2, spare1}                         OPTIONAL    -- Need N

}

-- TAG-EUTRA-PHYSCELLIDRANGE-STOP

-- ASN1STOP

#### –	EUTRA-PresenceAntennaPort1

The IE EUTRA-PresenceAntennaPort1 is used to indicate whether all the neighbouring cells use Antenna Port 1. When set to true, the UE may assume that at least two cell-specific antenna ports are used in all neighbouring cells.

EUTRA-PresenceAntennaPort1 information element

-- ASN1START

-- TAG-EUTRA-PRESENCEANTENNAPORT1-START

EUTRA-PresenceAntennaPort1 ::=              BOOLEAN

-- TAG-EUTRA-PRESENCEANTENNAPORT1-STOP

-- ASN1STOP

#### –	EUTRA-Q-OffsetRange

The IE EUTRA-Q-OffsetRange is used to indicate a cell, or frequency specific offset to be applied when evaluating triggering conditions for measurement reporting. The value in dB. Value dB-24 corresponds to -24 dB, value dB-22 corresponds to -22 dB and so on.

EUTRA-Q-OffsetRange information element

-- ASN1START

-- TAG-EUTRA-Q-OFFSETRANGE-START

EUTRA-Q-OffsetRange ::=                     ENUMERATED {

dB-24, dB-22, dB-20, dB-18, dB-16, dB-14,

dB-12, dB-10, dB-8, dB-6, dB-5, dB-4, dB-3,

dB-2, dB-1, dB0, dB1, dB2, dB3, dB4, dB5,

dB6, dB8, dB10, dB12, dB14, dB16, dB18,

dB20, dB22, dB24}

-- TAG-EUTRA-Q-OFFSETRANGE-STOP

-- ASN1STOP

#### –	IAB-IP-Address

The IE IAB-IP-Address is used to indicate the IP address/prefix.

IAB-IP-Address information element

-- ASN1START

-- TAG-IABIPADDRESS-START

IAB-IP-Address-r16 ::=  CHOICE {

iPv4-Address-r16                BIT STRING (SIZE(32)),

iPv6-Address-r16                BIT STRING (SIZE(128)),

iPv6-Prefix-r16                 BIT STRING (SIZE(64)),

...

}

-- TAG-IABIPADDRESS-STOP

-- ASN1STOP

IAB-IP-Address field descriptions

iPv4-Address This field is used to provide the allocated IPv4 address.

iPv6-Address This field is used to provide the allocated IPv6 address.

iPv6-Prefix This field is used to provide the allocated IPv6 prefix.

#### –	IAB-IP-AddressIndex

The IE IAB-IP-AddressIndex is used to identify a configuration of an IP address.

IAB-IP-AddressIndex information element

-- ASN1START

-- TAG-IABIPADDRESSINDEX-START

IAB-IP-AddressIndex-r16 ::= INTEGER (1..maxIAB-IP-Address-r16)

-- TAG-IABIPADDRESSINDEX-STOP

-- ASN1STOP

#### –	IAB-IP-Usage

The IE IAB-IP-Usage is used to indicate the usage of the assigned IP address/prefix.

IAB-IP-Usage information element

-- ASN1START

-- TAG-IAB-IP-USAGE-START

IAB-IP-Usage-r16 ::= ENUMERATED {f1-C, f1-U, non-F1, spare}

-- TAG-IAB-IP-USAGE-STOP

-- ASN1STOP

#### –	LoggingDuration

The LoggingDuration indicates the duration for which UE is requested to perform measurement logging. Value min10 corresponds to 10 minutes, value min20 corresponds to 20 minutes and so on.

LoggingDuration information element

-- ASN1START

-- TAG-LOGGINGDURATION-START

LoggingDuration-r16 ::=   ENUMERATED {

min10, min20, min40, min60, min90, min120, spare2, spare1}

-- TAG-LOGGINGDURATION-STOP

-- ASN1STOP

#### –	LoggingInterval

The LoggingInterval indicates the periodicity for logging measurement results. Value ms1280 corresponds to 1.28s, value ms2560 corresponds to 2.56s and so on. Value infinity means it is equal to the configured value of the LoggingDuration IE.

LoggingInterval information element

-- ASN1START

-- TAG-LOGGINGINTERVAL-START

LoggingInterval-r16 ::=   ENUMERATED {

ms320, ms640, ms1280, ms2560, ms5120, ms10240, ms20480,

ms30720, ms40960, ms61440 , infinity}

-- TAG-LOGGINGINTERVAL-STOP

-- ASN1STOP

#### –	LogMeasResultListBT

The IE LogMeasResultListBT covers measured results for Bluetooth.

LogMeasResultListBT information element

-- ASN1START

-- TAG-LOGMEASRESULTLISTBT-START

LogMeasResultListBT-r16 ::= SEQUENCE (SIZE (1..maxBT-IdReport-r16)) OF LogMeasResultBT-r16

LogMeasResultBT-r16 ::= SEQUENCE {

bt-Addr-r16             BIT STRING (SIZE (48)),

rssi-BT-r16             INTEGER (-128..127)        OPTIONAL,

...

}

-- TAG-LOGMEASRESULTLISTBT-STOP

-- ASN1STOP

LogMeasResultListBT field descriptions

bt-Addr This field indicates the Bluetooth public address of the Bluetooth beacon as defined in TS 37.355 [49].

rssi-BT This field provides the beacon received signal strength indicator (RSSI) in dBm as defined in TS 37.355 [49].

#### –	LogMeasResultListWLAN

The IE LogMeasResultListWLAN covers measured results for WLAN.

LogMeasResultListWLAN information element

-- ASN1START

-- TAG-LOGMEASRESULTLISTWLAN-START

LogMeasResultListWLAN-r16 ::=    SEQUENCE (SIZE (1..maxWLAN-Id-Report-r16)) OF LogMeasResultWLAN-r16

LogMeasResultWLAN-r16 ::=        SEQUENCE {

wlan-Identifiers-r16             WLAN-Identifiers-r16,

rssiWLAN-r16                     WLAN-RSSI-Range-r16          OPTIONAL,

rtt-WLAN-r16                     WLAN-RTT-r16                 OPTIONAL,

...

}

WLAN-Identifiers-r16 ::=         SEQUENCE {

ssid-r16                         OCTET STRING (SIZE (1..32))  OPTIONAL,

bssid-r16                        OCTET STRING (SIZE (6))      OPTIONAL,

hessid-r16                       OCTET STRING (SIZE (6))      OPTIONAL,

...

}

WLAN-RSSI-Range-r16 ::= INTEGER(0..141)

WLAN-RTT-r16 ::=                 SEQUENCE {

rttValue-r16                     INTEGER (0..16777215),

rttUnits-r16                     ENUMERATED {

microseconds,

hundredsofnanoseconds,

tensofnanoseconds,

nanoseconds,

tenthsofnanoseconds,

...},

rttAccuracy-r16                  INTEGER (0..255)             OPTIONAL,

...

}

-- TAG-LOGMEASRESULTLISTWLAN-STOP

-- ASN1STOP

LogMeasResultListWLAN field descriptions

Bssid Basic Service Set Identifier (BSSID) defined in IEEE 802.11-2012 [50].

Hessid Homogenous Extended Service Set Identifier (HESSID) defined in IEEE 802.11-2012 [50].

rssiWLAN Measured WLAN RSSI result in dBm. The IE WLAN-RSSI-Range specifies the value range used in WLAN RSSI measurements and thresholds. Integer value for WLAN RSSI measurements is according to mapping table in TS 36.133 [40]. Value 0 corresponds to –infinity, value 1 to -100dBm, value 2 to -99dBm, and so on (i.e. in steps of 1dBm) until value 140, which corresponds to 39dBm, while value 141 corresponds to +infinity.

rtt-WLAN This field provides the measured roundtrip time between the target device and WLAN AP and optionally the accuracy expressed as the standard deviation of the delay. Units for each of these are 1000ns, 100ns, 10ns, 1ns, and 0.1ns as defined in TS 37.355 [49].

rttValue This field specifies the Round Trip Time (RTT) measurement between the target device and WLAN AP in units given by the field rttUnits as defined in TS 37.355 [49].

rttUnits This field specifies the Units for the fields rttValue and rttAccuracy. The available Units are 1000ns, 100ns, 10ns, 1ns, and 0.1ns as defined in TS 37.355 [49].

rttAccuracy This field provides the estimated accuracy of the provided rttValue expressed as the standard deviation in units given by the field rttUnits as defined in TS 37.355 [49].

Ssid Service Set Identifier (SSID) defined in IEEE 802.11-2012 [50].

Wlan-Identifiers Indicates the WLAN parameters used for identification of the WLAN for which the measurement results are applicable.

#### –	MeasConfigAppLayerId

The IE MeasConfigAppLayerId identifies the application layer measurement.

MeasConfigAppLayerId information element

-- ASN1START

-- TAG-MEASCONFIGAPPLAYERID-START

MeasConfigAppLayerId-r17 ::= INTEGER (0..maxNrofAppLayerMeas-1-r17)

-- TAG-MEASCONFIGAPPLAYERID-STOP

-- ASN1STOP

#### –	OtherConfig

The IE OtherConfig contains configuration related to miscellaneous other configurations.

OtherConfig information element

-- ASN1START

-- TAG-OTHERCONFIG-START

OtherConfig ::=                 SEQUENCE {

delayBudgetReportingConfig  CHOICE{

release                 NULL,

setup                   SEQUENCE{

delayBudgetReportingProhibitTimer   ENUMERATED {s0, s0dot4, s0dot8, s1dot6, s3, s6, s12, s30}

}

}                                                                                                     OPTIONAL        -- Need M

}

OtherConfig-v1540 ::=           SEQUENCE {

overheatingAssistanceConfig     SetupRelease {OverheatingAssistanceConfig}                            OPTIONAL, -- Need M

...

}

OtherConfig-v1610 ::=                   SEQUENCE {

idc-AssistanceConfig-r16                SetupRelease {IDC-AssistanceConfig-r16}                       OPTIONAL, -- Need M

drx-PreferenceConfig-r16                SetupRelease {DRX-PreferenceConfig-r16}                       OPTIONAL, -- Need M

maxBW-PreferenceConfig-r16              SetupRelease {MaxBW-PreferenceConfig-r16}                     OPTIONAL, -- Need M

maxCC-PreferenceConfig-r16              SetupRelease {MaxCC-PreferenceConfig-r16}                     OPTIONAL, -- Need M

maxMIMO-LayerPreferenceConfig-r16       SetupRelease {MaxMIMO-LayerPreferenceConfig-r16}              OPTIONAL, -- Need M

minSchedulingOffsetPreferenceConfig-r16 SetupRelease {MinSchedulingOffsetPreferenceConfig-r16}        OPTIONAL, -- Need M

releasePreferenceConfig-r16             SetupRelease {ReleasePreferenceConfig-r16}                    OPTIONAL, -- Need M

referenceTimePreferenceReporting-r16    ENUMERATED {true}                                             OPTIONAL, -- Need R

btNameList-r16                          SetupRelease {BT-NameList-r16}                                OPTIONAL, -- Need M

wlanNameList-r16                        SetupRelease {WLAN-NameList-r16}                              OPTIONAL, -- Need M

sensorNameList-r16                      SetupRelease {Sensor-NameList-r16}                            OPTIONAL, -- Need M

obtainCommonLocation-r16                ENUMERATED {true}                                             OPTIONAL, -- Need R

sl-AssistanceConfigNR-r16               ENUMERATED{true}                                              OPTIONAL  -- Need R

}

OtherConfig-v1700 ::=                   SEQUENCE {

ul-GapFR2-PreferenceConfig-r17          ENUMERATED {true}                                             OPTIONAL, -- Need R

musim-GapAssistanceConfig-r17           SetupRelease {MUSIM-GapAssistanceConfig-r17}                  OPTIONAL, -- Need M

musim-LeaveAssistanceConfig-r17         SetupRelease {MUSIM-LeaveAssistanceConfig-r17}                OPTIONAL, -- Need M

successHO-Config-r17                    SetupRelease {SuccessHO-Config-r17}                           OPTIONAL, -- Need M

maxBW-PreferenceConfigFR2-2-r17         ENUMERATED {true}                                             OPTIONAL, -- Cond maxBW

maxMIMO-LayerPreferenceConfigFR2-2-r17  ENUMERATED {true}                                             OPTIONAL, -- Cond maxMIMO

minSchedulingOffsetPreferenceConfigExt-r17  ENUMERATED {true}                                         OPTIONAL, -- Cond minOffset

rlm-RelaxationReportingConfig-r17       SetupRelease {RLM-RelaxationReportingConfig-r17}              OPTIONAL, -- Need M

bfd-RelaxationReportingConfig-r17       SetupRelease {BFD-RelaxationReportingConfig-r17}              OPTIONAL, -- Need M

scg-DeactivationPreferenceConfig-r17    SetupRelease {SCG-DeactivationPreferenceConfig-r17}           OPTIONAL, -- Cond SCG

rrm-MeasRelaxationReportingConfig-r17   SetupRelease {RRM-MeasRelaxationReportingConfig-r17}          OPTIONAL, -- Need M

propDelayDiffReportConfig-r17           SetupRelease {PropDelayDiffReportConfig-r17}                  OPTIONAL  -- Need M

}

CandidateServingFreqListNR-r16 ::= SEQUENCE (SIZE (1..maxFreqIDC-r16)) OF ARFCN-ValueNR

MUSIM-GapAssistanceConfig-r17 ::= SEQUENCE {

musim-GapProhibitTimer-r17        ENUMERATED {s0, s0dot1, s0dot2, s0dot3, s0dot4, s0dot5, s1, s2, s3, s4, s5, s6, s7, s8, s9, s10}

}

MUSIM-LeaveAssistanceConfig-r17 ::=     SEQUENCE {

musim-LeaveWithoutResponseTimer-r17     ENUMERATED {ms10, ms20, ms40, ms60, ms80, ms100, spare2, spare1}

}

OtherConfig-v18xy ::=                   SEQUENCE {

musim-GapPriorityAssistanceConfig-r18           ENUMERATED {true}                                     OPTIONAL, -- Cond musimGapConfig

musim-CapabilityRestrictionConfig-r18            SetupRelease {MUSIM-CapabilityRestrictionConfig-r18} OPTIONAL -- Need M

}

Editor’s Note: FFS whether a separate control is needed for the UE assistance information of the “keep solution”, based on the RAN4 inputs on the capability signalings for “keep solution” and “priority-based solution”.

MUSIM-CapabilityRestrictionConfig-r18 ::=     SEQUENCE {

musim-candidateBandList-r18                  MUSIM-CandidateBandList-r18               OPTIONAL, -- Need M

musim-WaitTimer-r18     ENUMERATED {ms10, ms20, ms40, ms60, ms80, ms100, spare2, spare1},

musim-ProhibitTimer-r18     ENUMERATED {ms0, ms10, ms20, ms40, ms60, ms80, spare2, spare1}

}

MUSIM-CandidateBandbList-r18::= SEQUENCE (SIZE (1.. maxBands-MUSIM)) OF FreqBandIndicatorNR

maxBands-MUSIM      FFS

.

SuccessHO-Config-r17 ::=                SEQUENCE {

thresholdPercentageT304-r17             ENUMERATED {p40, p60, p80, spare5, spare4, spare3, spare2, spare1}      OPTIONAL, --Need R

thresholdPercentageT310-r17             ENUMERATED {p40, p60, p80, spare5, spare4, spare3, spare2, spare1}      OPTIONAL, --Need R

thresholdPercentageT312-r17             ENUMERATED {p20, p40, p60, p80, spare4, spare3, spare2, spare1}         OPTIONAL, --Need R

sourceDAPS-FailureReporting-r17         ENUMERATED {true}                                                       OPTIONAL, --Need R

...

}

OverheatingAssistanceConfig ::= SEQUENCE {

overheatingIndicationProhibitTimer    ENUMERATED {s0, s0dot5, s1, s2, s5, s10, s20, s30,

s60, s90, s120, s300, s600, spare3, spare2, spare1}

}

IDC-AssistanceConfig-r16 ::=    SEQUENCE {

candidateServingFreqListNR-r16  CandidateServingFreqListNR-r16                     OPTIONAL, -- Need R

...

}

DRX-PreferenceConfig-r16 ::=          SEQUENCE {

drx-PreferenceProhibitTimer-r16       ENUMERATED {

s0, s0dot5, s1, s2, s3, s4, s5, s6, s7,

s8, s9, s10, s20, s30, spare2, spare1}

}

MaxBW-PreferenceConfig-r16 ::=        SEQUENCE {

maxBW-PreferenceProhibitTimer-r16     ENUMERATED {

s0, s0dot5, s1, s2, s3, s4, s5, s6, s7,

s8, s9, s10, s20, s30, spare2, spare1}

}

MaxCC-PreferenceConfig-r16 ::=        SEQUENCE {

maxCC-PreferenceProhibitTimer-r16     ENUMERATED {

s0, s0dot5, s1, s2, s3, s4, s5, s6, s7,

s8, s9, s10, s20, s30, spare2, spare1}

}

MaxMIMO-LayerPreferenceConfig-r16 ::= SEQUENCE {

maxMIMO-LayerPreferenceProhibitTimer-r16 ENUMERATED {

s0, s0dot5, s1, s2, s3, s4, s5, s6, s7,

s8, s9, s10, s20, s30, spare2, spare1}

}

MinSchedulingOffsetPreferenceConfig-r16 ::=   SEQUENCE {

minSchedulingOffsetPreferenceProhibitTimer-r16 ENUMERATED {

s0, s0dot5, s1, s2, s3, s4, s5, s6, s7,

s8, s9, s10, s20, s30, spare2, spare1}

}

ReleasePreferenceConfig-r16 ::=       SEQUENCE {

releasePreferenceProhibitTimer-r16    ENUMERATED {

s0, s0dot5, s1, s2, s3, s4, s5, s6, s7,

s8, s9, s10, s20, s30, infinity, spare1},

connectedReporting                    ENUMERATED {true}                                               OPTIONAL  -- Need R

}

RLM-RelaxationReportingConfig-r17 ::= SEQUENCE {

rlm-RelaxtionReportingProhibitTimer   ENUMERATED {s0, s0dot5, s1, s2, s5, s10, s20, s30,

s60, s90, s120, s300, s600, infinity, spare2, spare1}

}

BFD-RelaxationReportingConfig-r17 ::= SEQUENCE {

bfd-RelaxtionReportingProhibitTimer   ENUMERATED {s0, s0dot5, s1, s2, s5, s10, s20, s30,

s60, s90, s120, s300, s600, infinity, spare2, spare1}

}

SCG-DeactivationPreferenceConfig-r17 ::=       SEQUENCE {

scg-DeactivationPreferenceProhibitTimer-r17    ENUMERATED {

s0, s1, s2, s4, s8, s10, s15, s30,

s60, s120, s180, s240, s300, s600, s900, s1800}

}

RRM-MeasRelaxationReportingConfig-r17 ::= SEQUENCE {

s-SearchDeltaP-Stationary-r17             ENUMERATED {dB2, dB3, dB6, dB9, dB12, dB15, spare2, spare1},

t-SearchDeltaP-Stationary-r17             ENUMERATED {s5, s10, s20, s30, s60, s120, s180, s240, s300, spare7, spare6, spare5,

spare4, spare3, spare2, spare1}

}

PropDelayDiffReportConfig-r17 ::= SEQUENCE {

threshPropDelayDiff-r17           ENUMERATED {ms0dot5, ms1, ms2, ms3, ms4, ms5, ms6 ,ms7, ms8, ms9, ms10, spare5,

spare4, spare3, spare2, spare1}                OPTIONAL,   -- Need M

neighCellInfoList-r17             SEQUENCE (SIZE (1..maxCellNTN-r17)) OF NeighbourCellInfo-r17         OPTIONAL    -- Need M

}

NeighbourCellInfo-r17  ::= SEQUENCE {

epochTime-r17                  EpochTime-r17,

ephemerisInfo-r17              EphemerisInfo-r17

}

-- TAG-OTHERCONFIG-STOP

-- ASN1STOP

OtherConfig field descriptions

bfd-RelaxationReportingConfig Configuration for the UE to report the relaxation state of BFD measurements.

candidateServingFreqListNR Indicates for each candidate NR serving cells, the center frequency around which UE is requested to report IDC issues.

connectedReporting Indicates that the UE can report a preference to remain in RRC_CONNECTED state following a report to leave RRC_CONNECTED state. If absent, the UE cannot report a preference to stay in RRC_CONNECTED state.

delayBudgetReportingProhibitTimer Prohibit timer for delay budget reporting. Value in seconds. Value s0 means prohibit timer is set to 0 seconds, value s0dot4 means prohibit timer is set to 0.4 seconds, and so on.

drx-PreferenceConfig Configuration for the UE to report assistance information to inform the gNB about the UE's DRX preferences for power saving.

drx-PreferenceProhibitTimer Prohibit timer for DRX preferences assistance information reporting. Value in seconds. Value s0 means prohibit timer is set to 0 seconds, value s0dot5 means prohibit timer is set to 0.5 seconds, value s1 means prohibit timer is set to 1 second and so on.

idc-AssistanceConfig Configuration for the UE to report assistance information to inform the gNB about UE detected IDC problem.

maxBW-PreferenceConfig Configuration for the UE to report assistance information to inform the gNB about the UE's preferred bandwidth for power saving.

maxBW-PreferenceProhibitTimer Prohibit timer for preferred bandwidth assistance information reporting. Value in seconds. Value s0 means prohibit timer is set to 0 seconds, value s0dot5 means prohibit timer is set to 0.5 seconds, value s1 means prohibit timer is set to 1 second and so on.

maxCC-PreferenceConfig Configuration for the UE to report assistance information to inform the gNB about the UE's preferred number of carriers for power saving.

maxBW-PreferenceConfigFR2-2 Configuration for the UE to report assistance information to inform the gNB about the UE's preferred bandwidth for power saving for FR2-2.

maxCC-PreferenceProhibitTimer Prohibit timer for preferred number of carriers assistance information reporting. Value in seconds. Value s0 means prohibit timer is set to 0 seconds, value s0dot5 means prohibit timer is set to 0.5 seconds, value s1 means prohibit timer is set to 1 second and so on.

maxMIMO-LayerPreferenceConfig Configuration for the UE to report assistance information to inform the gNB about the UE's preferred number of MIMO layers for power saving.

maxMIMO-LayerPreferenceConfigFR2-2 Configuration for the UE to report assistance information to inform the gNB about the UE's preferred number of MIMO layers for power saving for FR2-2.

maxMIMO-LayerPreferenceProhibitTimer Prohibit timer for preferred number of number of MIMO layers assistance information reporting. Value in seconds. Value s0 means prohibit timer is set to 0 seconds, value s0dot5 means prohibit timer is set to 0.5 seconds, value s1 means prohibit timer is set to 1 second and so on.

minSchedulingOffsetPreferenceConfig Configuration for the UE to report assistance information to inform the gNB about the UE's preferred minimumSchedulingOffset value for cross-slot scheduling for power saving.

minSchedulingOffsetPreferenceConfigExt Configuration for the UE to report assistance information to inform the gNB about the UE's preferred minimumSchedulingOffset value for cross-slot scheduling for power saving for SCS 480 kHz and/or 960 kHz.

minSchedulingOffsetPreferenceProhibitTimer Prohibit timer for preferred minimumSchedulingOffset assistance information reporting. Value in seconds. Value s0 means prohibit timer is set to 0 seconds, value s0dot5 means prohibit timer is set to 0.5 seconds, value s1 means prohibit timer is set to 1 second and so on.

musim-candidateBandList A list of candidate bands which UE is requested to report the forbidden or affected bands for MUSIM purpose. If the field is absent, the UE is allowed to report the forbidden or affected band using frequencyBandListFilter in the UECapabilityEnquiry.

musim-GapAssistanceConfig Configuration for the UE to report assistance information for gap preference.

musim-GapProhibitTimer Prohibit timer for MUSIM assistance information reporting for gap preference.

musim-LeaveAssistanceConfig Configuration for the UE to report assistance information for leaving RRC_CONNECTED for MUSIM purpose.

musim-LeaveWithoutResponseTimer Indicates the timer for the UE to enter RRC_IDLE for MUSIM purpose as defined in clause 5.3.8.6.

musim-needForGapsConfigNR Configuration for the UE to report measurement gap requirement information of NR target bands.

musim-ProhitTimer Indicates the timer for the UE to indicate temporary capability restriction for impacted band(s)/frequencies. Value in milliseconds. Value ms0 means prohibit timer is set to 0 milliseconds, value ms10 means prohibit timer is set to 10 milliseconds and so on.

musim-WaitTimer Indicates the timer for the UE to apply the suggested temporary capability restriction as defined in clause 5.3.5.x. Value in milliseconds. Value ms10 means prohibit timer is set to 10 milliseconds, value ms20 means prohibit timer is set to 20 milliseconds and so on.

obtainCommonLocation Requests the UE to attempt to have detailed location information available using GNSS. NR configures the field if includeCommonLocationInfo is configured for one or more measurements.

overheatingAssistanceConfig Configuration for the UE to report assistance information to inform the gNB about UE detected internal overheating.

overheatingIndicationProhibitTimer Prohibit timer for overheating assistance information reporting. Value in seconds. Value s0 means prohibit timer is set to 0 seconds, value s0dot5 means prohibit timer is set to 0.5 seconds, value s1 means prohibit timer is set to 1 second and so on.

propDelayDiffReportConfig Configuration for the UE to report service link propagation delay difference between serving cell and neighbour cell(s).

referenceTimePreferenceReporting If present, the field indicates the UE is configured to provide reference time assistance information.

releasePreferenceConfig Configuration for the UE to report assistance information to inform the gNB about the UE's preference to leave RRC_CONNECTED state.

rlm-RelaxationReportingConfig Configuration for the UE to report the relaxation state of RLM measurements.

releasePreferenceProhibitTimer Prohibit timer for release preference assistance information reporting. Value in seconds. Value s0 means prohibit timer is set to 0 seconds, value s0dot5 means prohibit timer is set to 0.5 seconds, value s1 means prohibit timer is set to 1 second and so on. Value infinity means that once a UE has reported a release preference, the UE cannot report a release preference again during the RRC connection.

s-SearchDeltaP-Stationary Parameter "SSearchDeltaP-StationaryConnected" in 5.7.4.4. Value dB2 corresponds to 2 dB, dB3 corresponds to 3 dB and so on.

scg-DeactivationPreferenceConfig Configuration of the UE to indicate its preference for SCG deactivation.

scg -StatePreferenceProhibitTimer Prohibit timer for UE indication of its preference for SCG deactivation. Value in seconds. Value s0 means prohibit timer is set to 0 seconds, value s1 means prohibit timer is set to 1 second and so on.

sensorNameList Configuration for the UE to report measurements from specific sensors.

sl-AssistanceConfigNR Indicate whether UE is configured to provide configured grant assistance information for NR sidelink communication.

sourceDAPS-FailureReporting This field indicates whether the UE shall generate the SHR upon successfully completing the DAPS handover to the target cell and if a radio link failure was experienced in the source PCell while executing the DAPS handover. This field is set in the otherConfig configured by the source cell of the DAPS handover.

successHO-Config Configuration for the UE to report the successful handover information to the network.

t-SearchDeltaP-Stationary Parameter "TSearchDeltaP-StationaryConnected" in 5.7.4.4. Value in seconds. Value s5 means 5 seconds, value s10 means 10 seconds and so on.

thresholdPercentageT304 This field indicates the threshold for the ratio in percentage between the elapsed T304 timer and the configured value of the T304 timer. Value p40 corresponds to 40%, value p60 corresponds to 60% and so on. This field is set in the otherConfig configured by the target cell of the handover.

thresholdPercentageT310 This field indicates the threshold for the ratio in percentage between the elapsed T310 timer and the configured value of the T310 timer. Value p40 corresponds to 40%, value p60 corresponds to 60% and so on. This field is set in the otherConfig configured by the source cell of the handover.

thresholdPercentageT312 This field indicates the threshold for the ratio in percentage between the elapsed T312 timer and the configured value(s) of the T312 timer. Value p20 corresponds to 20%, value p40 corresponds to 40% and so on. This field is set in the otherConfig configured by the source cell of the handover.

threshPropDelayDiff Threshold for service link propagation delay difference report as specified in 5.7.4.2.

ul-GapFR2-PreferenceConfig Indicates whether UE is configured to request for FR2 UL gap activation/deactivation and preferred FR2 UL gap pattern.

NeighbourCellInfo field descriptions

epochTime Indicates the epoch time used along with the ephemerisInfo to derive the propagation delay difference for the associated neighbour cell. The UE considers epoch time, indicated by the SFN and sub-frame number in this field, to be the frame nearest to the frame in which the message indicating the epoch time is received. This field is used based on the timing of the serving cell, i.e. the SFN and sub-frame number indicated in this field refers to the SFN and sub-frame of the serving cell.

| Conditional Presence | Explanation |
| --- | --- |
| maxBW | This field is optionally present, need R, if maxBW-PreferenceConfig-r16 is setup; otherwise it is absent, need R. |
| maxMIMO | This field is optionally present, need R, if maxMIMO-LayerPreferenceConfig-r16 is setup; otherwise it is absent, need R. |
| minOffset | This field is optionally present, need R, if minSchedulingOffsetPreferenceConfig-r16 is setup; otherwise it is absent, need R. |
| musimGapConfig | This field is optionally present, need R, if musim-GapAssistanceConfig-r17 is configured; otherwise it is absent, need R. |
| SCG | This field is optionally present, need M, in an RRCReconfiguration message not within mrdc-SecondaryCellGroup and received, either via SRB3 within DLInformationTransferMRDC or via SRB1. Otherwise, it is absent. |

#### –	PhysCellIdUTRA-FDD

The IE PhysCellIdUTRA-FDD is used to indicate the physical layer identity of the cell, i.e. the primary scrambling code, as defined in TS 25.331 [45].

PhysCellIdUTRA-FDD information element

-- ASN1START

-- TAG-PHYSCELLIDUTRA-FDD-START

PhysCellIdUTRA-FDD-r16 ::=        INTEGER (0..511)

-- TAG-PHYSCELLIDUTRA-FDD-STOP

-- ASN1STOP

#### –	RRC-TransactionIdentifier

The IE RRC-TransactionIdentifier is used, together with the message type, for the identification of an RRC procedure (transaction).

RRC-TransactionIdentifier information element

-- ASN1START

-- TAG-RRC-TRANSACTIONIDENTIFIER-START

RRC-TransactionIdentifier ::=       INTEGER (0..3)

-- TAG-RRC-TRANSACTIONIDENTIFIER-STOP

-- ASN1STOP

#### –	Sensor-NameList

The IE Sensor-NameList is used to indicate the names of the sensors which the UE is configured to measure.

Sensor-NameList information element

-- ASN1START

-- TAG-SENSORNAMELIST-START

Sensor-NameList-r16 ::= SEQUENCE {

measUncomBarPre-r16     ENUMERATED {true}            OPTIONAL,  -- Need R

measUeSpeed             ENUMERATED {true}            OPTIONAL,  -- Need R

measUeOrientation       ENUMERATED {true}            OPTIONAL   -- Need R

}

-- TAG-SENSORNAMELIST-STOP

-- ASN1STOP

Sensor-NameList field descriptions

measUncomBarPre If configured, the UE reports the uncompensated Barometeric pressure measurement as defined in TS 37.355 [49].

measUeSpeed If configured, the UE reports the UE speed measurement as defined in TS 37.355 [49].

measUeOrientation If configured, the UE reports the UE orientation information as defined in TS 37.355 [49].

#### –	TraceReference

The TraceReference contains parameter Trace Reference as defined in TS 32.422 [52].

TraceReference information element

-- ASN1START

-- TAG-TRACEREFERENCE-START

TraceReference-r16 ::= SEQUENCE {

plmn-Identity-r16      PLMN-Identity,

traceId-r16            OCTET STRING (SIZE (3))

}

-- TAG-TRACEREFERENCE-STOP

-- ASN1STOP

#### –	UE-MeasurementsAvailable

The IE UE-MeasurementsAvailable is used to indicate all relevant available indicators for UE measurements.

UE-MeasurementsAvailable information element

-- ASN1START

-- TAG-UE-MeasurementsAvailable-START

UE-MeasurementsAvailable-r16 ::=              SEQUENCE {

logMeasAvailable-r16                         ENUMERATED {true}               OPTIONAL,

logMeasAvailableBT-r16                       ENUMERATED {true}               OPTIONAL,

logMeasAvailableWLAN-r16                     ENUMERATED {true}               OPTIONAL,

connEstFailInfoAvailable-r16                 ENUMERATED {true}               OPTIONAL,

rlf-InfoAvailable-r16                        ENUMERATED {true}               OPTIONAL,

...,

[[

successHO-InfoAvailable-r17                  ENUMERATED {true}               OPTIONAL,

sigLogMeasConfigAvailable-r17                BOOLEAN                         OPTIONAL

]]

}

-- TAG-UE-MeasurementsAvailable-STOP

-- ASN1STOP

#### –	UTRA-FDD-Q-OffsetRange

The IE UTRA-FDD-Q-OffsetRange is used to indicate a frequency specific offset to be applied when evaluating triggering conditions for measurement reporting. The value is in dB. Value dB-24 corresponds to -24 dB, value dB-22 corresponds to -22 dB and so on.

UTRA-FDD-Q-OffsetRange information element

-- ASN1START

-- TAG-UTRA-FDD-Q-OFFSETRANGE-START

UTRA-FDD-Q-OffsetRange-r16 ::=              ENUMERATED {

dB-24, dB-22, dB-20, dB-18, dB-16, dB-14,

dB-12, dB-10, dB-8, dB-6, dB-5, dB-4, dB-3,

dB-2, dB-1, dB0, dB1, dB2, dB3, dB4, dB5,

dB6, dB8, dB10, dB12, dB14, dB16, dB18,

dB20, dB22, dB24}

-- TAG-UTRA-FDD-Q-OFFSETRANGE-STOP

-- ASN1STOP

#### –	VisitedCellInfoList

The IE VisitedCellInfoList includes the mobility history information of maximum of 16 most recently visited primary cells or time spent in any cell selection state and/or camped on any cell state in NR or E-UTRA and, in case of Dual Connectivity, the mobility history information of maxPSCellHistory most recently visited primary secondary cell group cells across all the primary cells included in the VisitedCellInfoList. The most recently visited cell is stored first in the list. The list includes cells visited in RRC_IDLE, RRC_INACTIVE and RRC_CONNECTED states for NR and RRC_IDLE and RRC_CONNECTED for E-UTRA.

VisitedCellInfoList information element

-- ASN1START

-- TAG-VISITEDCELLINFOLIST-START

VisitedCellInfoList-r16 ::= SEQUENCE (SIZE (1..maxCellHistory-r16)) OF VisitedCellInfo-r16

VisitedCellInfo-r16 ::=  SEQUENCE {

visitedCellId-r16        CHOICE {

nr-CellId-r16            CHOICE {

cgi-Info                 CGI-Info-Logging-r16,

pci-arfcn-r16            PCI-ARFCN-NR-r16

},

eutra-CellId-r16         CHOICE {

cellGlobalId-r16         CGI-InfoEUTRA,

pci-arfcn-r16                PCI-ARFCN-EUTRA-r16

}

}                                        OPTIONAL,

timeSpent-r16            INTEGER (0..4095),

...,

[[

visitedPSCellInfoListReport-r17    VisitedPSCellInfoList-r17                   OPTIONAL

]]

}

VisitedPSCellInfoList-r17 ::= SEQUENCE (SIZE (1..maxPSCellHistory-r17)) OF VisitedPSCellInfo-r17

VisitedPSCellInfo-r17 ::=    SEQUENCE {

visitedCellId-r17            CHOICE {

nr-CellId-r17                CHOICE {

cgi-Info-r17                 CGI-Info-Logging-r16,

pci-arfcn-r17                PCI-ARFCN-NR-r16

},

eutra-CellId-r17         CHOICE {

cellGlobalId-r17         CGI-InfoEUTRALogging,

pci-arfcn-r17            PCI-ARFCN-EUTRA-r16

}

}                                                 OPTIONAL,

timeSpent-r17            INTEGER (0..4095),

...

}

-- TAG-VISITEDCELLINFOLIST-STOP

-- ASN1STOP

VisitedCellInfoList field descriptions

timeSpent This field indicates the duration of stay in the cell or in any cell selection state and/or camped on any cell state in NR or E-UTRA approximated to the closest second. If included in VisitedPSCellInfo, it indicates the duration of stay in the PSCell or without any PSCell. If the duration of stay exceeds 4095s, the UE shall set it to 4095s.

visitedCellId This field indicates the visited cell id including NR and E-UTRA cells.

#### –	WLAN-NameList

The IE WLAN-NameList is used to indicate the names of the WLAN AP for which the UE is configured to measure.

WLAN-NameList information element

-- ASN1START

-- TAG-WLANNAMELIST-START

WLAN-NameList-r16 ::= SEQUENCE (SIZE (1..maxWLAN-Name-r16)) OF WLAN-Name-r16

WLAN-Name-r16 ::= OCTET STRING (SIZE (1..32))

-- ASN1STOP

-- TAG-WLANNAMELIST-STOP

WLAN-NameList field descriptions

WLAN-Name If configured, the UE only performs WLAN measurements according to the names identified. For each name, it refers to Service Set Identifier (SSID) defined in IEEE 802.11-2012 [50].

---------------------------------------------------------Skip Unchanged----------------------------------------------------------

## 6.4	RRC multiplicity and type constraint values

### –	Multiplicity and type constraint definitions

-- ASN1START

-- TAG-MULTIPLICITY-AND-TYPE-CONSTRAINT-DEFINITIONS-START

maxAdditionalRACH-r17                   INTEGER ::= 256     -- Maximum number of additional RACH configurations.

maxAI-DCI-PayloadSize-r16               INTEGER ::= 128      --Maximum size of the DCI payload scrambled with ai-RNTI

maxAI-DCI-PayloadSize-1-r16             INTEGER ::= 127      --Maximum size of the DCI payload scrambled with ai-RNTI minus 1

maxBandComb                             INTEGER ::= 65536   -- Maximum number of DL band combinations

maxBandsUTRA-FDD-r16                    INTEGER ::= 64      -- Maximum number of bands listed in UTRA-FDD UE caps

maxBH-RLC-ChannelID-r16                 INTEGER ::= 65536   -- Maximum value of BH RLC Channel ID

maxBT-IdReport-r16                      INTEGER ::= 32      -- Maximum number of Bluetooth IDs to report

maxBT-Name-r16                          INTEGER ::= 4       -- Maximum number of Bluetooth name

maxCAG-Cell-r16                         INTEGER ::= 16      -- Maximum number of NR CAG cell ranges in SIB3, SIB4

maxTwoPUCCH-Grp-ConfigList-r16          INTEGER ::= 32      -- Maximum number of supported configuration(s) of {primary PUCCH group

-- config, secondary PUCCH group config}

maxTwoPUCCH-Grp-ConfigList-r17          INTEGER ::= 16      -- Maximum number of supported configuration(s) of {primary PUCCH group

-- config, secondary PUCCH group config} for PUCCH cell switching

maxCBR-Config-r16                       INTEGER ::= 8       -- Maximum number of CBR range configurations for sidelink communication

-- congestion control

maxCBR-Config-1-r16                     INTEGER ::= 7       -- Maximum number of CBR range configurations for sidelink communication

-- congestion control minus 1

maxCBR-Level-r16                        INTEGER ::= 16      -- Maximum number of CBR levels

maxCBR-Level-1-r16                      INTEGER ::= 15      -- Maximum number of CBR levels minus 1

maxCellExcluded                         INTEGER ::= 16      -- Maximum number of NR exclude-listed cell ranges in SIB3, SIB4

maxCellGroupings-r16                    INTEGER ::= 32      -- Maximum number of cell groupings for NR-DC

maxCellHistory-r16                      INTEGER ::= 16      -- Maximum number of visited PCells reported

maxPSCellHistory-r17                    INTEGER ::= 16      -- Maximum number of visited PSCells across all reported PCells

maxCellInter                            INTEGER ::= 16      -- Maximum number of inter-Freq cells listed in SIB4

maxCellIntra                            INTEGER ::= 16      -- Maximum number of intra-Freq cells listed in SIB3

maxCellMeasEUTRA                        INTEGER ::= 32      -- Maximum number of cells in E-UTRAN

maxCellMeasIdle-r16                     INTEGER ::= 8       -- Maximum number of cells per carrier for idle/inactive measurements

maxCellMeasUTRA-FDD-r16                 INTEGER ::= 32      -- Maximum number of cells in FDD UTRAN

maxCellNTN-r17                          INTEGER ::= 4       -- Maximum number of NTN neighbour cells for which assistance information is

-- provided

maxCarrierTypePairList-r16              INTEGER ::= 16      -- Maximum number of supported carrier type pair of (carrier type on which

-- CSI measurement is performed, carrier type on which CSI reporting is

-- performed) for CSI reporting cross PUCCH group

maxCellAllowed                          INTEGER ::= 16      -- Maximum number of NR allow-listed cell ranges in SIB3, SIB4

maxEARFCN                               INTEGER ::= 262143  -- Maximum value of E-UTRA carrier frequency

maxEUTRA-CellExcluded                   INTEGER ::= 16      -- Maximum number of E-UTRA exclude-listed physical cell identity ranges

-- in SIB5

maxEUTRA-NS-Pmax                        INTEGER ::= 8       -- Maximum number of NS and P-Max values per band

maxFeatureCombPreamblesPerRACHResource-r17 INTEGER ::= 256  -- Maximum number of feature combination preambles.

maxLogMeasReport-r16                    INTEGER ::= 520     -- Maximum number of entries for logged measurements

maxMultiBands                           INTEGER ::= 8       -- Maximum number of additional frequency bands that a cell belongs to

maxNARFCN                               INTEGER ::= 3279165 -- Maximum value of NR carrier frequency

maxNR-NS-Pmax                           INTEGER ::= 8       -- Maximum number of NS and P-Max values per band

maxFreqIdle-r16                         INTEGER ::= 8       -- Maximum number of carrier frequencies for idle/inactive measurements

maxNrofServingCells                     INTEGER ::= 32      -- Max number of serving cells (SpCells + SCells)

maxNrofServingCells-1                   INTEGER ::= 31      -- Max number of serving cells (SpCells + SCells) minus 1

maxNrofAggregatedCellsPerCellGroup      INTEGER ::= 16

maxNrofAggregatedCellsPerCellGroupMinus4-r16 INTEGER ::= 12

maxNrofDUCells-r16                      INTEGER ::= 512     -- Max number of cells configured on the collocated IAB-DU

maxNrofAppLayerMeas-r17                 INTEGER ::= 16      -- Max number of simultaneous application layer measurements

maxNrofAppLayerMeas-1-r17               INTEGER ::= 15      -- Max number of simultaneous application layer measurements minus 1

maxNrofAvailabilityCombinationsPerSet-r16   INTEGER ::= 512 -- Max number of AvailabilityCombinationId used in the DCI format 2_5

maxNrofAvailabilityCombinationsPerSet-1-r16 INTEGER ::= 511 -- Max number of AvailabilityCombinationId used in the DCI format 2_5 minus 1

maxNrofIABResourceConfig-r17            INTEGER ::= 65536   -- Max number of IAB-ResourceConfigID used in MAC CE

maxNrofIABResourceConfig-1-r17          INTEGER ::= 65535   -- Max number of IAB-ResourceConfigID used in MAC CE minus 1

maxNrofSCellActRS-r17                   INTEGER ::= 255     -- Max number of RS configurations per SCell for SCell activation

maxNrofSCells                           INTEGER ::= 31      -- Max number of secondary serving cells per cell group

maxNrofCellMeas                         INTEGER ::= 32      -- Maximum number of entries in each of the cell lists in a measurement object

maxNrofCRS-IM-InterfCell-r17            INTEGER ::= 8       -- Maximum number of LTE interference cells for CRS-IM per UE

maxNrofRelayMeas-r17                    INTEGER ::= 32      -- Maximum number of L2 U2N Relay UEs to measure for each measurement object

-- on sidelink frequency

maxNrofCG-SL-r16                        INTEGER ::= 8       -- Max number of sidelink configured grant

maxNrofCG-SL-1-r16                      INTEGER ::= 7       -- Max number of sidelink configured grant minus 1

maxSL-GC-BC-DRX-QoS-r17                 INTEGER ::= 16      -- Max number of sidelink DRX configurations for NR

-- sidelink groupcast/broadcast communication

maxNrofSL-RxInfoSet-r17                 INTEGER ::= 4       -- Max number of sidelink DRX configuration sets in sidelink DRX assistant

-- information

maxNrofSS-BlocksToAverage               INTEGER ::= 16      -- Max number for the (max) number of SS blocks to average to determine cell measurement

maxNrofCondCells-r16                    INTEGER ::= 8       -- Max number of conditional candidate SpCells

maxNrofCondCells-1-r17                  INTEGER ::= 7       -- Max number of conditional candidate SpCells minus 1

maxNrofCSI-RS-ResourcesToAverage        INTEGER ::= 16      -- Max number for the (max) number of CSI-RS to average to determine cell measurement

maxNrofDL-Allocations                   INTEGER ::= 16      -- Maximum number of PDSCH time domain resource allocations

maxNrofDL-AllocationsExt-r17            INTEGER ::= 64      -- Maximum number of PDSCH time domain resource allocations for multi-PDSCH

-- scheduling

maxNrofPDU-Sessions-r17                 INTEGER ::= 256     -- Maximum number of PDU Sessions

maxNrofSR-ConfigPerCellGroup            INTEGER ::= 8       -- Maximum number of SR configurations per cell group

maxLCG-ID                               INTEGER ::= 7       -- Maximum value of LCG ID

maxLCG-ID-IAB-r17                       INTEGER ::= 255     -- Maximum value of LCG ID for IAB-MT

maxLC-ID                                INTEGER ::= 32      -- Maximum value of Logical Channel ID

maxLC-ID-Iab-r16                        INTEGER ::= 65855   -- Maximum value of BH Logical Channel ID extension

maxLTE-CRS-Patterns-r16                 INTEGER ::= 3       -- Maximum number of additional LTE CRS rate matching patterns

maxNrofTAGs                             INTEGER ::= 4       -- Maximum number of Timing Advance Groups

maxNrofTAGs-1                           INTEGER ::= 3       -- Maximum number of Timing Advance Groups minus 1

maxNrofBWPs                             INTEGER ::= 4       -- Maximum number of BWPs per serving cell

maxNrofCombIDC                          INTEGER ::= 128     -- Maximum number of reported MR-DC combinations for IDC

maxNrofSymbols-1                        INTEGER ::= 13      -- Maximum index identifying a symbol within a slot (14 symbols, indexed from 0..13)

maxNrofSlots                            INTEGER ::= 320     -- Maximum number of slots in a 10 ms period

maxNrofSlots-1                          INTEGER ::= 319     -- Maximum number of slots in a 10 ms period minus 1

maxNrofPhysicalResourceBlocks           INTEGER ::= 275     -- Maximum number of PRBs

maxNrofPhysicalResourceBlocks-1         INTEGER ::= 274     -- Maximum number of PRBs minus 1

maxNrofPhysicalResourceBlocksPlus1      INTEGER ::= 276     -- Maximum number of PRBs plus 1

maxNrofControlResourceSets              INTEGER ::= 12      -- Max number of CoReSets configurable on a serving cell

maxNrofControlResourceSets-1            INTEGER ::= 11      -- Max number of CoReSets configurable on a serving cell minus 1

maxNrofControlResourceSets-1-r16        INTEGER ::= 15      -- Max number of CoReSets configurable on a serving cell extended in minus 1

maxNrofCoresetPools-r16                 INTEGER ::= 2       -- Maximum number of CORESET pools

maxCoReSetDuration                      INTEGER ::= 3       -- Max number of OFDM symbols in a control resource set

maxNrofSearchSpaces-1                   INTEGER ::= 39      -- Max number of Search Spaces minus 1

maxNrofSearchSpacesLinks-1-r17          INTEGER ::= 39      -- Max number of Search Space links minus 1

maxNrofBFDResourcePerSet-r17            INTEGER ::= 64      -- Max number of reference signal in one BFD set

maxSFI-DCI-PayloadSize                  INTEGER ::= 128     -- Max number payload of a DCI scrambled with SFI-RNTI

maxSFI-DCI-PayloadSize-1                INTEGER ::= 127     -- Max number payload of a DCI scrambled with SFI-RNTI minus 1

maxIAB-IP-Address-r16                   INTEGER ::= 32      -- Max number of assigned IP addresses

maxINT-DCI-PayloadSize                  INTEGER ::= 126     -- Max number payload of a DCI scrambled with INT-RNTI

maxINT-DCI-PayloadSize-1                INTEGER ::= 125     -- Max number payload of a DCI scrambled with INT-RNTI minus 1

maxNrofRateMatchPatterns                INTEGER ::= 4       -- Max number of rate matching patterns that may be configured

maxNrofRateMatchPatterns-1              INTEGER ::= 3       -- Max number of rate matching patterns that may be configured minus 1

maxNrofRateMatchPatternsPerGroup        INTEGER ::= 8       -- Max number of rate matching patterns that may be configured in one group

maxNrofCSI-ReportConfigurations         INTEGER ::= 48      -- Maximum number of report configurations

maxNrofCSI-ReportConfigurations-1       INTEGER ::= 47      -- Maximum number of report configurations minus 1

maxNrofCSI-ResourceConfigurations       INTEGER ::= 112     -- Maximum number of resource configurations

maxNrofCSI-ResourceConfigurations-1     INTEGER ::= 111     -- Maximum number of resource configurations minus 1

maxNrofAP-CSI-RS-ResourcesPerSet        INTEGER ::= 16

maxNrOfCSI-AperiodicTriggers            INTEGER ::= 128     -- Maximum number of triggers for aperiodic CSI reporting

maxNrofReportConfigPerAperiodicTrigger  INTEGER ::= 16      -- Maximum number of report configurations per trigger state for aperiodic reporting

maxNrofNZP-CSI-RS-Resources             INTEGER ::= 192     -- Maximum number of Non-Zero-Power (NZP) CSI-RS resources

maxNrofNZP-CSI-RS-Resources-1           INTEGER ::= 191     -- Maximum number of Non-Zero-Power (NZP) CSI-RS resources minus 1

maxNrofNZP-CSI-RS-ResourcesPerSet       INTEGER ::= 64      -- Maximum number of NZP CSI-RS resources per resource set

maxNrofNZP-CSI-RS-ResourceSets          INTEGER ::= 64      -- Maximum number of NZP CSI-RS resource sets per cell

maxNrofNZP-CSI-RS-ResourceSets-1        INTEGER ::= 63      -- Maximum number of NZP CSI-RS resource sets per cell minus 1

maxNrofNZP-CSI-RS-ResourceSetsPerConfig INTEGER ::= 16      -- Maximum number of resource sets per resource configuration

maxNrofNZP-CSI-RS-ResourcesPerConfig    INTEGER ::= 128     -- Maximum number of resources per resource configuration

maxNrofZP-CSI-RS-Resources              INTEGER ::= 32      -- Maximum number of Zero-Power (ZP) CSI-RS resources

maxNrofZP-CSI-RS-Resources-1            INTEGER ::= 31      -- Maximum number of Zero-Power (ZP) CSI-RS resources minus 1

maxNrofZP-CSI-RS-ResourceSets-1         INTEGER ::= 15

maxNrofZP-CSI-RS-ResourcesPerSet        INTEGER ::= 16

maxNrofZP-CSI-RS-ResourceSets           INTEGER ::= 16

maxNrofCSI-IM-Resources                 INTEGER ::= 32      -- Maximum number of CSI-IM resources

maxNrofCSI-IM-Resources-1               INTEGER ::= 31      -- Maximum number of CSI-IM resources minus 1

maxNrofCSI-IM-ResourcesPerSet           INTEGER ::= 8       -- Maximum number of CSI-IM resources per set

maxNrofCSI-IM-ResourceSets              INTEGER ::= 64      -- Maximum number of NZP CSI-IM resource sets per cell

maxNrofCSI-IM-ResourceSets-1            INTEGER ::= 63      -- Maximum number of NZP CSI-IM resource sets per cell minus 1

maxNrofCSI-IM-ResourceSetsPerConfig     INTEGER ::= 16      -- Maximum number of CSI IM resource sets per resource configuration

maxNrofCSI-SSB-ResourcePerSet           INTEGER ::= 64      -- Maximum number of SSB resources in a resource set

maxNrofCSI-SSB-ResourceSets             INTEGER ::= 64      -- Maximum number of CSI SSB resource sets per cell

maxNrofCSI-SSB-ResourceSets-1           INTEGER ::= 63      -- Maximum number of CSI SSB resource sets per cell minus 1

maxNrofCSI-SSB-ResourceSetsPerConfig    INTEGER ::= 1       -- Maximum number of CSI SSB resource sets per resource configuration

maxNrofCSI-SSB-ResourceSetsPerConfigExt INTEGER ::= 2       -- Maximum number of CSI SSB resource sets per resource configuration

-- extended

maxNrofFailureDetectionResources        INTEGER ::= 10      -- Maximum number of failure detection resources

maxNrofFailureDetectionResources-1      INTEGER ::= 9       -- Maximum number of failure detection resources minus 1

maxNrofFailureDetectionResources-1-r17  INTEGER ::= 63      -- Maximum number of the enhanced failure detection resources minus 1

maxNrofFreqSL-r16                       INTEGER ::= 8       -- Maximum number of carrier frequency for NR sidelink communication

maxNrofSL-BWPs-r16                      INTEGER ::= 4       -- Maximum number of BWP for NR sidelink communication

maxFreqSL-EUTRA-r16                     INTEGER ::= 8       -- Maximum number of EUTRA anchor carrier frequency for NR sidelink communication

maxNrofSL-MeasId-r16                    INTEGER ::= 64      -- Maximum number of sidelink measurement identity (RSRP) per destination

maxNrofSL-ObjectId-r16                  INTEGER ::= 64      -- Maximum number of sidelink measurement objects (RSRP) per destination

maxNrofSL-ReportConfigId-r16            INTEGER ::= 64      -- Maximum number of sidelink measurement reporting configuration(RSRP) per destination

maxNrofSL-PoolToMeasureNR-r16           INTEGER ::= 8       -- Maximum number of resource pool for NR sidelink measurement to measure for

-- each measurement object (for CBR)

maxFreqSL-NR-r16                        INTEGER ::= 8       -- Maximum number of NR anchor carrier frequency for NR sidelink communication

maxNrofSL-QFIs-r16                      INTEGER ::= 2048    -- Maximum number of QoS flow for NR sidelink communication per UE

maxNrofSL-QFIsPerDest-r16               INTEGER ::= 64      -- Maximum number of QoS flow per destination for NR sidelink communication

maxNrofObjectId                         INTEGER ::= 64      -- Maximum number of measurement objects

maxNrofPageRec                          INTEGER ::= 32      -- Maximum number of page records

maxNrofPCI-Ranges                       INTEGER ::= 8       -- Maximum number of PCI ranges

maxPLMN                                 INTEGER ::= 12      -- Maximum number of PLMNs broadcast and reported by UE at establishment

maxTAC-r17                              INTEGER ::= 12      -- Maximum number of Tracking Area Codes to which a cell belongs to

maxNrofCSI-RS-ResourcesRRM              INTEGER ::= 96      -- Maximum number of CSI-RS resources per cell for an RRM measurement object

maxNrofCSI-RS-ResourcesRRM-1            INTEGER ::= 95      -- Maximum number of CSI-RS resources per cell for an RRM measurement object

-- minus 1.

maxNrofMeasId                           INTEGER ::= 64      -- Maximum number of configured measurements

maxNrofQuantityConfig                   INTEGER ::= 2       -- Maximum number of quantity configurations

maxNrofCSI-RS-CellsRRM                  INTEGER ::= 96      -- Maximum number of cells with CSI-RS resources for an RRM measurement object

maxNrofSL-Dest-r16                      INTEGER ::= 32      -- Maximum number of destination for NR sidelink communication and discovery

maxNrofSL-Dest-1-r16                    INTEGER ::= 31      -- Highest index of destination for NR sidelink communication and discovery

maxNrofSLRB-r16                         INTEGER ::= 512     -- Maximum number of radio bearer for NR sidelink communication per UE

maxSL-LCID-r16                          INTEGER ::= 512     -- Maximum number of RLC bearer for NR sidelink communication per UE

maxSL-SyncConfig-r16                    INTEGER ::= 16      -- Maximum number of sidelink Sync configurations

maxNrofRXPool-r16                       INTEGER ::= 16      -- Maximum number of Rx resource pool for NR sidelink communication and

-- discovery

maxNrofTXPool-r16                       INTEGER ::= 8       -- Maximum number of Tx resource pool for NR sidelink communication and

-- discovery

maxNrofPoolID-r16                       INTEGER ::= 16      -- Maximum index of resource pool for NR sidelink communication and

-- discovery

maxNrofSRS-PathlossReferenceRS-r16      INTEGER ::= 64      -- Maximum number of RSs used as pathloss reference for SRS power control.

maxNrofSRS-PathlossReferenceRS-1-r16    INTEGER ::= 63      -- Maximum number of RSs used as pathloss reference for SRS power control

-- minus 1.

maxNrofSRS-ResourceSets                 INTEGER ::= 16      -- Maximum number of SRS resource sets in a BWP.

maxNrofSRS-ResourceSets-1               INTEGER ::= 15      -- Maximum number of SRS resource sets in a BWP minus 1.

maxNrofSRS-PosResourceSets-r16          INTEGER ::= 16      -- Maximum number of SRS Positioning resource sets in a BWP.

maxNrofSRS-PosResourceSets-1-r16        INTEGER ::= 15      -- Maximum number of SRS Positioning resource sets in a BWP minus 1.

maxNrofSRS-Resources                    INTEGER ::= 64      -- Maximum number of SRS resources.

maxNrofSRS-Resources-1                  INTEGER ::= 63      -- Maximum number of SRS resources minus 1.

maxNrofSRS-PosResources-r16             INTEGER ::= 64      -- Maximum number of SRS Positioning resources.

maxNrofSRS-PosResources-1-r16           INTEGER ::= 63      -- Maximum number of SRS Positioning resources minus 1.

maxNrofSRS-ResourcesPerSet              INTEGER ::= 16      -- Maximum number of SRS resources in an SRS resource set

maxNrofSRS-TriggerStates-1              INTEGER ::= 3       -- Maximum number of SRS trigger states minus 1, i.e., the largest code point.

maxNrofSRS-TriggerStates-2              INTEGER ::= 2       -- Maximum number of SRS trigger states minus 2.

maxRAT-CapabilityContainers             INTEGER ::= 8       -- Maximum number of interworking RAT containers (incl NR and MRDC)

maxSimultaneousBands                    INTEGER ::= 32      -- Maximum number of simultaneously aggregated bands

maxULTxSwitchingBandPairs               INTEGER ::= 32      -- Maximum number of band pairs supporting dynamic UL Tx switching in a band

-- combination.

maxNrofSlotFormatCombinationsPerSet     INTEGER ::= 512     -- Maximum number of Slot Format Combinations in a SF-Set.

maxNrofSlotFormatCombinationsPerSet-1   INTEGER ::= 511     -- Maximum number of Slot Format Combinations in a SF-Set minus 1.

maxNrofTrafficPattern-r16               INTEGER ::= 8       -- Maximum number of Traffic Pattern for NR sidelink communication.

maxNrofPUCCH-Resources                  INTEGER ::= 128

maxNrofPUCCH-Resources-1                INTEGER ::= 127

maxNrofPUCCH-ResourceSets               INTEGER ::= 4       -- Maximum number of PUCCH Resource Sets

maxNrofPUCCH-ResourceSets-1             INTEGER ::= 3       -- Maximum number of PUCCH Resource Sets minus 1.

maxNrofPUCCH-ResourcesPerSet            INTEGER ::= 32      -- Maximum number of PUCCH Resources per PUCCH-ResourceSet

maxNrofPUCCH-P0-PerSet                  INTEGER ::= 8       -- Maximum number of P0-pucch present in a p0-pucch set

maxNrofPUCCH-PathlossReferenceRSs       INTEGER ::= 4       -- Maximum number of RSs used as pathloss reference for PUCCH power control.

maxNrofPUCCH-PathlossReferenceRSs-1     INTEGER ::= 3       -- Maximum number of RSs used as pathloss reference for PUCCH power control

-- minus 1.

maxNrofPUCCH-PathlossReferenceRSs-r16   INTEGER ::= 64      -- Maximum number of RSs used as pathloss reference for PUCCH power control

-- extended.

maxNrofPUCCH-PathlossReferenceRSs-1-r16 INTEGER ::= 63      -- Maximum number of RSs used as pathloss reference for PUCCH power control

-- minus 1 extended.

maxNrofPUCCH-PathlossReferenceRSs-1-r17 INTEGER ::= 7       -- Maximum number of RSs used as pathloss reference for PUCCH power control

-- minus 1.

maxNrofPUCCH-PathlossReferenceRSsDiff-r16 INTEGER ::= 60    -- Difference between the extended maximum and the non-extended maximum

maxNrofPUCCH-ResourceGroups-r16         INTEGER ::= 4       -- Maximum number of PUCCH resources groups.

maxNrofPUCCH-ResourcesPerGroup-r16      INTEGER ::= 128     -- Maximum number of PUCCH resources in a PUCCH group.

maxNrofPowerControlSetInfos-r17         INTEGER ::= 8       -- Maximum number of PUCCH power control set infos

maxNrofMultiplePUSCHs-r16               INTEGER ::= 8       -- Maximum number of multiple PUSCHs in PUSCH TDRA list

maxNrofP0-PUSCH-AlphaSets               INTEGER ::= 30      -- Maximum number of P0-pusch-alpha-sets (see TS 38.213 [13], clause 7.1)

maxNrofP0-PUSCH-AlphaSets-1             INTEGER ::= 29      -- Maximum number of P0-pusch-alpha-sets minus 1 (see TS 38.213 [13], clause 7.1)

maxNrofPUSCH-PathlossReferenceRSs       INTEGER ::= 4       -- Maximum number of RSs used as pathloss reference for PUSCH power control.

maxNrofPUSCH-PathlossReferenceRSs-1     INTEGER ::= 3       -- Maximum number of RSs used as pathloss reference for PUSCH power control

-- minus 1.

maxNrofPUSCH-PathlossReferenceRSs-r16   INTEGER ::= 64      -- Maximum number of RSs used as pathloss reference for PUSCH power control

-- extended

maxNrofPUSCH-PathlossReferenceRSs-1-r16 INTEGER ::= 63      -- Maximum number of RSs used as pathloss reference for PUSCH power control

-- extended minus 1

maxNrofPUSCH-PathlossReferenceRSsDiff-r16  INTEGER ::= 60   -- Difference between maxNrofPUSCH-PathlossReferenceRSs-r16 and

-- maxNrofPUSCH-PathlossReferenceRSs

maxNrofPathlossReferenceRSs-r17         INTEGER ::= 64      -- Maximum number of RSs used as pathloss reference for PUSCH, PUCCH, SRS

-- power control for unified TCI state operation

maxNrofPathlossReferenceRSs-1-r17       INTEGER ::= 63      -- Maximum number of RSs used as pathloss reference for PUSCH, PUCCH, SRS

-- power control for unified TCI state operation minus 1

maxNrofNAICS-Entries                    INTEGER ::= 8       -- Maximum number of supported NAICS capability set

maxBands                                INTEGER ::= 1024    -- Maximum number of supported bands in UE capability.

maxBandsMRDC                            INTEGER ::= 1280

maxBandsEUTRA                           INTEGER ::= 256

maxCellReport                           INTEGER ::= 8

maxDRB                                  INTEGER ::= 29      -- Maximum number of DRBs (that can be added in DRB-ToAddModList).

maxFreq                                 INTEGER ::= 8       -- Max number of frequencies.

maxFreqLayers                           INTEGER ::= 4       -- Max number of frequency layers.

maxFreqPlus1                            INTEGER ::= 9       -- Max number of frequencies for Slicing.

maxFreqIDC-r16                          INTEGER ::= 128     -- Max number of frequencies for IDC indication.

maxCombIDC-r16                          INTEGER ::= 128     -- Max number of reported UL CA for IDC indication.

maxFreqIDC-MRDC                         INTEGER ::= 32      -- Maximum number of candidate NR frequencies for MR-DC IDC indication

maxNrofCandidateBeams                   INTEGER ::= 16      -- Max number of PRACH-ResourceDedicatedBFR in BFR config.

maxNrofCandidateBeams-r16               INTEGER ::= 64      -- Max number of candidate beam resources in BFR config.

maxNrofCandidateBeamsExt-r16            INTEGER ::= 48      -- Max number of PRACH-ResourceDedicatedBFR in the CandidateBeamRSListExt

maxNrofPCIsPerSMTC                      INTEGER ::= 64      -- Maximum number of PCIs per SMTC.

maxNrofQFIs                             INTEGER ::= 64

maxNrofResourceAvailabilityPerCombination-r16 INTEGER ::= 256

maxNrOfSemiPersistentPUSCH-Triggers     INTEGER ::= 64      -- Maximum number of triggers for semi persistent reporting on PUSCH

maxNrofSR-Resources                     INTEGER ::= 8       -- Maximum number of SR resources per BWP in a cell.

maxNrofSlotFormatsPerCombination        INTEGER ::= 256

maxNrofSpatialRelationInfos             INTEGER ::= 8

maxNrofSpatialRelationInfos-plus-1      INTEGER ::= 9

maxNrofSpatialRelationInfos-r16         INTEGER ::= 64

maxNrofSpatialRelationInfosDiff-r16     INTEGER ::= 56      -- Difference between maxNrofSpatialRelationInfos-r16 and maxNrofSpatialRelationInfos

maxNrofIndexesToReport                  INTEGER ::= 32

maxNrofIndexesToReport2                 INTEGER ::= 64

maxNrofSSBs-r16                         INTEGER ::= 64      -- Maximum number of SSB resources in a resource set.

maxNrofSSBs-1                           INTEGER ::= 63      -- Maximum number of SSB resources in a resource set minus 1.

maxNrofS-NSSAI                          INTEGER ::= 8       -- Maximum number of S-NSSAI.

maxNrofTCI-StatesPDCCH                  INTEGER ::= 64

maxNrofTCI-States                       INTEGER ::= 128     -- Maximum number of TCI states.

maxNrofTCI-States-1                     INTEGER ::= 127     -- Maximum number of TCI states minus 1.

maxUL-TCI-r17                           INTEGER ::= 64      -- Maximum number of TCI states.

maxUL-TCI-1-r17                         INTEGER ::= 63      -- Maximum number of TCI states minus 1.

maxNrofAdditionalPCI-r17                INTEGER ::= 7       -- Maximum number of additional PCI

maxMPE-Resources-r17                    INTEGER ::= 64      -- Maximum number of pooled MPE resources

maxNrofUL-Allocations                   INTEGER ::= 16      -- Maximum number of PUSCH time domain resource allocations.

maxQFI                                  INTEGER ::= 63

maxRA-CSIRS-Resources                   INTEGER ::= 96

maxRA-OccasionsPerCSIRS                 INTEGER ::= 64      -- Maximum number of RA occasions for one CSI-RS

maxRA-Occasions-1                       INTEGER ::= 511     -- Maximum number of RA occasions in the system

maxRA-SSB-Resources                     INTEGER ::= 64

maxSCSs                                 INTEGER ::= 5

maxSecondaryCellGroups                  INTEGER ::= 3

maxNrofServingCellsEUTRA                INTEGER ::= 32

maxMBSFN-Allocations                    INTEGER ::= 8

maxNrofMultiBands                       INTEGER ::= 8

maxCellSFTD                             INTEGER ::= 3       -- Maximum number of cells for SFTD reporting

maxReportConfigId                       INTEGER ::= 64

maxNrofCodebooks                        INTEGER ::= 16      -- Maximum number of codebooks supported by the UE

maxNrofCSI-RS-ResourcesExt-r16          INTEGER ::= 16      -- Maximum number of codebook resources supported by the UE for eType2/Codebook combo

maxNrofCSI-RS-ResourcesExt-r17          INTEGER ::= 8       -- Maximum number of codebook resources for fetype2R1 and fetype2R2

maxNrofCSI-RS-Resources                 INTEGER ::= 7       -- Maximum number of codebook resources supported by the UE

maxNrofCSI-RS-ResourcesAlt-r16          INTEGER ::= 512     -- Maximum number of alternative codebook resources supported by the UE

maxNrofCSI-RS-ResourcesAlt-1-r16        INTEGER ::= 511     -- Maximum number of alternative codebook resources supported by the UE minus 1

maxNrofSRI-PUSCH-Mappings               INTEGER ::= 16

maxNrofSRI-PUSCH-Mappings-1             INTEGER ::= 15

maxSIB                                  INTEGER::= 32       -- Maximum number of SIBs

maxSI-Message                           INTEGER::= 32       -- Maximum number of SI messages

maxSIB-MessagePlus1-r17                 INTEGER::= 33       -- Maximum number of SIB messages plus 1

maxPO-perPF                             INTEGER ::= 4       -- Maximum number of paging occasion per paging frame

maxPEI-perPF-r17                        INTEGER ::= 4       -- Maximum number of PEI occasion per paging frame

maxAccessCat-1                          INTEGER ::= 63      -- Maximum number of Access Categories minus 1

maxBarringInfoSet                       INTEGER ::= 8       -- Maximum number of access control parameter sets

maxCellEUTRA                            INTEGER ::= 8       -- Maximum number of E-UTRA cells in SIB list

maxEUTRA-Carrier                        INTEGER ::= 8       -- Maximum number of E-UTRA carriers in SIB list

maxPLMNIdentities                       INTEGER ::= 8       -- Maximum number of PLMN identities in RAN area configurations

maxDownlinkFeatureSets                  INTEGER ::= 1024    -- (for NR DL) Total number of FeatureSets (size of the pool)

maxUplinkFeatureSets                    INTEGER ::= 1024    -- (for NR UL) Total number of FeatureSets (size of the pool)

maxEUTRA-DL-FeatureSets                 INTEGER ::= 256     -- (for E-UTRA) Total number of FeatureSets (size of the pool)

maxEUTRA-UL-FeatureSets                 INTEGER ::= 256     -- (for E-UTRA) Total number of FeatureSets (size of the pool)

maxFeatureSetsPerBand                   INTEGER ::= 128     -- (for NR) The number of feature sets associated with one band.

maxPerCC-FeatureSets                    INTEGER ::= 1024    -- (for NR) Total number of CC-specific FeatureSets (size of the pool)

maxFeatureSetCombinations               INTEGER ::= 1024    -- (for MR-DC/NR)Total number of Feature set combinations (size of the pool)

maxInterRAT-RSTD-Freq                   INTEGER ::= 3

maxGIN-r17                              INTEGER ::= 24      -- Maximum number of broadcast GINs

maxHRNN-Len-r16                         INTEGER ::= 48      -- Maximum length of HRNNs

maxNPN-r16                              INTEGER ::= 12      -- Maximum number of NPNs broadcast and reported by UE at establishment

maxNrOfMinSchedulingOffsetValues-r16    INTEGER ::= 2       -- Maximum number of min. scheduling offset (K0/K2) configurations

maxK0-SchedulingOffset-r16              INTEGER ::= 16      -- Maximum number of slots configured as min. scheduling offset (K0)

maxK2-SchedulingOffset-r16              INTEGER ::= 16      -- Maximum number of slots configured as min. scheduling offset (K2)

maxK0-SchedulingOffset-r17              INTEGER ::= 64      -- Maximum number of slots configured as min. scheduling offset (K0)

maxK2-SchedulingOffset-r17              INTEGER ::= 64      -- Maximum number of slots configured as min. scheduling offset (K2)

maxDCI-2-6-Size-r16                     INTEGER ::= 140     -- Maximum size of DCI format 2-6

maxDCI-2-7-Size-r17                     INTEGER ::= 43      -- Maximum size of DCI format 2-7

maxDCI-2-6-Size-1-r16                   INTEGER ::= 139     -- Maximum DCI format 2-6 size minus 1

maxNrofUL-Allocations-r16               INTEGER ::= 64      -- Maximum number of PUSCH time domain resource allocations

maxNrofP0-PUSCH-Set-r16                 INTEGER ::= 2       -- Maximum number of P0 PUSCH set(s)

maxOnDemandSIB-r16                      INTEGER ::= 8       -- Maximum number of SIB(s) that can be requested on-demand

maxOnDemandPosSIB-r16                   INTEGER ::= 32      -- Maximum number of posSIB(s) that can be requested on-demand

maxCI-DCI-PayloadSize-r16               INTEGER ::= 126     -- Maximum number of the DCI size for CI

maxCI-DCI-PayloadSize-1-r16             INTEGER ::= 125     -- Maximum number of the DCI size for CI minus 1

maxUu-RelayRLC-ChannelID-r17            INTEGER ::= 32      -- Maximum value of Uu Relay RLC channel ID

maxWLAN-Id-Report-r16                   INTEGER ::= 32      -- Maximum number of WLAN IDs to report

maxWLAN-Name-r16                        INTEGER ::= 4       -- Maximum number of WLAN name

maxRAReport-r16                         INTEGER ::= 8       -- Maximum number of RA procedures information to be included in the RA report

maxTxConfig-r16                         INTEGER ::= 64      -- Maximum number of sidelink transmission parameters configurations

maxTxConfig-1-r16                       INTEGER ::= 63      -- Maximum number of sidelink transmission parameters configurations minus 1

maxPSSCH-TxConfig-r16                   INTEGER ::= 16      -- Maximum number of PSSCH TX configurations

maxNrofCLI-RSSI-Resources-r16           INTEGER ::= 64      -- Maximum number of CLI-RSSI resources for UE

maxNrofCLI-RSSI-Resources-1-r16         INTEGER ::= 63      -- Maximum number of CLI-RSSI resources for UE minus 1

maxNrofCLI-SRS-Resources-r16            INTEGER ::= 32      -- Maximum number of SRS resources for CLI measurement for UE

maxCLI-Report-r16                       INTEGER ::= 8

maxNrofCC-Group-r17                     INTEGER ::= 16      -- Maximum number of CC groups for DC location report

maxNrofConfiguredGrantConfig-r16        INTEGER ::= 12      -- Maximum number of configured grant configurations per BWP

maxNrofConfiguredGrantConfig-1-r16      INTEGER ::= 11      -- Maximum number of configured grant configurations per BWP minus 1

maxNrofCG-Type2DeactivationState        INTEGER ::= 16      -- Maximum number of deactivation state for type 2 configured grants per BWP

maxNrofConfiguredGrantConfigMAC-1-r16   INTEGER ::= 31      -- Maximum number of configured grant configurations per MAC entity minus 1

maxNrofSPS-Config-r16                   INTEGER ::= 8       -- Maximum number of SPS configurations per BWP

maxNrofSPS-Config-1-r16                 INTEGER ::= 7       -- Maximum number of SPS configurations per BWP minus 1

maxNrofSPS-DeactivationState            INTEGER ::= 16      -- Maximum number of deactivation state for SPS per BWP

maxNrofPPW-Config-r17                   INTEGER ::= 4       -- Maximum number of Preconfigured PRS processing windows per DL BWP

maxNrofPPW-ID-1-r17                     INTEGER ::= 15      -- Maximum number of Preconfigured PRS processing windows minus 1

maxNrOfTxTEGReport-r17                  INTEGER ::= 256     -- Maximum number of UE Tx Timing Error Group Report

maxNrOfTxTEG-ID-1-r17                   INTEGER ::= 7       -- Maximum number of UE Tx Timing Error Group ID minus 1

maxNrofPagingSubgroups-r17              INTEGER ::= 8       -- Maximum number of paging subgroups per paging occasion

maxNrofPUCCH-ResourceGroups-1-r16       INTEGER ::= 3

maxNrofReqComDC-Location-r17            INTEGER ::= 128     -- Maximum number of requested carriers/BWPs combinations for DC location

-- report

maxNrofServingCellsTCI-r16              INTEGER ::= 32      -- Maximum number of serving cells in simultaneousTCI-UpdateList

maxNrofTxDC-TwoCarrier-r16              INTEGER ::= 64      -- Maximum number of UL Tx DC locations reported by the UE for 2CC uplink CA

maxNrofRB-SetGroups-r17                 INTEGER ::= 8       -- Maximum number of RB set groups

maxNrofRB-Sets-r17                      INTEGER ::= 8       -- Maximum number of RB sets

maxNrofEnhType3HARQ-ACK-r17             INTEGER ::= 8       -- Maximum number of enhanced type 3 HARQ-ACK codebook

maxNrofEnhType3HARQ-ACK-1-r17           INTEGER ::= 7       -- Maximum number of enhanced type 3 HARQ-ACK codebook minus 1

maxNrofPRS-ResourcesPerSet-r17          INTEGER ::= 64      -- Maximum number of PRS resources for one set

maxNrofPRS-ResourcesPerSet-1-r17        INTEGER ::= 63      -- Maximum number of PRS resources for one set minus 1

maxNrofPRS-ResourceOffsetValue-1-r17    INTEGER ::= 511

maxNrofGapId-r17                        INTEGER ::= 8       -- Maximum number of measurement gap ID is FFS

maxNrofPreConfigPosGapId-r17            INTEGER ::= 16      -- Maximum number of preconfigured positioning measurement gap

maxNrOfGapPri-r17                       INTEGER ::= 16      -- Maximum number of gap priority level

maxCEFReport-r17                        INTEGER ::= 4       -- Maximum number of CEF reports by the UE

maxNrofMultiplePDSCHs-r17               INTEGER ::= 8       -- Maximum number of PDSCHs in PDSCH TDRA list

maxSliceInfo-r17                        INTEGER ::= 8       -- Maximum number of NSAGs

maxCellSlice-r17                        INTEGER ::= 16      -- Maximum number of cells supporting the NSAG

maxNrofTRS-ResourceSets-r17             INTEGER ::= 64      -- Maximum number of TRS resource sets

maxNrofSearchSpaceGroups-1-r17          INTEGER ::= 2       -- Maximum number of search space groups minus 1

maxNrofRemoteUE-r17                     INTEGER ::= 32      -- Maximum number of connected L2 U2N Remote UEs

maxDCI-4-2-Size-r17                     INTEGER ::= 140     -- Maximum size of DCI format 4-2

maxFreqMBS-r17                          INTEGER ::= 16      -- Maximum number of MBS frequencies reported in MBSInterestIndication

maxNrofDRX-ConfigPTM-r17                INTEGER ::= 64      -- Max number of DRX configuration for PTM provided in MBS broadcast in a

-- cell

maxNrofDRX-ConfigPTM-1-r17              INTEGER ::= 63      -- Max number of DRX configuration for PTM provided in MBS broadcast in a

-- cell minus 1

maxNrofMBS-ServiceListPerUE-r17         INTEGER ::= 16      -- Maximum number of services which the UE can include in the  MBS interest

-- indication

maxNrofMBS-Session-r17                  INTEGER ::= 1024    -- Maximum number of MBS sessions provided in MBS broadcast in a cell

maxNrofMTCH-SSB-MappingWindow-r17       INTEGER ::= 16      -- Maximum number of MTCH to SSB beam mapping pattern

maxNrofMTCH-SSB-MappingWindow-1-r17     INTEGER ::= 15      -- Maximum number of MTCH to SSB beam mapping pattern minus 1

maxNrofMRB-Broadcast-r17                INTEGER ::= 4       -- Maximum number of broadcast MRBs configured for one MBS broadcast service

maxNrofPageGroup-r17                    INTEGER ::= 32      -- Maximum number of paging groups in a paging message

maxNrofPDSCH-ConfigPTM-r17              INTEGER ::= 16      -- Maximum number of PDSCH configuration groups for PTM

maxNrofPDSCH-ConfigPTM-1-r17            INTEGER ::= 15      -- Maximum number of PDSCH configuration groups for PTM minus 1

maxG-RNTI-r17                           INTEGER ::= 16      -- Maximum number of G-RNTI that can be configured for a UE.

maxG-RNTI-1-r17                         INTEGER ::= 15      -- Maximum number of G-RNTI that can be configured for a UE minus 1.

maxG-CS-RNTI-r17                        INTEGER ::= 8       -- Maximum number of G-CS-RNTI that can be configured for a UE.

maxG-CS-RNTI-1-r17                      INTEGER ::= 7       -- Maximum number of G-CS-RNTI that can be configured for a UE minus 1.

maxMRB-r17                              INTEGER ::= 32      -- Maximum number of multicast MRBs (that can be added in MRB-ToAddModLIst)

maxFSAI-MBS-r17                         INTEGER ::= 64      -- Maximum number of MBS frequency selection area identities

maxNeighCellMBS-r17                     INTEGER ::= 8       -- Maximum number of MBS broadcast neighbour cells

maxNrofPdcch-BlindDetectionMixed-1-r16  INTEGER ::= 7       -- Maximum number of combinations of mixed Rel-16 and Rel-15 PDCCH

-- monitoring capabilities minus 1

maxNrofPdcch-BlindDetection-r17         INTEGER ::= 16      -- Maximum number of combinations of PDCCH blind detection monitoring

-- capabilities

-- TAG-MULTIPLICITY-AND-TYPE-CONSTRAINT-DEFINITIONS-STOP

-- ASN1STOP

Editor's note: maxK0-SchedulingOffset and maxK0-SchedulingOffset need confirmation by RAN1.

### –	End of NR-RRC-Definitions

-- ASN1START

END

-- ASN1STOP

## 6.5	Short Message

Short Messages can be transmitted on PDCCH using P-RNTI with or without associated Paging message using Short Message field in DCI format 1_0 (see TS 38.212 [17], clause 7.3.1.2.1).

Table 6.5-1 defines Short Messages. Bit 1 is the most significant bit.

Table 6.5-1: Short Messages

| Bit | Short Message |
| --- | --- |
| 1 | systemInfoModification If set to 1: indication of a BCCH modification other than SIB6, SIB7 and SIB8. |
| 2 | etwsAndCmasIndication If set to 1: indication of an ETWS primary notification and/or an ETWS secondary notification and/or a CMAS notification. |
| 3 | stopPagingMonitoring This bit can be used for only operation with shared spectrum channel access and if nrofPDCCH-MonitoringOccasionPerSSB-InPO is present. If set to 1: indication that the UE may stop monitoring PDCCH occasion(s) for paging in this Paging Occasion as specified in TS 38.304 [20], clause 7.1. |
| 4 | systemInfoModification-eDRX If set to 1: indication of a BCCH modification other than SIB6, SIB7 and SIB8. This indication applies only to UEs using IDLE eDRX cycle longer than the BCCH modification period. |
| 5 – 8 | Not used in this release of the specification, and shall be ignored by UE if received. |

---------------------------------------------------------Skip Unchanged----------------------------------------------------------

# 7	Variables and constants

## 7.1	Timers

### 7.1.1	Timers (Informative)

| Timer | Start | Stop | At expiry |
| --- | --- | --- | --- |
| T300 | Upon transmission of RRCSetupRequest. | Upon reception of RRCSetup or RRCReject message, cell re-selection, relay reselection, and upon abortion of connection establishment by upper layers. | Perform the actions as specified in 5.3.3.7. |
| T301 | Upon transmission of RRCReestabilshmentRequest | Upon reception of RRCReestablishment or RRCSetup message as well as when the selected cell becomes unsuitable or the (re)selected L2 U2N Relay UE becomes unsuitable, upon reception of notificationMessageSidelink indicating relayUE-HO or relayUE-CellReselection. | Go to RRC_IDLE |
| T302 | Upon reception of RRCReject while performing RRC connection establishment or resume, upon reception of RRCRelease with waitTime. | Upon entering RRC_CONNECTED or RRC_IDLE, upon cell re-selection, upon cell change due to relay (re)selection, and upon reception of RRCReject message. | Inform upper layers about barring alleviation as specified in 5.3.14.4 |
| T304 | Upon reception of RRCReconfiguration message including reconfigurationWithSync for the MCG which does not include sl-PathSwitchConfig, or upon reception of RRCReconfiguration message including reconfigurationWithSync for the SCG not indicated as deactivated in the NR or E-UTRA message containing the RRCReconfiguration message or upon conditional reconfiguration execution i.e. when applying a stored RRCReconfiguration message including reconfigurationWithSync. | Upon successful completion of random access on the corresponding SpCell For T304 of SCG, upon SCG release | For T304 of MCG, in case of the handover from NR or intra-NR handover, or path switch from a L2 U2N Relay UE to a NR cell, initiate the RRC re-establishment procedure; In case of handover to NR, perform the actions defined in the specifications applicable for the source RAT. If any DAPS bearer is configured and if there is no RLF in source PCell, initiate the failure information procedure. For T304 of SCG, inform network about the reconfiguration with sync failure by initiating the SCG failure information procedure as specified in 5.7.3. |
| T310 | Upon detecting physical layer problems for the SpCell i.e. upon receiving N310 consecutive out-of-sync indications from lower layers. | Upon receiving N311 consecutive in-sync indications from lower layers for the SpCell, upon receiving RRCReconfiguration with reconfigurationWithSync for that cell group, upon reception of MobilityFromNRCommand, upon the reconfiguration of rlf-TimersAndConstant, upon initiating the connection re-establishment procedure, upon conditional reconfiguration execution i.e. when applying a stored RRCReconfiguration message including reconfigurationWithSync for that cell group, and upon initiating the MCG failure information procedure. Upon SCG release, if the T310 is kept in SCG. | If the T310 is kept in MCG: If AS security is not activated: go to RRC_IDLE else: initiate the MCG failure information procedure as specified in 5.7.3b or the connection re-establishment procedure as specified in 5.3.7 or the procedure as specified in 5.3.10.3 if any DAPS bearer is configured. If the T310 is kept in SCG, Inform E-UTRAN/NR about the SCG radio link failure by initiating the SCG failure information procedure as specified in 5.7.3. |
| T311 | Upon initiating the RRC connection re-establishment procedure | Upon selection of a suitable NR cell, or upon selection of a suitable L2 U2N Relay UE, or a cell using another RAT. | Enter RRC_IDLE |
| T312 | If T312 is configured in MCG: Upon triggering a measurement report for a measurement identity for which T312 has been configured and useT312 has been set to true, while T310 in PCell is running. If T312 is configured in SCG and useT312 has been set to true: Upon triggering a measurement report for a measurement identity for which T312 has been configured, while T310 in PSCell is running. | Upon receiving N311 consecutive in-sync indications from lower layers for the SpCell, receiving RRCReconfiguration with reconfigurationWithSync for that cell group, upon reception of MobilityFromNRCommand, upon initiating the connection re-establishment procedure, upon the reconfiguration of rlf-TimersAndConstant, upon initiating the MCG failure information procedure, upon conditional reconfiguration execution i.e. when applying a stored RRCReconfiguration message including reconfigurationWithSync for that cell group, and upon the expiry of T310 in corresponding SpCell. Upon SCG release, if the T312 is kept in SCG | If the T312 is kept in MCG initiate the MCG failure information procedure as specified in 5.7.3b or the connection re-establishment procedure. If the T312 is kept in SCG, Inform E-UTRAN/NR about the SCG radio link failure by initiating the SCG failure information procedure.as specified in 5.7.3. |
| T316 | Upon transmission of the MCGFailureInformation message | Upon receiving RRCRelease, RRCReconfiguration with reconfigurationwithSync for the PCell, MobilityFromNRCommand, or upon initiating the re-establishment procedure | Perform the actions as specified in 5.7.3b.5. |
| T319 | Upon transmission of RRCResumeRequest or RRCResumeRequest1 when the resume procedure is not initiated for SDT. | Upon reception of RRCResume, RRCSetup, RRCRelease, RRCRelease with suspendConfig or RRCReject message, upon cell re-selection or upon relay (re)selection. | Perform the actions as specified in 5.3.13.5. |
| T319a | Upon transmission of RRCResumeRequest or RRCResumeRequest1 when the resume procedure is initiated for SDT. | Upon reception of RRCResume, RRCSetup, RRCRelease, RRCReject message or upon failure to resume RRC connection for SDT as specified in 5.3.13.5 or upon cell reselection. | Perform the actions as specified in 5.3.13.5. |
| T320 | Upon reception of t320 or upon cell (re)selection to NR from another RAT with validity time configured for dedicated priorities (in which case the remaining validity time is applied). | Upon entering RRC_CONNECTED, upon reception of RRCRelease, when PLMN selection or SNPN selection is performed on request by NAS, when the UE enters RRC_IDLE from RRC_INACTIVE, or upon cell (re)selection to another RAT (in which case the timer is carried on to the other RAT). | Discard the cell reselection priority information provided by dedicated signalling. |
| T321 | Upon receiving measConfig including a reportConfig with the reportType set to reportCGI | Upon acquiring the information needed to set all fields of cgi-info, upon receiving measConfig that includes removal of the reportConfig with the reportType set to reportCGI and upon detecting that a cell is not broadcasting SIB1. | Initiate the measurement reporting procedure, stop performing the related measurements. |
| T322 | Upon receiving measConfig including reportConfigNR with the reportType set to reportSFTD and drx-SFTD-NeighMeas is set to true. | Upon acquiring the SFTD measurement results, upon receiving measConfig that includes removal of the reportConfig with the reportType set to reportSFTD. | Initiate the measurement reporting procedure, stop performing the related measurements. |
| T325 | Upon reception of RRCRelease message with deprioritisationTimer. |  | Stop deprioritisation of all frequencies or NR signalled by RRCRelease. |
| T330 | Upon receiving LoggedMeasurementConfiguration message | Upon log volume exceeding the suitable UE memory, upon initiating the release of LoggedMeasurementConfiguration procedure | Perform the actions specified in 5.5a.1.4 |
| T331 | Upon receiving RRCRelease message with measIdleDuration | Upon receiving RRCSetup, RRCResume, RRCRelease with idle/inactive measurement configuration, upon cell selection/reselection to a cell that does not belong to the validityArea (if configured), or upon cell re-selection to another RAT. | Perform the actions as specified in 5.7.8.3. |
| T342 | Upon transmitting UEAssistanceInformation message with DelayBudgetReport. | Upon releasing delayBudgetReportingConfig during the connection re-establishment/resume procedures, and upon receiving delayBudgetReportingConfig set to release. | No action. |
| T345 | Upon transmitting UEAssistanceInformation message with overheatingAssistance | Upon releasing overheatingAssistanceConfig during the connection re-establishment procedure, upon initiating the connection resumption procedure, and upon receiving overheatingAssistanceConfig set to release. | No action. |
| T346a (The UE maintains one instance of this timer per cell group) | Upon transmitting UEAssistanceInformation message with drx-Preference. | Upon releasing drx-PreferenceConfig during the connection re-establishment/resume procedures, upon receiving drx-PreferenceConfig set to release, or upon performing MR-DC release. | No action. |
| T346b (The UE maintains one instance of this timer per cell group) | Upon transmitting UEAssistanceInformation message with maxBW-Preference. | Upon releasing maxBW-PreferenceConfig during the connection re-establishment/resume procedures, upon receiving maxBW-PreferenceConfig set to release, or upon performing MR-DC release. | No action. |
| T346c (The UE maintains one instance of this timer per cell group) | Upon transmitting UEAssistanceInformation message with maxCC-Preference. | Upon releasing maxCC-PreferenceConfig during the connection re-establishment/resume procedures, upon receiving maxCC-PreferenceConfig set to release, or upon performing MR-DC release. | No action. |
| T346d (The UE maintains one instance of this timer per cell group) | Upon transmitting UEAssistanceInformation message with maxMIMO-LayerPreference. | Upon releasing maxMIMO-LayerPreferenceConfig during the connection re-establishment/resume procedures, upon receiving maxMIMO-LayerPreferenceConfig set to release, or upon performing MR-DC release. | No action. |
| T346e (The UE maintains one instance of this timer per cell group) | Upon transmitting UEAssistanceInformation message with minSchedulingOffsetPreference. | Upon releasing minSchedulingOffsetPreferenceConfig during the connection re-establishment/resume procedures, upon receiving minSchedulingOffsetPreferenceConfig set to release, or upon performing MR-DC release. | No action. |
| T346f | Upon transmitting UEAssistanceInformation message with releasePreference. | Upon releasing releasePreferenceConfig during the connection re-establishment/resume procedures, or upon receiving releasePreferenceConfig set to release. | No action. |
| T346g | Upon transmitting UEAssistanceInformation message with musim-PreferredRRC-State. | Upon receiving RRCRelease, or upon receiving musim-LeaveAssistanceConfig set to release. | Perform the actions as specified in 5.3.8.6. |
| T346h | Upon transmitting UEAssistanceInformation message with musim-GapPreferenceList or musim-GapPriorityPreferenceList Information. | Upon releasing musim-GapAssistanceConfig during the connection re-establishment/resume procedures, or upon receiving musim-GapAssistanceConfig set to release. | No action. |
| T346i | Upon transmitting UEAssistanceInformation message with scg-DeactivationPreference | Upon releasing scg-DeactivationPreferenceConfig during RRC connection re-establishment/resume or upon receiving scg-DeactivationPreferenceConfig set to release. | No action. |
| T346j (The UE maintains one instance of this timer per cell group) | Upon transmitting UEAssistanceInformation message with rlm-RelaxationReportingConfig. | Upon releasing rlm-RelaxationReportingConfig during the connection re-establishment/resume procedures, upon receiving rlm-RelaxationReportingConfig set to release, or upon performing MR-DC release. | No action. |
| T346k (The UE maintains one instance of this timer per cell group) | Upon transmitting UEAssistanceInformation message with bfd-RelaxationReportingConfig. | Upon releasing bfd-RelaxationReportingConfig during the connection re-establishment/resume procedures, upon receiving bfd-RelaxationReportingConfig set to release, or upon performing MR-DC release. | No action. |
| T3xx | Upon transmission of MUSIM temporary restriction of musim-CapRestriction for affected capabilities for serving cell(s), release of SCell or release of SCG. | Upon reception of RRCReconfiguration message that does not exceed UE temporary capability restriction transmitted via musim-CapRestriction. | Can apply the temporary UE capability restriction that the UE indicated |
| T3xy | Upon transmission of MUSIM temporary restriction of musim-CapRestriction for impacted band combination(s) | Upon releasing musim-CapabilityRestrictionConfig during the connection re-establishment/resume procedures, or upon receiving musim-CapabilityRestrictionConfig set to release. | No action. |
| T350 | Upon transmitting DedicatedSIBRequest message with requestedSIB-List and/or requestedPosSIB-List. | Upon acquiring the requested SIB(s) or posSIB(s), upon releasing onDemandSIB-Request during the connection re-establishment procedures, upon receiving onDemandSIB-Request set to release, upon reception of RRCRelease or upon successful change of PCell while in RRC_CONNECTED. | No action |
| T380 | Upon reception of t380 in RRCRelease. | Upon reception of RRCResume, RRCSetup or RRCRelease. | Perform the actions as specified in 5.3.13. |
| T390 | When access attempt is barred at access barring check for an Access Category. The UE maintains one instance of this timer per Access Category. | Upon cell (re)selection, upon relay (re)selection, upon entering RRC_CONNECTED, upon reception of RRCReconfiguration including reconfigurationWithSync, upon change of PCell while in RRC_CONNECTED, upon reception of MobilityFromNRCommand, or upon reception of RRCRelease. | Perform the actions as specified in 5.3.14.4. |
| T400 | Upon transmission of RRCReconfigurationSidelink | Upon reception of RRCReconfigurationFailureSidelink or RRCReconfigurationCompleteSidelink | Perform the Sidelink radio link failure related actions as specified in 5.8.9.3. |
| T420 | Upon reception of the RRCReconfiguration message including sl-PathSwitchConfig | Upon successfully sending RRCReconfigurationComplete message (i.e., PC5 RLC acknowledgement is received from target L2 U2N Relay UE) | Perform the RRC re-establishment procedure as specified in 5.3.7. |
| T430 | Start or restart from the subframe indicated by epochTime upon reception of SIB19, or upon reception of RRCReconfiguration message for the target cell including reconfigurationWithSync, or upon conditional reconfiguration execution i.e. when applying a stored RRCReconfiguration message for the target cell including reconfigurationWithSync. | Stop T430, if it is running, for the source cell upon reception of RRCReconfiguration message including reconfigurationWithSync, or upon conditional reconfiguration execution i.e. when applying a stored RRCReconfiguration message including reconfigurationWithSync. | Perform the actions as specified in 5.2.2.6. |

### 7.1.2	Timer handling

When the UE applies zero value for a timer, the timer shall be started and immediately expire unless explicitly stated otherwise.

## 7.2	Counters

| Counter | Reset | Incremented | When reaching max value |
| --- | --- | --- | --- |
| N310 | Upon reception of "in-sync" indication from lower layers; upon receiving RRCReconfiguration with reconfigurationWithSync for that cell group; upon initiating the connection re-establishment procedure. | Upon reception of "out-of-sync" from lower layer while the timer T310 is stopped. | Start timer T310 |
| N311 | Upon reception of "out-of-sync" indication from lower layers; upon receiving RRCReconfiguration with reconfigurationWithSync for that cell group; upon initiating the connection re-establishment procedure. | Upon reception of the "in-sync" from lower layer while the timer T310 is running. | Stop the timer T310. |

## 7.3	Constants

| Constant | Usage |
| --- | --- |
| N310 | Maximum number of consecutive "out-of-sync" indications for the SpCell received from lower layers |
| N311 | Maximum number of consecutive "in-sync" indications for the SpCell received from lower layers |

---------------------------------------------------------Skip Unchanged----------------------------------------------------------

### 11.2.2	Message definitions

---------------------------------------------------------Skip Unchanged----------------------------------------------------------

#### –	CG-Config

This message is used to transfer the SCG radio configuration as generated by the SgNB or SeNB. It can also be used by a CU to request a DU to perform certain actions, e.g. to request the DU to perform a new lower layer configuration.

Direction: Secondary gNB or eNB to master gNB or eNB, alternatively CU to DU.

CG-Config message

-- ASN1START

-- TAG-CG-CONFIG-START

CG-Config ::=                   SEQUENCE {

criticalExtensions                  CHOICE {

c1                                  CHOICE{

cg-Config                           CG-Config-IEs,

spare3 NULL, spare2 NULL, spare1 NULL

},

criticalExtensionsFuture            SEQUENCE {}

}

}

CG-Config-IEs ::=                   SEQUENCE {

scg-CellGroupConfig                 OCTET STRING (CONTAINING RRCReconfiguration)    OPTIONAL,

scg-RB-Config                       OCTET STRING (CONTAINING RadioBearerConfig)     OPTIONAL,

configRestrictModReq                ConfigRestrictModReqSCG                         OPTIONAL,

drx-InfoSCG                         DRX-Info                                        OPTIONAL,

candidateCellInfoListSN             OCTET STRING (CONTAINING MeasResultList2NR)     OPTIONAL,

measConfigSN                        MeasConfigSN                                    OPTIONAL,

selectedBandCombination             BandCombinationInfoSN                           OPTIONAL,

fr-InfoListSCG                      FR-InfoList                                     OPTIONAL,

candidateServingFreqListNR          CandidateServingFreqListNR                      OPTIONAL,

nonCriticalExtension                CG-Config-v1540-IEs                             OPTIONAL

}

CG-Config-v1540-IEs ::=             SEQUENCE {

pSCellFrequency                     ARFCN-ValueNR                                   OPTIONAL,

reportCGI-RequestNR                 SEQUENCE {

requestedCellInfo                   SEQUENCE {

ssbFrequency                        ARFCN-ValueNR,

cellForWhichToReportCGI             PhysCellId

}                                                                               OPTIONAL

}                                                                                   OPTIONAL,

ph-InfoSCG                          PH-TypeListSCG                                  OPTIONAL,

nonCriticalExtension                CG-Config-v1560-IEs                             OPTIONAL

}

CG-Config-v1560-IEs ::=             SEQUENCE {

pSCellFrequencyEUTRA                ARFCN-ValueEUTRA                                OPTIONAL,

scg-CellGroupConfigEUTRA            OCTET STRING                                    OPTIONAL,

candidateCellInfoListSN-EUTRA       OCTET STRING                                    OPTIONAL,

candidateServingFreqListEUTRA       CandidateServingFreqListEUTRA                   OPTIONAL,

needForGaps                         ENUMERATED {true}                               OPTIONAL,

drx-ConfigSCG                       DRX-Config                                      OPTIONAL,

reportCGI-RequestEUTRA              SEQUENCE {

requestedCellInfoEUTRA          SEQUENCE {

eutraFrequency                             ARFCN-ValueEUTRA,

cellForWhichToReportCGI-EUTRA              EUTRA-PhysCellId

}                                                                               OPTIONAL

}                                                                                   OPTIONAL,

nonCriticalExtension                CG-Config-v1590-IEs                             OPTIONAL

}

CG-Config-v1590-IEs ::=             SEQUENCE {

scellFrequenciesSN-NR               SEQUENCE (SIZE (1.. maxNrofServingCells-1)) OF  ARFCN-ValueNR          OPTIONAL,

scellFrequenciesSN-EUTRA            SEQUENCE (SIZE (1.. maxNrofServingCells-1)) OF  ARFCN-ValueEUTRA       OPTIONAL,

nonCriticalExtension                CG-Config-v1610-IEs                                                    OPTIONAL

}

CG-Config-v1610-IEs ::=             SEQUENCE {

drx-InfoSCG2                        DRX-Info2                                       OPTIONAL,

nonCriticalExtension                CG-Config-v1620-IEs                             OPTIONAL

}

CG-Config-v1620-IEs ::=             SEQUENCE {

ueAssistanceInformationSCG-r16      OCTET STRING (CONTAINING UEAssistanceInformation)  OPTIONAL,

nonCriticalExtension                CG-Config-v1630-IEs                                OPTIONAL

}

CG-Config-v1630-IEs ::=             SEQUENCE {

selectedToffset-r16                 T-Offset-r16                                       OPTIONAL,

nonCriticalExtension                CG-Config-v1640-IEs                                OPTIONAL

}

CG-Config-v1640-IEs ::=             SEQUENCE {

servCellInfoListSCG-NR-r16          ServCellInfoListSCG-NR-r16                      OPTIONAL,

servCellInfoListSCG-EUTRA-r16       ServCellInfoListSCG-EUTRA-r16                   OPTIONAL,

nonCriticalExtension                CG-Config-v1700-IEs                             OPTIONAL

}

CG-Config-v1700-IEs ::=             SEQUENCE {

candidateCellInfoListCPC-r17        CandidateCellInfoListCPC-r17                    OPTIONAL,

twoPHRModeSCG-r17                   ENUMERATED {enabled}                            OPTIONAL,

nonCriticalExtension                CG-Config-v1730-IEs                             OPTIONAL

}

CG-Config-v1730-IEs ::=             SEQUENCE {

fr1-Carriers-SCG-r17                INTEGER (1..32)                                 OPTIONAL,

fr2-Carriers-SCG-r17                INTEGER (1..32)                                 OPTIONAL,

nonCriticalExtension                SEQUENCE {}                                     OPTIONAL

}

ServCellInfoListSCG-NR-r16 ::=      SEQUENCE (SIZE (1.. maxNrofServingCells)) OF  ServCellInfoXCG-NR-r16

ServCellInfoXCG-NR-r16 ::=          SEQUENCE {

dl-FreqInfo-NR-r16                  FrequencyConfig-NR-r16                          OPTIONAL,

ul-FreqInfo-NR-r16                  FrequencyConfig-NR-r16                          OPTIONAL, -- Cond FDD

...

}

FrequencyConfig-NR-r16 ::=          SEQUENCE {

freqBandIndicatorNR-r16             FreqBandIndicatorNR,

carrierCenterFreq-NR-r16            ARFCN-ValueNR,

carrierBandwidth-NR-r16             INTEGER (1..maxNrofPhysicalResourceBlocks),

subcarrierSpacing-NR-r16            SubcarrierSpacing

}

ServCellInfoListSCG-EUTRA-r16 ::=   SEQUENCE (SIZE (1.. maxNrofServingCellsEUTRA)) OF ServCellInfoXCG-EUTRA-r16

ServCellInfoXCG-EUTRA-r16 ::=       SEQUENCE {

dl-CarrierFreq-EUTRA-r16            ARFCN-ValueEUTRA                                OPTIONAL,

ul-CarrierFreq-EUTRA-r16            ARFCN-ValueEUTRA                                OPTIONAL, -- Cond FDD

transmissionBandwidth-EUTRA-r16     TransmissionBandwidth-EUTRA-r16                 OPTIONAL,

...

}

TransmissionBandwidth-EUTRA-r16 ::= ENUMERATED {rb6, rb15, rb25, rb50, rb75, rb100}

PH-TypeListSCG ::=                  SEQUENCE (SIZE (1..maxNrofServingCells)) OF PH-InfoSCG

PH-InfoSCG ::=                      SEQUENCE {

servCellIndex                       ServCellIndex,

ph-Uplink                           PH-UplinkCarrierSCG,

ph-SupplementaryUplink              PH-UplinkCarrierSCG                             OPTIONAL,

...,

[[

twoSRS-PUSCH-Repetition-r17         ENUMERATED{enabled}                             OPTIONAL

]]

}

PH-UplinkCarrierSCG ::=             SEQUENCE{

ph-Type1or3                         ENUMERATED {type1, type3},

...

}

MeasConfigSN ::=                    SEQUENCE {

measuredFrequenciesSN               SEQUENCE (SIZE (1..maxMeasFreqsSN)) OF NR-FreqInfo  OPTIONAL,

...

}

NR-FreqInfo ::=                     SEQUENCE {

measuredFrequency                   ARFCN-ValueNR                                       OPTIONAL,

...

}

ConfigRestrictModReqSCG ::=         SEQUENCE {

requestedBC-MRDC                    BandCombinationInfoSN                               OPTIONAL,

requestedP-MaxFR1                   P-Max                                               OPTIONAL,

...,

[[

requestedPDCCH-BlindDetectionSCG    INTEGER (1..15)                                     OPTIONAL,

requestedP-MaxEUTRA                 P-Max                                               OPTIONAL

]],

[[

requestedP-MaxFR2-r16               P-Max                                               OPTIONAL,

requestedMaxInterFreqMeasIdSCG-r16  INTEGER(1..maxMeasIdentitiesMN)                     OPTIONAL,

requestedMaxIntraFreqMeasIdSCG-r16  INTEGER(1..maxMeasIdentitiesMN)                     OPTIONAL,

requestedToffset-r16                T-Offset-r16                                        OPTIONAL

]]

}

BandCombinationIndex ::= INTEGER (1..maxBandComb)

BandCombinationInfoSN ::=           SEQUENCE {

bandCombinationIndex                BandCombinationIndex,

requestedFeatureSets                FeatureSetEntryIndex

}

FR-InfoList ::= SEQUENCE (SIZE (1..maxNrofServingCells-1)) OF FR-Info

FR-Info ::= SEQUENCE {

servCellIndex       ServCellIndex,

fr-Type             ENUMERATED {fr1, fr2}

}

CandidateServingFreqListNR ::= SEQUENCE (SIZE (1.. maxFreqIDC-MRDC)) OF ARFCN-ValueNR

CandidateServingFreqListEUTRA ::= SEQUENCE (SIZE (1.. maxFreqIDC-MRDC)) OF ARFCN-ValueEUTRA

T-Offset-r16 ::= ENUMERATED {ms0dot5, ms0dot75, ms1, ms1dot5, ms2, ms2dot5, ms3, spare1}

CandidateCellInfoListCPC-r17 ::= SEQUENCE (SIZE (1..maxFreq)) OF CandidateCellInfo-r17

CandidateCellInfo-r17 ::=        SEQUENCE {

ssbFrequency-r17                 ARFCN-ValueNR,

candidateList-r17                SEQUENCE (SIZE (1..maxNrofCondCells-r16)) OF CandidateCell-r17

}

CandidateCell-r17 ::=            SEQUENCE {

physCellId-r17                   PhysCellId,

condExecutionCondSCG-r17         OCTET STRING (CONTAINING CondReconfigExecCondSCG-r17)               OPTIONAL

}

-- TAG-CG-CONFIG-STOP

-- ASN1STOP

CG-Config field descriptions

candidateCellInfoListCPC Contains information regarding candidate target cells for Conditional PSCell Change (CPC) that the source secondary gNB suggests the target secondary gNB to consider configuring for CPC.

candidateCellInfoListSN Contains information regarding cells that the source secondary node suggests the target secondary gNB to consider configuring.

candidateCellInfoListSN-EUTRA Includes the MeasResultList3EUTRA as specified in TS 36.331 [10]. Contains information regarding cells that the source secondary node suggests the target secondary eNB to consider configuring. This field is only used in NE-DC.

candidateServingFreqListNR, candidateServingFreqListEUTRA Indicates frequencies of candidate serving cells for In-Device Co-existence Indication (see TS 36.331 [10]).

configRestrictModReq Used by SN to request changes to SCG configuration restrictions previously set by MN to ensure UE capabilities are respected. E.g. can be used to request configuring an NR band combination whose use MN has previously forbidden. SN only includes this field in SN-initiated procedures.

drx-ConfigSCG This field contains the complete DRX configuration of the SCG. This field is only used in NR-DC.

drx-InfoSCG This field contains the DRX long and short cycle configuration of the SCG. This field is used in (NG)EN-DC and NE-DC.

drx-InfoSCG2 This field contains the drx-onDurationTimer configuration of the SCG. This field is only used in (NG)EN-DC.

fr-InfoListSCG Contains information of FR information of serving cells that include PScell and SCells configured in SCG.

fr1-Carriers-SCG, fr2-Carriers-SCG Indicates the number of FR1 or FR2 serving cells configured in SCG.

measuredFrequenciesSN Used by SN to indicate a list of frequencies measured by the UE.

needForGaps In NE-DC, indicates whether the SN requests gNB to configure measurements gaps.

ph-InfoSCG Power headroom information in SCG that is needed in the reception of PHR MAC CE of MCG

ph-SupplementaryUplink Power headroom information for supplementary uplink. In the case of (NG)EN-DC and NR-DC, this field is only present when two UL carriers are configured for a serving cell and one UL carrier reports type1 PH while the other reports type 3 PH.

ph-Type1or3 Type of power headroom for a certain serving cell in SCG (PSCell and activated SCells). Value type1 refers to type 1 power headroom, value type3 refers to type 3 power headroom. (See TS 38.321 [3]).

ph-Uplink Power headroom information for uplink.

pSCellFrequency, pSCellFrequencyEUTRA Indicates the frequency of PSCell in NR (i.e., pSCellFrequency) or E-UTRA (i.e., pSCellFrequencyEUTRA). In this version of the specification, pSCellFrequency is not used in NE-DC whereas pSCellFrequencyEUTRA is only used in NE-DC. pSCellFrequency indicates the absoluteFrequencySSB.

reportCGI-RequestNR, reportCGI-RequestEUTRA Used by SN to indicate to MN about configuring reportCGI procedure. The request may optionally contain information about the cell for which SN intends to configure reportCGI procedure. In this version of the specification, the reportCGI-RequestNR is used in (NG)EN-DC and NR-DC whereas reportCGI-RequestEUTRA is used only for NE-DC.

requestedBC-MRDC Used to request configuring a band combination and corresponding feature sets which are forbidden to use by MN (i.e. outside of the allowedBC-ListMRDC) to allow re-negotiation of the UE capabilities for SCG configuration.

requestedMaxInterFreqMeasIdSCG Used to request the maximum number of allowed measurement identities to configure for inter-frequency measurement. This field is only used in NR-DC.

requestedMaxIntraFreqMeasIdSCG Used to request the maximum number of allowed measurement identities to configure for intra-frequency measurement on each serving frequency.

requestedPDCCH-BlindDetectionSCG Requested value of the reference number of cells for PDCCH blind detection allowed to be configured for the SCG.

requestedP-MaxEUTRA Requested value for the maximum power for the serving cells the UE can use in E-UTRA SCG. This field is only used in NE-DC.

requestedP-MaxFR1 Requested value for the maximum power for the serving cells on frequency range 1 (FR1) in this secondary cell group (see TS 38.104 [12]) the UE can use in NR SCG.

requestedP-MaxFR2 Requested value for the maximum power for the serving cells on frequency range 2 (FR2) in this secondary cell group the UE can use in NR SCG. This field is only used in NR-DC.

requestedToffset Requests the new value for the time offset restriction used by the SN for scheduling SCG transmissions (i.e. Tproc,SCGmax, see TS 38.213 [13]). This field is used in NR-DC only when the fields nrdc-PC-mode-FR1-r16 or nrdc-PC-mode-FR2-r16 are set to dynamic. Value ms0dot5 corresponds to 0.5 ms, value ms0dot75 corresponds to 0.75 ms, value ms1 corresponds to 1ms and so on.

scellFrequenciesSN-EUTRA, scellFrequenciesSN-NR Indicates the frequency of all SCells with SSB configured in SCG. The field scellFrequenciesSN-EUTRA is used in NE-DC; the field scellFrequenciesSN-NR is used in (NG)EN-DC and NR-DC. In (NG)EN-DC, the field is optionally provided to the MN. scellFrequenciesSN-NR indicates absoluteFrequencySSB.

scg-CellGroupConfig Contains the RRCReconfiguration message (containing only secondaryCellGroup and/or measConfig and/or otherConfig and/or conditionalReconfiguration and/or bap-Config and/or iab-IP-AddressConfigurationList): - to be sent to the UE, used upon SCG establishment or modification (only when the SCG is not released by the SN), as generated (entirely) by the (target) SgNB. In this case, the SN sets the RRCReconfiguration message in accordance with clause 6 e.g. regarding the "Need" or "Cond" statements. or - including the current SCG configuration of the UE, when provided in response to a query from MN, or in SN triggered SN change in order to enable delta signaling by the target SN, or in SN triggered modification procedure in order to coordinate CHO or MN-initiated CPC with SCG reconfigurations (see TS 38.423 [35]). In this case, the SN sets the RRCReconfiguration message in accordance with clause 11.2.3. The field is absent if neither SCG (re)configuration nor SCG configuration query nor SN triggered modification procedure in order to coordinate CHO or MN-initiated CPC with SCG reconfigurations (see TS 38.423 [35]) nor SN triggered SN change is performed, e.g. at inter-node capability/configuration coordination which does not result in SCG (re)configuration towards the UE. The field is also absent upon an SCG release triggered by the SN. This field is not applicable in NE-DC.

scg-CellGroupConfigEUTRA Includes the E-UTRA RRCConnectionReconfiguration message as specified in TS 36.331 [10]. In this version of the specification, the E-UTRA RRC message can only include the field scg-Configuration: - to be sent to the UE, used to (re-)configure the SCG configuration upon SCG establishment or modification (only when the SCG is not released by the SN), as generated (entirely) by the (target) SeNB. In this case, the SN sets the scg-Configuration within the EUTRA RRCConnectionReconfiguration message in accordance with clause 6 in TS 36.331 [10] e.g. regarding the "Need" or "Cond" statements. or - including the current SCG configuration of the UE, when provided in response to a query from MN, or in SN triggered SN change in order to enable delta signalling by the target SN. The field is absent if neither SCG (re)configuration nor SCG configuration query nor SN triggered SN change is performed, e.g. at inter-node capability/configuration coordination which does not result in SCG (re)configuration towards the UE. The field is also absent upon an SCG release triggered by the SN. This field is only used in NE-DC.

scg-RB-Config Contains the IE RadioBearerConfig: - to be sent to the UE, used to (re-)configure the SCG RB configuration upon SCG establishment or modification, as generated (entirely) by the (target) SgNB or SeNB. In this case, the SN sets the RadioBearerConfig in accordance with clause 6, e.g. regarding the "Need" or "Cond" statements. or - including the current SCG RB configuration of the UE, when provided in response to a query from MN or in SN triggered SN change or in SN triggered SN release or bearer type change between SN terminated bearer to MN terminated bearer in order to enable delta signaling by the MN or target SN. In this case, the SN sets the RadioBearerConfig in accordance with clause 11.2.3. The field is absent if neither SCG (re)configuration nor SCG configuration query nor SN triggered SN change nor SN triggered SN release is performed, e.g. at inter-node capability/configuration coordination which does not result in SCG RB (re)configuration.

selectedBandCombination Indicates the band combination selected by SN in (NG)EN-DC, NE-DC, and NR-DC. The SN should inform the MN with this field whenever the band combination and/or feature set it selected for the SCG changes (i.e. even if the new selection concerns a band combination and/or feature set that is allowed by the allowedBC-ListMRDC)

selectedToffset Indicates the value used by the SN for scheduling SCG transmissions (i.e. Tproc,SCGmax, see TS 38.213 [13]). This field is used in NR-DC only when the fields nrdc-PC-mode-FR1-r16 or nrdc-PC-mode-FR2-r16 are set to dynamic. The SN can only indicate a value that is less than or equal to maxToffset received from MN. This field is used in NR-DC only when MN has included the field maxToffset in CG-ConfigInfo. Value ms0dot5 corresponds to 0.5 ms, value ms0dot75 corresponds to 0.75 ms, value ms1 corresponds to 1ms and so on.

servCellInfoListSCG-EUTRA Indicates the carrier frequency and the transmission bandwidth of the serving cell(s) in the SCG in intra-band NE-DC. The field is needed when MN and SN operate serving cells in the same band for either contiguous or non-contiguous intra-band band combination or LTE NR inter-band band combinations where the frequency range of the E-UTRA band is a subset of the frequency range of the NR band (as specified in Table 5.5B.4.1-1 of TS 38.101-3 [34]) in NE-DC.

servCellInfoListSCG-NR Indicates the frequency band indicator, carrier center frequency, UE specific channel bandwidth and SCS of the serving cell(s) in the SCG in intra-band (NG)EN-DC. The field is needed when MN and SN operate serving cells in the same band for either contiguous or non-contiguous intra-band band combination or LTE NR inter-band band combinations where the frequency range of the E-UTRA band is a subset of the frequency range of the NR band (as specified in Table 5.5B.4.1-1 of TS 38.101-3 [34]) in (NG)EN-DC.

twoPHRModeSCG Indicates if the power headroom for SCG shall be reported as two PHRs (each PHR associated with a SRS resource set) is enabled or not.

twoSRS-PUSCH-Repetition Indicates whether the indicated serving cell is configured for PUSCH repetition corresponding to two SRS resource sets configured in either srs-ResourceSetToAddModList or srs-ResourceSetToAddModListDCI-0-2 with usage 'codebook' or 'noncodebook'.

transmissionBandwidth-EUTRA Indicates the transmission bandwidth on an E-UTRA carrier frequency as defined by the parameter Transmission Bandwidth Configuration "NRB" TS 36.104 [33]. The values rb6, rb15, rb25, rb50, rb75, rb100 indicate 6, 15, 25, 50, 75 and 100 resource blocks respectively.

ueAssistanceInformationSCG Includes for each UE assistance feature associated with the SCG, the information last reported by the UE in the NR UEAssistanceInformation message for the SCG, if any.

BandCombinationInfoSN field descriptions

bandCombinationIndex In case of NR-DC, this field indicates the position of a band combination in the supportedBandCombinationList. In case of NE-DC, this field indicates the position of a band combination in the supportedBandCombinationList and/or supportedBandCombinationListNEDC-Only. In case of (NG)EN-DC, this field indicates the position of a band combination in the supportedBandCombinationList and/or supportedBandCombinationList-UplinkTxSwitch. Band combination entries in supportedBandCombinationList are referred by an index which corresponds to the position of a band combination in the supportedBandCombinationList. Band combination entries in supportedBandCombinationListNEDC-Only are referred by an index which corresponds to the position of a band combination in the supportedBandCombinationListNEDC-Only increased by the number of entries in supportedBandCombinationList. Band combination entries in supportedBandCombinationList-UplinkTxSwitch are referred by an index which corresponds to the position of a band combination in the supportedBandCombinationList-UplinkTxSwitch increased by the number of entries in supportedBandCombinationList.

requestedFeatureSets The position in the FeatureSetCombination which identifies one FeatureSetUplink/Downlink for each band entry in the associated band combination

| Conditional Presence | Explanation |
| --- | --- |
| FDD | This field is mandatory present if dl-FreqInfo-NR is included and concerns an FDD carrier; otherwise the field is absent. |

#### –	CG-ConfigInfo

This message is used by master eNB or gNB to request the SgNB or SeNB to perform certain actions e.g. to establish, modify or release an SCG. The message may include additional information e.g. to assist the SgNB or SeNB to set the SCG configuration. It can also be used by a CU to request a DU to perform certain actions, e.g. to establish, or modify an MCG or SCG.

Direction: Master eNB or gNB to secondary gNB or eNB, alternatively CU to DU.

CG-ConfigInfo message

-- ASN1START

-- TAG-CG-CONFIG-INFO-START

CG-ConfigInfo ::=               SEQUENCE {

criticalExtensions              CHOICE {

c1                              CHOICE{

cg-ConfigInfo               CG-ConfigInfo-IEs,

spare3 NULL, spare2 NULL, spare1 NULL

},

criticalExtensionsFuture        SEQUENCE {}

}

}

CG-ConfigInfo-IEs ::=           SEQUENCE {

ue-CapabilityInfo               OCTET STRING (CONTAINING UE-CapabilityRAT-ContainerList)          OPTIONAL,-- Cond SN-AddMod

candidateCellInfoListMN         MeasResultList2NR                                                 OPTIONAL,

candidateCellInfoListSN         OCTET STRING (CONTAINING MeasResultList2NR)                       OPTIONAL,

measResultCellListSFTD-NR       MeasResultCellListSFTD-NR                                         OPTIONAL,

scgFailureInfo                  SEQUENCE {

failureType                     ENUMERATED { t310-Expiry, randomAccessProblem,

rlc-MaxNumRetx, synchReconfigFailure-SCG,

scg-reconfigFailure,

srb3-IntegrityFailure},

measResultSCG                   OCTET STRING (CONTAINING MeasResultSCG-Failure)

}                                                                                                 OPTIONAL,

configRestrictInfo              ConfigRestrictInfoSCG                                             OPTIONAL,

drx-InfoMCG                     DRX-Info                                                          OPTIONAL,

measConfigMN                    MeasConfigMN                                                      OPTIONAL,

sourceConfigSCG                 OCTET STRING (CONTAINING RRCReconfiguration)                      OPTIONAL,

scg-RB-Config                   OCTET STRING (CONTAINING RadioBearerConfig)                       OPTIONAL,

mcg-RB-Config                   OCTET STRING (CONTAINING RadioBearerConfig)                       OPTIONAL,

mrdc-AssistanceInfo             MRDC-AssistanceInfo                                               OPTIONAL,

nonCriticalExtension            CG-ConfigInfo-v1540-IEs                                           OPTIONAL

}

CG-ConfigInfo-v1540-IEs ::=     SEQUENCE {

ph-InfoMCG                      PH-TypeListMCG                                                    OPTIONAL,

measResultReportCGI             SEQUENCE {

ssbFrequency                    ARFCN-ValueNR,

cellForWhichToReportCGI         PhysCellId,

cgi-Info                        CGI-InfoNR

}                                                                                                 OPTIONAL,

nonCriticalExtension            CG-ConfigInfo-v1560-IEs                                           OPTIONAL

}

CG-ConfigInfo-v1560-IEs ::=  SEQUENCE {

candidateCellInfoListMN-EUTRA       OCTET STRING                                                  OPTIONAL,

candidateCellInfoListSN-EUTRA       OCTET STRING                                                  OPTIONAL,

sourceConfigSCG-EUTRA               OCTET STRING                                                  OPTIONAL,

scgFailureInfoEUTRA                 SEQUENCE {

failureTypeEUTRA                    ENUMERATED { t313-Expiry, randomAccessProblem,

rlc-MaxNumRetx, scg-ChangeFailure},

measResultSCG-EUTRA                 OCTET STRING

}                                                                                                 OPTIONAL,

drx-ConfigMCG                       DRX-Config                                                    OPTIONAL,

measResultReportCGI-EUTRA               SEQUENCE {

eutraFrequency                      ARFCN-ValueEUTRA,

cellForWhichToReportCGI-EUTRA           EUTRA-PhysCellId,

cgi-InfoEUTRA                           CGI-InfoEUTRA

}                                                                                                 OPTIONAL,

measResultCellListSFTD-EUTRA        MeasResultCellListSFTD-EUTRA                                  OPTIONAL,

fr-InfoListMCG                      FR-InfoList                                                   OPTIONAL,

nonCriticalExtension                CG-ConfigInfo-v1570-IEs                                       OPTIONAL

}

CG-ConfigInfo-v1570-IEs ::=  SEQUENCE {

sftdFrequencyList-NR                SFTD-FrequencyList-NR                                         OPTIONAL,

sftdFrequencyList-EUTRA             SFTD-FrequencyList-EUTRA                                      OPTIONAL,

nonCriticalExtension                CG-ConfigInfo-v1590-IEs                                       OPTIONAL

}

CG-ConfigInfo-v1590-IEs ::=  SEQUENCE {

servFrequenciesMN-NR            SEQUENCE (SIZE (1.. maxNrofServingCells-1)) OF  ARFCN-ValueNR     OPTIONAL,

nonCriticalExtension            CG-ConfigInfo-v1610-IEs                                           OPTIONAL

}

CG-ConfigInfo-v1610-IEs ::=  SEQUENCE {

drx-InfoMCG2                 DRX-Info2                                                            OPTIONAL,

alignedDRX-Indication        ENUMERATED {true}                                                    OPTIONAL,

scgFailureInfo-r16                  SEQUENCE {

failureType-r16                     ENUMERATED { scg-lbtFailure-r16, beamFailureRecoveryFailure-r16,

t312-Expiry-r16, bh-RLF-r16,

beamFailure-r17, spare3, spare2, spare1},

measResultSCG-r16                   OCTET STRING (CONTAINING MeasResultSCG-Failure)

}                                                                                                 OPTIONAL,

dummy1                                  SEQUENCE {

failureTypeEUTRA-r16                    ENUMERATED { scg-lbtFailure-r16, beamFailureRecoveryFailure-r16,

t312-Expiry-r16, spare5,

spare4, spare3, spare2, spare1},

measResultSCG-EUTRA-r16                 OCTET STRING

}                                                                                                 OPTIONAL,

sidelinkUEInformationNR-r16      OCTET STRING (CONTAINING SidelinkUEInformationNR-r16)            OPTIONAL,

sidelinkUEInformationEUTRA-r16   OCTET STRING                                                     OPTIONAL,

nonCriticalExtension             CG-ConfigInfo-v1620-IEs                                          OPTIONAL

}

CG-ConfigInfo-v1620-IEs ::=             SEQUENCE {

ueAssistanceInformationSourceSCG-r16    OCTET STRING (CONTAINING UEAssistanceInformation)         OPTIONAL,

nonCriticalExtension                    CG-ConfigInfo-v1640-IEs                                   OPTIONAL

}

CG-ConfigInfo-v1640-IEs ::=             SEQUENCE {

servCellInfoListMCG-NR-r16              ServCellInfoListMCG-NR-r16                   OPTIONAL,

servCellInfoListMCG-EUTRA-r16           ServCellInfoListMCG-EUTRA-r16                OPTIONAL,

nonCriticalExtension                    CG-ConfigInfo-v1700-IEs                      OPTIONAL

}

CG-ConfigInfo-v1700-IEs ::=             SEQUENCE {

candidateCellListCPC-r17                CandidateCellListCPC-r17                     OPTIONAL,

twoPHRModeMCG-r17                       ENUMERATED {enabled}                         OPTIONAL,

lowMobilityEvaluationConnectedInPCell-r17 ENUMERATED {enabled}                       OPTIONAL,

nonCriticalExtension                    CG-ConfigInfo-v1730-IEs                      OPTIONAL

}

CG-ConfigInfo-v1730-IEs ::=             SEQUENCE {

fr1-Carriers-MCG-r17                    INTEGER (1..32)                              OPTIONAL,

fr2-Carriers-MCG-r17                    INTEGER (1..32)                              OPTIONAL,

nonCriticalExtension                    CG-ConfigInfo-v18xy-IEs                                  OPTIONAL

}

CG-ConfigInfo-v18xy-IEs ::=             SEQUENCE {

musim-GapConfigInfo-v18xy                 MUSIM-GapConfig-r17                          OPTIONAL,

nonCriticalExtension                    SEQUENCE {}                                  OPTIONAL

}

ServCellInfoListMCG-NR-r16 ::=          SEQUENCE (SIZE (1.. maxNrofServingCells)) OF  ServCellInfoXCG-NR-r16

ServCellInfoListMCG-EUTRA-r16 ::=       SEQUENCE (SIZE (1.. maxNrofServingCellsEUTRA)) OF ServCellInfoXCG-EUTRA-r16

SFTD-FrequencyList-NR ::=               SEQUENCE (SIZE (1..maxCellSFTD)) OF ARFCN-ValueNR

SFTD-FrequencyList-EUTRA ::=            SEQUENCE (SIZE (1..maxCellSFTD)) OF ARFCN-ValueEUTRA

ConfigRestrictInfoSCG ::=       SEQUENCE {

allowedBC-ListMRDC              BandCombinationInfoList                                           OPTIONAL,

powerCoordination-FR1               SEQUENCE {

p-maxNR-FR1                     P-Max                                                         OPTIONAL,

p-maxEUTRA                      P-Max                                                         OPTIONAL,

p-maxUE-FR1                     P-Max                                                         OPTIONAL

}                                                                                                 OPTIONAL,

servCellIndexRangeSCG           SEQUENCE {

lowBound                        ServCellIndex,

upBound                         ServCellIndex

}                                                                                                 OPTIONAL,   -- Cond SN-AddMod

maxMeasFreqsSCG                     INTEGER(1..maxMeasFreqsMN)                                    OPTIONAL,

dummy                               INTEGER(1..maxMeasIdentitiesMN)                               OPTIONAL,

...,

[[

selectedBandEntriesMNList        SEQUENCE (SIZE (1..maxBandComb)) OF SelectedBandEntriesMN        OPTIONAL,

pdcch-BlindDetectionSCG          INTEGER (1..15)                                                  OPTIONAL,

maxNumberROHC-ContextSessionsSN  INTEGER(0.. 16384)                                               OPTIONAL

]],

[[

maxIntraFreqMeasIdentitiesSCG     INTEGER(1..maxMeasIdentitiesMN)                                 OPTIONAL,

maxInterFreqMeasIdentitiesSCG     INTEGER(1..maxMeasIdentitiesMN)                                 OPTIONAL

]],

[[

p-maxNR-FR1-MCG-r16               P-Max                                                           OPTIONAL,

powerCoordination-FR2-r16         SEQUENCE {

p-maxNR-FR2-MCG-r16                P-Max                                                      OPTIONAL,

p-maxNR-FR2-SCG-r16                P-Max                                                      OPTIONAL,

p-maxUE-FR2-r16                    P-Max                                                      OPTIONAL

}                                                                                                 OPTIONAL,

nrdc-PC-mode-FR1-r16    ENUMERATED {semi-static-mode1, semi-static-mode2, dynamic}                OPTIONAL,

nrdc-PC-mode-FR2-r16    ENUMERATED {semi-static-mode1, semi-static-mode2, dynamic}                OPTIONAL,

maxMeasSRS-ResourceSCG-r16       INTEGER(0..maxNrofCLI-SRS-Resources-r16)                         OPTIONAL,

maxMeasCLI-ResourceSCG-r16       INTEGER(0..maxNrofCLI-RSSI-Resources-r16)                        OPTIONAL,

maxNumberEHC-ContextsSN-r16      INTEGER(0..65536)                                                OPTIONAL,

allowedReducedConfigForOverheating-r16      OverheatingAssistance                                 OPTIONAL,

maxToffset-r16                   T-Offset-r16                                                     OPTIONAL

]],

[[

allowedReducedConfigForOverheating-r17      OverheatingAssistance-r17                             OPTIONAL,

maxNumberUDC-DRB-r17             INTEGER(0..2)                                                    OPTIONAL,

maxNumberCPCCandidates-r17       INTEGER(0..maxNrofCondCells-1-r17)                               OPTIONAL

]]

}

SelectedBandEntriesMN ::=       SEQUENCE (SIZE (1..maxSimultaneousBands)) OF BandEntryIndex

BandEntryIndex ::=              INTEGER (0.. maxNrofServingCells)

PH-TypeListMCG ::=              SEQUENCE (SIZE (1..maxNrofServingCells)) OF PH-InfoMCG

PH-InfoMCG ::=                  SEQUENCE {

servCellIndex                       ServCellIndex,

ph-Uplink                           PH-UplinkCarrierMCG,

ph-SupplementaryUplink              PH-UplinkCarrierMCG                                           OPTIONAL,

...,

[[

twoSRS-PUSCH-Repetition-r17         ENUMERATED{enabled}                                           OPTIONAL

]]

}

PH-UplinkCarrierMCG ::=         SEQUENCE{

ph-Type1or3                         ENUMERATED {type1, type3},

...

}

BandCombinationInfoList ::=     SEQUENCE (SIZE (1..maxBandComb)) OF BandCombinationInfo

BandCombinationInfo ::=         SEQUENCE {

bandCombinationIndex            BandCombinationIndex,

allowedFeatureSetsList          SEQUENCE (SIZE (1..maxFeatureSetsPerBand)) OF FeatureSetEntryIndex

}

FeatureSetEntryIndex ::=        INTEGER (1.. maxFeatureSetsPerBand)

DRX-Info ::=                    SEQUENCE {

drx-LongCycleStartOffset        CHOICE {

ms10                            INTEGER(0..9),

ms20                            INTEGER(0..19),

ms32                            INTEGER(0..31),

ms40                            INTEGER(0..39),

ms60                            INTEGER(0..59),

ms64                            INTEGER(0..63),

ms70                            INTEGER(0..69),

ms80                            INTEGER(0..79),

ms128                           INTEGER(0..127),

ms160                           INTEGER(0..159),

ms256                           INTEGER(0..255),

ms320                           INTEGER(0..319),

ms512                           INTEGER(0..511),

ms640                           INTEGER(0..639),

ms1024                          INTEGER(0..1023),

ms1280                          INTEGER(0..1279),

ms2048                          INTEGER(0..2047),

ms2560                          INTEGER(0..2559),

ms5120                          INTEGER(0..5119),

ms10240                         INTEGER(0..10239)

},

shortDRX                            SEQUENCE {

drx-ShortCycle                      ENUMERATED  {

ms2, ms3, ms4, ms5, ms6, ms7, ms8, ms10, ms14, ms16, ms20, ms30, ms32,

ms35, ms40, ms64, ms80, ms128, ms160, ms256, ms320, ms512, ms640, spare9,

spare8, spare7, spare6, spare5, spare4, spare3, spare2, spare1 },

drx-ShortCycleTimer                 INTEGER (1..16)

}                                                                                             OPTIONAL

}

DRX-Info2 ::=          SEQUENCE {

drx-onDurationTimer    CHOICE {

subMilliSeconds INTEGER (1..31),

milliSeconds    ENUMERATED {

ms1, ms2, ms3, ms4, ms5, ms6, ms8, ms10, ms20, ms30, ms40, ms50, ms60,

ms80, ms100, ms200, ms300, ms400, ms500, ms600, ms800, ms1000, ms1200,

ms1600, spare8, spare7, spare6, spare5, spare4, spare3, spare2, spare1 }

}

}

MeasConfigMN ::= SEQUENCE {

measuredFrequenciesMN               SEQUENCE (SIZE (1..maxMeasFreqsMN)) OF NR-FreqInfo        OPTIONAL,

measGapConfig                       SetupRelease { GapConfig }                                OPTIONAL,

gapPurpose                          ENUMERATED {perUE, perFR1}                                OPTIONAL,

...,

[[

measGapConfigFR2                    SetupRelease { GapConfig }                                OPTIONAL

]],

[[

interFreqNoGap-r16                  ENUMERATED {true}                                         OPTIONAL

]]

}

MRDC-AssistanceInfo ::= SEQUENCE {

affectedCarrierFreqCombInfoListMRDC     SEQUENCE (SIZE (1..maxNrofCombIDC)) OF AffectedCarrierFreqCombInfoMRDC,

...,

[[

overheatingAssistanceSCG-r16            OCTET STRING (CONTAINING OverheatingAssistance)       OPTIONAL

]],

[[

overheatingAssistanceSCG-FR2-2-r17      OCTET STRING (CONTAINING OverheatingAssistance-r17)   OPTIONAL

]]

}

AffectedCarrierFreqCombInfoMRDC ::= SEQUENCE {

victimSystemType                    VictimSystemType,

interferenceDirectionMRDC           ENUMERATED {eutra-nr, nr, other, utra-nr-other, nr-other, spare3, spare2, spare1},

affectedCarrierFreqCombMRDC         SEQUENCE    {

affectedCarrierFreqCombEUTRA        AffectedCarrierFreqCombEUTRA                          OPTIONAL,

affectedCarrierFreqCombNR           AffectedCarrierFreqCombNR

}                                                                                             OPTIONAL

}

VictimSystemType ::= SEQUENCE {

gps                         ENUMERATED {true}               OPTIONAL,

glonass                     ENUMERATED {true}               OPTIONAL,

bds                         ENUMERATED {true}               OPTIONAL,

galileo                     ENUMERATED {true}               OPTIONAL,

wlan                        ENUMERATED {true}               OPTIONAL,

bluetooth                   ENUMERATED {true}               OPTIONAL

}

AffectedCarrierFreqCombEUTRA ::= SEQUENCE (SIZE (1..maxNrofServingCellsEUTRA)) OF ARFCN-ValueEUTRA

AffectedCarrierFreqCombNR ::= SEQUENCE (SIZE (1..maxNrofServingCells)) OF ARFCN-ValueNR

CandidateCellListCPC-r17 ::= SEQUENCE (SIZE (1..maxFreq)) OF CandidateCellCPC-r17

CandidateCellCPC-r17 ::=           SEQUENCE {

ssbFrequency-r17                   ARFCN-ValueNR,

candidateCellList-r17              SEQUENCE (SIZE (1..maxNrofCondCells-r16)) OF PhysCellId

}

-- TAG-CG-CONFIG-INFO-STOP

-- ASN1STOP

CG-ConfigInfo field descriptions

alignedDRX-Indication This field is signalled upon MN triggered CGI reporting by the UE that requires aligned DRX configurations between the MCG and the SCG (i.e. same DRX cycle and on-duration configured by MN completely contains on-duration configured by SN).

allowedBC-ListMRDC A list of indices referring to band combinations in MR-DC capabilities from which SN is allowed to select the SCG band combination. Each entry refers to: - a band combination numbered according to supportedBandCombinationList and supportedBandCombinationList-UplinkTxSwitch in the UE-MRDC-Capability (in case of (NG)EN-DC), or according to supportedBandCombinationList and supportedBandCombinationListNEDC-Only in the UE-MRDC-Capability (in case of NE-DC), or according to supportedBandCombinationList in the UE-NR-Capability (in case of NR-DC), - and the Feature Sets allowed for each band entry. All MR-DC band combinations indicated by this field comprise the MCG band combination, which is a superset of the MCG band(s) selected by MN.

allowedReducedConfigForOverheating Indicates the reduced configuration that the SCG is allowed to configure. reducedMaxCCs in allowedReducedConfigForOverheating indicates the maximum number of downlink/uplink PSCell/SCells that the SCG is allowed to configure. This field is used in (NG)EN-DC and NR-DC. reducedMaxBW-FR1 and reducedMaxBW-FR2 in allowedReducedConfigForOverheating indicates the maximum aggregated bandwidth across all downlink/uplink carriers of FR1 and FR2-1, respectively that the SCG is allowed to configure. reducedMaxBW-FR2-2 in allowedReducedConfigForOverheating-r17 indicates the maximum aggregated bandwidth across all downlink/uplink carriers of FR2-2 that the SCG is allowed to configure. This field is only used in NR-DC. reducedMaxMIMO-LayersFR1 and reducedMaxMIMO-LayersFR2 in allowedReducedConfigForOverheating indicates the maximum number of downlink/uplink MIMO layers of each serving cell operating on FR1 and FR2-1, respectively that the SCG is allowed to configure. reducedMaxMIMO-LayersFR2-2 in allowedReducedConfigForOverheating-r17 indicates the maximum number of downlink/uplink MIMO layers of each serving cell operating on FR2-2 that the SCG is allowed to configure. This field is only used in NR-DC.

candidateCellInfoListMN, candidateCellInfoListSN Contains information regarding cells that the master node or the source node suggests the target gNB or DU to consider configuring. In case of MN initiated CPA or CPC, the field candidateCellInfoListMN contains information regarding cells that the MN suggests the candidate target secondary node to consider configuring for MN initiated CPA or CPC. For (NG)EN-DC, including CSI-RS measurement results in candidateCellInfoListMN is not supported in this version of the specification. For NR-DC, including SSB and/or CSI-RS measurement results in candidateCellInfoListMN is supported.

candidateCellInfoListMN-EUTRA, candidateCellInfoListSN-EUTRA Includes the MeasResultList3EUTRA as specified in TS 36.331 [10]. Contains information regarding cells that the master node or the source node suggests the target secondary eNB to consider configuring. These fields are only used in NE-DC.

candidateCellListCPC Contains information regarding cells that the source secondary node suggests the candidate target secondary node to consider configuring for SN initiated Conditional PSCell Change (CPC).

configRestrictInfo Includes fields for which SgNB is explicitly indicated to observe a configuration restriction.

drx-ConfigMCG This field contains the complete DRX configuration of the MCG. This field is only used in NR-DC.

drx-InfoMCG This field contains the DRX long and short cycle configuration of the MCG. This field is used in (NG)EN-DC and NE-DC.

drx-InfoMCG2 This field contains the drx-onDurationTimer configuration of the MCG. This field is only used in (NG)EN-DC.

dummy, dummy1 These fields are not used in the specification and SN ignores the received value(s).

fr-InfoListMCG Contains information of FR information of serving cells that include PCell and SCell(s) configured in MCG.

fr1-Carriers-MCG, fr2-Carriers-MCG Indicates the number of FR1 or FR2 serving cells configured in MCG.

interFreqNoGap Indicates that the field interFrequencyConfig-NoGap-r16 has been included within the MeasConfig IE generated by the MN.

lowMobilityEvaluationConnectedInPCell Indicates if low mobility criterion has been configured in NR PCell.

maxInterFreqMeasIdentitiesSCG Indicates the maximum number of allowed measurement identities that the SCG is allowed to configure for inter-frequency measurement. The maximum value for this field is 10. If the field is absent, the SCG is allowed to configure inter-frequency measurements up to the maximum value. This field is only used in NR-DC.

maxIntraFreqMeasIdentitiesSCG Indicates the maximum number of allowed measurement identities that the SCG is allowed to configure for intra-frequency measurement on each serving frequency. The maximum value for this field is 9 (in case of (NG)EN-DC or NR-DC) or 10 (in case of NE-DC). If the field is absent, the SCG is allowed to configure intra-frequency measurements up to the maximum value on each serving frequency.

maxMeasCLI-ResourceSCG Indicates the maximum number of CLI RSSI resources that the SCG is allowed to configure.

maxMeasFreqsSCG Indicates the maximum number of NR inter-frequency carriers the SN is allowed to configure with PSCell for measurements.

maxMeasSRS-ResourceSCG Indicates the maximum number of SRS resources that the SCG is allowed to configure for CLI measurement.

maxNumberCPCCandidates Indicates the maximum numbers of conditional reconfigurations the SN is allowed to configure for SN initiated CPC. Value 0 indicates that the SN is not allowed to configure SN initiated CPC. If the field is absent, the SN is allowed to configure up to maxNrofCondCells-r16 conditional reconfigurations for SN-initiated CPC.

maxNumberROHC-ContextSessionsSN Indicates the maximum number of ROHC context sessions allowed to SN terminated bearer, excluding context sessions that leave all headers uncompressed.

maxNumberEHC-ContextsSN Indicates the maximum number of EHC contexts allowed to the SN terminated bearer. The field indicates the number of contexts in addition to CID = "all zeros", as specified in TS 38.323 [5].

maxNumberUDC-DRB Indicates the maximum number of UDC DRBs allowed to SN terminated bearer. This field is used in NGEN-DC, NR-DC and NE-DC.

maxToffset Indicates the maximum Toffset value the SN is allowed to use for scheduling SCG transmissions (see TS 38.213 [13]). This field is used in NR-DC only when the fields nrdc-PC-mode-FR1-r16 or nrdc-PC-mode-FR2-r16 are set to dynamic. Value ms0dot5 corresponds to 0.5 ms, value ms0dot75 corresponds to 0.75 ms, value ms1 corresponds to 1 ms and so on.

measuredFrequenciesMN Used by MN to indicate a list of frequencies measured by the UE.

measGapConfig Indicates the FR1 and perUE measurement gap configuration configured by MN.

measGapConfigFR2 Indicates the FR2 measurement gap configuration configured by MN.

mcg-RB-Config Contains all of the fields in the IE RadioBearerConfig used in MN, used by the SN to support delta configuration to UE (i.e. when MN does not use full configuration option), for bearer type change between MN terminated bearer with NR PDCP to SN terminated bearer. It is also used to indicate the PDCP duplication related information for MN terminated split bearer (whether duplication is configured and if so, whether it is initially activated) in SN Addition/Modification procedure. Otherwise, this field is absent.

measResultReportCGI, measResultReportCGI-EUTRA Used by MN to provide SN with CGI-Info for the cell as per SN′s request. In this version of the specification, the measResultReportCGI is used for (NG)EN-DC and NR-DC and the measResultReportCGI-EUTRA is used only for NE-DC.

measResultSCG-EUTRA This field includes the MeasResultSCG-FailureMRDC IE as specified in TS 36.331 [10]. This field is only used in NE-DC.

measResultSFTD-EUTRA SFTD measurement results between the PCell and the E-UTRA PScell in NE-DC. This field is only used in NE-DC.

mrdc-AssistanceInfo Contains the IDC assistance information for MR-DC reported by the UE (see TS 36.331 [10]).

nrdc-PC-mode-FR1 Indicates the uplink power sharing mode that the UE uses in NR-DC FR1 (see TS 38.213 [13], clause 7.6).

nrdc-PC-mode-FR2 Indicates the uplink power sharing mode that the UE uses in NR-DC FR2 (see TS 38.213 [13], clause 7.6).

overheatingAssistanceSCG Contains the UE's preference on reduced configuration for NR SCG to address overheating. This field is only used in (NG)EN-DC.

overheatingAssistanceSCG-FR2-2 Contains the UE's preference on reduced configuration for NR SCG on FR2-2 to address overheating. This field is only used in (NG)EN-DC.

p-maxEUTRA Indicates the maximum total transmit power to be used by the UE in the E-UTRA cell group (see TS 36.104 [33]). This field is used in (NG)EN-DC and NE-DC.

p-maxNR-FR1 For (NG)EN-DC and NE-DC, the field indicates the maximum total transmit power to be used by the UE in the NR cell group across all serving cells in frequency range 1 (FR1) (see TS 38.104 [12]). For NR-DC, it indicates the maximum total transmit power to be used by the UE in the NR cell group across all serving cells in frequency range 1 (FR1) (see TS 38.104 [12]) the UE can use in NR SCG.

p-maxUE-FR1 Indicates the maximum total transmit power to be used by the UE across all serving cells in frequency range 1 (FR1).

p-maxNR-FR1-MCG Indicates the maximum total transmit power to be used by the UE in the NR cell group across all serving cells in frequency range 1 (FR1) (see TS 38.104 [12]) the UE can use in NR MCG. This field is only used in NR-DC.

p-maxNR-FR2-SCG Indicates the maximum total transmit power to be used by the UE in the NR cell group across all serving cells in frequency range 2 (FR2) (see TS 38.104 [12]) the UE can use in NR SCG.

p-maxUE-FR2 Indicates the maximum total transmit power to be used by the UE across all serving cells in frequency range 2 (FR2).

p-maxNR-FR2-MCG Indicates the maximum total transmit power to be used by the UE in the NR cell group across all serving cells in frequency range 2 (FR2) (see TS 38.104 [12]) the UE can use in NR MCG.

pdcch-BlindDetectionSCG Indicates the maximum value of the reference number of cells for PDCCH blind detection allowed to be configured for the SCG.

ph-InfoMCG Power headroom information in MCG that is needed in the reception of PHR MAC CE in SCG.

ph-SupplementaryUplink Power headroom information for supplementary uplink. For UE in (NG)EN-DC, this field is absent.

ph-Type1or3 Type of power headroom for a serving cell in MCG (PCell and activated SCells). type1 refers to type 1 power headroom, type3 refers to type 3 power headroom. (See TS 38.321 [3]).

ph-Uplink Power headroom information for uplink.

powerCoordination-FR1 Indicates the maximum power that the UE can use in FR1.

powerCoordination-FR2 Indicates the maximum power that the UE can use in frequency range 2 (FR2). This field is only used in NR-DC.

scgFailureInfo Contains SCG failure type and measurement results. In case the sender has no measurement results available, the sender may include one empty entry (i.e. without any optional fields present) in measResultPerMOList. This field is used in (NG)EN-DC and NR-DC.

scg-RB-Config Contains all of the fields in the IE RadioBearerConfig used in SN, used to allow the target SN to use delta configuration to the UE, e.g. during SN change. The field is signalled upon change of SN unless MN uses full configuration option. Otherwise, the field is absent.

selectedBandEntriesMNList A list of indices referring to the position of a band entry selected by the MN, in each band combination entry in allowedBC-ListMRDC IE. BandEntryIndex 0 identifies the first band in the bandList of the BandCombination, BandEntryIndex 1 identifies the second band in the bandList of the BandCombination, and so on. This selectedBandEntriesMNList includes the same number of entries, and listed in the same order as in allowedBC-ListMRDC. The SN uses this information to determine which bands out of the NR band combinations in allowedBC-ListMRDC it can configure in SCG in NR-DC. The SN can use this information to determine for which band pair(s) it should check SimultaneousRxTxPerBandPair.

servCellIndexRangeSCG Range of serving cell indices that SN is allowed to configure for SCG serving cells.

servCellInfoListMCG-EUTRA Indicates the carrier frequency and the transmission bandwidth of the serving cell(s) in the MCG in intra-band (NG)EN-DC. The field is needed when MN and SN operate serving cells in the same band for either contiguous or non-contiguous intra-band band combination or LTE NR inter-band band combinations where the frequency range of the E-UTRA band is a subset of the frequency range of the NR band (as specified in Table 5.5B.4.1-1 of TS 38.101-3 [34]) in (NG)EN-DC.

servCellInfoListMCG-NR Indicates the frequency band indicator, carrier center frequency, UE specific channel bandwidth and SCS of the serving cell(s) in the MCG in intra-band NE-DC. The field is needed when MN and SN operate serving cells in the same band for either contiguous or non-contiguous intra-band band combination or LTE NR inter-band band combinations where the frequency range of the E-UTRA band is a subset of the frequency range of the NR band (as specified in Table 5.5B.4.1-1 of TS 38.101-3 [34]) in NE-DC.

servFrequenciesMN-NR Indicates the frequency of all serving cells that include PCell and SCell(s) with SSB configured in MCG. This field is only used in NR-DC. servFrequenciesMN-NR indicates absoluteFrequencySSB.

sftdFrequencyList-NR Includes a list of SSB frequencies. Each entry identifies the SSB frequency of a PSCell, which corresponds to one MeasResultCellSFTD-NR entry in the MeasResultCellListSFTD-NR.

sftdFrequencyList-EUTRA Includes a list of E-UTRA frequencies. Each entry identifies the carrier frequency of a PSCell, which corresponds to one MeasResultSFTD-EUTRA entry in the MeasResultCellListSFTD-EUTRA.

sidelinkUEInformationEUTRA This field contains the E-UTRA SidelinkUEInformation message as specified in TS 36.331 [10].

sidelinkUEInformationNR This field contains the NR SidelinkUEInformationNR message.

sourceConfigSCG Includes all of the current SCG configurations used by the target SN to build delta configuration to be sent to UE, e.g. during SN change. The field contains the RRCReconfiguration message, i.e. including secondaryCellGroup and measConfig. The field is signalled upon change of SN, unless MN uses full configuration option. Otherwise, the field is absent.

sourceConfigSCG-EUTRA Includes the E-UTRA RRCConnectionReconfiguration message as specified in TS 36.331 [10]. In this version of the specification, the E-UTRA RRC message can only include the field scg-Configuration. In this version of the specification, this field is absent when master gNB uses full configuration option. This field is only used in NE-DC.

twoPHRModeMCG Indicates if the power headroom for MCG shall be reported as two PHRs (each PHR associated with a SRS resource set) is enabled or not.

twoSRS-PUSCH-Repetition Indicates whether the indicated serving cell is configured for PUSCH repetition corresponding to two SRS resource sets configured in either srs-ResourceSetToAddModList or srs-ResourceSetToAddModListDCI-0-2 with usage 'codebook' or 'noncodebook'.

ueAssistanceInformationSourceSCG Includes for each UE assistance feature associated with the SCG, the information last reported by the UE in the NR UEAssistanceInformation message for the source SCG, if any.

ue-CapabilityInfo Contains the IE UE-CapabilityRAT-ContainerList supported by the UE (see NOTE 3). A gNB that retrieves MRDC related capability containers ensures that the set of included MRDC containers is consistent w.r.t. the feature set related information.

BandCombinationInfo field descriptions

allowedFeatureSetsList Defines a subset of the entries in a FeatureSetCombination. Each index identifies a position in the FeatureSetCombination, which corresponds to one FeatureSetUplink/Downlink for each band entry in the associated band combination.

bandCombinationIndex In case of NR-DC, this field indicates the position of a band combination in the supportedBandCombinationList. In case of NE-DC, this field indicates the position of a band combination in the supportedBandCombinationList and/or supportedBandCombinationListNEDC-Only. In case of (NG)EN-DC, this field indicates the position of a band combination in the supportedBandCombinationList and/or supportedBandCombinationList-UplinkTxSwitch. Band combination entries in supportedBandCombinationList are referred by an index which corresponds to the position of a band combination in the supportedBandCombinationList. Band combination entries in supportedBandCombinationListNEDC-Only are referred by an index which corresponds to the position of a band combination in the supportedBandCombinationListNEDC-Only increased by the number of entries in supportedBandCombinationList. Band combination entries in supportedBandCombinationList-UplinkTxSwitch are referred by an index which corresponds to the position of a band combination in the supportedBandCombinationList-UplinkTxSwitch increased by the number of entries in supportedBandCombinationList.

| Conditional Presence | Explanation |
| --- | --- |
| SN-AddMod | The field is mandatory present upon SN addition and SN change. It is optionally present upon SN modification and inter-MN handover without SN change. Otherwise, the field is absent. |

NOTE 3:	The following table indicates per MN RAT and SN RAT whether RAT capabilities are included or not in ue-CapabilityInfo.

| MN RAT | SN RAT | NR capabilities | E-UTRA capabilities | MR-DC capabilities |
| --- | --- | --- | --- | --- |
| E-UTRA | NR | Need not be included if the UE Radio Capability ID as specified in 23.502 [43] is used. Included otherwise | Not included | Need not be included if the UE Radio Capability ID as specified in 23.502 [43] is used. Included otherwise |
| NR | E-UTRA | Not included | Need not be included if the UE Radio Capability ID as specified in 23.502 [43] is used. Included otherwise | Need not be included if the UE Radio Capability ID as specified in 23.502 [43] is used. Included otherwise |
| NR | NR | Need not be included if the UE Radio Capability ID as specified in 23.502 [43] is used. Included otherwise | Not included | Not included |

---------------------------------------------------------Change End----------------------------------------

# Annex of meeting agreements

Highlighted below are the meeting agreements that have been considered for the CR.

RAN2#123bis Agreements

- For proactive UE temporary capability reporting, UE reporting of the information regarding its impacted frequency is sufficient. - UE can indicate impacted band(s)/frequencies in a BC for the proactive reporting, detailed signalling is FFS. - UE is allowed to only report the impacted band(s)/frequencies based on a frequency/band filter list (e.g. frequencies/bands), if configured by the network. - UAI based signalling is also used for proactive reporting of temporary UE capability restriction. - One configuration is used to control all temporary capabilities update - We will introduce ‘wait timer’ for the reactive approach - The UE starts the timer when the UE requests a temporary restriction to the network if the timer is configured. We assume network configures the length for this timer. - Stop: if UE receives reconfiguration that does not exceed the capabilities that UE suggested via capability restriction report - Expiry: UE can apply the temporary UE capability restriction upon the timer expiry. - We will introduce ‘prohibit timer’ for the proactive approach (Network can set zero value for this timer, details can be handled in spec drafting phase) - The UE can remove the MUSIM capability restriction information by not including any fields in capability restriction report (details will be handled in the specification drafting). - Working assumption: Early capability restriction indication is provided in Msg5. Detailed UE behaviour, if any, can be further discussed. - The UE can indicate the temporary capability restriction of measurement gap for R18 MUSIM purpose in the UAI by using the existing NeedForGapInfoNR. - It is confirmed that the previous agreement that Maximum MIMO layers restriction (and bandwidth restriction, if supported) is reported per CC at least applies for the reactive approach. - Baseline for the proactive approach: Maximum MIMO layers restriction (and bandwidth restriction, if supported) is reported per FSPC (per cc per BC). - For Rel-18 MUSIM dual active operation, UE is configured with the band-filter list by the NW A in the OtherConfig for forbidden/affected band signalling. - For Rel-18 MUSIM dual active operation, UE indicates its forbidden/affected band combinations (or band(s)) based on the network configured band-filter list, in the UAI signalling to NW A. - For Rel-18 MUSIM dual active operation, UE signals its temporary capabilities restrictions as forbidden band combinations with band indexed to the band-filter list and/or affected band combinations with band indexed to the band-filter list along with explicit fields for restricted (lower) capabilities e.g. maximum MIMO layers. - RAN2 confirms that no need to request gap priority or configure gap priority for aperiodic gap. Below editor note from running CR could be removed. - •Editor’s Note: FFS musim-GapPriorityToAddModList-r18 is for aperodic MUSIM gap. - Introduce single bit indication in MUSIM assistance information to indicate the UE preference of “keep” option. - Reuse existing control flag (i.e. musim-GapPriorityAssistanceConfig-r18 in running CR) to indicate whether the UE could include “keep” option for MUSIM gap. - The prohibit timer configuration for R17 MUSIM gap preference (i.e. musim-GapProhibitTimer) is also apply to R18 MUSIM gap priority preference. - FFS if any other configuration or related behaviour is needed. - For the proactive approach, the MN can indicate the forbidden/affected band information (for the MUSIM purpose) to the SN. FFS for the reactive case.

RAN2#123 Agreements

- Use Msg5 for early indication of MUSIM capability restriction for UEs in IDLE. - Using LCIDs would avoid any problems for RRC resume procedure. However, there are not many LCIDs left for UL and some other Rel-18 WIs also intend to use them. - FFS whether there is a need to use the LCIDs or whether we can reuse the legacy LCIDs. - Whether we can use the LCIDs (given that multiple WIs may be trying to use them) will be discussed in the main session. How to proceed LCID usage for MUSIM can be discussed in the next meeting based on the main session decision. - Continue discussion in Thursday session with proactive approach on whether UE can indicating frequency that it would prefer to use. - Discussion was not continued due to lack of time. Post-meeting email discussion (long, vivo) on this topic. - If a timer is introduced, RAN2 needs to define UE behaviour when timer expires and network response is not received. RAN2 also needs to define what “network response” means, i.e. is it a RRCReconfiguration message or a particular field or something else? - FFS whether a timer is needed (e.g. to avoid UE from doing something while the network response has not yet arrived) - Companies should provide Stage-3 details for the next meeting on UE behaviour when network does or does not respond to the UE request to restrict the capabilities due to MUSIM. - 3: UE sends early indication of MUSIM temporary capability restriction only if the network indicates that it is allowed in SIB1. - 1: The UE can indicate that some frequencies (e.g. frequency ranges, bands or BCs) are impacted by NW B so that they are: - 1) forbidden because of collision - 2) having restricted (lower) capabilities (e.g. with lower MIMO layer). - 4: The restrictions can apply to CA, DC and/or single CC. - 5: The UL/DL MIMO layer and/or the UL/DL supported bandwidth restriction (if supported) shall only work for the restricted frequencies for the proactive case. - 1 For dual active MUSIM, the UE can indicate the temporary maximum MIMO layers for specific serving cells for both UL and DL. - FFS whether there is a use case for the UE to indicate the temporary supported channel bandwidth for specific serving cells. - 3 Maximum MIMO layers/bandwidth restriction is reported per CC ((FFS how we signal this). - FFS whether we support indicating temporary capability restrictions on SRS Tx switching capability. FFS whether this could be already indicated by the MIMO layer restrictions. - 1. When requesting periodic MUSIM gap(s), UE indicates priority values (using R17 IE definition) for all or a subset periodic MUSIM gaps. - 2. When receiving priorities for periodic MUSIM gap(s), the UE may receive changed priority values. If network doesn’t retain the relative priorities among MUSIM gaps, UE behaviour is not specified. - Send LS to RAN4 informing them of this agreement. Offline 203 (LGE).

RAN2#121bis-e Agreements

- Consider “proactive” approach (wherein the UE can request capability restrictions which can be independent of current RRC configuration if allowed by the NW) to MUSIM capability restrictions in addition to the reactive approach (which has been agreed previously). Such a mechanism shall still be under NW control, i.e. it is up to network whether to allow such signalling. FFS on the details – should aim for a common framework for the reactive and proactive approach. FFS on UE capabilities

- Support “early indication” from UE to network during RRC connection setup/resume procedure.

- FFS how to indicate this and in which message. The indication will tell network that UE capabilities are temporarily restricted.

- FFS on details (i.e. when UE can indicate this, what does it indicate, how does it relate to UAI, etc.)

- No consensus to support UE-initiated SCell deactivation for MUSIM in Rel-18.

- 1: For Rel-18 MUSIM dual active operation, the maximum MIMO layer may be changed and the change can be indicated to the NW. FFS if this is only for NW A or also NW B.

- 3: For Rel-18 MUSIM dual active operation, the measurement gap requirement may be changed and the change can be indicated to the NW. FFS if this is only for NW A or also NW B.

- 4: For Rel-18 MUSIM dual active operation, the measurement gap requirement change is reported for each serving cells, and for target bands or all supported NR bands depending on whether target bands are configured by the NW. FFS on whether the reporting can reuse the current needForGapInfoNR in RRC reconfiguration complete or extend the similar function in UAI.  FFS if this is only for NW A or also NW B.

- 8: The maximum UL power may be changed due to Rel-18 MUSIM dual active operation, but there is no need to introduce any new UE behavior for reporting this change.

- 6: UE can explicitly request specific serving cells or serving cell group to be released for Rel-18 MUSIM purpose. FFS how/whether this works for the proactive case.

- 9: RAN2 should avoid duplicating all the capabilities that UE reports via the UECapabilityInformation in the UAI for R18 MUSIM purpose.

- RAN2 can discuss P2, P5 and P7 from R2-2304397 during RAN2#123.

- 1: Introduce 1 optional per-UE capability bit (without xDD/FRx differentiation) to indicate MUSIM gap priority configuration and preference. A UE supporting this feature shall also support musim-GapPreference-r17.

- 2: Introduce a new indication in the OtherConfig to indicate whether UE is allowed to report MUSIM gap priority preference via UAI.

- 4: The existing IE GapPriority-r17 is re-used to configure the priority for periodic MUSIM gap.

- 9: RAN2 assumes no RAN4 impact is expected on maximum UL power change due to R18 MUSIM. Can re-discuss if critical issues are found in RAN2.

RAN2#121 Agreements

- A2a: When the UE is in Connected mode in two NR networks, it is up to the UE implementation to select which NW to perform signaling for temporary UE capability restrictions.

- A2b: When the UE is in Connected mode in NR NW A and moving from Idle/Inactive to connected mode in NR NW B, the signaling for temporary UE capability restrictions can happen on NW A. FFS how to handle if UE is moving from IDLE/INACTIVE in NW A and is in CONNECTED with NW B.

- A2c: When the UE is in Connected mode in both networks and one is E-UTRAN, the signaling for temporary UE capability restrictions happens on the NR network.

- A3: The UE will request a temporary capability restrictions (e.g. via UAI) only after the NW signals via RRC that this is allowed. FFS whether the UE can indicate if it is already connecting with reduced capabilities during connection set-up/resume.

- A4: RAN2 to discuss whether prohibit timer is needed for the signaling of temporary UE capability restrictions This can wait until after progress is made on the signaling framework.

- A1: UAI can be used for the signaling of temporary UE capability changes for dual-active MUSIM. FFS if we have additional signalling (depends on e.g. SCell/SCG deactivation usability for MUSIM)

- A8: For dual-active MUSIM, at least the following type of UE capabilities can be expected to be impacted:

- •	Transmission and reception capabilities (e.g. MIMO layers)

- •	Measurement capabilities (e.g. gaps)

- •	Supported bandwidth

- •	Supported band-combinations (FFS whether this is CA or DC or both)

- FFS what is the granularity of reported temporary UE capability restrictions (also pending the band conflict discussion).

- FFS whether UE reports some or all of the above or whether we can do something simpler

- A6: For dual-active MUSIM, UE signaling will support the request for release (and reversal) of SCells and SCG. The signaling details (e.g. implicit or explicit request of each SCell or SCG) is FFS. FFS if we support deactivation (based on discussion in which case it can be used). It is up to network how to react to UE request.

- RAN2 does not intend to create new procedures for e.g. SCell/SCG deactivation for MUSIM purposes in Rel-18. Existing procedures can be used based on NW choice.

- B4: RAN2 considers the only RAN3 impact may be to support the UE request of SCG/SCell release via SRB3 (if supported) for MUSIM purpose (e.g. cause value).  If this can be done via inter-node messages, RAN2 expects no RAN3 impacts.

- 1: The UE is only allowed to provide MUSIM assistance information for Rel-17 MUSIM gap preference to NR MN and NR MN configures the UE with Re-17 MUSIM gap(s). This requires no specification impacts.

- Use inter-node messages to convey Rel-17 MUSIM gap configuration from MN to SN in NW A when UE is in NR-DC.

- RAN2 confirms that the band conflict scenarios will be covered by the temporary UE capability restrictions. FFS on signalling details.

RAN2#119bis Agreements

- The R18 MUSIM solution should work in DC/CA and RAN sharing scenarios (but need not be optimized for RAN sharing).

- RAN2 aims to address at least the Scenario 1: the UE in network A in RRC_CONNECTED indicates (i.e. adds/removes) its preference on temporary UE capability due start/stop connection in NW B. This can be e.g. CA/DC capability restriction.

- 2	The following is assumed when defining the solution:

- The two networks are independent (i.e. no inter-network communication);

- The Core Network is not aware of the temporary restrictions of the UE capability;

- 1: RAN2 can discuss NW A MN-SN coordination of Rel-18 MUSIM temporary capability restrictions due to UE being configured with NR-DC in NW A.

- RAN2 thinks MN-SN coordination for Rel-17 MUSIM gaps requires WI clarification in RAN

- RAN2 needs to discuss which UE capabilities can be impacted by sharing of resources between the MUSIM links.

- RAN2 aims to prioritize only few solutions and avoid multiple solutions for the same problem (FFS pending on solution details).

- A7: The UE can initiate signaling for UE capability restrictions on NW A if NW A allows it. The specification will not capture NW B events which can cause such need.

- A4: RAN2 to discuss whether the following UE capabilities (not a complete list) are impacted for dual-active MUSIM: MIMO layers, BC capabilities, Measurement capabilities, Bandwidth, srs-TxSwitch, UL tx power, Power Class.

- For proposals A1-A2, the solution details need more discussion. Other solutions are not precluded (requires company input with details). Will discuss further over email on the solutions (after this meeting) and which capabilities can be affected.

- For B1-B3, B5, the solution details need more discussion. May prioritize B1, B2 and B5. FFS on signalling details. Other solutions are not precluded (requires company input with details) and none of B1-B5 are agreed as solutions for this WI.

- Do not consider solution B4 in Rel-18 (since it may have CN impacts which are precluded in this WI)

- CX: RAN2 to continue evaluation of any Xn-AP, F1-AP or RAN4 impact due to dual-active MUSIM operation.

- 1: RAN2 can consider such Band conflict scenarios for MUSIM in CONNECTED to arrive at a graceful specification-based solution intended to mitigate such conflicts.

- Wait for RAN4 feedback on MUSIM gap priority.