3GPP TSG RAN WG2 #124	R2-23xxxxx

Chicago, USA, 13 – 17 Nov, 2023

Source: 	Ericsson

Title: 	[Post123bis][307][NR-NTN Enh] 38.331 running CR (Ericsson)

Agenda Item:	7.6.1

Document for:	Discussion and decision

# Introduction

This document captures the outcome of the following email discussion:

[Post123bis][307][NR-NTN Enh] 38.331 running CR (Ericsson)

Scope: running CR update and list of open issues

Intended outcome:

        - Endorsed running CR

        - List of open issues to be addressed by company Tdocs

Deadline: Medium

*** Detailed scope for all e-mail discussions on running CRs and open issues *** 1. Update the running CR with agreements from the meeting 2. Rapporteur to propose resolutions for straightforward open issues which can already be included in the running CR 3. For Stage 3 running CRs, get input on stage-3 issues that require further input from companies to make a decision: - Focus on stage-3 issues which are better handled via offline, e.g. signaling details, parameter values/ranges, NOT functionality discussion. For these issues, if any, the CR rapporteur should submit a separate report with proposals to the next meeting by the submission deadline, while input via company Tdocs should be avoided 4. Identify the remaining open issues that need to be solved for WI completion in the next meeting: - Company Tdocs for the next meeting should focus on these issues

# 2	Open Issue List

During RAN2#123, the following agreement was taken for RACH-less handover:

- The mapping between type-1 CG and SSBs in CG-SDT can be the baseline of how to configure pre-allocated grant mapped to SSBs (can rediscuss in case of different input from RAN1).

Consequently, the CG-SDT configuration was taken as a baseline to implement the pre-allocated uplink grant (CG type 1) for RACH-less handover. While some parameters such as ssb-Subset or RSRP-ThesholdSSB have been discussed in RAN2, others such as DMRS-Ports needs further clarification.

Editor’s Note: FFS whether the definition of DMRS port configuration and DMRS sequence configuration for NTN RACH-less handover needs revision.

CG-NTN-RACH-less-Configuration field descriptions

ntn-DMRS-Ports Indicates the set of DMRS ports for SSB to PUSCH mapping (see TS 38.213 [13]). The first (left-most / most significant) bit corresponds to DMRS port 0, the second most significant bit corresponds to DMRS port 1, and so on. A bit set to 1 indicates that this DMRS port is used for mapping.

ntn-NrofDMRS-Sequences Indicates the number of DMRS sequences for SSB to PUSCH mapping (see TS 38.213 [13]).

ntn-SSB-Subset Indicates SSB subset for SSB to CG PUSCH mapping within one CG configuration.

ntn-SSB-PerCG-PUSCH The number of SSBs per pre-allocated uplink grant PUSCH (see TS 38.213 [13]). Value one corresponds to 1 SSBs per pre-allocated uplink grant PUSCH, value two corresponds to 2 SSBs per pre-allocated uplink grant PUSCH and so on.

ntn-RSRP-ThresholdSSB An RSRP threshold configured for SSB selection for the pre-allocated uplink grant as specified in TS 38.321 [3].

Q1: Please share your views on whether DMRS port configuration and DMRS sequence configuration for RACH-less handover needs revision:

| Company | Yes / No | Comments |
| --- | --- | --- |
| Ericsson | Yes | These parameters are within RAN1’s scope. |

In addition, during the RRC review, a company raised the question whether the following parameters, which hold a special configuration/restrictions for CG-SDT are equally applicable for NTN RACH-less handover.

ConfiguredGrantConfig field descriptions

antennaPort Indicates the antenna port(s) to be used for this configuration, and the maximum bitwidth is 5. See TS 38.214 [19], clause 6.1.2, and TS 38.212 [17], clause 7.3.1. The UE ignores this field in case of CG-SDT.

cg-RetransmissionTimer Indicates the initial value of the configured retransmission timer (see TS 38.321 [3]) in multiples of periodicity. The value of cg-RetransmissionTimer is always less than or equal to the value of configuredGrantTimer. This field is always configured together with harq-ProcID-Offset. This field is not configured for operation in licensed spectrum or simultaneously with harq-ProcID-Offset2. The network does not configure this field for CG-SDT.

harq-ProcID-Offset For operation with shared spectrum channel access configured with cg-RetransmissionTimer-r16, this configures the range of HARQ process IDs which can be used for this configured grant where the UE can select a HARQ process ID within [harq-procID-offset, .., (harq-procID-offset + nrofHARQ-Processes – 1)]. harq-ProcID-Offset-v1730 is only applicable for operation with shared spectrum channel access in FR2-2. If the field harq-ProcID-Offset-v1730 is present, the UE shall ignore the harq-ProcID-Offset-r16. The network does not configure this field for CG-SDT.

pathlossReferenceIndex Indicates the reference signal index used as PUSCH pathloss reference (see TS 38.213 [13], clause 7.1.1). In case of CG-SDT, the UE does not use this field.

phy-PriorityIndex Indicates the PHY priority of CG PUSCH at least for PHY-layer collision handling. Value p0 indicates low priority and value p1 indicates high priority. The network does not configure this for CG-SDT.

precodingAndNumberOfLayers Indicates the precoding and number of layers (see TS 38.212 [17], clause 7.3.1.1.2, and TS 38.214 [19], clause 6.1.2.3). In case of CG-SDT, network sets this field to 1.

srs-ResourceIndicator Indicates the SRS resource to be used. The network does not configure this for CG-SDT.

uci-OnPUSCH Selection between and configuration of dynamic and semi-static beta-offset. For Type 1 UL data transmission without grant, uci-OnPUSCH should be set to semiStatic. The network does not configure this for CG-SDT.

Q2: Please share your views on whether the following parameters in the ConfiguredGrantConfig IE which have a special description for CG-SDT need revision for NTN RACH-less handover:

| Company | Yes / No | Comments |
| --- | --- | --- |
| Ericsson | Yes | These parameters are within RAN1’s scope. |

Q3: Do you think anything essential for WI completion is missed? Note that unchanged PCI functionality is discussed in a separate offline (312). Please elaborate on the missing issues if any.

| Company | Open issue | Comments |
| --- | --- | --- |
| Ericsson | In NTN RACH-less handover, how network indicates the beam for the monitoring of PDCCH in dynamic grant case | From the RRC review, rapporteur has proposed to use TCI state Id (which contains SSB indexes) to provide the beam. A few companies are unsure of how this would work. Others prefer SSB index. The issue would be better handled with company contributions next meeting. |

# 3	Conclusion

To be completed