3GPP TSG-RAN WG2 Meeting #124	R2-23xxxxx

# Chicago, USA, 13 – 17 November, 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.331 | CR | XXXX | rev | - | Current version: | 17.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Running CR 36331 for UE capability for R18 SONMDT |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon, CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_ENDC_SON_MDT_enh2-Core |  |  |  |  |  | Date: |  |  | 2023-09-29 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Introduction of R18 features on SON and MDT. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Based on RAN2#123bis agreements, R18 UE capabilities for SON and MDT are defined, which includes: 2: Introduce an optional feature without signalling for LTE RLF report for voice fallback in LTE. This feature is for the case an RLF occurs shortly after successful HO from NR to E-UTRAN for voice fallback. 9: Introduce a new UE capability bit (optional with signalling) for RACH report about NR RACH Report in LTE. This bit indicates whether the UE supports NR RACH report in LTE, upon request from the network. 12: Introduce a new UE capability bit (optional with signalling) for signalling based logged MDT override protection in LTE. This bit indicates whether the UE supports the override protection of the signalling based logged measurements configured in E-UTRA when going to NR. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | No R18 UE capabilities for SON and MDT are defined. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.3.6 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS 36.306 CRxxxx |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

6.3.6	Other information elements

<Partially omitted>

–	UE-EUTRA-Capability

The IE UE-EUTRA-Capability is used to convey the E-UTRA UE Radio Access Capability Parameters, see TS 36.306 [5], and the Feature Group Indicators for mandatory features (defined in Annexes B.1 and C.1) to the network. The IE UE-EUTRA-Capability is transferred in E-UTRA or in another RAT.

NOTE 0:	For (UE capability specific) guidelines on the use of keyword OPTIONAL, see Annex A.3.5.

UE-EUTRA-Capability information element

-- ASN1START

UE-EUTRA-Capability ::=			SEQUENCE {

accessStratumRelease			AccessStratumRelease,

ue-Category						INTEGER (1..5),

pdcp-Parameters					PDCP-Parameters,

phyLayerParameters				PhyLayerParameters,

rf-Parameters					RF-Parameters,

measParameters					MeasParameters,

featureGroupIndicators			BIT STRING (SIZE (32))					OPTIONAL,

interRAT-Parameters				SEQUENCE {

utraFDD							IRAT-ParametersUTRA-FDD				OPTIONAL,

utraTDD128						IRAT-ParametersUTRA-TDD128			OPTIONAL,

utraTDD384						IRAT-ParametersUTRA-TDD384			OPTIONAL,

utraTDD768						IRAT-ParametersUTRA-TDD768			OPTIONAL,

geran							IRAT-ParametersGERAN				OPTIONAL,

cdma2000-HRPD					IRAT-ParametersCDMA2000-HRPD		OPTIONAL,

cdma2000-1xRTT					IRAT-ParametersCDMA2000-1XRTT		OPTIONAL

},

nonCriticalExtension			UE-EUTRA-Capability-v920-IEs			OPTIONAL

}

-- Late non critical extensions

UE-EUTRA-Capability-v9a0-IEs ::=	SEQUENCE {

featureGroupIndRel9Add-r9			BIT STRING (SIZE (32))				OPTIONAL,

fdd-Add-UE-EUTRA-Capabilities-r9	UE-EUTRA-CapabilityAddXDD-Mode-r9	OPTIONAL,

tdd-Add-UE-EUTRA-Capabilities-r9	UE-EUTRA-CapabilityAddXDD-Mode-r9	OPTIONAL,

nonCriticalExtension				UE-EUTRA-Capability-v9c0-IEs		OPTIONAL

}

UE-EUTRA-Capability-v9c0-IEs ::=	SEQUENCE {

interRAT-ParametersUTRA-v9c0		IRAT-ParametersUTRA-v9c0		OPTIONAL,

nonCriticalExtension				UE-EUTRA-Capability-v9d0-IEs	OPTIONAL

}

UE-EUTRA-Capability-v9d0-IEs ::=	SEQUENCE {

phyLayerParameters-v9d0				PhyLayerParameters-v9d0			OPTIONAL,

nonCriticalExtension				UE-EUTRA-Capability-v9e0-IEs	OPTIONAL

}

UE-EUTRA-Capability-v9e0-IEs ::=	SEQUENCE {

rf-Parameters-v9e0					RF-Parameters-v9e0						OPTIONAL,

nonCriticalExtension				UE-EUTRA-Capability-v9h0-IEs			OPTIONAL

}

UE-EUTRA-Capability-v9h0-IEs ::=	SEQUENCE {

interRAT-ParametersUTRA-v9h0		IRAT-ParametersUTRA-v9h0				OPTIONAL,

-- Following field is only to be used for late REL-9 extensions

lateNonCriticalExtension			OCTET STRING							OPTIONAL,

nonCriticalExtension				UE-EUTRA-Capability-v10c0-IEs			OPTIONAL

}

UE-EUTRA-Capability-v10c0-IEs ::=	SEQUENCE {

otdoa-PositioningCapabilities-r10	OTDOA-PositioningCapabilities-r10		OPTIONAL,

nonCriticalExtension				UE-EUTRA-Capability-v10f0-IEs			OPTIONAL

}

UE-EUTRA-Capability-v10f0-IEs ::=	SEQUENCE {

rf-Parameters-v10f0					RF-Parameters-v10f0						OPTIONAL,

nonCriticalExtension				UE-EUTRA-Capability-v10i0-IEs			OPTIONAL

}

UE-EUTRA-Capability-v10i0-IEs ::=	SEQUENCE {

rf-Parameters-v10i0					RF-Parameters-v10i0						OPTIONAL,

-- Following field is only to be used for late REL-10 extensions

lateNonCriticalExtension			OCTET STRING (CONTAINING UE-EUTRA-Capability-v10j0-IEs)	OPTIONAL,

nonCriticalExtension				UE-EUTRA-Capability-v11d0-IEs			OPTIONAL

}

UE-EUTRA-Capability-v10j0-IEs ::=	SEQUENCE {

rf-Parameters-v10j0					RF-Parameters-v10j0						OPTIONAL,

nonCriticalExtension				SEQUENCE {}								OPTIONAL

}

UE-EUTRA-Capability-v11d0-IEs ::=	SEQUENCE {

rf-Parameters-v11d0					RF-Parameters-v11d0						OPTIONAL,

otherParameters-v11d0				Other-Parameters-v11d0					OPTIONAL,

nonCriticalExtension				UE-EUTRA-Capability-v11x0-IEs			OPTIONAL

}

UE-EUTRA-Capability-v11x0-IEs ::=	SEQUENCE {

-- Following field is only to be used for late REL-11 extensions

lateNonCriticalExtension			OCTET STRING								OPTIONAL,

nonCriticalExtension				UE-EUTRA-Capability-v12b0-IEs				OPTIONAL

}

UE-EUTRA-Capability-v12b0-IEs ::= SEQUENCE {

rf-Parameters-v12b0					RF-Parameters-v12b0						OPTIONAL,

nonCriticalExtension				UE-EUTRA-Capability-v12x0-IEs			OPTIONAL

}

UE-EUTRA-Capability-v12x0-IEs ::= SEQUENCE {

-- Following field is only to be used for late REL-12 extensions

lateNonCriticalExtension			OCTET STRING							OPTIONAL,

nonCriticalExtension				UE-EUTRA-Capability-v1370-IEs			OPTIONAL

}

UE-EUTRA-Capability-v1370-IEs ::= SEQUENCE {

ce-Parameters-v1370					CE-Parameters-v1370						OPTIONAL,

fdd-Add-UE-EUTRA-Capabilities-v1370	UE-EUTRA-CapabilityAddXDD-Mode-v1370	OPTIONAL,

tdd-Add-UE-EUTRA-Capabilities-v1370	UE-EUTRA-CapabilityAddXDD-Mode-v1370	OPTIONAL,

nonCriticalExtension				UE-EUTRA-Capability-v1380-IEs			OPTIONAL

}

UE-EUTRA-Capability-v1380-IEs ::= SEQUENCE {

rf-Parameters-v1380					RF-Parameters-v1380						OPTIONAL,

ce-Parameters-v1380					CE-Parameters-v1380,

fdd-Add-UE-EUTRA-Capabilities-v1380	UE-EUTRA-CapabilityAddXDD-Mode-v1380,

tdd-Add-UE-EUTRA-Capabilities-v1380	UE-EUTRA-CapabilityAddXDD-Mode-v1380,

nonCriticalExtension				UE-EUTRA-Capability-v1390-IEs			OPTIONAL

}

UE-EUTRA-Capability-v1390-IEs ::= SEQUENCE {

rf-Parameters-v1390					RF-Parameters-v1390						OPTIONAL,

nonCriticalExtension				UE-EUTRA-Capability-v13e0a-IEs			OPTIONAL

}

UE-EUTRA-Capability-v13e0a-IEs ::= SEQUENCE {

lateNonCriticalExtension			OCTET STRING (CONTAINING UE-EUTRA-Capability-v13e0b-IEs)							OPTIONAL,

nonCriticalExtension				UE-EUTRA-Capability-v1470-IEs			OPTIONAL

}

UE-EUTRA-Capability-v13e0b-IEs ::= SEQUENCE {

phyLayerParameters-v13e0			PhyLayerParameters-v13e0,

-- Following field is only to be used for late REL-13 extensions

nonCriticalExtension				SEQUENCE {}								OPTIONAL

}

UE-EUTRA-Capability-v1470-IEs ::= SEQUENCE {

mbms-Parameters-v1470				MBMS-Parameters-v1470					OPTIONAL,

phyLayerParameters-v1470			PhyLayerParameters-v1470				OPTIONAL,

rf-Parameters-v1470					RF-Parameters-v1470						OPTIONAL,

nonCriticalExtension				UE-EUTRA-Capability-v14a0-IEs			OPTIONAL

}

UE-EUTRA-Capability-v14a0-IEs ::= SEQUENCE {

phyLayerParameters-v14a0				PhyLayerParameters-v14a0,

nonCriticalExtension					UE-EUTRA-Capability-v14b0-IEs			OPTIONAL

}

UE-EUTRA-Capability-v14b0-IEs ::= SEQUENCE {

rf-Parameters-v14b0				RF-Parameters-v14b0				OPTIONAL,

nonCriticalExtension				UE-EUTRA-Capability-v14x0-IEs		OPTIONAL

}

UE-EUTRA-Capability-v14x0-IEs ::= SEQUENCE {

-- Following field is only to be used for late REL-14 extensions

lateNonCriticalExtension			OCTET STRING							OPTIONAL,

nonCriticalExtension				UE-EUTRA-Capability-v15x0-IEs			OPTIONAL

}

UE-EUTRA-Capability-v15x0-IEs ::= SEQUENCE {

-- Following field is only to be used for late REL-15 extensions

lateNonCriticalExtension			OCTET STRING							OPTIONAL,

nonCriticalExtension				UE-EUTRA-Capability-v16c0-IEs			OPTIONAL

}

UE-EUTRA-Capability-v16c0-IEs ::= SEQUENCE {

measParameters-v16c0 				MeasParameters-v16c0,

-- Following field is only to be used for late REL-16 extensions

lateNonCriticalExtension			OCTET STRING							OPTIONAL,

nonCriticalExtension				SEQUENCE {}							OPTIONAL

}

-- Regular non critical extensions

UE-EUTRA-Capability-v920-IEs ::=		SEQUENCE {

phyLayerParameters-v920					PhyLayerParameters-v920,

interRAT-ParametersGERAN-v920			IRAT-ParametersGERAN-v920,

interRAT-ParametersUTRA-v920			IRAT-ParametersUTRA-v920			OPTIONAL,

interRAT-ParametersCDMA2000-v920		IRAT-ParametersCDMA2000-1XRTT-v920	OPTIONAL,

deviceType-r9							ENUMERATED {noBenFromBatConsumpOpt}	OPTIONAL,

csg-ProximityIndicationParameters-r9	CSG-ProximityIndicationParameters-r9,

neighCellSI-AcquisitionParameters-r9	NeighCellSI-AcquisitionParameters-r9,

son-Parameters-r9						SON-Parameters-r9,

nonCriticalExtension					UE-EUTRA-Capability-v940-IEs		OPTIONAL

}

UE-EUTRA-Capability-v940-IEs ::=	SEQUENCE {

lateNonCriticalExtension			OCTET STRING (CONTAINING UE-EUTRA-Capability-v9a0-IEs)			OPTIONAL,

nonCriticalExtension				UE-EUTRA-Capability-v1020-IEs			OPTIONAL

}

UE-EUTRA-Capability-v1020-IEs ::=	SEQUENCE {

ue-Category-v1020					INTEGER (6..8)							OPTIONAL,

phyLayerParameters-v1020			PhyLayerParameters-v1020				OPTIONAL,

rf-Parameters-v1020					RF-Parameters-v1020						OPTIONAL,

measParameters-v1020				MeasParameters-v1020					OPTIONAL,

featureGroupIndRel10-r10			BIT STRING (SIZE (32))					OPTIONAL,

interRAT-ParametersCDMA2000-v1020	IRAT-ParametersCDMA2000-1XRTT-v1020		OPTIONAL,

ue-BasedNetwPerfMeasParameters-r10	UE-BasedNetwPerfMeasParameters-r10		OPTIONAL,

interRAT-ParametersUTRA-TDD-v1020	IRAT-ParametersUTRA-TDD-v1020			OPTIONAL,

nonCriticalExtension				UE-EUTRA-Capability-v1060-IEs			OPTIONAL

}

UE-EUTRA-Capability-v1060-IEs ::=	SEQUENCE {

fdd-Add-UE-EUTRA-Capabilities-v1060	UE-EUTRA-CapabilityAddXDD-Mode-v1060	OPTIONAL,

tdd-Add-UE-EUTRA-Capabilities-v1060	UE-EUTRA-CapabilityAddXDD-Mode-v1060	OPTIONAL,

rf-Parameters-v1060					RF-Parameters-v1060						OPTIONAL,

nonCriticalExtension				UE-EUTRA-Capability-v1090-IEs			OPTIONAL

}

UE-EUTRA-Capability-v1090-IEs ::=	SEQUENCE {

rf-Parameters-v1090					RF-Parameters-v1090						OPTIONAL,

nonCriticalExtension				UE-EUTRA-Capability-v1130-IEs			OPTIONAL

}

UE-EUTRA-Capability-v1130-IEs ::=	SEQUENCE {

pdcp-Parameters-v1130				PDCP-Parameters-v1130,

phyLayerParameters-v1130			PhyLayerParameters-v1130				OPTIONAL,

rf-Parameters-v1130					RF-Parameters-v1130,

measParameters-v1130				MeasParameters-v1130,

interRAT-ParametersCDMA2000-v1130	IRAT-ParametersCDMA2000-v1130,

otherParameters-r11					Other-Parameters-r11,

fdd-Add-UE-EUTRA-Capabilities-v1130	UE-EUTRA-CapabilityAddXDD-Mode-v1130	OPTIONAL,

tdd-Add-UE-EUTRA-Capabilities-v1130	UE-EUTRA-CapabilityAddXDD-Mode-v1130	OPTIONAL,

nonCriticalExtension				UE-EUTRA-Capability-v1170-IEs			OPTIONAL

}

UE-EUTRA-Capability-v1170-IEs ::=	SEQUENCE {

phyLayerParameters-v1170			PhyLayerParameters-v1170				OPTIONAL,

ue-Category-v1170					INTEGER (9..10)							OPTIONAL,

nonCriticalExtension				UE-EUTRA-Capability-v1180-IEs			OPTIONAL

}

UE-EUTRA-Capability-v1180-IEs ::=	SEQUENCE {

rf-Parameters-v1180					RF-Parameters-v1180						OPTIONAL,

mbms-Parameters-r11					MBMS-Parameters-r11						OPTIONAL,

fdd-Add-UE-EUTRA-Capabilities-v1180	UE-EUTRA-CapabilityAddXDD-Mode-v1180	OPTIONAL,

tdd-Add-UE-EUTRA-Capabilities-v1180	UE-EUTRA-CapabilityAddXDD-Mode-v1180	OPTIONAL,

nonCriticalExtension				UE-EUTRA-Capability-v11a0-IEs			OPTIONAL

}

UE-EUTRA-Capability-v11a0-IEs ::=	SEQUENCE {

ue-Category-v11a0					INTEGER (11..12)						OPTIONAL,

measParameters-v11a0				MeasParameters-v11a0					OPTIONAL,

nonCriticalExtension				UE-EUTRA-Capability-v1250-IEs			OPTIONAL

}

UE-EUTRA-Capability-v1250-IEs ::=	SEQUENCE {

phyLayerParameters-v1250				PhyLayerParameters-v1250				OPTIONAL,

rf-Parameters-v1250						RF-Parameters-v1250						OPTIONAL,

rlc-Parameters-r12						RLC-Parameters-r12						OPTIONAL,

ue-BasedNetwPerfMeasParameters-v1250	UE-BasedNetwPerfMeasParameters-v1250	OPTIONAL,

ue-CategoryDL-r12						INTEGER (0..14)							OPTIONAL,

ue-CategoryUL-r12						INTEGER (0..13)							OPTIONAL,

wlan-IW-Parameters-r12					WLAN-IW-Parameters-r12					OPTIONAL,

measParameters-v1250					MeasParameters-v1250					OPTIONAL,

dc-Parameters-r12						DC-Parameters-r12						OPTIONAL,

mbms-Parameters-v1250					MBMS-Parameters-v1250					OPTIONAL,

mac-Parameters-r12						MAC-Parameters-r12						OPTIONAL,

fdd-Add-UE-EUTRA-Capabilities-v1250		UE-EUTRA-CapabilityAddXDD-Mode-v1250	OPTIONAL,

tdd-Add-UE-EUTRA-Capabilities-v1250		UE-EUTRA-CapabilityAddXDD-Mode-v1250	OPTIONAL,

sl-Parameters-r12						SL-Parameters-r12						OPTIONAL,

nonCriticalExtension					UE-EUTRA-Capability-v1260-IEs			OPTIONAL

}

UE-EUTRA-Capability-v1260-IEs ::=	SEQUENCE {

ue-CategoryDL-v1260					INTEGER (15..16)						OPTIONAL,

nonCriticalExtension				UE-EUTRA-Capability-v1270-IEs			OPTIONAL

}

UE-EUTRA-Capability-v1270-IEs ::= SEQUENCE {

rf-Parameters-v1270					RF-Parameters-v1270						OPTIONAL,

nonCriticalExtension				UE-EUTRA-Capability-v1280-IEs			OPTIONAL

}

UE-EUTRA-Capability-v1280-IEs ::= SEQUENCE {

phyLayerParameters-v1280			PhyLayerParameters-v1280				OPTIONAL,

nonCriticalExtension				UE-EUTRA-Capability-v1310-IEs			OPTIONAL

}

UE-EUTRA-Capability-v1310-IEs ::= SEQUENCE {

ue-CategoryDL-v1310					ENUMERATED {n17, m1}					OPTIONAL,

ue-CategoryUL-v1310					ENUMERATED {n14, m1}					OPTIONAL,

pdcp-Parameters-v1310				PDCP-Parameters-v1310,

rlc-Parameters-v1310				RLC-Parameters-v1310,

mac-Parameters-v1310				MAC-Parameters-v1310					OPTIONAL,

phyLayerParameters-v1310			PhyLayerParameters-v1310				OPTIONAL,

rf-Parameters-v1310					RF-Parameters-v1310						OPTIONAL,

measParameters-v1310				MeasParameters-v1310					OPTIONAL,

dc-Parameters-v1310					DC-Parameters-v1310						OPTIONAL,

sl-Parameters-v1310					SL-Parameters-v1310						OPTIONAL,

scptm-Parameters-r13				SCPTM-Parameters-r13					OPTIONAL,

ce-Parameters-r13					CE-Parameters-r13						OPTIONAL,

interRAT-ParametersWLAN-r13			IRAT-ParametersWLAN-r13,

laa-Parameters-r13					LAA-Parameters-r13						OPTIONAL,

lwa-Parameters-r13					LWA-Parameters-r13						OPTIONAL,

wlan-IW-Parameters-v1310			WLAN-IW-Parameters-v1310,

lwip-Parameters-r13					LWIP-Parameters-r13,

fdd-Add-UE-EUTRA-Capabilities-v1310	UE-EUTRA-CapabilityAddXDD-Mode-v1310	OPTIONAL,

tdd-Add-UE-EUTRA-Capabilities-v1310	UE-EUTRA-CapabilityAddXDD-Mode-v1310	OPTIONAL,

nonCriticalExtension				UE-EUTRA-Capability-v1320-IEs			OPTIONAL

}

UE-EUTRA-Capability-v1320-IEs ::= SEQUENCE {

ce-Parameters-v1320					CE-Parameters-v1320						OPTIONAL,

phyLayerParameters-v1320			PhyLayerParameters-v1320				OPTIONAL,

rf-Parameters-v1320					RF-Parameters-v1320						OPTIONAL,

fdd-Add-UE-EUTRA-Capabilities-v1320	UE-EUTRA-CapabilityAddXDD-Mode-v1320	OPTIONAL,

tdd-Add-UE-EUTRA-Capabilities-v1320	UE-EUTRA-CapabilityAddXDD-Mode-v1320	OPTIONAL,

nonCriticalExtension				UE-EUTRA-Capability-v1330-IEs			OPTIONAL

}

UE-EUTRA-Capability-v1330-IEs ::= SEQUENCE {

ue-CategoryDL-v1330					INTEGER (18..19)						OPTIONAL,

phyLayerParameters-v1330			PhyLayerParameters-v1330				OPTIONAL,

ue-CE-NeedULGaps-r13				ENUMERATED {true}						OPTIONAL,

nonCriticalExtension				UE-EUTRA-Capability-v1340-IEs			OPTIONAL

}

UE-EUTRA-Capability-v1340-IEs ::= SEQUENCE {

ue-CategoryUL-v1340					INTEGER (15)							OPTIONAL,

nonCriticalExtension				UE-EUTRA-Capability-v1350-IEs			OPTIONAL

}

UE-EUTRA-Capability-v1350-IEs ::= SEQUENCE {

ue-CategoryDL-v1350					ENUMERATED {oneBis}						OPTIONAL,

ue-CategoryUL-v1350					ENUMERATED {oneBis}						OPTIONAL,

ce-Parameters-v1350					CE-Parameters-v1350,

nonCriticalExtension				UE-EUTRA-Capability-v1360-IEs			OPTIONAL

}

UE-EUTRA-Capability-v1360-IEs ::= SEQUENCE {

other-Parameters-v1360				Other-Parameters-v1360					OPTIONAL,

nonCriticalExtension				UE-EUTRA-Capability-v1430-IEs			OPTIONAL

}

UE-EUTRA-Capability-v1430-IEs ::= SEQUENCE {

phyLayerParameters-v1430			PhyLayerParameters-v1430,

ue-CategoryDL-v1430					ENUMERATED {m2}								OPTIONAL,

ue-CategoryUL-v1430					ENUMERATED {n16, n17, n18, n19, n20, m2}	OPTIONAL,

ue-CategoryUL-v1430b				ENUMERATED {n21}							OPTIONAL,

mac-Parameters-v1430				MAC-Parameters-v1430						OPTIONAL,

measParameters-v1430				MeasParameters-v1430						OPTIONAL,

pdcp-Parameters-v1430				PDCP-Parameters-v1430						OPTIONAL,

rlc-Parameters-v1430				RLC-Parameters-v1430,

rf-Parameters-v1430					RF-Parameters-v1430							OPTIONAL,

laa-Parameters-v1430				LAA-Parameters-v1430						OPTIONAL,

lwa-Parameters-v1430				LWA-Parameters-v1430						OPTIONAL,

lwip-Parameters-v1430				LWIP-Parameters-v1430						OPTIONAL,

otherParameters-v1430				Other-Parameters-v1430,

mmtel-Parameters-r14				MMTEL-Parameters-r14						OPTIONAL,

mobilityParameters-r14				MobilityParameters-r14						OPTIONAL,

ce-Parameters-v1430					CE-Parameters-v1430,

fdd-Add-UE-EUTRA-Capabilities-v1430	UE-EUTRA-CapabilityAddXDD-Mode-v1430		OPTIONAL,

tdd-Add-UE-EUTRA-Capabilities-v1430	UE-EUTRA-CapabilityAddXDD-Mode-v1430		OPTIONAL,

mbms-Parameters-v1430				MBMS-Parameters-v1430						OPTIONAL,

sl-Parameters-v1430					SL-Parameters-v1430							OPTIONAL,

ue-BasedNetwPerfMeasParameters-v1430	UE-BasedNetwPerfMeasParameters-v1430	OPTIONAL,

highSpeedEnhParameters-r14			HighSpeedEnhParameters-r14					OPTIONAL,

nonCriticalExtension				UE-EUTRA-Capability-v1440-IEs				OPTIONAL

}

UE-EUTRA-Capability-v1440-IEs ::= SEQUENCE {

lwa-Parameters-v1440				LWA-Parameters-v1440,

mac-Parameters-v1440				MAC-Parameters-v1440,

nonCriticalExtension				UE-EUTRA-Capability-v1450-IEs			OPTIONAL

}

UE-EUTRA-Capability-v1450-IEs ::= SEQUENCE {

phyLayerParameters-v1450			PhyLayerParameters-v1450		OPTIONAL,

rf-Parameters-v1450					RF-Parameters-v1450			OPTIONAL,

otherParameters-v1450				OtherParameters-v1450,

ue-CategoryDL-v1450					INTEGER (20)					OPTIONAL,

nonCriticalExtension					UE-EUTRA-Capability-v1460-IEs	OPTIONAL

}

UE-EUTRA-Capability-v1460-IEs ::= SEQUENCE {

ue-CategoryDL-v1460				INTEGER (21)							OPTIONAL,

otherParameters-v1460				Other-Parameters-v1460,

nonCriticalExtension				UE-EUTRA-Capability-v1510-IEs		OPTIONAL

}

UE-EUTRA-Capability-v1510-IEs ::= SEQUENCE {

irat-ParametersNR-r15					IRAT-ParametersNR-r15					OPTIONAL,

featureSetsEUTRA-r15					FeatureSetsEUTRA-r15					OPTIONAL,

pdcp-ParametersNR-r15					PDCP-ParametersNR-r15					OPTIONAL,

fdd-Add-UE-EUTRA-Capabilities-v1510		UE-EUTRA-CapabilityAddXDD-Mode-v1510	OPTIONAL,

tdd-Add-UE-EUTRA-Capabilities-v1510		UE-EUTRA-CapabilityAddXDD-Mode-v1510	OPTIONAL,

nonCriticalExtension					UE-EUTRA-Capability-v1520-IEs			OPTIONAL

}

UE-EUTRA-Capability-v1520-IEs ::= SEQUENCE {

measParameters-v1520					MeasParameters-v1520,

nonCriticalExtension					UE-EUTRA-Capability-v1530-IEs	OPTIONAL

}

UE-EUTRA-Capability-v1530-IEs ::= SEQUENCE {

measParameters-v1530					MeasParameters-v1530					OPTIONAL,

otherParameters-v1530					Other-Parameters-v1530					OPTIONAL,

neighCellSI-AcquisitionParameters-v1530	NeighCellSI-AcquisitionParameters-v1530	OPTIONAL,

mac-Parameters-v1530					MAC-Parameters-v1530					OPTIONAL,

phyLayerParameters-v1530				PhyLayerParameters-v1530				OPTIONAL,

rf-Parameters-v1530						RF-Parameters-v1530						OPTIONAL,

pdcp-Parameters-v1530					PDCP-Parameters-v1530					OPTIONAL,

ue-CategoryDL-v1530						INTEGER (22..26)						OPTIONAL,

ue-BasedNetwPerfMeasParameters-v1530	UE-BasedNetwPerfMeasParameters-v1530	OPTIONAL,

rlc-Parameters-v1530					RLC-Parameters-v1530					OPTIONAL,

sl-Parameters-v1530						SL-Parameters-v1530						OPTIONAL,

extendedNumberOfDRBs-r15				ENUMERATED {supported}					OPTIONAL,

reducedCP-Latency-r15					ENUMERATED {supported}					OPTIONAL,

laa-Parameters-v1530					LAA-Parameters-v1530					OPTIONAL,

ue-CategoryUL-v1530						INTEGER (22..26)						OPTIONAL,

fdd-Add-UE-EUTRA-Capabilities-v1530		UE-EUTRA-CapabilityAddXDD-Mode-v1530	OPTIONAL,

tdd-Add-UE-EUTRA-Capabilities-v1530		UE-EUTRA-CapabilityAddXDD-Mode-v1530	OPTIONAL,

nonCriticalExtension					UE-EUTRA-Capability-v1540-IEs			OPTIONAL

}

UE-EUTRA-Capability-v1540-IEs ::= SEQUENCE {

phyLayerParameters-v1540				PhyLayerParameters-v1540				OPTIONAL,

otherParameters-v1540					Other-Parameters-v1540,

fdd-Add-UE-EUTRA-Capabilities-v1540		UE-EUTRA-CapabilityAddXDD-Mode-v1540	OPTIONAL,

tdd-Add-UE-EUTRA-Capabilities-v1540		UE-EUTRA-CapabilityAddXDD-Mode-v1540	OPTIONAL,

sl-Parameters-v1540						SL-Parameters-v1540						OPTIONAL,

irat-ParametersNR-v1540					IRAT-ParametersNR-v1540					OPTIONAL,

nonCriticalExtension					UE-EUTRA-Capability-v1550-IEs			OPTIONAL

}

UE-EUTRA-Capability-v1550-IEs ::= SEQUENCE {

neighCellSI-AcquisitionParameters-v1550	NeighCellSI-AcquisitionParameters-v1550	OPTIONAL,

phyLayerParameters-v1550				PhyLayerParameters-v1550,

mac-Parameters-v1550					MAC-Parameters-v1550,

fdd-Add-UE-EUTRA-Capabilities-v1550		UE-EUTRA-CapabilityAddXDD-Mode-v1550,

tdd-Add-UE-EUTRA-Capabilities-v1550		UE-EUTRA-CapabilityAddXDD-Mode-v1550,

nonCriticalExtension					UE-EUTRA-Capability-v1560-IEs	OPTIONAL

}

UE-EUTRA-Capability-v1560-IEs ::= SEQUENCE {

pdcp-ParametersNR-v1560				PDCP-ParametersNR-v1560,

irat-ParametersNR-v1560				IRAT-ParametersNR-v1560,

appliedCapabilityFilterCommon-r15		OCTET STRING							OPTIONAL,

fdd-Add-UE-EUTRA-Capabilities-v1560	UE-EUTRA-CapabilityAddXDD-Mode-v1560,

tdd-Add-UE-EUTRA-Capabilities-v1560	UE-EUTRA-CapabilityAddXDD-Mode-v1560,

nonCriticalExtension					UE-EUTRA-Capability-v1570-IEs			OPTIONAL

}

UE-EUTRA-Capability-v1570-IEs ::= SEQUENCE {

rf-Parameters-v1570				RF-Parameters-v1570					OPTIONAL,

irat-ParametersNR-v1570			IRAT-ParametersNR-v1570				OPTIONAL,

nonCriticalExtension				UE-EUTRA-Capability-v15a0-IEs			OPTIONAL

}

UE-EUTRA-Capability-v15a0-IEs ::= SEQUENCE {

neighCellSI-AcquisitionParameters-v15a0	NeighCellSI-AcquisitionParameters-v15a0,

eutra-5GC-Parameters-r15				EUTRA-5GC-Parameters-r15				OPTIONAL,

fdd-Add-UE-EUTRA-Capabilities-v15a0	UE-EUTRA-CapabilityAddXDD-Mode-v15a0	OPTIONAL,

tdd-Add-UE-EUTRA-Capabilities-v15a0	UE-EUTRA-CapabilityAddXDD-Mode-v15a0	OPTIONAL,

nonCriticalExtension				UE-EUTRA-Capability-v1610-IEs			OPTIONAL

}

UE-EUTRA-Capability-v1610-IEs ::= SEQUENCE {

highSpeedEnhParameters-v1610			HighSpeedEnhParameters-v1610				OPTIONAL,

neighCellSI-AcquisitionParameters-v1610	NeighCellSI-AcquisitionParameters-v1610		OPTIONAL,

mbms-Parameters-v1610					MBMS-Parameters-v1610						OPTIONAL,

pdcp-Parameters-v1610					PDCP-Parameters-v1610						OPTIONAL,

mac-Parameters-v1610					MAC-Parameters-v1610						OPTIONAL,

phyLayerParameters-v1610				PhyLayerParameters-v1610					OPTIONAL,

measParameters-v1610					MeasParameters-v1610						OPTIONAL,

pur-Parameters-r16						PUR-Parameters-r16							OPTIONAL,

eutra-5GC-Parameters-v1610				EUTRA-5GC-Parameters-v1610					OPTIONAL,

otherParameters-v1610					Other-Parameters-v1610						OPTIONAL,

dl-DedicatedMessageSegmentation-r16		ENUMERATED {supported}						OPTIONAL,

mmtel-Parameters-v1610					MMTEL-Parameters-v1610,

irat-ParametersNR-v1610					IRAT-ParametersNR-v1610						OPTIONAL,

rf-Parameters-v1610						RF-Parameters-v1610							OPTIONAL,

mobilityParameters-v1610				MobilityParameters-v1610					OPTIONAL,

ue-BasedNetwPerfMeasParameters-v1610	UE-BasedNetwPerfMeasParameters-v1610,

sl-Parameters-v1610						SL-Parameters-v1610							OPTIONAL,

fdd-Add-UE-EUTRA-Capabilities-v1610		UE-EUTRA-CapabilityAddXDD-Mode-v1610		OPTIONAL,

tdd-Add-UE-EUTRA-Capabilities-v1610		UE-EUTRA-CapabilityAddXDD-Mode-v1610		OPTIONAL,

nonCriticalExtension					UE-EUTRA-Capability-v1630-IEs				OPTIONAL

}

UE-EUTRA-Capability-v1630-IEs ::= SEQUENCE {

rf-Parameters-v1630						RF-Parameters-v1630							OPTIONAL,

sl-Parameters-v1630						SL-Parameters-v1630							OPTIONAL,

earlySecurityReactivation-r16			ENUMERATED {supported}					OPTIONAL,

mac-Parameters-v1630					MAC-Parameters-v1630,

measParameters-v1630					MeasParameters-v1630						OPTIONAL,

fdd-Add-UE-EUTRA-Capabilities-v1630		UE-EUTRA-CapabilityAddXDD-Mode-v1630,

tdd-Add-UE-EUTRA-Capabilities-v1630		UE-EUTRA-CapabilityAddXDD-Mode-v1630,

nonCriticalExtension					UE-EUTRA-Capability-v1650-IEs		OPTIONAL

}

UE-EUTRA-Capability-v1650-IEs ::= SEQUENCE {

otherParameters-v1650				Other-Parameters-v1650			OPTIONAL,

nonCriticalExtension				UE-EUTRA-Capability-v1660-IEs		OPTIONAL

}

UE-EUTRA-Capability-v1660-IEs ::= SEQUENCE {

irat-ParametersNR-v1660			IRAT-ParametersNR-v1660,

nonCriticalExtension				UE-EUTRA-Capability-v1690-IEs		OPTIONAL

}

UE-EUTRA-Capability-v1690-IEs ::= SEQUENCE {

other-Parameters-v1690			Other-Parameters-v1690,

nonCriticalExtension				UE-EUTRA-Capability-v1700-IEs				OPTIONAL

}

UE-EUTRA-Capability-v1700-IEs ::= SEQUENCE {

measParameters-v1700					MeasParameters-v1700					OPTIONAL,

ue-BasedNetwPerfMeasParameters-v1700	UE-BasedNetwPerfMeasParameters-v1700	OPTIONAL,

phyLayerParameters-v1700				PhyLayerParameters-v1700,

ntn-Parameters-r17						NTN-Parameters-r17						OPTIONAL,

irat-ParametersNR-v1700				IRAT-ParametersNR-v1700				OPTIONAL,

mbms-Parameters-v1700					MBMS-Parameters-v1700,

nonCriticalExtension					UE-EUTRA-Capability-v1710-IEs		OPTIONAL

}

UE-EUTRA-Capability-v1710-IEs ::= SEQUENCE {

irat-ParametersNR-v1710					IRAT-ParametersNR-v1710,

neighCellSI-AcquisitionParameters-v1710	NeighCellSI-AcquisitionParameters-v1710	OPTIONAL,

sl-Parameters-v1710						SL-Parameters-v1710					OPTIONAL,

sidelinkRequested-r17						ENUMERATED {true}						OPTIONAL,

nonCriticalExtension						UE-EUTRA-Capability-v1720-IEs			OPTIONAL

}

UE-EUTRA-Capability-v1720-IEs ::= SEQUENCE {

ntn-Parameters-v1720						NTN-Parameters-v1720,

nonCriticalExtension						UE-EUTRA-Capability-v1730-IEs		OPTIONAL

}

UE-EUTRA-Capability-v1730-IEs ::= SEQUENCE {

phyLayerParameters-v1730					PhyLayerParameters-v1730,

nonCriticalExtension						UE-EUTRA-Capability-v18xy-IEs							OPTIONAL

}

UE-EUTRA-Capability-v18xy-IEs ::= SEQUENCE {

son-Parameters-v18xy						SON-Parameters-v18xy				OPTIONAL,

ue-BasedNetwPerfMeasParameters-v18xy		UE-BasedNetwPerfMeasParameters-v18xy	OPTIONAL,

nonCriticalExtension						SEQUENCE {}							OPTIONAL

}

UE-EUTRA-CapabilityAddXDD-Mode-r9 ::=	SEQUENCE {

phyLayerParameters-r9					PhyLayerParameters						OPTIONAL,

featureGroupIndicators-r9				BIT STRING (SIZE (32))					OPTIONAL,

featureGroupIndRel9Add-r9				BIT STRING (SIZE (32))					OPTIONAL,

interRAT-ParametersGERAN-r9				IRAT-ParametersGERAN					OPTIONAL,

interRAT-ParametersUTRA-r9				IRAT-ParametersUTRA-v920				OPTIONAL,

interRAT-ParametersCDMA2000-r9			IRAT-ParametersCDMA2000-1XRTT-v920		OPTIONAL,

neighCellSI-AcquisitionParameters-r9	NeighCellSI-AcquisitionParameters-r9	OPTIONAL,

...

}

UE-EUTRA-CapabilityAddXDD-Mode-v1060 ::=	SEQUENCE {

phyLayerParameters-v1060				PhyLayerParameters-v1020				OPTIONAL,

featureGroupIndRel10-v1060				BIT STRING (SIZE (32))					OPTIONAL,

interRAT-ParametersCDMA2000-v1060		IRAT-ParametersCDMA2000-1XRTT-v1020		OPTIONAL,

interRAT-ParametersUTRA-TDD-v1060		IRAT-ParametersUTRA-TDD-v1020			OPTIONAL,

...,

[[	otdoa-PositioningCapabilities-r10	OTDOA-PositioningCapabilities-r10		OPTIONAL

]]

}

UE-EUTRA-CapabilityAddXDD-Mode-v1130 ::=	SEQUENCE {

phyLayerParameters-v1130					PhyLayerParameters-v1130			OPTIONAL,

measParameters-v1130						MeasParameters-v1130				OPTIONAL,

otherParameters-r11							Other-Parameters-r11				OPTIONAL,

...

}

UE-EUTRA-CapabilityAddXDD-Mode-v1180 ::=	SEQUENCE {

mbms-Parameters-r11					MBMS-Parameters-r11

}

UE-EUTRA-CapabilityAddXDD-Mode-v1250 ::=	SEQUENCE {

phyLayerParameters-v1250			PhyLayerParameters-v1250			OPTIONAL,

measParameters-v1250				MeasParameters-v1250				OPTIONAL

}

UE-EUTRA-CapabilityAddXDD-Mode-v1310 ::=	SEQUENCE {

phyLayerParameters-v1310			PhyLayerParameters-v1310			OPTIONAL

}

UE-EUTRA-CapabilityAddXDD-Mode-v1320 ::=	SEQUENCE {

phyLayerParameters-v1320			PhyLayerParameters-v1320			OPTIONAL,

scptm-Parameters-r13				SCPTM-Parameters-r13				OPTIONAL

}

UE-EUTRA-CapabilityAddXDD-Mode-v1370 ::=	SEQUENCE {

ce-Parameters-v1370					CE-Parameters-v1370					OPTIONAL

}

UE-EUTRA-CapabilityAddXDD-Mode-v1380 ::=	SEQUENCE {

ce-Parameters-v1380					CE-Parameters-v1380

}

UE-EUTRA-CapabilityAddXDD-Mode-v1430 ::=	SEQUENCE {

phyLayerParameters-v1430			PhyLayerParameters-v1430			OPTIONAL,

mmtel-Parameters-r14				MMTEL-Parameters-r14				OPTIONAL

}

UE-EUTRA-CapabilityAddXDD-Mode-v1510 ::=	SEQUENCE {

pdcp-ParametersNR-r15						PDCP-ParametersNR-r15		OPTIONAL

}

UE-EUTRA-CapabilityAddXDD-Mode-v1530 ::=	SEQUENCE {

neighCellSI-AcquisitionParameters-v1530	NeighCellSI-AcquisitionParameters-v1530	OPTIONAL,

reducedCP-Latency-r15			ENUMERATED {supported}					OPTIONAL

}

UE-EUTRA-CapabilityAddXDD-Mode-v1540 ::=	SEQUENCE {

eutra-5GC-Parameters-r15					EUTRA-5GC-Parameters-r15		OPTIONAL,

irat-ParametersNR-v1540						IRAT-ParametersNR-v1540			OPTIONAL

}

UE-EUTRA-CapabilityAddXDD-Mode-v1550 ::=	SEQUENCE {

neighCellSI-AcquisitionParameters-v1550	NeighCellSI-AcquisitionParameters-v1550	OPTIONAL

}

UE-EUTRA-CapabilityAddXDD-Mode-v1560 ::=	SEQUENCE {

pdcp-ParametersNR-v1560					PDCP-ParametersNR-v1560

}

UE-EUTRA-CapabilityAddXDD-Mode-v15a0 ::=	SEQUENCE {

phyLayerParameters-v1530				PhyLayerParameters-v1530				OPTIONAL,

phyLayerParameters-v1540				PhyLayerParameters-v1540				OPTIONAL,

phyLayerParameters-v1550				PhyLayerParameters-v1550				OPTIONAL,

neighCellSI-AcquisitionParameters-v15a0	NeighCellSI-AcquisitionParameters-v15a0

}

UE-EUTRA-CapabilityAddXDD-Mode-v1610 ::= SEQUENCE {

phyLayerParameters-v1610					PhyLayerParameters-v1610				OPTIONAL,

pur-Parameters-r16							PUR-Parameters-r16						OPTIONAL,

measParameters-v1610						MeasParameters-v1610					OPTIONAL,

eutra-5GC-Parameters-v1610					EUTRA-5GC-Parameters-v1610				OPTIONAL,

irat-ParametersNR-v1610						IRAT-ParametersNR-v1610					OPTIONAL,

neighCellSI-AcquisitionParameters-v1610		NeighCellSI-AcquisitionParameters-v1610	OPTIONAL,

mobilityParameters-v1610					MobilityParameters-v1610				OPTIONAL

}

UE-EUTRA-CapabilityAddXDD-Mode-v1630 ::= SEQUENCE {

measParameters-v1630						MeasParameters-v1630

}

AccessStratumRelease ::=			ENUMERATED {

rel8, rel9, rel10, rel11, rel12, rel13,

rel14, rel15, ..., rel16, rel17}

FeatureSetsEUTRA-r15 ::=	SEQUENCE {

featureSetsDL-r15			SEQUENCE (SIZE (1..maxFeatureSets-r15)) OF FeatureSetDL-r15		OPTIONAL,

featureSetsDL-PerCC-r15		SEQUENCE (SIZE (1..maxPerCC-FeatureSets-r15)) OF FeatureSetDL-PerCC-r15		OPTIONAL,

featureSetsUL-r15			SEQUENCE (SIZE (1..maxFeatureSets-r15)) OF FeatureSetUL-r15		OPTIONAL,

featureSetsUL-PerCC-r15		SEQUENCE (SIZE (1..maxPerCC-FeatureSets-r15)) OF FeatureSetUL-PerCC-r15		OPTIONAL,

...,

[[	featureSetsDL-v1550		SEQUENCE (SIZE (1..maxFeatureSets-r15)) OF FeatureSetDL-v1550	OPTIONAL

]]

}

MobilityParameters-r14 ::=			SEQUENCE {

makeBeforeBreak-r14					ENUMERATED {supported}					OPTIONAL,

rach-Less-r14						ENUMERATED {supported}					OPTIONAL

}

MobilityParameters-v1610 ::=		SEQUENCE {

cho-r16								ENUMERATED {supported}					OPTIONAL,

cho-FDD-TDD-r16						ENUMERATED {supported}					OPTIONAL,

cho-Failure-r16						ENUMERATED {supported}					OPTIONAL,

cho-TwoTriggerEvents-r16			ENUMERATED {supported}					OPTIONAL

}

DC-Parameters-r12 ::=			SEQUENCE {

drb-TypeSplit-r12						ENUMERATED {supported}			OPTIONAL,

drb-TypeSCG-r12							ENUMERATED {supported}			OPTIONAL

}

DC-Parameters-v1310 ::=			SEQUENCE {

pdcp-TransferSplitUL-r13				ENUMERATED {supported}			OPTIONAL,

ue-SSTD-Meas-r13						ENUMERATED {supported}			OPTIONAL

}

MAC-Parameters-r12 ::=				SEQUENCE {

logicalChannelSR-ProhibitTimer-r12	ENUMERATED {supported}					OPTIONAL,

longDRX-Command-r12					ENUMERATED {supported}					OPTIONAL

}

MAC-Parameters-v1310 ::=				SEQUENCE {

extendedMAC-LengthField-r13		ENUMERATED {supported}				OPTIONAL,

extendedLongDRX-r13				ENUMERATED {supported}				OPTIONAL

}

MAC-Parameters-v1430 ::=				SEQUENCE {

shortSPS-IntervalFDD-r14			ENUMERATED {supported}				OPTIONAL,

shortSPS-IntervalTDD-r14			ENUMERATED {supported}				OPTIONAL,

skipUplinkDynamic-r14				ENUMERATED {supported}				OPTIONAL,

skipUplinkSPS-r14					ENUMERATED {supported}				OPTIONAL,

multipleUplinkSPS-r14				ENUMERATED {supported}				OPTIONAL,

dataInactMon-r14					ENUMERATED {supported}				OPTIONAL

}

MAC-Parameters-v1440 ::=				SEQUENCE {

rai-Support-r14					ENUMERATED {supported}			OPTIONAL

}

MAC-Parameters-v1530 ::=		SEQUENCE {

min-Proc-TimelineSubslot-r15	SEQUENCE (SIZE(1..3)) OF ProcessingTimelineSet-r15	OPTIONAL,

skipSubframeProcessing-r15			SkipSubframeProcessing-r15						OPTIONAL,

earlyData-UP-r15					ENUMERATED {supported}							OPTIONAL,

dormantSCellState-r15				ENUMERATED {supported}							OPTIONAL,

directSCellActivation-r15			ENUMERATED {supported}							OPTIONAL,

directSCellHibernation-r15			ENUMERATED {supported}							OPTIONAL,

extendedLCID-Duplication-r15		ENUMERATED {supported}							OPTIONAL,

sps-ServingCell-r15					ENUMERATED {supported}							OPTIONAL

}

MAC-Parameters-v1550 ::=				SEQUENCE {

eLCID-Support-r15					ENUMERATED {supported}			OPTIONAL

}

MAC-Parameters-v1610 ::=		SEQUENCE {

directMCG-SCellActivationResume-r16	ENUMERATED {supported}			OPTIONAL,

directSCG-SCellActivationResume-r16	ENUMERATED {supported}			OPTIONAL,

earlyData-UP-5GC-r16				ENUMERATED {supported}			OPTIONAL,

rai-SupportEnh-r16					ENUMERATED {supported}			OPTIONAL

}

MAC-Parameters-v1630 ::=		SEQUENCE {

directSCG-SCellActivationNEDC-r16	ENUMERATED {supported}			OPTIONAL

}

NTN-Parameters-r17 ::=		SEQUENCE {

ntn-Connectivity-EPC-r17		ENUMERATED {supported}			OPTIONAL,

ntn-TA-Report-r17				ENUMERATED {supported}			OPTIONAL,

ntn-PUR-TimerDelay-r17		ENUMERATED {supported}			OPTIONAL,

ntn-OffsetTimingEnh-r17		ENUMERATED {supported}			OPTIONAL,

ntn-ScenarioSupport-r17		ENUMERATED {ngso,gso}				OPTIONAL

}

NTN-Parameters-v1720 ::=		SEQUENCE {

ntn-SegmentedPrecompensationGaps-r17		ENUMERATED {sym1,sl1,sf1}		OPTIONAL

}

ProcessingTimelineSet-r15 ::=		ENUMERATED {set1, set2}

RLC-Parameters-r12 ::=				SEQUENCE {

extended-RLC-LI-Field-r12			ENUMERATED {supported}

}

RLC-Parameters-v1310 ::=				SEQUENCE {

extendedRLC-SN-SO-Field-r13				ENUMERATED {supported}				OPTIONAL

}

RLC-Parameters-v1430 ::=				SEQUENCE {

extendedPollByte-r14						ENUMERATED {supported}			OPTIONAL

}

RLC-Parameters-v1530 ::=				SEQUENCE {

flexibleUM-AM-Combinations-r15			ENUMERATED {supported}			OPTIONAL,

rlc-AM-Ooo-Delivery-r15					ENUMERATED {supported}			OPTIONAL,

rlc-UM-Ooo-Delivery-r15					ENUMERATED {supported}			OPTIONAL

}

PDCP-Parameters ::=				SEQUENCE {

supportedROHC-Profiles				ROHC-ProfileSupportList-r15,

maxNumberROHC-ContextSessions		ENUMERATED {

cs2, cs4, cs8, cs12, cs16, cs24, cs32,

cs48, cs64, cs128, cs256, cs512, cs1024,

cs16384, spare2, spare1}				DEFAULT cs16,

...

}

PDCP-Parameters-v1130 ::=		SEQUENCE {

pdcp-SN-Extension-r11					ENUMERATED {supported}			OPTIONAL,

supportRohcContextContinue-r11			ENUMERATED {supported}			OPTIONAL

}

PDCP-Parameters-v1310 ::=				SEQUENCE {

pdcp-SN-Extension-18bits-r13			ENUMERATED {supported}	OPTIONAL

}

PDCP-Parameters-v1430 ::=				SEQUENCE {

supportedUplinkOnlyROHC-Profiles-r14		SEQUENCE {

profile0x0006-r14						BOOLEAN

},

maxNumberROHC-ContextSessions-r14		ENUMERATED {

cs2, cs4, cs8, cs12, cs16, cs24, cs32,

cs48, cs64, cs128, cs256, cs512, cs1024,

cs16384, spare2, spare1}				DEFAULT cs16

}

PDCP-Parameters-v1530 ::=			SEQUENCE {

supportedUDC-r15					SupportedUDC-r15				OPTIONAL,

pdcp-Duplication-r15				ENUMERATED {supported}		OPTIONAL

}

PDCP-Parameters-v1610 ::=			SEQUENCE {

pdcp-VersionChangeWithoutHO-r16		ENUMERATED {supported}		OPTIONAL,

ehc-r16								ENUMERATED {supported}		OPTIONAL,

continueEHC-Context-r16				ENUMERATED {supported}		OPTIONAL,

maxNumberEHC-Contexts-r16			ENUMERATED {cs2, cs4, cs8, cs16, cs32, cs64, cs128, cs256,

cs512, cs1024, cs2048, cs4096, cs8192, cs16384,

cs32768, cs65536}	OPTIONAL,

jointEHC-ROHC-Config-r16			ENUMERATED {supported}		OPTIONAL

}

SupportedUDC-r15 ::=				SEQUENCE {

supportedStandardDic-r15			ENUMERATED {supported}		OPTIONAL,

supportedOperatorDic-r15			SupportedOperatorDic-r15	OPTIONAL

}

SupportedOperatorDic-r15 ::=		SEQUENCE {

versionOfDictionary-r15				INTEGER (0..15),

associatedPLMN-ID-r15				PLMN-Identity

}

PhyLayerParameters ::=				SEQUENCE {

ue-TxAntennaSelectionSupported		BOOLEAN,

ue-SpecificRefSigsSupported		BOOLEAN

}

PhyLayerParameters-v920 ::=		SEQUENCE {

enhancedDualLayerFDD-r9			ENUMERATED {supported}			OPTIONAL,

enhancedDualLayerTDD-r9			ENUMERATED {supported}			OPTIONAL

}

PhyLayerParameters-v9d0 ::=			SEQUENCE {

tm5-FDD-r9						ENUMERATED {supported}			OPTIONAL,

tm5-TDD-r9						ENUMERATED {supported}			OPTIONAL

}

PhyLayerParameters-v1020 ::=			SEQUENCE {

twoAntennaPortsForPUCCH-r10				ENUMERATED {supported}					OPTIONAL,

tm9-With-8Tx-FDD-r10					ENUMERATED {supported}					OPTIONAL,

pmi-Disabling-r10						ENUMERATED {supported}					OPTIONAL,

crossCarrierScheduling-r10				ENUMERATED {supported}					OPTIONAL,

simultaneousPUCCH-PUSCH-r10				ENUMERATED {supported}					OPTIONAL,

multiClusterPUSCH-WithinCC-r10			ENUMERATED {supported}					OPTIONAL,

nonContiguousUL-RA-WithinCC-List-r10	NonContiguousUL-RA-WithinCC-List-r10	OPTIONAL

}

PhyLayerParameters-v1130 ::=			SEQUENCE {

crs-InterfHandl-r11						ENUMERATED {supported}					OPTIONAL,

ePDCCH-r11								ENUMERATED {supported}					OPTIONAL,

multiACK-CSI-Reporting-r11				ENUMERATED {supported}					OPTIONAL,

ss-CCH-InterfHandl-r11					ENUMERATED {supported}					OPTIONAL,

tdd-SpecialSubframe-r11					ENUMERATED {supported}					OPTIONAL,

txDiv-PUCCH1b-ChSelect-r11				ENUMERATED {supported}					OPTIONAL,

ul-CoMP-r11								ENUMERATED {supported}					OPTIONAL

}

PhyLayerParameters-v1170 ::=			SEQUENCE {

interBandTDD-CA-WithDifferentConfig-r11	BIT STRING (SIZE (2))			OPTIONAL

}

PhyLayerParameters-v1250 ::=			SEQUENCE {

e-HARQ-Pattern-FDD-r12					ENUMERATED {supported}			OPTIONAL,

enhanced-4TxCodebook-r12				ENUMERATED {supported}			OPTIONAL,

tdd-FDD-CA-PCellDuplex-r12				BIT STRING (SIZE (2))			OPTIONAL,

phy-TDD-ReConfig-TDD-PCell-r12			ENUMERATED {supported}			OPTIONAL,

phy-TDD-ReConfig-FDD-PCell-r12			ENUMERATED {supported}			OPTIONAL,

pusch-FeedbackMode-r12					ENUMERATED {supported}			OPTIONAL,

pusch-SRS-PowerControl-SubframeSet-r12	ENUMERATED {supported}			OPTIONAL,

csi-SubframeSet-r12						ENUMERATED {supported}			OPTIONAL,

noResourceRestrictionForTTIBundling-r12	ENUMERATED {supported}			OPTIONAL,

discoverySignalsInDeactSCell-r12		ENUMERATED {supported}			OPTIONAL,

naics-Capability-List-r12				NAICS-Capability-List-r12		OPTIONAL

}

PhyLayerParameters-v1280 ::=			SEQUENCE {

alternativeTBS-Indices-r12				ENUMERATED {supported}			OPTIONAL

}

PhyLayerParameters-v1310 ::=			SEQUENCE {

aperiodicCSI-Reporting-r13				BIT STRING (SIZE (2))			OPTIONAL,

codebook-HARQ-ACK-r13					BIT STRING (SIZE (2))			OPTIONAL,

crossCarrierScheduling-B5C-r13			ENUMERATED {supported}			OPTIONAL,

fdd-HARQ-TimingTDD-r13					ENUMERATED {supported}			OPTIONAL,

maxNumberUpdatedCSI-Proc-r13			INTEGER(5..32)					OPTIONAL,

pucch-Format4-r13						ENUMERATED {supported}			OPTIONAL,

pucch-Format5-r13						ENUMERATED {supported}			OPTIONAL,

pucch-SCell-r13							ENUMERATED {supported}			OPTIONAL,

spatialBundling-HARQ-ACK-r13			ENUMERATED {supported}			OPTIONAL,

supportedBlindDecoding-r13				SEQUENCE {

maxNumberDecoding-r13					INTEGER(1..32)				OPTIONAL,

pdcch-CandidateReductions-r13			ENUMERATED {supported}		OPTIONAL,

skipMonitoringDCI-Format0-1A-r13		ENUMERATED {supported}		OPTIONAL

}																		OPTIONAL,

uci-PUSCH-Ext-r13						ENUMERATED {supported}			OPTIONAL,

crs-InterfMitigationTM10-r13			ENUMERATED {supported}			OPTIONAL,

pdsch-CollisionHandling-r13				ENUMERATED {supported}			OPTIONAL

}

PhyLayerParameters-v1320 ::=			SEQUENCE {

mimo-UE-Parameters-r13					MIMO-UE-Parameters-r13			OPTIONAL

}

PhyLayerParameters-v1330 ::=			SEQUENCE {

cch-InterfMitigation-RefRecTypeA-r13	ENUMERATED {supported}			OPTIONAL,

cch-InterfMitigation-RefRecTypeB-r13	ENUMERATED {supported}			OPTIONAL,

cch-InterfMitigation-MaxNumCCs-r13		INTEGER (1.. maxServCell-r13)	OPTIONAL,

crs-InterfMitigationTM1toTM9-r13		INTEGER (1.. maxServCell-r13)	OPTIONAL

}

PhyLayerParameters-v13e0 ::=			SEQUENCE {

mimo-UE-Parameters-v13e0				MIMO-UE-Parameters-v13e0

}

PhyLayerParameters-v1430 ::=			SEQUENCE {

ce-PUSCH-NB-MaxTBS-r14					ENUMERATED {supported}			OPTIONAL,

ce-PDSCH-PUSCH-MaxBandwidth-r14			ENUMERATED {bw5, bw20}			OPTIONAL,

ce-HARQ-AckBundling-r14					ENUMERATED {supported}			OPTIONAL,

ce-PDSCH-TenProcesses-r14				ENUMERATED {supported}			OPTIONAL,

ce-RetuningSymbols-r14					ENUMERATED {n0, n1}				OPTIONAL,

ce-PDSCH-PUSCH-Enhancement-r14			ENUMERATED {supported}			OPTIONAL,

ce-SchedulingEnhancement-r14			ENUMERATED {supported}			OPTIONAL,

ce-SRS-Enhancement-r14					ENUMERATED {supported}			OPTIONAL,

ce-PUCCH-Enhancement-r14				ENUMERATED {supported}			OPTIONAL,

ce-ClosedLoopTxAntennaSelection-r14		ENUMERATED {supported}			OPTIONAL,

tdd-SpecialSubframe-r14					ENUMERATED {supported}			OPTIONAL,

tdd-TTI-Bundling-r14					ENUMERATED {supported}			OPTIONAL,

dmrs-LessUpPTS-r14						ENUMERATED {supported}			OPTIONAL,

mimo-UE-Parameters-v1430				MIMO-UE-Parameters-v1430		OPTIONAL,

alternativeTBS-Index-r14				ENUMERATED {supported}			OPTIONAL,

feMBMS-Unicast-Parameters-r14			FeMBMS-Unicast-Parameters-r14	OPTIONAL

}

PhyLayerParameters-v1450 ::=			SEQUENCE {

ce-SRS-EnhancementWithoutComb4-r14		ENUMERATED {supported}			OPTIONAL,

crs-LessDwPTS-r14						ENUMERATED {supported}			OPTIONAL}

PhyLayerParameters-v1470 ::=			SEQUENCE {

mimo-UE-Parameters-v1470				MIMO-UE-Parameters-v1470		OPTIONAL,

srs-UpPTS-6sym-r14						ENUMERATED {supported}			OPTIONAL

}

PhyLayerParameters-v14a0 ::=			SEQUENCE {

ssp10-TDD-Only-r14						ENUMERATED {supported}			OPTIONAL

}

PhyLayerParameters-v1530 ::=			SEQUENCE {

stti-SPT-Capabilities-r15				SEQUENCE {

aperiodicCsi-ReportingSTTI-r15			ENUMERATED {supported}			OPTIONAL,

dmrs-BasedSPDCCH-MBSFN-r15				ENUMERATED {supported}			OPTIONAL,

dmrs-BasedSPDCCH-nonMBSFN-r15			ENUMERATED {supported}			OPTIONAL,

dmrs-PositionPattern-r15				ENUMERATED {supported}			OPTIONAL,

dmrs-SharingSubslotPDSCH-r15			ENUMERATED {supported}			OPTIONAL,

dmrs-RepetitionSubslotPDSCH-r15			ENUMERATED {supported}			OPTIONAL,

epdcch-SPT-differentCells-r15			ENUMERATED {supported}			OPTIONAL,

epdcch-STTI-differentCells-r15			ENUMERATED {supported}			OPTIONAL,

maxLayersSlotOrSubslotPUSCH-r15			ENUMERATED {oneLayer,twoLayers,fourLayers}

OPTIONAL,

maxNumberUpdatedCSI-Proc-SPT-r15		INTEGER(5..32)					OPTIONAL,

maxNumberUpdatedCSI-Proc-STTI-Comb77-r15		INTEGER(1..32)			OPTIONAL,

maxNumberUpdatedCSI-Proc-STTI-Comb27-r15		INTEGER(1..32)			OPTIONAL,

maxNumberUpdatedCSI-Proc-STTI-Comb22-Set1-r15	INTEGER(1..32)			OPTIONAL,

maxNumberUpdatedCSI-Proc-STTI-Comb22-Set2-r15	INTEGER(1..32)			OPTIONAL,

mimo-UE-ParametersSTTI-r15				MIMO-UE-Parameters-r13			OPTIONAL,

mimo-UE-ParametersSTTI-v1530			MIMO-UE-Parameters-v1430		OPTIONAL,

numberOfBlindDecodesUSS-r15				INTEGER(4..32)					OPTIONAL,

pdsch-SlotSubslotPDSCH-Decoding-r15		ENUMERATED {supported}			OPTIONAL,

powerUCI-SlotPUSCH						ENUMERATED {supported}			OPTIONAL,

powerUCI-SubslotPUSCH					ENUMERATED {supported}			OPTIONAL,

slotPDSCH-TxDiv-TM9and10				ENUMERATED {supported}			OPTIONAL,

subslotPDSCH-TxDiv-TM9and10				ENUMERATED {supported}			OPTIONAL,

spdcch-differentRS-types-r15			ENUMERATED {supported}			OPTIONAL,

srs-DCI7-TriggeringFS2-r15				ENUMERATED {supported}			OPTIONAL,

sps-cyclicShift-r15						ENUMERATED {supported}			OPTIONAL,

spdcch-Reuse-r15						ENUMERATED {supported}			OPTIONAL,

sps-STTI-r15							ENUMERATED {slot, subslot, slotAndSubslot}

OPTIONAL,

tm8-slotPDSCH-r15						ENUMERATED {supported}			OPTIONAL,

tm9-slotSubslot-r15						ENUMERATED {supported}			OPTIONAL,

tm9-slotSubslotMBSFN-r15				ENUMERATED {supported}			OPTIONAL,

tm10-slotSubslot-r15					ENUMERATED {supported}			OPTIONAL,

tm10-slotSubslotMBSFN-r15				ENUMERATED {supported}			OPTIONAL,

txDiv-SPUCCH-r15						ENUMERATED {supported}			OPTIONAL,

ul-AsyncHarqSharingDiff-TTI-Lengths-r15	ENUMERATED {supported}			OPTIONAL

}																			OPTIONAL,

ce-Capabilities-r15					SEQUENCE {

ce-CRS-IntfMitig-r15					ENUMERATED {supported}			OPTIONAL,

ce-CQI-AlternativeTable-r15				ENUMERATED {supported}			OPTIONAL,

ce-PDSCH-FlexibleStartPRB-CE-ModeA-r15	ENUMERATED {supported}			OPTIONAL,

ce-PDSCH-FlexibleStartPRB-CE-ModeB-r15	ENUMERATED {supported}			OPTIONAL,

ce-PDSCH-64QAM-r15						ENUMERATED {supported}			OPTIONAL,

ce-PUSCH-FlexibleStartPRB-CE-ModeA-r15	ENUMERATED {supported}			OPTIONAL,

ce-PUSCH-FlexibleStartPRB-CE-ModeB-r15	ENUMERATED {supported}			OPTIONAL,

ce-PUSCH-SubPRB-Allocation-r15			ENUMERATED {supported}			OPTIONAL,

ce-UL-HARQ-ACK-Feedback-r15				ENUMERATED {supported}			OPTIONAL

}	OPTIONAL,

shortCQI-ForSCellActivation-r15			ENUMERATED {supported}			OPTIONAL,

mimo-CBSR-AdvancedCSI-r15				ENUMERATED {supported}			OPTIONAL,

crs-IntfMitig-r15						ENUMERATED {supported}			OPTIONAL,

ul-PowerControlEnhancements-r15			ENUMERATED {supported}			OPTIONAL,

urllc-Capabilities-r15					SEQUENCE {

pdsch-RepSubframe-r15					ENUMERATED {supported}		OPTIONAL,

pdsch-RepSlot-r15						ENUMERATED {supported}		OPTIONAL,

pdsch-RepSubslot-r15					ENUMERATED {supported}		OPTIONAL,

pusch-SPS-MultiConfigSubframe-r15		INTEGER (0..6)				OPTIONAL,

pusch-SPS-MaxConfigSubframe-r15			INTEGER (0..31)				OPTIONAL,

pusch-SPS-MultiConfigSlot-r15			INTEGER (0..6)				OPTIONAL,

pusch-SPS-MaxConfigSlot-r15				INTEGER (0..31)				OPTIONAL,

pusch-SPS-MultiConfigSubslot-r15		INTEGER (0..6)				OPTIONAL,

pusch-SPS-MaxConfigSubslot-r15			INTEGER (0..31)				OPTIONAL,

pusch-SPS-SlotRepPCell-r15				ENUMERATED {supported}		OPTIONAL,

pusch-SPS-SlotRepPSCell-r15				ENUMERATED {supported}		OPTIONAL,

pusch-SPS-SlotRepSCell-r15				ENUMERATED {supported}		OPTIONAL,

pusch-SPS-SubframeRepPCell-r15			ENUMERATED {supported}		OPTIONAL,

pusch-SPS-SubframeRepPSCell-r15			ENUMERATED {supported}		OPTIONAL,

pusch-SPS-SubframeRepSCell-r15			ENUMERATED {supported}		OPTIONAL,

pusch-SPS-SubslotRepPCell-r15			ENUMERATED {supported}		OPTIONAL,

pusch-SPS-SubslotRepPSCell-r15			ENUMERATED {supported}		OPTIONAL,

pusch-SPS-SubslotRepSCell-r15			ENUMERATED {supported}		OPTIONAL,

semiStaticCFI-r15						ENUMERATED {supported}		OPTIONAL,

semiStaticCFI-Pattern-r15				ENUMERATED {supported}		OPTIONAL

}	OPTIONAL,

altMCS-Table-r15						ENUMERATED {supported}			OPTIONAL

}

PhyLayerParameters-v1540 ::=			SEQUENCE {

stti-SPT-Capabilities-v1540			SEQUENCE {

slotPDSCH-TxDiv-TM8-r15					ENUMERATED {supported}

}												OPTIONAL,

crs-IM-TM1-toTM9-OneRX-Port-v1540		ENUMERATED {supported}			OPTIONAL,

cch-IM-RefRecTypeA-OneRX-Port-v1540		ENUMERATED {supported}			OPTIONAL

}

PhyLayerParameters-v1550 ::=			SEQUENCE {

dmrs-OverheadReduction-r15				ENUMERATED {supported}			OPTIONAL

}

PhyLayerParameters-v1610 ::=			SEQUENCE {

ce-Capabilities-v1610	SEQUENCE {

ce-CSI-RS-Feedback-r16						ENUMERATED {supported}			OPTIONAL,

ce-CSI-RS-FeedbackCodebookRestriction-r16	ENUMERATED {supported}			OPTIONAL,

crs-ChEstMPDCCH-CE-ModeA-r16				ENUMERATED {supported}			OPTIONAL,

crs-ChEstMPDCCH-CE-ModeB-r16				ENUMERATED {supported}			OPTIONAL,

crs-ChEstMPDCCH-CSI-r16						ENUMERATED {supported}			OPTIONAL,

crs-ChEstMPDCCH-ReciprocityTDD-r16			ENUMERATED {supported}			OPTIONAL,

etws-CMAS-RxInConnCE-ModeA-r16				ENUMERATED {supported}			OPTIONAL,

etws-CMAS-RxInConnCE-ModeB-r16				ENUMERATED {supported}			OPTIONAL,

mpdcch-InLteControlRegionCE-ModeA-r16		ENUMERATED {supported}			OPTIONAL,

mpdcch-InLteControlRegionCE-ModeB-r16		ENUMERATED {supported}			OPTIONAL,

pdsch-InLteControlRegionCE-ModeA-r16		ENUMERATED {supported}			OPTIONAL,

pdsch-InLteControlRegionCE-ModeB-r16		ENUMERATED {supported}			OPTIONAL,

multiTB-Parameters-r16						CE-MultiTB-Parameters-r16		OPTIONAL,

resourceResvParameters-r16					CE-ResourceResvParameters-r16	OPTIONAL

}	OPTIONAL,

widebandPRG-Slot-r16				ENUMERATED {supported}			OPTIONAL,

widebandPRG-Subslot-r16				ENUMERATED {supported}			OPTIONAL,

widebandPRG-Subframe-r16			ENUMERATED {supported}			OPTIONAL,

addSRS-r16		SEQUENCE {

addSRS-FrequencyHopping-r16		ENUMERATED {supported}			OPTIONAL,

addSRS-AntennaSwitching-r16		ENUMERATED {useBasic}			OPTIONAL,

addSRS-CarrierSwitching-r16		ENUMERATED {supported}			OPTIONAL

} OPTIONAL,

virtualCellID-BasicSRS-r16			ENUMERATED {supported}			OPTIONAL,

virtualCellID-AddSRS-r16		ENUMERATED {supported}			OPTIONAL

}

PhyLayerParameters-v1700 ::=	SEQUENCE {

ce-Capabilities-v1700			SEQUENCE {

ce-PDSCH-14HARQProcesses-r17		ENUMERATED {supported}			OPTIONAL,

ce-PDSCH-14HARQProcesses-Alt2-r17	ENUMERATED {supported}			OPTIONAL,

ce-PDSCH-MaxTBS-r17					ENUMERATED {supported}			OPTIONAL

}	OPTIONAL

}

PhyLayerParameters-v1730 ::=	SEQUENCE {

csi-SubframeSet2ForDormantSCell-r17	ENUMERATED {supported}			OPTIONAL

}

MIMO-UE-Parameters-r13 ::=				SEQUENCE {

parametersTM9-r13						MIMO-UE-ParametersPerTM-r13		OPTIONAL,

parametersTM10-r13						MIMO-UE-ParametersPerTM-r13		OPTIONAL,

srs-EnhancementsTDD-r13					ENUMERATED {supported}			OPTIONAL,

srs-Enhancements-r13					ENUMERATED {supported}			OPTIONAL,

interferenceMeasRestriction-r13			ENUMERATED {supported}			OPTIONAL

}

MIMO-UE-Parameters-v13e0 ::=			SEQUENCE {

mimo-WeightedLayersCapabilities-r13		MIMO-WeightedLayersCapabilities-r13	OPTIONAL

}

MIMO-UE-Parameters-v1430 ::=			SEQUENCE {

parametersTM9-v1430						MIMO-UE-ParametersPerTM-v1430	OPTIONAL,

parametersTM10-v1430					MIMO-UE-ParametersPerTM-v1430	OPTIONAL

}

MIMO-UE-Parameters-v1470 ::=			SEQUENCE {

parametersTM9-v1470					MIMO-UE-ParametersPerTM-v1470,

parametersTM10-v1470					MIMO-UE-ParametersPerTM-v1470

}

MIMO-UE-ParametersPerTM-r13 ::=			SEQUENCE {

nonPrecoded-r13							MIMO-NonPrecodedCapabilities-r13	OPTIONAL,

beamformed-r13							MIMO-UE-BeamformedCapabilities-r13	OPTIONAL,

channelMeasRestriction-r13				ENUMERATED {supported}				OPTIONAL,

dmrs-Enhancements-r13					ENUMERATED {supported}				OPTIONAL,

csi-RS-EnhancementsTDD-r13				ENUMERATED {supported}				OPTIONAL

}

MIMO-UE-ParametersPerTM-v1430 ::=		SEQUENCE {

nzp-CSI-RS-AperiodicInfo-r14			SEQUENCE {

nMaxProc-r14							INTEGER(5..32),

nMaxResource-r14						ENUMERATED {n1, n2, n4, n8}

}																			OPTIONAL,

nzp-CSI-RS-PeriodicInfo-r14				SEQUENCE {

nMaxResource-r14						ENUMERATED {n1, n2, n4, n8}

}																			OPTIONAL,

zp-CSI-RS-AperiodicInfo-r14					ENUMERATED {supported}			OPTIONAL,

ul-dmrs-Enhancements-r14				ENUMERATED {supported}				OPTIONAL,

densityReductionNP-r14					ENUMERATED {supported}				OPTIONAL,

densityReductionBF-r14					ENUMERATED {supported}				OPTIONAL,

hybridCSI-r14							ENUMERATED {supported}				OPTIONAL,

semiOL-r14								ENUMERATED {supported}				OPTIONAL,

csi-ReportingNP-r14						ENUMERATED {supported}				OPTIONAL,

csi-ReportingAdvanced-r14				ENUMERATED {supported}				OPTIONAL

}

MIMO-UE-ParametersPerTM-v1470 ::=		SEQUENCE {

csi-ReportingAdvancedMaxPorts-r14		ENUMERATED {n8, n12, n16, n20, n24, n28}	OPTIONAL

}

MIMO-CA-ParametersPerBoBC-r13 ::=		SEQUENCE {

parametersTM9-r13						MIMO-CA-ParametersPerBoBCPerTM-r13		OPTIONAL,

parametersTM10-r13						MIMO-CA-ParametersPerBoBCPerTM-r13		OPTIONAL

}

MIMO-CA-ParametersPerBoBC-r15 ::=		SEQUENCE {

parametersTM9-r15						MIMO-CA-ParametersPerBoBCPerTM-r15	OPTIONAL,

parametersTM10-r15						MIMO-CA-ParametersPerBoBCPerTM-r15	OPTIONAL

}

MIMO-CA-ParametersPerBoBC-v1430 ::=		SEQUENCE {

parametersTM9-v1430						MIMO-CA-ParametersPerBoBCPerTM-v1430	OPTIONAL,

parametersTM10-v1430					MIMO-CA-ParametersPerBoBCPerTM-v1430	OPTIONAL

}

MIMO-CA-ParametersPerBoBC-v1470 ::=		SEQUENCE {

parametersTM9-v1470						MIMO-CA-ParametersPerBoBCPerTM-v1470,

parametersTM10-v1470						MIMO-CA-ParametersPerBoBCPerTM-v1470

}

MIMO-CA-ParametersPerBoBCPerTM-r13 ::=	SEQUENCE {

nonPrecoded-r13							MIMO-NonPrecodedCapabilities-r13	OPTIONAL,

beamformed-r13							MIMO-BeamformedCapabilityList-r13	OPTIONAL,

dmrs-Enhancements-r13					ENUMERATED {different}				OPTIONAL

}

MIMO-CA-ParametersPerBoBCPerTM-v1430 ::=	SEQUENCE {

csi-ReportingNP-r14						ENUMERATED {different}				OPTIONAL,

csi-ReportingAdvanced-r14				ENUMERATED {different}				OPTIONAL

}

MIMO-CA-ParametersPerBoBCPerTM-v1470 ::=	SEQUENCE {

csi-ReportingAdvancedMaxPorts-r14		ENUMERATED {n8, n12, n16, n20, n24, n28}	OPTIONAL

}

MIMO-CA-ParametersPerBoBCPerTM-r15 ::=	SEQUENCE {

nonPrecoded-r13							MIMO-NonPrecodedCapabilities-r13	OPTIONAL,

beamformed-r13							MIMO-BeamformedCapabilityList-r13	OPTIONAL,

dmrs-Enhancements-r13					ENUMERATED {different}				OPTIONAL,

csi-ReportingNP-r14						ENUMERATED {different}				OPTIONAL,

csi-ReportingAdvanced-r14				ENUMERATED {different}				OPTIONAL

}

MIMO-NonPrecodedCapabilities-r13 ::=	SEQUENCE {

config1-r13								ENUMERATED {supported}			OPTIONAL,

config2-r13								ENUMERATED {supported}			OPTIONAL,

config3-r13								ENUMERATED {supported}			OPTIONAL,

config4-r13								ENUMERATED {supported}			OPTIONAL

}

MIMO-UE-BeamformedCapabilities-r13 ::=		SEQUENCE {

altCodebook-r13							ENUMERATED {supported}			OPTIONAL,

mimo-BeamformedCapabilities-r13			MIMO-BeamformedCapabilityList-r13

}

MIMO-BeamformedCapabilityList-r13 ::=		SEQUENCE (SIZE (1..maxCSI-Proc-r11)) OF MIMO-BeamformedCapabilities-r13

MIMO-BeamformedCapabilities-r13 ::=		SEQUENCE {

k-Max-r13								INTEGER (1..8),

n-MaxList-r13							BIT STRING (SIZE (1..7))		OPTIONAL

}

MIMO-WeightedLayersCapabilities-r13 ::=		SEQUENCE {

relWeightTwoLayers-r13	ENUMERATED {v1, v1dot25, v1dot5, v1dot75, v2, v2dot5, v3, v4},

relWeightFourLayers-r13	ENUMERATED {v1, v1dot25, v1dot5, v1dot75, v2, v2dot5, v3, v4}	OPTIONAL,

relWeightEightLayers-r13	ENUMERATED {v1, v1dot25, v1dot5, v1dot75, v2, v2dot5, v3, v4}	OPTIONAL,

totalWeightedLayers-r13	INTEGER (2..128)

}

NonContiguousUL-RA-WithinCC-List-r10 ::= SEQUENCE (SIZE (1..maxBands)) OF NonContiguousUL-RA-WithinCC-r10

NonContiguousUL-RA-WithinCC-r10 ::=		SEQUENCE {

nonContiguousUL-RA-WithinCC-Info-r10	ENUMERATED {supported}					OPTIONAL

}

RF-Parameters ::=					SEQUENCE {

supportedBandListEUTRA				SupportedBandListEUTRA

}

RF-Parameters-v9e0 ::=					SEQUENCE {

supportedBandListEUTRA-v9e0				SupportedBandListEUTRA-v9e0				OPTIONAL

}

RF-Parameters-v1020 ::=				SEQUENCE {

supportedBandCombination-r10			SupportedBandCombination-r10

}

RF-Parameters-v1060 ::=				SEQUENCE {

supportedBandCombinationExt-r10			SupportedBandCombinationExt-r10

}

RF-Parameters-v1090 ::=					SEQUENCE {

supportedBandCombination-v1090			SupportedBandCombination-v1090			OPTIONAL

}

RF-Parameters-v10f0 ::=					SEQUENCE {

modifiedMPR-Behavior-r10					BIT STRING (SIZE (32))				OPTIONAL

}

RF-Parameters-v10i0 ::=					SEQUENCE {

supportedBandCombination-v10i0			SupportedBandCombination-v10i0			OPTIONAL

}

RF-Parameters-v10j0 ::=					SEQUENCE {

multiNS-Pmax-r10						ENUMERATED {supported}					OPTIONAL

}

RF-Parameters-v1130 ::=				SEQUENCE {

supportedBandCombination-v1130			SupportedBandCombination-v1130			OPTIONAL

}

RF-Parameters-v1180 ::=				SEQUENCE {

freqBandRetrieval-r11					ENUMERATED {supported}			OPTIONAL,

requestedBands-r11						SEQUENCE (SIZE (1.. maxBands)) OF FreqBandIndicator-r11						OPTIONAL,

supportedBandCombinationAdd-r11			SupportedBandCombinationAdd-r11		OPTIONAL

}

RF-Parameters-v11d0 ::=					SEQUENCE {

supportedBandCombinationAdd-v11d0		SupportedBandCombinationAdd-v11d0		OPTIONAL

}

RF-Parameters-v1250 ::=				SEQUENCE {

supportedBandListEUTRA-v1250				SupportedBandListEUTRA-v1250			OPTIONAL,

supportedBandCombination-v1250			SupportedBandCombination-v1250			OPTIONAL,

supportedBandCombinationAdd-v1250		SupportedBandCombinationAdd-v1250		OPTIONAL,

freqBandPriorityAdjustment-r12			ENUMERATED {supported}					OPTIONAL

}

RF-Parameters-v1270 ::=				SEQUENCE {

supportedBandCombination-v1270			SupportedBandCombination-v1270			OPTIONAL,

supportedBandCombinationAdd-v1270		SupportedBandCombinationAdd-v1270		OPTIONAL

}

RF-Parameters-v1310 ::=				SEQUENCE {

eNB-RequestedParameters-r13			SEQUENCE {

reducedIntNonContCombRequested-r13	ENUMERATED {true}						OPTIONAL,

requestedCCsDL-r13					INTEGER (2..32)							OPTIONAL,

requestedCCsUL-r13					INTEGER (2..32)							OPTIONAL,

skipFallbackCombRequested-r13		ENUMERATED {true}						OPTIONAL

}																				OPTIONAL,

maximumCCsRetrieval-r13					ENUMERATED {supported}					OPTIONAL,

skipFallbackCombinations-r13			ENUMERATED {supported}					OPTIONAL,

reducedIntNonContComb-r13				ENUMERATED {supported}					OPTIONAL,

supportedBandListEUTRA-v1310			SupportedBandListEUTRA-v1310			OPTIONAL,

supportedBandCombinationReduced-r13		SupportedBandCombinationReduced-r13		OPTIONAL

}

RF-Parameters-v1320 ::=				SEQUENCE {

supportedBandListEUTRA-v1320			SupportedBandListEUTRA-v1320			OPTIONAL,

supportedBandCombination-v1320			SupportedBandCombination-v1320			OPTIONAL,

supportedBandCombinationAdd-v1320		SupportedBandCombinationAdd-v1320		OPTIONAL,

supportedBandCombinationReduced-v1320	SupportedBandCombinationReduced-v1320	OPTIONAL

}

RF-Parameters-v1380 ::=				SEQUENCE {

supportedBandCombination-v1380			SupportedBandCombination-v1380			OPTIONAL,

supportedBandCombinationAdd-v1380		SupportedBandCombinationAdd-v1380		OPTIONAL,

supportedBandCombinationReduced-v1380	SupportedBandCombinationReduced-v1380	OPTIONAL

}

RF-Parameters-v1390 ::=				SEQUENCE {

supportedBandCombination-v1390			SupportedBandCombination-v1390			OPTIONAL,

supportedBandCombinationAdd-v1390		SupportedBandCombinationAdd-v1390		OPTIONAL,

supportedBandCombinationReduced-v1390	SupportedBandCombinationReduced-v1390	OPTIONAL

}

RF-Parameters-v12b0 ::=				SEQUENCE {

maxLayersMIMO-Indication-r12			ENUMERATED {supported}					OPTIONAL

}

RF-Parameters-v1430 ::=				SEQUENCE {

supportedBandCombination-v1430			SupportedBandCombination-v1430			OPTIONAL,

supportedBandCombinationAdd-v1430		SupportedBandCombinationAdd-v1430		OPTIONAL,

supportedBandCombinationReduced-v1430	SupportedBandCombinationReduced-v1430	OPTIONAL,

eNB-RequestedParameters-v1430			SEQUENCE {

requestedDiffFallbackCombList-r14		BandCombinationList-r14

}																				OPTIONAL,

diffFallbackCombReport-r14				ENUMERATED {supported}					OPTIONAL

}

RF-Parameters-v1450 ::=				SEQUENCE {

supportedBandCombination-v1450			SupportedBandCombination-v1450			OPTIONAL,

supportedBandCombinationAdd-v1450		SupportedBandCombinationAdd-v1450		OPTIONAL,

supportedBandCombinationReduced-v1450	SupportedBandCombinationReduced-v1450	OPTIONAL

}

RF-Parameters-v1470 ::=				SEQUENCE {

supportedBandCombination-v1470			SupportedBandCombination-v1470			OPTIONAL,

supportedBandCombinationAdd-v1470		SupportedBandCombinationAdd-v1470		OPTIONAL,

supportedBandCombinationReduced-v1470	SupportedBandCombinationReduced-v1470	OPTIONAL

}

RF-Parameters-v14b0 ::=				SEQUENCE {

supportedBandCombination-v14b0			SupportedBandCombination-v14b0			OPTIONAL,

supportedBandCombinationAdd-v14b0		SupportedBandCombinationAdd-v14b0		OPTIONAL,

supportedBandCombinationReduced-v14b0	SupportedBandCombinationReduced-v14b0	OPTIONAL

}

RF-Parameters-v1530 ::=				SEQUENCE {

sTTI-SPT-Supported-r15					ENUMERATED {supported}					OPTIONAL,

supportedBandCombination-v1530			SupportedBandCombination-v1530			OPTIONAL,

supportedBandCombinationAdd-v1530		SupportedBandCombinationAdd-v1530		OPTIONAL,

supportedBandCombinationReduced-v1530	SupportedBandCombinationReduced-v1530	OPTIONAL,

powerClass-14dBm-r15					ENUMERATED {supported}					OPTIONAL

}

RF-Parameters-v1570 ::=			SEQUENCE {

dl-1024QAM-ScalingFactor-r15			ENUMERATED {v1, v1dot2, v1dot25},

dl-1024QAM-TotalWeightedLayers-r15		INTEGER (0..10)

}

RF-Parameters-v1610 ::=				SEQUENCE {

supportedBandCombination-v1610			SupportedBandCombination-v1610			OPTIONAL,

supportedBandCombinationAdd-v1610		SupportedBandCombinationAdd-v1610		OPTIONAL,

supportedBandCombinationReduced-v1610	SupportedBandCombinationReduced-v1610	OPTIONAL

}

RF-Parameters-v1630 ::=				SEQUENCE {

supportedBandCombination-v1630			SupportedBandCombination-v1630			OPTIONAL,

supportedBandCombinationAdd-v1630		SupportedBandCombinationAdd-v1630		OPTIONAL,

supportedBandCombinationReduced-v1630	SupportedBandCombinationReduced-v1630	OPTIONAL

}

SkipSubframeProcessing-r15 ::=		SEQUENCE {

skipProcessingDL-Slot-r15			INTEGER (0..3)					OPTIONAL,

skipProcessingDL-SubSlot-r15		INTEGER (0..3)					OPTIONAL,

skipProcessingUL-Slot-r15			INTEGER (0..3)					OPTIONAL,

skipProcessingUL-SubSlot-r15		INTEGER (0..3)					OPTIONAL

}

SPT-Parameters-r15 ::=				SEQUENCE {

frameStructureType-SPT-r15			BIT STRING (SIZE (3))			OPTIONAL,

maxNumberCCs-SPT-r15				INTEGER (1..32)					OPTIONAL

}

STTI-SPT-BandParameters-r15 ::= SEQUENCE {

dl-1024QAM-Slot-r15						ENUMERATED {supported}			OPTIONAL,

dl-1024QAM-SubslotTA-1-r15				ENUMERATED {supported}			OPTIONAL,

dl-1024QAM-SubslotTA-2-r15				ENUMERATED {supported}			OPTIONAL,

simultaneousTx-differentTx-duration-r15	ENUMERATED {supported}			OPTIONAL,

sTTI-CA-MIMO-ParametersDL-r15			CA-MIMO-ParametersDL-r15		OPTIONAL,

sTTI-CA-MIMO-ParametersUL-r15			CA-MIMO-ParametersUL-r15,

sTTI-FD-MIMO-Coexistence				ENUMERATED {supported}			OPTIONAL,

sTTI-MIMO-CA-ParametersPerBoBCs-r15		MIMO-CA-ParametersPerBoBC-r13	OPTIONAL,

sTTI-MIMO-CA-ParametersPerBoBCs-v1530	MIMO-CA-ParametersPerBoBC-v1430	OPTIONAL,

sTTI-SupportedCombinations-r15			STTI-SupportedCombinations-r15	OPTIONAL,

sTTI-SupportedCSI-Proc-r15				ENUMERATED {n1, n3, n4}			OPTIONAL,

ul-256QAM-Slot-r15						ENUMERATED {supported}			OPTIONAL,

ul-256QAM-Subslot-r15					ENUMERATED {supported}			OPTIONAL,

...

}

STTI-SupportedCombinations-r15 ::=	SEQUENCE {

combination-22-r15					DL-UL-CCs-r15					OPTIONAL,

combination-77-r15					DL-UL-CCs-r15					OPTIONAL,

combination-27-r15					DL-UL-CCs-r15					OPTIONAL,

combination-22-27-r15				SEQUENCE (SIZE (1..2)) OF DL-UL-CCs-r15		OPTIONAL,

combination-77-22-r15				SEQUENCE (SIZE (1..2)) OF DL-UL-CCs-r15		OPTIONAL,

combination-77-27-r15				SEQUENCE (SIZE (1..2)) OF DL-UL-CCs-r15		OPTIONAL

}

DL-UL-CCs-r15 ::= SEQUENCE {

maxNumberDL-CCs-r15				INTEGER (1..32)						OPTIONAL,

maxNumberUL-CCs-r15				INTEGER (1..32)						OPTIONAL

}

SupportedBandCombination-r10 ::= SEQUENCE (SIZE (1..maxBandComb-r10)) OF BandCombinationParameters-r10

SupportedBandCombinationExt-r10 ::= SEQUENCE (SIZE (1..maxBandComb-r10)) OF BandCombinationParametersExt-r10

SupportedBandCombination-v1090 ::= SEQUENCE (SIZE (1..maxBandComb-r10)) OF BandCombinationParameters-v1090

SupportedBandCombination-v10i0 ::= SEQUENCE (SIZE (1..maxBandComb-r10)) OF BandCombinationParameters-v10i0

SupportedBandCombination-v1130 ::= SEQUENCE (SIZE (1..maxBandComb-r10)) OF BandCombinationParameters-v1130

SupportedBandCombination-v1250 ::= SEQUENCE (SIZE (1..maxBandComb-r10)) OF BandCombinationParameters-v1250

SupportedBandCombination-v1270 ::= SEQUENCE (SIZE (1..maxBandComb-r10)) OF BandCombinationParameters-v1270

SupportedBandCombination-v1320 ::= SEQUENCE (SIZE (1..maxBandComb-r10)) OF BandCombinationParameters-v1320

SupportedBandCombination-v1380 ::= SEQUENCE (SIZE (1..maxBandComb-r10)) OF BandCombinationParameters-v1380

SupportedBandCombination-v1390 ::= SEQUENCE (SIZE (1..maxBandComb-r10)) OF BandCombinationParameters-v1390

SupportedBandCombination-v1430 ::= SEQUENCE (SIZE (1..maxBandComb-r10)) OF BandCombinationParameters-v1430

SupportedBandCombination-v1450 ::= SEQUENCE (SIZE (1..maxBandComb-r10)) OF BandCombinationParameters-v1450

SupportedBandCombination-v1470 ::= SEQUENCE (SIZE (1..maxBandComb-r10)) OF BandCombinationParameters-v1470

SupportedBandCombination-v14b0 ::= SEQUENCE (SIZE (1..maxBandComb-r10)) OF BandCombinationParameters-v14b0

SupportedBandCombination-v1530 ::= SEQUENCE (SIZE (1..maxBandComb-r10)) OF BandCombinationParameters-v1530

SupportedBandCombination-v1610 ::= SEQUENCE (SIZE (1..maxBandComb-r10)) OF BandCombinationParameters-v1610

SupportedBandCombination-v1630 ::= SEQUENCE (SIZE (1..maxBandComb-r10)) OF BandCombinationParameters-v1630

SupportedBandCombinationAdd-r11 ::= SEQUENCE (SIZE (1..maxBandComb-r11)) OF BandCombinationParameters-r11

SupportedBandCombinationAdd-v11d0 ::= SEQUENCE (SIZE (1..maxBandComb-r11)) OF BandCombinationParameters-v10i0

SupportedBandCombinationAdd-v1250 ::= SEQUENCE (SIZE (1..maxBandComb-r11)) OF BandCombinationParameters-v1250

SupportedBandCombinationAdd-v1270 ::= SEQUENCE (SIZE (1..maxBandComb-r11)) OF BandCombinationParameters-v1270

SupportedBandCombinationAdd-v1320 ::= SEQUENCE (SIZE (1..maxBandComb-r11)) OF BandCombinationParameters-v1320

SupportedBandCombinationAdd-v1380 ::= SEQUENCE (SIZE (1..maxBandComb-r11)) OF BandCombinationParameters-v1380

SupportedBandCombinationAdd-v1390 ::= SEQUENCE (SIZE (1..maxBandComb-r11)) OF BandCombinationParameters-v1390

SupportedBandCombinationAdd-v1430 ::= SEQUENCE (SIZE (1..maxBandComb-r11)) OF BandCombinationParameters-v1430

SupportedBandCombinationAdd-v1450 ::= SEQUENCE (SIZE (1..maxBandComb-r11)) OF BandCombinationParameters-v1450

SupportedBandCombinationAdd-v1470 ::= SEQUENCE (SIZE (1..maxBandComb-r11)) OF BandCombinationParameters-v1470

SupportedBandCombinationAdd-v14b0 ::= SEQUENCE (SIZE (1..maxBandComb-r11)) OF BandCombinationParameters-v14b0

SupportedBandCombinationAdd-v1530 ::= SEQUENCE (SIZE (1..maxBandComb-r11)) OF BandCombinationParameters-v1530

SupportedBandCombinationAdd-v1610 ::= SEQUENCE (SIZE (1..maxBandComb-r11)) OF BandCombinationParameters-v1610

SupportedBandCombinationAdd-v1630 ::= SEQUENCE (SIZE (1..maxBandComb-r11)) OF BandCombinationParameters-v1630

SupportedBandCombinationReduced-r13 ::=	SEQUENCE (SIZE (1..maxBandComb-r13)) OF BandCombinationParameters-r13

SupportedBandCombinationReduced-v1320 ::=	SEQUENCE (SIZE (1..maxBandComb-r13)) OF BandCombinationParameters-v1320

SupportedBandCombinationReduced-v1380 ::=	SEQUENCE (SIZE (1..maxBandComb-r13)) OF BandCombinationParameters-v1380

SupportedBandCombinationReduced-v1390 ::=	SEQUENCE (SIZE (1..maxBandComb-r13)) OF BandCombinationParameters-v1390

SupportedBandCombinationReduced-v1430 ::=	SEQUENCE (SIZE (1..maxBandComb-r13)) OF BandCombinationParameters-v1430

SupportedBandCombinationReduced-v1450 ::=	SEQUENCE (SIZE (1..maxBandComb-r13)) OF BandCombinationParameters-v1450

SupportedBandCombinationReduced-v1470 ::=	SEQUENCE (SIZE (1..maxBandComb-r13)) OF BandCombinationParameters-v1470

SupportedBandCombinationReduced-v14b0 ::=	SEQUENCE (SIZE (1..maxBandComb-r13)) OF BandCombinationParameters-v14b0

SupportedBandCombinationReduced-v1530 ::=	SEQUENCE (SIZE (1..maxBandComb-r13)) OF BandCombinationParameters-v1530

SupportedBandCombinationReduced-v1610 ::=	SEQUENCE (SIZE (1..maxBandComb-r13)) OF BandCombinationParameters-v1610

SupportedBandCombinationReduced-v1630 ::=	SEQUENCE (SIZE (1..maxBandComb-r13)) OF BandCombinationParameters-v1630

BandCombinationParameters-r10 ::= SEQUENCE (SIZE (1..maxSimultaneousBands-r10)) OF BandParameters-r10

BandCombinationParametersExt-r10 ::= SEQUENCE {

supportedBandwidthCombinationSet-r10	SupportedBandwidthCombinationSet-r10	OPTIONAL

}

BandCombinationParameters-v1090 ::= SEQUENCE (SIZE (1..maxSimultaneousBands-r10)) OF BandParameters-v1090

BandCombinationParameters-v10i0::= SEQUENCE {

bandParameterList-v10i0			SEQUENCE (SIZE (1..maxSimultaneousBands-r10)) OF

BandParameters-v10i0	OPTIONAL

}

BandCombinationParameters-v1130 ::=	SEQUENCE {

multipleTimingAdvance-r11		ENUMERATED {supported}					OPTIONAL,

simultaneousRx-Tx-r11			ENUMERATED {supported}					OPTIONAL,

bandParameterList-r11			SEQUENCE (SIZE (1..maxSimultaneousBands-r10)) OF BandParameters-v1130	OPTIONAL,

...

}

BandCombinationParameters-r11 ::=	SEQUENCE {

bandParameterList-r11			SEQUENCE (SIZE (1..maxSimultaneousBands-r10)) OF

BandParameters-r11,

supportedBandwidthCombinationSet-r11	SupportedBandwidthCombinationSet-r10	OPTIONAL,

multipleTimingAdvance-r11		ENUMERATED {supported}					OPTIONAL,

simultaneousRx-Tx-r11			ENUMERATED {supported}					OPTIONAL,

bandInfoEUTRA-r11				BandInfoEUTRA,

...

}

BandCombinationParameters-v1250::= SEQUENCE {

dc-Support-r12					SEQUENCE {

asynchronous-r12				ENUMERATED {supported}			OPTIONAL,

supportedCellGrouping-r12		CHOICE {

threeEntries-r12				BIT STRING (SIZE(3)),

fourEntries-r12					BIT STRING (SIZE(7)),

fiveEntries-r12					BIT STRING (SIZE(15))

}																OPTIONAL

}																	OPTIONAL,

supportedNAICS-2CRS-AP-r12		BIT STRING (SIZE (1..maxNAICS-Entries-r12))		OPTIONAL,

commSupportedBandsPerBC-r12				BIT STRING (SIZE (1.. maxBands))		OPTIONAL,

...

}

BandCombinationParameters-v1270 ::= SEQUENCE {

bandParameterList-v1270			SEQUENCE (SIZE (1..maxSimultaneousBands-r10)) OF

BandParameters-v1270		OPTIONAL

}

BandCombinationParameters-r13 ::=	SEQUENCE {

differentFallbackSupported-r13	ENUMERATED {true}				OPTIONAL,

bandParameterList-r13			SEQUENCE (SIZE (1..maxSimultaneousBands-r10)) OF BandParameters-r13,

supportedBandwidthCombinationSet-r13	SupportedBandwidthCombinationSet-r10	OPTIONAL,

multipleTimingAdvance-r13		ENUMERATED {supported}				OPTIONAL,

simultaneousRx-Tx-r13			ENUMERATED {supported}				OPTIONAL,

bandInfoEUTRA-r13				BandInfoEUTRA,

dc-Support-r13					SEQUENCE {

asynchronous-r13			ENUMERATED {supported}				OPTIONAL,

supportedCellGrouping-r13		CHOICE {

threeEntries-r13				BIT STRING (SIZE(3)),

fourEntries-r13					BIT STRING (SIZE(7)),

fiveEntries-r13					BIT STRING (SIZE(15))

}																OPTIONAL

}																	OPTIONAL,

supportedNAICS-2CRS-AP-r13		BIT STRING (SIZE (1..maxNAICS-Entries-r12))	OPTIONAL,

commSupportedBandsPerBC-r13		BIT STRING (SIZE (1.. maxBands))		OPTIONAL

}

BandCombinationParameters-v1320 ::= SEQUENCE {

bandParameterList-v1320			SEQUENCE (SIZE (1..maxSimultaneousBands-r10)) OF

BandParameters-v1320		OPTIONAL,

additionalRx-Tx-PerformanceReq-r13		ENUMERATED {supported}					OPTIONAL

}

BandCombinationParameters-v1380 ::= SEQUENCE {

bandParameterList-v1380		SEQUENCE (SIZE (1..maxSimultaneousBands-r10)) OF

BandParameters-v1380		OPTIONAL

}

BandCombinationParameters-v1390 ::= SEQUENCE {

ue-CA-PowerClass-N-r13			ENUMERATED {class2}				OPTIONAL

}

BandCombinationParameters-v1430 ::= SEQUENCE {

bandParameterList-v1430			SEQUENCE (SIZE (1..maxSimultaneousBands-r10)) OF

BandParameters-v1430		OPTIONAL,

v2x-SupportedTxBandCombListPerBC-r14			BIT STRING (SIZE (1.. maxBandComb-r13))		OPTIONAL,

v2x-SupportedRxBandCombListPerBC-r14			BIT STRING (SIZE (1.. maxBandComb-r13))		OPTIONAL

}

BandCombinationParameters-v1450 ::= SEQUENCE {

bandParameterList-v1450			SEQUENCE (SIZE (1..maxSimultaneousBands-r10)) OF

BandParameters-v1450		OPTIONAL

}

BandCombinationParameters-v1470 ::= SEQUENCE {

bandParameterList-v1470			SEQUENCE (SIZE (1..maxSimultaneousBands-r10)) OF

BandParameters-v1470		OPTIONAL,

srs-MaxSimultaneousCCs-r14	INTEGER (1..31)				OPTIONAL

}

BandCombinationParameters-v14b0 ::= SEQUENCE {

bandParameterList-v14b0			SEQUENCE (SIZE (1..maxSimultaneousBands-r10)) OF

BandParameters-v14b0		OPTIONAL

}

BandCombinationParameters-v1530 ::= SEQUENCE {

bandParameterList-v1530		SEQUENCE (SIZE (1..maxSimultaneousBands-r10)) OF							BandParameters-v1530		OPTIONAL,

spt-Parameters-r15				SPT-Parameters-r15				OPTIONAL

}

-- If an additional band combination parameter is defined, which is supported for MR-DC,

--  it shall be defined in the IE CA-ParametersEUTRA in TS 38.331 [82].

BandCombinationParameters-v1610 ::= SEQUENCE {

measGapInfoNR-r16					MeasGapInfoNR-r16					OPTIONAL,

bandParameterList-v1610		SEQUENCE (SIZE (1..maxSimultaneousBands-r10)) OF							BandParameters-v1610		OPTIONAL,

interFreqDAPS-r16						SEQUENCE {

interFreqAsyncDAPS-r16					ENUMERATED {supported}		OPTIONAL,

interFreqMultiUL-TransmissionDAPS-r16	ENUMERATED {supported}		OPTIONAL

}																		OPTIONAL

}

BandCombinationParameters-v1630 ::= SEQUENCE {

v2x-SupportedTxBandCombListPerBC-v1630		BIT STRING (SIZE (1..maxBandCombSidelinkNR-r16))		OPTIONAL,

v2x-SupportedRxBandCombListPerBC-v1630		BIT STRING (SIZE (1..maxBandCombSidelinkNR-r16))		OPTIONAL,

scalingFactorTxSidelink-r16					SEQUENCE (SIZE (1..maxBandCombSidelinkNR-r16)) OF ScalingFactorSidelink-r16		OPTIONAL,

scalingFactorRxSidelink-r16					SEQUENCE (SIZE (1..maxBandCombSidelinkNR-r16)) OF ScalingFactorSidelink-r16		OPTIONAL,

interBandPowerSharingSyncDAPS-r16			ENUMERATED {supported}	OPTIONAL,

interBandPowerSharingAsyncDAPS-r16			ENUMERATED {supported}	OPTIONAL

}

ScalingFactorSidelink-r16 ::=						ENUMERATED {f0p4, f0p75, f0p8, f1}

SupportedBandwidthCombinationSet-r10 ::=	BIT STRING (SIZE (1..maxBandwidthCombSet-r10))

BandParameters-r10 ::= SEQUENCE {

bandEUTRA-r10					FreqBandIndicator,

bandParametersUL-r10			BandParametersUL-r10					OPTIONAL,

bandParametersDL-r10			BandParametersDL-r10					OPTIONAL

}

BandParameters-v1090 ::= SEQUENCE {

bandEUTRA-v1090					FreqBandIndicator-v9e0					OPTIONAL,

...

}

BandParameters-v10i0::= SEQUENCE {

bandParametersDL-v10i0		SEQUENCE (SIZE (1..maxBandwidthClass-r10)) OF CA-MIMO-ParametersDL-v10i0

}

BandParameters-v1130 ::= SEQUENCE {

supportedCSI-Proc-r11			ENUMERATED {n1, n3, n4}

}

BandParameters-r11 ::= SEQUENCE {

bandEUTRA-r11					FreqBandIndicator-r11,

bandParametersUL-r11			BandParametersUL-r10					OPTIONAL,

bandParametersDL-r11			BandParametersDL-r10					OPTIONAL,

supportedCSI-Proc-r11			ENUMERATED {n1, n3, n4}					OPTIONAL

}

BandParameters-v1270 ::= SEQUENCE {

bandParametersDL-v1270			SEQUENCE (SIZE (1..maxBandwidthClass-r10)) OF CA-MIMO-ParametersDL-v1270

}

BandParameters-r13 ::= SEQUENCE {

bandEUTRA-r13					FreqBandIndicator-r11,

bandParametersUL-r13				BandParametersUL-r13				OPTIONAL,

bandParametersDL-r13				BandParametersDL-r13				OPTIONAL,

supportedCSI-Proc-r13			ENUMERATED {n1, n3, n4}			OPTIONAL

}

BandParameters-v1320 ::= SEQUENCE {

bandParametersDL-v1320			MIMO-CA-ParametersPerBoBC-r13

}

BandParameters-v1380 ::=	SEQUENCE {

txAntennaSwitchDL-r13			INTEGER (1..32)					OPTIONAL,

txAntennaSwitchUL-r13			INTEGER (1..32)					OPTIONAL

}

BandParameters-v1430 ::= SEQUENCE {

bandParametersDL-v1430			MIMO-CA-ParametersPerBoBC-v1430	OPTIONAL,

ul-256QAM-r14						ENUMERATED {supported}		OPTIONAL,

ul-256QAM-perCC-InfoList-r14		SEQUENCE (SIZE (2..maxServCell-r13)) OF UL-256QAM-perCC-Info-r14		OPTIONAL,

srs-CapabilityPerBandPairList-r14		SEQUENCE (SIZE (1..maxSimultaneousBands-r10)) OF

SRS-CapabilityPerBandPair-r14	OPTIONAL

}

BandParameters-v1450 ::= SEQUENCE {

must-CapabilityPerBand-r14		MUST-Parameters-r14		OPTIONAL

}

BandParameters-v1470 ::= SEQUENCE {

bandParametersDL-v1470			MIMO-CA-ParametersPerBoBC-v1470	OPTIONAL

}

BandParameters-v14b0 ::= SEQUENCE {

srs-CapabilityPerBandPairList-v14b0		SEQUENCE (SIZE (1..maxSimultaneousBands-r10)) OF		SRS-CapabilityPerBandPair-v14b0		OPTIONAL

}

BandParameters-v1530 ::=	SEQUENCE {

ue-TxAntennaSelection-SRS-1T4R-r15				ENUMERATED {supported}	OPTIONAL,

ue-TxAntennaSelection-SRS-2T4R-2Pairs-r15		ENUMERATED {supported}	OPTIONAL,

ue-TxAntennaSelection-SRS-2T4R-3Pairs-r15		ENUMERATED {supported}	OPTIONAL,

dl-1024QAM-r15									ENUMERATED {supported}	OPTIONAL,

qcl-TypeC-Operation-r15							ENUMERATED {supported}	OPTIONAL,

qcl-CRI-BasedCSI-Reporting-r15					ENUMERATED {supported}	OPTIONAL,

stti-SPT-BandParameters-r15					STTI-SPT-BandParameters-r15	OPTIONAL

}

BandParameters-v1610 ::=	SEQUENCE {

intraFreqDAPS-r16		SEQUENCE {

intraFreqAsyncDAPS-r16					ENUMERATED {supported}		OPTIONAL,

dummy									ENUMERATED {supported}		OPTIONAL,

intraFreqTwoTAGs-DAPS-r16				ENUMERATED {supported}		OPTIONAL

}																	OPTIONAL,

addSRS-FrequencyHopping-r16 ENUMERATED {supported}			OPTIONAL,

addSRS-AntennaSwitching-r16	SEQUENCE {

addSRS-1T2R-r16			ENUMERATED {supported}			OPTIONAL,

addSRS-1T4R-r16			ENUMERATED {supported}			OPTIONAL,

addSRS-2T4R-2pairs-r16	ENUMERATED {supported}			OPTIONAL,

addSRS-2T4R-3pairs-r16	ENUMERATED {supported}			OPTIONAL

}				OPTIONAL,

srs-CapabilityPerBandPairList-v1610		SEQUENCE (SIZE (1..maxSimultaneousBands-r10)) OF

SRS-CapabilityPerBandPair-v1610	OPTIONAL

}

V2X-BandParameters-r14 ::= SEQUENCE {

v2x-FreqBandEUTRA-r14			FreqBandIndicator-r11,

bandParametersTxSL-r14			BandParametersTxSL-r14				OPTIONAL,

bandParametersRxSL-r14			BandParametersRxSL-r14				OPTIONAL

}

V2X-BandParameters-v1530 ::= SEQUENCE {

v2x-EnhancedHighReception-r15			ENUMERATED {supported}		OPTIONAL

}

BandParametersTxSL-r14 ::= SEQUENCE {

v2x-BandwidthClassTxSL-r14		V2X-BandwidthClassSL-r14,

v2x-eNB-Scheduled-r14			ENUMERATED {supported}				OPTIONAL,

v2x-HighPower-r14				ENUMERATED {supported}				OPTIONAL

}

BandParametersRxSL-r14 ::= SEQUENCE {

v2x-BandwidthClassRxSL-r14		V2X-BandwidthClassSL-r14,

v2x-HighReception-r14			ENUMERATED {supported}				OPTIONAL

}

V2X-BandwidthClassSL-r14 ::= SEQUENCE (SIZE (1..maxBandwidthClass-r10)) OF V2X-BandwidthClass-r14

UL-256QAM-perCC-Info-r14 ::= SEQUENCE {

ul-256QAM-perCC-r14			ENUMERATED {supported}				OPTIONAL

}

FeatureSetDL-r15 ::=	SEQUENCE {

mimo-CA-ParametersPerBoBC-r15	MIMO-CA-ParametersPerBoBC-r15			OPTIONAL,

featureSetPerCC-ListDL-r15	SEQUENCE (SIZE (1..maxServCell-r13)) OF FeatureSetDL-PerCC-Id-r15

}

FeatureSetDL-v1550 ::=	SEQUENCE {

dl-1024QAM-r15				ENUMERATED {supported}			OPTIONAL

}

FeatureSetDL-PerCC-r15 ::=	SEQUENCE {

fourLayerTM3-TM4-r15						ENUMERATED {supported}				OPTIONAL,

supportedMIMO-CapabilityDL-MRDC-r15		MIMO-CapabilityDL-r10					OPTIONAL,

supportedCSI-Proc-r15						ENUMERATED {n1, n3, n4}				OPTIONAL

}

FeatureSetUL-r15 ::=	SEQUENCE {

featureSetPerCC-ListUL-r15	SEQUENCE (SIZE(1..maxServCell-r13)) OF FeatureSetUL-PerCC-Id-r15

}

FeatureSetUL-PerCC-r15 ::=	SEQUENCE {

supportedMIMO-CapabilityUL-r15		MIMO-CapabilityUL-r10				OPTIONAL,

ul-256QAM-r15						ENUMERATED {supported}				OPTIONAL

}

FeatureSetDL-PerCC-Id-r15 ::=	INTEGER (0..maxPerCC-FeatureSets-r15)

FeatureSetUL-PerCC-Id-r15 ::=	INTEGER (0..maxPerCC-FeatureSets-r15)

BandParametersUL-r10 ::= SEQUENCE (SIZE (1..maxBandwidthClass-r10)) OF CA-MIMO-ParametersUL-r10

BandParametersUL-r13 ::= CA-MIMO-ParametersUL-r10

CA-MIMO-ParametersUL-r10 ::= SEQUENCE {

ca-BandwidthClassUL-r10				CA-BandwidthClass-r10,

supportedMIMO-CapabilityUL-r10		MIMO-CapabilityUL-r10				OPTIONAL

}

CA-MIMO-ParametersUL-r15 ::= SEQUENCE {

supportedMIMO-CapabilityUL-r15		MIMO-CapabilityUL-r10				OPTIONAL

}

BandParametersDL-r10 ::= SEQUENCE (SIZE (1..maxBandwidthClass-r10)) OF CA-MIMO-ParametersDL-r10

BandParametersDL-r13 ::= CA-MIMO-ParametersDL-r13

CA-MIMO-ParametersDL-r10 ::= SEQUENCE {

ca-BandwidthClassDL-r10				CA-BandwidthClass-r10,

supportedMIMO-CapabilityDL-r10		MIMO-CapabilityDL-r10				OPTIONAL

}

CA-MIMO-ParametersDL-v10i0 ::= SEQUENCE {

fourLayerTM3-TM4-r10				ENUMERATED {supported}				OPTIONAL

}

CA-MIMO-ParametersDL-v1270 ::= SEQUENCE {

intraBandContiguousCC-InfoList-r12			SEQUENCE (SIZE (1..maxServCell-r10)) OF IntraBandContiguousCC-Info-r12

}

CA-MIMO-ParametersDL-r13 ::= SEQUENCE {

ca-BandwidthClassDL-r13					CA-BandwidthClass-r10,

supportedMIMO-CapabilityDL-r13			MIMO-CapabilityDL-r10				OPTIONAL,

fourLayerTM3-TM4-r13						ENUMERATED {supported}				OPTIONAL,

intraBandContiguousCC-InfoList-r13		SEQUENCE (SIZE (1..maxServCell-r13)) OF IntraBandContiguousCC-Info-r12

}

CA-MIMO-ParametersDL-r15 ::= SEQUENCE {

supportedMIMO-CapabilityDL-r15			MIMO-CapabilityDL-r10				OPTIONAL,

fourLayerTM3-TM4-r15					ENUMERATED {supported}				OPTIONAL,

intraBandContiguousCC-InfoList-r15		SEQUENCE (SIZE (1..maxServCell-r13)) OF

IntraBandContiguousCC-Info-r12				OPTIONAL

}

IntraBandContiguousCC-Info-r12 ::= SEQUENCE {

fourLayerTM3-TM4-perCC-r12			ENUMERATED {supported}				OPTIONAL,

supportedMIMO-CapabilityDL-r12		MIMO-CapabilityDL-r10				OPTIONAL,

supportedCSI-Proc-r12				ENUMERATED {n1, n3, n4}				OPTIONAL

}

CA-BandwidthClass-r10 ::= ENUMERATED {a, b, c, d, e, f, ...}

V2X-BandwidthClass-r14 ::= ENUMERATED {a, b, c, d, e, f, ..., c1-v1530}

MIMO-CapabilityUL-r10 ::= ENUMERATED {twoLayers, fourLayers}

MIMO-CapabilityDL-r10 ::= ENUMERATED {twoLayers, fourLayers, eightLayers}

MUST-Parameters-r14 ::= SEQUENCE {

must-TM234-UpTo2Tx-r14						ENUMERATED {supported}		OPTIONAL,

must-TM89-UpToOneInterferingLayer-r14		ENUMERATED {supported}		OPTIONAL,

must-TM10-UpToOneInterferingLayer-r14		ENUMERATED {supported}		OPTIONAL,

must-TM89-UpToThreeInterferingLayers-r14	ENUMERATED {supported}		OPTIONAL,

must-TM10-UpToThreeInterferingLayers-r14	ENUMERATED {supported}		OPTIONAL

}

SupportedBandListEUTRA ::=			SEQUENCE (SIZE (1..maxBands)) OF SupportedBandEUTRA

SupportedBandListEUTRA-v9e0::=			SEQUENCE (SIZE (1..maxBands)) OF SupportedBandEUTRA-v9e0

SupportedBandListEUTRA-v1250 ::=		SEQUENCE (SIZE (1..maxBands)) OF SupportedBandEUTRA-v1250

SupportedBandListEUTRA-v1310 ::=		SEQUENCE (SIZE (1..maxBands)) OF SupportedBandEUTRA-v1310

SupportedBandListEUTRA-v1320 ::=		SEQUENCE (SIZE (1..maxBands)) OF SupportedBandEUTRA-v1320

SupportedBandEUTRA ::=				SEQUENCE {

bandEUTRA							FreqBandIndicator,

halfDuplex							BOOLEAN

}

SupportedBandEUTRA-v9e0 ::=		SEQUENCE {

bandEUTRA-v9e0						FreqBandIndicator-v9e0		OPTIONAL

}

SupportedBandEUTRA-v1250 ::=		SEQUENCE {

dl-256QAM-r12						ENUMERATED {supported}		OPTIONAL,

ul-64QAM-r12						ENUMERATED {supported}		OPTIONAL

}

SupportedBandEUTRA-v1310 ::=		SEQUENCE {

ue-PowerClass-5-r13			ENUMERATED {supported}		OPTIONAL

}

SupportedBandEUTRA-v1320 ::=		SEQUENCE {

intraFreq-CE-NeedForGaps-r13				ENUMERATED {supported}				OPTIONAL,

ue-PowerClass-N-r13			ENUMERATED {class1, class2, class4}		OPTIONAL

}

MeasParameters ::=					SEQUENCE {

bandListEUTRA						BandListEUTRA

}

MeasParameters-v1020 ::=			SEQUENCE {

bandCombinationListEUTRA-r10			BandCombinationListEUTRA-r10

}

MeasParameters-v1130 ::=			SEQUENCE {

rsrqMeasWideband-r11			ENUMERATED {supported}					OPTIONAL

}

MeasParameters-v11a0 ::=			SEQUENCE {

benefitsFromInterruption-r11			ENUMERATED {true}				OPTIONAL

}

MeasParameters-v1250 ::=			SEQUENCE {

timerT312-r12						ENUMERATED {supported}		OPTIONAL,

alternativeTimeToTrigger-r12		ENUMERATED {supported}		OPTIONAL,

incMonEUTRA-r12						ENUMERATED {supported}		OPTIONAL,

incMonUTRA-r12						ENUMERATED {supported}		OPTIONAL,

extendedMaxMeasId-r12				ENUMERATED {supported}		OPTIONAL,

extendedRSRQ-LowerRange-r12			ENUMERATED {supported}		OPTIONAL,

rsrq-OnAllSymbols-r12				ENUMERATED {supported}		OPTIONAL,

crs-DiscoverySignalsMeas-r12		ENUMERATED {supported}		OPTIONAL,

csi-RS-DiscoverySignalsMeas-r12		ENUMERATED {supported}		OPTIONAL

}

MeasParameters-v1310 ::=			SEQUENCE {

rs-SINR-Meas-r13						ENUMERATED {supported}		OPTIONAL,

allowedCellList-r13						ENUMERATED {supported}		OPTIONAL,

extendedMaxObjectId-r13					ENUMERATED {supported}		OPTIONAL,

ul-PDCP-Delay-r13						ENUMERATED {supported}		OPTIONAL,

extendedFreqPriorities-r13				ENUMERATED {supported}		OPTIONAL,

multiBandInfoReport-r13					ENUMERATED {supported}		OPTIONAL,

rssi-AndChannelOccupancyReporting-r13	ENUMERATED {supported}		OPTIONAL

}

MeasParameters-v1430 ::=			SEQUENCE {

ceMeasurements-r14						ENUMERATED {supported}		OPTIONAL,

ncsg-r14								ENUMERATED {supported}				OPTIONAL,

shortMeasurementGap-r14					ENUMERATED {supported}				OPTIONAL,

perServingCellMeasurementGap-r14		ENUMERATED {supported}				OPTIONAL,

nonUniformGap-r14						ENUMERATED {supported}				OPTIONAL

}

MeasParameters-v1520 ::=			SEQUENCE {

measGapPatterns-r15					BIT STRING (SIZE (8))		OPTIONAL

}

MeasParameters-v1530 ::=			SEQUENCE {

qoe-MeasReport-r15					ENUMERATED {supported}		OPTIONAL,

qoe-MTSI-MeasReport-r15				ENUMERATED {supported}		OPTIONAL,

ca-IdleModeMeasurements-r15				ENUMERATED {supported}		OPTIONAL,

ca-IdleModeValidityArea-r15				ENUMERATED {supported}		OPTIONAL,

heightMeas-r15							ENUMERATED {supported}			OPTIONAL,

multipleCellsMeasExtension-r15			ENUMERATED {supported}			OPTIONAL

}

MeasParameters-v1610 ::=		SEQUENCE {

bandInfoNR-v1610					SEQUENCE (SIZE (1..maxBands)) OF MeasGapInfoNR-r16	OPTIONAL,

altFreqPriority-r16					ENUMERATED {supported}							OPTIONAL,

ce-DL-ChannelQualityReporting-r16	ENUMERATED {supported}							OPTIONAL,

ce-MeasRSS-Dedicated-r16			ENUMERATED {supported}							OPTIONAL,

eutra-IdleInactiveMeasurements-r16			ENUMERATED {supported}		OPTIONAL,

nr-IdleInactiveMeasFR1-r16			ENUMERATED {supported}		OPTIONAL,

nr-IdleInactiveMeasFR2-r16			ENUMERATED {supported}		OPTIONAL,

idleInactiveValidityAreaList-r16		ENUMERATED {supported}		OPTIONAL,

measGapPatterns-NRonly-r16			ENUMERATED {supported}		OPTIONAL,

measGapPatterns-NRonly-ENDC-r16		ENUMERATED {supported}		OPTIONAL

}

MeasParameters-v1630 ::=		SEQUENCE {

nr-IdleInactiveBeamMeasFR1-r16		ENUMERATED {supported}		OPTIONAL,

nr-IdleInactiveBeamMeasFR2-r16		ENUMERATED {supported}		OPTIONAL,

ce-MeasRSS-DedicatedSameRBs-r16		ENUMERATED {supported}		OPTIONAL

}

MeasParameters-v16c0 ::=		SEQUENCE {

nr-CellIndividualOffset-r16			ENUMERATED {supported}		OPTIONAL

}

MeasParameters-v1700 ::=		SEQUENCE {

sharedSpectrumMeasNR-EN-DC-r17	SEQUENCE (SIZE (1..maxBandsNR-r15)) OF SharedSpectrumMeasNR-r17	OPTIONAL,

sharedSpectrumMeasNR-SA-r17		SEQUENCE (SIZE (1..maxBandsNR-r15)) OF SharedSpectrumMeasNR-r17	OPTIONAL

}

SharedSpectrumMeasNR-r17 ::=		SEQUENCE {

nr-RSSI-ChannelOccupancyReporting-r17                  BOOLEAN

}

MeasGapInfoNR-r16 ::= SEQUENCE {

interRAT-BandListNR-EN-DC-r16		InterRAT-BandListNR-r16				OPTIONAL,

interRAT-BandListNR-SA-r16			InterRAT-BandListNR-r16				OPTIONAL

}

BandListEUTRA ::=					SEQUENCE (SIZE (1..maxBands)) OF BandInfoEUTRA

BandCombinationListEUTRA-r10 ::=	SEQUENCE (SIZE (1..maxBandComb-r10)) OF BandInfoEUTRA

BandInfoEUTRA ::=					SEQUENCE {

interFreqBandList					InterFreqBandList,

interRAT-BandList					InterRAT-BandList		OPTIONAL

}

InterFreqBandList ::=				SEQUENCE (SIZE (1..maxBands)) OF InterFreqBandInfo

InterFreqBandInfo ::=				SEQUENCE {

interFreqNeedForGaps				BOOLEAN

}

InterRAT-BandList ::=				SEQUENCE (SIZE (1..maxBands)) OF InterRAT-BandInfo

InterRAT-BandListNR-r16 ::=				SEQUENCE (SIZE (1..maxBandsNR-r15)) OF InterRAT-BandInfoNR-r16

InterRAT-BandInfo ::=				SEQUENCE {

interRAT-NeedForGaps				BOOLEAN

}

InterRAT-BandInfoNR-r16 ::=			SEQUENCE {

interRAT-NeedForGapsNR-r16			BOOLEAN

}

IRAT-ParametersNR-r15 ::=		SEQUENCE {

en-DC-r15							ENUMERATED {supported}						OPTIONAL,

eventB2-r15						ENUMERATED {supported}						OPTIONAL,

supportedBandListEN-DC-r15		SupportedBandListNR-r15						OPTIONAL

}

IRAT-ParametersNR-v1540 ::=		SEQUENCE {

eutra-5GC-HO-ToNR-FDD-FR1-r15		ENUMERATED {supported}				OPTIONAL,

eutra-5GC-HO-ToNR-TDD-FR1-r15		ENUMERATED {supported}				OPTIONAL,

eutra-5GC-HO-ToNR-FDD-FR2-r15		ENUMERATED {supported}				OPTIONAL,

eutra-5GC-HO-ToNR-TDD-FR2-r15		ENUMERATED {supported}				OPTIONAL,

eutra-EPC-HO-ToNR-FDD-FR1-r15		ENUMERATED {supported}				OPTIONAL,

eutra-EPC-HO-ToNR-TDD-FR1-r15		ENUMERATED {supported}				OPTIONAL,

eutra-EPC-HO-ToNR-FDD-FR2-r15		ENUMERATED {supported}				OPTIONAL,

eutra-EPC-HO-ToNR-TDD-FR2-r15		ENUMERATED {supported}				OPTIONAL,

ims-VoiceOverNR-FR1-r15				ENUMERATED {supported}				OPTIONAL,

ims-VoiceOverNR-FR2-r15				ENUMERATED {supported}				OPTIONAL,

sa-NR-r15								ENUMERATED {supported}				OPTIONAL,

supportedBandListNR-SA-r15			SupportedBandListNR-r15				OPTIONAL

}

IRAT-ParametersNR-v1560 ::=		SEQUENCE {

ng-EN-DC-r15							ENUMERATED {supported}				OPTIONAL

}

IRAT-ParametersNR-v1570 ::=		SEQUENCE {

ss-SINR-Meas-NR-FR1-r15				ENUMERATED {supported}				OPTIONAL,

ss-SINR-Meas-NR-FR2-r15				ENUMERATED {supported}				OPTIONAL

}

IRAT-ParametersNR-v1610 ::=		SEQUENCE {

nr-HO-ToEN-DC-r16					ENUMERATED {supported}				OPTIONAL,

ce-EUTRA-5GC-HO-ToNR-FDD-FR1-r16	ENUMERATED {supported}				OPTIONAL,

ce-EUTRA-5GC-HO-ToNR-TDD-FR1-r16	ENUMERATED {supported}				OPTIONAL,

ce-EUTRA-5GC-HO-ToNR-FDD-FR2-r16	ENUMERATED {supported}				OPTIONAL,

ce-EUTRA-5GC-HO-ToNR-TDD-FR2-r16	ENUMERATED {supported}				OPTIONAL

}

IRAT-ParametersNR-v1660 ::=		SEQUENCE {

extendedBand-n77-r16				ENUMERATED {supported}				OPTIONAL

}

IRAT-ParametersNR-v1700 ::=		SEQUENCE {

eutra-5GC-HO-ToNR-TDD-FR2-2-r17			ENUMERATED {supported}				OPTIONAL,

eutra-EPC-HO-ToNR-TDD-FR2-2-r17			ENUMERATED {supported}				OPTIONAL,

ce-EUTRA-5GC-HO-ToNR-TDD-FR2-2-r17		ENUMERATED {supported}				OPTIONAL,

ims-VoiceOverNR-FR2-2-r17				ENUMERATED {supported}				OPTIONAL

}

IRAT-ParametersNR-v1710 ::=		SEQUENCE {

extendedBand-n77-2-r17				ENUMERATED {supported}				OPTIONAL

}

EUTRA-5GC-Parameters-r15 ::=		SEQUENCE {

eutra-5GC-r15								ENUMERATED {supported}			OPTIONAL,

eutra-EPC-HO-EUTRA-5GC-r15				ENUMERATED {supported}			OPTIONAL,

ho-EUTRA-5GC-FDD-TDD-r15					ENUMERATED {supported}			OPTIONAL,

ho-InterfreqEUTRA-5GC-r15					ENUMERATED {supported}			OPTIONAL,

ims-VoiceOverMCG-BearerEUTRA-5GC-r15	ENUMERATED {supported}			OPTIONAL,

inactiveState-r15							ENUMERATED {supported}			OPTIONAL,

reflectiveQoS-r15							ENUMERATED {supported}			OPTIONAL

}

EUTRA-5GC-Parameters-v1610 ::=	SEQUENCE {

ce-InactiveState-r16			ENUMERATED {supported}			OPTIONAL,

ce-EUTRA-5GC-r16				ENUMERATED {supported}			OPTIONAL

}

PDCP-ParametersNR-r15 ::=		SEQUENCE {

rohc-Profiles-r15					ROHC-ProfileSupportList-r15,

rohc-ContextMaxSessions-r15			ENUMERATED {

cs2, cs4, cs8, cs12, cs16, cs24, cs32,

cs48, cs64, cs128, cs256, cs512, cs1024,

cs16384, spare2, spare1}			DEFAULT cs16,

rohc-ProfilesUL-Only-r15				SEQUENCE {

profile0x0006-r15						BOOLEAN

},

rohc-ContextContinue-r15			ENUMERATED {supported}				OPTIONAL,

outOfOrderDelivery-r15				ENUMERATED {supported}				OPTIONAL,

sn-SizeLo-r15						ENUMERATED {supported}				OPTIONAL,

ims-VoiceOverNR-PDCP-MCG-Bearer-r15	ENUMERATED {supported}				OPTIONAL,

ims-VoiceOverNR-PDCP-SCG-Bearer-r15	ENUMERATED {supported}				OPTIONAL

}

PDCP-ParametersNR-v1560 ::=		SEQUENCE {

ims-VoNR-PDCP-SCG-NGENDC-r15			ENUMERATED {supported}				OPTIONAL

}

ROHC-ProfileSupportList-r15 ::=	SEQUENCE {

profile0x0001-r15					BOOLEAN,

profile0x0002-r15					BOOLEAN,

profile0x0003-r15					BOOLEAN,

profile0x0004-r15					BOOLEAN,

profile0x0006-r15					BOOLEAN,

profile0x0101-r15					BOOLEAN,

profile0x0102-r15					BOOLEAN,

profile0x0103-r15					BOOLEAN,

profile0x0104-r15					BOOLEAN

}

SupportedBandListNR-r15 ::=		SEQUENCE (SIZE (1..maxBandsNR-r15)) OF SupportedBandNR-r15

SupportedBandNR-r15 ::=			SEQUENCE {

bandNR-r15							FreqBandIndicatorNR-r15

}

IRAT-ParametersUTRA-FDD ::=		SEQUENCE {

supportedBandListUTRA-FDD			SupportedBandListUTRA-FDD

}

IRAT-ParametersUTRA-v920 ::=		SEQUENCE {

e-RedirectionUTRA-r9				ENUMERATED {supported}

}

IRAT-ParametersUTRA-v9c0 ::=		SEQUENCE {

voiceOverPS-HS-UTRA-FDD-r9						ENUMERATED {supported}		OPTIONAL,

voiceOverPS-HS-UTRA-TDD128-r9					ENUMERATED {supported}		OPTIONAL,

srvcc-FromUTRA-FDD-ToUTRA-FDD-r9				ENUMERATED {supported}		OPTIONAL,

srvcc-FromUTRA-FDD-ToGERAN-r9					ENUMERATED {supported}		OPTIONAL,

srvcc-FromUTRA-TDD128-ToUTRA-TDD128-r9			ENUMERATED {supported}		OPTIONAL,

srvcc-FromUTRA-TDD128-ToGERAN-r9				ENUMERATED {supported}		OPTIONAL

}

IRAT-ParametersUTRA-v9h0 ::=		SEQUENCE {

mfbi-UTRA-r9						ENUMERATED {supported}

}

SupportedBandListUTRA-FDD ::=		SEQUENCE (SIZE (1..maxBands)) OF SupportedBandUTRA-FDD

SupportedBandUTRA-FDD ::=			ENUMERATED {

bandI, bandII, bandIII, bandIV, bandV, bandVI,

bandVII, bandVIII, bandIX, bandX, bandXI,

bandXII, bandXIII, bandXIV, bandXV, bandXVI, ...,

bandXVII-8a0, bandXVIII-8a0, bandXIX-8a0, bandXX-8a0,

bandXXI-8a0, bandXXII-8a0, bandXXIII-8a0, bandXXIV-8a0,

bandXXV-8a0, bandXXVI-8a0, bandXXVII-8a0, bandXXVIII-8a0,

bandXXIX-8a0, bandXXX-8a0, bandXXXI-8a0, bandXXXII-8a0}

IRAT-ParametersUTRA-TDD128 ::=		SEQUENCE {

supportedBandListUTRA-TDD128		SupportedBandListUTRA-TDD128

}

SupportedBandListUTRA-TDD128 ::=	SEQUENCE (SIZE (1..maxBands)) OF SupportedBandUTRA-TDD128

SupportedBandUTRA-TDD128 ::=		ENUMERATED {

a, b, c, d, e, f, g, h, i, j, k, l, m, n,

o, p, ...}

IRAT-ParametersUTRA-TDD384 ::=		SEQUENCE {

supportedBandListUTRA-TDD384		SupportedBandListUTRA-TDD384

}

SupportedBandListUTRA-TDD384 ::=	SEQUENCE (SIZE (1..maxBands)) OF SupportedBandUTRA-TDD384

SupportedBandUTRA-TDD384 ::=		ENUMERATED {

a, b, c, d, e, f, g, h, i, j, k, l, m, n,

o, p, ...}

IRAT-ParametersUTRA-TDD768 ::=		SEQUENCE {

supportedBandListUTRA-TDD768		SupportedBandListUTRA-TDD768

}

SupportedBandListUTRA-TDD768 ::=	SEQUENCE (SIZE (1..maxBands)) OF SupportedBandUTRA-TDD768

SupportedBandUTRA-TDD768 ::=		ENUMERATED {

a, b, c, d, e, f, g, h, i, j, k, l, m, n,

o, p, ...}

IRAT-ParametersUTRA-TDD-v1020 ::=		SEQUENCE {

e-RedirectionUTRA-TDD-r10				ENUMERATED {supported}

}

IRAT-ParametersGERAN ::=			SEQUENCE {

supportedBandListGERAN				SupportedBandListGERAN,

interRAT-PS-HO-ToGERAN				BOOLEAN

}

IRAT-ParametersGERAN-v920 ::=		SEQUENCE {

dtm-r9								ENUMERATED {supported}			OPTIONAL,

e-RedirectionGERAN-r9				ENUMERATED {supported}			OPTIONAL

}

SupportedBandListGERAN ::=			SEQUENCE (SIZE (1..maxBands)) OF SupportedBandGERAN

SupportedBandGERAN ::=				ENUMERATED {

gsm450, gsm480, gsm710, gsm750, gsm810, gsm850,

gsm900P, gsm900E, gsm900R, gsm1800, gsm1900,

spare5, spare4, spare3, spare2, spare1, ...}

IRAT-ParametersCDMA2000-HRPD ::=	SEQUENCE {

supportedBandListHRPD				SupportedBandListHRPD,

tx-ConfigHRPD						ENUMERATED {single, dual},

rx-ConfigHRPD						ENUMERATED {single, dual}

}

SupportedBandListHRPD ::=			SEQUENCE (SIZE (1..maxCDMA-BandClass)) OF BandclassCDMA2000

IRAT-ParametersCDMA2000-1XRTT ::=	SEQUENCE {

supportedBandList1XRTT				SupportedBandList1XRTT,

tx-Config1XRTT						ENUMERATED {single, dual},

rx-Config1XRTT						ENUMERATED {single, dual}

}

IRAT-ParametersCDMA2000-1XRTT-v920 ::=	SEQUENCE {

e-CSFB-1XRTT-r9						ENUMERATED {supported},

e-CSFB-ConcPS-Mob1XRTT-r9			ENUMERATED {supported}			OPTIONAL

}

IRAT-ParametersCDMA2000-1XRTT-v1020 ::=	SEQUENCE {

e-CSFB-dual-1XRTT-r10				ENUMERATED {supported}

}

IRAT-ParametersCDMA2000-v1130 ::=		SEQUENCE {

cdma2000-NW-Sharing-r11					ENUMERATED {supported}		OPTIONAL

}

SupportedBandList1XRTT ::=			SEQUENCE (SIZE (1..maxCDMA-BandClass)) OF BandclassCDMA2000

IRAT-ParametersWLAN-r13 ::=		SEQUENCE {

supportedBandListWLAN-r13		SEQUENCE (SIZE (1..maxWLAN-Bands-r13)) OF WLAN-BandIndicator-r13					OPTIONAL

}

CSG-ProximityIndicationParameters-r9 ::=	SEQUENCE {

intraFreqProximityIndication-r9		ENUMERATED {supported}			OPTIONAL,

interFreqProximityIndication-r9		ENUMERATED {supported}			OPTIONAL,

utran-ProximityIndication-r9		ENUMERATED {supported}			OPTIONAL

}

NeighCellSI-AcquisitionParameters-r9 ::=	SEQUENCE {

intraFreqSI-AcquisitionForHO-r9		ENUMERATED {supported}			OPTIONAL,

interFreqSI-AcquisitionForHO-r9		ENUMERATED {supported}			OPTIONAL,

utran-SI-AcquisitionForHO-r9		ENUMERATED {supported}			OPTIONAL

}

NeighCellSI-AcquisitionParameters-v1530 ::=	SEQUENCE {

reportCGI-NR-EN-DC-r15					ENUMERATED {supported}			OPTIONAL,

reportCGI-NR-NoEN-DC-r15				ENUMERATED {supported}			OPTIONAL

}

NeighCellSI-AcquisitionParameters-v1550 ::=	SEQUENCE {

eutra-CGI-Reporting-ENDC-r15				ENUMERATED {supported}			OPTIONAL,

utra-GERAN-CGI-Reporting-ENDC-r15			ENUMERATED {supported}			OPTIONAL

}

NeighCellSI-AcquisitionParameters-v15a0 ::=	SEQUENCE {

eutra-CGI-Reporting-NEDC-r15				ENUMERATED {supported}			OPTIONAL

}

NeighCellSI-AcquisitionParameters-v1610 ::=	SEQUENCE {

eutra-SI-AcquisitionForHO-ENDC-r16			ENUMERATED {supported}			OPTIONAL,

nr-AutonomousGaps-ENDC-FR1-r16				ENUMERATED {supported}			OPTIONAL,

nr-AutonomousGaps-ENDC-FR2-r16				ENUMERATED {supported}			OPTIONAL,

nr-AutonomousGaps-FR1-r16					ENUMERATED {supported}			OPTIONAL,

nr-AutonomousGaps-FR2-r16					ENUMERATED {supported}			OPTIONAL

}

NeighCellSI-AcquisitionParameters-v1710 ::=	SEQUENCE {

gNB-ID-Length-Reporting-NR-EN-DC-r17			ENUMERATED {supported}			OPTIONAL,

gNB-ID-Length-Reporting-NR-NoEN-DC-r17		ENUMERATED {supported}			OPTIONAL

}

SON-Parameters-r9 ::=				SEQUENCE {

rach-Report-r9						ENUMERATED {supported}			OPTIONAL

}

SON-Parameters-v18xy ::=				SEQUENCE {

rach-Report-ForNR-r18						ENUMERATED {supported}			OPTIONAL

}

PUR-Parameters-r16 ::=				SEQUENCE {

pur-CP-5GC-CE-ModeA-r16				ENUMERATED {supported}			OPTIONAL,

pur-CP-5GC-CE-ModeB-r16				ENUMERATED {supported}			OPTIONAL,

pur-UP-5GC-CE-ModeA-r16				ENUMERATED {supported}			OPTIONAL,

pur-UP-5GC-CE-ModeB-r16				ENUMERATED {supported}			OPTIONAL,

pur-CP-EPC-CE-ModeA-r16				ENUMERATED {supported}			OPTIONAL,

pur-CP-EPC-CE-ModeB-r16				ENUMERATED {supported}			OPTIONAL,

pur-UP-EPC-CE-ModeA-r16				ENUMERATED {supported}			OPTIONAL,

pur-UP-EPC-CE-ModeB-r16				ENUMERATED {supported}			OPTIONAL,

pur-CP-L1Ack-r16					ENUMERATED {supported}			OPTIONAL,

pur-FrequencyHopping-r16			ENUMERATED {supported}			OPTIONAL,

pur-PUSCH-NB-MaxTBS-r16				ENUMERATED {supported}			OPTIONAL,

pur-RSRP-Validation-r16				ENUMERATED {supported}			OPTIONAL,

pur-SubPRB-CE-ModeA-r16				ENUMERATED {supported}			OPTIONAL,

pur-SubPRB-CE-ModeB-r16				ENUMERATED {supported}			OPTIONAL

}

UE-BasedNetwPerfMeasParameters-r10 ::=	SEQUENCE {

loggedMeasurementsIdle-r10				ENUMERATED {supported}		OPTIONAL,

standaloneGNSS-Location-r10				ENUMERATED {supported}		OPTIONAL

}

UE-BasedNetwPerfMeasParameters-v1250 ::=	SEQUENCE {

loggedMBSFNMeasurements-r12				ENUMERATED {supported}

}

UE-BasedNetwPerfMeasParameters-v1430 ::=	SEQUENCE {

locationReport-r14						ENUMERATED {supported}		OPTIONAL

}

UE-BasedNetwPerfMeasParameters-v1530 ::=	SEQUENCE {

loggedMeasBT-r15						ENUMERATED {supported}		OPTIONAL,

loggedMeasWLAN-r15						ENUMERATED {supported}		OPTIONAL,

immMeasBT-r15							ENUMERATED {supported}		OPTIONAL,

immMeasWLAN-r15							ENUMERATED {supported}		OPTIONAL

}

UE-BasedNetwPerfMeasParameters-v1610 ::=	SEQUENCE {

ul-PDCP-AvgDelay-r16						ENUMERATED {supported}		OPTIONAL

}

UE-BasedNetwPerfMeasParameters-v1700 ::=	SEQUENCE {

loggedMeasIdleEventL1-r17					ENUMERATED {supported}		OPTIONAL,

loggedMeasIdleEventOutOfCoverage-r17		ENUMERATED {supported}		OPTIONAL,

loggedMeasUncomBarPre-r17					ENUMERATED {supported}		OPTIONAL,

immMeasUncomBarPre-r17					ENUMERATED {supported}		OPTIONAL

}

UE-BasedNetwPerfMeasParameters-v18xy ::=	SEQUENCE {

sigBasedLTELoggedMeas-OverrideProtect-r18		ENUMERATED {supported}			OPTIONAL

}

OTDOA-PositioningCapabilities-r10 ::=	SEQUENCE {

otdoa-UE-Assisted-r10					ENUMERATED {supported},

interFreqRSTD-Measurement-r10			ENUMERATED {supported}		OPTIONAL

}

Other-Parameters-r11 ::=				SEQUENCE {

inDeviceCoexInd-r11						ENUMERATED {supported}		OPTIONAL,

powerPrefInd-r11						ENUMERATED {supported}		OPTIONAL,

ue-Rx-TxTimeDiffMeasurements-r11		ENUMERATED {supported}		OPTIONAL

}

Other-Parameters-v11d0 ::=				SEQUENCE {

inDeviceCoexInd-UL-CA-r11				ENUMERATED {supported}		OPTIONAL

}

Other-Parameters-v1360 ::=	SEQUENCE {

inDeviceCoexInd-HardwareSharingInd-r13		ENUMERATED {supported}		OPTIONAL

}

Other-Parameters-v1430 ::=			SEQUENCE {

bwPrefInd-r14					ENUMERATED {supported}		OPTIONAL,

rlm-ReportSupport-r14			ENUMERATED {supported}		OPTIONAL

}

OtherParameters-v1450 ::=	SEQUENCE {

overheatingInd-r14				ENUMERATED {supported}		OPTIONAL

}

Other-Parameters-v1460 ::=	SEQUENCE {

nonCSG-SI-Reporting-r14			ENUMERATED {supported}		OPTIONAL

}

Other-Parameters-v1530 ::=			SEQUENCE {

assistInfoBitForLC-r15			ENUMERATED {supported}		OPTIONAL,

timeReferenceProvision-r15		ENUMERATED {supported}		OPTIONAL,

flightPathPlan-r15				ENUMERATED {supported}		OPTIONAL

}

Other-Parameters-v1540 ::=			SEQUENCE {

inDeviceCoexInd-ENDC-r15		ENUMERATED {supported}		OPTIONAL

}

Other-Parameters-v1610 ::=		SEQUENCE {

resumeWithStoredMCG-SCells-r16	ENUMERATED {supported}		OPTIONAL,

resumeWithMCG-SCellConfig-r16	ENUMERATED {supported}		OPTIONAL,

resumeWithStoredSCG-r16			ENUMERATED {supported}		OPTIONAL,

resumeWithSCG-Config-r16		ENUMERATED {supported}		OPTIONAL,

mcgRLF-RecoveryViaSCG-r16		ENUMERATED {supported}		OPTIONAL,

overheatingIndForSCG-r16		ENUMERATED {supported}		OPTIONAL

}

Other-Parameters-v1650 ::=		SEQUENCE {

mpsPriorityIndication-r16			ENUMERATED {supported}		OPTIONAL

}

Other-Parameters-v1690 ::=		SEQUENCE {

ul-RRC-Segmentation-r16			ENUMERATED {supported}			OPTIONAL

}

MBMS-Parameters-r11 ::=				SEQUENCE {

mbms-SCell-r11							ENUMERATED {supported}		OPTIONAL,

mbms-NonServingCell-r11					ENUMERATED {supported}		OPTIONAL

}

MBMS-Parameters-v1250 ::=				SEQUENCE {

mbms-AsyncDC-r12						ENUMERATED {supported}		OPTIONAL

}

MBMS-Parameters-v1430 ::=				SEQUENCE {

fembmsDedicatedCell-r14				ENUMERATED {supported}		OPTIONAL,

fembmsMixedCell-r14					ENUMERATED {supported}		OPTIONAL,

subcarrierSpacingMBMS-khz7dot5-r14	ENUMERATED {supported}		OPTIONAL,

subcarrierSpacingMBMS-khz1dot25-r14	ENUMERATED {supported}		OPTIONAL

}

MBMS-Parameters-v1470 ::=		SEQUENCE {

mbms-MaxBW-r14					CHOICE {

implicitValue					NULL,

explicitValue					INTEGER(2..20)

},

mbms-ScalingFactor1dot25-r14		ENUMERATED {n3, n6, n9, n12}	OPTIONAL,

mbms-ScalingFactor7dot5-r14		ENUMERATED {n1, n2, n3, n4}		OPTIONAL

}

MBMS-Parameters-v1610 ::=		SEQUENCE {

mbms-ScalingFactor2dot5-r16		ENUMERATED {n2, n4, n6, n8}			OPTIONAL,

mbms-ScalingFactor0dot37-r16	ENUMERATED {n12, n16, n20, n24}		OPTIONAL,

mbms-SupportedBandInfoList-r16	SEQUENCE (SIZE (1..maxBands)) OF MBMS-SupportedBandInfo-r16

}

MBMS-Parameters-v1700 ::=		SEQUENCE {

mbms-SupportedBandInfoList-v1700		SEQUENCE (SIZE (1..maxBands)) OF MBMS-SupportedBandInfo-v1700		OPTIONAL

}

MBMS-SupportedBandInfo-r16 ::=		SEQUENCE {

subcarrierSpacingMBMS-khz2dot5-r16	ENUMERATED {supported}		OPTIONAL,

subcarrierSpacingMBMS-khz0dot37-r16	SEQUENCE {

timeSeparationSlot2-r16			ENUMERATED {supported}			OPTIONAL,

timeSeparationSlot4-r16			ENUMERATED {supported}			OPTIONAL

}	OPTIONAL

}

MBMS-SupportedBandInfo-v1700 ::=	SEQUENCE {

pmch-Bandwidth-n40-r17				ENUMERATED {supported}		OPTIONAL,

pmch-Bandwidth-n35-r17				ENUMERATED {supported}		OPTIONAL,

pmch-Bandwidth-n30-r17				ENUMERATED {supported}		OPTIONAL

}

FeMBMS-Unicast-Parameters-r14 ::=		SEQUENCE {

unicast-fembmsMixedSCell-r14			ENUMERATED {supported}		OPTIONAL,

emptyUnicastRegion-r14					ENUMERATED {supported}		OPTIONAL

}

SCPTM-Parameters-r13 ::=				SEQUENCE {

scptm-ParallelReception-r13					ENUMERATED {supported}		OPTIONAL,

scptm-SCell-r13								ENUMERATED {supported}		OPTIONAL,

scptm-NonServingCell-r13					ENUMERATED {supported}		OPTIONAL,

scptm-AsyncDC-r13							ENUMERATED {supported}		OPTIONAL

}

CE-Parameters-r13 ::=		SEQUENCE {

ce-ModeA-r13						ENUMERATED {supported}				OPTIONAL,

ce-ModeB-r13						ENUMERATED {supported}				OPTIONAL

}

CE-Parameters-v1320 ::=		SEQUENCE {

intraFreqA3-CE-ModeA-r13				ENUMERATED {supported}				OPTIONAL,

intraFreqA3-CE-ModeB-r13				ENUMERATED {supported}				OPTIONAL,

intraFreqHO-CE-ModeA-r13				ENUMERATED {supported}				OPTIONAL,

intraFreqHO-CE-ModeB-r13				ENUMERATED {supported}				OPTIONAL

}

CE-Parameters-v1350 ::=		SEQUENCE {

unicastFrequencyHopping-r13				ENUMERATED {supported}				OPTIONAL

}

CE-Parameters-v1370 ::=		SEQUENCE {

tm9-CE-ModeA-r13						ENUMERATED {supported}			OPTIONAL,

tm9-CE-ModeB-r13						ENUMERATED {supported}			OPTIONAL

}

CE-Parameters-v1380 ::=		SEQUENCE {

tm6-CE-ModeA-r13						ENUMERATED {supported}			OPTIONAL

}

CE-Parameters-v1430 ::=		SEQUENCE {

ce-SwitchWithoutHO-r14					ENUMERATED {supported}				OPTIONAL

}

CE-MultiTB-Parameters-r16 ::=	SEQUENCE {

pdsch-MultiTB-CE-ModeA-r16			ENUMERATED {supported}			OPTIONAL,

pdsch-MultiTB-CE-ModeB-r16			ENUMERATED {supported}			OPTIONAL,

pusch-MultiTB-CE-ModeA-r16			ENUMERATED {supported}			OPTIONAL,

pusch-MultiTB-CE-ModeB-r16			ENUMERATED {supported}			OPTIONAL,

ce-MultiTB-64QAM-r16				ENUMERATED {supported}			OPTIONAL,

ce-MultiTB-EarlyTermination-r16	ENUMERATED {supported}			OPTIONAL,

ce-MultiTB-FrequencyHopping-r16		ENUMERATED {supported}			OPTIONAL,

ce-MultiTB-HARQ-AckBundling-r16		ENUMERATED {supported}			OPTIONAL,

ce-MultiTB-Interleaving-r16			ENUMERATED {supported}			OPTIONAL,

ce-MultiTB-SubPRB-r16					ENUMERATED {supported}			OPTIONAL

}

CE-ResourceResvParameters-r16 ::=	SEQUENCE {

subframeResourceResvDL-CE-ModeA-r16	ENUMERATED {supported}			OPTIONAL,

subframeResourceResvDL-CE-ModeB-r16	ENUMERATED {supported}			OPTIONAL,

subframeResourceResvUL-CE-ModeA-r16	ENUMERATED {supported}			OPTIONAL,

subframeResourceResvUL-CE-ModeB-r16	ENUMERATED {supported}			OPTIONAL,

slotSymbolResourceResvDL-CE-ModeA-r16	ENUMERATED {supported}			OPTIONAL,

slotSymbolResourceResvDL-CE-ModeB-r16	ENUMERATED {supported}			OPTIONAL,

slotSymbolResourceResvUL-CE-ModeA-r16	ENUMERATED {supported}			OPTIONAL,

slotSymbolResourceResvUL-CE-ModeB-r16	ENUMERATED {supported}			OPTIONAL,

subcarrierPuncturingCE-ModeA-r16			ENUMERATED {supported}			OPTIONAL,

subcarrierPuncturingCE-ModeB-r16			ENUMERATED {supported}			OPTIONAL

}

LAA-Parameters-r13 ::=				SEQUENCE {

crossCarrierSchedulingLAA-DL-r13			ENUMERATED {supported}		OPTIONAL,

csi-RS-DRS-RRM-MeasurementsLAA-r13			ENUMERATED {supported}		OPTIONAL,

downlinkLAA-r13								ENUMERATED {supported}		OPTIONAL,

endingDwPTS-r13								ENUMERATED {supported}		OPTIONAL,

secondSlotStartingPosition-r13				ENUMERATED {supported}		OPTIONAL,

tm9-LAA-r13									ENUMERATED {supported}		OPTIONAL,

tm10-LAA-r13								ENUMERATED {supported}		OPTIONAL

}

LAA-Parameters-v1430 ::=				SEQUENCE {

crossCarrierSchedulingLAA-UL-r14			ENUMERATED {supported}		OPTIONAL,

uplinkLAA-r14								ENUMERATED {supported}		OPTIONAL,

twoStepSchedulingTimingInfo-r14				ENUMERATED {nPlus1, nPlus2, nPlus3}	OPTIONAL,

uss-BlindDecodingAdjustment-r14				ENUMERATED {supported}		OPTIONAL,

uss-BlindDecodingReduction-r14				ENUMERATED {supported}		OPTIONAL,

outOfSequenceGrantHandling-r14				ENUMERATED {supported}		OPTIONAL

}

LAA-Parameters-v1530 ::=				SEQUENCE {

aul-r15										ENUMERATED {supported}		OPTIONAL,

laa-PUSCH-Mode1-r15							ENUMERATED {supported}		OPTIONAL,

laa-PUSCH-Mode2-r15							ENUMERATED {supported}		OPTIONAL,

laa-PUSCH-Mode3-r15							ENUMERATED {supported}		OPTIONAL

}

WLAN-IW-Parameters-r12 ::=	SEQUENCE {

wlan-IW-RAN-Rules-r12					ENUMERATED {supported}		OPTIONAL,

wlan-IW-ANDSF-Policies-r12						ENUMERATED {supported}		OPTIONAL

}

LWA-Parameters-r13 ::=		SEQUENCE {

lwa-r13						ENUMERATED {supported}		OPTIONAL,

lwa-SplitBearer-r13			ENUMERATED {supported}		OPTIONAL,

wlan-MAC-Address-r13		OCTET STRING (SIZE (6))		OPTIONAL,

lwa-BufferSize-r13			ENUMERATED {supported}		OPTIONAL

}

LWA-Parameters-v1430 ::=		SEQUENCE {

lwa-HO-WithoutWT-Change-r14			ENUMERATED {supported}		OPTIONAL,

lwa-UL-r14							ENUMERATED {supported}		OPTIONAL,

wlan-PeriodicMeas-r14				ENUMERATED {supported}		OPTIONAL,

wlan-ReportAnyWLAN-r14				ENUMERATED {supported}		OPTIONAL,

wlan-SupportedDataRate-r14			INTEGER (1..2048)			OPTIONAL

}

LWA-Parameters-v1440 ::=		SEQUENCE {

lwa-RLC-UM-r14						ENUMERATED {supported}		OPTIONAL

}

WLAN-IW-Parameters-v1310 ::=	SEQUENCE {

rclwi-r13										ENUMERATED {supported}		OPTIONAL

}

LWIP-Parameters-r13 ::=		SEQUENCE {

lwip-r13					ENUMERATED {supported}				OPTIONAL

}

LWIP-Parameters-v1430 ::=		SEQUENCE {

lwip-Aggregation-DL-r14					ENUMERATED {supported}				OPTIONAL,

lwip-Aggregation-UL-r14					ENUMERATED {supported}				OPTIONAL

}

NAICS-Capability-List-r12 ::= SEQUENCE (SIZE (1..maxNAICS-Entries-r12)) OF NAICS-Capability-Entry-r12

NAICS-Capability-Entry-r12	::=	SEQUENCE {

numberOfNAICS-CapableCC-r12				INTEGER(1..5),

numberOfAggregatedPRB-r12				ENUMERATED {

n50, n75, n100, n125, n150, n175,

n200, n225, n250, n275, n300, n350,

n400, n450, n500, spare},

...

}

SL-Parameters-r12 ::=				SEQUENCE {

commSimultaneousTx-r12					ENUMERATED {supported}		OPTIONAL,

commSupportedBands-r12					FreqBandIndicatorListEUTRA-r12	OPTIONAL,

discSupportedBands-r12					SupportedBandInfoList-r12	OPTIONAL,

discScheduledResourceAlloc-r12			ENUMERATED {supported}		OPTIONAL,

disc-UE-SelectedResourceAlloc-r12		ENUMERATED {supported}		OPTIONAL,

disc-SLSS-r12							ENUMERATED {supported}		OPTIONAL,

discSupportedProc-r12					ENUMERATED {n50, n400}		OPTIONAL

}

SL-Parameters-v1310 ::=				SEQUENCE {

discSysInfoReporting-r13					ENUMERATED {supported}		OPTIONAL,

commMultipleTx-r13							ENUMERATED {supported}		OPTIONAL,

discInterFreqTx-r13							ENUMERATED {supported}		OPTIONAL,

discPeriodicSLSS-r13						ENUMERATED {supported}		OPTIONAL

}

SL-Parameters-v1430 ::=				SEQUENCE {

zoneBasedPoolSelection-r14				ENUMERATED {supported}				OPTIONAL,

ue-AutonomousWithFullSensing-r14		ENUMERATED {supported}				OPTIONAL,

ue-AutonomousWithPartialSensing-r14		ENUMERATED {supported}				OPTIONAL,

sl-CongestionControl-r14				ENUMERATED {supported}				OPTIONAL,

v2x-TxWithShortResvInterval-r14			ENUMERATED {supported}				OPTIONAL,

v2x-numberTxRxTiming-r14				INTEGER(1..16)						OPTIONAL,

v2x-nonAdjacentPSCCH-PSSCH-r14			ENUMERATED {supported}				OPTIONAL,

slss-TxRx-r14							ENUMERATED {supported}				OPTIONAL,

v2x-SupportedBandCombinationList-r14	V2X-SupportedBandCombination-r14	OPTIONAL

}

SL-Parameters-v1530 ::=				SEQUENCE {

slss-SupportedTxFreq-r15				ENUMERATED {single, multiple}		OPTIONAL,

sl-64QAM-Tx-r15						ENUMERATED {supported}				OPTIONAL,

sl-TxDiversity-r15						ENUMERATED {supported}				OPTIONAL,

ue-CategorySL-r15						UE-CategorySL-r15					OPTIONAL,

v2x-SupportedBandCombinationList-v1530	V2X-SupportedBandCombination-v1530	OPTIONAL

}

SL-Parameters-v1540 ::=				SEQUENCE {

sl-64QAM-Rx-r15						ENUMERATED {supported}				OPTIONAL,

sl-RateMatchingTBSScaling-r15			ENUMERATED {supported}				OPTIONAL,

sl-LowT2min-r15							ENUMERATED {supported}				OPTIONAL,

v2x-SensingReportingMode3-r15			ENUMERATED {supported}				OPTIONAL

}

SL-Parameters-v1610 ::=		SEQUENCE {

sl-ParameterNR-r16			OCTET STRING								OPTIONAL,

dummy						V2X-SupportedBandCombinationEUTRA-NR-r16	OPTIONAL

}

SL-Parameters-v1630 ::=					SEQUENCE {

v2x-SupportedBandCombinationListEUTRA-NR-r16	V2X-SupportedBandCombinationEUTRA-NR-v1630	OPTIONAL

}

SL-Parameters-v1710 ::=					SEQUENCE {

v2x-SupportedBandCombinationListEUTRA-NR-v1710	V2X-SupportedBandCombinationEUTRA-NR-v1710	OPTIONAL

}

UE-CategorySL-r15 ::=			SEQUENCE {

ue-CategorySL-C-TX-r15				INTEGER(1..5),

ue-CategorySL-C-RX-r15				INTEGER(1..4)

}

V2X-SupportedBandCombination-r14 ::=		SEQUENCE (SIZE (1..maxBandComb-r13)) OF V2X-BandCombinationParameters-r14

V2X-SupportedBandCombination-v1530	::=		SEQUENCE (SIZE (1..maxBandComb-r13)) OF V2X-BandCombinationParameters-v1530

V2X-BandCombinationParameters-r14 ::=	SEQUENCE (SIZE (1.. maxSimultaneousBands-r10)) OF V2X-BandParameters-r14

V2X-BandCombinationParameters-v1530 ::=	SEQUENCE (SIZE (1.. maxSimultaneousBands-r10)) OF V2X-BandParameters-v1530

V2X-SupportedBandCombinationEUTRA-NR-r16	::=	SEQUENCE (SIZE (1..maxBandCombSidelinkNR-r16)) OF V2X-BandParametersEUTRA-NR-r16

V2X-SupportedBandCombinationEUTRA-NR-v1630	::=	SEQUENCE (SIZE (1..maxBandCombSidelinkNR-r16)) OF V2X-BandCombinationParametersEUTRA-NR-v1630

V2X-SupportedBandCombinationEUTRA-NR-v1710 ::=	SEQUENCE (SIZE (1..maxBandCombSidelinkNR-r16)) OF V2X-BandCombinationParametersEUTRA-NR-v1710

V2X-BandCombinationParametersEUTRA-NR-v1630 ::=	SEQUENCE {

bandListSidelinkEUTRA-NR-r16					SEQUENCE (SIZE (1.. maxSimultaneousBands-r10)) OF V2X-BandParametersEUTRA-NR-r16,

bandListSidelinkEUTRA-NR-v1630					SEQUENCE (SIZE (1.. maxSimultaneousBands-r10)) OF V2X-BandParametersEUTRA-NR-v1630

}

V2X-BandCombinationParametersEUTRA-NR-v1710 ::=	SEQUENCE (SIZE (1..maxSimultaneousBands-r10)) OF V2X-BandParametersEUTRA-NR-v1710

V2X-BandParametersEUTRA-NR-r16 ::=	CHOICE {

eutra									SEQUENCE {

v2x-BandParameters1-r16				V2X-BandParameters-r14		OPTIONAL,

v2x-BandParameters2-r16				V2X-BandParameters-v1530		OPTIONAL

},

nr										SEQUENCE {

v2x-BandParametersNR-r16					OCTET STRING				OPTIONAL

}

}

V2X-BandParametersEUTRA-NR-v1630 ::=	CHOICE {

eutra									NULL,

nr										SEQUENCE {

tx-Sidelink-r16							ENUMERATED {supported}	OPTIONAL,

rx-Sidelink-r16							ENUMERATED {supported}	OPTIONAL

}

}

V2X-BandParametersEUTRA-NR-v1710 ::=	SEQUENCE {

v2x-BandParametersEUTRA-NR-v1710				OCTET STRING			OPTIONAL

}

SupportedBandInfoList-r12 ::=		SEQUENCE (SIZE (1..maxBands)) OF SupportedBandInfo-r12

SupportedBandInfo-r12 ::=			SEQUENCE {

support-r12								ENUMERATED {supported}	OPTIONAL

}

FreqBandIndicatorListEUTRA-r12 ::=		SEQUENCE (SIZE (1..maxBands)) OF FreqBandIndicator-r11

MMTEL-Parameters-r14 ::=			SEQUENCE {

delayBudgetReporting-r14					ENUMERATED {supported}		OPTIONAL,

pusch-Enhancements-r14						ENUMERATED {supported}		OPTIONAL,

recommendedBitRate-r14						ENUMERATED {supported}		OPTIONAL,

recommendedBitRateQuery-r14					ENUMERATED {supported}		OPTIONAL

}

MMTEL-Parameters-v1610 ::=				SEQUENCE {

recommendedBitRateMultiplier-r16			ENUMERATED {supported}			OPTIONAL

}

SRS-CapabilityPerBandPair-r14 ::= SEQUENCE {

retuningInfo				SEQUENCE {

rf-RetuningTimeDL-r14			ENUMERATED {n0, n0dot5, n1, n1dot5, n2, n2dot5, n3,

n3dot5, n4, n4dot5, n5, n5dot5, n6, n6dot5,

n7, spare1}		OPTIONAL,

rf-RetuningTimeUL-r14			ENUMERATED {n0, n0dot5, n1, n1dot5, n2, n2dot5, n3,

n3dot5, n4, n4dot5, n5, n5dot5, n6, n6dot5,

n7, spare1}		OPTIONAL

}

}

SRS-CapabilityPerBandPair-v14b0 ::= SEQUENCE {

srs-FlexibleTiming-r14				ENUMERATED {supported}		OPTIONAL,

srs-HARQ-ReferenceConfig-r14			ENUMERATED {supported}		OPTIONAL

}

SRS-CapabilityPerBandPair-v1610::= SEQUENCE {

addSRS-CarrierSwitching-r16				ENUMERATED {supported}		OPTIONAL

}

HighSpeedEnhParameters-r14 ::= SEQUENCE {

measurementEnhancements-r14		ENUMERATED {supported}		OPTIONAL,

demodulationEnhancements-r14	ENUMERATED {supported}		OPTIONAL,

prach-Enhancements-r14			ENUMERATED {supported}		OPTIONAL

}

HighSpeedEnhParameters-v1610 ::= SEQUENCE {

measurementEnhancementsSCell-r16	ENUMERATED {supported}		OPTIONAL,

measurementEnhancements2-r16		ENUMERATED {supported}		OPTIONAL,

demodulationEnhancements2-r16	ENUMERATED {supported}		OPTIONAL,

interRAT-enhancementNR-r16		ENUMERATED {supported}		OPTIONAL

}

-- ASN1STOP

| UE-EUTRA-Capability field descriptions |  | FDD/ TDD diff |
| --- | --- | --- |
| accessStratumRelease Set to rel17 in this version of the specification. NOTE 7. |  | - |
| additionalRx-Tx-PerformanceReq Indicates whether the UE supports the additional Rx and Tx performance requirement for a given band combination as specified in TS 36.101 [42]. |  | - |
| addSRS Presence of this field indicates the UE supports the additional SRS symbol(s) within the normal UL subframes in TDD as described in TS 36.213 [23]. |  | - |
| addSRS-1T2R Indicates whether the UE supports selecting one antenna among two antennas to transmit additional SRS symbol(s) for the corresponding band of the band combination as described in TS 36.213 [23]. |  | - |
| addSRS-1T4R Indicates whether the UE supports selecting one antenna among four antennas to transmit additional SRS symbol(s) for the corresponding band of the band combination as described in TS 36.213 [23]. |  | - |
| addSRS-2T4R-2Pairs Indicates whether the UE supports selecting one antenna pair between two antenna pairs to transmit additional SRS symbol(s) simultaneously for the corresponding band of the band combination as described in TS 36.213 [23]. |  | - |
| addSRS-2T4R-3Pairs Indicates whether the UE supports selecting one antenna pair among three antenna pairs to transmit additional SRS symbol(s) simultaneously for the corresponding band of the band combination as described in TS 36.213 [23]. |  | - |
| addSRS-AntennaSwitching (in addSRS) Value useBasic indicates the antenna switching capabilities for additional SRS symbol(s) for a band of band combination for which the capability is not signalled in bandParameterList-v1610 is the same as indicated by bandParameterList-v1380 and/or bandParameterList-v1530 for the concerned band of band combination. |  | - |
| addSRS-AntennaSwitching (in bandParameterList-v1610) If signalled, the field indicates the antenna switching capabilities for additional SRS symbol(s) for the concerned band of band combination. |  | - |
| addSRS-CarrierSwitching (in addSRS) Indicates whether carrier switching is supported for additional SRS symbol(s) for all band pairs of band combinations for which UE supports SRS carrier switching. This field is included only if srs-CapabilityPerBandPairList-r14 is included. If this field is included, addSRS-CarrierSwitching (in bandParameterList-v1610) is not included. |  | - |
| addSRS-CarrierSwitching (in bandParameterList-v1610) Indicates whether carrier switching is supported for additional SRS symbol(s) for the concerned band pair of band combination. This field is included only if srs-CapabilityPerBandPairList-r14 is included.If this field is included, addSRS-CarrierSwitching (in addSRS) is not included. |  | - |
| addSRS-FrequencyHopping (in addSRS) Indicates whether frequency hopping is supported for additional SRS symbol(s) for all bands of band combinations for which the capability is not signalled in bandParameterList-v1610. |  | - |
| addSRS-FrequencyHopping (in bandParameterList-v1610) If signalled, the field indicates whether frequency hopping is supported for additional SRS symbol(s) for the concerned band of band combination. |  | - |
| allowedCellList Indicates whether the UE supports EUTRA allowed-cell listing to limit the set of cells applicable for measurements. |  | - |
| alternativeTBS-Indices Indicates whether the UE supports alternative TBS indices ITBS 26A and 33A as specified in TS 36.213 [23]. |  | - |
| alternativeTBS-Index Indicates whether the UE supports alternative TBS index ITBS 33B as specified in TS 36.213 [23]. |  | No |
| alternativeTimeToTrigger Indicates whether the UE supports alternativeTimeToTrigger. |  | No |
| altFreqPriority Indicates whether the UE supports alternative cell reselection priority. |  | No |
| altMCS-Table Indicates whether the UE supports the 6-bit MCS table as specified in TS 36.212 [22] and TS 36.213 [23]. |  | Yes |
| aperiodicCSI-Reporting Indicates whether the UE supports aperiodic CSI reporting with 3 bits of the CSI request field size as specified in TS 36.213 [23], clause 7.2.1 and/or aperiodic CSI reporting mode 1-0 and mode 1-1 as specified in TS 36.213 [23], clause 7.2.1. The first bit is set to "1" if the UE supports the aperiodic CSI reporting with 3 bits of the CSI request field size. The second bit is set to "1" if the UE supports the aperiodic CSI reporting mode 1-0 and mode 1-1. |  | No |
| aperiodicCsi-ReportingSTTI Indicates whether the UE supports aperiodic CSI reporting for short TTI as specified in TS 36.213 [23], clause 7.2.1. |  | Yes |
| appliedCapabilityFilterCommon Contains the filter, applied by the UE, common for all MR-DC related capability containers that are requested and as defined by UE-CapabilityRequestFilterCommon IE in TS 38.331 [82]. |  | - |
| assistInfoBitForLC Indicates whether the UE supports assistance information bit for local cache. |  | - |
| aul Indicates whether the UE supports AUL as specified n TS 36.321 [6]. |  | - |
| bandCombinationListEUTRA One entry corresponding to each supported band combination listed in the same order as in supportedBandCombination. |  | - |
| BandCombinationParameters-v1090, BandCombinationParameters-v10i0, BandCombinationParameters-v1270 If included, the UE shall include the same number of entries, and listed in the same order, as in BandCombinationParameters-r10. |  | - |
| BandCombinationParameters-v1130 The field is applicable to each supported CA bandwidth class combination (i.e. CA configuration in TS 36.101 [42], clause 5.6A.1) indicated in the corresponding band combination. If included, the UE shall include the same number of entries, and listed in the same order, as in BandCombinationParameters-r10. |  | - |
| bandEUTRA E-UTRA band as defined in TS 36.101 [42] and TS 36.102 [113] for NTN capable UE. In case the UE includes bandEUTRA-v9e0 or bandEUTRA-v1090, the UE shall set the corresponding entry of bandEUTRA (i.e. without suffix) or bandEUTRA-r10 respectively to maxFBI. |  | - |
| bandInfoNR-v1610 One entry corresponding to each supported E-UTRA band listed in the same order as in supportedBandListEUTRA. If absent, network assumes gap is required when measurement is performed on any NR bands while UE is served by cell(s) belongs to a E-UTRA band listed in supportedBandListEUTRA except for the FR2 inter-RAT measurement which depends on the support of independentGapConfig. |  | - |
| bandListEUTRA One entry corresponding to each supported E-UTRA band listed in the same order as in supportedBandListEUTRA. |  | - |
| bandParameterList-v1380 If included, the UE shall include the same number of entries listed in the same order as the band entries in the corresponding band combination. |  | - |
| bandParametersUL, bandParametersDL Indicates the supported parameters for the band. Each of CA-MIMO-ParametersUL and CA-MIMO-ParametersDL can be included only once for one band in a single band combination entry. |  | - |
| beamformed (in MIMO-CA-ParametersPerBoBCPerTM) If signalled, the field indicates for a particular transmission mode, the UE capabilities concerning beamformed EBF/ FD-MIMO operation (class B) applicable for the concerned band combination. |  | - |
| beamformed (in MIMO-UE-ParametersPerTM) Indicates for a particular transmission mode, the UE capabilities concerning beamformed EBF/ FD-MIMO operation (class B) applicable for band combinations for which the concerned capabilities are not signalled. |  | Yes |
| benefitsFromInterruption Indicates whether the UE power consumption would benefit from being allowed to cause interruptions to serving cells when performing measurements of deactivated SCell carriers for measCycleSCell of less than 640ms, as specified in TS 36.133 [16]. |  | No |
| bwPrefInd Indicates whether the UE supports maximum PDSCH/PUSCH bandwidth preference indication. |  | - |
| ca-BandwidthClass The CA bandwidth class supported by the UE as defined in TS 36.101 [42], Table 5.6A-1. The UE explicitly includes all the supported CA bandwidth class combinations in the band combination signalling. Support for one CA bandwidth class does not implicitly indicate support for another CA bandwidth class. |  | - |
| ca-IdleModeMeasurements Indicates whether UE supports reporting measurements performed during RRC_IDLE. |  | - |
| ca-IdleModeValidityArea Indicates whether UE supports validity area for IDLE measurements during RRC_IDLE. |  | - |
| cch-IM-RefRecTypeA-OneRX-Port This field defines whether the DL Category 1bis or the DL Category M2 UE supports Type A downlink control channel interference mitigation (CCH-IM) receiver "LMMSE-IRC + CRS-IC" for PDCCH/PCFICH/PHICH/EPDCCH receive processing (Enhanced downlink control channel performance requirements Type A in TS 36.101 [6]). |  | No |
| cch-InterfMitigation-RefRecTypeA, cch-InterfMitigation-RefRecTypeB, cch-InterfMitigation-MaxNumCCs The field cch-InterfMitigation-RefRecTypeA defines whether the UE supports Type A downlink control channel interference mitigation (CCH-IM) receiver "LMMSE-IRC + CRS-IC" for PDCCH/PCFICH/PHICH/EPDCCH receive processing (Enhanced downlink control channel performance requirements Type A in the TS 36.101 [6]). The field cch-InterfMitigation-RefRecTypeB defines whether the UE supports Type B downlink CCH-IM receiver "E-LMMSE-IRC + CRS-IC" for PDCCH/PCFICH/PHICH receive processing in synchronous networks (Enhanced downlink control channel performance requirements Type B in the TS 36.101 [6]). The UE supporting the capability defined by cch-InterfMitigation-RefRecTypeB-r13 shall also support the capability defined by cch-InterfMitigation-RefRecTypeA-r13. If the UE sets one or more of the fields cch-InterfMitigation-RefRecTypeA and cch-InterfMitigation-RefRecTypeB to "supported", the UE shall include the parameter cch-InterfMitigation-MaxNumCCs to indicate that the UE supports CCH-IM on at least one arbitrary downlink CC for up to cch-InterfMitigation-MaxNumCCs downlink CC CA configuration. The UE shall not include the parameter cch-InterfMitigation-MaxNumCCs if neither cch-InterfMitigation-RefRecTypeA nor cch-InterfMitigation-RefRecTypeB is present. The UE may not perform CCH-IM on more than 1 DL CCs. For example, the UE sets "cch-InterfMitigation-MaxNumCCs = 3" to indicate that UE supports CCH-IM on at least one DL CC for supported non-CA, 2DL CA and 3DL CA configurations. For CA scenarios, the CCH-IM is guaranteed to be supported on at least one arbitrary component carrier. |  | - |
| cdma2000-NW-Sharing Indicates whether the UE supports network sharing for CDMA2000. |  | - |
| ce-ClosedLoopTxAntennaSelection Indicates whether the UE supports UL closed-loop Tx antenna selection in CE mode A, as specified in TS 36.212 [22]. |  | Yes |
| ce-CQI-AlternativeTable Indicates whether the UE supports alternative CQI table in CE mode A. See TS 36.213 [22]. |  | Yes |
| ce-CRS-IntfMitig Indicates whether UE supports CRS interference mitigation, i.e., value supported indicates UE does not rely on the CRS outside certain PRBs and subframes as defined in TS 36.133 [16], clauses 3.6.1.2 and 3.6.1.3, and TS 36.213 [23] when operating in coverage enhancement mode. |  | Yes |
| ce-CSI-RS-Feedback Indicates whether the UE supports CSI-RS based feedback when the UE is operating in CE mode A, as specified in TS 36.213 [23]. |  | Yes |
| ce-CSI-RS-FeedbackCodebookRestriction Indicates whether the UE supports CSI-RS based feedback with codebook subset restriction when the UE in CE is operating in CE mode A, as specified in TS 36.213 [23]. |  | Yes |
| ce-DL-ChannelQualityReporting Indicates whether UE operating in CE mode supports aperiodic DL channel quality reporting in RRC_CONNECTED. |  | Yes |
| ce-EUTRA-5GC Indicates whether the UE operating in CE mode A or B supports E-UTRA/5GC. |  | Yes |
| ce-EUTRA-5GC-HO-ToNR-FDD-FR1 Indicates whether the UE operating in CE mode A or B supports handover from E-UTRA/5GC to NR FDD FR1. |  | Yes |
| ce-EUTRA-5GC-HO-ToNR-TDD-FR1 Indicates whether the UE operating in CE mode A or B supports handover from E-UTRA/5GC to NR TDD FR1. |  | Yes |
| ce-EUTRA-5GC-HO-ToNR-FDD-FR2 Indicates whether the UE operating in CE mode A or B supports handover from E-UTRA/5GC to NR FDD FR2. |  | Yes |
| ce-EUTRA-5GC-HO-ToNR-TDD-FR2 Indicates whether the UE operating in CE mode A or B supports handover from E-UTRA/5GC to NR TDD FR2-1 as specified in TS 38.101-x [xx]. |  | Yes |
| ce-EUTRA-5GC-HO-ToNR-TDD-FR2-2 Indicates whether the UE operating in CE mode A or B supports handover from E-UTRA/5GC to NR TDD FR2-2 as specified in TS 38.101-x [xx]. |  | - |
| ce-HARQ-AckBundling Indicates whether the UE supports HARQ-ACK bundling in half duplex FDD in CE mode A, as specified in TS 36.212 [22] and TS 36.213 [23]. |  | - |
| ce-InactiveState Indicates whether UE operating in CE mode supports RRC_INACTIVE when connected to 5GC. A UE including this field also supports short eDRX cycles in RRC_INACTIVE when connected to 5GC. |  | No |
| ce-MeasRSS-Dedicated, ce-MeasRSS-DedicatedSameRBs Indicates whether the UE operating in CE mode A/B supports receiving neighbour cell RSS information in dedicated signalling and performing serving cell and neighbour cell measurements based on RSS in RRC_CONNECTED as specified in TS 36.306 [5] and TS 36.133 [16]. |  | Yes |
| ce-ModeA, ce-ModeB Indicates whether the UE supports operation in CE mode A and/or B, as specified in TS 36.211 [21] and TS 36.213 [23]. |  | - |
| crs-ChEstMPDCCH-CE-ModeA, crs-ChEstMPDCCH-CE-ModeB Indicates whether UE operating in CE mode A/B supports using CRS for improving MPDCCH channel estimation. |  | Yes |
| crs-ChEstMPDCCH-CSI Indicates whether UE operating in CE mode A supports CSI-based mapping for improving MPDCCH channel estimation. |  | Yes |
| crs-ChEstMPDCCH-ReciprocityTDD Indicates whether UE operating in CE mode A supports using CRS for improving MPDCCH channel estimation with reciprocity-based candidates in TDD. |  | No |
| ceMeasurements Indicates whether the UE supports intra-frequency RSRQ measurements and inter-frequency RSRP and RSRQ measurements in RRC_CONNECTED, as specified in TS 36.133 [16] and TS 36.304 [4]. |  | - |
| ce-MultiTB-64QAM Indicates whether the UE supports downlink 64QAM for multiple TB scheduling in connected mode for PDSCH when operating in CE mode A, as specified in TS 36.211 [21] and TS 36.213 [23]. This field can be included only if ce-PUSCH-SubPRB-Allocation is included. |  | Yes |
| ce-MultiTB-EarlyTermination Indicates whether the UE supports early termination of PUSCH transmission for multiple TB scheduling in connected mode, as specified in TS 36.211 [21] and TS 36.213 [23]. |  | Yes |
| ce-MultiTB-FrequencyHopping Indicates whether the UE supports frequency hopping for multiple TB scheduling for PDSCH/PUSCH in connected mode, as specified in TS 36.211 [21] and TS 36.213 [23]. |  | Yes |
| ce-MultiTB-HARQ-AckBundling Indicates whether the UE supports downlink HARQ-ACK bundling for multiple TB scheduling in connected mode when operating in CE mode A, as specified in TS 36.211 [21] and TS 36.213 [23]. |  | Yes |
| ce-MultiTB-Interleaving Indicates whether the UE supports TB interleaving for multiple TB scheduling in connected mode for PDSCH/PUSCH when operating in CE mode A or B, as specified in TS 36.211 [21] and TS 36.213 [23]. |  | Yes |
| ce-MultiTB-SubPRB Indicates whether the UE supports sub-PRB allocation for multiple TB scheduling for PUSCH in connected mode, as specified in TS 36.211 [21] and TS 36.213 [23]. This field can be included only if ce-PUSCH-SubPRB-Allocation is included. |  | Yes |
| ce-PDSCH-14HARQProcesses, ce-PDSCH-14HARQProcesses-Alt2 Indicates whether the UE supports 14-HARQ processes, as specified in TS 36.212 [22]. |  | - |
| ce-PDSCH-64QAM Indicates whether the UE supports 64QAM for non-repeated unicast PDSCH in CE mode A. |  | Yes |
| ce-PDSCH-FlexibleStartPRB-CE-ModeA, ce-PDSCH-FlexibleStartPRB-CE-ModeB, ce-PUSCH-FlexibleStartPRB-CE-ModeA, ce-PUSCH-FlexibleStartPRB-CE-ModeB This field indicates whether UE supports flexible starting PRB for PDSCH/PUSCH when operating in coverage enhancement mode A/B, as specified in TS 36.211 [21] and TS 36.213 [22]. |  | Yes |
| ce-PDSCH-MaxTBS Indicates whether the UE supports downlink TBS of 1736 bits, as specified in TS 36.212 [22]. |  | - |
| ce-PDSCH-PUSCH-Enhancement Indicates whether the UE supports new numbers of repetitions for PUSCH and modulation restrictions for PDSCH/PUSCH in CE mode A as specified in TS 36.212 [22] and TS 36.213 [23]. |  | No |
| ce-PDSCH-PUSCH-MaxBandwidth Indicates the maximum supported PDSCH/PUSCH channel bandwidth in CE mode A and B, as specified in TS 36.212 [22] and TS 36.213 [23]. Value bw5 corresponds to 5 MHz and value bw20 corresponds to 20 MHz. If the field is absent the maximum PDSCH/PUSCH channel bandwidth in CE mode A and B is 1.4 MHz. If the setting of this parameter is 20 MHz, the max supported PUSCH channel bandwidth in CE mode A is 5 MHz. The maximum PUSCH channel bandwidth in CE mode B is 1.4 MHz regardless of the setting of this parameter. Parameter: transmission bandwidth configuration, see TS 36.101 [42], table 5.6-1. |  | Yes |
| ce-PDSCH-TenProcesses Indicates whether the UE supports 10 DL HARQ processes in FDD in CE mode A. |  | Yes |
| ce-PUCCH-Enhancement Indicates whether the UE supports repetition levels 64 and 128 for PUCCH in CE Mode B, as specified in TS 36.211 [21] and in TS 36.213 [23]. |  | No |
| ce-PUSCH-NB-MaxTBS Indicates whether the UE supports 2984 bits max UL TBS in 1.4 MHz in CE mode A operation, as specified in TS 36.212 [22] and TS 36.213 [23]. |  | Yes |
| ce-PUSCH-SubPRB-Allocation Indicates whether the UE supports sub-PRB resource allocation for PUSCH in CE mode A or B, as specified in TS 36.211 [21], TS 36.212 [22] and TS 36.213 [23]. |  | Yes |
| ce-RetuningSymbols Indicates the number of retuning symbols in CE mode A and B as specified in TS 36.211 [21]. Value n0 corresponds to 0 retuning symbols and value n1 corresponds to 1 retuning symbol. If the field is absent the number of retuning symbols in CE mode A and B is 2. |  | No |
| ce-SchedulingEnhancement Indicates whether the UE supports dynamic HARQ-ACK delay for HD-FDD in CE mode A as specified in TS 36.212 [22] and TS 36.213 [23]. |  | No |
| ce-SRS-Enhancement Indicates whether the UE supports SRS coverage enhancement in TDD with support of SRS combs 2 and 4 as specified in TS 36.213 [23]. This field can be included only if ce-SRS-EnhancementWithoutComb4 is not included. |  | Yes |
| ce-SRS-EnhancementWithoutComb4 Indicates whether the UE supports SRS coverage enhancement in TDD with support of SRS comb 2 but without support of SRS comb 4 as specified in TS 36.213 [23]. This field can be included only if ce-SRS-Enhancement is not included. |  | - |
| ce-SwitchWithoutHO Indicates whether the UE supports switching between normal mode and enhanced coverage mode without handover. |  | - |
| ce-UL-HARQ-ACK-Feedback This field indicates whether UE supports uplink HARQ ACK feedback when operating in coverage enhancement, as specified in TS36.213 [22]. |  | Yes |
| channelMeasRestriction Indicates for a particular transmission mode whether the UE supports channel measurement restriction. |  | Yes |
| cho Indicates whether the UE supports conditional handover including execution condition, candidate cell configuration and maximum 8 candidate cells. |  | Yes |
| cho-Failure Indicates whether the UE supports conditional handover during re-establishment procedure when the selected cell is configured as candidate cell for condition handover. |  | Yes |
| cho-FDD-TDD Indicates whether the UE supports conditional handover between FDD and TDD cells. |  | No |
| cho-TwoTriggerEvents Indicates whether the UE supports 2 trigger events for same execution condition. It is mandatory supported if the UE suppors cho. |  | Yes |
| codebook-HARQ-ACK Indicates whether the UE supports determining HARQ ACK codebook size based on the DAI-ased solution and/or the number of configured CCs. The first bit is set to "1" if the UE supports the DAI-based codebook size determination. The second bit is set to "1" if the UE supports the codebook determination based on the number of configured CCs. |  | No |
| commMultipleTx Indicates whether the UE supports multiple transmissions of sidelink communication to different destinations in one SC period. If commMultipleTx-r13 is set to supported then the UE support 8 transmitting sidelink processes. |  | - |
| commSimultaneousTx Indicates whether the UE supports simultaneous transmission of EUTRA and sidelink communication (on different carriers) in all bands for which the UE indicated sidelink support in a band combination (using commSupportedBandsPerBC). |  | - |
| commSupportedBands Indicates the bands on which the UE supports sidelink communication, by an independent list of bands i.e. separate from the list of supported E-UTRA band, as indicated in supportedBandListEUTRA. |  | - |
| commSupportedBandsPerBC Indicates, for a particular band combination, the bands on which the UE supports simultaneous reception of EUTRA and sidelink communication. If the UE indicates support simultaneous transmission (using commSimultaneousTx), it also indicates, for a particular band combination, the bands on which the UE supports simultaneous transmission of EUTRA and sidelink communication. The first bit refers to the first band included in commSupportedBands, with value 1 indicating sidelink is supported. |  | - |
| configN (in MIMO-CA-ParametersPerBoBCPerTM) If signalled, the field indicates for a particular transmission mode whether the UE supports non-precoded EBF/ FD-MIMO (class A) related configuration N for the concerned band combination. |  | - |
| configN (in MIMO-UE-ParametersPerTM) Indicates for a particular transmission mode whether the UE supports non-precoded EBF/ FD-MIMO (class A) related configuration N for band combinations for which the concerned capabilities are not signalled. |  | Yes |
| continueEHC-Context Indicates that the UE supports EHC context continuation operation where the UE keeps the established EHC context(s) upon PDCP re-establishment, as specified in TS 36.323 [8]. |  | No |
| crossCarrierScheduling |  | Yes |
| crossCarrierScheduling-B5C Indicates whether the UE supports cross carrier scheduling beyond 5 DL CCs. |  | No |
| crossCarrierSchedulingLAA-DL Indicates whether the UE supports cross-carrier scheduling from a licensed carrier for LAA cell(s) for downlink. This field can be included only if downlinkLAA is included. |  | - |
| crossCarrierSchedulingLAA-UL Indicates whether the UE supports cross-carrier scheduling from a licensed carrier for LAA cell(s) for uplink. This field can be included only if uplinkLAA is included. |  | - |
| crs-DiscoverySignalsMeas Indicates whether the UE supports CRS based discovery signals measurement, and PDSCH/EPDCCH RE mapping with zero power CSI-RS configured for discovery signals. |  | Yes |
| crs-IM-TM1-toTM9-OneRX-Port Indicates whether the DL Cateogry 1bis UE ot the DL Category M2 UE supports CRS interference mitigation (IM) while operating in the following transmission modes (TM): TM 1, TM 2, …, TM 8 and TM 9. |  | No |
| crs-InterfHandl Indicates whether the UE supports CRS interference handling. |  | Yes |
| crs-InterfMitigationTM10 The field defines whether the UE supports CRS interference mitigation in transmission mode 10. The UE supporting the crs-InterfMitigationTM10 capability shall also support the crs-InterfHandl capability. |  | No |
| crs-InterfMitigationTM1toTM9 Indicates whether the UE supports CRS interference mitigation (IM) while operating in the following transmission modes (TM): TM 1, TM 2, …, TM 8 and TM 9. The UE shall not include the field if it does not support CRS IM in TMs 1-9. If the field is present, the UE supports CRS-IM on at least one arbitrary downlink CC for up to crs-InterfMitigationTM1toTM9-r13 downlink CC CA configuration. The UE signals crs-InterfMitigationTM1toTM9-r13 value to indicate the maximum crs-InterfMitigationTM1toTM9-r13 downlink CC CA configuration where UE may apply CRS IM. For example, the UE sets "crs-InterfMitigationTM1toTM9-r13 = 3" to indicate that the UE supports CRS-IM on at least one DL CC for supported non-CA, 2DL CA and 3DL CA configurations. The UE supporting the crs-InterfMitigationTM1toTM9-r13 capability shall also support the crs-InterfHandl-r11 capability. |  | - |
| crs-IntfMitig Indicate whether the UE supports CRS interference mitigation as specified in TS 36.133 [16], clause 3.6.1.1. |  | Yes |
| crs-LessDwPTS Indicates whether the UE supports TDD special subframe configuration 10 without CRS transmission on the 5th symbol of DwPTS, i.e. ssp10-CRS-LessDwPTS, as specified in TS 36.211 [17]. |  | - |
| csi-ReportingAdvanced, csi-ReportingAdvancedMaxPorts (in MIMO-CA-ParametersPerBoBCPerTM) If signalled, the field indicates that for a particular transmission mode, the maximum number of CSI-RS ports supported by the UE for advanced CSI reporting is different in the concerned band of band combination than the value indicated by the field csi-ReportingAdvanced or csi-ReportingAdvancedMaxPorts in MIMO-UE-ParametersPerTM. The UE shall not include both csi-ReportingAdvanced and csi-ReportingAdvancedMaxPorts for a particular transmission mode in the concerned band of band combination. |  | - |
| csi-ReportingAdvanced (in MIMO-UE-ParametersPerTM) Indicates for a particular transmission mode the maximum number of CSI-RS ports supported by the UE for advanced CSI reporting. The field csi-ReportingAdvanced indicates 32 CSI-RS ports. The UE shall not include both csi-ReportingAdvanced and csi-ReportingAdvancedMaxPorts for a particular transmission mode. |  | Yes |
| csi-ReportingAdvancedMaxPorts (in MIMO-UE-ParametersPerTM) Indicates for a particular transmission mode the maximum number of CSI-RS ports supported by the UE for advanced CSI reporting. The field csi-ReportingAdvancedMaxPorts indicates 8, 12, 16, 20, 24 or 28 CSI-RS ports. The UE shall not include both csi-ReportingAdvanced and csi-ReportingAdvancedMaxPorts for a particular transmission mode. |  | - |
| csi-ReportingNP (in MIMO-CA-ParametersPerBoBCPerTM) If signalled, value different indicates that for a particular transmission mode, the CSI reporting on non-precoded CSI-RS with 20, 24, 28 or 32 antenna ports for the concerned band of band combination is different than the value indicated by field csi-ReportingNP in MIMO-UE-ParametersPerTM. |  | - |
| csi-ReportingNP (in MIMO-UE-ParametersPerTM) Indicates for a particular transmission mode whether the UE supports CSI reporting on non-precoded CSI-RS with 20, 24, 28, or 32 antenna ports for band combinations for which the concerned capabilities are not signalled in MIMO-CA-ParametersPerBoBCPerTM, and the FD-MIMO processing capability condition as described in NOTE 8 is satisfied. |  | Yes |
| csi-RS-DiscoverySignalsMeas Indicates whether the UE supports CSI-RS based discovery signals measurement. If this field is included, the UE shall also include crs-DiscoverySignalsMeas. |  | Yes |
| csi-RS-DRS-RRM-MeasurementsLAA Indicates whether the UE supports performing RRM measurements on LAA cell(s) based on CSI-RS-based DRS. This field can be included only if downlinkLAA is included. |  | - |
| csi-RS-EnhancementsTDD Indicates for a particular transmission mode whether the UE supports CSI-RS enhancements applicable for TDD. |  | Yes |
| csi-SubframeSet Indicates whether the UE supports REL-12 DL CSI subframe set configuration, REL-12 DL CSI subframe set dependent CSI measurement/feedback, configuration of up to 2 CSI-IM resources for a CSI process with no more than 4 CSI-IM resources for all CSI processes of one frequency if the UE supports tm10, configuration of two ZP-CSI-RS for tm1 to tm9, PDSCH RE mapping with two ZP-CSI-RS configurations, and EPDCCH RE mapping with two ZP-CSI-RS configurations if the UE supports EPDCCH. This field is only applicable for UEs supporting TDD. |  | Yes |
| csi-SubframeSet2ForDormantSCell Indicates whether the UE supports second CSI subframe set for periodic CSI reporting for dormant serving cells. A UE that indicates support of this field shall also indicate support for dormantSCellState-r15. This field is only applicable for UEs supporting TDD. |  | - |
| dataInactMon Indicates whether the UE supports the data inactivity monitoring as specified in TS 36.321 [6]. |  | - |
| dc-Support Including this field indicates that the UE supports synchronous DC and power control mode 1. Including this field for a band combination entry comprising of single band entry indicates that the UE supports intra-band contiguous DC. Including this field for a band combination entry comprising of two or more band entries, indicates that the UE supports DC for these bands and that the serving cells corresponding to a band entry shall belong to one cell group (i.e. MCG or SCG). Including field asynchronous indicates that the UE supports asynchronous DC and power control mode 2. Including this field for a TDD/FDD band combination indicates that the UE supports TDD/FDD DC for this band combination. |  | - |
| delayBudgetReporting Indicates whether the UE supports delay budget reporting. |  | No |
| demodulationEnhancements This field defines whether the UE supports advanced receiver in SFN scenario (350 km/h) as specified in TS 36.101 [42]. |  | - |
| demodulationEnhancements2 This field defines whether the UE supports further enhanced receiver in HST-SFN scenario (up to 500 km/h velocity) as specified in TS 36.101 [42]. |  | - |
| densityReductionNP, densityReductionBF Indicates whether the UE supports CSI-RS density reduction with values 1, 1/2 and 1/3 for non-precoded CSI-RS and beamformed CSI-RS respectively. |  | Yes |
| deviceType UE may set the value to "noBenFromBatConsumpOpt" when it does not foresee to particularly benefit from NW-based battery consumption optimisation. Absence of this value means that the device does benefit from NW-based battery consumption optimisation. |  | - |
| diffFallbackCombReport Indicates that the UE supports reporting of UE radio access capabilities for the CA band combinations asked by the eNB as well as, if any, reporting of different UE radio access capabilities for their fallback band combination as specified in TS 36.306 [5]. The UE does not report fallback combinations if their UE radio access capabilities are the same as the ones for the CA band combination asked by the eNB. |  | - |
| differentFallbackSupported Indicates that the UE supports different capabilities for at least one fallback case of this band combination. |  | - |
| directMCG-SCellActivationResume Indicates whether the UE supports having an E-UTRA MCG SCell configured in activated SCell state. |  | - |
| directSCellActivation Indicates whether the UE supports having an E-UTRA SCell configured in activated SCell state in the RRCConnectionReconfiguration message. This field is applicable to both LTE standalone and LTE-DC. |  | - |
| directSCellHibernation Indicates whether the UE supports having an SCell configured in dormant SCell state. |  | - |
| directSCG-SCellActivationNEDC Indicates whether the UE supports having an E-UTRA SCG SCell configured in activated SCell state in the RRCConnectionReconfiguration message contained in the NR RRCReconfiguration message, as defined in TS 36.321 [6] and TS 38.331 [82]. If the UE indicates support of directSCG-SCellActivationNEDC-r16, the UE shall also indicate support of ne-dc as specified in TS 38.331 [82]. |  | - |
| directSCG-SCellActivationResume Indicates whether the UE supports having an E-UTRA SCG SCell configured in activated SCell state. |  | - |
| discInterFreqTx Indicates whether the UE support sidelink discovery announcements either a) on the primary frequency only or b) on other frequencies also, regardless of the UE configuration (e.g. CA, DC). The UE may set discInterFreqTx to supported when having a separate transmitter or if it can request sidelink discovery transmission gaps. |  | - |
| discoverySignalsInDeactSCell Indicates whether the UE supports the behaviour on DL signals and physical channels when SCell is deactivated and discovery signals measurement is configured as specified in TS 36.211 [21], clause 6.11A. This field is included only if UE supports carrier aggregation and includes crs-DiscoverySignalsMeas. |  | Yes |
| discPeriodicSLSS Indicates whether the UE supports periodic (i.e. not just one time before sidelink discovery announcement) Sidelink Synchronization Signal (SLSS) transmission and reception for sidelink discovery. |  | - |
| discScheduledResourceAlloc Indicates whether the UE supports transmission of discovery announcements based on network scheduled resource allocation. |  | - |
| disc-UE-SelectedResourceAlloc Indicates whether the UE supports transmission of discovery announcements based on UE autonomous resource selection. |  | - |
| disc-SLSS Indicates whether the UE supports Sidelink Synchronization Signal (SLSS) transmission and reception for sidelink discovery. |  | - |
| discSupportedBands Indicates the bands on which the UE supports sidelink discovery. One entry corresponding to each supported E-UTRA band, listed in the same order as in supportedBandListEUTRA. |  | - |
| discSupportedProc Indicates the number of processes supported by the UE for sidelink discovery. |  | - |
| discSysInfoReporting Indicates whether the UE supports reporting of system information for inter-frequency/PLMN sidelink discovery. |  | - |
| dl-256QAM Indicates whether the UE supports 256QAM in DL on the band. |  | - |
| dl-1024QAM Indicates whether the UE supports 1024QAM in DL on the band or on the band within the band combination. When dl-1024QAM-ScalingFactor and dl-1024QAM-TotalWeightedLayers are included, the UE supports 1024QAM in a set of CCs in a band combination if the CCs belong to bands indicated to support 1024QAM in that band combination and the 1024QAM processing capability condition as specified in equation 4.3.5.31-1 in TS 36.306 [5] is satisfied. |  | - |
| dl-1024QAM-ScalingFactor Indicates scaling factor for processing a CC configured with 1024QAM with respect to a CC not configured with 1024QAM as described in 4.3.5.31 in TS 36.306 [5]. Value v1 indicates 1, value v1dot2 indicates 1.2 and value v1dot25 indicates 1.25. |  | - |
| dl-1024QAM-TotalWeightedLayers Indicates total number of weighted layers the UE can process for 1024QAM as described in 4.3.5.31 in TS 36.306 [5]. Actual value = (10 + indicated value x 2), i.e., value 0 indicates 10 layers, value 1 indicates 12 layers and so on. |  | - |
| dl-1024QAM-Slot Indicates whether the UE supports 1024QAM in DL on the band for slot TTI operation. |  | - |
| dl-1024QAM-SubslotTA-1 Indicates whether the UE supports 1024QAM in DL on the band for subslot TTI operation with TA set 1. |  | - |
| dl-1024QAM-SubslotTA-2 Indicates whether the UE supports 1024QAM in DL on the band for subslot TTI operation with TA set 2, dmrsBasedSPDCCH-nonMBSFN |  | - |
| dl-DedicatedMessageSegmentation Indicates whether the UE supports reception of segmented DL RRC messages. |  | - |
| dmrs-BasedSPDCCH-MBSFN Indicates whether the UE supports sDCI monitoring in DMRS based SPDCCH for MBSFN subframe. If UE supports this, it also provides the corresponding DMRS based SPDCCH capability in min-Proc-TimelineSubslot. |  | Yes |
| dmrs-BasedSPDCCH-nonMBSFN Indicates whether the UE supports sDCI monitoring in DMRS based SPDCCH for non-MBSFN subframe. If UE supports this, it also provides the corresponding DMRS based SPDCCH capability in min-Proc-TimelineSubslot. |  | Yes |
| dmrs-Enhancements (in MIMO-CA-ParametersPerBoBCPerTM) If signalled, the field indicates for a particular transmission mode, that for the concerned band combination the DMRS enhancements are different than the value indicated by field dmrs-Enhancements in MIMO-UE-ParametersPerTM. |  | - |
| dmrs-Enhancements (in MIMO-UE-ParametersPerTM) Indicates for a particular transmission mode whether the UE supports DMRS enhancements for the indicated transmission mode. |  | Yes |
| dmrs-LessUpPTS Indicates whether the UE supports not to transmit DMRS for PUSCH in UpPTS. |  | No |
| dmrs-OverheadReduction Indicates whether the UE supports OCC4 for rank 3 and 4 transmission as specified in clause 5.3.3.1.5C of TS 36.212 [22]. |  | Yes |
| dmrs-PositionPattern Indicates whether the UE supports uplink DMRS position pattern 'D D D' in subslot #5 with application of the 1/6 as the TBS scaling factor. |  | Yes |
| dmrs-RepetitionSubslotPDSCH Indicates whether the UE supports back-to-back 3/4-layer DMRS reception in two consecutive subslots across subframe boundary for subslot-PDSCH. |  | Yes |
| dmrs-SharingSubslotPDSCH Indicates whether the UE supports DMRS sharing in two consecutive subslots across subframe boundary for subslot-PDSCH. |  | Yes |
| dormantSCellState Indicates whether UE supports Dormant SCell state (i.e. SCell state with CQI and RRM measurement reporting but no PDCCH monitoring). |  | - |
| downlinkLAA Presence of the field indicates that the UE supports downlink LAA operation including identification of downlink transmissions on LAA cell(s) for full downlink subframes, decoding of common downlink control signalling on LAA cell(s), CSI feedback for LAA cell(s), RRM measurements on LAA cell(s) based on CRS-based DRS. |  | - |
| drb-TypeSCG Indicates whether the UE supports SCG bearer. |  | - |
| drb-TypeSplit Indicates whether the UE supports split bearer except for PDCP data transfer in UL. |  | - |
| dtm Indicates whether the UE supports DTM in GERAN. |  | - |
| dummy This field is not used in the specification. It shall not be sent by the UE. |  | - |
| earlyData-UP Indicates whether the UE supports UP-EDT when connected to EPC. |  | - |
| earlyData-UP-5GC Indicates whether the UE supports UP-EDT when connected to 5GC. |  | - |
| earlySecurityReactivation Indicates whether the UE supports early security reactivation when resuming a suspended RRC connection. |  | - |
| e-CSFB-1XRTT Indicates whether the UE supports enhanced CS fallback to CDMA2000 1xRTT or not. |  | Yes |
| e-CSFB-ConcPS-Mob1XRTT Indicates whether the UE supports concurrent enhanced CS fallback to CDMA2000 1xRTT and PS handover/ redirection to CDMA2000 HRPD. |  | Yes |
| e-CSFB-dual-1XRTT Indicates whether the UE supports enhanced CS fallback to CDMA2000 1xRTT for dual Rx/Tx configuration. This bit can only be set to supported if tx-Config1XRTT and rx-Config1XRTT are both set to dual. |  | Yes |
| e-HARQ-Pattern-FDD Indicates whether the UE supports enhanced HARQ pattern for TTI bundling operation for FDD. |  | Yes |
| ehc Indicates that the UE supports Ethernet header compression and decompression using EHC protocol, as specified in TS 36.323 [8] and in Annex A of TS 38.323 [83]. The UE indicating this capability and indicating support for at least one ROHC profile, shall support simultaneous configuration of EHC and ROHC on different DRBs. |  | No |
| eLCID-Support Indicates whether the UE supports LCID "10000" and MAC PDU subheader containing the eLCID field as described in TS 36.321 [6]. |  | - |
| emptyUnicastRegion Indicates whether the UE supports unicast reception in subframes with empty unicast control region as described in TS 36.213 [23] clause 12. This field can be included only if unicast-fembmsMixedSCell and crossCarrierScheduling are included. |  | No |
| en-DC Indicates whether the UE supports EN-DC. |  | - |
| endingDwPTS Indicates whether the UE supports reception ending with a subframe occupied for a DwPTS-duration as described in TS 36.211 [21] and TS 36.213 [23]. This field can be included only if downlinkLAA is included. |  | - |
| Enhanced-4TxCodebook Indicates whether the UE supports enhanced 4Tx codebook. |  | No |
| enhancedDualLayerTDD Indicates whether the UE supports enhanced dual layer (PDSCH transmission mode 8) for TDD or not. |  | - |
| ePDCCH Indicates whether the UE can receive DCI on UE specific search space on Enhanced PDCCH. |  | Yes |
| epdcch-SPT-differentCells Indicates whether the UE supports EPDCCH and short processing time on different serving cells. |  | Yes |
| epdcch-STTI-differentCells Indicates whether the UE supports EPDCCH and sTTI on different serving cells. |  | Yes |
| e-RedirectionUTRA |  | Yes |
| e-RedirectionUTRA-TDD Indicates whether the UE supports enhanced redirection to UTRA TDD to multiple carrier frequencies both with and without using related SIB provided by RRCConnectionRelease or not. |  | Yes |
| etws-CMAS-RxInConnCE-ModeA, etws-CMAS-RxInConn Indicates whether the UE operating in CE mode A/B supports reception of ETWS/CMAS indication in RRC_CONNECTED mode as specified in TS 36.212 [22]. |  | - |
| eutra-5GC Indicates whether the UE supports E-UTRA/5GC. |  | Yes |
| eutra-5GC-HO-ToNR-FDD-FR1 Indicates whether the UE supports handover from E-UTRA/5GC to NR FDD FR1. |  | Yes |
| eutra-5GC-HO-ToNR-TDD-FR1 Indicates whether the UE supports handover from E-UTRA/5GC to NR TDD FR1. |  | Yes |
| eutra-5GC-HO-ToNR-FDD-FR2 Indicates whether the UE supports handover from E-UTRA/5GC to NR FDD FR2. |  | Yes |
| eutra-5GC-HO-ToNR-TDD-FR2 Indicates whether the UE supports handover from E-UTRA/5GC to NR TDD FR2-1 as specified in TS 38.101-x [xx]. |  | Yes |
| eutra-5GC-HO-ToNR-TDD-FR2-2 Indicates whether the UE supports handover from E-UTRA/5GC to NR TDD FR2-2 as specified in TS 38.101-x [xx]. |  | - |
| eutra-CGI-Reporting-ENDC Indicates whether the UE supports Intra-RAT report CGI procedure when it is configured with (NG) EN-DC wherein either MN and SN have different DRX cycles, or on-duration configured by MN does not contain on-duration configured by SN if their DRX cycles are same. |  | Yes |
| eutra-CGI-Reporting-NEDC Indicates whether the UE supports acquisition of relevant information from a neighbouring E-UTRA cell by reading the SI of the neighbouring cell and reporting the acquired information to the network when the NE-DC is configured. |  | Yes |
| eutra-EPC-HO-ToNR-FDD-FR1 Indicates whether the UE supports handover from E-UTRA/EPC to NR FDD FR1. |  | Yes |
| eutra-EPC-HO-ToNR-TDD-FR1 Indicates whether the UE supports handover from E-UTRA/EPC to NR TDD FR1. |  | Yes |
| eutra-EPC-HO-ToNR-FDD-FR2 Indicates whether the UE supports handover from E-UTRA/EPC to NR FDD FR2. |  | Yes |
| eutra-EPC-HO-ToNR-TDD-FR2 Indicates whether the UE supports handover from E-UTRA/EPC to NR TDD FR2-1 as specified in TS 38.101-x [xx]. |  | Yes |
| eutra-EPC-HO-ToNR-TDD-FR2-2 Indicates whether the UE supports handover from E-UTRA/EPC to NR TDD FR2-2 as specified in TS 38.101-x [xx]. |  | - |
| eutra-EPC-HO-EUTRA-5GC Indicates whether the UE supports handover between E-UTRA/EPC and E-UTRA/5GC. |  | Yes |
| eutra-IdleInactiveMeasurements Indicates whether UE supports reporting measurements performed during RRC_IDLE or RRC_INACTIVE. |  | No |
| eutra-SI-AcquisitionForHO-ENDC Indicates whether the UE supports, upon configuration of si-RequestForHO by the network, acquisition of relevant information from a neighbouring E-UTRA cell by reading the SI of the neighbouring cell using autonomous gaps and reporting the acquired information to the network. |  | Yes |
| eventB2 Indicates whether the UE supports event B2. A UE supporting NR SA operation shall set this bit to supported. |  | - |
| extendedBand-n77 This field is only applicable for UEs that indicate support for band n77. If present, the UE supports the restriction to 3450 - 3550 MHz and 3700 - 3980 MHz ranges of band n77 in the USA as specified in Note 12 of Table 5.2-1 in TS 38.101-1 [85]. If absent, the UE supports only restriction to the 3700 - 3980 MHz range of band n77 in the USA. A UE that indicates this field shall support NS value 55 as specified in TS 38.101-1 [85]. |  | - |
| extendedBand-n77-2 This field is only applicable for UEs that indicate support for band n77. If present, the UE supports the restriction to 3450 - 3650 MHz and 3650 - 3980 ranges of band n77 in Canada as specified in Note 12 of Table 5.2-1 in TS 38.101-1 [85]. If absent, the UE supports only restriction to the 3450 - 3650 MHz range of band n77 in Canada. A UE that indicates this field shall also support NS value 57 as specified in TS 38.101-1 [85]. |  | - |
| extendedFreqPriorities Indicates whether the UE supports extended E-UTRA frequency priorities indicated by cellReselectionSubPriority field. A UE supporting NR SA operation shall set this bit to supported. |  | - |
| extendedLCID-Duplication Indicates whether the UE supports use of extended LCIDs 32-38 for PDCP duplication. |  | - |
| extendedLongDRX Indicates whether the UE supports extended long DRX cycle values of 5.12s and 10.24s in RRC_CONNECTED. |  | - |
| extendedMAC-LengthField Indicates whether the UE supports the MAC header with L field of size 16 bits as specified in TS 36.321 [6], clause 6.2.1. |  | - |
| extendedMaxMeasId Indicates whether the UE supports extended number of measurement identies as defined by maxMeasId-r12. |  | No |
| extendedMaxObjectId Indicates whether the UE supports extended number of measurement object identies as defined by maxObjectId-r13. |  | No |
| extendedNumberOfDRBs Indicates whether the UE supports up to 15 DRBs. The UE shall support any combination of RLC AM and RLC UM entities for the configured DRBs. |  | - |
| extendedPollByte Indicates whether the UE supports extended pollByte values as defined by pollByte-r14. |  | - |
| extended-RLC-LI-Field Indicates whether the UE supports 15 bit RLC length indicator. |  | - |
| extendedRLC-SN-SO-Field Indicates whether the UE supports 16 bits of RLC sequence number and segmentation offset. |  | - |
| extendedRSRQ-LowerRange Indicates whether the UE supports the extended RSRQ lower value range from -34dB to -19.5dB in measurement configuration and reporting as specified in TS 36.133 [16]. |  | No |
| fdd-HARQ-TimingTDD Indicates whether UE supports FDD HARQ timing for TDD SCell when configured with TDD PCell. |  | Yes |
| featureGroupIndicators, featureGroupIndRel9Add, featureGroupIndRel10 The definitions of the bits in the bit string are described in Annex B.1 (for featureGroupIndicators and featureGroupIndRel9Add) and in Annex C.1 (for featureGroupIndRel10). |  | Yes |
| featureSetsDL-PerCC In MR-DC, indicates a set of features that the UE supports on one component carrier in a bandwidth class for a band in a given band combination. The UE shall hence include at least as many FeatureSetDL-PerCC-Id in this list as the number of carriers it supports according to the ca-bandwidthClassDL, except if indicating additional functionality by reducing the number of FeatureSetDownlinkPerCC-Id in the feature set. The order of the elements in this list is not relevant, i.e., the network may configure any of the carriers in accordance with any of the FeatureSetDL-PerCC-Id in this list. |  | - |
| FeatureSetDL-PerCC-Id In MR-DC, indicates the index position of the FeatureSetDL-PerCC-r15 in the featureSetsDL-PerCC-r15 list. Value 0 corresponds to the first element in the list, value 1 corresponds to the second element in the list, and so on. Value 32 is not used. |  | - |
| featureSetsUL-PerCC In MR-DC, indicates a set of features that the UE supports on one component carrier in a bandwidth class for a band in a given band combination. The UE shall hence include at least as many FeatureSetUL-PerCC-Id in this list as the number of carriers it supports according to the ca-bandwidthClassUL, except if indicating additional functionality by reducing the number of FeatureSetDownlinkPerCC-Id in the feature set. The order of the elements in this list is not relevant, i.e., the network may configure any of the carriers in accordance with any of the FeatureSetUL-PerCC-Id in this list. |  | - |
| FeatureSetUL-PerCC-Id In MR-DC, indicates the index position of the FeatureSetUL-PerCC-r15 in the featureSetsUL-PerCC-r15 list. Value 0 corresponds to the first element in the list, value 1 corresponds to the second element in the list, and so on. Value 32 is not used. |  | - |
| fembmsMixedCell Indicates whether the UE in RRC_CONNECTED supports MBMS reception with 15 kHz subcarrier spacings via MBSFN from FeMBMS/Unicast mixed cells on a frequency indicated in an MBMSInterestIndication message. |  |  |
| fembmsDedicatedCell Indicates whether the UE in RRC_CONNECTED supports MBMS reception with 15 kHz subcarrier spacings via MBSFN from MBMS-dedicated cells on a frequency indicated in an MBMSInterestIndication message. |  |  |
| flexibleUM-AM-Combinations Indicates whether the UE supports any combination of RLC UM and RLC AM bearers as long as the total number of bearers is at most 8, regardless of what FGI20 indicates. |  | - |
| flightPathPlan Indicates whether UE supports reporting of flight path plan information. |  | - |
| fourLayerTM3-TM4 Indicates whether the UE supports 4-layer spatial multiplexing for TM3 and TM4. |  | - |
| fourLayerTM3-TM4 (in FeatureSetDL-PerCC) Indicates whether the UE supports 4-layer spatial multiplexing for TM3 and TM4 for MR-DC within the indicated feature set. If this field is absent, UE supports two layer MIMO for TM3/TM4. |  | - |
| fourLayerTM3-TM4-perCC Indicates whether the UE supports 4-layer spatial multiplexing for TM3 and TM4 for the component carrier. |  | - |
| frameStructureType-SPT This field indicates the supported FS-type(s) for short processing time. The UE capability is reported per band combination. The reported FS-type(s) apply to the reported maxNumberCCs-SPT-r15 for the given band combination. |  | - |
| freqBandPriorityAdjustment Indicates whether the UE supports the prioritization of frequency bands in multiBandInfoList over the band in freqBandIndicator as defined by freqBandIndicatorPriority-r12. |  | - |
| freqBandRetrieval Indicates whether the UE supports reception of requestedFrequencyBands. |  | - |
| gNB-ID-Length-Reporting-NR-EN-DC Indicates whether the UE supports Inter-RAT gNB ID length reporting towards NR cell when it is configured with (NG)EN-DC. If the UE supports reportCGI-NR-EN-DC-r15, the UE shall support the gNB-ID-Length-Reporting-NR-EN-DC-r17. |  | - |
| gNB-ID-Length-Reporting-NR-NoEN-DC Indicates whether the UE supports Inter-RAT gNB ID length reporting towards cell when it is not configured with (NG)EN-DC. If the UE supports reportCGI-NR-NoEN-DC-r15, the UE shall support gNB-ID-Length-Reporting-NR-NoEN-DC-r17. |  | - |
| halfDuplex If halfDuplex is set to true, only half duplex operation is supported for the band, otherwise full duplex operation is supported. |  | - |
| heightMeas Indicates whether UE supports the measurement events H1/H2. |  | - |
| ho-EUTRA-5GC-FDD-TDD Indicates whether the UE supports handover between E-UTRA/5GC FDD and E-UTRA/5GC TDD. |  | No |
| ho-InterfreqEUTRA-5GC Indicates whether the UE supports inter frequency handover within E-UTRA/5GC. |  | Yes |
| hybridCSI Indicates whether the UE supports hybrid CSI transmission as described in TS 36.213 [23]. |  | Yes |
| idleInactiveValidityAreaList Indicates whether the UE supports list of validity areas for measurements during RRC_IDLE and RRC_INACTIVE. |  | No |
| immMeasBT Indicates whether the UE supports Bluetooth measurements in RRC connected mode. |  | - |
| immMeasUnComBarPre Indicates whether the UE supports uncompensated barometric pressure measurements in RRC connected mode. |  | - |
| immMeasWLAN Indicates whether the UE supports WLAN measurements in RRC connected mode. |  | - |
| ims-VoiceOverMCG-BearerEUTRA-5GC Indicates whether the UE supports IMS voice over NR PDCP for MCG bearer for E-UTRA/5GC. |  | No |
| ims-VoiceOverNR-FR1 Indicates whether the UE supports IMS voice over NR FR1. |  | No |
| ims-VoiceOverNR-FR2 Indicates whether the UE supports IMS voice over NR FR2-1 as specified in TS 38.101-x [xx]. |  | No |
| ims-VoiceOverNR-FR2-2 Indicates whether the UE supports IMS voice over NR FR2-2 as specified in TS 38.101-x [xx]. |  | - |
| ims-VoiceOverNR-PDCP-MCG-Bearer Indicates whether the UE supports IMS voice over NR PDCP with only MCG RLC bearer. |  | Yes |
| ims-VoiceOverNR-PDCP-SCG-Bearer Indicates whether the UE supports IMS voice over NR PDCP with only SCG RLC bearer when configured with EN-DC. |  | Yes |
| ims-VoNR-PDCP-SCG-NGENDC Indicates whether the UE supports IMS voice over NR PDCP with only SCG RLC bearer when configured with NGEN-DC. |  | Yes |
| inactiveState Indicates whether the UE supports RRC_INACTIVE. |  | No |
| incMonEUTRA Indicates whether the UE supports increased number of E-UTRA carrier monitoring in RRC_IDLE and RRC_CONNECTED, as specified in TS 36.133 [16]. |  | No |
| incMonUTRA Indicates whether the UE supports increased number of UTRA carrier monitoring in RRC_IDLE and RRC_CONNECTED, as specified in TS 36.133 [16]. |  | No |
| inDeviceCoexInd Indicates whether the UE supports in-device coexistence indication as well as autonomous denial functionality. |  | Yes |
| inDeviceCoexInd-ENDC Indicates whether the UE supports in-device coexistence indication for (NG)EN-DC operation. This field can be included only if inDeviceCoexInd is included. The UE supports inDeviceCoexInd-ENDC in the same duplexing modes as it supports inDeviceCoexInd. |  | - |
| inDeviceCoexInd-HardwareSharingInd Indicates whether the UE supports indicating hardware sharing problems when sending the InDeviceCoexIndication, as well as omitting the TDM assistance information. A UE that supports hardware sharing indication shall also indicate support of LAA operation. |  | - |
| inDeviceCoexInd-UL-CA Indicates whether the UE supports UL CA related in-device coexistence indication. This field can be included only if inDeviceCoexInd is included. The UE supports inDeviceCoexInd-UL-CA in the same duplexing modes as it supports inDeviceCoexInd. |  | - |
| interBandTDD-CA-WithDifferentConfig Indicates whether the UE supports inter-band TDD carrier aggregation with different UL/DL configuration combinations. The first bit indicates UE supports the configuration combination of SCell DL subframes are a subset of PCell and PSCell by SIB1 configuration and the configuration combination of SCell DL subframes are a superset of PCell and PSCell by SIB1 configuration; the second bit indicates UE supports the configuration combination of SCell DL subframes are neither superset nor subset of PCell and PSCell by SIB1 configuration. This field is included only if UE supports inter-band TDD carrier aggregation. |  | - |
| interBandPowerSharingAsyncDAPS Indicates whether the UE supports power sharing for asynchronous inter-band DAPS handovers. |  | - |
| interBandPowerSharingSyncDAPS Indicates whether the UE supports power sharing for synchronous inter-band DAPS handovers. |  | - |
| interferenceMeasRestriction Indicates whether the UE supports interference measurement restriction. |  | Yes |
| interFreqAsyncDAPS Indicates whether the UE supports asynchronous DAPS handover in source PCell and inter-frequency target PCell. |  | - |
| interFreqBandList One entry corresponding to each supported E-UTRA band listed in the same order as in supportedBandListEUTRA. |  | - |
| interFreqDAPS Indicates whether the UE supports DAPS handover in source PCell and inter-frequency target PCell, i.e. support of simultaneous DL reception of PDCCH and PDSCH from source and target cell. For a BC, the capability applies to every carrier pair for source and target. A UE indicating this capability shall also support synchronous DAPS handover, and single UL transmission for inter-frequency DAPS handover. |  | - |
| interFreqMultiUL-TransmissionDAPS Indicates that the UE supports simultaneous UL transmission in source PCell and inter-frequency target PCell. |  | - |
| interFreqNeedForGaps Indicates need for measurement gaps when operating on the E-UTRA band given by the entry in bandListEUTRA or on the E-UTRA band combination given by the entry in bandCombinationListEUTRA and measuring on the E-UTRA band given by the entry in interFreqBandList. |  | - |
| interFreqProximityIndication Indicates whether the UE supports proximity indication for inter-frequency E-UTRAN CSG member cells. |  | - |
| interFreqRSTD-Measurement Indicates whether the UE supports inter-frequency RSTD measurements for OTDOA positioning, as specified in TS 36.355 [54]. |  | Yes |
| interFreqSI-AcquisitionForHO Indicates whether the UE supports, upon configuration of si-RequestForHO by the network, acquisition and reporting of relevant information using autonomous gaps by reading the SI from a neighbouring inter-frequency cell. |  | Yes |
| interRAT-BandList One entry corresponding to each supported band of another RAT listed in the same order as in the interRAT-Parameters. The NR bands reported in SupportedBandListNR are excluded from this list. |  | - |
| interRAT-BandListNR-EN-DC One entry corresponding to each supported NR band listed in the same order as in the supportedBandListEN-DC-r15. If both interRAT-BandListNR-EN-DC and interRAT-BandListNR-SA are included, the UE shall set the same interRAT-NeedForGapsNR value for the same NR band. |  | - |
| interRAT-BandListNR-SA One entry corresponding to each supported NR band listed in the same order as in the supportedBandListNR-SA. If both interRAT-BandListNR-EN-DC and interRAT-BandListNR-SA are included, the UE shall set the same interRAT-NeedForGapsNR value for the same NR band. |  | - |
| interRAT-enhancementNR Indicates whether the UE supports enhanced inter-RAT NR measurement requirements to support high speed up to 500 km/h as specified in TS 36.133 [16], when EN-DC is not configured and when EN-DC is configured. |  | - |
| interRAT-NeedForGaps Indicates need for DL measurement gaps when operating on the E-UTRA band given by the entry in bandListEUTRA or on the E-UTRA band combination given by the entry in bandCombinationListEUTRA and measuring on the inter-RAT band given by the entry in the interRAT-BandList. |  | - |
| interRAT-NeedForGapsNR Indicates need for measurement gaps when operating on the E-UTRA band given by the entry in supportedBandListEUTRA or on the E-UTRA band combination given by the entry in supportedBandCombination-r10 or supportedBandCombinationAdd-r11 or supportedBandCombinationReduced-r13 and measuring on the NR band given by the entry in the InterRAT-BandListNR. |  | - |
| interRAT-ParametersWLAN Indicates whether the UE supports WLAN measurements configured by MeasObjectWLAN with corresponding quantity and report configuration in the supported WLAN bands. |  | - |
| interRAT-PS-HO-ToGERAN Indicates whether the UE supports inter-RAT PS handover to GERAN or not. |  | Yes |
| intraBandContiguousCC-InfoList Indicates, per serving carrier of which the corresponding bandwidth class includes multiple serving carriers (i.e. bandwidth class B, C, D and so on), the maximum number of supported layers for spatial multiplexing in DL and the maximum number of CSI processes supported. The number of entries is equal to the number of component carriers in the corresponding bandwidth class. The UE shall support the setting indicated in each entry of the list regardless of the order of entries in the list.The UE shall include the field only if it supports 4-layer spatial multiplexing in transmission mode3/4 for a subset of component carriers in the corresponding bandwidth class, or if the maximum number of supported layers for at least one component carrier is higher than supportedMIMO-CapabilityDL-r10 in the corresponding bandwidth class, or if the number of CSI processes for at least one component carrier is higher than supportedCSI-Proc-r11 in the corresponding band. This field may also be included for bandwidth class A but in such a case without including any sub-fields in IntraBandContiguousCC-Info-r12 (see NOTE 6). |  | - |
| intraFreqA3-CE-ModeA Indicates whether the UE when operating in CE Mode A supports eventA3 for intra-frequency neighbouring cells. |  | - |
| intraFreqA3-CE-ModeB Indicates whether the UE when operating in CE Mode B supports eventA3 for intra-frequency neighbouring cells. |  | - |
| intraFreq-CE-NeedForGaps Indicates need for measurement gaps when operating in CE on the E-UTRA band given by the entry in supportedBandListEUTRA. |  |  |
| intraFreqAsyncDAPS Indicates whether the UE supports asynchronous DAPS handover in source PCell and intra-frequency target PCell. |  | - |
| intraFreqDAPS Indicates whether UE supports DAPS handover in source PCell and intra-frequency target PCell, i.e. support of simultaneous DL reception of PDCCH and PDSCH from source and target cell. A UE indicating this capability shall also support synchronous DAPS handover, and single UL transmission for intra-frequency DAPS handover. |  | - |
| intraFreqHO-CE-ModeA Indicates whether the UE when operating in CE Mode A supports intra-frequency handover. |  | - |
| intraFreqHO-CE-ModeB Indicates whether the UE when operating in CE Mode B supports intra-frequency handover. |  | - |
| intraFreqProximityIndication Indicates whether the UE supports proximity indication for intra-frequency E-UTRAN CSG member cells. |  | - |
| intraFreqSI-AcquisitionForHO Indicates whether the UE supports, upon configuration of si-RequestForHO by the network, acquisition and reporting of relevant information using autonomous gaps by reading the SI from a neighbouring intra-frequency cell. |  | Yes |
| intraFreqTwoTAGs-DAPS Indicates whether the UE supports different timing advance groups in source PCell and intra-frequency target PCell. It is mandatory for intraFreqDAPS capable UE. |  | - |
| jointEHC-ROHC-Config Indicates whether the UE supports simultaneous configuration of EHC and ROHC protocols for the same DRB. |  | No |
| k-Max (in MIMO-CA-ParametersPerBoBCPerTM) If signalled, the field indicates for a particular transmission mode the maximum number of NZP CSI RS resource configurations supported within a CSI process applicable for the concerned band combination. |  | No |
| k-Max (in MIMO-UE-ParametersPerTM) Indicates for a particular transmission mode the maximum number of NZP CSI RS resource configurations supported within a CSI process applicable for band combinations for which the concerned capabilities are not signalled. |  | Yes |
| laa-PUSCH-Mode1 Indicates whether the UE supports LAA PUSCH mode 1 as defined in TS 36.213 [23]. |  | - |
| laa-PUSCH-Mode2 Indicates whether the UE supports LAA PUSCH mode 2 as defined in TS 36.213 [23]. |  | - |
| laa-PUSCH-Mode3 Indicates whether the UE supports LAA PUSCH mode 3 as defined in TS 36.213 [23]. |  | - |
| locationReport Indicates whether the UE supports reporting of its geographical location information to eNB. |  | - |
| loggedMBSFNMeasurements Indicates whether the UE supports logged measurements for MBSFN. A UE indicating support for logged measurements for MBSFN shall also indicate support for logged measurements in Idle mode. |  | - |
| loggedMeasBT Indicates whether the UE supports Bluetooth measurements in RRC idle mode. |  | - |
| loggedMeasIdleEventL1 Indicates whether the UE supports event triggered logged measurements for eventL1 in camped normally state. |  | - |
| loggedMeasIdleEventOutOfCoverage Indicates whether the UE supports event triggered logged measurements for outOfCoverage in any cell selection state. |  | - |
| loggedMeasUnComBarPre Indicates whether the UE supports uncompensated barometric pressure measurements in RRC_IDLE mode. |  | - |
| loggedMeasurementsIdle Indicates whether the UE supports logged measurements in Idle mode. |  | - |
| loggedMeasWLAN Indicates whether the UE supports WLAN measurements in RRC idle mode. |  | - |
| logicalChannelSR-ProhibitTimer Indicates whether the UE supports the logicalChannelSR-ProhibitTimer as defined in TS 36.321 [6]. |  | - |
| longDRX-Command Indicates whether the UE supports Long DRX Command MAC Control Element. |  | - |
| lwa Indicates whether the UE supports LTE-WLAN Aggregation (LWA). The UE which supports LWA shall also indicate support of interRAT-ParametersWLAN-r13. |  | - |
| lwa-BufferSize Indicates whether the UE supports the layer 2 buffer sizes for "with support for split bearers" as defined in Table 4.1-3 and 4.1A-3 of TS 36.306 [5] for LWA. |  | - |
| lwa-HO-WithoutWT-Change Indicates whether the UE supports handover where LWA configuration is retained without WT change and using LWA end-marker for PDCP key change indication for LWA operation. |  | - |
| lwa-RLC-UM Indicates whether the UE supports RLC UM for LWA bearer. |  | - |
| lwa-SplitBearer Indicates whether the UE supports the split LWA bearer (as defined in TS 36.300 [9]). |  | - |
| lwa-UL Indicates whether the UE supports UL transmission over WLAN for LWA bearer. |  | - |
| lwip Indicates whether the UE supports LTE/WLAN Radio Level Integration with IPsec Tunnel (LWIP). The UE which supports LWIP shall also indicate support of interRAT-ParametersWLAN-r13. |  | - |
| lwip-Aggregation-DL, lwip-Aggregation-UL Indicates whether the UE supports aggregation of LTE and WLAN over DL/UL LWIP. The UE that indicates support of LWIP aggregation over DL or UL shall also indicate support of lwip. |  | - |
| makeBeforeBreak Indicates whether the UE supports intra-frequency Make-Before-Break handover, and whether the UE which indicates dc-Parameters supports intra-frequency Make-Before-Break SeNB change, as defined in TS 36.300 [9]. |  | - |
| measGapPatterns-NRonly Indicates whether the UE supports gap patterns 2, 3 and 11 in LTE standalone when the frequencies to be measured within this measurement gap are all NR frequencies. |  | No |
| measGapPatterns-NRonly-ENDC Indicates whether the UE supports gap patterns 2, 3 and 11 in (NG)EN-DC when the frequencies to be measured within this measurement gap are all NR frequencies. |  | No |
| maximumCCsRetrieval Indicates whether UE supports reception of requestedMaxCCsDL and requestedMaxCCsUL. |  | - |
| maxLayersMIMO-Indication Indicates whether the UE supports the network configuration of maxLayersMIMO. If the UE supports fourLayerTM3-TM4 or intraBandContiguousCC-InfoList or FeatureSetDL-PerCC for MR-DC, UE supports the configuration of maxLayersMIMO for these cases regardless of indicating maxLayersMIMO-Indication. |  | - |
| maxLayersSlotOrSubslotPUSCH Indicates the maxiumum number of layers for slot-PUSCH or subslot-PUSCH transmission. |  | Yes |
| maxNumberCCs-SPT Indicates the maximum number of supported CCs for short processing time. The UE capability is reported per band combination. The reported number of carriers applies to all the FS-type(s) frameStructureType-SPT-r15 supported in a given band combination. Absence of the field indicates that 0 number of CCs are supported for short processing time. |  | - |
| maxNumberDL-CCs, maxNumberUL-CCs Indicates for each TTI combination "sTTI-SupportedCombinations", the maximum number of supported DL CCs/UL CCs for short TTI. Absence of the field indicates that 0 number of CCs are supported for short TTI. |  | - |
| maxNumberDecoding Indicates the maximum number of blind decodes in UE-specific search space per UE in one subframe for CA with more than 5 CCs as defined in TS 36.213 [23] which is supported by the UE. The number of blind decodes supported by the UE is the field value * 32. Only values 5 to 32 can be used in this version of the specification. |  | No |
| maxNumberEHC-Contexts Defines the maximum number of Ethernet header compression contexts supported by the UE across all DRBs and across UE's EHC compressor and EHC decompressor. The indicated number defines the number of contexts in addition to CID = "all zeros" as specified in Annex A of TS 38.323 [83]. |  | No |
| maxNumberROHC-ContextSessions Set to the maximum number of concurrently active ROHC contexts supported by the UE, excluding context sessions that leave all headers uncompressed. cs2 corresponds with 2 (context sessions), cs4 corresponds with 4 and so on. The network ignores this field if the UE supports none of the ROHC profiles in supportedROHC-Profiles. If the UE indicates both maxNumberROHC-ContextSessions and maxNumberROHC-ContextSessions-r14, same value shall be indicated. |  | - |
| maxNumberUpdatedCSI-Proc, maxNumberUpdatedCSI-Proc-SPT Indicates the maximum number of CSI processes to be updated across CCs. |  | No |
| maxNumberUpdatedCSI-Proc-STTI-Comb77, maxNumberUpdatedCSI-Proc-STTI-Comb27, maxNumberUpdatedCSI-Proc-STTI-Comb22-Set1, maxNumberUpdatedCSI-Proc-STTI-Comb22-Set2 Indicates the maximum number of CSI processes to be updated across CCs. Comb77 is applicable for {slot, slot}, Comb27 for {subslot, slot}, Comb22-Set1 for {subslot, subslot} processing timeline set 1 and the Comb22-Set2 for {subslot, subslot} processing timeline set 2. |  |  |
| mbms-AsyncDC Indicates whether the UE in RRC_CONNECTED supports MBMS reception via MRB on a frequency indicated in an MBMSInterestIndication message, where (according to supportedBandCombination) the carriers that are or can be configured as serving cells in the MCG and the SCG are not synchronized. If this field is included, the UE shall also include mbms-SCell and mbms-NonServingCell. The field indicates that the UE supports the feature for xDD if mbms-SCell and mbms-NonServingCell are supported for xDD. |  | - |
| mbms-MaxBW Indicates maximum supported bandwidth (T) for MBMS reception, see TS 36.213 [23]. clause 11.1. If the value is set to implicitValue, the corresponding value of T is calculated as specified in TS 36.213 [23], clause 11.1. If the value is set to explicitValue, the actual value of T = explicitValue * 40 MHz. |  | - |
| mbms-NonServingCell Indicates whether the UE in RRC_CONNECTED supports MBMS reception via MRB on a frequency indicated in an MBMSInterestIndication message, where (according to supportedBandCombination and to network synchronization properties) a serving cell may be additionally configured. If this field is included, the UE shall also include the mbms-SCell field. |  | Yes |
| mbms-ScalingFactor1dot25, mbms-ScalingFactor7dot5 Indicates parameter A(1.25 / A(7.5, i.e., scaling factor for processing one unit of bandwidth corresponding to subcarrier spacing of 1.25 kHz / 7.5 kHz, with respect to one unit of bandwidth corresponding to subcarrier spacing of 15 kHz. See TS 36.213 [23], clause 11.1. This field is included only if subcarrierSpacingMBMS-khz1dot25 / subcarrierSpacingMBMS-khz7dot5 is included. This field shall be included if mbms-MaxBW and subcarrierSpacingMBMS-khz1dot25 / subcarrierSpacingMBMS-khz7dot5 are included. |  | - |
| mbms-ScalingFactor0dot37, mbms-ScalingFactor2dot5 Indicates parameter A(0.37 / A(2..5, i.e., scaling factor for processing one unit of bandwidth corresponding to subcarrier spacing of 0.37 kHz / 2.5 kHz, with respect to one unit of bandwidth corresponding to subcarrier spacing of 15 kHz. See TS 36.213 [23], clause 11.1. This field is included only if fembmsMixedCell or fembmsDedicatedCell is included. This field shall be included if subcarrierSpacingMBMS-khz0dot37 / subcarrierSpacingMBMS-khz2dot5 is included for at least one E-UTRA band in mbms-SupportedBandInfoList. |  | - |
| mbms-SCell Indicates whether the UE in RRC_CONNECTED supports MBMS reception via MRB on a frequency indicated in an MBMSInterestIndication message, when an SCell is configured on that frequency (regardless of whether the SCell is activated or deactivated). |  | Yes |
| mbms-SupportedBandInfoList One entry corresponding to each supported E-UTRA band listed in the same order as in supportedBandListEUTRA. This list is included only if fembmsMixedCell or fembmsDedicatedCell is included. If mbms-SupportedBandInfoList-v1700 is included, the UE shall include the same number of entries, and listed in the same order, as in mbms-SupportedBandInfoList-r16. |  | - |
| mcgRLF-RecoveryViaSCG Indicates whether the UE supports recovery from MCG RLF via split SRB1 (if supported) and via SRB3 (if supported). |  | - |
| measGapPatterns-NRonly Indicates whether the UE supports gap patterns 2, 3 and 11 in LTE standalone when the frequencies to be measured within this measurement gap are all NR frequencies. |  | No |
| measGapPatterns-NRonly-ENDC Indicates whether the UE supports gap patterns 2, 3 and 11 in (NG)EN-DC when the frequencies to be measured within this measurement gap are all NR frequencies. |  | No |
| measurementEnhancements This field defines whether UE supports measurement enhancements in high speed scenario (350 km/h) as specified in TS 36.133 [16]. |  | - |
| measurementEnhancements2 This field defines whether UE supports measurement enhancements in high speed scenario (up to 500 km/h velocity) as specified in TS 36.133 [16]. |  | - |
| measurementEnhancementsSCell This field defines whether UE supports SCell measurement enhancements in high speed scenario (350 km/h) as specified in TS 36.133 [16]. |  | - |
| measGapPatterns Indicates whether the UE that supports NR supports gap patterns 4 to 11 in LTE standalone as specified in TS 36.133 [16], and for independent measurement gap configuration on FR1 and per-UE gap in (NG)EN-DC as specified in TS 38.133 [84]. The first/ leftmost bit covers pattern 4, and so on. Value 1 indicates that the UE supports the concerned gap pattern. |  | - |
| mfbi-UTRA It indicates if the UE supports the signalling requirements of multiple radio frequency bands in a UTRA FDD cell, as defined in TS 25.307 [65]. |  | - |
| MIMO-BeamformedCapabilityList A list of pairs of {k-Max, n-MaxList} values with the nth entry indicating the values that the UE supports for each CSI process in case n CSI processes would be configured. |  | No |
| MIMO-CapabilityDL The number of supported layers for spatial multiplexing in DL. The field may be absent for category 0 and category 1 UE in which case the number of supported layers is 1. |  | - |
| MIMO-CapabilityUL The number of supported layers for spatial multiplexing in UL. Absence of the field means that the number of supported layers is 1. |  | - |
| MIMO-CA-ParametersPerBoBC A set of MIMO parameters provided per band of a band combination. In case a subfield is absent, the concerned capabilities are the same as indicated at the per UE level (i.e. by MIMO-UE-ParametersPerTM). |  | - |
| mimo-CBSR-AdvancedCSI Indicates whether UE supports CBSR for advanced CSI reporting with and without amplitude restriction as defined in TS 36.213 [23], clause 7.2. |  | Yes |
| min-Proc-TimelineSubslot Minimum processing timeline for subslot operation. The minimum processing timeline can belong to one of two sets of associated processing and maximum TA operation. The sets supported can be different for 1os CRS-based SPDCCH, 2os CRS-based SPDCCH and DMRS-based SPDCCH. The sequence applies to: 1. 1os CRS based SPDCCH 2. 2os CRS based SPDCCH 3. DMRS based SPDCCH |  | - |
| modifiedMPR-Behavior Field encoded as a bit map, where at least one bit N is set to "1" if UE supports modified MPR/A-MPR behaviour N, see TS 36.101 [42]. All remaining bits of the field are set to "0". The leading / leftmost bit (bit 0) corresponds to modified MPR/A-MPR behaviour 0, the next bit corresponds to modified MPR/A-MPR behaviour 1 and so on. Absence of this field means that UE does not support any modified MPR/A-MPR behaviour. |  | - |
| mpdcch-InLteControlRegionCE-ModeA, mpdcch-InLteControlRegionCE-ModeB Indicates whether UE operating in CE mode A/B supports MPDCCH reception in LTE control channel region as specified in TS 36.211 [21]. |  | Yes |
| mpsPriorityIndication Indicates whether the UE supports mpsPriorityIndication on release with redirect. |  | - |
| multiACK-CSI-reporting Indicates whether the UE supports multi-cell HARQ ACK and periodic CSI reporting and SR on PUCCH format 3. |  | Yes |
| multiBandInfoReport Indicates whether the UE supports the acquisition and reporting of multi band information for reportCGI. |  | - |
| multiClusterPUSCH-WithinCC |  | Yes |
| multiNS-Pmax Indicates whether the UE supports the mechanisms defined for cells broadcasting NS-PmaxList. |  | - |
| multipleCellsMeasExtension Indicates whether the UE supports numberOfTriggeringCells in the report configuration. |  | - |
| multipleTimingAdvance Indicates whether the UE supports multiple timing advances for each band combination listed in supportedBandCombination. If the band combination comprised of more than one band entry (i.e., inter-band or intra-band non-contiguous band combination), the field indicates that the same or different timing advances on different band entries are supported. If the band combination comprised of one band entry (i.e., intra-band contiguous band combination), the field indicates that the same or different timing advances across component carriers of the band entry are supported. It is mandatory for UEs to support 2 TAGs for inter frequency DAPS handover. |  | - |
| multipleUplinkSPS Indicates whether the UE supports multiple uplink SPS and reporting SPS assistance information. A UE indicating multipleUplinkSPS shall also support V2X communication via Uu, as defined in TS 36.300 [9]. |  | - |
| must-CapabilityPerBand Indicates that UE supports MUST, as specified in 36.212 [22], clause 5.3.3.1, on the band in the band combination. |  | - |
| must-TM234-UpTo2Tx-r14 Indicates that the UE supports MUST operation for TM2/3/4 using up to 2Tx. |  | - |
| must-TM89-UpToOneInterferingLayer-r14 Indicates that the UE supports MUST operation for TM8/9 with assistance information for up to 1 interfering layer. |  | - |
| must-TM89-UpToThreeInterferingLayers-r14 Indicates that the UE supports MUST operation for TM8/9 with assistance information for up to 3 interfering layers. |  | - |
| must-TM10-UpToOneInterferingLayer-r14 Indicates that the UE supports MUST operation for TM10 with assistance information for up to 1 interfering layer. |  | - |
| must-TM10-UpToThreeInterferingLayers-r14 Indicates that the UE supports MUST operation for TM10 with assistance information for up to 3 interfering layers. |  | - |
| naics-Capability-List Indicates that UE supports NAICS, i.e. receiving assistance information from serving cell and using it to cancel or suppress interference of neighbouring cell(s) for at least one band combination. If not present, UE does not support NAICS for any band combination. The field numberOfNAICS-CapableCC indicates the number of component carriers where the NAICS processing is supported and the field numberOfAggregatedPRB indicates the maximum aggregated bandwidth across these of component carriers (expressed as a number of PRBs) with the restriction that NAICS is only supported over the full carrier bandwidth. The UE shall indicate the combination of {numberOfNAICS-CapableCC, numberOfNAICS-CapableCC} for every supported numberOfNAICS-CapableCC, e.g. if a UE supports {x CC, y PRBs} and {x-n CC, y-m PRBs} where n>=1 and m>=0, the UE shall indicate both. - For numberOfNAICS-CapableCC = 1, UE signals one value for numberOfAggregatedPRB from the range {50, 75, 100}; - For numberOfNAICS-CapableCC = 2, UE signals one value for numberOfAggregatedPRB from the range {50, 75, 100, 125, 150, 175, 200}; - For numberOfNAICS-CapableCC = 3, UE signals one value for numberOfAggregatedPRB from the range {50, 75, 100, 125, 150, 175, 200, 225, 250, 275, 300}; - For numberOfNAICS-CapableCC = 4, UE signals one value for numberOfAggregatedPRB from the range {50, 100, 150, 200, 250, 300, 350, 400}; - For numberOfNAICS-CapableCC = 5, UE signals one value for numberOfAggregatedPRB from the range {50, 100, 150, 200, 250, 300, 350, 400, 450, 500}. |  | No |
| ncsg Indicates whether the UE supports measurement NCSG Pattern Id 0, 1, 2 and 3, as specified in TS 36.133 [16]. If this field is included and the UE supports asynchronous DC, the UE shall support NCSG Pattern Id 0, 1, 2 and 3. If this field is included but the UE does not support asynchronous DC, only NCSG Pattern Id 0 and 1 shall be supported |  | No |
| ng-EN-DC Indicates whether the UE supports NGEN-DC. |  | - |
| n-MaxList (in MIMO-UE-ParametersPerTM) Indicates for a particular transmission mode the maximum number of NZP CSI RS ports supported within a CSI process applicable for band combinations for which the concerned capabilities are not signalled. For k-Max values exceeding 1, the UE shall include the field and signal k-Max minus 1 bits. The first bit indicates n-Max2, with value 0 indicating 8 and value 1 indicating 16. The second bit indicates n-Max3, with value 0 indicating 8 and value 1 indicating 16. The third bit indicates n-Max4, with value 0 indicating 8 and value 1 indicating 32. The fourth bit indicates n-Max5, with value 0 indicating 16 and value 1 indicating 32. The fifth bit indicates n-Max6, with value 0 indicating 16 and value 1 indicating 32. The sixt bit indicates n-Max7, with value 0 indicating 16 and value 1 indicating 32. The seventh bit indicates n-Max8, with value 0 indicating 16 and value 1 indicating 64. |  | Yes |
| n-MaxList (in MIMO-CA-ParametersPerBoBCPerTM) If signalled, the field indicates for a particular transmission mode the maximum number of NZP CSI RS ports supported within a CSI process applicable for band the concerned combination. Further details are as indicated for n-MaxList in MIMO-UE-ParametersPerTM. |  | No |
| NonContiguousUL-RA-WithinCC-List One entry corresponding to each supported E-UTRA band listed in the same order as in supportedBandListEUTRA. |  | No |
| nonPrecoded (in MIMO-UE-ParametersPerTM) Indicates for a particular transmission mode the UE capabilities concerning non-precoded EBF/ FD-MIMO operation (class A) for band combinations for which the concerned capabilities are not signalled in MIMO-CA-ParametersPerBoBCPerTM, and the FD-MIMO processing capability condition as described in NOTE 8 is satisfied. |  | Yes |
| nonPrecoded (in MIMO-CA-ParametersPerBoBCPerTM) If signalled, the field indicates for a particular transmission mode, the UE capabilities concerning non-precoded EBF/ FD-MIMO operation (class A) applicable for the concerned band combination. |  | - |
| nonUniformGap Indicates whether the UE supports measurement non uniform Pattern Id 1, 2, 3 and 4 in LTE standalone as specified in TS 36.133 [16]. |  | No |
| noResourceRestrictionForTTIBundling Indicate whether the UE supports TTI bundling operation without resource allocation restriction. |  | No |
| nonCSG-SI-Reporting Indicates whether UE will report PLMN list from non-CSG cells. |  | - |
| nr-AutonomousGaps-ENDC-FR1 Indicates whether the UE supports, upon configuration of useAutonomousGapsNR by the network, acquisition of relevant information from a neighbouring NR cell by reading the SI of the neighbouring cell on FR1 using autonomous gaps and reporting the acquired information to the network when it is configured with (NG)EN-DC. |  | Yes |
| nr-AutonomousGaps-ENDC-FR2 Indicates whether the UE supports, upon configuration of useAutonomousGapsNR by the network, acquisition of relevant information from a neighbouring NR cell by reading the SI of the neighbouring cell on FR2 using autonomous gaps and reporting the acquired information to the network when it is configured with (NG)EN-DC. |  | Yes |
| nr-AutonomousGaps-FR1 Indicates whether the UE supports, upon configuration of useAutonomousGapsNR by the network, acquisition of relevant information from a neighbouring NR cell by reading the SI of the neighbouring cell on FR1 using autonomous gaps and reporting the acquired information to the network when it is not configured with (NG)EN-DC. |  | Yes |
| nr-AutonomousGaps-FR2 Indicates whether the UE supports, upon configuration of useAutonomousGapsNR by the network, acquisition of relevant information from a neighbouring NR cell by reading the SI of the neighbouring cell on FR2 using autonomous gaps and reporting the acquired information to the network when it is not configured with (NG)EN-DC. |  | Yes |
| nr-CellIndividualOffset Indicates whether the UE supports use of cell specific offset for NR inter-RAT measurements. |  | No |
| nr-HO-ToEN-DC Indicates whether the UE supports inter-RAT handover from NR to EN-DC while NR-DC or NE-DC is not configured. This field is mandatory present if EN-DC is supported. |  | - |
| nr-IdleInactiveBeamMeasFR1 Indicates whether the UE supports performing eNB-configured SSB-based beam level RRM measurements for configured NR FR1 carrier(s) in RRC_IDLE and in RRC_INACTIVE as specified in TS 36.306 [5], clause 4.3.6.46. |  | No |
| nr-IdleInactiveBeamMeasFR2 Indicates whether the UE supports performing eNB-configured SSB-based beam level RRM measurements for configured NR FR2 carrier(s) in RRC_IDLE and in RRC_INACTIVE as specified in TS 36.306 [5], clause 4.3.6.47. |  | No |
| nr-IdleInactiveMeasFR1 Indicates whether UE supports reporting measurements performed on NR FR1 carrier(s) during RRC_IDLE and RRC_INACTIVE. |  | No |
| nr-IdleInactiveMeasFR2 Indicates whether UE supports reporting measurements performed on NR FR2 carrier(s) during RRC_IDLE and RRC_INACTIVE. |  | No |
| nr-RSSI-ChannelOccupancyReporting Indicates whether the UE supports performing measurements and reporting of RSSI and channel occupancy on the corresponding NR band. |  | - |
| ntn-Connectivity-EPC Indicates whether the UE supports NTN access when connected to EPC. If the UE indicates this capability, the UE shall support all NTN essential features as specified in TS 36.306 [5]. |  | - |
| ntn-OffsetTimingEnh Indicates whether the UE supports timing relationship enhancement using Differential Koffset as specified in TS 36.321 [6] and TS 36.213 [23]. |  | - |
| ntn-PUR-TimerDelay Indicates whether the UE supports delaying the start of the pur-ResponseWindowTimer for NTN, see TS 36.321 [6]. |  | - |
| ntn-SegmentedPrecompensationGaps Indicates the minumum supported gap length between segments for segmented uplink transmission. Value sym1 corresponds to 1 symbol, value sl1 corresponds to 1 slot, value sf1 corresponds to 1 subframe. |  | - |
| ntn-ScenarioSupport Indicates whether the UE supports NTN features only for GSO or NGSO scenario. If a UE does not include this field but includes ntn-Connectivity-EPC-r17, the UE supports the NTN features for both GSO and NGSO scenarios. |  | - |
| ntn-TA-report Indicates whether the UE supports timing advance reporting in RRC_CONNECTED, see TS 36.321 [6]. |  | - |
| numberOfBlindDecodesUSS Indicates the maximum number of blind decodes in UE specific search space in one subframe for CCs configured with sTTI operation supported by the UE. The number of blind decodes supported by the UE is the field value X*68. Field value ranges from 4 to 32. |  | Yes |
| nzp-CSI-RS-AperiodicInfo Indicates whether the UE supports aperiodic NZP CSI-RS transmission for the indicated transmission mode. |  | Yes |
| nzp-CSI-RS-PeriodicInfo Indicates whether the UE supports periodic NZP CSI-RS transmission for the indicated transmission mode. |  | Yes |
| otdoa-UE-Assisted Indicates whether the UE supports UE-assisted OTDOA positioning, as specified in TS 36.355 [54]. |  | Yes |
| outOfOrderDelivery Same as "outOfOrderDelivery" defined in TS 38.306 [87]. |  | No |
| outOfSequenceGrantHandling Indicates whether the UE supports PUSCH transmissions with out of sequence UL grants as defined in TS 36.213 [23]. This field can be included only if uplinkLAA is included. |  | - |
| overheatingInd Indicates whether the UE supports overheating assistance information. |  | No |
| overheatingIndForSCG Indicates whether the UE supports the inclusion of NR SCG reduced configuration in the overheating assistance information. The UE which indicates support of overheatingIndForSCG shall also indicate support of overheatingInd. |  | - |
| pdcch-CandidateReductions Indicates whether the UE supports PDCCH candidate reduction on UE specific search space as specified in TS 36.213 [23], clause 9.1.1. |  | No |
| pdcp-Duplication Indicates whether the UE supports PDCP duplication. |  | - |
| pdcp-SN-Extension Indicates whether the UE supports 15 bit length of PDCP sequence number. |  | - |
| pdcp-SN-Extension-18bits Indicates whether the UE supports 18 bit length of PDCP sequence number. |  | - |
| pdcp-TransferSplitUL Indicates whether the UE supports PDCP data transfer split in UL for the drb-TypeSplit as specified in TS 36.323 [8]. |  | - |
| pdcp-VersionChangeWithoutHO Indicates whether, the UE supports changing the PDCP version of DRBs, from LTE PDCP to NR PDCP and vice versa, with and without handover. A UE supporting PDCP version change shall signal field pdcp-Parameters-v1610. When the field pdcp-VersionChangeWithoutHO is not included and pdcp-Parameters-v1610 is included, it implies the UE supports PDCP version change only with handover. |  | - |
| pdsch-CollisionHandling Indicates whether the UE supports PDSCH collision handling as specified in TS 36.213 [23]. |  | No |
| pdsch-InLteControlRegionCE-ModeA, pdsch-InLteControlRegionCE-ModeB Indicates whether UE operating in CE mode A/B supports PDSCH reception in LTE control channel region as specified in TS 36.211 [21]. |  | Yes |
| pdsch-MultiTB-CE-ModeA, pdsch-MultiTB-CE-ModeB Indicates whether the UE supports multiple TB scheduling in connected mode for PDSCH when operating in CE mode A/B, as specified in TS 36.211 [21] and TS 36.213 [23]. |  | Yes |
| pdsch-RepSubframe Indicates whether the UE supports subframe PDSCH repetition. |  | Yes |
| pdsch-RepSlot Indicates whether the UE supports slot PDSCH repetition. |  | Yes |
| pdsch-RepSubslot Indicates whether the UE supports subslot PDSCH repetition. This field is only applicable for UEs supporting FDD. |  | - |
| pdsch-SlotSubslotPDSCH-Decoding Indicates whether the UE supports decoding of PDSCH and slot-PDSCH/subslot-PDSCH assigned with C-RNTI/SPS C-RNTI in the same subframe for a given carrier. |  | Yes |
| perServingCellMeasurementGap Indicates whether the UE supports per serving cell measurement gap indication, as specified in TS 36.133 [16]. |  | - |
| phy-TDD-ReConfig-FDD-PCell Indicates whether the UE supports TDD UL/DL reconfiguration for TDD serving cell(s) via monitoring PDCCH with eIMTA-RNTI on a FDD PCell, and HARQ feedback according to UL and DL HARQ reference configurations. This bit can only be set to supported only if the UE supports FDD PCell and phy-TDD-ReConfig-TDD-PCell is set to supported. |  | No |
| phy-TDD-ReConfig-TDD-PCell Indicates whether the UE supports TDD UL/DL reconfiguration for TDD serving cell(s) via monitoring PDCCH with eIMTA-RNTI on a TDD PCell, and HARQ feedback according to UL and DL HARQ reference configurations, and PUCCH format 3. |  | Yes |
| pmch-Bandwidth-n40, pmch-Bandwidth-n35, pmch-Bandwidth-n30 Indicates, for the E‑UTRA band corresponding to the entry in mbms-SupportedBandInfoList-v1700, whether the UE in RRC_CONNECTED supports MBMS reception via MBSFN from MBMS-dedicated cells in an MBSFN area with PMCH bandwidth of 40/ 35/ 30 PRBs as described in TS 36.211 [21] and TS 36.213 [23]. | - |  |
| pmi-Disabling |  | Yes |
| powerClass-14dBm Indicates whether the UE supports power class 14 dBm when operating in CE mode A or B for all the bands that are supported by the UE, as specified in TS 36.101 [42]. |  | - |
| powerPrefInd Indicates whether the UE supports power preference indication. |  | No |
| powerUCI-SlotPUSCH, powerUCI-SubslotPUSCH Indicates whether the UE supports BPRE derivation based on the actual derived O_CQI. The parameter uplinkPower-CSIPayload configures the UE to derive BPRE based on either the actual value of O_CQI or the largest value of O_CQI across all RI values. If the UE does not support the capability, the UE will derive BPRE based on the largest value of O_CQI across all RI values. |  | Yes |
| prach-Enhancements This field defines whether the UE supports random access preambles generated from restricted set type B in high speed scenoario as specified in TS 36.211 [21]. |  | - |
| processingTimelineSet Indicates, for each SPDCCH configuration, support for a set of TA values. Each set consists of two different processing timelines and associated maximum TA. Set 1 indicates support for n+4 and n+6 and set 2 indicates support for n+6 and n+8, see TS 36.211 [21], clause 8.1, The minimum processing timeline to use, out of the two options for a given set is configured by parameter proc-Timeline. Support of Set 1 implicitly means support of Set 2. |  | - |
| pucch-Format4 Indicates whether the UE supports PUCCH format 4. |  | Yes |
| pucch-Format5 Indicates whether the UE supports PUCCH format 5. |  | Yes |
| pucch-SCell Indicates whether the UE supports PUCCH on SCell. |  | No |
| pur-CP-EPC-CE-ModeA, pur-CP-EPC-CE-ModeB, pur-CP-5GC-CE-ModeA, pur-CP-5GC-CE-ModeB Indicates whether UE operating in CE mode A/B supports CP transmission using PUR when connected to EPC/ 5GC. |  | Yes |
| pur-CP-L1Ack Indicates whether UE supports L1 acknowledgement in response to CP transmission using PUR when connected to EPC/ 5GC. |  | Yes |
| pur-FrequencyHopping Indicates whether UE supports frequency hopping for transmission using PUR. |  | Yes |
| pur-PUSCH-NB-MaxTBS Indicates whether the UE supports 2984 bits max UL TBS in 1.4 MHz for transmission using PUR when operating in CE mode A, as specified in TS 36.212 [22] and TS 36.213 [23]. |  | Yes |
| pur-RSRP-Validation Indicates whether UE supports serving cell RSRP for TA validation for transmission using PUR when connected to EPC/ 5GC. |  | Yes |
| pur-SubPRB-CE-ModeA, pur-SubPRB-CE-ModeB Indicates whether UE supports subPRB resource allocation for PUSCH for transmission using PUR when operating in CE mode A/B. |  | Yes |
| pur-UP-EPC-CE-ModeA, pur-UP-EPC-CE-ModeB, pur-UP-5GC-CE-ModeA, pur-UP-5GC-CE-ModeB Indicates whether UE operating in CE mode A/B supports UP transmission using PUR when connected to EPC/ 5GC. |  | Yes |
| pusch-Enhancements Indicates whether the UE supports the PUSCH enhancement mode as specified in TS 36.211 [21] and TS 36.213 [23]. |  | Yes |
| pusch-FeedbackMode Indicates whether the UE supports PUSCH feedback mode 3-2. |  | No |
| pusch-MultiTB-CE-ModeA, pusch-MultiTB-CE-ModeB Indicates whether the UE supports multiple TB scheduling in connected mode for PUSCH when operating in CE mode A/B, as specified in TS 36.211 [21] and TS 36.213 [23]. |  | Yes |
| pusch-SPS-MaxConfigSlot Indicates the max number of SPS configurations across all cells for slot PUSCH. |  | Yes |
| pusch-SPS-MultiConfigSlot Indicates the number of multiple SPS configurations of slot PUSCH for each serving cell. |  | Yes |
| pusch-SPS-MaxConfigSubframe Indicates the max number of SPS configurations across all cells for subframe PUSCH. |  | Yes |
| pusch-SPS-MultiConfigSubframe Indicates the number of multiple SPS configurations of subframe PUSCH for each serving cell. |  | Yes |
| pusch-SPS-MaxConfigSubslot Indicates the max number of SPS configurations across all cells for subslot PUSCH. |  | - |
| pusch-SPS-MultiConfigSubslot Indicates the number of multiple SPS configurations of subslot PUSCH for each serving cell. This field is only applicable for UEs supporting FDD. |  | - |
| pusch-SPS-SlotRepPCell Indicates whether the UE supports SPS repetition for slot PUSCH for PCell. |  | Yes |
| pusch-SPS-SlotRepPSCell Indicates whether the UE supports SPS repetition for slot PUSCH for PSCell. |  | Yes |
| pusch-SPS-SlotRepSCell Indicates whether the UE supports SPS repetition for slot PUSCH for serving cells other than SpCell. |  | Yes |
| pusch-SPS-SubframeRepPCell Indicates whether the UE supports SPS repetition for subframe PUSCH for PCell. |  | Yes |
| pusch-SPS-SubframeRepPSCell Indicates whether the UE supports SPS repetition for subframe PUSCH for PSCell. |  | Yes |
| pusch-SPS-SubframeRepSCell Indicates whether the UE supports SPS repetition for subframe PUSCH for serving cells other than SpCell. |  | Yes |
| pusch-SPS-SubslotRepPCell Indicates whether the UE supports SPS repetition for subslot PUSCH for PCell. This field is only applicable for UEs supporting FDD. |  | - |
| pusch-SPS-SubslotRepPSCell Indicates whether the UE supports SPS repetition for subslot PUSCH for PSCell. This field is only applicable for UEs supporting FDD. |  | - |
| pusch-SPS-SubslotRepSCell Indicates whether the UE supports SPS repetition for subslot PUSCH for serving cells other than SpCell. This field is only applicable for UEs supporting FDD. |  | - |
| pusch-SRS-PowerControl-SubframeSet Indicates whether the UE supports subframe set dependent UL power control for PUSCH and SRS. This field is only applicable for UEs supporting TDD. |  | Yes |
| qcl-CRI-BasedCSI-Reporting Indicates whether the UE supports CRI based CSI feedback for the FeCoMP feature as specified in TS 36.213 [23], clause 7.1.10. |  | - |
| qcl-TypeC-Operation The UE uses this field to indicate the support of all of the following three features: QCL Type-C operation for FeCoMP, the capability to support separate PDSCH RE mapping for different PDSCH CWs in non-coherent joint transmission and the capability to support handling new DMRS port to MIMO layer mapping for the CWs, as specified in TS 36.213 [23], clause 7.1.10. |  | - |
| qoe-MeasReport Indicates whether the UE supports QoE Measurement Collection for streaming services. |  | - |
| qoe-MTSI-MeasReport Indicates whether the UE supports QoE Measurement Collection for MTSI services. |  |  |
| rach-Less Indicates whether the UE supports RACH-less handover, and whether the UE which indicates dc-Parameters supports RACH-less SeNB change, as defined in TS 36.300 [9]. |  | - |
| rach-Report Indicates whether the UE supports delivery of rach-Report. |  | - |
| rach-Report-ForNR Indicates whether the UE supports NR RACH report in LTE, upon request from the network. |  | - |
| rai-Support Defines whether the UE supports release assistance indication (RAI) as specified in TS 36.321 [6] for BL UEs. |  | No |
| rai-SupportEnh Indicates whether the UE supports 2-bit RAI when connected to EPC as specified in TS 36.321 [6]. |  | - |
| rclwi Indicates whether the UE supports RCLWI, i.e. reception of rclwi-Configuration. The UE which supports RLCWI shall also indicate support of interRAT-ParametersWLAN-r13. The UE which supports RCLWI and wlan-IW-RAN-Rules shall also support applying WLAN identifiers received in rclwi-Configuration for the access network selection and traffic steering rules when in RRC_IDLE. |  | - |
| recommendedBitRate Indicates whether the UE supports the bit rate recommendation message from the eNB to the UE as specified in TS 36.321 [6], clause 6.1.3.13. |  | No |
| recommendedBitRateMultiplier Indicates whether the UE supports the bit rate multiplier for recommended bit rate MAC CE as specified in TS 36.321 [6], clause 6.1.3.13. If this field is included, the UE shall also include the recommendedBitRate field. |  | - |
| recommendedBitRateQuery Indicates whether the UE supports the bit rate recommendation query message from the UE to the eNB as specified in TS 36.321 [6], clause 6.1.3.13. If this field is included, the UE shall also include the recommendedBitRate field. |  | No |
| reducedCP-Latency Indicates whether the UE supports reduced CP latency. |  | Yes |
| reducedIntNonContComb Indicates whether the UE supports receiving requestReducedIntNonContComb that requests the UE to exclude supported intra-band non-contiguous CA band combinations other than included in capability signalling as specified in TS 36.306 [5], clause 4.3.5.21. |  | - |
| reducedIntNonContCombRequested Indicates that the UE excluded supported intra-band non-contiguous CA band combinations other than included in capability signalling as specified in TS 36.306 [5,] clause 4.3.5.21. |  | - |
| reflectiveQoS Indicates whether the UE supports AS reflective QoS. |  | No |
| relWeightTwoLayers/ relWeightFourLayers/ relWeightEightLayers Indicates relative weight of processing FD-MIMO with 2/ 4/ 8 layers with respect to non-FD-MIMO with the same number of layers, see NOTE 8. Value v1 corresponds to relative weight of 1, value v1dot25 corresponds to relative weight of 1.25 and so on. This field can be included only if the UE supports the corresponding number of layers (i.e., 2/ 4/ 8 layers). |  | - |
| reportCGI-NR-EN-DC Indicates whether the UE supports Inter-RAT report CGI procedure towards NR cell when it is configured with (NG)EN-DC. |  | Yes |
| reportCGI-NR-NoEN-DC Indicates whether the UE supports Inter-RAT report CGI procedure towards NR cell when it is not configured with (NG)EN-DC. |  | Yes |
| resumeWithMCG-SCellConfig Indicates whether the UE supports (re-)configuration of E-UTRA MCG SCells. |  | - |
| resumeWithSCG-Config Indicates whether the UE supports (re-)configuration of an NR SCG. |  | - |
| resumeWithStoredMCG-SCells Indicates whether the UE supports not deleting the stored E-UTRA MCG SCell configuration when initiating the resume procedure. |  | - |
| resumeWithStoredSCG Indicates whether the UE supports not deleting the stored NR SCG configuration when initiating the resume procedure. |  | - |
| srs-CapabilityPerBandPairList Indicates, for a particular pair of bands, the SRS carrier switching parameters when switching between the band pair to transmit SRS on a PUSCH-less SCell as specified in TS 36.212 [22] and TS 36.213 [23]. If included, the UE shall include a number of entries as indicated in the following, and listed in the same order, as in bandParameterList for the concerned band combination: - For the first band, the UE shall include the same number of entries as in bandParameterList i.e. first entry corresponds to first band in bandParameterList and so on, - For the second band, the UE shall include one entry less i.e. first entry corresponds to the second band in bandParameterList and so on - And so on. |  | - |
| requestedBands Indicates the frequency bands requested by E-UTRAN. |  | - |
| requestedCCsDL, requestedCCsUL Indicates the maximum number of CCs requested by E-UTRAN. |  | - |
| requestedDiffFallbackCombList Indicates the CA band combinations for which report of different UE capabilities is requested by E-UTRAN. |  | - |
| rf-RetuningTimeDL Indicates the interruption time on DL reception within a band pair during the RF retuning for switching between the band pair to transmit SRS on a PUSCH-less SCell. n0 represents 0 OFDM symbols, n0dot5 represents 0.5 OFDM symbols, n1 represents 1 OFDM symbol and so on. This field is mandatory present if switching between the band pair is supported. |  | - |
| rf-RetuningTimeUL Indicates the interruption time on UL transmission within a band pair during the RF retuning for switching between the band pair to transmit SRS on a PUSCH-less SCell. n0 represents 0 OFDM symbols, n0dot5 represents 0.5 OFDM symbols, n1 represents 1 OFDM symbol and so on. This field is mandatory present if switching between the band pair is supported. |  | - |
| rlc-AM-Ooo-Delivery Indicates whether the UE supports out-of-order delivery from RLC to PDCP for RLC AM. |  | - |
| rlc-UM-Ooo-Delivery Indicates whether the UE supports out-of-order delivery from RLC to PDCP for RLC UM. |  | - |
| rlm-ReportSupport Indicates whether the UE supports RLM event and information reporting. |  | - |
| rohc-ContextContinue Same as "continueROHC-Context" defined in TS 38.306 [87]. |  | No |
| rohc-ContextMaxSessions Same as "maxNumberROHC-ContextSessions" defined in TS 38.306 [87]. |  | No |
| rohc-Profiles Same as "supportedROHC-Profiles" defined in TS 38.306 [87]. |  | No |
| rohc-ProfilesUL-Only Same as "uplinkOnlyROHC-Profiles" defined in TS 38.306 [87]. |  | No |
| rsrqMeasWideband Indicates whether the UE can perform RSRQ measurements with wider bandwidth. |  | Yes |
| rsrq-OnAllSymbols Indicates whether the UE can perform RSRQ measurement on all OFDM symbols and also support the extended RSRQ upper value range from -3dB to 2.5dB in measurement configuration and reporting as specified in TS 36.133 [16]. |  | No |
| rs-SINR-Meas Indicates whether the UE can perform RS-SINR measurements in RRC_CONNECTED as specified in TS 36.214 [48]. |  | - |
| rssi-AndChannelOccupancyReporting Indicates whether the UE supports performing measurements and reporting of RSSI and channel occupancy. This field can be included only if downlinkLAA is included. |  | - |
| sa-NR Indicates whether the UE supports standalone NR as specified in TS 38.331 [82]. |  | No |
| scalingFactorTxSidelink, scalingFactorRxSidelink Indicates, for a particular band combination of EUTRA, the scaling facor, as defined in TS 38.306 [87], for the PC5 band combination(s) v2x-SupportedBandCombinationListEUTRA-NR on which the UE supports simultaneous transmission/reception of EUTRA and NR sidelink communication respectively, or simultaneous transmission or reception of EUTRA and joint V2X sidelink communication and NR sidelink communication respectively (as indicated by v2x-SupportedTxBandCombListPerBC-v1630 / v2x-SupportedRxBandCombListPerBC-v1630). The leading / leftmost value corresponds to the first band combination included in v2x-SupportedBandCombinationListEUTRA-NR which is indicated with value 1 by v2x-SupportedTxBandCombListPerBC-v1630 / v2x-SupportedRxBandCombListPerBC-v1630, the next value corresponds to the second band combination included in v2x-SupportedBandCombinationListEUTRA-NR which is indicated with value 1 by v2x-SupportedTxBandCombListPerBC-v1630 / v2x-SupportedRxBandCombListPerBC-v1630 and so on. For each value of ScalingFactorSidelink-r16, value f0p4 indicates the scaling factor 0.4, f0p75 indicates 0.75, and so on. |  | - |
| scptm-AsyncDC Indicates whether the UE in RRC_CONNECTED supports MBMS reception via SC-MRB on a frequency indicated in an MBMSInterestIndication message, where (according to supportedBandCombination) the carriers that are or can be configured as serving cells in the MCG and the SCG are not synchronized. If this field is included, the UE shall also include scptm-SCell and scptm-NonServingCell. |  | Yes |
| scptm-NonServingCell Indicates whether the UE in RRC_CONNECTED supports MBMS reception via SC-MRB on a frequency indicated in an MBMSInterestIndication message, where (according to supportedBandCombination and to network synchronization properties) a serving cell may be additionally configured. If this field is included, the UE shall also include the scptm-SCell field. |  | Yes |
| scptm-Parameters Presence of the field indicates that the UE supports SC-PTM reception as specified in TS 36.306 [5]. |  | Yes |
| scptm-SCell Indicates whether the UE in RRC_CONNECTED supports MBMS reception via SC-MRB on a frequency indicated in an MBMSInterestIndication message, when an SCell is configured on that frequency (regardless of whether the SCell is activated or deactivated). |  | Yes |
| scptm-ParallelReception Indicates whether the UE in RRC_CONNECTED supports parallel reception in the same subframe of DL-SCH transport blocks transmitted using C-RNTI/Semi-Persistent Scheduling C-RNTI and using SC-RNTI/G-RNTI as specified in TS 36.306 [5]. |  | Yes |
| secondSlotStartingPosition Indicates whether the UE supports reception of subframes with second slot starting position as described in TS 36.211 [21] and TS 36.213 [23]. This field can be included only if downlinkLAA is included. |  | - |
| semiOL Indicates whether the UE supports semi-open-loop transmission for the indicated transmission mode. |  | Yes |
| semiStaticCFI Indicates whether the UE supports the semi-static configuration of CFI for subframe/slot/sub-slot operation. |  | Yes |
| semiStaticCFI-Pattern Indicates whether the UE supports the semi-static configuration of CFI pattern for subframe/slot/sub-slot operation. This field is only applicable for UEs supporting TDD. |  | - |
| sharedSpectrumMeasNR-EN-DC Indicates whether the UE supports performing measurements and reporting of RSSI and channel occupancy on each supported NR band in EN-DC. If included, the UE shall include the same number of entries, and listed in the same order as in supportedBandListEN-DC-r15. |  | - |
| sharedSpectrumMeasNR-SA Indicates whether the UE supports performing measurements and reporting of RSSI and channel occupancy on each supported NR band in NR SA. If included, the UE shall include the same number of entries, and listed in the same order as in supportedBandListNR-SA-r15. |  | - |
| shortCQI-ForSCellActivation Indicates whether the UE supports additional CQI reporting periodicity after SCell activation. |  | Yes |
| shortMeasurementGap Indicates whether the UE supports shorter measurement gap length (i.e. gp2 and gp3) in LTE standalone as specified in TS 36.133 [16], and for independent measurement gap configuration on FR1 and per-UE gap in (NG)EN-DC as specified in TS38.133 [84]. |  | No |
| shortSPS-IntervalFDD Indicates whether the UE supports uplink SPS intervals shorter than 10 subframes in FDD mode. |  | - |
| shortSPS-IntervalTDD Indicates whether the UE supports uplink SPS intervals shorter than 10 subframes in TDD mode. |  | - |
| sigBasedLTELoggedMeas-OverrideProtect Indicates whether the UE supports the override protection of the signalling based logged measurements configured in E-UTRA when entering RRC_CONNECTED state in NR. |  | - |
| simultaneousPUCCH-PUSCH Indicates whether the UE supports simultaneous transmission of PUSCH/PUCCH and SlotOrSubslotPUSCH/SPUCCH (if supported). |  | Yes |
| simultaneousRx-Tx Indicates whether the UE supports simultaneous reception and transmission on different bands for each band combination listed in supportedBandCombination. This field is only applicable for inter-band TDD band combinations. A UE indicating support of simultaneousRx-Tx and dc-Support-r12 shall support different UL/DL configurations between PCell and PSCell. |  | - |
| simultaneousTx-DifferentTx-Duration Indicates whether the UE supports simultaneous transmission of different transmission durations over different carriers. The different transmission durations can be of subframe, slot or subslot duration. |  | - |
| skipFallbackCombinations Indicates whether UE supports receiving requestSkipFallbackComb that requests UE to exclude fallback band combinations from capability signalling. |  | - |
| skipFallbackCombRequested Indicates whether requestSkipFallbackComb is requested by E-UTRAN. |  | - |
| skipMonitoringDCI-Format0-1A Indicates whether UE supports blind decoding reduction on UE specific search space by not monitoring DCI Format 0 and 1A as specified in TS 36.213 [23], clause 9.1.1. |  | No |
| skipSubframeProcessing This fields defines whether the UE supports aborting reception of PDSCH if the UE receives slot-PDSCH/subslot-PDSCH during an ongoing PDSCH reception and instead starts receiving the slot-PDSCH/subslot-PDSCH, as well as whether the UE supports aborting a PUSCH transmission if the UE gets a grant for a slot-PUSCH/ subslot-PUSCH transmission that overlaps with a grant received for a PUSCH transmission. The capability indicates the number of subframes that the UE may drop prior to the subframe in which it prioritizes the processing of slot/subslot PDSCH/PUSCH as described in TS 36.213 [23], clauses 7.1 and 8.0. Separate capability for UL and DL and per sTTI length in each direction: skipProcessingDL-Slot, skipProcessingDL-Subslot, skipProcessingUL-Slot and skipProcessingUL-Subslot. |  | - |
| skipUplinkDynamic Indicates whether the UE supports skipping of UL transmission for an uplink grant indicated on PDCCH if no data is available for transmission as described in TS 36.321 [6]. |  | - |
| skipUplinkSPS Indicates whether the UE supports skipping of UL transmission for a configured uplink grant if no data is available for transmission as described in TS 36.321 [6]. |  | - |
| sl-64QAM-Rx Indicates whether the UE supports 64QAM for the reception of V2X sidelink communication. |  | - |
| sl-64QAM-Tx Indicates whether the UE supports 64QAM for the transmission of V2X sidelink communication. |  | - |
| sl-CongestionControl Indicates whether the UE supports Channel Busy Ratio measurement and reporting of Channel Busy Ratio measurement results to eNB for V2X sidelink communication. |  | - |
| sl-LowT2min Indicates whether the UE supports 10ms as minimum value of T2 for resource selection procedure of V2X sidelink communication. |  | - |
| sl-ParameterNR Includes the SidelinkParametersNR IE as specified in TS 38.331 [82]. The field includes the sidelink capability for NR-PC5, where multipleSR-ConfigurationsSidelink, logicalChannelSR-DelayTimerSidelink and relayParameters are not applicable. |  | - |
| sl-RateMatchingTBSScaling Indicates whether the UE supports rate matching and TBS scalling for V2X sidelink communication. |  | - |
| slotPDSCH-TxDiv-TM8 Indicates whether the UE supports TX diversity transmission using ports 7 and 8 for TM8 for slot PDSCH. |  | - |
| slotPDSCH-TxDiv-TM9and10 Indicates whether the UE supports TX diversity transmission using ports 7 and 8 for TM9/10 for slot PDSCH. |  | Yes |
| slotSymbolResourceResvDL-CE-ModeA, slotSymbolResourceResvDL-CE-ModeB, slotSymbolResourceResvUL-CE-ModeA, slotSymbolResourceResvUL-CE-ModeB Indicates whether the UE supports slot/symbol-level time-domain resource reservation in downlink/uplink when operating in CE mode A/B, as specified in TS 36.211 [21] and TS 36.213 [23]. |  | Yes |
| slss-SupportedTxFreq Indicates whether the UE supports the SLSS transmission on single carrier or on multiple carriers in the case of sidelink carrier aggregation. |  | - |
| slss-TxRx Indicates whether the UE supports SLSS/PSBCH transmission and reception in UE autonomous resource selection mode and eNB scheduled mode in a band for V2X sidelink communication. |  | - |
| sl-TxDiversity Indicates whether the UE supports transmit diversity for V2X sidelink communication. See TS 36.101 [42]. |  | - |
| sn-SizeLo Same as "shortSN" defined in TS 38.306 [87]. |  | No |
| spatialBundling-HARQ-ACK Indicates whether UE supports HARQ-ACK spatial bundling on PUCCH or PUSCH as specified in TS 36.213 [23], clauses 7.3.1 and 7.3.2. |  | No |
| spdcch-differentRS-types Indicates whether the UE supports monitoring of sPDCCH on RB sets with different RS types within a TTI. |  | Yes |
| spdcch-Reuse Indicates whether the UE supports L1 based SPDCCH reuse. |  | Yes |
| sps-CyclicShift Indicates whether the UE supports RRC configuration of cyclic shift for DMRS for UL SPS using 1ms TTI. |  | Yes |
| sps-ServingCell Indicates whether the UE supports multiple UL/DL SPS configurations simultaneously active on different serving cells as specified in TS 36.321 [6]. |  | - |
| sps-STTI Indicates whether the UE supports SPS in DL and/or UL for slot or subslot based PDSCH and PUSCH, respectively. |  | Yes |
| srs-DCI7-TriggeringFS2 Indicates whether the UE supports SRS triggerring via DCI format 7 for FS2. |  | - |
| srs-Enhancements Indicates whether the UE supports SRS enhancements. |  | Yes |
| srs-EnhancementsTDD Indicates whether the UE supports TDD specific SRS enhancements. |  | Yes |
| srs-FlexibleTiming Indicates whether the UE supports configuration of soundingRS-FlexibleTiming-r14 for the corresponding band pair. For a TDD-TDD band pair, UE shall include at least one of srs-FlexibleTiming and/or srs-HARQ-ReferenceConfig when rf-RetuningTimeDL or rf-RetuningTimeUL corresponding to the band pair is larger than 1 OFDM symbol. |  | - |
| srs-HARQ-ReferenceConfig Indicates whether the UE supports configuration of harq-ReferenceConfig-r14 for the corresponding band pair. For a TDD-TDD band pair, UE shall include at least one of srs-FlexibleTiming and/or srs-HARQ-ReferenceConfig when rf-RetuningTimeDL or rf-RetuningTimeUL corresponding to the band pair is larger than 1 OFDM symbol. |  | - |
| srs-MaxSimultaneousCCs Indicates the maximum number of simultaneously configurable target CCs for SRS switching (i.e., CCs for which srs-SwitchFromServCellIndex is configured) supported by the UE. |  | - |
| srs-UpPTS-6sym Indicates whether the UE supports up to 6-symbol SRS in UpPTS. |  | - |
| srvcc-FromUTRA-FDD-ToGERAN Indicates whether UE supports SRVCC handover from UTRA FDD PS HS to GERAN CS. |  | - |
| srvcc-FromUTRA-FDD-ToUTRA-FDD Indicates whether UE supports SRVCC handover from UTRA FDD PS HS to UTRA FDD CS. |  | - |
| srvcc-FromUTRA-TDD128-ToGERAN Indicates whether UE supports SRVCC handover from UTRA TDD 1.28Mcps PS HS to GERAN CS. |  | - |
| srvcc-FromUTRA-TDD128-ToUTRA-TDD128 Indicates whether UE supports SRVCC handover from UTRA TDD 1.28Mcps PS HS to UTRA TDD 1.28Mcps CS. |  | - |
| ss-CCH-InterfHandl Indicates whether the UE supports synchronisation signal and common channel interference handling. |  | Yes |
| ss-SINR-Meas-NR-FR1, ss-SINR-Meas-NR-FR2 Indicates whether the UE can perform NR SS-SINR measurement for a frequency range (i.e. FR1 or FR2) as specified in TS 38.215 [89]. |  | - |
| ssp10-TDD-Only Indicates the UE supports special subframe configuration 10 when operating only in TDD carriers (i.e., not in TDD/FDD CA or TDD/FS3 CA). A UE including this field shall not include tdd-SpecialSubframe-r14. |  | - |
| standaloneGNSS-Location Indicates whether the UE is equipped with a standalone GNSS receiver that may be used to provide detailed location information in RRC measurement report and logged measurements. |  | - |
| sTTI-SPT-Supported Indicates whether the UE supports the features STTI and/or SPT. If the UE supports STTI and/or SPT features, the UE shall report the field sTTI-SPT-Supported set to supported in capability signalling, irrespective of whether requestSTTI-SPT-Capability field is present or not. |  | - |
| sTTI-FD-MIMO-Coexistence Indicates whether the UE supports CSI feedback for more than 8 NZP CSI-RS ports on subframe based PUSCH in any serving cell and supporting STTI in any serving cell. |  | - |
| sTTI-SupportedCombinations Indicates the different combinations of short TTI lengths, see field description for dl-STTI-Length and ul-STTI-Length, that the UE supports in a single PUCCH group or in two PUCCH groups. A short TTI length combination is reported for DL first followed by UL. In case of two PUCCH groups the support for the primary PUCCH group is indicated first. |  | - |
| subcarrierPuncturingCE-ModeA, subcarrierPuncturingCE-ModeB Indicates whether the UE supports subcarrier puncturing in downlink when operating in CE mode A/B, as specified in TS 36.211 [21] and TS 36.213 [23]. |  | Yes |
| subcarrierSpacingMBMS-khz7dot5, subcarrierSpacingMBMS-khz1dot25 Indicates the supported subcarrier spacings for MBSFN subframes in addition to 15 kHz subcarrier spacing. subcarrierSpacingMBMS-khz1dot25 and subcarrierSpacingMBMS-khz7dot5 indicates that the UE supports 1.25 and 7.5 kHz respectively for MBSFN subframes as described in TS 36.211 [21], clause 6.12. This field is included only if fembmsMixedCell or fembmsDedicatedCell is included. |  | - |
| subcarrierSpacingMBMS-khz2dot5, subcarrierSpacingMBMS-khz0dot37 Presence of this field indicates the supported subcarrier spacings of 2.5kHz / 0.37kHz for MBSFN subframes in addition to 15 kHz subcarrier spacing when operating on the E-UTRA band given by the entry in mbms-SupportedBandInfoList as described in TS 36.211 [21], clause 6.12. |  | - |
| subframeResourceResvDL-CE-ModeA, subframeResourceResvDL-CE-ModeB, subframeResourceResvUL-CE-ModeA, subframeResourceResvUL-CE-ModeB Indicates whether the UE supports Subframe-level time-domain resource reservation in downlink/uplink when operating in CE mode A/B, as specified in TS 36.211 [21] and TS 36.213 [23]. |  | Yes |
| subslotPDSCH-TxDiv-TM9and10 Indicates whether the UE supports TX diversity transmission using ports 7 and 8 for TM9/10 for subslot PDSCH. |  | Yes |
| supportedBandCombination Includes the supported CA band combinations, if any, and may include all the supported non-CA bands. |  | - |
| supportedBandCombinationAdd-r11 Includes additional supported CA band combinations in case maximum number of CA band combinations of supportedBandCombination is exceeded. |  | - |
| SupportedBandCombinationAdd-v11d0, SupportedBandCombinationAdd-v1250, SupportedBandCombinationAdd-v1270, SupportedBandCombinationAdd-v1320, SupportedBandCombinationAdd-v1380, SupportedBandCombinationAdd-v1390, SupportedBandCombinationAdd-v1430, SupportedBandCombinationAdd-v1450, SupportedBandCombinationAdd-v1470, SupportedBandCombinationAdd-v14b0, SupportedBandCombinationAdd-v1530, SupportedBandCombinationAdd-v1630 If included, the UE shall include the same number of entries, and listed in the same order, as in SupportedBandCombinationAdd-r11. |  | - |
| SupportedBandCombinationAdd-v1610 If included, the UE shall include the same number of entries, and listed in the same order, as in SupportedBandCombinationAdd-r11. If absent, network assumes gap is required when measurement is performed on any NR bands while UE is served by cell(s) belongs to an E-UTRA CA band combinations listed in SupportedBandCombinationAdd-r11 except for the FR2 inter-RAT measurement which depends on the support of independentGapConfig. |  | - |
| SupportedBandCombinationExt, SupportedBandCombination-v1090, SupportedBandCombination-v10i0, SupportedBandCombination-v1130, SupportedBandCombination-v1250, SupportedBandCombination-v1270, SupportedBandCombination-v1320, SupportedBandCombination-v1380, SupportedBandCombination-v1390, SupportedBandCombination-v1430, SupportedBandCombination-v1450, SupportedBandCombination-v1470, SupportedBandCombination-v14b0, SupportedBandCombination-v1530, SupportedBandCombination-v1630 If included, the UE shall include the same number of entries, and listed in the same order, as in supportedBandCombination-r10. |  | - |
| SupportedBandCombination-v1610 If included, the UE shall include the same number of entries, and listed in the same order, as in supportedBandCombination-r10. If absent, network assumes gap is required when measurement is performed on any NR bands while UE is served by cell(s) belongs to an E-UTRA CA band combinations listed in supportedBandCombination-r10 except for the FR2 inter-RAT measurement which depends on the support of independentGapConfig. |  | - |
| supportedBandCombinationReduced Includes the supported CA band combinations, and may include the fallback CA combinations specified in TS 36.101 [42], clause 4.3A. This field also indicates whether the UE supports reception of requestReducedFormat. |  | - |
| SupportedBandCombinationReduced-v1320, SupportedBandCombinationReduced-v1380, SupportedBandCombinationReduced-v1390, SupportedBandCombinationReduced-v1430, SupportedBandCombinationReduced-v1450, SupportedBandCombinationReduced-v1470, SupportedBandCombinationReduced-v14b0, SupportedBandCombinationReduced-v1530, SupportedBandCombinationReduced-v1630 If included, the UE shall include the same number of entries, and listed in the same order, as in supportedBandCombinationReduced-r13. |  | - |
| SupportedBandCombinationReduced-v1610 If included, the UE shall include the same number of entries, and listed in the same order, as in supportedBandCombinationReduced-r13. If absent, network assumes gap is required when measurement is performed on any NR bands while UE is served by cell(s) belongs to an E-UTRA CA band combinations listed in supportedBandCombinationReduced-r13 except for the FR2 inter-RAT measurement which depends on the support of independentGapConfig. |  | - |
| SupportedBandGERAN GERAN band as defined in TS 45.005 [20]. |  | No |
| SupportedBandList1XRTT One entry corresponding to each supported CDMA2000 1xRTT band class. |  | - |
| SupportedBandListEUTRA Includes the supported E-UTRA bands. This field shall include all bands which are indicated in BandCombinationParameters. |  | - |
| SupportedBandListEUTRA-v9e0, SupportedBandListEUTRA-v1250, SupportedBandListEUTRA-v1310, SupportedBandListEUTRA-v1320 If included, the UE shall include the same number of entries, and listed in the same order, as in supportedBandListEUTRA (i.e. without suffix). |  | - |
| SupportedBandListGERAN |  | No |
| SupportedBandListHRPD One entry corresponding to each supported CDMA2000 HRPD band class. |  | - |
| SupportedBandListNR-SA Includes the NR bands supported by the UE in NR-SA (for handover and redirection). The field is included in case the UE supports NR SA as specified in TS 38.331 [32] and not otherwise. The presence of this field also indicates that the UE can perform both NR SS-RSRP and SS-RSRQ measurement in the included NR band(s) as specified in TS 38.215 [89]. |  | No |
| supportedBandListEN-DC Includes the NR bands supported by the UE in (NG)EN-DC. The field is included in case the parameter en-DC or ng-EN-DC is present and set to supported and not otherwise. The presence of this field also indicates that the UE can perform both NR SS-RSRP and SS-RSRQ measurement in the included NR band(s) as specified in TS 38.215 [89]. |  | - |
| supportedBandListWLAN Indicates the supported WLAN bands by the UE. |  | - |
| SupportedBandUTRA-FDD UTRA band as defined in TS 25.101 [17]. |  | - |
| SupportedBandUTRA-TDD128 UTRA band as defined in TS 25.102 [18]. |  | - |
| SupportedBandUTRA-TDD384 UTRA band as defined in TS 25.102 [18]. |  | - |
| SupportedBandUTRA-TDD768 UTRA band as defined in TS 25.102 [18]. |  | - |
| supportedBandwidthCombinationSet The supportedBandwidthCombinationSet indicated for a band combination is applicable to all bandwidth classes indicated by the UE in this band combination. Field encoded as a bit map, where bit N is set to "1" if UE support Bandwidth Combination Set N for this band combination, see 36.101 [42]. The leading / leftmost bit (bit 0) corresponds to the Bandwidth Combination Set 0, the next bit corresponds to the Bandwidth Combination Set 1 and so on. The UE shall neither include the field for a non-CA band combination, nor for a CA band combination for which the UE only supports Bandwidth Combination Set 0. |  | - |
| supportedCellGrouping This field indicates for which mapping of serving cells to cell groups (i.e. MCG or SCG) the UE supports asynchronous DC. This field is only present for a band combination with more than two but less than six band entries where the UE supports asynchronous DC. If this field is not present but asynchronous operation is supported, the UE supports all possible mappings of serving cells to cell groups for the band combination. The bitmap size is selected based on the number of entries in the combinations, i.e., in case of three entries, the bitmap corresponding to threeEntries is selected and so on. A bit in the bit string set to 1 indicates that the UE supports asynchronous DC for the cell grouping option represented by the concerned bit position. Each bit position represents a different cell grouping option, as illustrated by a table, see NOTE 5. A cell grouping option is represented by a number of bits, each representing a particular band entry in the band combination with the left-most bit referring to the band listed first in the band combination, etc. Value 0 indicates that the carriers of the corresponding band entry are mapped to a first cell group, while value 1 indicates that the carriers of the corresponding band entry are mapped to a second cell group. It is noted that the mapping table does not include entries with all bits set to the same value (0 or 1) as this does not represent a DC scenario (i.e. indicating that the UE supports that all carriers of the corresponding band entry are in one cell group). |  | - |
| supportedCSI-Proc, sTTI-SupportedCSI-Proc Indicates the maximum number of CSI processes supported on a component carrier within a band. Value n1 corresponds to 1 CSI process, value n3 corresponds to 3 CSI processes, and value n4 corresponds to 4 CSI processes. If this field is included, the UE shall include the same number of entries listed in the same order as in BandParameters/STTI-SPT-BandParameters. If the UE supports at least 1 CSI process on any component carrier, then the UE shall include this field in all bands in all band combinations. |  | - |
| supportedCSI-Proc (in FeatureSetDL-PerCC) In MR-DC, indicates the number of CSI processes for the component carrier in the corresponding bandwidth class. If the UE supports at least 1 CSI process, then the UE shall include this field. |  | - |
| supportedMIMO-CapabilityDL-MRDC (in FeatureSetDL-PerCC) In MR-DC, indicates the maximum number of supported layers in TM9/10 for the component carrier in the corresponding bandwidth class. |  | - |
| supportedNAICS-2CRS-AP If included, the UE supports NAICS for the band combination. The UE shall include a bitmap of the same length, and in the same order, as in naics-Capability-List, to indicate 2 CRS AP NAICS capability of the band combination. The first/ leftmost bit points to the first entry of naics-Capability-List, the second bit points to the second entry of naics-Capability-List, and so on. For band combinations with a single component carrier, UE is only allowed to indicate {numberOfNAICS-CapableCC, numberOfAggregatedPRB} = {1, 100} if NAICS is supported. |  | - |
| supportedOperatorDic Indicates whether the UE supports operator defined dictionary. If UE supports operator defined dictionary, the UE shall report versionOfDictionary and associatedPLMN-ID of the stored operator defined dictionary. This parameter is not required to be present if the UE is in VPLMN. In this release of the specification, UE can only support one operator defined dictionary. The associatedPLMN-ID is only associated to the operator defined dictionary which has no relationship with UE's HPLMN ID. |  | - |
| supportRohcContextContinue Indicates whether the UE supports ROHC context continuation operation where the UE does not reset the current ROHC context upon handover. |  | - |
| supportedROHC-Profiles Indicates the ROHC profiles that UE supports in both uplink and downlink. |  | - |
| supportedUplinkOnlyROHC-Profiles Indicates the ROHC profiles that UE supports in uplink and not in downlink, see TS 36.323 [8] |  | - |
| supportedStandardDic Indicates whether the UE supports standard dictionary for SIP and SDP as specified in TS 36.323 [8]. |  | - |
| supportedUDC Indicates whether the UE supports UL data compression, see TS 36.323 [8]. |  | - |
| tdd-SpecialSubframe Indicates whether the UE supports TDD special subframe defined in TS 36.211 [21]. A UE shall indicate tdd-SpecialSubframe-r11 if it supports the TDD special subframes ssp7 and ssp9. A UE shall indicate tdd-SpecialSubframe-r14 if it supports the TDD special subframe ssp10, except when ssp10-TDD-Only-r14 is included. |  | Yes |
| tdd-FDD-CA-PCellDuplex The presence of this field indicates that the UE supports TDD/FDD CA in any supported band combination including at least one FDD band with bandParametersUL and at least one TDD band with bandParametersUL. The first bit is set to "1" if UE supports the TDD PCell. The second bit is set to "1" if UE supports FDD PCell. This field is included only if the UE supports band combination including at least one FDD band with bandParametersUL and at least one TDD band with bandParametersUL. If this field is included, the UE shall set at least one of the bits as "1". If this field is included with DC, then it is applicable within a CG, and the presence of this field indicates the capability of the UE to support TDD/FDD CA with at least one FDD band and at least one TDD band in the same CG, with the value indicating the support for TDD/FDD PCell (PSCell). |  | No |
| tdd-TTI-Bundling The presence of this field indicates whether the UE supporting TDD special subframe configuration 10 also supports TTI bundling for TDD configuration 2 and 3 when PUSCH transimission in UpPTS is configured, see TS 36.213 [23], clause 8.0. If this field is present, the tdd-SpecialSubframe-r14 or ssp10-TDD-Only-r14 shall be present. |  | Yes |
| timeReferenceProvision Indicates whether the UE supports provision of time reference in DLInformationTransfer message. |  | - |
| timeSeparationSlot2, timeSeparationSlot4 Indicates whether the UE supports time staggering length of 2 slots (MBSFN reference signal pattern type 2) / 4 slots (MBSFN reference signal pattern type 1) for MBSFN-RS associated with PMCH with subcarrier spacing of 0.37 kHz for MBSFN subframes when operating on the E-UTRA band given by the entry in mbms-SupportedBandInfoList as described in TS 36.211 [21], clause 6.10.2.2.4. |  | - |
| timerT312 Indicates whether the UE supports T312. |  | No |
| tm5-FDD Indicates whether the UE supports the PDSCH transmission mode 5 in FDD. |  | - |
| tm5-TDD Indicates whether the UE supports the PDSCH transmission mode 5 in TDD. |  | - |
| tm6-CE-ModeA Indicates whether the UE supports tm6 operation in CE mode A, see TS 36.213 [23], clause 7.2.3. This field can be included only if ce-ModeA is included. |  | Yes |
| tm8-slotPDSCH Indicates whether the UE supports configuration and decoding of TM8 for slot PDSCH in TDD. |  | - |
| tm9-CE-ModeA Indicates whether the UE supports tm9 operation in CE mode A, see TS 36.213 [23], clause 7.2.3. This field can be included only if ce-ModeA is included. |  | Yes |
| tm9-CE-ModeB Indicates whether the UE supports tm9 operation in CE mode B, see TS 36.213 [23], clause 7.2.3. This field can be included only if ce-ModeB is included. |  | Yes |
| tm9-LAA Indicates whether the UE supports tm9 operation on LAA cell(s). This field can be included only if downlinkLAA is included. |  | - |
| tm9-slotSubslot Indicates whether the UE supports configuration and decoding of TM9 for slot and/or subslot PDSCH for non-MBSFN. |  | Yes |
| tm9-slotSubslotMBSFN Indicates whether the UE supports configuration and decoding of TM9 for slot and/or subslot PDSCH for MBSFN. |  | Yes |
| tm9-With-8Tx-FDD Indicates whether the UE supports PDSCH transmission mode 9 with 8 CSI reference signal ports for FDD when not operating in CE mode. |  | Yes |
| tm10-LAA Indicates whether the UE supports tm10 operation on LAA cell(s). This field can be included only if downlinkLAA is included. |  | - |
| tm10-slotSubslot Indicates whether the UE supports configuration and decoding of TM10 for slot and/or subslot PDSCH for non-MBSFN. |  | Yes |
| tm10-slotSubslotMBSFN Indicates whether the UE supports configuration and decoding of TM10 for slot and/or subslot PDSCH for MBSFN. |  | Yes |
| totalWeightedLayers Indicates total number of weighted layers the UE can process for FD-MIMO. See NOTE 8. |  | - |
| twoAntennaPortsForPUCCH |  | No |
| twoStepSchedulingTimingInfo Presence of this field indicates that the UE supports uplink scheduling using PUSCH trigger A and PUSCH trigger B (as defined in TS 36.213 [23]). This field also indicates the timing between the PUSCH trigger B and the earliest time the UE supports performing the associated UL transmission. For reception of PUSCH trigger B in subframe N, value nPlus1 indicates that the UE supports performing the UL transmission in subframe N+1, value nPlus2 indicates that the UE supports performing the UL transmission in subframe N+2, and so on. This field can be included only if uplinkLAA is included. |  | - |
| txAntennaSwitchDL, txAntennaSwitchUL The presence of txAntennaSwitchUL indicates the UE supports transmit antenna selection for this UL band in the band combination as described in TS 36.213 [23], clauses 8.2 and 8.7. The field txAntennaSwitchDL indicates the entry number of the first-listed band with UL in the band combination that affects this DL. The field txAntennaSwitchUL indicates the entry number of the first-listed band with UL in the band combination that switches together with this UL. Value 1 means first entry, value 2 means second entry and so on. All DL and UL that switch together indicate the same entry number. For the case of carrier switching, the antenna switching capability for the target carrier configuration is indicated as follows: For UE configured with a set of component carriers belonging to a band combination Cbaseline = {b1(1),…,bx(1),…,by(0),…}, where "1/0" denotes whether the corresponding band has an uplink, if a component carrier in bx is to be switched to a component carrier in by (according to srs-SwitchFromServCellIndex), the antenna switching capability is derived based on band combination Ctarget = {b1(1),…,bx(0),…,by(1),…}. |  | - |
| txDiv-PUCCH1b-ChSelect Indicates whether the UE supports transmit diversity for PUCCH format 1b with channel selection. |  | Yes |
| txDiv-SPUCCH Indicates whether the UE supports Tx diversity on SPUCCH format 1/1a/1b/3. |  | Yes |
| tx-Sidelink, rx-Sidelink Indicates that the UE supports sidelink transmission/reception on the band in the band combination. For NR sidelink transmission, tx-Sidelink is only applicable if the UE supports at least one of sl-TransmissionMode1-r16 and sl-TransmissionMode2-r16 on the band as specified in TS 38.331 [82]. For NR sidelink reception, rx-Sidelink is only applicable if the UE supports sl-Reception-r16 on the band as specified in TS 38.331 [82]. |  | - |
| uci-PUSCH-Ext Indicates whether the UE supports an extension of UCI delivering more than 22 HARQ-ACK bits on PUSCH as specified in TS 36.212 [22], clause 5.2.2.6 and TS 36.213 [23], clause 8.6.3. |  | No |
| ue-AutonomousWithFullSensing Indicates whether the UE supports transmitting PSCCH/PSSCH using UE autonomous resource selection mode with full sensing (i.e., continuous channel monitoring) for V2X sidelink communication and the UE supports maximum transmit power associated with Power class 3 V2X UE, see TS 36.101 [42]. |  | - |
| ue-AutonomousWithPartialSensing Indicates whether the UE supports transmitting PSCCH/PSSCH using UE autonomous resource selection mode with partial sensing (i.e., channel monitoring in a limited set of subframes) for V2X sidelink communication and the UE supports maximum transmit power associated with Power class 3 V2X UE, see TS 36.101 [42]. |  | - |
| ue-Category UE category as defined in TS 36.306 [5]. Set to values 1 to 12 in this version of the specification. |  | - |
| ue-CategoryDL UE DL category as defined in TS 36.306 [5]. Value n17 corresponds to UE category 17, value m1 corresponds to UE category M1, value oneBis corresponds to UE category 1bis, value m2 corresponds to UE category M2. For ASN.1 compatibility, a UE indicating DL category 0, m1 or m2 shall also indicate any of the categories (1..5) in ue-Category (without suffix), which is ignored by the eNB, a UE indicating UE category oneBis shall also indicate UE category 1 in ue-Category (without suffix), and a UE indicating UE category m2 shall also indicate UE category m1. The field ue-CategoryDL is set to values 0, m1, oneBis, m2, 4, 6, 7, 9 to 16, n17, 18, 19, 20, 21, 22, 23, 24, 25, 26 in this version of the specification. |  | - |
| ue-CategorySL-C-TX UE SL category for V2X transmission as defined in TS 36.306 [5]. Set to values 1 to 5 in this version of the specification. |  | - |
| ue-CategorySL-C-RX UE SL category for V2X reception as defined in TS 36.306 [5]. Set to values 1 to 4 in this version of the specification. |  | - |
| ue-CategoryUL UE UL category as defined in TS 36.306 [5]. Value n14 corresponds to UE category 14, value n16 corresponds to UE category 16 and so on. Value m1 corresponds to UE category M1, value m2 corresponds to UE category M2, value oneBis corresponds to UE category 1bis. The field ue-CategoryUL is set to values m1, m2, 0, oneBis, 3, 5, 7, 8, 13, n14, 15, n16 to n21 or 22 to 26 in this version of the specification. |  | - |
| ue-CA-PowerClass-N Indicates whether the UE supports UE power class N in the E-UTRA band combination, see TS 36.101 [42] and TS 36.307 [78]. If ue-CA-PowerClass-N is not included, UE supports the default UE power class in the E-UTRA band combination, see TS 36.101 [42]. |  | - |
| ue-CE-NeedULGaps Indicates whether the UE needs uplink gaps during continuous uplink transmission in FDD as specified in TS 36.211 [21] and TS 36.306 [5]. |  | - |
| ue-PowerClass-N, ue-PowerClass-5 Indicates whether the UE supports UE power class 1, 2, 4 or 5 in the E-UTRA band, see TS 36.101 [42] and TS 36.307 [79] and TS 36.102 [113] for NTN capable UE. UE includes either ue-PowerClass-N or ue-PowerClass-5. If neither ue-PowerClass-N nor ue-PowerClass-5 is included, UE supports the default UE power class in the E-UTRA band, see TS 36.101 [42] and TS 36.102 [113] for NTN capable UE. |  | - |
| ue-Rx-TxTimeDiffMeasurements Indicates whether the UE supports Rx - Tx time difference measurements. |  | No |
| ue-SpecificRefSigsSupported |  | No |
| ue-SSTD-Meas Indicates whether the UE supports SSTD measurements between the PCell and the PSCell as specified in TS 36.214 [48] and TS 36.133 [16]. |  | - |
| ue-TxAntennaSelectionSupported Except for the supported band combinations for which bandParameterList-v1380 is included, TRUE indicates that the UE is capable of supporting UE transmit antenna selection such that all the supported bands in the band combination are affected by transmit antenna switching, as described in TS 36.213 [23], clause 8.7. E-UTRAN ignores this field for band combinations for which bandParameterList-v1380 is included. |  | Yes |
| ue-TxAntennaSelection-SRS-1T4R Indicates whether the UE supports selecting one antenna among four antennas to transmit SRS for the corresponding band of the band combination as described in TS 36.213 [23]. |  | - |
| ue-TxAntennaSelection-SRS-2T4R-2Pairs Indicates whether the UE supports selecting one antenna pair between two antenna pairs to transmit SRS simultaneously for the corresponding band of the band combination as described in TS 36.213 [23]. |  | - |
| ue-TxAntennaSelection-SRS-2T4R-3Pairs Indicates whether the UE supports selecting one antenna pair among three antenna pairs to transmit SRS simultaneously for the corresponding band of the band combination as described in TS 36.213 [23]. |  | - |
| ul-64QAM Indicates whether the UE supports 64QAM in UL on the band. This field is only present when the field ue-CategoryUL indicates UL UE category that supports UL 64QAM, see TS 36.306 [5], Table 4.1A-2. If the field is present for one band, the field shall be present for all bands including downlink only bands. |  | - |
| ul-256QAM Indicates whether the UE supports 256QAM in UL on the band in the band combination. This field is only present when the field ue-CategoryUL indicates UL UE category that supports 256QAM in UL, see TS 36.306 [5], Table 4.1A-2. The UE includes this field only if the field ul-256QAM-perCC-InfoList is not included. |  | - |
| ul-256QAM (in FeatureSetUL-PerCC) Indicates whether the UE supports 256QAM in UL for MR-DC within the indicated feature set. This field is only present when the field ue-CategoryUL indicates UL UE category that supports 256QAM in UL, see TS 36.306 [5], Table 4.1A-2. |  | - |
| ul-256QAM-perCC-InfoList Indicates, per serving carrier of which the corresponding bandwidth class includes multiple serving carriers (i.e. bandwidth class B, C, D and so on), whether the UE supports 256QAM in the band combination. The number of entries is equal to the number of component carriers in the corresponding bandwidth class. The UE shall support the setting indicated in each entry of the list regardless of the order of entries in the list. This field is only present when the field ue-CategoryUL indicates UL UE category that supports 256QAM in UL, see TS 36.306 [5], Table 4.1A-2. The UE includes this field only if the field ul-256QAM is not included. |  | - |
| ul-256QAM-Slot Indicates whether the UE supports 256QAM in UL for slot TTI operation on the band. |  | - |
| ul-256QAM-Subslot Indicates whether the UE supports 256QAM in UL for subslot TTI operation on the band. |  | - |
| ul-AsyncHarqSharingDiff-TTI-Lengths Indicates whether the UE supports UL asynchronous HARQ sharing between different TTI lengths for an UL serving cell. |  | Yes |
| ul-CoMP Indicates whether the UE supports UL Coordinated Multi-Point operation. |  | No |
| ul-dmrs-Enhancements Indicates whether the UE supports UL DMRS enhancements as defined in TS 36.211 [21], clause 6.10.3A. |  | Yes |
| ul-PDCP-AvgDelay Indicates whether the UE supports UL PDCP Packet Average Delay measurement (as specified in TS 38.314 [103]) and reporting in RRC_CONNECTED. |  | - |
| ul-PDCP-Delay Indicates whether the UE supports UL PDCP Packet Delay per QCI measurement as specified in TS 36.314 [71]. |  | - |
| ul-powerControlEnhancements Indicates whether UE supports UplinkPowerControlDedicated. |  | Yes |
| ul-RRC-Segmentation Indicates the UE supports uplink RRC segmentation of UECapabilityInformation. |  | - |
| uplinkLAA Presence of the field indicates that the UE supports uplink LAA operation. |  | - |
| uss-BlindDecodingAdjustment Indicates whether the UE supports blind decoding adjustment on UE specific search space as defined in TS 36.213 [22]. This field can be included only if uplinkLAA is included. |  | - |
| uss-BlindDecodingReduction Indicates whether the UE supports blind decoding reduction on UE specific search space by not monitoring DCI format 0A/0B/4A/4B as defined in TS 36.213 [22]. This field can be included only if uplinkLAA is included. |  | - |
| unicastFrequencyHopping Indicates whether the UE supports frequency hopping for unicast MPDCCH/PDSCH (configured by mpdcch-pdsch-HoppingConfig) and unicast PUSCH (configured by pusch-HoppingConfig). |  | - |
| unicast-fembmsMixedSCell Indicates whether the UE supports unicast reception from FeMBMS/Unicast mixed cell. This field is included only if UE supports carrier aggregation. |  | No |
| utra-GERAN-CGI-Reporting-ENDC Indicates whether the UE supports Inter-RAT report CGI procedure towards GERAN/UTRA cell when it is configured with (NG)EN-DC wherein either MN and SN have different DRX cycles, or on-duration configured by MN does not contain on-duration configured by SN if their DRX cycles are same. |  | Yes |
| utran-ProximityIndication Indicates whether the UE supports proximity indication for UTRAN CSG member cells. |  | - |
| utran-SI-AcquisitionForHO Indicates whether the UE supports, upon configuration of si-RequestForHO by the network, acquisition and reporting of relevant information using autonomous gaps by reading the SI from a neighbouring UMTS cell. |  | Yes |
| v2x-BandParametersNR Includes the NR BandParametersSidelink-r16 IE as specified in TS 38.331 [82]. The field includes the per-band per-band-combination sidelink capability for NR-PC5. |  | - |
| v2x-BandParametersEUTRA-NR-v1710 Includes the BandParametersSidelinkEUTRA-NR-v1710 IE as specified in TS 38.331 [82]. The field includes the per-band per-band-combination sidelink capability for NR-PC5. |  | - |
| v2x-BandwidthClassTxSL, v2x-BandwidthClassRxSL The bandwidth class for V2X sidelink transmission and reception supported by the UE as defined in TS 36.101 [42], Table 5.6G.1-3. The UE explicitly includes all the supported bandwidth class combinations for V2X sidelink transmission or reception in the band combination signalling. Support for one bandwidth class does not implicitly indicate support for another bandwidth class. |  | - |
| v2x-eNB-Scheduled Indicates whether the UE supports transmitting PSCCH/PSSCH using dynamic scheduling, SPS in eNB scheduled mode for V2X sidelink communication, reporting SPS assistance information and the UE supports maximum transmit power associated with Power class 3 V2X UE, see TS 36.101 [42] in a band. |  | - |
| v2x-EnhancedHighReception Indicates whether the UE supports reception of 30 PSCCH in a subframe and decoding of 204 RBs per subframe counting both PSCCH and PSSCH in a band for V2X sidelink communication. |  | - |
| v2x-HighPower Indicates whether the UE supports maximum transmit power associated with Power class 2 V2X UE for V2X sidelink transmission in a band, see TS 36.101 [42]. |  | - |
| v2x-HighReception Indicates whether the UE supports reception of 20 PSCCH in a subframe and decoding of 136 RBs per subframe counting both PSCCH and PSSCH in a band for V2X sidelink communication. |  | - |
| v2x-nonAdjacentPSCCH-PSSCH Indicates whether the UE supports transmission and reception in the configuration of non-adjacent PSCCH and PSSCH for V2X sidelink communication. |  | - |
| v2x-numberTxRxTiming Indicates the number of multiple reference TX/RX timings counted over all the configured sidelink carriers for V2X sidelink communication. |  | - |
| v2x-SensingReportingMode3 Indicates whether the UE supports sensing measurements and reporting of measurement results in eNB scheduled mode for V2X sidelink communication. |  | - |
| v2x-SupportedBandCombinationList Indicates the supported band combination list on which the UE supports simultaneous transmission and/or reception of V2X sidelink communication. |  |  |
| v2x-SupportedBandCombinationListEUTRA-NR Indicates the supported band combination list on which the UE supports simultaneous transmission and/or reception of NR sidelink communication only, or joint V2X sidelink communication and NR sidelink communication. |  | - |
| v2x-SupportedTxBandCombListPerBC, v2x-SupportedRxBandCombListPerBC Indicates, for a particular band combination of EUTRA, the supported band combination list among v2x-SupportedBandCombinationList on which the UE supports simultaneous transmission or reception of EUTRA and V2X sidelink communication respectively. The first bit refers to the first entry of v2x-SupportedBandCombinationList, with value 1 indicating V2X sidelink transmission/reception is supported. |  | - |
| v2x-SupportedTxBandCombListPerBC-v1630, v2x-SupportedRxBandCombListPerBC-v1630 Indicates, for a particular band combination of EUTRA, the supported band combination list among v2x-SupportedBandCombinationListEUTRA-NR on which the UE supports simultaneous transmission or reception of EUTRA and NR sidelink communication respectively, or simultaneous transmission or reception of EUTRA and joint V2X sidelink communication and NR sidelink communication respectively. The first bit refers to the first entry of v2x-SupportedBandCombinationListEUTRA-NR, with value 1 indicating V2X sidelink transmission/reception is supported. |  | - |
| v2x-TxWithShortResvInterval Indicates whether the UE supports 20 ms and 50 ms resource reservation periods for UE autonomous resource selection and eNB scheduled resource allocation for V2X sidelink communication. |  | - |
| virtualCellID-BasicSRS Indicates whether the UE supports virtual cell ID for basic SRS symbol(s). |  | - |
| virtualCellID-AddSRS This field indicates whether the UE supports virtual cell ID for additional SRS symbol(s). |  | - |
| voiceOverPS-HS-UTRA-FDD Indicates whether UE supports IMS voice according to GSMA IR.58 profile in UTRA FDD. |  | - |
| voiceOverPS-HS-UTRA-TDD128 Indicates whether UE supports IMS voice in UTRA TDD 1.28Mcps. |  | - |
| widebandPRG-Slot, widebandPRG-Subslot, widebandPRG-Subframe Indicates whether the UE supports wideband precoding resource block group size for slot/subslot/subframe operation as specified in TS 36.213 [23]. |  | - |
| wlan-IW-RAN-Rules Indicates whether the UE supports RAN-assisted WLAN interworking based on access network selection and traffic steering rules. |  | - |
| wlan-IW-ANDSF-Policies Indicates whether the UE supports RAN-assisted WLAN interworking based on ANDSF policies. |  | - |
| wlan-MAC-Address Indicates the WLAN MAC address of this UE. |  | - |
| wlan-PeriodicMeas Indicates whether the UE supports periodic reporting of WLAN measurements. |  | - |
| wlan-ReportAnyWLAN Indicates whether the UE supports reporting of WLANs not listed in the measObjectWLAN. |  | - |
| wlan-SupportedDataRate Indicates the maximum WLAN data rate supported by the UE over all LWA bearers. Actual value of supported data rate is field value * 10 Mbps (i.e., value 1 corresponds to 10 Mbps, value 2 corresponds to 20 Mbps and so on). |  | - |
| zp-CSI-RS-AperiodicInfo Indicates whether the UE supports aperiodic ZP-CSI-RS transmission for the indicated transmission mode. |  | Yes |

NOTE 1:	The IE UE-EUTRA-Capability does not include AS security capability information, since these are the same as the security capabilities that are signalled by NAS. Consequently, AS need not provide "man-in-the-middle" protection for the security capabilities.

NOTE 2:	The column FDD/ TDD diff indicates if the UE is allowed to signal, as part of the additional capabilities for an XDD mode i.e. within UE-EUTRA-CapabilityAddXDD-Mode-xNM, a different value compared to the value signalled elsewhere within UE-EUTRA-Capability (i.e. the common value, supported for both XDD modes). A '-' is used to indicate that it is not possible to signal different values (used for fields for which the field description is provided for other reasons). Annex E specifies for which TDD and FDD serving cells a UE supporting TDD/FDD CA shall support a capability for which it indicates support within the capability signalling.

NOTE 2a:	From REL-15 onwards, the UE is not allowed to signal different values for FDD and TDD unless yes is indicated in column FDD/ TDD diff (i.e. no need to introduce field description solely for the purpose of indicate no).

NOTE 3:	The BandCombinationParameters for the same band combination can be included more than once.

NOTE 4:	UE CA and measurement capabilities indicate the combinations of frequencies that can be configured as serving frequencies.

NOTE 5:	The grouping of the cells to the first and second cell group, as indicated by supportedCellGrouping, is shown in the table below. The leading / leftmost bit of supportedCellGrouping corresponds to the Bit String Position 1.

| Nr of Band Entries: | 5 | 4 | 3 |
| --- | --- | --- | --- |
| Length of Bit-String: | 15 | 7 | 3 |
| Bit String Position | Cell grouping option (0= first cell group, 1= second cell group) |  |  |
| 1 | 00001 | 0001 | 001 |
| 2 | 00010 | 0010 | 010 |
| 3 | 00011 | 0011 | 011 |
| 4 | 00100 | 0100 |  |
| 5 | 00101 | 0101 |  |
| 6 | 00110 | 0110 |  |
| 7 | 00111 | 0111 |  |
| 8 | 01000 |  |  |
| 9 | 01001 |  |  |
| 10 | 01010 |  |  |
| 11 | 01011 |  |  |
| 12 | 01100 |  |  |
| 13 | 01101 |  |  |
| 14 | 01110 |  |  |
| 15 | 01111 |  |  |

NOTE 6:	UE includes the intraBandContiguousCC-InfoList-r12 also for bandwidth class A because of the presence conditions in BandCombinationParameters-v1270. For example, if UE supports CA_1A_41D band combination, if UE includes the field intraBandContiguousCC-InfoList-r12 for band 41, the UE includes intraBandContiguousCC-InfoList-r12 also for band 1.

NOTE 6a:	For multiple BandParameters entries with the same bandEUTRA and same ca-BandwidthClassDL in a supported band combination, the UE capabilities indicated by BandParameters are agnostic to the order in which they are indicated in the bandParameterList, under the condition that the set of the capabilities indicated for the concerned bandEUTRA (e.g. bandParametersDL and bandParametersUL) are used together, and the concerned BandParameters correspond to the supportedBandwithCombinationSet for which set of channel bandwidths for carrier(s) is the same among sub-blocks, as defined in TS 36.101 [42], Table 5.6A.1-3, Table 5.6A.1-4, Table 5.6A.1-5.

NOTE 7:	For a UE that indicates release X in field accessStratumRelease but supports a feature specified in release X+ N (i.e. early UE implementation), the ASN.1 comprehension requirement are specified in Annex F.

NOTE 8:	For a UE that does not include mimo-WeightedLayersCapabilities-r13, or for the case with no CC configured with FD-MIMO, the FD-MIMO processing capability condition is not applicable (i.e. considered as satisfied). For a UE that includes mimo-WeightedLayersCapabilities-r13, the FD-MIMO processing capability condition is satisfied if the equation 4.3.28.13-1 in TS 36.306 [5] is satisfied.