3GPP TSG-RAN WG2 #125bis	R2-240xxxx

Changsha, China, April 2024

Agenda Item:	7.9.4

Source:	OPPO

Title:	-

Document for:	Discussion

Introduction

This paper is to collect companies comment for SRAP corrections for R18 Relay enhancement.

Discussion

In R2-2402587, it is proposed to add the following condition in 5.3a.1.2:

- 5.3a.1.2 UE ID fields and BEARER ID field determination For an U2U SRAP SDU received from upper layer, the SRAP entity shall: - If there is an entry in sl-LocalID-PairToAddModList, in which the sl-RemoteUE-L2Identity and the sl-PeerRemoteUE-L2Identity match the source L2 ID of the L2 U2U Remote UE and the destination L2 ID of the peer L2 U2U Remote UE: - Determine the UE ID (for SRC) field corresponding to sl-RemoteUE-LocalIdentity configured for the concerned sl-RemoteUE-L2Identity and UE ID (for DST) field corresponding to sl-PeerRemoteUE-LocalIdentity configured for the concerned sl-PeerRemoteUE-L2Identity as specified in TS 38.331 [3]; - Determine the egress link on PC5 interface towards the U2U relay UE based on the concerned sl-RemoteUE-L2Identity and sl-PeerRemoteUE-L2Identity as specified in TS 38.331 [3]; - Determine the BEARER ID field for SL-SRBs as the fixed value (i.e., set 0/1/2/3 for SL-SRB0/1/2/3 respectively) or for SL-DRBs as the 5 LSBs of slrb-PC5-ConfigIndex used in end-to-end SL DRB configuration procedure as specified in TS 38.331 [3].

During the online session, there are different views on whether this change is needed or the UE behaviour on determining local ID/egress link based on the corresponding L2 ID is already clear. So the following question is to check companies’ view on whether this change is needed.

Question-1: Do you think the above change in R2-2402587 is needed?

| Company | Yes/No | Comments |
| --- | --- | --- |

Question-2: Any comments on the drafted SRAP CR?

| Company | Clause | Comment/suggested change |
| --- | --- | --- |