3GPP TSG RAN meeting #105										     RP-24xxxx

Melbourne, Australia, September 9-12, 2024

## Status Report to TSG

Agenda item:			10.3.1

| WI / SI Name | Non-Terrestrial Networks (NTN) for Internet of Things (IoT) Phase 3 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| included in this status report | Study Item: No | Core part: Yes | Performance part: Yes |  | Testing part: No |
| Acronym | IoT_NTN_Ph3 |  |  |  |  |
| Unique ID | 1020096 |  |  |  |  |
| TSG Tdoc of latest approved WI/SI description (if any) | RP-241624 |  |  |  |  |
| Target Completion Date (indicate if changed) | Study Item: N/A | Core part: 09/2025 | Performance part: 03/2026 | Testing part: N/A |  |
| Overall Completion level | Study Item: N/A | Core part: 30% RAN1: 40% RAN2: 30% RAN3: 15% RAN4: 15% | Performance Part: 0% | Testing part: N/A |  |

Note: Overall completion level percentage numbers should use one of the colors below:

- xx%: Normal progress, no RAN plenary action needed

- xx%: Progress behind schedule, may need RAN plenary intervention. If so, SR should clearly define requested action

- xx%: Progress critically behind, RAN plenary shall intervene. SR should define requested action

Source:

| Leading WG |  | RAN WG2 |
| --- | --- | --- |
| Rapporteur | Name | Chun-Fan (Felix) Tsai |
|  | Company | MediaTek Inc. |
|  | Email | Chun-Fan.tsai@mediatek.com |

## 1	Work plan related evaluation

| Do you want to modify the time budget for this WI/SI compared to what was endorsed at the last RAN meeting? | No |
| --- | --- |

If you answered No:	Then please remove the Excel file from the zip file of this status report.

If you answered Yes:	Then please fill out the attached Excel template to request a modification of the time 		budgets for your WI /SI. The Excel table has to be filled out for all affected RAN WGs and 		up to the target date of the WI/SI. The basis are the endorsed time budgets of the last 		RAN meeting. Please highlight all changes of the values.
		One time unit (TU) corresponds to ~ 2 hours in the meeting.
		If this status report covers a WI with Core and Performance part, then please have one 		line for each in the attached Excel table.
		Note: If no Excel table is attached, then this means no time budget change.

Additional explanations/motivations for the time budget changes in the attached Excel table:

## 2.	Detailed progress in RAN WGs since last TSG meeting (for all involved WGs)

NOTE: Agreements and Open issues impacted cross-TSG aspects shall be explicitly highlighted

## 2.1	RAN1

#### 2.1.1	Agreements

###### RAN1#118, Aug’24

Agreement

RAN1 studies whether the following types of UL transmission gap will impact the design of OCC for IoT-NTN when considering e.g. phase continuity

- UL gaps for synchronization (from Rel-13)

- Gaps around NPRACH occasions

- UL timing adjustment gaps and segmentation for IoT-NTN (from Rel-17)

- TDM DMRS that are muted

- Guard periods for 3.75kHz UL transmissions

Agreement

The following combinations are considered for further simulation in RAN1 for 3.75kHz SCS OCC for NPUSCH format 1:

- Option 1: OCC2, Symbol-level, TDM DMRS

- Option 2: OCC2, Symbol-level, CDM DMRS with new pattern

- Option 3: OCC2, Slot-level, TDM DMRS

- Option 4: OCC2, Slot-level, CDM DMRS with legacy pattern

- Option 6: OCC4, Symbol-level, CDM DMRS with new pattern

The following combinations are considered for further simulation in RAN1 for 15kHz SCS OCC for NPUSCH format 1:

- Option 1: OCC2, Symbol-level, TDM DMRS

- Option 3: OCC2, Slot-level, TDM DMRS

- Option 4: OCC2, Slot-level, CDM DMRS with legacy pattern

- Option 5: OCC4, Symbol-level, TDM DMRS

- Option 7: OCC4, Slot -level, TDM DMRS

- Option 8: OCC4, Slot-level, CDM DMRS with legacy pattern

Note 1: For TDM, the legacy DMRS pattern, with DMRS symbols appropriately muted/blanked is used. Companies to report their assumption on whether spreading is applied to the legacy DMRS pattern for 15 kHz SCS.

Note 2: Companies to report DMRS sequence applied.

Agreement

For 3.75kHz SCS, NPUSCH format 1 simulations are performed using an appropriate MCS with SNR at least in the range of -8dB to 0dB.

###### RAN1#117, May’24

Agreement

For 3.75kHz single-tone OCC for NPUSCH format 1, RAN1 supports either symbol-level OCC or slot-level OCC. Other OCC schemes are not pursued.

For 15kHz single-tone OCC for NPUSCH format 1, RAN1 supports either symbol-level OCC or slot-level OCC. Other OCC schemes are not pursued.

Agreement

Inter-repetition OCC for NPRACH is not studied further in RAN1.

Agreement

- For the time-domain DMRS pattern (including blanked DMRS, if any):

  - For 15kHz single-tone, RAN1 strives to reuse the Rel-17 DMRS pattern

  - For 3.75kHz single-tone

    - RAN1 studies

      - Rel-17 DMRS pattern

      - A new DMRS pattern

  - The DMRS overhead (including blanked DMRS, if any) for OCC is the same as for Rel-17

Agreement

The Rel-17 guard period locations and length for NB-IoT 3.75kHz UL slot are preserved when OCC is applied to NPUSCH format 1.

###### RAN1#116bis, Apr’24

Agreement

For the NPUSCH evaluation assumptions, update the DMRS configuration, as follows:

| DMRS configuration | For baseline evaluations: OS#4 per slot for 3.75kHz OS#3 per slot for 15kHz For OCC evaluations: Up to proponent | For baseline evaluations: OS#3 per slot for 15kHz For OCC evaluations: Up to proponent |
| --- | --- | --- |

Agreement

At least the following NPRACH OCC schemes are considered by RAN1 for study:

- Intra-symbol group OCC

- Inter-symbol group(s) OCC

- Inter-repetition OCC

Agreement

The study of OCC for NPRACH does not consider NPRACH format 2.

Agreement

The following evaluation assumptions are used for the study of OCC for NPRACH:

|  | Parameter | value |
| --- | --- | --- |
| Scenario | Orbit and elevation angle | GEO at 12.5 degrees; LEO600 at 30 degrees |
| Channel and impairments | carrier frequency | 2GHz |
|  | Channel model | NTN-TDL-C The channels from different UE are independent. |
|  | Frequency error | Uniform random selection from [-0.1 ppm, +0.1 ppm] for all UEs Variation of frequency error is negligible. |
|  | Timing error | Uniform random selection from [-97Ts, +97Ts] for all UEs Timing drift 80us/s for LEO600 and 0 for GEO. |
|  | Power imbalance | Uniformly distributed between +Pimb and -Pimb for all UEs Proponent to report the value of Pimb (can be zero) and justification for the chosen value |
| Transmitter | NPRACH format | 1 or 0 |
|  | MIMO scheme | SISO |
|  | Number of repetitions (Nrep) | Up to proponent |
|  | OCC length | Up to proponent |
|  | OCC sequence | Up to proponent |
|  | Number of UE | Up to proponent |
|  | Velocity of UE | 3km/h |
|  | Total NPRACH time / frequency resource utilisation | To be reported by proponent. |
| KPI | Target detection probability | 99% |
|  | Target false alarm probability | 0.1% |
|  | SNR operating point | Report SNR where target detection probability and false alarm probability are reached for baseline and OCC schemes |

Agreement

OCC multiplexing is not supported between a UE using NPUSCH format 1 with 3.75kHz SCS and another UE using NPUSCH format 1 with 15kHz SCS.

Agreement

For OCC of NPUSCH format 1, RAN1 will not consider multiplexing more than 4 UEs.

Agreement

For single-tone DMRS when OCC is applied to NPUSCH format 1, RAN1 considers at least the following for further study:

- TDM of DMRS. The time domain locations of DMRS for different UEs are different. No OCC is applied for the DMRS of different UEs.

  - FFS: Detailed mapping

- CDM of DMRS. The time domain locations of DMRS for different UEs are the same. Different OCCs are applied for the DMRS of different UEs.

  - FFS: Detailed mapping

- Other schemes are not precluded, including combinations of the above

Agreement

For the NPUSCH evaluation assumptions, update the frequency error assumption, as follows.

| Frequency error | Uniform random selection from [-0.1 ppm, +0.1 ppm] for all UEs Variation of frequency error is negligible. For GEO, the same frequency error is applied to each subframe of a transport block. For LEO, the same frequency error is applied to each subframe of a segment (if applied in the evaluation). Companies to report their assumption on frequency error across segments. |
| --- | --- |

###### RAN1#116, Feb’24

Agreements on 9.11.4 IoT-NTN uplink capacity/throughput enhancement

Agreement

For single-tone NPUSCH format 1 transmissions with both 3.75kHz and 15kHz SCS, the following OCC schemes are considered by RAN1 for further study:

- Time domain OCC where OCC spreads across:

  - Symbol-level

  - Slot-level

  - Repetition-level

  - RV-level

For multi-tone NPUSCH format 1 transmissions, the following OCC schemes are considered by RAN1 for further study:

- Time domain OCC where OCC spreads across:

  - Symbol-level

  - Slot-level (including Nslot level)

  - Repetition-level

  - RV-level

- Intra-symbol pre-DFT spreading OCC

Agreement

The following evaluation assumptions are used for the study of OCC for NPUSCH format 1:

|  | Parameter | value |  |  |
| --- | --- | --- | --- | --- |
| scenario | orbit | GEO | LEO600 |  |
|  | Elevation angle | 12.5 degree | 30degree |  |
| Channel and impairments | carrier frequency | 2GHz |  |  |
|  | Channel model | NTN-TDL-C The channels from different UE are independent. |  |  |
|  | Frequency error | Uniform random selection from [-0.1 ppm, +0.1 ppm] for all UEs Variation of frequency error is negligible. |  |  |
|  | Timing error | Uniform random selection from [-97Ts, +97Ts] for all UEs Timing drift 80us/s for LEO600 and 0 for GEO. |  |  |
|  | Power imbalance | Uniformly distributed between +Pimb and -Pimb for all UEs Proponent to report the value of Pimb (can be zero) and justification for the chosen value |  |  |
| transmitter | SCS | 3.75KHz and 15KHz | 15kHz |  |
|  | Number of tones | Single tone | Single tone and multi tone up to 12 tones |  |
|  | Waveform | DFT-s-OFDM |  |  |
|  | Frequency hopping | w/o frequency hopping |  |  |
|  | MIMO scheme | SISO |  |  |
|  | DMRS configuration | For baseline evaluations: OS#3 per slot for 3.75kHz OS#4 per slot for 15kHz For OCC evaluations: Up to proponent |  | For baseline evaluations: OS#4 per slot for 15kHz For OCC evaluations: Up to proponent |
|  | Number of resource unit (NRU) | Up to proponent |  | Up to proponent |
|  | Modulation order (Qm) | Up to proponent |  | Up to proponent |
|  | TBS (ITBS) | Up to proponent |  | Up to proponent |
|  | Number of repetitions (Nrep) | Up to proponent |  |  |
|  | OCC length | Up to 4 |  |  |
|  | OCC sequence | Up to proponent |  |  |
|  | Number of UE | Up to 4 |  |  |
|  | Velocity of UE | 3km/h |  |  |
| receiver | Receiver algorithm | MMSE |  |  |
|  | Channel estimation | Real channel estimation |  |  |
| KPI | SNR at 10% BLER | Report for baseline and OCC schemes |  |  |
|  | Aggregated throughput | Total throughput of up to 4 UEs multiplexed |  |  |

#### 2.1.2	Remaining Open issues

Definition of necessary physical layer features enabling

- IoT-NTN uplink capacity/throughput enhancement

## 2.2	RAN2

#### 2.2.1	Agreements

###### RAN2#127, Aug’24

Support of Store & Forward

Agreements:

1.	RAN2 adopts the SA2 study conclusions on the possible S&F architectures as the baseline for further discussion (RAN2 will only consider the full CN and spit-MME payload options)

2.	RAN2 will consider both single satellite pass and multiple satellite pass scenarios

3.	RAN2 will consider both MO and MT data within scope

4.	UE is informed whether its serving satellite is currently operating in S&F via System Information broadcast (FFS if we also need a static indication that in general the NW supports the feature)

5.	RAN2 does not further discuss whether legacy UEs will always need to be barred in a S&F network, at least not before further progress in SA2 discussion. If there will be a need for this, mechanisms to bar legacy UEs are already in place and no further impact on RAN2 specs in expected.

7.	RAN2 agrees there will be a way to bar legacy UEs (using legacy cellBarred and/or cellBarred-NTN bit) and still allow R19 S&F capable UEs. FFS on the exact solution (e.g. new barring bits or whether this is linked to some other indication)

Reduce the necessary signaling to complete an Early Data Transmission

Agreements:

1.	RAN2 will continue working on a CB-msg3 EDT-like mechanism

2.	RAN2 assumes that a DSA based scheme would not have RAN1 impacts, while RAN2 thinks that a CRDSA based scheme would necessarily have RAN1 impacts

3.	In the next meeting continue the comparison (e.g. in terms of packet loss ratio, usage of UL/DL radio resources) between existing CB mechanism (Slotted Aloha) and other mechanisms (DSA, CRDSA, others) and try to decide on which way to go and in case whether/what to ask to RAN1.

4.	For DSA and CRDSA, RAN2 can consider in the evaluation how to integrate them with repetition.

###### RAN2#126, May’24

Support of Store & Forward

Agreements:

1.	For the uplink/downlink messages transmission for MO, from RAN2 perspective the following steps are taken as baseline for S&F satellite operation (in case only eNB is on the satellite):

1) The UE sends uplink data signalling to eNB when service link is available and the eNB stores it.

2) When feeder link is available, the eNB sends the uplink data/NAS signalling to the CN.

3) The eNB (same or different) receives the downlink data/NAS signalling from the CN and stores it when feeder link is available (and service link is not available).

4) The eNB (same or different) sends the downlink data/signalling to the UE when service link is available again

2.	S&F indication can be provided by SIB (FFS on the details). RAN2 assumes that no NAS indication is needed

3.	RAN2 understands legacy UEs may be barred by legacy cellBarred and cellBarredNTN

Reduce the necessary signaling to complete an Early Data Transmission

Agreements:

- RAN2 focusses the study on contention-based Msg3 transmission to complete an EDT-like transaction (FFS on the details of Msg3. FFS on the procedural steps, e.g. how much we reuse of EDT and PUR procedures. FFS on allocation of resources).

- RAN2 can continue the discussion on Diversity Slotted ALOHA (DSA) and Contention Resolution Diversity Slotted Aloha (CRDSA) for Msg3-EDT transmissions without msg1/ RAR, evaluating possible impacts on the specification, in the next RAN2 meeting (RAN2 might send an LS to RAN1 later on this)

- If an IoT NTN UE in IDLE state is to use the new R19 contention-based procedure, the UE needs to verify/update the uplink synchronization (e.g. get GNSS fix, acquire TA) just before sending msg3.

###### RAN2#125bis, April’24

Support of Store & Forward

RAN2 assumptions:

- S&F implies that at least the full eNB will be onboard

- An IoT NTN network shall be able to inform UE(s) whether S&F Satellite operation is applied, either via NAS or AS (wait for SA2 progress on this)

3.	The S&F satellite operation is common for NB-IoT and eMTC.

4.	The S&F satellite operation is applied to both CP solution and UP solution (for the UP solution pending on SA2 conclusions on the architecture)

Reduce the necessary signaling to complete an Early Data Transmission

Agreements:

- Both NB-IoT and eMTC are within scope of uplink capacity enhancements

- Both C-plane and U-plane solutions are within scope of uplink capacity enhancements.

- Only CIoT EPS is within scope of uplink capacity enhancements

#### 2.2.2	Remaining Open issues

- RAN2 impact due to S&F operation

- Detail procedure for EDT-like transmissions without msg1/ RAR

## 2.3	RAN3

#### 2.3.1	Agreements

###### RAN3#125, Aug’24

Agreement:

RAN3 decides to work on Split MME architecture and Full CN on board architecture. Whether eNB only on board architecture is feasible or not can be discussed later

#### 2.3.2	Remaining Open issues

- Further discussion on whether eNB only on board architecture is feasible or not

- Potential standard impact of Split MME architecture and Full CN on board architecture

## 2.4	RAN4

#### 2.4.1	Agreements

###### RAN4#112, Aug’24

# Topic #1: Work plan for Rel-19 IoT_NTN_Ph3

Issue 1-1: Work plan for Rel-19 IoT_NTN_Ph3

- Approve the workplan in R4-2413529

# Topic #2: RF core requirements

NOTE1: Enhancements to enable multiplexing of multiple UEs in a single 3.75 kHz or 15 kHz subcarrier via orthogonal cover codes (OCC) for NPUSCH format 1 and NPRACH.

#### Issue 2-1: UE RF requirement impact for NPUSCH with OCC feature

- Agreement:

  - No UE RF requirement impact from symbol-level for NPUSCH

  - FFS on UE RF requirement impact from slot-level for NPUSCH

#### Issue 2-2: UE RF requirement impact for NPRACH with OCC feature

- Agreement:

  - Wait RAN1 reach conclusions on OCC feature before RAN4 evaluates the UE RF impact.

#### Issue 2-3: SAN RF requirement impact for NPUSCH/NPRACH with OCC feature

- Agreement:

  - Wait RAN1 reach conclusions for OCC feature before RAN4 evaluates the SAN RF impact.

RF

### Sub-Topic 1-1: RRM impact

The work scope of RRM for Rel-19 IoT NTN is to discuss whether and how to define timing requirements for Msg3 transmission without msg1/ Random Access Response (RAR).

- Note: It can be revisited if the RRM impact from other objectives has been identified.

RRM

Besides, the following LS is approved.

- R4-2414114 Reply LS to RAN2 on UL synchronization for contention based Msg3 transmission without Msg1/Msg2, ZTE

#### 2.4.2	Remaining Open issues

- UE RF and SAN RF requirement impact due to OCC feature

- UE RRM requirements due to Msg3 transmission without msg1/ Random Access Response (RAR).

## 2.5	RAN5

#### 2.5.1	Agreements

#### 2.5.2	Remaining Open issues

#### 2.5.3	Remaining Open issues with cross-WG dependencies

## 2.6	RAN6

#### 2.6.1	Agreements

#### 2.6.2	Remaining Open issues

## 3.	Detailed progress in SA/CT WGs since last TSG meeting (for all involved WGs)

NOTE: This section only needs to be filled in for WI/SIs where there is a corresponding relevant WI/SI in SA/CT.

## 3.1	SAx/CTs

#### 3.1.1	Agreements with cross-TSG impacts

#### 3.1.2	Remaining Open issues with cross-TSG impacts

NOTE: This section should also flag any critical dependencies that need TSG attention.

## 4.	References

List of all related Tdocs in the affected WGs since last TSG.

## 4.1	RAN1

RAN1#117, Aug’24:

R1-2405842	Discussion on UL capacity enhancements for IoT NTN	Huawei, HiSilicon

R1-2405928	Discussion on IoT-NTN uplink capacity/throughput enhancement	Spreadtrum Communications

R1-2406006	Discussion on the IoT -NTN uplink capacity/throughput enhancements	CMCC

R1-2406077	Discussion on the IoT-NTN uplink capacity/throughput enhancements	TCL

R1-2406111	IoT-NTN uplink capacity/throughput enhancement	InterDigital, Inc.

R1-2406133	Discussion on UL capacity enhancement for IoT NTN	ZTE Corporation, Sanechips

R1-2406205	Discussion on IoT-NTN uplink capacity enhancement	vivo

R1-2406232	Discussion on IoT-NTN uplink capacity/throughput enhancement	OPPO

R1-2406278	Discussion on IoT-NTN uplink capacity enhancement	Xiaomi

R1-2406362	Discussion on UL capacity enhancement for IoT NTN	CATT

R1-2406427	IoT-NTN uplink capacity enhancement	Nokia, Nokia Shanghai Bell

R1-2406449	Discussion on IoT-NTN uplink capacity/throughput enhancement	LG Electronics

R1-2406512	Discussion on uplink capacity enhancement for IoT NTN	Lenovo

R1-2406556	IoT-NTN uplink capacity/throughput enhancement	NEC

R1-2406573	IoT NTN OCC methods for NPUSCH and NPRACH	Sharp

R1-2406673	Discussion on uplink capacity/throughput enhancement for IoT-NTN	Samsung

R1-2406741	Discussion on uplink capacity/throughput enhancement for IoT NTN	ETRI

R1-2406780	IoT-NTN - uplink capacity/throughput enhancemen	MediaTek Inc.

R1-2406809	On uplink capacity enhancements for IoT-NTN	Ericsson

R1-2406866	On IoT-NTN Uplink Capacity Enhancement	Apple

R1-2407052	IOT-NTN uplink capacity/throughput enhancement	Qualcomm Incorporated

R1-2407138	Views on UL Capacity Enh for IoT-NTN	Inmarsat, Viasat

R1-2407297	FL Summary #2 for IoT-NTN	Moderator (Sony)

## 4.2	RAN2

RAN2#127, Aug’24:

R2-2406240	LS on FS_5GSAT_Ph3_ARCH conclusions (S2-2407350; contact: OPPO)	SA2	LS in	Rel-19	FS_5GSAT_Ph3_ARCH	To:SA3, SA3-LI	Cc:RAN2

R2-2406245	Reply LS on FS_5GSAT_Ph3_ARCH conclusions (s3i240477; contact: Tencastle)	SA3-LI	LS in	Rel-19	FS_5GSAT_Ph3_ARCH	To:SA2	Cc:SA3, RAN2

R2-2406251	RAN2 Aspect for S&F Operation	vivo	discussion	Rel-19	IoT_NTN_Ph3-Core

R2-2406283	RAN2 aspects of the Store and Forward satellite operation	Huawei, HiSilicon, Turkcell	discussion	Rel-19	IoT_NTN_Ph3-Core

R2-2406326	Discussion on support of store and forward operation	CATT	discussion	IoT_NTN_Ph3-Core

R2-2406526	Discussion on information for Store & Forward	ASUSTeK	discussion	Rel-19	IoT_NTN_Ph3-Core

R2-2406536	Considerations on S&F operation from device perspective	Telit Communications S.p.A., Novamint, Sateliot, Thales	discussion	Rel-19	R2-2404979	Revised

R2-2406570	Discussion on the S&F indication	Google	discussion	Rel-19	IoT_NTN_Ph3-Core

R2-2406639	Support of S&F mode operation	Qualcomm Incorporated	discussion	Rel-19	IoT_NTN_Ph3-Core

R2-2406689	Support of S&F operation in IoT NTN	Apple	discussion	Rel-19	IoT_NTN_Ph3-Core

R2-2406771	Discussion on Store & Forward satellite operation	OPPO	discussion	Rel-19	IoT_NTN_Ph3-Core

R2-2406821	RAN2 impact on S&F mode	MediaTek Inc.	discussion	IoT_NTN_Ph3-Core	R2-2405132

R2-2406874	Store and Forward support in IoT NTN	Lenovo	discussion	Rel-19

R2-2406906	The design of radio interface for IoT NTN Store & Forward	China Telecom	discussion	Rel-19	IoT_NTN_Ph3-Core

R2-2406967	Discussion on IoT NTN Store and Forward	CMCC	discussion	Rel-19	IoT_NTN_Ph3-Core

R2-2407018	Support  of Store and Forward	NEC	discussion	Rel-19	IoT_NTN_Ph3-Core

R2-2407027	Discussion on support of Store&Forward	Transsion Holdings	discussion	Rel-19

R2-2407056	Discussion on the support of store and forward	Xiaomi	discussion

R2-2407075	Radio-Interface Impacts for IoT-NTN SF Operations	Nokia, Nokia Shanghai Bell	discussion

R2-2407152	Further consideration on S&F operation in IoT NTN	ZTE Corporation, Sanechips	discussion	Rel-19	IoT_NTN_Ph3-Core	R2-2404882

R2-2407233	AS Security for Store & Forward Satellite Operation	SHARP Corporation	discussion

R2-2407237	Support for store and forward in IoT NTN	Ericsson	discussion	IoT_NTN_Ph3-Core

R2-2407256	On SA2 progress and RAN2 aspects of Store and Forward	Samsung	discussion	Rel-19	IoT_NTN_Ph3-Core

R2-2407353	Discussion on the Store and Forward satellite operation	HONOR	discussion	Rel-19	IoT_NTN_Ph3-Core

R2-2407487	Considerations on S&F operation from device perspective	Telit Communications S.p.A., Novamint, Sateliot, Thales	discussion	Rel-19	R2-2406536

R2-2407491	Consideration on S&F operation	DENSO CORPORATION	discussion	IoT_NTN_Ph3-Core

R2-2407537	Support of Store & Forward	Sequans Communications	discussion	Rel-19	IoT_NTN_Ph3-Core

R2-2406252	Further Discussion on EDT Enhancement for IoT-NTN	vivo	discussion	Rel-19	IoT_NTN_Ph3-Core

R2-2406284	Way forward for RAN2 discussion on UL capacity enhancement	Huawei, HiSilicon, Turkcell	discussion	Rel-19	IoT_NTN_Ph3-Core

R2-2406327	Consideration on the feasibility of RAN2 scope for UL capacity enhancements	CATT	discussion	IoT_NTN_Ph3-Core

R2-2406592	Discussion on uplink capacity enhancements for IOT NTN	Beijing Xiaomi Mobile Software	discussion	Rel-19

R2-2406593	Performance of Advanced Random Access Protocols	DLR	discussion	Rel-19

R2-2406640	Discussion on EDT enhancements	Qualcomm Incorporated	discussion	Rel-19	IoT_NTN_Ph3-Core

R2-2406688	Uplink capacity enhancement in IoT NTN	Apple	discussion	Rel-19	IoT_NTN_Ph3-Core

R2-2406763	Uplink Capacity Enhancement for EDT transaction	Spreadtrum Communications	discussion	Rel-19

R2-2406766	Discussion on enhanced EDT for IoT NTN	OPPO	discussion	Rel-19	IoT_NTN_Ph3-Core

R2-2406868	Consideration on UL capacity enhancement for IoT-NTN	NEC Corporation.	discussion	Rel-19	IoT_NTN_Ph3-Core

R2-2406869	Discussion on enhanced EDT	MediaTek Inc.	discussion	IoT_NTN_Ph3-Core	R2-2405133

R2-2406875	EDT for uplink capacity enhancement in NTN	Lenovo	discussion	Rel-19

R2-2406907	Contention-based Msg3-EDT for IoT NTN capacity enhancement	China Telecom	discussion	Rel-19	IoT_NTN_Ph3-Core

R2-2406974	Considerations on uplink capacity enhancement for IoT-NTN	CMCC	discussion	Rel-19	IoT_NTN_Ph3-Core

R2-2407028	Discussion on uplink capacity enhancement	Transsion Holdings	discussion	Rel-19

R2-2407121	Discussion on Contention Resolution Diversity Slotted ALOHA	TOYOTA Info Technology Center	other	Rel-19	IoT_NTN_Ph3-Core

R2-2407139	Msg3 transmission without msg1/RAR	Interdigital, Inc.	discussion	Rel-19	IoT_NTN_Ph3-Core

R2-2407140	Efficient delivery (reduced overhead) of msg4 / RRCEarlyDataComplete	Interdigital, Inc.	discussion	Rel-19	IoT_NTN_Ph3-Core

R2-2407153	Further consideration on uplink capacity enhancements in IoT NTN	ZTE Corporation, Sanechips	discussion	Rel-19	IoT_NTN_Ph3-Core	R2-2404884

R2-2407167	Consideration on UL capacity enhancement for IoT NTN	Nokia, Nokia Shanghai Bell	discussion	Rel-19	IoT_NTN_Ph3-Core

R2-2407257	Procedures for uplink capacity enhancements for IoT NTN	Samsung	discussion	Rel-19	IoT_NTN_Ph3-Core

R2-2407502	Discussion on DSA and CRDSA Performance	ESA, Eutelsat Group, Viasat, Inmarsat, Novamint, Echostar, Sateliot	discussion	Rel-19

R2-2407546	Views on UL Capacity Enhancements for IoT-NTN	Inmarsat, Viasat	discussion	Rel-19	NR_NTN_Ph3-Core	Late

R2-2407552	UL capacity enhancements objectives for IoT NTN	Ericsson	discussion	Rel-19

=> Revised in R2-2407555

R2-2407555	UL capacity enhancements objectives for IoT NTN	Ericsson	discussion	Rel-19

## 4.3	RAN3

RAN3#125, Aug’24:

R3-244463	Work Plan for Rel-19 IoT NTN   ZTE Corporation, CATT, MediaTek Inc.   Work Plan

R3-244464     Initial considerations on support of store and forward satellite operation for IoT NTN   ZTE Corporation   discussion

R3-244516	Discussion on Support of Store and Forward in NTN   THALES, Novamint, Sateliot   discussion

R3-244277	Discussion about UE attachment in Store&Forward   CATT   discussion

R3-244196	Support of Store and Forward on IoT NTN   Xiaomi   discussion

R3-244245     Support for Store and Forward operation in IoT NTN   China Telecom   discussion

R3-244287	Discussion on Support for IoT NTN for Regenerative Payload    Qualcomm Incorporated   discussion

R3-244344	Discussion on the support of Store&Forward   Nokia, Nokia Shanghai Bell   discussion

R3-244366	Support store and forward for IoT NTN    Huawei   discussion

R3-244383     Support for Regenerative Payload in IoT NTN   Ericsson, Sateliot   other

R3-244554	Discussion on support of regenerative payload for IoT NTN   Samsung   discussion

R3-244570	Discussion on support of IoT NTN   CMCC   discussion

## 4.4	RAN4

RAN4#112, Aug’24:

Work Plan

R4-2413529 Work Plan for Rel-19 IoT NTN, MediaTek inc.

RF

R4-2413530 Way Forward for [112][316] IoT_NTN_Ph3, MediaTek inc.

R4-2411471 Discussion on RF requirement impact for IoT NTN phase 3, MediaTek inc.

R4-2412992 IoT NTN UE RF impact, Ericsson

R4-2413143 NTN IoT UL capacity enhancements, Qualcomm Incorporated

RRM

R4-2414039 WF on RRM requirements for R19 IoT NTN Phase 3, MediaTek inc.

R4-2414114 Reply LS to RAN2 on UL synchronization for contention based Msg3 transmission without Msg1/Msg2, ZTE

R4-2411822 Topic summary for [112][227] IoT_NTN_Ph3, Moderator (MediaTek)

R4-2411472 Discussion on RRM requirements for IoT NTN phase 3, MediaTek inc.

R4-2411618 Initial discussion on RRM core requirements in NTN for IoT Phase 3, Xiaomi

R4-2411764 (IoT_NTN_Ph3-Core) Discussion on LS from RAN2 of Msg3 transmission timing, CMCC

R4-2412207 Discussion on RRM impacts for R19 IoT NTN, Huawei, HiSilicon

R4-2412232 Discussion on RRM requirements on Non-Terrestrial Networks (NTN) for Internet of Things (IoT) Phase 3, Ericsson

R4-2412233 Rreply LS to RAN2 on UL synchronization for contention based Msg3 transmission without Msg1/Msg2, Ericsson

R4-2412602 Consideration on RRM impacts for R19 IoT NTN Phase 3, vivo

R4-2412603 Reply LS on UL synchronization for contention based Msg3 transmission without Msg1/Msg2, vivo

R4-2412865 Reply to RAN2 LS on contention based Msg3 transmission, Nokia

R4-2413187 (IoT_NTN_Ph3-Core) Impact on RRM requirements and response to RAN2 LS, Qualcomm Incorporated