3GPP TSG-RAN WG2 Meeting #128                                                                   R2-24xxxxx

Orlando, USA, Nov 18th – 22nd, 2024

Title:	[draft]LS on emergency services and eDRX

Response to:

Release:	Rel-17

Work Item:	[NR_newRAT-Core], NR_redcap-Core

Source:	ZTE Corporation [To be RAN2]

To:	SA2, CT1

Cc:	RAN3

Contact person:	Jing Liu

liu.jing30@zte.com.cn

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	None

1	Overall description

In RAN2#128, companies discussed the UE behaviour when UE is configured with eDRX and has PDU session associated with emergency services. RAN2 notice the below description in TS 23.501 states that Idle eDRX should not be used by the UE when the it has PDU session associated with emergency services. However, it is unclear how RAN configured eDRX should be handled in this case.

#### 5.31.7.2 Extended Discontinuous Reception (DRX) for CM-IDLE and CM-CONNECTED with RRC-INACTIVE ##### 5.31.7.2.1 Overview ##### // skipped irrelevant part When the UE has PDU Session associated with emergency services, the UE and AMF follow regular discontinuous reception as defined in clause 5.4.5 and shall not use the extended idle mode DRX. Extended idle mode DRX parameters may be negotiated while the UE has PDU Session associated with emergency services. When the PDU Session associated with emergency services is released, the UE and AMF shall reuse the negotiated extended idle mode DRX parameters in the last Registration Update procedure. // skipped irrelevant part

Based on current specification, RAN eDRX is configured by RAN node when releasing the UE to RRC_INACTIVE state. Note that RAN node configures RAN eDRX only if the UE is configured with Idle eDRX by CN.

However, based on current signalling design, RAN node is unaware whether a UE is configured with PDU session for emergency services, thus, it is difficult for RAN node to choose not to configure RAN eDRX when releasing the UE, and not to use RAN eDRX when sending RAN Paging message.

RAN2 would like to ask whether standard solution is needed to address above issue, and whether RAN2 needs to capture in spec that RRC_INACTIVE UE should ignore RAN eDRX when the UE has PDU session associated with emergency services.

2	Actions

To SA2, CT1 group

ACTION: 	RAN2 kindly asks SA2 and CT1 whether any standard solution is needed to make RAN node aware of PDU session associated with emergency services, and whether RAN2 needs to capture in spec that RRC_INACTIVE UE should ignore RAN eDRX when the UE has emergency PDU session associated with emergency services?

3	Dates of next TSG-RAN WG2 meetings

TSG-RAN2 Meeting #129	17 - 21 February 2025	Athens, GR

TSG-RAN2 Meeting #129bis             07 - 11 April 2025	                           China, CN