3GPP TSG-RAN2 Meeting #129	R2-25xxxxx

Athens, Greece, 17-21 February 2025

- Title:	Discussion on MCSt

- Source:	ZTE Corporation, Sanechips

- Agenda Item:	7.6

- Document for:	Discussion, Decision

# Introduction

In this paper, we discuss following email discussion:

- [POST129][119][SL] (ZTE)

Scope: Check the relation between resource (re)selection approach a/b and {MCSt + a resource pool with/without PSFCH}, prepare and discuss stage 2 CR.

Intended outcome: 38.300 CR in R2-2501488 to be agreed.

Deadline: Short email discussion

# Approach1&2

Below definitions for approach1&2 defined in R1-2304257 are for your information:

Approach 1: “best effort for multiple TBs” - Step 1: Higher layer triggers L1 resource selection for one TB with one set of parameters (, remaining PDB, and ) - R16/17 behavior. - Step 2: L1 report a set of candidate single-slot resource (SA) according to existing L1 resource allocation procedure - R16/17 behavior. - Step 3: Higher layer selects a set of resources either randomly (R16/17 behavior) or according to a consecutive-slots criterion (new behavior) to achieve MCSt. - Step 4: Repeat Step 1-3 for different TB if required. Approach 2: “guarantee MCSt for single TB and best effort for multiple TBs” - Step 1: Higher layer triggers L1 resource selection for one TB with one set of parameters (, remaining PDB, and ) + “number of slots for MCSt” which could be derived based on CAPC of the logical channel/TB or other means. - Step 2: L1 report a set of candidate multi-slot resource (SA) according to most of the existing L1 resource allocation procedure (FFS: RSRP calculation / threshold may need to change) - Step 3: Higher layer selects a candidate multi-slot resource either randomly (R16/17 behavior) or according to a consecutive-slots criterion (new behavior). - Step 4: Repeat Step 1-3 for different TB if required.

# Discussion

## Resource pool with PSFCH

According to the below RAN2#129 agreement, Rapporteur believe we do not need to re-discuss whether MCSt is supported on resource pool with PSFCH resource:

RAN2 123bis： 5. For a resource pool configured with PSFCH resource, UE can NOT select consecutive slots (i.e., MCSt) for transmissions of a single TB. RAN2 129: RAN2 keep the conclusion of allowing MCSt for multiple TBs in resource pool with PSFCH, and not pursue optimization for the case of PFSCH between two selected consecutive slots..

### Question1

- Question1: Regarding resource pool with PSFCH, do you agree to capture following statement in 38.300?

A UE using mode 2 resource allocation with single slot candidate resources and operating in a pool of resources with PSFCH configured supports resource selection for multiple TBs MCSt.

| Company | Agree or disagree | Comments |
| --- | --- | --- |
| ZTE | Agree |  |

## Resource pool without PSFCH

For resource pool without PSFCH, following cases can be discussed. Case1a and Case1b are approach2 defined in R1-2304257. Case2b and case2c is two different implementation ways of approach1. RAN1 does not mention case2a, but rapporteur think current MAC spec cannot prevent case2a due to the random resource selection.

| Multipe slots sensing: Take N=3 as an example, N=3 is indicated to PHY layer | Case1a(Approach2) | single TB | UE use the multiple slots candidate resources by filling the same TB. |
| --- | --- | --- | --- |
|  | Case1b(Approach2) | multi-TBs | UE use the multiple slots candidate resources by filling the same TB. And based on MAC implementation, two consecutive “multi slots candidate resource” are selected for different TBs. |
| Single slots sensing: N = 1 is indicated to PHY layer or no value N is indicated to PHY layer. | Case2a | single TB | Based on MAC implementation, the single slot candidate resources for same TB are consecutive. |
|  | Case2b(Approach1) | multi-TBs | Based on MAC implementation, the single slot candidate resources for different TBs are consecutive. |
|  | Case2c(Approach1) | multi-TBs | Based on MAC implementation, the single slot candidate resources for different TBs and multiple TBs are consecutive. |

Table I: candidate cases for MCSt on pool without PSFCH

### Question2a

- Question2a: Regarding resource pool without PSFCH, do you agree below case1a is supported and following statement is correct:

A UE using mode 2 resource allocation with multiple slots candidate resources and operating in a pool of resources without PSFCH configured supports resource selection for single TBs MCSt.

| Company | Agree or disagree | Comments |
| --- | --- | --- |
| ZTE | Agree |  |

### Question2b

- Question2b: Regarding resource pool without PSFCH, do you agree below case1b is supported and following statement is correct:

A UE using mode 2 resource allocation with multiple slots candidate resources and operating in a pool of resources without PSFCH configured supports resource selection for multiple TBs MCSt.

| Company | Agree or disagree | Comments |
| --- | --- | --- |
| ZTE | Agree |  |

### Question2c

- Question2c: Regarding resource pool without PSFCH, do you agree below case2a is supported and following statement is correct:

A UE using mode 2 resource allocation with single slots candidate resources and operating in a pool of resources without PSFCH configured supports resource selection for single TBs MCSt.

| Company | Agree or disagree | Comments |
| --- | --- | --- |
| ZTE | Agree |  |

### Question2d

- Question2d: Regarding resource pool without PSFCH, do you agree below case2b/c are supported and following statement is correct:

A UE using mode 2 resource allocation with single slots candidate resources and operating in a pool of resources without PSFCH configured supports resource selection for multiple TBs MCSt.

| Company | Agree or disagree | Comments |
| --- | --- | --- |
| ZTE | Agree |  |

### Question3

- Question3: if the case1a/b and case2a/b/c are supported, which one of below options is preferred to be adopted in 38.300?

- Option1:

A UE using mode 2 resource allocation with multiple/single slot candidate resources and operating in a pool of resources without PSFCH configured supports resource selection for multiple/single TB multiple consecutive slot transmission (MCSt)

- Option2:

A UE using mode 2 resource allocation operating in a pool of resources without PSFCH configured supports resource selection for multiple consecutive slot transmission (MCSt)

| Company | option | Comments |
| --- | --- | --- |
| ZTE | 1 or 2 |  |

# Conclusion