# 3GPP TSG RAN WG2 Meeting #129      					R2-250xxxx
 Wuhan, China, 7th - 11th April 2025

Agenda item:	8.4.1

Source:	ZTE Corporation, Sanechips

Title:	Discussion for 37.340 CR

WID/SID:	NR_LPWUS-Core – Release 19

Document for:	Discussion and Decision

1	Introduction

This is a discussion document on 37.340 related issues for LP-WUS:

[Post129][213][LPWUS] Running CR for TS 37.340 (ZTE)

Intended outcome: Running CR for submission to the next meeting

Deadline:  Long

2	Discussion

Issue 1: LP-WUS monitoring in NR-DC

The following agreements have been achieved in RAN1#118bis for the support of LP-WUS for DC case:

Agreement

LP-WUS is supported when UE is configured with NR-DC in RRC CONNECTED mode

- Above is supported for the case PDCCH monitoring is triggered by LP-WUS in the same cell group

- FFS: The cell(s) where PDCCH monitoring triggered by a LP-WUS is applicable

For NR-DC case when LP-WUS is supported, RAN1 has agreed that LP-WUS can only trigger PDCCH monitoring in the same cell group. We understand the LP-WUS operation would be per cell group.

Then within each cell group, it is worth considering the LP-WUS monitoring and the follow up PDCCH monitoring behaviors.

In NR-DC MCG, similar as the DCP monitoring, we understand the LP-WUS can only be configured to be monitored on the PCell.

Question 1: Do companies agree that in NR-DC MCG, the LP-WUS can only be configured to be monitored on the PCell?

| Company | Yes/No | Comments |
| --- | --- | --- |

For NR-DC SCG, similar as the DCP monitoring, we understand the LP-WUS can only be configured to be monitored on the PSCell.

Question 2: Do companies agree that in NR-DC SCG, the LP-WUS can only be configured to be monitored on the PSCell?

| Company | Yes/No | Comments |
| --- | --- | --- |

RAN1 has agreed that LP-WUS can only trigger PDCCH monitoring in the same cell group. We understand the LP-WUS operation would be per cell group and the LP-WUS in PCell of MCG and PSCell of SCG should be configured and to be monitored independently.

Question 3: Do companies agree that the LP-WUS in PCell of MCG and PSCell of SCG should be configured and to be monitored independently?

| Company | Yes/No | Comments |
| --- | --- | --- |

Issue 2: LP-WUS support in MR-DC

Question 4: For the following MR-DC scenarios, which scenario(s) companies would like to support LP-WUS?

- NE-DC

- EN-DC

- NGEN-DC

- All

- None

| Company | NE-DC/EN-DC/NGEN-DC/all/None? | Comments |
| --- | --- | --- |

3	Comments on the CR

| Company | Comments | Rapporteur Feedback |
| --- | --- | --- |

Conclusion

Based on post-meeting email discussion, Rapporteur identify the following stage 3 open issues: