# 3GPP TSG RAN WG2 Meeting #129      					R2-250xxxx
 Wuhan, China, 7th - 11th April 2025

Agenda item:	8.4.1

Source:	ZTE Corporation, Sanechips

Title:	Discussion for 37.340 CR

WID/SID:	NR_LPWUS-Core – Release 19

Document for:	Discussion and Decision

1	Introduction

This is a discussion document on 37.340 related issues for LP-WUS:

[Post129][213][LPWUS] Running CR for TS 37.340 (ZTE)

Intended outcome: Running CR for submission to the next meeting

Deadline:  Long

2	Discussion

Issue 1: LP-WUS monitoring in NR-DC

The following agreements have been achieved in RAN1#118bis for the support of LP-WUS for DC case:

Agreement

LP-WUS is supported when UE is configured with NR-DC in RRC CONNECTED mode

- Above is supported for the case PDCCH monitoring is triggered by LP-WUS in the same cell group

- FFS: The cell(s) where PDCCH monitoring triggered by a LP-WUS is applicable

For NR-DC case when LP-WUS is supported, RAN1 has agreed that LP-WUS can only trigger PDCCH monitoring in the same cell group. We understand the LP-WUS operation would be per cell group.

Then within each cell group, it is worth considering the LP-WUS monitoring and the follow up PDCCH monitoring behaviors.

In NR-DC MCG, similar as the DCP monitoring, we understand the LP-WUS can only be configured to be monitored on the PCell.

Question 1: Do companies agree that in NR-DC MCG, the LP-WUS can only be configured to be monitored on the PCell?

| Company | Yes/No | Comments |
| --- | --- | --- |
| Ericsson | Yes | We do not see the purpose of other configuration options. FYI: the RAN1 agreement from Athens is still open whether LP-WUS is only configured on PCell/PSCell. But it is clear that when LP-WUS is detected all serving cells of the cell group wake-up and enter Active Time: Agreement - For the case when UE is configured with NR-DC with CA without dual DRX groups or without CA in RRC CONNECTED mode, - LP-WUS can be configured on a serving cell per cell-group - This does not imply that LP-WUS has to be configured on both cell groups - LP-WUS indication is applicable to all serving cells in the cell-group - Note: There is no impact to PDCCH monitoring behaviour related to SCell dormancy/activation/deactivation - FFS whether to support this case as separate UE capability from CA case - FFS UE capability for monitoring LP-WUS on one cell group or both cell groups - FFS: Whether the serving cell on which the LP-WUS is configured is only PCell/PSCell or can be RRC configured cell other than Pcell/PSCell |
| Huawei, HiSilicon | Wait for RAN1 | RAN1 is discussing: FFS: Whether the serving cell on which the LP-WUS is configured is only PCell/PSCell or can be RRC configured cell other than Pcell/PSCell We understand at least Pcell/PSCell can be supported, but whether SCell can also configured with LP-WUS depends on further RAN1 conclusion. |
| Lenovo | Yes | We prefer to reuse the principle of DCP that LP-WUS is configured on the Pcell for NR-DC MCG, but since the issue is under RAN1’s discussion, we are wondering whether RAN2 needs to make such conclusions or pending on RAN1’s agreement. |
| Sharp | Yes | The same way for DCP can be reused. Considering RAN1 is discussing this issue, maybe RAN2 can make a working assumption first and then revisit it if needed. |

For NR-DC SCG, similar as the DCP monitoring, we understand the LP-WUS can only be configured to be monitored on the PSCell.

Question 2: Do companies agree that in NR-DC SCG, the LP-WUS can only be configured to be monitored on the PSCell?

| Company | Yes/No | Comments |
| --- | --- | --- |
| Ericsson | Yes | Same as for Q2 |
| Huawei, HiSilicon | Wait for RAN1 | Same as for Q2. |
| Lenovo | Yes |  |
| Sharp | Yes |  |

RAN1 has agreed that LP-WUS can only trigger PDCCH monitoring in the same cell group. We understand the LP-WUS operation would be per cell group and the LP-WUS in PCell of MCG and PSCell of SCG should be configured and to be monitored independently.

Question 3: Do companies agree that the LP-WUS in PCell of MCG and PSCell of SCG should be configured and to be monitored independently?

| Company | Yes/No | Comments |
| --- | --- | --- |
| Ericsson | Yes |  |
| Huawei, HiSilicon | Yes for intention | We understand LP-WUS configuration and operation for MCG and SCG is independent. But whether only Pcell/PSCell can be configured depends on the Q1/2. |
| Lenovo | Yes, with comments | We agree the principle that LP-WUS in PCell of MCG and PSCell of SCG should be configured and to be monitored independently. Additionally, since it has been agreed that different LP-WUS options (i.e., opti1-1 and opt1-2) for RRC-CONNECTED mode cannot be configured simultaneously for the same UE. Then, the case that MCG and SCG are configured with different LP-WUS options should be avoided. Whether the coordination on LP-WUS configuration between MN and SN is needed, which may need further discussion on RAN3. |
| Sharp | Yes | In our understanding, the restriction of Option 1-1 and Option 1-2 not simultaneously configured for the same UE is applicable for the same cell group, and MCG and SCG can configure different options. |

Issue 2: LP-WUS support in MR-DC

Question 4: For the following MR-DC scenarios, which scenario(s) companies would like to support LP-WUS?

- NE-DC

- EN-DC

- NGEN-DC

- All

- None

| Company | NE-DC/EN-DC/NGEN-DC/all/None? | Comments |
| --- | --- | --- |
| Huawei, HiSilicon | All | Similar as DCP. |
| Ericsson | All | Assuming that there is no additional specification impact, except capturing in 37.340 that these scenarios are supported with LP-WUS. |
| Lenovo | NE-DC, NGEN-DC | In principle, the LP-WUS can be configured in MCG if MN is a gNB and configured in SCG if SN is a gNB. Both NE-DC and NGEN-DC can be supported. While for EN-DC, not sure whether the incompatibility issue exists with the EPC network. |
| Sharp | All |  |

3	Comments on the CR

| Company | Comments | Rapporteur Feedback |
| --- | --- | --- |
| Ericsson | One minor comment/suggestion: For DCP it says: In MR-DC, for power saving purpose, the UE can be configured with DCP … For LP-WUS it says: In NR-DC, the UE can be configured with LP-WUS … We could also consider, dependent on whether option 1-1 or 1-2 is configured: In NR-DC, for power saving or latency purpose, the UE can be configured with LP-WUS … |  |

Conclusion

Based on post-meeting email discussion, Rapporteur identify the following stage 3 open issues: