3GPP TSG-RAN WG2 Meeting #129-bis	R2-250xxxx

Wuhan, China, 7th – 11th April, 2025

Agenda item:	8.11.1

Source: 	Huawei, HiSilicon (Rapporteur)

Title: 	Summary of [Post129][217][SBFD] List of open issues of RRC impact

Document for:	Discussion and decision

# Introduction

This is the summary of the following email discussion:

- [Post129][217][SBFD] List of open issues of RRC impact (Huawei)

Intended outcome: List of open issues of RRC impact to submit to the next meeting, taking into account related proposals in this meeting

Deadline:  Long

The target of this email discussion is to identify the open issues of RRC impact, to assist the discussion of related issues and the correspondent implementation in RRC spec if possible. Companies are encouraged to raise the open issues of RRC impact, preferably most relevant to the completion of this WI, and to propose solution options if applicable. Please consider RAN1 latest progress of RRC impact e.g. higher layer parameter list R1-2501645.

# Open issues related to Random access in SBFD

## CFRA RO type selection related signaling

Issue description:

It is agreed that for CFRA case, the RACH RO type is indicated by the network. The relate signalling design has yet to be discussed/agreed.

Solutions:

According to [1] P5/6/7, [3] P1 , [7] P8/9 and [16] P13, the following issues are open/can be discussed:

2.1.1: For CFRA triggered by Msg1-based SI request, the RO type is indicated in SI-RequestResources of SIB1.

2.1.2: For CFRA triggered by BFR, the RO type is indicated in BeamFailureRecoveryConfig.

2.1.3: For CFRA triggered by ReconfigurationwithSync, the RO type is indicated in RACH-ConfigDedicated.

2.1.4: Within the corresponding RACH configuration for the additional ROs, one bit is used to indicate the RO type for CFRA.

2.1.5: For RO type indication in CFRA, the following RRC and MAC spec impact should be considered:

- Add an RO type indication in BeamFailureRecoveryConfig to indicate whether the corresponding RACH-ConfigGeneric only contains valid SBFD RO;

- Add an RO type indication in rach-ConfigDedicated to indicate whether the corresponding RACH-ConfigGeneric only contains valid SBFD RO;

- Add an RO type indication in EarlyUL-SyncConfig to indicate whether the corresponding RACH-ConfigGeneric only contains valid SBFD RO;

- Add an RO type indication in LTM cell switch command MAC CE to indicate whether the RA triggered by the MAC CE can only be performed on SBFD RO. (MAC spec impact listed here for information)

2.1.6: For SI request, add an RO type indication in SI-RequestConfig and SI-RequestConfigRepetition to indicate whether the corresponding RACH-ConfigGeneric contains valid useable SBFD RO.

Further comments:

Companies can provide comments, e.g. any concern on above issue(s)/solution(s), specifically any duplication/overlapping between above solutions.

| Company | Comments |
| --- | --- |
| CATT | RAN2 can discuss how to indicate the RO type for each CFRA scenario. Our views on each CFRA scenario are as below: - Error: Reference source not found - Error: Reference source not found - Error: Reference source not found - Error: Reference source not found - Error: Reference source not found - Error: Reference source not found |
| Apple | For 2.1.1, first as shown in our contribution, we don’t see the need to extend SI-request RACH resource to SBFD symbols. If RAN2 do make an agreement to support it, we feel the extension to SBFD symbol should not be SI message specific (as proposed in 2.1.1), e.g., SI#x on non-SBFD symbols, SI#y on SBFD symbols. For this kind of configuration, seems the assumption is certain SI (SI#y) is only needed for SBFD capable UE(s). But RAN2 never discussed this is the case. At least we don't think there would be SIB(s) only applicable to SBFD capable UE(s) but not to legacy UE(s). Instead, we think there should be a separate configuration in parallel to SI-RequestConfig (similar as SI-RequestConfigRepetition). For 2.1.2/2.1.3/2.1.4: Our understanding on the proposal (using HO case as example) is UE combines the configuration in RACH-ConfigDedicated and SBFD RACH-ConfigCommon for target cell (in HO CMD) together, to comprehend the configuration option (Option 1 or Option 2) since UE needs to know it is Option 1 or Option 2 configuration. We think it should be better captured in the field description, making it clear those relevant fields also apply to contention free random access (RACH-ConfigDedicated), and to contention-based beam failure recovery (CB-BFR). But we also understand this is stage 3 details. |

## 2.2 CBRA RO type selection based on NT signalling(s) including UE behaviour when the signalling is absent

Issue description:

It has been discussed that for CBRA case, the RACH RO type can be indicated by the network. The signalling design and the UE behaviour when the signalling on the RO type and RSRP threshold are absent has yet to be discussed/agreed.

Solutions:

According to [3] P5/P6, the following issues are open/can be discussed:

2.2.1: UE selects the first available RO type(legacy RO or additional RO) among all of the RACH resources valid to the UE for its CBRA based access, when the indication of RO type from the network is absent.

2.2.2: For SI request and RA-SDT, the UE always selects the first available RO type (legacy RO or additional RO) among all of the RACH resources valid to the UE in case of CBRA, when the indication of RO type from the network is absent.

Further comments:

Companies can provide comments, e.g. any concern on above issue(s)/solution(s).

| Company | Comments |
| --- | --- |
| CATT | In our understanding，if no RO type indication is provided by NW, the NW can configure how to use the RSRP threshold, e.g., if the RSRP> threshold, which type of RO can be used. |
| Apple | When NW indication is absent, we think it should be better to follow the common RSRP based condition principle. |

## 2.3 Issues related to RACH config options

Issue description:

There are two RACH configuration options. Their RRC impact can be discussed together with RAN1 higher layer parameters related.

Solutions:

According to [5] P3, 4 , [7] P2, [8] P2 and [16] P4, the following issues are open/can be discussed:

2.3.1: For Option 1, one bit enabler and SBFD specific power control parameters are provided into RACH-ConfigCommon. FFS on SBFD specific SSB-RO mapping configuration.

2.3.2:  For Option 2, SBFD specific RACH resources (e.g., rach-SBFD-ConfigCommon) are provided into BWP-UplinkCommon and AdditionalRACH-Config.

2.3.3: For SBFD RACH configuration option 2 in CBRA, add a SBFD RACH configuration parallel to RACH-ConfigCommon in BWP-UplinkCommon, and add the SBFD RACH configuration in each AdditionalRACH-Config.

Further comments:

Companies can provide comments, e.g. any concern on above issue(s)/solution(s).

| Company | Comments |
| --- | --- |
| CATT | We can agree with the above solutions with RAN1 parameter list. |
| Apple | Proponent to 2.3.1 and 2.3.2. 2.3.3 seems the same as 2.3.2. |

## 2.4 Applicability of "Random access procedure in SBFD symbols"

Issue description:

If RAN2 confirms the working assumption "Random access procedure in SBFD symbols is supported for all the existing RACH trigger events. ", or on the contrary, if RAN2 singles out SI case to be not supported with SBFD symbols, the RRC impact needs to be discussed.

Solutions:

According to [5] O1/O2 and other companies' proposal on this WA, the following issues are open/can be discussed:

2.4.1: Confirm the WA, RACH resource configurations for (Position) SI request is provided separately from RACH-ConfigCommon.

2.4.2:  Do not support Position SI request and SI request based RACH resources on SBFD symbols, RRC impact (?)

Further comments:

Companies can provide comments, e.g. any concern on above issue(s)/solution(s).

| Company | Comments |
| --- | --- |
| CATT | We agree with the solutions mentioned in 2.4.1. RACH on SBFD symbol is not only used to reduce latency but also to improve coverage. At least for the cell edge UE, performing Position SI request and SI request on SBFD symbol can increase the SI request successful probability. Hence, it is unnecessary to prohibit using RACH resource on SBFD symbols for Position SI request and SI request. |
| Apple | We are open to discuss. Our opinion is there is no need to extend SI requested based RACH due to the observations in our contribution. Observation 1: The size of RACH resources for (Position) SI request is dependent on the number of on-demand request SI messages, but not on the number of UE(s) requesting for on demand SI. Extending them to SBFD symbols increases the on-demand SI RACH resource overhead without any benefit. Observation 2: RACH resource configurations for (Position) SI request is provided separately from RACH-ConfigCommon, therefore extending them to SBFD symbols requires more efforts on RRC CR drafting. |

## 2.5 RSRP threshold for CBRA RO selection

Issue description:

In legacy, the RSRP threshold is configured by the NT on SSB/CSI-RS selection; for CBRA RO type selection, the RSRP threshold is configured also by NT with RRC signalling.

Solutions:

According to [3] P10, the following issues are open/can be discussed:

2.5.1: RSRP threshold for SSB/CSI-RS selection can be the same or different for legacy RACH resources and additional RACH resources

Further comments:

Companies can provide comments, e.g. any concern on above issue(s)/solution(s).

| Company | Comments |
| --- | --- |
| CATT | In our understanding, these two RSRP thresholds can be two separate thresholds. |
| Apple | We think this proposal is talking about where to contain the RSRP threshold for SSB/CSI selection. In our understanding, it should be carried in each SBFD RACH resource configuration. Combining the discussion in 2.3, it means in rach-SBFD-ConfigCommon in BWP-UplinkCommon and AdditionalRACH-Config. |

## 2.x Issue...

Issue description:

Solutions:

2.x.1:

Further comments:

Companies can provide comments, e.g. any concern on above issue(s)/solution(s).

| Company | Comments |
| --- | --- |

# 3. Open issues related to other impacts

## 3.1 	CLI measurement resource configuration, triggering, reporting

Issue description:

In order to support UE-to-UE and gNB-to-gNB CLI measurements, the CLI resource configuration for SRS-RSRP and CLI-RSSI measurements based on Rel-16 SRS-ResourceConfigCLI and rssi-ResourceConfigCLI shall be discussed and concluded with ASN.1 design. It is understood that RAN1 will provide related higher layer parameters for this and the related triggering, reporting aspects.

Solutions:

For measurement resource configuration, [19] P1/2/3, [20] P3, [28] P2/9, [29] P1:

3.1.1: CLI RS resource configuration with e.g. CSI-SBFD-IM-Resource-r19;  using legacy ZP-CSI-RS-Resource with the addition of SBFD CLI ZP-CSI-RS for UE-to-UE CLI measurement.

For CLI reporting, RAN1 has made agreements and RAN2 can design the signalling accordingly,  [28] P5, [29] P3/4:

3.1.2: For L1 based UE-to-UE CLI reporting configuration, the resourcesForChannelMeasurement is reused for CLI measurement with the associated ID of CLI resource, i.e., SRS-RSRP resource and CLI-RSSI resource.

For the CLI reporting triggering configuration with RRC (other than MAC) impact, the following design for aperiodic reporting can be done according to RAN1 agreement [28] P6:

3.1.3: Reuse CSI-AperiodicTriggerStateList to configure the trigger state associated with one or more aperiodic CSI-ReportConfig.

Further comments:

Companies can provide comments, e.g. any concern on above issue(s)/solution(s).

| Company | Comments |
| --- | --- |
| CATT | ‘SRS-RSRP and CLI-RSSI measurements’ in the Issue description would be better revised to ‘L1 SRS-RSRP and CLI-RSSI measurements’ which is different from legacy SRS-RSRP and CLI-RSSI measurements. |
| Apple | 3.1.1: We didn’t see RAN1 agreement on using ZP-CSI-RS-Resource. Should be up to RAN1 to decide. 3.1.2: OK. 3.1.3: OK |

## 3.2 Inter gNB signaling of measurement resource configuration  for CLI mitigation

Issue description:

It is understood that RAN3 (R3-250888) concluded that the IE design on SBFD time and frequency configuration information for the SBFD information exchange among gNBs is up to RAN2.

According to [25] P7, the resource configuration can be signalled as below.

Solutions: For SBFD gNB-to-gNB CLI measurement, to include the set of one or more periodic NZP CSI-RS resources configuration into inter-node message MeasurementTimingConfiguration.

Further comments:

Companies can provide comments, e.g. any concern on above issue(s)/solution(s).

| Company | Comments |
| --- | --- |
| CATT | Agree |
| Apple | In general OK. |

## 3.3 Valid symbol type for CSI-RS based RLM/BFD/CBD measurements

Issue description:

[27] P2 proposed that the network configures the valid symbol type for CSI-RS based RLM/BFD/CBD measurement.  It can be discussed together with the related RAN1 higher layer parameters as this configuration is within RAN1 domain. [29] P7/8 discussed also on this issue.

Solutions:

3.3.1: The field indicating the valid symbol type is included in IE RadioLinkMonitoringConfig and BeamFailureRecoveryConfig.

3.3.2:    RAN2 consider to support the separate Layer3 measurement report for CSI-RS resources in SBFD symbol.

Further comments:

Companies can provide comments, e.g. any concern on above issue(s)/solution(s).

| Company | Comments |
| --- | --- |
| CATT | For option 3.3.1, RAN1 has concluded that Configuration 1 and Configuration 2 are not applicable to CSI-RS resources configuration and CSI-RS based RLM/BFD/CBD measurement. So valid symbol type only is configured for L1-RSRP/SINR. RAN1’s LS in R1-2501560 does not support option 3.3.1. Please find part of the LS as below: Issue#2: Behaviour for CSI-RS based L1-RSRP/SINR and RLM/BFD/CBD measurements For resource configuration and/or reporting configuration for CSI-RS based L1-RSRP/SINR measurement and RLM/BFD/CBD measurement, RAN1 would like to provide the following response: - According to the following RAN1 agreements, Configuration 1 and Configuration 2 are not applicable to CSI-RS resources configuration and CSI-RS based RLM/BFD/CBD measurement. - The following RAN1 agreements for CSI reporting are applicable to CSI-RS based L1-RSRP/SINR measurement/reporting. For option 3.3.2, separate CSI reports on SBFD and non-SBFD are for interference management from RAN1’s perspective. The value of L3 separate measurement seems limited and not in the WI scope… |
| Apple | 3.3.1: should this up to RAN1/RAN4 to decide? 3.3.2: We also feel RAN1/RAN4 decision is needed on this. For example, RAN4 already agreed for DU configuration: UE measurement requirements would be same as in legacy with the condition that all CSI-RS resources configured for measurement are available in DL subband of SBFD symbols and/or on non-SBFD DL symbols. FFS for DUD configuration. |

## 3.x Issue...

Issue description:

Solutions:

3.x.1:

Further comments:

Companies can provide comments, e.g. any concern on above issue(s)/solution(s).

| Company | Comments |
| --- | --- |
| CATT | SBFD subband configuration also has RRC impact. |

# Conclusion:

List of open issues of RRC impact:

# Reference

- R2-2500274	Discussion on Random Access in SBFD symbols	 CATT

- R2-2500884	SBFD RA aspects 	Ericsson

- R2-2500298	Random Access for SBFD Operation	NEC

- R2-2500339	Discussion on Random Access operation in SBFD	InterDigital, Inc.

- R2-2500588	Remaining issues for RACH in SBFD	Apple

- R2-2500298	Random Access for SBFD Operation	NEC

- R2-2500606	Discussion on random access procedure in SBFD	 ZTE Corporation

- R2-2501244	Discussion on Random Access in SBFD	LG Electronics Inc.

- R2-2500098	Impacts on the random access by the evolution of duplex operation		Huawei, HiSilicon

- R2-2500105	Random Access Operation of SBFD	Nokia Corporation

- R2-2500109	SBFD Configuration for initial random access and operations	Charter Communications, Inc

- R2-2500191	Discussion on RACH in SBFD	Xiaomi

- R2-2500337	Discussion on random access procedure in SBFD	vivo

- R2-2500376	Random Access Issues for SBFD	Sharp

- R2-2500751	Random access for SBFD Operation	Sony

- R2-2501029	Discussion on random access in SBFD	CMCC

- R2-2501129	Views on random access for SBFD	Qualcomm Incorporated

- R2-2501135	Random access in SBFD	Samsung

- R2-2500110	Cross link interference handling in SBFD networks	Charter Communications, Inc

- R2-2500275	Discussion on other aspects of SBFD	CATT

- R2-2500338	SBFD other aspects	vivo

- R2-2500340	Discussion on resource configuration in SBFD	InterDigital, Inc.

- R2-2500403	Other aspects of SBFD	Xiaomi

- R2-2500554	Other aspects of SBFD	Nokia

- R2-2500607	Discussion on L3 and L1 measurement in SBFD	ZTE Corporation

- R2-2500637	Other impacts by the evolution of duplex operation	Huawei, HiSilicon

- R2-2500885	CSI-RS measurements for RLM/BFD/CBD in SBFD	Ericsson

- R2-2501130	Other aspects of SBFD	Qualcomm Incorporated

- R2-2501170	Other Aspects of SBFD	Samsung