3GPP TSG-RAN WG2 Meeting #129-bis	R2-250xxxx

Wuhan, China, 7th – 11th April, 2025

Agenda item:	8.11.1

Source: 	Huawei, HiSilicon (Rapporteur)

Title: 	Summary of [Post129][217][SBFD] List of open issues of RRC impact

Document for:	Discussion and decision

# Introduction

This is the summary of the following email discussion:

- [Post129][217][SBFD] List of open issues of RRC impact (Huawei)

Intended outcome: List of open issues of RRC impact to submit to the next meeting, taking into account related proposals in this meeting

Deadline:  Long

The target of this email discussion is to identify the open issues of RRC impact, to assist the discussion of related issues and the correspondent implementation in RRC spec if possible. Companies are encouraged to raise the open issues of RRC impact, preferably most relevant to the completion of this WI, and to propose solution options if applicable. Please consider RAN1 latest progress of RRC impact e.g. higher layer parameter list R1-2501645.

# Open issues related to Random access in SBFD

## CFRA RO type selection related signaling

Issue description:

It is agreed that for CFRA case, the RACH RO type is indicated by the network. The relate signalling design has yet to be discussed/agreed.

Solutions:

According to [1] P5/6/7, [3] P1 , [7] P8/9 and [16] P13, the following issues are open/can be discussed:

2.1.1: For CFRA triggered by Msg1-based SI request, the RO type is indicated in SI-RequestResources of SIB1.

2.1.2: For CFRA triggered by BFR, the RO type is indicated in BeamFailureRecoveryConfig.

2.1.3: For CFRA triggered by ReconfigurationwithSync, the RO type is indicated in RACH-ConfigDedicated.

2.1.4: Within the corresponding RACH configuration for the additional ROs, one bit is used to indicate the RO type for CFRA.

2.1.5: For RO type indication in CFRA, the following RRC and MAC spec impact should be considered:

- Add an RO type indication in BeamFailureRecoveryConfig to indicate whether the corresponding RACH-ConfigGeneric only contains valid SBFD RO;

- Add an RO type indication in rach-ConfigDedicated to indicate whether the corresponding RACH-ConfigGeneric only contains valid SBFD RO;

- Add an RO type indication in EarlyUL-SyncConfig to indicate whether the corresponding RACH-ConfigGeneric only contains valid SBFD RO;

- Add an RO type indication in LTM cell switch command MAC CE to indicate whether the RA triggered by the MAC CE can only be performed on SBFD RO. (MAC spec impact listed here for information)

2.1.6: For SI request, add an RO type indication in SI-RequestConfig and SI-RequestConfigRepetition to indicate whether the corresponding RACH-ConfigGeneric contains valid useable SBFD RO.

Further comments:

Companies can provide comments, e.g. any concern on above issue(s)/solution(s), specifically any duplication/overlapping between above solutions.

| Company | Comments |
| --- | --- |
| CATT | RAN2 can discuss how to indicate the RO type for each CFRA scenario. Our views on each CFRA scenario are as below: - Error: Reference source not found - Error: Reference source not found - Error: Reference source not found - Error: Reference source not found - Error: Reference source not found - Error: Reference source not found |
| Apple | For 2.1.1, first as shown in our contribution, we don’t see the need to extend SI-request RACH resource to SBFD symbols. If RAN2 do make an agreement to support it, we feel the extension to SBFD symbol should not be SI message specific (as proposed in 2.1.1), e.g., SI#x on non-SBFD symbols, SI#y on SBFD symbols. For this kind of configuration, seems the assumption is certain SI (SI#y) is only needed for SBFD capable UE(s). But RAN2 never discussed this is the case. At least we don't think there would be SIB(s) only applicable to SBFD capable UE(s) but not to legacy UE(s). Instead, we think there should be a separate configuration in parallel to SI-RequestConfig (similar as SI-RequestConfigRepetition). For 2.1.2/2.1.3/2.1.4: Our understanding on the proposal (using HO case as example) is UE combines the configuration in RACH-ConfigDedicated and SBFD RACH-ConfigCommon for target cell (in HO CMD) together, to comprehend the configuration option (Option 1 or Option 2) since UE needs to know it is Option 1 or Option 2 configuration. We think it should be better captured in the field description, making it clear those relevant fields also apply to contention free random access (RACH-ConfigDedicated), and to contention-based beam failure recovery (CB-BFR). But we also understand this is stage 3 details. |
| LGE | For solution 2.1.1 and 2.1.6 First, it should be clarified that RA procedure initiated by Msg1-based SI request is not CFRA nor CBRA from MAC perspective, since common RACH resource is used for all UEs in the cell. If the Msg1-based SI request with SBFD RO is supported, it could be simple to indicate whether to use SBFD RO or legacy RO. However, it could be risky to dictate all SBFD-aware UEs to use the same type of RO regardless of the channel condition (e.g., whether the UE is in the cell-edge area or in cell-center area), considering CLI impacts based on channel condition of each UE. Note that it is different from other CFRA cases, since the network may indicate appropriate RO type for each UE. On the other hand, if the UE selects between SBFD RO and legacy RO based on its channel condition (similar to CBRA procedure), it would cause a lot of workload, in order to determine whether the Msg1-based SI request for RedCap/PosSI/Msg1 repetition can be applied with SBFD RO. Given that the benefits (latency, coverage) of using SBFD RO in Msg1-based SI request is not essential or it could be already handled by other feature (e.g., Msg1 repetition), additional discussion to support Msg1-based SI request is not preferred. For solution 2.1.5 If early sync procedure is supported using SBFD RO, it should be clarified as: - Add an RO type indication in EarlyUL-SyncConfig to indicate whether the corresponding RACH-ConfigGeneric only contains valid SBFD RO from network side; (similar to 1-bit indication to indicate whether RACH configuration Option 1 for SBFD random access operation is enabled or not for CBRA cases) Then, given that the early sync procedure is initiated by PDCCH order for an LTM candidate cell, the used RO type may be indicated by the PDCCH order. OK to discuss on other issues. |
| OPPO | We are open to discuss whether to have one bit indication for all CFRA cases or separate bits for different CFRA configurations. However, we should probably firstly decide which CFRA cases (e.g. SI request and early RACH) can support the additional RO. |
| Samsung | For Msg1-based SI requests (2.1.1, 2.1.6), we concur with Apple and LGE that there appears to be no advantage in incorporating SBFD. We have no objections to 2.1.2 and 2.1.3. Regarding 2.1.5, this matter relates to 'Mobility WI' and their MAC-CE design; therefore, consultation with them may be necessary. |
| Xiaomi | For 2.1.1, we agree with Apple / LGE / Samsung that it is not necessary to support Msg1 SI request resource on SBFD symbols since it is not essential to minimize the latency for legacy on-demand SI request. Note that coexistence of SBFD and Rel-19 NES on-demand SIB1 is a separate discussion. We are open to discuss other open issue listed above. |
| Ericsson | For 2.1.6, RAN2 has not made agreement, it is FFS. For UL early sync, RAN2 may need to check with RAN1 if DCI will include the RO type indicator, if it is the case, RAN2 may no need to include RO type in RRC. |
| ZTE | For SI request: We agree with LG that introducing RO type selection for SI request will cause lots of MAC spec impact. To minimum the spec effort, it is ok for ZTE to not support SBFD RO for SI request. For early sync: We also agree with LG and Ericsson that the RO type indication for early sync should at least be ‘whether there exists valid SBFD RO’, rather than ‘should mandatory use SBFD RO’ . We are also ok to only include this RO type indication in PDCCH order. For other CFRA cases: Solution 2.1.5 is ok. |
| Qualcomm | We basically support all CFRA case to have one-bit indication for the RO type. For SI request, we are fine with 2.1.6. |
| Sharp | We also think Msg1 SI request resource on SBFD symbols is not necessary. |
| Nokia | We are ok with 2.1.1 – 2.1.4 For 2.1.5, we don’t think the constrain ‘only’ is needed. Our understanding is that the RO type indication does not imply that the corresponding RACH configuration consists only of SBFD ROs. Instead, it serves as an indication for UEs to use a specific RO type (SBFD RO). The UE validates only the indicated RO type. |
| CMCC | We agree with CATT that RAN2 can discuss how to indicate the RO type for each CFRA event. For Position SI request and SI request, we think that it is technically feasible to support SBFD operation, but we are okay if SBFD operation is not supported for SI request (2.1.1, 2.1.6) to reduce MAC spec impact. For UL early sync, we agree with Ericsson that RAN2 may need to check with RAN1 whether the RO type indication will be included in PDCCH order. |

## 2.2 CBRA RO type selection based on NT signalling(s) including UE behaviour when the signalling is absent

Issue description:

It has been discussed that for CBRA case, the RACH RO type can be indicated by the network. The signalling design and the UE behaviour when the signalling on the RO type and RSRP threshold are absent has yet to be discussed/agreed.

Solutions:

According to [3] P5/P6, the following issues are open/can be discussed:

2.2.1: UE selects the first available RO type(legacy RO or additional RO) among all of the RACH resources valid to the UE for its CBRA based access, when the indication of RO type from the network is absent.

2.2.2: For SI request and RA-SDT, the UE always selects the first available RO type (legacy RO or additional RO) among all of the RACH resources valid to the UE in case of CBRA, when the indication of RO type from the network is absent.

Further comments:

Companies can provide comments, e.g. any concern on above issue(s)/solution(s).

| Company | Comments |
| --- | --- |
| CATT | In our understanding，if no RO type indication is provided by NW, the NW can configure how to use the RSRP threshold, e.g., if the RSRP> threshold, which type of RO can be used. |
| Apple | When NW indication is absent, we think it should be better to follow the common RSRP based condition principle. |
| LGE | For all solutions, it is not needed to further discuss. In RAN2#129 meeting, it is agreed to select the RO type based on RSRP threshold: - If no RO type indication is provided by the NW, a UE selects RO type based on a SSB RSRP threshold. FFS whether NW can further indicate whether to select the additional RO type below or above this SSB RSRP threshold. [LGE2] Regarding the RSRP threshold can be absent and the corresponding procedure, following options can only be acceptable from our side: - Option 1: RSRP threshold is mandatorily configured, if the legacy RO and additional RO are configured in the cell (similar to 2-step RA) - Option 2: if the RSRP threshold is not configured, the UE prioritize the additional RO, regardless of the channel condition Slightly prefer Option 2, but do not have strong view between these two options. |
| OPPO | Agree with CATT and LGE that we should follow the RAN2 agreement made in RAN2#129. |
| Samsung | Disagree with LGE and Oppo. This is not about previous agreements but about UE behavior when both the RO type indication and RSRP threshold are absent. We should discuss UE behavior in this case unless one of them is mandatory (which could be one option). We could consider the following three options: - UE selects the first available RO type. - Up to UE implementation. - UE always selects the legacy (or SBFD) RO type. For now, we support option 1. |
| Xiaomi | Agree with LGE. |
| Sony | We agree with Samsung, the issue is the UE behaviour when the signalling on the RO type and RSRP threshold are absent has yet to be discussed/agreed.  UE selects the first available RO type is reasonable. |
| Ericsson | If NW doesn’t provide any indication, it may mean that the network doesn’t see there is congestion for RACH. In this case, latency may be not an issue. So, it may be sufficient to leave to UE implementation to select RO type. |
| ZTE | -> NW indication is first priority, if absent: -> RSRP is the second priority, if absent: -> SBFD UE use the SBFD RO as a default solution. The benefit of using SBFD RO as a default solution is, SBFD UE can achieve early identification to gNB by choosing SBFD RO, to facilitate the further scheduling and reduce the whole RA latency. |
| Qualcomm | If the network signalling on the RO type selection and RSRP threshold configuration are absent, the SBFD-aware UE should prioritize the additional RO over the legacy RO. Always selecting the first available RO type may make SBFD-aware UE lose the opportunity to enjoy the benefits of using additional RO. |
| Sharp | RAN2 already agreed RO type indication and RSRP-based RO type selection. We see that those two options are sufficient. The case that either indication or threshold should be mandatory. |
| Nokia | We are ok with 2.2.1 and 2.2.2, with the additional condition indicating that UE selects the first available RO type (legacy RO or additional RO) among all of the RACH resources valid to the UE for its CBRA based access, when the indication of RO type or the RSRP values for RO type selection from the network is absent. |
| CMCC | We prefer the scheme that the RSRP threshold is mandatorily configured, if the both legacy RO and additional RO are configured in one cell. To this end, if no RO type is explicitly indicated by network, a UE selects the RO type based on the DL RSRP threshold as RAN2 agreed. However, if the mentioned scheme can not be agreed, the behavior of SBFD-aware UE depends on its implementation in case that both explicit indication and RSRP threshold are absent. |

## 2.3 Issues related to RACH config options

Issue description:

There are two RACH configuration options. Their RRC impact can be discussed together with RAN1 higher layer parameters related.

Solutions:

According to [5] P3, 4 , [7] P2, [8] P2 and [16] P4, the following issues are open/can be discussed:

2.3.1: For Option 1, one bit enabler and SBFD specific power control parameters are provided into RACH-ConfigCommon. FFS on SBFD specific SSB-RO mapping configuration.

2.3.2:  For Option 2, SBFD specific RACH resources (e.g., rach-SBFD-ConfigCommon) are provided into BWP-UplinkCommon and AdditionalRACH-Config.

2.3.3: For SBFD RACH configuration option 2 in CBRA, add a SBFD RACH configuration parallel to RACH-ConfigCommon in BWP-UplinkCommon, and add the SBFD RACH configuration in each AdditionalRACH-Config.

Further comments:

Companies can provide comments, e.g. any concern on above issue(s)/solution(s).

| Company | Comments |
| --- | --- |
| CATT | We can agree with the above solutions with RAN1 parameter list. |
| Apple | Proponent to 2.3.1 and 2.3.2. 2.3.3 seems the same as 2.3.2. |
| LGE | In our understanding, LS from RAN1 on higher layer parameter list is agreed in R1-2501644/ R1-2501645. This can be used to implement RRC parameters on SBFD RA procedure. Meanwhile, we are open to discuss on solution 2.3.2 and 2.3.4., i.e., additional the SBFD RACH configuration in each AdditionalRACH-Config, for RACH configuration Option 2. |
| OPPO | Agree with 2.3.1. For 2.3.2 and 2.3.3, the functional differences are unclear. If there is no functional difference, we can leave it to the running CR rapporteur. |
| Samsung | We also agree with 2.3.1. In general we are ok with 2.3.2 and 2.3.3. (not sure the difference) |
| Xiaomi | 2.3.1 can be discussed. For others, it can be handled when implementing RRC running CR. |
| Ericsson | The suggested solutions are fine. [Eri2] we agree with other companies that 2.3.3 seems the same as 2.3.2. |
| ZTE | For 2.3.1, suggest to wait for RAN1 parameter list Difference between 2.3.1 and 2.3.2 is signalling structure, whether to include option 2 into BWP-UplinkCommon (2.3.2) or parallel to BWP-UplinkCommon (2.3.3). 2.3.3 is preferred since it is similar design with 2-step RACH configuration. |
| Qualcomm | We are fine with 2.3.2 and 2.3.3. We can check the RAN1 parameter list for 2.3.1. |
| Sharp | Agree with other companies that 2.3.1 ad 2.3.2 are ok. |
| Nokia | OK with 2.3.1, 2.3.2, and 2.3.3 |
| CMCC | Agree with 2.3.1. In terms of 2.3.2 and 2.3.3, we are not sure about the difference as some companies. |

## 2.4 Applicability of "Random access procedure in SBFD symbols"

Issue description:

If RAN2 confirms the working assumption "Random access procedure in SBFD symbols is supported for all the existing RACH trigger events. ", or on the contrary, if RAN2 singles out SI case to be not supported with SBFD symbols, the RRC impact needs to be discussed.

Solutions:

According to [5] O1/O2 and other companies' proposal on this WA, the following issues are open/can be discussed:

2.4.1: Confirm the WA, RACH resource configurations for (Position) SI request is provided separately from RACH-ConfigCommon.

2.4.2:  Do not support Position SI request and SI request based RACH resources on SBFD symbols, RRC impact (?)

Further comments:

Companies can provide comments, e.g. any concern on above issue(s)/solution(s).

| Company | Comments |
| --- | --- |
| CATT | We agree with the solutions mentioned in 2.4.1. RACH on SBFD symbol is not only used to reduce latency but also to improve coverage. At least for the cell edge UE, performing Position SI request and SI request on SBFD symbol can increase the SI request successful probability. Hence, it is unnecessary to prohibit using RACH resource on SBFD symbols for Position SI request and SI request. |
| Apple | We are open to discuss. Our opinion is there is no need to extend SI requested based RACH due to the observations in our contribution. Observation 1: The size of RACH resources for (Position) SI request is dependent on the number of on-demand request SI messages, but not on the number of UE(s) requesting for on demand SI. Extending them to SBFD symbols increases the on-demand SI RACH resource overhead without any benefit. Observation 2: RACH resource configurations for (Position) SI request is provided separately from RACH-ConfigCommon, therefore extending them to SBFD symbols requires more efforts on RRC CR drafting. |
| LGE | It is related to issue 2.1 and we prefer to not support Msg1-based SI request using SBFD RO. See our comments on issue 2.1. |
| OPPO | We are open to discuss whether to support additional RO for SI request. If there is no specific technical blockage, we should probably allow additional RO for SI request, so that more RO resources can be used and split between SI request and other use cases. |
| Samsung | It is related to issue 2.1 and we prefer to not support Msg1-based SI request using SBFD RO. See our comments on issue 2.1. |
| Xiaomi | We support 2.4.2. See our comments on issue 2.1. |
| Ericsson | It is perhaps easier to have a common SBFD RACH framework for all RACH trigger events. although there will be additional specification efforts for (position) SI request, the changes will be similar/same as other RACH trigger events. it is manageable to do it for RAN2. |
| ZTE | We support 2.4.2. See our comments on issue 2.1. |
| Qualcomm | We are fine with 2.4.1. We observe the benefits to have additional ROs for SI request. |
| Sharp | We are ok with 2.4.1. |
| Nokia | We support 2.4.1. We see advantages to support Position SI request and SI request-based RACH resources on SBFD symbols |
| CMCC | We are open. See our comment on issue 2.1. |

## 2.5 RSRP threshold for CBRA RO selection

Issue description:

In legacy, the RSRP threshold is configured by the NT on SSB/CSI-RS selection; for CBRA RO type selection, the RSRP threshold is configured also by NT with RRC signalling.

Solutions:

According to [3] P10, the following issues are open/can be discussed:

2.5.1: RSRP threshold for SSB/CSI-RS selection can be the same or different for legacy RACH resources and additional RACH resources

Further comments:

Companies can provide comments, e.g. any concern on above issue(s)/solution(s).

| Company | Comments |
| --- | --- |
| CATT | In our understanding, these two RSRP thresholds can be two separate thresholds. |
| Apple | We think this proposal is talking about where to contain the RSRP threshold for SSB/CSI selection. In our understanding, it should be carried in each SBFD RACH resource configuration. Combining the discussion in 2.3, it means in rach-SBFD-ConfigCommon in BWP-UplinkCommon and AdditionalRACH-Config. |
| LGE | Similar to Apple, we also understand that this proposal is RSRP to select SSB/CSI-RS, not RSRP TH for selection between additional RO and legacy RO. We failed to find any needs to configure separated RSRP threshold for SSB/CSI-RS selection for RACH configuration Option 1. For RACH configuration Option 2, it naturally supports the separated RSRP threshold to select SSB/CSI-RS. |
| OPPO | Since the transmission coverage between additional RO and legacy RO is expected to be different, a separate RSRP threshold for SSB-selection is needed. |
| Samsung | Seems more of the RAN1 issue and have no strong opinion. |
| Xiaomi | Agree with LGE. |
| Ericsson | the issue/question is only relevant for RACH configuration option 2. In this case, it is up to the network decision/implementation to set the same or different RSRP threshold between legacy RACH configuration and additional RACH configuration. For RACH configuration option 1, there is no separate RSRP threshold for legacy RACH resource and additional RACH resource. |
| ZTE | Agree with Ericsson |
| Qualcomm | For RACH configuration option 2, the RSRP thresholds for legacy RACH resource and additional RACH resources are configured in different parameters and the value could be different. |
| Sharp | We can leave it up to NW configuration. |
| Nokia | Our view is that baseline should be to use the RSRP threshold configured in the corresponding RACH configuration for SSB/CSI-RS selection. Thus, for option 1 RACH configuration, same RSRP threshold is used for SSB/CSI-RS selection for both SBFD and legacy ROs. For option 2 RACH configuration, RSRP threshold configured in the SBFD RACH configuration can be used for SSB/CSI-RS selection. Our preference is as follows: Since the network will support either Option 1 or Option 2 RACH configuration at a time, the approach depends on the selected configuration. - If the network supports Option 1 RACH configuration, a separate RSRP threshold can be configured for SBFD RO to allow additional RO selection when the measured RSRP exceeds the configured ‘new’ RSRP threshold. - If the network supports Option 2 RACH configuration, the ‘new’ RSRP threshold can be used to select additional RO when the measured RSRP is below the configured threshold. |
| CMCC | Agree with Ericsson. |

## 2.x Issue...

Issue description:

Solutions:

2.x.1:

Further comments:

Companies can provide comments, e.g. any concern on above issue(s)/solution(s).

| Company | Comments |
| --- | --- |
| Xiaomi | - RA-RNTI collision issue should be discussed for RACH configuration 2, e.g., RO in flexible symbol. - The co-existence of SBFD RACH and R19 NES RACH activation should be discussed. |
| Sony | - RA-RNTI collision issue should be discussed for RACH configuration 2, e.g., RO in flexible symbol. Same view as Xiaomi. |
| Nokia | We share the same view as Xiaomi and Sony on RA-RNTI collision |

# 3. Open issues related to other impacts

## 3.1 	CLI measurement resource configuration, triggering, reporting

Issue description:

In order to support UE-to-UE and gNB-to-gNB CLI measurements, the CLI resource configuration for SRS-RSRP and CLI-RSSI measurements based on Rel-16 SRS-ResourceConfigCLI and rssi-ResourceConfigCLI shall be discussed and concluded with ASN.1 design. It is understood that RAN1 will provide related higher layer parameters for this and the related triggering, reporting aspects.

Solutions:

For measurement resource configuration, [19] P1/2/3, [20] P3, [28] P2/9, [29] P1:

3.1.1: CLI RS resource configuration with e.g. CSI-SBFD-IM-Resource-r19;  using legacy ZP-CSI-RS-Resource with the addition of SBFD CLI ZP-CSI-RS for UE-to-UE CLI measurement.

For CLI reporting, RAN1 has made agreements and RAN2 can design the signalling accordingly,  [28] P5, [29] P3/4:

3.1.2: For L1 based UE-to-UE CLI reporting configuration, the resourcesForChannelMeasurement is reused for CLI measurement with the associated ID of CLI resource, i.e., SRS-RSRP resource and CLI-RSSI resource.

For the CLI reporting triggering configuration with RRC (other than MAC) impact, the following design for aperiodic reporting can be done according to RAN1 agreement [28] P6:

3.1.3: Reuse CSI-AperiodicTriggerStateList to configure the trigger state associated with one or more aperiodic CSI-ReportConfig.

Further comments:

Companies can provide comments, e.g. any concern on above issue(s)/solution(s).

| Company | Comments |
| --- | --- |
| CATT | ‘SRS-RSRP and CLI-RSSI measurements’ in the Issue description would be better revised to ‘L1 SRS-RSRP and CLI-RSSI measurements’ which is different from legacy SRS-RSRP and CLI-RSSI measurements. |
| Apple | 3.1.1: We didn’t see RAN1 agreement on using ZP-CSI-RS-Resource. Should be up to RAN1 to decide. 3.1.2: OK. 3.1.3: OK |
| LGE | In our understanding, RAN1 parameter list is already agreed on R1-2501644/ R1-2501645. This can be used to implement RRC parameters on CLI handling. |
| OPPO | We prefer to follow the signalling structure of the parameter list provided by RAN1, unless some critical issues are found. |
| Samsung | 3.1.1: Based on RAN1 agreements, two CLI measurement resource set lists for L1 SRS-RSRP and L1 CLI-RSSI could be configured in CSI-ResourceConfig but we don’t see the ZP-CSI-RS-Resource is the correct place for this configuration. Note that ZP-CSI-RS-Resource is only used for rate-matching, so the new field could be used for this CLI measurements. As we provided in Annex - TP [29], two CLI measurement resource set lists for L1 SRS-RSRP and L1 CLI-RSSI could be configured directly under the CSI-ResourceConfig using the non-critical extension. 3.1.2: OK. Details could be further discussed e.g. how to handle the legacy mandatory fields in this IE. 3.1.3: OK. Details could be further discussed based on RAN1 agreements and following could be considered to be added. - Introduce the new resourceForChannel-r19 for SBFD CLI resource association. - Add the text to ignore the legacy mandatory field of resourceForChannel. |
| Xiaomi | We also prefer to follow RRC parameter list. |
| Ericsson | The suggested solutions are fine. We also prefer to follow RAN1 parameter list as a starting point. |
| ZTE | There is no RAN2-originated issue for CLI configuration and trigger. RAN2 should wait for RAN1’s parameter list |
| Qualcomm | 3.1.1 is not aligned with RAN1 parameter list and can not be agreed. We are the proponent of 3.1.2 and 3.1.3. |
| Sharp | We also prefer to follow parameter list provided by RAN1. |
| Nokia | 3.1.1 We don’t support this proposal. RAN1 has already agreed on defining new L1-RSR-RSRP and L1-CLI-RSSI resources. RAN1#118-bis agreement: Agreement Extend CSI-ResourceConfig to include two CLI measurement resource set lists for SRS-RSRP and CLI-RSSI measurements based on Rel-16 SRS-ResourceConfigCLI and rssi-ResourceConfigCLI defined in MeasObjectCLI for L3 based SRS-RSRP and CLI-RSSI measurement. 3.1.2 Support, this is in line with the RAN1#118-bis agreement: Agreement In CSI-ReportConfig, reuse resourcesForChannelMeasurement providing the ID of a CSI-ResourceConfig that contains configuration of measurement resources for L1-SRS-RSRP and/or L1-CLI-RSSI. 3.1.3 Support, this is in line with the RAN1#118-bis agreement: Agreement For aperiodic CLI reporting on PUSCH - Reuse CSI-AperiodicTriggerStateList to configure the trigger state associated with one or more aperiodic CSI-ReportConfig - Reuse the existing DCI field 'CSI request' to trigger an aperiodic CLI report - Introduce an indication in CSI-AssociatedReportConfigInfo that selects a set from the list of CLI-RSSI measurement resource sets or SRS-RSRP measurement resource sets configured in CSI-ResourceConfig |
| CMCC | 3.1.1: We share same view with Samsung. RAN1 has agreed to extend CSI-ResourceConfig to include two CLI measurement resource set lists for L1 SRS-RSRP and L1 CLI-RSSI. RAN1 has never considered using ZP-CSI-RS-Resource for UE-to-UE CLI measurement. 3.1.2: OK. It is aligned with RAN1 agreements. 3.1.3: OK. It is aligned with RAN1 agreements. |

## 3.2 Inter gNB signaling of measurement resource configuration  for CLI mitigation

Issue description:

It is understood that RAN3 (R3-250888) concluded that the IE design on SBFD time and frequency configuration information for the SBFD information exchange among gNBs is up to RAN2.

According to [25] P7, the resource configuration can be signalled as below.

Solutions: For SBFD gNB-to-gNB CLI measurement, to include the set of one or more periodic NZP CSI-RS resources configuration into inter-node message MeasurementTimingConfiguration.

Further comments:

Companies can provide comments, e.g. any concern on above issue(s)/solution(s).

| Company | Comments |
| --- | --- |
| CATT | Agree |
| Apple | In general OK. |
| LGE | First “IE design on SBFD time and frequency configuration information” mentioned in RAN3 LS should be separately discussed with “periodic NZP CSI-RS resources configuration.” These two aspects are discussed in RAN1/RAN3 separately with different purpose, i.e., - SBFD information exchange among gNBs is for Time/frequency coordination for gNB-to-gNB CLI handling; while - SSB and/or periodic NZP CSI-RS resources configuration exchange among gNBs is for spatial coordination. For IE design on SBFD time configuration information, RAN1 parameter list in R1-2501645 can be used as a baseline. For SBFD frequency configuration information, we can agree on the basic principle, i.e., one RIV is indicated for SBFD UL subband and one or two RIVs are indicated for SBFD DL subband, based on following RAN1#119 agreement: Agreement Confirm the following working assumption made in RAN1#118. Working Assumption For cell-specific configuration of frequency locations of SBFD UL subband, for each SCS configuration in SCS-SpecificCarrierList for UL, starting RB and bandwidth of SBFD UL subband are indicated by a RIV-based indication as defined in 38.214 setting =275. For cell-specific configuration of frequency locations of SBFD DL subband(s), for each SCS configuration in SCS-SpecificCarrierList for DL, starting RB and bandwidth of each SBFD DL subband are indicated by a RIV-based indication as defined in 38.214 setting =275. - One or two SBFD DL subbands can be configured. For periodic NZP CSI-RS resources configuration, in our understanding, RAN3 is already discussing between following two options: - ption 1: Define IE to convey NZP-CSI-RS-ResourceSet in Xn message - Option 2: Define NZP-CSI-RS-ResourceSet in inter-node message MeasurementTimingConfiguration In this sense, RAN2 may wait for the RAN3 discussion. |
| OPPO | This is fine for us. We can revisit this kind of measurement resource configuration when we receive more inputs from RAN1 or RAN3. |
| Samsung | In general OK. |
| Xiaomi | We are OK with the proposed solution. [Xiaomi2] After further check RAN3 discussion, we tend to agree with LGE. In RAN3 discussion (e.g. the offline summary R3-250803 and RAN3 Chair notes), SBFD configuration and measurement configuration (including NZP CSI-RS configuration) are separate issues. RAN3 LS is only about SBFD configuration. RAN3 is still discussing measurement configuration. Therefore RAN2 can wait for RAN3 discussion regarding NZP CSI-RS configuration. |
| Ericsson | The suggested solutions are fine. |
| ZTE | Ok with the proposed solution |
| Qualcomm | RAN2 should wait for RAN3 discussion and decision. There are already some ongoing discussions on different options in RAN3 as LGE mentioned. |
| Sharp | We can wait for RAN3 progress, as long as RAN3 is discussing the same issue. |
| Nokia | Support. It is natural to extend the content of the MeasurementTimingConfiguration IE with the NZP-CSI-RS configuration for gNB-to-gNB CLI measurements. The IE already includes CSI-RS for cell mobility. |
| CMCC | We are OK with the solution. |

## 3.3 Valid symbol type for CSI-RS based RLM/BFD/CBD measurements

Issue description:

[27] P2 proposed that the network configures the valid symbol type for CSI-RS based RLM/BFD/CBD measurement.  It can be discussed together with the related RAN1 higher layer parameters as this configuration is within RAN1 domain. [29] P7/8 discussed also on this issue.

Solutions:

3.3.1: The field indicating the valid symbol type is included in IE RadioLinkMonitoringConfig and BeamFailureRecoveryConfig.

3.3.2:    RAN2 consider to support the separate Layer3 measurement report for CSI-RS resources in SBFD symbol.

Further comments:

Companies can provide comments, e.g. any concern on above issue(s)/solution(s).

| Company | Comments |
| --- | --- |
| CATT | For option 3.3.1, RAN1 has concluded that Configuration 1 and Configuration 2 are not applicable to CSI-RS resources configuration and CSI-RS based RLM/BFD/CBD measurement. So valid symbol type only is configured for L1-RSRP/SINR. RAN1’s LS in R1-2501560 does not support option 3.3.1. Please find part of the LS as below: Issue#2: Behaviour for CSI-RS based L1-RSRP/SINR and RLM/BFD/CBD measurements For resource configuration and/or reporting configuration for CSI-RS based L1-RSRP/SINR measurement and RLM/BFD/CBD measurement, RAN1 would like to provide the following response: - According to the following RAN1 agreements, Configuration 1 and Configuration 2 are not applicable to CSI-RS resources configuration and CSI-RS based RLM/BFD/CBD measurement. - The following RAN1 agreements for CSI reporting are applicable to CSI-RS based L1-RSRP/SINR measurement/reporting. For option 3.3.2, separate CSI reports on SBFD and non-SBFD are for interference management from RAN1’s perspective. The value of L3 separate measurement seems limited and not in the WI scope… |
| Apple | 3.3.1: should this up to RAN1/RAN4 to decide? 3.3.2: We also feel RAN1/RAN4 decision is needed on this. For example, RAN4 already agreed for DU configuration: UE measurement requirements would be same as in legacy with the condition that all CSI-RS resources configured for measurement are available in DL subband of SBFD symbols and/or on non-SBFD DL symbols. FFS for DUD configuration. |
| LGE | It should be discussed in RAN1/RAN4, not needed in RAN2 perspective. RAN1/RAN4 will be more expertise on any impact on RLM/BFD operation in SBFD symbol. |
| OPPO | For both 3.3.1 and 3.3.2, we think that these can be left to RAN1/RAN4 to decide. |
| Samsung | 3.3.1: Introducing the new BFD/BFR procedure for SBFD symbols are not acceptable. The purpose of BFD/BFR is checking the radio link monitoring and we think even the separate link quality is monitored by SBFD symbols, the radio link monitoring should be handled unified. In that sense, RAN2 could consider the proposals in [27] that - Keep the legacy BFD and BFR procedure in the MAC layer, i.e., no need for the MAC entity to trigger BFD and BFR for non SBFD symbols and SBFD symbols separately - The network configures the valid symbol type for CSI-RS based RLM/BFD/CBD measurement i.e. SBFD symbols could be used for this RLM/BFD/CBD measurement. 3.3.2: Based on the above RAN1 agreements, L1 CSI-RS resources for SBFD symbols could be measured separately, then it is also possible the separate CSI-RS measurement is used for L3 measurement (i.e. RRM manner). It is good for gNB to know the separate measurement report for SBFD in order to decide the mobility (i.e. HO), the how to support could be considered. In short, the measurement of L1 CSI-RS resources for SBFD symbols separately is already supported, then only the new thing is reporting the measured results for L3 RRM manner. We think this feature has no big spec. impact and RAN2 could decide it. |
| Xiaomi | We think 3.3.1 and 3.3.2 should be decided by RAN1/RAN4. |
| Ericsson | Solution for 3.3.1. is fine, which is aligned with RAN1 LS reply to RAN4. [Eri2] After double checking RAN1 LS reply, we confirm that RAN1 has not indicated to include the valid symbol type for RadioLinkMonitoringConfig and BeamFailureRecoveryConfig. So we agree 3.3.1 is up to RAN1/RAN4 to decide. Solution for 3.3.2, it may be sufficient for the gNB to include valid symbol type in measurement configuration. E.g., we can extend the meas report such that UE can report measurements taken in SBFD slots and non-SBFD slots separately. In addition, it is necessary to further discuss if the UE needs to be signalled of the SBFD T/F configuration of each neighbour cell by the serving gNB. |
| ZTE | We do not think separate L3 report is beneficial, since L3 measurement is to collect and average several L1 measurements to reflect the cell quality. If L3 measurement is reported only for SBFD symbols, does this mean the L3 measurement is only generated by the L1 measurement on SBFD symbols? Since L1 measurement on SBFD symbols and non-SBFD symbols will have different level of RSRP due to interference/bandwidth/etc., in this case, the L3 measurement cannot evaluate the cell quality fairly/accurately. In R2-2500607, ZTE has analysed the issue and give a better solution as below: Proposal 4: gNB configures additional power threshold to UE, the additional power threshold is used for the consolidation of cell measurement from the beam measurements made on SBFD symbols. Another issue is, not only CSI-RS but also SSB will be impacted in the L3/L1 measurement. RAN1 has agreed SSB can be transmitted on SBFD symbols: RAN1#118-bis Working Assumption For SSB symbols configured with SBFD subbands, - Option 1: The SSB symbols configured with SBFD subbands are SBFD symbols. Only DL receptions within DL usable PRBs are allowed for SBFD aware UEs. Note: The SSB block is assumed to be within DL subband. |
| Qualcomm | RAN1 has already indicated in their reply LS R1-2501560 to RAN4 that the ‘Configuration 1 and Configuration 2 are not applicable to CSI-RS resources configuration and CSI-RS based RLM/BFD/CBD measurement.’. So, 3.3.1 is not aligned with RAN1 and can not be agreed. For 3.3.2, we think it is similar issue/configuration which is also clearly indicated in RAN1 LS. Given the current configuration areed by RAN1, the CSI measurements on SBFD symbols and non-SBFD symbols should have the similar situation and no additional interference conditions. So, there is no need to have separate L3 measurement report for SBFD symbol. |
| Sharp | We can leave it up to RAN1. |
| Nokia | 3.3.1 : In the last RAN1 meeting, an LS was sent to RAN4 (R1-2501560), where in Issue 2 RAN1 answered that configuration 1 and configuration 2 are not applicable to CSI-RS configuration for RLM/BFD. The configuration of a valid symbol type is applicable to L1-RSRP and SINR measurement reporting. Therefore, we believe that this issue should be discussed in RAN1 and RAN4. 3.3.2: Before agreeing on this solution, we think RAN2 should discuss whether the time-frequency resource configuration of the neighbour cell is known by the UE performing the layer 3 measurements. This issue is also being discussed in RAN4 |
| CMCC | Both the two solutions should be discussed and determined in RAN1/RAN4. |

## 3.x Issue...

Issue description:

Solutions:

3.x.1:

Further comments:

Companies can provide comments, e.g. any concern on above issue(s)/solution(s).

| Company | Comments |
| --- | --- |
| CATT | SBFD subband configuration also has RRC impact. |
| Ericsson | RAN2 should discuss if SBFD operation can be supported for CA and DC. |
| ZTE | Agree with Ericsson. RAN1 has already support multiple carrier scenario with SBFD. RAN2 should discuss the spec impact of the CA and DC work together with SBFD. |

# Conclusion:

List of open issues of RRC impact:

# Reference

- R2-2500274	Discussion on Random Access in SBFD symbols	 CATT

- R2-2500884	SBFD RA aspects 	Ericsson

- R2-2500298	Random Access for SBFD Operation	NEC

- R2-2500339	Discussion on Random Access operation in SBFD	InterDigital, Inc.

- R2-2500588	Remaining issues for RACH in SBFD	Apple

- R2-2500298	Random Access for SBFD Operation	NEC

- R2-2500606	Discussion on random access procedure in SBFD	 ZTE Corporation

- R2-2501244	Discussion on Random Access in SBFD	LG Electronics Inc.

- R2-2500098	Impacts on the random access by the evolution of duplex operation		Huawei, HiSilicon

- R2-2500105	Random Access Operation of SBFD	Nokia Corporation

- R2-2500109	SBFD Configuration for initial random access and operations	Charter Communications, Inc

- R2-2500191	Discussion on RACH in SBFD	Xiaomi

- R2-2500337	Discussion on random access procedure in SBFD	vivo

- R2-2500376	Random Access Issues for SBFD	Sharp

- R2-2500751	Random access for SBFD Operation	Sony

- R2-2501029	Discussion on random access in SBFD	CMCC

- R2-2501129	Views on random access for SBFD	Qualcomm Incorporated

- R2-2501135	Random access in SBFD	Samsung

- R2-2500110	Cross link interference handling in SBFD networks	Charter Communications, Inc

- R2-2500275	Discussion on other aspects of SBFD	CATT

- R2-2500338	SBFD other aspects	vivo

- R2-2500340	Discussion on resource configuration in SBFD	InterDigital, Inc.

- R2-2500403	Other aspects of SBFD	Xiaomi

- R2-2500554	Other aspects of SBFD	Nokia

- R2-2500607	Discussion on L3 and L1 measurement in SBFD	ZTE Corporation

- R2-2500637	Other impacts by the evolution of duplex operation	Huawei, HiSilicon

- R2-2500885	CSI-RS measurements for RLM/BFD/CBD in SBFD	Ericsson

- R2-2501130	Other aspects of SBFD	Qualcomm Incorporated

- R2-2501170	Other Aspects of SBFD	Samsung