3GPP TSG-RAN WG2 Meeting #130	R2-250XXXX

# St. Julian’s, Malta, May 19th – 23rd, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.305 | CR | Draft | rev | - | Current version: | 18.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | x | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | 38.305 running CR for AIML Positioning |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_AIML_air-Core |  |  |  |  |  | Date: |  |  | 2025-04-25 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Introduction of UE-based AI/ML positioning with UE-side model in Rel-19. Only the specification changes for AI/ML positioning Case 1 (UE-based positioning with UE-side model) are captured in this running CR. Note: RAN3 is working on AI/ML postioning Case 3a/3b, and RAN2 is supposed to merge RAN3 stage-2 TPs later. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduction of UE-based AI/ML positioning with UE-side model in Rel-19. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | UE-based AI/ML positioning with UE-side model is not supported in Rel-19. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 3.1, 3.2, 4.3.1, 4.3.X, 7.13, 8.X |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | x |  | Other core specifications |  |  |  | TS/TR 37.355 CR xxxx |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | RAN2#129bis: initial version in R2-2501920 RAN2#130: updated based on [POST129bis][014][AI PHY] 38.305 Running CR (CATT), and the main changes include: - Remove separate sub-clause for applicability reporting, and add the corresponding description to “Capability Transfer Procedure” - Remove the corresponding description for positioning integrity - Add the content of “Information that may be transferred from the LMF to UE” based on available RAN1 agreement |  |  |  |  |  |  |  |  |

START OF CHANGES

3.1	Definitions

For the purposes of the present document, the terms and definitions given in TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in TR 21.905 [1].

As used in this document, the suffixes "-based" and "-assisted" refer respectively to the node that is responsible for making the positioning calculation (and which may also provide measurements) and a node that provides measurements (but which does not make the positioning calculation). Thus, an operation in which measurements are provided by the UE to the LMF to be used in the computation of a position estimate is described as "UE-assisted" (and could also be called "LMF-based"), while one in which the UE computes its own position is described as "UE-based". For sidelink positioning, an operation in which measurements are provided by a SL Target UE to a server (SL Server UE or LMF) to be used in the computation of a position estimate is described as "SL Target UE-assisted" (and could also be called "server-based"), while one in which the SL Target UE computes its own position is described as "SL Target UE-based".

Alert Limit (AL): The maximum allowable positioning error for the purpose of integrity. If the positioning error is beyond this limit, the integrity results of the calculated location may not meet the integrity requirement.

Editor's Note: FFS whether we need a definition for applicable functionality, considering the term “(positioning) functionality” is already used at multiple places in TS 38.305.

Mobile TRP: a TRP belonging to a mobile IAB-node.

Positioning integrity: A measure of the trust in the accuracy of the position-related data and the ability to provide associated alerts.

Pre-configured assistance data: Refers to the DL-PRS assistance data (with associated validity criteria) that can be provided to the UE (before or during an ongoing LPP positioning session), to be then utilized for potential positioning measurements at a future time (e.g. for deferred MT-LR). Pre-configured DL-PRS assistance data may consist of multiple instances, where each instance is applicable to a different area within the network.

Protection Level (PL): A statistical upper-bound of the Positioning Error (PE) that ensures that, the probability per unit of time of the true error being greater than the AL and the PL being less than or equal to the AL, for longer than the TTA, is less than the TIR, i.e., the PL satisfies the following inequality: 
	Prob per unit of time [((PE>AL) & (PL<=AL)) for longer than TTA] < TIR
When the PL bounds the positioning error in the horizontal plane or on the vertical axis then it is called Horizontal Protection Level (HPL) or Vertical Protection Level (VPL) respectively.
A specific equation for the PL is not specified as this is implementation-defined. For the PL to be considered valid, it must simply satisfy the inequality above.

NOTE 1:	the PL inequality is valid for all values of the AL.

NOTE 2:	the TIR may correspond to the achievable TIR in the case that the requested TIR cannot be satisfied.

PRS-only TP: A TP which only transmits PRS, DL-PRS signals and is not associated with a cell.

PRS Processing Window (PPW): The PRS Processing Window is configured by the network to a UE for NR DL-PRS measurements without measurement gap.

Ranging/SL Positioning Protocol (RSPP): RSPP comprises SLPP messages defined in TS 38.355 [47], Supplementary Services messages defined in TS 24.080 [53], and Supplementary RSPP signalling messages defined in TS 24.514 [54].

Ranging: Refers to the determination of the distance between two UEs or more UEs and/or the direction of one UE from another UE via sidelink interface.

Reception Point (RP): A set of geographically co-located receive antennas (e.g. antenna array (with one or more antenna elements)) for one cell, part of one cell or one UL-SRS-only RP. Reception Points can include base station (ng-eNB or gNB) antennas, remote radio heads, a remote antenna of a base station, an antenna of a UL-SRS-only RP, etc. One cell can include one or multiple reception points. For a homogeneous deployment, each reception point may correspond to one cell.

Relative Position: An estimate of the UE position relative to other network elements or relative to other UEs.

Rx Time Delay: From a signal reception perspective, there will be a time delay from the time when the RF signal arrives at the Rx antenna to the time when the signal is digitized and time-stamped at the baseband.

Rx Timing Error: Result of Rx time delay involved in the reception of a signal before reporting measurements that are obtained from the signal. It is the uncalibrated Rx time delay, or the remaining delay after the UE/TRP internal calibration/compensation of the Rx time delay, involved in the reception of the DL-PRS/UL SRS signals. The calibration/compensation may also include the calibration/compensation of the relative time delay between different RF chains in the same UE/TRP and may also possibly consider the offset of the Rx antenna phase centre to the physical antenna centre.

Sidelink Positioning: A functionality which determines geographical or relative location and possibly velocity using sidelink measurements.

SL Anchor UE: A UE, supporting positioning of target UE, e.g. by transmitting and/or receiving reference signals for positioning, providing positioning-related information, etc. using Sidelink.

SL Server UE: A UE offering position method determination, assistance data distribution and/or location calculation functionalities for sidelink positioning and ranging based services. It interacts with other UEs over PC5 as necessary in order to determine a ranging/SL position method, distribute assistance data and calculate the location of the target UE. A Target UE or SL Anchor UE can act as SL Server UE if any of the functionalities is supported.

SL Target UE: A UE whose distance, direction and/or position is measured with the support from one or multiple SL Anchor UEs using sidelink.

SRS-only RP: An RP which only receives UL-SRS signals and is not associated with a cell.

Transmission Point (TP): A set of geographically co-located transmit antennas (e.g. antenna array (with one or more antenna elements)) for one cell, part of one cell or one DL-PRS-only TP. Transmission Points can include base station (ng-eNB or gNB) antennas, remote radio heads, a remote antenna of a base station, an antenna of a DL-PRS-only TP, etc. One cell can include one or multiple transmission points. For a homogeneous deployment, each transmission point may correspond to one cell.

Transmission-Reception Point (TRP): A set of geographically co-located antennas (e.g. antenna array (with one or more antenna elements)) supporting TP and/or RP functionality.

TRP Rx 'Timing Error Group' (TRP Rx TEG): Rx timing errors, associated with TRP reporting of one or more UL measurements, that are within a certain margin.

TRP RxTx 'Timing Error Group' (TRP RxTx TEG): Rx timing errors and Tx timing errors, associated with TRP reporting of one or more gNB Rx-Tx time difference measurements, which have the 'Rx timing errors+Tx timing errors' differences within a certain margin.

TRP Tx 'Timing Error Troup' (TRP Tx TEG): Tx timing errors, associated with TRP transmissions on one or more DL-PRS resources, that are within a certain margin.

Tx Time Delay: From a signal transmission perspective, the time delay from the time when the digital signal is generated at baseband to the time when the RF signal is transmitted from the Tx antenna.

Tx Timing Error: Result of Tx time delay involved in the transmission of a signal. It is the uncalibrated Tx time delay, or the remaining delay after the TRP/UE internal calibration/compensation of the Tx time delay, involved in the transmission of the DL-PRS/UL SRS signals. The calibration/compensation may also include the calibration/compensation of the relative time delay between different RF chains in the same TRP/UE and may also possibly consider the offset of the Tx antenna phase centre to the physical antenna centre.

U2N Relay UE: A UE that provides functionality to support connectivity to the network for U2N Remote UE(s).

U2N Remote UE: A UE that communicates with the network via a U2N Relay UE.

UE Rx 'Timing Error Group' (UE Rx TEG): Rx timing errors, associated with UE reporting of one or more DL measurements (RSTD), that are within a certain margin.

UE RxTx 'Timing Error Group' (UE RxTx TEG): Rx timing errors and Tx timing errors, associated with UE reporting of one or more UE Rx-Tx time difference measurements, which have the 'Rx timing errors+Tx timing errors' differences within a certain margin.

UE Tx 'Timing Error Group' (UE Tx TEG): Tx timing errors, associated with UE transmissions on one or more UL SRS resources for positioning purpose, that are within a certain margin.

NEXT CHANGE

3.2	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1].

5GC	5G Core Network

5GS	5G System

A-AoA	Azimuth-Angle of Arrival

ADR	Accumulated Delta Range

AI	Artificial Intelligence

AL	Alert Limit

AoA	Angle of Arrival

AP	Access Point

APC	Antenna Phase Center

ARP	Antenna Reference Point

BDS	BeiDou Navigation Satellite System

BSSID	Basic Service Set Identifier

CID	Cell-ID (positioning method)

CLAS	Centimetre Level Augmentation Service

DL-AoD	Downlink Angle-of-Departure

DL-PRS	Downlink Positioning Reference Signal

DL-RSCP	Downlink Reference Signal Carrier Phase

DL-RSCPD	Downlink Reference Signal Carrier Phase Difference

DL-TDOA	Downlink Time Difference Of Arrival

DNU	Do Not Use

DRX	Discontinuous Reception

E-SMLC	Enhanced Serving Mobile Location Centre

E-CID	Enhanced Cell-ID (positioning method)

ECEF	Earth-Centered, Earth-Fixed

ECI	Earth-Centered-Inertial

eDRX	Extended Discontinuous Reception

EGNOS	European Geostationary Navigation Overlay Service

E-UTRAN	Evolved Universal Terrestrial Radio Access Network

FDMA	Frequency Division Multiple Access

FKP	Flächenkorrekturparameter (Engl: Area Correction Parameters)

GAGAN	GPS Aided Geo Augmented Navigation

GCS	Global Coordinate System (as defined in TR 38.901 [55])

GLONASS	GLObal'naya NAvigatsionnaya Sputnikovaya Sistema (Engl.: Global Navigation Satellite System)

GMLC	Gateway Mobile Location Centre

GNSS	Global Navigation Satellite System

GPS	Global Positioning System

GRS80	Geodetic Reference System 1980

HESSID	Homogeneous Extended Service Set Identifier

IOD	Issue of Data

LCS	LoCation Services

Local Coordinate System (as defined in TR 38.901 [55])

LCS-UP	Location Services User Plane

LCS-UPP	Location Services User Plane Protocol

LMF	Location Management Function

LPP	LTE Positioning Protocol

MAC	Master Auxiliary Concept

MBS	Metropolitan Beacon System

ML	Machine Learning

MO-LR	Mobile Originated Location Request

MT-LR	Mobile Terminated Location Request

Multi-RTT	Multi-Round Trip Time

NavIC	NAVigation with Indian Constellation

NG-C	NG Control plane

NG-AP	NG Application Protocol

NI-LR	Network Induced Location Request

N-RTK	Network – Real-Time Kinematic

NRPPa	NR Positioning Protocol A

NTN	Non-Terrestrial Network

OTDOA	Observed Time Difference Of Arrival

PCO	Phase Center Offset

PCV	Phase Center Variation

PDU	Protocol Data Unit

posSI	Positioning System Information

posSIB	Positioning SIB

PPP	Precise Point Positioning

PPP-RTK	Precise Point Positioning – Real-Time Kinematic

PRS	Positioning Reference Signal (for E-UTRA)

PRU	Positioning Reference Unit

QZSS	Quasi-Zenith Satellite System

RP	Reception Point

RRM	Radio Resource Management

RSPP	Ranging/SL Positioning Protocol

RSRP	Reference Signal Received Power

RSRPP	Reference Signal Received Path Power

RSRQ	Reference Signal Received Quality

RSSI	Received Signal Strength Indicator

RSTD	Reference Signal Time Difference

RTD	Relative Time Difference

RTK	Real-Time Kinematic

SBAS	Space Based Augmentation System

SDT	Small Data Transmission

SET	SUPL Enabled Terminal

SIB	System Information Block

SL	Sidelink

SL-PRS	Sidelink Positioning Reference Signal

SL-PRS-RSRP	Sidelink PRS Reference Signal Received Power

SL-PRS-RSRPP	Sidelink PRS Reference Signal Received Path Power

SL-RSTD	Sidelink Reference Signal Time Difference

SL-RTOA	Sidelink Relative Time of Arrival

SLP	SUPL Location Platform

SLPP	Sidelink Positioning Protocol

SP	Semi-Persistent

SRS	Sounding Reference Signal

SSB	Synchronization Signal Block

SSID	Service Set Identifier

SSR	State Space Representation

STEC	Slant TEC

SUPL	Secure User Plane Location

TADV	Timing Advance

TBS	Terrestrial Beacon System

TEC	Total Electron Content

TEG	Timing Error Group

TP	Transmission Point

TRP	Transmission-Reception Point

TTA	Time To Alert

TxTEG	Tx Timing Error Group

UE	User Equipment

UL-AoA	Uplink Angle of Arrival

UL-RSCP	Uplink Reference Signal Carrier Phase

UL-RTOA	Uplink Relative Time of Arrival

UL-SRS	Uplink Sounding Reference Signal

UL-TDOA	Uplink Time Difference of Arrival

UPF	User Plane Function

URA	User Range Accuracy

WAAS	Wide Area Augmentation System

WGS-84	World Geodetic System 1984

WLAN	Wireless Local Area Network

Z-AoA	Zenith Angles of Arrival

NEXT CHANGE

4.3	Standard UE Positioning Methods

4.3.1	Introduction

The standard positioning methods supported for NG-RAN access are:

-	network-assisted GNSS methods;

-	observed time difference of arrival (OTDOA) positioning based on LTE signals;

-	enhanced cell ID methods based on LTE signals;

-	WLAN positioning;

-	Bluetooth positioning;

-	terrestrial beacon system (TBS) positioning;

-	sensor based methods:

-	barometric Pressure Sensor;

-	motion sensor.

-	NR enhanced cell ID methods (NR E-CID) based on NR signals;

-	Multi-Round Trip Time Positioning (Multi-RTT based on NR signals);

-	Downlink Angle-of-Departure (DL-AoD) based on NR signals;

-	Downlink Time Difference of Arrival (DL-TDOA) based on NR signals;

-	Uplink Time Difference of Arrival (UL-TDOA) based on NR signals;

-	Uplink Angle-of-Arrival (UL-AoA), including A-AoA and Z-AoA based on NR signals;

-	SL positioning and Ranging based on sidelink signals, incl.:

-	Sidelink Round Trip Time Positioning (SL-RTT);

-	Sidelink Angle-of-Arrival (SL-AoA);

-	Sidelink Time Difference of Arrival (SL-TDOA);

-	Sidelink Time of Arrival (SL-TOA).

-	AI/ML positioning based on NR signals;

Hybrid positioning using multiple methods from the list of positioning methods above is also supported.

Standalone mode (e.g. autonomous, without network assistance) using one or more methods from the list of positioning methods above is also supported.

These positioning methods (except SL positioning and ranging methods) may be supported in UE-based, UE-assisted/LMF-based, and NG-RAN node assisted versions. Table 4.3.1-1 indicates which of these versions are supported in this version of the specification for the standardised positioning methods.

Table 4.3.1-1: Supported versions of UE positioning methods

| Method | UE-based | UE-assisted, LMF-based | NG-RAN node assisted | SUPL Note 8 |
| --- | --- | --- | --- | --- |
| A-GNSS | Yes | Yes | No | Yes |
| OTDOA Note1, Note 2 | No | Yes | No | Yes |
| E-CID Note 4, Note 7 | No | Yes | Yes | Yes for E-UTRA |
| Sensor | Yes | Yes | No | No |
| WLAN | Yes | Yes | No | Yes |
| BluetoothNote 9 | Yes | Yes | No | No |
| TBS Note 5 | Yes | Yes | No | Yes (MBS) |
| DL-TDOA | Yes | Yes | No | Yes |
| DL-AoD | Yes | Yes | No | Yes |
| Multi-RTT | No | Yes | Yes | Yes |
| NR E-CID | No | Yes | Yes | Yes (DL NR E-CID) |
| UL-TDOA | No | No | Yes | Yes |
| UL-AoA | No | No | Yes | Yes |
| AI/MLNote10 | Yes | No | FFS | N/A |
| NOTE 1: This includes TBS positioning based on PRS signals. NOTE 2: In this version of the specification only OTDOA based on LTE signals is supported. NOTE 3: Void NOTE 4: This includes Cell-ID for NR method when UE is served by gNB. NOTE 5: In this version of the specification only for TBS positioning based on MBS signals. NOTE 6: Void NOTE 7: Enhanced Cell ID based on LTE signals. NOTE 8: This shows whether the positioning method is supported by SUPL ULP [16]. NOTE 9: UE-based positioning mode is supported only for Bluetooth AoD positioning method. NOTE10: AI/ML model resides in UE for UE-based AI/ML positioning. |  |  |  |  |

Sensor, WLAN, Bluetooth, and TBS positioning methods based on MBS signals are also supported in standalone mode, as described in the corresponding clauses.

The SL positioning and ranging methods may be supported in SL-Target UE-based or SL-Target UE-assisted/server-based mode, where "server" may be a SL Server UE or LMF. Table 4.3.1-2 indicates which of these versions are supported in this version of the specification for the SL positioning and ranging methods.

Table 4.3.1-2: Supported versions of SL positioning and ranging methods.

| Method | SL-Target UE-based | SL-Target UE-assisted, server-based |
| --- | --- | --- |
| SL-RTTNOTE 1 | Yes | Yes |
| SL-AoANOTE 2 | Yes | Yes |
| SL-TDOA | Yes | Yes |
| SL-TOA | Yes | Yes |
| NOTE 1: The SL-RTT method may also be used for ranging between UEs. NOTE 2: The SL-AoA method may also be used to obtain direction between UEs. |  |  |

NEXT CHANGE

4.3.X	AI/ML positioning

The AI/ML positioning method makes use of the downlink signals received from multiple TPs to determine AI/ML model input, at the UE. The UE performs AI/ML model inference using assistance data received from the positioning server, and other configuration information to locate the UE in relation to the neighbouring TPs.

The operation of the UE-based AI/ML positioning method is described in clause 8.X.

NEXT CHANGE

NEXT CHANGE

8	Positioning methods and Supporting Procedures

8.X	UE-based AI/ML positioning

8.X.1	General

In the AI/ML positioning method, the UE position is predicted by AI/ML model , .

How AI/ML model is used for inference of UE position is up to implementation and beyond the scope of this specification.

8.X.2	Information to be transferred between NG-RAN/5GC Elements

8.X.2.0	General

This clause defines the information that may be transferred between LMF and UE/gNB.

8.X.2.1	Information that may be transferred from the LMF to UE

8.X.2.1.0	General

Editor's note: Information that may be transferred from the LMF to UE depends on RAN1 agreements and future parameter list.

The information that may be transferred from the LMF to the UE are listed in table 8.X.2.1.0-1.

Table 8.X.2.1.0-1: Assistance data that may be transferred from LMF to the UE

| Information | UE-based |
| --- | --- |
| Physical cell IDs (PCIs), global cell IDs (GCIs), ARFCN, and PRS IDs of candidate NR TRPs for measurement | Yes |
| Timing relative to the serving (reference) TRP of candidate NR TRPs | Yes |
| DL-PRS configuration of candidate NR TRPs | Yes |
| Indication of which DL-PRS Resource Sets across DL-PRS positioning frequency layers are linked for DL-PRS bandwidth aggregation | Yes |
| SSB information of the TRPs (the time/frequency occupancy of SSBs) | Yes |
| Spatial direction information (e.g. azimuth, elevation etc.) of the DL-PRS Resources of the TRPs served by the gNB | Yes |
| Fine Timing relative to the serving (reference) TRP of candidate NR TRPs | Yes |
| PRS-only TP indication | Yes |
| The association information of DL-PRS resources with TRP Tx TEG ID | Yes |
| LOS/NLOS indicators | Yes |
| On-Demand DL-PRS-Configurations, possibly together with information on which configurations are available for DL-PRS bandwidth aggregation | Yes |
| Validity Area of the Assistance Data | Yes |
| PRU measurements together with the location information of the PRU | Yes |
| Data facilitating the integrity results determination of the calculated location | Yes |

8.X.2.2	Information that may be transferred from the UE to LMF

Editor's note: FFS what information may need to be transferred from the UE to LMF, e.g., the UE location and time stamp, which depends on RAN1 parameter list or RAN2 further discussion.

8.X.2.3	Information that may be transferred from the gNB to LMF

Editor's note: Information that may be transferred from the gNB to LMF depends on RAN1 and RAN3 input.

8.X.3	AI/ML Positioning Procedures

8.X.3.0	General

The procedures described in this clause support UE-based AI/ML positioning.

8.X.3.1	Procedures between LMF and UE

8.X.3.1.1	Capability Transfer Procedure

The Capability Transfer procedure for AI/ML positioning is described in clause 7.1.2.1.

The unsolicited capability transfer procedure is also to enable the UE to provide information of whether the AI/ML positioning method is applicable to the LMF (e.g., as part of a positioning procedure).

Figure 8.X.3.1.1-1 shows the Applicability reporting Transfer operations for the AI/ML positioning method when the procedure is initiated by the UE.

Figure 8.X.3.1.1-1: UE-initiated Applicability reporting Transfer Procedure

(1)	UE may send an unsolicited LPP Provide Capabilities message to the LMF when the applicability of AI/ML positioning method changes.

Editor's note: Whether the “general principle” agreed for BM in RAN2#127 can also be applied for POS case 1, i.e., The UE determines the applicable functionalities based on NW-side additional conditions (if provided), UE-side additional conditions (internally known by UE) and model availability in device.

Editor’s note: whether the agreement for BM made in RAN2#129 can also be applicable to AI/ML positioning Case 1, i.e., “Support the explicit reporting of applicability/inapplicability in initial report and subsequent reporting it reports only applicability it changed. FFS if we report explicit cause”

8.X.3.1.2	Assistance Data Transfer Procedure

Editor's note: RAN2 needs to discuss whether the procedure described in 8.12.3.1.2 (for DL-TDOA) can be reused.

Editor's note: Whether the “general principle” agreed for BM in RAN2#127 can also be applied for POS case 1, i.e., The UE determines the applicable functionalities based on NW-side additional conditions (if provided), UE-side additional conditions (internally known by UE) and model availability in device.

Editor’s note: whether the agreement for BM made in RAN2#129 can also be applicable to AI/ML positioning Case 1, i.e., “Support the explicit reporting of applicability/inapplicability in initial report and subsequent reporting it reports only applicability it changed. FFS if we report explicit cause”

8.X.3.1.4	Location Information Transfer Procedure

8.X.3.1.4.0	General

The purpose of this procedure is to enable the LMF to request location estimate from the UE.

8.X.3.1.4.1	LMF-initiated Location Information Transfer Procedure

Figure 8.X.3.1.4.1-1 shows the Location Information Transfer operations for the AI/ML positioning method when the procedure is initiated by the LMF.

Figure 8.X.3.1.4.1-1: LMF-initiated Location Information Transfer Procedure

(1)	The LMF sends an LPP Request Location Information message to the UE. This request includes an indication of AI/ML model inference for positioning.

(2)	The UE then sends an LPP Provide Location Information message to the LMF, and includes the inferred location coordinates. If the UE is unable to perform the AI/ML positioning, the UE returns LPP Provide Location Information message with error cause

Editor’s note: whether the UE behaviour for activating AI/ML positioning functionality needs to be captured in stage-2.

8.X.3.2	Procedures between LMF and gNB

Editor's note: depending on further RAN1/RAN3 input.

8.X.3.2.1	Assistance Data Delivery procedure

8.X.3.2.1.0	General

8.X.3.2.1.1	LMF-initiated assistance data delivery to the LMF

END OF CHANGES

Annex of meeting agreements:

RAN2#125bis

Agreements:

- For POS, RAN2 assumes gNB or LMF could perform performance monitoring for case 3a and LMF is responsible for the performance monitoring for case 3b and wait for any further inputs from other WGs

- For POS, RAN2 assumes that NRPPa is used for the signalling between gNB and LMF for case 3a and 3b and the detailed signalling design is up to RAN3.

Agreements

- Which AI/ML-enabled Features/FGs and functionalities are supported should be standardized. The details wait for RAN1’s progress.   “supported” means that the UE is capable of supporting the functionality and doesn’t mean neccesarily that the UE has the model available.  FFS what functionality refers to.

- Supported AI/ML-enabled Features/FGs and supported functionalities are included in UE capability.

Agreements for positioning and beam management

- Support proactive reporting of UE-sided applicable functionality, e.g., the UE reports its applicable AI/ML functionalities via UAI message/LPP message.

- Support reactive reporting of UE-sided applicable functionality.  The NW configures AI/ML functionalities via RRC/LPP message.  FFS what the configuration contains. FFS how to report applicable functionality and what is applicable functionality

3	FFS how the two approaches will be specified and whether we can combine them into one procedure.    FFS how to report applicable functionality, what is applicable functionality, how the UE determines which function is applicable or not (if it is needed)

RAN2#126

Agreements

2	We will work offline on the definitions for functionality types and define what is availability.

3	The UE will indicate the gNB/LMF whether the AI/ML functionality is available/applicable.   For a functionality to be applicable at least there should at least one model available within it.   FFS other details on what is applicability/non-applicability.

Agreements for NWside model

1	RAN2 to await RAN1 progress to determine need for any specification work on AI/ML positioning Case 3a and Case 3b.

Agreements

- The LPP Capability Transfer procedures (RequestCapabilities/ProvideCapabilities messages) are used to indicate supported AI/ML positioning capabilities.  FFS how to handle dynamic capabilities, depending on further RAN1 progress and understanding of the functionality.

- wait for RAN1 for associate ID discussion

- At least for Case 1, existing LPP procedures related to Location Information Transfer (RequestLocationInformation/ ProvideLocationInformation messages) are used for providing the results of the UE sided model inference operation.  FFS further details on signaling enhancements

RAN2#127

“General principle” without distinguishing use case in BM case item:

Agreements on definitions

- Supported functionalities refer to functionalities that UE can indicate by using UE capability information (via RRC/LPP signalling)

- Applicable functionalities refers to functionalities that the UE is ready to apply for inference

- Activated functionalities refers to functionalities already enabled for performing inference

Agreements on definitions

- UE decides the applicable functionalities based on NW-side additional conditions (if provided), UE-side additional conditions (internally known by UE) and model availability in device. FFS whether other configuration can considered by UE (e.g. inference configuration).  FFS  how the applicable functionality is decided if NW-side additional condition is not provided in step 3.

RAN2#127bis

Agreements:

1: 	The following procedures for LCM for UE sided model for AI positioning case 1 is the baseline:

Step 1: LMF may request the UE to report the supported functionalities at the UE side by LPP request capabilities message.

Step 2: UE sends LPP provide capabilities message to LMF with the supported functionalities at the UE side.

Step 3: LMF sends the LPP provide assistance data message (which may contain network side additional condition).

Step 4: UE reports the applicable functionality to the LMF by the LPP provide capabilities message.

Step 5: The LMF requests the inferred location information using the LPP request location information message.

Step 6: UE reports the inferred location using LPP provide location information message.

2: 	Whether the inference configuration is provided in step 3 or/and step 5 is FFS (to be revised based on RAN1 progress).

3: 	Whether network side additional condition is needed and what it contains is FFS (to be revised based on RAN1 progress).

4: 	FFS whether LMF controls the UE sending unsolicited LPP provide capabilities (i.e. whether step4 is sent reactively or proactively).  FFS the signalling details.

5:   RAN2 will decide whether AI positioning will be a new method after further details from RAN1 are received.

RAN2#128

Agreements

1	For POS Case 1, RAN2 confirm that the existing unsolicited UE capability report mechanism in LPP can support UE to report the applicable functionality in both “proactive” and “reactive” as a baseline.

- 	Proactive case: When the applicability change, UE can send an unsolicited LPP ProvideCapabilities message to LMF .

- 	Reactive case: If the applicability changes based on the configuration in LPP ProvideAssistanceData message in step 3, UE can send an unsolicited LPP ProvideCapabilities message to LMF.  Configuration details are FFS

2  As a baseline, If the AIML based positioning method becomes non-applicable when LMF requests UE location estimation, UE cannot perform the AIML based positioning, and reply with LPP Providelocationinformation message with error cause.  FFS if other fallback options are considered

RAN2#129

Agreements

1:	Introduce AI/ML positioning Case 1 as a new positioning method.

2:	Existing LPP procedures related to Location Information Transfer (RequestLocationInformation/ ProvideLocationInformation messages) are used for providing and requesting the results of the UE sided model inference operation. The detail stage 3 message extention can be disucssed while drafting the stage 3 CR.

3:	FFS UE autonomous switching between AI/ML and non-AI/ML methods is not allowed.  FFS if this is unconditional or linked to condition of multiple positioning method are not configured in RequestLocationInformation,

4: 	The content of error cause is discussed while drafting stage3 CRs.

5: 	As a baseline, UE receives the needed assistance data for calculating UE location for AI/ML in step3 (ProvideAssistanceData) and UE receives the instruction to perform the inference in step 5 (RequestLocationInformation). The content of Assistance Data and the content of request location information is based upon RAN1 parameter list.

6: 	UE reports the applicable functionality to the LMF by the LPP provide capabilities message if there is a change of applicable functionality.   FFS if any additional LMF control is needed.

RAN2#129bis

Agreements for positioning case 1

- LMF is responsible for functionality management

- UE reports the applicable functionality to the LMF by the LPP provide capabilities message without any additional LMF control.

- Switching/fallback to non-AI/ML positioning can be supported by including multiple positioning methods in a LPP Request Location Information message. No additional specification work is foreseen specifically for supporting "switching/fallback operation".

- An AIML positioning functionality is considered “activated” once UE receives an LPP RequestLocationInformation from the LMF requesting inferred location information.

- For triggered and periodical reporting, we rely on existing positioning framework mechanisms to deactivate AI/ML positioning (no spec impact is foreseen)