3GPP TSG-RAN WG2 Meeting #130	R2-25xxxxx

St Julian’s, Malta, May 19 - 23, 2025

Agenda Item:	8.1.1

Source:	Qualcomm Incorporated (Rapporteur)

Title:	LPP open issues for feature "AI/ML for NR air interface"

Document for:	Discussion, Decision

- Introduction

This document includes a list of open issues according to the following email discussion:

[POST129bis][015][AI PHY] 37.355 Running CR (Qualcomm)

Intended outcome:

- Update CR based on agreements from RAN2#129bis

- List of remaining open issues

Deadline:  Long

Companies are invited to provide feedback on open issue list by: 2nd May 2025

Open issues for specification 37.355 (LPP)

Common NR Positioning Information Elements (Clause 6.4.3)

Open issue LPP-1: Applicability of dl-PRS-ResourcePrioritySubset

Issue description:

The IE NR-DL-PRS-Info also includes method specific information (for DL-AoD). If DL-PRS assistance data are needed for NR AI/ML positioning, it needs to be clarified that the dl-PRS-ResourcePrioritySubset is not applicable to NR AI/ML positioning.

Status in running CR:

Captured as "Editor's Note".

Relevant Agreements:

From RAN1#119 (see Annex in LPP running CR [1] for the full list of RAN1 and RAN2 agreements for Case 1):

Agreement For AI/ML based positioning Case 1, all assistance information from legacy UE-based DL-TDOA, other than info #7, can be provided from LMF to UE. For info #7, RAN1 study, if necessary, choose one alternative from the following: […]

Proposed resolution:

Given that all assistance information for UE-based DL-TDOA (other than info #7) are applicable to NR AI/ML positioning Case 1, the IE NR-DL-PRS-Info is also applicable. This implies that the dl-PRS-ResourcePrioritySubset (which provides a priority for measurement reporting in UE-assisted DL-AoD) is not applicable to NR AI/ML positioning Case 1.

Proposal 1:	The field dl-PRS-ResourcePrioritySubset in IE NR-DL-PRS-Info should be ignored for NR AI/ML positioning. Remove corresponding 'Editor's Note' from the running CR.

Companies are invited to provide feedback regarding the above open issue and proposed resolution:

| Company | Agree to proposal? | Other comments |
| --- | --- | --- |

Open issue LPP-2: Applicability of IE NR-DL-PRS-ProcessingCapability

Issue description:

In the case of capabilities for multiple NR positioning methods are provided, the IE NR-DL-PRS-ProcessingCapability applies across the NR positioning methods and the target device shall indicate the same values for the capabilities in IEs NR-DL-TDOA-ProvideCapabilities, NR-DL-AoD-ProvideCapabilities, and NR-Multi-RTT-ProvideCapabilities.

If the IE NR-DL-PRS-ProcessingCapability is applicable to NR AI/ML positioning, it needs to be clarified whether the above also applies to NR-AI-ML-PositioningProvideCapabilities.

Status in running CR:

Captured as "Editor's Note" (without changes to the current specification text).

Relevant Agreements:

R2-25xxxxx (R1-2502979): LS on updated Rel-19 RAN1 UE features lists for NR after RAN1#120bis.

The above RAN1 feature list includes some DL-PRS Resource capabilities (58-2-3/3a/3b), however, all in yellow highlight (indicating that they are not yet final).

Whether the NR-DL-PRS-ProcessingCapability are applicable to NR AI/ML positioning Case 1 or not is currently unclear (most capabilities in this IE are primarily for UE-assisted mode). In addition, if the capabilities should be applicable, it is unclear whether the same values as in IEs NR-DL-TDOA-ProvideCapabilities, NR-DL-AoD-ProvideCapabilities, and NR-Multi-RTT-ProvideCapabilities shall be indicated.

Proposed resolution:

Wait for further RAN1 input and keep the "Editor's Note" in the running CR.

Proposal 2:	Regarding the applicability of IE NR-DL-PRS-ProcessingCapability to NR AI/ML positioning Case 1, wait for further RAN1 input and keep the current "Editor's Note" in the running CR for now.

Companies are invited to provide feedback regarding the above open issue and proposed resolution:

| Company | Agree to proposal? | Other comments |
| --- | --- | --- |

Open issue LPP-3: Applicability of NR-DL-PRS-QCL-ProcessingCapability

Issue description:

In the case of capabilities for multiple NR positioning methods are provided, the IE NR-DL-PRS-QCL-ProcessingCapability applies across the NR positioning methods and the target device shall indicate the same values for the capabilities in IEs NR-DL-TDOA-ProvideCapabilities, NR-DL-AoD-ProvideCapabilities, and NR-Multi-RTT-ProvideCapabilities.

If the IE NR-DL-PRS-QCL-ProcessingCapability is applicable to NR AI/ML positioning, it needs to be clarified whether the above also applies to NR-AI-ML-PositioningProvideCapabilities.

Status in running CR:

Captured as "Editor's Note" (without changes to the current specification text).

Relevant Agreements:

R2-25xxxxx (R1-2502979): LS on updated Rel-19 RAN1 UE features lists for NR after RAN1#120bis.

The above RAN1 feature list includes some DL-PRS Resource capabilities (58-2-3/3a/3b), however, all in yellow highlight (indicating that they are not yet final).

Whether the NR-DL-PRS-QCL-ProcessingCapability are applicable to NR AI/ML positioning Case 1 or not is currently unclear (most capabilities are primarily for UE-assisted mode). In addition, if the capabilities should be applicable, it is unclear whether the same values as in IEs NR-DL-TDOA-ProvideCapabilities, NR-DL-AoD-ProvideCapabilities, and NR-Multi-RTT-ProvideCapabilities shall be indicated.

Proposed resolution:

Wait for further RAN1 input and keep the "Editor's Note" in the running CR.

Proposal 3:	Regarding the applicability of IE NR-DL-PRS-QCL-ProcessingCapability to NR AI/ML positioning Case 1, wait for further RAN1 input and keep the current "Editor's Note" in the running CR for now.

Companies are invited to provide feedback regarding the above open issue and proposed resolution:

| Company | Agree to proposal? | Other comments |
| --- | --- | --- |

Open issue LPP-4: Applicability of NR-DL-PRS-ResourcesCapability

Issue description:

The IE NR-DL-PRS-ResourcesCapability defines the DL-PRS Resources capability for each positioning method.

However, most capabilities are primarily for UE-assisted mode. Whether and which DL-PRS Resources capabilities are needed for UE-based direct NR AI/ML positioning depends on RAN1.

Status in running CR:

Captured as "Editor's Note" (without changes to the current specification text).

Relevant Agreements:

R2-25xxxxx (R1-2502979): LS on updated Rel-19 RAN1 UE features lists for NR after RAN1#120bis.

The above RAN1 feature list includes some DL-PRS Resource capabilities (58-2-3/3a/3b), however, all in yellow highlight (indicating that they are not yet final).

However, this indicates that DL-PRS Resource capabilities specifically for NR AI/ML Positioning Case 1 are going to be defined.

Proposed resolution:

Wait for further RAN1 input and keep the "Editor's Note" in the running CR.

Proposal 4:	Regarding the applicability of IE NR-DL-PRS-ResourcesCapability to NR AI/ML positioning Case 1, wait for further RAN1 input and keep the current "Editor's Note" in the running CR for now.

Companies are invited to provide feedback regarding the above open issue and proposed resolution:

| Company | Agree to proposal? | Other comments |
| --- | --- | --- |

Open issue LPP-5: Applicability of NR-On-Demand-DL-PRS-Configurations-Selected-IndexList

Issue description:

In the case of available on-demand DL-PRS configurations for multiple NR positioning methods are provided, the IE NR-On-Demand-DL-PRS-Configurations shall be present in only one of NR-Multi-RTT-ProvideAssistanceData, NR-DL-AoD-ProvideAssistanceData, or NR-DL-TDOA-ProvideAssistanceData.

If on-demand DL-PRS is applicable to NR AI/ML positioning, it needs to be clarified whether the above also applies to NR-AI-ML-PositioningProvideAssistanceData.

Status in running CR:

Captured as "Editor's Note" in IEs NR-On-Demand-DL-PRS-Configurations-Selected-IndexList, NR-DL-TDOA-ProvideAssistanceData, NR-DL-AoD-ProvideAssistanceData.

Relevant Agreements:

From RAN1#119 (see Annex in LPP running CR [1] for the full list of RAN1 and RAN2 agreements):

Agreement For AI/ML based positioning Case 1, all assistance information from legacy UE-based DL-TDOA, other than info #7, can be provided from LMF to UE. For info #7, RAN1 study, if necessary, choose one alternative from the following: […]

Proposed resolution:

Given that all assistance information for UE-based DL-TDOA (other than info #7) are applicable to NR AI/ML positioning Case1, the on-demand DL-PRS feature, and associated assistance data and signalling is also applicable to NR AI/ML positioning Case 1. Since the on-demand DL-PRS configurations can then be included in the assistance data for each NR positioning method (incl. NR AI/ML positioning), they need to be provided only once in the case of multiple NR positioning methods are requested and an index indicates the applicable configuration(s) for each method. There seems no reason why NR AI/ML positioning Case 1 should not follow the same logic.

Proposal 5:	The IE NR-On-Demand-DL-PRS-Configurations-Selected-IndexList is also applicable to NR AI/ML positioning Case 1. The corresponding Editor's Notes in clause 6.4.3, 6.5.10.1, and 6.5.11.1 can be removed.

Companies are invited to provide feedback regarding the above open issue and proposed resolution:

| Company | Agree to proposal? | Other comments |
| --- | --- | --- |

Open issue LPP-6: Applicability of NR-PRU-DL-Info

Issue description:

Currently, the IE NR-PRU-DL-Info is primarily used for NR carrier phase positioning.

If the IE NR-PRU-DL-Info is also applicable to NR AI/ML positioning, the IE description needs to be generalized.

Status in running CR:

Captured as "Editor's Note" in IE NR-PRU-DL-Info.

Relevant Agreements:

From RAN1#119 (see Annex in LPP running CR [1] for the full list of RAN1 and RAN2 agreements):

Agreement For AI/ML based positioning Case 1, all assistance information from legacy UE-based DL-TDOA, other than info #7, can be provided from LMF to UE. For info #7, RAN1 study, if necessary, choose one alternative from the following: […]

Proposed resolution:

Given that all assistance information for UE-based DL-TDOA (other than info #7) are applicable to NR AI/ML positioning Case1, the IE NR-PRU-DL-Info is also applicable:

| 14 | PRU measurements together with the location information of the PRU |
| --- | --- |

Therefore, the IE description should be generalized (i.e., not specific to carrier phase measurements).

Proposal 6:	The IE NR-PRU-DL-Info is also applicable to NR AI/ML positioning Case 1. The corresponding Editor's Notes in clause 6.4.3 can be removed.

Companies are invited to provide feedback regarding the above open issue and proposed resolution:

| Company | Agree to proposal? | Other comments |
| --- | --- | --- |

Open issue LPP-7: Applicability of NR-SelectedDL-PRS-IndexList

Issue description:

In the case of assistance data for multiple NR positioning methods are provided, the IE NR-DL-PRS-AssistanceData shall be present in only one of NR-Multi-RTT-ProvideAssistanceData, NR-DL-AoD-ProvideAssistanceData, or NR-DL-TDOA-ProvideAssistanceData.

If the IE NR-DL-PRS-AssistanceData is applicable to NR AI/ML positioning, it needs to be clarified whether the above also applies to NR-AI-ML-PositioningProvideAssistanceData.

Status in running CR:

Captured as "Editor's Note" in IEs NR-SelectedDL-PRS-IndexList, NR-DL-TDOA-ProvideAssistanceData, NR-DL-AoD-ProvideAssistanceData.

Relevant Agreements:

From RAN1#119 (see Annex in LPP running CR [1] for the full list of RAN1 and RAN2 agreements):

Agreement For AI/ML based positioning Case 1, all assistance information from legacy UE-based DL-TDOA, other than info #7, can be provided from LMF to UE. For info #7, RAN1 study, if necessary, choose one alternative from the following: […]

Proposed resolution:

Given that all assistance information for UE-based DL-TDOA (other than info #7) are applicable to NR AI/ML positioning Case1, the NR-DL-PRS-AssistanceData and associated signalling is also applicable to NR AI/ML positioning Case 1. Since the NR-DL-PRS-AssistanceData can then be included in the assistance data for each NR positioning method (incl. NR AI/ML positioning), they need to be provided only once in the case of multiple NR positioning methods are requested and an index indicates the applicable configuration(s) for each method. There seems no reason why NR AI/ML positioning Case 1 should not follow the same logic.

Proposal 7:	The IE NR-SelectedDL-PRS-IndexList is also applicable to NR AI/ML positioning Case 1. The corresponding Editor's Notes in clause 6.4.3, 6.5.10.1, and 6.5.11.1 can be removed.

Companies are invited to provide feedback regarding the above open issue and proposed resolution:

| Company | Agree to proposal? | Other comments |
| --- | --- | --- |

NR AI/ML Positioning Information Elements (Clause 6.5.13)

Open issue LPP-8: Details of IE NR-AI-ML-PositioningProvideAssistanceData

Issue description:

This IE defines the assistance data for NR AI/ML positioning Case 1. According to RAN1#119 agreements, the assistance data should be analogous to DL-TDOA assistance data:

Agreement For AI/ML based positioning Case 1, all assistance information from legacy UE-based DL-TDOA, other than info #7, can be provided from LMF to UE. For info #7, RAN1 study, if necessary, choose one alternative from the following: - Alternative 1. Info #7 is provided implicitly via associated ID. - Associated ID is signaled by LMF to indicate whether info #7 is consistent between training and inference. - Alternative 2. Info #7 can be provided either implicitly or explicitly by LMF. Note: no UE capability is introduced on whether info #7 is provided implicitly or explicitly, and the UE can request info #7 to be provided explicitly or implicitly. - If provided implicitly, associated ID is signaled by LMF to indicate whether info #7 is consistent between training and inference. - Alternative 3. Info #7 is not be provided from LMF to UE. - If info #7 is not provided, UE may assume info #7 is consistent between training and inference. - Alternative 4. Info #7 is provided explicitly from LMF to UE. 7; Geographical coordinates of the TRPs served by the gNB (include a transmission reference location for each DL-PRS Resource ID, reference location for the transmitting antenna of the reference TRP, relative locations for transmitting antennas of other TRPs).

Status in running CR:

The running CR currently includes all assistance data from UE-based DL-TDOA, except the NR-PeriodicAssistData (which is only for carrier phase positioning and not included in the RAN1 agreement), together with an Editor's Note. The RAN1 agreement refers to the information in Stage 2 (38.305):

|  | Information |
| --- | --- |
| 1 | Physical cell IDs (PCIs), global cell IDs (GCIs), ARFCN, and PRS IDs of candidate NR TRPs for measurement |
| 2 | Timing relative to the serving (reference) TRP of candidate NR TRPs |
| 3 | DL-PRS configuration of candidate NR TRPs |
| 4 | Indication of which DL-PRS Resource Sets across DL-PRS positioning frequency layers are linked for DL-PRS bandwidth aggregation |
| 5 | SSB information of the TRPs (the time/frequency occupancy of SSBs) |
| 6 | Spatial direction information (e.g. azimuth, elevation etc.) of the DL-PRS Resources of the TRPs served by the gNB |
| 7 | Geographical coordinates of the TRPs served by the gNB (include a transmission reference location for each DL-PRS Resource ID, reference location for the transmitting antenna of the reference TRP, relative locations for transmitting antennas of other TRPs) |
| 8 | Fine Timing relative to the serving (reference) TRP of candidate NR TRPs |
| 9 | PRS-only TP indication |
| 10 | The association information of DL-PRS resources with TRP Tx TEG ID |
| 11 | LOS/NLOS indicators |
| 12 | On-Demand DL-PRS-Configurations, possibly together with information on which configurations are available for DL-PRS bandwidth aggregation |
| 13 | Validity Area of the Assistance Data |
| 14 | PRU measurements together with the location information of the PRU |
| 15 | Data facilitating the integrity results determination of the calculated location |
| 16 | TRP beam/antenna information (including azimuth angle, zenith angle and relative power between PRS resources per angle per TRP) |
| 17 | Expected Angle Assistance information |
| 18 | PRS priority list |

[1] Table 8.12.2.1.0-1 in 38.305, Use equipment (UE) positioning in NG-RAN (Release 18), v18.3.0

[2] Table 8.11.2.1.0-1 in 38.305, Use equipment (UE) positioning in NG-RAN (Release 18), v18.3.0

NOTE: Items #7, 16-18 (assistance data for DL-AoD) are still under discussion in RAN1.

Proposed resolution:

Given that all assistance information for UE-based DL-TDOA (other than info #7) are applicable to NR AI/ML positioning Case1, the NR-AI-ML-PositioningProvideAssistanceData should include (at least) all the assistance data IEs currently defined for UE-based DL-TDOA. This needs to be revised once additional RAN1 agreements are available.

Proposal 8:	The IE NR-AI-ML-PositioningProvideAssistanceData contains (at least) all assistance data elements from UE-based DL-TDOA as starting point. This will be revised when additional RAN1 input is available. The current Editor's Note is kept for now.

Companies are invited to provide feedback regarding the above open issue and proposed resolution:

| Company | Agree to proposal? | Other comments |
| --- | --- | --- |

Open issue LPP-9: Details of IE NR-AI-ML-PositioningRequestAssistanceData

Issue description:

The IE NR-AI-ML-PositioningRequestAssistanceData is used by the target device to request assistance data from a location server for NR AI/ML direct positioning (e.g., during an ongoing LPP session or via MO-LR).

This IE should be analogous to the NR-AI-ML-PositioningProvideAssistanceData. I.e., the "Request" must match the "Provide".

For the NR-AI-ML-PositioningProvideAssistanceData, see issue #LPP-8.

Status in running CR:

The running CR currently includes all assistance data from UE-based DL-TDOA, except the NR-PeriodicAssistData (which is only for carrier phase positioning), together with an Editor's Note (see issue #LPP-8).

Proposed resolution:

Given that all assistance information for UE-based DL-TDOA (other than info #7) are applicable to NR AI/ML positioning Case1, the NR-AI-ML-PositioningRequestAssistanceData should include (at least) all the assistance data IEs currently defined for UE-based DL-TDOA. This needs to be revised once additional RAN1 agreements are available.

Proposal 9:	The IE NR-AI-ML-PositioningRequestAssistanceData contains (at least) all assistance data elements from UE-based DL-TDOA as starting point. This will be revised when additional RAN1 input is available. The current Editor's Note is kept for now.

Companies are invited to provide feedback regarding the above open issue and proposed resolution:

| Company | Agree to proposal? | Other comments |
| --- | --- | --- |

Open issue LPP-10: Details of IE NR-AI-ML-PositioningProvideLocationInformation

Issue description:

The IE NR-AI-ML-PositioningProvideLocationInformation provides the results of the inference operation. This should comprise the location coordinates (which are reported in CommonIEsProvideLocationInformation) together with the time stamp (as for any UE-based method). However, for the NR positioning methods, batch reporting was introduced in Rel-17 (i.e., up to 32 instances (location results) can be reported at once). One issue here is whether batch reporting should also be applicable to NR AI/ML positioning.

Status in running CR:

Captured as "Editor's Note" in IE NR-AI-ML-PositioningProvideLocationInformation.

Proposed resolution:

The IE NR-AI-ML-PositioningProvideLocationInformation includes at least the location coordinates together with time stamp (as for any UE-based method). Any additional elements depend on further RAN1 progress.

Since NR AI/ML positioning also falls into the category of "NR positioning methods" (e.g., measurements are based on NR DL-PRS), it seems consequent that batch reporting is also applicable to NR AI/ML positioning (in the same way as UE-based DL-TDOA and DL-AoD).

Proposal 10:	The IE NR-AI-ML-PositioningProvideLocationInformation contains (at least) the time stamp for the location coordinates (which are reported in CommonIEsProvideLocationInformation). This will be revised when additional RAN1 input is available. The current Editor's Note is kept for now.

Companies are invited to provide feedback regarding the above open issue and proposed resolution:

| Company | Agree to proposal? | Other comments |
| --- | --- | --- |

Companies are requested to provide their view on whether batch reporting (i.e., up to 32 instances (location results) can be reported at once) should also be applicable to NR AI/ML positioning Case 1.

| Company | Applicable/Not Applicable | Comment/Reason |
| --- | --- | --- |

Open issue LPP-11: Details of IE NR-AI-ML-PositioningRequestLocationInformation

Issue description:

The IE NR-AI-ML-PositioningRequestLocationInformation includes information on the location request. Per agreement from RAN2#129:

- Existing LPP procedures related to Location Information Transfer (RequestLocationInformation/ ProvideLocationInformation messages) are used for providing and requesting the results of the UE sided model inference operation. The detail stage 3 message extension can be discussed while drafting the stage 3 CR.

Status in running CR:

The running CR currently includes only the common AssistanceAvailability flag (which is included in all LPP positioning methods for which assistance data are defined), together with an "Editor's Note" in IE NR-AI-ML-PositioningRequestLocationInformation.

Proposed resolution:

Companies to provide contributions to the following meeting on additional positioning instructions for IE NR-AI-ML-PositioningRequestLocationInformation (if any).

Proposal 11:	The IE NR-AI-ML-PositioningRequestLocationInformation contains (at least) the AssistanceAvailability flag. Additional details/information can be discussed via company contributions.

Companies are invited to provide feedback regarding the above open issue and proposed resolution:

| Company | Agree to proposal? | Other comments |
| --- | --- | --- |

Open issue LPP-12: Details of IE NR-AI-ML-PositioningProvideCapabilities

Issue description:

The IE NR-AI-ML-PositioningProvideCapabilities indicates support for NR AI/ML direct positioning (i.e., Case 1) and provides the NR AI/ML positioning capabilities ("applicable functionality") to the location server.

RAN2#126: - The LPP Capability Transfer procedures (RequestCapabilities/ProvideCapabilities messages) are used to indicate supported AI/ML positioning capabilities. FFS how to handle dynamic capabilities, depending on further RAN1 progress and understanding of the functionality. RAN2#128: - For POS Case 1, RAN2 confirm that the existing unsolicited UE capability report mechanism in LPP can support UE to report the applicable functionality in both “proactive” and “reactive” as a baseline. - Proactive case: When the applicability change, UE can send an unsolicited LPP ProvideCapabilities message to LMF. - Reactive case: If the applicability changes based on the configuration in LPP ProvideAssistanceData message in step 3, UE can send an unsolicited LPP ProvideCapabilities message to LMF. Configuration details are FFS. RAN2#129bis: - UE reports the applicable functionality to the LMF by the LPP provide capabilities message without any additional LMF control.

Status in running CR:

The IE NR-AI-ML-PositioningProvideCapabilities currently includes the common LPP capabilities and assistance data supported (per NR-AI-ML-PositioningProvideAssistanceData (which are the same as in NR-DL-TDOA-ProvideAssistanceData)).

DL-PRS capabilities depend on further RAN1 agreements and are not included yet (see #LPP-2/3/4).

Captured as "Editor's Note" in IE NR-AI-ML-PositioningProvideCapabilities.

Proposed resolution:

Given that all assistance information for UE-based DL-TDOA (other than info #7) are applicable to NR AI/ML positioning Case1, the NR-AI-ML-PositioningProvideCapabilities should include (at least) all the capabilities currently defined for UE-based DL-TDOA.

Proposal 12:	The IE NR-AI-ML-PositioningProvideCapabilities contains (at least) all capabilities from UE-based DL-TDOA as starting point, except the capability related to DL-PRS processing (see #LPP-2/3/4). This will be revised when additional RAN1 input is available. The current Editor's Note is kept for now.

Companies are invited to provide feedback regarding the above open issue and proposed resolution:

| Company | Agree to proposal? | Other comments |
| --- | --- | --- |

Open issue LPP-13: Location server error causes

Issue description:

The IE NR-AI-ML-Positioning-Error is used by the location server or target device to provide NR AI/ML positioning error reasons to the target device or location server, respectively.

RAN2#128: - As a baseline, if the AIML based positioning method becomes non-applicable when LMF requests UE location estimation, UE cannot perform the AIML based positioning, and reply with LPP Providelocationinformation message with error cause. FFS if other fallback options are considered. RAN2#129 - The content of error cause is discussed while drafting stage3 CRs.

Status in running CR:

The IE NR-DL-AI-ML-LocationServerErrorCauses currently includes the error causes from DL-TDOA.

Captured as "Editor's Note" in IE NR-DL-AI-ML-LocationServerErrorCauses.

Proposed resolution:

Companies to provide contributions to the following meeting on error causes for IE NR-DL-AI-ML-LocationServerErrorCauses (the proposed error causes should be accompanied by some justification (e.g., why needed? Expected receiver behaviour? etc.)

Open issue LPP-14: Target device error causes

Issue description:

The IE NR-AI-ML-Positioning-Error is used by the location server or target device to provide NR AI/ML positioning error reasons to the target device or location server, respectively.

RAN2#128: - As a baseline, if the AIML based positioning method becomes non-applicable when LMF requests UE location estimation, UE cannot perform the AIML based positioning, and reply with LPP Providelocationinformation message with error cause. FFS if other fallback options are considered. RAN2#129 - The content of error cause is discussed while drafting stage3 CRs.

Status in running CR:

The IE NR-DL-AI-ML-TargetDeviceErrorCauses currently includes the error causes from DL-TDOA.

Captured as "Editor's Note" in IE NR-DL-AI-ML-TargetDeviceErrorCauses.

Proposed resolution:

Companies to provide contributions to the following meeting on error causes for IE NR-DL-AI-ML-TargetDeviceErrorCauses (the proposed error causes should be accompanied by some justification (e.g., why needed? Expected receiver behaviour? etc.)

Other identified open issues

Companies are invited to describe any other identified open issues not currently included within this document.

| Company | Other identified open issues? (please describe) |
| --- | --- |
| CATT | Suggested open issue: whether positioning Integrity is supported for AI/ML positioning method (i.e., Case 1)? Per RAN1 agreement: "For AI/ML based positioning Case 1, all assistance information from legacy UE-based DL-TDOA, other than info #7, can be provided from LMF to UE. “ The RAN1 agreement already includes info #15 for positioning integrity: Data facilitating the integrity results determination of the calculated location So it seems that RAN1 already agreed positioning Integrity is supported for AI/ML positioning method, but there is still no RAN2 discussion on this topic. And according to the comments received in [POST129bis][014][AI PHY] 38.305 Running CR, companies have different views on this understanding. |

Conclusions

<To be filled after companies have provided feedback to the proposed resolutions for simple issues only. Please include the number of supporting companies (e.g., 18/20]) in brackets within the proposal>

The following proposals have been provided based on feedback to the above document:

[Proposals for easy agreement]

<List all proposals with consensus and/or may be easily agreed based on Rapporteur’s opinion>

[Proposals for discussion]

<List all proposals which will likely require further online/offline discussion to resolve>

References

R2-250xxxx_(Running CR 37355-i40)_v01.docx: "Running CR for AI/ML Positioning Accuracy Enhancements";
provided in the 'docs' sub-folder for this email discussion.