3GPP TSG-RAN WG2 Meeting #130	R2-250xxxx

St Julian’s, Malta, May 19 - 23, 2025

Agenda item:	8.1.1

Source: 	Qualcomm Incorporated (Rapporteur)

Title: 	Open Issues List for Running CR to TS 37.355 (LPP)

Document for:		Discussion

# 1.	Introduction

This document summarizes the open issues for the running CR to TS 37.355 (LPP) to support the AI/ML Positioning Accuracy Enhancements.

# 2.	Open Issues List

| # | Clause/IE | Brief Description | Description/Comment | Status |
| --- | --- | --- | --- | --- |
| 1 | 6.4.3 NR-DL-PRS-Info | Applicability of dl-PRS-ResourcePrioritySubset | The IE NR-DL-PRS-Info also includes method specific information (for DL-AoD). If DL-PRS assistance data are needed for AI/ML positioning, it needs to be clarified that the dl-PRS-ResourcePrioritySubset is not applicable to AI/ML positioning. | Open |
| 2 | 6.4.3 NR-DL-PRS-ProcessingCapability | Applicability of IE NR-DL-PRS-ProcessingCapability | In the case of capabilities for multiple NR positioning methods are provided, the IE NR-DL-PRS-ProcessingCapability applies across the NR positioning methods and the target device shall indicate the same values for the capabilities in IEs NR-DL-TDOA-ProvideCapabilities, NR-DL-AoD-ProvideCapabilities, and NR-Multi-RTT-ProvideCapabilities. Most capabilities are primarily for UE-assisted mode. If the IE NR-DL-PRS-ProcessingCapability is applicable to AI/ML positioning, it needs to be clarified whether the above also applies to NR-AI-ML-PositioningProvideCapabilities. | Open |
| 3 | 6.4.3 NR-DL-PRS-QCL-ProcessingCapability | Applicability of NR-DL-PRS-QCL-ProcessingCapability | In the case of capabilities for multiple NR positioning methods are provided, the IE NR-DL-PRS-QCL-ProcessingCapability applies across the NR positioning methods and the target device shall indicate the same values for the capabilities in IEs NR-DL-TDOA-ProvideCapabilities, NR-DL-AoD-ProvideCapabilities, and NR-Multi-RTT-ProvideCapabilities. Most capabilities are primarily for UE-assisted mode. If the IE NR-DL-PRS-QCL-ProcessingCapability is applicable to AI/ML positioning, it needs to be clarified whether the above also applies to NR-AI-ML-PositioningProvideCapabilities. | Open |
| 4 | 6.4.3 NR-DL-PRS-ResourcesCapability | Applicability of NR-DL-PRS-ResourcesCapability | The IE NR-DL-PRS-ResourcesCapability defines the DL-PRS Resources capability for each positioning method. However, most capabilities are primarily for UE-assisted mode. Whether and which DL-PRS Resources capabilities are needed for UE-based direct AI/ML positioning depends on RAN1. | Open |
| 5 | 6.4.3 NR-On-Demand-DL-PRS-Configurations-Selected-IndexList 6.5.10.1 NR-DL-TDOA-ProvideAssistanceData 6.5.11.1 NR-DL-AoD-ProvideAssistanceData | Applicability of NR-On-Demand-DL-PRS-Configurations-Selected-IndexList | In the case of available on-demand DL-PRS configurations for multiple NR positioning methods are provided, the NR-On-Demand-DL-PRS-Configurations shall be present in only one of NR-Multi-RTT-ProvideAssistanceData, NR-DL-AoD-ProvideAssistanceData, or NR-DL-TDOA-ProvideAssistanceData. If on-demand PRS is applicable to AI/ML positioning, it needs to be clarified whether the above also applies to NR-AI-ML-PositioningProvideAssistanceData. | Open |
| 6 | 6.4.3 NR-PRU-DL-Info | Applicability of NR-PRU-DL-Info | Currently, the IE NR-PRU-DL-Info is primarily used for NR carrier phase positioning. If the IE NR-PRU-DL-Info is also applicable to AI/ML positioning, the IE description needs to be generalized. | Open |
| 7 | 6.4.3 NR-SelectedDL-PRS-IndexList 6.5.10.1 NR-DL-TDOA-ProvideAssistanceData 6.5.11.1 NR-DL-AoD-ProvideAssistanceData | Applicability of NR-SelectedDL-PRS-IndexList | In the case of assistance data for multiple NR positioning methods are provided, the IE NR-DL-PRS-AssistanceData shall be present in only one of NR-Multi-RTT-ProvideAssistanceData, NR-DL-AoD-ProvideAssistanceData, or NR-DL-TDOA-ProvideAssistanceData. If the IE NR-DL-PRS-AssistanceData is applicable to AI/ML positioning, it needs to be clarified whether the above also applies to NR-AI-ML-PositioningProvideAssistanceData. | Open |
| 8 | 6.5.13.1 (new) NR-AI-ML-PositioningProvideAssistanceData | Details of IE NR-AI-ML-PositioningProvideAssistanceData | May be similar to DL-TDOA assistance data per RAN1 agreement from RAN1#119: "For AI/ML based positioning Case 1, all assistance information from legacy UE-based DL-TDOA, other than info #7, can be provided from LMF to UE." Note: Info#7 is "Geographical coordinates of the TRPs served by the gNB (include a transmission reference location for each DL-PRS Resource ID, reference location for the transmitting antenna of the reference TRP, relative locations for transmitting antennas of other TRPs)" | Open |
| 9 | 6.5.13.2 (new) NR-AI-ML-PositioningRequestAssistanceData | Details of IE NR-AI-ML-PositioningRequestAssistanceData | Will be based on IE NR-AI-ML-PositioningProvideAssistanceData | Open |
| 10 | 6.5.13.3 (new) NR-AI-ML-PositioningProvideLocationInformation | Details of NR-AI-ML-PositioningProvideLocationInformation | The main issue here is whether batch reporting is applicable to AI/ML positioning. | Open |
| 11 | 6.5.13.5 (new) NR-AI-ML-PositioningRequestLocationInformation | Details of NR-AI-ML-PositioningRequestLocationInformation |  | Open |
| 12 | 6.5.13.6 (new) NR-AI-ML-PositioningProvideCapabilities | Details of NR-AI-ML-PositioningProvideCapabilities | May be similar to DL-TDOA per RAN1 agreement from RAN1#119: "For AI/ML based positioning Case 1, all assistance information from legacy UE-based DL-TDOA, other than info #7, can be provided from LMF to UE." DL-PRS capabilities (if any) depends on RAN1 (see also issue #2, #3, #4). | Open |
| 13 | 6.5.13.8 (new) NR-DL-AI-ML-LocationServerErrorCauses | Location server error causes | Current list is based on DL-TDOA. | Open |
| 14 | 6.5.13.8 (new) NR-DL-AI-ML-TargetDeviceErrorCauses | Target device error causes | Current list is based on DL-TDOA. | Open |