3GPP TSG-RAN WG2 #130	                                  R2-250xxxx

# St. Julians, Malta, May 19 – May 23, 2025

Agenda item:	8.7.1 (NR_XR_Ph3-Core)

Source:	LG Electronics

Title: 	Discussion of [POST129bis][504][XR] PDCP running CR (LGE)

Document for:	Report

# 1.	Introduction

This document summarizes the discussion of the following offline discussion.

[POST129bis][504][XR] PDCP running CR and open issues (LGE)

Scope:

- Update and review the CR

- List open issues related to the CR

Intended outcome:

- Running CR for endorsement in the next meeting

- List of open issues for discussion at the next meeting

Deadline:  April 29, 10:00 UTC

# 2.	Contact information

| Company | Name | E-mail |
| --- | --- | --- |
| LG Electronics | SeungJune Yi | seungjune.yi@lge.com |
| OPPO | Zhe Fu | fuzhe@OPPO.com |
| Huawei, HiSilicon | Seau Sian Lim | seau.sian.lim@huawei.com |
| ZTE | Eswar Kalyan Vutukuri | eswar.vutukuri@zte.com.cn |
| Futurewei | Yunsong Yang | yyang1@futurewei.com |
| vivo | Chenli | Chenli5g@vivo.com |
| Nokia | Sunyoung Lee | sunyoung.lee@nokia.com |

# 3.	Comments to the PDCP running CR v01

Companies are invited to list their comments on v01, using comment identifier (company ID and number), e.g. LGE001. The rapporteur will provide update based on the comments in proper time.

| Comment identifier | Section | Comments and/or change suggestions | Rapporteur resolution |
| --- | --- | --- | --- |
| OPPO001 | 3.1 | We understand that the Rapporteur intends to resolve the COUNT issue mentioned by companies in the last meeting. However, we think the following text somehow depends on when the UE assigns SN. Note that the non-delay-reporting data ahead of delay-reporting data can include both the data with low importance associated with the same channel and the data associated with other channel with higher LCH priority. Non-delay-reporting PDCP SDU: a non-delay-reporting PDCP SDU associated with the i:th dsr-ReportingThreshold is a PDCP SDU that will be transmitted prior to the PDCP SDU with the largest COUNT value among the delay-reporting PDCP SDUs associated with the i:th dsr-ReportingThreshold. Thus, we suggest the text below for a more generic description of Non-delay-reporting PDCP SDU: Non-delay-reporting PDCP SDU: a non-delay-reporting PDCP SDU associated with the i:th dsr-ReportingThreshold is a PDCP SDU that will be transmitted prior to any one of the delay-reporting PDCP SDUs associated with the i:th dsr-ReportingThreshold. |  |
| HW001 | 3.1 | Similar comment to Opp001. We are just wondering whether we need to keep the ‘largest COUNT’ since we are not comparing with another COUNT value of the PDCP SDU in the previous version. We think we can simplifiy it as follow : Non-delay-reporting PDCP SDU: a non-delay-reporting PDCP SDU associated with the i:th dsr-ReportingThreshold is a PDCP SDU that will be transmitted prior to each of the PDCP SDU with the largest COUNT value among the delay-reporting PDCP SDUs associated with the i:th dsr-ReportingThreshold. Oppo suggestion is also fine with us. |  |
| HW002 | 5.x | On the Editor’s Note, we think this is not needed as we think that it does not impact the triggering on PDCP, as only RLC knows whether all PDUs within the PDU set has been transmitted. Basically, Tx PDCP entity can indicate the SDU that requires auto-retx or polling. But whether to perform the auto-retx or polling can be left to RLC layer to decide, including whether to handle PDU-set based processing if pduSet-Discard is enabled. Hence the Editor’s Note here can be removed and will comment |  |
| Z001 | 5.16.1 | When stopReTxObsoleteSDU is set to enabled, and indicated from upper layer (e.g. PDCP) to discard a particular RLC SDU, the RLC SDU or SDU segment will not be transmitted or retransmitted, and the peer PDCP entity cannot receive the PDCP SDU. In this case, the discarded PDCP SDU should be included in the PDCP SN gap report. So, suggest to make the following change: ### 5.16.1 Transmit operation For UM DRBs and AM DRBs configured by upper layers to send a PDCP SN gap report in the uplink (sn-GapReport in TS 38.331 [3]), the transmitting PDCP entity shall trigger a PDCP SN gap report when: - the PDCP SDU(s) are discarded as specified in clause 5.3; and - there is at least one stored PDCP SDU(s) which is associated with a COUNT value larger than the COUNT value associated to the discarded PDCP SDU(s); and - stopReTxObsoleteSDU is not set and the discarded PDCP SDU(s) have not been submitted by RLC to lower layers, or stopReTxObsoleteSDU is set to enabled. |  |
| FW001 | 5.x | Theoretically speaking, for both auto retx and polling, the “if” condition should include that the corresponding PDCP Data PDU has already been submitted to lower layers, as below: if a PDCP SDU for which the remaining time till discardTimer expiry becomes less than the [AutoRetxThresshold] and the corresponding PDCP Data PDU has already been submitted to lower layers: if a PDCP SDU for which the remaining time till discardTimer expiry becomes less than the [AutoRetxThresshold] and the corresponding PDCP Data PDU has already been submitted to lower layers: |  |
| FW002 | 5.x | There are two issues in “indicate the trigger of RLC autonomous retransmission for the corresponding PDCP Data PDU to the associated RLC entity(-ies).” - PDCP entity merely indicates that condition for remaining-time-based RLC autonomous retransmission has been met. It is up to the RLC entity to decide whether to trigger the RLC autonomous retransmission. - If “associated RLC entity(-ies)” means RLC entity(-ies) associated with the particular PDCP Data PDU, it is fine. However, if “associated RLC entity(-ies)” means the RLC entities associated with the PDCP entity, they may not have all received the PDCP Data PDU from the PDCP entity, e.g. in the cases of split bearer or DAPS bearer. There is no need in the specification to mandate the PDCP entity to indicate to all associated RLC entities. Implementation can choose to indicate to all RLC entities associated with the PDCP entity or only to those that the corresponding PDCP Data PDU had previously been submitted to (in the former case, each associated RLC entity ignores the indication if the PDCP SN is not recognized). Besides, in the case of SL, the PDCP entity is directly associated with SRAP entity. So, the indication may be indicated to the SRAP entity and through it to the RLC entity. Suggest using similar language as the discard indication, i.e., “indicate to lower layers”, to avoid all these problems. Therefore, suggest the following changes: indicate to lower layers that condition for remaining-time-based RLC autonomous retransmission has been met . Similar issues for polling. Hence, suggest the following changes: indicate to lower layers that condition for remaining-time-based RLC polling has been met . |  |
| FW003 | 5.x | We agree with HW002 that the Editor’s Note is not needed. |  |
| V001 | 5.15 | Consider to change “are not considered as delay-reporting PDCP data volume…” to “have not been considered as delay-reporting PDCP data volume..” in several places in 5.15 |  |
| V002 | 5.15 | In the note, the current wording means when the the delay-reporting PDCP SDU changes its associated dsr-ReportingThreshold, the PDCP will provide a delay reporting indication. But the truth is the indication should be an update. Suggest to change it as: “The transmitting PDCP entity provides an updated delay-reporting indication for the PDCP Data PDU to lower layers when the delay-reporting PDCP SDU changes its associated dsr-ReportingThreshold.” |  |
| V003 | 5.x | Similar comment as HW002. The Editor’s Note can be removed, because PDCP SDU for which remaining time less than the threshold will be indicated to RLC, and the RLC will decide whether to perform auto retransmission or polling enh., no matter whether pdu-SetDiscard is configured or not. |  |
| N001 | 5.x | In the trigging conditions, ‘if’ sentence has only the subject and is missing a verb, while the prerequisite that the corresponding PDCP Data PDU has been delivered to the lower layer appears to be missing from the condition. It would be good to align the writing style with that used for specifying the existing delay-critical indication. Our suggestion is, for instance: For the purpose of RLC autonomous retransmission, for the PDCP Data PDU already submitted to lower layers, the transmitting PDCP entity shall indicate [the trigger of RLC autonomous retransmission] to the associated RLC entity(-ies) when: - for the corresponding PDCP SDU, the remaining time till discardTimer expiry becomes less than the [AutoRetxThreshold]. |  |
| N002 | 5.x | Regarding FW002, we also recognize that indicating whether the triggering condition is met in PDCP would be sufficient. |  |

# 4.	Comments to the PDCP running CR v02

| Comment identifier | Section | Comments and/or change suggestions | Rapporteur resolution |
| --- | --- | --- | --- |

# 5.	Comments to the PDCP running CR v03

| Comment identifier | Section | Comments and/or change suggestions | Rapporteur resolution |
| --- | --- | --- | --- |

# 6.	Open issues

The following editor’s NOTE have been kept in the current running CR

- Issue 1: It is FFS which delay-reporting PDCP data volume shall consider PDCP Control PDUs, the PDCP SDUs to be retransmitted, and the PDCP Data PDUs to be retransmitted.

- Issue 2: It is FFS for which PDCP SDU the transmitting PDCP entity shall trigger RLC autonomous retransmission and polling, if pdu-SetDiscard is configured.

In addition, following open issues are identified during e-mail discussion.

…