3GPP TSG RAN WG2 Meeting 129Bis		R2-25xxxxx

Wuhan, China, 7 - 11 April, 2025

Title:	Draft LS on LP-WUS in RRC_CONNECTED

Response to:	-

Release:	Rel-19

Work Item:	NR_LPWUS-Core

Source:	InterDigital (to be RAN WG2)

To:	RAN WG1

Cc:	-

Contact person:	Samuli Turtinen

samuli.turtinen@interdigital.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	-

1	Overall description

1.1	Short DRX

RAN2 discussed the Short DRX cycle applicability with LP-WUS in RRC_CONNECTED mode and made the following agreement and a working assumption:

For Option 1-1, the UE does not monitor LP-WUS when Short DRX cycle is used.

Working assumption: For option 1-2, it is up to network configuring short DRX cycle with LP-WUS. The UE monitors LP-WUS outside the Active Time regardless of if Short DRX cycle or Long DRX cycle is used.

1.2	LP-WUS monitoring

RAN2 discussed whether there are any scenarios on when the UE would not be able to monitor LP-WUS and made the following working assumption:

Working assumption for the case of potential collision (if any): In Option 1-1, when the UE is not able to monitor the LP-WUS occasion(s) the UE should start the drx-OnDurationTimer (as if LP-WUS was detected). FFS for Option 1-2.

Based on this WA, RAN2 would like to ask RAN1:

1. Whether there are any cases/scenarios on when the UE could not monitor LP-WUS?

2. Can the UE monitor LR and MR simultaneously?

1.3	UE time offset preference

RAN2 agreed that if configured, the UE can signal a preferred time offset via UAI (UE Assistance Information) signalling, and RAN2 would like to know whether there were any further conclusions in RAN1 on what information the preferred time offset via UAI should provide?

2	Actions

To RAN WG1

ACTION: 	RAN2 respectfully asks RAN1 to take the above agreements into account and provide responses to questions asked in sections 1.2 and 1.3.

3	Dates of next TSG RAN WG2 meetings

RAN2#130	2025-05-19 – 2025-05-23		Malta, Malta

RAN2#131	2025-08-25 – 2025-08-29		Bengaluru, India