3GPP TSG-RAN WG2 #131	R2-25xxxxx

Bengaluru, India, 25th – 29th August, 2025

Agenda Item:	8.1.x

Source:	Ericsson, ZTE

Title:	Report of email discussion [POST130][031][AI PHY] NW side data collection

Document for:	Discussion

# Introduction

This paper summarizes the following email discussion:

[POST130][031][AI PHY] NW side data collection (Ericsson/ZTE)

Intended outcome: provide two TP(s) for data logging and configuration in RRC on how to capture this in a simple way to RAN2.  Discuss impacts to RAN1 for each solution and RAN3.

Deadline:  long

The deadline for providing comments is 8 August 2025, 10:00 UTC.

The rapporteur will provide two TPs for two solutions for data logging and configuration in RRC, as well as a summary of foreseen RAN1 and RAN3 impacts for each solution, based on the outcome of this email discussion.

Companies providing input to this email discussion are requested to leave contact information below.

| Company | Name | Email Address |
| --- | --- | --- |

# Discussion

RAN2 has made the following agreements regarding logging and configuration for NW side data collection, that are relevant to this email discussion:

From RAN2#130: - „As a starting point, the data logging is captured in RRC specs.“ „Data is collected on per data logging configuration basis and UE indicates data logging configuration ID. An indication of the “gap” is needed. “Gap” is time interval larger than the configured logging periodicity. FFS if timestamp and relative time stamp for each group is needed per “group”.“ „The UE should report the CGI of the serving cell whenever feasible. If CGI is unavailable, the UE shall log PCI-ARFCN as a fallback.“ From RAN2#129bis: „The measurement configuration of AI/ML data collection can configure measurements for multiple sets of resources and use cases (e.g. BM, Mobility, etc)“ From RAN2#129: „Support the use of L3 measurement event triggered (i.e. L3 serving cell measurements becoming worse/better than a threshold for TTT) to determine whether the UE performs logging or not. L1 measurement event triggered will not be supported. FFS what to log“ From RAN2#127bis: „For data collection for both NW-sided/UE sided BM model training, at least L1-RSRPs and/or beam-IDs needs to be collected by UE. FFS if other data needs to be collected based on RAN1 progress“

Based on the agreements above, two approaches were proposed in RAN2#130 for introducing the logging configuration for the beam management use case in RRC:

- The logging configuration is introduced within the L1 CSI measurement framework, e.g. as a new list of configurations under CSI-MeasConfig, cf. Figure 1 and [1].

- The logging configuration is introduced in a new L3 measurement framework, at the same level as MeasConfig and CellGroupConfig, cf. Figure 2 and [2].

The corresponding TPs for these two approaches are provided as a complement to this document and are based on the RRC changes from the endorsed running CR [R2-2504349], where the additional changes for the two approaches in this email discussion are marked with tracked changes (from Ericsson in the TP for approach 1 and from ZTE in the TP for approach 2).

The two TPs are intended to indicate the main directions of the changes and not to provide the final wording for the procedural text or the final ASN.1. Therefore, companies are kindly asked to focus on the major logging procedures and logging configuration content. Details like IE/parameter names will be revised by the RRC running CR rapporteur when merging the final TP with the running CR and thus do not need to be addressed in this email discussion.

Figure 1 RRC L1 logging configuration structure for approach (1).

Figure 2 RRC L3 logging configuration structure for approach (2).

## 2.1 Content of TPs for RRC

In this section we discuss the contents of the two TPs for RRC.

The TP for approach (1) captures the logging procedures in RRC in a new clause 5.5c, similarly as legacy logging in clause 5.5a. The TP for approach (2) captures the logging procedures in RRC in a new clause 5.5x.

###### Q1-1: For approach (1), from RRC perspective, do you agree that it is sufficient to capture the logging procedures in the new clause 5.5c? Please comment if you think that the logging procedures should be moved elsewhere in the RRC specs or if you think that something is missing in the RRC procedures in the TP.

| Company | Yes/No | Comment |
| --- | --- | --- |

###### Q1-2: For approach (2), from RRC perspective, do you agree that it is sufficient to capture the logging procedures in the new clause 5.5x? Please comment if you think that the logging procedures should be moved elsewhere in the RRC specs or if you think that something is missing in the procedures in the TP.

| Company | Yes/No | Comment |
| --- | --- | --- |

In approach (1), for capturing event-triggered logging based on L3 measurements (i.e. L3 serving cell measurements becoming worse/better than a threshold for TTT), the threshold and timeToTrigger were added in the logging configuration under CSI-MeasConfig. Additionally, it was captured how to evaluate the entering and leaving conditions for the event in:

- changes to clauses 5.5.4.2 and 5.5.4.3 and field description, or alternatively,

- only in field description for eventTriggeredConfig in CSI-LoggedMeasurementConfig (similarly to the field description of eventType in the LoggedMeasurementConfiguration message for legacy logging).

###### Q2-1: For approach (1), from RRC perspective, do you agree that, to configure the event-triggered logging based on L3 measurements, it is sufficient to add threshold and timeToTrigger in the logging configuration under CSI-MeasConfig? If no, please explain the reason and where the threshold and timeToTrigger should be included. Please comment also on whether you prefer option a) or b) for capturing the event evaluation.

| Company | Yes/No | Preferred option for capturing event evaluation: a)/b) | Comment |
| --- | --- | --- | --- |

In approach (2), for capturing the L3 measurement event that triggers logging (i.e. L3 serving cell measurements becoming worse/better than a threshold for TTT), two new events N1 and N2 were added in clauses 5.5.4.x and 5.5.4.y, respectively, and in the logging configuration (in eventTriggeredLogging, in BM-DataLoggingConfig).

###### Q2-2: For approach (2), do you agree that, to configure the event-triggered logging based on L3 measurements, it is sufficient to define and configure the new events N1 and N2 as in the TP? If no, what do you think would be missing in the RRC specs?

| Company | Yes/No | Comment |
| --- | --- | --- |

RAN2#129 agreed explicitly that event-based logging is based on a threshold and TTT. The hysteresis is another typical parameter for legacy events, but there is no explicit RAN2 agreement to capture it for event-based logging for NW-side data collection.

###### Q3: For both approaches, do you agree that the hysteresis should be configured and used (alongside threshold and timeToTrigger) for event-triggered logging? If no, please explain why.

| Company | Yes/No | Comment |
| --- | --- | --- |

###### Q4: For approaches (1) and (2), are there any other aspects that you think are missing from any of the two TPs? If yes, please explain what you think is missing.

| Company | TP for approach (1) Yes/No | TP for approach (2) Yes/No | Comment |
| --- | --- | --- | --- |

## 2.2 Impacts on RAN1

In both approaches for sending the logging configuration for beam management, upon receiving a logging configuration, the UE needs to log CSI related measurements (L1-RSRP and beam index) for the serving cell, without sending any measurements at L1.

###### Q5: For approaches (1) and (2), do you think there may be RAN1 impact (e.g. in TS 38.214) for ensuring that the UE performs measurements according to the logging configuration and does not trigger L1 reports? If yes, please comment on what RAN1 impacts you foresee.

| Company | Approach (1) Yes/No | Approach (2) Yes/No | Comment |
| --- | --- | --- | --- |

## 2.3 Impacts on RAN3

The logging configuration (for both approaches) needs to contain:

- references to the resources to be measured for logging (for both periodic and event-triggered logging), which for the beam management use case are CSI-ResourceConfigId(s); and

- event-triggered logging configuration based on L3 measurements, including threshold and TTT.

Furthermore, with approach (1) the logging configuration would be generated by the gNB-DU under CSI-MeasConfig, whereas with approach (2) the logging configuration would be generated by the gNB-CU at L3.

###### Q6: For approaches (1) and (2), do you think there may be RAN3 impact? If the answer is yes, please describe. Possible aspects to consider are: 1) CU-DU interaction for configuring the event; 2) CU-DU interaction for configuring the measurement resources; 3) CU-DU interaction for retrieving logged data, 4) CU-DU interaction for de-configuring logging configurations upon low power state indication, etc. In the comments, companies also can provide other potential RAN3 impacts in addition to above mentioned.

Note: In the rapporteur’s view, RAN3 impacts are present also in other components of AIML for PHY (besides NW-side data collection).

| Company | Approach (1) Yes/No | Approach (2) Yes/No | Comment |
| --- | --- | --- | --- |

## 2.4 Final questions

###### Q7: Among approach (1) and (2), considering the complexities and impacts of the approaches, which one is acceptable/not acceptable?

| Company | Acceptable (approach 1/2) | Not acceptable (approach 1/2) |
| --- | --- | --- |

# Conclusion

Based on the discussion during the offline meeting, captured in the previous section, we propose the following:

- <TBD>

# References

- R2-2504644, Ericsson, Nokia, Huawei, T-Mobile USA, BT Plc., “Discussion on NW-side data collection framework”, 3GPP TSG-RAN WG2 #130, Malta, May, 2025.

- R2-2503849, ZTE Corporation, Apple, MediaTek, Samsung, OPPO, Lenovo, Xiaomi, CMCC, China Telecom, vivo, NTT DOCOMO, Sanechips, “Discussion On the NW Side Data Collection RRC Framework”, 3GPP TSG RAN2 Meeting #130, Malta, May, 2025.