3GPP TSG-RAN WG2 Meeting #130	R2-250xxxx

# St Julian’s, Malta, May 19th – 23rd, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.304 | CR | draftCR | rev | - | Current version: | 18.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Running 38.304 CR for network energy saving |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Apple (Rapporteur) |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | Netw_Energy_NR_enh-Core |  |  |  |  |  | Date: |  |  | 2025-04-18 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Introduction of Rel-19 network energy saving to TS 38.304. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Change is introduced to support the following objectives: - On-demand SIB1 for idle/inactive UEs - Adaptation of common channels/signals - Only paging adaptation is introduced for now - Potential spec impact on RACH adaptation on reception of paging DCI - No specfication impacts is foreseen for SSB adaptation The table below presents the agreements and if/how they have been captured. And the following color code is used to categorize agreements interms of if/how they have been captured. Fully implemented Already specified in 38.304 Not/partially implemented but additional agreements/FFSs needed before conclusion Doesn’t impact 38.304 It will be removed when the CR is finalized. On-demand SIB1 for idle/inactive UEs. RAN2#125bis. At least RAN2 starts scenario 1a. Other scenarios are not excluded. - Scenario 1a: Cell A SIB assisted intra-cell WUS. And WUS and SIB1 is sent to/from NES cell. with below potential RAN2 impacts: - Add WUS configuration in SIB of cell A. - Cell reselection from cell A to NES cell, including trigger condition and cell barring changes. - Whether allow camping, paging and SIB update in NES cell. - Cell reselection from NES cell to cell A or normal cell.; Captured in new section X. EN 4: details of UL WUS configuration and whether/how to capture the details.. Contents of UL WUS - RAN2 assumes that RACH procedure is reused for UE to request on-demand SIB1. - UL WUS configuration includes at least below information: - RACH configuration - A UE needs to know a UL WUS configuration to request SIB1 of which cell.; Captured in new section X. EN 4: details of UL WUS configuration and whether/how to capture the details.. On-demand SIB1 acquisition procedure - Existing Msg 1 based on-demand procedure is reused for on-demand SIB1 acquisition procedure. FFS on Msg 3. FFS if / when the UE monitors the OD-SIB1 upon reception of RAR. FFS:T whether introduce specified UE behavior if RACH failure of OD-SIB1 request.; N/A. The UE first should acquire valid SIB1 (e.g. via SIB1 request) for camping to NES cell (if the UE knows the cell doesn’t broadcast SIB1 and supports on-demand SIB1).; N/A. RAN2#126. Study on-demand SIB1 provisioning for NES Cell(s) in versions of Scenario 1a with multiple Cells A and/or NES Cells: - More than one Cell A may provide configuration for the same NES cell. - The same Cell A may assist more than one NES Cells.; N/A. RRC release message assisted intra-cell WUS can be discussed as option of signaling details in stage 3.; N/A. Can use the PCI and frequency of a NES Cell to associate the UL WUS configuration with a NES Cell.; N/A. For Message 1 based on-demand SIB1 request, the on-demand SI request configuration that currently included in SIB1 may be used as the design baseline.; N/A. Cell A’s SIB can be used to configure on-demand SIB1 related configuration for neighbour NES cells, e.g., via new SIB or the existing SIB.; N/A (updated by agreement of SIB-X). If the UE chooses the NES cell using legacy intra-F/inter-F cell re-selection procedure (as baseline), the UE triggers WUS transmission.; Captured in new Section X. After UE successfully receives OD-SIB1 for that NES Cell and if it is a suitable cell, UE camps in the NES Cell “similar” to a legacy Cell.; Captured in new Section X. RAN2 not to support on-demand SIB1 request that is combined with an initial access to perform RRC connection establishment/resume on the NES cell.; N/A. NW need to bar the legacy UE from accessing the on-demand SIB1 cell (e.g. based on the existing barring mechanism).; Rely on existing barring of 38.304.. How to avoid/deprioritize the legacy UE camping at the Cell A attempting to switch to the NES Cell (but allowing the R19 NES UE to do that).; N/A. If the NES UE is unable to acquire the SIB1 of NES Cell before UE initiates OD-SIB1 procedure, it will not consider it as barred at that moment. R19 NES UE bars the cell if the UE fails to acquire SIB1 via on-demand SIB1 for NES cell.; Captured in new section 5.3.1X.. After Rel-19 NES UEs camp in NES cell, the UE behaviour is same as the one defined as legacy normal camped state, e.g. paging reception, SIB1 update, etc.; Captured in new section 5.3.1X.. RAN2 assumes the UE is expected to receive the RAR responding to the preamble transmission for Msg1-based on-demand SIB1 procedure, as the baseline.; N/A. As baseline, upon random access procedure failure of OD-SIB1 request, UE regards OD-SIB1 can’t be acquired in the NES cell and considers it as barred. It doesn’t exclude the option to leave the determination to the UE implementation.; Captured in new section 5.3.1X.. Once the NES UE camps on the NES cell, if the UE receives SIB change notification, the UE is expected to receive SIB1 from NES cell.; Captured in new Section X (refer to Section 5.2.5).. RAN2 to wait for RAN1’s progress whether to support scenario 3.; N/A. RAN2#127. No consensus for RAN1 case 3 in RAN2.; N/A. We can rely on legacy UE behaviour to ensure UE has valid SIB1 for the cell (i.e. UE reacquire SIB1 whenever (re)selecting cell).; N/A. Further UE keeps SIB1 updated while on cell via regular SI modification procedure (confirmation of earlier RAN2 agreement).; Captured in new Section X (refer to Section 5.2.5).. Once Rel-19 NES UE camps on the NES cell, the UE expects to receive UL WUS configuration updates from the NES Cell, e.g., via legacy SI modification procedures.; Captured in new Section X (refer to SI modification of 38.331).. Msg 3 based OD-SI procedure is not supported for on-demand SIB1 request in case 2 (for requesting to the NES Cell).; N/A. Following example options to handle legacy UEs (i.e. UEs not supporting OD-SIB1) can be considered in normative work. Details and further analysis need to be further discussed in normative work. Other existing options are not excluded. - Option 1: Legacy UEs bar the OD-SIB1 cell based on cellBarred bit set to barred in MIB. - Option 2: Legacy UEs bar the OD-SIB1 cell based on no SIB1 indication via ssb-SubcarrierOffset in MIB. - Option 3: Network includes cells supporting OD-SIB1 to list of excluded cells.; N/A. RAN2 understands the NW can avoid impact on legacy RRC connected UE and R19 RRC connected UE due to on-demand SIB1 (e.g. NES cell with OD-SIB1 is measured by legacy RRC_CONNECTED UE and can be configured as its PSCell/SCells/target cell).; N/A. RAN2 conclude that on-demand SIB1 is feasible from RAN2 perspective and recommend normative work of case 2 for on-demand SIB1.; N/A. RAN2#127bis. We will inherit all agreements made during SI phase to WI phase.; N/A. A NES cell can include neighbouring NES cell’s WUS configuration.; N/A. NES cell’s WUS configuration, it is included in a new SIB (including its own WUS configuration).; Captured in Section X. Editor’s note 3: need to update IE name of SIB-X according to running RRC CR.. In on-demand SIB1 procedure, the UE considers RACH failure when PREAMBLE_TRANSMISSION_COUNTER = preambleTransMax + 1.; N/A. The MAC layer will indicate the RACH failure for SI request to upper layers. FFS: after that the UE upper layer will consider the cell as barred.; N/A. The legacy UE behaviour can be reused upon on-demand SIB1 acquisition failure, i.e., the NES UE should follow the intraFreqReselection in MIB of NES cell.; Captured in new Section X.. A cell for which SIB1 request configuration is available, can periodically broadcast SIB1.; N/A. If UE has SIB1 request configuration of a cell, UE needs to check if SIB1 is currently being broadcasted or provided on demand for that cell before requesting SIB1 of that cell.; Captured In new Section X EN 5: details on how the OD-SIB1 UE determines the target OD-SIB1 cell doesn’t broadcast SIB1.. Legacy UEs bar the OD-SIB1 cell based on no SIB1 indication in MIB e.g. via ssb-SubcarrierOffset. Detailed solution is up to RAN1. If this works, no separate barring bit for R19 NES UEs is introduced.; Rely on existing barring of 38.304.. NES UEs should be allowed to reselect to cells that are prevented from legacy UEs (e.g. by excluded cell list, reselection priorities).; N/A (updated by later agreement). RAN2 will not start the discussion on the issue when the UL WUS configuration update in Cell A is not synchronised with the UL WUS configuration update in the NES cell, unless we’re asked to do that by other WG, e.g. RAN3.; N/A. RAN2#128. Let’s wait for more RAN1 progress on Kssb discussion.; N/A. NES UE with SIB1 request configuration of a NES cell assumes that a NES cell, with SSB containing K_SSB < 24 for FR1 and K_SSB < 12 for FR2, will acquire SIB1 as in legacy.; N/A. New NES-specific reselection priority parameters for NES UEs are defined for the purpose of prioritizing/deprioritizing a NES frequency.; Captured in new Section 5.2.4.1X.. Introduce new IntraFreqExcludedCellList-NES / InterFreqExcludedCellList-NES IEs enable proper reselection behaviour of legacy and NES UEs.; Captured in new Section 5.2.4.1X.. Reuse legacy cell reselection criterion as trigger condition of OD-SIB1 acquisition. No need to specify other conditions (e.g. a new RSRP threshold).; Captured in new Section X.. The existing 1-second rule in the cell reselection criteria is still applied to the triggering condition of UL WUS transmission.; Rely on existing behavior of 38.304.. The UE considers the cell as barred after MAC indicates max number of preamble transmission for the OD-SIB1 request.; Captured in new Section 5.3.1X.. If MSG2 (ACK) is received, but UE fails to receive SIB1 then the UE may consider this cell as barred, no spec impact.; N/A. A UE bars the NES/SIB1 less cell and/or excludes it as a candidate for reselection since the UE had no corresponding UL WUS configuration, the UE would treat this cell as if cell status is “not barred” and consider it as candidate for cell reselection once it has received a UL-WUS configuration to request SIB1 for this cell.; Captured in new Section 5.3.1X.. UE considers WUS configuration only valid in directly succeeding cell reselection from cell where UE acquired WUS configuration. FFS on SI validity area rather than cell level.; Captured in new Section X.. Wait one cycle of RAN discussion (i.e. to see whether WID is updated or not to handle RRC connected UEs); N/A. Working assumption: UL-WUS configuration in RRC release is not supported.; N/A. RAN2#129. There is no need for additional barring mechanisms (in addition to the k_ssb signaling “no SIB1” indication in MIB) to handle legacy to be able to bar cell using OD-SIB1.; N/A. Specify the following UE behavior to allow the UEs in RRC_CONNECTED state to acquire OD-SIB1 when T311 is running: - When T311 is running, the UE can trigger the OD-SIB1 acquisition procedure with stored UL WUS configuration in SIB-X, if it is still valid. - The legacy cell selection criteria are reused as the trigger condition of OD-SIB1 acquisition. - The OD-SIB1 acquisition behavior is same as that of RRC_IDLE/IANCTIV UEs.; Captured in Section X. EN 4: details of UL WUS configuration and whether/how to capture the details.. The UE follows the legacy validity principle of stored SIB.; Captured in Section X.. For Rel-19 NES UE in RRC_CONNECTED, rely on the NW dedicated RRC for SIB1 delivery if searchSpaceSIB1 is not configured. It is legacy UE behavior and no spec change is expected.; N/A. SIB-x can be cell specific or area specific, as legacy.; N/A. Upon reception of RAR, the UE monitors OD-SIB1 in the window agreed by RAN1.; N/A. Proposal 5: For Rel-19 NES UE in RRC_CONNECTED, if searchSpaceSIB1 is configured and SIB1 is not broadcasting, rely on the NW implementation to deliver SIB1 as baseline. We will make conclusion on P5 next meeting.; EN 6: whether any specification impacts on the case that if searchSpaceSIB1 is configured and SIB1 is not broadcasting for an OD-SIB1 UE in RRC_CONNECTED state.. RAN2#129bis. NW ensures that the RRC connected UE has the latest SIB1 (e.g. dedicated RRC message to deliver SIB1 or not configure searchSpaceSIB1), as baseline. UE understands that the stored SIB1 is the latest SIB1.; N/A. When the cell supporting on demand SIB1 is broadcasting SIB1 (e.g. upon SIB1 request), legacy UE can camp on the cell if the legacy UE is able to acquire the broadcasted SIB1. No specification impact is foreseen.; N/A. We do not need a separate new triggering condition of OD-SIB1 acquisition.; N/A. When timer T311 is running, SIB1 acquisition triggering condition is same as legacy. No additional spec impact is foreseen.; N/A. RAN2#130. RAN2#131. Adaptation of common channels/signals. RAN2#125bis. From the UE point of view, UE will monitor one PEI/PO every paging DRX cycle, i.e. the UE doesn’t skip PO in paging DRX cycle.; Captured in Section 7.1. For adaptation of paging occasions in time domain, RAN2 to study a) bundle paging frames and b) extend the values of N to have increased interval between PFs (e.g. T/64, T/128 ...) and compensating decrease in number of PFs by increasing POs per PF.; N/A. For Paging adaptation, R2 discusses the following options on compatibility of legacy RRC_IDLE/RRC_INACTIVE UE: - Option 1: Prevent the access of legacy UE via barring; - Option 2: Separate paging resources for legacy UEs and Rel-19 NES UEs (assuming there are legacy UEs); N/A. RAN2#126. RAN2 counts on RAN1 evaluation and conclusion.; N/A. RAN2#127. Option-a) is about the bundling of PF for R19 NES UEs.; N/A. R2 observe that the option-a) and option-b) can be designed to configure the PO:s at same time position.; N/A. RAN2#127bis. Select option-b as baseline for R19 NES paging enhancement.; N/A. R2 should aim at signaling overhead minimization (e.g., Ns=8 and FFS for other larger values); N/A. Allowing legacy and R19 UEs to co-ex in the same PF/PO is possible, based on NW configuration.; Captured in Section 7.1.. Legacy UE is not barred.; N/A. Rel-19 UEs only monitor the PO(s) according to Rel-19 paging configuration.; Captured in Section 7.1. Editor’s note 1: details of Rel-19 paging configuration and whether/how to capture the details.. RAN2#128. Introduce value for Ns=8. FFS on 16.; N/A. Introduce value for N= T/32. FFS on T/64, T/128, T/256.; N/A. RAN2#129. - For N, values smaller than T/32 are not supported.; N/A. - The maximum possible value for Ns is 8.; N/A. - Introduce a separate PEI configuration.; Captured in Section 7.2. Editor’s note 2: details of Rel-19 PEI configuration and whether/how to capture the details.. - Paging adaptations are configured semi-statically and updated via system information update notification.; Captured in Section 7.2.. - A new UE capability is added for R19 NES paging enhancement, and the new capability is included in UE-RadioPagingInfo. FFS on whether we have a common capability for all NES features.; N/A. - Legacy UEs and UEs non-capable of time domain PRACH adaptation are expected to use legacy PRACH resources per legacy configuration and procedures. No barring is needed.; N/A. RAN2 starts CBRA for RACH adaptation. - RAN2 starts 4-step RACH adaptation; N/A. From R2 perspective, not apply time-domain RACH adaptation to RACH resources for MSG1-based SI request. From R2 perspective, not apply time-domain RACH adaptation to IAB RACH resources.; N/A. From R2 perspective, RACH adaptation is not modelled as RA feature(s). From R2 perspective, RACH partitioning with all the features, i.e. RedCap, SDT, and Slicing, and feature combinations, are supported for PRACH adaption in time domain.; N/A. Will follow legacy mechanism regarding how to select RACH resource.; N/A. RAN2#129bis. For the case when both pei-Config-r17 and pagingAdaptationPEI-Config-r19 are configured, R19 UE supporting paging adaption should monitor PEI according to pagingAdaptationPEI-Config-r19 while other UE should monitor PEI according to pei-Config-r17.; Captured in Section 7.2.1. For the case when pei-Config-r17 is configured and pagingAdaptationPEI-Config-r19 is absent, both R19 UE supporting paging adaption and other UE should monitor PEI according to pei-Config-r17.; N/A. Paging clustering / bundling / adaptation is not supported/applied in RRC_CONNECTED.; N/A. RAN2#130. Rel-19 excluded cell list can be configured as an empty list. No new UE behavior is introduced (i.e. the UE supporting OD-SIB1 ignores legacy excluded cell list only if NES excluded cell list is present).; Revise text in Section 5.2.4.1. Same as the other 2 agreed cases of OD-SIB1 acquisition failure, capture the failure case of OD-SIB1 window expiry in TS 38.304. RAN2 confirm no other cases need to be considered and specified in TS 38.304.; Captured in Section 5.3.1. Introduce a new optional firstPDCCH-MonitoringOccasionOfPO-r19 field parameter for Rel-19 UEs that support adaptive paging.; Captured in Section 7.1. Introduce a new optional firstPDCCH-MonitoringOccasionOfPEI-O-r19 field parameter for Rel-19 UEs that support adaptive pagin; Captured in Section 7.2.1. RAN2#131. |
| Consequences if not approved: |  | Release-19 network energy saving is not supported |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 3.1, 3.2, 5.2.4.1, 5.3.1, 7.1, 7.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS 38.300 |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS 38.331 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

First change

## 3.1	Definitions

For the purposes of the present document, the following terms and definitions apply:

2Rx XR UE: two antenna port XR UE as specified in TS 38.101-1 [15].

Acceptable Cell: A cell that satisfies certain conditions as specified in 4.5.

Allowed CAG list: A per-PLMN list of CAG Identifiers the UE is allowed to access (see TS 23.501 [10]).

Available PLMN(s): One or more PLMN(s) for which the UE has found at least one cell and read its PLMN identity(ies).

Available SNPN(s): One or more SNPN(s) for which the UE has found at least one cell and read its SNPN identity(ies).

Barred Cell: A cell a UE is not allowed to camp on.

CAG cell: A cell broadcasting at least one Closed Access Group Identifier.

Camped on a cell: UE has completed the cell selection/reselection process and has chosen a cell. The UE monitors system information and (in most cases) paging information.

Camped on any cell: UE is in idle mode and has completed the cell selection/reselection process and has chosen a cell irrespective of PLMN identity.

Closed Access Group Identifier: Identifier of a CAG within a PLMN.

Commercial Mobile Alert System: Public Warning System that delivers Warning Notifications provided by Warning Notification Providers to CMAS capable UEs.

Earth-fixed cell: An NTN cell fixed with respect to a certain geographic area on the earth all the time. It can be provisioned by beam(s) continuously covering the same geographical areas all the time (e.g., the case of GSO satellites).

Earth-moving cell: An NTN cell moving on the ground. It can be provisioned by beam(s) whose coverage area slides over the Earth surface (e.g., the case of NGSO satellites generating fixed or non-steerable beams).

eCall Only Mode: A UE configuration option that allows the UE to register at 5GC and register in IMS to perform only eCall Over IMS, and a non-emergency IMS call for test and/or terminal reconfiguration services.

EHPLMN: Any of the PLMN entries contained in the Equivalent HPLMN list TS 23.122 [9].

Equivalent PLMN list: List of PLMNs considered as equivalent by the UE for cell selection, cell reselection, and handover according to the information provided by the NAS.

Equivalent SNPN list: List of SNPNs considered as equivalent by the UE for cell selection, cell reselection, and handover according to the information provided by the NAS.

eRedCap UE: A UE with enhanced reduced capabilities as specified in clause 4.2.22 in TS 38.306 [24].

Home PLMN: A PLMN where the Mobile Country Code (MCC) and Mobile Network Code (MNC) of the PLMN identity are the same as the MCC and MNC of the IMSI.

HSDN cell: A cell that has higher priority than other cells for cell reselection for HSDN capable UE in a High-mobility state.

Mobile-IAB cell: As defined in TS 38.300 [2].

Network Identifier: Identifier of an SNPN in combination with a PLMN ID (TS 23.501 [10]).

Non-Public Network: A network deployed for non-public use, as defined in TS 22.261 [12].

Non-terrestrial network: An NG-RAN consisting of gNBs, which provides non-terrestrial NR access to UEs by means of an NTN payload embarked on an airborne or space-borne NTN vehicle and an NTN Gateway.

NR sidelink communication: AS functionality enabling at least V2X Communication as defined in TS 23.287 [16] and/or A2X Communication as defined in TS 23.256 [26], and ProSe communication (including ProSe non-Relay, UE-to-Network Relay communication and, UE-to-UE Relay communication including UE-to-UE Relay communication with integrated discovery) as defined in TS 23.304 [22], between two or more nearby UEs, using NR technology but not traversing any network node.

NR sidelink discovery: AS functionality enabling ProSe non-Relay Discovery, ProSe UE-to-Network Relay discovery and ProSe UE-to-UE Relay discovery for Proximity based Services as defined in TS 23.304 [22] between two or more nearby UEs, using NR technology but not traversing any network node.

Paging adaptation: As defined in TS 38.300 [2].

Process: A local action in the UE invoked by an RRC procedure or an RRC_IDLE or RRC_INACTIVE state procedure.

Quasi-Earth fixed cell: An NTN cell fixed with respect to a certain geographic area on the earth during a certain time duration. It can be provisioned by beam(s) covering one geographic area for a limited period and a different geographic area during another period (e.g., the case of NGSO satellites generating steerable beams).

Radio Access Technology: Type of technology used for radio access, for instance NR or E-UTRA.

Ranging/Sidelink Positioning: AS functionality enabling ranging-based services and sidelink positioning as defined in TS 23.586 [25].

RedCap UE: A UE with reduced capabilities as specified in clause 4.2.21 in TS 38.306 [24].

Registration Area: (NAS) registration area is an area in which the UE may roam without a need to perform location registration, which is a NAS procedure.

Registered PLMN: This is the PLMN on which certain Location Registration outcomes have occurred, as specified in TS 23.122 [9].

Registered SNPN: This is the SNPN on which certain Location Registration outcomes have occurred, as specified in TS 23.122 [9].

Reserved Cell: A cell on which camping is not allowed, except for particular UEs, if so indicated in the system information.

Selected PLMN: This is the PLMN that has been selected by the NAS, either manually or automatically.

Selected SNPN: This is the SNPN that has been selected by the NAS, either manually or automatically.

Serving cell: The cell on which the UE is camped.

Sidelink: UE to UE interface for V2X sidelink communication defined in TS 23.287[16].

SNPN Access Mode: Mode of operation wherein UE only selects SNPNs (as defined in TS 23.501 [10]).

SNPN identity: An identifier of an SNPN comprising of a PLMN ID and an NID combination.

Strongest cell: The cell on a particular frequency that is considered strongest according to the layer 1 cell search procedure (TS 38.213 [4], TS 38.215 [11]).

Suitable Cell: This is a cell on which a UE may camp. For NR cell, the criteria are defined in clause 4.5, for E-UTRA cell in TS 36.304 [7].

U2N Relay UE: a UE that provides functionality to support connectivity to the network for U2N Remote UE(s).

U2N Remote UE: a UE that communicates with the network via a U2N Relay UE.

U2U Relay UE: a UE that provides functionality to support connectivity for U2U Remote UE(s).

U2U Remote UE: a UE that communicates with other UE(s) via a U2U Relay UE.

V2X sidelink communication: AS functionality enabling V2X Communication as defined in TS 23.285 [17], between nearby UEs, using E-UTRA technology but not traversing any network node.

Next change

## 3.2	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1].

AS	Access Stratum

ATG	Air To Ground

CAG	Closed Access Group

CAG-ID	Closed Access Group Identifier

CMAS	Commercial Mobile Alert System

CN	Core Network

DCI	Downlink Control Information

DRX	Discontinuous Reception

DTX	Discontinuous Transmission

eDRX	Extended DRX

ETWS	Earthquake and Tsunami Warning System

E-UTRA	Evolved UMTS Terrestrial Radio Access

E-UTRAN	Evolved UMTS Terrestrial Radio Access Network

GIN	Group ID for Network selection

H-SFN	Hyper System Frame Number

HRNN	Human-Readable Network Name

HSDN	High Speed Dedicated Network

IAB	Integrated Access and Backhaul

IMSI	International Mobile Subscriber Identity

L2	Layer-2

MBS	Multicast/Broadcast Services

MBS FSAI	MBS Frequency Selection Area Identity

MCC	Mobile Country Code

MCCH	MBS Control Channel

MICO	Mobile Initiated Connection Only

MRB	MBS Radio Bearer

MTCH	MBS Traffic Channel

NAS	Non-Access Stratum

NCR	Network-Controlled Repeater

NCR-Fwd	NCR Forwarding

NCR-MT	NCR Mobile Termination

NES	Network Energy Savings

NID	Network Identifier

NPN	Non-Public Network

NR	NR Radio Access

NSAG	Network Slice AS Group

NTN	Non-Terrestrial Network

OD-SIB1             On Demand SIB1

PEI	Paging Early Indication

PEI-O	Paging Early Indication-Occasion

PH	Paging Hyperframe

PLMN	Public Land Mobile Network

PTW	Paging Time Window

RAT	Radio Access Technology

RNA	RAN-based Notification Area

RNAU	RAN-based Notification Area Update

RRC	Radio Resource Control

SDT	Small Data Transmission

SL	Sidelink

SNPN	Stand-alone Non-Public Network

TN	Terrestrial Network

TRS	Tracking Reference Signal

U2N	UE-to-Network

U2U	UE-to-UE

UAC	Unified Access Control

UE	User Equipment

UMTS	Universal Mobile Telecommunications System

VSAT	Very Small Aperture Terminal

V2X	Vehicle to Everything

Next change

#### 5.2.4.1	Reselection priorities handling

Absolute priorities of different NR frequencies or inter-RAT frequencies may be provided to the UE in the system information, in the RRCRelease message, or by inheriting from another RAT at inter-RAT cell (re)selection. In the case of system information, an NR frequency or inter-RAT frequency may be listed without providing a priority (i.e. the field cellReselectionPriority is absent for that frequency). If any fields with cellReselectionPriority or nsag-CellReselectionPriority are provided in dedicated signalling, the UE shall ignore any fields with cellReselectionPriority and nsag-CellReselectionPriority provided in system information.

When UE is in camped normally state, if it supports slice-based cell reselection and has received the network slice(s) and NSAG information from NAS to be used for cell reselection, UE shall derive reselection priorities according to clause 5.2.4.11.

NOTE 00: UE derives reselection priorities according to clause 5.2.4.11 also in case SIB16 (see TS 38.331 [3]) is not broadcast in the camped cell.

If dedicated frequency priority parameters (odsib1-CellReselectionPriority, odsib1-CellReselectionSubPriority) are provided in system information, the UE supporting OD-SIB1 ignores the cellReselectionPriority and cellReselectionSubPriority in the system information and applies the dedicated ones to determine frequency prioritization. If intraFreqODSIB1-ExcludedCellList is provided in system information, the UE supporting OD-SIB1 ignores intraFreqExcludedCellList and doesn’t consider the cell(s) (if any) in intraFreqODSIB1-ExcludedCellList as candidates for cell reselection. If interFreqODSIB1-ExcludedCellList is provided in system information, the UE supporting OD-SIB1 ignores interFreqExcludedCellList and doesn’t consider the cell(s) (if any) in interFreqODSIB1-ExcludedCellList as candidates for cell reselection.

If UE is in camped on any cell state, UE shall only apply the priorities provided by system information from current cell, and the UE preserves priorities provided by dedicated signalling and deprioritisationReq received in RRCRelease unless specified otherwise. When the UE in camped normally state, has only dedicated priorities other than for the current frequency, the UE shall consider the current frequency to be the lowest priority frequency (i.e. lower than any of the network configured values). When the HSDN capable UE is in High-mobility state, the UE shall always consider the HSDN cells to be the highest priority (i.e., higher than any other network configured priorities). When the HSDN capable UE is not in High-mobility state, the UE shall always consider HSDN cells to be the lowest priority (i.e., lower than any other network configured priorities). If the UE is configured to perform both NR sidelink communication and V2X sidelink communication, the UE may consider the frequency providing both NR sidelink communication configuration and V2X sidelink communication configuration to be the highest priority. If the UE is configured to perform NR sidelink communication and not perform V2X communication, the UE may consider the frequency providing NR sidelink communication configuration to be the highest priority. If the UE is configured to perform V2X sidelink communication and not perform NR sidelink communication, the UE may consider the frequency providing V2X sidelink communication configuration to be the highest priority. If the UE is configured to perform ranging/sidelink positioning, the UE may consider the frequency providing ranging/sidelink positioning configuration to be the highest priority.

A UE on a vehicle with a mobile-IAB cell may consider the frequency for which a mobile-IAB cell is the best cell to be the highest priority. The UE identifies a mobile-IAB cell by mobileIAB-Cell in SIB1 (see TS 38.331 [3]). The UE may narrow its search scope for mobile-IAB cell(s) by mobileIAB-CellList if broadcasted in SIB4 (see TS 38.331 [3]). A non-mobile-IAB cell may be excluded from this mobile IAB frequency prioritization for up to 300 seconds.

NOTE 0a:	The frequency only providing the anchor frequency configuration should not be prioritized for V2X service during cell reselection, as specified in TS 38.331[3].

NOTE 0b:	When UE is configured to perform NR sidelink communication or V2X sidelink communication performs cell reselection, it may consider the frequencies providing the intra-carrier and inter-carrier configuration have equal priority in cell reselection.

NOTE 0c:	The prioritization among the frequencies which UE considers to be the highest priority frequency is left to UE implementation unless otherwise stated.

NOTE 0d:	The UE is configured to perform V2X sidelink communication or NR sidelink communication, if it has the capability and is authorized for the corresponding sidelink operation.

NOTE 0e:	When UE is configured to perform both NR sidelink communication and V2X sidelink communication, but cannot find a frequency which can provide both NR sidelink communication configuration and V2X sidelink communication configuration, UE may consider the frequency providing either NR sidelink communication configuration or V2X sidelink communication configuration to be the highest priority.

NOTE 0f:	Void.

NOTE 0g:	How the UE determines itself to be on a vehicle with a mobile-IAB cell is left to the UE's implementation.

The UE shall only perform cell reselection evaluation for NR frequencies and inter-RAT frequencies that are given in system information and for which the UE has a priority provided.

If the MBS broadcast capable UE is receiving or interested to receive an MBS broadcast service(s) and can only receive this MBS broadcast service(s) by camping on a frequency on which it is provided, the UE may consider that frequency to be the highest priority during the MBS broadcast session as specified in TS 38.300 [2] as long as the two following conditions are fulfilled:

1)	SIB1 scheduling information of the cell reselected by the UE due to frequency prioritization for MBS contains SIB20;

2)	Either:

-	One or more MBS FSAI(s) of that frequency is indicated in SIB21 of the serving cell and the same MBS FSAI(s) is also indicated for this MBS broadcast service in MBS User Service Description (USD) as specified in TS 26.517 [20], or

-	SIB21 is not provided in the serving cell and that frequency is included in the USD of this service, or

-	SIB21 is provided in the serving cell but does not provide the frequency mapping for the concerned service, and that frequency is included in the USD of this service.

NOTE 0g: It is up to UE implementation which frequency to select, when the USD provides multiple frequencies for the service the UE is interested in.

If the MBS broadcast capable UE is receiving or interested to receive an MBS broadcast service, the UE may consider cell reselection candidate frequencies at which it cannot receive the MBS broadcast service to be of the lowest priority during the MBS broadcast session as specified in TS 38.300 [2], as long as SIB1 scheduling information of the cell contains SIB20 on the MBS frequency which the UE monitors and as long as the condition 2) above is fulfilled for the serving cell.

NOTE 0h:	Example scenarios in which such down-prioritisation may be needed include the cases where camping is not possible for the UE on the MBS broadcast frequency (e.g. the MBS broadcast frequency belongs to a PLMN different from UE's registered PLMN) while the UE can receive the MBS broadcast service when camped on another frequency than the MBS broadcast frequency or current frequency.

NOTE 0i:	The frequency prioritization for MBS broadcast, NR sidelink communication, or V2X sidelink communication may override the re-selection priorities for slice-based cell reselection.

In case UE receives RRCRelease with deprioritisationReq, UE shall consider current frequency and stored frequencies due to the previously received RRCRelease with deprioritisationReq or all the frequencies of NR to be the lowest priority frequency (i.e. lower than any of the network configured values) while T325 is running irrespective of camped RAT. The UE shall delete the stored deprioritisation request(s) when a PLMN selection or SNPN selection is performed on request by NAS (TS 23.122 [9]).

NOTE 1:	UE should search for a higher priority layer for cell reselection as soon as possible after the change of priority. The minimum related performance requirements specified in TS 38.133 [8] are still applicable.

NOTE 1a:	The UE does not consider MBS broadcast, NR sidelink communication or V2X sidelink communication functionality to replace cell reselection priorities caused by HSDN or deprioritisationReq functionality.

The UE shall delete priorities provided by dedicated signalling when:

-	the UE enters a different RRC state; or

-	the optional validity time of dedicated priorities (T320) expires; or

-	the UE receives an RRCRelease message with the field cellReselectionPriorities absent; or

-	a PLMN selection or SNPN selection is performed on request by NAS (TS 23.122 [9]).

NOTE 2:	Equal priorities between RATs are not supported.

The UE shall not consider any exclude-listed cells as candidate for cell reselection.

The UE shall consider only the allow-listed cells, if configured, as candidates for cell reselection.

NCR-MT may be configured with additional allowed cell list and/or forbidden cell list, see TS 38.300 [2]. The NCR-MT shall consider only the allowed cell list, if configured by OAM as candidates for cell reselection (ignore above exclude-listed cells and/or allow-listed cells). The NCR-MT shall not consider the cells for cell reselection in the forbidden cell list, if configured by OAM.

The UE in RRC_IDLE state shall inherit the priorities provided by dedicated signalling and the remaining validity time (i.e. T320 in NR and E-UTRA), if configured, at inter-RAT cell (re)selection.

NOTE 3:	The network may assign dedicated cell reselection priorities for frequencies not configured by system information.

Next change

### 5.3.1	Cell status and cell reservations

Cell status and cell reservations are indicated in the MIB or SIB1 message as specified in TS 38.331 [3] by means of the following fields:

-	cellBarred (IE type: "barred" or "not barred") 
Indicated in MIB message. In case of multiple PLMNs or NPNs indicated in SIB1, this field is common for all PLMNs and NPNs. This field is ignored by UEs supporting NTN for NTN access, or by UEs supporting ATG for ATG access.

-	cellBarredATG (IE type: "barred" or "not barred")
Indicated in SIB1 message. In case of multiple PLMNs or NPNs indicated in SIB1, this field is common for all PLMNs and NPNs. This field is only applicable to ATG UEs.

-	cellBarred2RxXR (IE type: "barred")
Indicated in SIB1 message. In case of multiple PLMNs or NPNs indicated in SIB1, this field is common for all PLMNs and NPNs. This field is only applicable to 2Rx XR UEs.

-	cellBarred-eRedCap1Rx (IE type: "barred" or "not barred")
Indicated in SIB1 message. In case of multiple PLMNs or NPNs indicated in SIB1, this field is common for all PLMNs and NPNs. This field is only applicable to eRedCap UEs.

-	cellBarred-eRedCap2Rx (IE type: "barred" or "not barred")
Indicated in SIB1 message. In case of multiple PLMNs or NPNs indicated in SIB1, this field is common for all PLMNs and NPNs. This field is only applicable to eRedCap UEs.

-	cellBarredFixedVSAT (IE type: "barred" or "not barred")
Indicated in SIB1 message. In case of multiple PLMNs indicated in SIB1, this field is common for all PLMNs. This field is only applicable to VSAT UEs using NTN access.

-	cellBarredMobileVSAT (IE type: "barred" or "not barred")
Indicated in SIB1 message. In case of multiple PLMNs indicated in SIB1, this field is common for all PLMNs. This field is only applicable to VSAT UEs using NTN access.

-	cellBarredNES (IE type: "not barred")
Indicated in SIB1 message. In case of multiple PLMNs or NPNs indicated in SIB1, this field is common for all PLMNs and NPNs. This field is only applicable to UEs indicating any of the values in nes-CellDTX-DRX as specified in TS 38.306 [24].

-	cellBarredNTN (IE type: "barred" or "not barred")
Indicated in SIB1 message. In case of multiple PLMNs indicated in SIB1, this field is common for all PLMNs. This field is ignored if the UE does not support NTN access.

-	cellBarredRedCap1Rx (IE type: "barred" or "not barred")
Indicated in SIB1 message. In case of multiple PLMNs or NPNs indicated in SIB1, this field is common for all PLMNs and NPNs. This field is only applicable to RedCap UEs.

-	cellBarredRedCap2Rx (IE type: "barred" or "not barred")
Indicated in SIB1 message. In case of multiple PLMNs or NPNs indicated in SIB1, this field is common for all PLMNs and NPNs. This field is only applicable to RedCap UEs.

-	cellReservedForOperatorUse (IE type: "reserved" or "not reserved") 
Indicated in SIB1 message. In case of multiple PLMNs or NPNs indicated in SIB1, this field is specified per PLMN or per SNPN.

-	cellReservedForOtherUse (IE type: "true") 
Indicated in SIB1 message. In case of multiple PLMNs indicated in SIB1, this field is common for all PLMNs.

-	cellReservedForFutureUse (IE type: "true") 
Indicated in SIB1 message. In case of multiple PLMNs or NPNs indicated in SIB1, this field is common for all PLMNs and NPNs.

NOTE 0:	IAB-MT ignores the cellBarred, cellReservedForOperatorUse, cellReservedForFutureUse, and intraFreqReselection (i.e. treats intraFreqReselection as if it was set to allowed) as defined in TS 38.331 [3]. IAB-MT also ignores cellReservedForOtherUse for cell barring determination (i.e. NPN capable IAB-MT considers cellReservedForOtherUse for determination of an NPN-only cell) as defined in TS 38.331 [3].

NOTE 0a: NCR-MT ignores the cellBarred, cellReservedForOperatorUse, cellReservedForFutureUse, and intraFreqReselection (i.e. treats intraFreqReselection as if it was set to allowed) as defined in TS 38.331 [3]. NCR-MT also ignores cellReservedForOtherUse for cell barring determination (i.e. NPN capable NCR-MT considers cellReservedForOtherUse for determination of an NPN-only cell) as defined in TS 38.331 [3].

-	halfDuplexRedCapAllowed (IE type: "true")
Indicated in SIB1 message. In case of multiple PLMNs or NPNs indicated in SIB1, this field is common for all PLMNs and NPNs. This field is only applicable to (e)RedCap UEs.

-	iab-Support (IE type: "true")
Indicated in SIB1 message. In case of multiple PLMNs or NPNs indicated in SIB1, this field is specified per PLMN or per SNPN. This field is only applicable to IAB-MT.

-	ncr-Support (IE type: "true")
Indicated in SIB1 message. In case of multiple PLMNs or NPNs indicated in SIB1, this field is common for all PLMNs and NPNs. This field is only applicable to NCR-MT.

-	mobileIAB-Support (IE type: "true")
Indicated in SIB1 message. In case of multiple PLMNs or NPNs indicated in SIB1, this field is specified per PLMN or per SNPN. This field is only applicable to mobile IAB-MT.

When cell status is indicated as "not barred" and "not reserved" for operator use and not "true" for other use and not "true" for future use,

-	UEs shall treat this cell as candidate during the cell selection and cell reselection procedures.

When cell broadcasts any CAG-IDs or NIDs and the cell status is indicated as "not barred" and "not reserved" for operator use and "true" for other use, and not "true" for future use:

-	All NPN-capable UEs shall treat this cell as candidate during the cell selection and cell reselection procedures, other UEs shall treat this cell as if cell status is "barred".

When cell status is indicated as "true" for other use, and either cell does not broadcast any CAG-IDs or NIDs or does not broadcast any CAG-IDs and the UE is not operating in SNPN Access Mode,

-	The UE shall treat this cell as if cell status is "barred".

When cell status is indicated as "true" for future use,

-	The UE shall treat this cell as if cell status is "barred".

When cellBarredNES is absent and cellBarred is set to "barred",

-	The UE indicating any of the values in nes-CellDTX-DRX shall treat this cell as if cell status is "barred".

When cellBarredNTN is not broadcast in this cell,

-	For NTN access, the UE shall treat this cell as if cell status is "barred".

When halfDuplexRedCapAllowed is not broadcast in this cell,

-	The (e)RedCap UE only capable of operating in half-duplex for FDD shall treat this cell as if cell status is "barred".

When cellBarredATG is not broadcast in this cell,

-	For ATG access, the UE shall treat this cell as if cell status is "barred".

When cellBarredFixedVSAT is not broadcast in this cell,

-	For NTN access, the fixed VSAT UE shall treat this cell as if cell status is "barred".

When cellBarredMobileVSAT is not broadcast in this cell,

-	For NTN access, the mobile VSAT UE shall treat this cell as if cell status is "barred".

When cellBarred2RxXR is broadcast in this cell,

-	The 2Rx XR UE shall treat this cell as if cell status is "barred".

When intraFreqReselectionRedCap is not broadcast in this cell,

-	The RedCap UE shall treat this cell as if cell status is "barred".

When intraFreqReselection-eRedCap is not broadcast in this cell,

-	The eRedCap UE shall treat this cell as if cell status is "barred".

When cellBarredRedCap1Rx is set to "barred" and barringExemptEmergencyCall is present in SIB1, if the cell will not be treated as barred by the UE for any reason other than the cellBarredRedCap1Rx being set to "barred" (see TS 38.331 [3]), and cell selection criteria are fulfilled as defined in clause 5.2.3,

-	The RedCap UE that supports barring exemption for emergency call (see TS 38.306 [24]) and only 1Rx branch shall treat this cell as an acceptable cell.

When cellBarredRedCap2Rx is set to "barred" and barringExemptEmergencyCall is present in SIB1, if the cell will not be treated as barred by the UE for any reason other than the cellBarredRedCap2Rx being set to "barred" (see TS 38.331 [3]), and cell selection criteria are fulfilled as defined in clause 5.2.3,

-	The RedCap UE that supports barring exemption for emergency call (see TS 38.306 [24]) and 2Rx branches shall treat this cell as an acceptable cell.

When cellBarred-eRedCap1Rx is set to "barred" and barringExemptEmergencyCall is present in SIB1, if the cell will not be treated as barred by the UE for any reason other than the cellBarred-eRedCap1Rx being set to "barred" (see TS 38.331 [3]), and cell selection criteria are fulfilled as defined in clause 5.2.3,

-	The eRedCap UE that supports barring exemption for emergency call (see TS 38.306 [24]) and only 1Rx branch shall treat this cell as an acceptable cell.

When cellBarred-eRedCap2Rx is set to "barred" and barringExemptEmergencyCall is present in SIB1, if the cell will not be treated as barred by the UE for any reason other than the cellBarred-eRedCap2Rx being set to "barred" (see TS 38.331 [3]), and cell selection criteria are fulfilled as defined in clause 5.2.3,

-	The eRedCap UE that supports barring exemption for emergency call (see TS 38.306 [24]) and 2Rx branches shall treat this cell as an acceptable cell.

When cellBarred2RxXR and barringExemptEmergencyCall are both present in SIB1, if the cell will not be treated as barred by the UE for any reason other than the cellBarred2RxXR being present in SIB1 (see TS 38.331 [3]), and cell selection criteria are fulfilled as defined in clause 5.2.3,

-	The 2Rx XR UE that supports barring exemption for emergency call (see TS 38.306 [24]) shall treat this cell as an acceptable cell.

When cell status is indicated as "not barred" and "reserved" for operator use for any PLMN/SNPN and not "true" for other use and not "true" for future use,

-	UEs assigned to Access Identity 11 or 15 operating in their HPLMN/EHPLMN shall treat this cell as candidate during the cell selection and reselection procedures if the field cellReservedForOperatorUse for that PLMN set to "reserved".

-	UEs assigned to Access Identity 11 or 15 shall treat this cell as candidate during the cell selection and reselection procedures if the field cellReservedForOperatorUse for selected/registered SNPN is set to "reserved".

-	UEs assigned to an Access Identity 0, 1, 2 and 12 to 14 shall behave as if the cell status is "barred" in case the cell is "reserved for operator use" for the registered PLMN/SNPN or the selected PLMN/SNPN.

-	UEs assigned to Access Identity 3 shall behave as if the cell status is "barred" in case the cell is "reserved for operator use" for the registered PLMN or the selected PLMN.

NOTE 1:	Access Identities 11, 15 are only valid for use in the HPLMN/ EHPLMN and registered/selected SNPN; Access Identities 12, 13, 14 are only valid for use in the home country and registered/selected SNPN as specified in TS 22.261 [12].

NOTE 1a:	Access Identity 3 is only valid for PLMNs that indicate to potential Disaster Inbound Roamers that the UEs can access the PLMN as specified in TS 22.261 [12].

When cell status "barred" is indicated or to be treated as if the cell status is "barred",

-	The UE is not permitted to select/reselect this cell, not even for emergency calls;

-	The UE shall select another cell according to the following rule:

-	If the cell is to be treated as if the cell status is "barred" due to being unable to acquire the MIB:

-	the UE may exclude the barred cell as a candidate for cell selection/reselection for up to 300 seconds.

-	the UE may select another cell on the same frequency if the selection criteria are fulfilled.

-	else:

-	If the UE is a RedCap UE, the UE shall acquire SIB1 and, in the remainder of this procedure, consider 'intraFreqReselection in MIB' to be 'intraFreqReselectionRedCap in SIB1', if available; or,

-	If the UE is an eRedCap UE, the UE shall acquire SIB1 and, in the remainder of this procedure, consider 'intraFreqReselection in MIB' to be 'intraFreqReselection-eRedCap in SIB1', if available; or,

-	If the UE is a 2Rx XR UE, the UE shall acquire SIB1 and, in the remainder of this procedure, consider 'intraFreqReselection in MIB' to be 'intraFreqReselection2RxXR in SIB1', if available:

-	If the cell is to be treated as if the cell status is "barred" due to being unable to acquire the SIB1:

-	the UE may exclude the barred cell as a candidate for cell selection/reselection for up to 300 seconds.

-	the UE may select another cell on the same frequency if the selection criteria are fulfilled.

-	If the cell status "barred" is indicated in MIB but the UE is unable to acquire the SIB1; or

-	If the UE is a RedCap UE and intraFreqReselectionRedCap in SIB1 is not available; or

-	If the UE is an eRedCap UE and intraFreqReselection-eRedCap in SIB1 is not available:

-	the UE shall exclude the barred cell as a candidate for cell selection/reselection for 300 seconds.

-	the UE may select another cell on the same frequency if re-selection criteria are fulfilled.

-	If the UE is neither a RedCap UE nor an eRedCap UE; or

-	if the UE is a RedCap UE and intraFreqReselectionRedCap in SIB1 is available; or

-	if the UE is an eRedCap UE and intraFreqReselection-eRedCap in SIB1 is available:

-	If the field intraFreqReselection in MIB message is set to "allowed":

-	the UE may select another cell on the same frequency if re-selection criteria are fulfilled;

-	If the cell is to be treated as if the cell status is "barred" due to no available SIB1 request configuration for the UE supporting OD-SIB1 as defined in section 5.2.2.3.1 of TS 38.331 [3]; or

-	If the cell is to be treated as if the cell status is "barred" due to maximum number of PRACH attempts is reached for the UE supporting OD-SIB1 as defined in section 5.2.2.3.3x of TS 38.331 [3]; or

-	If the cell is to be treated as if the cell status is "barred" due to failing to acquire the SIB1 upon the expiry of the SIB1 monitoring window as defined in [4] for the UE supporting OD-SIB1; or

-	If the cell is to be treated as if the cell status is "barred" due to being unable to acquire the SIB1:

-	the UE may exclude the barred cell as a candidate for cell selection/reselection for up to 300 seconds;

-	else:

-	the UE shall exclude the barred cell as a candidate for cell selection/reselection for 300 seconds.

-	If the field intraFreqReselection in MIB message is set to "not allowed":

-	If the cell is to be treated as if the cell status is "barred" due to no available SIB1 request configuration for the UE supporting OD-SIB1 as defined in section 5.2.2.3.1 of TS 38.331 [3]; or

-	If the cell is to be treated as if the cell status is "barred" due to maximum number of PRACH attempts is reached for the UE supporting OD-SIB1 as defined in section 5.2.2.3.3x of TS 38.331 [3]; or

-	If the cell is to be treated as if the cell status is "barred" due to failing to acquire the SIB1 upon the expiry of the SIB1 monitoring window as defined in [4] for the UE supporting OD-SIB1; or

-	If the cell is to be treated as if the cell status is "barred" due to being unable to acquire the SIB1:

-	the UE may exclude the barred cell as a candidate for cell selection/reselection for up to 300 seconds;

-	If the cell operates in licensed spectrum:

-	the UE shall not re-select to another cell on the same frequency as the barred cell and exclude such cell(s) as candidate(s) for cell selection/reselection for 300 seconds;

-	else:

-	the UE may select to another cell on the same frequency if the reselection criteria are fulfilled.

-	else:

-	If the cell operates in licensed spectrum, or if this cell belongs to a PLMN which is indicated as being equivalent to the registered PLMN or the selected PLMN of the UE, or if this cell belongs to an SNPN which is equal to or indicated as being equivalent to the registered SNPN or the selected SNPN of the UE:

-	the UE shall not re-select to another cell on the same frequency as the barred cell and exclude such cell(s) as candidate(s) for cell selection/reselection for 300 seconds;

-	else:

-	the UE may select to another cell on the same frequency if the reselection criteria are fulfilled.

-	the UE shall exclude the barred cell as a candidate for cell selection/reselection for 300 seconds.

The cell selection of another cell may also include a change of RAT.

If a UE supporting OD-SIB1 barred a cell due to no available SIB1 request configuration as defined in section 5.2.2.3.1 of TS 38.331 [3], it considers the cell is no longer barred once SIB1 request configuration of the cell is acquired.

NOTE 2:	If barring of a cell is triggered by the condition of trackingAreaCode and trackingAreaList not being provided, as specified in TS 38.331 [3], the barring only applies to this PLMN and the UE can re-evaluate the barring condition again due to selection of another PLMN.

Next change

## 7.1	Discontinuous Reception for paging

The UE may use Discontinuous Reception (DRX) in RRC_IDLE and RRC_INACTIVE state in order to reduce power consumption. The UE monitors one paging occasion (PO) per DRX cycle. A PO is a set of PDCCH monitoring occasions and can consist of multiple time slots (e.g. subframe or OFDM symbol) where paging DCI can be sent (TS 38.213 [4]). One Paging Frame (PF) is one Radio Frame and may contain one or multiple PO(s) or starting point of a PO. A L2 U2N Relay UE monitors the paging occasions of its PC5-RRC connected L2 U2N Remote UEs. In this case, the DRX cycle and UE ID mentioned in this clause refer to those of the L2 U2N Remote UE.

In multi-beam operations, the UE assumes that the same paging message and the same Short Message are repeated in all transmitted beams and thus the selection of the beam(s) for the reception of the paging message and Short Message is up to UE implementation. The paging message is same for both RAN initiated paging and CN initiated paging.

The UE initiates RRC Connection Resume procedure upon receiving RAN initiated paging. If the UE receives a CN initiated paging in RRC_INACTIVE state, the UE moves to RRC_IDLE and informs NAS. However, if a L2 U2N Relay UE in RRC_INACTIVE state receives a CN initiated paging for a L2 U2N Remote UE, the L2 U2N Relay UE does not move to RRC_IDLE state.

NOTE 0a:	The L2 U2N Remote UE does not need to monitor the PO in order to receive the paging message.

NOTE 0b:	While the SDT procedure is ongoing in RRC_INACTIVE state, the UE monitors the PO in order to receive only the Short Message as specified in TS 38.331 [3].

The PF and PO for paging are determined by the following formulae:

SFN for the PF is determined by:

(SFN + PF_offset) mod T = (T div N)*(UE_ID mod N)

Index (i_s), indicating the index of the PO is determined by:

i_s = floor (UE_ID/N) mod Ns

The PDCCH monitoring occasions for paging are determined according to pagingSearchSpace as specified in TS 38.213 [4] and firstPDCCH-MonitoringOccasionOfPO (or firstPDCCH-MonitoringOccasionOfPO-r19 for paging adaptation, if configured) and nrofPDCCH-MonitoringOccasionPerSSB-InPO if configured as specified in TS 38.331 [3]. When SearchSpaceId = 0 is configured for pagingSearchSpace, the PDCCH monitoring occasions for paging are same as for RMSI as defined in clause 13 in TS 38.213 [4].

When SearchSpaceId = 0 is configured for pagingSearchSpace, Ns is either 1 or 2. For Ns = 1, there is only one PO which starts from the first PDCCH monitoring occasion for paging in the PF. For Ns = 2, PO is either in the first half frame (i_s = 0) or the second half frame (i_s = 1) of the PF.

When SearchSpaceId other than 0 is configured for pagingSearchSpace, the UE monitors the (i_s + 1)th PO. A PO is a set of 'S*X ' consecutive PDCCH monitoring occasions where 'S' is the number of actual transmitted SSBs determined according to ssb-PositionsInBurst in SIB1 and X is the nrofPDCCH-MonitoringOccasionPerSSB-InPO if configured or is equal to 1 otherwise. The [x*S+K]th PDCCH monitoring occasion for paging in the PO corresponds to the Kth transmitted SSB, where x=0,1,…,X-1, K=1,2,…,S. The PDCCH monitoring occasions for paging which do not overlap with UL symbols (determined according to tdd-UL-DL-ConfigurationCommon) are sequentially numbered from zero starting from the first PDCCH monitoring occasion for paging in the PF. When firstPDCCH-MonitoringOccasionOfPO (or firstPDCCH-MonitoringOccasionOfPO-r19 for paging adaptation, if configured) is present, the starting PDCCH monitoring occasion number of (i_s + 1)th PO is the (i_s + 1)th value of the firstPDCCH-MonitoringOccasionOfPO (or firstPDCCH-MonitoringOccasionOfPO-r19 for paging adaptation, if configured) parameter; otherwise, it is equal to i_s * S*X. If X > 1, when the UE detects a PDCCH transmission addressed to P-RNTI within its PO, the UE is not required to monitor the subsequent PDCCH monitoring occasions for this PO.

NOTE 1:	A PO associated with a PF may start in the PF or after the PF.

NOTE 2:	The PDCCH monitoring occasions for a PO can span multiple radio frames. When SearchSpaceId other than 0 is configured for paging-SearchSpace the PDCCH monitoring occasions for a PO can span multiple periods of the paging search space.

The following parameters are used for the calculation of PF and i_s above:

T: DRX cycle of the UE.

If the UE does not operate in eDRX as defined in clause 7.4:

-	T is determined by the shortest of the UE specific DRX value configured by RRC (if any), the UE specific DRX value configured by upper layers (if any), and a default DRX value broadcast in system information. For L2 U2N Relay UE, T for a L2 U2N Remote UE is determined by the shortest of the UE specific DRX value provided in PC5-RRC signalling and a default DRX value broadcast in system information.

In RRC_IDLE state, if the UE operates in eDRX and eDRX is configured by upper layers, i.e., TeDRX, CN, according to clause 7.4:

-	If TeDRX, CN is no longer than 1024 radio frames:

-	T = TeDRX, CN;

-	else:

-	During CN configured PTW, T is determined by the shortest of UE specific DRX value, if configured by upper layers, and the default DRX value broadcast in system information.

In RRC_INACTIVE state, if the UE operates in eDRX and eDRX is configured by RRC, i.e., TeDRX, RAN (if any), and upper layers, i.e., TeDRX, CN, as defined in clause 7.4:

-	If both TeDRX, CN and used TeDRX, RAN are no longer than 1024 radio frames:

-	T = min{TeDRX, RAN, TeDRX, CN}.

-	If TeDRX, CN is no longer than 1024 radio frames and no TeDRX, RAN is configured or used:

-	T is determined by the shortest of UE specific DRX value configured by RRC and TeDRX, CN.

-	If TeDRX, CN is longer than 1024 radio frames:

-	If TeDRX, RAN is not configured or used:

-	During CN configured PTW, T is determined by the shortest of the UE specific DRX value configured by RRC, the UE specific DRX value configured by upper layers (if any), and a default DRX value broadcast in system information. Outside the CN configured PTW, T is determined by the UE specific DRX value configured by RRC;

-	else if used TeDRX, RAN is no longer than 1024 radio frames:

-	During CN configured PTW, T is determined by the shortest of the UE specific DRX value, if configured by upper layers and TeDRX, RAN, and a default DRX value broadcast in system information. Outside the CN configured PTW, T is determined by TeDRX, RAN;

-	else if used TeDRX, RAN is longer than 1024 radio frames:

-	During the overlapped part of CN configured PTW and RAN configured PTW, T is determined by the shortest of the UE specific DRX value configured by RRC, the UE specific DRX value configured by upper layers (if any), and a default DRX value broadcast in system information;

-	During CN configured PTW and outside RAN configured PTW, T is determined by the shortest of the UE specific DRX value configured by upper layers (if any), and a default DRX value broadcast in system information;

-	Outside CN configured PTW and during RAN configured PTW, T is determined by the UE specific DRX value configured by RRC.

N: number of total paging frames in T

Ns: number of paging occasions for a PF

PF_offset: offset used for PF determination

UE_ID:

If the UE operates in eDRX as specified in clause 7.4:

-	5G-S-TMSI mod 4096

else:

-	5G-S-TMSI mod 1024

Parameters Ns, nAndPagingFrameOffset, nrofPDCCH-MonitoringOccasionPerSSB-InPO, and the length of default DRX Cycle are signaled in SIB1. The values of N and PF_offset are derived from the parameter nAndPagingFrameOffset as defined in TS 38.331 [3]. The parameter firstPDCCH-MonitoringOccasionOfPO is signalled in SIB1 for paging in the BWP configured by initialDownlinkBWP. For paging in a DL BWP other than the BWP configured by initialDownlinkBWP, the parameter first-PDCCH-MonitoringOccasionOfPO is signaled in the corresponding BWP configuration.

For a UE in RRC_IDLE and RRC_INACTIVE and supporting paging adaptation, if pagingAdaptation-NS and pagingAdaptationNAndPagingFrameOffset are signaled in SIB1, it determines the value of Ns from pagingAdaptation-NS, N and PF_offset from the parameter pagingAdaptationNAndPagingFrameOffset as defined in TS 38.331 [3], and only monitors the PO(s) derived from these paging parameters. If parameter firstPDCCH-MonitoringOccasionOfPO-r19 is signalled in SIB1, the UE uses it to determine the PDCCH monitoring occasions for paging as specified in TS 38.331 [3].

If the UE has no 5G-S-TMSI, for instance when the UE has not yet registered onto the network, the UE shall use as default identity UE_ID = 0 in the PF and i_s formulas above.

5G-S-TMSI is a 48 bit long bit string as defined in TS 23.501 [10]. 5G-S-TMSI shall in the formulae above be interpreted as a binary number where the left most bit represents the most significant bit.

In RRC_INACTIVE state, if the UE supports inactiveStatePO-Determination and the network broadcasts ranPagingInIdlePO with value "true", the UE shall use the same i_s as for RRC_IDLE state. Otherwise, the UE determines the i_s based on the parameters and formula above.

In RRC_INACTIVE state, if used eDRX value configured by upper layers is no longer than 1024 radio frames, the UE shall use the same i_s as for RRC_IDLE state.

In RRC_INACTIVE state, if used eDRX value configured by upper layers is longer than 1024 radio frames, during CN PTW, the UE shall use the same i_s as for RRC_IDLE state. Outside CN PTW, the UE shall use the i_s for RRC_INACTIVE state.

Next change

### 7.2.1	Paging Early Indication reception

The UE may use Paging Early Indication (PEI) in RRC_IDLE and RRC_INACTIVE states in order to reduce power consumption. If PEI configuration is provided in system information, the UE in RRC_IDLE or RRC_INACTIVE state supporting PEI (except for the UEs expecting MBS group notification) can monitor PEI using PEI parameters in system information according to the procedure described below.

If lastUsedCellOnly is configured in system information of a cell, the UE monitors PEI in this cell only if the UE most recently received RRCRelease without noLastCellUpdate in this cell. Otherwise (i.e., if lastUsedCellOnly is not configured in system information of a cell), the UE monitors PEI in the camped cell.

The UE monitors one PEI occasion per DRX cycle. A PEI occasion (PEI-O) is a set of PDCCH monitoring occasions (MOs) and can consist of multiple time slots (e.g. subframes or OFDM symbols) where PEI can be sent (TS 38.213 [4]). In multi-beam operations, the UE assumes that the same PEI is repeated in all transmitted beams and thus the selection of the beam(s) for the reception of the PEI is up to UE implementation.

The time location of PEI-O for UE's PO is determined by a reference point and an offset:

-	The reference point is the start of a reference frame determined by a frame-level offset from the start of the first PF of the PF(s) associated with the PEI-O, provided by pei-FrameOffset in SIB1;

-	The offset is a symbol-level offset from the reference point to the start of the first PDCCH MO of this PEI-O, provided by firstPDCCH-MonitoringOccasionOfPEI-O in SIB1.

If one PEI-O is associated with POs of two PFs, the two PFs are consecutive PFs calculated by the parameters PF_offset, T, Ns, and N. The first PF of the PFs associated with the PEI-O is provided by (SFN for PF) - floor (iPO/Ns)*T/N, where SFN for PF is determined in clause 7.1, iPO is defined in clause 10.4a in TS 38.213[4], T, Ns, and N are determined in clause 7.1.

The PDCCH MOs for PEI are determined as specified in TS 38.213 [4] according to pei-SearchSpace, pei-FrameOffset, firstPDCCH-MonitoringOccasionOfPEI-O and nrofPDCCH-MonitoringOccasionPerSSB-InPO if configured as specified in TS 38.331 [3]. When SearchSpaceId = 0 is configured for pei-SearchSpace, the PDCCH MOs for PEI are same as for RMSI as defined in clause 13 in TS 38.213 [4]. UE determines first PDCCH MO for PEI-O based on pei-FrameOffset and firstPDCCH-MonitoringOccasionOfPEI-O, as for the case with SearchSpaceId > 0 configured.

When SearchSpaceId = 0 is configured for pei-SearchSpace, the UE monitors the PEI-O according to searchSpaceZero. When SearchSpaceId other than 0 is configured for pei-SearchSpace, the UE monitors the PEI-O according to the search space with the configured SearchSpaceId.

A PEI occasion is a set of 'S*X' consecutive PDCCH MOs, where 'S' is the number of actual transmitted SSBs determined according to ssb-PositionsInBurst in SIB1, and X is the nrofPDCCH-MonitoringOccasionPerSSB-InPO if configured or is equal to 1 otherwise. The [x*S+K]th PDCCH MO for PEI in the PEI-O corresponds to the Kth transmitted SSB, where x=0,1,…,X-1, K=1,2,…,S. The PDCCH MOs for PEI which do not overlap with UL symbols (determined according to tdd-UL-DL-ConfigurationCommon) are sequentially numbered from zero starting from the first PDCCH MO for PEI in the PEI-O. When the UE detects a PEI within its PEI-O, the UE is not required to monitor the subsequent MO(s) associated with the same PEI-O.

If the UE detects PEI and the PEI indicates the subgroup the UE belongs to monitor its associated PO, as specified in clause 10.4a in TS 38.213 [4], the UE monitors the associated PO as specified in clause 7.1. If the UE does not detect PEI on the monitored PEI occasion or the PEI does not indicate the subgroup the UE belongs to monitor its associated PO, as specified in clause 10.4a in TS 38.213 [4], the UE is not required to monitor the associated PO as specified in clause 7.1.

If the UE is unable to monitor the PEI occasion (i.e. all valid PDCCH MO for PEI) corresponding to its PO, e.g. during cell re-selection, the UE monitors the associated PO according to clause 7.1.

In RRC_INACTIVE state, when the UE uses the same i_s as for RRC_IDLE state as specified in clause 7.1, the UE shall use the same iPO as for RRC_IDLE state. Otherwise, the UE determines the iPO based on the formula defined in clause 10.4a in TS 38.213 [4].

For a UE supporting paging adaptation and PEI, if pagingAdaptationPEI-Config is signaled in system information, the UE in RRC_IDLE and RRC_INACTIVE state can monitor the PEI occasion according to pagingAdaptationPEI-Config and firstPDCCH-MonitoringOccasionOfPEI-O-r19 (if configured).

End of change