3GPP TSG-RAN WG2 Meeting #131	R2-250xxxx

Bangalore, India, 25th – 29th Aug. 2025

Agenda item:		8.4.1

Source:				CATT

Title:				  	Discission of [Post130][211][LPWUS] Running CR for 38.304 (CATT)

Document for:	Discussion and Decision

- Introduction

This document is the report of the following discussion:

[Post130][211][LPWUS] Running CR for 38.304 (CATT)

Intended outcome: Updated and reviewed the CR for endorsement, update the open issue list if needed, can also discuss open issues to form proposals to the next meeting

Deadline:  Long

- Discussion

  - Easily addressed open issues

    - Open issue 38304-1: FFS on the terminology LP-WUS UE

In RAN2#130, we agreed:

On UE capabilities A UE indicating support of LP-WUS reception in IDLE/INACTIVE shall support UE-ID based subgrouping. From R2 point of view, RRM measurement relaxation and RRM measurement fully offloading are defined as RAN2 capability without UE capability signalling. UE supporting LP-WUS reception shall also support RRM measurement relaxation and RRM measurement fully offloading

Based on the agreements on UE capabilities, the possible options for the FFS on the terminology LP-WUS UE can be:

- Option A: In the running CR, UE supporting LP-WUS reception is used instead of LP-WUS UE.

- Option B: LP-WUS UE is introduced in clause 3.1 in TS 38.304. E.g. LP-WUS UE: A UE with LP-WUS reception capability as specified in TS 38.306.

Companies are invited to provide their preference.

Q1: Which option is preferred for the FFS on the terminology LP-WUS UE?

- Option A: In the running CR, UE supporting LP-WUS reception is used instead of LP-WUS UE.

- Option B: LP-WUS UE is introduced in clause 3.1 in TS 38.304. E.g. LP-WUS UE: A UE with LP-WUS reception capability as specified in TS 38.306.

| Company | Preferred option (A or B) | Comments |
| --- | --- | --- |
| Samsung | A | The LP-WUS function could be viewed as one of the UE behaviors that a Rel-19 UE with the capability could support, rather than requiring the introduction of a new UE type. |
| NEC | B | Using LP-WUS UE is more convenient in the spec, it doesn’t mean this is a new type of UE as long as we give a clear definition on what LP-WUS UE is. |
| Vivo | A | Totally agree with Samsung. LP-WUS/WUR is a Rel-19 feature. Usually, we only specify the UE supporting xx feature, but not xx UE, which looks like a new UE type. We should avoid such potential mis-understanding. Besides, RAN1 has sent an LS to RAN2 in last meeting. One intention is LP-WUS function as a sub-functional UE behavior that a Rel-19 UE with the capability could support, rather than introducing a significant burden, such as defining a new LP-RAT. |
| Huawei/HiSilicon | Please see comment | We favour Option A but think the wording needs modification. May be “UE can use LP-WUS”? (Our intention is to say that UE supports LP-WUS and the camped cell configures LP-WUS). |
| Lenovo | B | Once a clear definition is included in clause 3.1 for the terminology of LP-WUS UE, it can be used in the spec for simplicity. |
| OPPO | A | Option A is clearer as LP-WUS is a new optional feature with UE capability rather than a new UE type. |
| Ericsson | A, see comments | No strong view, i.e. both can work. For option A prefer to shorten to “UE supporting LP-WUS”. |
| DOCOMO | A | Share similar views with Samsung and vivo. It is better to avoid misunderstanding as a new UE type by Option B. For Option A, we think alignment of terminology is necessary depending on the outcome from the on-going post email discussion titled “[Post130][210][LPWUS] Running CR for 38.331”. It is discussed whether to algin the terminologies related to LP-WUS among specs. |

Summary:

    - Open issue 38304-7: higher priority frequency

Open issue 38304-7: Whether Relaxed measurement and offloading measurement can be performed when there is NR inter-frequency and/or NR inter-RAT frequency with reselection priority higher than that of the camped frequency. (Same as the open issue in RRC, i.e., FFS on whether/how RRM relaxation is applicable for high priority frequency)

RAN2 discussed the open issue online without conclusion in RAN2#130.

R2-2504623 Remaining issues for LP-WUS RRM ZTE Corporation, Sanechips discussion Rel-19 NR_LPWUS-Core Noted # existence of higher priority frequency Proposal 8 RAN2 to discuss which threshold to be applied to RRM relaxation for high priority frequencies 1. Reuse the threshold for lower or equal priority frequency. 2. Reuse Offloading condition. Discussion - OPPO think we can reuse offloading condition. - vivo point out that in this meeting R4 agree for both case 1 and case 3. - Ericsson fine to follow R4 conclusion and think these only applies when NW configure R19 RRM rlx/offloading. - CATT suggest to take the new agreements from R4 into account in the post meeting email discussions.

And in RAN4#115, the agreements on high priority frequency were:

- For case 1: - MR is expected to perform relaxed higher priority frequency layer measurement with K2*Thigher_priority_search and K2 = 60 - Note: RAN4 assumes Srxlev > SnonIntraSearchP and Squal > SnonIntraSearchQ is always met for case 1. - For case 3: - When Srxlev > SnonIntraSearchP and Squal > SnonIntraSearchQ, MR is expected to perform relaxed higher priority frequency layer measurement with K2*Thigher_priority_search and K2 = 60 - When the condition of Srxlev > SnonIntraSearchP and Squal > SnonIntraSearchQ is NOT met, the same requirement for higher priority, equal priority and lower priority carriers: - 16 times of Tdetect,NR_Inter, Tmeasure,NR_Inter and Tevaluate,NR_Inter are applied

According to RAN4 agreement, UE is expected to perform relaxed higher priority frequency layer measurements in both serving cell measurement offloading (i.e., Case 1) and relaxed serving cell/neighbouring cell measurements (i.e., Case 3).

General descriptions of serving cell measurement offloading (i.e., Case 1) and relaxed serving cell/neighbouring cell measurements (i.e., Case 3) have already been captured in 38.304 running CR for LP-WUS, as shown below:

5.2.4.x.1 Relaxed measurement rules for LP-WUS UE LP-WUS UE may choose to perform relaxed serving cell and neighbouring cell measurements on MR according to requirements specified in TS 38.133 [8] if the entry condition for measurement relaxation in clause 5.2.4.x.2 is fulfilled. //skip 5.2.4.x.3 Serving cell measurement offloading rules for LP-WUS UE LP-WUS UE may choose to perform serving cell measurement offloading (i.e., serving cell measurement fully offloaded to LR and no serving cell measurement via MR is required) according to requirements specified in TS 38.133 [8] if the entry condition for serving cell measurement offloading in clause 5.2.4.x.4 is fulfilled. LP-WUS UE is not required to perform serving cell measurement offloading according to requirements specified in TS 38.133 [8] if the exit condition for serving cell measurement offloading in clause 5.2.4.x.4 is fulfilled.

Rapporteur understands general descriptions covers all cases of neighbouring cell measurements on MR. Considering the relaxed requirements are referred to TS 38.133 directly, we don’t need to specify anything on high priority frequency for serving cell measurement offloading or measurement relaxation with LP-WUS in 38.304 running CR.

Q2: Do companies agree that nothing is needed on high priority frequency for serving cell measurement offloading or measurement relaxation with LP-WUS in 38.304 running CR? And if it is needed to specify something for high priority frequency in 38.304 running CR, please provide your suggestion.

| Company | Yes/No | Comments |
| --- | --- | --- |
| Samsung | Yes | Nothing is needed. |
| NEC | Yes | We can leave higher priority frequency measurements to RAN4. |
| vivo | See comment | Yes by now. We agree with Rapporteur the corresponding relaxation requirements need to be captured in TS 38.133. It seems nothing is needed in TS 38.304 if the corresponding higher priority frequency relaxation has been captured in RAN4 specifications. With this, we think there is no need to have any proposal/conclusion on this part for safety. Let’s review the RAN2/RAN4 specification to check whether anything is missing. |
| Huawei/HiSilicon |  | Agree with Vivo |
| Lenovo | Yes | Refer to follow spec TS28.133 is enough. |
| OPPO | Yes | The measurement for higher priority frequency can rely on RAN4 spec. In current spec, it is specified that the measurement for higher priority frequency is according to TS38.133: - For a NR inter-frequency or inter-RAT frequency with a reselection priority higher than the reselection priority of the current NR frequency, the UE shall perform measurements of higher priority NR inter-frequency or inter-RAT frequencies according to TS 38.133 [8]. |
| Ericsson | Yes, see comment | We agree with the principle, but we need to check how/what RAN4 has captured in 38.133. |
| DOCOMO |  | Agree with vivo and Huawei. |

    - Open issue 38304-11: FFS on the determination of RRM measurement relaxation/offloading conditions if UE supports both measurement types

In RAN2#130, it was agreed:

It is up to UE implementation to choose whether SSB measurement based or OOK LP-SS measurement based conditions are used for LP-WUS monitoring entry/exit condition, if UE support both measurement types.

In RAN2#129bis, it was agreed:

RAN2 assumes for the entry/ exit conditions of serving cell measurement offloading and serving cell RRM measurement relaxation: separate MR thresholds (according to RAN1 agreement)/LR thresholds can be configured for different types of LP WUR if a cell supports both types of LRs (can revisit based on RAN1 and RAN 4 progress, if any).

Rapporteur wonders if the above agreement in RAN2#130 can be applied to conditions of serving cell measurement offloading and serving cell & neighbouring cell measurement relaxation, i.e. it is up to UE implementation to choose whether SSB measurement based or OOK LP-SS measurement based are used for RRM relaxation/offloading conditions, if UE supports both measurement types. Or do RAN2 need to clarify that the same measurement type is used for conditions of LP-WUS monitoring and RRM relaxation/offloading?

Companies are invited to provide their preference.

Q3: Which option is preferred if UE support both measurement types?

- Option A: Same as LP-WUS monitoring, it is up to UE implementation to choose whether SSB measurement based or OOK LP-SS measurement based are used for RRM relaxation/offloading conditions if UE supports both measurement types.

- Option B: It is clarified that the same measurement type is used for conditions of LP-WUS monitoring and RRM relaxation/offloading if UE supports both measurement types.

| Company | Preferred option (A or B) | Comments |
| --- | --- | --- |
| Samsung | A | We prefer A, but don’t have a strong opinion. We are open to aligning with the majority view if it leans toward option B. |
| NEC | No strong view | Both options can work… |
| vivo | A | LP-WUS monitoring and RRM relaxation are two separate features for UE supporting LP-WUS/WUR. Reasonable UE behaviour should choose the same RS type measurement for LP-WUS monitoring and RRM relaxation. Considering it is up to UE implementation to choose which RS type measurement is used for RRM relaxation, we should apply the same principle for LP-WUS monitoring. |
| Huawei/HiSilicon | A |  |
| Lenovo | A | Prefer to follow the principle as agreed for LP-WUS monitoring, can be left to UE implementation. |
| OPPO | A | We can use the same principle for LP-WUS monitoring. Leave it up to UE implementation to determine. |
| Ericsson | A |  |
| DOCOMO | A |  |

Summary:

  - Comments on TS 38.304 running CR

Companies can provide comments and suggestions to the uploaded running CR in this table. Please do not add changes, suggestions, or comments directly to the draft CR document.

| Company + Issue Number (e.g., C001) | Detailed comments | Rapporteur response |
| --- | --- | --- |
| SS001 | We suggest discussing the alignment of terminologies in 38.304 with those in RAN1 CRs (38.212 ~ 215). For further discussion, please refer to Section 2.3.4 below. |  |
| SS002 | - Running CR 7.x.0 General The UE monitors one LP-WUS occasion per DRX cycle. A LO is a set of LP-WUS monitoring occasions (LP-WUS MOs). In multi-beam operations, the UE assumes that the same LP-WUS is repeated in all transmitted beams and thus the selection of the beam(s) for the reception of the LP-WUS is up to UE implementation. The time location of an LO for UE’s PO is determined by a reference PF and the configured frame-level offset: - The reference PF is the start of the PF, or the first PF of the PF(s) (if mapping of POs from multiple PFs to one LO is supported), associated with the LO. The reference PF for the LO of a PO is provided by (SFN for PF) – floor(iPO/NS) * T/N, where SFN for PF is determined in clause 7.1, iPO is defined in clause 10.4C in TS 38.213[4], T, Ns, and N are determined in clause 7.1. - The frame-level offset between the LO and the reference PF is provided by lo-Offset in SIB1. If single value is configured for lo-Offset, and if the gap between the LO and the corresponding PO is no less than the wake-up delay that a UE supports, the UE monitors the LO associated with the offset, otherwise the UE follows the paging monitoring procedure as described in clause 7.1 or 7.2. If more than one values are configured for lo-Offset, and if the gap between the LO associated with the largest offset and the corresponding PO is no less than the wake-up delay that a UE supports, the UE monitors the LO associated with the smallest offset value that has a gap between the LO and the PO associated with the offset no less than the wake-up delay, otherwise the UE follows the paging monitoring procedure as described in clause 7.1or 7.2. - Comment The details regarding the locations, offsets, and UE behaviors related to LO (LP-WUS Occasion) monitoring are already defined in TS 38.213[4], as follows. [TS 38.213 CR] A UE assumes that WUS occasions occur with a periodicity equal to the I-DRX cycle in the RRC_IDLE/RRC_INACTIVE state [17, TS 38.304]. The UE determines WUS occasions associated with a paging occasion based on PO-to-LO association. A reference frame of a WUS occasion starts a number of frames prior to the first of a number of paging frames associated with the WUS occasion. Each number of frames is provided by LO-FrameOffsets. The first WUS monitoring occasion of a WUS occasion starts at an offset provided by offset_firstMO_withinLO relative to the start of the reference frame. If multiple values for the number of frames provided by LO-FrameOffsets are larger than or equal to the value of XYZ, the UE monitors WUS starting at a WUS occasion corresponding to the smallest of the multiple values. If all values for the number of frames provided by LO-FrameOffsets are smaller than the value of XYZ, the UE monitors PDCCH according to Type2-PDCCH CSS sets associated with the paging occasion and does not monitor WUS. A paging occasion associated with a WUS occasion has index iPO=UE_ID mod N⋅NS+i_s mod NPOWO where NPOWO is a number of paging occasions associated with a WUS occasion, N, NS, iSG, and i_s are defined in [17, TS 38.304], and UE_ID is defined in clause 7.1 of [17, TS 38.304]. If a number of NSGPO subgroups per paging occasion, provided by subgroupNumber-PO-WUS, is NSGPO>1, the codepoint for the subgroup index iSG in a PO iPO is iPO*NSGPO+1+iSG, and the codepoint for all subgroups in the PO is (iPO+1)*NSGPO+1-1; otherwise, the codepoint for the PO iPO is iPO.. Therefore, we could consider avoiding redundancy by not redefining these aspects here in 38.304. - Suggestion What if we simplify the text as follows: “The UE monitors one LP-WUS occasion per DRX cycle. A LO is a set of LP-WUS monitoring occasions (LP-WUS MOs) and is defined in clause 10.4C in TS 38.213[4]. In multi-beam operations, the UE assumes that the same LP-WUS is repeated in all transmitted beams and thus the selection of the beam(s) for the reception of the LP-WUS is up to UE implementation.” |  |
| NEC (W001) | 7.x LP-WUS monitoring 7.x.0 General If the UE detects LP-WUS and the LP-WUS is associated with the UE as specified in clause 10.xx in TS 38.213 [4], the UE monitors the associated PO as specified in clause 7.1 or monitors PEI as specified in clause 7.2, which is up to UE implementation if PEI is supported and related configuration is provided in system information. If UE does not detect a LP-WUS on the monitored LP-WUS occasion (LO) or the LP-WUS is not associated with the UE as specified in clause 10.xx in TS 38.213 [4], the UE is not required to monitor the associated PO as specified in clause 7.1. Comment: when we look at RAN1 spec, the similar wording is used: 10.4C PDCCH monitoring activation by WUS in RRC_IDLE/RRC_INACTIVE A paging occasion associated with a WUS occasion has index iPO=UE_ID mod N⋅NS+i_s mod NPOWO where NPOWO is a number of paging occasions associated with a WUS occasion, N, NS, iSG, and i_s are defined in [17, TS 38.304], and UE_ID is defined in clause 7.1 of [17, TS 38.304]. If a number of NSGPO subgroups per paging occasion, provided by subgroupNumber-PO-WUS, is NSGPO>1, the codepoint for the subgroup index iSG in a PO iPO is iPO*NSGPO+1+iSG, and the codepoint for all subgroups in the PO is (iPO+1)*NSGPO+1-1; otherwise, the codepoint for the PO iPO is iPO. If, in a WUS monitoring occasion, a UE determines a codepoint associated with the UE [17, TS 38.304], the UE performs PDCCH monitoring according to Type2-PDCCH CSS sets for the paging occasion associated with the WUS monitoring occasion when a time from the end of the WUS reception to the start of the PDCCH monitoring occasion is not smaller than the value of XYZ; otherwise, the UE is not required to perform the PDCCH monitoring. The UE may also perform PDCCH monitoring for Type2A-PDCCH CSS sets for DCI format 2_7, if provided. Comment: both RAN1 and RAN2 use the wording “associated” but not mention how to map to subgroup, it is a little ambiguous. Therefore, we suggest RAN2 to modify the spec as below: If the UE detects LP-WUS and the LP-WUS is associated with the subgroup the UE belongs to or is associated with all subgroups as specified in clause 10.4C in TS 38.213 [4], the UE monitors the associated PO as specified in clause 7.1 or monitors PEI as specified in clause 7.2, which is up to UE implementation if PEI is supported and related configuration is provided in system information. If UE does not detect a LP-WUS on the monitored LP-WUS occasion (LO) or the LP-WUS is not associated with the subgroup the UE belongs to and is not associated with all subgroups as specified in clause 10.4C in TS 38.213 [4], the UE is not required to monitor the associated PO as specified in clause 7.1. |  |
| NEC (W002) | 7.y.2 UE_ID based subgrouping for LP-WUS SubgroupID = (floor(UE_ID/(N*Ns*Np)) mod lp-SubgroupsNumForUEID) + (lp-SubgroupsNumPerPO – lp-SubgroupsNumForUEID), where: Np is the number of subgroupsNumForUEID for PEI, if configured and UE supports PEI; otherwise, Np is 1 lp-SubgroupsNumForUEID and lp-SubgroupsNumPerPO are the subgroup number for UE_ID based subgrouping for LP-WUS and the total subgroup number for LP-WUS, respectively Comment: no strong view, but think this is broadcast signalling. It would be better to modify as below (similar to the PEI description): Np is the number of subgroupsNumForUEID for PEI, if broadcasted in system information and UE supports PEI; otherwise, Np is 1 |  |
| Vivo (v001) | Proposed change: - LP-WUS UE may perform further relaxed serving cell and neighbouring cell measurement on MR as specified in clause 5.2.4.x.1 or serving cell measurement offloading from MR to LR as specified in clause 5.2.4.x.3. Reason: “further perform” may be mis-interpreted as LP-WUS UE should first perform legacy relaxation and further perform Rel-19 relaxation. The trueth is LP-WUS could perform either legacy relaxation (as in legacy) or Rel-19 further relaxation defined in RAN4. |  |
| Vivo (v002) | 5.2.4.x Relaxed measurement and measurement offloading for LP-WUS UE Reason: to keep the consistent with below description. |  |
| Vivo (v003) | 5.2.4.x.1 Relaxed measurement rules 5.2.4.x.2 Relaxed measurement criterion 5.2.4.x.3 Serving cell measurement offloading rules 5.2.4.x.4 Serving cell measurement offloading criterion Reason: as it is already mentioned “the LP-WUS UE” in title of 5.2.4.x |  |
| Vivo (V004) | 7.x.0 General The time location of an LO for UE’s PO is determined by a reference PF and the configured frame-level offset: - The reference PF is the start of the PF, or the first PF of the PF(s) (if mapping of POs from multiple PFs to one LO is configured), associated with the LO. The reference PF for the LO of a PO is provided by (SFN for PF) – floor(iPO/NS) * T/N, where SFN for PF is determined in clause 7.1, iPO is defined in clause 10.4C in TS 38.213[4], T, Ns, and N are determined in clause 7.1. - The frame-level offset between the LO and the reference PF is provided by lpwus-LoOffset in SIB1. Comment: The “reference PF” is not aligned with current RRC CR (from RAN1 RRC parameters), it should be “reference PF/PO” or “reference point”. Reason: to align with RRC. |  |
| Vivo (v005) | 7.x.0 General If the UE detects LP-WUS and the LP-WUS is associated with the UE as specified in clause 10.xx in TS 38.213 [4], the UE monitors the associated PO as specified in clause 7.1 or monitors PEI as specified in clause 7.2 if PEI is supported and related configuration is provided in system information, which is up to UE implementation. If UE does not detect a LP-WUS on the monitored LP-WUS occasion (LO) or the LP-WUS is not associated with the UE as specified in clause 10.xx in TS 38.213 [4], the UE is not required to monitor the associated PO as specified in clause 7.1. |  |
| Vivo (v006) | 7.x.1 These thresholds can be configured separately for LR measurments based on LP-SS and LR measurements based on SSB if a cell supports both measurement types as specified in TS 38.331 [3]. If UE supports both measurement types, it is up to UE implementation to choose whether LR measurments are based on LP-SS or based on SSB for the determination of the LP-WUS monitoring entry/exit conditions. Comments: this sentence should be moved after exit condition, as it is also applied to exit condition. |  |
| Vivo (v007) | 7.x.1 Condition for LP-WUS monitoring The entry condition for LP-WUS monitoring is fulfilled when: - Srxlev > SLP_WUS_EntryThresholdP_MR, and, - Qrxlevmeas_lr > QLP_WUS_EntryThresholdP_LR, if QLP_WUS_EntryThresholdP_LR is configured, and, - Squal > SLP_WUS_EntryThresholdQ_MR, if SLP_WUS_EntryThresholdQ_MR is configured, and - Qqualmeas_lr > QLP_WUS_EntryThresholdQ_LR, if QLP_WUS_EntryThresholdQ_LR is configured. The exit condition for LP-WUS monitoring is fulfilled when: - Qrxlevmeas_lr < QLP_WUS_ExitThresholdP_LR or, - Qqualmeas_lr < QLP_WUS_ExitThresholdQ_LR, if QLP_WUS_ExitThresholdQ_LR is configured. Where: Reason: to keep the consistent with other places. |  |
| Vivo (v008) | 7.y.1 CN assigned subgrouping 7.y.2 UE_ID based subgrouping Reason: as it is already mentioned “for LP-WUS” in title of 7.y. similar to what captured in 7.3 |  |
| HW (001) | “- LP-WUS UE may further perform relaxed serving cell and neighbouring cell measurement on MR as specified in clause 5.2.4.x.1 or serving cell measurement offloading from MR to LR as specified in clause 5.2.4.x.3.” Editorial: “measurement” to “measurements” |  |
| HW (002) | 5.2.4.x.3 Serving cell measurement offloading rules for LP-WUS UE LP-WUS UE may choose to perform serving cell measurement offloading (i.e., serving cell measurement fully offloaded to LR and no serving cell measurement via MR is required) according to requirements specified in TS 38.133 [8] if the entry condition for serving cell measurement offloading in clause 5.2.4.x.4 is fulfilled. LP-WUS UE is not required to perform serving cell measurement offloading according to requirements specified in TS 38.133 [8] if the exit condition for serving cell measurement offloading in clause 5.2.4.x.4 is fulfilled. Proposed change (editorial): ((i.e., serving cell measurement is fully offloaded to LR and no serving cell measurement via MR is required)) |  |
| HW (003) | 5.2.4.x.3 Serving cell measurement offloading rules for LP-WUS UE LP-WUS UE may choose to perform serving cell measurement offloading (i.e., serving cell measurement fully offloaded to LR and no serving cell measurement via MR is required) according to requirements specified in TS 38.133 [8] if the entry condition for serving cell measurement offloading in clause 5.2.4.x.4 is fulfilled. LP-WUS UE is not required to perform serving cell measurement offloading according to requirements specified in TS 38.133 [8] if the exit condition for serving cell measurement offloading in clause 5.2.4.x.4 is fulfilled. Proposed change: “LP-WUS UE is not required to perform should exit serving cell measurement offloading .....” |  |
| HW (004) | 7.x.0 General The UE may monitor LP-WUS in RRC_IDLE and RRC_INACTIVE states in order to reduce power consumption. If LP-WUS configuration is provided in system information, the UE in RRC_IDLE or RRC_INACTIVE state supporting LP-WUS (except for the UEs expecting MBS group notification) may start LP-WUS monitoring using LP-WUS parameters in system information according to the procedure described below if the entry condition in clause 7.x.1 is fulfilled. The UE may stop LP-WUS monitoring if the exit condition in clause 7.x.1 is fulfilled. If the UE detects LP-WUS and the LP-WUS is associated with the UE as specified in clause 10.xx in TS 38.213 [4], the UE monitors the associated PO as specified in clause 7.1 or monitors PEI as specified in clause 7.2, which is up to UE implementation if PEI is supported and related configuration is provided in system information. If UE does not detect a LP-WUS on the monitored LP-WUS occasion (LO) or the LP-WUS is not associated with the UE as specified in clause 10.xx in TS 38.213 [4], the UE is not required to monitor the associated PO as specified in clause 7.1. The UE monitors one LP-WUS occasion per DRX cycle. A LO is a set of LP-WUS monitoring occasions (LP-WUS MOs). In multi-beam operations, the UE assumes that the same LP-WUS is repeated in all transmitted beams and thus the selection of the beam(s) for the reception of the LP-WUS is up to UE implementation. Proposed changes (editorial): - “If the UE does not detect a LP-WUS on the monitored LP-WUS occasion (LO)” - “The UE monitors one LP-WUS occasion LO per DRX cycle” (motivation: as LO is already mentioned in the above paragraph) |  |
| HW (005) | 7.x.0 General “If more than one values are configured for lo-Offset, and if the gap between the LO associated with the largest offset” Editorial: “one value is” |  |
| Len001 | 5.2.4.9.x Relaxed measurement criterion for LP-WUS UE …….. Editor’s NOTE: The detailed parameters for RRM measurement relaxation for LP-WUS will be aligned with RRC specification. Editor’s NOTE: FFS for the metrics of serving cell RRM relaxation (i.e. RSRP and/or RSRQ). Editor’s NOTE: FFS serving cell quality by MR is existing Srxlev/ Squal or Qrxlevmeas/ Qqualmeas (i.e. measured value). Editor’s NOTE: FFS serving cell quality by LR is measured value. Editor’s NOTE: FFS on exit condition for serving cell RRM relaxation, e.g., whether a separate exit condition other than ‘not fulfilling the entry condition’ is needed, or whether exit condition include MR and/or LR-based measurements. Editor’s NOTE: FFS if the entry condition for serving cell RRM measurement relaxation is the same as neighbour cell RRM measurement relaxation. Editor’s NOTE: FFS whether/how to capture separate thresholds for different UE types (to be aligned with RRC specification). Editor’s NOTE: FFS relaxed measurement criteria is different from LP-WUS monitoring entry criteria. 5.2.4.9.z Serving cell measurement oOffloading measurement criterion for LP-WUS UE …… Editor’s NOTE: The detailed parameters for RRMserving cell measurement offloading will be aligned with RRC specification. Editor’s NOTE: FFS for the metrics of serving cellRRM measurement offloading. Editor’s NOTE: FFS serving cell quality by MR is existing Srxlev or Qrxlevmeas (i.e. measured value). Editor’s NOTE: FFS serving cell quality by LR is measured value. Editor’s NOTE: FFS whether/how to capture separate thresholds for different UE types (to be aligned with RRC specification). Comments: Above FFS note with strikethrough can be removed and replaced by corresponding RAN2 agreements. |  |
| Len 002 | 5.2.4.9.y Serving cell oOffloading measurement rules LP-WUS UE may choose to perform RRM serving cell measurement offloading (i.e., serving cell measurement fully offloaded to LR and no serving cell measurement via MR is required) according to requirements specified in TS 38.133 [8] if the entry condition for RRM measurementserving cell measurement offloading in clause 5.2.4.9.z is fulfilled. LP-WUS UE is not allowed required to perform serving cellRRM measurement offloading according to requirements specified in TS 38.133 [8] if the exit condition for serving cellRRM measurement offloading in clause 5.2.4.9.z is fulfilled. 5.2.4.9.z Serving cell measurement oOffloading measurement criterion for LP-WUS UE The entry condition for MR RRM serving cell RRM measurerement offloading is fulfilled when: …… Comments: Fixed wording issues. For 5,2,4,9.z added MR to align with 5.2.4.9. x |  |
| OPPO001 | - Qrxlevmeas_lr = current measured cell RX level value of the serving cell based on LR (RSRP). - Qqualmeas_lr = current measured cell quality value of the serving cell based on LR (RSRQ). Comment: Since these two parameters are measured by LR, maybe we need to use LR-RSRP and LR-RSRQ to avoid the confusion. |  |

  - Open issue list

    - Closed open issue

RRM relaxation/offloading

Open issue 38304-4: FFS relaxed measurement criteria/RRM offloading criteria is different from LP-WUS monitoring criteria. (Same as the open issue in RRC, i.e., FFS on whether/how to reduce the threshold number for LP-WUS/WUR)

LP-WUS in idle/inactive mode

Open issue 38304-5: FFS the UEs expecting MBS group notification should monitor its PO to receive the MBS group notification regardless of LP-WUS.

    - Remaining open issues on LP-WUS in idle/inactive mode

| Index | Issue description | Rapporteur suggestion |
| --- | --- | --- |
| 38304-8 | Whether LP-WUS is only used in the last used cell or in any cell | Issue Type: not essential not important How to address it: can be discussed based on companies’ contribution |
| 38304-9 | FFS the SubgroupID for LP-WUS used outside CN PTW in RRC_INACTIVE state with CN configured PTW Note: the open issue was discussed online in RAN2#130 (R2-2504677) without conclusion. | Issue Type: not essential not important How to address it: can be discussed based on companies’ contribution |
| 38304-10 | FFS whether/how LP-WUS with SDT is supported Note: the open issue was discussed online in RAN2#130 (R2-2504264) without conclusion. | Issue Type: not essential not important How to address it: can be discussed based on companies’ contribution |

    - Remaining open issues on RRM relaxation/offloading

| Index | Issue description | Rapporteur suggestion |
| --- | --- | --- |
| 38304-1 | FFS on the terminology LP-WUS UE | Issue Type: not essential not important How to address it: can be discussed in clause 2.1 as an easily addressed open issue. |
| 38304-2 | FFS (if needed) on enhancements based on R16 criteria (e.g., based on the LR measurements) for the case when MR serving cell measurement results are not available. | Issue Type: not essential not important How to address it: can be discussed based on companies’ contribution |
| 38304-3 | FFS on exit condition for serving cell RRM relaxation, e.g., whether a separate exit condition other than ‘not fulfilling the entry condition’ is needed, or whether exit condition include MR and/or LR-based measurements. (Same as the open issue in RRC, i.e., FFS on exit condition for serving cell RRM relaxation) | Issue Type: not essential but important How to address it: can be discussed based on companies’ contribution |
| 38304-6 | Whether UE low mobility criterion or stationary criterion should be considered for RRM relaxation/offloading. (Same as the open issue in RRC, i.e., FFS on low mobility criteria) | Issue Type: not essential not important How to address it: can be discussed based on companies’ contribution |
| 38304-7 | Whether Relaxed measurement and offloading measurement can be performed when there is NR inter-frequency and/or NR inter-RAT frequency with reselection priority higher than that of the camped frequency. (Same as the open issue in RRC, i.e., FFS on whether/how RRM relaxation is applicable for high priority frequency) | Issue Type: not essential but important How to address it: can be discussed in clause 2.1 as an easily addressed open issue. |
| 38304-11 | FFS on the determination of RRM measurement relaxation/offloading conditions if UE supports both measurement types | Issue Type: not essential not important How to address it: can be discussed in clause 2.1 as an easily addressed open issue |

    - Other open issues

In addition to the above open issues, please provide your comments on any other RAN2 open issues of 38.304 running CR for LP-WUS, and Rapporteur will response.

| Company | Open issue | Rapporteur response |
| --- | --- | --- |
| Samsung | Whether/ How to align terminologies among WGs. Now RAN1 has finalized the LP-WUS CRs in 38.212, 213, 214, and 215 with using the following terminologies: - LPSS Low power synchronization signal - WUS Wake-Up Signal - WUR Wake-Up Receiver Additionally, the LS R1-2504888 from RAN1 explicitly states that RAN1 does not intend to specify LR or MR in Release 19. As far as we understand, RAN1 views the LP-WUS function as a sub-functional UE behavior that a Rel-19 UE with the capability could support, rather than introducing a significant burden, such as defining a new LP-RAT. Therefore, they are reluctant to explicitly distinguish LR and MR. Based on this context, we suggest discussing the following proposals: P1. Whether to align the terminologies among TSs/WGs. If the answer of P1 is yes, we propose: P2. Modify the following terminologies (Straightforward changes) - LP-WUS → WUS - LP-SS → LPSS - LO (LP-WUS Occasion) → WUS Occasion. P3. Discuss how to modify/remove the LR and MR. As an example, we could consider the following changes: - LR → WUR - MR → removed P4. Modify parameter names such as: - Q_rxlevmeas_lr → Q_rxlevmeas_wur - based on LR → based on WUR - lpxxx → wurxx - … |  |

Summary:

- Conclusion

According to feedback on clause 2.1, we propose:

And the following stage 3 open issues of 38.304 running CR for LP-WUS are identified: