3GPP TSG-WG2 # 131	R2-250xxxx

# Bengaluru, India, 25 - 29 August 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.321 | CR | DraftCR | rev | - | Current version: | 18.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | MAC Running CR for LP-WUS |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Apple |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_LPWUS-Core |  |  |  |  |  | Date: |  |  | 2025-07-07 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Introduction of Rel-19 LP-WUS in TS 38.321 |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduction of Rel-19 LP-WUS in TS 38.321 - Support of LP-WUS in CONNECTED state |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Rel-19 LP-WUS is not captured in TS 38.321. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 3.2, 5.7 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | Y |  | Other core specifications |  |  |  | TS 38.300 CRxxxx TS 38.331 CRxxxx |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | R2-2502307, initial version based on version 18.5.0 R2-2503807, updated version to include the RAN2#129bis agreement R2-250xxxx, updated version to include the RAN2#130 agreement, based on version 18.6.0 |  |  |  |  |  |  |  |  |

Start of change

## 3.2	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1].

A2X	Aircraft-to-Everything

AP	Aperiodic

BFR	Beam Failure Recovery

BRID	Broadcast Remote Identification

BSR	Buffer Status Report

BWP	Bandwidth Part

CE	Control Element

CG	Cell Group

CG-SDT	Configured Grant-based SDT

CI-RNTI	Cancellation Indication RNTI

CSI	Channel State Information

CSI-IM	CSI Interference Measurement

CSI-RS	CSI Reference Signal

CS-RNTI	Configured Scheduling RNTI

DAA	Detect And Avoid

DAPS	Dual Active Protocol Stack

DCP	DCI with CRC scrambled by PS-RNTI

DL-PRS	DownLink-Positioning Reference Signal

DSR	Delay Status Report

DTX	Discontinuous Transmission

G-CS-RNTI	Group Configured Scheduling RNTI

G-RNTI	Group RNTI

IAB	Integrated Access and Backhaul

INT-RNTI	Interruption RNTI

LBT	Listen Before Talk

LCG	Logical Channel Group

LCP	Logical Channel Prioritization

LTM	L1/L2 Triggered Mobility

LP-WUS	Low Power-Wake up Signal

MBS	Multicast/Broadcast Services

MCCH	MBS Control Channel

MCCH-RNTI	MBS Control Channel RNTI

MCG	Master Cell Group

MO-SDT	Mobile Originated SDT

MPE	Maximum Permissible Exposure

MTCH	MBS Traffic Channel

MT-SDT	Mobile Terminated SDT

N3C	Non-3GPP Connection

NCD-SSB	Non Cell Defining SSB

NCR	Network-Controlled Repeater

NSAG	Network Slice AS Group

NUL	Normal Uplink

NZP CSI-RS	Non-Zero Power CSI-RS

PDB	Packet Delay Budget

PEI-RNTI	Paging Early Indication RNTI

PHR	Power Headroom Report

PQI	PC5 QoS Identifier

PS-RNTI	Power Saving RNTI

PSI	PDU Set Importance

PTAG	Primary Timing Advance Group

PTM	Point to Multipoint

PTP	Point to Point

QCL	Quasi-colocation

PPW	PRS Processing Window

PRS	Positioning Reference Signal

RA-SDT	Random Access-based SDT

RRH	Remote Radio Head

RS	Reference Signal

SCG	Secondary Cell Group

SDT	Small Data Transmission

SFI-RNTI	Slot Format Indication RNTI

SI	System Information

SL-PRS-CS-RNTI	SL-PRS-Configured Scheduling-RNTI

SL-PRS-RNTI	SL-PRS-RNTI

SL-CS-RNTI	Sidelink-Configured Scheduling-RNTI

SL-PRS	Sidelink-PRS

SL-RNTI	Sidelink-RNTI

SpCell	Special Cell

SP	Semi-Persistent

SP-CSI-RNTI	Semi-Persistent CSI RNTI

SPS	Semi-Persistent Scheduling

SR	Scheduling Request

SRI	SRS Resource Indicator

SS	Synchronization Signals

SSB	Synchronization Signal Block

STAG	Secondary Timing Advance Group

STx2P	Simultaneous Transmission with 2 Panels

SUL	Supplementary Uplink

TAG	Timing Advance Group

TCI	Transmission Configuration Indicator

TPC-SRS-RNTI	Transmit Power Control-Sounding Reference Signal-RNTI

TRIV	Time Resource Indicator Value

TRP	Transmit/Receive Point

TRS	CSI-RS for tracking

U2N	UE-to-Network

U2U	UE-to-UE

UCI	Uplink Control Information

UTO-UCI	Unused Transmission Occasion - UCI

UTW	Uplink Time Window

V2X	Vehicle-to-Everything

ZP CSI-RS	Zero Power CSI-RS

## 5.7	Discontinuous Reception (DRX)

The MAC entity may be configured by RRC with a DRX functionality that controls the UE's PDCCH monitoring activity for the MAC entity's C-RNTI, CI-RNTI, CS-RNTI, INT-RNTI, SFI-RNTI, SP-CSI-RNTI, TPC-PUCCH-RNTI, TPC-PUSCH-RNTI, TPC-SRS-RNTI, AI-RNTI, SL-RNTI, SL-CS-RNTI, SL-PRS-RNTI, SL-PRS-CS-RNTI, SL Semi-Persistent Scheduling V-RNTI and cellDTRX-RNTI. When using DRX operation, the MAC entity shall also monitor PDCCH according to requirements found in other clauses of this specification. When in RRC_CONNECTED, if DRX is configured, for all the activated Serving Cells, the MAC entity may monitor the PDCCH discontinuously using the DRX operation specified in this clause; otherwise the MAC entity shall monitor the PDCCH as specified in TS 38.213 [6].

NOTE 1:	Void

RRC controls DRX operation by configuring the following parameters:

-	drx-onDurationTimer: the duration at the beginning of a DRX cycle;

-	drx-SlotOffset: the delay before starting the drx-onDurationTimer;

-	drx-InactivityTimer: the duration after the PDCCH occasion in which a PDCCH indicates a new UL, DL or SL transmission for the MAC entity;

-	drx-RetransmissionTimerDL (per DL HARQ process except for the broadcast process): the maximum duration until a DL retransmission is received;

-	drx-RetransmissionTimerUL (per UL HARQ process): the maximum duration until a grant for UL retransmission is received;

-	drx-LongCycleStartOffset: the Long DRX cycle and drx-StartOffset which defines the subframe where the Long and Short DRX cycle starts;

-	drx-NonIntegerLongCycleStartOffset (optional): the Long DRX cycle and drx-StartOffset which defines the subframe where the Long and Short DRX cycle start, when the length of the Long DRX cycle and/or the short DRX cycle is not an integer;

-	drx-ShortCycle (optional): the Short DRX cycle;

-	drx-NonIntegerShortCycle (optional): the Short DRX cycle whose length is not an integer;

-	drx-ShortCycleTimer (optional): the duration the UE shall follow the Short DRX cycle;

-	drx-HARQ-RTT-TimerDL (per DL HARQ process except for the broadcast process): the minimum duration before a DL assignment for HARQ retransmission is expected by the MAC entity;

-	drx-HARQ-RTT-TimerUL (per UL HARQ process): the minimum duration before a UL HARQ retransmission grant is expected by the MAC entity;

-	drx-RetransmissionTimerSL (per sidelink process): the maximum duration until a grant for SL retransmission is received;

-	drx-HARQ-RTT-TimerSL (per sidelink process): the minimum duration before an SL retransmission grant is expected by the MAC entity;

-	drx-LastTransmissionUL (optional): the configuration to start drx-HARQ-RTT-TimerUL after the last transmission within a bundle;

-	ps-Wakeup (optional): the configuration to start associated drx-onDurationTimer in case DCP is monitored but not detected;

-	ps-TransmitOtherPeriodicCSI (optional): the configuration to report periodic CSI that is not L1-RSRP on PUCCH during the time duration indicated by drx-onDurationTimer in case DCP is configured but associated drx-onDurationTimer is not started;

-	ps-TransmitPeriodicL1-RSRP (optional): the configuration to transmit periodic CSI that is L1-RSRP on PUCCH during the time duration indicated by drx-onDurationTimer in case DCP is configured but associated drx-onDurationTimer is not started;

-	downlinkHARQ-FeedbackDisabled (optional): the configuration to disable HARQ feedback per DL HARQ process;

-	uplinkHARQ-Mode (optional): the configuration to set HARQmodeA or HARQmodeB per UL HARQ process;

-	disableCG-RetransmissionMonitoring (optional): the configuration to disable starting drx-HARQ-RTT-TimerUL for UL transmission over a configured uplink grant;

-	drx-TimeReferenceSFN (optional): the configuration to indicate how UE initializes of DRX_SFN_COUNTER;

-	lpwus-TransmitOtherPeriodicCSI (optional): the configuration to report periodic CSI that is not L1-RSRP on PUCCH during the time duration indicated by drx-onDurationTimer in case LP-WUS is configured but associated drx-onDurationTimer is not started;

-	lpwus-TransmitPeriodicL1-RSRP (optional): the configuration to transmit periodic CSI that is L1-RSRP on PUCCH during the time duration indicated by drx-onDurationTimer in case LP-WUS is configured but associated drx-onDurationTimer is not started;

-	lpwus-PDCCHMonitoringTimer (optional): the duration of the UE's PDCCH monitoring activity for the MAC entity after receiving the LP-WUS indication.

Editor’s NOTE:	The parameter name may be further updated to align with the name used in RRC specification.

The following UE variable is used for the DRX operation if drx-NonIntegerLongCycleStartOffset is configured:

-	DRX_SFN_COUNTER: the counter that increments when SFN changes to 0. The maximum value of this counter is at least 65535.

Serving Cells of a MAC entity may be configured by RRC in two DRX groups with separate DRX parameters. When RRC does not configure a secondary DRX group, there is only one DRX group and all Serving Cells belong to that one DRX group. When two DRX groups are configured, each Serving Cell is uniquely assigned to either of the two groups. The DRX parameters that are separately configured for each DRX group are: drx-onDurationTimer, drx-InactivityTimer. The DRX parameters that are common to the DRX groups are: drx-SlotOffset, drx-RetransmissionTimerDL, drx-RetransmissionTimerUL, drx-LongCycleStartOffset, drx-NonIntegerLongCycleStartOffset, drx-ShortCycle (optional), drx-NonIntegerShortCycle (optional), drx-ShortCycleTimer (optional), drx-HARQ-RTT-TimerDL, and drx-HARQ-RTT-TimerUL.

When DRX is configured, the Active Time for Serving Cells in a DRX group includes the time while:

-	drx-onDurationTimer, drx-InactivityTimer or lpwus-PDCCHMonitoringTimer configured for the DRX group is running; or

Editor’s NOTE:	FFS whether lpwus-PDCCHMonitoringTimer is configured per DRX group or common to DRX groups.

-	drx-RetransmissionTimerDL, drx-RetransmissionTimerUL or drx-RetransmissionTimerSL is running on any Serving Cell in the DRX group; or

-	ra-ContentionResolutionTimer (as described in clause 5.1.5) or msgB-ResponseWindow (as described in clause 5.1.4a) is running; or

-	a Scheduling Request is sent on PUCCH and is pending (as described in clause 5.4.4 or 5.22.1.5). If this Serving Cell is part of a non-terrestrial network, the Active Time is started after the Scheduling Request transmission that is performed when the SR_COUNTER is 0 for all the SR configurations with pending SR(s) plus the UE-gNB RTT; or

-	a PDCCH indicating a new transmission addressed to the C-RNTI of the MAC entity has not been received after successful reception of a Random Access Response for the Random Access Preamble not selected by the MAC entity among the contention-based Random Access Preamble (as described in clauses 5.1.4 and 5.1.4a); or

-	there is an ongoing RACH-less LTM cell switch; or

-	there is an ongoing RACH-less handover in a terrestrial network.

The following MAC timers are used for DRX operation in a non-terrestrial network:

-	HARQ-RTT-TimerDL-NTN (per DL HARQ process configured with HARQ feedback enabled): the minimum duration before a DL assignment for HARQ retransmission is expected by the MAC entity;

-	HARQ-RTT-TimerUL-NTN (per UL HARQ process configured with HARQModeA): the minimum duration before a UL HARQ retransmission grant is expected by the MAC entity.

When DRX is not configured and multicast DRX is configured for a G-RNTI or G-CS-RNTI, the MAC entity shall:

1>	monitor the PDCCH as specified in TS 38.213 [6];

1>	if a MAC PDU is received in a configured downlink assignment for unicast; or

1>	if the PDCCH indicates a DL unicast transmission:

2>	stop the drx-RetransmissionTimerDL-PTM for the corresponding HARQ process.

When DRX is configured, the MAC entity shall:

1>	if a MAC PDU is received in a configured downlink assignment for unicast:

2>	if this Serving Cell is configured with downlinkHARQ-FeedbackDisabled:

3>	if the corresponding HARQ process is configured with HARQ feedback enabled:

4>	set HARQ-RTT-TimerDL-NTN for the corresponding HARQ process equal to drx-HARQ-RTT-TimerDL plus the latest available UE-gNB RTT value;

4>	start the HARQ-RTT-TimerDL-NTN for the corresponding HARQ process in the first symbol after the end of the corresponding transmission carrying the DL HARQ feedback.

2>	else:

3>	start the drx-HARQ-RTT-TimerDL for the corresponding HARQ process in the first symbol after the end of the corresponding transmission carrying the DL HARQ feedback.

NOTE 1a:	Void.

NOTE 1b:	Void.

2>	stop the drx-RetransmissionTimerDL for the corresponding HARQ process;

2>	stop the drx-RetransmissionTimerDL-PTM for the corresponding HARQ process.

1>	if a MAC PDU is transmitted in a configured uplink grant and LBT failure indication is not received from lower layers:

2>	if this Serving Cell is configured with uplinkHARQ-Mode:

3>	if the corresponding HARQ process is configured as HARQModeA:

4>	set HARQ-RTT-TimerUL-NTN for the corresponding HARQ process equal to drx-HARQ-RTT-TimerUL plus the latest available UE-gNB RTT value;

4>	if drx-LastTransmissionUL is configured:

5>	start the HARQ-RTT-TimerUL-NTN for the corresponding HARQ process in the first symbol after the end of the last transmission (within a bundle) of the corresponding PUSCH transmission.

4>	else:

5>	start the HARQ-RTT-TimerUL-NTN for the corresponding HARQ process in the first symbol after the end of the first transmission (within a bundle) of the corresponding PUSCH transmission.

2>	else:

3>	if disableCG-RetransmissionMonitoring is not configured for the configured uplink grant:

4>	if drx-LastTransmissionUL is configured:

5>	start the drx-HARQ-RTT-TimerUL for the corresponding HARQ process in the first symbol after the end of the last transmission (within a bundle) of the corresponding PUSCH transmission.

4>	else:

5>	start the drx-HARQ-RTT-TimerUL for the corresponding HARQ process in the first symbol after the end of the first transmission (within a bundle) of the corresponding PUSCH transmission.

2>	stop the drx-RetransmissionTimerUL for the corresponding HARQ process at the first transmission (within a bundle) of the corresponding PUSCH transmission.

1>	if a MAC PDU is transmitted in a configured sidelink grant:

2>	if the PUCCH resource is configured:

3>	start the drx-HARQ-RTT-TimerSL for the corresponding HARQ process in the first symbol after the end of the corresponding PUCCH transmission carrying the SL HARQ feedback; or

3>	start the drx-HARQ-RTT-TimerSL for the corresponding HARQ process in the first symbol after the end of the corresponding PUCCH resource for the SL HARQ feedback when the PUCCH is not transmitted;

3>	stop the drx-RetransmissionTimerSL for the corresponding HARQ process.

2>	else:

3>	start the drx-HARQ-RTT-TimerSL for the corresponding HARQ process at the first symbol after the end of the corresponding PSSCH transmission;

3>	stop the drx-RetransmissionTimerSL for the corresponding HARQ process.

1>	if a drx-HARQ-RTT-TimerDL expires:

2>	if the data of the corresponding HARQ process was not successfully decoded:

3>	start the drx-RetransmissionTimerDL for the corresponding HARQ process in the first symbol after the expiry of drx-HARQ-RTT-TimerDL.

1>	if a HARQ-RTT-TimerDL-NTN expires:

2>	if the data of the corresponding HARQ process was not successfully decoded:

3>	start the drx-RetransmissionTimerDL for the corresponding HARQ process in the first symbol after the expiry of HARQ-RTT-TimerDL-NTN.

1>	if a drx-HARQ-RTT-TimerUL expires:

2>	start the drx-RetransmissionTimerUL for the corresponding HARQ process in the first symbol after the expiry of drx-HARQ-RTT-TimerUL.

1>	if a HARQ-RTT-TimerUL-NTN expires:

2>	start the drx-RetransmissionTimerUL for the corresponding HARQ process in the first symbol after the expiry of HARQ-RTT-TimerUL-NTN.

1>	if a drx-HARQ-RTT-TimerSL expires:

2>	if a HARQ NACK feedback for the corresponding HARQ process is transmitted on PUCCH; or

2>	if a HARQ NACK feedback for the corresponding HARQ process is generated but not transmitted on PUCCH; or

2>	if the PUCCH resource is not configured for the SL grant:

3>	start the drx-RetransmissionTimerSL for the corresponding HARQ process in the first symbol after the expiry of drx-HARQ-RTT-TimerSL.

NOTE 1c:	The UE handles the drx-RetransmissionTimerSL operation when sl-PUCCH-Config is configured by RRC but PUCCH resource is not scheduled same as when sl-PUCCH-Config is not configured.

1>	if a DRX Command MAC CE indicated by PDCCH addressed to C-RNTI or CS-RNTI, or by a configured downlink assignment for unicast transmission or a Long DRX Command MAC CE is received:

2>	stop drx-onDurationTimer for each DRX group;

2>	stop drx-InactivityTimer for each DRX group;

2>	stop lpwus-PDCCHMonitoringTimer for each DRX group.

1>	if drx-InactivityTimer for a DRX group expires:

2>	if the Short DRX cycle is configured:

3>	start or restart drx-ShortCycleTimer for this DRX group in the first symbol after the expiry of drx-InactivityTimer;

3>	use the Short DRX cycle for this DRX group.

2>	else:

3>	use the Long DRX cycle for this DRX group.

1>	if a DRX Command MAC CE indicated by PDCCH addressed to C-RNTI or CS-RNTI, or by a configured downlink assignment for unicast transmission is received

2>	if the Short DRX cycle is configured:

3>	start or restart drx-ShortCycleTimer for each DRX group in the first symbol after the end of DRX Command MAC CE reception;

3>	use the Short DRX cycle for each DRX group.

2>	else:

3>	use the Long DRX cycle for each DRX group.

1>	if drx-ShortCycleTimer for a DRX group expires:

2>	use the Long DRX cycle for this DRX group.

1>	if a Long DRX Command MAC CE is received:

2>	stop drx-ShortCycleTimer for each DRX group;

2>	use the Long DRX cycle for each DRX group.

1>	if the drx-NonIntegerLongCycleStartOffset is configured:

2>	increment DRX_SFN_COUNTER by 1 in the first symbol of a slot in which SFN changes to 0;

2>	if DRX is (re-)configured by RRC:

3>	if drx-TimeReferenceSFN is included in the RRC (re-)configuration which is received during the first half of a hyper frame (i.e., SFN is between 0 and 511):

4>	set DRX_SFN_COUNTER to 1.

3>	else:

4>	set DRX_SFN_COUNTER to 0.

1>	if the Short DRX cycle is used for a DRX group and the drx-NonIntegerShortCycle is not configured, and [(SFN × 10) + subframe number] modulo (drx-ShortCycle) = (drx-StartOffset) modulo (drx-ShortCycle); or

1>	if the Short DRX cycle is used for a DRX group and the drx-NonIntegerShortCycle is configured, and floor([(DRX_SFN_COUNTER × 10240) + (SFN × 10) + subframe number  drx-StartOffset] modulo (drx-NonIntegerShortCycle)) = 0:

2> if the lpwus-PDCCHMonitoringTimer is not configured:

3>	start drx-onDurationTimer for this DRX group after drx-SlotOffset from the beginning of the subframe.

Editor’s NOTE:	The case where lpwus-PDCCHMonitoringTimer is not configured includes legacy DRX operation and LP-WUS option 1-1, but not LP-WUS option 1-2.

1>	if the Long DRX cycle is used for a DRX group and the drx-NonIntegerLongCycleStartOffset is not configured, and [(SFN × 10) + subframe number] modulo (drx-LongCycle) = drx-StartOffset; or

1>	if the Long DRX cycle is used for a DRX group and the drx-NonIntegerLongCycleStartOffset is configured, and floor([(DRX_SFN_COUNTER × 10240) + (SFN × 10) + subframe number] modulo (drx-NonIntegerLongCycle)) = drx-StartOffset:

2>	if DCP monitoring is configured for the active DL BWP as specified in TS 38.213 [6], clause 10.3:

3>	if DCP indication associated with the current DRX cycle received from lower layer indicated to start drx-onDurationTimer, as specified in TS 38.213 [6]; or

3>	if all DCP occasion(s) in time domain, as specified in TS 38.213 [6], associated with the current DRX cycle occurred in Active Time considering grants/assignments/DRX Command MAC CE/Long DRX Command MAC CE received and Scheduling Request sent until 4 ms prior to start of the last DCP occasion, or during a measurement gap, or when the MAC entity monitors for a PDCCH transmission on the search space indicated by recoverySearchSpaceId of the SpCell identified by the C-RNTI while the ra-ResponseWindow is running (as specified in clause 5.1.4); or

3>	if ps-Wakeup is configured with value true and DCP indication associated with the current DRX cycle has not been received from lower layers:

4>	start drx-onDurationTimer after drx-SlotOffset from the beginning of the subframe.

2>	else if LP-WUS monitoring is configured:

3>	if lpwus-PDCCHMonitoringTimer is not configured:

4>	if LP-WUS indication associated with the current DRX cycle received from lower layer indicated to start drx-onDurationTimer, as specified in TS 38.213 [6]; or

4>	if all LP-WUS monitoring occasion(s) in time domain, as specified in TS 38.213 [6], associated with the current DRX cycle occurred in Active Time considering grants/assignments/DRX Command MAC CE/Long DRX Command MAC CE received and Scheduling Request sent until 4 ms prior to start of the last LP-WUS occasion, or during a measurement gap, or when the MAC entity monitors for a PDCCH transmission on the search space indicated by recoverySearchSpaceId of the SpCell identified by the C-RNTI while the ra-ResponseWindow is running (as specified in clause 5.1.4):

5>	start drx-onDurationTimer after drx-SlotOffset from the beginning of the subframe.

Editor’s NOTE:	The case where LP-WUS monitoring is configured without lpwus-PDCCHMonitoringTimer is LP-WUS Option 1-1.

Editor’s NOTE:	The DRX operation in LP-WUS Option 1-1 takes DCP description as baseline.

Editor’s NOTE:	The working assumption for UE operation under collision for Option 1-1 needs to be confirmed.

2>	else:

3>	start drx-onDurationTimer for this DRX group after drx-SlotOffset from the beginning of the subframe.

1>	if LP-WUS monitoring is configured and the lpwus-PDCCHMonitoringTimer for this DRX group is configured

2>	if LP-WUS indication is received from lower layer indicated to start lpwus-PDCCHMonitoringTimer, as specified in TS 38.213 [6]:

3>	start lpwus-PDCCHMonitoringTimer from the beginning of the subframe indicated from lower layer.

Editor’s NOTE:	The case where lpwus-PDCCHMonitoringTimer is configured is LP-WUS Option 1-2.

Editor’s NOTE:	The LP-WUS based DRX model is that LP-WUS monitoring and sending LP-WUS indication (together with the timepoint to start timer in Option 1-2) to MAC is captured in RAN1 spec (38.213), and the DRX operation based on the LP-WUS indication is captured in MAC spec.

Editor’s NOTE:	The relationship between UE's LP-WUS monitoring and DRX active time is assumed to be reflected in RAN1 spec (38.213), so we will not capture this part in MAC spec.

Editor’s NOTE:	FFS in Option 1-2 whether the UE should start the lpwus_PDCCHMonitoringTimer (as if LP-WUS was detected) when the UE is not able to monitor the LP-WUS occasion(s).

NOTE 2:	In case of unaligned SFN across carriers in a cell group, the SFN of the SpCell is used to calculate the DRX duration.

1>	if a DRX group is in Active Time:

2>	monitor the PDCCH on the Serving Cells in this DRX group as specified in TS 38.213 [6];

2>	if the PDCCH indicates a DL transmission; or

2>	if the PDCCH indicates a one-shot HARQ feedback as specified in clause 9.1.4 of TS 38.213 [6]; or

2>	if the PDCCH indicates a retransmission of HARQ feedback as specified in clause 9.1.5 of TS 38.213 [6]:

3>	if this Serving Cell is configured with downlinkHARQ-FeedbackDisabled:

4>	if at least one of the corresponding HARQ process(es) is configured with HARQ feedback enabled:

5>	set HARQ-RTT-TimerDL-NTN for the corresponding HARQ process(es) equal to drx-HARQ-RTT-TimerDL plus the latest available UE-gNB RTT value;

5>	if the UE is configured with one-shot HARQ Feedback:

6>	start or restart the HARQ-RTT-TimerDL-NTN for the corresponding HARQ process(es) whose HARQ feedback is enabled and reported in the first symbol after the end of the corresponding transmission carrying the DL HARQ feedback.

5>	else:

6>	start the HARQ-RTT-TimerDL-NTN for the corresponding HARQ process in the first symbol after the end of the corresponding transmission carrying the DL HARQ feedback.

3>	else:

4>	start or restart the drx-HARQ-RTT-TimerDL for the corresponding HARQ process(es) whose HARQ feedback is reported in the first symbol after the end of the corresponding transmission carrying the DL HARQ feedback.

NOTE 3:	When HARQ feedback is postponed by PDSCH-to-HARQ_feedback timing indicating an inapplicable k1 value, as specified in TS 38.213 [6], the corresponding transmission opportunity to send the DL HARQ feedback is indicated in a later PDCCH requesting the HARQ-ACK feedback.

3>	stop the drx-RetransmissionTimerDL for the corresponding HARQ process(es) whose HARQ feedback is reported;

3>	stop the drx-RetransmissionTimerDL-PTM for the corresponding HARQ process;

3>	if the PDSCH-to-HARQ_feedback timing indicate an inapplicable k1 value as specified in TS 38.213 [6]:

4>	start the drx-RetransmissionTimerDL in the first symbol after the (end of the last) PDSCH transmission (within a bundle) for the corresponding HARQ process.

2>	if the PDCCH indicates a UL transmission:

3>	if this Serving Cell is configured with uplinkHARQ-Mode:

4>	if the corresponding HARQ process is configured as HARQModeA:

5>	set HARQ-RTT-TimerUL-NTN for the corresponding HARQ process equal to drx-HARQ-RTT-TimerUL plus the latest available UE-gNB RTT value;

5>	if drx-LastTransmissionUL is configured:

6>	start the HARQ-RTT-TimerUL-NTN for the corresponding HARQ process in the first symbol after the end of the last transmission (within a bundle) of the corresponding PUSCH transmission.

5>	else:

6>	start the HARQ-RTT-TimerUL-NTN for the corresponding HARQ process in the first symbol after the end of the first transmission (within a bundle) of the corresponding PUSCH transmission.

3>	else:

4>	if drx-LastTransmissionUL is configured:

5>	start the drx-HARQ-RTT-TimerUL for the corresponding HARQ process in the first symbol after the end of the last transmission (within a bundle) of the corresponding PUSCH transmission.

4>	else:

5>	start the drx-HARQ-RTT-TimerUL for the corresponding HARQ process in the first symbol after the end of the first transmission (within a bundle) of the corresponding PUSCH transmission.

3>	stop the drx-RetransmissionTimerUL for the corresponding HARQ process.

2>	if the PDCCH indicates an SL transmission:

3>	if the PUCCH resource is configured:

4>	start the drx-HARQ-RTT-TimerSL for the corresponding HARQ process in the first symbol after the end of the corresponding PUCCH transmission carrying the SL HARQ feedback; or

4>	start the drx-HARQ-RTT-TimerSL for the corresponding HARQ process in the first symbol after the end of the corresponding PUCCH resource for the SL HARQ feedback when the PUCCH is not transmitted;

4>	stop the drx-RetransmissionTimerSL for the corresponding HARQ process.

3>	else:

4>	start the drx-HARQ-RTT-TimerSL for the corresponding HARQ process at the first symbol after end of PDCCH occasion;

4>	stop the drx-RetransmissionTimerSL for the corresponding HARQ process.

2>	if the PDCCH indicates a new transmission (DL, UL or SL) on a Serving Cell in this DRX group:

3>	start or restart drx-InactivityTimer for this DRX group in the first symbol after the end of the PDCCH reception.

NOTE 3a:	A PDCCH indicating activation of SPS, configured grant type 2, or configured sidelink grant of configured grant Type 2 is considered to indicate a new transmission.

NOTE 3b:	If the PDCCH reception includes two PDCCH candidates from corresponding search spaces, as described in clause 10.1 in TS 38.213 [6], start or restart drx-InactivityTimer for this DRX group in the first symbol after the end of the PDCCH candidate that ends later in time.

2>	if a HARQ process receives downlink feedback information and acknowledgement is indicated:

3>	stop the drx-RetransmissionTimerUL for the corresponding HARQ process.

1>	if DCP monitoring is configured for the active DL BWP as specified in TS 38.213 [6], clause 10.3, or if LP-WUS monitoring is configured as specified in TS 38.213 [6], clause 10.X; and

1>	if the current symbol n occurs within drx-onDurationTimer duration; and

1>	if drx-onDurationTimer associated with the current DRX cycle is not started as specified in this clause:

2>	if the MAC entity would not be in Active Time considering grants/assignments/DRX Command MAC CE/Long DRX Command MAC CE received and Scheduling Request sent until 4 ms prior to symbol n when evaluating all DRX Active Time conditions as specified in this clause; and

2>	if allowCSI-SRS-Tx-MulticastDRX-Active is not configured, or if cfr-ConfigMulticast is not configured for any of the active BWP(s) of the Serving Cell(s), or if all multicast DRXes would not be in Active Time considering multicast assignments/DRX Command MAC CE for MBS multicast received until 4 ms prior to symbol n when evaluating all DRX Active Time conditions as specified in Clause 5.7b and all multicast sessions are configured with multicast DRX:

3>	not transmit periodic SRS and semi-persistent SRS defined in TS 38.214 [7];

3>	not report semi-persistent CSI configured on PUSCH;

3>	not report semi-persistent CSI on PUCCH;

3>	if neither ps-TransmitPeriodicL1-RSRP nor  lpwus-TransmitPeriodicL1-RSRP is configured with value true:

4>	not report periodic CSI that is L1-RSRP on PUCCH.

3>	if neither ps-TransmitOtherPeriodicCSI nor lpwus-TransmitOtherPeriodicCSI is configured with value true:

4>	not report periodic CSI that is not L1-RSRP on PUCCH.

1>	else:

2>	in current symbol n, if a DRX group would not be in Active Time considering grants/assignments scheduled on Serving Cell(s) in this DRX group and DRX Command MAC CE/Long DRX Command MAC CE received and Scheduling Request sent until 4 ms prior to symbol n when evaluating all DRX Active Time conditions as specified in this clause; and

2>	if allowCSI-SRS-Tx-MulticastDRX-Active is not configured, or if cfr-ConfigMulticast is not configured for any of the active BWP(s) of the Serving Cell(s), or, in current symbol n, if all multicast DRXes corresponding to the DRX group would not be in Active Time considering multicast assignments/DRX Command MAC CE for MBS multicast received until 4 ms prior to symbol n when evaluating all DRX Active Time conditions as specified in Clause 5.7b and all multicast sessions corresponding to the DRX group are configured with multicast DRX:

3>	not transmit periodic SRS and semi-persistent SRS defined in TS 38.214 [7] in this DRX group;

3>	not report CSI on PUCCH and semi-persistent CSI configured on PUSCH in this DRX group.

2>	if CSI masking (csi-Mask) is setup by upper layers:

3>	in current symbol n, if drx-onDurationTimer of a DRX group would not be running considering grants/assignments scheduled on Serving Cell(s) in this DRX group and DRX Command MAC CE/Long DRX Command MAC CE received until 4 ms prior to symbol n when evaluating all DRX Active Time conditions as specified in this clause; and

3>	if allowCSI-SRS-Tx-MulticastDRX-Active is not configured, or if cfr-ConfigMulticast is not configured for any of the active BWP(s) of the Serving Cell(s), or, in current symbol n, if drx-onDurationTimerPTM(s) of all multicast DRXes corresponding to the DRX group would not be running considering DRX Command MAC CE for MBS multicast received until 4 ms prior to symbol n when evaluating all DRX Active Time conditions as specified in Clause 5.7b and all multicast sessions corresponding to the DRX group are configured with multicast DRX:

4>	not report CSI on PUCCH in this DRX group.

NOTE 4:	If a UE multiplexes a CSI configured on PUCCH with other overlapping UCI(s) according to the procedure specified in TS 38.213 [6] clause 9.2.5 and this CSI multiplexed with other UCI(s) would be reported on a PUCCH resource either outside DRX Active Time of the DRX group in which this PUCCH is configured or outside the on-duration period of the DRX group in which this PUCCH is configured if CSI masking is setup by upper layers, it is up to UE implementation whether to report this CSI multiplexed with other UCI(s).

The MAC entity shall ensure no rounding error is generated when performing the modulus operation with drx-NonIntegerShortCycle or drx-NonIntegerLongCycle as the divisor.

Regardless of whether the MAC entity is monitoring PDCCH or not on the Serving Cells in a DRX group, the MAC entity transmits HARQ feedback, aperiodic CSI on PUSCH, and aperiodic SRS defined in TS 38.214 [7] on the Serving Cells in the DRX group when such is expected.

The MAC entity needs not to monitor the PDCCH if it is not a complete PDCCH occasion (e.g. the Active Time starts or ends in the middle of a PDCCH occasion).

When drx-LastTransmissionUL is configured, drx-HARQ-RTT-TimerUL or HARQ-RTT-TimerUL-NTN is started after the last PUSCH transmission occasion of a bundle regardless of whether that last PUSCH transmission occasion is used for a PUSCH transmission for that bundle or not.

End of change

Annex A – RAN2 agreements

RAN2#126 Agreements

- In RRC_CONNECTED mode, RAN2 to further discuss the impacts of LP-WUS operation methods identified in RAN1. - For Option 1-1 (as described in RAN1 agreement), the LP-WUS monitoring occasion locates at a configured time offset before the start of drx-onDurationTimer. The range of time offset can be determined by RAN1. - For Option 1-1, RAN2 assumes the solutions/ operations introduced for DCP mechanism is taken as baseline. - RAN2 assume that legacy DCP and Option 1-1 is not configured simultaneously for a UE. - The LP-WUS related configuration for RRC CONNECTED state UE is provided via dedicated RRC message.

RAN2#127 Agreements on Option 1-2

- After LP-WUS triggers the UE to perform PDCCH monitoring, the UE starts one timer. When the timer is running, the UE monitors PDCCH. FFS on the timer (e.g., newly defined timer or legacy timer.) - The timer is started at a time offset after receiving the LP-WUS indication for PDCCH monitoring. The range of time offset is left for RAN1.

RAN2#127bis Agreements

- For Option 1-2, LP-WUS monitoring is performed at least outside legacy C-DRX Active Time. FFS if the legacy drx-onDurationTimer is started or not if the new timer is configured in Option 1-2. - In option 1-2, a new timer triggered by LPWUS is introduced. When this new timer is running, UE is in C-DRX active time. When UE is not in C-DRX active time, UE goes back to LPWUS monitoring. - When UE is in C-DRX active time, UE PDCCH monitoring behaviors related to other legacy DRX timers (except for drx-onDurationTimer) are not affected.

RAN2#128 Agreements

- The drx-onDurationTimer is not started with Option 1-2 LP-WUS. - For Option 1-2, network can configure whether UE reports periodic CSI/L1-RSRP during the time given by the configured drx-onDurationTimer, for the case when UE is outside C-DRX active time. - For option 1-2, if UE receives DRX command MAC CE or Long DRX command MAC CE, UE stops the new timer triggered by LP-WUS. - Don’t support Option 1-1 and Option 1-2 simultaneously configured for the same UE.

RAN2#129 Agreements

- For Option 1-1, UE monitors LP-WUS outside C-DRX active time at least when long DRX cycle is used. FFS whether short DRX cycle is used. - RAN2 confirm the (Long) DRX command MAC CE can be used with option 1-1 to stop drx-onDurationTimer and drx-InactivityTimer. - RAN2 confirm the (Long) DRX command MAC CE can be used with option 1-2 to stop the new timer and drx-InactivityTimer.

RAN2#129bis Agreements

On short DRX cycle - For Option 1-1, the UE does not monitor LP-WUS when Short DRX cycle is used. - Working assumption: For option 1-2, it is up to network configuring short DRX cycle with LP-WUS. The UE monitors LP-WUS outside the Active Time regardless of if Short DRX cycle or Long DRX cycle is used. - Send an LS to RAN1 to inform the above conclusions, can revisit if needed based on RAN1 feedback. Other aspects related to the procedure (e.g., collision handling, UAI, etc.) - Working assumption for the case of potential collision (if any): In Option 1-1, when the UE is not able to monitor the LP-WUS occasion(s) the UE should start the drx-OnDurationTimer (as if LP-WUS was detected). FFS for Option 1-2. - Send LS to inform this working assumption, can also ask a) what are the cases when UE cannot monitor LP-WUS, b) whether UE can monitor LR and MR simultaneously. - If configured, the UE can signal a preferred time offset via UAI signalling. - Ask RAN1 for further information regarding their conclusions. Dual DRX group - FFS whether/how to support LP-WUS (including Option 1-1 and 1-2) and dual DRX group MRDC - For NR-DC, the LP-WUS can be configured to be monitored at least on the PCell and PSCell. Wait for RAN1 progress on whether to allow LP-WUS configuration and monitoring on other Cells. - For NR-DC, the LP-WUS in MCG and SCG can be configured independently. - Apart from NR-DC, LP-WUS can also be supported in NE-DC, EN-DC, NGEN-DC. And proposal 1 and 2 also apply to NE-DC, EN-DC, NGEN-DC.

RAN2#130 Agreements

- Working assumption: LP-WUS can be configured on the PCell with secondary DRX. LP-WUS with secondary DRX is supported with option 1-1 and 1-2, i.e. the UE monitors LP-WUS before the on-duration occasion or periodically outside ActiveTime. When LP-WUS is detected, then UE starts the drx-onDurationTimer (with option 1-1) or the lpwus-PDCCHMonitoringTimer (with option 1-2) in both DRX groups. - Check whether we need to capture in MAC that UE is not expected to monitor LP-WUS if not in Cell DTX active period.

Annex B – Related RAN1 agreements

RAN1#121 Agreements

- Working assumption: LP-WUS can be configured on the PCell with secondary DRX. LP-WUS with secondary DRX is supported with option 1-1 and 1-2, i.e. the UE monitors LP-WUS before the on-duration occasion or periodically outside ActiveTime. When LP-WUS is detected, then UE starts the drx-onDurationTimer (with option 1-1) or the lpwus-PDCCHMonitoringTimer (with option 1-2) in both DRX groups. - Check whether we need to capture in MAC that UE is not expected to monitor LP-WUS if not in Cell DTX active period. Agreement For LP-WUS MOs in connected mode for Option 1-1/1-2, the time offset1/3 in previous agreement are configured relative to the start of SFN0 where the offset is a symbol-level offset Agreement As the initial reply to RAN2 LS in R1-2503616, RAN1 confirms that at least the collision with Active Time, measurement gap, and RAR window monitoring for BFR can be considered for the cases/scenarios on when the UE is not able to monitor LP-WUS. Agreement For LP-WUS monitoring in RRC CONNECTED mode, when Rel-17 unified TCI framework is NOT configured or UE does NOT support Rel-17 unified TCI framework - Alt1: RRC provides the CORESET ID that UE shall derive the active TCI state for LP-WUS Conclusion There is no consensus in RAN1 whether to specify UE autonomous fallback to PDCCH monitoring when UE monitors LP-WUS in RRC CONNECTED mode Agreement As the reply to RAN2 LS in R1-2503616, RAN1 assumes that UE is not able to operate LR and MR simultaneously in Rel-19. RAN1 understanding is that the terminology of LR and MR operations are for discussion purpose and will not be specified - LR operation is the UE operation for LP-WUS monitoring - MR operation is the UE operation for all other NR signals/channels transmissions/receptions in connected mode Conclusion From RAN1 perspective, for the case of potential collision (if any) in Option 1-2, when the UE is not able to monitor all the LP-WUS MO(s) in a LP-WUS periodicity, - It is up to RAN2 to further discuss and finalize the specification support, if any. Agreement For the UE capability report on the minimum time gap between the end of the last symbol of LP-WUS and the time where MR starts PDCCH monitoring regardless of SCS, the same candidate values {V1, V2, V3} are supported for different receiver types - V1=5ms - V2=13ms - V3=37ms Agreement For the UAI of preferred time offset for LP-WUS monitoring in RRC connected mode, - The candidate values are same as those for the UE capability of minimum time gap, i.e., {V1=5, V2=13, V3=37}ms - The reported UAI value is equal to or longer than the minimum time gap reported by UE capability for a UE Note: There is no change to the RAN1 agreed definition for UAI Agreement Nominal MO duration (X1, in unit of OFDM symbols) and actual LP-WUS duration (X2, in unit of OFDM symbols) are configured for LP-WUS in connected mode. (Alt C) - A LP-WUS MO spans the nominal MO duration (i.e., the LP-WUS MO duration is the same as the nominal MO duration.) - If the number of available OFDM symbols within the nominal MO duration is no less than the actual LP-WUS duration, UE monitors LP-WUS on the first X2 available symbols within the LP-WUS MO. - Otherwise, UE does not monitor LP-WUS in this MO (i.e., the MO is dropped). - Note: Any symbols that are not defined as unavailable are available symbols for LP-WUS. - Further discuss possible introduction of UE capability to restrict configuration of LP-WUS MO and LP-WUS duration. For example, - A LP-WUS spans a number of consecutive OFDM symbols according to the configured LP-WUS duration - If there is at least one OFDM symbol unavailable for the LP-WUS MO within the OFDM symbols where the LP-WUS would span, the UE does not monitor the LP-WUS in the MO Agreement For UE to determine whether a symbol is available for LP-WUS in connected mode, at least the following is supported: - A unit level bitmap with a periodicity 10, 20, or 40 units and a 14-bit or 28-bit symbol-level bitmap that covers 1 or 2 slots can be configured, where each unit is 1 or 2 slots for 14-bit or 28-bit symbol-level bitmap, respectively, with a maximum periodicity of 40ms. - ‘1’ in unit-level bitmap means the symbol level bitmap is applied to determine which symbols are unavailable in the unit for LP-WUS (‘0’ means unavailable). - ‘0’ in unit-level bitmap means: all the symbols in the unit are unavailable for LP-WUS - If the slot-level bitmap is not configured, UE assumes all 1’s for the bitmap. - If the symbol-level bitmap is not configured, UE assumes all 1’s for the bitmap. Agreement For the handling of at least PCell/PSCell SSB for connected mode, the SSB symbols are considered as unavailable for LP-WUS Agreement The UL symbols/slots configured in tdd-UL-DL-configurationCommon or tdd-UL-DL-ConfigurationDedicated are considered as unavailable for LP-WUS in connected mode