Comments collection for Rel-19 IoT NTN RRC running CR

# 1. Contacts

| Company | Name | Email |
| --- | --- | --- |
| MediaTek | Felix Tsai | Chun-fan.tsai@mediatek.com |
| Qualcomm | Bharat Shrestha | bshresth@qti.qualcomm.com |
| Samsung | Jonas Sedin | j.sedin@samsung.com |
| vivo | Yitao Mo (Stephen) | yitao.mo@vivo.com |
| Apple | Yuqin Chen | yuqin_chen@apple.com |
| Nokia | Ping Yuan | Ping.1.Yuan@nokia-sbell.com |

# 2. Comments

Please provide your comments in the table following similar format as the example:

| Company | Section/clause/IE | Comments/Suggested Change | Rapp Response |
| --- | --- | --- | --- |
| MTK01 vivo01 | 5.3.3.1x Conditions for initiating CB-Msg3 EDT in NTN | We should use CB-Msg3-EDT rather than CB-Msg3 EDT to align with the RAN2 agreement and 36.300/36.321 vivo additional comment: From stage-2 spec, CB-Msg3 is a kind of MO-EDT, the conditions should be added in 5.3.3.1b. I.E., #### 5.3.3.1b Conditions for initiating EDT A BL UE, UE in CE or NB-IoT UE can initiate EDT using the random access procedure when all of the following conditions are fulfilled: … A BL UE, UE in CE Mode A or NB-IoT UE can initiate EDT using the CN-Msg3-EDT procedure when all of the following conditions are fulfilled: |  |
| MTK02 | 5.3.3.1x A BL UE, UE in CE Mode A or NB-IoT UE can initiate CB-Msg3 EDT transmission when all of the following conditions are fulfilled: | The T in EDT is for transmission, this additional transmission is not needed. Suggest to delete it. vivo agrees with it. |  |
| MTK03 | 5.3.3.1x the measured RSRP satisfies the conditions specified in TS 36.321 [6], clause X; | We think it is sufficient to check the minimum RSRP threshold of initialing CB-Msg3-EDT in RRC. There seems no need referring to 36.321. Suggest some like “ the measured RSRP is larger than the minimum RSRP threshold configured in cb-Msg3-MinRSRP-Threshold (in cb-Msg3-MinRSRP-Threshold-NB for NB-IoT) ” |  |
| MTK04 | 5.3.3.1x 1> the size of the resulting MAC PDU including the total UL data is expected to be smaller than or equal to the TBS signalled in [FFS parameter name], as specified in TS 36.321 [6], clause X; | Unlike the legacy EDT, UE does not move to the next CE level when the number of max re-attempt has been reached, there will be no TBS check after that point. The only TBS check occurs before the procedure is for initialization. We suggest that the TBS check is only captured in RRC spec, and the reference to MAC can be removed. Nokia03: The maximum TBS is defined per CE level while the CE level determination is captured in MAC spec. In this sense, referring to MAC spec seems reasonable (as legacy) |  |
| MTK05 | – CB-Msg3-ConfigSIB cb-Msg3-MaxAttemptNum-r19 ENUMERATED {n3, n4, n5, n6, n7, n8, n10} | It better starts with n2. If this IE is absent, no re-attempt should be assumed. |  |
| MTK06 | CB-MSG3-MPDCCH-Config-r19 ::= SEQUENCE { mpdcch-Narrowband-r19 INTEGER (1..maxAvailNarrowBands-r13), mpdcch-PRB-PairsConfig-r19 SEQUENCE{ numberPRB-Pairs-r19 ENUMERATED {n2, n4, n6, spare1}, resourceBlockAssignment-r19 BIT STRING (SIZE(4)) }, | Indentation issue on numberPRB-Pairs-r19 and resourceBlockAssignment-r19. |  |
| MTK07 | maxCE-Level-NB-r19 INTEGER ::= 3 | It should be placed at 6.7.4 |  |
| MTK08 | CB-Msg3-ProbabilityAnchorList-NB-r19 | RAN2 agrees that a new probability parameter for anchor carrier is introduced in SIB22-NB. RAN2 can further discuss whether it should be a single value or a list of value for each CE level. |  |
| MTK09 | cb-Msg3-RSRP-CE-Level-NB-r19 CB-Msg3-RSRP-CE-Level-NB-r19 | It should be cb-Msg3-RSRP-CE-Level-List-NB-r19. |  |
| MTK10 | npusch-SubCarrierSetIndex-r19 CHOICE { khz15 INTEGER (0..18), khz3dot75 INTEGER (0..47) }, | According to the RAN1 LS1, it should be defined as a set. - The following parameters can be supported: - npusch-NumRUsIndex-r16 - npusch-NumRepetitionsIndex-r16 - npusch-SubCarrierSetIndex-r16 (but defining this as a set) - npusch-MCS-r16 |  |
| MTK11 | ack-NumRepetitions-NB-r19 ACK-NACK-NumRepetitions-NB-r13, | This IE could be optional. It is absent, the same value in SIB2 for NPRACH can be used. If this field is absent, the UE apply the value ack-NACK-NumRepetitions-r13 configured in SystemInformationBlockType2-NB. |  |
| MTK12 | cb-Msg3-MaxAttemptNum-NB-r19 ENUMERATED {n3, n4, n5, n6, n7, n8, n10, spare1}, | Same comment to cb-Msg3-MaxAttemptNum-r19. And this IE should be optional. |  |
| MTK13 | CB-Msg3-ConfigList-NB CB-Msg3 EDT configuration for each CE level applicable to a UE performing CB-Msg3 EDT. The first entry in the list is the CB-Msg3 EDT configuration for CE level 0, the second entry in the list is the CB-Msg3 EDT configuration for CE level 1, and so on. | It is a legacy UE behavior that numbers of CE levels in the anchor carrier and non-anchor carrier are the same. I think we can stick it. Here we can add: For the CB-Msg3-ConfigList-NB in SystemInformationBlockType22-NB, E-UTRAN includes the same number of entries, and listed in the same order, as in CB-Msg3-ConfigList-NB in SystemInformationBlockType2-NB. |  |
| MTK14 | cb-Msg3-ResponseWindow-NB MPDCCH search space window duration. See TS 36.321 [6] and TS 36.213 [23]. Value pp1 corresponds to 1 PDCCH period, pp2 corresponds to 2 PDCCH periods and so on. The value considered by the UE is: mac-ContentionResolutionTimer = Min (signaled value x PDCCH period, 10.24s). | The MPDCCH should be NPDCCH. mac-ContentionResolutionTimer cleary is a mistake. |  |
| MTK15 | npdcch-CarrierIndex Indicates the non-anchor carrier for receiving Msg4. If this field is absent, UE receives Msg4 on the anchor carrier. | We suggest a clearer text: Indicates the carrier in the list of DL non-anchor carriers for receiving CB-Msg4. If this field is absent, UE receives CB-Msg4 on the anchor carrier. |  |
| MTK16 | cb-Msg3-NumOfReplicas-NB Indicates the number of replicas that UE should send for CB-Msg3 EDT. | Suggest to modify as: Indicates the number of replicas that UE should send within one attempt of CB-Msg3 EDT. |  |
| Qualcomm | 6.3.2 CB-Msg3-ConfigSIB CB-Msg3-Config-r19 ::= SEQUENCE { cb-Msg3-TBS-r19 ENUMERATED {b328, b408, b504, b600, b712, b808, b936, b1000}, cb-Msg3-NumOfReplicas-r19 INTEGER(1..4), cb-Msg3-TimeResource-r19 SEQUENCE { pusch-Periodicity-r19 ENUMERATED {sf2, sf4, sf8, sf16, sf32, sf64, sf128, sf256}, pusch-StartSFN-r19 INTEGER (0..1023), pusch-StartSubframe-r19 INTEGER (0..9) }, cb-Msg3-MPDCCH-Config-r19 CB-Msg3-MPDCCH-Config-r19, cb-Msg3-PUCCH-Config-r19 CB-Msg3-PUCCH-Config-r19, cb-Msg3-PUSCH-Config-r19 CB-Msg3-PUSCH-Config-r19, cb-Msg3-TxWindow-r19 SEQUENCE { windowStartSFN-r19 INTEGER (0..1023), windowStartSubframe-r19 INTEGER (0..9), windowSize-r19 ENUMERATED {FFS}, windowPeriodicity-r19 ENUMERATED {FFS} } | 1. In procedural text, at least CP and UP indication can be added now. For example, AS security enabling is not applicable in CP solution. 2. Configuration We suggest remove redundancies. The cb-Msg3-TimeResource-r19 can be moved into cb-Msg3-TxWindow-r19. Start of PUSCH and start of window should be same. cb-Msg3-TxWindow-r19 SEQUENCE { windowStartSFN-r19 INTEGER (0..1023), windowStartSubframe-r19 INTEGER (0..9), windowSize-r19 ENUMERATED {FFS}, windowPeriodicity-r19 ENUMERATED {FFS}, pusch-Periodicity-r19 ENUMERATED {sf2, sf4, sf8, sf16, sf32, sf64, sf128, sf256} } |  |
| SS01 | SystemInformationBlockType31 information element -- ASN1START SystemInformationBlockType31-r17 ::= SEQUENCE { servingSatelliteInfo-r17 ServingSatelliteInfo-r17, lateNonCriticalExtension OCTET STRING OPTIONAL, ..., [[ servingSatelliteInfo-v1820 ServingSatelliteInfo-v1820 OPTIONAL -- Need OR ]], [[ t-ModeSwitching-r19 TimeOffsetUTC-r17 OPTIONAL -- Need OR ]] } | Introduce a ServingSatelliteInfo-v19xy in SystemInformationBlockType31(-NB). This helps to help to improve clarity and does not cost anything in terms of overhead. |  |
| SS02 | 1> if the SI message was not possible to decode from the accumulated SI message transmissions by the end of the SI-window, continue reception and accumulation of SI message transmissions on DL-SCH in the next SI-window occasion for the concerned SI message; | There was a discussion on this for release 17 for NTN-related SIBs, but there are similar issues for this for PWS-related SIB. If PWS is segmented to multiple SIBs, then the UE should not accumulate SI messages across SI-windows. This was probably not considered in the past as neither LTE-M nor NB-IoT was designed to receive PWS SIB. How to handle this needs to be discussed in an open issue. |  |
| SS03 | Section 5.2.1.3 | We believe that it would be useful for someone implementing NB-IoT PWS that the following agreement should be captured in some manner: Agreements: 1. In case of PWS notification the network may release a RRC_CONNECTED UE to idle, in case the UE reports the capability to receive PWS in idle (no other spec impact other than the introduction of a UE capability for supporting PWS reception in idle) An example could be: In RRC_CONNECTED, BL UEs or UEs in CE or NB-IoT UEs are not required to acquire system information except when T311 is running, or upon handover where the UE is only required to acquire the MasterInformationBlock in the target PCell, or for UEs in CE to receive ETWS/CMAS information, or upon expiry of T317 where the UE is only required to acquire the SystemInformationBlockType31 (SystemInformationBlockType31-NB in NB-IoT). For an NB-IoT UE capable of receiving ETWS/CMAS, the E-UTRAN may initiate connection release for UE to acquire ETWS/CMAS. In RRC_IDLE, E-UTRAN may notify BL UEs or UEs in CE or NB-IoT UEs about SI update, ETWS and CMAS notification, and may notify BL UEs or UEs in CE about EAB modification and UAC modification, using Direct Indication information, as specified in 6.6 (or 6.7.5 in NB-IoT) and TS 36.212 [22]. |  |
| SS04 | cb-Msg3-MaxAttemptNum-r19 | It is more efficient to have a parameter range that includes the configurable values rather than relying on optionality. Remember that OPTIONAL costs one bit and makes ASN1 implementations more complicated. Better to have a parameter with 2^n values that is not optional, which includes all the values. We also think that n1 and n2 makes sense here as compared to preamble transmissions, as it keeps the latency low. So we propose the values [n1, n2, n3, n4, n5, n6, n8, n10] and that the field is not optional. Also there is an extreme comma “,” after the need-code. |  |
| SS05 | [[ npdsch-ConfigDedicated-v1900 NPDSCH-ConfigDedicated-NB-v1900 OPTIONAL -- Need ON ]] | Should be changed to npusch-ConfigDedicated-v1900 |  |
| SS06 vivo02 | CB-Msg3-EDT modelling | As we have discussed in the past, currently the introduction of CB-Msg3-EDT causes a lot of changes to duplicate CP/UP-EDT procedures. This is despite the fact that CB-Msg3-EDT is to a large degree a MAC-enhancement and that the higher layer procedures are largely the same – i.e same type of RRC messages and same type of content in the RRC messages. We think it is nevertheless an open issue that we think should be raised and discussed. vivo additional comments: similar view as Samsung. Changes in 5.3.3.2, 5.3.3.3b, 5.3.3.18 is not needed as the new CB-Msg3 is still covered by legacy EDT. |  |
| SS07 |  | The following agreement does not seem to have been implemented: 3. A CB-Msg4 without RRC message (but with contention resolution identity) is allowed as the complete response to the CB-Msg3 in CP solution. It should be implemented in a similar fashion to section 5.3.3.3d: For CP transmission using PUR, upon indication from lower layers that transmission using PUR is successfully completed, the UE shall perform the actions as specified in 5.3.3.4b as if an empty RRCEarlyDataComplete message was received. |  |
| SS08 | #### 5.3.3.3b Actions related to transmission of RRCEarlyDataRequest message The UE shall set the contents of RRCEarlyDataRequest message as follows: . . . 1> if the UE is a NB-IoT UE: 2> if the UE supports DL channel quality reporting and cqi-Reporting is present in SystemInformationBlockType2-NB: 3> set the cqi-NPDCCH to include the latest results of the downlink channel quality measurements of the carrier where the random access response is received as specified in TS 36.133 [16]; | It needs to be discussed how to deal with setting the cqi-NPDCCH field for CB-Msg3-EDT. Currently it is specified that the UE shall measure the random access response, but in CB-Msg3-EDT there is obviously no random access response. Either cqi-NPDCCH is not reported for CB-Msg3-EDT or it is specified that the UE uses CQI based on other measurements. |  |
| SS09 | ta-Report When this field is included in SystemInformationBlockType2, it indicates reporting of timing advance is enabled during Random Access due to RRC connection establishment, RRC connection resume or RRC connection reestablishment. When this field is included in MobilityControlInfo, it indicates TA reporting is enabled during Random Access due to handover, see TS 36.321 [6], clause 5.4.9. | According to the field description, it is not clear whether TAR MAC CE will be included for a CB-Msg3-EDT transmission, but in the current procedures a TAR MAC CE will be triggered for CB-Msg3-EDT as long as the ta-Report is configured. This is potentially wasteful as the UE will only transmit a short message. This should be added to the open issue list. |  |
| ERI01 | Segmentation can be applied for the delivery of a secondary notification and, if present, the associated warning area coordinates. | Segmentation can be applied to the warning area coordinates only in the secondary notification. |  |
| ERI02 | For NB-IoT, the possible boundaries of modification for SystemInformationBlockType1-NB are defined by SFN values for which (H-SFN * 1024 + SFN) mod 4096 = 0. | Similar to legacy, upon an emergency, the network should be able to update SIB1-NB regardless the modification period boundaries. |  |
| ERI03 | 4> if the UE is in CE: 5> start acquiring SystemInformationBlockType10; 4> else if the UE is an NB-IoT UE: 5> start acquiring SystemInformationBlockType10-NB immediately; | A NB-IoT can operate in CE mode A or B. Thus, it would enter the first branch. In addition, RAN2 should discuss if the acquisition starts “immediately”. |  |
| ERI04 vivo03 | 1> forward the t-ModeSwitching to upper layers, if present. | It is uncertain whether upper layers make use of this parameter. We suggest waiting for the LS response from CT1. vivo additional comment: we are fine to keep this as this inforamtion may be used at the APP layer. In addition, the mode information is also intended to be indicated to determine the mode switching status. Merely having UTC timing info is insufficient to determine the mode switcing state (e.g. whether the switching is from S&F to regenerative or from regenerative to S&F). |  |
| ERI05 | sf-OperationIdication Indicates that the cell is operating in Store and Forward mode. If this field is present, UEs supporting the Store and Forward operation ignores cellBarred-NTN and cellBarred. Value 'barred' means the cell is barred for NTN connectivity with the Store and Forward operation, as defined in TS 36.304 [4]. Value 'notBarred' means the cell allows UEs supporting the Store and Forward operation to access. If this field is absent, the NTN cell is operating in normal mode, i.e., not in the Store and Forward mode and UEs supporting the Store and Forward operation follow cellBarred-NTN. | The detailed description pertains 304. We prefer to simplify the text in 331. There is no need to capture both in 331 and 304 the UE behaviour upon each of the values that the parameter may take. |  |
| ERI06 | ]], [[ t-ModeSwitching-r19 TimeOffsetUTC-r17 OPTIONAL -- Need OR ]] | Following previous releases conventions, it could be included under the extension marker. |  |
| Apple01 | Section 5.3.3.3a 1> if the UE is initiating CB-Msg3 EDT in accordance with conditions in 5.3.3.1x; or | The same sentence is used for MO and MT access, it should be more specific on MO and MT. |  |
| Apple02 | Section 6.7.3.2, IE CB-Msg3-ConfigSIB-NB | cb-Msg3-RSRP-CE-Level-NB-r19 can be renamed as a list. |  |
| Apple03 | Section 6.7.3.2, IE CB-Msg3-ConfigSIB-NB | I guess npusch-SubCarrierSetIndex-r19 should be a list, to indicate multiple frequency domain resources (as RAN1 also confirmed)? |  |
| Apple04 | Section 6.7.3.2, IE PhysicalConfigDedicated-NB [[ npdsch-ConfigDedicated-v1900 NPDSCH-ConfigDedicated-NB-v1900 OPTIONAL -- Need ON ]] | It should be npusch not npdsch. |  |
| Apple05 | Section 5.2.2.4 4> else if the UE is an NB-IoT UE: 5> start acquiring SystemInformationBlockType10-NB immediately; | Why not just merge it to the bullet above? |  |
| Apple06 | Section 5.3.3.18 - the UE is initiating CB-Msg3 EDT in accordance with conditions in 5.3.3.1x; | It should be made clear this is only for UP because CP solution does not require early security reactivation. |  |
| Apple07 | Section 6.2.2, SystemInformationBlockType1 sf-OperationIdication | The name in field description is not the same as in ASN.1. |  |
| Apple08 | Section 6.4 maxCE-Level-NB-r19 | It should be put into NB section 6.7.4 |  |
| Apple09 | Section 6.7.3.1 SIB22-NB E-UTRAN includes the same number of entries, and listed in the same order, as in CB-Msg3-ConfigList-NB in SystemInformationBlockType2-NB. | This sentence should be better put into cb-Msg3-ProbabilityAnchorList-NB. |  |
| Nokia01 | Section 5.2.2.4 1> if the UE is ETWS capable: 2> upon entering a cell during RRC_IDLE, following successful handover or upon connection re-establishment: 3> discard any previously buffered warningMessageSegment; | Per agreement in last meeting, need to further discuss continued inter-cell PWS reception in intra-eNB scenario. |  |
| Nokia02 | #### – CB-Msg3-ConfigSIB CB-MSG3-MPDCCH-Config-r19 ::= SEQUENCE { mpdcch-Narrowband-r19 INTEGER (1..maxAvailNarrowBands-r13), | Whether the narrowband configuration should be defined as a set (not one single narrow band) has been raised to RAN1 in LS R2-2503175 question2. Since there is no consensus in RAN1 (LS response R1-2504905), RAN2 should further discuss the issue and take decision in next meeting. |  |