3GPP TSG-RAN WG2 meeting #131	R2-25xxxxx

# Bengaluru, India, 25 - 29 Aug, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.331 | CR | Draft | rev | - | Current version: | 18.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Running 36.331 CR for R19 SONMDT |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_ENDC_SON_MDT_Ph4-Core |  |  |  |  |  | Date: |  |  | 2025-05-29 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Introduction of R19 SON/MDT features in TS 36.331. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduction of R19 SON/MDT features in TS 36.331. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | R19 SON/MDT features are not suported in TS 36.331. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.3.3.4, 5.6.13a.3, 6.2.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS 36.306 CRxxxx |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

5.3.3.4	Reception of the RRCConnectionSetup by the UE

NOTE 1:	Prior to this, lower layer signalling is used to allocate a C-RNTI. For further details see TS 36.321 [6];

The UE shall:

1>	except when the UE connected to 5GC is a BL UE or UE in CE, if the RRCConnectionSetup is received in response to an RRCConnectionResumeRequest from a suspended RRC connection:

2>	if the UE is resuming an RRC connection after early security reactivation in accordance with conditions in 5.3.3.18:

3>	discard any current AS security context including the KRRCenc key, the KRRCint key, the KUPint key and the KUPenc key;

2>	release all radio resources, including release of the RLC entity, the MAC configuration and the associated PDCP entity for all established or suspended RBs, except for SRB0;

2>	discard the stored UE AS context and resumeIdentity;

2>	if stored, discard the stored nextHopChainingCount;

2>	if stored, discard the stored drb-ContinueROHC;

2>	indicate to upper layers fallback of the RRC connection;

1>	if the RRCConnectionSetup is received in response to an RRCConnectionResumeRequest from RRC_INACTIVE:

2>	stop T380 if running;

2>	discard the stored UE Inactive AS context;

2> release rrc-InactiveConfig, if configured;

1>	if the UE connected to 5GC is a BL UE or UE in CE, and the RRCConnectionSetup is received in response to an RRCConnectionResumeRequest from a suspended RRC connection:

2>	discard the stored UE AS context and resumeIdentity;

2>	if stored, discard the stored nextHopChainingCount;

2>	if stored, discard the stored drb-ContinueROHC;

1>	if the RRCConnectionSetup is received in response to an RRCConnectionResumeRequest from RRC_INACTIVE; or

1>	if the UE connected to 5GC is a BL UE or UE in CE, and the RRCConnectionSetup is received in response to an RRCConnectionResumeRequest from a suspended RRC connection:

2>	discard any current AS security context including the KRRCenc key, the KRRCint key, the KUPint key and the KUPenc key;

2>	release radio resources for all established RBs except SRB0, including release of the RLC entities, of the associated PDCP entities and of SDAP entities;

2>	release the RRC configuration except for the default L1 parameter values, default MAC main configuration and CCCH;

2>	apply the default NR PDCP configuration as specified in TS 38.331 [82], clause 9.2.1.1 for SRB1;

2>	use NR PDCP for all subsequent messages received and sent by the UE via SRB1;

2>	indicate to upper layers fallback of the RRC connection;

1>	if the RRCConnectionSetup is received in response to an RRCEarlyDataRequest or RRCConnectionResumeRequest for transmission using PUR:

2>	instruct the associated MAC entity to start timeAlignmentTimer;

1>	perform the radio resource configuration procedure in accordance with the received radioResourceConfigDedicated and as specified in 5.3.10.0;

1>	if stored, discard the cell reselection priority information provided by the idleModeMobilityControlInfo or inherited from another RAT;

1>	if stored, discard the altFreqPriorities provided by the RRCConnectionRelease;

1>	if stored, discard the dedicated offset provided by the redirectedCarrierOffsetDedicated;

1>	stop timer T300;

1>	if T302 is running:

2>	stop timer T302;

2>	if the UE is connected to 5GC:

3>	perform the actions as specified in 5.3.16.4;

1>	stop timer T303, if running;

1>	stop timer T305, if running;

1>	stop timer T306, if running;

1>	stop timer T308, if running;

1>	perform the actions as specified in 5.3.3.7;

1>	stop timer T320, if running;

1>	stop timer T350, if running;

1>	perform the actions as specified in 5.6.12.4;

1>	release rclwi-Configuration, if configured, as specified in 5.6.16.2;

1>	stop timer T360, if running;

1>	stop timer T322, if running;

1>	if timer T331 is running:

2>	stop timer T331;

2>	perform the actions as specified in 5.6.20.3;

1>	stop timer T323, if running;

1>	forward the dedicatedInfoNAS, if received, to the upper layers;

1>	if T309 is running:

2>	stop timer T309 for all access categories;

2>	perform the actions as specified in 5.3.16.4.

1>	enter RRC_CONNECTED;

1>	stop the cell re-selection procedure;

1>	consider the current cell to be the PCell;

1>	except for NB-IoT:

2>	if the UE supports RLF report for inter-RAT MRO EUTRA as defined in TS 38.306 [87], and if the UE has radio link failure or handover failure information available in VarRLF-Report of TS 38.331 [82] and if the RPLMN is included in plmn-IdentityList stored in VarRLF-Report of TS 38.331 [82]:

3>	if reconnectCellId in VarRLF-Report of TS 38.331 [82] is not set, and if the UE failed to perform reestablishment; or

3>	if reconnectCellId in VarRLF-Report of TS 38.331 [82] is not set, and if the UE selected the current PCell immediately after failure in performing MobilityFromNRCommand:

4>	if the selected PCell is an acceptable cell as defined in TS 36.304 [4]:

5>	set timeUntilReconnection in VarRLF-Report of TS 38.331 [82] to the time that elapsed since the MobilityFromNRCommand failure;

4>	if the selected PCell is a suitable cell as defined in TS 36.304 [4]:

5>	[if the UE supports RLF-Report for LTM] and if ltmCellId in VarRLF-Report of TS 38.331 [82] is set:

6>	set timeUntilReconnection in VarRLF-Report of TS 38.331 [82] to the time that elapsed since the radio link failure or reconfiguration with sync failure experienced in the failedPCellID stored in VarRLF-Report of TS 38.331 [82];

5>	else:

6>	set timeUntilReconnection in VarRLF-Report of TS 38.331 [82] to the time that elapsed since the last radio link failure or handover failure;

5>	set eutraReconnectCellId in reconnectCellId in VarRLF-Report of TS 38.331 [82] to the global cell identity and the tracking area code of the PCell;

2>	if the UE radio link failure or handover failure information available in VarRLF-Report and if the RPLMN is included in plmn-IdentityList stored in VarRLF-Report:

3>	if reconnectCellId in VarRLF-Report is not set, and if the UE failed to perform reestablishment:

4>	set timeUntilReconnection in VarRLF-Report to the time that elapsed since the last radio link failure or handover failure;

4>	set eutraReconnectCellId in reconnectCellId in VarRLF-Report to the global cell identity and the tracking area code of the PCell;

1>	set the content of RRCConnectionSetupComplete message as follows:

2>	if the RRCConnectionSetup is received in response to an RRCConnectionResumeRequest:

3>	if upper layers provide an S-TMSI:

4>	set the s-TMSI to the value received from upper layers;

3>	else if upper layers provide a 5G-S-TMSI:

4>	if the UE is a NB-IoT UE:

5>	set the ng-5G-S-TMSI to the value received from upper layers;

4>	else:

5>	set the ng-5G-S-TMSI-Bits to ng-5G-S-TMSI with the value received from upper layers;

2>	else if upper layers provide a 5G-S-TMSI:

3>	except for NB-IoT, set the ng-5G-S-TMSI-Bits to ng-5G-S-TMSI-Part2 to the leftmost 8 bits of 5G-S-TMSI received from upper layers;

2>	set the selectedPLMN-Identity to the PLMN selected by upper layers (see TS 23.122 [11], TS 24.301 [35] for E-UTRA/EPC and TS 24.501 [95] for E-UTRA/5GC) from the PLMN(s) included in the plmn-IdentityList in SystemInformationBlockType1 (or SystemInformationBlockType1-NB in NB-IoT);

2>	if upper layers provide the 'Registered MME', include and set the registeredMME as follows:

3>	if the PLMN identity of the 'Registered MME' is different from the PLMN selected by the upper layers:

4>	include the plmnIdentity in the registeredMME and set it to the value of the PLMN identity in the 'Registered MME' received from upper layers;

3>	set the mmegi and the mmec to the value received from upper layers;

2>	if upper layers provided the 'Registered MME':

3>	include and set the gummei-Type to the value provided by the upper layers;

2>	if upper layers provide the 'Registered AMF', include and set the registeredAMF as follows:

3>	if the PLMN identity of the 'Registered AMF' is different from the PLMN selected by the upper layers:

4>	include the plmnIdentity in the registeredAMF and set it to the value of the PLMN identity in the 'Registered AMF' received from upper layers;

3>	set the amf-Identifier to AMF Identifier of the 'Registered AMF' received from upper layers;

2>	if upper layers provided the 'Registered AMF':

3>	include and set the guami-Type to the value provided by the upper layers;

2>	if upper layers provide one or more S-NSSAI (see TS 23.003 [27]):

3>	include the s-NSSAI-list and set the content to the values provided by the upper layers;

2>	if the UE supports CIoT EPS optimisation(s):

3>	include attachWithoutPDN-Connectivity if received from upper layers;

3>	include up-CIoT-EPS-Optimisation if received from upper layers;

3>	except for NB-IoT, include cp-CIoT-EPS-Optimisation if received from upper layers;

2>	if the UE supports CIoT 5GS optimisation(s):

3>	for NB-IoT, include ng-U-DataTransfer if received from upper layers;

3>	except for NB-IoT, include cp-CIoT-5GS-Optimisatoin if received from upper layers;

2>	if connecting as an RN:

3>	include the rn-SubframeConfigReq;

2>	if the RRCConnectionSetup is received in response to RRCEarlyDataRequest:

3>	set the dedicatedInfoNAS to a zero-length octet string;

2>	else:

3>	set the dedicatedInfoNAS to include the information received from upper layers;

2>	if the RRCConnectionSetup is not in response to transmission using PUR and the UE has a stored pur-Config including pur-ConfigID:

3>	include the stored pur-ConfigID;

2>	if the UE is connected to EPC:

3>	except for NB-IoT:

4>	include the mobilityState and set it to the mobility state (as specified in TS 36.304 [4]) of the UE just prior to entering RRC_CONNECTED state;

3>	for NB-IoT:

4>	if the UE has radio link failure information available in VarRLF-Report-NB and if the RPLMN is included in plmn-IdentityList stored in VarRLF-Report-NB:

5>	include rlf-InfoAvailable;

4>	if the UE has ANR measurements information available in VarANR-MeasReport-NB and if the RPLMN is included in plmn-IdentityList stored in VarANR-MeasReport-NB:

5>	include anr-InfoAvailable;

3>	include dcn-ID if a DCN-ID value (see TS 23.401 [41]) is received from upper layers;

2>	else (i.e. the UE is connected to 5GC):

3>	if the UE is a BL UE:

4>	include lte-M;

2>	except for NB-IoT:

3>	if the UE has radio link failure or handover failure information available in VarRLF-Report and if the RPLMN is included in plmn-IdentityList stored in VarRLF-Report:

4>	include rlf-InfoAvailable;

3>	if the UE has MBSFN logged measurements available for E-UTRA and if the RPLMN is included in plmn-IdentityList stored in VarLogMeasReport:

4>	include logMeasAvailableMBSFN;

3>	if the UE has logged measurements available for E-UTRA and if the RPLMN is included in plmn-IdentityList stored in VarLogMeasReport:

4>	include logMeasAvailable;

4>	if Bluetooth measurement results are included in the logged measurements the UE has available:

5>	include logMeasAvailableBT;

4>	if WLAN measurement results are included in the logged measurements the UE has available:

5>	include logMeasAvailableWLAN;

3>	if the UE has connection establishment failure information available in VarConnEstFailReport and if the RPLMN is equal to plmn-Identity stored in VarConnEstFailReport:

4>	include connEstFailInfoAvailable;

3>	if the UE has flight path information available:

4>	include flightPathInfoAvailable;

3>	if the UE supports storage of mobility history information and the UE has mobility history information available in VarMobilityHistoryReport:

4>	include the mobilityHistoryAvail;

3>	if the SIB2 contains idleModeMeasurements and the UE has E-UTRA idle/inactive measurement information concerning cells other than the PCell available in VarMeasIdleReport; or

3>	if the SIB2 contains idleModeMeasurementsNR and the UE has NR idle/inactive measurement information available in VarMeasIdleReport:

4>	include the idleMeasAvailable;

3>	if upper layers indicate that access to RLOS is initiated (see TS 23.401 [41] clause 4.3.8.3):

4>	set rlos-Request to true;

2>	if UE needs UL gaps during continuous uplink transmission:

3>	include ue-CE-NeedULGaps;

2>	for NB-IoT:

3>	if the UE supports serving cell idle mode measurements reporting and servingCellMeasInfo is present in SystemInformationBlockType2-NB:

4>	set the measResultServCell to include the measurements of the serving cell;

NOTE 2:	The UE includes the latest results of the serving cell measurements as used for cell selection/ reselection evaluation, which are performed in accordance with the performance requirements as specified in TS 36.133 [16].

2>	if connecting as an IAB-node:

3>	include iab-NodeIndication;

2>	if the UE is connected to NTN:

3>	include gnss-validityDuration in accordance with the remaining time of the GNSS validity duration;

3>	if UE supports GNSS position fix in RRC_CONNECTED and gnss-PositionFixDurationReporting is present in SystemInformationBlockType2(-NB):

4>	include gnss-PositionFixDuration in accordance with the time duration required for the UE to acquire a GNSS position;

2>	if UE supports uplink RRC Segmentation of UECapabilityInformation according to the network indication rrc-SegAllowed:

3>	except for NB-IoT, may include ul-RRC-Segmentation if upper layers indicate that they are performing an Attach or TA Update;

2>	if the UE supports uplink RRC Segmentation of UECapabilityInformation according to the network indication rrc-MaxCapaSegAllowed:

3>	except for NB-IoT, include the ul-RRC-MaxCapaSegments if upper layers indicate that they are performing an Attach or TA Update;

1>	submit the RRCConnectionSetupComplete message to lower layers for transmission;

1>	for NB-IoT:

2>	if the UE supports connected mode measurements and connMeasConfig is present in SystemInformationBlockType3-NB:

3>	perform measurements as specified in 5.5.8.

1>	the procedure ends.

<Next modification>

5.6.13a	NR SCG failure information

5.6.13a.1	General

Figure 5.6.13a.1-1: NR SCG failure information

The purpose of this procedure is to inform E-UTRAN about an SCG failure the UE has experienced (e.g. SCG radio link failure, failure to successfully complete an SCG reconfiguration with sync), as specified in TS 38.331 [82], clause 5.7.3.2.

5.6.13a.2	Initiation

A UE initiates the procedure to report NR SCG failures when neither E-UTRA MCG nor NR SCG transmission is not suspended and in accordance with TS 38.331 [82], clause 5.7.3.2. Actions the UE shall perform upon initiating the procedure, other than related to the transmission of the SCGFailureInformationNR message are specified in TS 38.331 [82], clause 5.7.3.2.

5.6.13a.3	Actions related to transmission of SCGFailureInformationNR message

The UE shall set the contents of the SCGFailureInformationNR message as follows:

1>	include failureType within failureReportSCG-NR and set it to indicate the SCG failure in accordance with TS 38.331 [82], clause 5.7.3.3;

NOTE 1:	This may involve including both failureType-r15 and failureType-v1610, see TS 38.331 [82], clause 5.7.3.3.

1>	include and set measResultSCG in accordance with TS 38.331 [82], clause 5.7.3.4:

1>	for each NR frequency the UE is configured to measure by measConfig for which measurement results are available:

2>	set the measResultFreqListNR to include the best measured cells, ordered such that the best cell is listed first using RSRP to order if RSRP measurement results are available for cells on this frequency, otherwise using RSRQ to order if RSRQ measurement results are available for cells on this frequency, otherwise using SINR to order, and based on measurements collected up to the moment the UE detected the failure, and for each cell that is included, include the optional fields that are available;

NOTE 2:	Field measResultSCG is used to report available results for NR frequencies the UE is configured to measure by NR RRC signalling.

1>	if detailed location information is available, set the content of the locationInfo as follows:

2>	include the locationCoordinates;

2>	include the horizontalVelocity, if available;

1>	if available, set the logMeasResultListWLAN to include the WLAN measurement results, in order of decreasing RSSI for WLAN APs;

1>	if available, set the logMeasResultListBT to include the Bluetooth measurement results, in order of decreasing RSSI for Bluetooth beacons;

1>	[if the UE supports SCG failure for mobility robustness optimization]:

2>	if the failureType is set to synchReconfigFailureSCG; or

2>	if the failureType is set to randomAccessProblem and the SCG failure was declared while T304 was running:

3>	set perRA-InfoListNR to indicate the performed random access procedure related information as specified in 5.7.10.5 of TS 38.331.

3>	set the failedPSCellId to the physical cell identity and carrier frequency of the target PSCell of the failed PSCell change or failed PSCell addition;

2>	else:

3>	set the failedPSCellId to the physical cell identity and carrier frequency of the PSCell in which the SCG failure was declared;

2>	set the previousPSCellId to the physical cell identity and carrier frequency of the source PSCell associated to the last received RRCReconfiguration message including reconfigurationWithSync for the SCG, if available;

2>	set the timeSCG-Failure to the elapsed time since the last execution of RRCReconfiguration message including the reconfigurationWithSync for the SCG until declaring the SCG failure.

The UE shall submit the SCGFailureInformationNR message to lower layers for transmission.

<Next modification>

6.2.2	Message definitions

<Partially omitted >

–	SCGFailureInformationNR

The SCGFailureInformationNR message is used to provide information regarding NR SCG failures detected by the UE.

Signalling radio bearer: SRB1

RLC-SAP: AM

Logical channel: DCCH

Direction: UE to E-UTRAN

SCGFailureInformationNR message

-- ASN1START

SCGFailureInformationNR-r15 ::=		SEQUENCE {

criticalExtensions					CHOICE {

c1									CHOICE {

scgFailureInformationNR-r15			SCGFailureInformationNR-r15-IEs,

spare3 NULL, spare2 NULL, spare1 NULL

},

criticalExtensionsFuture			SEQUENCE {}

}

}

SCGFailureInformationNR-r15-IEs ::=	SEQUENCE {

failureReportSCG-NR-r15				FailureReportSCG-NR-r15				OPTIONAL,

nonCriticalExtension					SCGFailureInformationNR-v1590-IEs	OPTIONAL

}

SCGFailureInformationNR-v1590-IEs ::=	SEQUENCE {

lateNonCriticalExtension					OCTET STRING					OPTIONAL,

nonCriticalExtension						SEQUENCE {}					OPTIONAL

}

FailureReportSCG-NR-r15 ::=		SEQUENCE {

failureType-r15						ENUMERATED {

t310-Expiry, randomAccessProblem,

rlc-MaxNumRetx,

synchReconfigFailureSCG, scg-reconfigFailure,

srb3-IntegrityFailure, dummy},

measResultFreqListNR-r15				MeasResultFreqListFailNR-r15		OPTIONAL,

measResultSCG-r15						OCTET STRING						OPTIONAL,

...,

[[	locationInfo-r16				LocationInfo-r10						OPTIONAL,

logMeasResultListBT-r16			LogMeasResultListBT-r15					OPTIONAL,

logMeasResultListWLAN-r16		LogMeasResultListWLAN-r15				OPTIONAL,

failureType-v1610				ENUMERATED {t312-Expiry, scg-lbtFailure,

beamFailureRecoveryFailure, bh-RLF-r16,

beamFailure-r17,

spare3, spare2, spare1}	OPTIONAL

]],

[[

previousPSCellId-r19			SEQUENCE {

physCellId-r19                       PhysCellIdNR-r15,

carrierFreq-r19                      ARFCN-ValueNR-r15

}                                                    		OPTIONAL,

failedPSCellId-r19				SEQUENCE {

physCellId-r19                       PhysCellIdNR-r15,

carrierFreq-r19                      ARFCN-ValueNR-r15

}                                                    		OPTIONAL,

timeSCG-Failure-r19                   INTEGER (0..1023)        OPTIONAL,

perRA-InfoListNR-r19				SEQUENCE {

perRA-InfoList-r16			OCTET STRING		OPTIONAL,

perRA-InfoList-v1660			OCTET STRING		OPTIONAL,

perRA-InfoList-v1800			OCTET STRING		OPTIONAL

}

]]

}

MeasResultFreqListFailNR-r15 ::=	SEQUENCE (SIZE (1..maxFreqNR-r15)) OF MeasResultFreqFailNR-r15

MeasResultFreqFailNR-r15 ::=		SEQUENCE {

carrierFreq-r15						ARFCN-ValueNR-r15,

measResultCellList-r15				MeasResultCellListNR-r15			OPTIONAL,

...

}

-- ASN1STOP

SCGFailureInformationNR field descriptions

failedPSCellId This field indicates the physical cell id and carrier frequency of the cell in which SCG failure is detected or the target PSCell of the failed PSCell change or failed PSCell addition.

failureType Indicates the cause of the SCG failure. When the field failureType-v1610 is included, the network ignores the field failureType-r15.

measResultFreqListNR The field contains available results of measurements on NR frequencies the UE is configured to measure by measConfig.

measResultSCG Includes the NR MeasResultSCG-Failure IE as specified in TS 38.331 [82]. The field contains available results of measurements on NR frequencies the UE is configured to measure by the NR RRCConfiguration message.

previousPSCellId This field indicates the physical cell id and carrier frequency of the cell that is the source PSCell of the last PSCell change. In case of PSCell addition failure, this field is absent.

perRA-InfoListNR This field is used to indicate per RA information for NR RACH. The perRA-InfoList-r16 IE includes PerRAInfoList-r16, and the perRA-InfoList-v1660 IE includes PerRAInfoList-v1660, and the perRA-InfoList-v1800 includes PerRAInfoList-v1800, which are specified in TS 38.331 [82].

timeSCG-Failure This field is used to indicate the time elapsed since the last execution of RRCReconfiguration with reconfigurationWithSync for the SCG until the SCG failure. Actual value = field value * 100ms. The maximum value 1023 means 102.3s or longer.

MRO for MR-DC SCG failure

RAN2#130

For open issues for running 36.331 CR, RAN2 made the following agreements:

Agreements - Remove the condition:3> if the last RRCReconfiguration message including the reconfigurationWithSync for the SCG was received to enter the PSCell in which the SCG failure was declared. - When the LTM based recovery fails and the UE selects E-UTRA cell and receives RRCConnectionSetup, timeUntilReconnection need to refer to the first failure. - Specify separate containers for all the fields i.e. PerRAInfoList-r16, PerRAInfoList-v1660 and PerRAInfoList-v1800.

RAN2#129bis

For open issue list, the discussion and agreements are listed as below:

R2-2502788	Open issue list for running 36.331 CR for R19 SONMDT	Huawei, HiSilicon	discussion	Rel-19	NR_ENDC_SON_MDT_Ph4-Core

- Noted

Proposal 1: For ASN.1 definitions in TS 36.331, it is proposed to introduce the perRAInfoListNR IE, and it includes PerRAInfoList-r16, PerRAInfoList-v1660 and PerRAInfoList-v1800 (which are specified in TS 38.331).

Proposal 2: It is proposed to keep the condition for UE setting timeSCGFailure and previousPSCellId, i.e. 3>	if the last RRCReconfiguration message including the reconfigurationWithSync for the SCG was received to enter the PSCell in which the SCG failure was declared.

On P2:

-	Samsung wonders if it covers PSCell change and or PSCell addition? Huawei is OK to leave this as FFS.

-	Nokia wonders if we can instead have a container which carries all the fields (PerRAInfoList-r16, PerRAInfoList-v1660 and PerRAInfoList-v1800)? Samsung think we can use a container (OCTET STRING) and then in the field description we clarify the content.

Agreements For ASN.1 definitions in TS 36.331, it is proposed to introduce the perRAInfoListNR IE, and it includes PerRAInfoList-r16, PerRAInfoList-v1660 and PerRAInfoList-v1800 (which are specified in TS 38.331).

RAN2#129

No agreements.

RAN2#128

No agreements.

RAN2#127-bis

4: Close the stage-2 work on MRO for MR-DC SCG failure.

- Add reporting of the following parameters for SCG failure report in EN-DC scenario:

⁻	For failedPSCellId and previousPSCellId: frequency and the PCI of the PSCell;

⁻	For timeSCGFailure: value range 0-1023;

⁻	For failureType: Reuse the legacy field.

⁻	perRA-InfoList

RAN2#127

- To support MRO for SCG failure in EN-DC, enhance SCGFailureInformationNR message to include previousPSCellId, failedPSCellId, timeSCGFailure.

RAN2#126

- Reply to RAN3 that we will only do EN-DC. RAN2 understands that whether also supporting (NG)EN-DC has no additional RAN2 impact hence RAN3 can decide. If later we get time we can consider other options.

R2-2405846	Reply LS on support of MRO for MR-DC SCG failure	RAN2

- Approved

RAN2#125-bis

No agreements.