3GPP TSG-RAN2 Meeting #130	R2-2504295

# St. Julian, 19th – 23th May 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.331 | CR |  | rev |  | Current version: | 18.5.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Running CR for SONMDT features |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson, ZTE |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_ENDC_SON_MDT_Ph4-Core |  |  |  |  |  | Date: |  |  | 2025-05-08 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | This running CR implements all the agreements related to the Re-19 SON/MDT WI that affects RRC technical specification up to the meeting RAN2#130. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The SONMDT agreements up to RAN2#130 affecting the RRC spec are captured in the running CR. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Rel-19 RRC specification will not include the new functionalities agreed for the Rel-19 SON/MDT for NR air interface functionalities. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.3.3.4, 5.3.5.3, 5.3.5.18.6,5.3.7.3, 5.3.10.5, 5.7.3.5, 5.7.10.3, 5.7.10.4, 5.7.10.5, 5.7.10.6, 5.7.10.7, 6.2.2, 6.3.2, 7.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

FIRST CHANGE

#### 5.3.3.4	Reception of the RRCSetup by the UE

The UE shall perform the following actions upon reception of the RRCSetup:

1>	if the RRCSetup is received in response to an RRCReestablishmentRequest; or

1>	if the RRCSetup is received in response to an RRCResumeRequest or RRCResumeRequest1:

2>	if the UE is NCR-MT:

3>	indicate to NCR-Fwd to cease forwarding;

2>	if sdt-MAC-PHY-CG-Config is configured:

3>	instruct the MAC entity to stop the cg-SDT-TimeAlignmentTimer, if it is running;

3>	instruct the MAC entity to start the timeAlignmentTimer associated with the PTAG indicated by tag-Id, if it is not running;

2>	if srs-PosRRC-Inactive is configured:

3>	instruct the MAC entity to stop the inactivePosSRS-TimeAlignmentTimer, if it is running;

2>	if srs-PosRRC-InactiveValidityAreaPreConfigList or srs-PosRRC-InactiveValidityAreaNonPreConfig is configured:

3>	instruct the MAC entity to stop the inactivePosSRS-ValidityAreaTAT, if it is running;

2>	if the UE is configured to receive MBS multicast in RRC_INACTIVE:

3>	reset MAC;

2>	discard any stored UE Inactive AS context and suspendConfig;

2>	discard any current AS security context including the KRRCenc key, the KRRCint key, the KUPint key and the KUPenc key;

2>	release radio resources for all established RBs except SRB0 and broadcast MRBs, including release of the RLC entities, of the associated PDCP entities and of SDAP;

2>	release the RRC configuration except for the default L1 parameter values, default MAC Cell Group configuration, CCCH configuration and broadcast MRBs;

2>	indicate to upper layers fallback of the RRC connection;

2>	for each application layer measurement configuration with appLayerIdleInactiveConfig absent:

3>	forward the measConfigAppLayerId and inform upper layers about the release of the application layer measurement configuration;

3>	release the application layer measurement configuration;

3>	discard any application layer measurement reports which were not yet fully submitted to lower layers for transmission;

3>	consider itself not to be configured to send application layer measurement reports for the measConfigAppLayerId;

2>	stop timer T380, if running;

1>	perform the cell group configuration procedure in accordance with the received masterCellGroup and as specified in 5.3.5.5;

1>	perform the radio bearer configuration procedure in accordance with the received radioBearerConfig and as specified in 5.3.5.6;

1>	if stored, discard the cell reselection priority information provided by the cellReselectionPriorities or inherited from another RAT;

1>	stop timer T300, T301, T319;

1>	if T319a is running:

2>	stop T319a;

2>	consider SDT procedure is not ongoing;

1>	if T390 is running:

2>	stop timer T390 for all access categories;

2>	perform the actions as specified in 5.3.14.4;

1>	if T302 is running:

2>	stop timer T302;

2>	perform the actions as specified in 5.3.14.4;

1>	stop timer T320, if running;

1>	if the RRCSetup is received in response to an RRCResumeRequest, RRCResumeRequest1 or RRCSetupRequest:

2>	if T331 is running:

3>	stop timer T331;

3>	perform the actions as specified in 5.7.8.3;

2>	enter RRC_CONNECTED;

2>	stop the cell re-selection procedure;

2>	stop relay (re)selection procedure if any for L2 U2N Remote UE;

1>	consider the current cell to be the PCell;

1>	perform the L2 U2N Remote UE configuration procedure in accordance with the received sl-L2RemoteUE-Config as specified in 5.3.5.16;

1>	perform the sidelink dedicated configuration procedure in accordance with the received sl-ConfigDedicatedNR as specified in 5.3.5.14;

1>	if the UE has radio link failure or handover failure information available in VarRLF-Report and if the RPLMN is included in plmn-IdentityList stored in VarRLF-Report; or

1>	if the UE has radio link failure or handover failure information available in VarRLF-Report and if the current registered SNPN identity is included in snpn-IdentityList stored in VarRLF-Report:

2>	if reconnectCellId in VarRLF-Report is not set after failing to perform reestablishment and if this is the first RRCSetup received by the UE after declaring the failure:

3>	if the UE supports RLF-Report for conditional handover and if choCellId in VarRLF-Report is set; or

3>	if the UE supports RLF-Report for MCG LTM cell switch and if ltm-RecoveryCellId in VarRLF-Report is set:

4>	set timeUntilReconnection in VarRLF-Report to the time that elapsed since the radio link failure or reconfiguration with sync failure experienced in the failedPCellId stored in VarRLF-Report;

3>	else:

4>	set timeUntilReconnection in VarRLF-Report to the time that elapsed since the last radio link failure or handover failure;

3>	set nrReconnectCellId in reconnectCellId in VarRLF-Report to the global cell identity and the tracking area code of the PCell;

1>	if the UE supports RLF report for inter-RAT MRO NR as defined in TS 36.306 [62], and if the UE has radio link failure or handover failure information available in VarRLF-Report of TS 36.331 [10] and if the RPLMN is included in plmn-IdentityList stored in VarRLF-Report of TS 36.331 [10]:

2>	if reconnectCellId in VarRLF-Report of TS 36.331[10] is not set after failing to perform reestablishment and if this is the first RRCSetup received by the UE after declaring the failure:

3>	set timeUntilReconnection in VarRLF-Report of TS 36.331[10] to the time that elapsed since the last radio link failure or handover failure in LTE;

3>	set nrReconnectCellId in reconnectCellId in VarRLF-Report of TS 36.331[10] to the global cell identity and the tracking area code of the PCell;

1>	for each application layer measurement configuration with appLayerIdleInactiveConfig configured:

2>	if the RPLMN is not included in plmn-IdentityList in VarAppLayerPLMN-ListConfig:

3>	forward the measConfigAppLayerId and inform upper layers about the release of the application layer measurement configuration;

3>	release the application layer measurement configuration including its fields in the UE variables VarAppLayerIdleConfig and VarAppLayerPLMN-ListConfig;

3>	discard any application layer measurement reports which were not yet fully submitted to lower layers for transmission;

3>	consider itself not to be configured to send application layer measurement reports for the measConfigAppLayerId;

1>	set the content of RRCSetupComplete message as follows:

2>	if upper layers provide a 5G-S-TMSI:

3>	if the RRCSetup is received in response to an RRCSetupRequest:

4>	set the ng-5G-S-TMSI-Value to ng-5G-S-TMSI-Part2;

3>	else:

4>	set the ng-5G-S-TMSI-Value to ng-5G-S-TMSI;

2>	if upper layers selected an SNPN or a PLMN and in case of PLMN UE is either allowed or instructed to access the PLMN via a cell for which at least one CAG ID is broadcast:

3>	set the selectedPLMN-Identity from the npn-IdentityInfoList;

2>	else:

3>	set the selectedPLMN-Identity to the PLMN selected by upper layers from the plmn-IdentityInfoList;

2>	if upper layers provide the 'Registered AMF':

3>	include and set the registeredAMF as follows:

4>	if the PLMN identity of the 'Registered AMF' is different from the PLMN selected by the upper layers:

5>	include the plmnIdentity in the registeredAMF and set it to the value of the PLMN identity in the 'Registered AMF' received from upper layers;

4>	set the amf-Identifier to the value received from upper layers;

3>	include and set the guami-Type to the value provided by the upper layers;

2>	if upper layers provide one or more S-NSSAI (see TS 23.003 [21]):

3>	include the s-NSSAI-List and set the content to the values provided by the upper layers;

2>	if upper layers provide onboarding request indication:

3>	include the onboardingRequest;

2>	set the dedicatedNAS-Message to include the information received from upper layers;

2>	if connecting as an IAB-node but not as a mobile IAB-node:

3>	include the iab-NodeIndication;

2>	else if connecting as a mobile IAB-node:

3>	include the mobileIAB-NodeIndication;

2>	if connecting as an NCR-node:

3>	include the ncr-NodeIndication;

2>	if the SIB1 contains idleModeMeasurementsNR and the UE has NR idle/inactive measurement information concerning cells other than the PCell available in VarMeasIdleReport; or

2>	if the SIB1 contains idleModeMeasurementsEUTRA and the UE has E-UTRA idle/inactive measurement information available in VarMeasIdleReport:

3>	include the idleMeasAvailable;

2>	if the SIB1 contains reselectionMeasurementsNR and the UE has valid NR reselection measurements available for any frequency listed in measReselectionCarrierListNR in VarMeasReselectionConfig:

3>	include the reselectionMeasAvailable;

2>	if the UE has logged measurements available for NR and if the RPLMN is included in plmn-IdentityList stored in VarLogMeasReport; or

2>	if the UE has logged measurements available for NR and if the current registered SNPN identity is included in snpn-ConfigID-List stored in VarLogMeasReport:

3>	include the logMeasAvailable in the RRCSetupComplete message;

3>	if Bluetooth measurement results are included in the logged measurements the UE has available for NR:

4>	include the logMeasAvailableBT in the RRCSetupComplete message;

3>	if WLAN measurement results are included in the logged measurements the UE has available for NR:

4>	include the logMeasAvailableWLAN in the RRCSetupComplete message;

2>	if the sigLoggedMeasType in VarLogMeasReport is included; or

2>	if the UE supports the override protection of the signalling based logged MDT for inter-RAT (i.e. LTE to NR), and if the sigLoggedMeasType in VarLogMeasReport of TS 36.331 [10] is included:

3>	if T330 timer is running (associated to the logged measurement configuration for NR or for LTE):

4>	set sigLogMeasConfigAvailable to true in the RRCSetupComplete message;

3>	else:

4>	if the UE has logged measurements in VarLogMeasReport or in VarLogMeasReport of TS 36.331 [10]:

5>	set sigLogMeasConfigAvailable to false in the RRCSetupComplete message;

2>	if the UE has connection establishment failure or connection resume failure information available in VarConnEstFailReport or VarConnEstFailReportList and if the RPLMN is equal to plmn-Identity stored in VarConnEstFailReport or in at least one of the entries of VarConnEstFailReportList; or

2>	if the UE has connection establishment failure information or connection resume failure information available in VarConnEstFailReport or VarConnEstFailReportList and if the current registered SNPN identity is equal to snpn-Identity stored in VarConnEstFailReport or any entry of VarConnEstFailReportList:

3>	include connEstFailInfoAvailable in the RRCSetupComplete message;

2>	if the UE has radio link failure or handover failure information available in VarRLF-Report and if the RPLMN is included in plmn-IdentityList stored in VarRLF-Report, or

2>	if the UE has radio link failure or handover failure information available in VarRLF-Report of TS 36.331 [10], and if the UE is capable of cross-RAT RLF reporting and if the RPLMN is included in plmn-IdentityList stored in VarRLF-Report of TS 36.331 [10]; or

2>	if the UE has radio link failure or handover failure information available in VarRLF-Report and if the current registered SNPN identity is included in snpn-IdentityList stored in the VarRLF-Report:

3>	include rlf-InfoAvailable in the RRCSetupComplete message;

2>	if the UE has successful handover information available in VarSuccessHO-Report and if the RPLMN is included in plmn-IdentityList stored in VarSuccessHO-Report; or

2>	if the UE has successful handover information available in VarSuccessHO-Report and if the current registered SNPN identity is included in snpn-IdentityList stored in the VarSuccessHO-Report:

3>	include successHO-InfoAvailable in the RRCSetupComplete message;

2>	if the UE has successful PSCell change or addition information available in VarSuccessPSCell-Report and if the RPLMN is included in plmn-IdentityList stored in VarSuccessPSCell-Report; or

2>	if the UE has successful PSCell change or addition information available in VarSuccessPSCell-Report and if the current registered SNPN identity is included in snpn-IdentityList stored in the VarSuccessPSCell-Report:

3>	include successPSCell-InfoAvailable in the RRCSetupComplete message;

2>	if the UE supports storage of mobility history information and the UE has mobility history information available in VarMobilityHistoryReport:

3>	include the mobilityHistoryAvail in the RRCSetupComplete message;

2>	if the UE has at least one stored application layer measurement configuration with appLayerIdleInactiveConfig configured:

3>	include measConfigReportAppLayerAvailable in the RRCSetupComplete message;

2>	if the UE supports uplink RRC message segmentation of UECapabilityInformation according to the network indication rrc-SegAllowed:

3>	may include the ul-RRC-Segmentation in the RRCSetupComplete message;

2>	if the UE supports uplink RRC message segmentation of UECapabilityInformation according to the network indication rrc-MaxCapaSegAllowed:

3>	include the ul-RRC-MaxCapaSegments in the RRCSetupComplete message;

2>	if the RRCSetup is received in response to an RRCResumeRequest, RRCResumeRequest1 or RRCSetupRequest:

3>	if speedStateReselectionPars is configured in the SIB2:

4>	include the mobilityState in the RRCSetupComplete message and set it to the mobility state (as specified in TS 38.304 [20]) of the UE just prior to entering RRC_CONNECTED state;

2>	if SIB1 contains musim-CapRestrictionAllowed:

3>	if supported, include the musim-CapRestrictionInd in the RRCSetupComplete message upon determining it has temporary capability restriction;

2>	if the UE has flight path information available:

3>	include flightPathInfoAvailable;

1>	submit the RRCSetupComplete message to lower layers for transmission, upon which the procedure ends.

NOTE:	Upon reception of musim-CapRestrictionInd in RRCSetupComplete, it is up to network implementation to configure the UE with a limited configuration that is used until network sends RRCReconfiguration based on the actual restricted UE capabilities included in UEAssistanceInformation.

#### 5.3.5.3	Reception of an RRCReconfiguration by the UE

The UE shall perform the following actions upon reception of the RRCReconfiguration, upon execution of the conditional reconfiguration (CHO, CPA, CPC, or subsequent CPAC), or upon execution of an LTM cell switch:

1>	if the RRCReconfiguration is applied due to a conditional reconfiguration execution upon cell selection performed while timer T311 was running, as defined in 5.3.7.3:

2>	remove all the entries in the condReconfigList within the MCG and the SCG VarConditionalReconfig except for the entries in which subsequentCondReconfig is present, if any;

1>	if the RRCReconfiguration includes the daps-SourceRelease:

2>	reset the source MAC and release the source MAC configuration;

2>	for each DAPS bearer:

3>	release the RLC entity or entities as specified in TS 38.322 [4], clause 5.1.3, and the associated logical channel for the source SpCell;

3>	reconfigure the PDCP entity to release DAPS as specified in TS 38.323 [5];

2>	for each SRB:

3>	release the PDCP entity for the source SpCell;

3>	release the RLC entity as specified in TS 38.322 [4], clause 5.1.3, and the associated logical channel for the source SpCell;

2>	release the physical channel configuration for the source SpCell;

2>	discard the keys used in the source SpCell (the KgNB key, the KRRCenc key, the KRRCint key, the KUPint key and the KUPenc key), if any;

1>	if the RRCReconfiguration is received while the timer T348 is running:

2>	if the configuration does not exceed UE temporary capability restriction indicated via musim-CapRestriction included in the last transmission of UEAssistanceInformation:

3>	stop the timer T348;

1>	if the RRCReconfiguration is received via other RAT (i.e., inter-RAT handover to NR):

2>	if the RRCReconfiguration does not include the fullConfig and the UE is connected to 5GC (i.e., delta signalling during intra 5GC handover):

3>	re-use the source RAT SDAP and PDCP configurations if available (i.e., current SDAP/PDCP configurations for all RBs from source E-UTRA RAT prior to the reception of the inter-RAT HO RRCReconfiguration message);

1>	else:

2>	if the RRCReconfiguration includes the fullConfig:

3>	perform the full configuration procedure as specified in 5.3.5.11;

1>	if the RRCReconfiguration includes the masterCellGroup:

2>	perform the cell group configuration for the received masterCellGroup according to 5.3.5.5;

1>	if the RRCReconfiguration includes the masterKeyUpdate:

2>	perform AS security key update procedure as specified in 5.3.5.7;

1>	if the RRCReconfiguration includes the sk-Counter:

2>	perform security key update procedure as specified in 5.3.5.7;

1>	if the RRCReconfiguration includes the secondaryCellGroup:

2>	perform the cell group configuration for the SCG according to 5.3.5.5;

1>	if the RRCReconfiguration includes the mrdc-SecondaryCellGroupConfig:

2>	if the mrdc-SecondaryCellGroupConfig is set to setup:

3>	if the mrdc-SecondaryCellGroupConfig includes mrdc-ReleaseAndAdd:

4>	perform MR-DC release as specified in clause 5.3.5.10;

3>	if the received mrdc-SecondaryCellGroup is set to nr-SCG:

4>	perform the RRC reconfiguration according to 5.3.5.3 for the RRCReconfiguration message included in nr-SCG;

3>	if the received mrdc-SecondaryCellGroup is set to eutra-SCG:

4>	perform the RRC connection reconfiguration as specified in TS 36.331 [10], clause 5.3.5.3 for the RRCConnectionReconfiguration message included in eutra-SCG;

2>	else (mrdc-SecondaryCellGroupConfig is set to release):

3>	perform MR-DC release as specified in clause 5.3.5.10;

NOTE 00:	If the UE receives, within an LTM candidate configuration, an mrdc-SecondaryCellGroupConfig set to release even if no SCG is currently configured at the UE, the UE does not consider this as an invalid configuration.

1>	if the RRCReconfiguration message includes the radioBearerConfig:

2>	perform the radio bearer configuration according to 5.3.5.6;

1>	if the RRCReconfiguration message includes the radioBearerConfig2:

2>	perform the radio bearer configuration according to 5.3.5.6;

1>	if the RRCReconfiguration message includes the measConfig:

2>	perform the measurement configuration procedure as specified in 5.5.2;

1>	if the RRCReconfiguration message includes the dedicatedNAS-MessageList:

2>	forward each element of the dedicatedNAS-MessageList to upper layers in the same order as listed;

1>	if the RRCReconfiguration message includes the dedicatedSIB1-Delivery:

2>	perform the action upon reception of SIB1 as specified in 5.2.2.4.2;

NOTE 0:	If this RRCReconfiguration is associated to the MCG and includes reconfigurationWithSync in spCellConfig and dedicatedSIB1-Delivery, the UE initiates (if needed) the request to acquire required SIBs, according to clause 5.2.2.3.5, only after the random access procedure or the LTM cell switch execution towards the target SpCell is completed.

1>	if the RRCReconfiguration message includes the dedicatedSystemInformationDelivery:

2>	perform the action upon reception of System Information as specified in 5.2.2.4;

2>	if all the SIB(s) and/or posSIB(s) requested in DedicatedSIBRequest message have been acquired:

3>	stop timer T350, if running;

1>	if the RRCReconfiguration message includes the dedicatedPosSysInfoDelivery:

2>	perform the action upon reception of the contained posSIB(s), as specified in clause 5.2.2.4.16;

2>	if all the SIB(s) and/or posSIB(s) requested in DedicatedSIBRequest message have been acquired:

3>	stop timer T350, if running;

1>	if the RRCReconfiguration message includes the otherConfig:

2>	perform the other configuration procedure as specified in 5.3.5.9;

1>	if the RRCReconfiguration message includes the bap-Config:

2>	perform the BAP configuration procedure as specified in 5.3.5.12;

1>	if the RRCReconfiguration message includes the iab-IP-AddressConfigurationList:

2>	if iab-IP-AddressToReleaseList is included:

3>	perform release of IP address as specified in 5.3.5.12a.1.1;

2>	if iab-IP-AddressToAddModList is included:

3>	perform IAB IP address addition/update as specified in 5.3.5.12a.1.2;

1>	if the RRCReconfiguration message includes the conditionalReconfiguration:

2>	perform conditional reconfiguration as specified in 5.3.5.13;

1>	if the RRCReconfiguration message includes the needForGapsConfigNR:

2>	if needForGapsConfigNR is set to setup:

3>	consider itself to be configured to provide the measurement gap requirement information of NR target bands;

2>	else:

3>	consider itself not to be configured to provide the measurement gap requirement information of NR target bands;

1>	if the RRCReconfiguration message includes the needForGapNCSG-ConfigNR:

2>	if needForGapNCSG-ConfigNR is set to setup:

3>	consider itself to be configured to provide the measurement gap and NCSG requirement information of NR target bands;

2>	else:

3>	consider itself not to be configured to provide the measurement gap and NCSG requirement information of NR target bands;

1>	if the RRCReconfiguration message includes the needForGapNCSG-ConfigEUTRA:

2>	if needForGapNCSG-ConfigEUTRA is set to setup:

3>	consider itself to be configured to provide the measurement gap and NCSG requirement information of E-UTRA target bands;

2>	else:

3>	consider itself not to be configured to provide the measurement gap and NCSG requirement information of E-UTRA target bands;

1>	if the RRCReconfiguration message includes the onDemandSIB-Request:

2>	if onDemandSIB-Request is set to setup:

3>	consider itself to be configured to request SIB(s) or posSIB(s) in RRC_CONNECTED in accordance with clause 5.2.2.3.5;

2>	else:

3>	consider itself not to be configured to request SIB(s) or posSIB(s) in RRC_CONNECTED in accordance with clause 5.2.2.3.5;

3>	stop timer T350, if running;

1>	if the RRCReconfiguration message includes the sl-ConfigDedicatedNR:

2>	perform the sidelink dedicated configuration procedure as specified in 5.3.5.14;

NOTE 0a:	If the sl-ConfigDedicatedNR was received embedded within an E-UTRA RRCConnectionReconfiguration message, the UE does not build an NR RRCReconfigurationComplete message for the received sl-ConfigDedicatedNR.

1>	if the RRCReconfiguration message includes the sl-L2RelayUE-Config:

2>	perform the L2 U2N or U2U Relay UE configuration procedure as specified in 5.3.5.15;

1>	if the RRCReconfiguration message includes the sl-L2RemoteUE-Config:

2>	perform the L2 U2N or U2U Remote UE configuration procedure as specified in 5.3.5.16;

1>	if the RRCReconfiguration message includes the dedicatedPagingDelivery:

2>	perform the Paging message reception procedure as specified in 5.3.2.3;

1>	if the RRCReconfiguration message includes the sl-ConfigDedicatedEUTRA-Info:

2>	perform related procedures for V2X sidelink communication in accordance with TS 36.331 [10], clause 5.3.10 and clause 5.5.2;

1>	if the RRCReconfiguration message includes the ul-GapFR2-Config:

2>	perform the FR2 UL gap configuration procedure as specified in 5.3.5.13c;

1>	if the RRCReconfiguration message includes the musim-GapConfig:

2>	perform the MUSIM gap configuration procedure as specified in 5.3.5.9a;

1>	if the RRCReconfiguration message includes the appLayerMeasConfig:

2>	for each application layer measurement configuration with appLayerIdleInactiveConfig configured:

3>	if the RPLMN is not included in plmn-IdentityList in VarAppLayerPLMN-ListConfig:

4>	forward the measConfigAppLayerId and inform upper layers about the release of the application layer measurement configuration;

4>	release the application layer measurement configuration including its fields in the UE variables VarAppLayerIdleConfig and VarAppLayerPLMN-ListConfig;

4>	discard any application layer measurement reports which were not yet fully submitted to lower layers for transmission;

4>	consider itself not to be configured to send application layer measurement report for the measConfigAppLayerId;

2>	if idleInactiveReportAllowed is included in the RRCReconfiguration message:

3> if the UE is configured with at least one application layer measurement configuration with appLayerIdleInactiveConfig configured:

4>	initiate the procedure in 5.7.16.2 after the RRCReconfigurationComplete has been transmitted;

2>	else:

3>	for each application layer measurement configuration with appLayerIdleInactiveConfig configured:

4>	forward the measConfigAppLayerId and inform upper layers about the release of the application layer measurement configuration;

4>	release the application layer measurement configuration including its fields in the UE variables VarAppLayerIdleConfig and VarAppLayerPLMN-ListConfig;

4>	discard any application layer measurement reports which were not yet fully submitted to lower layers for transmission;

4>	consider itself not to be configured to send application layer measurement reports for the measConfigAppLayerId;

2>	perform the application layer measurement configuration procedure as specified in 5.3.5.13d;

1>	if the RRCReconfiguration message includes the ue-TxTEG-RequestUL-TDOA-Config:

2>	if ue-TxTEG-RequestUL-TDOA-Config is set to setup:

3>	perform the UE positioning assistance information procedure as specified in 5.7.14;

2>	else:

3>	release the configuration of UE positioning assistance information;

1>	if the RRCReconfiguration message includes the aerial-Config:

2>	(re)configure the aerial parameters in accordance with the included aerial-Config;

1>	if the RRCReconfiguration message includes the sl-IndirectPathAddChange:

2>	perform the SL indirect path specific configuration procedure as specified in 5.3.5.17.2.2;

1>	if the RRCReconfiguration message includes the n3c-IndirectPathAddChange:

2>	perform configuration procedure for the remote UE part of N3C indirect path as specified in 5.3.5.17.3.2;

1>	if the RRCReconfiguration message includes the n3c-IndirectPathConfigRelay:

2>	perform the configuration procedure for the relay UE part of N3C indirect path as specified in 5.3.5.17.3.3;

1>	if the RRCReconfiguration message includes the ltm-Config:

2>	if the ltm-Config is set to setup:

3>	perform the LTM configuration procedure as specified in 5.3.5.18.1;

2>	else:

3>	perform the LTM configuration release procedure as specified in clause 5.3.5.18.7;

1>	if the RRCReconfiguration message includes the srs-PosResourceSetLinkedForAggBWList:

2>	if srs-PosResourceSetLinkedForAggBWList is set to setup:

3>	perform the SRS for positioning transmission using bandwidth aggregation provided in configuration srs-PosResourceSetLinkedForAggBW as specified in TS 38.211 [16];

2>	else:

3>	release all the configuration of srs-PosResourceSetLinkedForAggBW;

1>	set the content of the RRCReconfigurationComplete message as follows:

2>	if the RRCReconfiguration includes the masterCellGroup containing the reportUplinkTxDirectCurrent:

3>	include the uplinkTxDirectCurrentList for each MCG serving cell with UL;

3>	include uplinkDirectCurrentBWP-SUL for each MCG serving cell configured with SUL carrier, if any, within the uplinkTxDirectCurrentList;

2>	if the RRCReconfiguration includes the masterCellGroup containing the reportUplinkTxDirectCurrentTwoCarrier:

3>	include in the uplinkTxDirectCurrentTwoCarrierList the list of uplink Tx DC locations for the configured intra-band uplink carrier aggregation in the MCG;

2>	if the RRCReconfiguration includes the masterCellGroup containing the reportUplinkTxDirectCurrentMoreCarrier:

3>	include in the uplinkTxDirectCurrentMoreCarrierList the list of uplink Tx DC locations for the configured intra-band uplink carrier aggregation in the MCG;

2>	if the RRCReconfiguration includes the secondaryCellGroup containing the reportUplinkTxDirectCurrent:

3>	include the uplinkTxDirectCurrentList for each SCG serving cell with UL;

3>	include uplinkDirectCurrentBWP-SUL for each SCG serving cell configured with SUL carrier, if any, within the uplinkTxDirectCurrentList;

2>	if the RRCReconfiguration includes the secondaryCellGroup containing the reportUplinkTxDirectCurrentTwoCarrier:

3>	include in the uplinkTxDirectCurrentTwoCarrierList the list of uplink Tx DC locations for the configured intra-band uplink carrier aggregation in the SCG;

2>	if the RRCReconfiguration includes the secondaryCellGroup containing the reportUplinkTxDirectCurrentMoreCarrier:

3>	include in the uplinkTxDirectCurrentMoreCarrierList the list of uplink Tx DC locations for the configured intra-band uplink carrier aggregation in the SCG;

NOTE 0b:	The UE does not expect that the reportUplinkTxDirectCurrentTwoCarrier or reportUplinkTxDirectCurrentMoreCarrier is received in both masterCellGroup and in secondaryCellGroup. Network only configures at most one of reportUplinkTxDirectCurrent, reportUplinkTxDirectCurrentTwoCarrier or reportUplinkTxDirectCurrentMoreCarrier in one RRC message.

2>	if the RRCReconfiguration message includes the mrdc-SecondaryCellGroupConfig with mrdc-SecondaryCellGroup set to eutra-SCG:

3>	include in the eutra-SCG-Response the E-UTRA RRCConnectionReconfigurationComplete message in accordance with TS 36.331 [10] clause 5.3.5.3;

2> if the RRCReconfiguration message includes the mrdc-SecondaryCellGroupConfig with mrdc-SecondaryCellGroup set to nr-SCG:

3>	include in the nr-SCG-Response the SCG RRCReconfigurationComplete message;

3>	if the RRCReconfiguration message is applied due to conditional reconfiguration execution and the RRCReconfiguration message does not include the reconfigurationWithSync in the masterCellGroup:

4>	include in the selectedCondRRCReconfig the condReconfigId for the selected cell of conditional reconfiguration execution;

4>	if a new sk-Counter value has been selected due to the conditional reconfiguration execution for subsequent CPAC:

5>	include selectedSK-Counter and set its value to the selected sk-Counter value;

3>	if the RRCReconfiguration message is applied due to conditional reconfiguration execution and condExecutionCondPSCell is configured for the selected PSCell:

4>	include in the selectedPSCellForCHO-WithSCG and set it to the information of the selected PSCell;

2>	if the RRCReconfiguration includes the reconfigurationWithSync in spCellConfig of an MCG:

3>	if the UE has logged measurements available for NR and if the RPLMN is included in plmn-IdentityList stored in VarLogMeasReport; or

3>	if the UE has logged measurements available for NR and if the current registered SNPN identity is included in snpn-ConfigID-List stored in the VarLogMeasReport:

4>	include the logMeasAvailable in the RRCReconfigurationComplete message;

2> if the UE has not previously sent successHO-InfoAvailable for the current content of VarSuccessHO-Report since the UE entered the serving cell in RRC_CONNECTED state:

3>	if the UE has successful handover information available in VarSuccessHO-Report and if the RPLMN is included in plmn-IdentityList stored in VarSuccessHO-Report; or

3>	if the UE has successful handover information available in VarSuccessHO-Report and if the current registered SNPN identity is included in snpn-IdentityList stored in the VarSuccessHO-Report:

4>	include successHO-InfoAvailable in the RRCReconfigurationComplete message;

4>	if Bluetooth measurement results are included in the logged measurements the UE has available for NR:

5>	include the logMeasAvailableBT in the RRCReconfigurationComplete message;

4>	if WLAN measurement results are included in the logged measurements the UE has available for NR:

5>	include the logMeasAvailableWLAN in the RRCReconfigurationComplete message;

3>	if the sigLoggedMeasType in VarLogMeasReport is included; or

3>	if the UE supports the override protection of the signalling based logged MDT for inter-RAT (i.e. LTE to NR), and if the sigLoggedMeasType in VarLogMeasReport of TS 36.331 [10] is included:

4>	if T330 timer is running (associated to the logged measurement configuration for NR or for LTE):

5>	set sigLogMeasConfigAvailable to true in the RRCReconfigurationComplete message;

4>	else:

5>	if the UE has logged measurements in VarLogMeasReport or in VarLogMeasReport of TS 36.331 [10]:

6>	set sigLogMeasConfigAvailable to false in the RRCReconfigurationComplete message;

3>	if the UE has connection establishment failure or connection resume failure information available in VarConnEstFailReport or VarConnEstFailReportList and if the RPLMN is equal to plmn-Identity stored in VarConnEstFailReport or in at least one of the entries of VarConnEstFailReportList; or

3>	if the UE has connection establishment failure information or connection resume failure information available in VarConnEstFailReport or VarConnEstFailReportList and if the registered SNPN identity is equal to snpn-Identity in networkIdentity stored in VarConnEstFailReport or any entry of VarConnEstFailReportList:

4>	include connEstFailInfoAvailable in the RRCReconfigurationComplete message;

3>	if the UE has radio link failure or handover failure information available in VarRLF-Report and if the RPLMN is included in plmn-IdentityList stored in VarRLF-Report; or

3>	if the UE has radio link failure or handover failure information available in VarRLF-Report of TS 36.331 [10] and if the UE is capable of cross-RAT RLF reporting and if the RPLMN is included in plmn-IdentityList stored in VarRLF-Report of TS 36.331 [10]; or

3>	if the UE has radio link failure or handover failure information available in VarRLF-Report and if the current registered SNPN identity is included in snpn-IdentityList stored in VarRLF-Report:

4>	include rlf-InfoAvailable in the RRCReconfigurationComplete message;

3>	if the UE was configured with successHO-Config when connected to the source PCell:

4>	if the applied RRCReconfiguration is not due to a conditional reconfiguration execution upon cell selection performed while timer T311 was running, as defined in 5.3.7.3, and the applied RRCReconfiguration is not due to an LTM cell switch execution upon cell selection performed while timer T311 was running, as defined in 5.3.7.3; or

4>	if the applied RRCReconfiguration is not received when T316 was running:

5>	perform the actions for the successful handover report determination as specified in clause 5.7.10.6, upon successfully completing the Random Access procedure triggered for the reconfigurationWithSync in spCellConfig of the MCG, or upon an indication from lower layer that the LTM cell switch execution has been successfully completed;

4>	if applied RRCReconfiguration is received when T316 was running:

5>	release successHO-Config configured by the source PCell and thresholdPercentageT304 if configured by the target PCell;

3>	if the UE has successful handover information available in VarSuccessHO-Report and if the RPLMN is included in plmn-IdentityList stored in VarSuccessHO-Report; or

3>	if the UE has successful handover information available in VarSuccessHO-Report and if the current registered SNPN identity is included in snpn-IdentityList stored in the VarSuccessHO-Report:

4>	include successHO-InfoAvailable in the RRCReconfigurationComplete message;

3>	release successPSCell-Config configured by the source PCell, if available;

3>	if the UE has successful PSCell change or addition information available in VarSuccessPSCell-Report and if the RPLMN is included in plmn-IdentityList stored in VarSuccessPSCell-Report; or

3>	if the UE has successful PSCell change or addition information available in VarSuccessPSCell-Report and if the current registered SNPN identity is included in snpn-IdentityList stored in the VarSuccessPSCell-Report:

4>	include successPSCell-InfoAvailable in the RRCReconfigurationComplete message;

2>	if the RRCReconfiguration message was received via SRB1, but not within mrdc-SecondaryCellGroup or E-UTRA RRCConnectionReconfiguration or E-UTRA RRCConnectionResume:

3>	if the UE is configured to provide the measurement gap requirement information of NR target bands:

4>	if the RRCReconfiguration message includes the needForGapsConfigNR; or

4>	if the NeedForGapsInfoNR information is changed compared to last time the UE reported this information; or

4>	if the RRCReconfiguration message includes the needForInterruptionConfigNR and set it to enabled; or

4>	if the needForInterruptionConfigNR is enabled and the NeedForInterruptionInfoNR information is changed compared to last time the UE reported this information:

5>	include the NeedForGapsInfoNR and set the contents as follows:

6>	include intraFreq-needForGap and set the gap requirement information of intra-frequency measurement for each NR serving cell;

6>	if requestedTargetBandFilterNR is configured:

7>	for each supported NR band that is also included in requestedTargetBandFilterNR, include an entry in interFreq-needForGap and set the gap requirement information for that band;

6>	else:

7>	include an entry in interFreq-needForGap and set the corresponding gap requirement information for each supported NR band;

5>	if the needForInterruptionConfigNR is enabled:

6>	include the needForInterruptionInfoNR and set the contents as follows:

7>	include intraFreq-needForInterruption with the same number of entries, and listed in the same order, as in intraFreq-needForGap;

7> for each entry in intraFreq-needForInterruption:

8>	include interruptionIndication and set the interruption requirement information if the corresponding entry in intraFreq-needForGap is set to no-gap;

7>	include interFreq-needForInterruption with the same number of entries, and listed in the same order, as in interFreq-needForGap;

7> for each entry in interFreq-needForInterruption:

8>	include interruptionIndication and set the interruption requirement information if the corresponding entry in interFreq-needForGap is set to no-gap;

3>	if the UE is configured to provide the measurement gap and NCSG requirement information of NR target bands:

4>	if the RRCReconfiguration message includes the needForGapNCSG-ConfigNR; or

4>	if the needForGapNCSG-InfoNR information is changed compared to last time the UE reported this information:

5>	include the NeedForGapNCSG-InfoNR and set the contents as follows:

6>	include intraFreq-needForNCSG and set the gap and NCSG requirement information of intra-frequency measurement for each NR serving cell;

6>	if requestedTargetBandFilterNCSG-NR is configured:

7>	for each supported NR band included in requestedTargetBandFilterNCSG-NR, include an entry in interFreq-needForNCSG and set the NCSG requirement information for that band;

6>	else:

7>	include an entry for each supported NR band in interFreq-needForNCSG and set the corresponding NCSG requirement information;

3>	if the UE is configured to provide the measurement gap and NCSG requirement information of E-UTRA target bands:

4>	if the RRCReconfiguration message includes the needForGapNCSG-ConfigEUTRA; or

4>	if the needForGapNCSG-InfoEUTRA information is changed compared to last time the UE reported this information:

5>	include the NeedForGapNCSG-InfoEUTRA and set the contents as follows:

6>	if requestedTargetBandFilterNCSG-EUTRA is configured, for each supported E-UTRA band included in requestedTargetBandFilterNCSG-EUTRA, include an entry in needForNCSG-EUTRA and set the NCSG requirement information for that band; otherwise, include an entry for each supported E-UTRA band in needForNCSG-EUTRA and set the corresponding NCSG requirement information;

2>	if the UE has (updated) flight path information available:

3>	if the UE had not provided a flight path information since last entering RRC_CONNECTED state; or

3>	if at least one waypoint or a timestamp corresponding to a waypoint location that was not previously provided since last entering RRC_CONNECTED state is available; or

3>	if at least one upcoming waypoint or a timestamp corresponding to a waypoint location that was previously provided since last entering RRC_CONNECTED state is to be removed; or

3>	if flightPathUpdateDistanceThr is configured and, for at least one waypoint, the 3D distance between the previously provided location and the new location is more than the distance threshold configured by flightPathUpdateDistanceThr; or

3> if flightPathUpdateTimeThr is configured and, for at least one waypoint, the time difference between the previously provided timestamp and the new timestamp, if available, is more than the time threshold configured by flightPathUpdateTimeThr:

4>	include flightPathInfoAvailable;

NOTE 0c:	If neither flightPathUpdateDistanceThr nor flightPathUpdateTimeThr is configured, it is up to UE implementation whether to include flightPathInfoAvailable when updated flight path information is available.

2>	if the UE has at least one stored application layer measurement configuration with appLayerIdleInactiveConfig configured which has not been successfully transmitted since entering RRC_CONNECTED state:

3>	include measConfigReportAppLayerAvailable;

2>	if this RRCReconfiguration message is applied due to an LTM cell switch execution procedure according to clause 5.3.5.18.6:

3>	include in the appliedLTM-CandidateId the LTM-CandidateId of the applied LTM candidate configuration;

1>	if the UE is configured with E-UTRA nr-SecondaryCellGroupConfig (UE in (NG)EN-DC):

2>	if the RRCReconfiguration message was received via E-UTRA SRB1 as specified in TS 36.331 [10]; or

2>	if the RRCReconfiguration message was received via E-UTRA RRC message RRCConnectionReconfiguration within MobilityFromNRCommand (handover from NR standalone to (NG)EN-DC);

3>	if the RRCReconfiguration is applied due to a conditional reconfiguration execution for CPC which is configured via conditionalReconfiguration contained in nr-SecondaryCellGroupConfig specified in TS 36.331 [10]:

4>	submit the RRCReconfigurationComplete message via the E-UTRA MCG embedded in E-UTRA RRC message ULInformationTransferMRDC as specified in TS 36.331 [10], clause 5.6.2a.

3>	else if the RRCReconfiguration message was included in E-UTRA RRCConnectionResume message:

4>	submit the RRCReconfigurationComplete message via E-UTRA embedded in E-UTRA RRC message RRCConnectionResumeComplete as specified in TS 36.331 [10], clause 5.3.3.4a;

3>	else:

4>	submit the RRCReconfigurationComplete via E-UTRA embedded in E-UTRA RRC message RRCConnectionReconfigurationComplete as specified in TS 36.331 [10], clause 5.3.5.3/5.3.5.4/5.4.2.3;

3>	if the scg-State is not included in the E-UTRA message (RRCConnectionReconfiguration or RRCConnectionResume) containing the RRCReconfiguration message:

4>	perform SCG activation as specified in 5.3.5.13a;

4>	if reconfigurationWithSync was included in spCellConfig of an SCG:

5>	initiate the Random Access procedure on the PSCell, as specified in TS 38.321 [3];

4>	else if the SCG was deactivated before the reception of the E-UTRA RRC message containing the RRCReconfiguration message:

5>	if bfd-and-RLM was not configured to true before the reception of the E-UTRA RRCConnectionReconfiguration or RRCConnectionResume message containing the RRCReconfiguration message or if lower layers indicate that a Random Access procedure is needed for SCG activation:

6>	initiate the Random Access procedure on the SpCell, as specified in TS 38.321 [3];

5>	else the procedure ends;

4>	else the procedure ends;

3>	else:

4>	perform SCG deactivation as specified in 5.3.5.13b;

4>	the procedure ends;

2>	if the RRCReconfiguration message was received within nr-SecondaryCellGroupConfig in RRCConnectionReconfiguration message received via SRB3 within DLInformationTransferMRDC:

3>	submit the RRCReconfigurationComplete via E-UTRA embedded in E-UTRA RRC message RRCConnectionReconfigurationComplete as specified in TS 36.331 [10], clause 5.3.5.3/5.3.5.4;

3>	if the scg-State is not included in the RRCConnectionReconfiguration:

4>	if reconfigurationWithSync was included in spCellConfig of an SCG:

5>	initiate the Random Access procedure on the SpCell, as specified in TS 38.321 [3];

4>	else the procedure ends;

3>	else:

4>	perform SCG deactivation as specified in 5.3.5.13b;

4>	the procedure ends;

NOTE 1:	The order the UE sends the RRCConnectionReconfigurationComplete message and performs the Random Access procedure towards the SCG is left to UE implementation.

2>	else (RRCReconfiguration was received via SRB3) but not within DLInformationTransferMRDC:

3>	submit the RRCReconfigurationComplete message via SRB3 to lower layers for transmission using the new configuration;

NOTE 2:	In (NG)EN-DC and NR-DC, in the case RRCReconfiguration is received via SRB1 or within DLInformationTransferMRDC via SRB3, the random access is triggered by RRC layer itself as there is not necessarily other UL transmission. In the case RRCReconfiguration is received via SRB3 but not within DLInformationTransferMRDC, the random access is triggered by the MAC layer due to arrival of RRCReconfigurationComplete.

1>	else if the RRCReconfiguration message was received via SRB1 within the nr-SCG within mrdc-SecondaryCellGroup (UE in NR-DC, mrdc-SecondaryCellGroup was received in RRCReconfiguration or RRCResume via SRB1):

2>	if the RRCReconfiguration is applied due to a conditional reconfiguration execution for CPC or subsequent CPAC which is configured via conditionalReconfiguration contained in nr-SCG within mrdc-SecondaryCellGroup; or

2>	if the RRCReconfiguration is applied due to an LTM cell switch execution:

3>	submit the RRCReconfigurationComplete message via SRB1 embedded in NR RRC message ULInformationTransferMRDC as specified in clause 5.7.2a.3.

2>	if the scg-State is not included in the RRCReconfiguration or RRCResume message containing the RRCReconfiguration message:

3>	perform SCG activation as specified in 5.3.5.13a;

3>	if reconfigurationWithSync was included in spCellConfig in nr-SCG:

4>	if the RRCReconfiguration message is not applied due to an LTM cell switch execution for which lower layer indicate to skip the Random Access procedure:

5>	initiate the Random Access procedure on the PSCell, as specified in TS 38.321 [3];

4>	if the UE was configured with successPSCell-Config when connected to the source PSCell (for PSCell change) or to the PCell (for PSCell addition or change):

5>	perform the actions for the successful PSCell change or addition report determination as specified in clause 5.7.10.7, upon successfully completing the Random Access procedure triggered for the reconfigurationWithSync in spCellConfig of the SCG;

3>	else if the SCG was deactivated before the reception of the NR RRC message containing the RRCReconfiguration message:

4>	if bfd-and-RLM was not configured to true before the reception of the RRCReconfiguration or RRCResume message containing the RRCReconfiguration message; or

4>	if lower layers indicate that a Random Access procedure is needed for SCG activation:

5>	initiate the Random Access procedure on the PSCell, as specified in TS 38.321 [3];

4>	else the procedure ends;

3>	else the procedure ends;

2>	else

3>	perform SCG deactivation as specified in 5.3.5.13b;

3>	the procedure ends;

NOTE 2a:	The order in which the UE sends the RRCReconfigurationComplete message and performs the Random Access procedure towards the SCG is left to UE implementation.

1>	else if the RRCReconfiguration message was received via SRB3 (UE in NR-DC):

2>	if the RRCReconfiguration message was received within DLInformationTransferMRDC:

3>	if the RRCReconfiguration message was received within the nr-SCG within mrdc-SecondaryCellGroup (NR SCG RRC Reconfiguration):

4>	if the scg-State is not included in the RRCReconfiguration message containing the RRCReconfiguration message:

5>	if reconfigurationWithSync was included in spCellConfig in nr-SCG:

6>	initiate the Random Access procedure on the PSCell, as specified in TS 38.321 [3];

6>	if the UE was configured with successPSCell-Config when connected to the source PSCell (for PSCell change) or to the PCell (for PSCell addition or change):

7>	perform the actions for the successful PSCell change report determination as specified in clause 5.7.10.7, upon successfully completing the Random Access procedure triggered for the reconfigurationWithSync in spCellConfig of the SCG;

5>	else:

6>	the procedure ends;

4>	else:

5>	perform SCG deactivation as specified in 5.3.5.13b;

5>	the procedure ends;

3>	else:

4>	if the RRCReconfiguration does not include the mrdc-SecondaryCellGroupConfig:

5>	if the RRCReconfiguration includes the scg-State:

6>	perform SCG deactivation as specified in 5.3.5.13b;

4>	submit the RRCReconfigurationComplete message via SRB1 to lower layers for transmission using the new configuration;

2>	else:

3>	if the RRCReconfiguration includes the reconfigurationWithSync in spCellConfig for the SCG; and

3>	if the UE was configured with successPSCell-Config when connected to the source PSCell (for PSCell change):

4>	perform the actions for the successful PSCell change report determination as specified in clause 5.7.10.7, upon successfully completing the Random Access procedure triggered for the reconfigurationWithSync in spCellConfig of the SCG;

3>	if the UE has successful PSCell change or addition information available in VarSuccessPSCell-Report and if the RPLMN is included in plmn-IdentityList stored in VarSuccessPSCell-Report; or

3>	if the UE has successful PSCell change or addition information available in VarSuccessPSCell-Report and if the current registered SNPN identity is included in snpn-IdentityList stored in the VarSuccessPSCell-Report:

4>	include successPSCell-InfoAvailable in the RRCReconfigurationComplete message;

3>	submit the RRCReconfigurationComplete message via SRB3 to lower layers for transmission using the new configuration;

1>	else (RRCReconfiguration was received via SRB1):

2>	if the UE is in NR-DC and;

2>	if the RRCReconfiguration does not include the mrdc-SecondaryCellGroupConfig:

3>	if the RRCReconfiguration includes the scg-State:

4>	perform SCG deactivation as specified in 5.3.5.13b;

3>	else:

4>	perform SCG activation without SN message as specified in 5.3.5.13b1;

2>	if the reconfigurationWithSync was included in spCellConfig of an MCG:

3>	if ta-Report or ta-ReportATG is configured with value enabled and the UE supports TA reporting:

4>	indicate TA report initiation to lower layers;

2>	submit the RRCReconfigurationComplete message via SRB1 to lower layers for transmission using the new configuration;

2>	if this is the first RRCReconfiguration message after successful completion of the RRC re-establishment procedure:

3>	resume SRB2, SRB4, DRBs, multicast MRB, and BH RLC channels for IAB-MT, and Uu Relay RLC channels for L2 U2N Relay UE, that are suspended;

1>	if sl-IndirectPathAddChange was included in RRCReconfiguration message:

2>	if SRB1 is configured as split SRB and pdcp-Duplication is configured:

3>	when successfully sending RRCReconfigurationComplete message via SL indirect path (i.e., PC5 RLC acknowledgement is received from target L2 U2N Relay UE):

4>	stop timer T421;

2> else (i.e. split SRB1 with duplication is not configured):

3> when receiving RRCReconfigurationCompleteSidelink message from target L2 U2N Relay UE:

4>	stop timer T421;

1>	if reconfigurationWithSync was included in spCellConfig of an MCG or SCG and when MAC of an NR cell group successfully completes a Random Access procedure triggered above; or,

1>	if sl-PathSwitchConfig was included in reconfigurationWithSync included in spCellConfig of an MCG, and when successfully sending RRCReconfigurationComplete message (i.e., PC5 RLC acknowledgement is received from target L2 U2N Relay UE); or,

1>	if rach-LessHO was included in reconfigurationWithSync included in spCellConfig of an MCG, and upon indication from lower layers that the RACH-less handover has been successfully completed; or,

1>	if reconfigurationWithSync was included in spCellConfig of an MCG or SCG and the RRCReconfiguration message is applied due to an LTM cell switch execution and upon an indication from lower layer that the LTM cell switch execution has been successfully completed:

2>	stop timer T304 for that cell group if running;

2>	if rach-LessHO was included in reconfigurationWithSync included in spCellConfig of an MCG, and upon indication from lower layers that the RACH-less handover has been successfully completed; or,

2>	if reconfigurationWithSync was included in spCellConfig of an MCG or SCG and the RRCReconfiguration message is applied due to an LTM cell switch execution and upon an indication from lower layer that the LTM cell switch execution has been successfully completed:

3>	release dedicated preambles provided in rach-ConfigDedicated within reconfigurationWithSync, if configured;

3>	release dedicated msgA PUSCH resources provided in rach-ConfigDedicated within reconfigurationWithSync, if configured;

2>	if sl-PathSwitchConfig was included in reconfigurationWithSync:

3>	if the sl-IndirectPathMaintain is not included in reconfigurationWithSync:

4>	stop timer T420;

4>	release all radio resources, including release of the RLC entities and the MAC configuration at the source side;

4>	reset MAC used in the source cell;

3>	else (sl-IndirectPathMaintain is included):

4>	release radio resources on the direct path, including release of the RLC entities and the MAC configuration;

4>	reset MAC used in the source cell;

2>	if rach-LessHO was included in reconfigurationWithSync and cg-RRC-Configuration was configured:

3>	release the uplink grant configured for RACH-less handover;

NOTE 2b:	PDCP and SDAP configured by the source prior to the path switch that are reconfigured and re-used by target when delta signalling is used, are not released as part of this procedure.

2>	stop timer T310 for source SpCell if running;

2>	apply the parts of the CSI reporting configuration, the scheduling request configuration and the sounding RS configuration that do not require the UE to know the SFN of the respective target SpCell, if any;

2>	apply the parts of the measurement and the radio resource configuration that require the UE to know the SFN of the respective target SpCell (e.g. measurement gaps, periodic CQI reporting, scheduling request configuration, sounding RS configuration), if any, upon acquiring the SFN of that target SpCell;

2>	for each DRB configured as DAPS bearer, request uplink data switching to the PDCP entity, as specified in TS 38.323 [5];

2>	if the reconfigurationWithSync was included in spCellConfig of an MCG:

3>	if T390 is running:

4>	stop timer T390 for all access categories;

4>	perform the actions as specified in 5.3.14.4.

3>	if T350 is running:

4>	stop timer T350;

3>	if RRCReconfiguration does not include dedicatedSIB1-Delivery and

3>	if the active downlink BWP, which is indicated by the firstActiveDownlinkBWP-Id for the target SpCell of the MCG, has a common search space configured by searchSpaceSIB1:

4>	acquire the SIB1, which is scheduled as specified in TS 38.213 [13], of the target SpCell of the MCG;

4>	upon acquiring SIB1, perform the actions specified in clause 5.2.2.4.2;

2>	if the RRCReconfiguration message is applied due to a conditional reconfiguration execution and the RRCReconfiguration message is contained in an entry in MCG VarConditionalReconfig that includes the subsequentCondReconfig:

3>	for each entry in the condReconfigList within the MCG VarConditionalReconfig:

4>	if there is an entry in condExecutionCondToAddModList within the subsequentCondReconfig that has subsequentCondReconfigId matching the condReconfigId in the entry of the condReconfigList:

5>	if subsequentCondExecutionCondSCG is included in the entry of the condExecutionCondToAddModList:

6>	store in the condExecutionCondSCG in the entry of the condReconfigList the value of subsequentCondExecutionCondSCG in the entry of the condExecutionCondToAddModList;

2>	if the RRCReconfiguration message is applied due to a conditional reconfiguration execution and the RRCReconfiguration message is contained in an entry in SCG VarConditionalReconfig that includes the subsequentCondReconfig:

3>	for each entry in the condReconfigList within the SCG VarConditionalReconfig:

4>	if there is an entry in condExecutionCondToAddModList within the subsequentCondReconfig that has subsequentCondReconfigId matching the condReconfigId in the entry of the condReconfigList:

5>	if subsequentCondExecutionCond is included in the entry of the condExecutionCondToAddModList:

6>	store in the condExecutionCond in the entry of the condReconfigList the value of subsequentCondExecutionCond in the entry of the condExecutionCondToAddModList;

2>	if the reconfigurationWithSync was included in spCellConfig of an MCG; or

2>	if the reconfigurationWithSync was included in spCellConfig of an SCG and the CPA, CPC, or subsequent CPAC was configured:

3>	remove all the entries in the condReconfigList within the MCG and the SCG VarConditionalReconfig except for the entries in which subsequentCondReconfig is present, if any;

3>	remove all the entries within VarConditionalReconfiguration as specified in TS 36.331 [10], clause 5.3.5.9.6, if any;

3>	for each measId of the MCG measConfig, if configured, and for each measId of the SCG measConfig, if configured, if the associated reportConfig has a reportType set to condTriggerConfig:

4>	if the reportConfigId is not associated with any measId indicated by the condExecutionCond or the condExecutionCondSCG in an entry of condReconfigList in VarConditionalReconfig in which subsequentCondReconfig is included:

5>	remove the entry with the matching reportConfigId from the reportConfigList within the VarMeasConfig;

4>	if the associated measObjectId is only associated to a reportConfig with reportType set to condTriggerConfig; and

4>	if the measObjectId is not associated with any measId indicated by the condExecutionCond or the condExecutionCondSCG in an entry of condReconfigList in VarConditionalReconfig in which subsequentCondReconfig is included:

5>	remove the entry with the matching measObjectId from the measObjectList within the VarMeasConfig;

4>	remove the entry with the matching measId from the measIdList within the VarMeasConfig;

2>	if reconfigurationWithSync was included in masterCellGroup or secondaryCellGroup:

3>	if the UE initiated transmission of a UEAssistanceInformation message for the corresponding cell group during the last 1 second, and the UE is still configured to provide the concerned UE assistance information for the corresponding cell group; or

3>	if the RRCReconfiguration message is applied due to a conditional reconfiguration execution or an LTM cell switch procedure, and the UE is configured to provide UE assistance information for the corresponding cell group, and the UE has initiated transmission of a UEAssistanceInformation message for the corresponding cell group since it was configured to do so in accordance with 5.7.4.2:

4>	initiate transmission of a UEAssistanceInformation message for the corresponding cell group in accordance with clause 5.7.4.3 to provide the concerned UE assistance information;

4>	start or restart the prohibit timer (if exists) associated with the concerned UE assistance information with the timer value set to the value in corresponding configuration;

4>	start or restart the leave without response timer (if exists) with the timer value set to the value in the musim-LeaveAssistanceConfig or the wait timer (if exists) with the timer value set to the value in musim-CapabilityRestrictionConfig;

3>	if SIB12 is provided by the target PCell, and the UE initiated transmission of a SidelinkUEInformationNR message indicating a change of NR sidelink communication/discovery related parameters relevant in target PCell (i.e. change of sl-RxInterestedFreqList or sl-TxResourceReqList) during the last 1 second preceding reception of the RRCReconfiguration message including reconfigurationWithSync in spCellConfig of an MCG; or

3>	if the RRCReconfiguration message is applied due to a conditional reconfiguration execution and the UE is capable of NR sidelink communication/discovery and SIB12 is provided by the target PCell, and the UE has initiated transmission of a SidelinkUEInformationNR message since it was configured to do so in accordance with 5.8.3.2:

4>	initiate transmission of the SidelinkUEInformationNR message in accordance with 5.8.3.3;

3>	if any application layer measurement report container has been received from upper layers for which the successful transmission of the MeasurementReportAppLayer message or at least one segment of the message via SRB4 (if reconfigurationWithSync was included in masterCellGroup) or SRB5 (if reconfigurationWithSync was included in secondaryCellGroup) has not been confirmed by lower layers:

4>	if RRC segmentation was used for the MeasurementReportAppLayer message:

5>	if RRC segmentation is enabled based on the field rrc-SegAllowedSRB4 or rrc-SegAllowedSRB5 for the reportingSRB (or SRB4 if reportingSRB is not configured):

6>	re-submit all segments of the MeasurementReportAppLayer message to lower layers for transmission via the reportingSRB (or SRB4 if reportingSRB is not configured);

5>	else:

6>	discard all segments of the MeasurementReportAppLayer message;

4>	else:

5>	re-submit the MeasurementReportAppLayer message to lower layers for transmission via the reportingSRB (or SRB4 if reportingSRB is not configured);

2>	if reconfigurationWithSync was included in masterCellGroup and SRB4 is configured in the target cell:

3>	for each application layer measurement configuration in the UE:

4>	if the RRCReconfiguration message is applied due to a conditional reconfiguration execution, if transmissionOfSessionStartStop is set to true for the application layer measurement configuration and if the session status has changed since the UE was configured with the conditional reconfiguration:

5>	initiate transmission of a MeasurementReportAppLayer message including appLayerSessionStatus, via SRB4 for the application layer measurement in accordance with 5.7.16.2;

2>	if reconfigurationWithSync was included in masterCellGroup and the target cell provides SIB21 or provides SIB1 including nonServingCellMII:

3>	if the UE initiated transmission of an MBSInterestIndication message during the last 1 second preceding reception of this RRCReconfiguration message; or

3>	if the RRCReconfiguration message is applied due to a conditional reconfiguration execution, and the UE has initiated transmission of an MBSInterestIndication message after having received this RRCReconfiguration message:

4>	initiate transmission of an MBSInterestIndication message in accordance with clause 5.9.4;

2>	the procedure ends.

NOTE 3:	The UE is only required to acquire broadcasted SIB1 if the UE can acquire it without disrupting unicast or MBS multicast data reception, i.e. the broadcast and unicast/MBS multicast beams are quasi co-located.

NOTE 4: The UE sets the content of UEAssistanceInformation according to latest configuration (i.e. the configuration after applying the RRCReconfiguration message) and latest UE preference. The UE may include more than the concerned UE assistance information within the UEAssistanceInformation according to 5.7.4.2. Therefore, the content of UEAssistanceInformation message might not be the same as the content of the previous UEAssistanceInformation message.

NEXT CHANGE

#### 5.3.5.18	LTM configuration and execution

##### 5.3.5.18.6	LTM cell switch execution

Upon the indication by lower layers that an LTM cell switch procedure is triggered, or upon performing LTM cell switch following cell selection performed while timer T311 was running, as specified in 5.3.7.3, the UE shall:

1>	if the LTM cell switch is triggered on the MCG:

2>	release/clear all current dedicated and common radio configurations which have neither been received via SRB1 within mrdc-SecondaryCellGroup, nor via SRB3 except for the following:

-	the radio bearer configuration (configured via RadioBearerConfig)

-	the logicalChannelIdentity and logicalChannelIdentityExt of RLC bearers configured in RLC-BearerConfig and the associated RLC entities, their state variables, buffers, and timers, except for triggering the associated RLC entities to reset the variable RETX_COUNT its initial value, as specified in TS 38.322 [4];

-	the bh-LogicalChannelIdentity of BH RLC channels configured in BH-RLC-ChannelConfig and the associated RLC entities, their state variables, buffers, and timers, except for triggering the associated RLC entities to reset the variable RETX_COUNT its initial value, as specified in TS 38.322 [4];

-	the UE variables VarLTM-ServingCellNoResetID and VarLTM-ServingCellUE-MeasuredTA-ID;

-	the ltm-Config;

-	the MCG C-RNTI;

-	the AS security configurations associated with the master key;

-	the logged measurement configuration;

-	the successHO-Config;

1>	else, if the LTM cell switch is triggered on the SCG:

2>	release/clear all current dedicated and common radio configurations which have been received either via SRB1 within mrdc-SecondaryCellGroup, or via SRB3 except for the following:

-	the radio bearer configuration (configured via RadioBearerConfig IE)

-	the logicalChannelIdentity and logicalChannelIdentityExt of RLC bearers configured in RLC-BearerConfig and the associated RLC entities, their state variables, buffers, and timers, except for triggering the associated RLC entities to reset the variable RETX_COUNT its initial value, as specified in TS 38.322 [4];

-	the bh-LogicalChannelIdentity of BH RLC channels configured in BH-RLC-ChannelConfig and the associated RLC entities, their state variables, buffers, and timers, except for triggering the associated RLC entities to reset the variable RETX_COUNT its initial value, as specified in TS 38.322 [4];

-	the UE variables VarLTM-ServingCellNoResetID and VarLTM-ServingCellUE-MeasuredTA-ID;

-	the ltm-Config;

-	the AS security configurations associated with the secondary key;

1>	for each SRB/DRB in the current UE configuration:

2>	if the LTM cell switch is triggered on the MCG and the SRB/DRB using the master key; or

2>	if the LTM cell switch is triggered on the SCG and the SRB/DRB using the secondary key:

3>	keep the associated PDCP and SDAP entities, their state variables, buffers and timers;

3>	release all fields related to the SRB/DRB configuration except for srb-Identity and drb-Identity;

NOTE 00:	For all radio bearers and RLC bearers included in the LTM candidate configuration to be applied at an LTM cell switch execution (i.e., as derived from the LTM reference configuration and the LTM candidate configuration), even if those radio bearers and RLC bearers were configured before the LTM cell switch execution, the network includes fields as specified for the initial setup of radio bearers and RLC bearers and sets the values previously stored by the UE for the fields that cannot be modified according to presence conditions or field descriptions.

1>	apply the default L1 parameter values as specified in corresponding physical layer specifications except for the parameters for which values are provided in SIB1;

1>	use the default values specified in 9.2.3 for timers T310, T311 and constants N310, N311 associated with the cell group for which the LTM cell switch procedure is triggered, where T310, N310, and N311 are for both MCG and SCG, and T311 is only for the MCG;

1>	apply the default MAC Cell Group configuration as specified in 9.2.2 for the cell group for which the LTM cell switch procedure is triggered;

1>	for each srb-Identity in the current UE configuration:

2>	apply the default SRB configuration defined in 9.2.1 for the corresponding SRB;

1>	if the LTM-Candidate IE in ltm-Config indicated by lower layers or for the selected cell in accordance with 5.3.7.3 does not contain the field ltm-NoResetID and if the UE does not have any value stored of ltm-ServingCellNoResetID within VarLTM-ServingCellNoResetID; or

1>	if the value of field ltm-NoResetID contained within the LTM-Candidate IE in ltm-Config indicated by lower layers or for the selected cell in accordance with 5.3.7.3 is not equal to the value of ltm-ServingCellNoResetID within VarLTM-ServingCellNoResetID:

2>	for each logicalChannelIdentity and logicalChannelIdentityExt that is part of the current UE configuration for the cell group for which the LTM cell switch procedure is triggered:

3>	if servedRadioBearer is set to drb-Identity:

4>	after the end of this procedure, re-establish the corresponding RLC entity as specified in TS 38.322 [4], after applying the LTM configuration in ltm-CandidateConfig within the LTM-Candidate IE in ltm-Config;

2>	for each bh-LogicalChannelIdentity that is part of the current UE configuration for the cell group for which the LTM cell switch procedure is triggered:

3>	after the end of this procedure, re-establish the corresponding RLC entity as specified in TS 38.322 [4], after applying the LTM configuration in ltm-CandidateConfig within the LTM-Candidate IE in ltm-Config;

2>	for each drb-Identity value that is part of the current UE configuration:

3>	if this DRB is an AM DRB:

4>	after the end of this procedure, trigger the PDCP entity of this DRB to perform data recovery as specified in TS 38.323 [5], after applying the LTM configuration in ltm-CandidateConfig within LTM-Candidate IE in ltm-Config;

2>	if the value of field ltm-NoResetID contained within the LTM-Candidate IE in ltm-Config indicated by lower layers or for the selected cell in accordance with 5.3.7.3 is not equal to the value of ltm-ServingCellNoResetID within VarLTM-ServingCellNoResetID:

3>	replace the value of ltm-ServingCellNoResetID in VarLTM-ServingCellNoResetID with the value of ltm-NoResetID in the LTM-Candidate in ltm-Config indicated by lower layers or for the selected cell in accordance with 5.3.7.3;

1> if the LTM-Candidate IE in ltm-Config indicated by lower layers or for the selected cell in accordance with 5.3.7.3 contains the field ltm-UE-MeasuredTA-ID:

2>	if the value of ltm-UE-MeasuredTA-ID is not equal to the value of ltm-ServingCellUE-MeasuredTA-ID within VarLTM-ServingCellUE-MeasuredTA-ID:

3>	replace the value of ltm-ServingCellUE-MeasuredTA-ID in VarLTM-ServingCellUE-MeasuredTA-ID with the value received within ltm-UE-MeasuredTA-ID;

3>	for each LTM-Candidate IE in ltm-Config:

4>	if the value of ltm-UE-MeasuredTA-ID within LTM-Candidate IE is equal to the value of ltm-ServingCellUE-MeasuredTA-ID within VarLTM-ServingCellUE-MeasuredTA-ID:

5>	inform lower layers that the UE is configured with UE-based TA measurements for the LTM-Candidate;

4>	else:

5>	inform lower layers that the UE is not configured with UE-based TA measurements for the LTM-Candidate;

NOTE 0:	The UE is not expected to perform UE-based TA measurements for an SpCell.

1>	if ltm-ConfigComplete is not included within the LTM-Candidate IE in ltm-Config indicated by lower layers or for the selected cell in accordance with 5.3.7.3:

2>	consider ltm-ReferenceConfiguration in ltm-Config, associated with the cell group for which the LTM cell switch procedure is triggered, to be the current UE configuration for the fields and configurations to be released by the actions above in this procedure;

2>	if measConfig is included within ltm-ReferenceConfiguration in ltm-Config;

3>	perform the measurement configuration procedure as specified in clause 5.5.2 by considering the measConfig within ltm-ReferenceConfiguration in ltm-Config as the received measConfig:

NOTE 1:	When the UE considers the reference configuration to be the current UE configuration, the UE should store fields and configurations that are part of the reference configuration but should not execute any actions or procedures triggered by the reception of an RRCReconfiguration message which are described in clause 5.3.5.3, unless specified otherwise in this clause.

1>	if the LTM cell switch is triggered by an indication from lower layers:

2>	apply the RRCReconfiguration message in ltm-CandidateConfig within LTM-Candidate IE in ltm-Config identified by the LTM candidate configuration identity received from lower layers according to clause 5.3.5.3;

1>	else (LTM cell switch triggered upon cell selection performed while timer T311 was running):

2>	apply the RRCReconfiguration message in ltm-CandidateConfig within LTM-Candidate IE in ltm-Config related to the LTM candidate configuration identity for the selected cell (i.e., in accordance with 5.3.7.3) according to clause 5.3.5.3;

1>	release the radio bearer(s) and the logical channel(s) that were part of the UE configuration before of this LTM cell switch procedure but not part of the LTM candidate configuration either indicated by lower layers or for the selected cell in accordance with 5.3.7.3, or the LTM reference configuration (in case the LTM candidate configuration does not include ltm-ConfigComplete).

NOTE 2:	When ltm-ConfigComplete is not included for an LTM candidate configuration, before an LTM cell switch is triggered a UE implementation may generate and store an RRCReconfiguration message by applying the received LTM candidate configuration on top of the LTM reference configuration, and the stored RRCReconfiguration message is applied when the LTM cell switch is triggered. It is up to the UE to ensure that the RRC reconfiguration applied at the time of LTM cell switch is in accordance with the latest LTM reference configuration and LTM candidate configuration.

NEXT CHANGE

#### 5.3.7.3	Actions following cell selection while T311 is running

Upon selecting a suitable NR cell, the UE shall:

1>	ensure having valid and up to date essential system information as specified in clause 5.2.2.2;

1>	stop timer T311;

1>	if T390 is running:

2>	stop timer T390 for all access categories;

2>	perform the actions as specified in 5.3.14.4;

1>	stop the relay (re)selection procedure, if ongoing;

1>	if the cell selection is triggered by detecting radio link failure of the MCG or re-configuration with sync failure of the MCG, except for an LTM cell switch procedure following cell selection performed while timer T311 was running, as specified in 5.3.7.3, or mobility from NR failure, and

1>	if attemptCondReconfig is configured; and

1>	if the selected cell is not configured with CondEventT1, or the selected cell is configured with CondEventT1 and leaving condition has not been fulfilled; and

1>	if the selected cell is one of the candidate cells for which the reconfigurationWithSync is included in the masterCellGroup in the MCG VarConditionalReconfig and the condExecutionCondPSCell is not configured for the corresponding condReconfigId in the MCG VarConditionalReconfig:

2>	if the UE supports RLF-Report for conditional handover, set the choCellId in the VarRLF-Report to the global cell identity, if available, otherwise to the physical cell identity and carrier frequency of the selected cell;

2>	apply the stored condRRCReconfig associated to the selected cell and perform actions as specified in 5.3.5.3;

NOTE 1:	It is left to network implementation to how to avoid keystream reuse in case of CHO based recovery after a failed handover without key change.

1>	if the cell selection is triggered by detecting radio link failure of the MCG or re-configuration with sync failure of the MCG for an LTM cell switch procedure triggered upon the indication by lower layers as specified in clause 5.3.5.18.6; and

1>	if attemptLTM-Switch is configured; and

1>	if the selected cell is one of the LTM candidate cells in the LTM-Candidate IE within ltm-Config associated with the MCG:

2>	if the UE supports RLF-Report for MCG LTM cell switch, set the ltm-RecoveryCellId in the VarRLF-Report to the global cell identity, if available, otherwise to the physical cell identity and carrier frequency of the selected cell;

2>	perform the LTM cell switch procedure for the selected LTM candidate cell according to the actions specified in 5.3.5.18.6;

NOTE 2:	In case both attemptCondReconfig and attemptLTM-Switch are configured, it is left to the UE implementation which procedure to execute.

1>	else:

2>	if UE is configured with attemptCondReconfig; or

2>	if UE is configured with attemptLTM-Switch:

3>	reset MAC;

3>	release spCellConfig, if configured;

3>	release the MCG SCell(s), if configured;

3>	release delayBudgetReportingConfig, if configured and stop timer T342, if running;

3>	release overheatingAssistanceConfig , if configured and stop timer T345, if running;

3>	if MR-DC is configured:

4>	perform MR-DC release, as specified in clause 5.3.5.10;

3>	release idc-AssistanceConfig, if configured;

3>	release btNameList, if configured;

3>	release wlanNameList, if configured;

3>	release sensorNameList, if configured;

3>	release drx-PreferenceConfig for the MCG, if configured and stop timer T346a associated with the MCG, if running;

3>	release maxBW-PreferenceConfig for the MCG, if configured and stop timer T346b associated with the MCG, if running;

3>	release maxCC-PreferenceConfig for the MCG, if configured and stop timer T346c associated with the MCG, if running;

3>	release maxMIMO-LayerPreferenceConfig for the MCG, if configured and stop timer T346d associated with the MCG, if running;

3>	release minSchedulingOffsetPreferenceConfig for the MCG, if configured and stop timer T346e associated with the MCG, if running;

3>	release rlm-RelaxationReportingConfig for the MCG, if configured and stop timer T346j associated with the MCG, if running;

3>	release bfd-RelaxationReportingConfig for the MCG, if configured and stop timer T346k associated with the MCG, if running;

3>	release releasePreferenceConfig, if configured and stop timer T346f, if running;

3>	release onDemandSIB-Request if configured, and stop timer T350, if running;

3>	release referenceTimePreferenceReporting, if configured;

3>	release sl-AssistanceConfigNR, if configured;

3>	release obtainCommonLocation, if configured;

3>	release scg-DeactivationPreferenceConfig, if configured, and stop timer T346i, if running;

3>	release musim-GapAssistanceConfig, if configured and stop timer T346h, if running;

3>	release musim-GapPriorityAssistanceConfig, if configured;

3>	release musim-LeaveAssistanceConfig, if configured;

3>	release musim-CapabilityRestrictionConfig, if configured and stop timer T346n, if running;

3>	release propDelayDiffReportConfig, if configured;

3>	release ul-GapFR2-PreferenceConfig, if configured;

3>	release rrm-MeasRelaxationReportingConfig, if configured;

3>	release maxBW-PreferenceConfigFR2-2, if configured;

3>	release maxMIMO-LayerPreferenceConfigFR2-2, if configured;

3>	release minSchedulingOffsetPreferenceConfigExt, if configured;

3>	release aerial-FlightPathAvailabilityConfig, if configured;

3>	release ul-TrafficInfoReportingConfig, if configured, and stop all instances of timer T346l, if running;

3>	suspend all RBs, and BH RLC channels for the IAB-MT, except SRB0 and broadcast MRBs;

2>	remove all the entries within the MCG VarConditionalReconfig, if any;

2>	perform the LTM configuration release procedure for the MCG and the SCG as specified in clause 5.3.5.18.7;

2>	for each measId, if the associated reportConfig has a reportType set to condTriggerConfig:

3>	for the associated reportConfigId:

4>	remove the entry with the matching reportConfigId from the reportConfigList within the VarMeasConfig;

3>	if the associated measObjectId is only associated to a reportConfig with reportType set to condTriggerConfig:

4>	remove the entry with the matching measObjectId from the measObjectList within the VarMeasConfig;

3>	remove the entry with the matching measId from the measIdList within the VarMeasConfig;

2>	remove the servingSecurityCellSetId within the VarServingSecurityCellSetID, if any;

2>	release the PC5 RLC entity for SL-RLC0, if any;

2>	start timer T301;

2>	apply the default L1 parameter values as specified in corresponding physical layer specifications except for the parameters for which values are provided in SIB1;

2>	apply the default MAC Cell Group configuration as specified in 9.2.2;

2>	apply the CCCH configuration as specified in 9.1.1.2;

2>	apply the timeAlignmentTimerCommon included in SIB1;

2>	initiate transmission of the RRCReestablishmentRequest message in accordance with 5.3.7.4;

NOTE 2a:	This procedure applies also if the UE returns to the source PCell.

NOTE 3:	A L2 U2N Relay UE may re-establish (e.g. via release and establish) the SL-RLC0 and SL-RLC1 of the connected L2 U2N Remote UE(s).

Upon selecting an inter-RAT cell, the UE shall:

1>	perform the actions upon going to RRC_IDLE as specified in 5.3.11, with release cause 'RRC connection failure'.

NEXT CHANGE

#### 5.3.10.5	RLF report content determination

The UE shall determine the content in the VarRLF-Report as follows:

1>	clear the information included in VarRLF-Report, if any;

1>	if the UE is not in SNPN access mode, set the plmn-IdentityList to include the list of EPLMNs stored by the UE (i.e. including the RPLMN);

1>	else if the UE is in SNPN access mode, set the snpn-IdentityList to include the list of equivalent SNPNs stored by the UE (i.e., including the registered SNPN identity);

1>	set the measResultLastServCell to include the cell level RSRP, RSRQ and the available SINR, of the source PCell (in case HO failure) or PCell (in case RLF) based on the available SSB and CSI-RS measurements collected up to the moment the UE detected failure;

1>	if the UE was configured with condExecutionCond and condExecutionCondPScell, set the measResultLastServPSCell to include the cell level RSRP, RSRQ and the available SINR, of the source PSCell (in case of PSCell change) or PSCell (in case of no PSCell change) based on the available SSB and CSI-RS measurements collected up to the moment the UE detected the failure;

1>	if T316 is not configured:

2>	set pSCellId to the of the source PSCell (in case of PSCell change) or PSCell (in case of no PSCell change);

1>	if measRSSI-ReportConfig is configured for the measObject indicated as the servingCellMO of the source PCell (in case HO failure) or PCell (in case of RLF), set the measResultLastServCellRSSI to the linear average of the available RSSI sample value(s) provided by lower layers for the frequency of the source PCell (in case HO failure) or PCell (in case of RLF) up to the moment the UE detected the failure;

1>	if the SS/PBCH block-based measurement quantities are available:

2>	set the rsIndexResults in measResultLastServCell to include all the available measurement quantities of the source PCell (in case HO failure) or PCell (in case RLF), ordered such that the highest SS/PBCH block RSRP is listed first if SS/PBCH block RSRP measurement results are available, otherwise the highest SS/PBCH block RSRQ is listed first if SS/PBCH block RSRQ measurement results are available, otherwise the highest SS/PBCH block SINR is listed first, based on the available SS/PBCH block based measurements collected up to the moment the UE detected failure;

2>	set the rsIndexResults in measResultLastServPSCell to include all the available measurement quantities of  the source PSCell (in case of PSCell change) or PSCell (in case of no PSCell change) if the UE was configured with condExecutionCond and condExecutionCondPScell, ordered such that the highest SS/PBCH block RSRP is listed first if SS/PBCH block RSRP measurement results are available, otherwise the highest SS/PBCH block RSRQ is listed first if SS/PBCH block RSRQ measurement results are available, otherwise the highest SS/PBCH block SINR is listed first, based on the available SS/PBCH block based measurements collected up to the moment the UE detected failure;

1>	if the CSI-RS based measurement quantities are available:

2>	set the rsIndexResults in measResultLastServCell to include all the available measurement quantities of the source PCell (in case HO failure) or PCell (in case RLF), ordered such that the highest CSI-RS RSRP is listed first if CSI-RS RSRP measurement results are available, otherwise the highest CSI-RS RSRQ is listed first if CSI-RS RSRQ measurement results are available, otherwise the highest CSI-RS SINR is listed first, based on the available CSI-RS based measurements collected up to the moment the UE detected failure;

2>	if the UE was configured with condExecutionCond and condExecutionCondPScell, set the rsIndexResults in measResultLastServPSCell to include all the available measurement quantities of the source PSCell (in case of PSCell change) or PSCell (in case of no PSCell change), ordered such that the highest CSI-RS RSRP is listed first if CSI-RS RSRP measurement results are available, otherwise the highest CSI-RS RSRQ is listed first if CSI-RS RSRQ measurement results are available, otherwise the highest CSI-RS SINR is listed first, based on the available CSI-RS based measurements collected up to the moment the UE detected failure;

1>	if the UE supports RLF-Report for MCG LTM cell switch and if the UE was configured with ltm-Config associated with the MCG when connected to the source PCell (in case HO failure) or PCell (in case RLF )and if the SS/PBCH block-based L1 measurement quantities are available:

2>	set the resultsSSB-Indexes in measResultL1-LastServCell to include all the available measurement quantities of the source PCell (in case HO failure) or PCell (in case RLF), ordered such that the highest SS/PBCH block L1-RSRP is listed first, based on the available SS/PBCH block-based L1 measurements collected up to the moment the UE detected failure;

1>	for each of the configured measObjectNR in which measurements are available or in which the associated reportConfigNR is configured as conditional handover with time-based or location-based trigger condition:

2>	if the SS/PBCH block-based measurement quantities are available:

3>	set the measResultListNR in measResultNeighCells to include all the available measurement quantities of the best measured cells, other than the source PCell (in case HO failure) or PCell (in case RLF), and other than the source PSCell (in case of PSCell change) or PSCell (in case of no PSCell change) if the UE was configured with condExecutionCond and condExecutionCondPScell ordered such that the cell with highest SS/PBCH block RSRP is listed first if SS/PBCH block RSRP measurement results are available, otherwise the cell with highest SS/PBCH block RSRQ is listed first if SS/PBCH block RSRQ measurement results are available, otherwise the cell with highest SS/PBCH block SINR is listed first, based on the available SS/PBCH block based measurements collected up to the moment the UE detected failure;

3>	for each neighbour cell included, include the optional fields that are available;

NOTE 0a:	For the neighboring cells included in measResultListNR in measResultNeighCells ordered based on the SS/PBCH block measurement quantities, UE also includes the CSI-RS based measurement quantities, if available.

2>	if the CSI-RS based measurement quantities are available:

3>	set the measResultListNR in measResultNeighCells to include all the available measurement quantities of the best measured cells, other than the source PCell (in case HO failure) or PCell (in case RLF), and other than the source PSCell (in case of PSCell change) or PSCell (in case of no PSCell change) if the UE was configured with condExecutionCond and condExecutionCondPScell ordered such that the cell with highest CSI-RS RSRP is listed first if CSI-RS RSRP measurement results are available, otherwise the cell with highest CSI-RS RSRQ is listed first if CSI-RS RSRQ measurement results are available, otherwise the cell with highest CSI-RS SINR is listed first, based on the available CSI-RS based measurements collected up to the moment the UE detected radio link failure;

3>	for each neighbour cell included, include the optional fields that are available;

NOTE 0b:	For ordering the neighboring cells based on the CSI-RS measurement quantities, UE includes measurements only for the cells not yet included in measResultListNR in measResultNeighCells to avoid overriding SS/PBCH block-based ordered measurements.

2>	if measurement quantities are not available:

3>	set physCellId in measResultListNR in measResultNeighCells to include the physical cell identity of the neighbour cells that are candidate cells for which the reconfigurationWithSync is included in the masterCellGroup in the MCG VarConditionalReconfig at the moment of the detected failure;

3>	for each neighbour cell included, include the optional fields that are available;

2>	for each neighbour cell, if any, included in measResultListNR in measResultNeighCells:

3>	if the UE supports RLF-Report for conditional handover with measurement-based trigger condition and if the neighbour cell is one of the candidate cells for which the reconfigurationWithSync is included in the masterCellGroup in the MCG VarConditionalReconfig at the moment of the detected failure and if the related MCG VarConditionalReconfig only concerns measurement-based trigger condition; or

3>	if the UE supports RLF-Report for conditional handover with time-based and location-based trigger condition and if the neighbour cell is one of the candidate cells for which the reconfigurationWithSync is included in the masterCellGroup in the MCG VarConditionalReconfig at the moment of the detected failure; or

3>	if the UE supports RLF-Report for conditional handover with candidate SCG and if the neighbour cell is one of the candidate cells for which the reconfigurationWithSync is included in the masterCellGroup in the MCG VarConditionalReconfig at the moment of the detected failure:

4>	set choConfig in MeasResult2NR to the execution condition for each measId within condTriggerConfig associated to the neighbour cell within the MCG VarConditionalReconfig;

4>	if the first entry of choConfig corresponds to a fulfilled execution condition at the moment of handover failure, or radio link failure; or

4>	if the second entry of choConfig, if available, corresponds to a fulfilled execution condition at the moment of handover failure, or radio link failure:

5>	set firstTriggeredEvent to the execution condition condFirstEvent corresponding to the first entry of choConfig or to the execution condition condSecondEvent corresponding to the second entry of choConfig, whichever execution condition was fulfilled first in time;

5>	set timeBetweenEvents to the elapsed time between the point in time of fulfilling the condition in choConfig that was fulfilled first in time, and the point in time of fulfilling the condition in choConfig that was fulfilled second in time, if both the first execution condition corresponding to the first entry and the second execution condition corresponding to the second entry in the choConfig were fulfilled;

4>	if the UE supports RLF-Report for conditional handover with time-based or location-based trigger condition and if one entry of choConfig concerns to condEventD2;

5>	set distanceFromReference1 to the measured distance between UE and the serving cell moving reference location, and set distanceFromReference2 to the measured distance between UE and the moving reference location of the neighbour cell, at the moment of handover failure, or radio link failure;

1>	for each entry of condReconfigList in the MCG VarConditionalReconfig including both condExecutionCond and condExecutionCondPSCell, include an entry in choWithCandidateSCGInfoList and set the values as follows:

2>	if all triggering events of both condExecutionCond and condExecutionCondPSCell of the concerned entry of condReconfigList are fulfilled:

3>	set firstFulfilledConfig to cho or cpc, whichever was fulfilled first;

3>	set timeBetweenFulfillment to the elapsed time between the fulfillments of the last triggering events of the two execution conditions;

2>	else if all triggering events of only one of the condExecutionCond or condExecutionCondPSCell of the concerned entry of condReconfigList are fulfilled:

3>	set firstFulfilledConfig to cho or cpc, whichever was fulfilled;

3>	set timeBetweenLastFulfillmentAndEvent to the elapsed time between the point in time of fulfilling the last triggering event of the fulfilled execution condition and the RLF;

2>	set the pCellId to the global cell identity and tracking area code, if available, and otherwise the physical cell identity and carrier frequency, of the target candidate PCell stored in the condRRCReconfig of the concerned entry of condReconfigList;

2>	set the psCellId to the global cell identity and tracking area code, if available, and otherwise the physical cell identity and carrier frequency, of the target candidate PSCell stored in the condRRCReconfig of the concerned entry of condReconfigList;

1>	if the UE supports RLF-Report for MCG LTM cell switch, for each neighbour MCG LTM candidate cell:

2>	if SS/PBCH block-based L1-RSRP measurement quantities are available:

3>	set the measResultL1-NeighCells to include all the available SS/PBCH block-based L1-RSRP measurement results, ordered such that the cell with highest SS/PBCH block-based L1-RSRP (of all SS/PBCH block-based L1-RSRP measurement results for the cell) is listed first;

1>	for each of the configured measObjectNR associated with neighboring cells if the associated reportConfigNR includes measRSSI-ReportConfig:

2>	set the measResultNeighFreqRSSI in the measResultNeighFreqListRSSI to the linear average of the available RSSI sample value(s) provided by lower layers for the frequencies other than the frequency of the source PCell (in case HO failure) or of the PCell (in case RLF), up to the moment the UE detected failure:

3>	for each neighbour frequency included, include the optional fields that are available;

1>	for each of the configured EUTRA frequencies in which measurements are available;

2>	set the measResultListEUTRA in measResultNeighCells to include the best measured cells ordered such that the cell with highest RSRP is listed first if RSRP measurement results are available, otherwise the cell with highest RSRQ is listed first, and based on measurements collected up to the moment the UE detected failure;

3>	for each neighbour cell included, include the optional fields that are available;

NOTE 1:	The measured quantities are filtered by the L3 filter as configured in the mobility measurement configuration. The measurements are based on the time domain measurement resource restriction, if configured. Exclude-listed cells are not required to be reported.

1>	set the c-RNTI to the C-RNTI used in the source PCell (in case HO failure) or PCell (in case RLF);

1>	if the failure is detected due to reconfiguration with sync failure as described in 5.3.5.8.3, set the fields in VarRLF-report as follows:

2>	set the connectionFailureType to hof;

2>	if the UE supports RLF-Report for DAPS handover and if any DAPS bearer was configured while T304 was running:

3>	set lastHO-Type to daps;

3>	if radio link failure was detected in the source PCell, according to clause 5.3.10.3:

4>	set timeConnSourceDAPS-Failure to the time between the initiation of the DAPS handover execution and the radio link failure detected in the source PCell while T304 was running;

4>	set the rlf-Cause to the trigger for detecting the source radio link failure in accordance with clause 5.3.10.4;

2>	if the UE supports RLF-Report for conditional handover and if configuration of the conditional handover is available in the MCG VarConditionalReconfig at the moment of the handover failure:

3>	if the UE executed a conditional handover toward target PCell according to the condRRCReconfig of the target PCell:

4>	set timeSinceCHO-Reconfig to the time elapsed between the execution of the last RRCReconfiguration message including reconfigurationWithSync for the target PCell of the failed conditional handover, and the reception in the source PCell of the last conditionalReconfiguration including the condRRCReconfig of the target PCell of the failed conditional handover;

3>	else:

4>	set timeSinceCHO-Reconfig to the time elapsed between the execution of the last RRCReconfiguration message including reconfigurationWithSync for the target PCell of the failed handover, and the reception in the source PCell of the last conditionalReconfiguration including the condRRCReconfig;

3>	set choCandidateCellList to include the global cell identity, if available, and otherwise to the physical cell identity and carrier frequency of each of the candidate target cells for conditional handover included in condRRCReconfig within the MCG VarConditionalReconfig at the time of the failed handover, excluding the candidate target cells included in measResultNeighCells;

2>	if the UE supports RLF-Report for conditional handover and if the last executed RRCReconfiguration message including reconfigurationWithSync was concerning a conditional handover:

3>	set lastHO-Type to cho;

2>	else if the UE supports RLF-Report for MCG LTM cell switch and the last executed RRCReconfiguration message including reconfigurationWithSync was concerning an LTM cell switch:

3>	set lastHO-Type to ltm;

2>	if the UE supports RLF-Report for conditional handover with candidate SCG and if the last executed RRCReconfiguration message including reconfigurationWithSync included both condExecutionCond and condExecutionCondPSCell:

3>	set lastHO-Type to choWithCandidateSCG;

2>	set the nrFailedPCellId in failedPCellId to the global cell identity and tracking area code, if available, and otherwise to the physical cell identity and carrier frequency of the target PCell of the failed handover or a failed LTM cell switch;

2>	include nrPreviousCell in previousPCellId and set it to the global cell identity and tracking area code of the PCell where the last RRCReconfiguration message including reconfigurationWithSync was received;

2>	set the timeConnFailure to the elapsed time since the execution of the last RRCReconfiguration message including the reconfigurationWithSync;

1>	else if the failure is detected due to Mobility from NR failure as described in 5.4.3.5, set the fields in VarRLF-report as follows:

2>	set the connectionFailureType to hof;

2>	if last MobilityFromNRCommand concerned a failed inter-RAT handover from NR to E-UTRA and if the UE supports Radio Link Failure Report for Inter-RAT MRO EUTRA (NR to EUTRA):

3>	set the eutraFailedPCellId in failedPCellId to the global cell identity and tracking area code, if available, and otherwise to the physical cell identity and carrier frequency of the target PCell of the failed handover;

2>	include nrPreviousCell in previousPCellId and set it to the global cell identity and tracking area code of the PCell where the last MobilityFromNRCommand message was received;

2>	set the timeConnFailure to the elapsed time since the initialization of the handover associated to the last MobilityFromNRCommand message;

2>	if the UE supports RLF report for inter-system handover for voice fallback and if voiceFallbackIndication is included in the last MobilityFromNRCommand:

3>	include the voiceFallbackHO;

1>	else if the failure is detected due to radio link failure as described in 5.3.10.3, set the fields in VarRLF-report as follows:

2>	set the connectionFailureType to rlf;

2>	set the rlf-Cause to the trigger for detecting radio link failure in accordance with clause 5.3.10.4;

2>	set the nrFailedPCellId in failedPCellId to the global cell identity and the tracking area code, if available, and otherwise to the physical cell identity and carrier frequency of the PCell where radio link failure is detected;

2>	if an RRCReconfiguration message including the reconfigurationWithSync was received before the connection failure:

3>	if the last successfully executed RRCReconfiguration message including the reconfigurationWithSync concerned an intra NR handover or an LTM cell switch and the target cell of the intra NR handover or LTM cell switch was the PCell where radio link failure is detected; and

3>	if T316 was not running before entering the PCell in which the radio link failure was detected; and

3>	if T311 was not running before entering the PCell in which the radio link failure was detected:

4>	include the nrPreviousCell in previousPCellId and set it to the global cell identity and the tracking area code of the source PCell of the intra NR handover or LTM cell switch concerning the last successfully executed RRCReconfiguration message including reconfigurationWithSync ;

4>	if the UE supports RLF-Report for DAPS handover and if the last executed RRCReconfiguration message including reconfigurationWithSync was concerning a DAPS handover:

5>	set lastHO-Type to daps;

4>	else if the UE supports RLF-Report for conditional handover and if the last executed RRCReconfiguration message including reconfigurationWithSync was concerning a conditional handover:

5>	set lastHO-Type to cho;

4>	else if the UE supports RLF-Report for MCG LTM cell switch and the last executed RRCReconfiguration message including reconfigurationWithSync was concerning an LTM cell switch:

5>	set lastHO-Type to ltm;

4>	if the UE supports RLF-Report for conditional handover with candidate SCG and if the last executed RRCReconfiguration message including reconfigurationWithSync included both condExecutionCond and condExecutionCondPSCell:

5>	set lastHO-Type to choWithCandidateSCG;

4>	set the timeConnFailure to the elapsed time since the execution of the last RRCReconfiguration message including the reconfigurationWithSync;

3>	else if the last RRCReconfiguration message including the reconfigurationWithSync concerned a handover to NR from E-UTRA and if the UE supports Radio Link Failure Report for Inter-RAT MRO EUTRA:

4>	include the eutraPreviousCell in previousPCellId and set it to the global cell identity and the tracking area code of the E-UTRA PCell where the last RRCReconfiguration message including reconfigurationWithSync was received embedded in E-UTRA RRC message MobilityFromEUTRACommand message as specified in TS 36.331 [10] clause 5.4.3.3;

4>	set the timeConnFailure to the elapsed time since reception of the last RRCReconfiguration message including the reconfigurationWithSync embedded in E-UTRA RRC message MobilityFromEUTRACommand message as specified in TS 36.331 [10] clause 5.4.3.3;

2>	if configuration of the conditional handover is available in the MCG VarConditionalReconfig at the moment of declaring the radio link failure:

3>	set timeSinceCHO-Reconfig to the time elapsed between the detection of the radio link failure, and the reception, in the source PCell, of the last conditionalReconfiguration including the condRRCReconfig message;

3>	set choCandidateCellList to include the global cell identity if available, and otherwise to the physical cell identity and carrier frequency of each of all the candidate target cells for conditional handover included in condRRCReconfig within the MCG VarConditionalReconfig at the time of radio link failure, excluding the candidate target cells included in measResultNeighCells;

1>	if connectionFailureType is rlf and the rlf-Cause is set to randomAccessProblem or beamFailureRecoveryFailure; or

1>	if connectionFailureType is rlf and the rlf-Cause is set to lbtFailure and the radio link failure is detected during the random access procedure; or

1>	if connectionFailureType is hof and if the failed handover is an intra-RAT reconfiguration with synch and if a random-access procedure was trigged for the failed reconfiguration with synch; or

2>	set the ra-InformationCommon to include the random-access related information as described in clause 5.7.10.5;

1>	if connectionFailureType is rlf and the rlf-Cause is set to lbtFailure, and the radio link failure is not detected during the random access procedure:

2>	set the locationAndBandwidth and subcarrierSpacing in bwp-Info associated to the UL BWP in which the consistent uplink LBT failure was detected;

1>	if the rlf-Cause is set to t310-Expiry or t312-Expiry:

2>	set the ssbRLMConfigBitmap and/or csi-rsRLMConfigBitmap in measResultLastServCell to include the radio link monitoring configuration of the last serving cell, if available;

1>	if available, set the locationInfo as in 5.3.3.7.

The UE may discard the radio link failure information or handover failure information, i.e. release the UE variable VarRLF-Report, 48 hours after the radio link failure/handover failure is detected.

NOTE 2:	In this clause, the term 'handover failure' has been used to refer to 'reconfiguration with sync failure'.

NEXT CHANGE

## 5.5a	Logged Measurements

### 5.5a.1	Logged Measurement Configuration

#### 5.5a.1.1	General

Figure 5.5a.1.1-1: Logged measurement configuration

The purpose of this procedure is to configure the UE to perform logging of measurement results while in RRC_IDLE and RRC_INACTIVE. The procedure applies to logged measurements capable UEs that are in RRC_CONNECTED.

NOTE:	NG-RAN may retrieve stored logged measurement information by means of the UE information procedure.

#### 5.5a.1.2	Initiation

NG-RAN initiates the logged measurement configuration procedure to UE in RRC_CONNECTED by sending the LoggedMeasurementConfiguration message.

#### 5.5a.1.3	Reception of the LoggedMeasurementConfiguration by the UE

Upon receiving the LoggedMeasurementConfiguration message the UE shall:

1>	discard the logged measurement configuration as well as the logged measurement information as specified in 5.5a.2;

1>	store the received loggingDuration, reportType and areaConfiguration, if included, in VarLogMeasConfig;

1>	If the UE is in SNPN access mode:

2>	if the LoggedMeasurementConfiguration message includes snpn-ConfigList:

3>	set the snpn-ConfigID-List in VarLogMeasReport to include the current registered SNPN identity as well as SNPN identities in snpn-Config-List;

2>	else:

3>	set the snpn-ConfigID-List in VarLogMeasReport to include the current registered SNPN identity;

1>	else if the LoggedMeasurementConfiguration message includes plmn-IdentityList:

2>	set plmn-IdentityList in VarLogMeasReport to include the RPLMN as well as the PLMNs included in plmn-IdentityList;

1>	else:

2>	set plmn-IdentityList in VarLogMeasReport to include the RPLMN;

1>	store the received absoluteTimeInfo, traceReference, traceRecordingSessionRef, and tce-Id in VarLogMeasReport;

1>	store the received bt-NameList, if included, in VarLogMeasConfig;

1>	store the received wlan-NameList, if included, in VarLogMeasConfig;

1>	store the received sensor-NameList, if included, in VarLogMeasConfig;

1>	start timer T330 with the timer value set to the loggingDuration;

1>	store the received sigLoggedMeasType, if included, in VarLogMeasReport;

1>	store the received earlyMeasIndication, if included, in VarLogMeasConfig;

1>	store the received intendedAreaScopeList, if included, in VarLogMeasConfig;

#### 5.5a.1.4	T330 expiry

Upon expiry of T330 the UE shall:

1>	release VarLogMeasConfig;

The UE is allowed to discard stored logged measurements, i.e. to release VarLogMeasReport, 48 hours after T330 expiry.

### 5.5a.2	Release of Logged Measurement Configuration

#### 5.5a.2.1	General

The purpose of this procedure is to release the logged measurement configuration as well as the logged measurement information.

#### 5.5a.2.2	Initiation

The UE shall initiate the procedure upon receiving a logged measurement configuration in same or another RAT. The UE shall also initiate the procedure upon power off or upon deregistration.

The UE shall:

1>	stop timer T330, if running;

1>	if stored, discard the logged measurement configuration as well as the logged measurement information, i.e. release the UE variables VarLogMeasConfig and VarLogMeasReport.

### 5.5a.3	Measurements logging

#### 5.5a.3.1	General

This procedure specifies the logging of available measurements by a UE in RRC_IDLE and RRC_INACTIVE that has a logged measurement configuration. The actual process of logging within the UE, takes place in RRC_IDLE state could continue in RRC_INACTIVE state or vice versa.

#### 5.5a.3.2	Initiation

While T330 is running and SDT procedure is not ongoing, the UE shall:

1>	if measurement logging is suspended:

2>	if during the last logging interval the IDC problems detected by the UE is resolved, resume measurement logging;

1>	if intendedAreaScopeList is included in VarLogMeasConfig:

2> if location information, if available, is outside of all areas indicated by intendedAreaScopeList:

3> skip the execution of the remainder of clause 5.5a.3.2 for the current logging interval (i.e. do not perform measurement logging for this interval);

1>	if not suspended, perform the logging in accordance with the following:

2>	if the reportType is set to periodical in the VarLogMeasConfig:

3>	if the UE is in any cell selection state (as specified in TS 38.304 [20]):

4>	perform the logging at regular time intervals, as defined by the loggingInterval in the VarLogMeasConfig;

3>	if the UE is in camped normally state on an NR cell and if the RPLMN is included in plmn-IdentityList stored in VarLogMeasReport; or

3>	if the UE is in camped normally state on an NR cell and if the registered SNPN identity is included in snpn-ConfigID-List stored in VarLogMeasReport:

4>	if areaConfiguration is not included in VarLogMeasConfig; or

4>	if the serving cell is part of the area indicated by areaConfig in areaConfiguration in VarLogMeasConfig; or

4>	if the serving cell is part of the area indicated by cag-ConfigList in areaConfiguration in VarLogMeasConfig; or

4>	if the serving cell is part of the area indicated by snpn-ConfigList in areaConfiguration in VarLogMeasConfig:

5>	perform the logging at regular time intervals, as defined by the loggingInterval in the VarLogMeasConfig;

2>	else if the reportType is set to eventTriggered, and eventType is set to outOfCoverage:

3>	perform the logging at regular time intervals as defined by the loggingInterval in VarLogMeasConfig only when the UE is in any cell selection state;

3>	upon transition from any cell selection state to camped normally state in NR:

4>	if the RPLMN is included in plmn-IdentityList stored in VarLogMeasReport, or if the registered SNPN identity is included in snpn-ConfigID-List stored in VarLogMeasReport; and

4>	if areaConfiguration is not included in VarLogMeasConfig or if the current camping cell is part of the area indicated by areaConfig of areaConfiguration in VarLogMeasConfig, or if the current camping cell is part of the area indicated by cag-ConfigList of areaConfiguration in VarLogMeasConfig, or if the current camping cell is part of the area indicated by snpn-ConfigList of areaConfiguration in VarLogMeasConfig:

5>	perform the logging;

2>	else if the reportType is set to eventTriggered and eventType is set to eventL1:

3>	if the UE is in camped normally state on an NR cell and if the RPLMN is included in plmn-IdentityList stored in VarLogMeasReport; or

3>	if the UE is in camped normally state on an NR cell and if the registered SNPN identity is included in snpn-ConfigID-List stored in VarLogMeasReport:

4>	if areaConfiguration is not included in VarLogMeasConfig; or

4>	if the serving cell is part of the area indicated by areaConfig in areaConfiguration in VarLogMeasConfig; or

4>	if the current serving cell is part of the area indicated by cag-ConfigList of areaConfiguration in VarLogMeasConfig, or if the current camping cell is part of the area indicated by snpn-ConfigList of areaConfiguration in VarLogMeasConfig;

5>	perform the logging at regular time intervals as defined by the loggingInterval in VarLogMeasConfig only when the conditions indicated by the eventL1 are met;

2>	when performing the logging:

3> if InterFreqTargetInfo is configured and if the UE detected IDC problems on at least one of the frequencies included in InterFreqTargetInfo or any inter-RAT frequency during the last logging interval, or

3>	if InterFreqTargetInfo is not configured and if the UE detected IDC problems during the last logging interval:

4>	if measResultServingCell in the VarLogMeasReport is not empty:

5>	include inDeviceCoexDetected;

5>	suspend measurement logging from the next logging interval;

4>	else:

5>	suspend measurement logging;

3>	set the relativeTimeStamp to indicate the elapsed time since the moment at which the logged measurement configuration was received;

3>	if location information became available during the last logging interval, set the content of the locationInfo as in 5.3.3.7:

3>	if the UE is in any cell selection state (as specified in TS 38.304 [20]):

4>	set anyCellSelectionDetected to indicate the detection of no suitable or no acceptable cell found;

4>	if the reportType is set to eventTriggered in the VarLogMeasConfig; and

4>	if the RPLMN at the time of entering the any cell selection state is included in plmn-IdentityList stored in VarLogMeasReport or if the registered SNPN identity at the time of entering the any cell selection state is included in snpn-ConfigID-List stored in VarLogMeasReport; and

4>	if areaConfiguration is not included in VarLogMeasConfig or if the last suitable cell that the UE was camping on is part of the area indicated by areaConfig of areaConfiguration in VarLogMeasConfig, or if last suitable cell that the UE was camping on is part of the area indicated by cag-ConfigList of areaConfiguration in VarLogMeasConfig, or if last suitable cell that the UE was camping on is part of the area indicated by snpn-ConfigList of areaConfiguration in VarLogMeasConfig:

5>	set the servCellIdentity to indicate global cell identity of the last suitable cell that the UE was camping on;

5>	set the measResultServingCell to include the quantities of the last suitable cell the UE was camping on;

4>	else if the reportType is set to periodical in the VarLogMeasConfig:

5>	set the servCellIdentity to indicate global cell identity of the last logged cell that the UE was camping on;

5>	set the measResultServingCell to include the quantities of the last logged cell the UE was camping on;

3>	else:

4>	set the servCellIdentity to indicate global cell identity of the cell the UE is camping on;

4>	set the measResultServingCell to include the quantities of the cell the UE is camping on;

3>	if available, set the measResultNeighCells, in order of decreasing ranking-criterion as used for cell re-selection, to include measurements of neighbouring cell that became available during the last logging interval and according to the following:

4>	include measurement results for at most 6 neighbouring cells on the NR serving frequency and for at most 3 cells per NR neighbouring frequency and for the NR neighbouring frequencies in accordance with the following:

5>	if interFreqTargetInfo is included in VarLogMeasConfig:

6>	if earlyMeasIndication is included in VarLogMeasConfig;

7>	include measurement results for NR neighbouring frequencies that are included in both interFreqTargetInfo and either in measIdleCarrierListNR (within the VarMeasIdleConfig) or SIB4;

6>	else:

7>	include measurement results for NR neighbouring frequencies that are included in both interFreqTargetInfo and SIB4;

5>	else:

6>	if earlyMeasIndication is included in VarLogMeasConfig;

7>	include measurement results for NR neighbouring frequencies that are included in either measIdleCarrierListNR (within the VarMeasIdleConfig) or SIB4;

6>	else:

7>	include measurement results for NR neighbouring frequencies that are included in SIB4;

4>	include measurement results for at most 3 neighbours per inter-RAT frequency in accordance with the following:

5>	if earlyMeasIndication is included in VarLogMeasConfig:

6>	include measurement results for inter-RAT neighbouring frequencies that are included in either measIdleCarrierListEUTRA (within the VarMeasIdleConfig) or SIB5;

5>	else:

6>	include measurement results for inter-RAT frequencies that are included in SIB5;

4>	for each neighbour cell included, include the optional fields that are available;

NOTE 1:	The UE includes the latest results of the available measurements as used for cell reselection evaluation in RRC_IDLE or RRC_INACTIVE, which are performed in accordance with the performance requirements as specified in TS 38.133 [14].

NOTE 2:	For logging the measurements on frequencies (indicated in measIdleCarrierListNR/ measIdleCarrierListEUTRA) in the logged measurement, the qualityThreshold in measIdleConfig should not be applied, and how the UE logs the measurements on the frequencies is left to the UE implementation.

2>	when the memory reserved for the logged measurement information becomes full, stop timer T330 and perform the same actions as performed upon expiry of T330, as specified in 5.5a.1.4.

Editor’s Note: FFS if the UE logs the MDT data or not when it cannot obtain its location when geographical area scope is configured.

NEXT CHANGE

### 5.7.9	Mobility history information

#### 5.7.9.1	General

This procedure specifies how the mobility history information is stored by the UE, covering RRC_IDLE, RRC_INACTIVE and RRC_CONNECTED and released by the UE upon deregistration as specified in TS 23.502 [43].

#### 5.7.9.2	Initiation

If the UE supports storage of mobility history information, the UE shall:

1>	If the UE supports PSCell mobility history information and upon addition of a PSCell:

2>	include an entry in visitedPSCellInfoList in variable VarMobilityHistoryReport possibly after performing the following, if necessary:

3>	if visitedPSCellInfoListReport is available in the visitedCellInfoList in variable VarMobilityHistoryReport:

4>	for the oldest PCell entry in visitedCellInfoList including visitedPSCellInfoListReport;

5>	remove the oldest entry in the visitedPSCellInfoListReport;

3>	else:

4>	remove the oldest entry in visitedPSCellInfoList in variable VarMobilityHistoryReport;

2>	for the included entry:

3>	set the field timeSpent of the entry according to following:

4>	if this is the first PSCell entry for the current PCell since entering the current PCell in RRC_CONNECTED:

5>	include the entry as the time spent with no PSCell since entering the current PCell in RRC_CONNECTED;

4>	else:

5>	include the time spent with no PSCell since last PSCell release since entering the current PCell in RRC_CONNECTED;

1>	If the UE supports PSCell mobility history information and upon change, or release of a PSCell while being connected to the current PCell, or upon release of a PSCell while entering 'camped normally' state or 'any cell selection' state or 'camped on any cell' state:

2>	include an entry in visitedPSCellInfoList of the variable VarMobilityHistoryReport possibly after performing the following, if necessary:

3>	if visitedPSCellInfoListReport is available in the visitedCellInfoList in variable VarMobilityHistoryReport:

4>	for the oldest PCell entry in visitedCellInfoList including visitedPSCellInfoListReport;

5>	remove the oldest entry in the visitedPSCellInfoListReport;

3>	else:

4>	remove the oldest entry in visitedPSCellInfoList in variable VarMobilityHistoryReport;

2>	for the included entry:

3>	if the global cell identity of the previous PSCell is available:

4>	include the global cell identity of that cell in the field visitedCellId of the entry;

3>	else:

4>	include the physical cell identity and carrier frequency of that cell in the field visitedCellId of the entry;

3>	if the PSCell is changed or released while being connected to the current PCell, or if the PSCell is released while entering 'camped normally' state in the same serving cell which was previously the PCell:

4>	set the field timeSpent of the entry as the time spent in the previous PSCell while being connected to the current PCell/serving cell;

4>	if the UE supports storing and reporting SCG activation information in mobility history information:

5> set the field scgActiveDuration of the entry to the accumulated time spent in the previous PSCell with SCG state set to activated during the stay in the PSCell while being connected to the current PCell/serving cell, if available;

3>	if the PSCell is released while entering 'any cell selection' state or 'camped on any cell' state from a suitable cell in RRC_CONNECTED state in NR or LTE, or if the PSCell is released while entering 'camped normally' state while previously in RRC_CONNECTED state in a PCell different from the current serving cell:

4>	set the field timeSpent of the entry as the time spent in the previous PSCell while being connected to the previous PCell;

4>	if the UE supports storing and reporting SCG activation information in mobility history information:

5> set the field scgActiveDuration of the entry to the accumulated time spent in the previous PSCell with SCG state set to activated during the stay in the PSCell while being connected to the previous PCell, if available ;

1>	Upon change of suitable cell, consisting of PCell in RRC_CONNECTED (for NR or E-UTRA cell) or serving cell in RRC_INACTIVE (for NR cell) or in RRC_IDLE (for NR or E-UTRA cell), to another NR or E-UTRA cell, or when entering 'any cell selection' state from 'camped normally' state in NR or LTE or when entering 'any cell selection' state or 'camped on any cell' state from a suitable cell in RRC_CONNECTED state in NR or LTE:

NOTE:	Change of suitable cell encompasses also simultaneous change of serving cell and RRC state change.

2>	include an entry in visitedCellInfoList of the variable VarMobilityHistoryReport possibly after removing the oldest entry, if necessary, according to following:

3>	if the global cell identity of the previous PCell/serving cell is available:

4>	include the global cell identity of that cell in the field visitedCellId of the entry;

3>	else:

4>	include the physical cell identity and carrier frequency of that cell in the field visitedCellId of the entry;

3>	set the field timeSpent of the entry as the time spent in the previous PCell/serving cell;

3>	if the UE supports PSCell mobility history information and if the UE continues to be connected to the same PSCell during the change of the PCell in RRC_CONNECTED; or

3>	if the UE supports PSCell mobility history information and if the UE changes PSCell at the same time as the change of the PCell in RRC_CONNECTED; or

3>	if the UE supports PSCell mobility history information and if the PSCell is released at the same time as the change of the PCell in RRC_CONNECTED:

4>	include an entry in visitedPSCellInfoList of the variable VarMobilityHistoryReport possibly after performing the following, if necessary:

5>	if visitedPSCellInfoListReport is available in the visitedCellInfoList in variable VarMobilityHistoryReport:

6>	for the oldest PCell entry in visitedCellInfoList including visitedPSCellInfoListReport;

7>	remove the oldest entry in the visitedPSCellInfoListReport;

5>	else:

6>	remove the oldest entry in visitedPSCellInfoList in variable VarMobilityHistoryReport;

4>	for the included entry:

5>	if the global cell identity of the PSCell (in case the UE continues to be connected to the same PSCell) or the previous PSCell (in case the UE changes PSCell, or in case PSCell is released) is available:

6>	include the global cell identity of that cell in the field visitedCellId of the entry;

5>	else:

6>	include the physical cell identity and carrier frequency of that cell in the field visitedCellId of the entry;

5>	set the field timeSpent of the entry as the time spent in the PSCell, while being connected to previous PCell;

5>	if the UE supports storing and reporting SCG activation information in mobility history information:

6> set the field scgActiveDuration of the entry to the accumulated time spent in the PSCell with SCG state set to activated while being connected to the previous PCell, if available;

3>	if the UE supports PSCell mobility history information and if the UE was not configured with a PSCell at the time of change of PCell in RRC_CONNECTED; or

3>	if the UE supports PSCell mobility history information and if the UE was not configured with a PSCell at the time of entering 'camped normally' state while previously in RRC_CONNECTED state in a PCell different from the current serving cell; or

3>	if the UE supports PSCell mobility history information and if the UE was not configured with a PSCell at the time of entering 'any cell selection' state or 'camped on any cell' state from a suitable cell in RRC_CONNECTED state in NR or LTE:

4>	include an entry in visitedPSCellInfoList in variable VarMobilityHistoryReport after performing the following, if necessary;

5>	if visitedPSCellInfoListReport is available in the visitedCellInfoList in variable VarMobilityHistoryReport:

6>	for the oldest PCell entry in visitedCellInfoList including visitedPSCellInfoListReport;

7>	remove the oldest entry in the visitedPSCellInfoListReport;

5>	else:

6>	remove the oldest entry in visitedPSCellInfoList in variable VarMobilityHistoryReport;

4>	for the included entry:

5>	set the field timeSpent of the entry as the time without PSCell according to the following:

6>	if the UE experienced a PSCell release since entering the previous PCell in RRC_CONNECTED:

7>	include the time spent with no PSCell since last PSCell release since entering the previous PCell in RRC_CONNECTED;

6>	else:

7>	include the time spent with no PSCell since entering the previous PCell in RRC_CONNECTED;

3>	if the UE supports PSCell mobility history information and if visitedPSCellInfoList exists in VarMobilityHistoryReport:

4>	include visitedPSCellInfoList in VarMobilityHistoryReport in the visitedPSCellInfoListReport within the entry of the visitedCellInfoList associated to the latest PCell entry;

4>	remove visitedPSCellInfoList from the variable VarMobilityHistoryReport;

1>	if the UE supports PSCell mobility history information and upon entering 'camped normally' state in NR (in RRC_IDLE or RRC_INACTIVE) or E-UTRA (in RRC_IDLE) while previously in RRC_CONNECTED state NR or LTE in the same serving cell which was previously the PCell while not connected to a PSCell:

2>	include an entry in visitedPSCellInfoList in variable VarMobilityHistoryReport after performing the following, if necessary;

3>	if visitedPSCellInfoListReport is available in the visitedCellInfoList in variable VarMobilityHistoryReport:

4>	for the oldest PCell entry in visitedCellInfoList including visitedPSCellInfoListReport;

5>	remove the oldest entry in the visitedPSCellInfoListReport;

3>	else:

4>	remove the oldest entry in visitedPSCellInfoList in variable VarMobilityHistoryReport;

2>	for the included entry:

3>	set the field timeSpent of the entry as the time without PSCell according to the following:

4>	if the UE experienced a PSCell release since entering the current serving cell in RRC_CONNECTED:

5>	include the time spent with no PSCell since last PSCell release after entering the current serving cell in RRC_CONNECTED;

4>	else:

5>	include the time spent with no PSCell since entering the current serving cell in RRC_CONNECTED;

1>	upon entering 'camped normally' state in NR (in RRC_IDLE or RRC_INACTIVE) or E-UTRA (in RRC_IDLE) while previously in 'any cell selection' state or 'camped on any cell' state in NR or LTE:

2>	include an entry in visitedCellInfoList in variable VarMobilityHistoryReport possibly after removing the oldest entry, if necessary, according to following:

3>	set the field timeSpent of the entry as the time spent in 'any cell selection' state and/or 'camped on any cell' state in NR or LTE.

#### 5.7.9.3	Release of Mobility History Information

If the UE supports storage of mobility history information, the UE shall:

1>	if stored, discard the mobility history information, i.e. release the UE variable VarMobilityHistoryReport upon deregistration from the network as specified in TS 23.502 [43].

NEXT CHANGE

#### 5.7.3.5	Actions related to transmission of SCGFailureInformation message

The UE shall set the contents of the SCGFailureInformation message as follows:

1>	if the UE initiates transmission of the SCGFailureInformation message due to T310 expiry:

2>	set the failureType as t310-Expiry;

1>	else if the UE initiates transmission of the SCGFailureInformation message due to T312 expiry:

2>	set the failureType as other and set the failureType-v1610 as t312-Expiry;

1>	else if the UE initiates transmission of the SCGFailureInformation message to provide reconfiguration with sync failure information for an SCG:

2>	set the failureType as synchReconfigFailureSCG;

1>	else if the UE initiates transmission of the SCGFailureInformation message to provide random access problem indication from SCG MAC:

2>	if the random access procedure was initiated for beam failure recovery:

3>	set the failureType as other and set the failureType-v1610 as beamFailureRecoveryFailure;

2>	else:

3>	set the failureType as randomAccessProblem;

1>	else if the UE initiates transmission of the SCGFailureInformation message to provide indication from SCG RLC that the maximum number of retransmissions has been reached:

2>	set the failureType as rlc-MaxNumRetx;

1>	else if the UE initiates transmission of the SCGFailureInformation message due to SRB3 IP check failure:

2>	set the failureType as srb3-IntegrityFailure;

1>	else if the UE initiates transmission of the SCGFailureInformation message due to Reconfiguration failure of NR RRC reconfiguration message:

2>	set the failureType as scg-reconfigFailure;

1>	else if the UE initiates transmission of the SCGFailureInformation message due to consistent uplink LBT failures:

2>	set the failureType as other and set the failureType-v1610 as scg-lbtFailure;

1>	else if connected as an IAB-node and the SCGFailureInformation is initiated due to the reception of a BH RLF indication on BAP entity from the SCG:

2>	set the failureType as other and set failureType-v1610 as bh-RLF;

1>	else if the UE initiates transmission of the SCGFailureInformation message due to beam failure of the PSCell while the SCG is deactivated:

2>	set the failureType as other and set failureType-v1610 as beamFailure;

1> include and set MeasResultSCG-Failure in accordance with 5.7.3.4;

1>	for each MeasObjectNR configured by a MeasConfig associated with the MCG, and for which measurement results are available:

2>	include an entry in measResultFreqList;

2>	if there is a measId configured with the MeasObjectNR and a reportConfig which has rsType set to ssb:

3>	set ssbFrequency in measResultFreqList to the value indicated by ssbFrequency as included in the MeasObjectNR;

2>	if there is a measId configured with the MeasObjectNR and a reportConfig which has rsType set to csi-rs:

3>	set refFreqCSI-RS in measResultFreqList to the value indicated by refFreqCSI-RS as included in the associated measurement object;

2>	if a serving cell is associated with the MeasObjectNR:

3>	set measResultServingCell in measResultFreqList to include the available quantities of the concerned cell and in accordance with the performance requirements in TS 38.133 [14];

2>	set the measResultNeighCellList in measResultFreqList to include the best measured cells, ordered such that the best cell is listed first, and based on measurements collected up to the moment the UE detected the failure, and set its fields as follows;

3>	ordering the cells with sorting as follows:

4>	based on SS/PBCH block if SS/PBCH block measurement results are available and otherwise based on CSI-RS;

4>	using RSRP if RSRP measurement results are available, otherwise using RSRQ if RSRQ measurement results are available, otherwise using SINR;

3>	if the UE supports SCG failure information for mobility robustness optimization for conditional PSCell change or addition, for each neighbour cell, if any, included in measResultListNR in measResultFreqList:

4>	if the neighbour cell is one of the candidate cells for which the reconfigurationWithSync is included in the secondaryCellGroup in the MCG VarConditionalReconfig (for CPA or inter-SN CPC in NR-DC) or SCG VarConditionalReconfig (for intra-SN CPC) at the moment of the detected SCG failure (radio link failure at PSCell or PSCell change or addition failure):

5>	if the first entry of condExecutionCond or condExecutionCondSCG associated to the neighbour cell corresponds to a fulfilled execution condition at the moment of SCG failure; or

5>	if the second entry of condExecutionCond or condExecutionCondSCG associated to the neighbour cell, if available, corresponds to a fulfilled execution condition at the moment of SCG failure:

6>	set firstTriggeredEvent to the execution condition condFirstEvent corresponding to the first entry of condExecutionCond or condExecutionCondSCG associated to the neighbour cell or to the execution condition condSecondEvent corresponding to the second entry of condExecutionCond or condExecutionCondSCG associated to the neighbour cell, whichever execution condition was fulfilled first in time;

6>	set timeBetweenEvents to the elapsed time between the point in time of fulfilling the condition in condExecutionCond or condExecutionCondSCG associated to the neighbour cell that was fulfilled first in time, and the point in time of fulfilling the condition in condExecutionCond or condExecutionCondSCG associated to the neighbour cell that was fulfilled second in time, if both the first execution condition corresponding to the first entry and the second execution condition corresponding to the second entry in the condExecutionCond or condExecutionCondSCG associated to the neighbour cell were fulfilled;

3>	if the UE supports SCG failure information for mobility robustness optimization for conditional handover with candidate SCG, for each neighbour cell, if any, included in measResultListNR in measResultFreqList:

4>	if the neighbour cell is one of the candidate cells for which the reconfigurationWithSync is included in the masterCellGroup in the MCG VarConditionalReconfig at the moment of the detected SCG failure (radio link failure at PSCell or PSCell change or addition failure):

5>	set choConfig in MeasResult2NR to the execution condition for each measId within condTriggerConfig associated to the neighbour cell within the MCG VarConditionalReconfig;

5> if the first entry of choConfig corresponds to a fulfilled execution condition at the moment of handover failure, or radio link failure; or

5> if the second entry of choConfig, if available, corresponds to a fulfilled execution condition at the moment of handover failure, or radio link failure:

6>	set firstTriggeredEvent to the execution condition condFirstEvent corresponding to the first entry of choConfig or to the execution condition condSecondEvent corresponding to the second entry of choConfig, whichever execution condition was fulfilled first in time;

6>	set timeBetweenEvents to the elapsed time between the point in time of fulfilling the condition in choConfig that was fulfilled first in time, and the point in time of fulfilling the condition in choConfig that was fulfilled second in time, if both the first execution condition corresponding to the first entry and the second execution condition corresponding to the second entry in the choConfig were fulfilled;

3>	for each neighbour cell included:

4>	include the optional fields that are available.

NOTE 1:	The measured quantities are filtered by the L3 filter as configured in the mobility measurement configuration. The measurements are based on the time domain measurement resource restriction, if configured. Exclude-listed cells are not required to be reported.

NOTE 2:	Field measResultSCG-Failure is used to report available results for NR frequencies the UE is configured to measure by SCG RRC signalling.

1>	for each entry of condReconfigList in the MCG VarConditionalReconfig including both condExecutionCond and condExecutionCondPSCell, include an entry in choWithCandidateSCGInfoList and set the values as follows:

2>	if all triggering events of both condExecutionCond and condExecutionCondPSCell of the concerned entry of condReconfigList are fulfilled:

3>	set firstFulfilledConfig to cho or cpc, whichever was fulfilled first;

3>	set timeBetweenFulfillment to the elapsed time between the fulfillments of the last triggering events of the two execution conditions;

2>	else if all triggering events of only one of the condExecutionCond or condExecutionCondPSCell of the concerned entry of condReconfigList are fulfilled:

3>	set firstFulfilledConfig to cho or cpc, whichever was fulfilled;

3>	set timeBetweenLastFulfillmentAndEvent to the elapsed time between the point in time of fulfilling the last triggering event of the fulfilled execution condition and the SCG failure;

2>	set the pCellId to the global cell identity and tracking area code, if available, and otherwise the physical cell identity and carrier frequency, of the target candidate PCell stored in the condRRCReconfig of the concerned entry of condReconfigList;

2>	set the psCellId to the global cell identity and tracking area code, if available, and otherwise the physical cell identity and carrier frequency, of the target candidate PSCell stored in the condRRCReconfig of the concerned entry of condReconfigList;

1>	if available, set the locationInfo as in 5.3.3.7 according to the otherConfig associated with the NR MCG.

1>	if the UE supports SCG failure for mobility robustness optimization:

2>	if the failureType is set to synchReconfigFailureSCG; or

2>	if the failureType is set to randomAccessProblem and the SCG failure was declared while T304 was running:

3>	set perRAInfoList to indicate the performed random access procedure related information as specified in 5.7.10.5.

3>	set the failedPSCellId to the physical cell identity and carrier frequency of the target PSCell of the failed PSCell change or failed PSCell addition;

3>	if the failure occurred during a subsequent CPC:

4>	set the previousPSCellId to the physical cell identity and carrier frequency of the source PSCell of the failed PSCell change;

3>	else:

4>	set the previousPSCellId to the physical cell identity and carrier frequency of the source PSCell associated to the last received RRCReconfiguration message including reconfigurationWithSync for the SCG, if available;

3>	set the timeSCGFailure to the elapsed time since the last execution of RRCReconfiguration message including the reconfigurationWithSync for the SCG until declaring the SCG failure;

2>	else:

3>	set the failedPSCellId to the physical cell identity and carrier frequency of the PSCell in which the SCG failure was declared;

3>	if the last RRCReconfiguration message including the reconfigurationWithSync for the SCG was received to enter the PSCell in which the SCG failure was declared:

4>	set the timeSCGFailure to the elapsed time since the last execution of RRCReconfiguration message including the reconfigurationWithSync for the SCG until declaring the SCG failure;

4>	set the previousPSCellId to the physical cell identity and carrier frequency of the source PSCell associated to the last received RRCReconfiguration message including reconfigurationWithSync for the SCG;

1>	release successPSCell-Config configured by the source PSCell, if available.

The UE shall submit the SCGFailureInformation message to lower layers for transmission.

NEXT CHANGE

#### 5.7.10.3	Reception of the UEInformationRequest message

Upon receiving the UEInformationRequest message, the UE shall, only after successful security activation:

1>	if the idleModeMeasurementReq is included in the UEInformationRequest and the UE has stored VarMeasIdleReport that contains measurement information concerning cells other than the PCell:

2>	if validatedMeasurementsReq is included in the UEInformationRequest and measIdleValidityDuration is included in VarEnhMeasIdleConfig;

3>	set the measResultIdleEUTRA in the UEInformationResponse message to the value of measReportIdleEUTRA in the VarMeasIdleReport for any valid measurement results, if available, and set validityStatus to the value of measIdleValidityDuration in VarEnhMeasIdleConfig for each reported measurement;

3>	set the measResultIdleNR in the UEInformationResponse message to the value of measReportIdleNR in the VarMeasIdleReport for any valid measurement results, if available, and set validityStatus to the value of measIdleValidityDuration in VarEnhMeasIdleConfig for each reported measurement;

3>	discard the VarMeasIdleReport upon successful delivery of the UEInformationResponse message confirmed by lower layers;

2>	else:

3>	set the measResultIdleEUTRA in the UEInformationResponse message to the value of measReportIdleEUTRA in the VarMeasIdleReport, if available;

3>	set the measResultIdleNR in the UEInformationResponse message to the value of measReportIdleNR in the VarMeasIdleReport, if available;

3>	discard the VarMeasIdleReport upon successful delivery of the UEInformationResponse message confirmed by lower layers;

1>	if the reselectionMeasurementReq is included in the UEInformationRequest:

2>	if validatedMeasurementsReq is included in the UEInformationRequest and measReselectionValidityDuration is included in VarMeasReselectionConfig;

3>	if measReselectionCarrierListNR is present in VarMeasReselectionConfig:

4>	set the measResultReselectionNR in the UEInformationResponse message the valid NR measurement results, if available for any frequency listed in measReselectionCarrierListNR in VarMeasReselectionConfig and set validityStatus to the value of measIdleValidityDuration in VarMeasReselectionConfig for each reported measurement;

3> else:

4>	set the measResultReselectionNR in the UEInformationResponse message to any valid NR measurement results, if available, and set validityStatus to the value of measIdleValidityDuration in VarMeasReselectionConfig;

2>	else:

3>	if measReselectionCarrierListNR is present in VarMeasReselectionConfig:

4>	set the measResultReselectionNR in the UEInformationResponse message the NR measurement results, if available for any frequency listed in measReselectionCarrierListNR in VarMeasReselectionConfig;

3>	else:

4>	set the measResultReselectionNR in the UEInformationResponse message to any NR measurement results, if available;

1>	if the logMeasReportReq is present and if the RPLMN is included in plmn-IdentityList stored in VarLogMeasReport, or if the current registered SNPN identity is included in snpn-ConfigID-List stored in VarLogMeasReport:

2>	if VarLogMeasReport includes one or more logged measurement entries, set the contents of the logMeasReport in the UEInformationResponse message as follows:

3>	include the absoluteTimeStamp and set it to the value of absoluteTimeInfo in the VarLogMeasReport;

3>	include the traceReference and set it to the value of traceReference in the VarLogMeasReport;

3>	include the traceRecordingSessionRef and set it to the value of traceRecordingSessionRef in the VarLogMeasReport;

3>	include the tce-Id and set it to the value of tce-Id in the VarLogMeasReport;

3>	include the logMeasInfoList and set it to include one or more entries from the VarLogMeasReport starting from the entries logged first, and for each entry of the logMeasInfoList that is included, include all information stored in the corresponding logMeasInfoList entry in VarLogMeasReport;

3>	if the VarLogMeasReport includes one or more additional logged measurement entries that are not included in the logMeasInfoList within the UEInformationResponse message:

4>	include the logMeasAvailable;

4>	if bt-LocationInfo is included in locationInfo of one or more of the additional logged measurement entries in VarLogMeasReport that are not included in the logMeasInfoList within the UEInformationResponse message:

5>	include the logMeasAvailableBT;

4>	if wlan-LocationInfo is included in locationInfo of one or more of the additional logged measurement entries in VarLogMeasReport that are not included in the logMeasInfoList within the UEInformationResponse message:

5>	include the logMeasAvailableWLAN;

1>	if ra-ReportReq is set to true and the UE has random access related information available in VarRA-Report and if the RPLMN is included in plmn-IdentityList stored in VarRA-Report; or

1>	if ra-ReportReq is set to true and the UE has random access related information available in VarRA-Report and if the registered SNPN identity is included in snpn-IdentityList stored in VarRA-Report:

2> for each ra-ReportList in VarRA-Report that consists of failed SDT information:

3> set timeSinceSdt-Execution to the time that elapsed since SDT execution;

2>	set the ra-ReportList in the UEInformationResponse message to the value of ra-ReportList in VarRA-Report;

2>	discard the ra-ReportList from VarRA-Report upon successful delivery of the UEInformationResponse message confirmed by lower layers;

1>	if rlf-ReportReq is set to true:

2>	if the UE has radio link failure information or handover failure information available in VarRLF-Report and if the RPLMN is included in plmn-IdentityList stored in VarRLF-Report; or

2>	if the UE has radio link failure information or handover failure information available in VarRLF-Report and if the current registered SNPN identity is included in snpn-IdentityList stored in VarRLF-Report:

3>	set timeSinceFailure in VarRLF-Report to the time that elapsed since the last radio link failure or handover failure in NR;

3>	set the rlf-Report in the UEInformationResponse message to the value of rlf-Report in VarRLF-Report;

3>	discard the rlf-Report from VarRLF-Report upon successful delivery of the UEInformationResponse message confirmed by lower layers;

2>	else if the UE is capable of cross-RAT RLF reporting as defined in TS 38.306 [26] and has radio link failure information or handover failure information available in VarRLF-Report of TS 36.331 [10] and if the RPLMN is included in plmn-IdentityList stored in VarRLF-Report of TS 36.331 [10]:

3>	set timeSinceFailure in VarRLF-Report of TS 36.331 [10] to the time that elapsed since the last radio link failure or handover failure in EUTRA;

3>	set failedPCellId-EUTRA in the rlf-Report in the UEInformationResponse message to indicate the PCell in which RLF was detected or the source PCell of the failed handover in the VarRLF-Report of TS 36.331 [10];

3>	set the measResult-RLF-Report-EUTRA in the rlf-Report in the UEInformationResponse message to the value of rlf-Report in VarRLF-Report of TS 36.331 [10];

3>	discard the rlf-Report from VarRLF-Report of TS 36.331 [10] upon successful delivery of the UEInformationResponse message confirmed by lower layers;

1>	if connEstFailReportReq is set to true and the UE has connection establishment failure or connection resume failure information in VarConnEstFailReport or VarConnEstFailReportList and if the RPLMN is equal to plmn-Identity stored in VarConnEstFailReport or in at least one of the entries of VarConnEstFailReportList:

1>	if connEstFailReportReq is set to true and if the UE has connection establishment failure information or connection resume failure information available in VarConnEstFailReport or VarConnEstFailReportList and if the registered SNPN identity is equal to snpn-Identity in networkIdentity stored in VarConnEstFailReport or any entry of VarConnEstFailReportList:

2>	set timeSinceFailure in VarConnEstFailReport to the time that elapsed since the last connection establishment failure or connection resume failure in NR;

2>	set the connEstFailReport in the UEInformationResponse message to the value of connEstFailReport in VarConnEstFailReport;

2>	if the UE supports multiple CEF report:

3>	for each connEstFailReport in the connEstFailReportList in VarConnEstFailReportList:

4>	set timeSinceFailure to the time that elapsed since the associated connection establishment failure or connection resume failure in NR;

2>	for each connEstFailReport in the connEstFailReportList in the UEInformationResponse message, set the value to the value of connEstFailReport in VarConnEstFailReport in VarConnEstFailReportList;

2>	discard the connEstFailReport from VarConnEstFailReport and VarConnEstFailReportList upon successful delivery of the UEInformationResponse message confirmed by lower layers;

1>	if the mobilityHistoryReportReq is set to true:

2>	include the mobilityHistoryReport and set it to include visitedCellInfoList from VarMobilityHistoryReport;

2>	include in the mobilityHistoryReport an entry for the current PCell, possibly after removing the oldest entry if required, and set its fields as follows:

3>	set visitedCellId to the global cell identity or the physical cell identity and carrier frequency of the current PCell:

3>	set field timeSpent to the time spent in the current PCell;

3>	if the UE supports PSCell mobility history information and if visitedPSCellInfoList is present in VarMobilityHistoryReport:

4>	for the newest entry of the PCell in the mobilityHistoryReport, include visitedPSCellInfoList from VarMobilityHistoryReport;

4>	if the UE is configured with a PSCell:

5>	for the newest entry of the PCell in the mobilityHistoryReport, include the current PSCell information in the visitedPSCellInfoListReport, possibly after removing the oldest PSCell entry of a PCell in the mobilityHistoryReport, if required, and set its fields as follows:

6>	set visitedCellId to the global cell identity or the physical cell identity and carrier frequency of the current PSCell:

6>	set field timeSpent to the time spent in the current PSCell while being connected to the current PCell;

6>	if the UE supports storing and reporting SCG activation information in mobility history information:

7> set the field scgActiveDuration of the entry to the accumulated time spent in the current PSCell with SCG state set to activated during the stay in the PSCell while being connected to the current PCell, if available;

4>	else:

5>	for the newest entry of the PCell in the mobilityHistoryReport, include a new entry in the visitedPSCellInfoListReport, possibly after removing the oldest PSCell entry of a PCell in the mobilityHistoryReport, if required, and set its fields as follows:

6>	set field timeSpent to the time spent without PSCell in the current PCell since last PSCell release since connected to the current PCell in RRC_CONNECTED;

3>	else if the UE supports PSCell mobility history information:

4>	if the UE is configured with a PSCell:

5>	for the newest entry of the PCell in the mobilityHistoryReport, include the current PSCell information in the visitedPSCellInfoListReport, possibly after removing the oldest PSCell entry of a PCell in the mobilityHistoryReport, if required, and set its fields as follows:

6>	set visitedCellId to the global cell identity or the physical cell identity and carrier frequency of the current PSCell:

6>	set field timeSpent to the time spent in the current PSCell while being connected to the current PCell;

6>	if the UE supports storing and reporting SCG activation information in mobility history information:

7> set the field scgActiveDuration of the entry to the accumulated time spent in the current PSCell with SCG state set to activated while being connected to the current PCell, if available;

4>	else:

5>	for the newest entry of the PCell in the mobilityHistoryReport, include a new entry in the visitedPSCellInfoListReport, possibly after removing the oldest PSCell entry of a PCell in the mobilityHistoryReport, if required, and set its fields as follows:

6>	set field timeSpent to the time spent without PSCell in the current PCell since connected to the current PCell in RRC_CONNECTED;

1>	if the successHO-ReportReq is set to true and if the UE has successful handover related information available in VarSuccessHO-Report and if the RPLMN is included in the plmn-IdentityList stored in VarSuccessHO-Report; or

1>	if the successHO-ReportReq is set to true and if the UE has successful handover related information available in VarSuccessHO-Report and if the current registered SNPN identity is included in snpn-IdentityList if stored in the VarSuccessHO-Report:

2>	if the successHO-Report in the VarSuccessHO-Report concerns a DAPS handover and if a PDCP PDU has been received from the source cell of the concerned HO and a non-duplicated PDCP PDU has been received from the target cell of the concerned HO:

3>	set upInterruptionTimeAtHO in VarSuccessHO-Report to include the time elapsed between the time of arrival of the last PDCP PDU received from the source cell of the concerned handover and the time of arrival of the first non-duplicate PDCP PDU received from the target cell of the concerned handover, as measured at the time of arrival of the first non-duplicate PDCP PDU received from the target cell;

2>	if the successHO-Report in the VarSuccessHO-Report concerns a mobilityFromNRCommand:

3>	set timeSinceSHR in VarSuccessHO-Report to the time that elapsed since the successful handover report determination as specified in 5.7.10.6;

2>	set the successHO-Report in the UEInformationResponse message to the value of successHO-Report in the VarSuccessHO-Report, if available;

2>	discard the VarSuccessHO-Report upon successful delivery of the UEInformationResponse message confirmed by lower layers;

1>	if the successPSCell-ReportReq is set to true and if the UE has successful PSCell change or addition information available in VarSuccessPSCell-Report and if the RPLMN is included in plmn-IdentityList stored in VarSuccessPSCell-Report; or

1>	if the successPSCell-ReportReq is set to true and if the UE has successful PSCell change or addition information available in VarSuccessPSCell-Report and if the current registered SNPN identity is included in snpn-IdentityList if stored in the VarSuccessPSCell-Report:

2>	set the successPSCell-Report in the UEInformationResponse message to the value of successPSCell-Report in the VarSuccessPSCell-Report;

2>	discard the VarSuccessPSCell-Report upon successful delivery of the UEInformationResponse message confirmed by lower layers;

1>	if the coarseLocationRequest is set to true:

2>	include coarseLocationInfo, if available;

1>	if the flightPathInfoReq is included in the UEInformationRequest and the UE has (updated) flight path information available, set the flightPathInfoReport in the UEInformationResponse message as follows:

2>	include the list of up to maxWayPointNumber waypoints, if any, along the flight path;

2>	if the includeTimeStamp is set to true, for each included waypoint:

3>	if available, set the field timestamp to the time when UE intends to arrive at the waypoint;

1>	if the logMeasReport is included in the UEInformationResponse:

2>	submit the UEInformationResponse message to lower layers for transmission via SRB2;

2>	discard the logged measurement entries included in the logMeasInfoList from VarLogMeasReport upon successful delivery of the UEInformationResponse message confirmed by lower layers;

1>	else:

2>	submit the UEInformationResponse message to lower layers for transmission via SRB1.

#### 5.7.10.4	Actions for the Random Access report determination

Upon successfully performing random-access procedure initialized with 4-step or 2-step RA type, or upon failed or successfully completed on-demand system information acquisition procedure in RRC_IDLE or RRC_INACTIVE state, or upon failed or successfully completed RA-SDT operation as specified in clause 5.3.13.5, the UE shall:

1>	if the RPLMN or the PLMN selected by upper layers (see TS24.501 [23]) from the PLMN(s) included in the plmn-IdentityList in SIB1 is not included in plmn-IdentityList stored in a non-empty VarRA-Report; or

1>	if the registered SNPN identity or the SNPN identity selected by upper layers (see TS24.501 [23]) from the list of SNPN(s) included in the NPN-IdentityInfoList in SIB1is not included in snpn-IdentityList stored in a non-empty VarRA-Report:

2>	clear the information included in VarRA-Report;

1>	if the UE is not in SNPN access mode and if the number of RA-Report entries stored in the ra-ReportList in VarRA-Report is less than maxRAReport:

2>	if the number of PLMN entries in plmn-IdentityList in identityList stored in VarRA-Report is less than maxPLMN; or

2>	if the number of PLMN entries in plmn-IdentityList in identityList stored in VarRA-Report is equal to maxPLMN and the list of EPLMNs is subset of or equal to the plmn-IdentityList stored in VarRA-Report:

3>	append the following contents associated to the random-access procedure as a new entry in the VarRA-Report:

4>	if the list of EPLMNs has been stored by the UE:

5>	set the plmn-IdentityList to include the list of EPLMNs stored by the UE (i.e. includes the RPLMN) without exceeding the limit of maxPLMN;

4>	else:

5>	set the plmn-Identity, in plmn-IdentityList, to the PLMN selected by upper layers (see TS 24.501 [23]) from the PLMN(s) included in the plmn-IdentityInfoList in SIB1;

4>	set the cellId to the global cell identity and the tracking area code, if available, otherwise to the physical cell identity and carrier frequency of the cell in which the corresponding random-access preamble was transmitted;

4>	if the UE supports spCell ID indication:

5>	if the corresponding random-access procedure was performed on an SCell of MCG:

6>	set the spCellId to the global cell identity of the PCell;

5>	if the corresponding random-access procedure was performed on an SCell of SCG:

6>	set the spCellId to the global cell identity of the PSCell, if available, otherwise, set the spCellId to the global cell identity of the PCell;

5>	if the corresponding random-access procedure was performed on PSCell:

6>	if the cellId is not set to the global cell identity of the PSCell, set the spCellId to the global cell identity of the PCell;

4>	if the random-access procedure is initiated for SDT and the SDT transmission was failed as defined in TS 38.300 [2]:

5>	include the sdt-Failed;

5>	set the sdt-FailureCause to the cause of SDT failure;

4>	if the conditions to initiate MO-SDT were evaluated and not fullfilled according to TS 38.321 [3]:

5>	set the sdt-DL-RsrpInfo to the RSRP value measured at the time of SDT evaluation as specified in TS 38.321 [3];

5>	set the sdt-UL-DataVolume to the UL data volume at the time of SDT evaluation as specified in TS 38.321 [3];

4>	set the raPurpose to include the purpose of triggering the random-access procedure;

4>	set the ra-InformationCommon as specified in clause 5.7.10.5.

1>	if the UE is in SNPN access mode and if the number of RA-Report entries stored in the ra-ReportList in VarRA-Report is less than maxRAReport:

2>	if the number of SNPN identity entries in snpn-IdentityList in identityList stored in VarRA-Report is less than maxNPN; or

2>	if the number of SNPN identity entries in snpn-IdentityList in identityList stored in VarRA-Report is equal to maxNPN and the list of equivalent SNPN(s) is subset of or equal to the snpn-IdentityList stored in VarRA-Report:

3>	append the following contents associated to the random-access procedure as a new entry in the VarRA-Report:

4>	if the list of equivalent SNPN(s) has been stored by the UE:

5>	set the snpn-IdentityList to include the list of equivalent SNPN(s) stored by the UE (i.e. includes the registered SNPN identity) without exceeding the limit of maxNPN;

4>	else:

5>	set the snpn-Identity, in snpn-IdentityList, to the SNPN identity selected by upper layers (see TS 24.501 [23]) from the SNPN identities included in the NPN-IdentityInfoList in SIB1;

4>	set the cellId to the global cell identity and the tracking area code, if available, otherwise to the physical cell identity and carrier frequency of the cell in which the corresponding random-access preamble was transmitted;

4>	if the UE supports spCell ID indication:

5>	if the corresponding random-access procedure was performed on an SCell of MCG:

6>	set the spCellId to the global cell identity of the PCell;

5>	if the corresponding random-access procedure was performed on an SCell of SCG:

6>	set the spCellId to the global cell identity of the PSCell, if available, otherwise, set the spCellId to the global cell identity of the PCell;

5>	if the corresponding random-access procedure was performed on PSCell:

6>	if the cellId is not set to the global cell identity of the PSCell, set the spCellId to the global cell identity of the PCell;

4>	if the random-access procedure is initiated for SDT and the SDT transmission was failed as defined in TS 38.300 [2]:

5>	include the sdt-Failed;

5>	set the sdt-FailureCause to the cause of SDT failure;

4>	if conditions to initiate MO-SDT were evaluated and not fullfilled according to TS 38.321 [3]:

5>	set the sdt-DL-RsrpInfo to the RSRP value measured at the time of SDT evaluation as specified in TS 38.321 [3];

5>	set the sdt-UL-DataVolume to the UL data volume at the time of SDT evaluation as specified in TS 38.321 [3];

4>	set the raPurpose to include the purpose of triggering the random-access procedure;

4>	set the ra-InformationCommon as specified in clause 5.7.10.5.

The UE may discard the random access report information, i.e. release the UE variable VarRA-Report, 48 hours after the last successful random access procedure or the failed or successfully completed on-demand system information acquisition procedure or the failed or successfully completed RA-SDT procedure related information is added to the VarRA-Report.

NOTE 1:	Void.

#### 5.7.10.5	RA information determination

The UE shall, for the last successfully completed or last failed random-access procedure, set the content in ra-InformationCommon as follows:

1>	set the absoluteFrequencyPointA to indicate the absolute frequency of the reference resource block associated to the random-access resources used in the random-access procedure;

1>	set the locationAndBandwidth and subcarrierSpacing associated to the UL BWP of the random-access resources used in the random-access procedure;

1>	if contention based random-access resources are used in the random-access procedure:

2>	set the msgA-RO-FrequencyStart and msgA-RO-FDM and msgA-SubcarrierSpacing associated to the 2 step random- access resources if used in the random-access procedure;

2>	if msgA-SubcarrierSpacing associated to the 2 step random-access resources used in the random-access procedure is available:

3>	set the msgA-SubcarrierSpacing associated to the 2 step random-access resources used in the random-access procedure;

2>	else if only 2 step random-access resources are available in the UL BWP used in the random-access procedure:

3>	set the msgA-SCS-From-prach-ConfigurationIndex to the subcarrier spacing as derived from the msgA-PRACH-ConfigurationIndex used in the 2-step random-access procedure;

2>	else:

3>	set the msg1-SubcarrierSpacing associated to the 4 step random-access resources used in the random-access procedure;

2>	set the msg1-FrequencyStart associated to the 4 step random-access resources if used in the random-access procedure, and if its value is different from the value of msgA-RO-FrequencyStart if it is included in the ra-InformationCommon;

2>	set the msg1-FDM associated to the 4 step random-access resources if used in the random-access procedure, and if its value is different from the value of msgA-RO-FDM if it is included in the ra-InformationCommon;

2>	if msg1-SubcarrierSpacing associated to the 4 step random-access resources used in the random-access procedure is available, and if its value is different from the value of msgA-SubcarrierSpacing if it is included in the ra-InformationCommon:

3>	set the msg1-SubcarrierSpacing associated to the 4 step random-access resources used in the random-access procedure;

2> else:

3>	set the msg1-SCS-From-prach-ConfigurationIndex to the subcarrier spacing as derived from the prach-ConfigurationIndex used in the 4-step random-access procedure, and if its value is different from the value of msgA-SCS-From-prach-ConfigurationIndex if it is included in the ra-InformationCommon;

1>	if contention free random-access resources are used in the random-access procedure:

2>	set the msg1-FrequencyStartCFRA and msg1-FDMCFRA associated to the 4 step random-access resources if used in the random-access procedure;

2> if msg1-SubcarrierSpacing associated to the 4 step random-access resources used in the random-access procedure is available:

3>	set the msg1-SubcarrierSpacingCFRA associated to the 4 step random-access resources used in the random-access procedure;

2> else:

3>	set the msg1-SCS-From-prach-ConfigurationIndexCFRA to the subcarrier spacing as derived from the prach-ConfigurationIndex used in the 4 step random-access procedure;

2>	set the msgA-RO-FrequencyStartCFRA and msgA-RO-FDMCFRA associated to the 2 step contention free random access resources if used in the random-access procedure;

2>	set the msgA-MCS, the nrofPRBs-PerMsgA-PO, the msgA-PUSCH-TimeDomainAllocation, the frequencyStartMsgA-PUSCH, the nrofMsgA-PO-FDM associated to the 2 step random-access resources if used in the random-access procedure;

2>	if msgA-SubcarrierSpacing associated to the 2 step random-access resources used in the random-access procedure is available:

3>	set the msgA-SubcarrierSpacing associated to the 2 step random-access resources used in the random-access procedure;

2>	else if only 2 step random-access resources are available in the UL BWP used in the random-access procedure:

3>	set the msgA-SCS-From-prach-ConfigurationIndex to the subcarrier spacing as derived from the msgA-PRACH-ConfigurationIndex used in the 2-step random-access procedure;

2>	else:

3>	set the msg1-SubcarrierSpacing associated to the 4 step random-access resources used in the random-access procedure;

1>	if the random access procedure is initialized with RA_TYPE set to 2-stepRA as described in TS 38.321 [3]:

2>	set the dlPathlossRSRP to the measeured RSRP of the DL pathloss reference obtained at the time of RA_Type selection stage of the initialization of the RA procedure as captured in TS 38.321 [3];

2>	if the configuration for the random access msgA-TransMax was configured in RACH-ConfigDedicated for this random access procedure, and raPurpose is set to reconfigurationWithSync or ltm:

3>	set msgA-TransMax to the value of msgA-TransMax in RACH-ConfigDedicated;

2>	else if msgA-TransMax was configured in RACH-ConfigCommonTwoStepRA:

3>	set msgA-TransMax to the value of msgA-TransMax in RACH-ConfigCommonTwoStepRA;

2>	set the msgA-PUSCH-PayloadSize to the size of the overall payload available in the UE buffer at the time of initiating the 2 step RA procedure;

1>	if the purpose of the random access procedure is to request on-demand system information (i.e., if the raPurpose is set to requestForOtherSI or msg3RequestForOtherSI):

2>	set the intendedSIBs to indicate the SIB(s) the UE wanted to receive as a result of the SI request;

2>	set the ssbsForSI-Acquisition to indicate the SSB(s) used to receive the SI message;

2>	if the on-demand system information acquisition was successful:

3>	set the onDemandSISuccess to true;

1>	if one or more of the features including RedCap and/or Slicing and/or SDT and/or MSG3 repetitions and/or MSG1 repetitions and/or eRedCap are applicable for this random-access procedure as specified in clause 5.1.1b of TS 38.321[3]:

2>	set the triggeredFeatureCombination to indicate all the features triggering this random-access procedure as below:

3>	if this random-access procedure is triggered by RedCap, include redCap;

3>	if this random-access procedure is triggered by SDT, include smallData;

3>	if this random-access procedure is triggered by Msg3 repetitions, include msg3-Repetitions;

3>	if this random-access procedure is triggered by Msg1 repetitions, include msg1-Repetitions;

3>	if this random-access procedure is triggered by eRedCap, include eRedCap;

3>	if this random-access procedure is triggered by slicing, set nsag to the NSAG-ID(s) applied in the random-access procedure and set the triggered-S-NSSAI-List to include all the S-NSSAI(s) associated to the slices triggering the access attempt in the random-access procedure;

2>	if the value of used feature or combination of features is different from the triggeredFeatureCombination:

3>	set the usedFeatureCombination to indicate one or more features of FeatureCombination associated to the random-access resource used in the random-access procedure as below:

4>	if RedCap is part of the used FeatureCombination, include redCap;

4>	if SDT is part of the used FeatureCombination, include smallData;

4>	if Msg3 repetitions is part of the used FeatureCombination, include msg3-Repetitions;

4>	if Msg1 repetitions is part of the used FeatureCombination, include msg1-Repetitions;

4>	if eRedCap is part of the used FeatureCombination, include eRedCap;

4>	if NSAG(s) is part of the used FeatureCombination, set nsag to include the NSAG-ID(s) configured for the used FeatureCombination;

2>	set the startPreambleForThisPartition to the value of startPreambleForThisPartition in FeatureCombinationPreambles associated to the used FeatureCombination;

2>	set the numberOfPreamblesPerSSB-ForThisPartition to the value of numberOfPreamblesPerSSB-ForThisPartition in FeatureCombinationPreambles associated to the used FeatureCombination;

1>	set the parameters associated to the successive random-access attempts associated to the selected beam in the perRAInfoList as follows:

2>	if the random-access resource used is associated to a SS/PBCH block, set the associated random-access parameters for the successive random-access attempts associated to the same SS/PBCH block for one or more random-access attempts as follows:

3>	set the ssb-Index to include the SS/PBCH block index associated to the used random-access resource;

3>	set the numberOfPreamblesSentOnSSB to indicate the number of successive random-access attempts associated to the SS/PBCH block;

3>	if all preamble transmissions for the successive random-access attempts associated to this SS/PBCH block were blocked by LBT:

4>	include allPreamblesBlocked;

3>	else:

4>	if LBT failure indication was received from lower layers for the last random-access preamble transmission attempt in the SS/PBCH block associated to the ssb-Index, before changing the SS/PBCH block for random access preamble transmission:

5>	include lbt-Detected;

3>	for each random-access attempt performed on the random-access resource, except the random-access attempts for which LBT failure indication was received from lower layers, include the following parameters in the chronological order of the random-access attempt:

4>	if the random-access attempt is performed on the contention based random-access resource and if raPurpose is not equal to 'requestForOtherSI', include contentionDetected as follows:

5>	if contention resolution was not successful as specified in TS 38.321 [6] for the transmitted preamble:

6>	set the contentionDetected to true;

5>	else:

6>	set the contentionDetected to false;

4>	if the random access attempt is a 2-step random access attempt:

5>	if fallback from 2-step random access to 4-step random access occurred during the random access attempt:

6>	set fallbackToFourStepRA to true;

4>	if the random-access attempt is performed on the contention based random-access resource; or

4>	if the random-access attempt is performed on the contention free random-access resource and if the random-access procedure was initiated due to the PDCCH ordering:

5>	if the random access attempt is a 4-step random access attempt and the SS/PBCH block RSRP of the SS/PBCH block corresponding to the random-access resource used in the random-access attempt is above rsrp-ThresholdSSB; or

5>	if the random access attempt is a 2-step random access attempt and the SS/PBCH block RSRP of the SS/PBCH block corresponding to the random-access resource used in the random-access attempt is above msgA-RSRP-ThresholdSSB:

6>	set the dlRSRPAboveThreshold to true;

5>	else:

6>	set the dlRSRPAboveThreshold to false;

2>	else if the random-access resource used is associated to a CSI-RS, set the associated random-access parameters for the successive random-access attempts associated to the same CSI-RS for one or more random-access attempts as follows:

3>	set the csi-RS-Index to include the CSI-RS index associated to the used random-access resource;

3>	set the numberOfPreamblesSentOnCSI-RS to indicate the number of successive random-access attempts associated to the CSI-RS;

3>	if all preamble transmissions for the successive random-access attempts associated to this CSI-RS were blocked by LBT:

4>	include allPreamblesBlocked;

3>	else:

4>	if LBT failure indication was received from lower layers for the last random-access preamble transmission attempt in the CSI-RS associated to the csi-RS-Index, before changing the CSI-RS for random access preamble transmission:

5>	include lbt-Detected;

1>	if at least one LBT failure indication has been received from lower layers during the random-access procedure:

2>	set the numberOfLBT-Failures to indicate the total number of random-access attempts for which LBT failure indications have been received from lower layers in the random-access procedure.

The UE shall, for all the BWPs in which consistent LBT failures are triggered and not cancelled (according to TS 38.321 [3]) prior to successful RA completion or prior to RLF/HOF, set the below parameters in attemptedBWP-InfoList in the chronological order of BWP selection:

1>	set the locationAndBandwidth and subcarrierSpacing associated to the UL BWP.

NOTE 1:	Void.

NOTE 2:	If allPreamblesBlocked is included, it is left to UE implementation how to set the numberOfPreamblesSentOnSSB-r16, numberOfPreamblesSentOnCSI-RS-r16 and the perRAAttemptInfoList-r16.

#### 5.7.10.6	Actions for the successful handover report determination

The UE shall for the PCell:

1>	if the procedure is triggered due to successful completion of reconfiguration with sync, and if the ratio between the value of the elapsed time of the timer T304 and the configured value of the timer T304, included in the last applied RRCReconfiguration message including the reconfigurationWithSync, is greater than thresholdPercentageT304 if included in the successHO-Config received before executing the last reconfiguration with sync; or

1>	if the procedure is triggered due to successful completion of reconfiguration with sync, and if the ratio between the value of the elapsed time of the timer T310 and the configured value of the timer T310, configured while the UE was connected to the source PCell before executing the last reconfiguration with sync, is greater than thresholdPercentageT310 included in the successHO-Config if configured by the source PCell before executing the last reconfiguration with sync; or

1>	if the procedure is triggered due to successful completion of reconfiguration with sync, and if the T312 associated to the measurement identity of the target cell was running at the time of initiating the execution of the reconfiguration with sync procedure and if the ratio between the value of the elapsed time of the timer T312 and the configured value of the timer T312, configured while the UE was connected to the source PCell before executing the last reconfiguration with sync, is greater than thresholdPercentageT312 included in the successHO-Config if configured by the source PCell before executing the last reconfiguration with sync; or

1>	if the procedure is triggered due to successful completion of reconfiguration with sync, and if sourceDAPS-FailureReporting is included in the successHO-Config before executing the last reconfiguration with sync and is set to true and if the last executed handover was a DAPS handover and if an RLF occurred at the source PCell during the DAPS handover while T304 was running; or:

1>	if the procedure is triggered due to successful completion of Mobility from NR to E-UTRA, and if the ratio between the value of the elapsed time of the timer T310 and the configured value of the timer T310, configured while the UE was connected to the source PCell before executing the last Mobility from NR to E-UTRA, is greater than thresholdPercentageT310 included in the successHO-Config if configured by the source PCell before executing the last Mobility from NR to E-UTRA; or

1>	if the procedure is triggered due to successful completion of Mobility from NR to E-UTRA, and if the T312 associated to the measurement identity of the target cell was running at the time of initiating the execution of the Mobility from NR to E-UTRA and if the ratio between the value of the elapsed time of the timer T312 and the configured value of the timer T312, configured while the UE was connected to the source PCell before executing the last Mobility from NR to E-UTRA, is greater than thresholdPercentageT312 included in the successHO-Config if configured by the source PCell before executing the last Mobility from NR to E-UTRA:

2>	store the successful handover information in VarSuccessHO-Report and determine the content in VarSuccessHO-Report as follows:

3>	clear the information included in VarSuccessHO-Report, if any;

3>	if the UE is not in SNPN access mode, set the plmn-IdentityList to include the list of EPLMNs stored by the UE (i.e., includes the RPLMN);

3>	else if the UE is in SNPN access mode, set the snpn-IdentityList to include the list of equivalent SNPNs stored by the UE (i.e., including the registered SNPN identity), if available;

3>	for intra-NR handover, set the c-RNTI to the C-RNTI assigned by the target PCell of the handover;

3>	if the procedure is triggered due to successful completion of reconfiguration with sync, for the source PCell in which the last RRCReconfiguration message including reconfigurationWithSync was applied; or

3>	if the procedure is triggered due to successful completion of Mobility from NR to E-UTRA, for the source PCell in which the last MobilityFromNRCommand concerning an inter-RAT handover from NR to E-UTRA was applied:

4>	set the sourcePCellID in sourceCellInfo to the global cell identity and tracking area code, if available, of the source PCell;

4>	set the sourceCellMeas in sourceCellInfo to include the cell level RSRP, RSRQ and the available SINR, of the source PCell based on the available SSB and CSI-RS measurements collected up to the moment the UE sends RRCReconfigurationComplete message if the procedure is triggered due to successful completion of reconfiguration with sync, or up to the moment the UE sends the EUTRA RRCConnectionReconfigurationComplete message if the procedure is triggered due to successful completion of Mobility from NR to E-UTRA;

4>	set the rsIndexResults in sourceCellMeas to include all the available SSB and CSI-RS measurement quantities of the source PCell collected up to the moment the UE sends RRCReconfigurationComplete message if the procedure is triggered due to successful completion of reconfiguration with sync, or up to the moment the UE sends the EUTRA RRCConnectionReconfigurationComplete message if the procedure is triggered due to successful completion of Mobility from NR to E-UTRA;

4>	if the UE supports successful handover report for MCG LTM cell switch and if the UE was configured with ltm-Config associated with the MCG when connected to the source PCell:

5>	set the resultsSSB-Indexes in sourceCellMeasL1 to include all the available SS/PBCH block L1-RSRP measurement quantities of the source PCell collected up to the moment the UE sends RRCReconfigurationComplete message;

4>	if the last executed handover was a DAPS handover and if an RLF occurred at the source PCell during the DAPS handover while T304 was running:

5>	set the rlf-InSourceDAPS in sourceCellInfo to true;

3>	if the procedure is triggered due to successful completion of reconfiguration with sync and if the UE was configured with condExecutionCond and condExecutionCondPScell, for the source PSCell in which the last RRCReconfiguration message including reconfigurationWithSync was applied;

4>	set the sourcePSCellId in sourcePSCellInfo to the global cell identity and tracking area code, if available, of the source PSCell;

4>	set the sourcePSCellMeas in sourcePSCellInfo to include the available cell level RSRP, RSRQ and the SINR, of the source PSCell based on the available SSB and CSI-RS measurements collected up to the moment the UE sends RRCReconfigurationComplete message if the procedure is triggered due to successful completion of reconfiguration with sync;

4>	set the rsIndexResults in sourcePSCellMeas to include all the available SSB and CSI-RS measurement quantities of the source PSCell collected up to the moment the UE sends RRCReconfigurationComplete message if the procedure is triggered due to successful completion of reconfiguration with sync;

4>	set the targetPSCellId to the global cell identity and tracking area code, if available, of the target PSCell, and otherwise to the physical cell identity and carrier frequency of the target PSCell;

3>	if the procedure is triggered due to successful completion of reconfiguration with sync, for the target PCell indicated in the last applied RRCReconfiguration message including reconfigurationWithSync:

4>	set the targetPCellID in targetCellInfo to the global cell identity and tracking area code, if available, of the target PCell; otherwise, set the targetCell-PCI-ARFCN to the physical cell identity and carrier frequency of the target PCell;

NOTE 00:	If targetCell-PCI-ARFCN is included, it is left to UE implementation how to set the targetPCellID.

4>	set the targetCellMeas in targetCellInfo to include the cell level RSRP, RSRQ and the available SINR, of the target PCell based on the available SSB and CSI-RS measurements collected up to the moment the UE sends RRCReconfigurationComplete message;

4>	set the rsIndexResults in targetCellMeas to include all the available SSB and CSI-RS measurement quantities of the target PCell collected up to the moment the UE sends RRCReconfigurationComplete message;

4>	if the UE supports successful handover report for MCG LTM cell switch and if the UE was configured with ltm-Config associated with the MCG when connected to the source PCell:

5>	set the resultsSSB-Indexes in targetCellMeasL1 to include all the available SS/PBCH block L1-RSRP measurement quantities of the target PCell collected up to the moment the UE sends RRCReconfigurationComplete message;

4>	if the last applied RRCReconfiguration message including reconfigurationWithSync was included in the stored condRRCReconfig:

5>	set the timeSinceCHO-Reconfig to the time elapsed between the initiation of the execution of conditional reconfiguration for the target PCell and the reception of the last applied  conditionalReconfiguration including the condRRCReconfig of the target PCell in the source PCell;

3>	if the procedure is triggered due to successful completion of Mobility from NR to E-UTRA, for the target PCell indicated in the last applied MobilityFromNRCommand concerning an inter-RAT handover from NR to E-UTRA:

4>	set the targetPCellId in eutra-TargetCellInfo to the global cell identity and tracking area code, if available, and otherwise to the physical cell identity and carrier frequency of the target PCell;

4>	set the targetCellMeas in eutra-TargetCellInfo to include the cell level RSRP, RSRQ and the available SINR, of the target PCell based on the available measurements collected up to the moment the UE sends RRCConnectionReconfigurationComplete message;

NOTE 0:	If eutra-TargetCellInfo is included, it is left to UE implementation how to set the targetCellInfo.

3>	if the procedure is triggered due to successful completion of reconfiguration with sync and if the ratio between the value of the elapsed time of the timer T304 and the configured value of the T304 timer, included in the last applied RRCReconfiguration message including the reconfigurationWithSync, is greater than thresholdPercentageT304 if included in the successHO-Config received before executing the last reconfiguration with sync:

4>	set t304-cause in shr-Cause to true;

4> if the procedure is triggered due to successful completion of RACH-based reconfiguration with sync:

5>	set the ra-InformationCommon to include the random-access related information associated to the random access procedure in the target PCell, as specified in clause 5.7.10.5;

3>	if the ratio between the value of the elapsed time of the timer T310 and the configured value of the T310 timer, configured while the UE was connected to the source PCell before executing the last reconfiguration with sync or the last Mobility from NR to E-UTRA, is greater than thresholdPercentageT310 included in the successHO-Config if configured by the source PCell before executing the last reconfiguration with sync or Mobility from NR to E-UTRA:

4>	set t310-cause in shr-Cause to true;

3>	if the T312 associated to the measurement identity of the target cell was running at the time of initiating the execution of the reconfiguration with sync procedure or Mobility from NR to E-UTRA, and if the ratio between the value of the elapsed time of the timer T312 and the configured value of the T312 timer, configured while the UE was connected to the source PCell before executing the last reconfiguration with sync or Mobility from NR to E-UTRA, is greater than thresholdPercentageT312 included in the successHO-Config if configured by the source PCell before executing the last reconfiguration with sync, or Mobility from NR to E-UTRA:

4>	set t312-cause in shr-Cause to true;

3>	if the procedure is triggered due to successful completion of reconfiguration with sync and if sourceDAPS-FailureReporting included in the successHO-Config if configured by the source PCell before executing the last reconfiguration with sync is set to true, and if the last executed handover was a DAPS handover and if an RLF occurred at the source PCell during the DAPS handover while T304 was running:

4>	set sourceDAPS-Failure in shr-Cause to true;

3>	if the procedure is triggered due to successful completion of reconfiguration with sync, for each of the measObjectNR, configured by the source PCell, in which the last RRCReconfiguration message including reconfigurationWithSync was applied;or:

3>	if the procedure is triggered due to successful completion of Mobility from NR to E-UTRA, for each of the measObjectNR, configured by the source PCell, in which the last MobilityFromNRCommand concerning an inter-RAT handover from NR to E-UTRA was applied:

4>	if measRSSI-ReportConfig is configured for the frequency of the source PCell:

5>	if the procedure is triggered due to successful completion of reconfiguration with sync:

6>	set the measResultServCellRSSI to the linear average of the available RSSI sample value(s) provided by lower layers for the frequency of the source PCell up to the moment the UE sends the RRCReconfigurationComplete message;

5>	else if the procedure is triggered due to successful completion of Mobility from NR to E-UTRA:

6>	set the measResultServCellRSSI to the linear average of the available RSSI sample value(s) provided by lower layers for the frequency of the source PCell up to the moment the UE sends the EUTRA RRCConnectionReconfigurationComplete message;

4>	for each of the configured measObjectNR if measRSSI-ReportConfig is configured for the configured frequency:

5>	if the procedure is triggered due to successful completion of reconfiguration with sync:

6>	set the measResultNeighFreqRSSI in the measResultNeighFreqListRSSI to the linear average of the available RSSI sample value(s) provided by lower layers for the associated neighbouring frequency up to the moment the UE sends the RRCReconfigurationComplete message:

7>	for each neighbour frequency included, include the optional fields that are available;

5>	else if the procedure is triggered due to successful completion of Mobility from NR to E-UTRA:

6>	set the measResultNeighFreqRSSI in the measResultNeighFreqListRSSI to the linear average of the available RSSI sample value(s) provided by lower layers for the associated neighbouring frequency up to the moment the UE sends the EUTRA RRCConnectionReconfigurationComplete message:

7>	for each neighbour frequency included, include the optional fields that are available;

4>	if measurements are available for the measObjectNR:

5>	if the SS/PBCH block-based measurement quantities are available:

6>	set the measResultListNR in measResultNeighCells to include all the available measurement quantities of the best measured cells, other than the source PCell or target PCell, ordered such that the cell with highest SS/PBCH block RSRP is listed first if SS/PBCH block RSRP measurement results are available, otherwise the cell with highest SS/PBCH block RSRQ is listed first if SS/PBCH block RSRQ measurement results are available, otherwise the cell with highest SS/PBCH block SINR is listed first, based on the available SS/PBCH block based measurements collected up to the moment the UE sends the RRCReconfigurationComplete message if the procedure is triggered due to successful completion of reconfiguration with sync, or up to the moment the UE sends the EUTRA RRCConnectionReconfigurationComplete message if the procedure is triggered due to successful completion of Mobility from NR to E-UTRA;

6>	for each neighbour cell included, include the optional fields that are available;

NOTE 1:	For the neighboring cells set included in measResultListNR in measResultNeighCells ordered based on the SS/PBCH block measurement quantities, the UE includes also the CSI-RS based measurement quantities, if available.

5>	if the CSI-RS measurement quantities are available:

6>	set the measResultListNR in measResultNeighCells to include all the available measurement quantities of the best measured cells, other than the source PCell and target PCell, ordered such that the cell with highest CSI-RS RSRP is listed first if CSI-RS RSRP measurement results are available, otherwise the cell with highest CSI-RS RSRQ is listed first if CSI-RS RSRQ measurement results are available, otherwise the cell with highest CSI-RS SINR is listed first, based on the available CSI-RS based measurements collected up to the moment the UE sends the RRCReconfigurationComplete message if the procedure is triggered due to successful completion of reconfiguration with sync, or up to the moment the UE sends the EUTRA RRCConnectionReconfigurationComplete message if the procedure is triggered due to successful completion of Mobility from NR to E-UTRA;

6>	for each neighbour cell included, include the optional fields that are available;

NOTE 2:	For the neighboring cells set ordered based on the CSI-RS measurement quantities, the UE includes measurements only for the cells not yet included in measResultListNR in measResultNeighCells to avoid overriding SS/PBCH block-based ordered measurements.

3>	if the UE supports successful handover report for MCG LTM cell switch and if the UE was configured with ltm-Config associated with the MCG when connected to the source PCell:

4>	for each neighbour MCG LTM candidate cell:

5>	if SS/PBCH block-based L1-RSRP measurement quantities are available:

6>	set the neighCellsMeasL1ListNR to include all the available SS/PBCH block-based L1-RSRP measurement results of the best measured cells, other than the source PCell or target PCell, ordered such that the cell with highest SS/PBCH block-based L1-RSRP (of all SS/PBCH block-based L1-RSRP measurement results for the cell) is listed first, based on the available SS/PBCH block-based L1-RSRP measurements collected up to the moment the UE sends the RRCReconfigurationComplete message;

3>	if the procedure is triggered due to successful completion of reconfiguration with sync, for each of the measObjectEUTRA, configured by the source PCell in which the last RRCReconfiguration message including reconfigurationWithSync was applied; or:

3>	if the procedure is triggered due to successful completion of Mobility from NR to E-UTRA, for each of the measObjectEUTRA, configured by the source PCell in which the last MobilityFromNRCommand concerning an inter-RAT handover from NR to E-UTRA was applied:

4>	if measurements are available for the measObjectEUTRA:

5>	set the measResultListEUTRA in measResultNeighCells to include the best measured cells ordered such that the cell with highest RSRP is listed first if RSRP measurement results are available, otherwise the cell with highest RSRQ is listed first, based on measurements collected up to the moment the UE sends the RRCReconfigurationComplete message if the procedure is triggered due to successful completion of reconfiguration with sync, or up to the moment the UE sends the EUTRA RRCConnectionReconfigurationComplete message if the procedure is triggered due to successful completion of Mobility from NR to E-UTRA;

5>	for each neighbour cell included, include the optional fields that are available;

3>	for each of the neighbour cells included in measResultNeighCells:

4>	if the cell was a candidate target cell included in the condRRCReconfig within the conditionalReconfiguration configured by the source PCell, in which the last RRCReconfiguration message including reconfigurationWithSync was applied:

5>	set the choCandidate to true in measResultNR;

4>	if the UE supports successful handover report for MCG LTM cell switch and the cell was an LTM candidate cell in the LTM-Candidate IE within ltm-Config associated with the MCG when connected to the source PCell:

5>	set the ltm-Candidate to true in measResultNR;

3>	if the UE supports successful handover report for MCG LTM cell switch and the procedure is triggered due to successful completion of reconfiguration with sync concerning an LTM cell switch:

4>	if the last executed LTM cell switch is a RACH-less LTM cell switch:

5>	include the rach-Less;

3>	for each entry of condReconfigList in the MCG VarConditionalReconfig including both condExecutionCond and condExecutionCondPSCell, include an entry in choWithCandidateSCGInfoList and set the values as follows:

4>	set firstFulfilledConfig to cho or cpc, whichever was fulfilled first in time;

4>	if all triggering events of both condExecutionCond and condExecutionCondPSCell of the concerned entry of condReconfigList are fulfilled:

5>	set timeBetweenFulfillment to the elapsed time between the fulfillments of the last triggering events of the two execution conditions;

4>	set the pCellId to the global cell identity and tracking area code, if available, and otherwise the physical cell identity and carrier frequency, of the target candidate PCell stored in the condRRCReconfig of the concerned entry of condReconfigList;

4>	set the psCellId to the global cell identity and tracking area code, if available, and otherwise the physical cell identity and carrier frequency, of the target candidate PSCell stored in the condRRCReconfig of the concerned entry of condReconfigList;

3>	if available, set the locationInfo as in 5.3.3.7;

1>	release successHO-Config configured by the source PCell and thresholdPercentageT304 if configured by the target PCell.

The UE may discard the successful handover information, i.e., release the UE variable VarSuccessHO-Report, 48 hours after the last successful handover information is added to the VarSuccessHO-Report.

#### 5.7.10.7	Actions for the successful PSCell change or addition report determination

The UE shall for the PSCell:

1>	if the ratio between the value of the elapsed time of the timer T304 and the configured value of the timer T304, included in the last applied RRCReconfiguration message for the SCG including the reconfigurationWithSync, is greater than thresholdPercentageT304-SCG if included in the successPSCell-Config received before executing the last reconfiguration with sync for the SCG; or

1>	if sn-InitiatedPSCellChange associated to the last applied RRCReconfiguration with reconfigurationWithSync for the SCG is configured and if the ratio between the value of the elapsed time of the timer T310 and the configured value of the timer T310, configured while the UE was connected to the source PSCell before executing the last reconfiguration with sync for the SCG, is greater than thresholdPercentageT310-SCG included in the successPSCell-Config if configured by the source PSCell before executing the last reconfiguration with sync for the SCG; or

1>	if sn-InitiatedPSCellChange associated to the last applied RRCReconfiguration with reconfigurationWithSync for the SCG is configured and if the T312 associated to the measurement identity of the target PSCell included in a measConfig associated with the SCG was running at the time of initiating the execution of the reconfiguration with sync procedure for the SCG and if the ratio between the value of the elapsed time of the timer T312 and the configured value of the timer T312, configured while the UE was connected to the source PSCell before executing the last reconfiguration with sync, is greater than thresholdPercentageT312-SCG included in the successPSCell-Config if configured by the source PSCell before executing the last reconfiguration with sync for the SCG:

1>	if sn-InitiatedPSCellChange associated to the last applied RRCReconfiguration with reconfigurationWithSync for the SCG is not configured and if the ratio between the value of the elapsed time of the timer T310 and the configured value of the timer T310, configured while the UE was connected to the source PSCell before executing the last reconfiguration with sync for the SCG, is greater than thresholdPercentageT310-SCG included in the successPSCell-Config if configured by the PCell before executing the last reconfiguration with sync for the SCG; or

1>	if sn-InitiatedPSCellChange associated to the last applied RRCReconfiguration with reconfigurationWithSync for the SCG is not configured and if the T312 associated to the measurement identity of the target PSCell included in a measConfig associated with the SCG was running at the time of initiating the execution of the reconfiguration with sync procedure for the SCG and if the ratio between the value of the elapsed time of the timer T312 and the configured value of the timer T312, configured while the UE was connected to the source PSCell before executing the last reconfiguration with sync, is greater than thresholdPercentageT312-SCG included in the successPSCell-Config if configured by the PCell before executing the last reconfiguration with sync for the SCG:

2>	clear the information included in VarSuccessPSCell-Report, if any;

2>	store the successful PSCell change or addition information in VarSuccessPSCell-Report and determine the content in VarSuccessPSCell-Report as follows:

3>	if the UE is not in SNPN access mode, set the plmn-IdentityList to include the list of EPLMNs (including the RPLMN) stored by the UE;

3>	else if the UE is in SNPN access mode, set the snpn-IdentityList to include the list of equivalent SNPN identities (including the registered SNPN identity) stored by the UE, if available;

3>	if the procedure is triggered due to successful completion of CHO with candidate SCG:

4>	set the pCellId to the global cell identity and if available the tracking area code of the source PCell;

4>	set the targetPCellId to the global cell identity and tracking area code, if available, of the target PCell, and otherwise to the physical cell identity and carrier frequency of the target PCell;

3>	else:

4>	set the pCellId to the global cell identity and tracking area code, if available, of the PCell;

3>	for the source PSCell (in case of PSCell change procedure) in which the last RRCReconfiguration message for the SCG including reconfigurationWithSync was applied:

4>	set the sourcePSCellId in sourcePSCellInfo to the global cell identity and tracking area code, and otherwise to the physical cell identity and carrier frequency of the source PSCell;

4>	set the sourcePSCellMeas in sourcePSCellInfo to include the cell level RSRP, RSRQ and the available SINR, of the source PSCell based on the available SSB and CSI-RS measurements collected up to the moment the UE successfully completed the random access procedure for the SCG;

4>	set the rsIndexResults in sourcePSCellMeas to include all the available SSB and CSI-RS measurement quantities of the source PSCell collected up to the moment the UE successfully completed the random access procedure for the SCG;

3>	for the target PSCell indicated in the last applied RRCReconfiguration message for the SCG including reconfigurationWithSync:

4>	set the targetPSCellID in targetPSCellInfo to the global cell identity and tracking area code, if available, and otherwise to the physical cell identity and carrier frequency of the target PSCell;

4>	set the targetPSCellMeas in targetPSCellInfo to include the cell level RSRP, RSRQ and the available SINR, of the target PSCell based on the available SSB and CSI-RS measurements collected up to the moment the UE successfully completed the random access procedure for the SCG;

4>	set the rsIndexResults in targetPSCellMeas to include all the available SSB and CSI-RS measurement quantities of the target PSCell collected up to the moment the UE successfully completed the random access procedure for the SCG;

4>	if the last applied RRCReconfiguration message for the SCG including reconfigurationWithSync was included in the stored condRRCReconfig:

5>	set the timeSinceCPAC-Reconfig to the time elapsed between the initiation of the execution of conditional reconfiguration for the target PSCell and the reception of the last applied conditionalReconfiguration for the SCG including the condRRCReconfig of the target PSCell;

3>	if triggering threshold for storing the successful PSCell change or addition information in VarSuccessPSCell-Report based on the thresholdPercentageT304-SCG is met:

4>	set t304-cause in spr-Cause to true;

4>	set the ra-InformationCommon to include the random-access related information associated to the random access procedure in the target PSCell, as specified in clause 5.7.10.5;

3>	if triggering threshold for storing the successful PSCell change or addition information in VarSuccessPSCell-Report based on the thresholdPercentageT310-SCG is met:

4>	set t310-cause in spr-Cause to true;

3>	if triggering threshold for storing the successful PSCell change or addition information in VarSuccessPSCell-Report based on the thresholdPercentageT312-SCG is met:

4>	set t312-cause in spr-Cause to true;

3>	if sn-InitiatedPSCellChange associated to the last applied RRCReconfiguration with reconfigurationWithSync for the SCG is configured:

4>	consider all measObjectNR configured by the source PSCell;

3>	else:

4>	consider all measObjectNR configured by the PCell;

3>	for each of the considered measObjectNR:

4>	if measurements are available for the measObjectNR:

5>	if the SS/PBCH block-based measurement quantities are available:

6>	include in the measResultListNR in measResultNeighCells all the available measurement quantities of the best measured cells, other than the source PSCell (in case of PSCell change procedure) or target PSCell, ordered such that the cell with highest SS/PBCH block RSRP is listed first if SS/PBCH block RSRP measurement results are available, otherwise the cell with highest SS/PBCH block RSRQ is listed first if SS/PBCH block RSRQ measurement results are available, otherwise the cell with highest SS/PBCH block SINR is listed first, based on the available SS/PBCH block based measurements collected up to the moment the UE successfully completed the random access procedure;

6>	for each neighbour cell included, include the optional fields that are available (including the CSI-RS based measurement quantities, if available);

5>	if the CSI-RS measurement quantities are available for the cells not yet included in measResultListNR in measResultNeighCells:

6>	include in the measResultListNR in measResultNeighCells all the available measurement quantities of the best measured cells, other than the source PSCell (in case of PSCell change procedure) and target PSCell, ordered such that the cell with highest CSI-RS RSRP is listed first if CSI-RS RSRP measurement results are available, otherwise the cell with highest CSI-RS RSRQ is listed first if CSI-RS RSRQ measurement results are available, otherwise the cell with highest CSI-RS SINR is listed first, based on the available CSI-RS based measurements collected up to the moment the UE successfully completed the random access procedure;

6>	for each neighbour cell included, include the optional fields that are available;

3>	for each of the neighbour cells included in measResultNeighCells:

4>	if the cell was a candidate target cell included in the condRRCReconfig within the conditionalReconfiguration, configured by the source PCell including the condExecutionCond within the conditionalReconfiguration associated to condEventA4 or by the source PSCell (in case of PSCell change) when the last RRCReconfiguration message for the SCG including reconfigurationWithSync was applied:

5>	set the choCandidate to true in measResultNR;

3>	for each entry of condReconfigList in the MCG VarConditionalReconfig including both condExecutionCond and condExecutionCondPSCell, include an entry in choWithCandidateSCGInfoList and set the values as follows:

4>	set firstFulfilledConfig to cho or cpc, whichever was fulfilled first in time;

4>	if all triggering events of both condExecutionCond and condExecutionCondPSCell of the concerned entry of condReconfigList are fulfilled:

5>	set timeBetweenFulfillment to the elapsed time between the fulfillments of the last triggering events of the two execution conditions;

4>	set the pCellId to the global cell identity and tracking area code, if available, and otherwise the physical cell identity and carrier frequency, of the target candidate PCell stored in the condRRCReconfig of the concerned entry of condReconfigList;

4>	set the psCellId to the global cell identity and tracking area code, if available, and otherwise the physical cell identity and carrier frequency, of the target candidate PSCell stored in the condRRCReconfig of the concerned entry of condReconfigList;

3>	if sn-InitiatedPSCellChange associated to the last applied RRCReconfiguration with reconfigurationWithSync for the SCG is configured:

4>	if available, set the locationInfo as in 5.3.3.7 according to the otherConfig associated with the source PSCell;

4>	include sn-InitiatedPSCellChange;

3>	else:

4>	if available, set the locationInfo as in 5.3.3.7 according to the otherConfig associated with the PCell;

1>	release successPSCell-Config configured by the source PSCell if available and thresholdPercentageT304 if configured by the target PSCell.

The UE may discard the successful PSCell change or addition information, i.e., release the UE variable VarSuccessPSCell-Report, 48 hours after the last successful PSCell change or addition information is added to the VarSuccessPSCell-Report or upon deregistration from the network as specified in TS 23.502 [43].

NEXT CHANGE

### 6.2.2	Message definitions

#### –	LoggedMeasurementConfiguration

The LoggedMeasurementConfiguration message is used to perform logging of measurement results while in RRC_IDLE or RRC_INACTIVE. It is used to transfer the logged measurement configuration for network performance optimisation.

Signalling radio bearer: SRB1

RLC-SAP: AM

Logical channel: DCCH

Direction: Network to UE

LoggedMeasurementConfiguration message

-- ASN1START

-- TAG-LOGGEDMEASUREMENTCONFIGURATION-START

LoggedMeasurementConfiguration-r16 ::=  SEQUENCE {

criticalExtensions                      CHOICE {

loggedMeasurementConfiguration-r16      LoggedMeasurementConfiguration-r16-IEs,

criticalExtensionsFuture                SEQUENCE {}

}

}

LoggedMeasurementConfiguration-r16-IEs ::=  SEQUENCE {

traceReference-r16                          TraceReference-r16,

traceRecordingSessionRef-r16                OCTET STRING (SIZE (2)),

tce-Id-r16                                  OCTET STRING (SIZE (1)),

absoluteTimeInfo-r16                        AbsoluteTimeInfo-r16,

areaConfiguration-r16                       AreaConfiguration-r16                    OPTIONAL,  --Need R

plmn-IdentityList-r16                       PLMN-IdentityList2-r16                   OPTIONAL,  --Need R

bt-NameList-r16                             SetupRelease {BT-NameList-r16}           OPTIONAL,  --Need M

wlan-NameList-r16                           SetupRelease {WLAN-NameList-r16}         OPTIONAL,  --Need M

sensor-NameList-r16                         SetupRelease {Sensor-NameList-r16}       OPTIONAL,  --Need M

loggingDuration-r16                         LoggingDuration-r16,

reportType                                  CHOICE {

periodical                                  LoggedPeriodicalReportConfig-r16,

eventTriggered                              LoggedEventTriggerConfig-r16,

...

},

lateNonCriticalExtension                    OCTET STRING                             OPTIONAL,

nonCriticalExtension                        LoggedMeasurementConfiguration-v1700-IEs OPTIONAL

}

LoggedMeasurementConfiguration-v1700-IEs ::= SEQUENCE {

sigLoggedMeasType-r17                       ENUMERATED {true}                        OPTIONAL, -- Need R

earlyMeasIndication-r17                     ENUMERATED {true}                        OPTIONAL, -- Need R

areaConfiguration-r17                       AreaConfiguration-r17                    OPTIONAL,  --Need R

nonCriticalExtension                        LoggedMeasurementConfiguration-v1800-IEs OPTIONAL

}

LoggedMeasurementConfiguration-v1800-IEs ::= SEQUENCE {

areaConfiguration-v1800                     AreaConfiguration-v1800                  OPTIONAL,  --Need R

nonCriticalExtension                        LoggedMeasurementConfiguration-v1900-IEs OPTIONAL

}

LoggedMeasurementConfiguration-v1900-IEs ::= SEQUENCE {

intendedAreaScopeList-r19                   IntendedAreaScopeList-r19                   OPTIONAL,  -- Need R

nonCriticalExtension                        SEQUENCE {}                               OPTIONAL

}

LoggedPeriodicalReportConfig-r16 ::=            SEQUENCE {

loggingInterval-r16                             LoggingInterval-r16,

...

}

LoggedEventTriggerConfig-r16 ::=                SEQUENCE {

eventType-r16                                   EventType-r16,

loggingInterval-r16                             LoggingInterval-r16,

...

}

EventType-r16 ::= CHOICE {

outOfCoverage     NULL,

eventL1           SEQUENCE {

l1-Threshold      MeasTriggerQuantity,

hysteresis        Hysteresis,

timeToTrigger     TimeToTrigger

},

...

}

IntendedAreaScopeList-r19 ::=   SEQUENCE (SIZE (1..8)) OF IntendedAreaScopeInfo-r19

IntendedAreaScopeInfo-r19 ::=   SEQUENCE {

areaCoordinates-r19             CHOICE {

polygonArea                   OCTET STRING,

circleArea                    SEQUENCE {

referenceLocation-r19       ReferenceLocation-r17,

distanceRadius-r19          INTEGER(0..65535)

}

}

}

-- TAG-LOGGEDMEASUREMENTCONFIGURATION-STOP

-- ASN1STOP

LoggedMeasurementConfiguration field descriptions

absoluteTimeInfo Indicates the absolute time in the current cell.

areaConfiguration Used to restrict the area in which the UE performs measurement logging to cells broadcasting any of the included cell identities, the included tracking area codes/ frequencies, the included PNI-NPN identities or the SNPN identities. If areaConfiguration-r17 is present, the UE shall ignore areaConfiguration-r16. The areaConfiguration-v1800 is a non-critical extension of areaConfiguration-r17. See NOTE 1.

earlyMeasIndication If included, the field indicates the UE is allowed to log measurements on early measurement related frequencies in logged measurements.

eventType The value outOfCoverage indicates the UE to perform logging of measurements when the UE enters any cell selection state, and the value eventL1 indicates the UE to perform logging of measurements when the triggering condition (similar as event A2 as specified in 5.5.4.3) as configured in the event is met for the camping cell in camped normally state.

IntendedAreaScopeList Contains a list of intended area scope (additional geographical information) for NTN deployment.

plmn-IdentityList Indicates a set of PLMNs defining when the UE performs measurement logging as well as the associated status indication and information retrieval i.e. the UE performs these actions when the RPLMN is part of this set of PLMNs. The network does not include this field when the UE is configured with MDT configuration in SNPN access mode.

polygonArea Parameter type Polygon defined in TS 37.355 [49]. The first/leftmost bit of the first octet contains the most significant bit.

sigLoggedMeasType If included, the field indicates a signalling based logged measurement configuration (See TS 37.320 [61]).

tce-Id Parameter Trace Collection Entity Id: See TS 32.422 [52].

traceRecordingSessionRef Parameter Trace Recording Session Reference: See TS 32.422 [52].

reportType Parameter configures the type of MDT configuration, specifically Periodic MDT configuration or Event Triggerd MDT configuration.

NOTE 1:	The UE should perform measurement logging based on the following area configuration limitations:

-	If the areaConfiguration-r16/areaConfiguration-r17 is present, and the cag-ConfigList is absent, the UE should perform logging in both PN and PNI-NPN based on areaConfiguration-r16/areaConfiguration-r17, if any;

-	If the areaConfiguration-r17 and the cag-ConfigList are present simultaneously, the UE should perform logging in PN within the areaConfig-r16/areaConfig-r17 and perform logging in PNI-NPN within cag-ConfigList;

-	If the snpn-ConfigList is present, the UE should perform logging only in SNPN based on snpn-ConfigList. The snpn-ConfigList should not be configured together with PN or PNI-NPN area configurations.

Editor’s note: FFS on the coesistence of areaConfiguration-r18 and intendedAreaScopeList-r19.

NEXT CHANGE

#### –	SCGFailureInformation

The SCGFailureInformation message is used to provide information regarding NR SCG failures detected by the UE.

Signalling radio bearer: SRB1

RLC-SAP: AM

Logical channel: DCCH

Direction: UE to Network

SCGFailureInformation message

-- ASN1START

-- TAG-SCGFAILUREINFORMATION-START

SCGFailureInformation ::=                SEQUENCE {

criticalExtensions                       CHOICE {

scgFailureInformation                    SCGFailureInformation-IEs,

criticalExtensionsFuture                 SEQUENCE {}

}

}

SCGFailureInformation-IEs ::=            SEQUENCE {

failureReportSCG                         FailureReportSCG                    OPTIONAL,

nonCriticalExtension                     SCGFailureInformation-v1590-IEs     OPTIONAL

}

SCGFailureInformation-v1590-IEs ::=      SEQUENCE {

lateNonCriticalExtension                 OCTET STRING                        OPTIONAL,

nonCriticalExtension                     SEQUENCE {}                         OPTIONAL

}

FailureReportSCG ::=                     SEQUENCE {

failureType                              ENUMERATED {

t310-Expiry, randomAccessProblem,

rlc-MaxNumRetx,

synchReconfigFailureSCG, scg-ReconfigFailure,

srb3-IntegrityFailure, other-r16, spare1},

measResultFreqList                       MeasResultFreqList                                    OPTIONAL,

measResultSCG-Failure                    OCTET STRING (CONTAINING MeasResultSCG-Failure)       OPTIONAL,

...,

[[

locationInfo-r16                         LocationInfo-r16                                      OPTIONAL,

failureType-v1610                        ENUMERATED {scg-lbtFailure-r16, beamFailureRecoveryFailure-r16,

t312-Expiry-r16, bh-RLF-r16, beamFailure-r17, spare3, spare2, spare1} OPTIONAL

]],

[[

previousPSCellId-r17                 SEQUENCE {

physCellId-r17                       PhysCellId,

carrierFreq-r17                      ARFCN-ValueNR

}                                                             OPTIONAL,

failedPSCellId-r17                   SEQUENCE {

physCellId-r17                       PhysCellId,

carrierFreq-r17                      ARFCN-ValueNR

}                                                            OPTIONAL,

timeSCGFailure-r17                   INTEGER (0..1023)        OPTIONAL,

perRAInfoList-r17                    PerRAInfoList-r16          OPTIONAL

]],

[[

perRAInfoList-v17b0                  PerRAInfoList-v1660      OPTIONAL

]],

[[

perRAInfoList-v1840                  PerRAInfoList-v1800      OPTIONAL

]],

[[

choWithCandidateSCGInfoList-r19::=       SEQUENCE (SIZE (1..maxNrofCondCells-r16)) OF ChoWithCandidateSCGInfo-r19        OPTIONAL

]]

}

MeasResultFreqList ::=                   SEQUENCE (SIZE (1..maxFreq)) OF MeasResult2NR

-- TAG-SCGFAILUREINFORMATION-STOP

-- ASN1STOP

SCGFailureInformation field descriptions

measResultFreqList The field contains available results of measurements on NR frequencies the UE is configured to measure by measConfig.

measResultSCG-Failure The field contains the MeasResultSCG-Failure IE which includes available results of measurements on NR frequencies the UE is configured to measure by the NR SCG RRCReconfiguration message.

previousPSCellId This field indicates the physical cell id and carrier frequency of the cell that is the source PSCell of the last PSCell change. In case of PSCell addition failure, this field is absent.

failedPSCellId This field indicates the physical cell id and carrier frequency of the cell in which SCG failure is detected or the target PSCell of the failed PSCell change or failed PSCell addition.

timeSCGFailure This field is used to indicate the time elapsed since the last execution of RRCReconfiguration with reconfigurationWithSync for the SCG until the SCG failure. Actual value = field value * 100ms. The maximum value 1023 means 102.3s or longer.

NEXT CHANGE

#### –	UEInformationRequest

The UEInformationRequest message is used by the network to retrieve information from the UE.

Signalling radio bearer: SRB1

RLC-SAP: AM

Logical channel: DCCH

Direction: Network to UE

UEInformationRequest message

-- ASN1START

-- TAG-UEINFORMATIONREQUEST-START

UEInformationRequest-r16 ::=     SEQUENCE {

rrc-TransactionIdentifier        RRC-TransactionIdentifier,

criticalExtensions               CHOICE {

ueInformationRequest-r16         UEInformationRequest-r16-IEs,

criticalExtensionsFuture         SEQUENCE {}

}

}

UEInformationRequest-r16-IEs ::= SEQUENCE {

idleModeMeasurementReq-r16       ENUMERATED{true}                         OPTIONAL, -- Need N

logMeasReportReq-r16             ENUMERATED {true}                        OPTIONAL, -- Need N

connEstFailReportReq-r16         ENUMERATED {true}                        OPTIONAL, -- Need N

ra-ReportReq-r16                 ENUMERATED {true}                        OPTIONAL, -- Need N

rlf-ReportReq-r16                ENUMERATED {true}                        OPTIONAL, -- Need N

mobilityHistoryReportReq-r16       ENUMERATED {true}                        OPTIONAL, -- Need N

lateNonCriticalExtension         OCTET STRING                             OPTIONAL,

nonCriticalExtension             UEInformationRequest-v1700-IEs           OPTIONAL

}

UEInformationRequest-v1700-IEs ::= SEQUENCE {

successHO-ReportReq-r17          ENUMERATED {true}                        OPTIONAL, -- Need N

coarseLocationRequest-r17        ENUMERATED {true}                        OPTIONAL, -- Need N

nonCriticalExtension             UEInformationRequest-v1800-IEs           OPTIONAL

}

UEInformationRequest-v1800-IEs ::= SEQUENCE {

flightPathInfoReq-r18            FlightPathInfoReportConfig-r18           OPTIONAL, -- Need N

successPSCell-ReportReq-r18      ENUMERATED {true}                        OPTIONAL, -- Need N

reselectionMeasurementReq-r18    ENUMERATED {true}                        OPTIONAL, -- Need N

validatedMeasurementsReq-r18     ENUMERATED {true}                        OPTIONAL, -- Need N

nonCriticalExtension             SEQUENCE {}                              OPTIONAL

}

FlightPathInfoReportConfig-r18 ::= SEQUENCE {

maxWayPointNumber-r18             INTEGER (1..maxWayPoint-r18),

includeTimeStamp-r18              ENUMERATED {true}                       OPTIONAL  -- Need N

}

-- TAG-UEINFORMATIONREQUEST-STOP

-- ASN1STOP

UEInformationRequest-IEs field descriptions

coarseLocationRequest This field is used to request UE to report coarse location information.

connEstFailReportReq This field is used to indicate whether the UE shall report information about the connection failure.

flightPathInfoReq This field is used to indicate whether the UE shall report the flight path information, if available, and to specify the flight path information report configuration.

idleModeMeasurementReq This field indicates that the UE shall report the idle/inactive measurement information, if available, to the network in the UEInformationResponse message.

logMeasReportReq This field is used to indicate whether the UE shall report information about logged measurements.

mobilityHistoryReportReq This field is used to indicate whether the UE shall report information about mobility history information.

ra-ReportReq This field is used to indicate whether the UE shall report information about the random access procedure.

reselectionMeasurementReq This field indicates that the UE shall report the reselection measurement information, if available, to the network in the UEInformationResponse message.

rlf-ReportReq This field is used to indicate whether the UE shall report information about the radio link failure.

successHO-ReportReq This field is used to indicate whether the UE shall report information about the successful handover report.

successPSCell-ReportReq This field is used to indicate whether the UE shall report information about the successful PSCell change or addition report.

FlightPathInfoReportConfig field descriptions

includeTimeStamp Indicates whether time stamp of each way point can be reported in the flight path information report if time stamp information is available at the UE.

maxWayPointNumber Indicates the maximum number of way points UE can include in the flight path information report if this information is available at the UE.

#### –	UEInformationResponse

The UEInformationResponse message is used by the UE to transfer information requested by the network.

Signalling radio bearer: SRB1 or SRB2 (when logged measurement information is included)

RLC-SAP: AM

Logical channel: DCCH

Direction: UE to network

UEInformationResponse message

-- ASN1START

-- TAG-UEINFORMATIONRESPONSE-START

UEInformationResponse-r16 ::=        SEQUENCE {

rrc-TransactionIdentifier            RRC-TransactionIdentifier,

criticalExtensions                   CHOICE {

ueInformationResponse-r16            UEInformationResponse-r16-IEs,

criticalExtensionsFuture             SEQUENCE {}

}

}

UEInformationResponse-r16-IEs ::=    SEQUENCE {

measResultIdleEUTRA-r16              MeasResultIdleEUTRA-r16             OPTIONAL,

measResultIdleNR-r16                 MeasResultIdleNR-r16                OPTIONAL,

logMeasReport-r16                    LogMeasReport-r16                   OPTIONAL,

connEstFailReport-r16                ConnEstFailReport-r16               OPTIONAL,

ra-ReportList-r16                    RA-ReportList-r16                   OPTIONAL,

rlf-Report-r16                       RLF-Report-r16                      OPTIONAL,

mobilityHistoryReport-r16            MobilityHistoryReport-r16           OPTIONAL,

lateNonCriticalExtension             OCTET STRING                        OPTIONAL,

nonCriticalExtension                 UEInformationResponse-v1700-IEs     OPTIONAL

}

UEInformationResponse-v1700-IEs ::=  SEQUENCE {

successHO-Report-r17                 SuccessHO-Report-r17                OPTIONAL,

connEstFailReportList-r17            ConnEstFailReportList-r17           OPTIONAL,

coarseLocationInfo-r17               OCTET STRING                        OPTIONAL,

nonCriticalExtension                 UEInformationResponse-v1800-IEs     OPTIONAL

}

UEInformationResponse-v1800-IEs ::=  SEQUENCE {

flightPathInfoReport-r18             FlightPathInfoReport-r18            OPTIONAL,

successPSCell-Report-r18             SuccessPSCell-Report-r18            OPTIONAL,

measResultReselectionNR-r18          MeasResultIdleNR-r16                OPTIONAL,

nonCriticalExtension                 SEQUENCE {}                         OPTIONAL

}

FlightPathInfoReport-r18 ::=         SEQUENCE (SIZE (0..maxWayPoint-r18)) OF WayPoint-r18

WayPoint-r18 ::=                     SEQUENCE {

wayPointLocation-r18                 OCTET STRING,

timeStamp-r18                        AbsoluteTimeInfo-r16                OPTIONAL

}

LogMeasReport-r16 ::=                SEQUENCE {

absoluteTimeStamp-r16                AbsoluteTimeInfo-r16,

traceReference-r16                   TraceReference-r16,

traceRecordingSessionRef-r16         OCTET STRING (SIZE (2)),

tce-Id-r16                           OCTET STRING (SIZE (1)),

logMeasInfoList-r16                  LogMeasInfoList-r16,

logMeasAvailable-r16                 ENUMERATED {true}                   OPTIONAL,

logMeasAvailableBT-r16               ENUMERATED {true}                   OPTIONAL,

logMeasAvailableWLAN-r16             ENUMERATED {true}                   OPTIONAL,

...

}

LogMeasInfoList-r16 ::=              SEQUENCE (SIZE (1..maxLogMeasReport-r16)) OF LogMeasInfo-r16

LogMeasInfo-r16 ::=                  SEQUENCE {

locationInfo-r16                     LocationInfo-r16                    OPTIONAL,

relativeTimeStamp-r16                INTEGER (0..7200),

servCellIdentity-r16                 CGI-Info-Logging-r16                OPTIONAL,

measResultServingCell-r16            MeasResultServingCell-r16           OPTIONAL,

measResultNeighCells-r16             SEQUENCE {

measResultNeighCellListNR            MeasResultListLogging2NR-r16    OPTIONAL,

measResultNeighCellListEUTRA         MeasResultList2EUTRA-r16        OPTIONAL

},

anyCellSelectionDetected-r16         ENUMERATED {true}                   OPTIONAL,

...,

[[

inDeviceCoexDetected-r17             ENUMERATED {true}                   OPTIONAL

]]

}

ConnEstFailReport-r16 ::=            SEQUENCE {

measResultFailedCell-r16             MeasResultFailedCell-r16,

locationInfo-r16                     LocationInfo-r16                    OPTIONAL,

measResultNeighCells-r16             SEQUENCE {

measResultNeighCellListNR            MeasResultList2NR-r16               OPTIONAL,

measResultNeighCellListEUTRA         MeasResultList2EUTRA-r16            OPTIONAL

},

numberOfConnFail-r16                 INTEGER (1..8),

perRAInfoList-r16                            PerRAInfoList-r16,

timeSinceFailure-r16                 TimeSinceFailure-r16,

...

}

ConnEstFailReportList-r17 ::= SEQUENCE (SIZE (1..maxCEFReport-r17)) OF ConnEstFailReport-r16

MeasResultServingCell-r16 ::=        SEQUENCE {

resultsSSB-Cell                      MeasQuantityResults,

resultsSSB                           SEQUENCE{

best-ssb-Index                       SSB-Index,

best-ssb-Results                     MeasQuantityResults,

numberOfGoodSSB                      INTEGER (1..maxNrofSSBs-r16)

}                                                                        OPTIONAL

}

MeasResultFailedCell-r16 ::=         SEQUENCE {

cgi-Info                             CGI-Info-Logging-r16,

measResult-r16                       SEQUENCE {

cellResults-r16                      SEQUENCE{

resultsSSB-Cell-r16                  MeasQuantityResults

},

rsIndexResults-r16                   SEQUENCE{

resultsSSB-Indexes-r16               ResultsPerSSB-IndexList

}

}

}

RA-ReportList-r16 ::= SEQUENCE (SIZE (1..maxRAReport-r16)) OF RA-Report-r16

RA-Report-r16 ::=                    SEQUENCE {

cellId-r16                           CHOICE {

cellGlobalId-r16                     CGI-Info-Logging-r16,

pci-arfcn-r16                        PCI-ARFCN-NR-r16

},

ra-InformationCommon-r16             RA-InformationCommon-r16                         OPTIONAL,

raPurpose-r16                        ENUMERATED {accessRelated, beamFailureRecovery, reconfigurationWithSync, ulUnSynchronized,

schedulingRequestFailure, noPUCCHResourceAvailable, requestForOtherSI,

msg3RequestForOtherSI-r17, lbt-Failure-r18, ltm-r19, spare6, spare5, spare4, spare3,

spare2, spare1},

...,

[[

spCellID-r17                         CGI-Info-Logging-r16                             OPTIONAL

]],

[[

sdt-Failed-r18                       ENUMERATED {true}                                OPTIONAL

]],

[[

sdt-FailureCause-r19                 ENUMERATED {t319a-expiry, maxRetxThreshold, preambleTransMax, configuredGrantTimer, cg-SDT-TimeAlignmentTimer, cellReselection, spare2, spare1}                                       OPTIONAL,

sdt-DL-RsrpInfo-r19                  RSRP-Range                                       OPTIONAL,

sdt-UL-DataVolume-r19                INTEGER (0..96000)                                OPTIONAL,

timeSinceSdt-Execution-r19           TimeSinceSdt-Execution-r19                             OPTIONAL

]]

}

RA-InformationCommon-r16 ::=         SEQUENCE {

absoluteFrequencyPointA-r16          ARFCN-ValueNR,

locationAndBandwidth-r16             INTEGER (0..37949),

subcarrierSpacing-r16                SubcarrierSpacing,

msg1-FrequencyStart-r16              INTEGER (0..maxNrofPhysicalResourceBlocks-1)     OPTIONAL,

msg1-FrequencyStartCFRA-r16          INTEGER (0..maxNrofPhysicalResourceBlocks-1)     OPTIONAL,

msg1-SubcarrierSpacing-r16           SubcarrierSpacing                                OPTIONAL,

msg1-SubcarrierSpacingCFRA-r16       SubcarrierSpacing                                OPTIONAL,

msg1-FDM-r16                         ENUMERATED {one, two, four, eight}               OPTIONAL,

msg1-FDMCFRA-r16                     ENUMERATED {one, two, four, eight}               OPTIONAL,

perRAInfoList-r16                    PerRAInfoList-r16,

...,

[[

perRAInfoList-v1660                  PerRAInfoList-v1660                              OPTIONAL

]],

[[

msg1-SCS-From-prach-ConfigurationIndex-r16  ENUMERATED {kHz1dot25, kHz5, spare2, spare1}  OPTIONAL

]],

[[

msg1-SCS-From-prach-ConfigurationIndexCFRA-r16  ENUMERATED {kHz1dot25, kHz5, spare2, spare1} OPTIONAL

]],

[[

msgA-RO-FrequencyStart-r17           INTEGER (0..maxNrofPhysicalResourceBlocks-1)     OPTIONAL,

msgA-RO-FrequencyStartCFRA-r17       INTEGER (0..maxNrofPhysicalResourceBlocks-1)     OPTIONAL,

msgA-SubcarrierSpacing-r17           SubcarrierSpacing                                OPTIONAL,

msgA-RO-FDM-r17                      ENUMERATED {one, two, four, eight}               OPTIONAL,

msgA-RO-FDMCFRA-r17                  ENUMERATED {one, two, four, eight}               OPTIONAL,

msgA-SCS-From-prach-ConfigurationIndex-r17  ENUMERATED {kHz1dot25, kHz5, spare2, spare1}  OPTIONAL,

msgA-TransMax-r17                    ENUMERATED {n1, n2, n4, n6, n8, n10, n20, n50, n100, n200}  OPTIONAL,

msgA-MCS-r17                         INTEGER (0..15)                                   OPTIONAL,

nrofPRBs-PerMsgA-PO-r17              INTEGER (1..32)                                  OPTIONAL,

msgA-PUSCH-TimeDomainAllocation-r17  INTEGER (1..maxNrofUL-Allocations)               OPTIONAL,

frequencyStartMsgA-PUSCH-r17         INTEGER (0..maxNrofPhysicalResourceBlocks-1)     OPTIONAL,

nrofMsgA-PO-FDM-r17                  ENUMERATED {one, two, four, eight}               OPTIONAL,

dlPathlossRSRP-r17                   RSRP-Range                                       OPTIONAL,

intendedSIBs-r17                     SEQUENCE (SIZE (1..maxSIB)) OF SIB-Type-r17      OPTIONAL,

ssbsForSI-Acquisition-r17            SEQUENCE (SIZE (1..maxNrofSSBs-r16)) OF SSB-Index    OPTIONAL,

msgA-PUSCH-PayloadSize-r17           BIT STRING (SIZE (5))                            OPTIONAL,

onDemandSISuccess-r17                ENUMERATED {true}                                OPTIONAL

]],

[[

usedFeatureCombination-r18           ReportedFeatureCombination-r18                   OPTIONAL,

triggeredFeatureCombination-r18      ReportedFeatureCombination-r18                   OPTIONAL,

startPreambleForThisPartition-r18    INTEGER (0..63)                                  OPTIONAL,

numberOfPreamblesPerSSB-ForThisPartition-r18  INTEGER (1..64)                         OPTIONAL,

attemptedBWP-InfoList-r18            SEQUENCE (SIZE (1..maxNrofBWPs)) OF AttemptedBWP-Info-r18  OPTIONAL,

numberOfLBT-Failures-r18             INTEGER (1..128)                                 OPTIONAL,

perRAInfoList-v1800                  PerRAInfoList-v1800                              OPTIONAL,

intendedSIBs-r18                     SEQUENCE (SIZE (1..maxSIB)) OF SIB-Type-r18      OPTIONAL

]]

}

AttemptedBWP-Info-r18 ::=            SEQUENCE {

locationAndBandwidth-r18             INTEGER (0..37949),

subcarrierSpacing-r18                SubcarrierSpacing

}

ReportedFeatureCombination-r18 ::=   SEQUENCE {

redCap-r18                           ENUMERATED {true}                                OPTIONAL,

smallData-r18                        ENUMERATED {true}                                OPTIONAL,

nsag-r18                             NSAG-List-r17                                    OPTIONAL,

msg3-Repetitions-r18                 ENUMERATED {true}                                OPTIONAL,

msg1-Repetitions-r18                 ENUMERATED {true}                                OPTIONAL,

eRedCap-r18                          ENUMERATED {true}                                OPTIONAL,

triggered-S-NSSAI-List-r18           SEQUENCE (SIZE (1..maxNrofS-NSSAI)) OF S-NSSAI   OPTIONAL

}

PerRAInfoList-r16 ::= SEQUENCE (SIZE (1..200)) OF PerRAInfo-r16

PerRAInfoList-v1660 ::= SEQUENCE (SIZE (1..200)) OF PerRACSI-RSInfo-v1660

PerRAInfo-r16 ::=                    CHOICE {

perRASSBInfoList-r16                 PerRASSBInfo-r16,

perRACSI-RSInfoList-r16              PerRACSI-RSInfo-r16

}

PerRAInfoList-v1800 ::= SEQUENCE (SIZE (1..200)) OF PerRAInfo-v1800

PerRAInfo-v1800 ::=                  CHOICE {

perRASSBInfoList-v1800               PerRASSBInfo-v1800,

perRACSI-RSInfoList-v1800            PerRACSI-RSInfo-v1800

}

PerRASSBInfo-r16 ::=                 SEQUENCE {

ssb-Index-r16                        SSB-Index,

numberOfPreamblesSentOnSSB-r16       INTEGER (1..200),

perRAAttemptInfoList-r16             PerRAAttemptInfoList-r16

}

PerRASSBInfo-v1800 ::=               SEQUENCE {

allPreamblesBlocked                  ENUMERATED {true}                                OPTIONAL,

lbt-Detected-r18                     ENUMERATED {true}                                OPTIONAL,

...

}

PerRACSI-RSInfo-r16 ::=              SEQUENCE {

csi-RS-Index-r16                     CSI-RS-Index,

numberOfPreamblesSentOnCSI-RS-r16    INTEGER (1..200)

}

PerRACSI-RSInfo-v1660 ::=            SEQUENCE {

csi-RS-Index-v1660                   INTEGER (1..96)                                  OPTIONAL

}

PerRACSI-RSInfo-v1800 ::=            SEQUENCE {

allPreamblesBlocked                  ENUMERATED {true}                                OPTIONAL,

lbt-Detected-r18                     ENUMERATED {true}                                OPTIONAL,

...

}

PerRAAttemptInfoList-r16 ::=         SEQUENCE (SIZE (1..200)) OF PerRAAttemptInfo-r16

PerRAAttemptInfo-r16 ::=             SEQUENCE {

contentionDetected-r16               BOOLEAN                OPTIONAL,

dlRSRPAboveThreshold-r16             BOOLEAN                OPTIONAL,

...,

[[

fallbackToFourStepRA-r17             ENUMERATED {true}      OPTIONAL

]]

}

SIB-Type-r17 ::= ENUMERATED {sibType2, sibType3, sibType4, sibType5, sibType9, sibType10, sibType11, sibType12,

sibType13, sibType14, posSIB-v1810, spare5, spare4, spare3, spare2, spare1}

SIB-Type-r18 ::= ENUMERATED {sibType15, sibType16, sibType17, sibType18, sibType19, sibType20,

sibType21, sibType22, sibType23, sibType24, sibType25, spare5, spare4,

spare3, spare2, spare1}

RLF-Report-r16 ::=                   CHOICE {

nr-RLF-Report-r16                    SEQUENCE {

measResultLastServCell-r16           MeasResultRLFNR-r16,

measResultNeighCells-r16             SEQUENCE {

measResultListNR-r16                 MeasResultList2NR-r16       OPTIONAL,

measResultListEUTRA-r16              MeasResultList2EUTRA-r16    OPTIONAL

}                                                OPTIONAL,

c-RNTI-r16                           RNTI-Value,

previousPCellId-r16                  CHOICE {

nrPreviousCell-r16                   CGI-Info-Logging-r16,

eutraPreviousCell-r16                CGI-InfoEUTRALogging

}                                                                    OPTIONAL,

failedPCellId-r16                    CHOICE {

nrFailedPCellId-r16                  CHOICE {

cellGlobalId-r16                     CGI-Info-Logging-r16,

pci-arfcn-r16                        PCI-ARFCN-NR-r16

},

eutraFailedPCellId-r16           CHOICE {

cellGlobalId-r16                 CGI-InfoEUTRALogging,

pci-arfcn-r16                    PCI-ARFCN-EUTRA-r16

}

},

reconnectCellId-r16                  CHOICE {

nrReconnectCellId-r16                CGI-Info-Logging-r16,

eutraReconnectCellId-r16             CGI-InfoEUTRALogging

}                                                                                        OPTIONAL,

timeUntilReconnection-r16            TimeUntilReconnection-r16                           OPTIONAL,

reestablishmentCellId-r16            CGI-Info-Logging-r16                                OPTIONAL,

timeConnFailure-r16                  INTEGER (0..1023)                                   OPTIONAL,

timeSinceFailure-r16                 TimeSinceFailure-r16,

connectionFailureType-r16            ENUMERATED {rlf, hof},

rlf-Cause-r16                        ENUMERATED {t310-Expiry, randomAccessProblem, rlc-MaxNumRetx,

beamFailureRecoveryFailure, lbtFailure-r16,

bh-rlfRecoveryFailure, t312-expiry-r17, spare1},

locationInfo-r16                     LocationInfo-r16                                    OPTIONAL,

noSuitableCellFound-r16              ENUMERATED {true}                                   OPTIONAL,

ra-InformationCommon-r16             RA-InformationCommon-r16                            OPTIONAL,

...,

[[

csi-rsRLMConfigBitmap-v1650          BIT STRING (SIZE (96))                              OPTIONAL

]],

[[

lastHO-Type-r17                      ENUMERATED {cho, daps, ltm, choWithCandidateSCG}    OPTIONAL,

timeConnSourceDAPS-Failure-r17       TimeConnSourceDAPS-Failure-r17                      OPTIONAL,

timeSinceCHO-Reconfig-r17            TimeSinceCHO-Reconfig-r17                           OPTIONAL,

choCellId-r17                        CHOICE {

cellGlobalId-r17                     CGI-Info-Logging-r16,

pci-arfcn-r17                        PCI-ARFCN-NR-r16

}                                                                                        OPTIONAL,

choCandidateCellList-r17             ChoCandidateCellList-r17                            OPTIONAL

]],

[[

pSCellId-r18                         CHOICE {

cellGlobalId-r18                     CGI-Info-Logging-r16,

pci-arfcn-r18                        PCI-ARFCN-NR-r16

}                                                                                        OPTIONAL,

mcg-RecoveryFailureCause-r18         ENUMERATED {t316-Expiry, scg-Deactivated, spare2, spare1}  OPTIONAL,

scg-FailureCause-r18                 ENUMERATED {t310-Expiry, randomAccessProblem, rlc-MaxNumRetx,

synchReconfigFailureSCG, scg-ReconfigFailure,

srb3-IntegrityFailure, scg-lbtFailure, beamFailureRecoveryFailure,

t312-Expiry, bh-RLF, beamFailure, spare5, spare4, spare3, spare2, spare1 }

OPTIONAL,

elapsedTimeSCG-Failure-r18           ElapsedTimeSCG-Failure-r18                          OPTIONAL,

voiceFallbackHO-r18                  ENUMERATED {true}                                   OPTIONAL,

measResultLastServCellRSSI-r18     RSSI-Range-r16                                      OPTIONAL,

measResultNeighFreqListRSSI-r18    MeasResultNeighFreqListRSSI-r18                     OPTIONAL,

bwp-Info-r18                         AttemptedBWP-Info-r18                               OPTIONAL,

elapsedTimeT316-r18                  ElapsedTimeT316-r18                                 OPTIONAL,

scg-FailedAfterMCG-r18               ENUMERATED {true}                                   OPTIONAL
        ]],

[[

measResultL1-LastServCell-r19         MeasResultL1-r19,                                    OPTIONAL,

measResultL1-NeighCells-r19           MeasResultList3NR-r19                                OPTIONAL,

ltm-RecoveryCellId-r19                CHOICE {

cellGlobalId-r19                     CGI-Info-Logging-r16,

pci-arfcn-r19                        PCI-ARFCN-NR-r16

}                                                                                           OPTIONAL,

measResultLastServPSCell-r19          MeasResultRLFNR-r16                                   OPTIONAL,

choWithCandidateSCGInfoList-r19::=      SEQUENCE (SIZE (1..maxNrofCondCells-r16)) OF ChoWithCandidateSCGInfo-r19  OPTIONAL

]]

},

eutra-RLF-Report-r16                 SEQUENCE {

failedPCellId-EUTRA                  CGI-InfoEUTRALogging,

measResult-RLF-Report-EUTRA-r16      OCTET STRING,

...,

[[

measResult-RLF-Report-EUTRA-v1690    OCTET STRING                                        OPTIONAL

]]

}

}

SuccessHO-Report-r17 ::=                 SEQUENCE {

sourceCellInfo-r17                       SEQUENCE {

sourcePCellId-r17                        CGI-Info-Logging-r16,

sourceCellMeas-r17                       MeasResultSuccessHONR-r17                       OPTIONAL,

rlf-InSourceDAPS-r17                     ENUMERATED {true}                               OPTIONAL

},

targetCellInfo-r17                       SEQUENCE {

targetPCellId-r17                        CGI-Info-Logging-r16,

targetCellMeas-r17                       MeasResultSuccessHONR-r17                       OPTIONAL

},

measResultNeighCells-r17                 SEQUENCE {

measResultListNR-r17                     MeasResultList2NR-r16                           OPTIONAL,

measResultListEUTRA-r17                  MeasResultList2EUTRA-r16                        OPTIONAL

}                                                                                            OPTIONAL,

locationInfo-r17                         LocationInfo-r16                                    OPTIONAL,

timeSinceCHO-Reconfig-r17                TimeSinceCHO-Reconfig-r17                           OPTIONAL,

shr-Cause-r17                            SHR-Cause-r17                                       OPTIONAL,

ra-InformationCommon-r17                 RA-InformationCommon-r16                            OPTIONAL,

upInterruptionTimeAtHO-r17               UPInterruptionTimeAtHO-r17                          OPTIONAL,

c-RNTI-r17                               RNTI-Value                                          OPTIONAL,

...,

[[

targetCell-PCI-ARFCN-r17                 PCI-ARFCN-NR-r16                                    OPTIONAL

]],

[[

eutra-TargetCellInfo-r18                 SEQUENCE {

targetPCellId-r18                        CHOICE {

cellGlobalId-r18                         CGI-Info-Logging-r16,

pci-arfcn-r18                            PCI-ARFCN-EUTRA-r16

},

targetCellMeas-r18                       MeasQuantityResultsEUTRA                       OPTIONAL

}                                                                                           OPTIONAL,

measResultServCellRSSI-r18                   RSSI-Range-r16                                 OPTIONAL,

measResultNeighFreqListRSSI-r18              MeasResultNeighFreqListRSSI-r18                OPTIONAL,

eutra-C-RNTI-r18                             EUTRA-C-RNTI                                   OPTIONAL,

timeSinceSHR-r18                             TimeSinceSHR-r18                               OPTIONAL

]],

[[

sourceCellMeasL1-r19                         MeasResultL1-r19                               OPTIONAL,

targetCellMeasL1-r19                         MeasResultL1-r19                               OPTIONAL,

neighCellsMeasL1ListNR-r19                   MeasResultList3NR-r19                          OPTIONAL,

rach-Less-r19                                ENUMERATED {true}                              OPTIONAL,

sourcePSCellInfo-r19                       SEQUENCE {

sourcePSCellId-r19                        CGI-Info-Logging-r16,

sourcePSCellMeas-r19                      MeasResultSuccessHONR-r17                       OPTIONAL

}                                                                                             OPTIONAL,

choWithCandidateSCGInfoList-r19::=             SEQUENCE (SIZE (1..maxNrofCondCells-r16)) OF ChoWithCandidateSCGInfo-r19  OPTIONAL,

targetPSCellID-19 ::=                     CHOICE {

cellGlobalId-r19                       CGI-Info-Logging-r16,

pci-arfcn-r19                          PCI-ARFCN-NR-r16

}                                                                                            OPTIONAL

]]

}

SuccessPSCell-Report-r18 ::=             SEQUENCE {

pCellId-r18                              CGI-Info-Logging-r16,

sourcePSCellInfo-r18                     SEQUENCE {

sourcePSCellId-r18                       CHOICE {

cellGlobalId-r18                         CGI-Info-Logging-r16,

pci-arfcn-r18                            PCI-ARFCN-EUTRA-r16

},

sourcePSCellMeas-r18                     MeasResultSuccessHONR-r17                       OPTIONAL

}                                                                                            OPTIONAL,

targetPSCellInfo-r18                     SEQUENCE {

targetPSCellId-r18                       CHOICE {

cellGlobalId-r18                     CGI-Info-Logging-r16,

pci-arfcn-r18                        PCI-ARFCN-NR-r16

},

targetPSCellMeas-r18                     MeasResultSuccessHONR-r17                       OPTIONAL

},

measResultNeighCells-r18                 SEQUENCE {

measResultListNR-r18                     MeasResultList2NR-r16                           OPTIONAL,

measResultListEUTRA-r18                  MeasResultList2EUTRA-r16                        OPTIONAL

}                                                                                            OPTIONAL,

spr-Cause-r18                            SPR-Cause-r18                                       OPTIONAL,

timeSinceCPAC-Reconfig-r18               TimeSinceCPAC-Reconfig-r18                          OPTIONAL,

locationInfo-r18                         LocationInfo-r16                                    OPTIONAL,

ra-InformationCommon-r18                 RA-InformationCommon-r16                            OPTIONAL,

sn-InitiatedPSCellChange-r18             ENUMERATED {true}                                   OPTIONAL,

...,

[[

choWithCandidateSCGInfoList-r19::=             SEQUENCE (SIZE (1..maxNrofCondCells-r16)) OF ChoWithCandidateSCGInfo-r19  OPTIONAL,

targetPCellID-19::=                      CHOICE {

cellGlobalId-r19                       CGI-Info-Logging-r16,

pci-arfcn-r19                          PCI-ARFCN-NR-r16

}                                                                                            OPTIONAL

]]

}

MeasResultNeighFreqListRSSI-r18 ::=      SEQUENCE(SIZE (1..maxFreq)) OF MeasResultNeighFreqRSSI-r18

MeasResultNeighFreqRSSI-r18 ::=          SEQUENCE {

ssbFrequency-r18                         ARFCN-ValueNR                                       OPTIONAL,

ssbSubcarrierSpacing-r18                 SubcarrierSpacing                                   OPTIONAL,

refFreqCSI-RS-r18                        ARFCN-ValueNR                                       OPTIONAL,

measResult-RSSI-r18                      RSSI-Range-r16                                      OPTIONAL

}

MeasResultList2NR-r16 ::=            SEQUENCE(SIZE (1..maxFreq)) OF MeasResult2NR-r16

MeasResultList2EUTRA-r16 ::=         SEQUENCE(SIZE (1..maxFreq)) OF MeasResult2EUTRA-r16

MeasResult2NR-r16 ::=                SEQUENCE {

ssbFrequency-r16                     ARFCN-ValueNR                                           OPTIONAL,

refFreqCSI-RS-r16                    ARFCN-ValueNR                                           OPTIONAL,

measResultList-r16                   MeasResultListNR

}

MeasResultListLogging2NR-r16 ::=     SEQUENCE(SIZE (1..maxFreq)) OF MeasResultLogging2NR-r16

MeasResultLogging2NR-r16 ::=         SEQUENCE {

carrierFreq-r16                      ARFCN-ValueNR,

measResultListLoggingNR-r16          MeasResultListLoggingNR-r16

}

MeasResultListLoggingNR-r16 ::=      SEQUENCE (SIZE (1..maxCellReport)) OF MeasResultLoggingNR-r16

MeasResultLoggingNR-r16 ::=          SEQUENCE {

physCellId-r16                       PhysCellId,

resultsSSB-Cell-r16                  MeasQuantityResults,

numberOfGoodSSB-r16                  INTEGER (1..maxNrofSSBs-r16) OPTIONAL

}

MeasResult2EUTRA-r16 ::=             SEQUENCE {

carrierFreq-r16                      ARFCN-ValueEUTRA,

measResultList-r16                   MeasResultListEUTRA

}

MeasResultRLFNR-r16 ::=              SEQUENCE {

measResult-r16                       SEQUENCE {

cellResults-r16                      SEQUENCE{

resultsSSB-Cell-r16                  MeasQuantityResults                             OPTIONAL,

resultsCSI-RS-Cell-r16               MeasQuantityResults                             OPTIONAL

},

rsIndexResults-r16                   SEQUENCE{

resultsSSB-Indexes-r16               ResultsPerSSB-IndexList                         OPTIONAL,

ssbRLMConfigBitmap-r16               BIT STRING (SIZE (64))                          OPTIONAL,

resultsCSI-RS-Indexes-r16            ResultsPerCSI-RS-IndexList                      OPTIONAL,

csi-rsRLMConfigBitmap-r16            BIT STRING (SIZE (96))                          OPTIONAL

}                                                                                    OPTIONAL

}

}

MeasResultSuccessHONR-r17::=         SEQUENCE {

measResult-r17                       SEQUENCE {

cellResults-r17                      SEQUENCE{

resultsSSB-Cell-r17                  MeasQuantityResults                             OPTIONAL,

resultsCSI-RS-Cell-r17               MeasQuantityResults                             OPTIONAL

},

rsIndexResults-r17                   SEQUENCE{

resultsSSB-Indexes-r17               ResultsPerSSB-IndexList                         OPTIONAL,

resultsCSI-RS-Indexes-r17            ResultsPerCSI-RS-IndexList                      OPTIONAL

}

}

}

ChoCandidateCellList-r17 ::=         SEQUENCE(SIZE (1..maxNrofCondCells-r16)) OF ChoCandidateCell-r17

ChoCandidateCell-r17 ::=             CHOICE {

cellGlobalId-r17                     CGI-Info-Logging-r16,

pci-arfcn-r17                        PCI-ARFCN-NR-r16

}

SHR-Cause-r17 ::=                    SEQUENCE {

t304-cause-r17                       ENUMERATED {true}                                       OPTIONAL,

t310-cause-r17                       ENUMERATED {true}                                       OPTIONAL,

t312-cause-r17                       ENUMERATED {true}                                       OPTIONAL,

sourceDAPS-Failure-r17               ENUMERATED {true}                                       OPTIONAL,

...

}

SPR-Cause-r18 ::=                    SEQUENCE {

t304-cause-r18                       ENUMERATED {true}                                       OPTIONAL,

t310-cause-r18                       ENUMERATED {true}                                       OPTIONAL,

t312-cause-r18                       ENUMERATED {true}                                       OPTIONAL,

...

}

MeasResultL1-r19 ::=                 SEQUENCE {

resultsSSB-Indexes-r19               ResultsPerSSB-IndexList                                 OPTIONAL,

...

}

MeasResultList3NR-r19 ::=            SEQUENCE (1..maxFreq) OF MeasResult3NR-r19

MeasResult3NR-r19 ::=                SEQUENCE {

ssbFrequency-r16                     ARFCN-ValueNR                                           OPTIONAL,

l1-MeasResultList-r19                L1-MeasResultList-r19                                   OPTIONAL,

...

}

L1-MeasResultList-r19 ::=             SEQUENCE (1..maxCellReport) OF L1-MeasResultPerCell-r19

L1-MeasResultPerCell-r19 ::=          SEQUENCE {

physCellId                            PhyCellId,

resultsSSB-Indexes                    ResultsPerSSB-IndexList                                 OPTIONAL,

...

}

TimeSinceFailure-r16 ::= INTEGER (0..172800)

MobilityHistoryReport-r16 ::= VisitedCellInfoList-r16

TimeUntilReconnection-r16 ::= INTEGER (0..172800)

TimeSinceCHO-Reconfig-r17 ::= INTEGER (0..1023)

TimeSinceCPAC-Reconfig-r18 ::= INTEGER (0.. 1023)

TimeConnSourceDAPS-Failure-r17 ::= INTEGER (0..1023)

UPInterruptionTimeAtHO-r17 ::= INTEGER (0..1023)

ElapsedTimeT316-r18 ::= INTEGER (0..2000)

ElapsedTimeSCG-Failure-r18 ::= INTEGER (0..1023)

TimeSinceSHR-r18 ::= INTEGER (0..172800)

TimeSinceSdt-Execution-r19 ::= INTEGER (0..172800)

-- TAG-UEINFORMATIONRESPONSE-STOP

-- ASN1STOP

UEInformationResponse-IEs field descriptions

coarseLocationInfo Parameter type Ellipsoid-Point defined in TS 37.355 [49]. The first/leftmost bit of the first octet contains the most significant bit. The least significant bits of degreesLatitude and degreesLongitude are set to 0 to meet the accuracy requirement corresponds to a granularity of approximately 2 km. It is up to UE implementation how many LSBs are set to 0 to meet the accuracy requirement.

connEstFailReport This field is used to provide connection establishment failure or connection resume failure information.

connEstFailReportList This field is used to provide the list of connEstFailReport that are stored by the UE for the past up to maxCEFReport-r17.

flightPathInfoReport This field is used to provide the flight path information as list of waypoints and, if available, corresponding timestamps. List of size zero indicates the previously provided flight path information is no longer valid.

logMeasReport This field is used to provide the measurement results stored by the UE associated to logged MDT.

measResultIdleEUTRA EUTRA measurement results performed during RRC_INACTIVE or RRC_IDLE.

measResultIdleNR NR measurement results performed during RRC_INACTIVE or RRC_IDLE.

ra-ReportList This field is used to provide the list of RA reports that is stored by the UE for up to maxRAReport-r16 number of random access procedures. If the UE is an eRedCap UE, this field is used to provide the list of RA reports that is stored by the UE for up to 2 number of random access procedures.

rlf-Report This field is used to indicate the RLF report related contents.

successHO-Report This field is used to provide the successful handover report if triggered based on the successful handover configuration.

successPSCell-Report This field is used to provide the successful PSCell change or addition report if triggered based on the successful PSCell change or addition report configuration.

LogMeasReport field descriptions

absoluteTimeStamp Indicates the absolute time when the logged measurement configuration logging is provided, as indicated by NR within absoluteTimeInfo.

anyCellSelectionDetected This field is used to indicate the detection of any cell selection state, as defined in TS 38.304 [20]. The UE sets this field when performing the logging of measurement results in RRC_IDLE or RRC_INACTIVE and there is no suitable cell or no acceptable cell.

inDeviceCoexDetected Indicates that measurement logging is suspended due to IDC problem detection.

measResultServingCell This field refers to the log measurement results taken in the Serving cell.

numberOfGoodSSB Indicates the number of good beams (beams that are above absThreshSS-BlocksConsolidation, if configured by the network) associated to the cells within the R value range (which is configured by network for cell reselection) of the highest ranked cell as part of the beam level measurements. If the UE has no SSB of a neighbour cell whose measurement quantity is above the absThreshSS-BlocksConsolidation or if the network has not configured the absThreshSS-BlocksConsolidation, then the UE does not include numberOfGoodSSB for the corresponding neighbour cell. If the UE has no SSB of the serving cell whose measurement quantity is above the absThreshSS-BlocksConsolidation or if the network has not configured the absThreshSS-BlocksConsolidation, then the UE shall set the numberOfGoodSSB for the serving cell to one.

relativeTimeStamp Indicates the time of logging measurement results, measured relative to the absoluteTimeStamp. Value in seconds.

tce-Id Parameter Trace Collection Entity Id: See TS 32.422 [52].

traceRecordingSessionRef Parameter Trace Recording Session Reference: See TS 32.422 [52].

ConnEstFailReport field descriptions

measResultFailedCell This field refers to the last measurement results taken in the cell, where connection establishment failure or connection resume failure happened.

measResultNeighCells This field refers to the neighbour cell measurements when connection establishment failure or connection resume failure happened.

numberOfConnFail This field is used to indicate the latest number of consecutive failed RRCSetup or RRCResume procedures in the same cell independent of RRC state transition.

timeSinceFailure This field is used to indicate the time that elapsed since the connection (establishment or resume) failure. Value in seconds. The maximum value 172800 means 172800s or longer.

RA-InformationCommon field descriptions

absoluteFrequencyPointA This field indicates the absolute frequency position of the reference resource block (Common RB 0).

allPreamblesBlocked This field is included when the all the preamble transmission attempts in the corresponding beam (SSB or CSI-RS) are blocked by failed LBT.

attemptedBWP-InfoList This field indicates locationAndBandwidth and subcarrierSpacing of all the bandwidth parts in which the consistent LBT failures are triggered at the moment of successful RA completion.

locationAndBandwidth Frequency domain location and bandwidth of the bandwidth part associated to the random-access resources used by the UE or of the bandwidth part in which the consistent LBT failures is triggered and not cancelled prior to successful completion of random access procedure (if this field is included in attemptedBWP-InfoList) or prior to RLF/HOF (if this field is included in attemptedBWP-InfoList or bwp-Info).

numberOfLBT-Failures This field is used to indicate the total number of preamble transmission attempts for which LBT failure indication is received in the RA procedure. If the number of LBT failure indications received from lower layers during the RA procedure exceeds or equals to 128, UE sets the field to 128.This field is optional present when there is at least one preamble transmission attempt for which LBT failure indication is received during the RA procedure, otherwise it is absent.

numberOfPreamblesPerSSB-ForThisPartition This field determines how many consecutive preambles are associated to the used feature or combination of features starting from the starting preamble(s) per SSB.

perRAInfoList, perRAInfoList-v1660 This field provides detailed information about each of the random access attempts in the chronological order of the random access attempts. If perRAInfoList-v1660 is present, it shall contain the same number of entries, listed in the same order as in perRAInfoList-r16.

startPreambleForThisPartition This field indicates the first preamble associated with the used feature or combination of features.

subcarrierSpacing Subcarrier spacing used in the bandwidth part associated to the random-access resources used by the UE or of the bandwidth part in which the consistent LBT failures is triggered and not cancelled prior to successful completion of random access procedure (if this field is included in attemptedBWP-InfoList) or prior to RLF/HOF (if this field is included in attemptedBWP-InfoList or bwp-Info).

triggeredFeatureCombination One or more features (e.g., RedCap, Slicing, SDT and MSG3 repetition) that triggers the random-access procedure. When triggered feature is Slicing, UE includes all the S-NSSAIs associated to the slices triggering the access attempt in the random-access procedure.

usedFeatureCombination The feature or combination of features (e.g., redCap, smallData, nsag and msg3-Repetitions) associated to the used random-access resources as specified in TS 38.321[3].

RA-Report field descriptions

cellID This field indicates the CGI of the cell in which the associated random access procedure was performed.

contentionDetected This field is used to indicate that contention was detected for the transmitted preamble in the given random access attempt or not. This field is not included when the UE performs random access attempt is using contention free random-access resources or when the raPurpose is set to requestForOtherSI or when the RA attempt is a 2-step RA attempt and fallback to 4-step RA did not occur (i.e. fallbackToFourStepRA is not included).

csi-RS-Index, csi-RS-Index-v1660 This field is used to indicate the CSI-RS index corresponding to the random access attempt. If the random access procedure is for beam failure recovery, the field indicates the NZP-CSI-RS-ResourceId. For CSI-RS index larger than maxNrofCSI-RS-ResourcesRRM-1, the index value is the sum of csi-RS-Index (without suffix) and csi-RS-Index-v1660.

dlPathlossRSRP Measeured RSRP of the DL pathloss reference obtained at the time of RA_Type selection stage of the RA procedure as captured in TS 38.321 [3].

dlRSRPAboveThreshold In 4 step random access procedure, this field is used to indicate whether the DL beam (SSB) quality associated to the random access attempt was above or below the threshold rsrp-ThresholdSSB in beamFailureRecoveryConfig in UL BWP configuration of UL BWP selected for random access procedure initiated for beam failure recovery; Otherwise, if the UE has received rsrp-ThresholdSSB in FeatureCombinationPreambles used for the feature specific random access, the field is used to indicate whether DL beam (SSB) quality associated to the random access attempt was above or below this rsrp-ThresholdSSB-r17, else rsrp-ThresholdSSB in rach-ConfigCommon in UL BWP configuration of UL BWP selected for random access procedure. In 2 step random access procedure, if the UE has received msgA-RSRP-ThresholdSSB in FeatureCombinationPreambles used for the feature specific random access, the field is used to indicate whether DL beam (SSB) quality associated to the random access attempt was above or below this rsrp-ThresholdSSB-r17, else this field is used to indicate whether the DL beam (SSB) quality associated to the random access attempt was above or below the threshold msgA-RSRP-ThresholdSSB in rach-ConfigCommonTwoStepRA in UL BWP configuration of UL BWP selected for random access procedure.

fallbackToFourStepRA This field indicates if a fallback indication in MsgB is received (according to TS 38.321 [3]) for the 2-step random access attempt.

intendedSIBs This field indicates the SIB(s) the UE wanted to receive as a result of the on demand SI request (when the RA procedure is a used as a SI request) initiated by the UE. That is, it indicates the one(s) of the SIB(s) in the SI message(s) requested to be broadcast that the UE was interested in.

lbt-Detected This field is included when there is at least one LBT failure indication received prior to change of beam for preamble transmission during RA procedure, otherwise this field is absent.

msg1-SCS-From-prach-ConfigurationIndex This field is set by the UE with the corresponding SCS for CBRA as derived from the prach-ConfigurationIndex in RACH-ConfigGeneric when the msg1-SubcarrierSpacing is absent; otherwise, this field is absent.

msg1-SCS-From-prach-ConfigurationIndexCFRA This field is set by the UE with the corresponding SCS for CFRA as derived from the prach-ConfigurationIndex in RACH-ConfigGeneric when the msg1-SubcarrierSpacing is absent; otherwise, this field is absent.

msgA-PUSCH-PayloadSize This field indicates the size of the overall payload available in the UE buffer at the time of initiating the 2 step RA procedure. The value refers to the index of TS 38.321 [3], table 6.1.3.1-1, corresponding to the UE buffer size.

msgA-RO-FDM This field indicates the number of msgA PRACH transmission occasions Frequency-Division Multiplexed in one time instance for the PRACH resources configured for 2-step CBRA..

msgA-RO-FDMCFRA This field indicates the number of msgA PRACH transmission occasions Frequency-Division Multiplexed in one time instance for the PRACH resources configured for 2-step CFRA.

msgA-RO-FrequencyStart This field indicates the lowest resource block of the contention based random-access resources for 2-step CBRA in the random-access procedure. The indication has the form of the offset of the lowest PRACH transmissions occasion with respect to PRB 0 in the frequency domain.

msgA-RO-FrequencyStartCFRA This field indicates the lowest resource block of the contention free random-access resources for the 2-step CFRA in the random-access procedure. The indication has the form of the offset of the lowest PRACH transmissions occasion with respect to PRB 0 in the frequency domain.

msgA-SCS-From-prach-ConfigurationIndex This field is set by the UE with the corresponding SCS as derived from the msgA-PRACH-ConfigurationIndex in RACH-ConfigGenericTwoStepRA (see tables Table 6.3.3.1-1, Table 6.3.3.1-2, Table 6.3.3.2-2 and Table 6.3.3.2-3, TS 38.211 [16]) when the msgA-SubcarrierSpacing is absent and when only 2-step random-access resources are available in the UL BWP used in the random-access procedure; otherwise, this field is absent.

numberOfPreamblesSentOnCSI-RS This field is used to indicate the total number of successive RA preambles that were transmitted on the corresponding CSI-RS.

numberOfPreamblesSentOnSSB This field is used to indicate the total number of successive RA preambles that were transmitted on the corresponding SS/PBCH block.

onDemandSISuccess This field is set to true when the RA report entry is included because of either msg1 based on demand SI request or msg3 based on demand SI request and if the on-demand SI request is successful. Otherwise, the field is absent.

perRAAttemptInfoList This field provides detailed information about a random access attempt.

perRACSI-RSInfoList This field provides detailed information about the successive random access attempts associated to the same CSI-RS.

perRASSBInfoList This field provides detailed information about the successive random access attempts associated to the same SS/PBCH block.

ra-InformationCommon This field is used to provide information on random access attempts. This field is mandatory present.

raPurpose This field is used to indicate the RA scenario for which the RA report entry is triggered. The RA accesses associated to Initial access from RRC_IDLE, RRC re-establishment procedure, transition from RRC-INACTIVE. The indicator beamFailureRecovery is used in case of successful beam failure recovery related RA procedure in the SpCell [3]. The indicator reconfigurationWithSync is used if the UE executes a reconfiguration with sync. The indicator ltm is used if the UE executes a RACH based LTM cell switch. The indicator ulUnSynchronized is used if the random access procedure is initiated in a SpCell by DL or UL data arrival during RRC_CONNECTED when the timeAlignmentTimer is not running in the PTAG or if the RA procedure is initiated in a serving cell by a PDCCH order [3]. The indicator schedulingRequestFailure is used in case of SR failures [3]. The indicator noPUCCHResourceAvailable is used when the UE has no valid SR PUCCH resources configured [3]. The indicator requestForOtherSI is used for MSG1 based on demand SI request. The indicator msg3RequestForOtherSI is used in case of MSG3 based SI request. The indication lbtFailure is used when the UE initiates RACH in SpCell due to consistent uplink LBT failures [3]. The field can also be used for the SCG-related RA-Report when the raPurpose is set to beamFailureRecovery, reconfigurationWithSync, ltm, ulUnSynchronized, schedulingRequestFailure, noPUCCHResourceAvailable and lbtFailure.

sdt-Failed This field is included when the RA report entry is included because of SDT and if the SDT transmission failed. Otherwise, the field is absent.

sdt-FailureCause This field is included when the RA report entry is included because of SDT and if the SDT procedure failed. Otherwise, the field is absent. This field indicates the SDT failure cause. The field is set to t319a-expiry upon expiration of T319a timer. If the UE upper layers receive maxRetxThreshold reached indication from RLC while SDT procedure is ongoing, this field is set to maxRetxThreshold. It is set to preambleTransMax upon the UE upper layer receiving indication of reaching preambleTransMax from the MAC layer. Upon expiration of cg-SDT-TimeAlignmentTimer from the MAC, the field is set to cg-SDT-TimeAlignmentTimer. The field is set to configuredGrantTimer upon reception of indication that configuration grant timer has been expired from the MAC. The field is set to cellReselection upon SDT failure due to UE’s cell re-selection.

sdt-DL-RsrpInfo This field logs the RSRP value measured by UE during evaluation of SDT procedure. This field is included when the RA report entry is included because of SDT initiation failure. Otherwise, the field is absent.

sdt-UL-DataVolume This field logs the buffered data volume in the UE for the radio bearer configured for the SDT during evaluation of SDT procedure. This field is included when the RA report entry is included because of SDT initiation failure and if the SDT procedure failed. Otherwise, the field is absent. Value in bytes, the maximum value 96000 means 96000 bytes or larger.

spCellID This field is used to indicate the CGI of the SpCell of the cell group associated to the SCell in which the associated random access procedure was performed. If the UE performs RA procedure on a SCell associated to the MCG, then this field is set to the CGI of the PCell and if the UE performs RA procedure on a SCell associated to the SCG, then this field is set to the CGI of the PSCell. If the CGI of the PSCell is not available at the UE for the RA procedure performed on a SCell associated to the SCG or for the RA procedure on the PSCell, this field is set to the CGI of the PCell. Otherwise, the field is absent.

ssb-Index This field is used to indicate the SS/PBCH index of the SS/PBCH block corresponding to the random access attempt.

ssbsForSI-Acquisition This field indicates the SSB(s) (in the form of SSB index(es)) that the UE used to receive the requested SI message(s). The field is present if the purpose of the random access procedure was to request on-demand SI (i.e. if the raPurpose is set to requestForOtherSI or msg3RequestForOtherSI). Otherwise, the field is absent.

timeSinceSdt-Execution This field logs the elapsed time since the execution of RA-SDT. Value in seconds. The maximum value is 172800 seconds.

RLF-Report field descriptions

bwp-Info This field is used to indicate the BWP information in which the UE detected consistent uplink LBT failure. This field is set only when the detected consistent uplink LBT failure did not trigger the random access procedure.

choCandidateCellList This field is used to indicate the list of candidate target cells for conditional handover included in condRRCReconfig at the time of connection failure. The field does not include the candidate target cells included in measResultNeighCells.

choCellId This field is used to indicate the candidate target cell for conditional handover included in condRRCReconfig that the UE selected for CHO based recovery while T311 is running.

connectionFailureType This field is used to indicate whether the connection failure is due to radio link failure or handover failure.

csi-rsRLMConfigBitmap,csi-rsRLMConfigBitmap-v1650 These fields are used to indicate the CSI-RS indexes configured in the RLM configurations for the active BWP when the UE declares RLF or HOF. The UE first fills in the csi-rsRLMConfigBitmap-r16 to indicate the first 96 CSI-RS indexes and then csi-rsRLMConfigBitmap-v1650 to indicate the latter 96 CSI-RS indexes. The first/leftmost bit in csi-rsRLMConfigBitmap-r16 corresponds to CSI-RS index 0, the second bit corresponds to CSI-RS index 1. The first/leftmost bit in csi-rsRLMConfigBitmap-v1650 corresponds to CSI-RS index 96, the second bit corresponds to CSI-RS index 97. These fields are included only if the RadioLinkMonitoringConfig for the respective BWP is configured.

c-RNTI This field indicates the C-RNTI used in the PCell upon detecting radio link failure or the C-RNTI used in the source PCell upon handover failure.

elapsedTimeSCG-Failure This field is used to indicate the time elapsed between the SCG failure and the MCG failure. The maximum value 1023 means 1023ms or longer.

elapsedTimeT316 This field is used to indicate the value of the elapsed time of the timer T316. Value in milliseconds.

failedPCellId This field is used to indicate the PCell in which RLF is detected or the target PCell of the failed handover. For intra-NR handover nrFailedPCellId is included and for the handover from NR to EUTRA eutraFailedPCellId is included. The UE sets the ARFCN according to the frequency band used for transmission/ reception when the failure occurred.

failedPCellId-EUTRA This field is used to indicate the PCell in which RLF is detected or the source PCell of the failed handover in an E-UTRA RLF report.

lastHO-Type This field is used to indicate the type of the last executed handover or LTM cell switch before the last detected connection failure. The field is set to cho if the last executed handover was initiated by a conditional reconfiguration execution. The field is set to daps if the last executed handover was a DAPS handover. The field is set to ltm if the last executed RRCReconfiguration message including reconfigurationWithSync was an LTM cell switch.This field is set to choWithCandidateSCG if the last executed RRCReconfiguration contained both condExecutionCond and condExecutionCondPSCell.

ltm-RecoveryCellId This field is used to indicate the candidate target cell for LTM cell switch included in ltm-Config associated with the MCG that the UE selected for LTM based recovery while T311 was running.

mcg-RecoveryFailureCause This field is used to indicate the cause of the fast MCG recovery failure.

measResultL1-LastServCell This field contains the last L1 measurement results collected in the PCell upon detecting radio link failure or the source PCell upon reconfiguration with sync failure if the UE was configured with ltm-Config associated with the MCG when connected to the source PCell (in case HO failure) or PCell (in case RLF).

measResultL1-NeighCells This field contains the last L1 measurement results collected in neighbour MCG LTM candidate cells upon reconfiguration with sync failure or upon detecting radio link failure.

measResultListEUTRA This field refers to the last measurement results taken in the neighboring EUTRA Cells, when the radio link failure or handover failure happened.

measResultListNR This field refers to the last measurement results taken in the neighboring NR Cells, when the radio link failure or handover failure happened.

measResultLastServCell This field refers to the log measurement results taken in the PCell upon detecting radio link failure or the source PCell upon handover failure.

measResultLastServCellRSSI This field refers to the log RSSI measurement results in dBm (see TS 38.215 [9]) taken for the frequency of the PCell upon detecting radio link failure or source PCell upon detecting handover failure.

measResultNeighFreqListRSSI This field is used to log the RSSI measurement results in dBm (see TS 38.215 [9]) taken for the neighbouring frequencies upon detecting radio link failure or handover failure, when UE operates in shared spectrum.

measResult-RLF-Report-EUTRA Includes the E-UTRA RLF-Report-r9 IE as specified in TS 36.331 [10].

measResult-RLF-Report-EUTRA-v1690 Includes the E-UTRA RLF-Report-v9e0 IE as specified in TS 36.331 [10].

noSuitableCellFound This field is set by the UE when the T311 expires.

previousPCellId This field is used to indicate the source PCell of the last handover (source PCell when the last executed RRCReconfiguration message including reconfigurationWithSync was received). For intra-NR handover nrPreviousCell is included and for the handover from EUTRA to NR eutraPreviousCell is included.

pSCellId This field is used to indicate the PSCell in which the UE failed to perform fast MCG recovery procedure or the UE successfully performed fast MCG recovery procedure.

ra-InformationCommon This field is optionally included when connectionFailureType is set to 'hof' or when connectionFailureType is set to 'rlf' and the rlf-Cause equals to 'randomAccessProblem' or 'beamRecoveryFailure'; otherwise this field is absent.

reconnectCellId This field is used to indicate the cell in which the UE comes back to connected after connection failure and after failing to perform reestablishment, or to indicate the suitable cell in which the UE reconnects after failure in performing MobilityFromNRCommand for voice fallback (without initiating re-establishment procedure). If the UE comes back to RRC CONNECTED in an NR cell then nrReconnectCellID is included and if the UE comes back to RRC CONNECTED in an LTE cell then eutraReconnectCellID is included.

reestablishmentCellId If the UE was not configured with conditionalReconfiguration at the time of re-establishment attempt, or if the cell selected for the re-establishment attempt is not a candidate target cell for conditional reconfiguration, this field is used to indicate the cell in which the re-establishment attempt was made after connection failure.

rlf-Cause This field is used to indicate the cause of the last radio link failure that was detected. In case of handover failure information reporting (i.e., the connectionFailureType is set to 'hof'), the UE is allowed to set this field to any value, except for the case in which a radio link failure was detected in the source PCell while performing a DAPS handover.

scg-FailedAfterMCG This field is set if for the SCG failure is detected after MCG failure while T316 is running.

ssbRLMConfigBitmap This field is used to indicate the SS/PBCH block indexes configured in the RLM configurations for the active BWP when the UE declares RLF or HOF.The first/leftmost bit corresponds to SSB index 0, the second bit corresponds to SSB index 1. This field is included only if the RadioLinkMonitoringConfig for the respective BWP is configured.

timeConnFailure This field is used to indicate the time elapsed since the last HO or LTM cell switch execution until connection failure. Actual value = field value * 100ms. The maximum value 1023 means 102.3s or longer.

timeConnSourceDAPS-Failure This field is used to indicate the time that elapsed between the last DAPS handover execution and the radio link failure detected in the source cell while T304 is running. Value in milliseconds. The maximum value 1023 means 1023ms or longer.

timeSinceFailure This field is used to indicate the time that elapsed since the connection (radio link, handover or LTM cell switch) failure. Value in seconds. The maximum value 172800 means 172800s or longer. In the case of failure(s) (either at source or at target or at both) associated to DAPS handover, this field indicates the time elapsed since the latest connection (radio link, handover or LTM cell switch) failure.

timeSinceCHO-Reconfig In case of handover failure, this field is used to indicate the time elapsed between the initiation of the last handover execution towards the target cell and the reception of the latest conditional reconfiguration. In case of radio link failure, this field is used to indicate the time elapsed between the radio link failure and the reception of the latest conditional reconfiguration while connected to the source PCell. Actual value = field value * 100ms. The maximum value 1023 means 102.3s or longer.

timeUntilReconnection This field is used to indicate the time that elapsed between the connection (radio link, handover or LTM cell switch) failure and the next time the UE comes to RRC CONNECTED in an NR or EUTRA cell, after failing to perform reestablishment. Value in seconds. The maximum value 172800 means 172800s or longer.

voiceFallbackHO This field is set if for the failed mobility from NR, the voiceFallbackIndication was included in the MobilityFromNRCommand message.

SuccessHO-Report field descriptions

c-RNTI This field indicates the C-RNTI assigned by the target PCell of the handover for which the successful HO report was generated.

eutra-TargetCellInfo This field is used to indicate the target EUTRA PCell and the last measurement results of the target PCell of a handover in which the successful handover triggers the SuccessHO-Report.

eutra-C-RNTI This field indicates the C-RNTI assigned by the E-UTRA target PCell of the mobility from NR command for which the successful HO report was generated.

measResultListNR This field refers to the last measurement results taken in the neighboring NR Cells when a successful handover is executed.

measResultNeighFreqListRSSI This field is used to log the RSSI measurement results in dBm (see TS 38.215 [9]) taken for the neighbouring frequencies upon successful handover execution.

measResultServCellRSSI This field refers to the log RSSI measurement results in dBm (see TS 38.215 [9]) taken for the frequency of the source PCell upon successful handover execution.

neighCellsMeasL1ListNR This field refers to the last L1 measurement results taken in the neighboring MCG LTM candidate Cells when a successful reconfiguration with sync is executed if the UE was configured with ltm-Config associated with the MCG when connected to the source PCell.

rach-Less This field is set if the successful HO report is trigged by RACH-less LTM cell switch.

rlf-InSourceDAPS This field indicates whether a radio link failure occurred at the source cell while T304 was running.

shr-Cause This field is used to indicate the cause of the successful HO report.

sourceCellMeas This field refers to the last measurement results taken in the source PCell of a handover in which the successful handover triggers the SuccessHO-Report.

sourceCellMeasL1 This field refers to the last L1 measurement results taken in the source PCell of a reconfiguration with sync if the UE was configured with ltm-Config associated with the MCG when connected to the source PCell.

sourcePCellId This field is used to indicate the source PCell of a handover in which the successful handover triggers the SuccessHO-Report.

targetPCellId This field is used to indicate the target PCell of a handover in which the successful handover triggers the SuccessHO-Report.

targetCellMeas This field refers to the last measurement results taken in the target PCell of a handover in which the successful handover triggers the SuccessHO-Report.

targetCellMeasL1 This field refers to the last L1 measurement results taken in the target PCell of a reconfiguration with sync if the UE was configured with ltm-Config associated with the MCG when connected to the source PCell.

timeSinceCHO-Reconfig This field is used to indicate the time elapsed between the initiation of the last conditional reconfiguration execution towards the target cell and the reception of the latest conditional reconfiguration for this target cell. Actual value = field value * 100ms. The maximum value 1023 means 102.3s or longer.

timeSinceSHR This field is used to indicate the time elapsed since the execution of the last MobilityFromNRCommand towards the target EUTRA cell. Value in seconds. The maximum value 172800 means 172800s or longer.

upInterruptionTimeAtHO This field is used to indicate the time elapsed between the time of arrival of the last PDCP PDU received from the source cell for any data radio bearer and the time of arrival of the first non-duplicate PDCP PDU received from the target cell for any data radio bearer, and it is measured at the time of arrival of the first non-duplicate PDCP PDU received from the target cell for any data radio bearer. The field is set only in case of DAPS handover. Value in milliseconds. The maximum value 1023 means 1023ms or longer.

FlightPathInfoReport field descriptions

timeStamp Time stamp that describes estimated time of arrival, if available, of the UE at the corresponding wayPointLocation.

wayPointLocation Location coordinates of the planned waypoint. Parameter type LocationCoordinates defined in TS 37.355 [49]. The first/leftmost bit of the first octet contains the most significant bit.

SuccessPSCell-Report field descriptions

measResultListNR This field refers to the last measurement results according to the initiating node configuration taken in the neighboring NR Cells when a successful PSCell change/addition is executed.

pCellId This field is used to indicate the PCell to which the UE was connected when the successful PSCell change or addition triggers the SuccessPSCell-Report. Alternatively. this field indicates the source PCell to which the UE was connected to before executing CHO with candidate SCG in which the SuccessPSCell-Report was triggered.

sn-InitiatedPSCellChange This field indicates whether the PSCell change procedure for which the successful PSCell change report is logged is SN initiated or not.

spr-Cause This field is used to indicate the cause of the successful PSCell change or addition report.

sourcePSCellId This field is used to indicate the source PSCell of a PSCell change in which the successful PSCell change triggers the SuccessPSCell-Report.

sourcePSCellMeas This field refers to the last measurement results taken in the source PSCell of a PSCell change in which the successful PSCell change triggers the SuccessPSCell-Report.

targetPCellId This field is used to indicate the target PCell of a CHO with candidate SCG procedure in which the SuccessPSCell-Report was triggered.

targetPSCellId This field is used to indicate the target PSCell of a PSCell change/addition in which the successful PSCell change or addition triggers the SuccessPSCell-Report.

targetPSCellMeas This field refers to the last measurement results taken in the target PSCell of a PSCell change/addition in which the successful PSCell change or addition triggers the SuccessPSCell-Report.

timeSinceCPAC-Reconfig This field is used to indicate the time elapsed between the initiation of the last conditional reconfiguration execution towards the target PSCell and the reception of the latest conditional reconfiguration for this target PSCell. Actual value = field value * 100ms. The maximum value 1023 means 102.3s or longer.

NEXT CHANGE

## 6.3	RRC information elements

### 6.3.2	Radio resource control information elements

#### –	CGI-Info-Logging

The IE CGI-Info-Logging indicates the NR Cell Global Identifier (NCGI) for logging purposes (e.g. RLF report), the globally unique identity, and the TAC information of a cell in NR.

CGI-Info-Logging information element

-- ASN1START

-- TAG-CGI-INFO-LOGGING-START

CGI-Info-Logging-r16 ::=     SEQUENCE {

plmn-Identity-r16                    PLMN-Identity,

cellIdentity-r16                     CellIdentity,

trackingAreaCode-r16                 TrackingAreaCode               OPTIONAL

}

-- TAG-CGI-INFO-LOGGING-STOP

-- ASN1STOP

CGI-Info-Logging field descriptions

cellIdentity Unambiguously identify a cell within the context of the PLMN. It belongs the first PLMN-IdentityInfo IE of PLMN-IdentityInfoList in SIB1.

plmn-Identity Identifies the PLMN of the cell for the reported cellIdentity: the first PLMN entry of plmn-IdentityList (in SIB1) in the instance of PLMN-IdentityInfoList that contained the reported cellIdentity.

trackingAreaCode Indicates Tracking Area Code to which the cell indicated by cellIdentity field belongs. The first entry from trackingAreaList is logged in case the cell is configured with multiple tracking areas, e.g., NTN cell.

NEXT CHANGE

#### –	MeasResults

The IE MeasResults covers measured results for intra-frequency, inter-frequency, inter-RAT mobility and measured results for NR sidelink communication/discovery/positioning.

MeasResults information element

-- ASN1START

-- TAG-MEASRESULTS-START

MeasResults ::=                         SEQUENCE {

measId                                  MeasId,

measResultServingMOList                 MeasResultServMOList,

measResultNeighCells                    CHOICE {

measResultListNR                        MeasResultListNR,

...,

measResultListEUTRA                     MeasResultListEUTRA,

measResultListUTRA-FDD-r16              MeasResultListUTRA-FDD-r16,

sl-MeasResultsCandRelay-r17             OCTET STRING        -- Contains PC5 SL-MeasResultListRelay-r17

}                                                                                                                   OPTIONAL,

...,

[[

measResultServFreqListEUTRA-SCG         MeasResultServFreqListEUTRA-SCG                                             OPTIONAL,

measResultServFreqListNR-SCG            MeasResultServFreqListNR-SCG                                                OPTIONAL,

measResultSFTD-EUTRA                    MeasResultSFTD-EUTRA                                                        OPTIONAL,

measResultSFTD-NR                       MeasResultCellSFTD-NR                                                       OPTIONAL

]],

[[

measResultCellListSFTD-NR               MeasResultCellListSFTD-NR                                                   OPTIONAL

]],

[[

measResultForRSSI-r16                   MeasResultForRSSI-r16                                                       OPTIONAL,

locationInfo-r16                        LocationInfo-r16                                                            OPTIONAL,

ul-PDCP-DelayValueResultList-r16        UL-PDCP-DelayValueResultList-r16                                            OPTIONAL,

measResultsSL-r16                       MeasResultsSL-r16                                                           OPTIONAL,

measResultCLI-r16                       MeasResultCLI-r16                                                           OPTIONAL

]],

[[

measResultRxTxTimeDiff-r17              MeasResultRxTxTimeDiff-r17                                                  OPTIONAL,

sl-MeasResultServingRelay-r17           OCTET STRING                                                                OPTIONAL,

-- Contains PC5 SL-MeasResultRelay-r17

ul-PDCP-ExcessDelayResultList-r17       UL-PDCP-ExcessDelayResultList-r17                                           OPTIONAL,

coarseLocationInfo-r17                  OCTET STRING                                                                OPTIONAL

]],

[[

altitudeUE-r18                          Altitude-r18                                                                OPTIONAL,

cellsMetReportOnLeaveList-r18           SEQUENCE (SIZE (1..maxCellReport)) OF PhysCellId                            OPTIONAL

]]

}

MeasResultServMOList ::=                SEQUENCE (SIZE (1..maxNrofServingCells)) OF MeasResultServMO

MeasResultServMO ::=                    SEQUENCE {

servCellId                              ServCellIndex,

measResultServingCell                   MeasResultNR,

measResultBestNeighCell                 MeasResultNR                                                                OPTIONAL,

...

}

MeasResultListNR ::=                    SEQUENCE (SIZE (1..maxCellReport)) OF MeasResultNR

MeasResultNR ::=                        SEQUENCE {

physCellId                              PhysCellId                                                                  OPTIONAL,

measResult                              SEQUENCE {

cellResults                             SEQUENCE{

resultsSSB-Cell                         MeasQuantityResults                                                 OPTIONAL,

resultsCSI-RS-Cell                      MeasQuantityResults                                                 OPTIONAL

},

rsIndexResults                          SEQUENCE{

resultsSSB-Indexes                      ResultsPerSSB-IndexList                                             OPTIONAL,

resultsCSI-RS-Indexes                   ResultsPerCSI-RS-IndexList                                          OPTIONAL

}                                                                                                               OPTIONAL

},

...,

[[

cgi-Info                                CGI-InfoNR                                                                  OPTIONAL

]],

[[

choCandidate-r17                        ENUMERATED {true}                                                           OPTIONAL,

choConfig-r17                           SEQUENCE (SIZE (1..2)) OF CondTriggerConfig-r16                             OPTIONAL,

triggeredEvent-r17                      SEQUENCE {

timeBetweenEvents-r17                   TimeBetweenEvent-r17                                                    OPTIONAL,

firstTriggeredEvent-r17                 ENUMERATED {condFirstEvent, condSecondEvent}                            OPTIONAL

}                                                                                                               OPTIONAL

]],

[[

entering-r18                            ENUMERATED {true}                                                           OPTIONAL

]] ,

[[

ltm-Candidate-r19                        ENUMERATED {true}                                                           OPTIONAL,

distanceFromReference1-r19               INTEGER(0.. 65535)                                                          OPTIONAL,

distanceFromReference2-r19               INTEGER(0.. 65535)                                                          OPTIONAL

]]

}

MeasResultListEUTRA ::=                 SEQUENCE (SIZE (1..maxCellReport)) OF MeasResultEUTRA

MeasResultEUTRA ::=                     SEQUENCE {

eutra-PhysCellId                        PhysCellId,

measResult                              MeasQuantityResultsEUTRA,

cgi-Info                                CGI-InfoEUTRA                                                               OPTIONAL,

...

}

MultiBandInfoListEUTRA ::=              SEQUENCE (SIZE (1..maxMultiBands)) OF FreqBandIndicatorEUTRA

MeasQuantityResults ::=                 SEQUENCE {

rsrp                                    RSRP-Range                                                                  OPTIONAL,

rsrq                                    RSRQ-Range                                                                  OPTIONAL,

sinr                                    SINR-Range                                                                  OPTIONAL

}

MeasQuantityResultsEUTRA ::=            SEQUENCE {

rsrp                                    RSRP-RangeEUTRA                                                             OPTIONAL,

rsrq                                    RSRQ-RangeEUTRA                                                             OPTIONAL,

sinr                                    SINR-RangeEUTRA                                                             OPTIONAL

}

ResultsPerSSB-IndexList::=              SEQUENCE (SIZE (1..maxNrofIndexesToReport2)) OF ResultsPerSSB-Index

ResultsPerSSB-Index ::=                 SEQUENCE {

ssb-Index                               SSB-Index,

ssb-Results                             MeasQuantityResults                                                         OPTIONAL

}

ResultsPerCSI-RS-IndexList::=           SEQUENCE (SIZE (1..maxNrofIndexesToReport2)) OF ResultsPerCSI-RS-Index

ResultsPerCSI-RS-Index ::=              SEQUENCE {

csi-RS-Index                            CSI-RS-Index,

csi-RS-Results                          MeasQuantityResults                                                         OPTIONAL

}

MeasResultServFreqListEUTRA-SCG ::= SEQUENCE (SIZE (1..maxNrofServingCellsEUTRA)) OF MeasResult2EUTRA

MeasResultServFreqListNR-SCG ::= SEQUENCE (SIZE (1..maxNrofServingCells)) OF MeasResult2NR

MeasResultListUTRA-FDD-r16 ::=          SEQUENCE (SIZE (1..maxCellReport)) OF MeasResultUTRA-FDD-r16

MeasResultUTRA-FDD-r16 ::=              SEQUENCE {

physCellId-r16                          PhysCellIdUTRA-FDD-r16,

measResult-r16                          SEQUENCE {

utra-FDD-RSCP-r16                       INTEGER (-5..91)          OPTIONAL,

utra-FDD-EcN0-r16                       INTEGER (0..49)           OPTIONAL

}

}

MeasResultForRSSI-r16 ::=        SEQUENCE {

rssi-Result-r16                  RSSI-Range-r16,

channelOccupancy-r16             INTEGER (0..100)

}

MeasResultCLI-r16 ::=            SEQUENCE {

measResultListSRS-RSRP-r16       MeasResultListSRS-RSRP-r16                                                         OPTIONAL,

measResultListCLI-RSSI-r16       MeasResultListCLI-RSSI-r16                                                         OPTIONAL

}

MeasResultListSRS-RSRP-r16 ::=   SEQUENCE (SIZE (1.. maxCLI-Report-r16)) OF MeasResultSRS-RSRP-r16

MeasResultSRS-RSRP-r16 ::=       SEQUENCE {

srs-ResourceId-r16               SRS-ResourceId,

srs-RSRP-Result-r16              SRS-RSRP-Range-r16

}

MeasResultListCLI-RSSI-r16 ::=   SEQUENCE (SIZE (1.. maxCLI-Report-r16)) OF MeasResultCLI-RSSI-r16

MeasResultCLI-RSSI-r16 ::=       SEQUENCE {

rssi-ResourceId-r16              RSSI-ResourceId-r16,

cli-RSSI-Result-r16              CLI-RSSI-Range-r16

}

UL-PDCP-DelayValueResultList-r16 ::= SEQUENCE (SIZE (1..maxDRB)) OF UL-PDCP-DelayValueResult-r16

UL-PDCP-DelayValueResult-r16 ::= SEQUENCE {

drb-Id-r16                       DRB-Identity,

averageDelay-r16                 INTEGER (0..10000),

...

}

UL-PDCP-ExcessDelayResultList-r17 ::= SEQUENCE (SIZE (1..maxDRB)) OF UL-PDCP-ExcessDelayResult-r17

UL-PDCP-ExcessDelayResult-r17 ::= SEQUENCE {

drb-Id-r17                        DRB-Identity,

excessDelay-r17                   INTEGER (0..31),

...

}

TimeBetweenEvent-r17 ::= INTEGER (0..1023)

-- TAG-MEASRESULTS-STOP

-- ASN1STOP

MeasResultEUTRA field descriptions

eutra-PhysCellId Identifies the physical cell identity of the E-UTRA cell for which the reporting is being performed. The UE reports a value in the range 0..503, other values are reserved.

MeasResultNR field descriptions

averageDelay Indicates average delay for the packets during the reporting period, as specified in TS 38.314 [53]. Value 0 corresponds to 0 millisecond, value 1 corresponds to 0.1 millisecond, value 2 corresponds to 0.2 millisecond, and so on.

cellResults Cell level measurement results.

cellsMetReportOnLeaveList This field indicates the list of cells which met the event leaving condition.

choCandidate This field indicates whether the associated cell is a candidate target cell for conditional handover or conditional PSCell change or addition. This field may be included only in the SuccessHO-Report or SuccessPSCell-Report within UEInformationResponse message.

choConfig If the associated cell is a candidate target cell for conditional handover, this field indicates the conditional handover execution condition for each measId within condTriggerConfig associated to the cell. This field may be included only in the rlf-report within UEInformationResponse message.

drb-Id Indicates DRB value for which uplink PDCP delay ratio or value is provided, according to TS 38.314 [53].

distanceFromReference1, distanceFromReference2 This field indicates the measured distances between UE and the moving reference locations of the serving cell and of the associated neighbour cell if the conditional handover is based on condEventD2.

entering This field indicates if the event entering condition for the cell is satisfied and the cell has been just added within cellsTriggeredList.

firstTriggeredEvent This field is set to condFirstEvent if the execution condition associated to the first entry of choConfig was fulfilled first in time. This field is set to condSecondEvent if the execution condition associated to the second entry of choConfig was fulfilled first in time. This field may be included in rlf-report within UEInformationResponse message or in SCGFailureInformation message.

locationInfo Positioning related information and measurements.

ltm-Candidate This field indicates whether the associated cell is an LTM candidate cell. This field may be included only in the SuccessHO-Report within UEInformationResponse message.

physCellId The physical cell identity of the NR cell for which the reporting is being performed.

resultsSSB-Cell Cell level measurement results based on SS/PBCH related measurements.

resultsSSB-Indexes Beam level measurement results based on SS/PBCH related measurements.

resultsCSI-RS-Cell Cell level measurement results based on CSI-RS related measurements.

resultsCSI-RS-Indexes Beam level measurement results based on CSI-RS related measurements.

rsIndexResults Beam level measurement results.

timeBetweenEvents Indicates the time elapsed between fulfilling the conditional execution conditions included in choConfig. Value in milliseconds. The maximum value 1023 means 1023ms or longer. This field may be included in the reports associated to UEInformationResponse message, e.g., rlf-Report or in the SCGFailureInformation message.

MeasResultUTRA-FDD field descriptions

physCellId The physical cell identity of the UTRA-FDD cell for which the reporting is being performed.

utra-FDD-EcN0 According to CPICH_Ec/No in TS 25.133 [46] for FDD.

utra-FDD-RSCP According to CPICH_RSCP in TS 25.133 [46] for FDD.

MeasResults field descriptions

coarseLocationInfo This field indicates the coarse location information reported by the UE. This field is coded as the Ellipsoid-Point defined in TS 37.355 [49]. The first/leftmost bit of the first octet contains the most significant bit. The least significant bits of degreesLatitude and degreesLongitude are set to 0 to meet the accuracy requirement corresponds to a granularity of approximately 2 km. It is up to UE implementation how many LSBs are set to 0 to meet the accuracy requirement

excessDelay Indicates the ratio of packets in UL per DRB exceeding the configured delay threshold among the UL PDCP SDUs, according to the UL PDCP Excess Packet Delay per DRB mapping table, as defined in TS 38.314 [53], Table 4.3.1.e-1.

measId Identifies the measurement identity for which the reporting is being performed.

measQuantityResults The value sinr is not included when it is used for LogMeasReport-r16.

measResultCellListSFTD-NR SFTD measurement results between the PCell and the NR neighbour cell(s) in NR standalone.

measResultCLI CLI measurement results.

measResultEUTRA Measured results of an E-UTRA cell.

measResultForRSSI Includes measured RSSI result in dBm (see TS 38.215 [9]) and channelOccupancy which is the percentage of samples when the RSSI was above the configured channelOccupancyThreshold for the associated reportConfig.

measResultListEUTRA List of measured results for the maximum number of reported best cells for an E-UTRA measurement identity.

measResultListNR List of measured results for the maximum number of reported best cells for an NR measurement identity.

measResultListUTRA-FDD List of measured results for the maximum number of reported best cells for a UTRA-FDD measurement identity.

measResultNR Measured results of an NR cell.

measResultServFreqListEUTRA-SCG Measured results of the E-UTRA SCG serving frequencies: the measurement result of PSCell and each SCell, if any, and of the best neighbouring cell on each E-UTRA SCG serving frequency.

measResultServFreqListNR-SCG Measured results of the NR SCG serving frequencies: the measurement result of PSCell and each SCell, if any, and of the best neighbouring cell on each NR SCG serving frequency.

measResultServingMOList Measured results of measured cells with reference signals indicated in the serving cell measurement objects including measurement results of SpCell, configured SCell(s) and best neighbouring cell within measured cells with reference signals indicated in on each serving cell measurement object. If the sending of the MeasurementReport message is triggered by a measurement configured by the field sl-ConfigDedicatedForNR received within an E-UTRA RRCConnectionReconfiguration message (i.e. CBR measurements), this field is not applicable and its contents is ignored by the network.

measResultSFTD-EUTRA SFTD measurement results between the PCell and the E-UTRA PScell in NE-DC.

measResultSFTD-NR SFTD measurement results between the PCell and the NR PScell in NR-DC.

measResultsSL CBR measurements results for NR sidelink communication/discovery/positioning.

measResultUTRA-FDD Measured result of a UTRA-FDD cell.

sl-MeasResultsCandRelay Measurement result(s) of candiate L2 U2N relay UE(s).

sl-MeasResultServingRelay Measurement result of serving L2 U2N relay UE.

NEXT CHANGE

#### –	ChoWithCandidateSCGInfo

The IE ChoWithCandidateSCGInfo contains information regarding events of CHO with Candidate SCG.

ChoWithCandidateSCGInfo information element

-- ASN1START

-- TAG-CHOWITHCANDIDATESCGINFO-START

ChoWithCandidateSCGInfo::=                     SEQUENCE{

firstFulfilledConfig-r19                       ENUMERATED {cho, cpc}                             OPTIONAL,

timeBetweenFulfillment-r19                     TimeBetweenEvent-r17                              OPTIONAL,

timeBetweenLastFulfillmentAndEvent-r19         TimeBetweenEvent-r17                              OPTIONAL,

pCellId-r19              CHOICE {

cellGlobalId-r19         CGI-Info-Logging-r16,

pci-arfcn-r19            PCI-ARFCN-NR-r16

}                                                                                                OPTIONAL,

psCellId-r19             CHOICE {

cellGlobalId-r19         CGI-Info-Logging-r16,

pci-arfcn-r19            PCI-ARFCN-NR-r16

}                                                                                                OPTIONAL,

...

}

-- TAG-CHOWITHCANDIDATESCGINFO-STOP

-- ASN1STOP

ChoWithCandidateSCGInfo field descriptions

firstFulfilledConfig This field indicates if the execution conditions for conditional handover or conditional PSCell change/addition was fulfilled first.

timeBetweenFulfillment This field logs the time between fulfilment of conditional handover and conditional PSCell change or addition execution conditions.

timeBetweenLastFulfillmentAndEvent This field logs the time between fulfilment of either conditional handover and conditional PSCell change or addition execution conditions and failure for RLF and SCG failure, or the time between fulfilment of either conditional handover and conditional PSCell change or addition execution conditions and handover execution for SHR

NEXT CHANGE

#### –	VisitedCellInfoList

The IE VisitedCellInfoList includes the mobility history information of maximum of 16 most recently visited primary cells or time spent in any cell selection state and/or camped on any cell state in NR or E-UTRA and, in case of Dual Connectivity, the mobility history information of maxPSCellHistory most recently visited primary secondary cell group cells across all the primary cells included in the VisitedCellInfoList. The most recently visited cell is stored first in the list. The list includes cells visited in RRC_IDLE, RRC_INACTIVE and RRC_CONNECTED states for NR and RRC_IDLE and RRC_CONNECTED for E-UTRA.

VisitedCellInfoList information element

-- ASN1START

-- TAG-VISITEDCELLINFOLIST-START

VisitedCellInfoList-r16 ::= SEQUENCE (SIZE (1..maxCellHistory-r16)) OF VisitedCellInfo-r16

VisitedCellInfo-r16 ::=  SEQUENCE {

visitedCellId-r16        CHOICE {

nr-CellId-r16            CHOICE {

cgi-Info                 CGI-Info-Logging-r16,

pci-arfcn-r16            PCI-ARFCN-NR-r16

},

eutra-CellId-r16         CHOICE {

cellGlobalId-r16         CGI-InfoEUTRA,

pci-arfcn-r16                PCI-ARFCN-EUTRA-r16

}

}                                        OPTIONAL,

timeSpent-r16            INTEGER (0..4095),

...,

[[

visitedPSCellInfoListReport-r17    VisitedPSCellInfoList-r17                   OPTIONAL

]]

}

VisitedPSCellInfoList-r17 ::= SEQUENCE (SIZE (1..maxPSCellHistory-r17)) OF VisitedPSCellInfo-r17

VisitedPSCellInfo-r17 ::=    SEQUENCE {

visitedCellId-r17            CHOICE {

nr-CellId-r17                CHOICE {

cgi-Info-r17                 CGI-Info-Logging-r16,

pci-arfcn-r17                PCI-ARFCN-NR-r16

},

eutra-CellId-r17         CHOICE {

cellGlobalId-r17         CGI-InfoEUTRALogging,

pci-arfcn-r17            PCI-ARFCN-EUTRA-r16

}

}                                                 OPTIONAL,

timeSpent-r17            INTEGER (0..4095),

...,

[[

scgActiveDuration-r19            INTEGER (0..4095)                   OPTIONAL

]]

}

-- TAG-VISITEDCELLINFOLIST-STOP

-- ASN1STOP

VisitedCellInfoList field descriptions

timeSpent This field indicates the duration of stay in the cell or in any cell selection state and/or camped on any cell state in NR or E-UTRA approximated to the closest second. If included in VisitedPSCellInfo, it indicates the duration of stay in the PSCell or without any PSCell. If the duration of stay exceeds 4095s, the UE shall set it to 4095s.

visitedCellId This field indicates the visited cell id including NR and E-UTRA cells.

scgActiveDuration This field indicates the accumulated SCG active duration the stay in the PSCell. If the duration of activation exceeds 4095s, the UE shall set it to 4095s.

NEXT CHANGE

## 7.4	UE variables

#### –	VarLogMeasConfig

The UE variable VarLogMeasConfig includes the configuration of the logging of measurements to be performed by the UE while in RRC_IDLE, RRC_INACTIVE, covering intra-frequency, inter-frequency and inter-RAT mobility related measurements. The UE performs logging of measurements only while in RRC_IDLE and RRC_INACTIVE.

VarLogMeasConfig UE variable

-- ASN1START

-- TAG-VARLOGMEASCONFIG-START

VarLogMeasConfig-r16 ::=     SEQUENCE {

areaConfiguration-r16        AreaConfiguration-r16        OPTIONAL,

bt-NameList-r16              BT-NameList-r16              OPTIONAL,

wlan-NameList-r16            WLAN-NameList-r16            OPTIONAL,

sensor-NameList-r16          Sensor-NameList-r16          OPTIONAL,

loggingDuration-r16          LoggingDuration-r16,

reportType                   CHOICE {

periodical                   LoggedPeriodicalReportConfig-r16,

eventTriggered               LoggedEventTriggerConfig-r16

},

earlyMeasIndication-r17      ENUMERATED {true}            OPTIONAL,

areaConfiguration-r17        AreaConfiguration-r17        OPTIONAL,

areaConfiguration-v1800      AreaConfiguration-v1800      OPTIONAL,

intendedAreaScopeList-r19    IntendedAreaScopeList-r19    OPTIONAL

}

-- TAG-VARLOGMEASCONFIG-STOP

-- ASN1STOP

END OF CHANGES

# Annex

## MRO enhancements for Rel-18 mobility features – LTM

### RAN2#125-bis

For LTM MRO, RAN2 considers the following three connection failure cases:

- -	Too late LTM

- -	Too early LTM

- -	LTM to wrong cell

For too late LTM, the following sub-cases are considered but we may down prioritize later (not limiting):

- -	Case 1a: the UE detects RLF in source cell after receiving LTM candidate configurations and performs reestablishment procedure.

- -	Case 1b: the UE detects RLF in source cell after receiving LTM candidate configurations, selects an LTM candidate cell, detects HOF with the selected LTM cell.

- -	Case 1c: the UE detects RLF in source cell after receiving LTM candidate configurations, and successfully completes LTM execution with the selected LTM candidate cell.

For too early LTM, the following sub-cases are considered but we may down prioritize later (not limiting):

- -	Case 2a: the UE detects HOF/RLF in the LTM target cell and performs reestablishment procedure with the source cell.

- -	Case 2b: the UE detects HOF/RLF in the LTM target cell, selects the source cell which is also an LTM candidate cell, detects HOF with the source cell, and performs reestablishment procedure.

- -	Case 2c: the UE detects HOF/RLF in the LTM target cell, and successfully completes LTM execution with the selected source cell which is also an LTM candidate cell.

LTM to wrong cell, the following sub-cases are considered but we may down prioritize later (not limiting):

- -	Case 3a: the UE detects HOF/RLF in the LTM target cell and performs reestablishment procedure with the source cell.

- -	Case 3b: the UE detects HOF/RLF in the LTM target cell, selects an LTM candidate cell which is different from the source or target one, detects HOF with the selected LTM candidate cell, and performs reestablishment procedure.

- -	Case 3c: the UE detects HOF/RLF in the LTM target cell, and successfully completes LTM execution with the selected LTM candidate cell which is different from the source or target one.

RAN2 considers SHR, RA report and RLF for MCG LTM SON.

RAN2 will start work on MCG LTM.

### RAN2#126

We correct the definition of scenario 3a as follows: Case 3a: the UE detects HOF/RLF in the LTM target cell and performs reestablishment procedure in a cell other than the source cell and the target cell.

If available, log the L1 measurements for serving cell, target cell and other LTM candidate cells in RLF report, upon RLF or mobility failure.

Reuse the existing approach of using timeConnFailure and the reconnectCellId in RLF-report also for LTM failures, details TBD.

Log the LTM cell ID upon performing recovery an LTM candidate cell, details TBD e.g. which field.

Extend lastHO-Type in RLF-Report to indicate the LTM cell switch as last executed mobility procedure.

### RAN2#127

Only the field description associated to the timeConnFailure IE needs to be updated accordingly. Otherwise, we don’t expect any further specification impact for timeConnFailure and reconnectCellId is foreseen, TBC.

introduce a new field in RLF report to indicate the LTM recovery cell id.

RAN2 include the specific access type in the RLF report, i.e. whether it is RA-based or RA-less cell switch. FFS details, e.g. if explicit or implicitly signalled.

We do not include in the RLF report, the time since the last reception of PDCCH order towards the target cell till reception of cell switch command.

RA-based access will not be a new triggering condition for the SHR report.

HO interruption time will not be a new triggering condition for the SHR report

Reuse the existing approach of using timeUntilReconnection in RLF-report also for LTM failure scenarios.

We will not define and log timeSinceLTM-Reconfig (like timeSinceCHO-Reconfig) within RLF-report and SHR in LTM failure and near failure cases.

We will not log interruption time in SON reports (e.g. SHR) for LTM.

We aim to log some info to deduce the ltmCandidate (similar like choCandidate) in SHR to indicate whether a neighbour cell is an LTM candidate cell or not, TBD if explicit/implicit.

Log L3 measurements for serving cell, target cell and other LTM candidate cells in RLF report, upon RLF or mobility failure. RAN2 assumes this is already possible with existing spec.

### RAN2#127-bis

- The UE shall log cell IDs such as reestablishment cell ID, source, failed, reconnect cell ID, following the same principle as RLF, HOF and successful recovery, incl. the time between UE executing the LTM command and the failure.

- If RA-based LTM failure happens the UE logs and reports RACH info in the RLF report. Additional information is TBD.

- Unless RAN3 defines a NW-based solution: The UE logs and reports whether and how the UE got the TA value used for a failed LTM switch (gNB indicated or UE determined).

- Include an explicit indicator in SHR whether the successful LTM execution was RACH-less or RACH-based. Can sort out the details during stage-3 implementation.

- Unless RAN3 defines a NW-based solution: Introduce an explicit indication in RLF-Report to indicate whether a neighbour cell is an LTM candidate cell.

- UE logs available L1 measurement results for the serving cell, the target cell and other LTM candidate cells when a successful LTM cell switch triggers SHR.

### RAN2#128

- No need to include the specific access type (i.e. RACH-based or RACH-less) in the RLF report explicitly. It can be known implicitly. This can be revisited if RAN2 concludes that there are issues.

- Reuse the existing ra-InformationCommon for the RA-based LTM failure.

- If RAN3 does not address this meaning that we need a RAN2 solution, add a list indicating which LTM candidates the UE had at RLF.

### RAN2#129

Send an LS to RAN3 indicating scenario of beam failure recovery shortly after successful LTM (“near failure”), also say that we will not specify a UE based solution for this, but that RAN3 can consider a NW based solution if they want. RAN3 can see if/how the solution (if specified) could also be used for the case of LTM failure.

### RAN2#129-bis

- We will not add support for including the unnecessary LTM configuration, e.g. LTM candidate cell(s) unused, in SON reports, for both failure and near failure cases.

- Do not specify support for reporting difference between the TA value received from the LTM cell switch command and TA value measured by the UE

- Do not specify support for reporting of elapsed time between UE obtains the target cell TA and UE receives the cell switch command, or an indication to indicate whether the UE obtains TA before UE receives LTM cell switch command

- For LTM-related RA reports, the UE sets the cellId to the CGI if available, otherwise sets it based on ltm-CandidatePCI and ssb-Frequency in corresponding LTM-Candidate.

- Network is informed in the next RRC complete message about the SHR availability in case the UE performs RACH-less LTM cell switch. No need to change the current formulation of determining the SHR in the current running CR.

- Follow CHO like mechanism (i.e., a single rlf-Report for the RLF/HOF, including LTM recovery cell ID) for a consecutive LTM cell switch failure, i.e., an RLF/HOF at source or target cell followed by an HOF during LTM recovery. Relevant FFS can be removed from the running CR. No change is needed in the relevant text of running CR.

### RAN2#130

- We will not introduce UE reporting regarding TAs that were “unnecessarily” acquisitioned.

- We will not add TA difference between TA provided by network in Cell Switch Command for which the RACH-less LTM failed, and TA received in RAR for recovery/re-establishment.

- Introduce new values of raPurpose for RACH-based LTM-related RA reports.

- UE logs available L1 measurement results in the SHR for the serving, target and and neighbouring cells when the UE performs L3 handover and has an LTM configuration.

- We will not enhance MHI to include a “HO-type” (i.e., LTM or L3 HO).

## MRO enhancements for Rel-18 mobility features – CHO with candidate SCGs

### RAN2#125-bis

RAN2 to study failure and near failure scenarios for CHO with candidate SCGs.

### RAN2#126

RAN2 to enhance the RLF report with additional information regarding the state of the two execution conditions. We see later if we also can enhance the SCGFailureInformation report.

### RAN2#127

UE includes following information in RLF report:

- b.	Time information regarding condition fulfilment for CHO with candidate SCGs. Details are FFS. We consider both the case when both CHO condition and associated CPC condition are fulfilled, and the case when CHO (or CPC) is fulfilled but CPC (or CHO) conditions are not fulfilled.

- c.	Measurement results of PCells and PSCells.

### RAN2#127-bis

- UE reports the time gap between the first met condition (CHO or CPAC) and the second met condition (CPAC or CHO), and the first met execution condition (as agreed by RAN3), for a failed CHO with candidate SCGs. Details FFS.

- Include the elapsed time between the point in time of the first fulfilled condition and RLF in RLF report. Details FFS.

### RAN2#128

- RAN2 understands that current agreements is that the UE shall logs (in RLF report, SHR report and SCGFailureInformation) time from the last triggered event for the PCell (or PSCell) to the time to the last triggered event for the PSCell (or PCell). We don’t intend to do further or more granular enhancements.

- Measurement results of PCells and PSCells and the time information (as agreed for RLF) are included in SHR and SCGFailureInformation also. We will check what the spec impact of this is, e.g. something in the spec today may already make the UE log this.

### RAN2#129

Enhance RLF report for CHO with candidate SCGs to include the information for each CHO, i.e., first fulfilled event and time duration between two events fulfilled, if any.

Enhance RLF report for CHO with candidate SCGs to include the associations between CHO and CPAC.

Enhance RLF report for CHO with candidate SCGs to include at least the following information:

-	Identifier of candidate PCell(s) which met the configured CHO execution conditions when the RLF is encountered;

-	Identifier of candidate PSCell(s) which met the configured CPAC execution conditions when the RLF is encountered;

-	The Identifier of candidate PCell(s) or PSCell(s) that fulfilled execution conditions before the RLF is encountered.

Enhance SCGFailureInformation for CHO with candidate SCGs to include the information for each CHO, i.e., first fulfilled event and time duration between two events fulfilled, if any.

We should avoid specifying the procedure in a way that the UE sends redundant information

### RAN2#129-bis

- No new triggering conditions such as time gap between the first met condition (CHO or CPAC) and the second met condition (CPAC or CHO) is above a threshold, can be considered for SHR/SPR procedure.

- No new triggering conditions for SHR/SPR procedure for CHO with candidate SCG.

- UE includes the target PSCell ID in SHR for successful CHO with candidate SCGs.

- UE includes the target PCell ID in SPR for successful CHO with candidate SCG.

- In general, and where applicable, agreements valid for SHR, RLF reports and SCG failure info applies also to SPR.

Agreements

- For CHO with candidate SCGs, RAN2 to reuse the current stage-3 description, i.e.  include candidate PCell and candidate PSCell measurements results in neighbor measurements within RLF reports (or SCGFailureInformation).

- For CHO with candidate SCGs, RAN2 to clarify that the time duration between two fulfilled events (i.e., timeBetweenFulfillmen) shall only be included when both CHO and CPAC conditions are satisfied for failure cases.

- For CHO with candidate SCGs, RAN2 explicitly define a new lastHO-Type for CHO with candidate SCGs.

- For CHO with candidate SCGs, logging of elapsed time between fulfilling the last triggering event and handover execution in SHR is not required when only one condition (CHO or CPAC) is fulfilled.

- For CHO with candidate SCGs, RAN2 to agree to include “firstFulfilledConfig”, “timeBetweenFulfillment”, and “timeBetweenLastFulfillmentAndEvent” in the choWithCandidateSCGInfoList IE within the SCGFailureInformation message. Keep the current ASN.1 structure and no change are needed.

- FFS whether it is needed to avoid duplication of information in case of two reports being generated CHO with candidate SCGs, any redundancy (e.g., measurements) are recorded in the reports for PCell (i.e., in SHR, SPR).

## MRO enhancements for Rel-18 mobility features – S-CPAC

### RAN2#125-bis

No agreements.

### RAN2#126

RAN2 will look into failure and near failure scenarios for subsequent execution of SCPAC and see if/what enhancements are needed.

### RAN2#127

No agreements.

### RAN2#127-bis

No agreements.

### RAN2#128

No agreements.

### RAN2#129

No agreements.

## RACH Optimization for SDT

### RAN2#125-bis

No agreements.

### RAN2#126

For the purpose of SON enhancements for SDT, include RSRP/data volume related information

Downlink RSRP value and buffered uplink data volume at the time when the UE evaluates if it should perform SDT.

When SDT failure happens, the UE can indicate the failure cause of SDT to the network, e.g. T319a expiration. Details are TBD, e.g. if RSRP and data volume can also be included in such report.

### RAN2#127

Do not add logging of sdt-RSRP-Threshold, since already agreed by RAN3 to not support it.

UE logs and reports the failure cause for SDT to the network. FFS the details, e.g. if we down select some of the failure causes.

### RAN2#127-bis

- For failed SDT case, UE includes the DL RSRP and UL data volume at the time of SDT evaluation in SON report. For successful SDT procedure, the UE does not log.

- RAN2 understands for SON/MDT R19 the SDT enhancements only relate to the RA-SDT procedure. This does not rule out the case when the UE falls back from RA-SDT.

### RAN2#128

- The UE logs the DL RSRP and UL data volume for the failed to initiate of RA-SDT in RA report.

- The following failure causes can be logged for failed SDT in RA report:

⁻	upon receiving indication from the MCG RLC that the maximum number of retransmissions has been reached while SDT procedure is ongoing;

⁻	upon random access problem indication is received from MCG MAC while SDT procedure is ongoing;

⁻	upon T319a expires;

⁻	upon the lower layers indicate that cg-SDT-TimeAlignmentTimer or the configuredGrantTimer expired before receiving network response for the UL CG-SDT transmission with CCCH message while SDT procedure is ongoing;

⁻	FFS: unsuccessfully completed upon cell re-selection.

### RAN2#129

Include cell re-selection as a failure cause for failed RA-SDT in RA-Report

Introduce a new field in RA-Report to indicate the elapsed time since the execution of RA-SDT. Value in seconds. The maximum value is 172800 seconds.

## MHI Enhancements for SCG Deactivation/Activation

### RAN2#125-bis

No agreements.

### RAN2#126

No agreements.

### RAN2#127

It is beneficial for the network to have information about time spent in the PSCell in activated state vs. deactivated state.

Send an LS to RAN3 to ask if a NW solution is good enough to achieve the agreement above, or if a UE based solution is needed.

### RAN2#127-bis

No agreements.

### RAN2#128

No agreements.

### RAN2#129

We will introduce a UE based solution for SCG act/deact MHI, as an optional UE feature.

UE reports the time the UE has spent in PSCell with SCG activated, TBD details, e.g. percentage of time or absolute time, etc.

### RAN2#129bis

UE reports the absolute time it has spent in PSCell with SCG activated in MHI.

## MRO for MR-DC SCG failure

### RAN2#125-bis

No agreements.

### RAN2#126

Reply to RAN3 that we will only do EN-DC. RAN2 understands that whether also supporting (NG)EN-DC has no additional RAN2 impact hence RAN3 can decide. If later we get time we can consider other options.

### RAN2#127

To support MRO for SCG failure in EN-DC, enhance SCGFailureInformationNR message to include previousPSCellId, failedPSCellId, timeSCGFailure.

### RAN2#127-bis

- Add reporting of the following parameters for SCG failure report in EN-DC scenario:

⁻	For failedPSCellId and previousPSCellId: frequency and the PCI of the PSCell;

⁻	For timeSCGFailure: value range 0-1023;

⁻	For failureType: Reuse the legacy field.

-	perRA-InfoList

### RAN2#128

No agreements.

### RAN2#129

No agreements.

## SON/MDT for NTN

### RAN2#125-bis

No agreements.

### RAN2#126

No agreements.

### RAN2#127

No agreements.

### RAN2#127-bis

No agreements.

### RAN2#128

No agreements.

### RAN2#129

Existing IEs for CHO are used to report the fulfilled CHO trigger conditions before RLF occurs in case of “time and measurement based trigger condition” or “location and measurement based trigger condition”.

### RAN2#129-bis

1.     RAN2 assumes that the signaling structure agreed for encoding geographical area in case of MBS NTN can be used as a reference for encoding geographical area based logged MDT for NTN.

2.     Support reference location/radius and polygon-based area indication.

3.     Use dedicated signalling to provide area scope configuration to the UE

4.     Introduce a UE capability to support geographic area scope checking. FFS if with or without signalling.

5.     Only geographic area scope is used to indicate applicable logging area to the UE.

6.     Only if the UE can obtain location information in IDLE/Inactive state, the UE performs geographical area checking for logged MDT. FFS if, when the UE is in IDLE/INACTIVE, the UE logs or not if the UE cannot obtain its location.

### RAN2#130

- We go with option 2 (distance), unless we find critical issues with granularity.

- RAN2 to add logging of the first entry from trackingAreaList for the support of MRO for NTN.

- If configured with additional geographical information, FFS if the UE logs the MDT data or not when it cannot obtain its location.

- RAN2 will not work on unchanged PCI satellite mobility unless RAN3 tells us to do so.