3GPP TSG-RAN WG2 Meeting #131	Draft R2-2506346

# Bengaluru, India, August 25 - 29, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.306 | CR | 1920 | rev | 1 | Current version: | 18.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Introduction of LTE-based 5G Broadcast Phase 2 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon, Qualcomm Incorporated |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_terr_bcast_Ph2-Core |  |  |  |  |  | Date: |  |  | 2025-09-05 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Introduction of LTE-based 5G Broadcast Phase 2 to 36.306: The time-frequency interleavers for MBMS-dedicated cells is introduced in Rel-19.This CR is to capture the UE capabilities of time interleaving and frequency interleaving based on R1-2506427. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduce the UE capabilities for time interleaving and frequency interleaving. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | LTE-based 5G Broadcast Phase 2 is not supported. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.3.17 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS 36.211 CR 0576 TS 36.212 CR 0376 TS 36.213 CR 1448 TS 36.300 CR 1428 TS 36.331 CR 5143 TS 36.321 CR 1593 |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Start of Changes

### 4.3.17	MBMS parameters

#### 4.3.17.w	timeInterleaving-r19

This parameter defines, for the corresponding E-UTRA band, whether the UE supports MBMS reception from an MBMS-dedicated cell configured with time interleaving. With time interleaving, one TB is mapped to N non-consecutive subframes and two transmissions of the same TB are separated by (M-1) subframes. A UE that supports this feature shall support the following components:

- PMCH transmission pattern, excluding MCCH and MSI, with time interleaving for a set of PMCH numerologies indicated by xxx as specified in TS 36.331 [5];

- TBS determination for the scaled TB;

- determining the starting point for reading from the circular buffer (k0) for each subframe;

- extended MSI periodicities.

A UE which supports this feature shall also support fembmsDedicatedCell-r14 as specified in TS 36.331 [5].

#### 4.3.17.x	cyclicShiftFixedAlpha-r19

This parameter defines, for the corresponding E-UTRA band, whether the UE supports cyclic shift of PMCH with fixed alpha for the bit sequence in Section 6.3.1 of TS 36.211 for the ith subframe of the time-interleaved TB by X_i bits. A UE which supports this feature shall also support timeInterleaving-r19 as specified in TS 36.331 [5].

#### 4.3.17.y	cyclicShift-r19

This parameter defines, for the corresponding E-UTRA band, whether the UE supports cyclic shift of PMCH for the bit sequence in Section 6.3.1 of TS 36.211 for the ith subframe of the time-interleaved TB by X_i bits. A UE which supports this feature shall also support timeInterleaving-r19 as specified in TS 36.331 [5].

#### 4.3.17.z	frequencyInterleaving-r19

This parameter defines, for the corresponding E-UTRA band, whether the UE supports MBMS reception from an MBMS-dedicated cell configured with frequency interleaving. A UE that supports this feature shall support frequency-interleaving for MTCH for a set of PMCH numerologies indicated by xxx as specified in TS 36.331 [5].

A UE which supports this feature shall also support fembmsDedicatedCell-r14 as specified in TS 36.331 [5].

End of Changes