3GPP TSG-RAN WG2 #133	Tdoc R2-26xxxxx

Gothenburg, Sweden, Feb. 9th-13th 2026

Agenda Item:	x.x.x.x

Source:	Ericsson

Title:	[POST132][018][6G] ASN.1 structure

Document for:	Discussion, Decision

# 1	Introduction

This document captures the progress and outcome of the email discussion [POST132][018] on the 6G ASN.1 structure. According to the agreements at RAN2-132 it is supposed to “identify possible ways to address the issues agreed in the meeting and capture any additional observation”.

In the first phase of this email discussion companies should review the listed problems and observations. Comments should preferably be provided in the tables at the end of the respective sub-section. Companies may also add new sub-sections for problem areas that fit into the scope of this email discussion and that haven’t been mentioned yet.

Based on the contributions to RAN2-132 most companies seemed to be well aligned regarding the problems observed in 5G NR. Hence, the rapporteur proposes that the first phase ends by December 12th.

In the second phase this email discussion should aim to discuss and “identify possible ways to address the issues”. The second phase shall end by January 23rd the latest. However, companies are encouraged to provide and review inputs earlier and continuously to facilitate a fruitful discussion.

# 2	Contact Information

Please fill in the following table for contact information:

| Company | Email |
| --- | --- |
| Ericsson | Henning.Wiemann [at] ericsson (dot) com |
| Toyota ITC | Erik.Sunell [at] toyota (dot) com |

# 3	Problem areas

The following sub-sections describe problems that companies observed with the 5G ASN.1 structure and that they described in their contributions to RAN2-132.

## 3.1	Delta signalling

Several contributions (e.g. R2-2508618 (Huawei), R2-2508450 (Apple), R2-2508614 (Ericsson), R2-2508080 (Xiaomi), R2-2508115 (OPPO), R2-2508098 (CATT), R2-2508386 (InterDigital)…) observed that “the need code introducing additional restraints (e.g., Need S) and conditions (e.g. conditional presence) are the main causes of implementation complexity and compatibility issues in delta configuration”. Especially the textual rules whether and when a UE shall maintain or release fields makes it impossible to automate delta signalling. The required manual parsing makes the solution error prone.

It has also been mentioned (R2-2508614 (Ericsson), R2-2508618 (Huawei)) that these properties of NR’s delta signalling are particularly problematic for inter-node reconfigurations from network implementation perspective. The challenge is that the target node must be able to release branches of the source configuration which belongs to features that it does not support itself.

R2-2508406 (ZTE) highlighted the problem that NR’s AddMod/Release-Lists are difficult to implement and even more difficult to extend in subsequent releases. Extending the length or the individual elements tends to result in complex structures with parallel- or extension lists tends to result in implementation problems.

| Company Name | Comment on problem |
| --- | --- |
| Ericsson | We would like to echo what ZTE raised about the AddMod/Release lists. Those lists are a particularly important tool in the context of delta signalling since they were meant to allow the network to (re-)configure potentially long lists of potentially large elements in a signalling-efficient manner. But while AddMod/Release lists follow similar patterns and rules, their realizations are too different to automate their use in practice. And this is especially the case when those lists are extended (in size or element-type) in subsequent releases. We discovered e.g. related problems when trying to add or modify dedicated BWPs. In summary, we think that NR’s AddMod/Release lists suffer from the same ambiguity problem that several companies confirmed for the single-element cases (“Need M”, “Need S” and “Cond”). |
| Toyota ITC | We believe that the main limitation of Need codes and delta signalling is that Need codes are included as comment lines within the ASN.1 syntax rather than being integrated into the formal compiled ASN.1 schema. While the network side generally complies with these codes and restrictions, the UE implementation must still account for all possible encoding outcomes, including those that may never occur. Since signalling relies on the designer’s interpretation of these codes, errors and ambiguities still remain a risk. The current approach is based on failure handling, but recovery is never immediate. Such delays and errors can have serious consequences for critical applications, such as vehicle safety systems. Therefore, exploring alternative and more robust methods integrated in the formal ASN.1 schema would be beneficial and help these problems. The possibility of releasing unsupported source configuration branches suggests that the configuration release mechanisms should be improved but as of now we do not have any proposal. |

## 3.2	Conditional presence/absence rules

Several contributions (R2-2508112 (MediaTek), R2-2508649 (Toyota), R2-2508450 (Apple)…) raised that the specifications describe configuration constraints inconsistently, incompletely and ambiguously. And some companies mentioned that this may result in errors where “functionally mandatory UE configuration parameters can be absent in over-the-air RRC messages for initial configuration of a feature/functionality" or that "UE configuration parameters which shouldn't be modified after initial configuration of a feature/functionality can be sent in subsequent over-the-air RRC messages with new values”.

R2-2508386 (InterDigital) and R2-2508614 (Ericsson) observed that continued extensions of large IEs tend to make it ambiguous which combination of parameters and values is valid and thereby contributes to the above-mentioned ambiguities.

| Company Name | Comment on problem |
| --- | --- |
| Ericsson | We observe that RAN2 used conditions primarily for two reasons: 1) A constraint of the functionality being configured. For example, changing the RLC-SN could screw up the ARQ window and is hence forbidden unless the NW reestablishes the RLC entity. However, there are also cases (e.g. “SearchSpace-> controlResourceSetId -- Cond SetupOnly”) where it is not as obvious why the NW could not change the mapping in a later reconfiguration. It should be noted that there are other constraints which were not reflected in NR’s “Cond” tags: UE capabilities restrict also which fields the NW may include and which values it may set them to. What is the relation between “conditions” and “UE capabilities”? Any constraint that could differ among different UEs should be described (entirely?) in UE capabilities. RAN2 should avoid describing related constraints partially in conditions and partially in UE capabilities. 2) Re-use of (larger) IEs in several places where some fields within the IE may only be present in one context but not in others (e.g. PCell vs. SCell). RAN2 should consider to introduce separate IE types that are tailor-made for the specific purpose. It seems advisable to re-use only low-level (primitive) ASN.1 types. Of course, this comes also at an additional implementation cost for UE and NW and was therefore often avoided in NR. |
| Toyota ITC | We are willing to introduce additional, distinct information element types to address specific purposes, even if this results in a larger ASN.1 codebase. This approach would reduce reliance on textual descriptions and help maintain a concise and readable specification. One possible approach is leveraging ASN.1 sub-types with explicitly declared presence and absence. Extensions to information elements and messages should not be limited to non-critical extensions. Critical extensions should also be considered to avoid overly long, complex, and fragmented structures. The best practice is to evaluate extension types on a case-by-case basis for each Release, rather than applying a universal rule where only one extension type is used. |

## 3.3	Dependencies between common- and dedicated signalling

Several companies (R2-2508112 (MediaTek), R2-2508614 (Ericsson), …) consider dependencies between the common configuration (which the UE acquires via MIB/SIB1) and the dedicated configuration (which the gNB provides in the RRCReconfiguration) as a problem. R2-2508112 (MediaTek) explains that they “make the 5G UE configuration structure complex for both the UE and the network. From UE's perspective, the split of the UE configuration to common and dedicated parts has no functional significance”.

| Company Name | Comment on problem |
| --- | --- |
| Toyota ITC | We have no strong opinion on this topic, but the separation between common and dedicated configurations likely originates from UMTS legacy that continues to influence the current thinking. Given that the logical channel structure already clearly defines common and dedicated signalling, the types of information elements and configurations carried within these channels do not necessarily require such a distinction. |

## 3.4	Nested ASN.1 structure

Several companies (e.g. R2-2508874 (Samsung), R2-2508080 (Xiaomi), R2-2508386 (InterDigital), R2-2508139 (LG), R2-2508614 (Ericsson), R2-2508406 (ZTE), R2-2508450 (Apple)) indicated that NR’s deeply nested (tree-like) configuration structure makes future extensions and delta signalling difficult.

| Company Name | Comment on problem |
| --- | --- |
| Toyota ITC | We agree that the NR structures are deeply nested and complex. However, this issue assumes that 6GR ASN.1 will follow the same design methodology. Discussions around 6GR extension types and signalling restrictions, for example, may suggest different structuring approaches. |

## 3.5	Overhead due to IDs

NR uses IDs for to manage elements in AddMod/Release lists which are one of the main building blocks of NR’s delta signalling (see 3.1). Those IDs are also used to reference from one IE to another IE. While this is useful and efficient in principle, there are occasions where this led to substantial overhead.

| Company Name | Comment on problem |
| --- | --- |
| Ericsson | In our view, linking by references (IDs) is a much better practice than linking by hierarchy. Links by IDs can be multi-dimensional which isn’t possible in a hierarchical tree structure. However, if the IEs are small, the size of their ID (integer) can be significant compared to the size of their actual information. In addition, to the list where the elements and their IDs are defined, the IDs recur also in the IEs that refer to the elements. This may still be a good trade-off, if the (small) elements are defined once but re-used (= referenced) many times. But if that is not the case, such structure can be both complex and signalling-heavy. A prominent example is the CSI-RS resource configuration. CSI-RS resources, resource sets and resource configurations are small in actual size but refer to each other by means of IDs. We see examples in FR2 where this structure is by far the most substantial contributor (>50%) to the overall DL RRC message size. Smaller but still significant in size is the configuration of “TCI states”. They are associated with IDs and refer to SSBs or CSI-RSs by their respective IDs. And, like most of the above-mentioned CSI-RS configuration, they recur for each serving cell. Naturally, configuration flexibility comes with a cost in terms of complexity and overhead. But when designing 6G, RAN2 should look more carefully at real-world configuration examples, identify the actual overhead contributors and aim to eliminate those in 6G. The good news is that the configuration size of most of NR’s protocol layers and physical channels was already fairly small in practice! |
| Toyota ITC | We agree that linking with references is a better practice than linking by hierarchy because it decouples the hierarchy from the identities which is generally desirable. We should solve the problems first and then think about encoding size optimizations, not the other way around. |

## 3.6	…

# 4	Solution Directions

<TO BE ADDED FOR SECOND PHASE>

# 5	Conclusion

<TO BE ADDED WHEN THE EMAIL-DISCUSSION ENDS>

# References

Tdoc Number, Title, Source, Meeting, Date

Spec number, Title, Source, Version, Date