3GPP TSG-RAN WG2 #132	R2-250xxxx

# Dallas, USA, 17th – 21st November 2025

Agenda Item:	7.0.2.21

Source:	China Telecom

Title:	NCD-SSB configuration for serving cell in RRC_INACTIVE (outcome of [POST132][401][POS])

Document for: Discussion and Decision

Introduction

This document is the report of the following discussion:

- [Post132][401][POS] NCD-SSB configuration for serving cell in RRC_INACTIVE (China Telecom)

Scope: Evaluate what is needed to implement the RAN1 agreements on using NCD-SSB of the serving cell as a pathloss reference in RRC_INACTIVE.

Intended outcome: Agreeable CR

Deadline: Long

First round: invite companies to provide views on the information of the NCD-SSB of the serving cell as a pathloss reference in RRC_INACTIVE.

Deadline of first round: Thursday 2026-01-8 10:00 UTC

Second round: provide comments on the draft CRs based on the outcome of first round

Deadline of second round: Thursday 2026-01-22 10:00 UTC

Companies are invited to provide contact details on the table below.

| Company | Name | Email |
| --- | --- | --- |
| China Telecom | Hua Xu | xuh41@chinatelecom.cn |
| Samsung | Taeseop Lee | Taeseop.lee@samsung.com |
| CATT | Jianxiang Li | Lijianxiang@catt.cn |

Comments

After RAN2#132 meeting discussion, it’s agreed that the time offset for NCD-SSB of the serving cell needs to be available in RRCRelease for the benefit of a UE transitioning to RRC_INACTIVE.

Agreements:

RAN2 understand that under the RAN1 agreements for Rel-18, NCD-SSB of a neighbour cell is not supported as the reference signal for pathloss.

The time offset for NCD-SSB of the serving cell needs to be available in RRCRelease for the benefit of a UE transitioning to RRC_INACTIVE.  To be checked offline if some new parameter is needed to capture it.

Q1: Companies are invited to indicate whether any new parameters other than ssb-TimeOffset are required in RRCRelease to capture the necessary NCD-SSB information for use in RRC_INACTIVE.

| Company | Y/N | Comments |
| --- | --- | --- |
| Samsung | N | In our view, there is no need to introduce any new parameters including ssb-TimeOffset. The existing fields in SSB-Configuration-r16 IE (i.e., sfn-Offset-r16, integerSubframeOffset-r16, sfn-SSB-Offset-r16) seem enough to indicate the Tx timing of NCD-SSB. |
| CATT | N | There is no need to introduce any new parameters including ssb-TimeOffset. 1. According to the RAN1 parameter list in R1-2312697, it is clear that SSB-InfoNCell is updated to clarify NCD-SSB of serving cell can be configured in RRC Release. The existing RRC message and data structure already align with RRC parameter list. SRS for positioning in multiple cells within validity area for RRC_INACTIVE UE; PhysCellId; Update to the current description in 38.331: Indicates the physical cell ID of the neighbour cell or NCD-SSB of the serving cell for which SSB configuration is provided.; In SSB-InfoNcell in pathlossReferenceRS-Pos configured for srsPos resource sets in RRCRelease; Agreement For indication of the NCD-SSB as the pathloss reference RS for the positioning SRS resource set configured in the RRCRelease message, the fields PhysCellId and ssb-IndexNcell pertaining to the IE SSB-InfoNCell need to be updated to clarify NCD-SSB can be configured, from RAN1 perspective, for example, physicalCellId This field specifies the physical cell ID of the neighbour cell or NCD-SSB of the serving cell for which SSB configuration is provided. ssb-IndexNcell This field specifies the index of the SSB for a neighbour cell or of a NCD-SSB of the serving cell. See TS 38.213 [13]. If this field is absent, the UE determines the ssb-IndexNcell of the physicalCellId based on its SSB measurement from the cell.. SRS for positioning in multiple cells within validity area for RRC_INACTIVE UE; ssb-IndexNcell; Update to the current description in 38.331: Indicates the index of the SSB for a neighbour cell or of a NCD-SSB of the serving cell. See TS 38.213 [13]. If this field is absent, the UE determines the ssb-IndexNcell of the physicalCellId based on its SSB measurement from the cell.; In SSB-InfoNcell in pathlossReferenceRS-Pos configured for srsPos resource sets in RRCRelease. BTW, RAN1 doesn’t indicate the ssb-TimeOffset is required in their RRC Parameter list. 2. The SSB-Configuration-r16 in SSB-InfoNcell-r16 is a full configuration which can indicate NCD-SSB directly. There is no any reason to introduce any additional information to indicate NCD-SSB of serving cell. SSB-Configuration-r16 ::= SEQUENCE { ssb-Freq-r16 ARFCN-ValueNR, halfFrameIndex-r16 ENUMERATED {zero, one}, ssbSubcarrierSpacing-r16 SubcarrierSpacing, ssb-Periodicity-r16 ENUMERATED { ms5, ms10, ms20, ms40, ms80, ms160, spare2,spare1 } OPTIONAL, -- Need S sfn0-Offset-r16 SEQUENCE { sfn-Offset-r16 INTEGER (0..1023), integerSubframeOffset-r16 INTEGER (0..9) OPTIONAL -- Need R } OPTIONAL, -- Need R sfn-SSB-Offset-r16 INTEGER (0..15), ss-PBCH-BlockPower-r16 INTEGER (-60..50) OPTIONAL -- Cond Pathloss } SSB-InfoNcell-r16 ::= SEQUENCE { physicalCellId-r16 PhysCellId, ssb-IndexNcell-r16 SSB-Index OPTIONAL, -- Need S ssb-Configuration-r16 SSB-Configuration-r16 OPTIONAL -- Need S } ssb-Configuration This field specifies the full configuration of the SSB. If this field is absent, the UE obtains the configuration for the SSB from nr-SSB-Config received as part of DL-PRS assistance data in LPP, see TS 37.355 [49], by looking up the corresponding SSB configuration using the field physicalCellId. |

Rapp summary:

Additionally, R2-2508240 proposed to update the description of ssb-Ncell in order to align with SSB-InfoNCell field descriptions:

SSB-InfoNCell field descriptions

physicalCellId This field specifies the physical cell ID of the neighbour cell or NCD-SSB of the serving cell for which SSB configuration is provided.

ssb-IndexNcell This field specifies the index of the SSB for a neighbour cell or NCD-SSB of the serving cell. See TS 38.213 [13]. If this field is absent, the UE determines the ssb-IndexNcell of the physicalCellId based on its SSB measurement from the cell.

ssb-Configuration This field specifies the full configuration of the SSB. If this field is absent, the UE obtains the configuration for the SSB from nr-SSB-Config received as part of DL-PRS assistance data in LPP, see TS 37.355 [49], by looking up the corresponding SSB configuration using the field physicalCellId.

So the change on ssb-Ncell is:

ssb-Ncell This field indicates a SSB configuration from neighboring cell and the index of the SSB for a neighbour cell or the NCD-SSB for the serving cell.

Q2: Do companies agree the description of ssb-Ncell needs to be clarified or updated to explicitly cover both serving-cell and neighbour-cell cases?

| Company | Yes/No | Other comments |
| --- | --- | --- |
| Samsung | Yes |  |
| CATT | Proponent | According to the comment in Q1, only the description of ssb-Ncell is updated to align with RAN1 RRC parameter list in Rel-18. It seems this change would better work from Rel-18. Companies can share your comments if this CR should start from Rel-18. |

Rapp summary:

Conclusions