3GPP TSG-RAN WG3 Meeting #118	R3-22XXXX

Toulouse, France, 14th – 18th Nov 2022

Agenda item:	12.2.2.1

Source:	Samsung

Title:	TP to TS 38.423 for the procedure used for reporting of AI/ML related information for NG-RAN

Document for:	Discussion & Decision

# Introduction

This paper contains a TP for the procedure used for reporting of AI/ML related information.

# TP to TS 38.423

<<<<<<<<<<<<<<<<<<<< Start of the Change >>>>>>>>>>>>>>>>>>>>

Editor’s note: FFS on the names of new introduced procedures and messages.

## 8.1	Elementary procedures

In the following tables, all EPs are divided into Class 1 and Class 2 EPs.

**************** skip unchanged part *******************

| Resource Status Reporting Initiation | RESOURCE STATUS REQUEST | RESOURCE STATUS RESPONSE | RESOURCE STATUS FAILURE |
| --- | --- | --- | --- |
| Mobility Settings Change | MOBILITY CHANGE REQUEST | MOBILITY CHANGE ACKNOWLEDGE | MOBILITY CHANGE FAILURE |
| IAB Transport Migration Management | IAB TRANSPORT MIGRATION MANAGEMENT REQUEST | IAB TRANSPORT MIGRATION MANAGEMENT RESPONSE | IAB TRANSPORT MIGRATION MANAGEMENT REJECT |
| IAB Transport Migration Modification | IAB TRANSPORT MIGRATION MODIFICATION REQUEST | IAB TRANSPORT MIGRATION MODIFICATION RESPONSE |  |
| IAB Resource Coordination | IAB RESOURCE COORDINATION REQUEST | IAB RESOURCE COORDINATION RESPONSE |  |
| Partial UE Context Transfer | PARTIAL UE CONTEXT TRANSFER | PARTIAL UE CONTEXT TRANSFER ACKNOWLEDGE | PARTIAL UE CONTEXT TRANSFER FAILURE |
| AI/ML Information Reporting Initiation (FFS on the name) | AI/ML INFORMATION REQUEST (FFS on the name) | AI/ML INFORMATION RESPONSE (FFS on the name) | AI/ML INFORMATION FAILURE (FFS on the name) |

Table 8.1-2: Class 2 Elementary Procedures

| Elementary Procedure | Initiating Message |
| --- | --- |
| Handover Cancel | HANDOVER CANCEL |
| SN Status Transfer | SN STATUS TRANSFER |
| RAN Paging | RAN PAGING |
| Xn-U Address Indication | XN-U ADDRESS INDICATION |
| S-NG-RAN node Reconfiguration Completion | S-NODE RECONFIGURATION COMPLETE |
| S-NG-RAN node Counter Check | S-NODE COUNTER CHECK REQUEST |
| UE Context Release | UE CONTEXT RELEASE |
| RRC Transfer | RRC TRANSFER |
| Error Indication | ERROR INDICATION |
| Notification Control Indication | NOTIFICATION CONTROL INDICATION |
| Activity Notification | ACTIVITY NOTIFICATION |
| Secondary RAT Data Usage Report | SECONDARY RAT DATA USAGE REPORT |
| Trace Start | TRACE START |
| Deactivate Trace | DEACTIVATE TRACE |
| Handover Success | HANDOVER SUCCESS |
| Conditional Handover Cancel | CONDITIONAL HANDOVER CANCEL |
| Early Status Transfer | EARLY STATUS TRANSFER |
| Failure Indication | FAILURE INDICATION |
| Handover Report | HANDOVER REPORT |
| Resource Status Reporting | RESOURCE STATUS UPDATE |
| Access And Mobility Indication | ACCESS AND MOBILITY INDICATION |
| Cell Traffic Trace | CELL TRAFFIC TRACE |
| RAN Multicast Group Paging | RAN MULTICAST GROUP PAGING |
| SCG Failure Information Report | SCG FAILURE INFORMATION REPORT |
| SCG Failure Transfer | SCG FAILURE TRANSFER |
| F1-C Traffic Transfer | F1-C TRAFFIC TRANSFER |
| Retrieve UE Context Confirm | RETRIEVE UE CONTEXT CONFIRM |
| Conditional PSCell Change Cancel | CONDITIONAL PSCELL CHANGE CANCEL |
| AI/ML Information Reporting (FFS on the name) | AI/ML INFORMATION UPDATE (FFS on the name) |

### 8.4.AA AI/ML Information Reporting Initiation (FFS on the name)

#### 8.4.AA.1	General

This procedure is used by an NG-RAN node to request the reporting of AI/ML related information to another NG-RAN node.

The procedure uses non UE-associated signalling.

Editor’s Note: FFS other information that can be requested using this procedure.

Editor’s Note: FFS content of AL/ML related information.

#### 8.4.AA.2	Successful Operation

Figure 8.4.AA.2-1: AI/ML Information Reporting Initiation, successful operation

NG-RAN node1 initiates the procedure by sending the AI/ML INFORMATION REQUEST message to NG-RAN node2 to start AI/ML related  information reporting, stop AI/ML related  information reporting or add cells to report AI/ML related information. Upon receipt, NG-RAN node2:

-	shall initiate the requested AI/ML related information reporting according to the parameters given in the request in case the Registration Request IE is set to "start"; or

-	shall stop all cells AI/ML related information reporting and terminate the reporting in case the Registration Request IE is set to "stop"; or

-	shall add cells indicated in the Cell To Report List IE to the AI/ML related information reporting initiated before for the given Measurement IDs (FFS on the name), in case the Registration Request IE is set to "add". If AI/ML related information reporting is already initiated for a cell indicated in the Cell To Report List IE, this information shall be ignored.

If the Registration Request IE is set to "start" in the AI/ML INFORMATION REQUEST message and the Report Characteristics IE indicates cell specific AI/ML related information reporting, the Cell To Report List IE shall be included.

If Registration Request IE is set to "add" in the AI/ML INFORMATION REQUEST message, the Cell To Report List IE shall be included.

If NG-RAN node2 is capable to provide all requested information, it shall initiate the AI/ML related information reporting as requested by NG-RAN node1 and respond with the AI/ML INFORMATION RESPONSE message.

If the Reporting Periodicity IE in the AI/ML INFORMATION REQUEST is present, this indicates the periodicity for the reporting of periodic AI/ML related information. The NG-RAN node2 shall report only once, unless otherwise requested within the Reporting Periodicity IE.

#### 8.4.AA.3	Unsuccessful Operation

Figure 8.4.AA.3-1: AI/ML Information Reporting Initiation, unsuccessful operation

If any of the requested AI/ML related information reporting cannot be initiated, NG-RAN node2 shall send the AI/ML INFORMATION FAILURE message with an appropriate cause value.

#### 8.4.AA.4	Abnormal Conditions

FFS

### 8.4.BB	AI/ML Information Reporting (FFS on the name)

#### 8.4.BB.1	General

This procedure is initiated by an NG-RAN node to report AI/ML related information admitted by the NG-RAN node following a successful AI/ML Information Reporting Initiation procedure.

The procedure uses non UE-associated signalling.

Editor’s Note: FFS other information that can be reported using this procedure.

Editor’s Note: FFS content of AL/ML related information.

#### 8.4.BB.2	Successful Operation

Figure 8.4.11.2-1: AI/ML Information Reporting, successful operation

NG-RAN node2 shall report the admitted AI/ML related information in AI/ML INFORMATION UPDATE message. The admitted AI/ML related information is the information that was successfully initiated during the preceding AI/ML Information Reporting Initiation procedure.

#### 8.4.BB.3	Unsuccessful Operation

Not applicable.

#### 8.4.BB.4	Abnormal Conditions

Void

#### 9.1.3.CC	AI/ML INFORMATION REQUEST (FFS on the name)

This message is sent by NG-RAN node1 to NG-RAN node2 to initiate the requested AI/ML related information reporting according to the parameters given in the message.

Direction: NG-RAN node1  NG-RAN node2.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| NG-RAN node1 Measurement ID (FFS on the name) | M |  | INTEGER (1..4095,...) | Allocated by NG-RAN node1 | YES | reject |
| NG-RAN node2 Measurement ID (FFS on the name) | C-ifRegistrationRequestStoporAdd |  | INTEGER (1..4095,...) | Allocated by NG-RAN node2 | YES | ignore |
| Registration Request | M |  | ENUMERATED(start, stop, add, …) | Type of request for which the AI/ML related information is required. | YES | reject |
| Report Characteristics | C-ifRegistrationRequestStart |  | BITSTRING (SIZE(32)) | Each position in the bitmap indicates the object the NG-RAN node2 is requested to report. FFS on the coding | YES | reject |
| Cell To Report List |  | 0..1 |  | Cell ID list to which the request applies. | YES | ignore |
| >Cell To Report Item |  | 1 .. <maxnoofCellsinNG-RANnode> |  |  | – |  |
| >>Cell ID | M |  | Global NG-RAN Cell Identity 9.2.2.27 |  | – |  |
| Reporting Periodicity | O |  | ENUMERATED(500ms, 1000ms, 2000ms, 5000ms, 10000ms, …) | Periodicity that can be used for reporting of requested objects. Also used as the averaging window length for all objects if supported. | YES | ignore |

| Condition | Explanation |
| --- | --- |
| ifRegistrationRequestStoporAdd | This IE shall be present if the Registration Request IE is set to the value "stop" or "add". |
| ifRegistrationRequestStart | This IE shall be present if the Registration Request IE is set to the value "start". |

| Range bound | Explanation |
| --- | --- |
| maxnoofCellsinNG-RANnode | Maximum no. cells that can be served by a NG-RAN node. Value is 16384. |

#### 9.1.3.DD	AI/ML INFORMATION RESPONSE (FFS on the name)

This message is sent by NG-RAN node2 to NG-RAN node1 to indicate that the requested AI/ML related information, for all or parts of the objects included in the reporting is successfully initiated.

Direction: NG-RAN node2  NG-RAN node1

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| NG-RAN node1 Measurement ID (FFS on the name) | M |  | INTEGER (1..4095,...) | Allocated by NG-RAN node1 | YES | reject |
| NG-RAN node2 Measurement ID (FFS on the name) | M |  | INTEGER (1..4095,...) | Allocated by NG-RAN node2 | YES | reject |
| Reporting Characteristics | M |  | BITSTRING (SIZE(32)) | Each position in the bitmap indicates the object the NG-RAN node2 is able to report. FFS on the coding | YES | reject |
| Criticality Diagnostics | O |  | 9.2.3.3 |  | YES | ignore |

#### 9.1.3.EE	AI/ML INFORMATION FAILURE (FFS on the name)

This message is sent by the NG-RAN node2 to NG-RAN node1 to indicate that for any of the requested objects the reporting cannot be initiated.

Direction: NG-RAN node2  NG-RAN node1.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| NG-RAN node1 Measurement ID (FFS on the name) | M |  | INTEGER (1..4095,...) | Allocated by NG-RAN node1 | YES | reject |
| NG-RAN node2 Measurement ID (FFS on the name) | M |  | INTEGER (1..4095,...) | Allocated by NG-RAN node2 | YES | reject |
| Cause | M |  | 9.2.3.2 |  | YES | ignore |
| Criticality Diagnostics | O |  | 9.2.3.3 |  | YES | ignore |

#### 9.1.3.FF	AI/ML INFORMATION UPDATE (FFS on the name)

This message is sent by NG-RAN node2 to NG-RAN node1 to report the requested AI/ML related information.

Direction: NG-RAN node2  NG-RAN node1.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | ignore |
| NG-RAN node1 Measurement ID (FFS on the name) | M |  | INTEGER (1..4095,...) | Allocated by NG-RAN node1 | YES | reject |
| NG-RAN node2 Measurement ID (FFS on the name) | M |  | INTEGER (1..4095,...) | Allocated by NG-RAN node2 | YES | reject |
| Cell AI/ML Info Result |  | 1 |  |  | YES | ignore |
| >Cell AI/ML Info Result Item |  | 1 .. < maxnoofCellsinNG-RANnode > |  |  |  |  |
| >>Cell ID | M |  | Global NG-RAN Cell Identity 9.2.2.27 |  | – |  |
| >>Predicted Radio Resources Status | O |  | 9.2.2.50 |  | – |  |
| >>Predicted Number of Active UEs | O |  | 9.2.2.62 |  | –- |  |
| >>Predicted RRC Connections | O |  | 9.2.2.56 |  | – |  |
| >>FFS | O |  |  |  | - | - |

| Range bound | Explanation |
| --- | --- |
| maxnoofCellsinNG-RANnode | Maximum no. cells that can be served by a NG-RAN node. Value is 16384. |

<<<<<<<<<<<<<<<<<<<< End of the Change >>>>>>>>>>>>>>>>>>>>