SA WG2 Meeting #139E	draft S2-200xxxx

25 – 29 May 2020	(Revision of S2-200xxxx)

Source:	Samsung

Title:	Solution for KI#7: RFSP index based on Requested NSSAI

Document for:	Approval

Agenda Item:	8.8

Work Item / Release:	FS_eNS_Ph2 / Rel-17

Abstract of the contribution: This contribution proposes a solution for KI#7.

- 1. Proposal

It is proposed to adopt the following solution in TR.23.700-40 for KI #7.

*** 1st change (all new text) ***

## 6.X	Solution #<X>: RFSP index based on Requested NSSAI

### 6.X.1	Introduction

This solution addresses the Key Issue #7 in the perspective of supporting 5GC assisted cell selection to access network slice.

### 6.X.2	High-level Description

In Rel-15/Rel-16, the UE would have to attempt to select 5G-AN with no awareness as to whether a 5G-AN support a S-NSSAI. If the S-NSSAI in Requested NSSAI is not supported by the 5G-AN, the AMF doesn’t include the S-NSSAI in Allowed NSSAI. Meanwhile, AMF/PCF determines RFSP index by taking into account of the Allowed NSSAI so that the 5GC cannot steer the UE to a 5G-AN that can support the Requested NSSAI.

The solution proposes to take Requested NSSAI (i.e. including S-NSSAIs in Allowed NSSAI and rejected S-NSSAIs) into account when the AMF/PCF determines RFSP index.

### 6.X.3	Procedures

The Npcf_AMPolicyControl_Create message and Npcf_AMPolicyControl_Update message in TS 23.502 [6] are updated to include Requested NSSAI in order for the PCF to take the Requested NSSAI into account to determine RFSP index for the UE.

### 6.X.4	Impacts on existing services and interfaces

AMF:

- Takes Requested NSSAI into account when the AMF determines RFSP index.

- Provides Requested NSSAI to the PCF as an input for RFSP index decision.

PCF:

- Takes Requested NSSAI into account when the PCF determines RFSP index.

*** End of the changes ***