SA WG2 Meeting #139E	S2- 200xxxx

25 - 29 May, 2020, Elbonia 	(revision of S2-20xxxx)

Source: 	Ericsson

Title: 	KI#1, New Sol: Solution to support SNPN access using 3rd party credentials via external Credential Provider.

Document for: 	Agreement

Agenda Item: 	8.3

Work Item / Release:	FS_eNPN / Rel-17

Abstract of the contribution: This paper proposes a solution to address the Key Issue #1 on SNPN access using 3rd party credentials via external Credential Provider.

# Proposal

This pCR proposes to add the following solution to TR 23.700-07.

*** BEGIN CHANGES ***

## 6.0	Mapping Solutions to Key Issues

Table 6.0-1: Mapping of Solutions to Key Issues

|  | Key Issues |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Solutions | 1 | 2 | 3 | 4 | 5 | 6 |
| 1 | X |  |  |  |  |  |
| 2 | X |  |  |  |  |  |
| 3 | X |  |  |  |  |  |
| 4 | X |  |  |  |  |  |
| 5 |  |  |  | X |  |  |
| 6 |  |  |  | X |  |  |
| 7 |  |  |  | X |  |  |
| X | X |  |  |  |  |  |

*** NEXT CHANGES ***

## 6.X	Solution #X: SNPN access using 3rd party credentials via external Credential Provider

### 6.X.1	Introduction

This solution addresses key issue 1 "Enhancements to Support SNPN along with credentials owned by an entity separate from the SNPN ".

The solution enables UEs to access an SNPN which makes use of a credential management system managed by a credential provider external to the SNPN 5GS.

### 6.X.2	Functional Description

#### 6.X.2.1	Definitions

The following definitions apply to this solution:

SNPN: An SNPN which enables access for UEs using credentials owned by an entity separate from the SNPN.

Credentials Provider (CdP): An entity, separate from the SNPN that supports that its credentials are used to access an SNPN.

CdP-ID: Identifies the CdP that issued the credentials that a UE is using to access an SNPN.

#### 6.X.2.2	Architecture

Figure 6.x.2.2-1 depicts the architecture for the solution, i.e. the SNPN includes a complete 5GS SNPN network and the CdP provides credential management type of functionality.

Figure 6.x.2.2-1: Access to SNPN services using credentials from Credential Provider (CdP) for authentication in the SNPN

#### 6.X.2.3 High level principles of the solution

This solution enables UEs to access an SNPN which makes use of a credential management system managed by a credential provider external to the SNPN. The credential management functionality provided by the CdP includes handling of identifiers and corresponding security material used to identify the devices used within the SNPN and to mutually authenticate these devices and the SNPN 5GS. The credential provider will typically correspond with an already existing credential management system owned by the vertical owner of the SNPN 5GS.

The UE is provisioned with credentials managed by the CdP which include an identifier and related security information (e.g. certificates) and the CdP Identifier. The UE initiates registration in the SNPN using a SUPI based on the UE id provided by the CdP and provisioned in the UE.

The AMF initiates primary authentication, registration and subscription management procedures for the UE towards the AUSF and UDM of the SNPN based on existing procedures defined in 3GPP TS 33.501 [7]. It is assumed that there are AUSF and UDM instances within the SNPN supporting the SUPIs of the CdPs (e.g. SUPI ranges or CdP ID) the SNPN has agreements with.

For the primary authentication procedure, the UDM allows the UE to run primary authentication with credentials owned by a certain CdP. The UDM indicates to the AUSF to proceed with primary authentication involving the corresponding CdP.

-	One possibility is for the SNPN to delegate the authentication server role to the CdP (i.e. the CdP supports AAA functionality). In this case, the AUSF interacts with the CdP to execute the primary authentication procedure using the CdP credentials. The AUSF uses a AAA-P/IWF to interact with the CdP. The AAA-P/IWF undertakes any AAA protocol interworking between SBI services used by the AUSF and the AAA protocol supported by the CdP. This allows the AUSF to remain a full SBA entity within the 5GC architecture.

-	For some EAP methods, it could still be possible for the AUSF in the SNPN to remain as the EAP-Server. For example, for EAP-TLS the AUSF in the SNPN could behave as EAP-Server and the CdP could be used as Certificate Authority to check validity of the UE certificates.

For the registration and subscription management procedures, it is assumed that the SNPN has provisioned individual subscriptions for the UEs that use credentials from the CdP in the UDM/UDR. Alternatively, the SNPN may use common subscription profiles for these UEs.

NOTE:	Details of how subscriptions for UEs that use credentials from the CdP are provisioned/managed within the UDM/UDR are OOS of this solution.

### 6.X.3	Procedures

The following figure shows the interactions to enable UEs to access an SNPN which makes use of a credential management system managed by a Credential Provider external to the SNPN proposed in this solution.

Figure 6.X.3-1: UE registration in SNPN using credentials external to the SNPN

- The UE is configured with credentials from the CdP e.g. UE id, CdP ID and security information (e.g. certificates), and optionally a list of SNPNs that the CdP has an agreement/SLA with.

It is also assumed that there are AUSF and UDM instances within the SNPN that support the SUPIs of the CdPs (e.g. SUPI ranges or CdP ID) the SNPN has agreements with. The AMF selects these AUSF/UDM instances based on information locally configured in the AMF or provided by the SNPN NRF.

- The UE selects the SNPN and initiates UE registration in the SNPN. The UE creates a SUCI based on the UE id provided by the CdP and provisioned in the UE.

NOTE 1:	It is assumed that the UE id provisioned by the CdP is on NAI format and includes also the CdP ID in the domain part of the NAI, e.g. UEID@CdPID. Whether the SUPI within the SUCI is encrypted is FFS in SA3.

- The AMF within the SNPN initiates primary authentication for the UE using a Nausf_UEAuthentication_Authenticate service operation with the AUSF as currently specified in 3GPP TS 33.501 [7]. The AMF selects an AUSF based on the SUCI presented by the UE as specified in 3GPP TS 23.501 [4].

- The AUSF checks with UDM within the SNPN for the authentication method to be executed for the UE using a Nudm_UEAuthentication_Get service operation as currently specified in 3GPP TS 33.501 [7]. The AUSF selects a UDM also using the SUCI provided by the AMF as specified in 3GPP TS 23.501 [4]. The UDM resolves the SUCI to the SUPI before checking the authentication method applicable for the UE.

- The UDM provides the AUSF with the UE SUPI and the applicable authentication method for the UE. In this case, the UDM indicates to the AUSF to run primary authentication with credentials owned by a certain CdP. The UDM provides the AUSF also with the address of the CdP-AAA if required.

NOTE 2:	The SNPN may support credentials managed by different CdPs.

- The AUSF interacts with the CdP to execute the primary authentication procedure. If the SNPN delegates the authentication server role to the CdP, the AUSF uses a AAA-P/IWF to interact with the CdP. Otherwise the AUSF remains as the authentication server but may still contact the CdP for other purposes (e.g. certificate validation check).

- The UE executes the applicable EAP authentication method with the AUSF or the CdP.

- After successful authentication, the AMF is provided with the successful indication together with the SUPI of the UE and the resulting security Kseaf.

NOTE 3:	Details of the authentication procedure are for FFS in SA3. This includes potential impacts on key hierarchy when using AUSF as authenticator instead of EAP server and how UE is aware of key hierarchy to be used.

- After successful authentication, the AMF continues with the registration procedure for the UE in the SNPN. The AMF selects a UDM based on the UE’s SUPI to perform AMF registration and subscription management procedures with UDM. The SNPN may have provisioned individual subscriptions for the UEs that use credentials from the CdP in the UDM/UDR. Alternatively, the SNPN may use common subscription profiles for these UEs.

NOTE 4:	The details of how subscriptions for UEs that use credentials from the CdP are provisioned/managed within the UDM/UDR are OOS of this solution.

- The AMF completes the registration procedure in the SNPN. The security keys result from the primary authentication procedure using CdP credentials are used for subsequent security procedures within the SNPN (i.e. Security Mode Command).

### 6.X.4	Impacts on existing entities and interfaces

UE impacts:

-	CdP Credentials storage.

-	SUPI/SUCI generation based on CdP-UE Id

-	Possible impact on key hierarchy (to be confirmed by SA3).

AMF impacts:

None identified.

AUSF impacts:

-	Interworking with CdP. In case SNPN delegates authenticator role to the CdP, the AUSF behaves as EAP Authenticator role (i.e. forwarding auth requests to CdP acting as EAP server). Otherwise the AUSF may still contact the CdP for other purposes (e.g. certificate validation check).

-	Possible impact on key hierarchy (to be confirmed by SA3).

UDM impacts:

-	Possible impact on SUCI to SUPI de-concealment (to be confirmed by SA3).

-	Indication of EAP method using external CdP.

AAA-IWF/P impacts:

-	New NF supporting protocol interworking with CdP acting as e.g. external EAP server.

*** END CHANGES ***