3GPP TSG-SA2 Meeting #143-E 	S2-200YYYY

# Online, 24th Feb 2021 – 9th Mar 2021

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.502 | CR | YYYY | rev | - | Current version: | 16.7.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | DCAMP related update of BSF services (23.502) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Oracle Corporation |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI17_DCAMP |  |  |  |  |  | Date: |  |  | 2020-02-24 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | This paper proposes DCAMP related update of BSF services. The purpose of the update is to: 1. Allow discovery of PCF-AM by an AF/NEF (This covers the case where the UE is already registered at the time when the AF/NEF wishes to submit DCAMP related requests to PCF-AM). 2. Allow discovery of PCF-SM by PCF-AM (This covers the case where the UE already has an SM Policy Association for the relevant S-NSSAI/DNN at the time when the PCF-AM receives DCAMP related request from the NEF/AF (either directly or via the UDR). Consequently, this will allow PCF-AM to subscribe at the PCF-SM for detection of ‘start/stop of application’ event). 3. Allow BSF to notify PCF-AM about a newly registered PCF-SM (This covers the case where the UE does not have an SM Policy Association for the relevant S-NSSAI/DNN at the time when the PCF-AM receives DCAMP related request). |  |  |  |  |  |  |  |  |
| Summary of change: |  | 1. Update of Nbsf_Management_Register and Nbsf_Management_Discovery, such that it will support registration and discovery of a PCF-AM. 2. Add new service operations to Nbsf_Management service. The new service operations are subscribe, unsubscribe and notify. This will allow PCF-AM to get notified about a new PCF-SM serving the combination of a SUPI, DNN, S-NSSAI. 3. Add PCF-AM as a consumer of the BSF for the sake of discovery (or subscription for the status) of PCF-SM). Key changes: 1. PCF registration type attribute is added to PCF registration at the BSF. The value of this attribute can be ‘PCF-SM’ or ‘PCF-AM’. (This will indicate to the BSF whether the registration is PDU Session related (PCF-SM) or whether it is Access and Mobility related (PCF-AM)) 2. The category of some attributes (eg UE address) is changed from mandatory to conditional based on the registration type (eg In Nbsf_Management_Register service operation UE address is a required input attribute only if PCF registration type is ‘PCF-SM’. (The reason for this change is: so far we used to have only PCF-SM performing registration at the BSF. Now we also have PCF-AM performing registration at the BSF) |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | DCAMP functionality will not work properly, because: 1. PCF-AM will not be able to get notified when a new PCF-SM serves a combination of a SUPI, DNN, S-NSSAI. Consequently, PCF-AM will not know which PCF-SM to contact for sake of getting notified when a DCAMP related application starts or stops. 2. AF/NEF will not be able to discover PCF-AM, which serves an AM Association for a SUPI or GPSI. Consequently, AF/NEF will not be able to influence PCF-AM decisions for RFSP index value and/or SAR when specific services are in use by a UE. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2.13.1, 5.2.13.2.1, 5.2.13.2.2, 5.2.13.2.3, 5.2.13.2.4, 5.2.13.2.6, 5.2.13.2.7, 5.2.13.2.8 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

1st change

### 5.2.13	BSF Services

#### 5.2.13.1	General

The following table shows the BSF Services and Service Operations:

Table 5.2.13.1-1: NF services provided by the BSF

| Service Name | Service Operations | Operation Semantics | Example Consumer(s) |
| --- | --- | --- | --- |
| Nbsf_management | Register | Request/Response | PCF |
|  | Deregister | Request/Response | PCF |
|  | Discovery | Request/Response | NEF, AF, NWDAF, PCF |
|  | Update | Request/Response | PCF |
|  | Susbcsribe | Subscribe/Notify | PCF |
|  | Unsubscribe | Subscribe/Notify | PCF |
|  | Notify | Subscribe/Notify | PCF |

#### 5.2.13.2	Nbsf_Management service

##### 5.2.13.2.1	General

The Nbsf provides the Nbsf Management Register, Nbsf Management Deregister , the Nbsf Management Discovery, the Nbsf Management Update, the Nbsf Management Subscribe, the Nbsf Management Unsubscribe and the Nbsf Management Notify service operations.

##### 5.2.13.2.2	Nbsf_Management_Register service operation

Service Operation name: Nbsf_Management_Register

Description: Registers the tuple (UE address(es), SUPI, GPSI, DNN, S-NSSAI, PCF address(es), PCF instance id, PCF Set ID, level of Binding, PCF registration type) for an SM Policy Association or for an AM Policy Association.

NOTE 1: In some cases only subset of these parameters may be registered (eg UE address(es) will be registered only if PCF registration is for an SM Policy Association).

Inputs, Required: UE address(es) [Conditional, if PCF registration is for an SM Policy Association], SUPI [Conditional, if PCF registration is for AM Policy Association], GPSI [Conditional, if PCF registration is for AM Policy Association and if available], PCF address(es).

UE address can contain IP address/prefix or MAC address as defined in TS 23.501 [2]. It can optionally include Framed Route information. W-5GAN specific UE IP address information is specified in TS 23.316 [53].

Framed Route information is defined in Table 5.2.3.3.1-1.

NOTE 4:	For TSN scenarios the UE address contains the DS-TT port MAC address.

Inputs, Optional: DNN, SUPI, GPSI, S-NSSAI, PCF instance ID and PCF Set ID, level of Binding (see clause 6.3.1.0 of TS 23.501 [2]), PCF registration type.

NOTE 2:	PCF registration type can be either PCF-SM or PCF-AM.

NOTE 3:	The default value of PCF registration type is PCF-SM.

NOTE 5:	DNN and S-NSSAI are not applicable in the case where PCF registration type is PCF-AM.

Outputs, Required: Result indication, Binding Identifier for an SM Policy Association or for an AM Policy Association.

Outputs, Optional: None.

##### 5.2.13.2.3	Nbsf_Management_Deregister service operation

Service Operation name: Nbsf_Management_Deregister

Description: Removes the binding information for an SM Policy Association or for an AM Policy Association.

Inputs, Required: Binding Identifier for an SM Policy Association or for an AM Policy Association.

W-5GAN specific UE IP address information are specified in TS 23.316 [53].

Inputs, Optional:

Outputs, Required: Result indication.

Outputs, Optional: None.

##### 5.2.13.2.4	Nbsf_Management_Discovery service operation

Service Operation name: Nbsf_Management Discovery

Description: Discovers the PCF and PCF set selected for the tuple (UE address(es), SUPI, GPSI, DNN, S-NSSAI) in case where PCF for an SM Policy Association is to be discovered or the tuple (SUPI, GPSI) in case where PCF for an AM Policy Association is to be discovered.

Inputs, Required: UE address (i.e. IP address or MAC address) [Conditional if PCF discovery type is PCF-SM and if consumer is AF or NEF], DNN [Conditional, if PCF discovery type is PCF-SM and if available], S-NSSAI [Conditional, if PCF discovery type is PCF-SM and if available], SUPI [Conditional, if PCF discovery type is PCF-AMand if available], GPSI [Conditional, if PCF discovery type is PCF-AM and if available].

NOTE 1:	PCF discovery type can be either PCF-SM or PCF-AM.

NOTE 2:	The default value of PCF discovery type is PCF-SM.

NOTE 3:	For TSN scenarios the UE address contains the DS-TT port MAC address.

NOTE 4:	if PCF discovery type is PCF-AM at least one of the parameters SUPI or either GPSI has to be provided.

Inputs, Optional: SUPI, GPSI, PCF discovery type].

Outputs, Required: PCF address(es), PCF instance ID [Conditional, if available] and PCF Set ID [Conditional, if available], level of Binding [Conditional, if available] (see clause 6.3.1.0 of TS 23.501 [2]), UE address(es) (i.e. IP address(es) or MAC address(es)) [Conditional if PCF discovery type is PCF-SM and if consumer is PCF-AM].

Outputs, Optional: None.

##### 5.2.13.2.5	Nbsf_Management_Update service operation

Service Operation name: Nbsf_Management_Update

Description: Replaces the list of UE address(es) for an SM Policy Association  or replace PCF id for an SM Policy Association or for an AM Policy Association.

NOTE:	For example, PCF-2 may update its PCF id in cases where level of binding is NF Instance and PCF-1 fails and PCF-2 is the new NF Instance handling the SM Policy Association or the AM Policy Association.

Inputs, Required: Binding Identifier for the PDU Session.

UE address can contain IP address/prefix or Ethernet address as defined in TS 23.501 [2].

NOTE:	For TSN scenarios the UE address contains the DS-TT port MAC address.

Inputs, Optional: UE address(es), PCF id, PCF address(es).

Outputs, Required: Result indication.

Outputs, Optional: None.

##### 5.2.13.2.6	Nbsf_Management_Susbcribe service operation

Service Operation name: Nbsf_Management Susbcribe

Description: PCF-AM can subscribe to be notified of newly registered PCF-SM or of Deregistered PCF-SM.

Inputs, Required: SUPI [Conditional if available], GPSI [Conditional if available], callback URI.

NOTE:	At least one of the parameters SUPI or either GPSI has to be provided.

Inputs, Optional: DNN, S-NSSAI,

Outputs, Required: When the subscription is accepted: Subscription Correlation ID.

Outputs, Optional: None.

##### 5.2.13.2.7	Nbsf_Management_Unsusbcribe service operation

Service Operation name: Nbsf_Management Unsusbcribe

Description: PCF-AM can undo a previous subscribtion.

Inputs, Required: Subscription Correlation ID.

Inputs, Optional: None.

Outputs, Required: result.

Outputs, Optional: None.

##### 5.2.13.2.8	Nbsf_Management_Notify service operation

Service Operation name: Nbsf_Management Notify

Description: BSF can notify PCF-AM of newly registered PCF-SM or of Deregistered PCF-SM.

Inputs, Required: Notification Correlation Information, UE address(es), PCF address(es), PCF instance ID [Conditional, if available] and PCF Set ID [Conditional, if available], level of Binding [Conditional, if available] (see clause 6.3.1.0 of TS 23.501 [2]).

Inputs, Optional: None.

Outputs, Required: None.

Outputs, Optional: None.

End of changes