# 3GPP TSG-SA3 Meeting #115, Athens	S3-24xxxx

# 26 Feb – 01 Mar 2024

# Source:	Johns Hopkins University APL, US National Security Agency

# Title:	Updates to skeleton TR 33.757

# Document for:	Approval

Agenda Item:	5.3

# Decision/action requested

This pCR is proposing to add a new Key Issue for IPSec child security association coordination into TR 33.757

# 2	References

[1]	3GPP TS 33.501: “Security architecture and procedures for 5G system”

# 3	Rationale

The purpose of this pCR is to propose adding a new Key Issue for IPSec child security association coordination into TR 33.757.

# 4.	Detailed proposal

It is suggested to approve the following change.

**** START OF CHANGE ****

# 5	Key issues

## 5.X	Key Issue #X: IPSec Child-SA policy coordination between PLMN and NPN customers

### 5.X.1	Key issue details

### TS 33.501 [1] clause 9.1.3 requires a Child-SA for each QoS traffic class. IPSec configurations will reside in separate operational domains of the PLMN and PNI-NPN, therefore inter-domain coordination of Child-SA policies is required to support QoS for User Plane interfaces between PLMN and PNI-NPN customer domains.

### 5.X.2	Security Threats

### Incompatible IPSec Child-SA policy configuration between PLMN and PNI-NPN customers may result in failure of Child-SA associations, thereby exposing PLMN and PNI-NPN to operating without the required Child-SAs.

### 5.X.3	Potential security requirements

### 5GS should include Child-SA policy coordination across PLMN and PNI-NPN customer domains.

**** END OF CHANGE ****