3GPP TSG-SA3 Meeting #111 	S3-23xxxx

# Berlin, Germany, 22 -26 May 2023					(revision of S3-yyxxxx)

# Source:	Ericsson

# Title:	Update to conclusion for KI#2

# Document for:	Approval

Agenda Item:	5.3

# 1	Decision/action requested

This paper proposes an update to conclusion for KI#2.

# 2	References

[1]	3GPP TR 23.700-33

[2]	3GPP TR 33.740

# 3	Rationale

This paper proposes an update to conclusion for KI#2.

It is proposed to clarify that the conclusions apply as well to PC5 link establishment for Direct Communication with integrated discovery.

# 4	Detailed proposal

**** START OF CHANGE ****

# 7	Conclusions

## 7.2	Key Issue #2: Security of UE-to-UE Relay

For Key Issue #2, the following statements are agreed:

NOTE 1:	The choice and co-existence of the security mechanisms in different use cases (i.e., U2U Relay in and out of coverage) will be decided in normative phase.

Regarding End-to-End security:

- For L2 relay, the End UEs (Source UE and/or Target UE) reuse the unicast mode security mechanism defined in clause 5.3.3.1 of TS 33.536 [6] to establish a secure connection via the Relay UE.

- For L3 relay, End-to-End security can be supported in the application layer which is out of 3GPP scope.

Regarding hop-by-hop security during PC5 link establishment in Layer 3/Layer 2 UE-to-UE Relay Communication:

- When the Layer 3/Layer 2 UE-to-UE Relay is in 3GPP coverage, the End UE and the UE-to-UE Relay can establish a secure PC5 link with network assistance. The similar security procedure as PC5 security for 5G ProSe Communication via 5G ProSe Layer-3 UE to-Network Relay as defined in clause 6.3 in TS 33.503 [6] can be reused.

- The solutions for UE-to-UE Relay authorisation and security can be classified as user-plane (UP) or controlled-plane (CP) based solutions. It is concluded that both control plane and user plane solutions are supported for 5G ProSe Layer 2 UE-to-UE Relay and 5G ProSe Layer 3 UE-to-UE relay.

- When the Layer 3/Layer 2 UE-to-UE Relay is out of 3GPP coverage, the End UE and the UE-to-UE Relay can establish a secure PC5 link without network assistance. The similar security procedure as PC5 security for unicast mode 5G ProSe Direct Communication as defined in clause 6.2.3 in TS 33.503 [6] can be reused.

The conclusions above apply as well to PC5 link establishment for Direct Communication with integrated discovery.

**** END OF CHANGE ****