3GPP TSG-SA3 Meeting #111 	S3-23xxxx

# Berlin, Germany, 22 -26 May 2023

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.512 | CR | <CR#> | rev | <Rev#> | Current version: | 17.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification of NSSAA revocation |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Federal Office for Information Security (BSI) |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S3 |  |  |  |  |  |  |  |  |  |
| Work item code: | eSCAS_5G |  |  |  |  |  | Date: |  |  | 2023-05-08 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | The UE state (REGISTERED) required for the test case is not present in the Pre-Conditions. The Requirement Description mentions, that the Deregistration Request message shall include the list of rejected S-NSSAIs, each of them with the appropriate rejection cause value. This is currently missing and should be part of the Expected Results. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added UE state to Pre-conditions. Added required content of Deregistration Request to Expected Results. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The state of the UE may not be correct while testing. Requirement Description is not covered fully. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.2.2.9.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

********** START OF 1st CHANGE **********

##### 4.2.2.9.1	NSSAA revocation

Requirement Name: NSSAA revocation

Requirement Reference: TS 33.501 [7], clause 16.5

Requirement Description: " If no S-NSSAI is left in Allowed NSSAI for an access after the revocation, and no Default NSSAI can be provided to the UE in the Allowed NSSAI or a previous NSSAA failed for the Default NSSAI over this access, then the AMF shall execute the Network-initiated Deregistration procedure for the access as described in subclause 4.2.2.3.3 in TS 23.502 [8], and it shall include in the explicit De-Registration Request message the list of Rejected S-NSSAIs, each of them with the appropriate rejection cause value. "

as specified in TS 33.501[7], clause 16.5

Threat References: TR 33.926, clause K.2.X

Test Case:

Test Name: TC_NSSAA_REVOCATION

Purpose:

Verify that AMF deregisters UE when, after slice specific authorization revocation, there is no allowed NSSAI or Default NSSAI that can be used by UE.

Pre-Conditions:

-	Test environment with UE. The UE may be simulated.

-	The UE is registered at the AMF.

-	The AMF under test is configured with one S-NSSAI in the Allowed NSSAI and no default S-NSSAI.

Execution Steps

A message requesting the AMF under test to revoke the authorization of the S-NSSAI in the Allowed NSSAI is simulated and sent the AMF under test.

Expected Results:

The Deregistration Request message is sent by the AMF under test to the UE.

The Deregistration Request message includes the list of rejected S-NSSAIs, each of them with the appropriate rejection cause value.

Expected format of evidence:

Evidence suitable for the interface, e.g., Screenshot containing the operational results.

NOTE 1:	This test case is only applicable to AMF supporting Network Slice Specific Authentication and Authorization.

********** END OF CHANGE **********