3GPP TSG-SA3 Meeting #111 	S3-23xxxx

# Berlin, Germany, 22 - 26 May 2023

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.514 | CR | XXXX | rev | - | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | SCAS release reference corrections |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S3 |  |  |  |  |  |  |  |  |  |
| Work item code: | SCAS_5G_Ph2 |  |  |  |  |  | Date: |  |  | 2023-05-22 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | SA3 has been adding the release numbers explicitly to any of the references pertaining to the network function targeted by the SCAS work, for example reference 2 in TS 33.511. This is because the SCAS work has always been one "release late" since it is challenging to develop the SCAS requirements and tests in parallel to targeted new features within the same release timeline. The references have not been regularly updated and some SCAS specifications include more than one reference to the same specification, for example references 2 and 7 in TS 33.512. This practice is neither future proof nor it is documented anywhere. Furthermore, for SCAS evaluation of network products, this dependency on previous releases in SCAS documents turned out to be not very useful anyway. This issue has been discussed several times in previous SA3 meetings and the proposed resolution is documented in S3-231050. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Removal of the release number from the relevant references and minor reformulations to avoid verbatim content copies from other specifications |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Unnecessary dependencies on previous releases and risk for confusion on scope of SCAS specifications |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2, 4.2.1.1, 4.2.2.1, 4.2.2.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

**** Start of Changes****

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 33.501: "Security architecture and procedures for 5G system".

[3]	3GPP TS 33.117: "Catalogue of general security assurance requirements".

[4]	3GPP TR 33.926 "Security Assurance Specification (SCAS) threats and critical assets in 3GPP network product classes".

[5]	3GPP TS 23.501: "System Architecture for the 5G System (5GS)".

[6]	3GPP TS 23.003: "Numbering, addressing and identification".

[x]	3GPP TS 33.102: "3G security; Security architecture".

**** Next Changes****

#### 4.2.1.1	De-concealment of SUPI from the SUCI based on the protection scheme used to generate the SUCI.

Requirement Name: De-concealment of SUPI from the SUCI based on the protection scheme used to generate the SUCI.

Requirement Reference: TS 33.501 [2], clause 5.8.2.

Requirement Description: The SIDF is expected to resolve the SUPI from the SUCI based on the protection scheme used to generate the SUCI as specified in TS 33.501 [2], clause 5.8.2.

Threat References: TR 33.926 [4], clause E.2.2.1, Incorrect SUCI de-concealment.

TEST CASE:

Test Name: TC_DE-CONCEAL_SUPI_from_SUCI_UDM

Purpose:

Verify that the SIDF De-conceals the SUPI from the SUCI based on the protection scheme used to generate the SUCI.

Procedure and execution steps:

Pre-Condition:

-	UDM network product is connected in simulated/real network environment including an AUSF and AMF.

-	Tester shall have access to the subscription data stored in UDR.

-	Tester shall record the SUPI from the UE.

Execution Steps:

Test case:

Tester shall capture the entire authentication procedure between UE and AMF over N1, N12 and N13 interface using any network analyser.

1.	Tester shall filter the Nudm_Authentication_Get Response message sent from UDM to AUSF over N13 interface containing the SUPI.

2.	Tester shall compare the SUPI gotten from UE and the SUPI retrieved from Nudm_Authentication_Get Response message.

NOTE:	The tester may filter the Nausf_UEAutentication_Authenticate Response message sent from the UDM/AUSF to the AMF over N12 interface containing the SUPI, if the UDM and AUSF network products are collocated without an open N13 interface.

Expected Results:

SIDF resolves the SUPI from the SUCI based on the protection scheme used to generate the SUCI.

Expected format of evidence:

Evidence suitable for the interface, e.g., evidence can be presented in the form of screenshot/screen-capture.

**** Next Changes****

#### 4.2.2.1	Synchronization failure handling

Requirement Name: Synchronization failure handling

Requirement Reference: TS 33.501 [2], clause 6.1.3.3.2.

Requirement Description: When the UDM/ARPF receives an Nudm_UEAuthentication_Get Request message with a "synchronisation failure indication" it acts as described in TS 33.102 [x], clause 6.3.5 where ARPF is mapped to HE/AuC. The UDM/ARPF sends an Nudm_UEAuthentication_Get Response message with a new authentication vector for either EAP-AKA' or 5G-AKA depending on the authentication method applicable for the user to the AUSF as specified in TS 33.501 [2], clause 6.1.3.3.2.

Threat References: TR 33.926 [4], clause E.2.2.2, Synchronization failure.

Test Case:

Purpose:

Verify that synchronization failure is recovered correctly in the home network.

Pre-Conditions:

Test environment with an AUSF. The AUSF or AMF may be simulated.

Execution Steps

1.	The AUSF sends an Nudm_UEAuthentication_Get Request message to the UDM with a "synchronisation failure indication" and parameters RAND and AUTS.

2.	The UDM/ARPF performs steps 1-5 as described in TS 33.102, clause 6.3.5.

Expected Results:

The UDM sends an Nudm_UEAuthentication_Get Response message with a new authentication vector to the AUSF.

NOTE:	The expected results would be that the UDM/AUSF sends an Nausf_UEAuthentication_Authenticate Response message with EAP Request/AKA’-Challenge for EAP AKA’, or 5G SE AV for 5G AKA to the AMF, if the UDM and AUSF network products are collocated without an open N13 interface.

**** Next Changes****

#### 4.2.2.2	Storing of authentication status of UE by UDM.

Requirement Name: Storing of authentication status of UE by UDM.

Requirement Reference: TS 33.501 [2], clause 6.1.4.1a

Requirement Description: The UDM is expected to store the authentication status of the UE (SUPI, authentication result, timestamp, and the serving network name) after authentication as specified in TS 33.501 [2], clause 6.1.4.1a.

Threat References: TR 33.926 [4], clause E.2.2.3, Failure to store of authentication status.

TEST CASE:

Test Name: TC_AUTH_STATUS_STORE_UDM

Purpose:

Verify that the UDM under test stores the authentication status of UE, which is identical to the UE authentication information sent to/from the AUSF and the AMF.

Procedure and execution steps:

Pre-Condition:

-	UDM network product is connected with an AUSF in simulated/real network environment involving AMF, eNB.

-	The tester shall have access to all the authentication specific data sent over N1 interface, N12 interface and N13 interface.

-	The tester shall have access to the UDM under test.

Execution Steps:

1.	The tester shall capture the entire authentication procedure and authentication confirmation procedure over N12 and N13 interface using any network analyser.

2.	the tester shall filter the Nudm_UEAuthentication_Get Request message sent over the N13 interface to retrieve serving network name.

3.	The ester shall filter the Nudm_Authentication_Get Response message sent over N13 interface to find the SUPI.

4.	The tester shall filter the Nausf_UEAuthentication_Authenticate Response message sent over N12 interface to retrieve the Authentication result (EAP success/failure for EAP-AKA' or Result for 5G AKA).

5.	The tester shall filter the Nudm_UEAuthentication_ResultConfirmation Request message to retrieve the authentication result and time of authentication procedure sent from the AUSF to the UDM over N13 interface.

6.	The tester shall compare the serving network name stored in the UDM against the serving network name retrieved from the Nudm_Authentication_Get Request message and the serving network name retrieved from the Nudm_UEAuthentication_ResultConfirmation Request message.

7.	The tester shall compare the authentication status stored in the UDM against the authentication result retrieved from N12 interface.

8.	The tester shall compare the SUPI stored in the UDM against the SUPI retrieved from the Nudm_Authentication_Get Response message and the SUPI  retrieved from the Nudm_UEAuthentication_ResultConfirmation Request message.

9.	The tester shall compare the timestamp stored in the UDM against the time of authentication procedure retrieved from the Nudm_UEAuthentication_ResultConfirmation Request message.

Expected Results:

The storing of authentication status (SUPI, authentication result, timestamp, and the serving network name) of UE at the UDM is verified.

Expected format of evidence:

Evidence suitable for the interface, e.g., evidence can be presented in the form of screenshot/screen-capture.

NOTE:	this test case does not apply to the deployment scenario where the UDM and AUSF network products are collocated without an open N13 interface.

**** End of Changes****