3GPP TSG-SA3 Meeting #116	S3-24xxxx

Jeju, South Korea,  20th - 24th May 2024

- Source:	Samsung

- Title:	Conclusion for KI#1 in TR 33.701

- Document for:	Approval

Agenda Item:	5.6

# 1	Decision/action requested

It is proposed to approve this contribution to conclude KI#1.

# 2	References

[1]	3GPP TR 33.701 Study on mitigations against bidding down attacks

# 3	Rationale

It is proposed to approve this conclusion for key issue#1.

# 4	Detailed proposal

*** Start of Change ***

# 6	Conclusions

## 6.X	Conclusion for Key issue#1

1. If the HPLMN does not support GERAN/UTRAN:

The UICC is pre-configured with the information on restricted RAT types (i.e., GERAN/UTRAN) for the entire HPLMN. If the information is available in the UICC, then the UE excludes the combination of HPLMN ID and restricted RAT types of the HPLMN for any PLMN/Cell (re)selection procedures.

2. If the UE is in the VPLMN and/or if the PLMN performs phased decommissioning of GERAN/UTRAN:

During attach or registration procedure the network indicates to the UE about the information on restricted RAT types (i.e., GERAN/UTRAN) in attach or registration accept message for the TAI or included list of TAIs or for the entire PLMN. If the information is available in the UE, then the UE excludes the combination of PLMN ID, TAI (if indicated) and restricted RAT types of the PLMN for any PLMN/Cell (re)selection procedures.

*** End of Change ***