3GPP TSG-SA3 Meeting #117	S3-242xxx

Maastricht, Netherlands  19 - 23 August 2024

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.511 | CR | DraftCR | rev | - | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | New test for vendor certificate checking at gNB |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei; HiSilicon, CAICT(?), CMCC(?),CTCC(?), Nokia(?), Keysight(?) |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S3 |  |  |  |  |  |  |  |  |  |
| Work item code: | SCAS_5G_Maint |  |  |  |  |  | Date: |  |  | 2024-08-17 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Addition of a new test for local certificate checking that corresponds to a new threat introduced in a parallel contribution S3-242325 to the gNB network product class annex in TR 33.926. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Addition of a new testcase for local certificate checking by gNB. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incomplete SCAS coverage of the certificate enrolment functionality. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.2.2.1.x(new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

************* 1st of Change*************

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 33.501: "Security architecture and procedures for 5G system".

[3]	3GPP TS 33.117: "Catalogue of general security assurance requirements".

[4]	Void

[5]	3GPP TR 33.926: "Security Assurance Specification (SCAS) threats and critical assets in 3GPP network product classes".

[6]	3GPP TS 38.331: "NR; Radio Resource Control (RRC) protocol specification".

[7]	3GPP TS 23.501: "System Architecture for 5G System (5GS)".

[8]	3GPP TS 38.300: "NR and NG-RAN Overall Description".

[9]	3GPP TS 33.523: "5G Security Assurance Specification (SCAS); split gNB product classes".

[10]	3GPP TS 33.310: "Network Domain Security (NDS); Authentication Framework (AF)".

*********** End of 1st of Change***********

************* 2nd of Change*************

4.2.2.1.x	Vendor certificate checking

Requirement Name: vendor certificate checking at base station

Requirement Reference: In accordance with industry best practice

Requirement Description:

The gNB shall have the capability to check the certificate that is preconfigured by a vendor.

Test case:

Test Name: TC_LOCAL_CERT_CHCK

Purpose:

Verify that the base station can check the certificate that is configured by the vendore is valid or not.

Procedure and execution steps:

Pre-Conditions:

-	The gNB under test supports handling certificate as defined in clause 9 of TS 33.310[x].

-	A UPF or AMF or a security gateway may be real/emulated serving as a peer for the gNB network product.

-	The necessary documentation describing how to handle the case when a gNB checks the certificate preconfigured by vendoer is invalid.

Execution Steps

1. The tester configures the gNB with a certificate provided by vendor.

2. The tester creates a situation that the certificate is invalid, for example, by modifying the certificate or modifying the configuration of gNB.

3. The tester may need to trigger a connection establishement with the peer.

Expected Results:

-	The gNB reacts as described in the document.

Expected format of evidence:

The logs and the communication flow in a .pcap file.

************* End of Change*************