3GPP TSG-SA3 Meeting #123	S3-25aaaa

- Goteborg, Japan, 25 - 29 August 2025

Source:	Sony, OPPO?, Lenovo?, QualComm Inc?, Huawei?,…???

Title:	Procedure for Protection of AIoT device identifier privacy

Document for:	Approval

Agenda item:	4.1.1

Spec:	3GPP TS 33.369

Version:	0.2.0

Work Item:	AIOT_Sec

Comments

Based on the conclusion in TR 33.713 the following text is proposed.

Proposed Changes

* * * First Change * * * *

## 5.4	Protection of AIoT device identifier privacy

### 5.4.1	Procedure for AIoT Device identifier protection during Group Inventory

Figure 5.4.1-1:  Privacy protection of AIoT device ID during Group Inventory

1.	Step 1-6 of clause 6.2.2 (Procedure for Inventory) or clause 6.2.3 (Procedure for command) of TS 23.369 [2] is performed. The AIoT_Inventory request or the AIoT_Command_request contains the filtering information.

Editor’s Note: issues related filtering information is FFS.

2.	The AIOTF requests the ADM to derive RANDAIOT_n. The ADM provides the AIOTF the RANDAIOT_n as specified in clause 5.2.2..

Editor’s Note: How to address paging all case (i.e. paging message does not contain filtering information) is FFS.

Editor’s Note: whether and how to address Replay attack (e.g., replay RANDAIOT_n to track the same T-ID) or DoS attack (e.g., energy depletion in AIoT device) is FFS.

3.	The AIoTF shall include the filtering information and RANDAIOT_n in an Inventory Request and send the Inventory Request to the NG-RAN.

4.	The NG-RAN shall send a Paging message containing the filtering information and RANDAIOT_n.

5.	Upon receiving the Inventory request, the AIoT device shall check if the filtering information matches with the configured filtering information. If they match, the AIoT device shall compute a T-ID as specified in clause X.Z. If the filtering information does not match, the AIoT device shall stop further processing of the Inventory Request, i.e., the rest of the steps are not performed.

6.	The AIoT device shall include the T-ID in an D2R message and send it to the NG-RAN.

7.	The NG-RAN shall send an Inventory Report containing the T-ID(s) to the AIOTF.

8.	Upon receiving the Inventory response, the AIOTF shall forward the T-ID(s), filtering information and RANDAIOT_n to the ADM. The ADM compute the list of T-IDs for the AIoT device IDs as specified in clause X.Z and find a matching T-ID(s). If there is a match, the ADM shall provide the AIoT Device ID to the AIOTF.

9.	The AIOTF shall construct an AIoT_Inventory Notify including the AIoT device ID that corresponds to the received T-ID. The AIOTF continues with the procedures as specified in clause 6.2.2 or clause 6.2.3 of TS 23.369 [2].

* * * Second Change * * * *

# X.Z	Temporary Identity derivation function

When deriving a new temporary identity Temp ID_n+1 from KAIOT, the following parameters shall be used to form the input S to the  KDF:

-	FC = 0xCC,

-	P0 = RANDAIOT_n,

-	L0 = length of RANDAIOT_n (i.e. 0x00  0x10),

-	P1 = AIoT device ID.

-	L1 = length of AIoT device ID (i.e. 0x00  0x10),

The input key KEY shall be KAIOT.

* * * End of Changes * * * *