| 3GPP TR 33.858 V0.3.0 (2022-11) |  |
| --- | --- |
| Technical Report |  |
| 3rd Generation Partnership Project; Technical Specification Group Services and System Aspects; Study on security aspects of enhanced support of Non-Public Networks phase 2 (Release 18) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet http://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2022, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	5

Introduction	6

1	Scope	7

2	References	7

3	Definitions of terms, symbols and abbreviations	8

3.1	Terms	8

3.2	Symbols	8

3.3	Abbreviations	8

4	Assumptions	8

5	Key issues	8

5.1	Key issue #1: Security of non-3GPP access for SNPN	8

5.1.1	Key issue details	8

5.1.2	Threats	9

5.1.3	Potential security requirements	9

5.2	Key issue #2: Authentication for UE access to hosting network	9

5.2.1	Key issue details	9

5.2.2	Threats	9

5.2.3	Potential security requirements	9

5.X	Key issue #X: <Title>	9

5.X.1	Key issue details	9

5.X.2	Threats	9

5.X.3	Potential security requirements	9

6	Proposed solutions	10

6.0	Mapping of solutions to key issues	10

6.1	Solution #1: Authentication mechanism for untrusted non-3GPP Access in SNPN scenarios	10

6.1.1	Introduction	10

6.1.2	Solution details	10

6.1.3	System impact	11

6.1.4	Evaluation	11

6.2	Solution #2: Authentication mechanism for trusted non-3GPP Access in SNPN scenarios	11

6.2.1	Introduction	11

6.2.2	Solution details	11

6.2.3	System impact	12

6.2.4	Evaluation	12

6.3	Solution #3: Use of anonymous SUCI in trusted non-3GPP access for SNPN	12

6.3.1	Introduction	12

6.3.2	Solution details	12

6.3.3	System impact	12

6.3.4	Evaluation	13

6.4	Solution #4: Authentication for devices that do not support 5GC NAS over WLAN access in SNPN scenarios	13

6.4.1	Introduction	13

6.4.2	Solution details	13

6.4.3	System impact	13

6.4.4	Evaluation	13

6.5	Solution #5: Anonymous authentication during connection establishment in trusted non-3GPP network access.	14

6.5.1	Introduction	14

6.5.2	Solution details	14

6.5.3	System impact	14

6.5.4	Evaluation	14

6.6	Solution #6: Trusted non-3GPP Access for SNPN	14

6.6.1	Introduction	14

6.6.2	Solution details	14

6.6.3	System impact	15

6.6.4	Evaluation	15

6.7	Solution #7: Untrusted non-3GPP Access for SNPN	15

6.7.1	Introduction	15

6.7.2	Solution details	15

6.7.3	System impact	15

6.7.4	Evaluation	15

6.8	Solution #8: Reusing Existing N3GPP Security for SNPN	15

6.8.1	Introduction	15

6.8.2	Solution details	16

6.8.3	Evaluation	16

6.9	Solution #9: NSWO support in SNPN using any key-generating EAP-method	16

6.9.1	Introduction	16

6.9.2	Solution details	17

6.9.3	System impact	17

6.9.4	Evaluation	18

6.10	Solution #10: Access to localized services using existing mechanisms	18

6.10.1	Introduction	18

6.10.2	Solution details	18

6.10.2.1 	Solution for access to localized services based on Home Network Credentials	18

6.10.2.2 	Solution for access to localized services based on Onboarding Mechanism	18

6.10.3	System Impact	21

6.10.4	Evaluation	21

6.11	Solution #11: High-level solution on authentication for UE access to hosting network	21

6.11.1	Introduction	21

6.11.2	Solution details	21

6.11.3	System impact	22

6.11.4	Evaluation	22

6.12	Solution #12: Localised service authentication through onboarding procedure and registration afterwards.	22

6.12.1	Introduction	22

6.12.2	Solution details	22

6.12.3	System impact	23

6.12.4	Evaluation	23

6.13	Solution #13: Home network primary authentication – secondary authentication towards localised service	23

6.13.1	Introduction	23

6.13.2	Solution details	23

6.13.3	System impact	24

6.13.4	Evaluation	24

6.A	Solution #A: <Title>	24

6.A.1	Introduction	24

6.A.2	Solution details	24

6.A.3	System impact	24

6.A.4	Evaluation	24

7	Conclusions	25

Annex <A>: <Informative annex title for a Technical Report>	26

Annex X: Change history	26

# Foreword

This Technical Report has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

In the present document, modal verbs have the following meanings:

shall		indicates a mandatory requirement to do something

shall not	indicates an interdiction (prohibition) to do something

The constructions "shall" and "shall not" are confined to the context of normative provisions, and do not appear in Technical Reports.

The constructions "must" and "must not" are not used as substitutes for "shall" and "shall not". Their use is avoided insofar as possible, and they are not used in a normative context except in a direct citation from an external, referenced, non-3GPP document, or so as to maintain continuity of style when extending or modifying the provisions of such a referenced document.

should		indicates a recommendation to do something

should not	indicates a recommendation not to do something

may		indicates permission to do something

need not	indicates permission not to do something

The construction "may not" is ambiguous and is not used in normative elements. The unambiguous constructions "might not" or "shall not" are used instead, depending upon the meaning intended.

can		indicates that something is possible

cannot		indicates that something is impossible

The constructions "can" and "cannot" are not substitutes for "may" and "need not".

will		indicates that something is certain or expected to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

will not		indicates that something is certain or expected not to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

might	indicates a likelihood that something will happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

might not	indicates a likelihood that something will not happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

In addition:

is	(or any other verb in the indicative mood) indicates a statement of fact

is not	(or any other negative verb in the indicative mood) indicates a statement of fact

The constructions "is" and "is not" do not indicate requirements.

# Introduction

This clause is optional. If it exists, it shall be the second unnumbered clause.

# 1	Scope

The aim of this work is to study the security aspects for any potential enhancements to be developed based on the outcome of the study in TR 23.700-08 [2]. For each of the objectives in the scope of the study in TR 23.700-08 [2], potential security aspects that are to be covered in this study are as follows:

-	Support for enhanced mobility by enabling support for idle and connected mode mobility between SNPNs without new network selection.

-	Study if existing security mechanisms for mobility between PLMNs can be reused for SNPNs or if new security mechanisms are needed.

-	Support for non-3GPP access for SNPN

-	Study if existing security mechanisms for enabling non-3GPP access in a PLMN can be reused for enabling non-3GPP access in an SNPN or if new security mechanisms are needed.

-	Address new requirements (e.g., TS 22.261 [3] requirements for Providing Access to Local Services) related to NPN

-	Study the trust model for the resulting architecture for enabling Localized Services via a local hosting NPN.

-	Study if existing mechanisms for a UE to access an NPN can be reused for enabling a UE to authenticate with and access the local hosting NPN and the localized services via the hosting NPN with proper authorization, or if new security mechanisms are needed.

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TR 23.700-08: "Study on enhanced support of Non-Public Networks; Phase 2".

[3]	3GPP TS 22.261: "Service requirements for the 5G system".

[4]	3GPP TS 33.501: "Security architecture and procedures for 5G system"

[5]	IETF RFC 7296: "Internet Key Exchange Protocol Version 2 (IKEv2)"

[6]	3GPP TS 23.501: "System architecture for the 5G System (5GS)"

[7]	3GPP TS 23.502: "Procedures for the 5G System (5GS)"

…

[x]	<doctype> <#>[ ([up to and including]{yyyy[-mm]|V<a[.b[.c]]>}[onwards])]: "<Title>".

# 3	Definitions of terms, symbols and abbreviations

This clause and its three subclauses are mandatory. The contents shall be shown as "void" if the TS/TR does not define any terms, symbols, or abbreviations.

## 3.1	Terms

For the purposes of the present document, the terms given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

For the purposes of the present document, the following terms and definitions given in TR 23.700-08 [2] apply:

Local service, Localized service: Service, which is localized (i.e. provided at specific/limited area) and/or can be bounded in time. The service can be realized via applications (e.g. live or on-demand audio/video stream, electric game, IMS, etc), or connectivity (e.g. UE to UE, UE to Data Network, etc.).

Hosting network: A network providing access to Local/Localized services.

## 3.2	Symbols

For the purposes of the present document, the following symbols apply:

Symbol format (EW)

<symbol>	<Explanation>

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

Abbreviation format (EW)

<ABBREVIATION>	<Expansion>

# 4	Assumptions

This clause contains assumptions for the study. If there are no assumptions at the end of the study, the clause will be removed before sending for approval.

# 5	Key issues

## 5.1	Key issue #1: Security of non-3GPP access for SNPN

### 5.1.1	Key issue details

TR 23.700-08 [2] studies "Key Issue #2: Support of Non-3GPP access for SNPN". Clause 5.2.1 of TR 23.700-08 [2] states: "Currently the 3GPP specifications do not support direct connection to SNPN via non-3GPP access networks" and "One objective of this key issue is to enable the 5GS to support direct connection of non-3GPP access networks to the SNPN's 5GC."

The intention of this key issue is to study if existing security mechanisms for enabling non-3GPP access in a PLMN can be reused for enabling non-3GPP access in an SNPN, or if new security mechanisms are needed.

### 5.1.2	Threats

If non-3GPP access in an SNPN does not provide mutual authentication between UE and SNPN, it is possible to impersonate the UE or SNPN.

If communication between UE and SNPN via non-3GPP access is not confidentiality, integrity or replay-protected, it is possible to disclose, tamper or replay the communication.

### 5.1.3	Potential security requirements

The 5G system shall provide the means for UE and SNPN to mutually authenticate if non-3GPP access is used.

The 5G system shall provide the means to confidentiality, integrity and replay protect communication between UE and SNPN, if non-3GPP access is used.

Editor's Note: Threats and requirements for devices that are not UEs (e.g. FN-RG or N5GC device behind RG) are ffs.

## 5.2	Key issue #2: Authentication for UE access to hosting network

### 5.2.1	Key issue details

The terms "localized service" and "hosting network" are explained in clause 3.1 of this document.

TR 23.700-08 [2] studies "Key Issue #3: Enabling NPN as hosting network for providing access to localized services" and "Key Issue #4: Enabling UE to discover, select and access NPN as hosting network and receive localized services".

The intention of this key issue is to study authentication of UE access to a hosting network, if existing security mechanisms can be reused or new security mechanisms are needed.

### 5.2.2	Threats

If the UE is not authenticated towards the network, it is possible to impersonate the UE.

If the network is not authenticated towards the UE, it is possible to impersonate the network.

### 5.2.3	Potential security requirements

The UE and the hosting network shall support mutual authentication between the UE and the network.

## 5.X	Key issue #X: <Title>

### 5.X.1	Key issue details

### 5.X.2	Threats

### 5.X.3	Potential security requirements

# 6	Proposed solutions

## 6.0	Mapping of solutions to key issues

Table 6.0-1: Mapping of solutions to key issues

| Solutions | KI#1: Security of non-3GPP access for SNPN | KI#2: Authentication for UE access to hosting network |
| --- | --- | --- |
| Solution #1: Authentication mechanism for untrusted non-3GPP Access in SNPN scenarios | X |  |
| Solution #2: Authentication mechanism for trusted non-3GPP Access in SNPN scenarios | X |  |
| Solution #3: Use of anonymous SUCI in trusted non-3GPP access for SNPN | X |  |
| Solution #4: Authentication for devices that do not support 5GC NAS over WLAN access in SNPN scenarios | X |  |
| Solution #5: Anonymous authentication during connection establishment in trusted non-3GPP network access | X |  |
| Solution #6: Trusted non-3GPP Access for SNPN | X |  |
| Solution #7: Untrusted non-3GPP Access for SNPN | X |  |
| Solution #8: Reusing Existing N3GPP Security for SNPN | X |  |
| Solution #9: NSWO support in SNPN using any key-generating EAP-method | X |  |
| Solution #10: Access to localized services using existing mechanisms |  | X |
| Solution #11: High-level solution on authentication for UE access to hosting network |  | X |
| Solution #12: Localised service authentication through onboarding procedure and registration afterwards. |  | X |
| Solution #13: Home network primary authentication – secondary authentication towards localised service |  | X |

## 6.1	Solution #1: Authentication mechanism for untrusted non-3GPP Access in SNPN scenarios

### 6.1.1	Introduction

This solution addresses key issue #1.

TR 23.700-08 [2] studies "Key Issue #2: Support of Non-3GPP access for SNPN". Clause 5.2.1 of TR 23.700-08 [2] states: "Currently the 3GPP specifications do not support direct connection to SNPN via non-3GPP access networks" and "One objective of this key issue is to enable the 5GS to support direct connection of non-3GPP access networks to the SNPN's 5GC."

To access to SNPN via no-3GPP access, on the one hand, the UE may need to handle the new identity (e.g., onboarding SUCI/onboarding SUPI) and the new Registration Type (i.e.  SNPN Onboarding) to N3IWF. On the other hand, the UE may use anonymous value SUCI during the registration procedure, which will result in N3IWF failing to locate KN3IWF to authenticate the identity of UE.

This solution is proposed to address the aforementioned problem and enables the UE to access SNPN via an untrusted non-3GPP access network.

### 6.1.2	Solution details

This solution reuses the untrusted non-3GPP access authentication procedure in PLMN scenarios in clause 7.2.1 of TS 33.501 [4] with the following modifications:

-	In SNPN scenarios, if the construction of SUCI as described in clause 6.12 of TS 33.501 [4] cannot be used and if the employed EAP method supports SUPI privacy, then the UE can send an anonymous value SUCI to N3IWF based on configuration. And SNPN identifier, which consists of PLMN ID and NID, should also be included in AN parameters, which are sent to the N3IWF. Moreover, to fulfill the onboarding requirements, the UE may also send onboarding SUCI to the N3IWF.

-	The AMF can choose 5G AKA, EAP-AKA', or any other key-generating EAP authentication method to authenticate UE as described in clause 6.1.3 or clause I.2.2 of TS 33.501 [4].

-	If EAP-AKA' or key-generating EAP authentication method is used for authentication as described in clause 6.1.3.1 and clause I.2.2 of TS 33.501 [4], the AUSF shall include the EAP-Success in step 7.

Editor’s Note: The need for including SUPI privacy case is FFS.

### 6.1.3	System impact

TBD

### 6.1.4	Evaluation

TBD

## 6.2	Solution #2: Authentication mechanism for trusted non-3GPP Access in SNPN scenarios

### 6.2.1	Introduction

This solution addresses key issue #1.

Specifically, in SNPN scenarios, the UE may register for onboarding, therefore the UE needs to send new registration type to TNAN. Moreover, TNGF may need to leverage IDi to identify KTNGF, which can authenticate the identity of the UE. And IDi can be set as the SUCI/onboarding SUCI. However, in some cases, the UE may send anonymous value SUCI to TNGF in the registration request, making TNGF not able to locally link the identity of the UE with the corresponding KTNGF. Without the mapping between UE identity and KTNGF, TNGF cannot authenticate the identity of the UE.

To access to SNPN, on the one hand, the UE may provide the new identity (e.g. onboarding SUCI/onboarding SUPI) and the new Registration Type (i.e.  SNPN Onboarding) to TNAN. On the other hand, the UE may use anonymous value SUCI during the registration procedure, which results in the consequence that TNGF cannot locate KTNGF to authenticate the identity of UE.

This solution enables the UE to access SNPN via trusted non-3GPP access network.

### 6.2.2	Solution details

This solution reuses the authentication mechanism in clause 7A.2.1 of TS 33.501 [4] with the following modifications.

-	The UE may send SUCI/onboarding SUCI to the TNAP/TNGF. If the construction of SUCI as described in clause 6.12 of TS 33.501 [X] cannot be used and if the employed EAP method supports SUPI privacy, then the UE can send an anonymous value SUCI to TNAP/TNGF. If the UE indents to access SNPN, AN parameters should also include SNPN identifier, which consists of PLMN ID and NID. The authentication mechanisms could be 5G AKA, EAP-AKA’, and any other key-generating EAP authentication method as described in clause I.2 of TS 33.501 [X].

-	For the IKE_AUTH exchange part in step 13a of clause 7.2.1 of TS 33.501 [X], names in the ID payloads should correspond to the keys used to generate the AUTH payload. In case the UE utilizes the anonymous value SUCI in step 5, the UE shall initiate an IKE_AUTH exchange and shall include the SUCI/onboarding SUCI in ID payloads rather than anonymous value SUCI. To help TNGF identify KTNGF, the AMF should send the newly generated KTNGF and the corresponding SUCI/onboarding SUCI to the TNGF. The AMF may obtain the SUCI/onboarding SUCI from the AUSF.

NOTE: The UE can only provide SUCI/Onboarding SUCI in case it is configured with SUPI privacy parameters.

Editor’s Note: The need for including SUPI privacy case is FFS.

Editor’s Note: The usage of Onboarding SUCI in step 13 for a non 3gpp access is FFS.

### 6.2.3	System impact

TBD

### 6.2.4	Evaluation

TBD

## 6.3	Solution #3: Use of anonymous SUCI in trusted non-3GPP access for SNPN

## 6.3.1	Introduction

This solution solves Key issue #1 in the case of using anonymous SUCI in trusted non-3GPP access.

When introducing non-3GPP access in SNPN it is assumed that most security procedures can be reused. However, the use of anonymous SUCI is only applicable to SNPNs so there are not yet any procedures specified for this case in relation to non-3GPP access.

In the current procedures for trusted non-3GPP access in clause 7A.2.1 of TS 33.501 [4], it is specified to use the SUCI/GUTI to map the user to the correct KTNGF in step 13. When using anonymous SUCI, this is not a good solution since an anonymous SUCI is not unique. Instead, another identifier is needed. This solution proposes to use a hash of the key KTNGF as identifier in case anonymous SUCI is used during the authentication towards the SNPN.

This solution defines adaptations of existing procedures needed to support the use of anonymous SUCI in trusted access for SNPN.

### 6.3.2	Solution details

Procedures in clause 7A.2.1 of TS 33.501 [4] are reused with the following exception:

-	In step 13, if the construction of SUCI as described in clause 6.12 of TS 33.501 cannot be used, then a new type of identifier is used. The new identifier is proposed to be a hash of the key KTNGF. (potentially using some additional input). It is proposed to send the new identifier using the IDi payload.

It is already specified in section 3.5 of RFC 7296 [5] that the ID payload used for transport of IDi can be used to transfer a key identifier by setting the ID Type to ID_KEY_ID. Support of this ID Type is mandatory. The RFC does not specify how such a key identifier is generated. The proposal here is thus to use a hash of the key KTNGF potentially using some additional input to create a key identifier.

### 6.3.3	System impact

This solution has impact on UE and TNGF.

### 6.3.4	Evaluation

This solution solves Key issue #1 in the case of using anonymous SUCI in trusted non-3GPP access. If anonymous SUCI is not supported in trusted non-3GPP access, methods like EAP-TLS can only be used if the UE is also configured with a public key for SUCI concealment.

## 6.4	Solution #4: Authentication for devices that do not support 5GC NAS over WLAN access in SNPN scenarios

### 6.4.1	Introduction

This solution addresses key issue #1.

TR 23.700-08 [2] studies "Key Issue #2: Support of Non-3GPP access for SNPN". Clause 5.2.1 of TR 23.700-08 [2] states: "Currently the 3GPP specifications do not support direct connection to SNPN via non-3GPP access networks" and "One objective of this key issue is to enable the 5GS to support direct connection of non-3GPP access networks to the SNPN's 5GC."

Devices that do not support 5GC NAS signalling over WLAN access (referred to as "Non-5G-Capable over WLAN" devices, or N5CW devices for short) may access 5GC in a SNPN via a trusted WLAN Access Network that supports a Trusted WLAN Interworking Function (TWIF).

This solution enables N5CW to access SNPN via trusted non-3GPP WLAN Access Network.

### 6.4.2	Solution details

For N5CW device in a PLMN, the authentication procedure only include EAP-AKA’, which is defined in clause 6.1.3.1 of TS 33.501 [4]. However, in SNPN scenarios, N5CW supports key-generating EAP authentication methods.

This solution reuses the authentication procedure in clause 7A.2.4 of TS 33.501 [4] with the following modifications.

-	As described in clause I.2 of TS 33.501 [4], the authentication mechanisms utilized in step 8 of clause 7A.2.4 of TS 33.501 should include key-generating EAP authentication methods.

-	N5CW sends UE identity (e.g. SUCI/on boarding SUCI) and AN parameters to the TWAP/TWIF. And SNPN identifier, which consists of PLMN ID and NID, should be included in AN parameters. Moreover, in SNPN scenarios, if the construction of SUCI as described in clause 6.12 of TS 33.501 cannot be used and if the employed EAP method supports SUPI privacy, the UE can send an anonymous value SUCI based on configuration.

-	If the UE is accessing 5GS for Onboarding, the AN parameters sent from UE to TWAP /TWIF shall include Onboarding indication. And the Registration Type should set as "SNPN Onboarding".

-	The TWIF shall create a 5GC Registration Request message on behalf of the N5CW device. The TWIF shall use UE identity, AN parameters, Registration Type that are receieved from N5CW.

Editor’s Note: Whether onboarding for N5CW devices is in scope is FFS.

Editor’s Note: The need for including SUPI privacy case is FFS.

### 6.4.3	System impact

TBD

### 6.4.4	Evaluation

TBD

## 6.5	Solution #5: Anonymous authentication during connection establishment in trusted non-3GPP network access.

### 6.5.1	Introduction

This is a solution to KI#1.

When a UE access a trusted non-3gpp access network, it used either SUCI or 5G-GUTI for identification. In case of an NPN deployment, the UE might use an anonymous identifier when the EAP method supports its, as specified in TS 33.501 [4] clause I.5. The anonymous identifier will protect the identity of the UE and makes it impossible to differentiate between a group of UE’s using the same identifier namely the anonymous identifier. As the authentication and key derivation steps are independent of the IPsec establishment, the TNGF cannot link the authentication and derived key to a IKE_AUTH request – As the same identifier is used for multiple devices.

This solution provides a method to fill the gap caused by introducing the anonymous identifier which is already standardised in TS 33.501 [4] clause I.5. The solution proposes, that the TNGF creates a unique temporary identifier, shares it after authentication alongside other information necessary to establish the IPsec connection (e.g., TNGF address), to the UE. When the UE initiates the establishment of the IPSec channel, the UE uses the temporary identifier as identifier and thereby enables the TNGF to identify the correct key material (KTNGF) for the session.

The temporary identifier is only applicable when the anonymous identifier is used, therefore it’s proposed as an optional parameter.

### 6.5.2	Solution details

Procedures in clause 7A.2.1 of TS 33.501 [4] are reused with the following exception:

-	In step 9b, when an anonymous identifier has been used in step 5, transfer a unique temporary identifier, allocated by the TNGF, to the UE alongside the TNGF address.

-	In step 13b, use the unique temporary identifier provided in step 9b as IDi, in case an anonymous identifier was used in step 5.

The allocation of a temporary identifier by the TNGF, distributed to the UE, enables the TNGF to identify the KTNGF which is used in the IKE_AUTH procedure in step 13b and c.

### 6.5.3	System impact

This solution impacts TNGF and UE.

### 6.5.4	Evaluation

## 6.6	Solution #6: Trusted non-3GPP Access for SNPN

### 6.6.1	Introduction

This solution addresses key issue #1.

The normal trusted access procedures are used, only if the UE sends an anonymous SUCI, then the TNGF and the UE use the assigned IP address, which is unique within the TNGF, as identifier in the IDi according to RFC 7296.

### 6.6.2	Solution details

This solution reuses the trusted non-3GPP access authentication procedure in PLMN scenarios in clause 7A.2.1 of TS 33.501 [4] with the following modifications:

If the UE sends an anonymous SUCI in step 5 of the procedure, then the TNGF will use the IP address, which the TNGF assigns to the UE as unique identifier to bind the security key. In step 13, the UE shall include the ID_IPV4_ADDR or ID_IPV6_ADDR with the assigned IP address in the IDi. The TNGF uses the received IP address to locate the KTIPSe for the connection.

### 6.6.3	System impact

There is no system impact.

### 6.6.4	Evaluation

The assigned IP address is unique within an TNGF and can be used as identifier to locate the security key.

Editor’s Note: Further Evaluation is FFS

## 6.7	Solution #7: Untrusted non-3GPP Access for SNPN

### 6.7.1	Introduction

This solution addresses key issue #1.

The normal untrusted access procedures are used, only if the UE sends an anonymous SUCI, then the N3IWF assigns a unique identifier and the UE uses the unique identifier in the IDi when sending the IKE_AUTH with AUTH.

### 6.7.2	Solution details

This solution reuses the untrusted non-3GPP access authentication procedure in PLMN scenarios in clause 7.2.1 of TS 33.501 [4] with the following modifications:

If the UE sends an anonymous SUCI in step 5 of the procedure, then the N3IWF assigns a unique identifier, which is unique within the N3IWF and used to bind the communication and the security key received in step 12. The N3IWF provides the assigned unique identifier to the UE in an IKE_AUTH_Response and the UE shall include the unique identifier in the IDi when sending the IKE_AUTH with AUTH in step 14. The N3IWF uses the received unique identifier to locate the KN3IWF for the connection.

### 6.7.3	System impact

The N3IWF assigns a unique identifier to the UE when sending an anonymous SUCI, the UE sends the unique identifier with the IKE_AUTH with AUTH.

### 6.7.4	Evaluation

The identifier is unique within the N3IWF and used to bind the communication and the security key to the UE.

Editor’s Note: Further Evaluation is FFS

Editor’s Note: It is FFS whether an untrusted solution is required

## 6.8	Solution #8: Reusing Existing N3GPP Security for SNPN

### 6.8.1	Introduction

This solution addresses key issue #1 on Security of non-3GPP access for SNPN.

### 6.8.2	Solution details

For support of untrusted non-3GPP access for SNPN, it is proposed to reuse the security mechanisms defined in clause 7 in TS 33.501 [4].

For support of trusted non-3GPP access for SNPN, it is proposed to reuse the security mechanisms defined in clause 7A in TS 33.501 [4].

For support of wireline access for SNPN, it is proposed to reuse the security mechanisms defined in clause 7B in TS 33.501 [4].

For support of NSWO for SNPN, it is proposed to reuse the security mechanisms defined in Annex S in TS 33.501 [4].

NOTE: The solution does not address the case of anonymous SUCI.

### 6.8.3	Evaluation

This solution addresses the requirement of key issue #1.

The solution is based on existing N3GPP Security mechanisms and procedures and hence has no additional standard impact.

Editor’s Note: Further evaluation is ffs

## 6.9	Solution #9: NSWO support in SNPN using any key-generating EAP-method

### 6.9.1	Introduction

This solution solves Key issue #1 in the case of NSWO using any key-generating EAP-method.

Current procedures for NSWO are only defined to use EAP-AKA'. This solution extends the NSWO procedures to be able to use any key-generating EAP-method in SNPN.

The proposed procedure is based on the current procedures in Annex S.2.3 of TS 33.501[4]. The procedures assume access to subscribed SNPN.

### 6.9.2	Solution details

Figure: 6.9.2-1: Authentication procedure for NSWO in SNPN

Steps 1-2 are performed as described in Annex S.3.2 of TS 33.501 [4].

Step 3 is performed as described in Annex S.3.2 of TS 33.501 [4] with the following addition: If the EAP method supports privacy and the UE is configured to use anonymous SUCI, the UE may send an anonymous value SUCI based on configuration.

Steps 4-6 are performed as described in Annex S.3.2 of TS 33.501 [4].

7. Upon reception of the Nudm_UEAuthentication_Get Request, the UDM invokes SIDF to de-conceal SUCI to gain SUPI.

Based on the subscription the UDM selects the appropriate EAP method to be used. The UDM includes an indicator of the selected EAP method and the SUPI to AUSF in a Nudm_UEAuthentication_Get Response message.

8. Authentication is performed between the AUSF and UE using the selected EAP method. After a successful authentication the AUSF derives the MSK key. The decision to use MSK instead of EMSK is based on the NSWO indicator received in step 5.

Steps 9-11 are performed as described in steps 16-18 of Annex S.3.2 of TS 33.501 [4].

### 6.9.3	System impact

The solution has impact on UE and AUSF.No impact on WLAN AP, NSWOF or UDM.

### 6.9.4	Evaluation

TBD

## 6.10	Solution #10: Access to localized services using existing mechanisms

### 6.10.1	Introduction

This solution addresses Key Issue #2 (Authentication for UE access to hosting network). The proposed solutions use existing mechanisms in 33.501[4].

Editor’s Note: Usecases or scenarios for access to the hosting network need to be specified.

As per the conclusion in 23.700-08[2], clause 8.4, when UE accesses the Hosting network using the subscription/credentials of its Home network, only two cases are considered:

-	If the Home network is PLMN, the Hosting network can be PNI-NPN or SNPN.

-	If the Home network is SNPN, the Hosting network can be only SNPN.

In both cases, solutions to these scenarios can be considered in the following categories

-	UE uses home network credentials to access the hosting network.

-	UE uses credentials obtained using the onboarding procedure as defined in 33.501[4] Annex I.9("Security of UE onboarding in SNPNs" to access the hosting network.

Note: Onboarding procedures only provide connectivity to obtain credentials. The credentials provisioning protocol is out of scope.

Following clause details how UE can use existing mechanisms and methodologies to access localized services.

### 6.10.2	Solution details

#### 6.10.2.1 	Solution for access to localized services based on Home Network Credentials

To access the hosting network, the UE determines whether it can reuse home network credentials. As per 23.700-08[2], the UE concludes that a home network credential can be utilized if the SNPN ID of the hosting network is contained in the SNPN priority lists connected with the home network subscription and the hosting network shows support for CH credentials. Existing 33.501[4] primary authentication can be reused in this case. No normative work is needed from the SA3 WG perspective.

#### 6.10.2.2 	Solution for access to localized services based on Onboarding Mechanism

Figure 6.10.2.2-1 shows a general overview of the access to localized services based on the onboarding mechanism. If UE uses credentials other home network credentials, then credentials obtained using the onboarding procedure as defined in 33.501[4] Annex I.9("Security of UE onboarding in SNPNs" can be used to access the hosting network.

Figure 6.10.2.2-1: Overview of Access to localized services based on Onboarding Mechanism per Annex I.9.2

Figure 6.10.2.2-2 shows the procedures for access to localized services.

Figure 6.10.2.2-2: Procedures to Access localized services based on Onboarding

1.	The Localized Service Provider (LSP) establishes a service agreement with the operator of a Hosting Network. The LSP also establishes a service agreement with the UE to enable the UE to receive the information needed to discover/access Hosting Network and the localized service. The hosting network is configured based on the service agreement, e.g., DNN/S-NSSAI configuration for access to localized service, QoS, number of end-users, time, location, whether home network services can be accessed via the hosting network, etc. The configuration of the Hosting Network is performed by means that are outside of the 3GPP scope. The UE performs the procedures defined in 3GPP TS 23.502 to get the 5GC network access.

2.	Localized service advertisements prompt UE's user using default credentials where UE authenticates with the DCS (e.g., per TS 33.501[4] Annex I.9.2). The procedures are similar to the existing 23.501[BB] clause 5.30.2.10.2. e.g., After a successful onboarding procedure, the LSP sends a Security Profile corresponding to the UE and LSP certificate to the LSP access. The Security Profile associated with the LSP contains all the necessary data to establish a TLS tunnel between the LSP and UE.

Note: Aspects related to information configuration, such as PVS IP address and PVS FQDN, depend on WG SA2’s conclusions.

3.	After step 2, the UE needs to disconnect from the network (so far in the role of onboarding network) and reconnect again (now in the role of Hosting network). The user performs a selection of the Hosting Network. UE connects to the Hosting Network and is authenticated by the LSP in the role of Credential Holder (e.g., using a AAA server) using credentials downloaded in 2.	UE requests a PDU Session and accesses the localized service of the LSP via the Hosting Network. Upon expiry of the time-restricted credentials, the LSP in the role of Credential Holder requests a release of the UE. When the localized service agreement is terminated, the Hosting Network removes the configured information by means that are outside of the 3GPP scope.

### 6.10.3	System Impact

For 6.10.2.1, We do not see any system impact on existing security architecture per 33.501[4].

For 6.10.2.2, , the existing UE onboarding procedure cannot be used as is for access to localized services. As per  23.501[6] clause 5.30.2.10.4.2 (Onboarding configuration for the UE), The PVS information, such as PVS IP address and PVS FQDN(s)  provided by the onboarding network takes precedence over any such information stored in the UE. In contrast, the PVS information provided by the DCS takes precedence over the information stored in the onboarding network.

### 6.10.4	Evaluation

### For 6.10.2.1, With the proposed solutions above, we do not see any system impact on existing security architecture per 33.501[4]. Solution addresses key issue 2 and fulfils the requirements of mutual authentication between UE and Hosting Network.

Editor’s Note: Further Evaluation is FFS

## 6.11	Solution #11: High-level solution on authentication for UE access to hosting network

### 6.11.1	Introduction

This solution addresses Key Issue #2 "Authentication for UE access to hosting network". It provides a high-level exploration of the solution space, based on the analysis of the different possible use and business cases.

### 6.11.2	Solution details

The study in TR 23.700-08 [2] has considered different scenarios for UE access to hosting network. From an authentication point of view, the scenarios can be broadly classified into the following two cases:

1)	There is a business relationship between hosting network and home network.

2)	There is no business relationship between hosting network and home network.

For case 1), the business relationship between hosting network and home network can have different implications on the authentication architecture. At least the following cases are possible:

1a) Home network is Credentials Holder using AUSF/UDM.

1b) Hosting network is PNI-NPN and home network is the underlying PLMN.

1c) Hosting network is PNI-NPN and the underlying PLMN has a roaming relationship with the home network.

For case 1a), the authentication to the hosting network is already specified in Annex I.2 of TS 33.501 [4]. For case 1b), the authentication to the hosting network can be based on the PLMN-credentials of the home network. Additional secondary or slice-specific authentication to access the hosting network is also possible. For case 1c), the authentication to the hosting network can also be based on the PLMN-credentials of the home network. In addition to primary authentication, secondary authentication to access the hosting network (PNI-NPN deployed by the VPLMN) is possible with local breakout.

For case 2), the following cases need to be considered:

2a) Hosting network is SNPN and does not have a business relationship with the home network.

2b) Hosting network is PNI-NPN and the underlying PLMN does not have a roaming relationship (or other business relationship) with the home network: This case does not seem to be possible, since PLMNs usually have a roaming relationship.

In case 2a), the UE needs new credentials to authenticate to the hosting network. Hosting network credentials can be provisioned to UE using existing user plane mechanisms, over visited or home network connectivity or using onboarding as specified in TS 23.501 [6] clause 5.30.2.10, TS 23.502 [7] clause 4.2.2.2.4 and TS 33.501 [4] Annex I.9. Onboarding requires default UE credentials to be pre-configured on the UE.

Editor's Note: It is ffs whether the solution covers all scenarios concluded in SA2.

### 6.11.3	System impact

The high-level solutions described in the solution details do not have normative specification impact.

### 6.11.4	Evaluation

This solution addresses the security requirements of Key Issue #2 "Authentication for UE access to hosting network". It does not require new normative work.

Editor's Note: Further evaluation is ffs.

## 6.12	Solution #12: Localised service authentication through onboarding procedure and registration afterwards.

### 6.12.1	Introduction

This is a solution to KI#2.

The solution proposes to use the onboarding procedures defined in TS 33.501 [4] clause I.9 and TS 23.501 [6] clause 5.30.2.10 to provision the credentials, which afterwards are used to authenticate to the hosting network and hereby get access to the localised service.

The solutions assumes that the service provider and user have a service agreement and as part of that agreement the UE also have the default credentials provisioned. The license agreement and provisioning of default credentials is out of scope.

The onboarding of the UE is done towards the hosting network, which might host the DCS or the DCS can be located at the service provider. The hosting network enables the UE to get onboarded through the onboarding service provided, implying credentials to the hosting network/service is installed.

To access the localised service, the UE registers to the hosting network using the installed credentials.

### 6.12.2	Solution details

Procedures in this solution is based the procedures defined in TS 33.501 [4] clause I and TS 23.501 [6] clause 5.30.2.10 concerning UE onboarding.

- The service provider and user contractual agrees to enable the service. This implies that the service provider, provides the UE with the default UE credentials. The provisioning of default UE credentials is out of scope.

- The UE initiates an onboarding procedure towards the hosting network which in this case act like an onboarding network. The DCS may be hosted in the hosting network or by the service provider. The onboarding procedures follow the already standardised solution in TS 23 23.501 [Y] clause 5.30.2.10 and TS 33.501 [4] clause I.9

NOTE: Provisioning of credentials is out of scope.

- The UE registers to the hosting network and authenticates using credentials provisioned in step 2. After registration the UE has access to the localised service.

### 6.12.3	System impact

No system impacts.

### 6.12.4	Evaluation

## 6.13	Solution #13: Home network primary authentication – secondary authentication towards localised service

### 6.13.1	Introduction

This is a solution to KI#2.

This solution proposes to use home routed primary authentication towards the home network to establish the connection and secondary authentication towards the localised service provider to authenticate and get access to the service. The solution reuses already existing methods to authenticate the UE, namely home routed authentication, and local break out (break out from hosting network) secondary authentication towards the localised service provider.

### 6.13.2	Solution details

Procedures in this solution is based the procedures defined in TS 33.501 [4] clause 11 concerning secondary authentication.

- The hosting network, home network and service provider agree on a service agreement.

- The UE registers to the hosting network which “home routes” the authentication request to the home network which performs the primary authentication of the subscriber and establishes the key hierarchy as defined in TS 33.501 clause 6. The hosting network will act as a visited network and the home network as the home network.

- The SMF in the hosting network will initiate the secondary authentication as described in TS 33.501 [4] clause 11, as defined for the local breakout configuration. The AAA can either be managed by the service provider or the hosting network.

### 6.13.3	System impact

No system impacts.

### 6.13.4	Evaluation

## 6.A	Solution #A: <Title>

### 6.A.1	Introduction

### 6.A.2	Solution details

### 6.A.3	System impact

### 6.A.4	Evaluation

# 7	Conclusions

Annex <A>:
<Informative annex title for a Technical Report>

Annex X:
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2022-05 | SA3#107-e | S3-220957 |  |  |  | Skeleton | 0.0.0 |
| 2022-07 | SA3#107e AdHoc | S3-221674 |  |  |  | Version after incorporating changes from S3-221492 and S3-221681 | 0.1.0 |
| 2022-10 | SA3#108Adhoc-e | S3-223120 |  |  |  | Version after incorporating changes from S3-222931, S3-222965, S3-222990, S3-222931, S3-223118 | 0.2.0 |
| 2022-11 | SA3#109 | S3-224036 |  |  |  | Version after incorporating changes from S3-224034, S3-224035, S3-223804, S3-224037, S3-223669, S3-223668, S3-224043, S3-224044, S3-224045, S3-224046 | 0.3.0 |