- 3GPP TSG SA WG3 Meeting #104-e                            	S3-21xxxx

- e-meeting, 16 – 27 August 2021	                         revision S3-21abcd

- Source:	Huawei, HiSilicon

- Title:	Conclusion on User Consent Check

- Document for:	Approval

Agenda Item:	5.14

# Decision/action requested

This contribution proposes a new conclusion in TR 33.867

# References

# Rational

The contribution proposes a new conclusion on how to check user consent.

# Detailed proposal

*************** Start of 1st Change ****************

- 8.X	User Consent Check

The following generic aspects are concluded for normative work:

- How to check user consent shall be specified in Annex of 3GPP TS 33.501 [xx] in order to provide guideline if a new use case needs to check user consent.

- If a service is invoked by NF service consumer directly, NF service provider checks user consent based on user consent parameters in UE context, if it is missing, the NF service provider uses Nudm_SDT_Get service as depicted in 8.Y to retrieve user consent parameters.

- If there is an intermediate NF which is involved in service invocation, e.g. NWDAF, NEF, etc., the intermediate NF checks user consent based on user consent parameters in UE context, if it is missing, the intermediate NF invokes Nudm_SDT_Get service as depicted in 8.Y. The intermediate NF determines whether to allow the NF service consumers’ processing of data subject to user consent based on user consent parameters.

The following specific aspects for use case#1 are concluded for normative work:

- Procedure of user consent check for use case#1 shall be specified as pCRs in potential WID of eNA.

- NWDAF performs role of intermediate NF which may additionally check user consent when NF service consumer triggers a data analysis service.

The following specific aspects for use case#2 are concluded for normative work:

- Procedure of user consent check for use case#2 shall be specified as pCRs in WID of MEC.

- NEF performs role of intermediate NF which may additionally check user consent when AF triggers a data process service.

*************** End of 1st Change ****************