| 3GPP TR 28.881 V0.3.0 (2025-11) |  |
| --- | --- |
| Technical Report |  |
| 3rd Generation Partnership Project; Technical Specification Group Services and System Aspects; Management and orchestration; Study on intent driven management service for mobile network phase 4 (Release 20) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet https://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2025, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	8

Introduction	9

1	Scope	10

2	References	10

3	Definitions of terms, symbols and abbreviations	11

3.1	Terms	11

3.2	Symbols	11

3.3	Abbreviations	11

4	Use cases and potential solutions for new areas	11

4.1 Use case #1: Enhancement of radio service delivering and assurance scenarios	11

4.1.1 Description	11

4.1.2 Potential requirements	11

4.1.3 Potential solutions	12

4.1.4 Evaluation of potential solutions	12

4.2 Use case #2: Enhancement of radio network performance assurance scenarios	12

4.2.1 Description	12

4.2.2 Potential requirements	13

4.2.3 Potential solution	13

4.2.4 Evaluation of potential solutions	13

4.3 Use case #3:  Assisting and reporting intent decomposition across intent handling functions	13

4.3.1	Description	13

4.3.2	Potential requirements	14

4.3.3	Potential solutions	14

4.3.3.1	Potential solution #1	14

4.3.4	Evaluation of potential solutions	14

4.4 Use case #4: Intent traceability	14

4.4.1 Description	14

4.4.2 Potential requirements	15

4.4.3	Potential solutions	15

4.4.3.1 Potential solution #1	15

4.4.3.1 Potential solution #2	16

4.4.4	Evaluation of potential solutions	16

4.5	Use case #5: Invariant Guidance in Intent Contexts	17

4.5.1 Description	17

4.5.2 Potential requirements	17

4.5.3 Potential solutions	17

4.5.4  Evaluation of potential solutions	17

4.6	Use case #6: Intent Interpretation Assistance Information	18

4.6.1 Description	18

4.6.2 Potential requirements	18

4.6.3 Potential solutions	18

4.6.4  Evaluation of potential solutions	19

4.7 Use case #7: Enhancement of intent exploration	19

4.7.1 Description	19

4.7.2 Potential requirements	19

4.7.3 Potential solution	19

4.7.4 Evaluation of potential solutions	20

4.8 Use case #8: Support to express guarantee requirements in an intent	20

4.8.1 Description	20

4.8.2 Potential requirements	21

4.8.3 Potential solutions	21

4.8.3.1 Potential solution #1	21

4.8.3.2 Potential solution #2	21

4.8.4  Evaluation of potential solutions	22

4.9 Use case #9: Intent handling capability configuration, registration and discovery	22

4.9.1 Description	22

4.9.1.1 Intent handling capability description	22

4.9.1.2 Intent handling capability registration and discovery	22

4.9.1.3 Intent handling capability configuration	22

4.9.2 Potential requirements	22

4.9.3 Potential solution	23

4.9.4 Evaluation of potential solutions	23

4.10 Use Case#10: Radio network delivering in transient overload scenario	23

4.10.1 Description	23

4.10.2 Potential requirements	23

4.10.3 Potential solution	24

4.10.4 Evaluation of potential solutions	24

4.11	Use case #11: Enhancing intent feasibility check capability	24

4.11.1	Description	24

4.11.2	Potential requirements	24

4.11.3	Potential solutions	24

4.11.3.1	Potential solution #1 for a new procedure of feasibility check with exploration	24

4.11.3.2	Potential solution #2 Extend inFeasibleTarget with recommended values	25

4.11.4 Evaluation of potential solutions	25

The implementation of both potential solutions is not complex. Therefore, both potential solutions described in clause 4.11.3 are feasible solutions.	25

4.12 Use case #12: Documentation for the overview of intent driven management functionalities	25

4.12.1 Description	25

4.12.2 Potential requirements	25

4.12.3 Potential solution	25

4.12.4 Evaluation of potential solutions	26

4.13 Use case #13: Investigation on the intent lifecycle management state transition	26

4.13.1 Description	26

4.13.2 Potential requirements	27

4.13.3 Potential solutions	27

4.13.3.1 Potential solution #1	27

4.13.3.2 Potential solution #2	29

4.13.4 Evaluation of potential solutions	30

4.14 Use case #14:  intent expectation satisfied information	31

4.14.1	Description	31

4.14.2	Potential requirements	31

4.14.3	Potential solutions	31

4.13.4 Evaluation of potential solutions	31

4.15	Use case #15: Relation and Interaction with MnS producers for AI/ML Management	31

4.15.1	Description	31

4.15.2	Potential requirements	32

4.15.3	Potential solutions	32

4.15.4 Evaluation of potential solutions	32

4.16 Use case #16: Investigation on the applicability and potential impacts to support natural language intents translation	32

4.16.1 Description	32

4.17 Use case #17: Enhancement of core network and service delivering and assurance scenarios	33

4.17.1 Description	33

4.17.2 Potential requirements	33

4.17.3 Potential solutions	34

4.17.4 Evaluation of potential solutions	34

4.18 Use case #18: The relation and the interactions between intent handling function and NDTFunction	34

4.18.1 Description	34

4.18.2 Potential requirements	34

4.18.3 Potential solutions	34

4.18.4 Evaluation of potential solutions	34

4.19 Use case #19: Enhancement of radio service scenarios for service protection	34

4.19.1 Description	34

4.20	Use case #20:  New deployment scenario for Intent Handling Function at ManagedElement	35

4.20.1	Description	35

4.20.2	Potential requirements	35

4.20.3	Potential solutions	35

4.20.4	Evaluation of potential solutions	35

5	Conclusions and Recommendations	35

5.1 Use case #1: Enhancement of radio service delivering and assurance scenarios	35

5.2 Use case #2: Enhancement of radio network performance assurance scenarios	36

5.3 Use case #3: Assisting and reporting intent decomposition across intent handling functions	36

5.4 Use case #4: Intent traceability	36

5.5 Use case #5: Invariant Guidance in Intent Contexts	36

5.6 Use case #6: Intent Interpretation Assistance Information	36

5.7 Use case #7: Enhancement of intent exploration	36

5.8 Use case #8: Support to express guarantee requirements in an intent	37

5.9 Use case #9: Intent handling capability configuration, registration and discovery	37

5.10 Use Case#10: Radio network delivering in transient overload scenario	37

5.11 Use case #11: Enhancing intent feasibility check capability	37

5.12 Use case #12: Documentation for the overview of intent driven management functionalities	38

5.13 Use case #13: Improve intent life cycle documentation	38

5.14 Use case #14: Intent expectation satisfied information	38

5.15 Use case #15 Relation and Interaction with MnS producers for AI/ML Management	38

5.16 Use case #16: Investigation on the applicability and potential impacts to support natural language intents translation	38

4.17 Use case #17: Enhancement of core network and service delivering and assurance scenarios	39

4.18 Use case #18: The relation and the interactions between intent handling function and NDTFunction	39

4.19 Use case #19: Enhancement of radio service scenarios for service protection	39

4.20	Use case #20: New deployment scenario for Intent Handling Function at ManagedElement	39

Annex A: Change history	40

# Foreword

This Technical Report has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

In the present document, modal verbs have the following meanings:

shall	indicates a mandatory requirement to do something

shall not	indicates an interdiction (prohibition) to do something

The constructions "shall" and "shall not" are confined to the context of normative provisions, and do not appear in Technical Reports.

The constructions "must" and "must not" are not used as substitutes for "shall" and "shall not". Their use is avoided insofar as possible, and they are not used in a normative context except in a direct citation from an external, referenced, non-3GPP document, or so as to maintain continuity of style when extending or modifying the provisions of such a referenced document.

should	indicates a recommendation to do something

should not	indicates a recommendation not to do something

may	indicates permission to do something

need not	indicates permission not to do something

The construction "may not" is ambiguous and is not used in normative elements. The unambiguous constructions "might not" or "shall not" are used instead, depending upon the meaning intended.

can	indicates that something is possible

cannot	indicates that something is impossible

The constructions "can" and "cannot" are not substitutes for "may" and "need not".

will	indicates that something is certain or expected to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

will not	indicates that something is certain or expected not to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

might	indicates a likelihood that something will happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

might not	indicates a likelihood that something will not happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

In addition:

is	(or any other verb in the indicative mood) indicates a statement of fact

is not	(or any other negative verb in the indicative mood) indicates a statement of fact

The constructions "is" and "is not" do not indicate requirements.

# Introduction

# 1	Scope

The present document studies a number of potential enhancements to the intent driven management services for 5G Advanced network.  These include the definition of new targets and contexts for existing scenarios, negotiation enhancements, and utility function enhancements.

New functionality includes the introduction of intent guarantee requirements, support for intent traceability and support for natural language expression in intents.

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TS 28.312: "Management and orchestration; Intent driven management services for mobile networks".

[2]	3GPP TS 28.541: "Management and orchestration; 5G Network Resource Model (NRM); Stage 2 and stage 3".

[3]	3GPP TS 29.572: "5G System; Location Management Services; Stage 3"

[4]	3GPP TS 38.304: "NR; User Equipment (UE) procedures in Idle mode and in RRC Inactive state".

[5]	3GPP TS 38.331: "NR; Radio Resource Control (RRC); Protocol specification".

[6]	3GPP TS 28.537: “Management and orchestration; Management capabilities”.

[7]	3GPP TS 38.300: “NR; NR and NG-RAN Overall description; Stage-2”.

[8]	3GPP TS 28.554: "Management and orchestration; 5G end to end Key Performance Indicators (KPI)".

[9]	3GPP TR 28.914: "Study on intent driven management service for mobile network phase 3".

[10]	3GPP TS 28.105: "Management and orchestration; Artificial Intelligence/ Machine Learning (AI/ML) management".

[11]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[12]	3GPP TS 28.561: "Management and orchestration; Management aspects of network digital twins".

[13]	3GPP TS 33.501: “Security architecture and procedures for 5G system”

# 3	Definitions of terms, symbols and abbreviations

## 3.1	Terms

For the purposes of the present document, the terms given in TR 21.905 [11] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in TR 21.905 [11].

example: text used to clarify abstract rules by applying them literally.

## 3.2	Symbols

For the purposes of the present document, the following symbols apply:

<symbol>	<Explanation>

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [11] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [11].

<ABBREVIATION>	<Expansion>

# 4	Use cases and potential solutions for new areas

## 4.1 Use case #1: Enhancement of radio service delivering and assurance scenarios

### 4.1.1 Description

In 3GPP TS 28.312 [1], the existing use case and requirements for intent containing an expectation for delivering a radio service is described in clause 5.1.2. The RadioServiceExpectation is defined to represent MnS consumer's expectations for radio service delivering and assurance in the specified area. However, following scenarios are not supported:

- MnS consumer expresses the radio service delivering and assurance expectation with service reliability information.

- MnS consumer expresses the radio service delivering and assurance intent expectation for a specified area described in the form of civic location. The civic area can be represented by a civic address (e.g. country A, Building B, ROOM C) or location name (e.g. hotel name, school name). For example, MnS consumer wants to ensure the radio service targets (dLThptPerUETarget and dLLatencyTarget) for a specific civic address (e.g., the CivicAddress defined in clause 6.1.6.2.14 in TS 29.572 [3]).

- MnS consumer expresses the radio service delivering and assurance expectation for a specified assurance time duration. Using the concert as an example, MnS consumer expresses the intent indicating a radio service to be delivered and assured for two hours, or before concert, MnS consumer request to explore the supported maximum number of UE for the Radio Service which to be assured for two hours after it being delivered.

### 4.1.2 Potential requirements

REQ-IDMS_RadioServiceIntent -CON-1: The intent driven MnS producer for radio service should have capabilities enabling the MnS consumer to express service reliability requirements.

REQ-IDMS_RadioServiceIntent -CON-2: The intent driven MnS producer for radio service should have capabilities enabling the MnS consumer to express radio service delivering and assurance for a specified area described in the form of civic location.

REQ-IDMS_RadioServiceIntent -CON-3: The intent driven MnS producer for radio service should have capabilities enabling the MnS consumer to express radio service assurance time duration requirements.

### 4.1.3 Potential solutions

This solution proposes to reuse and enhance the existing RadioServiceExpectation defined in 3GPP TS 28.312 [1].

Enhancement Aspect1: Add following attributes as the ExpectationTargets for the RadioServiceExpectation to enable the MnS consumer to express service reliability requirements

- ReliabilityTarget, it represents the reliability target for the radio service that the intent expectation is applied.

Enhancement Aspect2: Add following attributes as the ObjectContexts for the RadioServiceExpectation to enable the MnS consumer to express radio service delivering and assurance requirements for a specified area described in the form of civic address.

- CivicAreaContext, the coverage areas for the Radio Service that the intent expectation is applied in the form of civic area (i.e. represented by CivicArea <<dataType>>). The CivicArea <<dataType>> can support civic address (e.g. streets, districts, seats, etc.) or location label (e.g. hotel name, school name). The detailed definition for civic address reuses the CivicAddress Data Type defined in clause 6.1.6.2.14 in TS 29.572 [3]. The type for locationLabel is string. Following is the proposed definition for CivicArea <<dataType>>.

| Attribute Name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| civicAddress | O | T | T | F | F |
| locationLabel | O | T | T | F | F |

NOTE: The locationLabel can be used if the location cannot be expressed by civic address. The locationLabel should be used without ambiguity.

Enhancement Aspect3: Add following attributes as the Expectation Contexts for the RadioServiceExpectation to support assurance duration information:

- AssuranceDurationContext, it describes the time duration at which the radio service instance should be scheduled to be delivered and available. The type is integer and unit is hour.

### 4.1.4 Evaluation of potential solutions

Only one potential solution has been identified, which is feasible.

## 4.2 Use case #2: Enhancement of radio network performance assurance scenarios

### 4.2.1 Description

In 3GPP TS 28.312 [1], the existing use case and requirements for intent containing an expectation on radio network performance to be assured and intent containing an expectation for RAN energy saving are described in clause 5.1.5 and clause 5.1.7. The RadioNetworkExpectation is defined in clause 6.2.2.1.1 to represent MnS consumer's expectations for radio network delivering and performance assurance. However, following scenarios are not supported:

- MnS consumer expresses radio network performance assurance expectation for a specific RAN feature (e.g., RedCap). For example, MnS consumer may express the radio network performance targets (e.g. weakRSRPRatioTarget, highUlPrbLoadRatioTarget and aveULRANUEThptTarget) to be assured for RedCap UEs in the specified areas. The detailed RAN feature need further investigation on TS 38.304 [4] and TS 38.331[5].

- MnS consumer expresses relative values for several performance targets for the specified areas. For example, MnS consumer may express the expectation on RAN energy consumption reduction ratio (as percentage) for RAN SubNetwork that the intent expectation is applied to illustrates the difference between the energy consumption before and after performing energy saving actions.

### 4.2.2 Potential requirements

REQ-IDMS_RadioNetworkIntent -CON-1: The intent driven MnS producer for radio network should have capabilities enabling the MnS consumer to express radio network performance assurance for a specific RAN feature

### 4.2.3 Potential solution

This solution proposes to reuse and enhance the existing RadioNetworkExpectation defined in 3GPP TS 28.312 [1] to enable the MnS consumer to express radio network performance assurance for a specific RAN feature.

Enhancement Aspect1: Add following attributes as the ObjectContexts for the RadioNetworkExpectation:

- rANFeatureContext, it represents the expected specific RAN feature for RAN Subnetwork that the intent expectation is applied.  Following are the allowed values for RAN features:

-	REDCAP: it represents the support of Reduced Capability (RedCap) and enhanced Reduced Capability (eRedCap) NR devices as defined in TS 38.300 [7].

-	XR: it represents the support of eXtended Reality (XR) services that require high data rate and low latency communications as defined in TS 38.300 [7].

-	AERIAL_UE_COMMUNICATION: it represents the Aerial UE Communication as defined in TS 38.300 [7].

-	V2X: it represents the V2X communication as defined in TS 38.300 [7].

NOTE: allowed values can be exteneded based on RAN specifications.4.2.4 Evaluation of potential solutions

### Note: allowed values can be exteneded based on RAN specifications.4.2.4 Evaluation of potential solutions

### 4.2.4 Evaluation of potential solutions

Only one potential solution has been identified, which is feasible.

## 4.3 Use case #3:  Assisting and reporting intent decomposition across intent handling functions

### 4.3.1	Description

An intent-driven MnS producer can translate an intent and decompose it into one or more new intents. If these newly generated intent(s) cannot be handled by the intent-driven MnS producer, it can submit them to another intent-driven MnS producer(s). In this case, the former intent-driven MnS producer acts as the consumer of the latter intent driven MnS producer(s), to which the newly generated intents are submitted.

The issue arises when the decomposition of an intent into new intents is carried out by an intent driven MnS producer, these decomposed intents may be submitted to intent-driven MnS producers of managed domains that are not desired by the original intent-driven MnS consumer. The intent-driven MnS consumer needs to be able to assist the intent driven MnS producer regarding the decomposition of intents to other MnS producers. Furthermore, the intent driven MnS consumer needs to receive a report regarding this decomposition. This is also important for troubleshooting an intent-driven MnS producer, to check whether it is acting as expected.

### 4.3.2	Potential requirements

REQ-Intent_DCMP-1: The intent driven MnS producer should have the capability to allow an MnS consumer to specify a context to be able to assist the intent-driven MnS producer regarding the decomposition of an intent to other MnS producers.

REQ-Intent_DCMP-2: The intent driven MnS producer should have the capability to allow an intent-driven MnS consumer to receive a report regarding the decomposition of an intent to other MnS producers.

### 4.3.3	Potential solutions

#### 4.3.3.1	Potential solution #1

This solution proposes to reuse and enhance the existing intent expectations and intent report information models defined in 3GPP TS 28.312 [1].

Intent expectations are enhanced with:

- A new expectation context that specifies the constraints applicable in the intent decomposition performed by the intent-diven MnS producer, such as a list of Intent Handling Functions identity among those under MnS consumer’s authorization to which the intent decomposition is not recommended by the intent-driven MnS consumer.

The IntentReport IOC in 3GPP TS 28.312 [1] is enhanced with following information:

- The identity of Intent Handling Functions to which the decomposed intents are sent.

- The identity of each intent and expectation resulting from the decomposition.

### 4.3.4	Evaluation of potential solutions

Only one potential solution provided in clause 4.3.3.1 is identified. This potential solution proposes enhancing the existing intent expectations by adding a new expectation context that allows the MnS consumer to specify intent handling functions that are not recommended for intent decomposition. This allows an MnS consumer to assist the MnS producer regarding decomposition of an intent. It also proposes enhancing the existing intent report to include information regarding decomposition of an intent. The implementation of this potential solution is not complex as it proposes enhancements to existing intent expectations and intent report information models.

Therefore, the potential solution described in clause 4.3.3.1 is a feasible solution to satisfy the requirements in clause 4.3.2.

## 4.4 Use case #4: Intent traceability

### 4.4.1 Description

An intent consumer (owner) submits an intent to a single intent handling function. In some cases, in order to fulfil the intent an intent handling function may need to submit additional intent(s) to other intent handling functions.  Such handling can occur multiple times across intent handling functions across multiple management and/or domain layers.

An operator/administrator needs visibility of the relationships between intents which have been created by the system. Since these are being created within/by the management systems (and not explicit consumers the operator may implement) it’s important to know where they came from to allow ‘trace-back’ to the original consumer intent which started the cascade of subsequent intents. There is however no identified method, standardized or otherwise, which allows for such traceability.

Since intents can result in new intent(s) to multiple intent handling functions, likely with different implementations, it is insufficient to rely on external mechanisms such as logging or local network management audit tools to trace the intent.  The information identifying each intent handling function which has handled the intent must be preserved along with the intent itself and accessible/meaningful within the content of each intent handling function. The intent consumer (owner) (e.g. Consumer A in figure 4.4.1-1) should be enabled to indicate whether they agree that their information be propagated beyond the recipient intent producer (e.g. Producer 1 in figure 4.1.1-1) to other intent producers (e.g. Producer 2 or 3 in figure 4.4.1-1).

The following figure provides an overview of such information and its handling:

Figure 4.4.1-1: Intent traceability information handling

### 4.4.2 Potential requirements

REQ-Intent_Trace-1: The intent driven MnS producer should provide information in the intent to identify that an intent has been handled by a particular intent handling function.

REQ-Intent_Trace-2: The intent driven MnS producer should provide information (as defined in REQ-Intent_Trace-1) to identify any subsequent intents created by it as part of fulfilment.

REQ-Intent_Trace-3: The intent driven MnS producer should provide information (as defined in REQ-Intent_Trace-1) identifying the intent handling functions to subsequent intent(s) to allow traceability of the intent across multiple intent handling functions.

REQ-Intent_Trace-4: The intent driven MnS producer should provide a capability allowing the intent MnS consumer (owner) to indicate whether the MnS consumer agrees that their information can be propagated in case of intent decomposition beyond the recipient intent producer to other intent producers.

### 4.4.3	Potential solutions

#### 4.4.3.1 Potential solution #1

To address REQ-Intent_Trace-1, REQ-Intent_Trace-2 and REQ-Intent_Trace-3 this solution proposes adding a new data structure to the intent model definition to define the information to trace the decomposition of an intent into subsequent intents.  The information is updated and propagated as new intent(s) are created as part of fulfilment.

The content proposed for the intent model definition would include

- the identity of each intent handling function which decomposed the intent

- the identity of each intent resulting from the decomposition

The handling of the information would be:

- a new intent created by an MnS Consumer is not required to populate this value, i.e. default is empty string.

- upon receipt of an intent, the Intent Handling Function updates the information to indicate it handled the intent by adding its identity to the incoming intent

- in the event the Intent Handling Function determines intent decomposition is required it create new intent(s) and propagates the information from the source intent to the new intent(s)

The visibility of this information is subject to the same access control as any other information exchanged between an authorised MnS consumer and producer.  For example, the originating Intent Owner Consumer A in Figure 4.4.1-1 will have access/visibility to its intents, including Intent A at Producer 1.  Producer 1 is an authorised MnS consumer of and will have access/visibility of its intents at Producer 2 and Producer 3, including Intent A.1 and A.2 resulting from the decomposition of Intent A. When no further decomposition is required, no further update is made to the information.

#### 4.4.3.1 Potential solution #2

To address REQ-Intent_Trace-4, this solution proposes adding a new attribute to the intent model definition to allow the MnS consumer indicate its preference for the propagation of intent traceability information.

The content proposed for the intent model definition would include:

- an attribute to indicate intent decomposition, e.g. ‘includeTraceInfo’ of type boolean

- default value of True

- value would be invariant

- configurable by MnS Consumer

The handling of includeTraceInfo would be:

- a new intent will have includeTraceInfo default value of True

- MnS Consumer may modify value of includeTraceInfo

- if includeTraceInfo =True, the recipient MnS Producer may include intent traceability information from the originating intent.

- if includeTraceInfo =False, the recipient MnS Producer may not include intent traceability information from the originating intent

- In the event of intent decomposition, the value of includeTraceInfo may or may not be propagated to subsequent intents at the discretion of each MnS Producer, i.e. in its role as MnS Consumer towards the subsequent MnS Producer(s).

### 4.4.4	Evaluation of potential solutions

TBD

## 4.5	Use case #5: Invariant Guidance in Intent Contexts

### 4.5.1 Description

TS 28.312 describes a scenario where for an instantiated intent, the intent handler may need to decompose the intent into multiple derivative intents, each to be fulfilled by a separate other intent handler. There may be contexts in the intent which the MnS consumer desires that the other intent handlers understand them exactly as they were provided, i.e., without any modification by the first intent handler in the decomposition process.

As an example, the MNO may, for some decomposition use-cases, want to only use certain certified hardware (e.g., with certain security, privacy, energy consumption, other types of quality guarantees) to be used in the fulfillment of RAN or CN operations. This additional guidance to use this specified hardware or the quality constraints on the resources may be provided as part of the intent context.

The MnS consumer should be enabled to indicate the context which should be transmitted to other intent handlers without modification.

### 4.5.2 Potential requirements

REQ-Intent_InvarGui-1: The intent driven MnS should include a capability enabling the MnS consumer to indicate the requirements, goals and contexts which the MnS consumer recommends be copied into decomposed intents and transmitted to other intent handlers without modification.

### 4.5.3 Potential solutions

The “context” dataType, as defined in TS 28.312, can be extended to include information about invariant guidance.

-	add a description for invariant guidance e.g. as: The Context can be used to indicate invariant guidance, e.g., which context is recommended to be propagated to all intent handler functions in the case where the intent is decomposed into multiple sub intents. The attribute contextInvariant is a flag to indicate if a specific context is recommended to be considered as in invariant guidance or not. The flag applies to context at the expectation level and intent target level.

-	The following attribute would be added to the Context <<dataType>>, i.e.:

Addition to Table 6.2.1.3.4.2-1

| Attribute Name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| contextInvariant | O | T | F | T | T |

The new attribute can be defined as:

| Attribute Name | Documentation and Allowed Values | Properties |
| --- | --- | --- |
| contextInvariant | It indicates whether the specific context recommended to be considered as invariant guidance to be transferred to subsequent intent handling functions without any modification. | type: Boolean multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |

### 4.5.4  Evaluation of potential solutions

TBA

## 4.6	Use case #6: Intent Interpretation Assistance Information

### 4.6.1 Description

A similar intent can be instantiated on several intent handlers, e.g., on several management systems each responsible for a for a different domain. TS28.312 supports intent negotiation that enables interaction between the MnS producer and MnS consumer to clarify the ability to fulfill intent requirements that may otherwise not be realizable owing to their abstract nature. For the same intents that are sent to several MnS producers, negotiation with the several MnS producers to clarify the intents can be resource wasteful.

The intent driven management service can minimize the number of negotiation exchanges while allowing effective interpretation of intents by reusing the interpretation information. The MnS consumer compiles prior interactions, e.g. previous IntentNegotiationReports or intentNegotiationConsumerFeedback, and shares those prior interactions as intent interpretation assistance information with the handler. The Mns consumer should be enabled to provide this intent interpretation assistance information that can be used by the intent handler to support the interpretation of the intent. As an example, if during intent negotiation, a set of options were provided by MnS producer and the MnS consumer selects one option, the MnS consumer may want the MnS producer to reuse this information, so the MnS consumer can provide it to the MnS producer for reuse.

NOTE: The intent interpretation assistance information can be visualized as a “frequently asked questions (FAQ)” section used in many human-oriented service descriptions (e.g. on service providers’ websites).

### 4.6.2 Potential requirements

REQ-Intent_ Interpretation_Assistance_1: The intent driven MnS producer should have the capability enabling the MnS consumer to provide information containing prior intent negotiations to the handler that can then be used by the intent handler to support the interpretation of the intent.

### 4.6.3 Potential solutions

- 	Add to the Intent IOC a description of the information about intent interpretation e.g., as:

-	The intent IOC includes intentInterpretationAssistanceInfo which contains prior negotiation interactions from the MnS consumer that are provided to the intent handling function and can be used by the intent handling function to support the interpretation of the intent. The intentInterpretationAssistanceInfo is a pair that captures the previous MnS producer questions, requests or reports and the corresponding consumer responses. It may include following information related to a timestamp, applicable intent context, and the previous MnS producer to MnS consumer interactions.

-		For example, the pair may include a key that is "the set of previous intent fulfillment options that were provided by MnS producer" as well as the corresponding value which is " the option that was selected by the MnS consumer".

- 	Introduce a data type and a corresponding attribute on the Intent IOC for intent interpretation assistance information say named intentInterpretationAssistanceInfo. The attribute is readable and writable.

- 	The intentInterpretationAssistanceInfo is a pair that captures the previous questions, requests or reports and the corresponding consumer responses. Examples can include the following attributes/ information:

- A timestamp

- Conditions: the conditions under which the intent was previously sent

- Producer report: the request sent by the MnS producer

- Consumer response: the responses provided by the MnS consumer for the specific MnS producer report.

Editor's note: An example of the exchanged information to be added

### 4.6.4  Evaluation of potential solutions

TBA

## 4.7 Use case #7: Enhancement of intent exploration

### 4.7.1 Description

In 3GPP TS 28.312 [2], the existing use case and requirements for intent Exploration is defined in clause 5.3.5. The IntentExplorationReport is defined in clause 6.2.1.3.19 to represent the intent exploration result, which includes the list of expectation exploration results. However, following scenarios are not supported:

- MnS consumer requests intent handling function to periodically report the best target values. For example, MnS consumer request intent handling function to report the best value for UENumberTarget of RadioServiceExpectation by the period of an hour. In this case, MnS producer need to execute the exploration process and send the intent exploration report with the best values for UENumberTarget to MnS consumer every hour. As the intent exploration process is complex and time-consuming (e.g. MnS producer may perform simulation activities to explore the best values), the MnS consumer needs to obtain the status (e.g., RUNNING, FINISHED, FAILED) of the intent exploration process for specific intent expectation (s).

- MnS consumer requests intent handling function to explore best target values for individual objects (e.g. area, cells) indicated in one IntentExpectation. For example, MnS consumer request to obtain the best values for UENumberTarget applied to ExpectationObject which represents a list of cells. In the request, MnS consumer wants to obtain the best values for UENumberTarget for each cell in addition to the best value for the list of cells. In this case, MnS producer needs to provide the following information in the intent exploration report:

-	The best UENumberTarget value for the list of cells represented by the ExpectationObject.

-	The best UENumberTarget value for each item of the list of cells represented by the ExpectationObject.

### 4.7.2 Potential requirements

REQ-IDMS-IntentExploration-CON-1: The intent driven MnS producer should have the capability enabling the MnS consumer to request to periodically obtain the exploration report.

REQ- IDMS-IntentExploration-CON-2: The intent driven MnS producer should have the capability enabling the MnS consumer to obtain the best target values for individual object represented by the ExpectationObject in an IntentExpectation.

### 4.7.3 Potential solution

This solution proposes to reuse and enhance the existing Intent and IntentReport IOC defined in 3GPP TS 28.312 [1].

Enhancement Aspect1: Extend the IntentReportControl <<dataType>> with following aspects:

- Extend the attribute "observationPeriod" in IntentReportControl <<dataType>> to indicate the time period for which the IntentExplorationReport is observed and reported. If the attribute "observationPeriod" is presented in the Intent instance with intentMgmtPurpose = EXPLORATION, MnS producer periodically explore the best target values and send the IntentExplorationReport to MnS consumer at the end of each observation period. If the attribute "observationPeriod" is absent in the Intent instance with intentMgmtPurpose = EXPLORATION, MnS producer performs the exploration process one time and send the IntentExplorationReport after finishing exploration process.

Enhancement Aspect2: For the ExpectationTarget <<dataType>> used for the attribute targetExplorationResult, the targetContext can be cellContext, coverageAreaPolygonContext. In this case, multiple TargetExplorationResult instances can be created for the same target name (e.g. UENumberTarget) with different cellContexts or coverageAreaPolygonContext. Following is one example for TargetExplorationResult instances in one IntentExplorationResult.

[UENumberTarget, IS_LESS_THAN, 10]

[UENumberTarget, IS_LESS_THAN, 4, Cell1]

[UENumberTarget, IS_LESS_THAN, 3, Cell2]

[UENumberTarget, IS_LESS_THAN, 3, Cell3]

Enhancement Aspect3: extend the IntentExplorationReport <<dataType>> with following aspects:

- Add the attribute "expectationExplorationStatus" to represent the status of intent exploration. The allowed values can be NOT_STARTED, RUNNING, FINISHED, FAILED.

### 4.7.4 Evaluation of potential solutions

Only one potential solution has been identified, which is feasible.

## 4.8 Use case #8: Support to express guarantee requirements in an intent

### 4.8.1 Description

In today’s highly dynamic and heterogeneous network environment, Intent-Driven Management Service (IDMS) is emerging as the foundational paradigm for advanced network operations and automation.  Nevertheless, practical deployments reveal a critical capability gap: the absence of robust support for dynamic resource reservation and release.

Intent expression, translation, negotiation, and fulfilment are intrinsically coupled to resource allocation—e.g., bandwidth, capacity and spectrum.  When the IDMS is used for communication service assurance in a specific time window, the MnS consumer should be allowed to express the guarantee requirements in the intent. Then, the MnS Producer can perform some actions (e.g., resource reservation) to guarantee the intent fulfilment in the future. Because these resources are highly time-variant, the current IDMS lacks the ability for the MnS Consumer to proactively reserve or release them on demand.  This shortcoming results in inefficient resource utilisation and reduced flexibility in intent fulfilment. Additionally, after an MnS Producer has classified an intent as “FEASIBLE” following the initial feasibility check, subsequent changes in network resource can render the intent “INFEASIBLE” before intent fulfilment is requested without actions taken for guarantee. This temporal inconsistency, caused by the absence of continuous feasibility validation, will lead to fulfilment failures, undermining the reliability of intent-based operations and degrading overall network performance.

An intent defines one or more requirements, goals and constraints that should be ensured for the intent to be considered fulfilled. The requirements are evaluated against the state of the network during the intent fulfilment feasibility check. Even if the result is feasible, no guarantees are explicitly provided by the MnS producer to ensure that the requirements will be continuously fulfilled over time. While the MnS producer may find a solution that meets the requirements when the intent is accepted, it may later degrade due to over-allocation, faults, or other issues.

To tackle this issue, the concept of guaranteed requirements is introduced. Such requirements should be considered for forward-looking sustained compliance. The MnS Producer should be able to utilize different methods to guarantee the intent fulfilment in the future.

NOTE: The guaranteed requirements may apply to all or part of an intent - that is, to the entire intent or to specific expectations. This study will determine which aspects of an intent should be covered by guaranteed requirements.

Since in reality there is no way to guarantee something all the time (i.e., there is always a probability that a requirements will be breached), Guaranteed requirements are associated with a confidence level that expresses the probability of successfully in fulfilling the requirements. The confidence level is specified as a probability threshold and is provided by the MnS producer. How to express confidence may depend on the context of the intent, e.g. for a connectivity service-related intent, the confidence may be expressed as service availability and service reliability. How the MnS producer will provide the confidence level to the requirements is implementation specific.

Since guarantees may not extend indefinitely, the MnS consumer may specify one or more guarantee periods over which the MnS producer provides the confidence level. The MnS consumer may specify multiple guarantee periods for the guarantee if it wants to have multiple time intervals where the expectations should be guaranteed. For example, if MnS Consumer wants to have expectations guaranteed only on weekdays, during business hours, etc..

### 4.8.2 Potential requirements

REQ-IDMS_Guarantee-1: The intent driven MnS producer should have the capability to allow MnS consumer to express an intent to be guaranteed and which expectations requirement in an intent should be guaranteed.

REQ-IDMS_Guarantee-2: The intent driven MnS producer should have the capability to allow MnS consumer to specify the guarantee period.

REQ-IDMS_Guarantee-3: The intent driven MnS producer should have the capability to report to the MnS consumer about the confidence level of fulfilling the guaranteed expectations at time of reporting.

### 4.8.3 Potential solutions

### 4.8.3.1 Potential solution #1

To support to express guarantee requirements in an intent, the following enhancements are proposed:

Add a new attribute guaranteeIndicator in the Intent IOC to allow the MnS Consumer to specify Guarantee Requirements in the intent. guaranteeIndicator attribute can be on intent expectation level.

### 4.8.3.2 Potential solution #2

Enhancement Aspect 1:

A new generic type of intentContext and expectationContext is proposed to be added to the Intent <<IOC>> and IntentExpectation <<dataType>>, respectively.

The presence of this new context indicates that the associated requirements should be guaranteed by the MnS producer. When the context is set at the intent level (intentContext), all expectation within that intent should be guaranteed. When it is set only to some of the intent expectations (expectationContext), only those expectations should be guaranteed.

The new guarantee period context specifies the time intervals for which the MnS Producer shall evaluate the confidence level of fulfilment, as explained in clause 4.8.1.

Allowed values:
- contextAttribute: "guaranteePeriod"
- contextCondition: " IS_ALL_OF "

- contextValueRange: a list of SchedulingTime <<choice>> defined in TS 28.622 [6]

Enhancement Aspect 2:

The confidence level that is calculated by the MnS Producer considering all guarantee periods should be reported as part of the intentFulfilmentReport, following the reporting period set by observationPeriod attribute of IntentReportControl <<dataType>>.

A new optional attribute guaranteeConfidenceLevel is proposed to be added to the IntentFulfilmentReport <<dataType>> and to the ExpectationFulfilmentResult <<dataType>>, to indicate the guarantee confidence level calculated by the MnS Producer.

Even though the guarantee level is reported as part of the fulfilment report, the reported value shall be interpreted as the confidence level associated with the guaranteed expectations. For example, if an intent expectation is defined as a guarantee expectation for a maximum delay of 10 ms, and the corresponding reported confidence level is 90%, this indicates that there is a 90% confidence that the latency will remain below 10 ms during the guarantee periods, even if the current observed fulfilment temporarily exceeds 10 ms.

The new attribute is provided when the corresponding intent or intent expectation is set for guarantee.

### 4.8.4  Evaluation of potential solutions

### Only one potential solution has been identified, which is feasible.

## 4.9 Use case #9: Intent handling capability configuration, registration and discovery

### 4.9.1 Description

#### 4.9.1.1 Intent handling capability description

In TS 28.312 [1], the existing use case, requirements and solution (including IntentHandlingCapability <<dataType>>) for intent handling capability obtaining are defined, which allows the MnS consumer to query the intent handling capabilities for a specific intent handling function. The use case also describes that different intent handling functions are deployed to support different areas of the same intent expectation object domain. However, the IntentHandlingCapability <<dataType>> (including supportedExpectationObjectType and supportedExpectationTargetInfoList) is not allowed to describe the supported area information. In additional, for radio network intent, multiple intent handling functions can be deployed to support different radio access technologies (e.g. EUTRAN, NR) and/or different frequencies. It is important to describe the supported radio access technologies (e.g. and frequencies for the radio network intent handling functions.

Several optional intent negotiation functionalities (including Intent Feasibility check, Intent Exploration and Intent Fulfilment Negotiation) are introduced in the TS 28.312 [1]. One intent handling function may support all the negotiation functionalities or part of the negotiation functionalities. For example, intent handling function A support both Intent Feasibility check and Intent Exploration functionalities, while intent handling function B only support Intent Feasibility check functionality. So, it is important to allow MnS consumer to know which negotiation capabilities can be provided by a specific intent handling function.

#### 4.9.1.2 Intent handling capability registration and discovery

In 3GPP TS 28.537 [6], the existing use case, requirements and solution (including MnSInfo IOC) for MnS discovery are defined, which allows MnS consumer to retrieve the list of MnS instances which can provide the intent driven management capability from MnS Registry. In this case, MnS consumer needs to send the query request to each intent handling function (represented by mnsAddress of the retrieved MnS instances from MnS Registry) to obtain the detailed intent handling capabilities.

However, in some scenarios, MnS consumer may need to request to retrieve a MnS instance with a specific intent handling capability from MnS Registry.

#### 4.9.1.3 Intent handling capability configuration

As TS 28.312 [1] described, multiple intent handling functions maybe deployed to support different expectation objects or to support different areas of the same intent expectation object domain. Operator may need to configuration the intent handling capability for each intent handling function with different responsibilities. For example, intent handling function A is configured to support handling radio service intent in the Venue A, while intent handling function B is configured to support handling radio service intent in the Venue B.

### 4.9.2 Potential requirements

REQ-IDMS_IHCO-CON-1: The intent driven MnS producer should have capabilities enabling an MnS consumer to obtain intent handling capabilities of each intent handling function, including supported contexts.

Note: Examples of contexts to be included as part of the intent handling capabilities are the ObjectContexts and the ExpectationContexts as defined in clause 6.2.2.1 of 3GPP TS 28.312 [2].

REQ-IDMS_IHCO-CON-2: The intent driven MnS producer should have capabilities enabling an MnS consumer to obtain intent negotiation capability of a specific intent handling function.

REQ-IDMS_IHCO-CON-3: The 3GPP management system should have capabilities enabling the configuration of the intent handling capability for a specific intent handling function.

Editor’s Note: which content of the intent handling capability can be configured needs further investigation.

### 4.9.3 Potential solution

This solution proposes to reuse and enhance the existing IntentHandlingFunction defined in 3GPP TS 28.312 [2].

Enhancement Aspect1: Add following attributes for IntentHandlingCapability <<dataType>>:

- supportedObjectContextInfoList, it represents the list of specific ObjectContexts supported by the intent handling function. The new SupportedObjectContextInfo datatype includes the supportedObjectContextAttribute, supportedObjectContextCondition and supportedObjectContextValueRange attributes.

- supportedExpectationContextInfoList – it represents the list of specific ExpectationContexts supported by the intent handling function. The new SupportedExpectationContextInfo datatype includes the supportedExpectationContextAttribute, supportedExpectationContextCondition and supportedExpectationContextValueRange attributes.

Enhancement Aspect2: Add following attributes for IntentHandlingFunction <<IOC>>:

- supportedNegotiationFunctionalities, it represents the list of intent negotiation functionalities supported by the intent handling function. The type is ENUM with allowed values: FEASIBILITYCHECK, EXPLORATION and FULFILMENT_WITH_NEGOTIATION. If the value is absent, none of the intent negotiation capabilities are not supported by the intent handling function.

- intentHandlingScope, representing the scope of received intent that the intent handling function can support to address, the allowed values can be RAN, or CN.

### 4.9.4 Evaluation of potential solutions

Only one potential solution has been identified, which is feasible.

## 4.10 Use Case#10: Radio network delivering in transient overload scenario

### 4.10.1 Description

This use case describes a scenario where a MnS consumer express intent containing an expectation for ensuring a radio network in transient overload scenarios (e.g., high-speed rail or subway systems) to a MnS producer. In transient overload scenarios, the high load caused by massive user access and bursty traffic typically lasts only a few seconds specifically when trains pass through a cell, while traffic volume remains extremely low during other periods.

In 3GPP TS 28.312 [1], the existing use case and requirements for intent containing an expectation for delivering a radio network is described in clause 5.1.2. The RadioServiceExpectation is defined to represent MnS consumer's expectations for radio network delivering in the specified area, and it serves to support service assurance efforts. However, transient overload scenario is not supported.

-	MnS consumer expresses the radio network delivering expectation with transient load information. It can help MnS producer identify the actual high load information when these shot-duration, high-impact events occur thereby supporting network expansion decision-making.

### 4.10.2 Potential requirements

REQ-IDMS_RadioServiceIntent -CON-1: The intent driven MnS producer for radio network should have capabilities enabling the MnS consumer to express transient load requirements.

### 4.10.3 Potential solution

This solution proposes to reuse and enhance the existing RadioNetworkExpectation defined in 3GPP TS 28.312 [2].

Enhancement Aspect1: Add following attributes as the ExpectationTargets for the RadioNetworkExpectation to support transient overload requirements:

- PrbHighLoadRatio, it represents the load target for the radio network that the intent expectation is applied. The detailed definition for PrbHighLoadRatio see TS 28.554 [8]. The type is Real.

### 4.10.4 Evaluation of potential solutions

TBD

## 4.11	Use case #11: Enhancing intent feasibility check capability

### 4.11.1	Description

This use case describes proposed enhancements for feasibility check, enabling intent-driven MnS consumer to receive information regarding how to make an infeasible intent feasible together with feasibility check in case the intent is deemed as infeasible. This will support MnS consumers to understand the changes needed for a feasible intent.

In 3GPP TS 28.312 [1], clause 5.3.3.1 states the following:

“In case the result of intent fulfillment feasibility check is infeasible, MnS producer notifies the MnS consumer the reason of infeasibility and corresponding recommendations, then the MnS consumer decides how to handle the issue that intent is infeasible, e.g. update the intent, suspend the intent, delete the intent, etc.”

However, the issue is the IntentFeasibilityCheckReport specified in 3GPP TS 28.312 [1], clause 6.2.1.3.10 does not include corresponding recommendations. The MnS consumer may request exploration as specified in 3GPP TS 28.312 [1] Clause 5.3.5, based on inFeasibleExpectationInfos which is an optional attribute indicating the infeasible expectations including infeasible targets.  However, if the inFeasibleExpectationInfos attribute is not supported, then it is very difficult for the intent-driven MnS consumer to update an infeasible intent as they do not have the recommendation on how to make it feasible.

### 4.11.2	Potential requirements

REQ-Intent_FEAS1: The intent driven MnS producer should have the capability to enable the authorized MnS consumer to receiverecommended values for infeasible targets .

### 4.11.3	Potential solutions

#### 4.11.3.1	Potential solution #1 for a new procedure of feasibility check with exploration

This solution proposes to reuse the existing generic intent exploration report and intent feasibility check report defined in 3GPP TS 28.312 [1] and to enhance the Intent <<IOC>>.

In order to enable an MnS consumer to request a feasibility check with corresponding recommendations, a procedure is proposed with ensuring feasibility check with exploration is supported and enabled by a new allowed value “FEASIBILITYCHECK_WITH_EXPLORATION” that is to be included for intentMgmtPurpose attribute of Intent <<IOC>> which is specified in 3GPP TS 28.312, clause 6.2.1.4. When feasibility check with exploration is requested by the intent-driven MnS consumer, the intent driven MnS producer can report corresponding recommendations for infeasible expectations with existing IntentExplorationReport <<dataType>>, together with existing IntentFeasibilityCheckReport <<dataType>>.

This solution does not have any impact on the intent lifecycle management states studied in clause 4.13 of the present document.

#### 4.11.3.2	Potential solution #2 Extend inFeasibleTarget with recommended values

This solution proposes to reuse and enhance the existing IntentFeasibilityCheckReport<<dataType>>defined in 3GPP TS 28.312 [1].

Enhancement Aspect1: Extend the type for attribute “inFeasibleTargets” from “String” to “inFeasibleTargetInfo <<dataType>>”, which includes following attributes:

- targetName, represents the targetName for the inFeasibleTarget.

- recommendedValue, represents the recommended value for the inFeasibleTarget.

### 4.11.4 Evaluation of potential solutions

Two potential solutions are identified in clause 4.11.3. Both potential solution #1 and potential solution #2 support the capability to allow MnS consumer to receive recommended values for infeasible targets in an infeasible intent.

## The implementation of both potential solutions is not complex. Therefore, both potential solutions described in clause 4.11.3 are feasible solutions.

## 4.12 Use case #12: Documentation for the overview of intent driven management functionalities

### 4.12.1 Description

In TS 28.312 [1], several intent management functionalities (including but not limited to Intent handling capability obtaining, intent exploration, intent feasibility check, intent report management) are introduced in different releases. However, the overview of intent driven management functionalities and corresponding usage for different phases are missing. It is important and useful to illustrate the intent driven management functionalities in TS 28.312 [1].

### 4.12.2 Potential requirements

### 4.12.3 Potential solution

It proposes to introduce a new concept section in clause 4 in TS 28.312 [1] as showed below to illustrate the overview of intent driven management functionalities.

4.X Overview of intent driven management functionalities for different phases

4.X.1 Introduction

The intent management functionalities for different phases include:

- Intent investigation and pre-evaluation

- Intent fulfilment

Above intent management functionalities applied for all intent categorizes, including Intent-CSC, Intent-CSP and Intent-NOP.

4.X.2 Intent investigation and pre-evaluation

In this phase, the MnS consumer finds out what intent content (a list of expectations) is feasible before expressing the intent expectations to be fulfilled by the intent handling function. The network (including NEs) will not be changed during this phase. This includes three functionalities:

- Intent handling capability obtaining. The functionality allows MnS consumer to find the intent handling function which has the necessary domain responsibilities and supports their required intent expectations. The MnS consumer can subscribe or query the intent handling capabilities (e.g., supported ExpectationObject, supportedExpectationTarget) for a specific intent handing function.

- Intent exploration. The functionality allows MnS Consumer to explore what the results of the wanted intent expectations would be and what is the best result the intent handling function can achieve. The MnS consumer can explore the best value(s) for specific intent target(s) (e.g. NumberofUEs, RANEnergyConsumption).

- Intent feasibility check. The functionality allows MnS consumer to verify or check the feasibility whether the proposed intent expectation is possible for an intent handling function.

4.X.3 Intent fulfilment:

In this phase, the intent handling function verifies the received intent and fulfils the given intent within its domains of responsibility. This includes following functionalities:

- Intent translation and execution. The functionality enables the intent handling function to translate the received intent to executable actions and performing the actions.

- Intent fulfilment evaluation. The functionality enables the intent handling function to evaluate the result about the intent fulfilment and generate the intent fulfilment reports. The intent handling function needs to continuously perform intent translation, execution and evaluation to ensure the intent fulfilment.

- Intent fulfilment negotiation. The functionality enables the intent handling function and the MnS consumer to agree on the best way to fulfil an intent.

- Intent conflict management. The functionalities enable the intent handling function to detect the intent related conflicts and collaborate with MnS consumer to resolve the conflicts. Following mechanism can be used to resolve the conflicts to improve the intent fulfilment:

- Intent priority and expectation preference. The functionality enable intent handling function resolve intent related conflicts based on intent priority and expectation preference specified by MnS consumer.

- Intent utility function. The functionality defines a method by which MnS consumers can express the relative value of an intent's expectations to assist the intent handling functions in fulfilling their intents in the most acceptable manner.

Editor’s Note: The above content maybe updated based on R20 progress.

Note: A mapping between the different phases described in 4.X.2 and 4.X.3 and the high-level phases described in Annex B in TS 28.312 [1] should be discussed and addressed during R20 normative work.

### 4.12.4 Evaluation of potential solutions

Only one potential solution has been identified, which is feasible.

## 4.13 Use case #13: Investigation on the intent lifecycle management state transition

### 4.13.1 Description

3GPP TS 28.312 [1] studies the state transitions during fulfilment phase. However, the intent Lifecycle management states that describes the pre-evaluation phase (i.e. intent feasibility check and intent exploration) and negotiation in intent fulfilment phase is missing. The main difference between the pre-evaluation phase, fulfilment phase and negotiation in intent fulfilment phase is on the intentMgmtPurpose. Thus, investigation on the intent lifecycle management state transition should be made, including the intent lifecycle management state transition diagram and the state transition events based on the intentMgmtPurpose.

In clause 4.7 of 3GPP TS 28.312 [1], the state transitions have been described in Figure 4.7-1. However, how intent negotiation, feasibility check, and validity are mapped to this figure and represented in intent state diagram is missing. Additionally, it is not clear that the state diagram and state transition events are combined in one figure. Thus, further enhancements should be made in intent handling state and lifecycle management from several aspects, including but not limited to the following:

-	Addressing the issue of the unclear combination of the state diagram and state transition events in one figure by explicitly defining new states and state transitions.

-	Clarifying the issues with the state transition events.

### 4.13.2 Potential requirements

### 4.13.3 Potential solutions

### 4.13.3.1 Potential solution #1

It proposes to add intent lifecycle management state transition diagram and intent lifecycle management state transition table to indicate the state transitions. Figure 4.13.2.1 shows the intent lifecycle management state diagram, where the number in the Figure identify the changes to the intent lifecycle management state. The explanations for the changes are described in Table 4.13.2.1.

Figure 4.13.2.1: intent lifecycle management state transition diagram

The transition numbers in the first column represent the intent lifecycle management state changes in Figure 4.13.2.1. The interactions specified under the column " Intent lifecycle management state transition events" of Table 4.13.2.1 shall be present for the transition.

NOTE 1:	The transition numbers do not indicate any strict order and not correspond to procedure steps.

NOTE 2:	Figure 4.13.2.1 may not be complete, and other states and transitions may be added.

Table 4.13.2.1: The intent lifecycle management state transition diagram table

| Transition number | Intent lifecycle management state transition events | Intent lifecycle management state |
| --- | --- | --- |
| 1 | The MnS producer creates the intent instances for intent exploration based on the received intent creation request, with the intentMgmtPurpose specified as "EXPLORATION" and the value of specific intent targets or contexts that need to be explored initially set to 'NULL'. | EXPLORATION |
| 2 | The MnS producer creates the intent instances for intent feasibility check based on the received intent creation request, with the intentMgmtPurpose specified as "FEASIBILITYCHECK" or "FEASIBILITYCHECK_WITH_EXPLORATION".. | FEASIBILITYCHECK |
| 3 | The MnS producer triggers the exploration process when intentMgmtPurpose is specified as "FEASIBILITYCHECK_WITH_EXPLORATION" and the feasibilityCheckResult is infeasible. | EXPLORATION |
| 4 | The MnS producer creates or modify the intent instances during fulfilment phase based on the received request, with the intentMgmtPurpose specified as " FULFILMENT_WITH_NEGOTIATION" or " FULFILMENT_ WITHOUT _NEGOTIATION ". | FULFILMENT |
| 5 | The MnS producer triggers the negotiation process. The process only can be triggered when intentMgmtPurpose is specified as " FULFILMENT_WITH_NEGOTIATION". | NEGOTIATION |
| 6 | The MnS producer finishes the negotiation process. | FULFILMENT |

NOTE 3: The Table 4.13.2.1 also provides information regarding the impact of the potential solution provided in clause 4.11.3.1 for enhancing intent feasibility check capability. The potential solution provided in clause 4.11.3.1 does not require a new intent lifecycle management state in potential solutions studied in clause 4.13.3.

### 4.13.3.2 Potential solution #2

It proposes to add a new intent handling state diagram based on Figure 4.13.3.2-1. New FULFILLING and EXPLORING states are defined to represent the full lifecycle of the intent and Figure 4.13.3.2-1 shows the intent handling state diagram, where the numbers identify the state changes. The explanations for the state changes are described in Table 4.13.3.2-1.

Figure 4.13.3.2-1: Intent handling state diagram

Table 4.13.3.2-1: The intent handling state transition table

| Transition number | The state transition events | State |
| --- | --- | --- |
| 1 | Upon receiving an intent creation request, the MnS Producer creates the corresponding intent instance. The MnS Producer verifies the intent syntax, assess its compliance with the MnS Producer’s supported capabilities, and validates its overall correctness. If the intent creation request is accepted, the intent state transitions to RECEIVED. If the request is rejected, the intent state transitions to TERMINATED. | RECEIVED |
| 2 | The MnS Producer may perform a feasibility check of the intent instance by transitioning it to the EXPLORING state. In this state, the MnS Producer and MnS Consumer may exchange negotiation or modification requests, or terminate the intent. The transition to EXPLORING may occur after the intent has been received, before the fulfilment phase to assess its feasibility, or upon a specific request from the MnS Consumer. The request can be issued by the MnS Consumer when the intent state is FULFILING, DEGRADED or FULFILED. | EXPLORING |
| 3 | If the intent is correct and is accepted, it may directly transition to FULFILLING state, or it may do so after the EXPLORING state or upon a specific request from the MnS Consumer. In the FULFILLING state, the intent handling function initiates the necessary actions to meet the intent requirements. | FULFILLING |
| 4 | If the MnS Producer determines that it is unable to take any further actions to fulfil the intent, the intent state transitions to DEGRADED. | DEGRADED |
| 5 | The MnS producer evaluates that the intent has been fulfilled. | FULFILLED |
| 6 | TThe MnS Producer determines that the intent, which was previously not fulfilled, has recovered and is now considered FULFILLED. | FULFILLED |
| 7 | The MnS Producer determines that the intent, which was previously considered FULFILLED, is no longer meeting the stated requirements after a period of observation. | DEGRADED |
| 8 | Upon request by the MnS Consumer, the MnS Producer configures the intentAdminState attribute with the value "DEACTIVATED" and the intent transitions to SUSPENDED. | SUSPENDED |
| 9 | The MnS Producer modifies the intent instance based on the received intent modification request. Correspondingly, the intent state shall transition from EXPLORING, SUSPENDED, FULFILLING, or DEGRADED to RECEIVED. | RECEIVED |
| 10 | The MnS producer deletes the intent instances upon receiving the intent deletion request. | TERMINATED |

### 4.13.4 Evaluation of potential solutions

Two solutions have been proposed concerning the lifecycle management state transitions. The evaluation considers how well each solution aligns with the intent states already defined in TS 28.312, the clarity and completeness of the state transitions, and the implementation complexity. TS 28.312 already defines lifecycle states ACKNOWLEDGED, FULFILLED, DEGRADED, SUSPENDED and TERMINATED.

Potential solution #1 introduces an intent lifecycle management state diagram defineing discrete states for EXPLORATION, FEASIBILITYCHECK, FULFILMENT_WITH_NEGOTIATION and FULFILMENT_WITHOUT_NEGOTIATION. This explicit split between exploration, feasibility check and fulfilment clarifies the existence of different management purposes for the intent instance, but the proposal does not clearly explain how transitions occur when the intent is modified (the management purpose of an existing intent instance is changed by the MnS consumer). The value for adding extra states for each of the management purposes is also not clear.

Potential solution #2 proposes an intent handling state diagram with states RECEIVED, EXPLORING, FULFILLING, DEGRADED, FULFILLED, SUSPENDED and TERMINATED. These states map naturally to those in TS 28.312: RECEIVED corresponds to ACKNOWLEDGED (initial state after creation), while FULFILLED, DEGRADED, SUSPENDED and TERMINATED mirror the normative states. The EXPLORING state groups together feasibility checking and negotiation. This reflects the TS 28.312 description that the MnS producer performs an automatic feasibility check upon receiving or modifying an intent and reports the result to the consumer, who may modify, suspend or delete the intent. By combining pre‑evaluation and fulfilment phases and introducing a separate FULFILLING state for execution, solution #2 offers a simple and implementable state machine.

In summary, solution #1 provides detailed pre‑fulfilment phases but diverges from existing TS 28.312 states and introduces extra complexity. Solution #2 maintains fidelity with existing specification states, unifies feasibility checking and negotiation, and introduces only two transient states (EXPLORING and FULFILLING) between intent creation and completion. This simplicity makes the state machine easier to understand and implement while still supporting negotiation and fulfilment handling.

## 4.14 Use case #14:  intent expectation satisfied information

### 4.14.1	Description

In 3GPP TS 28.312 [1], the existing intentReport allows the IDMS consumer to obtain the values of KPI as indicated by the corresponding expectation targets. However, this information may not accurately reflect the actual situation. For example, considering an intent that includes a radio network expectation with the target aveDLRANUEthptTarget, the existing intent report provides the average value of the downlink RAN UE throughput and tells whether the intent expectation is fulfilled or not based on the average value. However, in the case where the average value is impacted by some extreme values, the fulfilled intent report will miss the information that the majority of UE experience doesn’t fulfil the UE throughput specified in the target. Such information matters because it helps the network operator to have a better understanding on the intent expectation satisfaction. The report can include the information besides the average information, which can be entity based, or time based, and is derived through statistical measurement or calculation, such as distributions, range or std deviation.

### 4.14.2	Potential requirements

REQ-Intent_IESI-1: The intent driven MnS producer should have a capability providing information besides average value in the intent report.

### 4.14.3	Potential solutions

TBD

### 4.13.4 Evaluation of potential solutions

TBD

## 4.15	Use case #15: Relation and Interaction with MnS producers for AI/ML Management

### 4.15.1	Description

This use case describes a scenario where an intent driven MnS producer interacts and coordinates with MnS producers for AI/ML management which is specified in 3GPP TS 28.105 [10]. Such coordination can enable the intent-driven MnS producer to both leverage AI/ML for its own internal intent handling tasks, as well as for when AI/ML processing might be needed in the network as an outcome of processing the intents.

As an example of leveraging AI/ML, in order for the intent-driven MnS producer to fulfil an expectation, the intent driven MnS producer may request to control the inference, e.g., activate/deactivate the ML model/models, configure the allowed ranges of the inference output parameters, request fine-tuning of an ML model etc. These are possible cases whereby the intent driven MnS producer can use an ML model and interact as an MnS consumer with MnS producers described in 3GPP TS 28.105 [10], clause 6 for ML model lifecycle management, such as ML model training, ML testing, ML model deployment, AI/ML inference, AIML Inference emulation.

In the case of AI/ML being leveraged as an outcome of the intent handling, an example can be considered whereby in order for the intent-driven MnS producer to check the feasibility of an intent or exploration of an intent in pre-evaluation phase as specified in 3GPP TS 28.312 [1], the intent-driven MnS producer may request, for example, AIML Inference emulation before the ML model is applied for fulfilment in production network.

### 4.15.2	Potential requirements

REQ-Intent_AIML-1: The intent driven MnS producer should have the capability to interact with MnS producers for ML model lifecycle management in both intent pre-evaluation phase and intent fulfilment phase.

### 4.15.3	Potential solutions

This potential solution considers enabling an intent-driven MnS producer to interact with MnS producers for ML model lifecycle management. The solution does not require updating the NRM or already specified interfaces, and it only necessitates of updating the 3GPP TS 28.312 [1] to describe that the intent driven MnS producer,  if necessary, can interact with MnS producers for ML model lifecycle management in both intent pre-evaluation phase and intent fulfilment phase, with reference to 3GPP TS 28.105 [10].

### 4.15.4 Evaluation of potential solutions

The potential solution proposed in clause 4.15.3.1 satisfies the requirement identified in clause 4.15.2. The solution does not require updates to the NRM and/or interfaces, only describing the possible interactions between intent-driven MnS producer and MnS producer for ML model lifecycle management in 3GPP TS 28.312 [1], and hence, it is a feasible solution.

## 4.16 Use case #16: Investigation on the applicability and potential impacts to support natural language intents translation

### 4.16.1 Description

In TR 28.914 [9], clause 5.6 described the use case for investigating potential impacts to support natural language intents translation. It also states that there are some use cases where the use of natural language for intent expression would be preferred. For example, on the human-machine interface presenting and expressing intent using natural language can lead to a more intuitive experience for human users. A concrete example of natural language RAN ES intent can be “Reduce 10% RAN energy consumption without impact on user experience for Region X”. The use case also described that if an intent interpreter is introduced to process intents described in natural language, it is necessary to clarify its position within the existing intent management framework, especially its relationship with the MnS consumer.

Scnenario#1: Intent Interpreter as a separate function outside the intent handling function. In this scenario, Intent interpreter receive the natural language intent and translate it into a formal intent (the formal intent contains the attributes and corresponding values to enable the intent handling function to decide the actions to fulfil the intent expectation) according to intent model defined in TS 28.312 [1] and consume the 3GPP Intent driven MnS with formal intent model provided by intent handling function.

Figure 4.16-1 Deployment scnenario#1: Intent Interpreter as a separate function outside the intent handling function.

Scnenario#2: Intent Interpreter is a function integrated in intent handling function. In this scenario, Intent Interpreter receives the natural language intent from consumer and translates it into detailed actions to be executed to fulfil the intent.

Based on above two deployment scenarios, this study can investigate whether there is a need for a potential solution to enhance the intent-driven management service to support natural language.

Figure 4.16-2 Deployment scnenario#2: Intent Interpreter is a function integrated in intent handling function.

## 4.17 Use case #17: Enhancement of core network and service delivering and assurance scenarios

### 4.17.1 Description

This use case proposes to add the scenarios on core network and service delivering and assurance to support the SLA of core network services. In 3GPP TS 28.312 [1], the existing use case and requirements for intent containing an expectation for 5GC network is described in clause 5.1.8, enabling MnS consumers to express intentions for 5GC network delivery (e.g., deploying 5GC sub-networks in specified areas) and performance assurance (e.g., maximum number of PDU sessions, number of registered users). Additionally, the 5GC Network Expectation defined in Clause 6.2.2.1.4 covers basic parameters like NF type, location, PLMN, and TAI. However, the existing capabilities fail to meet the following service experience--specific requirements:

- MnS consumer expresses core network SLA-level latency assurance expectation for specific service flows.

- MnS consumer expresses the preference on NF selection (e.g., UPF) and binding expectation for core network services.

### 4.17.2 Potential requirements

REQ-IDMS_CNServiceIntent-1: The intent driven MnS producer for core network service should have capabilities enabling the MnS consumer to express service latency requirements.

REQ-IDMS_CNServiceIntent-2: The intent driven MnS producer for core network service should have capabilities enabling the MnS consumer to express the preference on NF selection (e.g., UPF).

### 4.17.3 Potential solutions

TBD

### 4.17.4 Evaluation of potential solutions

TBD

## 4.18 Use case #18: The relation and the interactions between intent handling function and NDTFunction

### 4.18.1 Description

This use case proposes a scenario where intent-driven management services (IDMS) are enabled through the Network Digital Twin (NDT). A Network Digital Twin, as referenced in 3GPP TR 28.561 [12], is used as a replica of a mobile network, in order to learn how an actual mobile network would behave in certain scenarios, without causing any impacts to the real network. This capability is crucial for enabling intent handling function to validate intent feasibility and explore the best values for corresponding targets.

For example, during the pre-evaluation phase, as specified in 3GPP TS 28.312 [1], when received an intent from the MnS consumer, the intent handling function can invoke NDT’s simulation capabilities to perform an intent feasibility check or explore best values for intent targets, validating the impact of these intents. This helps validate whether an intent (e.g., "assuring 500 concurrent UEs in a stadium during a concert") can be fulfilled before initiating intent fulfilment or explore the best values for intent targets within the optimal network parameters (e.g., adjusting UE group priorities to avoid resource conflicts) in the NDT.

### 4.18.2 Potential requirements

REQ-Intent_NDT-1: The intent driven MnS producer should have the capability to interact with NDTFunction to validate the feasibility of intents or explore the best values of intent targets in intent pre-evaluation phase.

REQ-Intent_NDT-2: The intent driven MnS producer should have the capability to allow MnS consumer to obtain the information of NDTFunctions as enabler information used for intent feasibility check and intent exploration.

### 4.18.3 Potential solutions

### 4.18.4 Evaluation of potential solutions

TBD

## 4.19 Use case #19: Enhancement of radio service scenarios for service protection

### 4.19.1 Description

In 3GPP TS 28.312 [1], the existing use case and requirements for intent containing an expectation for delivering a radio service is described in clause 5.1.2. The RadioServiceExpectation is defined to represent MnS consumer's expectations for radio service delivering and assurance in the specified area. However, current definitions do not provide means for expressing service-related protection expectations, i.e., requirements related to the level of protection or security controls to be applied to the managed service or resources.

NOTE: Even though this use case proposes to enhance the Radio Service scenario in 28.312 [1], the service-related protection expectations are scenario-agnostic and could be applicable to other use cases.

Service protection in this context refers to the set of security mechanisms (e.g., access control, traffic filtering, traffic encryption, traffic integrity) that aim to prevent malicious or unauthorized access to the different attack surfaces exposed by managed services or resources.

Clause 5 of TS 33.501 [13] lists the security requirements and features for 5G, which cover different service-related protection aspects, e.g., integrity protection, confidentiality protection, replay protection, privacy protection, etc.

## 4.20	Use case #20:  New deployment scenario for Intent Handling Function at ManagedElement

### 4.20.1	Description

This task investigates the need for a new deployment scenario for IHF at ManagedElement layer.

Discussion to date has not identified the need for such a scenario in 5GA.  As such, the topic concludes with no potential requirements nor potential solutions proposed in this study.

### 4.20.2	Potential requirements

FFS

### 4.20.3	Potential solutions

### FFS

### 4.20.4	Evaluation of potential solutions

Not applicable.

# 5	Conclusions and Recommendations

## 5.1 Use case #1: Enhancement of radio service delivering and assurance scenarios

The use case of enhancement of radio service delivering and assurance scenarios is introduced in clause 4.1 and following management capabilities are identified:

- MnS consumer expresses the radio service delivering and assurance intent expectation with service reliability information.

- MnS consumer expresses the radio service delivering and assurance intent expectation for a specified area described in the form of civic location.

- MnS consumer expresses the radio service delivering and assurance expectation for a specified assurance time duration.

It is recommended to enhance the RadiServiceExpectation defined in 3GPP TS 28.312 [1] to support above identified management capabilities:

- add "ReliabilityTarget " as the ExpectationTarget.

- add "CivicAreaContext" as the ObjectContext, which support both civic address and location label.

- add "AssuranceDurationContext" as the ExpectationContext.

The detailed solution in clause 4.1.3 is used as baseline for normative work.

## 5.2 Use case #2: Enhancement of radio network performance assurance scenarios

The use case of enhancement of radio network performance assurance scenarios is introduced in clause 4.2 and following management capabilities are identified:

- MnS consumer expresses radio network performance assurance expectation for a specific RAN feature.

It is recommended to enhance the RadiNetworkExpectation and generic ExpectationTarget defined in 3GPP TS 28.312 [1] to support above identified management capabilities:

- add "rANFeatureContext" as ObjectContexts for the RadioNetworkExpectation to represent the expected specific RAN feature for RAN Subnetwork that the intent expectation is applied.

## The detailed solution in clause 4.2.3 is used as baseline for normative work.

## 5.3 Use case #3: Assisting and reporting intent decomposition across intent handling functions

The use case description, requirements and a potential solution for assisting and reporting intent decomposition across intent handling functions are described in clause 4.3. This use case enables an MnS consumer to specify the intent handling functions that are not recommended for intent decomposition as an expectation context as well as to receive an intent report regarding intent decomposition.

The potential solution described in clause 4.3.3.1, which proposes enhancing the existing intent expectations with adding a new context as well as enhancing the intent report with information on decomposition of an intent, can be used as baseline for normative work.

NOTE: During normative phase, enhancing the intent report with information on intent decomposition can be considered with the solution proposed for intent traceability in clause 4.4.3.1 of present document.

## 5.4 Use case #4: Intent traceability

## 5.5 Use case #5: Invariant Guidance in Intent Contexts

## 5.6 Use case #6: Intent Interpretation Assistance Information

## 5.7 Use case #7: Enhancement of intent exploration

The use case of enhancement of intent exploration is introduced in clause 4.7 and following management capabilities are identified:

- The capability enabling the MnS consumer to request to periodically obtain the exploration report.

- The capability enabling the MnS consumer to obtain the best target values for individual object represented by the ExpectationObject in an IntentExpectation.

It is recommended to enhance the generic information model definition for intent driven management defined in 3GPP TS 28.312 [1] to support above identified management capabilities:

- Extend the attribute "observationPeriod" in IntentReportControl <<dataType>> to indicate the time period for which the IntentExplorationReport is observed and reported.

- Add cellContext, coverageAreaPolygonContext as targetContexts for the ExpectationTarget <<dataType>> which is used for the attribute targetExplorationResult.

- Extend the IntentExplorationReport <<dataType>> by adding the attribute "expectationExplorationStatus".

The detailed solution in clause 4.7.3 is used as baseline for normative work.

## 5.8 Use case #8: Support to express guarantee requirements in an intent

## 5.9 Use case #9: Intent handling capability configuration, registration and discovery

The use case of Intent handling capability configuration, registration and discovery is described in clause 4.11 and following management capabilities are identified:

- The capability enabling an MnS consumer to obtain intent handling capabilities of each intent handling function, including supported contexts.

- The capability enabling an MnS consumer to obtain intent negotiation capability of a specific intent handling function.

It is recommended to enhance the generic information model definition for intent driven management defined in 3GPP TS 28.312 [1] to support above identified management capabilities:

- Add attribute “supportedObjectContextInfoList” and “supportedExpectationContextInfoList” as attributes of IntentHandlingCapability <<dataType>>.

- Add attribute “supportedNegotiationFunctionalities” and “intentHandlingScope” as attributes of IntentHandlingCapability <<dataType>>.

The detailed solution in clause 5.9.3 is used as baseline for normative work.

## 5.10 Use Case#10: Radio network delivering in transient overload scenario

## 5.11 Use case #11: Enhancing intent feasibility check capability

The use case of enhancement of intent feasibility check capability is introduced in clause 4.11 and following management capabilities are identified:

- The capability enabling the MnS consumer to receive recommended values for infeasible targets.

The potential solution described in clause 4.11.3.1 proposes a new allowed value “FEASIBILITYCHECK_WITH_EXPLORATION” that is to be included for intentMgmtPurpose attribute.

The potential solution described in clause 4.11.3.2 proposes to extend the type for attribute “inFeasibleTargets” from “String” to “inFeasibleTargetInfo <<dataType>>”, which includes targetName and recommendedValue.

## 5.12 Use case #12: Documentation for the overview of intent driven management functionalities

The overview of intent driven management functionalities and corresponding usage for different phases is introduced in 4.12.3, including intent investigation and pre-evaluation phase and intent fulfilment.

It is recommended to add a new concept section in clause 4 in TS 28.312 [1] to illustrate the overview of intent driven management functionalities. The detailed solution in clause 4.12.3 is used as baseline for normative work.

## 5.13 	Use case #13: Improve intent life cycle documentation

The evaluation shows that potential solution #2 (clause 4.13.3.2) aligns most closely with the intent state definitions and behaviours described in TS 28.312. States like FULFILLED, DEGRADED and SUSPENDED are already defined. By introducing only two new transient states - EXPLORING for feasibility check and negotiation and FULFILLING for the fulfilment phase - solution #2 provides a clear separation between non-fulfilment and fulfilment activities without creating additional observable states. The EXPLORING state captures the automatic feasibility check and any negotiation that occurs before the intent progresses to fulfilment or is deleted. The FULFILLING state explicitly represents ongoing execution, allowing the system to report progress and to transition to DEGRADED, FULFILLED, SUSPENDED or TERMINATED as appropriate.

The detailed potential solution #2 in clause 4.13.3.2 should be used as baseline for normative work. The new description and state diagram should replace the content in clause 4.7 of TS 28.312 [1].

## 5.14 Use case #14: Intent expectation satisfied information

## 5.15 	Use case #15: Relation and Interaction with MnS producers for AI/ML Management

The use case description, requirements and a potential solution for relation and interaction with MnS producers for AI/ML Management are described in clause 4.15. This use case clarifies the relation and interaction of intent-driven MnS producers with MnS producers for ML model lifecycle management in both intent pre-evaluation phase and intent fulfilment phase.

The potential solution described in clause 4.15.3.1, which proposes adding text in 3GPP TS 28.312 [1] in order to describe possible interactions between intent-driven MnS producer and MnS producer for ML model lifecycle management can be used as a baseline for normative work. The exact text and the clause to which it belongs can be decided during the normative phase.

## 5.16 Use case #16: Investigation on the applicability and potential impacts to support natural language intents translation

Regarding the Deployment scnenario#1(Intent Interpreter as a separate function outside the intent handling function), the Intent Interpreter is MnS consumer’s internal implementation and the existing intent driven management service can be used to satisfy the interaction between intent handling function and MnS consumer with Intent Interpreter embedded.

Regarding the Deployment scnenario#2(Intent Interpreter is a function integrated in intent handling function), needs further investigation and discussion in 6G to identify corresponding requirements.

## 4.17 Use case #17: Enhancement of core network and service delivering and assurance scenarios

## 4.18 Use case #18: The relation and the interactions between intent handling function and NDTFunction

## 4.19 Use case #19: Enhancement of radio service scenarios for service protection

## 4.20 Use case #20: New deployment scenario for Intent Handling Function at ManagedElement

Annex A:
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2025-08 | SA5#162 |  |  |  |  | Initial version. | 0.0.0 |
| 2025-09 | SA5#162 | S5-254121 | draftTR |  |  | Draft TR v0.1.0 S5-254032 Add structure proposal S5-254033 Add new issue for enhancement of radio service delivering and assurance scenarios S5-254034 Add new issue for enhancement of radio network performance assurance scenarios S5-254035 Add issue description, requirement and solution for intent decomposition S5-254036 Add issue, requirements and potential solution for intent traceability S5-254037 Invariant Guidance in Intent Contexts Intent interpretation information S5-254038 Add new issue for Intent exploration enhancement S5-254039 Add Key Issue on Intent feasibility check S5-254040 enhancement to support resource reservation S5-254042 Add new issue for Intent handling capability registration and discovery | 0.1.0 |
| 2025-10 | SA5#163 | S5‑254896 | draftTR |  |  | Draft TR v0.2.0 S5-254645 Add solution for enhancement of radio network performance assurance scenarios S5-254646 Add solution for enhancement of radio service delivering and assurance scenarios S5-254647 Add new requirements and solution for enhancement radio service delivering and assurance scenarios S5-254648 Add new use case for radio service assurance in transient overload scenarios S5-254649 Add use-case description, requirement and solution for enhancing feasibility check S5-254650 Add Solution to Support to Express Guarantee Requirements in an Intent S5-254652 Add solution for Intent exploration enhancement S5-254653 Documentation for the overview of intent driven management functionalities S5-254654 Add use case for the investigation on the transition of intent Lifecycle management state S5-254655 Add Use case on intent expectation satisfaction information S5-254656 Add solution for Intent handling capability configuration, registration and discovery S5-254657 Add use-case description, requirement and solution for relation and interaction with AIML S5-254658 Add use case for the Investigation on the applicability and potential impacts to support natural language intents translation S5-254659 Add solution for use case#3 S5-254660 Add potential solution for UC#4 intent traceability S5-254884 Add description and requirements for intent guarantee UC#8 | 0.2.0 |
| 2025-11 | SA5#164 | S5-255470 | draftTR |  |  | S5-255071: Add evaluation and conclusion for Use case #7 Enhancement of intent exploration S5-255245: Add potential solution, evaluation and recommendation for Use Case#15 S5-255386: Add description and conclusion for WT 2.1 S5-255387: Add scope S5-255491: Add requirements and solution for expressing the relative values for the target S5-255492: Add evaluation and conclusion for Use case #1 Enhancement of radio service delivering and assurance scenarios S5-255494: Add use case for enhancement of Core network delivering and assurance scenarios S5-255495: Update Solution for enhancing intent feasibility check capability S5-255496: Enhance solution for Use-Case#11 and add evaluation, recommendations S5-255497: Add evaluation and conclusion for Use case #12 Documentation for the overview of intent driven management functionalities S5-255500: Enhance solution for Use case#13 Intent Lifecycle Management State Transition S5-255501: Add description and solution to intent LCM states UC#13 S5-255502: Update Use case #9 Intent handling capability configuration, registration and discovery S5-255503: Add solution for intent guarantee UC#8 S5-255504: Add use case for the relation and the interactions between intent handling function and NDT S5-255625: Intent Interpretation Assistance Information Solution S5-255626 : Update Use case #16 Investigation on the applicability and potential impacts to support natural language intents translation S5-255628: Invariant Guidance in Intent Contexts Solution S5-255629: Add evaluation and recommendations for Use case#3 S5-255683: Add use case on Enhancement of radio service for protection level | 0.3.0 |