| 3GPP TR 28.888 V0.3.0 (2026-02) |  |
| --- | --- |
| Technical Report |  |
| 3rd Generation Partnership Project; Technical Specification Group Services and System Aspects; Management and orchestration; Study on Enhanced exposure of management services (Release 20) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet https://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2025, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	4

Introduction	5

1	Scope	6

2	References	6

3	Definitions of terms, symbols and abbreviations	6

3.1	Terms	6

3.2	Symbols	7

3.3	Abbreviations	7

4	Concepts and background	7

4.1	Overview	7

4.2	Authorization of external MnS consumers using CAPIF	7

4.2.1	Introduction	7

4.2.2	Authorization in CAPIF Framework	8

4.3 Access control on notifications	8

5	Use cases and potential solutions	9

5.1	Use case #1: Authorization of the service API invocation request from the external MnS consumer using CAPIF	9

5.1.1	Description	9

5.1.2	Potential requirements	9

5.1.3	Potential solutions	9

5.1.4	Evaluation of potential solutions	9

5.2	Use case #2: Access control on notifications	9

5.2.1	Description	9

5.2.2	Potential requirements	10

5.2.3	Potential solutions	10

5.2.4	Evaluation of potential solutions	10

5.X	Use case #3: Transformation of MnS information for external MnS consumers	10

5.3.1	Description	10

5.3.2	Potential requirements	11

5.3.3	Potential solutions	11

5.3.4	Evaluation of potential solutions	11

6	Conclusions and recommendations	11

6.X	Use case #<X>: <use case title>	11

# Foreword

This Technical Report has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

In the present document, modal verbs have the following meanings:

shall	indicates a mandatory requirement to do something

shall not	indicates an interdiction (prohibition) to do something

The constructions "shall" and "shall not" are confined to the context of normative provisions, and do not appear in Technical Reports.

The constructions "must" and "must not" are not used as substitutes for "shall" and "shall not". Their use is avoided insofar as possible, and they are not used in a normative context except in a direct citation from an external, referenced, non-3GPP document, or so as to maintain continuity of style when extending or modifying the provisions of such a referenced document.

should	indicates a recommendation to do something

should not	indicates a recommendation not to do something

may	indicates permission to do something

need not	indicates permission not to do something

The construction "may not" is ambiguous and is not used in normative elements. The unambiguous constructions "might not" or "shall not" are used instead, depending upon the meaning intended.

can	indicates that something is possible

cannot	indicates that something is impossible

The constructions "can" and "cannot" are not substitutes for "may" and "need not".

will	indicates that something is certain or expected to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

will not	indicates that something is certain or expected not to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

might	indicates a likelihood that something will happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

might not	indicates a likelihood that something will not happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

In addition:

is	(or any other verb in the indicative mood) indicates a statement of fact

is not	(or any other negative verb in the indicative mood) indicates a statement of fact

The constructions "is" and "is not" do not indicate requirements.

# Introduction

# 1	Scope

The present document …

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 28.533: "Management and orchestration; Architecture framework".

[3]	3GPP TS 28.579: "Management services exposure to external consumers through CAPIF".

[4]	3GPP TS 28.319: "Access control for management services".

[5]	3GPP TS 28.537: "Management and Orchestration; Management capabilities".

[6]	3GPP TS 28.622: "Telecommunication management; Generic Network Resource Model (NRM) Integration Reference point (IRP); Information Service (IS)"

[7]	3GPP TS 33.122: "Security aspects of Common API Framework for 3GPP Northbound APIs "

[8]	3GPP TS 29.222: " Common API Framework for 3GPP Northbound APIs".

[9]	3GPP TS 23.501: "System architecture for the 5G System (5GS)".

[10]	3GPP TS 23.502: "Procedures for the 5G System (5GS)".

[11]	3GPP TS 28.541: "Management and orchestration; 5G Network Resource Model (NRM); Stage 2 and stage 3".

[12]	3GPP TS 28.552: "Management and orchestration; 5G performance measurements".

[13]	3GPP TS 28.530: "Management and orchestration; Concepts, use cases and requirements".

…

[x]	<doctype> <#>[ ([up to and including]{yyyy[-mm]|V<a[.b[.c]]>}[onwards])]: "<Title>".

# 3	Definitions of terms, symbols and abbreviations

## 3.1	Terms

For the purposes of the present document, the terms given in TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in TR 21.905 [1].

Management Service (MnS): defined in TS 28.533 [2].

External MnS consumer: defined in TS 28.533 [2].

Internal MnS consumer: defined in TS 28.533 [2].

## 3.2	Symbols

For the purposes of the present document, the following symbols apply:

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1].

# 4	Concepts and background

Editor’s note: This clause provides a description of concepts and background.

## 4.1	Overview

The study builds upon the management services exposure scenarios defined in clause 5.6 of TS 28.533 [2], which serves as the primary reference for exposing management services to external consumers. In addition, the Management Exposure framework described in TS 28.579 [3] introduces an optional approach based on CAPIF.

The enhanced exposure concept includes the ability to abstract sensitive internal identifiers, enforce data sharing permissions, and align with industry-standard exposure frameworks.

Key motivations and considerations include:

- Cross-domain integration: As operators increasingly collaborate with external entities, there is a need to expose selected management services in a controlled and standardized manner.

- Security and privacy: Exposure of collected data must comply with security requirements, for the case where these data conveys sensitive data that the operator does not want to expose as-is outside their admin domain.

- Access control enhancement: Improve mechanisms of existing specifications (e.g., TS 28.319 [4]) to provide notification-level access control, which is essential for dynamic service interactions.

## 4.2	Authorization of external MnS consumers using CAPIF

### 4.2.1	Introduction

As noted in clause 5.6 of TS 28.533 [2], an operator can decide to expose 3GPP management system capabilities to external MnS consumers using existing MnS interfaces (e.g. external MnS consumer Y1), or using CAPIF (e.g. external MnS consumer Y2).

When CAPIF is not used, an external MnS consumer gets access to MnS APIs using 3GPP management system mechanisms defined for discovery (see TS 28.537 [5] and TS 28.622 [6]) and access control (see clause 4.9 of TS 28.533 [2]), in the same way as an internal MnS consumer. The access control capability is provided by authentication and authorization service producers, which are responsible for authenticating and authorizing MnS consumers (see Annex D of TS 28.533 [2]), based on operator-configured information. The operator can configure this information using the MSAC framework defined in TS 28.319 [4].

To expose service APIs using CAPIF, the 3GPP management system needs to be registered as an CAPIF API provider domain. To that end, the 3GPP management system provides required API provider domain functionalities (AMF, APF, AEF) in a framework called Management Service Exposure Domain (MSED), see TS 28.579 [3].

### 4.2.2	Authorization in CAPIF Framework

3GPP TS 33.122 [7] specifies security aspects of the CAPIF Framework.

CAPIF has defined three possible formats for access token’s scope. These formats are specified in the “scope” attribute in AccessTokenReq (see clause 8.5.4.2.6 of TS 29.222 [8]) and AccessTokenRsp (see clause 8.5.4.2.7 of TS 29.222 [8]).

## 4.3 Access control on notifications

Currently, the 3GPP management system supports notification subscription via the NtfSubscriptionControl IOC (see clause 4.3.22 of TS 28.622[6]), enabling MnS consumers to subscribe to receive notifications from MnS producers.

The NtfSubscriptionControl IOC (as defined in clause 4.3.22 of TS 28.622[6]) includes the following attributes:

Table 4.3-1: NtfSubscriptionControl properties

| Attribute Name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| notificationRecipientAddress | M | T | T | F | T |
| notificationTypes | O | T | T | F | T |
| scope | O | T | T | F | T |
| notificationFilter | O | T | T | F | T |
| lastSequenceNo | O | T | F | F | F |
| operationalState | O | T | F | F | T |
| availabilityStatus | O | T | F | F | T |

Further, clause 7.3 of TS 28.319[4] defines the AccessRule class associated to a given MnS consumer identified by their Identity and Role classes. The AccessRule class includes the following attributes (shown in Table 4.3-2) as defined in clause 7.3.3 of TS 28.319[4]):

Table 4.3-2: AccessRule properties

| Attribute Name | S |
| --- | --- |
| ruleName | M |
| dataNodeSelector | M |
| operations | M |
| actions | O |
| componentCData | O |

In clause 7.3.3 of TS 28.319[4], componentCData is defined as an optional attribute which specifies notification types and performance metric names.

# 5	Use cases and potential solutions

## 5.1	Use case #1: Authorization of the service API invocation request from the external MnS consumer using CAPIF

### 5.1.1	Description

Editor's note:	This clause provides a description of use case.

For an external MnS consumer to be able to invoke service APIs at the MSED AEF, first they send the service API authorization request (over the CAPIF-1e interfaced) to the CCF to request for the authorization to access one or more service APIs. If the service API authorization request is successful, the CCF issues this authorization to the API invoker in the form of an access token. This token contains the permissions granted to the API invoker for requested service APIs. These permissions are carried as part of the token’s scope, whose format is one of the three formats specified in the scope attribute of the AccessTokenReq (see clause 8.5.4.2.6 of TS 29.222[8]) and AccessTokenRsp (see clause 8.5.4.2.7 of TS 29.222[8]). For further details on the service API authorization procedure, see clause 6.5.2.3 of TS 33.122[7].

Accordingly, once an external MnS consumer has a valid access token from the CCF, they proceed to perform the service API invocation request to the AEF over the CAPIF-2e interface (see clause 6.5.2.3 of TS 33.122[7] for more details). As a pre-condition for the service API invocation request to be successful, the AEF needs to be able to understand the access token format issued by the CCF to enable the access token validation.

This use case describes how the service API invocation request from the external MnS consumer can be authorized at the MSED AEF.

### 5.1.2	Potential requirements

Editor's note:	This clause provides potential requirements for the corresponding use case.

PREQ-FS_EnExpo-AuthCCF-01: The exposure of management services using CAPIF shall provide the capability enabling the authorization of the service API invocation request(s) at the MSED AEF.

### 5.1.3	Potential solutions

Editor's note:	This clause provides one or more solutions. Further (sub-)clause(s) may be added to capture details.

### 5.1.4	Evaluation of potential solutions

Editor's note:	This clause provides evaluation of potential solutions.

## 5.2	Use case #2: Access control on notifications

### 5.2.1	Description

Editor's note:	This clause provides a description of use case.

When an MnS consumer subscribes to receive notifications using the NtfSubscriptionControl IOC (see clause 4.3), the 3GPP management system needs to be able to determine whether a given MnS consumer is authorized to receive notifications on a set of managed object(s). Additionally, an MnS consumer could subscribe to receive notifications by specifying a notification scope that exceeds its authorized notification scope, the 3GPP management system needs to be able to handle such requests.

### 5.2.2	Potential requirements

Editor's note:	This clause provides potential requirements for the corresponding use case.

PREQ-FS_EnExpo-Ntf-01: The 3GPP Management System should provide the capability to configure, per role (as defined in clause 7.3.2 of TS 28.319[4]), the set of notification types that the MnS consumer is authorized to receive.

PREQ-FS_EnExpo-Ntf-02: The 3GPP Management System should provide the capability to enforce the restrictions configured in requirement PREQ-FS_EnExpo-Ntf-01.

PREQ-FS_EnExpo-Ntf-03: The 3GPP Management System should provide the capability to configure, per role, the information content that the MnS consumer is authorized to receive in a notification.

Editor’s note: If the proposed solution is deemed too complicated, then this requirement could be removed.

Editor’s note: The access control rules for reading data via CRUD and access control rules for receiving data via notifications should be common.

Editor’s note: The relationship between the entity that subscribes to receive notifications and the entity that receives the notifications needs to be clarified.

PREQ-FS_EnExpo-Ntf-04: The 3GPP Management System should provide the capability to enforce the restrictions configured in requirement PREQ-FS_EnExpo-Ntf-03.

### 5.2.3	Potential solutions

Editor's note:	This clause provides one or more solutions. Further (sub-)clause(s) may be added to capture details.

#### 5.2.3.1 Configuration of access control on notifications

In this proposed solution, a new NotificationAccessRule is proposed as another access rule type to be added to the class diagram in clause 7.2 of of TS 28.319[4].  The NotificationAccessRule provides access control on a specific notification type or all notification types. As illustrated in Figure 5.2.3.1-1, each NotificationAccessRule is associated to one or more Role(s). The attributes of the NotificationAccessRule are presented in Table 5.2.3.1-1.

Table 5.2.3.1-1: Attributes of the NotificationAccessRule

| Attribute Name | S |
| --- | --- |
| ruleName | M |
| dataNodeSelector | O |
| notificationTypes | M |
| action | O |

In the NotificationAccessRule, the notificationType attribute represents the notification type message for which the access rule applies, whereas the ruleName, dataNodeSelector and actions attribute definitions are as defined in clause 7.4.1 of TS 28.319[4]. Accordingly, for an MnS consumer to be able to receive a given notification type event from the MnS producer, one of the Role(s) associated to the MnS consumer should be configured with the corresponding NotificationAccessRule and the action of the NotificationAccessRule should be permitted (i.e., by setting the value of the action attribute to “ALLOW”). If provided, the NotificationAccessRule.dataNodeSelector attribute can be used to explicitly set the notification access control rules on one or more MOIs.

Figure 5.2.3.1-1: Classes for role-based access control including the notification access rule (See Figure 7.2-1 of TS 28.319[4])

The following definition for the notificationType attribute for the NotificationAccessRule is provided:

| Attribute Name | Documentation and Allowed Values | Properties |
| --- | --- | --- |
| notificationType | It describes the notification type that the NotificationAccessRule applies to. The absence of a value for this attribute means that the NotificationAccessRule applies for all notification types. | type: SupportedNotifications.notificationTypes. (see clause 4.3.61 of TS 28.622[6]) multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |

#### 5.2.3.2 Enforcement of the access control on notifications

In the following clauses, it is explained how the proposed solution in clause 5.2.3.1 can be used by the MnS producer when it comes to enforcement on notification subscriptions and sending considering different scenarios. The proposed solution in this clause assumes that the MnS consumer is authorized to create the NtfSubscriptionCtrl MOI as per the access control model described in clause 7 of TS 28.319[4].

##### 5.2.3.2.1 Enhancement of the NtfSubscriptionCtrl <<IOC>>

For the MnS producer to be able to know which MnS consumer requested to receive a set of notifications using the NtfSubscriptionCtrl IOC, there is a need to have the identity of the MnS consumer stored as part of the NtfSubscriptionCtrl IOC.

It is proposed to add a new attribute “consumerId” to the NtfSubscriptionCtrl IOC. This attribute is equivalent to the Identity.identityName attribute associated to the MnS consumer in the Identity class defined in clause 7.3.1 of TS 28.319[4]. The properties of the consumerId attribute is provided in Table 5.2.3.2.1-1.

Table 5.2.3.2.1-1: Properties of the consumerId attribute

| Attribute Name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| consumerId | M | T | F | F | T |

##### 5.2.3.2.2 Notification subscription handling

When an MnS consumer subscribes to receive notifications using the NtfSubscriptionControl IOC(see clause 4.3.22 of TS 28.622[6]). To this end,  the MnS producer evaluates the subscription request by first checking NtfSubscriptionCtrl.consumerId attribute. Using this attribute, the MnS producer checks if there are existing NotificationAccessRule(s)where:

- the Role associated to the NotificationAccessRule is the same as one of the Role(s) associated to the MnS consumer (i.e., the Role(s) associated to the NtfSubscriptionCtrl.consumerId).

- the NotificationAccessRule.notificationTypes includes all the notification types specified in the NtfSubscriptionCtrl.notificationTypes attribute.

- If the NotificationAccessRule.DataNodeSelector attribute is present, the values in NtfSubscriptionCtrl.scope and NtfSubscriptionCtrl.notificationFilter attributes in the request are a subset or correspond exactly to the values of the NotificationAccessRule.DataNodeSelector.

- If the NotificationAccessRule.DataNodeSelector attribute is supported but absent, checks if the, DataNodeAccessRule(s) allow read access to the scope defined by NtfSubscriptionCtrl.scope and NtfSubscriptionCtrl.notificationFilter attributes in the request.

- the value of the NotificationAccessRule.action is “ALLOW”

If all the checks (i) to (v) are true, the notification subscription request succeeds. If no such rule is found, (as a default behaviour) the creation/modification of the NtfSubscriptionControl MOI is denied. However, if checks (ii), (iii) and (iv) are false and checks (i) and (v) are true, then two different behaviours of the MnS producer are proposed, i.e., the strict scope and partial scope enforcement models:

- Strict scope enforcement model: here the MnS producer informs the MnS consumer that the notification subscription request wasn’t successful and provides the appropriate reason. Accordingly, in the strict scope enforcement model, the MnS consumer is not allowed to subscribe to receive notification types beyond the configured NotificationAccessRule.notificationTypes, NotificationAccessRule.dataNodeSelector and DataNodeAccessRule. dataNodeSelector authorized for the Role(s) associated to the MnS consumer.

- Partial scope enforcement model: here the MnS producer informs the MnS consumer that the notification subscription request has been successfully created. The MnS producer can indicate to the MnS consumer that the subscription is accepted but the notifications will be delivered only for the authorized partial scope “XYZ” or subset of the notification types of NotificationAccessRule.notificationTypes.

NOTE: It’s upto implementation which enforcement model is applied by the MnS producer.

##### 5.2.3.2.3 Notification sending

In this scenario, when a notification message is generated the MnS producer does similar checks as described in clause 5.2.3.2.2, but instead of checking the NtfSubscriptionCtrl.scope and NtfSubscriptionCtrl.notificationFilter, only the objectInstance in the common notification header is checked against the NotificationAccessRule.dataNodeSelector and DataNodeAccessRule. dataNodeSelector.  If statements (i) to (v) in clause 5.2.3.2.2 are true, then the notification message will be sent to the notificationRecipeintAddress (this attribute was provided by the MnS consumer(identified by the NtfSubscriptionCtrl.consumerId) during the NtfSubcriptionCtrl MOI creation request). However if any of the above statements is false, no notification will be sent out.

Further, in case some notifications that were earlier sucessfully subcribed to by the MnS consumer can nolonger be sent, then the MnS producer will inform the MnS consumer of such notification subcription changes including the reason. One possible reason for the change in the notification events received by the MnS consumer could be due to the modification(s) of the NotificationAccessRule(s) associated to the Role(s)of the MnS consumer after the notification subscription

NOTE:     The proposed solution assumes that the MnS producer’s responsibility is limited to ensuring that the MnS consumer is authorized to subscribe and receive given notification type(s). So, it’s upto the MnS consumer to ensure that the entity that receives the notifications (identified by the notificationRecipientAddress attribute of the NtfSubscriptionControl IOC) is authorized to receive such notification type(s).

### 5.2.4	Evaluation of potential solutions

Editor's note:	This clause provides evaluation of potential solutions.

The proposed solution extends the existing access control model defined in TS 28.319[4] by adding a new access control rule for notifications called NotificationAccessRule. Further, the proposed solution specifies how the defined NotificationAccessRule(s) and the DataNodeAccessRule(s)can be used to provide access control for  notification subscription handling and sending.

## 5.3	Use case #3: Transformation of MnS information for external MnS consumers

### 5.3.1	Description

Editor's note:	This clause provides a description of use case.

When an external MnS consumer which is located outside the PLMN trust domain requests access to network information, 3GPP management system needs to ensure that internal 5G information is not revealed. For instance, the relevant "S-NSSAI" need to be transformed/mapped to an AF-Service-Identifier before exposure, rather than being exposed to an untrusted AF directly. 3GPP 5GC control plane uses the same transformation/mapping approach for internal 5GC information. For example, to report the number of UEs and PDU Sessions per network slice to AF, the S-NSSAI is used in case of a trusted AF and AF-Service-Identifier is used for an untrusted AF, see " Translation of internal-external information" in clause 6.2.5.0 of TS 23.501 [9] and "Map the AF-Service-Identifier into DNN and S-NSSAI combination, determined by local configuration" in clause 4.3.6.1 of TS 23.502 [10] for further detail.

To manage the untrusted AFs, the related NRM information has been specified in TS 28.541 [11], which includes the NRM supporting for AF-Service-Identifier for an untrusted AF, see the attribute unTrustAfInfoList of NefInfo in clause 5.3.164 of TS 28.541 [11] for further detail.

The MnS consumer can get the performance measurements of a network slice. Performance measurements specified in TS 28.552 [12] can be split into sub-counters per S-NSSAI. 3GPP management system can use these sub-counters to distinguish performance measurements for different consumers/trusted AFs, which might be required when performance measurements are exposed as part of Network Slice as a Service (NSaaS) specified in TS 28.530 [13]. So far, no transformation/mapping approach of an S-NSSAI to an AF-Service-Identifier is specified for the case of an external MnS consumer/untrusted AF in performance management specifications. This use case is to study potential solutions to support transformation/mapping of an S-NSSAI to an AF-Service-Identifier for network slicing performance managements.

### 5.3.2	Potential requirements

Editor's note:	This clause provides potential requirements for the corresponding use case.

REQ-EnExpo-Trans-01: The 3GPP management system should support a transformation to replace S‑NSSAI with AF‑Service‑Identifier before exposure to external MnS consumers.

### 5.3.3	Potential solutions

Editor's note:	This clause provides one or more solutions. Further (sub-)clause(s) may be added to capture details.

This clause identifies potential solutions to support the transformation of internal slice identifiers (S-NSSAI) into external identifiers (AF-Service-Identifier) when exposing MnS information to external MnS consumers located outside the PLMN trust domain. The objective is to ensure that internal 5G system information is not revealed while still enabling the exposure of network slice related management information.

Reuse of existing configuration management models to support the transformation of internal slice identifiers (S NSSAI) into external identifiers (AF Service Identifier)

TS 28.541 [11] already defines the necessary NRM support for representing AF-Service-Identifier for untrusted AFs, including the unTrustAfInfoList attribute of NefInfo in clause 5.3.164. This existing NRM structure is sufficient to support the encapsulation and transformation of internal slice identifiers (S-NSSAI) into external identifiers (AF-Service-Identifier). No additional CM information models or attributes are required to support the transformation of S-NSSAI into AF-Service-Identifier.

Need for new encapsulated performance measurement definitions to support the transformation of internal slice identifiers (S NSSAI) into external identifiers (AF Service Identifier)

While configuration management is already supported, the current performance management (PM) specifications do not provide a mechanism to expose slice related performance measurements using AF-Service-Identifier. TS 28.552 [12] defines performance measurements that can be split into sub-counters per S-NSSAI, but no mapping or transformation mechanism exists for the case of an external MnS consumer or untrusted AF.

To address this gap, the following solution is proposed:

For those PMs that can be calculated per supported S-NSSAI subcounter, where in this case, only the S-NSSAI-id is mapped to the AF service identifier (before exposing to external MnS consumer).

### 5.3.4	Evaluation of potential solutions

Editor's note:	This clause provides evaluation of potential solutions.

# 6	Conclusions and recommendations

## 6.X	Use case #<X>: <use case title>

Editor's note:	This clause provides conclusions and recommendations for the corresponding use case.

Annex A (informative):
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2025-10 | SA5 #163 | S5-254449 S5-254694 |  |  |  | Added the TR structure for enhanced exposure of management services. Added concepts and background. | 0.1.0 |
| 2025-11 | SA5#164 | S5-255566 S5-255567 S5-255684 S5-255703 |  |  |  | Add use case on transformation of MnS information for external MnS consumers Add concepts, use case and requirements on access control on notifications Add concepts for authorization of external MnS consumers using CAPIF Add use case and requirements for authorization of the external MnS consumers at the CCF | 0.2.0 |
| 2026-02 | SA5#165 | S5-260734 S5-260735 |  |  |  | Add solution for transformation of MnS information for external MnS consumers Add potential solution and evaluation on access control on notifications | 0.3.0 |