| 3GPP TR 32.801-01 V0.1.0 (2026-02) |  |
| --- | --- |
| Technical Report |  |
| 3rd Generation Partnership Project; Technical Specification Group Services and System Aspects; Management and orchestration; Study on 6G Management and Orchestration (Release 20) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet https://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2026, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword.	5

Introduction	6

1	Scope	7

2	References	7

3	Definitions of terms, symbols and abbreviations	7

3.1	Terms	7

3.2	Symbols	7

3.3	Abbreviations	8

4	Overview	8

5	6G Management Architecture Principles	8

6	6G Management Scenarios	8

6.1	Identified 6G Management Scenarios	9

6.1.1	<Management Scenario Category#1>	9

6.1.1.1	Management Scenario#1: Mobile service delivery and assurance for specific events	9

6.1.1.1.1	Description	9

6.1.2	<Management Scenario Category#2>	10

6.1.2.1	Management scenarios#1: OAM-centric issues for Individual Service Complaint Handling	10

6.1.2.1.1	Description	10

6.1.3	<Management Scenario Category#3>	11

6.1.3.1	Management scenarios#1: Network issues handling	11

6.1.3.1.1	Description	11

6.1.4	Management Scenario for Energy Saving and Energy Efficiency	12

6.1.4.1	Management Scenario #1: RAN energy saving	12

6.1.4.1.1	Description	12

6.1.4.1.2	Potential Requirements	13

6.1.4.2	Management Scenario #2: Carbon emission control to reduce carbon emission	13

6.1.4.2.1	Description	13

6.1.4.2.2	Potential Requirements	14

6.1.5	Management Architecture for 6G	14

6.1.5.1	Management Scenario #1: Cloud Aspects of Management and Orchestration	14

6.1.5.1.1	Description	14

6.1.6	Data Management Framework (DMFW)	14

6.1.6.1	Management Scenario #1: Data Management Framework in the Management System	14

6.1.6.1.1	Description	14

6.1.7	Autonomous Agent	15

6.1.7.1	Management Scenario #1: Autonomous agent within 3GPP management system	15

6.1.7.1.1	Description	15

6.1.7.2	Management Scenario #2: Use case for management exposure to agents external to 3GPP system	15

6.1.7.2.1	Description	15

6.1.8	<Management Scenario Category#8>	16

6.1.8.1	Management scenario#1: Systematic Root Cause Analysis and End-to-End Observability	16

6.1.8.1.1	Description	16

6.1.9	Intent Driven Management	16

6.1.9.1	Management Scenario #1: Efficient intent handling by inputting intent before the events	16

6.1.9.1.1	Description	16

6.2	Management Scenarios for Support of 6G Services	17

6.2.1	System and Operation Aspects	17

6.2.1.1	Management Scenario #1: Management support for the use case on 6G network digital twin	17

6.2.1.1.1	Description	17

6.2.1.2	Management Scenario #2: Cloud native management scenario	17

6.2.1.2.1	Description	17

6.2.2	AI	18

6.2.3	Integrated Sensing and Communication	18

6.2.4	Ubiquitous Connectivity	18

6.2.4.1	Management Scenario #1: Management of TN-NTN Interworking	18

6.2.4.1.1	Description	18

6.2.4.1.2	Potential Requirements	18

6.2.5	Immersive Communication	18

6.2.6	Massive Communication	18

6.2.7	Further Use Cases on Industry and Verticals	18

6.2.8	Other Use Cases	18

7	Management Features	18

7.1	Data Management Framework (DMFW)	19

7.1.1	Key Issue #1: DMFW Capabilities	19

7.2	Data and Knowledge Management	19

7.2.1	Key Issue #1: Knowledge/semantic representation and management	19

7.2.2	Key Issue #2: Data management – Concepts and terminology	20

7.3	Management Agent	20

7.3.1	Key Issue #1 Concept of agent in management domain	20

7.4	Energy Saving and Energy Efficiency	20

7.4.1	Key Issue #1: Energy efficiency management architectural framework	20

7. 5	Cloud Aspects of Management and Orchestration	21

7.5.1	Key Issue #1: Improvements to LCM based on NF Deployments	21

7.5.1.1	Description	21

7.5.1.2	Potential Requirements	21

8	Solutions	22

8.1	Potential Solution #1: <title>	22

8.2	Potential Solution #2: <title>	22

9	Conclusions and Recommendations	22

Annex A: Management Scenarios, Key Issues, and Solutions Mapping	23

A.1	Mapping between Management Scenarios and Key Issues	23

A.2	Mapping between Key Issues and Solutions	23

Anne B: Telecom operators design principles for the 6G Study	24

B.1	Introduction	24

B.2	Telecom Operators’ View for the 6G 3GPP Management System	24

Annex X (informative): Change history	27

Foreword.

This Technical Report has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

In the present document, modal verbs have the following meanings:

shall	indicates a mandatory requirement to do something

shall not	indicates an interdiction (prohibition) to do something

The constructions "shall" and "shall not" are confined to the context of normative provisions, and do not appear in Technical Reports.

The constructions "must" and "must not" are not used as substitutes for "shall" and "shall not". Their use is avoided insofar as possible, and they are not used in a normative context except in a direct citation from an external, referenced, non-3GPP document, or so as to maintain continuity of style when extending or modifying the provisions of such a referenced document.

should	indicates a recommendation to do something

should not	indicates a recommendation not to do something

may	indicates permission to do something

need not	indicates permission not to do something

The construction "may not" is ambiguous and is not used in normative elements. The unambiguous constructions "might not" or "shall not" are used instead, depending upon the meaning intended.

can	indicates that something is possible

cannot	indicates that something is impossible

The constructions "can" and "cannot" are not substitutes for "may" and "need not".

will	indicates that something is certain or expected to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

will not	indicates that something is certain or expected not to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

might	indicates a likelihood that something will happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

might not	indicates a likelihood that something will not happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

In addition:

is	(or any other verb in the indicative mood) indicates a statement of fact

is not	(or any other negative verb in the indicative mood) indicates a statement of fact

The constructions "is" and "is not" do not indicate requirements.

Introduction

1	Scope

The present document …

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	SP-251653 Study on 6G Management and Orchestration.

[3] 	TR 22.870 Study on 6G Use Cases and Service Requirements.

[4]	3GPP TS 28.531: "Technical Specification Group Services and System Aspects; Management and orchestration; Provisioning".

[5]	3GPP TS 28.533: "Technical Specification Group Services and System Aspects; Management and orchestration; Architecture framework".

[6]	SP-251697: New WID on Life Cycle Management (LCM) of NF Deployment

…

[x]	<doctype> <#>[ ([up to and including]{yyyy[-mm]|V<a[.b[.c]]>}[onwards])]: "<Title>".

3	Definitions of terms, symbols and abbreviations

3.1	Terms

For the purposes of the present document, the terms given in TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in TR 21.905 [1].

example: text used to clarify abstract rules by applying them literally.

Knowledge: structured and semantically related data.

3.2	Symbols

For the purposes of the present document, the following symbols apply:

<symbol>	<Explanation>

3.3	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1].

<ABBREVIATION>	<Expansion>

4	Overview

Editor's note: This clause will provide an overview of 6G management capabilities and features.

5	6G Management Architecture Principles

Editor's note: This clause will contain the common 6G management architecture principles identified for the study.

The overarching design goal of 6G management architecture aims to achieve highly autonomous networks with following motivations:

- Facilitate business growth and improve user experience.

- Improve operational efficiency.

- Reduce operator integration efforts in multi-vendor environment.

For the 6G management architecture, it is crucial to thoroughly analyse whether and how the architecture of 6G OAM should be restructured or redesigned to support the management of the 6G network, enable new services and features of 6G, and support the new services and features of network management itself.

The 6G management architecture shall:

- Support management and orchestration through a management framework in 6G, capable of managing both 6G and existing 5G networks.

Editor's note: The Unified Management Interface for Multi-RAT support (UMMR) work for 4G/5G systems should be considered in 6G depends on conclusion in 3GPP TR 28.892.

- Support multi-vendor management and orchestration interoperability.

- Support the utilization of intelligent management capabilities to support a higher level of autonomy.

- Ensure management capabilities have clearly defined scope, responsibilities, and interaction mechanisms.

- Support cloud-native aspects of OAM capabilities.

- Support the service-based management architecture to support modular architecture design.

- Support the data management framework in the management system to support management of all the domains (e.g., RAN, Core).

6	6G Management Scenarios

Editor's note: This clause will contain brief description of management scenarios for 3GPP 6G networks and the related requirements to enable the management scenarios.

Editor's note: Drafting guidelines for the requirements and management scenarios in this clause: The requirements should be formulated as requirements for the 3GPP management system. The management scenarios should contain explanatory text only for concepts that are new and not already defined or known in SA5.

Editor’s note: The location of documentation of the requirements is to be decided.

6.1	Identified 6G Management Scenarios

Editor's note: This clause will contain identified 6G management scenarios and the related requirements.

6.1.1	<Management Scenario Category#1>

6.1.1.1	Management Scenario#1: Mobile service delivery and assurance for specific events

6.1.1.1.1	Description

This management scenario focus on the scenarios for 6G OAM systems supporting mobile service delivery and assurance for specific events. Following uses the Large-scale music festival scenario as an example, in which an opening ceremony and subsequent live performances are supported across multiple geographically distributed venues. During the opening ceremony, real-time interaction between virtual hosts and audiences requires the mobile network to satisfy strict requirements for latency on both uplink and downlink transmissions. During the performance period, multiple fixed and mobile video capture devices are deployed to support live streaming, which requires the mobile network to satisfy strict requirements on both latency and bandwidth for uplink transmission under mobility conditions.  It is important for the 6G OAM systems to provide and coordinate multiple management capabilities (e.g. network pre-evaluation, network resource adjustment, network performance monitoring and network performance optimization) by utilizing automation and intelligent mechanisms to support mobile service delivery and assurance. The mobile services in this scenario are 3GPP services, examples of 3GPP services can be eMBB services and URLLC service and other 6G new services investigated in TR 22.870.

Following illustrates the service flows (including pre-event evaluation and planning phase, event assurance phase and post-event evaluation phase) for the mobile service delivery and assurance for specific events.

Figure 6.1.1.1.1-1: OAM-centric issue handling for Service complaint management

Editor’s Note: For the interaction between customer and operator in the context of mobile service delivering and assurance, the boundary and collaboration between 3GPP and TM Forum need further investigation.

Pre-event evaluation and planning phase:

1. The music festival organizer expresses the mobile service deliver and assurance requirements to the operator, which includes the overall event plan (e.g. event times, venue names) and corresponding service requirements (e.g. service type, service targets) to the operator to offer corresponding mobile service packages.

2. The operator express the requirements to corresponding 6G OAM systems to generate a detailed management plan to support the mobile service delivery and assurance and pre-evaluate of mobile network capacity for specific services(e.g. live streaming service). Iteratively negotiations may happen between operator and 6G OAM systems to finalize the management plan to support mobile service delivery and assurance. Regarding the event covering multiple areas in which mobile network managed by different 6G OAM systems, operator needs to send the request to different 6G OAM systems.

3. The 6G OAM systems predicts the traffic load and network performance, evaluates the mobile network capacity  and performs evaluation for multiple network performance optimization tasks (e.g. load balance optimization among different cells, energy saving optimization) for the specified areas to release more network resources for the requested mobile services. If the existing mobile network cannot provide enough resources, 6G OAM systems may recommend operator to provide the mobile communication equipment (e.g. emergency communication vehicles, mobile wireless access base stations) for specific areas to expand mobile network capacity for the requested mobile services.

4. Operator confirms the detailed management plan to support mobile service delivery and assurance (e.g., recommendation for network optimization activities, network adjustment activities, mobile communication equipment deployment) and execute the user subscription policy based on the mobile network capacity evaluation result. The management plan will be evaluated and tested until it works smoothly before the event.

Event assurance phase

5. The operator express the requirements to the 6G OAM systems to monitor and assure the network and service performance for the specific areas and UE groups covered by the specific event.

6. 6G OAM systems collects the network and service performance metrics from the 6G network, analyzing the network and service performance to identify performance issues (e.g. Poor-performing cell, low UL/RL UE throughput) and predict the performance deterioration risk for corresponding areas and UE groups. Multiple network optimization and adjustment actions need to be taken in a collaborative way to resolve the identified issues and predicted risks to minimize the impact on user experience. 6G OAM systems also needs to provide the proactive fault intervention and recovery to avoid the service outage. 6G OAM systems may interact with operator to finalize the network optimization and fault recovery actions.

7. The 6G OAM systems also provides network and service performance metrics for operator for the specific areas and UE groups to monitor the network and service.

Post-event evaluation phase

8. After completion of the event, operator request the 6G OAM systems to provide the post-event analysis and evaluation report.

9. 6G OAM systems performs post-event evaluation by analyzing corresponding network and service performance for specific areas or UE groups and generate the evaluation report for the operator. The evaluation report also includes the presentation of dynamic network resource adjustments and the fulfillment result, thereby providing evidence of the effectiveness of automated and intelligent service assurance capabilities of 6G OAM systems. The post-evaluation results can be used as input for the 6G OAM systems to learn and refine the pre-event evaluation and event assurance for other events in the future.

10. Based on the post-event evaluation report from the 6G OAM systems, operator generate the fulfilment report (include the service delivery and assurance successful ratio) for the mobile service delivery and assurance for specific events to the music festival organizer.

6.1.2	<Management Scenario Category#2>

6.1.2.1	Management scenarios#1: OAM-centric issues for Individual Service Complaint Handling

6.1.2.1.1	Description

With the advent of 6G, networks are expected to achieve unprecedented levels of intelligence, enabling them to autonomously manage and resolve individual service complaints. An individual service complaint refers to a customer or consumer expressing dissatisfaction with network services, such as unstable gaming latency, poor call quality, or interrupted streaming experiences. Currently, the resolution of such complaints heavily depends on human intervention. For instance, identifying the root cause of a customer’s complaint often requires network engineers to manually collect network metrics, analyse the data, and diagnose the issue. This manual process is time-consuming, resource-intensive, and may result in delayed responses, ultimately impacting customer experience. Therefore, there is a pressing need for mechanisms that allow the network to autonomously detect, analyse, and address individual service complaints in real time, reducing human effort while improving service reliability and customer satisfaction.

Following figure 6.1.2.1.1-1 illustrates the end to end service flows:

Figure 6.1.2.1.1-1: OAM-centric issue handling for Service complaint management

- Network Diagnostics Initiation:

- Multiple customer complaints about mobile game latency prompt an operator to analyse network data, revealing a potential OAM-related issue.

- Coordination diagnostics:

- The operator determines that the observed latency may be caused by OAM-centric issues (e.g. high packet loss rate) and send request to corresponding 6G OAM systems to identify the cause and locate the OAM-centric issues.

- The 6G OAM system coordinates diagnostics across domains, and isolates the OAM-centric issues from different areas/domains (e.g. RAN, CN domains) and identifies the area/domain causing the issue.

- Autonomous Remediation

- After that, 6G OAM system autonomously perform diagnosis and root cause analysis for the OAM-centric issues for the different areas/domains that impact service quality and provide the result.

- Monitoring and evaluation:

- The 6G OAM system continuously monitors and evaluates the network performance for the specific UE groups related to identified OAM-centric issues affecting service quality to prevent the recurrence and provide the results to the operator.

6.1.3	<Management Scenario Category#3>

6.1.3.1	Management scenarios#1: Network issues handling

6.1.3.1.1	Description

Network issues, including network performance and fault management are the critical concern for operators. These issues may affect the normal operation of the network and the quality of service. The operators need to ensure that network issues can be automatically identified, and network services can recover automatically, enabling operators' real-time intelligent diagnosis, predictive analytics, and autonomous recovery, ensuring seamless network operations and superior Quality of Service (QoS).

This use case considers intelligent network diagnosis and self-recovery to restore operators 6G service, leveraging machine learning, data analytics, to realize closed-loop automation across two key scenarios:

1) Intelligent Network performance issues diagnostic analysis, prediction and recovery

- Autonomous diagnosis and mitigation of performance issues upon the specific network issues identified and requested by the Operator; The network issues can be weak coverage, overlapping coverage, high interference, low uplink/downlink traffic rate, high network load, voice quality problems (low access rate, high call drop rate, low handover success rate, high packet loss rate), etc. Predictive analytics identify potential degradations before they impact users, enabling self-recovery to dynamically adjust parameters (e.g., power allocation, or load balancing).

2) Intelligent Fault diagnostic analysis, prediction and recovery

- Autonomous root cause analysis and automated remediation for faults identified and requested by the Operator; The Fault can be radio unit failures, wireless link disruptions, or core network anomalies, ensuring uninterrupted essential services (e.g., data/voice connectivity).

Following Figure 6.1.3.1.1-1 illustrates the end to end service flows:

Figure 6.1.3.1.1-1: Network issue handling management

- The 6G OAM systems detects anomalies (e.g., KPI degradation: high interference, poor coverage), then report the anomalies to Operator.

- Operator reviews the anomalies report and performs issues identification, then requests issue diagnostics and recovery to 6G OAM systems

- The 6G OAM systems perform issue diagnostic analysis and recovery and report the issue diagnostic analysis and recovery result to Operator. For example, Issue ID, Root Cause: Weak Coverage: Building shadowing (RSRP <-115dBm), Overlapping Coverage: 3 cells with PCI mod3 conflict, Actions: Adjusted down tilt (Cells 12/15), Optimized PCI (Cells 8/9/11 → new PCI 42/45/48).

The monitoring of progress or status for the network issue diagnostic and recovery by the Operator to make sure whether the PMs and KPIs of the network are satisfying or not.

6.1.4	Management Scenario for Energy Saving and Energy Efficiency

6.1.4.1	Management Scenario #1: RAN energy saving

6.1.4.1.1	Description

Energy saving and efficiency is a key focus for 6G, as networks are expected to support exponentially higher data rates and device densities. For 6G network evolution, the network energy efficiency optimization still plays an important role in operator network management. Operators are aiming at decreasing power consumption in 6G networks to lower their operational expense with energy saving management solutions.

One typical scenario of energy saving is to switch off capacity boosters when the traffic demand is low, and re-activated them on a need basis. In 6G context, the scenarios will become more complex considering the interworking among multiple RAT.

Figure 6.1.4.1.1-1: Example of 6G RAN energy saving

Figure 6.1.x.1.1-1 shows that a 6G RAN capacity booster cell is overlaid by coverage cell(s) which can be in the same RAT or different RAT. There may be two typical cases of energy saving:

-	 Intra-System energy saving

-	 Inter-System energy saving

6G OAM system collects the traffic load performance measurements from the 6G RAN capacity booster cell and coverage cells, and may request the capacity booster cell to enter the energy saving mode when its traffic is below certain threshold. The capacity booster cell can be re-activated when additional capacity is needed.

For the third case, 6G OAM system collects the traffic load performance measurements from both Base station #A and #B and may configure a policy to indicate the traffic load transfer from Base Station #B to Base Station #A when Base Station #A traffic is below certain threshold to reduce the carbon emission caused by Base Station #B.

6.1.4.1.2	Potential Requirements

6.1.4.2	Management Scenario #2: Carbon emission control to reduce carbon emission

6.1.4.2.1	Description

Besides the energy saving management, the attention on reduction of carbon emission is raised in the communication industry for 6G. The operators can power base stations using energy supplies where energy is produced using renewable or non-renewable energy sources, and with varying carbon emission factors. There is the scenario where 6G base stations powered by solar PV and/or Energy Grid both exist and their coverage area may overlap as shown in Figure 6.1.x.2.1-1. The Base Station #A to which the coverage cell #A belongs uses energy having less carbon footprint which is generated by its attached solar PV, while its neighbour, Base Station #B to which the coverage cell #B, uses the traditional powering method – Energy Grid, which may use fossil energy and emits more carbon dioxide.

Figure 6.1.4.2.1-1: Example of 6G RAN Carbon emission control to reduce carbon emission

Figure 6.1.4.2.1-1 shows that a 6G RAN capacity booster cell is overlaid by coverage cell(s) which is either powered by energy supplies Grid or locally generated etc. The typical cases of energy efficiency optimization is:

-	Carbon emission control to reduce carbon emission

6G OAM system collects the traffic load performance measurements from both Base station #A and #B and may configure a policy to indicate the traffic load transfer from Base Station #B to Base Station #A when Base Station #A traffic is below certain threshold to reduce the carbon emission caused by Base Station #B.

6.1.4.2.2	Potential Requirements

6.1.5	Management Architecture for 6G

6.1.5.1	Management Scenario #1: Cloud Aspects of Management and Orchestration

6.1.5.1.1	Description

The 6G management system needs to evolve to support NF Deployments in 5G (for Network Slices and Sub-Networks) and 6G networks. For 5G, existing specifications have to be enhanced to enable these deployments.

6.1.6	Data Management Framework (DMFW)

6.1.6.1	Management Scenario #1: Data Management Framework in the Management System

6.1.6.1.1	Description

With the evolution of 6G mobile networks, operators are required to manage a highly distributed and complex network. This environment spans core and radio access networks including those comprising cloud implementations, multiple layers of edge computing, and a wide range of network slices. These domains continually generate large volumes of heterogeneous data with differing semantics, granularity, and temporal characteristics.

As operators progress toward higher levels of autonomy in network operation, management data becomes increasingly central to supporting intelligent network management capabilities. AI-assisted network optimization, proactive analysis, and real-time synchronization with network digital twins rely on timely, accurate, and semantically consistent data. However, existing data collection, storage, and exposure mechanisms are fragmented and insufficient to support the needs of AI-driven and autonomous network management. There are some differences today in how data is produced and managed and the aim is to provide consistent management of all data used to fulfil 6G use cases for operators. Today’s data management is too use case-specific and does not account for data characteristics, e.g., latency, volume, etc.

To address these limitations, a Data Management Framework (DMFW) is needed as a key enabler for the intelligent evolution of network management in 6G. The DMFW provides standardized mechanisms across the lifecycle of data for identified 6G use cases and existing networks.

By ensuring consistent, timely, and high-quality management data availability, the DMFW enables the transition from reactive monitoring to proactive optimization and supports the realization of advanced levels of autonomy in 6G network management.

6.1.7	Autonomous Agent

6.1.7.1	Management Scenario #1: Autonomous agent within 3GPP management system

6.1.7.1.1	Description

1. General description

6G networks introduce new capabilities that significantly broaden the scope of network including services such as ISAC, which introduce more diverse managed entities and more complex operating environments. These scenarios greatly increase the heterogeneity and operational complexity of 6G networks and place significantly higher demands on the management system.

It should be explored how to support collaborative invocation of management capabilities to optimize and support the above scenarios through management Agents.

An Autonomous Agents is a continuously operating intelligent entity capable of perception, reasoning & planning, action execution, and iterative learning. It can dynamically adapt to changes, generalize to unfamiliar situations, and continuously refine its behavior based on feedback.

6.1.7.2	Management Scenario #2: Use case for management exposure to agents external to 3GPP system

6.1.7.2.1	Description

The 3GPP SA1 Rel-20 study (TR 22.870) has identified use cases for 6G related to application scenarios, roles, service requirements, and associated management needs of AI Agents including support for AI Agents that are external to the 3GPP management system.

Such Agents may require to access the management services provided by the 3gpp management system.

In the context of existing 3GPP management services exposure, there are different exposure approaches defined in TS 28.533.

6.1.8	<Management Scenario Category#8>

6.1.8.1		Management scenario#1: Systematic Root Cause Analysis and End-to-End Observability

6.1.8.1.1	Description

This management scenario describes how a causality context, established at the point of initiation of a management action and propagated across all participating domains, enables two complementary capabilities: systematic root cause analysis and end-to-end observability. The causality context is carried alongside management operations and attached to the telemetry (logs, metrics, traces, events) produced by each domain.

Telecommunication operators already face significant challenges when correlating the effects of management actions across their existing network domains. When a management operation, such as a configuration change or a scaling action, triggers a chain of downstream activities, operators have no standardised mechanism to trace the causal relationship between the initiating action and its effects. Instead, they rely on timestamp-based heuristics and vendor-specific correlation methods to reconstruct what happened. These approaches are inherently fragile: clock skew introduces ambiguity, parallel operations create false correlations, and boundaries between management domains break the chain of association entirely.

This problem becomes substantially worse as operators adopt cloud-based and disaggregated network deployments. In such environments, a single management action may traverse the 3GPP management domain, one or more network function domains, and external cloud infrastructure platforms, each producing its own telemetry independently. The number of domain boundaries increases, the diversity of telemetry sources grows, and the correlation challenge scales accordingly. Root cause analysis becomes labour-intensive and unreliable, and end-to-end observability across the full management stack remains impractical.

1) Example 1: Systematic Root Cause Analysis of a management action

An OAM system initiates a configuration change targeting a network function. As the request passes through the 3GPP management domain, the network function processes the change and emits a completion event. The management system determines that additional resources are required and issues a scaling request to the cloud infrastructure platform. The cloud platform processes the scaling request, and if it encounters a failure, it emits a failure event.

When the operator investigates the original configuration failure, they need to understand the causal relationship between the management operation and the infrastructure failure.

2) Example 2: End-to-End Observability of a management action

A management operation (e.g. performance metric collection) is initiated for a specific network slice instance. As the operation progresses, a network function involved in the slice emits performance data. Similarly, the underlying cloud infrastructure supporting that network function likewise emits infrastructure metrics.

There is no clear mechanism that enables the 3GPP management system to associate the data that was produced in the network function (related to the network slice) and the cloud infrastructure.

6.1.9	Intent Driven Management

6.1.9.1	Management Scenario #1: Efficient intent handling by inputting intent before the events

6.1.9.1.1	Description

When a sports match is held in a certain area, a large number of people gather around the stadium, and traffic becomes highly concentrated locally. A mobile network operator uses intent-driven operations to optimize radio service delivery in that area—e.g., to maintain user experience while also minimizing base station power consumption. However, severe weather (e.g., heavy rain) causes cancellation of a sports match, the resulting rapid change in traffic distribution may render the original intent infeasible or suboptimal. Under current procedures, the MnS consumer deletes and recreates the intent or perform ad-hoc modifications, which may delay response and risk service degradation.

To address this challenge, MnS consumers should be able to predefine intent expectations associated with specific context conditions (e.g., large sports game cancellation, emergency alerts, traffic surge in adjacent areas) and register them with the MnS producer in advance. When the specified context occurs, the MnS producer can automatically switch to the predefined alternative intent without requiring manual intervention and the MnS producer reports the activation of a predefined alternative intent to MnS producer. This capability enables rapid, automated adaptation to dynamic conditions while preserving the intent-driven principle of focusing on “what” rather than “how.”

In summary, enabling MnS consumers to send intents for event contexts significantly improves responsiveness and automation in intent-driven management, reduces operational overhead, and ensures that service-level objectives can be maintained even under abrupt environmental or traffic changes.

It should be studied if enhancements to the existing intent modelling is necessary to support new 6G scenarios.

6.2	Management Scenarios for Support of 6G Services

Editor's note: This clause will contain management scenarios for support of Use Cases for 6G service captured in SA1 TR 22.870, and the related requirements identified.

Editor's note: The management-scenario categories in clause 6.2.x level follow the SA1 UC grouping as place holders and may be updated by future contributions; some categories have dependency on the work in other WGs—e.g., “6.3 Management support for ISAC”.

6.2.1	System and Operation Aspects

6.2.1.1	Management Scenario #1: Management support for the use case on 6G network digital twin

6.2.1.1.1	Description

This use case describes the management support for the use case on Network Digital Twin (NDT) in the 6G network defined in 3GPP TR 22.870 [3]. Autonomous Networks is a key objective of the 6G management system, aiming for efficient and low-intervention network operation. NDT is critical to supporting the realization of Autonomous Networks.

As a digital virtual representation of physical networks, NDT serves as a critical enabler for the 6G management system to optimize the management system and support the new services. For example, NDT enables comprehensive service awareness and network traffic restoration analysis for the system, laying a solid foundation for in-depth QoE evaluation and high-quality service provision. NDT provides simulation-based validation capabilities, allowing the system to test optimization solutions (e.g., QoS policy adjustment) in a virtual environment before deployment in physical network, thus realizing network autonomy.

6.2.1.2	Management Scenario #2: Cloud native management scenario

6.2.1.2.1	Description

To achieve objective of 6G resiliency use case in clause 5.6.3 of TR 22.870 [3], the 3GPP management system should follow cloud native principles to design and achieve management capabilities, for instance, adapt the decoupling characteristics of cloud-nativeness, and support elastic scaling and independent upgrading of network functions themselves. The use case assumes that some 6G capabilities (e.g., core network functions and related services) are designed as a collection of workloads (e.g., NF Deployment) on top of the cloud native infrastructure.

In various disruption scenarios, such as physical damage, cyber-attacks, and natural disasters, the 3GPP management system supports elastic management of cloud-native workloads, including coordinating with external orchestration and management systems to enable dynamic scheduling of 6G NF workloads and infrastructure resources, activating redundant workloads, triggering service traffic rerouting, etc., thereby improving network resilience and OAM efficiency.

This scenario focuses on cloud-native management capabilities within the scope of SA5, and achieves synergy with the 6G system architecture in SA2 and the use case requirements in SA1.

6.2.2	AI

6.2.3	Integrated Sensing and Communication

6.2.4	Ubiquitous Connectivity

According to 3GPP TR 22.870 [3], Ubiquitous Connectivity in 6G is defined as a full-space, full-time and full-scenario connectivity capability, aiming to provide continuous digital services with high quality and high reliability to users globally. Ubiquitous Connectivity is no longer limited to the extension of terrestrial base station coverage, but integrates communication, sensing and non-terrestrial technologies, enabling service continuity across diverse environments and mobility conditions.

From a management perspective, such connectivity introduces a highly distributed and heterogeneous system composed of terrestrial and non-terrestrial network components. The 3GPP management system is required to support service awareness, service assurance and operational coordination across multiple access technologies, spatial domains and administrative domains, while maintaining scalability and operational efficiency for large-scale 6G deployments.

6.2.4.1	Management Scenario #1: Management of TN-NTN Interworking

6.2.4.1.1	Description

In 6G, Ubiquitous Connectivity is jointly supported by terrestrial networks (TN) and non-terrestrial networks (NTN), including multi-orbit satellites (VLEO, LEO, MEO, GSO), High Altitude Platform Systems (HAPS), and Unmanned Aerial Vehicles (UAV), as described in TR 22.870[3], TR 23.801[y] and TR 38.914[z]. These network components operate at different altitudes, coverage scales and mobility characteristics, and may be deployed and operated by different administrative entities.

Services enabled by Ubiquitous Connectivity may dynamically traverse TN and multiple NTN segments, resulting in frequent changes in access type, topology and performance characteristics. Mobility across terrestrial and non-terrestrial access, as well as dynamic changes in network conditions, directly impacts service performance.

From a management perspective, the 3GPP management system is required to support TN-NTN connectivity in a consistent manner, enabling service continuity and predictable service experience without relying on static configuration or manual, domain-specific operations.

6.2.4.1.2	Potential Requirements

6.2.5	Immersive Communication

6.2.6	Massive Communication

6.2.7	Further Use Cases on Industry and Verticals

6.2.8	Other Use Cases

7	Management Features

Editor's note: This clause will contain the identified management features to enable the 6G management scenarios in clause 6 and the related potential Key Issues. The Mapping table in Annex A will reflect the relation between the management scenarios and Key Issues.

7.1	Data Management Framework (DMFW)

7.1.1	Key Issue #1: DMFW Capabilities

The DMFW should support the following capabilities:

- Data lifecycle management

- Data collection: the capability to collect data which includes:

  - Existing management data : Performance Measurements (PM), Key Performance Indicators (KPI), Fault/Alarm data, Configuration Management (CM) related data, Trace and MDT data, Quality of Experience (QoE) data, system logs, external management data, etc.

  - New 6G management data: the new data for network and service management. The integration of relevant external management data sources may also be considered.

- Data reporting:

- Data processing: includes capabilities such as data normalization, validation, quality assessment, aggregation, correlation, data enrichment.

- Data discovery

- Data registration

- Data delivery

- Data storage

- Data repository

- Job management:   the capability to support unified transport protocols (i.e., data request, data subscription and notifications) and data models to ensure consistent management control and data reporting, meeting data delivery QoS (e.g., latency, throughput), avoiding duplicate collection.

- Access control and data exposure: the capability to enable secure data access, data lineage, and exposure. Additionally, the capability to enable the data consumers to consume the data from multiple instances of DMFW and share data across them.  To supply authorized consumers with validated  data in the required timeline and format.

Editor’s note: Coordination with SA3 may be needed for data access control and exposure

- Integration with autonomous management services: the capability to support autonomous management services as required in their roles as data producers and/or consumers.

Editor’s note: the DMFW capabilities and the descriptions in this subclause are subject to further study.

7.2	Data and Knowledge Management

7.2.1		Key Issue #1: Knowledge/semantic representation and management

6G management system should understand meaning and relationships of data, in addition to processing syntactic data by making semantics more explicit. Knowledge representations should be used for 6G network management.

Knowledge requires data, the modelling of Knowledge and data should be related.

6G management system should support capabilities to use and manage knowledge. This includes:

- Formalize of Knowledge into structured, and machine-interpretable model.

- Exchange of Knowledge among management entities

- Create new Knowledge using existing Knowledge

The Knowledge representation can be done using graphs.

Editor Note: The standardization aspects of modelling the relationship between Knowledge and data is FFS.

7.2.2	Key Issue #2: Data management – Concepts and terminology

This key issue is about a common concept and terminology for data modelling and data management. A common concept and terminology ensures consistent and clear specifications, thus helping to increase the interoperability of solutions from different vendors. It also makes the process of writing standards more efficient because it allows for efficient communication between delegates and reduces the risk of misunderstandings.

The following concepts are proposed:

data point: an individual item in a data set

data set: a collection of data points, typically sharing common properties

Editor's note: These concepts are just a starting point. More will be added.

Editor's note: It is ffs if some of these terms should not be crisper, such as data segment or bucket instead of data set.

7.3	Management Agent

There are three types of agent:

1. Agent external to 3GPP system, e.g., 3rd party application Agent

2. Management Agent within 3GPP management system

3. Agent in 3GPP network

Clause 7.3.2 focuses on the concept of management agent within 3GPP management system.

Editor’s Note: Use of “Management Agent” in bullet 2 to be revisited.

Editor’s Note: Use of “Agent” in bullet 3 to be aligned with SA2.

7.3.1	Key Issue #1 Concept of agent in management domain

Management Agent: an entity that operates autonomously to achieve a specific goal for service and network management and orchestration. Management Agent may be intent driven, task driven, adaptive, capable of continuously learning, reasoning, decision-making and able to adapt its behaviours based on the changing conditions in the environment. The autonomy capabilities of the Management Agent may be governed by itself based on assigned intents/tasks or requiring minimal to no human intervention.

Management agents can realize, dynamically integrate or collaboratively invoke the management capabilities, e.g., IHF, MDA, NDT, CCL, etc.

Editor’s Note: Interaction between agents and whether any existing functions will become agents is FFS.

7.4	Energy Saving and Energy Efficiency

7.4.1	Key Issue #1: Energy efficiency management architectural framework

The 6G management system should support the following Energy Saving (ES) and Energy Efficiency (EE) management scenarios. This includes the MnS capabilities that need to be studied to be supported in 6G to enable the support of the same set of capabilities as in 5G, while allowing extensions for 6G.

1)	A Centralized energy saving scenario, to save energy in the network, that includes:

-	an MnS capability for Energy saving management (in the management system) that can make Network Elements and/or Network Functions (e.g., Cell, Core NF etc) to enter or exit energy saving state directly (e.g., by modifying the energy saving state of the Network Element or Network Function) based on configured control parameters (e.g., traffic load thresholds). Any MnF playing the role of the MnS Producer supporting this MnS capability for Energy saving management and would also play the role of MnS Consumer of a Provisioning MnS Producer and a Performance Assurance MnS Producer of the Managed Elements and Managed Functions, e.g., for RAN and Core.

2)	A Distributed energy saving scenario, to save energy in the network, that includes:

-	a Distributed energy saving functionality within the Network Element (e.g., within the RAN node) that can perform autonomous switch-off and transition the Network element/Network Function (e.g., cells) to enter or exit energy saving state. The definition of this functionality is not in the scope SA5 and is defined by other WGs (e.g., 3GPP RAN WGs for RAN or 3GPP SA2 WG for Core).

-	an MnS capability for Energy saving management (in the management system) that can configure the Network Element or Network Function (e.g., NG-RAN Node) with load thresholds, ES policy, etc, to be utilized by Distributed energy saving functionality within the Network Element to achieve energy saving. Any MnF playing the role of the MnS Producer supporting this MnS capability for Energy saving management and would also play the role of MnS Consumer of a Provisioning MnS Producer of the Managed Elements and Managed Functions, e.g., for RAN and Core.

3)	An Energy Saving and Energy Efficiency optimization scenario, to optimize network energy saving and energy efficiency, that includes:

-	an MnS capability for Energy Saving and Energy Efficiency optimization (in the management system) responsible optimizing the energy efficiency and improving energy savings across multiple Network Elements and Network Functions (e.g., RAN nodes, cells, Core Network Functions etc), and which can be used to support optimizing the configuration used in the Centralized energy saving scenario and Distributed energy saving scenario (e.g., by collecting and analysing the performance of the Network Elements and/or Network Functions such as traffic loads, identifying a group of cells with coverage overlaps for energy saving, and identifying the sequence in which these Network Elements or Network Function should enter or exit energy saving state etc). Any MnF playing the role of the MnS Producer offering this MnS capability for Energy saving and energy efficiency optimization and would also play the role of MnS Consumer of an MnS Producer supporting MnS capability for Energy saving management, a Provisioning MnS Producer and a Performance Assurance MnS Producer of the Managed Elements and Managed Functions, e.g., for RAN and Core.

7. 5	Cloud Aspects of Management and Orchestration

7.5.1	Key Issue #1: Improvements to LCM based on NF Deployments

7.5.1.1	Description

SA5 has conducted a study on LCM of NF Deployments in Rel-19. The scope of the study did not include the impact of NF Deployments on higher layer use cases like Sub-Network, Network Slice Subnets.

As part of the normative work in Rel-20, the 5GA WID ( SP-251697 [6]) for NF Deployment LCM has created tasks to specify the new deployment management reference point in TS 28.533 [5] and the corresponding NF procedures in TS 28.531 [4]. The lifecycle impacts of 3GPP sub-networks and Network Slice Subnets for NF deployments, not addressed in 5G/5GA, need to be studied as part of 6G.

7.5.1.2	Potential Requirements

MREQ-CMO-1: 3GPP management system should support LCM of Network Slice Subnets for NF Deployments.

MREQ-CMO-2: 3GPP management system should support LCM of Sub-Network for NF Deployments.

Editor’s note: It is FFS if more requirements need to be specified. This is dependent on the progress of the 5GA Rel-20 normative work for NF Deployment LCM.

8	Solutions

Editor's note: This clause will contain potential solutions related to the Key Issues in clause 7. The Mapping table in the Annex A will reflect the relation between potential Key Issues and solutions.

8.1	Potential Solution #1: <title>

8.2	Potential Solution #2: <title>

9	Conclusions and Recommendations

Editor's note: This clause will contain conclusions and recommendations for the Key Issues identified in clause 7.

Annex A:
Management Scenarios, Key Issues, and Solutions Mapping

A.1	Mapping between Management Scenarios and Key Issues

Table A.2-1: Mapping of Key Issues to Management Scenarios

| Key Issues | Management Scenarios |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 |
| #1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| #2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| #3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

A.2	Mapping between Key Issues and Solutions

Table A.3-1: Mapping of Solutions to Key Issues

| Solutions | Key Issues |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | #P | #Q |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| #X |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| #Y |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Anne B: Telecom operators design principles for the 6G Study

B.1	Introduction

This Annex describes guiding principles and expectations related to several study topics for the evolution of the 3GPP management system, tailored to the needs of telecom operators, to be considered during the 6G study phase in Release 20.

The goal is not to formulate specific potential requirements and functionalities, as these are proposed to be studied by the 6G SID [2]. The goal is to describe high-level expectations and guidelines from the operators’ perspective for each work task identified in the 6G study.

In practise all the operators are using some common tools, methodologies and principles however their OSS have different capabilities by means of supported functionalities. The suggested guidelines can be used to form a baseline that can drive the development of the 6G 3GPP management system that is also able to consider aspects for 6G FCAPS; related to AI quality management; and functionalities for automatic operator workflows including AI-based troubleshooting capabilities.

The 6G management system needs to promote improving upon the issues of 5G deployments and provide the management system solutions that will enable network operators to make 6G a success technically and commercially.

B.2	Telecom Operators’ View for the 6G 3GPP Management System

WT#1: Investigate the overall management architecture for 6G as collection of capabilities and high-level functionalities focusing strictly on architectural enablers. This WT provides the framework into which applicable features addressed under WT#2 will map.

|  | Operators view & guidelines for fulfilling the requirements |
| --- | --- |
| 1.1. Study high-level architectural requirements, principles and scope for 6G network management, including managed network scope (manage both 6G and 5G network or 6G network only), management layers, management functionalities and deployments. | Guidelines - Support management and orchestration through a management framework in 6G, capable of managing both 6G and existing 5G networks. - Consider practical deployment and operational aspects of SBMA early in the architectural design, including potential implementation challenges of pure SBA related to troubleshooting, observability and AI operational considerations. Simplicity and clarity should be guiding principles in the management architecture design, with conscious effort to avoid introducing unnecessary architectural or operational complexity. - The management architecture needs to support true interoperability, multi-technology and multi-vendor implementations. - Interoperability is not only about interactions between MnSs point of view but also between 3GPP management system and NFs and between external management systems. - New capabilities need to have clearly defined scope, responsibilities, and interaction mechanisms. - Avoid introducing optional fields and vendor-specific extensions extensively in the design of the MnS IoCs. Even with standardized MnS IoCs, many optional fields cause integration problems. Editor's note: A Unified Management Interface for Multi-RAT support work for 4G/5G systems should be considered in 6G is to be confirmed. |
| 1.2. Study how the high-level architectural requirements, principles and scope captured in WT#1.1 impact the existing SBMA framework including identification of new or existing management services, interfaces and management functions, and their applicability to specific management layers. |  |
| 1.3. Study potential architectural alignment and interaction points between the 6G network management architecture (developed in WT#1.1) and the network architecture defined in other groups (e.g., SA2 and RAN3), including the identification of relevant reference points, services, and management functions needed to enable interoperability. | Guidelines - Efforts should focus on defining clear responsibilities of 6G management plane, minimizing gaps between functional design developed by other groups and their management realization by SA5, while avoiding overlaps and inconsistent assumptions between control and management planes. - Using consistent definitions for concepts across groups can reduce misalignment and improve interoperability of specifications. |
| 1.4. Study whether and how to support MnF and service orchestration, lifecycle management and FCAPS. | Guidelines - MnFs can serve as a mechanism to help operators better integrate the 3GPP management system with external OSSs, while also enabling clearer and more structured interactions with control plane entities. - Management functions should be generic and, avoiding excessive specialization that increases architectural and operational complexity. - A modular architectural approach with clearly defined functional boundaries is encouraged, with clear functional limits to prevent redundant management functions and overlapping responsibilities. - Management functions in 6G need to provide clear visibility into the 6G system that is managed to support efficient operations and fault isolation. |
| 1.5. Study whether and how the agents are used within a part of 6G management architecture, e.g., agent management and orchestration, multi-agent collaboration and interactions, enabling agent to utilize/access 6G management services. | Guidelines - Use the term “Agent” consciously, in a precise and well-scoped manner to represent specific managed entities or MnF after analysis rather as a general term that represents any possible managed object or automation component. - It is encouraged that autonomous agents design considers aspects related to explainability, accuracy, sustainability, security and auditability, reliability. - Study of agents and their specification is recommended to prioritize key network operations based on operator requirements. - Use of standardized interfaces and well-defined communication patterns are recommended for agent interactions. - Agents are expected to be designed with the capability to operate across both 6G and existing 5G networks, enabling consistent management behaviour. - The use of agents in real networks is expected to take privacy, security and trust considerations into account as an integral part of the design, particularly when agents operate autonomously or across administrative and technology boundaries. - The standards should enable decisions taken by agents to be traceable and understandable to support effective troubleshooting. |
| 1.6. Study the data management framework within SA5 scope. Note1: Coordinate with SA2 and other working groups. Note2: SA5 scope of data is to be part of the study based on identified 6G use cases. | Guidelines - Cooperate with other SDOs and organizations in the data solutions domain to avoid duplication of work and promote harmonization. - It is important to study the gaps in existing data types to identify operator requirements on observability of MnS producers that provides automation, e.g. whether they behave as expected, execute authorized actions etc. |
| 1.7. Study how to handle information and knowledge considering relation with data. | Guidelines - Clearly identify and describe the relationship between the data management framework and the framework used for semantic and knowledge management, aiming for minimal architectural complexity. |
| 1.8. Study the architectural enablers (e.g., management services, operations and APIs) required to support network slice LCM in 6G. This includes considering enhancements to existing network slice management solutions to address 6G-specific slice aspects, in cooperation with SA2 as necessary. | Guidelines - It is encouraged to investigate how network slicing management procedures can become more autonomous to support 6G scenarios and address the increased complexity in 5G. |

Annex X (informative):
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2026-01 | SA5#165 |  |  |  |  | Initial version | 0.0.0 |
| 2026-02 | SA5#165 | S5-260679 S5-260683 S5-260685 S5-260686 S5-260687 S5-260711 S5-260727 S5-260728 S5-260766 S5-260767 S5-260768 S5-260769 S5-260772 S5-260814 S5-260836 S5-260838 S5-260841 S5-260845 S5-260846 S5-260847 | pCR pCR pCR pCR pCR pCR pCR pCR pCR pCR pCR pCR pCR pCR pCR pCR pCR pCR pCR pCR |  |  | Add the 6G management architecture design principles Add use case on individual mobile service delivery and assurance for specific events Add use case on RAN energy saving and energy efficiency optimization Add use case for individual service complaint handling management Add use case on network issue handling management Add key issue for data management – Concepts and terminology Add key issue for management agent Add principle, use case and key issue for data management framework Add Clause Structure Add use case for management support for the use case on 6G network digital twin Add new use case for cloud native management scenario for 6G resiliency Add use case, key issue for cloud aspects of management and orchestration Add use case on Ubiquitous Connectivity Add Telecom operators design principles for the 6G Study Add use case for autonomous agent within 3GPP management system Add use case for systematic root cause analysis and End-to-End observability Add key issue for energy efficiency management architectural framework Add term and add key issue for knowledge/semantic representation and management Add use case for management exposure to agents external to 3GPP system Add use case for efficient intent handling by inputting intent before the events | 0.1.0 |