3GPP TSG SA WG5 Meeting 137-e		S5-213167

- electronic meeting, online, 10 - 19 May 2021

- Source:	Huawei, China Telecom? China Mobile?

- Title:	Add concrete RadioNetworkExpectation

- Document for:	Approval

Agenda Item:	6.4.10

# 1	Decision/action requested

The group is asked to discuss and approval.

# 2	References

[1]	3GPP draft TS 28.312: “Management and orchestration; Intent driven management services for mobile networks v0.5.0”.

# 3	Rationale

This contribution proposes to add concrete IntentExpectation for radio network which can be used for both network deployment scenario and radio network optimization scenario.

# 4	Detailed proposal

It proposes to make the following changes to TS 28.312[1].

1st Change

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 28.531: "Management and orchestration; Provisioning".

[3]	3GPP TS 28.532: " Management and orchestration; Generic management services"

[X]	3GPP TS 28.541: "Management and orchestration; 5G Network Resource Model (NRM); Stage 2 and stage 3".

2nd Change

##### 6.2.1.2.X	RadioNetworkExpectation <<IOC>>

###### 6.2.1.2.X.1	Definition

This IOC represents the properties of an IntentExpectation for radio network.  The RadioNetworkExpectation represent the MnS consumer’s expectation on a radio network to be deployed or optimized.

A RadioNetworkExpectation can have one or multiple radio network targets, including rANCoverageTarget, rANCapacityTarget, rANUEThptTarget.

Editor’s Note: whther other radio network target is needed is FFS.

A RadioNetworkExpectation my have radio network constraint, including radioConstraint, frequencyConstraint and transportConstraint.

The attribute “expectedAreas” is used to represent a list of coverage areas to deploy or optimize the radio network. Coverage area is used as intent driven object for the intent expectation expressed by a MnS consumer. The attribute “expectedAreas” can be implemented by multiple choices, e.g. CoverageAreaPolygon, TrackingAreaCode.

###### 6.2.1.2.X.2	Attribute

The RadioNetworkExpectation includes attributes inherited from Top_ <<IOC>> and the following attributes:

| Attribute Name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| expectedAreas | M | T | T | F | T |
| rANCoverageTargets | M | T | T | F | T |
| rANCapacityTargets | M | T | T | F | T |
| rANUEThptTargets | M | T | T | F | T |
| rANConstraint | M | T | T | F | T |

3rd Change

##### 6.2.1.3.Y	CoverageArea <<choice >>

###### 6.2.1.3.Y.1	Definition

The <<choice>> defines a coverage area for the expected radio network.  Coverage area is used as intent driven object for the intent expectation expressed by a MnS consumer. The Coverage Areas can be implemented by multiple choices, e.g. CoverageAreaPolygon, TrackingAreaCode.

###### 6.2.1.3.Y.2	Attribute

| Attribute Name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| CHOICE_1 coverageAreaPolygon | CM | T | T | F | T |
| CHOICE_2 tAC | CM | T | T | F | T |

###### 6.2.1.3.Y.3	Attribute constraints

| Name | Definition |
| --- | --- |
| CHOICE_1 coverageAreaPolygon | This attribute shall be supported, when the coverage area in the intent expectation is in the form of polygon. |
| CHOICE_2 tAC | This attribute shall be supported, when the coverage area in the intent expectation is in the form of tracking area code. |

##### 6.2.1.3.A	RANCoverageTarget <<dataType>>

###### 6.2.1.3.A.1	Definition

This <<dataType>> represents the properties of coverage quality for the radio network.

###### 6.2.1.3.A.2	Attribute

| Attribute Name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| targetWeakRSRPRatioInfos | M | T | T | F | T |
| targetLowSINRRatioInfos | M | T | T | F | T |

###### 6.2.1.3.A.3	Attribute constraints

None

##### 6.2.1.3.B	RANCapacityTarget <<dataType>>

###### 6.2.1.3.B.1	Definition

This <<dataType>> represents the properties of network capacity for the radio network.

###### 6.2.1.3.B.2	Attribute

| Attribute Name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| targetMaximumUENumber | M | T | T | F | T |
| targetActivityFactor | M | T | T | F | T |

###### 6.2.1.3.B.3	Attribute constraints

None

##### 6.2.1.3.C	RANUEThptTarget <<dataType>>

###### 6.2.1.3.C.1	Definition

This <<dataType>> represents the properties of the RAN UE throughput for the radio network.

###### 6.2.1.3.C.2	Attribute

| Attribute Name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| targetAveULRANUEThpt | M | T | T | F | T |
| targetAveDLRANUEthpt | M | T | T | F | T |
| targetLowULRANUEThptRatioInfos | M | T | T | F | T |
| targetLowDLRANUEThptRatioInfos | M | T | T | F | T |

###### 6.2.1.3.C.3	Attribute constraints

None

##### 6.2.1.3.D	RANConstraint <<dataType>>

###### 6.2.1.3.D.1	Definition

This <<dataType>> represents the properties of a set of constraint for the radio network.

###### 6.2.1.3.D.2	Attribute

| Attribute Name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| radioConstraints | M | T | T | F | T |
| frequencyConstraints | M | T | T | F | T |
| transportConstraints | M | T | T | F | T |

###### 6.2.1.3.D.3	Attribute constraints

None

##### 6.2.1.3.E	RadioConstraint <<dataType>>

###### 6.2.1.3.E.1	Definition

This <<dataType>> represents the properties of a set of radio constraints for the for the radio network.

###### 6.2.1.3.E.2	Attribute

| Attribute Name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| cellIdRanges | M | T | T | F | T |
| gNBIdRanges | M | T | T | F | T |
| tACRanges | M | T | T | F | T |
| pLMNList | M | T | T | F | T |
| nRPCIRanges | M | T | T | F | T |

###### 6.2.1.3.E.3	Attribute constraints

None

##### 6.2.1.3.F	FrequencyConstraint <<dataType>>

###### 6.2.1.3.F.1	Definition

This <<dataType>> represents the properties of a set of frequency constraint for radio network.

###### 6.2.1.3.F.2	Attribute

| Attribute Name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| arfcnDL | M | T | T | F | T |
| arfcnUL | M | T | T | F | T |
| bSChannelBwDL | M | T | T | F | T |
| bSChannelBwUL | M | T | T | F | T |

###### 6.2.1.3.F.3	Attribute constraints

None

##### 6.2.1.3.G	TransportConstraint <<dataType>>

###### 6.2.1.3.G.1	Definition

This <<dataType>> represents the properties of a set of transport constraints to be used in the radio network.

###### 6.2.1.3.G.2	Attribute

| Attribute Name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| ngcLocalIpAddressList | M | T | T | F | T |
| nguRemoteIpAddressList | M | T | T | F | T |
| logicInterfaceId | M | T | T | F | T |
| nextHopInfo | M | T | T | F | T |

###### 6.2.1.3.G.3	Attribute constraints

None

4th Change

##### 6.2.2.2.4	Attribute definition

#### 6.2.1.4	Attribute definition

| Attribute Name | Documentation and Allowed Values | Properties |
| --- | --- | --- |
| userLabel | A user-friendly (and user assignable) name of the intent. allowedValues: Not Applicable | type: String multiplicity: 1 isOrdered: F isUnique: F defaultValue: None isNullable: False |
| intentExpectation | It indicates the expectations including requirements, goals and constraints given to a 3GPP system. The intentExpectation includes the intentAreas which describes a list of coverage areas for the intent to be applied. Editor’s Note: the detailed intentExpectation is for further discussion | type: FFS multiplicity: * isOrdered: F isUnique: F defaultValue: None isNullable: False |
| intentFulfilStatus | It describes the status of the intent fulfilment result, which is configured by MnS producer and can be read by MnS consumer. allowedValues: "FULFILLED", “NOT_FULFILLED Editor’s Note: whether other allowed values should be supported is FFS, and the name for the attribute intentFulfilStatus is FFS. | type: ENUM multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| expectedAreas | It describes a list of coverage area for the expected radio network | type: CoverageArea multiplicity: 1..* isOrdered: N/A isUnique: N/A defaultValue: False isNullable: True |
| coverageAreaPolygon | It describes the coverage area in the form of polygon. | type: string multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: False isNullable: True |
| rANCapacityTarget | It describes the properties of network capacity for the expected radio network. | type: RANCapacityTarget multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: False isNullable: True |
| rANCoverageTarget | It describes the properties of coverage quality for the expected radio network | type: RANCoverageTarget multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: False isNullable: True |
| rANUEThptTarget | It describes the properties of RAN UE throughput for the expected radio network | type: RANUEThpt multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: False isNullable: True |
| rANConstraint | It describes a set of constraints for the expected radio network. | type: RadioConstraint multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: False isNullable: True |
| radioConstraints | It describes a set of radio configuration constraints for the expected radio network. | type: RadioConstraint multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: False isNullable: True |
| frequencyConstraints | It describes a set of frequency constraints for the expected radio network. | type: FrequencyInfo multiplicity: 1..* isOrdered: N/A isUnique: N/A defaultValue: False isNullable: True |
| transportConstraints | It describes a set of transport constraints for the expected radio network. | type: TransportInfo multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: False isNullable: True |
| cellIdRanges | It describes a range of cell identity which can be assigned to the radio network. CellIdRange includes the following two items: - beginCellId - endCellId AllowedValues: Not Applicable The detailed definition of cell identity see TS 28.541[X]. | type: CellIdRange multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| gNBIdRanges | It describes a region of gNB Id which can be assigned to the radio network. GNBIdRange includes the following two items: - beginGNBId - endGNBId The detailed definition of gNB Id see TS 28.541[X]. | type: GNBIdRange multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| tACRanges | It describes a region of nRTAC which can be assigned to the radio network. TACRange includes the following two items: - beginTAC - endTAC The detailed definition of nRTAC see TS 28.541[X]. | type: TACRange multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| pLMNList | It describes a list of PLMN Id which can be assigned to the radio network. The detailed definition of PLMN Id see TS 28.541[X]. | type: PLMNId multiplicity: * isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| nRPCIRanges | It describes a range of nRPCI which can be assigned to the radio network. NRPCIRange includes the following two items: - beginNRPCI - endNRPCI The detailed definition of nRPCI Id see TS 28.541[X]. | type: NRPCIRange multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| arfcnDL | It describes the arfcnDL for the radio network to be deployed. The detailed definition of arfcnDL see TS 28.541[X]. | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| arfcnUL | It describes the arfcnUL for the radio network to be deployed. The detailed definition of arfcnUL see TS 28.541[X]. | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| bSChannelBwDL | It describes the bSChannelBwDL for the radio network to be deployed. The detailed definition of bSChannelBwDL see TS 28.541[X]. | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| bSChannelBwUL | It describes the bSChannelBwUL of the radio network to be deployed. The detailed definition for bSChannelBwUL see TS 28.541[X]. | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| ngcLocalIpAddressList | It describes a list of ngcLocalIpAddress of the NG control plane interface (NG-C) for the radio network to be deployed. | type: String multiplicity: 1..* isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| nguRemoteIpAddressList | It describes a list of nguLocalIpAddress of the NG user plane interface (NG-U) for the radio network to be deployed. | type: String multiplicity: 1..* isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| logicInterfaceId | It describes a logical transport interface for the radio network to be deployed The detailed definition of logicInterfaceId see TS 28.541. | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| nextHopInfoList | It describes the ingress transport node for the radio network to be deployed The detailed definition of nextHopInfolist see TS 28.541[X]. | type: String multiplicity: * isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| targetMaximumUENumber | It describes the maximum number of should be supported by the radio network to be deployed. | type: Integer multiplicity: * isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| targetActivityFactor | It describes the percentage value of the amount of simultaneous active UEs to the total number of UEs where active means the UEs are exchanging data with the radio network. | type: Real multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: False isNullable: True |
| targetAveULRANUEThpt | It describes the average UL RAN UE throughput for the radio network to be deployed. | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: False isNullable: True |
| targetAveDLRANUEThpt | It describes the average UL RAN UE throughput for the radio network to be deployed. | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: False isNullable: True |
| targeLowULRANUEThptRatioInfos | It describes the list of target low uplink RAN UE throughput ratio for the network to be optimized in the specified area as optimization target. Each TargeLowULRANUEThptRatioInfo specifics the low UL RAN UE throughput for a specified LowULRANUEThptThreshold, which includes following two items: - LowULRANUEThptThreshold - LowULRANUEThptRatio AllowedValues: Not Applicable | type: TargeLowULRANUEThptRatioInfo multiplicity: 1. * isOrdered: N/A isUnique: N/A defaultValue: False isNullable: True |
| targeLowDLRANUEThptRatioInfos | It describes the list of target low downlink RAN UE throughput ratio for the network to be optimized as optimization target. Each TargeLowDLRANUEThptRatioInfo specifics the low DL RAN UE throughput for a specified LowDLRANUEThptThreshold, which includes following two items: - LowDLRANUEThptThreshold - LowDLRANUEThptRatio AllowedValues: Not Applicable | type: TargeLowDLRANUEThptRatioInfo multiplicity: 1. * isOrdered: N/A isUnique: N/A defaultValue: False isNullable: True |
| targetWeakRSRPRatioInfos | It describes the list of target Weak RSRP Ratio for the network in the specified areas to be optimized as optimization target. Each TargetWeakRSRPRatioInfo includes following two items: -WeakRSRPThreshold -WeakRSRPRatio AllowedValues: Not Applicable | type: TargetWeakRSRPRatioInfo multiplicity: 1. * isOrdered: N/A isUnique: N/A defaultValue: False isNullable: True |
| targetLowSINRRatioInfos | It describes the list of target low SINR Ratio for the network in the specified areas to be optimized as optimization target. Each TargetLowSINRRatioInfo includes following two items: - LowSINRThreshold - LowSINRRatio AllowedValues: Not Applicable | type: TargetLowSINRRatioInfo multiplicity: 1. * isOrdered: N/A isUnique: N/A defaultValue: False isNullable: True |

End of changes