SA WG2 Meeting #165 	S2-2409627

# 14 - 18 October 2024, Hyderabad, India

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.288 | CR | 1190 | rev | - | Current version: | 18.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on Nnwdaf, Ndccf, Nmfaf related services |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNA_Ph3 |  |  |  |  |  | Date: |  |  | 2024-09-01 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | This CR aim to resolve misalignment issues of Nnwdaf, Ndccf, Nmfaf related services mentioned in LS in from CT3(C3-244437/S2-2409568). |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding OAM, CEF and LMF as consumers of Nmfaf_3caDataManagement service operation. Adding Nnwdaf_AnalyticsSubscription_Subscribe in step 6b of clause 6.2B.3 and related unsubscribe service operation(Historical Data and Analytics Storage via Notifications procedure). Editorial modifications. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Misalignment in the specification. Problems raised from CT3 are not solve. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2B.3, 9.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * Start of Change * * *

6.2B.3	Historical Data and Analytics Storage via Notifications

The procedure depicted in figure 6.2B.3-1 is used by consumers (NWDAF, DCCF) to store received notifications in the ADRF. The consumer requests the ADRF to initiate a subscription for data and/or analytics. Data and/or analytics provided in notifications as a result of the subsequent subscription by the ADRF are stored in the ADRF.

The consumer may include in the subscription request the DataSetTag attribute defined in Table 6.2B-1.

NWDAF

DCC

F

ADRF

2a

.

N

adrf

_

DataManagement_Storage

Subscription

Request

MFAF

Data or Analytics

Consumer

1a

.

N

dccf

_

DataManagement

_Subscribe

6a

.

N

dccf

_

DataManagement_

Subscribe

2b

.

N

adrf

_

DataManagement_Storage

Subscription

Request

6b

.

N

nwdaf

_

DataManagement

_Subscribe/Nnwdaf_AnalyticsSubscription_Subscribe

9a

.

N

adrf

_

DataManagement_

StorageSubscriptionRemoval request

9b

.

N

adrf

_

DataManagement_

StorageSubscriptionRemoval request

10b

.

N

nwdaf

_

DataManagement

_UnSubscribe/Nnwdaf_AnalyticsSubscription_Unsubscribe

10a

.

N

dccf

_

DataManagement

_UnSubscribe

1d. DCCF Determines Data

and/or

Analytics

is to be stored in ADRF

1c. NWDAF Determines Data

and/or

Analytics

is to be stored in ADRF

7. Notifications Stored in ADRF

8b. DCCF

Determines to stop

storing data in the ADRF

8a. NWDAF Determines to stop

storing data in the ADRF

1b

.

N

nwdaf

_

DataManagement

_Subscribe

3

.

ADRF Determines Data and/or Analytics

are already stored or being stored

4

a

.

N

adrf

_

DataManagement_Storage

Subscription

Request

Response

4

b

.

N

adrf

_

DataManagement_Storage

Subscription

Request

Response

0a

–

_

Configure Storage Policy

0b

–

_

Configure Storage Policy

0c

–

_

Configure Storage Policy

3a NWDAF Determines Storage Approach

3b DCCF Determines Storage Approach

3c ADRF Determines Storage Approach

11. ADRF Determines Data is to

be deleted from ADRF

19b. DCCF Determines Data is

to be deleted from ADRF

19a. NWDAF Determines Data

is to be deleted from ADRF

5a

.

N

dccf

_

DataManagement

_Subscribe response

5b

.

N

nwdaf

_

DataManagement

_Subscribe response

26. ADRF Deletes Stored Data

12

.

N

adrf_DataManagement_Retrieval Notify

(conditional) if ADRF is Managing Storage Approach

15a

.

N

nwdaf

_

_DataManagement

_Notify

15b

.

N

dccf

_

DataManagement

_Notify

16a

.

N

wdaf

_

DataManagement

_Notify  response

16b

.

N

dccf

_

DataManagement

_Notify response

14. DCCF or NWDAF retrieves Stored Data prior to Deletion from ADRF

18. Consumer Retrieves Stored Data prior to Deletion from ADRF

(conditional)

if Storage Approach is based on Polices Provisioned on DCCF or NWDAF

13

.

N

adrf

_

DataManagement

_Retreival_Notify response

(conditional)

if Storage Approach is based on request from Consumer

17

.

N

adrf

_

DataManagement

_Retreival_Notify response

(conditional) if DCCF or NWDAF is Managing Storage Approach

22a

.

N

wdaf

_

DataManagement

_Notify

22b

.

N

dccf

_

DataManagement

_Notify

23a

.

N

wdaf

_

DataManagement

_Notify  response

23b

.

N

dccf

_

DataManagement

_Notify response

20. DCCF or NWDAF retreives Stored Data prior to Deletion from ADRF

24. Consumer Retreives Stored Data prior to Deletion from ADRF

(conditional)

if Storage Approach is based on Polices Provisioned on DCCF or NWDAF

(conditional)

if Storage Approach is based on request from Consumer

25

.

N

adrf

_

DataManagement

_Delete

21

.

N

adrf

_

DataManagement

_Delete

(conditional) if

NWDAF or DCCF Determines to stop storing data

Figure 6.2B.3-1: Historical Data and Analytics Storage via Notifications

0a-c	NWDAF, DCCF or ADRF are configured with default operator storage policies as described in clause 5B.1.

1a-d.	Based on provisioning or based on reception of a DataManagement subscription request (e.g. see clause 6.2.6.3.2), the DCCF or the NWDAF determines that notifications are to be stored in an ADRF. The subscription request may contain Storage Handling information with a requested ADRF storage lifetime and a request to be notified before data are deleted from the ADRF.

2a-b.	The DCCF or the NWDAF determines the ADRF where data and/or analytics needs to be stored and requests that the ADRF subscribes to receive notifications. The determination may be made based on configuration or information supplied by the data consumer as described in clauses 6.1.4 and 6.2.6.3. The request to the ADRF specifies the data and/or analytics to which the ADRF will subscribe by invoking the Nadrf_DataManagement_StorageSubscriptionRequest service operation. The request may include the DataSetTag attribute which the data records are to be associated to when stored by ADRF. If the Storage Policy is not configured on the NWDAF or DCCF, the NWDAF or DCCF sends the Storage Handling Request to the ADRF. The NWDAF or DCCF may provide notification endpoint information to the ADRF for use by the ADRF to send notifications (implicit subscription) alerting the DCCF or NWDAF that data are about to be deleted (see step 12).

3.	[Optional] The ADRF may, based on implementation, determine whether the same data and/or analytics is already stored or being stored, based on the information sent in step 2 by the consumer. If the DataSetTag attribute is included for data and/or analytics already stored or being stored, then ADRF associates the data records with such DataSetTag.

3a-c.	[Optional] Based on Storage Handling information and Storage Policy, the ADRF, DCCF or NWDAF determines the Storage Approach (lifetime for storing data and whether consumer is notified prior to data deletion).

4.	[Optional] If the data and/or analytics is already stored and/or being stored in the ADRF, the ADRF sends Nadrf_DataManagement_StorageSubscriptionRequest Response message to the consumer indicating that data and/or analytics is stored. The ADRF includes the Storage Approach if determined in step 3c.

5a-b.	The DCCF or NWDAF sends a subscription response to the Data or Analytics Consumer. The response may contain the Storage Approach determined by the ADRF, NWDAF or DCCF.

6a-b.	ADRF subscribes to the DCCF or the NWDAF to receive notifications, providing its notification endpoint address and a notification correlation ID.

7.	The DCCF, the MFAF or the NWDAF sends Analytics or Data notifications containing the notification correlation ID provided by the ADRF to ADRF notification endpoint address. The Analytics or Data notifications shall contain timestamp. The ADRF stores the notifications.

Conditional on DCCF or NWDAF determining to stop storing data or analytics

8a-b.	The DCCF or the NWDAF determines to stop storing notifications in the ADRF.

9a-b.	The DCCF or the NWDAF requests that the ADRF unsubscribes to receive notifications.

10a-b.	The ADRF sends a request to the DCCF or the NWDAF to unsubscribe to data notifications.

The NWDAF may interact with the Data Source and the DCCF may interact with the Data Source and/or MFAF. Delivery of notifications from the DCCF/MFAF or NWDAF to the ADRF are subsequently halted.

Conditional on ADRF Managing the Storage Approach

11.	The ADRF determines that the lifetime of the stored data or analytics has expired (according to the Storage Approach).

12.	If indicated by the Storage Approach, the ADRF sends a notification alerting the DCCF or NWDAF that data are about to be deleted.

NOTE:	This is an implicit subscription.

Conditional if the Storage Appoach is based on Default Operator Policies provisioned on the NWDAF or DCCF (see step 3a-c and clause 5B.1)

13.	The DCCF or NWDAF indicate in the response to the ADRF whether they will retrieve the Data or Analytics

14.	The DCCF or NWDAF may retrieve the Data or Analytics from the ADRF

Conditional if the Storage Appoach is based on a Storage Handling Request received from the Data or Analytics Consumer (see step 3a-c and clause 5B.1)

15a-b. The NWDAF or DCCF sends a notification to the Data or Analytics Consumer alerting it that the data or analytics are about to be deleted.

16a-b. The Data or Analytics Consumer indicates in the response to the NWDAF or DCCF whether it will retrieve the Data or Analytics

17. The DCCF or NWDAF indicate in the response to the ADRF whether the consumer will retrieve the Data or Analytics

18.	The Data or Analytics Consumer retrieves the stored data or analytics

Conditional on DCCF or NWDAF Managing the Storage Approach

19a-b.	The NWDAF or DCCF determines that the lifetime of the stored data has expired (according to the Storage Approach).

Conditional if the Storage Appoach is based on Default Operator Policies provisioned on the NWDAF or DCCF (see step 3a-c and clause 5B.1)

20.	The NWDAF or DCCF optionally retrieve the data or analytics from the ADRF

21.	The NWDAF or DCCF request that the data or analytics be deleted from the ADRF

Conditional if the Storage Appoach is based on a Storage Handling Request received from the Data or Analytics Consumer (see step 3a-c and clause 5B.1)

22a-b.	The NWDAF or DCCF sends a notification to the Data or Analytics Consumer alerting it that the data or analytics are about to be deleted.

23a-b.	The Data or Analytics Consumer indicates in the response to the NWDAF or DCCF whether it will retrieve the Data or Analytics

24.	The Data or Analytics Consumer retrieves the stored data or analytics

25.	The NWDAF or DCCF request that the data or analytics be deleted from the ADRF

26.	The ADRF deletes the data or analytics if:

-	the Storage Approach in the ADRF indicates alerting the consumer is not required prior to data or analytics deletion;

-	in Steps 13 or 17 the DCCF or NWDAF indicated data or analytics will not be retrieved prior to deletion;

-	data or analytics retrieval in steps 14 or 18 has completed or

-	A request to delete data or analytics was received in steps 21 or 25.

If the ADRF received a response from the NWDAF or DCCF in steps 14 or 17 indicating data or analytics will be retrieved but retrieval is not initiated before an adequate fixed time has elapsed, the ADRF may autonomously delete the data or analytics.

* * * Next Change * * *

9.1	General

Table 9.1-1 shows the MFAF services and MFAF service operations.

Table 9.1-1: NF services provided by MFAF

| Service Name | Service Operations | Operation Semantics | Example Consumer(s) |
| --- | --- | --- | --- |
| Nmfaf_3daDataManagement | Configure | Request / Response | DCCF, NWDAF |
|  | Deconfigure | Request / Response | DCCF, NWDAF |
| Nmfaf_3caDataManagement | Notify | Subscribe / Notify | NWDAF, PCF, NSSF, AMF, SMF, NEF, AF, ADRF, OAM,CEF, LMF |
|  | Fetch | Request / Response | NWDAF, PCF, NSSF, AMF, SMF, NEF, AF, ADRF, OAM,CEF, LMF |

* * * End of Changes * * *