5G Media Action Group (5G-MAG)

liaison statement to 3GPP TSG SA WG4 (3GPP SA4)

| Title of the Liaison Statement: | Clarifications and fixes to 3gpp ts 26.512 |
| --- | --- |
| Objective: | For information and comments |
| Origin: | 5G Media Action Group |
| Contacts: | Jordi J. Giménez, Head of Technology 5G-MAG |
| Date: | October 31, 2022 |

5G-MAG has kicked-off the implementation roadmap of the 5G-MAG Reference Tools for 2023: https://www.5g-mag.com/target2023. A key requirement demanded by several use cases is the implementation of 5G Downlink Media Streaming Architecture components, with the initial focus on AF, AS and the 5GMSd client. 5G-MAG is maintaining a GitHub repository where issues related to standards are raised and discussed within the community with the aim to speed-up their resolution (https://github.com/5G-MAG/Standards). We would kindly like to inform 3GPP SA4 about the status of this work, and to invite 3GPP members to discuss the related CRs below.

5G-MAG's desire is also to set up a regular dialogue with SA4 domain experts. A first exchange with SA4 experts was held on 29th September. We would like to thank the SA4 delegates for attending. Our proposal is to convene a meeting at the mid-point between SA4's main Working Group meetings (four/five per year). Therefore, we would like to propose a date for the next one early in the New Year, e.g. Thursday, 12th January at 16:30–17:30 CET.

Resolution of issues found in 3GPP TS 26.512

In the process of implementing AF and AS a series of issues/incorrections related to 3GPP TS 26.512 have been identified.

| Justificaiton of the proposed changes | Link |
| --- | --- |
| Cardinality of DistributionConfigurations.domainNameAlias is ambiguous in Content Hosting Configuration | #16 |
| Cardinality of distributionConfigurations.canonicalDomainName does not match described usage in Create Content Hosting Configuration requests | #17 |
| Cardinality of ingestConfiguration.path is incorrect in Content Hosting Configuration | #18 |
| Use of path part of URL in IngestConfiguration.entryPoint for pull ingest | #19 |
| Inconsistency for parameter name requestPathPattern | #27 |
| ContentHostingConfiguration.distributionConfigurations.supplementaryDistributionNetworks inconsistencies. | #28 |
| DASH Media Player – APIs and Functions - M8 interface between application and provider | #29 |

Potential resolution to the aforementioned issues is provided in:

S4-221250 - [5GMS3] Rel-16 clarifications and corrections (CR0027 to TS 26.512)

S4-221251 - [5GMS3] Rel-17 clarifications and corrections (CR0028 to TS 26.512)

About the 5G Media Action Group and 5G-MAG Reference Tools

The 5G Media Action Group (5G-MAG) is a cross-industry association bridging the media and ICT industries. 5G-MAG provides a framework for members to drive collaboration on solutions for the connected media world, from content creation to consumption. In its role as 3GPP Market Representation Partner, 5G-MAG brings into 3GPP a consensus view of market requirements for the implementation and deployment of 5G technologies for media applications. One of the core efforts coordinated by 5G-MAG is the 5G-MAG Reference Tools development programme which develops common open software reference tools to support the implementation and interoperability of 5G Media technologies. More information at: https://developer.5g-mag.com

For technical queries please contact:

Jordi J. Giménez

Head of Technology, 5G-MAG Project Office

Email: gimenez@5g-mag.com

5G-MAG Association

17A Ancienne Route

CH-1218 Grand Saconnex

Geneva (Switzerland)

Phone: +41-22-717 2707

www.5g-mag.com