3GPP TSG-CT WG4 Meeting #128	C4-251487

Wuhan, China; 07th – 11th April 2025	was C4-251370, C4-251157

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 29.572 | CR | 0334 | rev | 2 | Current version: | 19.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Abnormal Handling for LMF Data Exposure Service |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson, Nokia, CEWit, vivo, Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | CT4 |  |  |  |  |  |  |  |  |  |
| Work item code: | AIML_CN |  |  |  |  |  | Date: |  |  | 2025-04-10 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | This CR introduces the following abnormal handling for LMF Data Exposure Service: 1. SA2 has agreed CR0685 of TS 23.273 has required that the LMF may send a cause code to the NWDAF containing MTLF when the requested number of sampling data cannot be met: ### 6.22.4 Input data collection by NWDAF for AI/ML positioning ML model training or ML model performance monitoring … 5. The LMF sends the collected data samples (i.e. location measurement data from PRUs/UEs and/or the NG-RAN, the corresponding ground truth data (i.e. location of PRUs or UEs) and the quality indicator of the ground truth data) to the NWDAF by invoking Nlmf_DataExposure_Notify service operation. The LMF may send a cause code to the NWDAF when the requested number of data samples cannot be met. The LMF may also send the location estimation of PRU(s)/UE(s) calculated in step 4 (if applicable). Then the NWDAF trains the ML model or performs ML model performance monitoring based on the data samples received from the LMF. 2. When the subscription is for performance monitoring and ML model identifier is provided in the subscription, but the ML model is not supported by the LMF, the LMF can return an application error indicating unsupported ML model. |  |  |  |  |  |  |  |  |
| Summary of change: |  | 1/ Define cause code for requested number of sampling data cannot be met 2/ Define application error for supported ML model. 3/ Update OpenAPI accordingly. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Missed abnormal scenario handling for LMF Data Exposure service. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.4.2.4, 6.3.3.2.3.1, 6.3.6.1, 6.3.6.2.4, 6.3.6.3.x(New), A.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This CR introduces backward compatible feature to the following APIs: TS29520_Nnwdaf_AnalyticsInfo.yaml TS29520_Nnwdaf_DataManagement.yaml TS29520_Nnwdaf_RoamingData.yaml TS29572_Nlmf_DataExposure.yaml TS29574_Ndccf_ContextManagement.yaml TS29574_Ndccf_DataManagement.yaml TS29575_Nadrf_DataManagement.yaml TS29576_Nmfaf_3caDataManagement.yaml TS29576_Nmfaf_ContextManagement.yaml |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Rev1: 1/ Co-source companies added 2/ Update other comments 3/ Add Editor's Note to capture that more cause codes may be defined in future. Rev2: Un-number the new clause number |  |  |  |  |  |  |  |  |

* * * First Change * * * *

#### 5.4.2.4	LMF Data Exposure Notification

This procedure allows a consumer NF to modify the notification about the input data used for ML model training or ML model performance monitoring for LMF-based AI/ML Positioning.

Figure 5.4.2.4-1: Notification for LMF data exposure

1.	LMF shall send a POST request to the NF Service Consumer with the information on Notification target address. The request body shall include sampling data and notification correlation ID, if received.

The LMF may provide a cause code in the report to indicate additional information to the NF consumer, e.g. when the requested number of sampling data cannot be met within the Area of Interest during the request time window.

2a.	On success, "204 No content" shall be returned by the NF Service Consumer.

2b.	On failure or redirection, one of the appropriate HTTP status codes listed in Table 6.3.5.2.3-2 shall be returned. For a 4xx/5xx response, the message body should contain a ProblemDetails structure indicating appropriate additional error information.

* * * Next Change * * * *

6.3.3.2.3.1	POST

This method shall support the URI query parameters specified in table 6.3.3.2.3.1-1.

Table 6.3.3.2.3.1-1: URI query parameters supported by the POST method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

This method shall support the request data structures specified in table 6.3.3.2.3.1-2 and the response data structures and response codes specified in table 6.3.3.2.3.1-3.

Table 6.3.3.2.3.1-2: Data structures supported by the POST Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| LmfDataExposureSubscription | M | 1 | LMF Data Exposure Subscription to be created |

Table 6.3.3.2.3.1-3: Data structures supported by the POST Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| LmfDataExposureSubscription | M | 1 | 201 Created | Represents successful creation of an LMF Subscription |
| RedirectResponse | O | 0..1 | 307 Temporary Redirect | Temporary redirection. |
| RedirectResponse | O | 0..1 | 308 Permanent Redirect | Permanent redirection. |
| ProblemDetails | O | 0..1 | 403 Forbidden | The "cause" attribute may be used to indicate the following application error: - UNSUPPORTED_ML_MODEL See table 6.3.7.x-1 for the description of these errors. |
| NOTE: The mandatory HTTP error status code for the POST method listed in Table 5.2.7.1-1 of 3GPP TS 29.500 [4] also apply, with response body containing an object of ProblemDetails data type (see clause 5.2.7 of 3GPP TS 29.500 [4]). |  |  |  |  |

* * * Next Change * * * *

#### 6.3.6.1	General

This clause specifies the application data model supported by the API.

Table 6.3.6.1-1 specifies the data types defined for the Nlmf_DataExposure service-based interface protocol.

Table 6.3.6.1-1: Nlmf_DataExposure specific Data Types

| Data type | Clause defined | Description |
| --- | --- | --- |
| LmfDataExposureSubscription | 6.3.6.2.2 | Represents an individual data collection subscription resource on LMF |
| LmfDataExposureNotification | 6.3.6.2.4 | Notify within an LMF exposure for data collection notification request. |
| LmfDataExposureReport | 6.3.6.2.5 | Represents the data collection exposure report. |
| LmfDataExposureCause | 6.3.6.3.x | Cause code for LMF Data Exposure Service |

Table 6.3.6.1-2 specifies data types re-used by the Nlmf_DataExposure service based interface protocol from other specifications, including a reference to their respective specifications and when needed, a short description of their use within the Nlmf_DataExposure service based interface.

Table 6.3.6.1-2: Nlmf_DataExposure re-used Data Types

| Data type | Reference | Description |
| --- | --- | --- |
| PresenceInfo | 3GPP TS 29.571 [8] | Area of Interest |
| Uri | 3GPP TS 29.571 [8] | URI |
| RedirectResponse | 3GPP TS 29.571 [8] | Redirect Response |
| DateTime | 3GPP TS 29.571 [8] | Date time |
| PatchItem | 3GPP TS 29.571 [8] | Patch item |
| TimeWindow | 3GPP TS 29.122 [29] | Time window |

##### 6.3.6.2.4	Type: LmfDataExposureReport

Table 6.3.6.2.4-1: Definition of type LmfDataExposureReport

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| timeStamp | DateTime | M | 1 | This IE shall contain the UTC time. |  |
| samplingData | FFS | FFS | FFS | FFS |  |
| cause | LmfDataExposureCause | O | 0..1 | When present, this IE shall indicate the cause code for the report. |  |

* * * Next Change * * * *

##### 6.3.6.3.x	Enumeration: LmfDataExposureCause

The enumeration LmfDataExposureCause represents the cause code for the LMF Data Exposure reports.

Table 6.3.6.3.x-1: Enumeration LmfDataExposureCause

| Enumeration value | Description |
| --- | --- |
| "NOT_ENOUGH_SAMPLES" | Indicates that the requested number of sampling data cannot be met within the Area of Interest during the requested time window. |

Editor's Note:	Whether more cause codes to be defined is FFS.

* * * Next Change * * * *

#### 6.3.7.x	Application Errors

The common application errors defined in Table 5.2.7.2-1 in 3GPP TS 29.500 [4] may be used for the Nlmf_DataExposure service. The application errors defined for the Nlmf_DataExposure service are listed in Table 6.3.7.x-1.

Table 6.3.7.x-1: Application errors

| Application Error | HTTP status code | Description |
| --- | --- | --- |
| UNSUPPORTED_ML_MODEL | 403 Forbidden | The requested ML Model for data collection is not supported by the LMF. |

* * * Next Change * * * *

# A.4	Nlmf_DataExposure API

**************** Text Skippef for Clarity *****************

schemas:

#

# STRUCTURED TYPES

#

LmfDataExposureSubscription:

description: Describes subscription operation of LMF data exposure

type: object

properties:

notificationUri:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uri'

notifyCorrelationId:

$ref: 'TS29572_Nlmf_Location.yaml#/components/schemas/CorrelationID'

aoi:

$ref: 'TS29571_CommonData.yaml#/components/schemas/PresenceInfo'

timeWindow:

$ref: 'TS29122_CommonData.yaml#/components/schemas/TimeWindow'

numOfSamples:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uinteger'

required:

- notificationUri

- notifyCorrelationId

- aoi

LmfDataExposureNotification:

description: Describes Notification for LMF data collection

properties:

notifyCorrelationId:

$ref: 'TS29572_Nlmf_Location.yaml#/components/schemas/CorrelationID'

reports:

type: array

items:

$ref: '#/components/schemas/LmfDataExposureReport'

minItems: 1

required:

- notifyCorrelationId

- reports

LmfDataExposureReport:

description: Describes an LMF data collection exposure report

properties:

timestamp:

$ref: 'TS29571_CommonData.yaml#/components/schemas/DateTime'

cause:

$ref: '#/components/schemas/LmfDataExposureCause'

required:

- timestamp

LmfDataExposureCause:

description: The cause code for LMF Data Exposure Report.

anyOf:

- type: string

enum:

- NOT_ENOUGH_SAMPLES

- type: string

**************** Text Skippef for Clarity *****************

* * * End of Changes * * * *