3GPP TSG-CT Meeting #109	CP-252210

# Beijing, P.R. China; 15th – 16th September 2025	(revision of C4-253247)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 29.505 | CR | 0539 | rev | 1 | Current version: | 19.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | PEI change timestamp in PeiUpdateInfo |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Work item code: | SBIProtoc19 |  |  |  |  |  | Date: |  |  | 2025-08-04 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The event type "CHANGE_OF_SUPI_PEI_ASSOCIATION" can be requested as an immediate report, to be delivered as part of the subscription response. In that case, UDM shall return the last known PEI (stored at UDR); in order to make this information more useful to the consumer, it should include the timestamp when the PEI was actually changed (which might have occurred long ago). The object where this information is stored in UDR ("PeiUpdateInfo") does not currently allow to store a timestamp. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Extend the data structure used to store and retrieve PEI update information (resource "PeiInfo") with the following optional attributes: - Timestamp with the last time when the PEI (any PEI type) was updated - Timestamp with the last time when the IMEI was updated (this is useful for the case where the consumer subscribes to this monitoring event type using the association type "IMEI_CHANGE", indicating that it is not interested in PEI changes that only correspond to SV changes) |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | It is not possible for UDM to know the point in time when the PEI change occurred (and report it accordingly in immediate monitoring reports). |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.4.1, 5.4.2.xx (new), 5.4.2.yy (new), A.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This CR introduces backwards-compatible new features with impacts on the following APIs: - TS29504_Nudr_DR.yaml |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Rev. 1 (to CT#109 plenary): - In the OpenAPI Annex, correct the reference of PeiUpdateInfo, which is defined in Nudm_UECM, rather than Nudm_SDM |  |  |  |  |  |  |  |  |

# * * * First Change * * * *

### 5.4.1	General

This clause specifies the application data model supported by the API.

Table 5.4.1-1 specifies the data types defined for the Nudr service based interface protocol.

# Table 5.4.1-1: Nudr Subscriber Data specific Data Types

| Data type | Clause defined | Description |
| --- | --- | --- |
| AuthenticationSubscription | 5.4.2.2 | A UE's authentication data |
| OperatorSpecificDataContainer | 5.4.2.3 | Container for operator specific data |
| SmfRegList | 5.4.2.4 | The list of all the SMF registrations of a UE |
| SubscriptionDataSubscriptions | 5.4.2.5 | A subscription to notifications. |
| DataChangeNotify | 5.4.2.6 | Container for data which have changed and notification was requested when changed. |
| IdentityData | 5.4.2.7 | Identity data corresponds to the provided ueId |
| ProvisionedDataSets | 5.4.2.8 | Contains the provisioned data sets |
| SorData | 5.4.2.9 | Used to store the status of the latest SOR data update |
| UpuData | 5.4.2.9A | Used to store the status of the latest UPU data update |
| NssaiAckData | 5.4.2.9B | Used to store the status of the latest NSSAI data update |
| CagAckData | 5.4.2.9C | Used to store the status of the latest CAG data update |
| AmfSubscriptionInfo | 5.4.2.19 | Information the UDR stores and retrieves related to active subscriptions at the AMF(s) |
| EeProfileData | 5.4.2.20 | Event Exposure Profile Data |
| ContextDataSets | 5.4.2.22 | Contains the context data sets |
| SequenceNumber | 5.4.2.23 | Contains the SQN |
| MessageWaitingData | 5.4.2.24 | Message Waiting Data list |
| SmscData | 5.4.2.25 | Addresses of SM-Service Center entities with SMS waiting to be delivered to the UE |
| SmfSubscriptionInfo | 5.4.2.26 | Information the UDR stores and retrieves related to active subscriptions at the SMF(s) |
| SmfSubscriptionItem | 5.4.2.27 | Contains info about a single SMF event subscription |
| MtcProvider | 5.4.2.28 | MTC provider information |
| HssSubscriptionInfo | 5.4.2.29 | Information the UDR stores related to active subscriptions at the HSS(s) |
| HssSubscriptionItem | 5.4.2.30 | Contains info about a single HSS event subscription |
| EeGroupProfileData | 5.4.2.31 | The Event Exposure Profile Data for a group of UEs |
| Pp5gVnGroupProfileData | 5.4.2.32 | The Parameter Provision Profile Data for 5G VN groups |
| PpProfileData | 5.4.2.33 | The Parameter Provision Profile Data |
| AllowedMtcProviderInfo | 5.4.2.34 | Allowed MTC Providers or AFs Information to provision parameters for UE. |
| GroupIdentifiers | 5.4.2.35 | External or Internal Group Identifier with a list of group members |
| AuthorizationData | 5.4.2.36 | Authorization Information |
| NiddAuthorizationInfo | 5.4.2.37 | Information the UDR stores and retrieves related to active NIDD authorizations |
| PpDataEntryList | 5.4.2.38 | A list of the Provisioned Parameters entries. |
| UeSubscribedDataSets | 5.4.2.39 | Contains the subscribed data sets of a UE |
| ServiceSpecificAuthorizationInfo | 5.4.2.40 | Information the UDR stores and retrieves related to active Service Specific authorizations |
| NfIdentifier | 5.4.2.41 | Network Function Identifier |
| EeSubscriptionExt | 5.4.2.42 | Extended Event Exposure Subscription |
| AdditionalEeSubsInfo | 5.4.2.43 | Additional EE-Subscription Information |
| ImmediateReport | 5.4.2.44 | Immediate Report |
| Pp5gMbsGroupProfileData | 5.4.2.45 | The Parameter Provision Profile Data for 5G MBS groups |
| AllowedMbsInfo | 5.4.2.46 | Allowed AFs Information to provision parameters for UE. |
| UeUpdConfData | 5.4.2.47 | UE Update Confirmation Data |
| AdditionalDataRef | 5.4.2.48 | Additional Data Reference |
| PeiUpdateInfo | 5.4.2.xx | PEI update information |
| PeiUpdateInfoExt | 5.4.2.yy | Additional attributes to the object PeiUpdateInfo |
| VarPlmnId | 5.4.3.2 | String identifying a PLMN ID or an SNPN ID. |
| AuthMethod | 5.4.3.3 | Contains the Authentication Method |
| ProvisionedDataSetName | 5.4.3.4 | The name of data set |
| ContextDataSetName | 5.4.3.6 | The name of context data set |
| SqnScheme | 5.4.3.7 | Scheme for generation of Sequence Numbers |
| Sign | 5.4.3.8 | Sign of the DIF value |
| UeUpdateStatus | 5.4.3.9 | Status of the procedure |
| PpDataType | 5.4.3.10 | The data type of the Provisioned Parameters |
| UeSubscribedDataSetName | 5.4.3.11 | The name of subscribed data set |

Table 5.4.1-2 specifies data types re-used by the Nudr service based interface protocol from other specifications, including a reference to their respective specifications and when needed, a short description of their use within the Nudr service based interface.

# Table 5.4.1-2: Nudr re-used Data Types

| Data type | Reference | Comments |
| --- | --- | --- |
| AccessAndMobilitySubscriptionData | 3GPP TS 29.503 [6] | Access and Mobility Subscription Data |
| SmfSelectionSubscriptionData | 3GPP TS 29.503 [6] | SMF Selection Subscription Data |
| SmSubsData | 3GPP TS 29.503 [6] | Session Management Subscription Data |
| Amf3GppAccessRegistration | 3GPP TS 29.503 [6] | The complete set of information relevant to the AMF where the UE has registered via 3GPP access. |
| AmfNon3GppAccessRegistration | 3GPP TS 29.503 [6] | The complete set of information relevant to the AMF where the UE has registered via non 3GPP access. |
| SmfRegistration | 3GPP TS 29.503 [6] | The complete set of information relevant to an SMF serving the UE |
| SmsfRegistration | 3GPP TS 29.503 [6] | The complete set of information relevant to the SMSF serving the UE. |
| SmsManagementSubscriptionData | 3GPP TS 29.503 [6] | SMS Management Subscription Data |
| SupportedFeatures | 3GPP TS 29.571 [3] | Used to negotiate the applicability of the optional features |
| ProblemDetails | 3GPP TS 29.571 [3] | Detailed information about the status code. |
| PpData | 3GPP TS 29.503 [6] | Parameter Provision Data |
| PpDataEntry | 3GPP TS 29.503 [6] | Provisioned Parameter Data Entry |
| SmsSubscriptionData | 3GPP TS 29.503 [6] | SMS Subscription Data |
| PatchItem | 3GPP TS 29.571 [3] | Data structure used for JSON patch. |
| SdmSubscription | 3GPP TS 29.503 [6] | A subscription to notifications |
| EeSubscription | 3GPP TS 29.503 [6] | A subscription to Notifications of event exposure |
| Uri | 3GPP TS 29.571 [3] | URI |
| TraceData | 3GPP TS 29.571 [3] | Trace Data |
| Dnn | 3GPP TS 29.571 [3] | Data Network Name |
| Snssai | 3GPP TS 29.571 [3] | Single NSSAI |
| VarUeId | 3GPP TS 29.571 [3] | String represents the SUPI or GPSI. |
| NfInstanceId | 3GPP TS 29.571 [3] | NF Instance Identifier |
| NotifyItem | 3GPP TS 29.571 [3] | Common data type used for data change notification. |
| OdbData | 3GPP TS 29.571 [3] | Operator Determined Barring Data |
| EventType | 3GPP TS 29.503 [6] | Event Type |
| ExtGroupId | 3GPP TS 29.503 [6] | External Group Identifier |
| PduSessionId | 3GPP TS 29.571 [3] | PDU Session ID which identifies a PDU session |
| DateTime | 3GPP TS 29.571 [3] | Date Time |
| UpuMac | 3GPP TS 29.509 [15] | MAC value for protecting UPU procedure (UPU-MAC-IAUSF and UPU-MAC-IUE) |
| 5GVnGroupConfiguration | 3GPP TS 29.503 [6] | 5G VN Group Configuration data |
| PatchResult | 3GPP TS 29.571 [3] | Patch Result |
| SupportedFeatures | 3GPP TS 29.571 [3] | Supported Features |
| AppPortId | 3GPP TS 29.503 [6] | Application Port Identifier |
| LcsPrivacyData | 3GPP TS 29.503 [6] | LCS Privacy Subscription Data |
| LcsMoData | 3GPP TS 29.503 [6] | LCS Mobile Originated Subscription Data |
| MtcProviderInformation | 3GPP TS 29.571 [3] | An ID uniquely identifying MTC provider information |
| EnhancedCoverageRestrictionData | 3GPP TS 29.503 [6] | Enhanced Coverage Restriction Data |
| LocationInfo | 3GPP TS 29.503 [6] | Location Information |
| V2xSubscriptionData | 3GPP TS 29.503 [6] | V2X Subscription Data |
| E164Number | 3GPP TS 29.503 [6] | E164Number |
| NetworkNodeDiameterAddress | 3GPP TS 29.503 [6] | Diameter Address of an Network Node |
| IpSmGwRegistration | 3GPP TS 29.503 [6] | The complete set of information relevant to the IP-SM-GW where the UE has registered. |
| LcsBroadcastAssistanceTypesData | 3GPP TS 29.503 [6] | LCS Broadcast Assistance Data Types |
| ContextInfo | 3GPP TS 29.503 [6] | Contains the headers received by an NF. |
| NfGroupId | 3GPP TS 29.571 [3] | NF Group ID |
| ProseSubscriptionData | 3GPP TS 29.503 [6] | ProSe Service Subscription Data |
| PlmnId | 3GPP TS 29.571 [3] | PLMN ID |
| AppPortId | 3GPP TS 29.503 [6] | Application Port Id |
| AuthorizationInfo | 3GPP TS 29.503 [6] | Contains Authorization Information |
| MbsSubscriptionData | 3GPP TS 29.503 [6] | Contains 5MBS Subscription Data |
| ServiceType | 3GPP TS 29.503 [6] | Service Type |
| RoamingInfoUpdate | 3GPP TS 29.503 [6] | Roaming Information in EPC domain |
| UcSubscriptionData | 3GPP TS 29.503 [6] | User Consent Data |
| UcPurpose | 3GPP TS 29.503 [6] | User Consent Purpose |
| PeiUpdateInfo | 3GPP TS 29.503 [6] | PEI Update Information |
| SharedData | 3GPP TS 29.503 [6] | Shared Data |
| TimeSyncSubscriptionData | 3GPP TS 29.503 [6] | Contains Time Synchronization Subscription Data |
| LcsSubscriptionData | 3GPP TS 29.503 [6] | UE’s other LCS Subscription Data |
| RangingSlPosSubscriptionData | 3GPP TS 29.503 [6] | Ranging and Sidelink Positioning Service Subscription Data |
| SmfRegistrationInfo | 3GPP TS 29.503 [6] | SMF Registration Information |
| A2xSubscriptionData | 3GPP TS 29.503 [6] | A2X Subscription Data |
| RangingSlPrivacyData | 3GPP TS 29.503 [6] | Ranging and Sidelink Positioning Privacy Subscription Data |

# * * * Next Change * * *

#### 5.4.2.xx	Type: PeiUpdateInfo

# Table 5.4.2.xx-1: Definition of type PeiUpdateInfo as a list of "to be combined data types"

| Data type | Cardinality | Description |
| --- | --- | --- |
| PeiUpdateInfo | 1 | see 3GPP TS 29.503 [6] |
| PeiUpdateInfoExt | 0..1 |  |

#### 5.4.2.yy	Type: PeiUpdateInfoExt

# Table 5.4.2.yy-1: Definition of type PeiUpdateInfoExt

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| lastPeiChangeTimestamp | DateTime | O | 0..1 | Timestamp when the PEI of the UE was last changed. |
| lastImeiChangeTimestamp | DateTime | O | 0..1 | Timestamp when the IMEI of the UE was last changed. |

# * * * Next Change * * *

# A.2	Nudr_DataRepository API for Subscription Data

(... text not shown for clarity ...)

/subscription-data/{ueId}/context-data/pei-info:

put:

summary: Update the PEI Information of the 5GC/EPC domains

operationId: CreateOrUpdatePeiInformation

tags:

- Update the PEI Information of the 5GC/EPC domains (Document)

security:

- {}

- oAuth2ClientCredentials:

- nudr-dr

- oAuth2ClientCredentials:

- nudr-dr

- nudr-dr:subscription-data

parameters:

- name: ueId

in: path

description: UE id

required: true

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/VarUeId'

requestBody:

content:

application/json:

schema:

# $ref: '#/components/schemas/PeiUpdateInfo'

required: true

responses:

'201':

description: Created

content:

application/json:

schema:

$ref: '#/components/schemas/PeiUpdateInfo'

headers:

Location:

description: 'Contains the URI of the newly created resource, according to the structure: {apiRoot}/nudr-dr/<apiVersion>/subscription-data/{ueId}/context-data/pei-info'

required: true

schema:

type: string

'204':

description: Upon success, an empty response body shall be returned

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29571_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29571_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29571_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'502':

$ref: 'TS29571_CommonData.yaml#/components/responses/502'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

description: Unexpected error

get:

summary: Retrieves the PEI Information of the 5GC/EPC domains

operationId: QueryPeiInformation

tags:

- Retrieve the Roaming Information of the 5GC/EPC domains (Document)

security:

- {}

- oAuth2ClientCredentials:

- nudr-dr

- oAuth2ClientCredentials:

- nudr-dr

- nudr-dr:subscription-data

parameters:

- name: ueId

in: path

description: UE id

required: true

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/VarUeId'

responses:

'200':

description: Expected response to a valid request

content:

application/json:

schema:

$ref: '#/components/schemas/PeiUpdateInfo'

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'406':

$ref: 'TS29571_CommonData.yaml#/components/responses/406'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'502':

$ref: 'TS29571_CommonData.yaml#/components/responses/502'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

(... text not shown for clarity ...)

AllowedMbsInfo:

type: object

properties:

afId:

type: string

TraceDataOrSharedTraceDataId:

oneOf:

- $ref: 'TS29571_CommonData.yaml#/components/schemas/TraceData'

- $ref: 'TS29503_Nudm_SDM.yaml#/components/schemas/SharedDataId'

PeiUpdateInfo:

allOf:

- $ref: 'TS29503_Nudm_UECM.yaml#/components/schemas/PeiUpdateInfo'

- $ref: '#/components/schemas/PeiUpdateInfoExt'

PeiUpdateInfoExt:

description: Additional attributes to the object PeiUpdateInfo

type: object

properties:

lastPeiChangeTimestamp:

$ref: 'TS29571_CommonData.yaml#/components/schemas/DateTime'

lastImeiChangeTimestamp:

$ref: 'TS29571_CommonData.yaml#/components/schemas/DateTime'

# * * * End of Changes * * * *